PART IV - KEYBOARD COMMANDS

It is intended that the operation of the paKet program should at all

times be intuitive and as easy to use as possible. However, you still

need to know about some of the special keys, details of which are given

below.

F1               Display Help Screen.

Display a help screen showing most of paKet's special keys and

their functions.

You may select any function by pressing the key shown on the Help

Screen - it is not necessary to remove the Help Window first. If

you do not want to use any of those functions at this time, press

<Esc> to return to normal communications.

This Help Screen is intended as a memory jogger. Full details are

included here in this Manual which may be viewed by pressing

<Alt-F1>.

Page 109

F2               Turn Log File on/off.

Pressing this key will switch the Log File on or off.

The Log File has three possible states:

- ON    When the log file is ON, the Status Window will display

the name of the file currently used as a log file and

all Communications Window activity will be written to

that log file.

If the Log File is not ON, its state will be either:

- OFF   The disk log facility is not currently active so

nothing is written to any log file.

or

- AUTO  (This will only occur if you set the AUTO LOG option ON).

In this state the disk log facility is OFF, but paKet

will automatically activate the disk log, using the

default file name, when a connection is established, and

will automatically close it upon disconnection.

The effect of this is the same as if you press <F2> on

connection and then press <F2> again on disconnection.

So, if the Log File is currently OFF (or AUTO), pressing <F2> will

turn it ON.

A Directory window will appear, together with a suggested file

name. The default file name will be today's date in the form

MMMDD.LOG, for example: NOV23.LOG but you may use any name you

like. Using the default name means you can easily keep a separate

log for each day's activities.

If the selected file already exists on the disk, it will be

preserved and incoming data will be appended to that file.

If, when you press <F2>, the Log File is currently ON, it will be

closed and further logging activity will cease. The Log status

will return to either OFF or AUTO, depending on the AUTO LOG

option.

Some operators prefer to use the log file for all captured data

including ASCII file transfers, and use their favourite editor to

extract desired messages, files, etc from the log file later.

Page 110

F3               Turn REMOTE Mode on/off

REMOTE Mode is discussed more fully in its own section in this

Manual, but briefly it is a facility that allows another (remote)

user to operate our paKet system's PMS or File Transfer systems

even if our station is unattended.

There are three ways to get this system into REMOTE Mode:

1. You can configure paKet to automatically switch into REMOTE

Mode when another station connects;

2. The other station could send the REMOTE Trigger code.

This code would normally be the default <Ctrl-]>, but is

configurable via the "Configuration - REMOTE Options"

Window;

3. You can activate our REMOTE Mode by pressing <F3>.

When paKet enters REMOTE Mode it will issue the REMOTE Menu to the

other station.

If this system is already in REMOTE Mode, there are three ways to

switch that Mode OFF:

1. The other station could enter B (Bye) to log off REMOTE Mode

and Disconnect the TNC session; or he may just disconnect.

2. The other station could enter T (Talk) to switch off REMOTE

Mode but retain the connection, continuing in normal

communications.

3. You could press <F3> to switch off REMOTE Mode returning to

normal communications (same effect as if the other station

sent "T").

Page 111

F4               Toggle Word Wrap, Justified and Single Line formats

Word Wrap:

paKet has a Word Wrap facility that operates on data entered in

the Type Ahead window. If a word does not entirely fit on the

current line, the whole word will be moved to the next line.

Word Wrapped text is much tidier for the receiver to read and I

expect this option to be the norm. Perhaps you might want to

turn it off where you are typing a long message such as a CTEXT

message and you don't want paKet to interfere when you reach

the end of the line.

The status of the Word Wrap toggle is shown to the right of the

Type Ahead header line.

Justified:

Justified text  is that  which contains  sufficient  additional

spaces to ensure the last word on each line ends exactly on the

right  hand  margin  column.   The effect  is  an aligned right

margin such  as  you  see  in  this  paragraph.   The technique

works much better with a graphic display and proportional fonts

where the  spacing can  be varied  in very small increments all

across  the line,  but with the text  display used  by paKet it

must be accomplished with space  characters inserted at various

points in the line.

paKet supports justified text in the Type Ahead Buffer so if

you like this format press <F4> until you see "Justified"

displayed in the Type Ahead header line.  This implies Word

Wrap too.

Single Line:

If Word Wrap or Justified is ON, you also have a Single Line

option.

With the Single line option OFF, the data you type will be held

in the Type Ahead buffer until you press the <Enter> key, at

which time the contents of the buffer will be transferred to

the Communications window and the data sent to the TNC.

If Single Line option is ON, the data will still be entered

into the Type Ahead buffer, but when the line is filled (i.e.

when the last word is wrapped to the next line) that full line

will be sent to the TNC.

The <F4> key cycles through the Word Wrap, Justified and Single

Line options if the Type Ahead window is displayed.

My personal preference?   I like to use Single Line mode with Word

Wrap.

Page 112

F5               Send an ASCII text file

With this option you can send the contents of an ASCII text file

to the TNC.

An example of this might be where you want to send a lengthy

message to someone, or if you want to send the same message to

more than one station, you could prepare a disk file with the

desired text before you connect to that station. Then when you are

ready to enter the message, just press <F5> to send the file.

Of course this option is also used to transfer a data file or

source program file to another station, provided the file contains

only ASCII text. While it is POSSIBLE to transmit a binary file

using this command, it is prone to problems and NOT recommended!

You should use one of the Binary File Transfer methods to send a

file that contains non-ASCII data.

When you press <F5>, a Directory Window will appear.  Select the

desired file by moving the highlight bar to the desired file and

pressing <Enter>.  You can enter the filename manually if you

prefer, by selecting that option at the top of the Directory

Window.

While the file is being sent an 'S' will appear in the Status

Window to the right of the Time display, along with a counter

showing the number of bytes actually sent to the TNC.

When the entire file has been transmitted, paKet will optionally

add a <Ctrl-Z> character indicating end-of-file to the other

station. If you do not want this end-of-file code added, you can

disable the option via the Configuration - File Transfer options.

It should be noted that if you have configured more than one

Communications Window, paKet will keep the data separate and will

send only data from THIS file to the station connected on THIS

window.  You can change to another window and press <F5> again to

send ANOTHER file to the TNC from THAT window. There is no limit

to the number of files open at any one time, provided you have

specified enough FILES in your CONFIG.SYS (refer to your DOS

manual for details of the FILES parameter).

Page 113

F6               Receive ASCII text file

When you press <F6>, paKet will ask you for the name of the file

you want to open.

From then, all data received in this Communications Window will be

written to this file.

If the other station sends an end-of-file character, <Ctrl-Z>, the

file will be closed. Or you can close it yourself by pressing <F6>

again.

While the file is open, an 'R' will appear in the Status Window to

the right of the Time display, along with a counter showing the

number of bytes actually received into this file so far.

It should be noted that if you have configured more than one

Communications Window, paKet will keep the data separate and will

write only data for THIS window to this file.  You can change to

another window and press <F6> again to create ANOTHER file for

data received into THAT window. There is no limit to the number of

files open at any one time, provided you have specified enough

FILES in your CONFIG.SYS (refer to your DOS manual for details of

the FILES parameter).

Page 114

F7               Send a Binary File using either pP or YAPP Protocol.

To use this feature the other station must be able to handle

either YAPP protocol or pP (paKet Protocol).

Normally the standard YAPP protocol will be used; the program will

automatically select and use the advanced paKet-Protocol if

necessary.  You do not have to concern yourself with the use of

the more advanced protocol; paKet will take care of that itself.

If you want to send a Binary File to a BayCom or Digicom system,

refer to the <Alt-F7> key. And if you prefer to send a Binary File

to the other station with no protocol at all (you will have to

synchronise it manually), refer to the RAW Binary mode with the

<Alt-F5> key.

When you wish to send (or "upload") a file to the other station,

first of all arrange for that station to be ready to receive the

file.  You do this by sending a "Yapp Upload" command to the other

station.  For example, if you are connected to a BBS which is

waiting for a command you type:

"YU PROG.EXE"

(This is the BBS command to upload the PROG.EXE file using YAPP

protocol.)

If you are connected to another paKet station which is in REMOTE

Mode, you simply type:

"YU"

(the paKet program at the receiving end is smart enough to figure

out the name of the file!)

The other station will respond with something like:

"Ready to Receive with YAPP protocol"

Of course various other systems might require a slightly different

command and might respond with a different prompt, but you get the

idea.  If in doubt, ask the operator of the other station.

Now press <F7> and tell it the name of your file you want to send.

A Directory Window will pop up to help you select the file.

Then paKet will:

prepare the TNC for the Binary Transfer;

display the Binary File Transfer Window;

synchronise with the other station; and

begin the transfer.

If all goes well, you do nothing - just sit back and watch - and

the file will be sent. The program returns to normal

communications when the transfer is completed.

Page 115

Any problems with the file transfer could be:

1. a problem with the other station (not in Binary Receive mode;

not enough room on disk there; etc) in which case you need to

try again when that station is ready to receive the Binary

file transfer; or

2. there could be a problem in the handshaking between your

computer and your TNC. If you are using Hardware Handshaking

(you will see RTS CTS etc in the right hand side of the

Status Window), try using Software Handshaking as there could

be a problem in the cable you are using between the computer

and the TNC.

To change to Software Handshaking, refer to the

"Configuration Windows - Serial Port" section of this

manual.

Check both the TNC and paKet's Serial Port Configuration to

ensure the Handshaking options correspond:

If you have specified Software Handshaking to paKet, set

XFLOW ON in your TNC;

If you have specified Hardware Handshaking to paKet, set

XFLOW OFF in your TNC.

3. There is a mismatch in your settings for STREAMDBL.

paKet works best if STREAMDBL is ON in both the paKet

Configuration AND in the TNC, but the important thing is to

ensure they are BOTH set the same way!

If your TNC does not have a STREAMDBL option, set paKet's

option OFF.

Refer to the section, Configuration Windows - Multi User

options, for a full discussion on the STREAMDBL settings.

4. Some TNCs cannot handle a PACLEN of 256 (or 0 which, to the

TNC, is equivalent to 256).

Reduce PACLEN for binary operations to 252 in the

Configuration - File Transfer options.

5. Check Serial Port settings.  It is essential you have

specified 8 data bits and No Parity for the Serial Port

parameters.  They must be set the same way in the TNC too so

check your TNC settings with the appropriate commands,

usually AWLEN and PARITY.

Page 116

F8               Receive Binary file using either pP or YAPP protocol.

To use this feature the other station must be able to handle

either YAPP Protocol or pP (paKet Protocol).  Normally the

standard YAPP Protocol will be used; the advanced pP will be

activated by the program only when necessary.

If you want to receive a Binary File from a BayCom or Digicom

system, use the <Alt-F8> key. And if you prefer to receive a

Binary File from the other station with no protocol at all (you

will have to synchronise it manually), refer to the RAW Binary

mode with the <Alt-F6> key.

Receiving (or "Downloading") a Binary File is even simpler than

sending one.

To use this feature, prepare the other station for the file

transfer then press <F8>.  That is all!

For example, if you are logged on to a BBS system, or to a paKet

REMOTE system, you could type:

"YD PROG.EXE"    (The command to download a YAPP file)

and it would respond with something like:

"Ready to Send with YAPP protocol"

Then you press <F8> and watch as the file PROG.EXE (or whatever

you have specified) is transferred to your disk.

paKet will:

prepare the TNC for the Binary Transfer;

display the Binary File Transfer Window;

synchronise with the other station; and

begin the transfer.

Once again, I suggest you check with the operator of the other

station if this command does not appear to be working on that

system.

Page 117

F9               DOS Gateway

Pressing this key will provide access to DOS so you can run other

DOS jobs without leaving paKet. A Message Window will pop up

asking you for a DOS command.

There are two ways to perform a DOS command:

1.  You may enter the command into the message window beside

the ">" prompt.

If you do this, the command will be executed (if possible)

and you will be invited to "Press any key to continue"

before paKet resumes where it left off.  The "Press any key

to continue" message appears briefly over the DOS display

to allow you time to peruse the result of the job and to

read any messages etc that may have come from the DOS

command.

2.  Press <Enter> instead of entering a command, and the screen

will be cleared before control is passed over to DOS and

the familiar DOS Prompt.

When you have finished the DOS operations, type EXIT to

return to paKet.  Do NOT type "PAKET" again as the system

will attempt to load a second copy of paKet into memory!

Please keep in mind that paKet is still in memory at this stage so

you will have considerably less free memory in the Base 640K for

your other DOS programs. You may even find some programs will not

run due to insufficient memory.

If you are running paKet under a multi tasking system (for example:

DESQview, OS/2, Windows Enhanced Mode, Windows NT, etc) you might

find it better to start another DOS session instead of using this

Gateway to perform your DOS operations.  That way you will get a

full complement of memory and not be constrained by the memory

occupied by the paKet system.

If you are running plain vanilla DOS and find insufficient memory

to run your DOS job, the only remedy for this I am afraid, is to

exit the paKet program (with the <Alt-X> or <Alt-F4> key) so you

can release the memory occupied by the paKet program.

If using the Gateway and are "away" in DOS, paKet's Interrupt

Handler continues to operate so any input from the TNC during that

time will be captured and stored in paKet's 32KB System Input

Buffer.  Unless your DOS jobs take a looonnngg time, you should

not lose any input data.  Please note though, paKet is not a multi

tasking program and the captured data will not be processed until

you return to the program.

Page 118

F10              Display TNC Help File commands

This feature will load the TNC Help file and will display a window

showing the available commands for your TNC.

The TNC Help file that is displayed is the one specified in the

Configuration - Directories and Files Window. Several TNC Help

Files are supplied with paKet so be sure to specify the file best

suited to your TNC.

The TNC Help File Window will display a list of all the commands

for this TNC and this may be all you need to choose the desired

command. If so, press <Esc> to return to normal communications.

For more detail on any of these commands, highlight the desired

command and press <Enter>. When finished with this Detail display

press <Esc> to return to the main TNC Help file list. Then select

another item to display or press <Esc> to return to normal

communications.

Further details of this display are available in the section on

"TNC Help Window".

Page 119

Alt-F1           Display the Online Manual

Pressing this key will bring up paKet's Online Manual Window which

reveals the entire contents of this Manual.

The Manual is indexed and the index will appear for you to select

a topic. Move the highlight bar with the <Up-Arrow>, <Down-Arrow>,

<PageUp>, <PageDown>, <Home> or <End> keys to the desired topic,

then press <Enter> to select that topic.

Once a particular topic is selected the full text for that section

of the Manual will be displayed and you may scroll up or down as

desired.

When you have finished with that section, press <Esc> to return to

the Index to select another topic. If you have finished with the

Online Manual, press <Esc> to return to normal communications.

Page 120

Alt-F3           Call the BBS for Mail Forwarding

Mail Forwarding is discussed in the PMS section of this Manual.

Usually the Mail Forwarding with your chosen BBS is done at

regular times during the day or night and does not require your

manual intervention.  However, if you wish, you can force the

paKet system to call the BBS now and conduct a Forwarding session

to send and receive any mail that might be waiting.

To do this, just press <Alt-F3> and paKet will do the rest.

The program will initiate a Connect to the BBS using the Connect

Path (or Script file) specified in your PMS Configuration.  If the

BBS has any mail for you, that mail will be sent and stored in

paKet's PMS. And if you have any outgoing mail in there waiting to

be sent, it will be sent to the BBS during this session.

While connected to the BBS, whether for Mail Forwarding or for a

normal session, the symbol "BBS" will be displayed in the Status

Window.

When there is no more mail to be sent or received, the system will

disconnect from the BBS.

Page 121

Alt-F4           Exit the paKet program

This command returns you to the operating system. Any files that

were open will be closed automatically.

To guard against inadvertent exits, paKet will ask for your

confirmation that you really do want to exit.

If you have specified any End-Auto Commands (Refer the

Configuration section for details) they will be sent to the TNC

and if paKet finds an AUTOEND.SCP file in the current directory

that Script will be executed before paKet returns to the operating

system.

The <Alt-X> key achieves the same as <Alt-F4>.

Page 122

Alt-F5           Send a RAW Binary File

This option is similar to the other Binary File Transfer options

in that paKet will put the TNC into Transparent Mode and will

display the Binary File Transfer Window while the file is being

transferred.   The main difference here is that no protocol at all

will be used. If there is any synchronisation required with the

other station, you will have to do that manually.

This facility is provided to cover those situations where you want

to send a data file containing non-ASCII codes to another station

(or non-ASCII commands to the TNC). If the other station cannot

handle one of the Binary Transfer protocols supported by paKet and

you are unable to use the standard ASCII Text file transfer (<F5>)

because of the non-ASCII codes in the file, this might be your

last hope!

paKet will ask you for the name of the file you wish to send. A

Directory Window will pop up to help you select the file.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

begin the transfer.

Remember, with this RAW method, you will have to perform any

synchronisation required and to manually issue any commands to

start and to end the file transfer.  paKet does nothing to help -

this time it is your responsibility!

Page 123

Alt-F6           Receive a RAW Binary File

As with the <Alt-F5> command to Send a RAW Binary File, this

option is similar to the other Binary File Transfer options in

that paKet will put the TNC into Transparent Mode and will display

the Binary File Transfer Window while the file is being

transferred.   The main difference here is that no protocol at all

will be used. If there is any synchronisation required with the

other station, you will have to do that manually.

This facility is provided to cover those situations where you want

to transfer a data file containing non-ASCII codes from another

station which cannot handle one of the Binary Transfer protocols

supported by paKet.

paKet will ask you for the name of the file you wish to open to

hold the captured data.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

open the nominated file;

write any data received into that file.

You will have to close the file manually by pressing <Alt-F6>

again.

Remember, with this RAW method, you will have to perform any

synchronisation required and to manually issue any commands to

start and to end the file transfer.  paKet does nothing to help -

this time it is your responsibility!

Page 124

Alt-F7           Send a Binary File to a BayCom system

The BayCom system supports Binary Transfers via its //WPRG and

//RPRG commands but does not currently support any Binary File

Transfer PROTOCOLS. However, with just one reservation, paKet is

able to effect a reliable Binary File Transfer to a BayCom system.

This paKet facility is provided primarily for Binary Transfers to

a BayCom system although it should work into a Digicom system as

well.

When you press <Alt-F7> the Disk Directory window will appear for

you to select the file to send. Then paKet will ask for the name

(and path if that is required) this file is to have in the BayCom

system.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send BayCom's //WPRG command;

transfer the contents of the file;

send BayCom's <CR>//WPRG OFF<CR> command at end-of-file.

If all goes well, you do nothing - just sit back and watch - and

the file will be sent. The program returns to normal

communications when the transfer is completed.

As the BayCom system does not have a defined File Transfer

Protocol, the only way to indicate the end-of-file is with the

<CR>//WPRG OFF<CR> command. The trouble is, the BayCom system will

add this string of characters to the file BEFORE recognising it as

the command to close the file! So the file now on the BayCom

system will be 12 bytes longer than the one we sent. This is a

flaw in the BayCom system and there is nothing paKet can do about

it.

Fortunately it is not a big problem: I have found it usually does

not matter that the additional characters are there.  Executable

program files and archived (compressed) files seem to ignore the

additional characters.  And if required, the BayCom operator can

easily edit the additional bytes from the end of the file. Small

price to pay for the convenience of having Binary Transfer

capability!

Page 125

Alt-F8           Receive a Binary File from a BayCom system

The BayCom system supports Binary Transfers via its //WPRG and

//RPRG commands but does not currently support any Binary File

Transfer PROTOCOLS. However with some operator input, paKet is

able to effect a reliable Binary File Transfer from a BayCom

system.

This paKet facility is provided primarily for Binary Transfers

from a BayCom system although it should work from a Digicom system

as well.

The main difficulty in transferring a Binary File from a BayCom

system is the absence of any end-of-file indicator. The BayCom

system will send the Binary File in response to its //RPRG command

and will simply stop sending when it has reached its end-of-file.

paKet needs to know when to close the file. There are four ways

this can be done:

1. paKet will count each byte received and, provided it knows

the size of the file, it will close the file when that

number of bytes has been received.

2. If paKet receives BayCom's <CR>//WPRG OFF<CR> string it will

assume that is the end of the file. That string will NOT be

written to our file.

This means the Binary File being transferred cannot have

this string embedded in the file. If the file does happen to

contain this string, I suggest you ask the BayCom operator

to compress the file with LHA or similar and send the

compressed version of the file - this should change the file

so the string is no longer apparent.

If the file being received was previously transferred to the

BayCom system by some other user (or by us for that matter),

that file will probably have the <CR>//WPRG OFF<CR> string

on the end of the file.   paKet would recognise that and

close the file at the right time! (Refer to the comments on

sending a Binary File to a BayCom system, under <Alt-F7>).

The BayCom operator could send us this "command" when that

system has finished sending the file, although if the BayCom

operator is present we should know the file size and will

probably have closed the file by now.

3. You can press <Alt-F8> during the transfer to close the file

manually.

4. paKet will close the file if nothing has been received for a

period of 2 minutes.

Page 126

As the BayCom system will stop sending when it has finished

and will wait for our next command, this option might be

used if the BayCom system is unattended and you do not know

the filesize.

If the radio frequency is not busy, it should be rather

obvious when the other system has stopped sending so you

might close the file manually with <Alt-F8> (option 3 above)

before the 2 minutes expires.

Before attempting the transfer, you should get the details of

the file from the BayCom operator. You will need the filename

of course and if it is not in the BayCom system's current

directory you will need to know its path as well. If possible

get the file size in bytes too.

When you press <Alt-F8> paKet will ask you for the name of the

file in the BayCom system. This name will form part of the

//RPRG command paKet sends, so be sure to specify any path

details if required.   If you are transferring a file from a

Digicom system, it might be an unusual file name because the

Commodore 64 can cope with longer file names including embedded

blanks! You will have to specify the file name as it appears in

that Digicom system. Of course the file name you choose for this

file on our system will have to conform to the DOS requirements.

Then paKet will ask you for the file size in bytes. If you do

not know the file size, enter 0. This means the first of the

above options will not apply and you will probably have to

close the file manually or wait for the 2 minute timeout.

If you are transferring a file from a Digicom system, you will

probably not know the file size in bytes because the Commodore

64 system displays its file sizes in blocks rather than bytes.

In this case, enter -1 for the file size. This will tell paKet

to expect the Digicom responses (which are a little different

from BayCom's) as well as indicating the file is to be closed

manually.

Finally, paKet will ask you for the filename you wish to use

here. As you will most likely use the same name as that on the

BayCom system, paKet offers this name as a default. You can

change the details if you wish or simply press <Enter> to

accept the same name.

Then the transfer begins.  paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send the BayCom //RPRG command;

begin the transfer;

When the file is closed by one of the above methods, paKet

removes the Binary File Transfer window and returns to normal

communications.

Page 127

Alt-A            Turn Alerts on/off

This toggle enables or disables the Alerts facility which can

sound a Bell or a coded sound sequence (using Morse Code) if one

of the predefined strings is detected in the Communications Window.

Refer to the Configuration - Alerts window to see how to specify

the desired search strings.

<Alt-A> is a three-way toggle.  Press this key combination to

toggle from "No Alerts" to "Standard Alert tones" to "Morse Code

Alert tones" to "No Alerts"...etc.

No Alerts

If this is the current setting, paKet will ignore the Alerts

facility and will not even look for a matching Alert string.

Standard tones

When the Alerts facility is enabled for standard tones, an "A"

will appear in the Status Window to the left of the Time display.

If one of the Alert strings is detected, paKet will sound the bell

using the standard Bell tone (or audio frequency) you have

specified in the Configuration Window for Communications Window

Settings.

Morse Code tones

When the Alerts facility is enabled for the coded sounds, an "M"

will appear in the Status Window to the left of the Time display.

If this Morse option is selected, a unique set of tones will be

sent for each of the Alert strings you have defined.  The first

Alert specified in the Alerts Configuration Window will be

reported with the Morse Code signal for the letter 'A';  the

second Alert recorded in that Configuration Window will be

reported with the Morse letter 'B'; etc.

You can override those simple Morse signals with even more

sophisticated Morse sequences.  This is discussed in the

Configuration section where the Alerts Window is discussed.

Use the <Alt-J> key to adjust the Morse Code speed.

Page 128

Alt-B            Send a BREAK signal to TNC - return to Command Mode

This is the preferred method of returning your TNC to Command Mode.

Pressing <Alt-B> will send to the TNC either a BREAK signal, three

<Ctrl-C>s, or an <Esc> depending on the setting you have chosen in

your Serial Port configuration.

Most TNCs support the BREAK signal and will return to Command mode

when this signal is received.

If the BREAK is not supported, the TNC's "Command" character

(usually <Ctrl-C>) must be used.

On a few non-standard TNCs you must send the <Esc> code prior to

your command. For example the old, original TNC-1 works like this.

I have found the PacComm PacTOR Controller also uses the <Esc> code

for Command Mode!  So this option is provided in case your TNC works

this way.

But, regardless of the TNC used and regardless of the way you have

configured this option, you can consider the <Alt-B> function the

correct way to prepare your TNC for a Command.

Page 129

Alt-C            Connect to another station

This feature allows you to more easily make a connection to your

regular contacts, especially where there are long path details to

be entered.

Pressing <Alt-C> will bring up paKet's Connect Window which is

fully described in the section on the Connect Window.

To make a call, simply move the cursor to the desired item and

press <Enter>.

You can change an existing entry by moving the cursor to the

character to be changed, and insert the new data. Pressing the

<Insert> key allows you to toggle Insert Mode so you can choose

between Inserting new data or Overtyping existing data.

Use the <Tab> key (or the <Right-Arrow> key if you prefer) to move

from the Name field to the Callsign field.

To insert a new entry, move the highlight bar to the desired line

and press <Ctrl-Ins>. The entry on that line and on all following

lines will move down making room for your new entry.

To remove an existing entry, move the highlight bar to that entry

and press <Ctrl-Del>.

Nodes and Digipeaters

For those who wish to connect to stations outside normal range you

can specify digipeaters in the path along with the callsign.

For example:

VK2DHU via VK2DIG-1 VK2RPT-4

You might want to connect via a Node (where you must firstly

connect to the Node then, after you are connected, issue a second

connect command to ask the Node to establish contact with the

desired station). In that case you may specify both connect paths

in the Connect line separated by a semi-colon.  For example:

VK2NOD via VK2DIG-1 VK2RPT-4 ; VK2DHU via VK2XXX

When this format is used, paKet will generate a CONNECT command

using the path specified up to the ';' then it will generate a

second CONNECT command using the path specified after the ';'.  So

using the example above, it will generate the following commands:

CONNECT VK2NOD v VK2DIG-1 VK2RPT-4

CONNECT VK2DHU v VK2XXX

Page 130

Please note it will not wait nor perform any intelligent processing

of the first connect command.  It will ensure the TNC is in Command

Mode before the first "CONNECT" is sent and then it simply generates

and sends both these lines to the TNC.

You will need NEWMODE ON in your TNC for this technique to work

because we want the second line to be processed as data, not as a

command for your TNC!   (Have a look in your TNC Manual for more

information on the NEWMODE parameter.  And check paKet's Multi

User Configuration Window to ensure paKet's NEWMODE setting is

configured the same as your TNC's setting).

The first "CONNECT" is processed by the TNC as a CONNECT command

to connect to the Node.  With NEWMODE ON the TNC switches

immediately into Converse Mode, so our second line is treated as

data and is transmitted to the Node.

I think we need an example.  Let's use that same sample we used

above:

- the TNC would initiate a connect to the Node (which is VK2NOD

in our example) via two digipeaters;

- the second line is sent as data by our TNC to that Node;

- that data will be recognised by the Node as a Connect

command; so  the connection to VK2DHU via VK2XXX is then

processed by the Node!

Scripts

If that seems rather complicated or maybe with your TNC it doesn't

work too well, the last resort, and the most flexible of all, is

to use a Script to establish the connection.  You can specify a

Script file name in the Connect Window in place of the callsign

and path details.  Just record the Script file name in Pointer

Brackets.  As an example, say you had a Script file called

BBSCONN.SCP. You could specify:

<BBSCONN>

instead of the callsign and when that item is selected paKet

recognises that entry as a Script and will load and run that

Script file.

The Script can provide all the flexibility you need with multiple

connects and delays etc.  Refer to the Script section of this

Manual for details on how to create or change a Script file.

Page 131

Alt-D            Display a Disk Directory

Pressing <Alt-D> will bring up the Disk Directory Window which is

fully described in the Windows section of this Manual.

You may scroll up and down this window using the <Up-Arrow> or

<Down-Arrow> keys, the <PageUp> or <PageDown> keys, or the <Home>

or <End> keys. Or you can use paKet's hot-key typing by entering

part or all of the file name - paKet will highlight the first

filename that matches the characters you have typed so far. Often

all you need to type is one or two characters to move to the file

you want.

If you press the <Enter> key to select the highlighted entry, the

action taken depends on the type of line highlighted. If the

highlight bar is on:

(Enter file name manually)

The Message Window pops up to ask you for the desired file

name. When you enter the file name the contents of that file

are displayed as if you selected that file with the <Enter>

key (see below).

HINT: Press <Home> to jump quickly to this entry at the top

of the Disk Directory Window.

(Change to another drive)

The Message Window pops up to ask you which drive you wish

to change to. Then the directory display changes to show the

root directory for that drive.

When entering the new drive letter, the colon is optional.

For example, you could enter either:

A  or

a:

to select drive A:. Note upper or lower case is acceptable.

<Previous directory>

The directory display changes to show the contents of the

"Parent" directory, that is the directory above this one.

Hint:

Usually it will be quicker to type a "." to highlight this

entry instead of using the arrows to move the highlight

bar.

<DIR>

If there is an entry with "<DIR>" instead of the file size,

this represents a sub directory below this one. So pressing

<Enter> on this line, causes that subdirectory to be

displayed.

Page 132

(filename)

If you press <Enter> with any file highlighted, paKet will

display the contents of that file. The command used to

create the display is configurable and is defined in

Configuration Windows - Directories and Files.

I prefer something like the LIST program, but you could use

the DOS "TYPE" command if you wish.  You could even specify

your favourite Text Editor there if you prefer!  When the

display program has finished, press any key to return to the

Directory Window. If you don't want to  display any more

files, press <Esc> to return to normal communications.

While you are viewing the file, paKet's Interrupt Handler

continues to operate so any input from the TNC during that

time will be captured and stored in paKet's 32KB System

Input Buffer.

If you press the <Delete> key while a file is highlighted, paKet

will delete that file (after seeking your confirmation).   You may

also delete a sub directory entry, provided that sub directory is

empty.

If you press the <Alt-R> key while a file is highlighted, paKet

will ask you for the new filename then will Rename that file.  You

can specify a new directory with the new filename and paKet will

Move the file to the new directory.

Page 133

Alt-E            Edit a data file with your preferred System Editor

This feature allows you to edit a file using your favourite text

editor.

Details of the System Editor program are defined in Configuration

Windows - Directories and Files and may be any editor or word

processor you choose. Keep in mind however, that paKet with all

its buffers and data is still in memory, so you will probably not

be able to load and run one of the BIG Word Processing packages

using this feature.

When you press <Alt-E>, paKet will display the Disk Directory

Window so you can identify the file you wish to edit. Then it will

load the nominated editor program.

When you have finished the edit and exit from that program, you

will be returned to paKet where you left off.

While you are using the System Editor, paKet's Interrupt Handler

continues to operate so any input from the TNC during that time

will be captured and stored in paKet's 32KB System Input Buffer.

If you are running paKet from one of the multi tasking systems

such as Windows Enhanced Mode, OS/2, DESQview, Windows NT, etc it

might be more convenient to start up another task and do the

editing there, leaving paKet running in its own window.

Page 134

Alt-F/Alt-L      Find text in Flashback/Continue Find in Flashback

Pressing <Alt-F> will commence a search backward through the

Flashback buffer, looking for a particular string of characters.

A Message Window will pop up asking you to enter the desired

search string. You can enter this in either upper or lower case

because the search is not case sensitive.

If found, the line containing the string is highlighted and paKet

remains in Flashback mode.

Pressing <Alt-L> will continue the search, looking for an earlier

occurrence of the same string.

Further details on this option are found in the Communications

Windows - Flashback section.

Page 135

Alt-G            Get last message back again

Sometimes a message disappears before you have had time to digest

it or perhaps you thought you understood it but have forgotten

some important detail ("what was that filename again"?)

You can increase the message duration in the Miscellaneous

Configuration options, but it is possible you might want to

take a little more time with just an occasional message.

This <Alt-G> command will re-display the last message displayed in

the Message Window.  The re-displayed message will remain for the

normal message duration but you can type <Alt-G> as many times as

you like.

Page 136

Alt-I            Initialise Communications Windows

This option would not be used in normal circumstances but has been

provided mainly to "fix" problems that were discovered while

testing with a KAM TNC. Other Kantronics TNCs were not tested but

it is likely the problems will occur with those models as well.

These TNCs do not behave in the same way as other TNCs when

multiple streams are used as they do not support the STREAMDBL

command.

If, when monitoring the frequency, the program receives a STREAMSW

character from the TNC, it has no way of knowing whether it is a

data character that somebody has transmitted or whether it is

really a change of stream.  Other TNCs resolve this issue with the

STREAMDBL facility. If we are going to be able to handle multiple

streams, the program cannot ignore this STREAMSW character - it

must process it as a change of stream!

Suppose we have decided to use the vertical bar ("|") as our

STREAMSW character and while monitoring the frequency, we see one

of these characters come in.  Without the benefit of the STREAMDBL

facility paKet must assume it is a change of Stream and will

change over to another Communications Window!  So any further

monitored data will start appearing in that other Window.

If you are using one of these Kantronics TNCs and find data is

going into some other Communications Window, you will need to

reset the Streams, clear any responses in the other Windows and

get everything back into order.  This is what this <Alt-I> command

will do for you.

If you are using some other brand of TNC this command should not

be required although it will not cause any problems. It will

simply clear any backlog in the other Communications Windows and

reset the TNC to the Stream specified for the first Communications

Window.

Page 137

Alt-J            adJust Morse speed

There is a Morse Code option in paKet's Alerts facility.  This is

provided to make some unique sounds for the different alert strings

that might appear.

When you type <Alt-J> a Message Window pops up showing the current

Morse speed setting in WPM (words per minute).  You can change

this by typing the new WPM value or if you prefer, you can press

the <Up-Arrow> or the <Down-Arrow> keys to adjust the speed up or

down.   Press <Enter> to complete the change.

The speed will automatically be saved and the Morse Alerts will

now come out at this new speed.

Hev a look at the <Alt-A> command and the Alerts Window in the

Configuration section for more details.

Page 138

Alt-M            Activate the Personal Message System (PMS)

When you press this key you get the Sysop PMS menu:

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

While the menu is active all other communications will be held in

the input buffers until you have finished. You don't have to do

anything - paKet will automatically switch <ScrollLock> on and

will switch it off again when you have finished.

The Menu options are used as follows:

B  (Bye)       - End Sysop PMS Processing

C  (Copy)      - Copy a message to a file (eg C 3)

E  (Edit)      - Edit a message header (eg E 3)

H  (Help)      - Display these details

K  (Kill)      - Kill a message (eg K 3)

KM (Kill Mine) - Kill all messages addressed to me

L  (List)      - List available messages

LM (List Mine) - list all unread messages addressed to me

R  (Read)      - Read a message (eg R 3)

RM (Read Mine) - Read all unread messages addressed to me

S  (Send)      - Send a message (eg S VK2XYZ)

?  (Help)      - Display these details

The Personal Message System is discussed fully in its own section

of this Manual.

Page 139

Alt-P            Turn Print Log on/off

This is a toggle, meaning you press <Alt-P> to turn the Print Log

ON and press it again to turn the Print Log OFF again.

While the Print Log is ON, a 'P' will be displayed in the Status

Window to the left of the Windows display and all Communications

data will be echoed to the printer on LPT1.

If the Print Log is OFF, no data will be sent to the Printer.

Page 140

Alt-Q            Quiet Mode on/off

When Quiet Mode is enabled, paKet will make NO sounds at all.  It

doesn't matter where the sounds would normally come from, whether

from a new Connection, or from an Alert, or from another station's

<BEL> character, or whatever.  Quiet Mode will silence them all.

This might be convenient if you want to leave the system running

at night and do not want to be disturbed by the various sounds

that can emanate from this system.

This is a toggle switch, meaning you press <Alt-Q> to activate

Quiet Mode and you press <Alt-Q> again to switch it off.

When Quiet Mode is active, a "Q" will be displayed to the left of

the time in the Status Window.

Page 141

Alt-R            Rename/Move a data file  or  Register this paKet program

When the Disk Directory Window is displayed, <Alt-R> may be used

to Rename a data file or Move it into another directory.

Move the Highlight bar to the file you want to rename or move

before pressing <Alt-R>.

The Message Window will then pop up asking you to enter the new

filename.

If you want to simply change the file's name, leaving the file in

its existing directory, just type the new filename and press

<Enter>. The filename will be changed and the Directory Window

will be re-sorted and re-displayed.

If you want to Move the file into another directory, enter the

full path details into the Message Window (you may also enter a

different filename if you wish) and paKet will transfer the file

to the new directory.  The new Directory Window will then be

displayed, showing that file in the new directory.

When a file is Moved into another directory, the data is not

actually transferred, it is just the directory entry that is

changed. DOS does not permit a "Move" to another drive, so if you

want the file on a different drive, you'll have to COPY the file

yourself.  Refer to the <F9> DOS Command for access to DOS to

perform the COPY.

If you have not yet Registered this copy of paKet, you can do so

by pressing <Alt-R> at any time (except when the Disk Directory

Window is displayed!) and paKet will ask you for your Serial

Number.  If your number is entered correctly, it will be recorded

and your callsign will appear in the top left corner of the

display, indicating your copy of paKet is now registered.

Page 142

Alt-S            Script processing - load and run a Script file

This is the command you use to activate a paKet Script file.

(Script Processing is discussed fully in its own section in this

Manual).

When you select this option, a Disk Directory Window will appear

with the default Script file name already highlighted. If you

decide to enter a file name manually and you type a file name

without an extension, paKet will assume an extension of ".SCP".

When you select the desired Script file, that file will be loaded

and Script processing will commence.   The word "SCRIPT" will be

displayed in the Status Window to the left of the time display

while the Script is running.

When the Script has completed, or is aborted by <Esc>, the

indicator in the Status Window will disappear and you will be

returned to normal communications.

Page 143

Alt-T            Set TNC's Time and date

If for any reason, you need to reset your TNC, you can easily

reset the date and time with this function. paKet will generate a

"DAY" command using the computer system's date and time as it does

during initialisation..

The format of the generated command includes the time in Hours,

Minutes and Seconds. It has been brought to my notice that some

TNCs cannot handle that format and will accept the time as Hours

and Minutes only. So, if you hold the Ctrl key while typing this

<Alt-T> command (i.e. <Ctrl-Alt-T>), paKet will generate the DAY

command without the seconds.

Page 144

Alt-V            Verify the TNC's Connected status

When a Connection is established with another station, paKet will

try to determine the callsign of that station from the TNC's

"Connected" message when it appears in the Communications Window.

paKet will then display that callsign in the Communication

Window's Header line.

Some software packages operating in Host Mode or KISS Mode can

interrogate the TNC to get this information, but these packages

are usually more specific to certain TNCs. paKet, on the other

hand was designed to run on all TAPR-compatible TNCs and so relies

on the data flowing from the TNC rather than on some special TNC

codes.

paKet's method works well most of the time, but there are

occasions where the callsign displayed is incorrect. For example,

if you reload paKet after a TNC connection has already been

established, the newly loaded copy of paKet will not see a

"Connected" message and so will think there is no connection!

This <Alt-V> command will issue a "C" (Connect) command to the TNC

and will look at its reply to determine the connected status. A

Message Window will pop up to advise you of the result and, if

necessary, paKet will change the displayed callsign in the

Communications Window Header.

Page 145

Alt-W            Write contents of the Flashback buffer to disk

If you have received some information which you wanted to save and

is still in the Flashback buffer, but did not have the disk Log

File on at the time, you can select this option to write some or

all of the Flashback buffer to disk.

A Message Window will pop up asking you to:

"Move to the start of the block and press <Enter>"

The program will automatically enter Flashback Mode allowing you

to use the arrow keys, <PageUp>, <PageDown>, etc to move the

highlight bar to the start of the block of data you want to write

to disk.  If you want to capture the entire Flashback buffer,

press <Ctrl-PageUp> to move to the start of the buffer.

Then press <Enter> to identify that as the first line to be

captured.   Another message will pop up:

"Move to the end of the block and press <Enter>"

Using the same technique, move down to the last line you want to

capture (<Ctrl-PageDown> to move quickly to the last line) and

press <Enter> to mark that as the last line to be captured.

Note, the block of data to be written will be highlighted as you

move through the Flashback buffer.

Now that we know what is to be written to disk, paKet will ask you

for the desired file name, via a Disk Directory Window.

When that data is written to the file, processing continues in

normal conversation mode.

Page 146

Alt-X            Exit the paKet program

This command returns you to the operating system. Any files that

were open will be closed automatically.

To guard against inadvertent exits, paKet will ask for your

confirmation that you really do want to exit.

If you have specified any End-Auto Commands (Refer the

Configuration section for details) they will be sent to the TNC

and if paKet finds an AUTOEND.SCP file in the current directory

that Script will be executed before paKet returns to the operating

system.

The <Alt-F4> key achieves the same as <Alt-X>.

Page 147

Alt-Y            Yawn

Typing this key combination inserts a special code into the text

which causes a half second delay.  It may be added to your text as

you type in the Type Ahead buffer and the delay will occur when

the data is sent to the Communications Window and of course to the

TNC.  It may be typed into other strings too such as Keyboard

Macros.

Page 148

Alt-Z            Configuration

This option allows you to configure paKet's many options to set up

an environment especially to suit yourself.

When you press <Alt-Z> the System Configuration Window will appear

to show the major items that may be configured.  When you select

one of those options additional Configuration Windows will appear

to allow you to select the desired options.

Full details of all configurable options are shown and explained

in the section on Configuration Windows in this Manual.  If you

are new to this, start with the Configuration Windows - Overview

and work your way through all the options from there.

There is some additional help available with the Online

Configuration.  When the System Configuration Window appears,

another Window should appear below it offering some additional

comments on the Configuration Window currently being displayed.

This is called the Configuration Help and is optional.  If it is

not being displayed, you can activate it by setting that option to

Y in the Miscellaneous options.

To change that Configuration Help option, after pressing <Alt-Z>

type 9 and you will see in the Miscellaneous options window an

item in there that says "Display Configuration Help ?" and you can

set that to Y or N. Set it to Y and you should now see the

additional Help windows while you are using the Online

Configuration.  You will need the file PAKETCFG.HLP in your

default PAKET directory.

Page 149

Insert           Toggle Insert Mode

The <Insert> key toggles the Insert Mode on and off.

When Insert Mode is on, the cursor will be a Block shape and any newly

typed data characters will be inserted between existing data

characters at the cursor position.

When Insert Mode is off, (i.e. Overwrite Mode) the cursor will be an

Underscore shape and newly typed data will overwrite any data

previously stored at the cursor position.

If the Type Ahead Window is not being used, you will not be able to

"insert" characters (or move the cursor back) because in this mode

each character you type into the Communications Window is sent to the

TNC as you type. It is too late to insert something else!

Delete           Delete a character or delete a file

When entering text the <Delete> key will remove the character under

the cursor.

If a Disk Directory Window is currently displayed, you can delete a

file from the disk by highlighting the unwanted file then pressing the

<Delete> key.

Bksp             Backspace - delete the character before the cursor

When entering text the <Backspace> key will remove the character to

the left of the cursor.

Ctrl-Del         Delete a line

When entering text the <Ctrl-Del> key will remove all text currently in

the Type Ahead Buffer or on the line being entered.

Ctrl-Ins         Insert a blank line

In the Connect Window and in the Configuration Windows for the

Keyboard Macros, Auto Commands and Alerts, you might want to insert an

entry in the Window between two existing entries.

Move the cursor or highlight bar to the line which you want moved down

and press <Ctrl-Ins>.  This will move that line and everything below

it down to leave a blank line where the cursor is.

Page 150

Esc              Return to previous operation

The <Esc> key is used in a number of places in paKet, each time in a

manner of "exiting" or "returning" from the current function. It is

your way of saying "I don't want to do this any more".

For example, if in Flashback Mode, the <Esc> key will return the

system to normal communications.

And if displaying a Disk Directory Window the <Esc> key will return

the system to normal communications.

If there are several "levels" involved in the current function, for

example when using the Configuration Windows you could have two or

even three levels of Configuration Window displayed. The <Esc> key

will take you back to the previous level. If you are already at the

first level, it will take you back to normal communications.

You may note the <Esc> code cannot therefore be sent to the TNC from

the keyboard. If you need to send an <Esc> code to the TNC you could

define it in a Keyboard Macro as <Ctrl-[> (written as "^[" which is

caret and left-square-bracket). <Ctrl-[> generates the same code as

<Esc>. If you enter it this way paKet will process that as data,

sending the code to the TNC. Refer to the Special Key Codes section

for more information on the use of the '^' symbol for generating

<Ctrl- > key sequences.

ScrollLock       Hold all input data in the buffers

The <ScrollLock> key in paKet operates as a Pause key for input data.

Keyboard activity remains unaffected, so you may continue to send to

the TNC.

While the system is "ScrollLock"ed, any input data coming in from the

TNC will be held in paKet's input buffers until the Scroll Lock key is

pressed again to release it.  While ScrollLock is ON, an indicator

appears to the right of the Communications Window Header line.

You may have difficulty finding the <ScrollLock> function on some

machines such as the Tandy 1000 that are not quite compatible with the

Industry Standard.  Refer to your manual!

Left-Arrow       Move the cursor left one position

When entering text this key is used to move the cursor one character

position to the left. The text remains unchanged.

Right-Arrow      Move the cursor right one position

When entering text this key is used to move the cursor one character

position to the right. The text remains unchanged.

Page 151

Up/Shift-Up      Move UP one line or Type Ahead Recall

The function of this key depends on the situation you are in when it

is used.

It is generally used in the sense of "moving up one line".  So if you

have a Disk Directory Window displayed, this key will move the

highlight bar up to the previous line.  Same for the Online Manual,

TNC Help, and just about everything else.

The main exception to this is the Type Ahead Buffer.  If you have

configured multiple lines of Type Ahead, you must use the <Left-Arrow>

or <Ctrl-Left-Arrow> or <Home> to move back through that Type-Ahead

data.  That is, you cannot move up a line in a multi line Type Ahead

buffer.  You get there by moving left through the buffer, wrapping

around at the beginning of the line.

If one of the subsidiary Windows is not currently on display, the

<Up-Arrow> is used either to move back through the Flashback Buffer or

to Recall previous lines from the Type Ahead Buffer.   Whew!   Trouble

is... it can do one or the other but can't do both at once!

The previous version of paKet used this key to Recall from Type Ahead

and many people complained they wanted it for Flashback!  Well, in

this version you can make your own choice.  You can choose whether you

would like to use the <Up-Arrow> for moving up in the Flashback buffer

or whether you would rather use it for recalling lines from the Type

Ahead Buffer.   This selection is made in the Miscellaneous options

Configuration Window.

No matter which way you set that option, the OTHER option is available

with the Shift key.  So if you configure the <Up-Arrow> as a Flashback

key, you can recall Type Ahead lines with <Shift-Up-Arrow>. And vice

versa.

Down/Shift-Down  Move DOWN one line or Type Ahead Recall

The function of this key depends on the situation you are in when it

is used and is similar to the way the <Up-Arrow> is used.

It is generally used in the sense of "moving down one line".  So if

you have a Disk Directory Window displayed, this key will move the

highlight bar down to the next line.  Same for the Online Manual, TNC

Help, and just about everything else.

The main exception to this is the Type Ahead Buffer.  If you have

configured multiple lines of Type Ahead, you must use the

<Right-Arrow> or <Ctrl-Right-Arrow> or <End> to move forward through

that Type-Ahead data.  That is, you cannot move down a line in a multi

line Type Ahead buffer.  You get there by moving forward through the

buffer, wrapping around at the end of the line.

Page 152

If one of the subsidiary Windows is not currently on display, the

<Down-Arrow> is used either to move down through the Flashback Buffer

or to Recall the next line from the Type Ahead Buffer.   Whew!

Trouble is... it can do one or the other but can't do both at once!

The previous version of paKet used this key to Recall from Type Ahead

and many people complained they wanted it for Flashback!  Well, in

this version you can make your own choice.  You can choose whether you

would like to use the <Down-Arrow> for moving down the Flashback

buffer or whether you would rather use it for recalling lines from the

Type Ahead Buffer.   This selection is made in the Miscellaneous

options Configuration Window.

No matter which way you set that option, the OTHER option is available

with the Shift key.  So if you configure <Down-Arrow> as a Flashback

key, you can recall Type Ahead lines with <Shift-Down-Arrow>. And vice

versa.

Home             Move to the start of the text or the window

When entering text the <Home> key moves the cursor to the beginning of

the text. The text remains unchanged.

When viewing a Disk Directory Window, Configuration Window, the Online

Manual or the TNC Help File, the <Home> key is used to move to the

first of the items on display.

End              Move to the end of the text or the window

When entering text the <End> key moves the cursor to the end of the

text. The text remains unchanged.

When viewing a Disk Directory Window, a Configuration Window, the

Online Manual or the TNC Help File, the <End> key is used to move to

the last of the items on display.

PageUp           Move display up one page

When viewing a Disk Directory Window, the Online Manual or the TNC

Help File, the <PageUp> key is used to move up the list of items on

display a page at a time.

At other times this key is used for Flashback operations, moving up

one page at a time.

A "page" here is considered to be the size of the window being

displayed.

Page 153

PageDown         Move display down one page

When viewing a Disk Directory Window, the Online Manual or the TNC

Help File, the <PageDown> key is used to move down the list of items

on display a page at a time.

At other times this key is used for Flashback operations, moving down

one page at a time. If you reach the end of the Flashback Buffer and

there is some Input data waiting to be displayed, paKet will display

one page of that Input data and remain in Flashback mode. If there is

less than a page waiting, it will return to normal communications.

Ctrl-PgUp        Move to beginning of the Flashback buffer

This key is used for Flashback operations, scrolling to the beginning

of the Flashback buffer. While the scrolling is in progress, pressing

any key including <Ctrl-PageUp> again will stop it at that point.

Ctrl-PgDown      Move to end of the Flashback buffer

This key is used for Flashback operations, moving to the end of the

Flashback buffer. While the scrolling is in progress, pressing

any key including <Ctrl-PageDown> again will stop it at that point.

Shift-Left       Change to previous Communications Window

<Shift-Left-Arrow> tells paKet to change to the previous

Communications Window if available.

You might prefer to use the <Shift-Fn> keys to change Communications

Windows (eg: <Shift-F3> to change to Window 3) but whichever method

you choose, paKet will automatically issue the change of stream

commands to the TNC when you change Windows.

Shift-Right      Change to next Communications Window

<Shift-Right-Arrow> tells paKet to change to the next

Communications Window if available.

You might prefer to use the <Shift-Fn> keys to change Communications

Windows (eg: <Shift-F3> to change to Window 3) but whichever method

you choose, paKet will automatically issue the change of stream

commands to the TNC when you change Windows.

Page 154

Shift-Fn         Select another Communications window.

You may select another Communications Window, that is select another

Stream, by pressing Shift and a Function key.

<Shift-F1> is the first Communications Window, <Shift-F2> the

second, etc. For example, you can select Window 4 by pressing

<Shift-F4>.

Another way to change to another Communications Window is to use the

<Shift-Left-Arrow> or <Shift-Right-Arrow> keys to move, one-by-one

to the desired window.

Page 155

(This page intentionally blank)

Page 156