PAKET - VERSION 6

Tony Lonsdale

VK2DHU

TABLE OF CONTENTS

PART I - INTRODUCTION

Welcome to paKet ..............................................   1

Computers - how it all began ..................................   3

Digital Communications ........................................   8

Morse Code .................................................   8

RTTY .......................................................   8

AMTOR ......................................................  11

PACTOR .....................................................  12

Packet Radio ...............................................  14

Error Free Communications ...............................  14

Frequency Sharing .......................................  14

Multiple Repeater Operation .............................  15

Networks ................................................  17

TCP/IP ...............................................  18

NET/ROM ..............................................  20

ROSE .................................................  22

The BBS Network and Mail Forwarding .....................  25

Mail Forwarding ......................................  26

The role of the BBS and where paKet fits in ..........  26

Bulletins ............................................  27

Other Communications Modes ....................................  28

The paKet Documentation .......................................  28

PART II - paKet's WINDOWS

Overview ......................................................  29

Binary File Transfer Window ...................................  30

Communications Windows ........................................  31

Flashback ..............................  34

Flashback Text Find ....................  35

Write Flashback to disk ................  36

Configuration Windows  Overview ...............................  37

System Configuration ...................  38

Serial Port ............................  39

Multi User options .....................  43

Communications Windows .................  47

Colours ................................  52

Keyboard Macros/Auto commands (etc) ....  54

Keyboard Macros .....................  55

Begin Auto Commands .................  59

End Auto Commands ...................  60

Alerts ..............................  61

Control Codes to PASS ...............  63

KISS Mode OFF sequence ..............  65

REMOTE / PMS options ...................  66

REMOTE options ......................  66

PMS options .........................  68

File Transfer options ..................  73

Directories and Files ..................  75

Miscellaneous options ..................  80

Connect Window ................................................  87

Disk Directory Window .........................................  90

Help Window ...................................................  92

Message Window ................................................  93

Status Window .................................................  94

TNC Help Window ...............................................  99

Type Ahead Window ............................................. 102

PART III - SPECIAL KEY CODES

Overview ...................................................... 105

Ctrl-key combinations ......................................... 105

Alt-key combinations .......................................... 106

Function keys ................................................. 107

Other commonly used command keys .............................. 107

Macro substitutions ........................................... 108

PART IV - KEYBOARD COMMANDS

F1            Display Help Screen ............................. 109

F2            Turn Log File on/off ............................ 110

F3            Turn REMOTE Mode on/off ......................... 111

F4            Toggle Word Wrap and Single Line on/off ......... 112

F5            Send ASCII text file ............................ 113

F6            Receive ASCII text file ......................... 114

F7            Send Binary File using pP or YAPP ............... 115

F8            Receive Binary File using pP or YAPP ............ 117

F9            DOS Gateway ..................................... 118

F10           Display TNC Help File commands .................. 119

Alt-F1        Display Online Manual ........................... 120

Alt-F3        Call the BBS for Mail Forwarding ................ 121

Alt-F4        Exit the paKet program .......................... 122

Alt-F5        Send RAW Binary File ............................ 123

Alt-F6        Receive RAW Binary File ......................... 124

Alt-F7        Send Binary File to BayCom (or Digicom) ......... 125

Alt-F8        Receive Binary File from BayCom (or Digicom) .... 126

Alt-A         Turn Alerts on/off .............................. 128

Alt-B         Send BREAK to the TNC - return to Command Mode .. 129

Alt-C         Connect to another station ...................... 130

Alt-D         Display a Disk Directory ........................ 132

Alt-E         Edit a data file ................................ 134

Alt-F/Alt-L   Find text in Flashback .......................... 135

Alt-G         Get last message back again ..................... 136

Alt-I         Initialise Communications Windows ............... 137

Alt-J         adJust Morse speed .............................. 138

Alt-M         Personal Message System ......................... 139

Alt-P         Turn Print Log on/off ........................... 140

Alt-Q         Turn Quiet mode on/off .......................... 141

Alt-R         Rename/Move a data file (or Register) ........... 142

Alt-S         Script Processing ............................... 143

Alt-T         Set TNC's Time and Date ......................... 144

Alt-V         Verify TNC's Connected status ................... 145

Alt-W         Write contents of Flashback to disk ............. 146

Alt-X         Exit the paKet program .......................... 147

Alt-Y         Yawn (half second delay) ........................ 148

Alt-Z         Configuration ................................... 149

Insert        Toggle Insert Mode on/off ....................... 150

Delete        Delete a character or delete a file ............. 150

Backspace     Delete the character before the cursor .......... 150

Ctrl-Del      Delete a line ................................... 150

Ctrl-Ins      Insert a blank line ............................. 150

Esc           Return to previous operation .................... 151

ScrollLock    Hold all input data (pause) ..................... 151

Left-Arrow    Move the cursor left ............................ 151

Right-Arrow   Move the cursor right ........................... 151

Up/Shift Up   Move cursor up or Type Ahead recall ............. 152

Down/Sh. Down Move cursor down or Type Ahead recall ........... 152

Home          Move to start of text or window ................. 153

End           Move to end of text or window ................... 153

PageUp        Flashback - Move back a page .................... 153

PageDown      Flashback - Move forward a page ................. 154

Ctrl-PageUp   Flashback - Move to beginning of buffer ......... 154

Ctrl-PageDown Flashback - MOve to end of buffer ............... 154

Shift-Left    Change to previous Communications Window ........ 154

Shift-Right   Change to next Communications Window ............ 154

Shift-Fn      Change to another Communications Window ......... 155

PART V - Messages in the paKet system

Overview ...................................................... 157

Messages starting with A ...................................... 158

B and C ................................ 163

D and E ................................ 167

F, G and H ............................. 172

I ...................................... 174

"I can't" .............................. 175

I (cont) ............................... 180

K, L and M ............................. 186

N ...................................... 189

P ...................................... 192

R ...................................... 197

S ...................................... 201

T ...................................... 205

U, V and W ............................. 216

Y and Z ................................ 220

PART VI - REMOTE Mode

Overview ...................................................... 223

Activating REMOTE Mode ........................................ 224

Leaving REMOTE Mode ........................................... 225

The REMOTE Menu Commands A to L commands ...................... 226

M (BayCom commands) .................. 230

P to W commands ...................... 233

Y (YAPP or pP commands) .............. 237

Other REMOTE commands ......................................... 239

PART VII - SCRIPT PROCESSING

Overview ...................................................... 241

Running a Script File ......................................... 242

Script Tutorial - S command ................................... 243

B and D commands ............................ 245

W and Q commands ............................ 247

G and : commands ............................ 249

* and E commands ............................ 251

C and R commands ............................ 254

AD, AU and AC commands ...................... 258

YD and YU commands .......................... 258

! command ................................... 262

T command ................................... 264

FC, FS and FT commands ...................... 269

Variables ................................... 270

M command ................................... 270

Passed Parameters ........................... 271

Scope of Variables .......................... 273

I command ................................... 275

? and J commands ............................ 275

L, P and X commands ......................... 277

The Script File syntax ........................................ 278

Script Command Summary ........................................ 279

PART VIII - PERSONAL MESSAGE SYSTEM (PMS)

Overview ...................................................... 285

The Sysop PMS Menu ............................................ 285

PMS commands in the REMOTE Menu ............................... 286

Accessing the PMS - B, C and E commands ....................... 287

H, K and L commands ....................... 292

R and S commands .......................... 296

Mail Forwarding ............................................... 299

Making Auto Mail Forwarding work for you ...................... 301

PART IX - paKet PROTOCOL (pP)

What is pP ? .................................................. 309

Using pP ...................................................... 310

The Technical details of the paKet Protocol ................... 312

PART X - TECHNICAL

Hardware vs Software Handshaking .............................. 317

Preferred TNC Settings ........................................ 320

Kantronics TNCs ............................................... 326

PacComm PacTOR Controller ..................................... 329

DSP-12 ........................................................ 330

Running paKet under Microsoft Windows ......................... 332

Running paKet under OS/2 ...................................... 334

The PAKET.CFG file ............................................ 336

PART XI - ADMINISTRATION DETAIL

The cost of paKet ............................................. 343

Authorised distributors ....................................... 343

Updates ....................................................... 344

Contacting the author ......................................... 344

Acknowledgements .............................................. 345

Dedication .................................................... 346

PART I - INTRODUCTION

Welcome to paKet.

paKet is one of the world's most popular programs for amateur radio

communications and if this is your first look at paKet I hope you will

soon agree it IS "the best Packet Radio software in the world".

paKet is a communications program developed especially for use with

Packet Radio, although it can be used for other modes with the

appropriate equipment.  It is designed to run on any IBM compatible

computer system and to communicate through a standard TAPR-compatible

Terminal Node Controller (TNC).

The program is very easy to use, even for newcomers to this field.

As you gain experience with the system, you can explore some of the

more sophisticated features which are provided to meet the needs of

even the most demanding packet operators.  It is simple if that is what

you want; and it is sophisticated if that is what you want.

The modern trend is towards Graphical Environments with the likes

of Windows, or OS/2.  paKet is written to run as a plain DOS program so

if you have an old XT with a CGA or even a Mono display system

gathering dust in the corner, paKet will run on that just fine. Packet

radio is an excellent application for the older systems that are not

much use for anything else nowadays, so it is quite feasible to

dedicate an older system to running paKet 24 hours per day.  What else

do we do with those older systems now?  We used to give them to the

wife or children to use as a Word Processor, play games, etc but many

modern Word Processors need at least 4MB RAM, 10 to 20 MB of disk,

80386, etc... The good games are not much better.   Thank Goodness for

packet radio!  If you are running Windows, OS/2, DESQview, etc you will

find paKet runs quite happily in the background as a simple DOS task.

In addition to providing the usual Terminal Mode where you can send and

receive data via the TNC, the program provides many features and

facilities to enhance the operation of your Packet Radio system. Such

features include:

Full Personal Message System with automatic Mail Forwarding to the BBS

Flashback feature to review earlier communications

Disk and/or printer logging

ASCII text file transfers

Exchange computer programs and other disk files with:

paKet-Protocol, an advanced Binary File Transfer protocol

YAPP Binary File transfer protocol

BayCom Binary File transfers (Digicom supported too)

Full online TNC help for your TNC commands

Online Manual - this entire Manual is available in a pop-up window

Interrupt driven communications, so you don't lose anything even when

paKet is running in a Multi Tasking environment.

Runs on COM1, COM2, COM3 or COM4

Both Hardware and Software Handshaking supported

Multiple windows for up to 10 simultaneous connections

REMOTE Mode - (unattended access)

Access to DOS, your favourite editor, and file LISTer from within paKet

Automatic Script processing..............     etc etc etc

Page 1

paKet is fully configurable so you can customise a system especially to

suit yourself.   Setting up your system is but a matter of a few

keystrokes - see the Configuration Windows section of this Manual for

details.

paKet uses the standard Command Mode (sometimes called Terminal Mode)

of the TNC.  Some other programs require HOST Mode or KISS Mode and so

they will often run only on specific TNCs.   But paKet will run with

any TAPR-compatible TNC and that includes just about all TNCs on the

market today.

paKet is a sophisticated communications system and it may take you a

while to become familiar with all its features, especially if this is

your first look at paKet, but it is designed to be "friendly" and as

easy to use as possible.  You do not need to use all its features if

you would rather keep it simple to start with.  Help is always

available at the touch of the <F1> key if you forget what key to press.

However, you really should read through the entire Manual to get the

most from the paKet system.

In the remainder of this Introductory section I would like to talk a

little about computers and where they fit into the scene.  And I will

cover packet radio principles generally, especially for those who are

new to this mode of operation.  I will also cover some other digital

modes because we often find the same equipment used for packet can also

be used on some other modes as well.

The remaining chapters of the Manual are a reference for the paKet

program, explaining the various part of the system and helping you to

get the most out of it.

This Manual has been fully revised, so if you are upgrading from an

earlier version of paKet you should scan through the entire Manual.

Some of it was found to be still relevant and has been retained without

change; where necessary, sections have been edited to accommodate

improvements in version 6; and some sections have been significantly

rewritten.

I hope you like paKet and find it adds to your pleasure when operating

your equipment in this exciting new generation of radio communications.

Tony Lonsdale, VK2DHU

Page 2

Computers - how it all began

From the mail I have received over the years, I gather there are many

paKet users who might be experienced with radio and electronics, but

are relatively new to Computers.  So I though it would be worthwhile to

provide a brief introduction to Computing.  In this section I will give

you a bit of background and history so you can see how it all came into

being and how we got to where we are today.

Although computers are commonplace now in Amateur Radio shacks, it is a

relatively new technology and there are a large number of Amateur Radio

operators who are embracing this technology for the first time when

they explore the world of digital communications, such as packet radio.

It was during the 1970s that we saw the dawning of this new Computer

Age.  Until then, computers were the exclusive domain of the Scientific

Community and Large Business or Government organisations.  To have a

computer in the home was unheard of, even in the early 70s. Some

amateurs who also happened to be computer enthusiasts, might have been

involved from the beginning, but for most it is only since the mid 80s

that computers have made their presence felt.

Actually it all started back in the 1940s.  That is, if we don't count

the "calculating machines" designed by people like Leonardo da Vinci

(circa 1500), Blaise Pascal (1653), Gottfried Leibnitz (1673) or

Charles Babbage (1820).  But in was in England in the early 1940s that

a group of scientists and mathematicians developed a series of

special-purpose computers including the "Colossus", to crack the secret

message codes being used by the Germans during the War.

The first ones used lots of electromechanical relays and occupied vast

rooms, generating lots of noise and heat.  They were relatively slow

too.

Then came the development of the Valve or Tube.  Remember the old

radios and TVs that had to warm up before they came on?  (You are

showing your age if you do!)  This technology enabled technicians to

get rid of the slow and noisy relay switches and build a much faster

and more reliable computing machine. This was the real beginning of

Electronic Computing as we know it today.

In 1944 at Harvard University in the USA the first general purpose

PROGRAMMABLE digital computer was produced using both relays and tubes.

It was called the "Mark I".

In 1946 ENIAC, the first fully electronic computer with about 18,000

valves/tubes arrived on the scene.  It weighed around 30 tons, covered

150 square yards of floor space (they didn't use the metric system),

and needed its own power sub-station as it used 150 KILOWATTS!   This

like "Mark I" was programmed by hard wiring.  That is, to change the

program, you had to rewire the computer!  ENIAC is reported to have

cost $600,000 which was a lot of money in 1946.  The first stored-

program computers, EDSDAC and BINAC, arrived a few years later.

Page 3

The first commercially available stored-program computer, the UNIVAC-1,

became available in 1951 and was used to process the voting for the

1952 Presidential elections.  It announced President Eisenhower as the

winner less than an hour after voting closed.

The transistor was discovered in 1947 and its potential was realised as

it began to appear in all types of electronic devices from the portable

radio (the "Trannie") to a new generation of computers.  These "solid

state" computers, known as the Second Generation of Computers, appeared

in the early 1960s and were faster and smaller than their predecessors

and even more reliable.

In 1958 Texas Instruments developed the first Integrated Circuit or

"IC", sometimes called a chip because it is manufactured on a chip of

silicon.  These miniature circuits whose detail can be seen only with

the aid of a microscope, can contain many thousands of transistors and

other components in a circuit smaller than the size of a small finger

nail.  With this technology, more computing power could be built in a

smaller space with not only cost savings but also with lower power

requirements meaning less heat and less cooling requirements.  Things

were really happening now!  These computers were known as the Third

Generation.

In more recent times we have seen the development of the VLSI (Very

Large Scale Integration), and ASIC (Application Specific Integrated

Circuit).  These chips pack even more electronic circuitry into a

smaller space than before and one wonders how far we can go with this

miniaturisation.  It is getting to the stage where we have to build the

components smaller so the electric current doesn't have so far to

travel - the closer they are, the less time the electricity takes to

get there.  In other words, the computer can be held up by the Speed of

Light!!!

Computers - Personal Computers

The computers developed up to the mid sixties were not really

classified into big and small computers.  They were all BIG, some

bigger than others.  But in 1965 Digital Equipment Corporation (DEC)

announced the first Mini-Computer, the PDP-8.  This was a much smaller

computer with less computing power but it sold for less than $20,000.

It became one of the Computer Industry's biggest success stories.  At

last ordinary scientists and engineers could get access to computers,

not just for calculations but for controlling and monitoring lab

experiments.

In 1973 the first Micro Processor was developed.  Most of the parts for

the processor section of a computer were reduced to the size of a

single chip.   Developing technology enabled the cost of these chips to

be reduced from hundreds of dollars to around twenty five dollars

within a few years.  The same technology developments produced higher

density memory chips whose prices were also falling dramatically.

It was now possible to produce a general purpose Micro Computer that

was even more powerful than the first Mini Computers for less than

$1,000.

Page 4

It took a little while for the world to appreciate the potential of

these little Micro Processors.  It was the enthusiasts, the hobbyists,

the amateurs, who started to build their own Micro Computer around the

mid seventies.  Note, they had to BUILD it; there was nowhere to BUY

one of these things at that time.  Of course, in those days there was

no software for these computers so any program had to be fed manually

into memory via the front panel switches.  But these WERE enthusiasts!

Around this time, it started to take off.  There were several events

that I believe triggered the whole Personal Computer Industry.

In the mid seventies IBM were promoting the use of a diskette (8 inches

diameter, single sided, 128KB capacity) in some of their business

machines.  A Mr. Gary Kildall, who worked for Intel Corporation, the

manufacturers of the Micro Processor chips, saw the potential of these

diskette storage devices and developed some software to connect them to

his Micro Processor at Intel.  It was far superior to the Cassette Tape

storage medium used until then.  He asked the company to support him in

this software development.  They didn't agree so he left and formed his

own company to develop and market this software.  That company became

known as Digital Research Inc. and the software, CP/M, was the most

popular operating system for Personal Computers for many years and one

of the early success stories.  Digital Research was a rival to

Microsoft in the early 80s and almost established its domination of the

Personal Computer Operating System market.  If just a couple of phone

calls and hurried meetings with IBM had gone differently, Bill Gates

might not be the wealthy man he is today.  Anyway, in the late 70s and

early 80s, CP/M was the dominant system and, because CP/M was available

on a variety of computer systems, it established a standard that

software developers could use when developing their software.

In 1977 there were some other success stories, the development and

marketing of the Apple Personal Computer and the Tandy TRS-80 Personal

Computer.  Here we saw the first of the consumer-targetted Personal

Computers.  You just bought it, plugged it in, and used it.  There was

still not a lot of software available for these things but at least you

didn't have to build it!  For the first time we saw computers that

didn't have front panel switches, flashing lights or spinning wheels.

They didn't look like the computers Science Fiction fans knew so well.

By now some good software was being developed for these machines and

the better software generated more interest in the computers so the

market grew which meant more software development which meant... etc.

The next big event in the development of the Personal Computer was in

1981 when IBM, the giant of the Computer Industry, introduced their own

version of the Personal Computer.  In fact it was called the "PC" and

that term is now used interchangeably with any Personal Computer.

With IBM involved, the business world took a closer look at what was

often regarded as a toy or games machine.  The potential was once again

realised and as the Personal Computers began to find their way into

Businesses, we saw the development of more sophisticated software.

Everyone has been struggling to keep up ever since.

Page 5

Because of the enormous size of IBM, their established presence in the

Computer world, and the prestige associated with the name, everyone sat

up and took notice of the IBM PC.  The design and the technology they

implemented for their PC was rather conservative, but it was "IBM"!

When the system was released, IBM also released full technical details

of the system, including details of the hardware and the firmware.

Some pundits saw this as a foolish move, thinking IBM failed to

appreciate the significance of this "little home computer". (I am sure

the pundits were correct in that IBM did fail to appreciate that

significance). But perhaps, unwittingly, releasing the technical

information proved to be a very successful move, at least for us, the

computer users and for the industry generally.

As a result of this information being freely available (in the Public

Domain if you like) and with the instant success of the IBM model, we

began to see other manufacturers building computers that were based on

the original IBM design.  They called them "clones" but those of you

who experienced some of those early clones will remember they were not

exactly clones at all - many of them bore little resemblance to the IBM

machine.  But it was the start of an industry within an industry.

Nowadays the clones have taken over to the point where we tend to use

the term "Industry Standard" or "Intel machines" (because Intel have a

stranglehold on the "IBM clone" market).  To many people a "PC" is any

model of any IBM Compatible computer.  Some even refer to these systems

as "DOS" machines because of the most popular operating system running

on these computers.  The degree of compatibility is no longer a problem

and all modern IBM-compatible clones conform to the evolving industry

standards.  In some ways, IBM's own PS/2 range are the only ones not

entirely "IBM Compatible!".

paKet requires an IBM Compatible computer, but I am happy to say it

will run on a IBM PS/2 as well.

There are other types of Personal Computer.  The biggest competitor to

the IBM clone models is the Apple Macintosh.  This is a very nice

machine with a Graphic/Mouse interface that "Mac" users claim is an

easier way to use the computer.  The Mac uses a Motorola processor

which is different to the Intel (or compatible) processor used in the

IBM clones.  So the Mac programs do not run on the IBM and the IBM

programs (including paKet) do not run on the Mac.   It is possible to

add some optional equipment to the Mac to allow it to run IBM software

but if that is your aim, you would probably be better off to get an IBM

Compatible maachine to start with.

Other computers include the Amiga which again has lots of nice features

but unless you buy some added equipment, they too will not run IBM

compatible programs.

There is still a very loyal band of Commodore 64/128 users.  This is

perhaps the cheapest way into packet radio, but these systems are

beginning to show their age as they generally offer fewer facilities

than we get from more powerful systems such as an IBM Compatible

computer running paKet.

Page 6

The quality, availability, performance and value for money of modern

Personal Computer systems are constantly improving so that the humble

PC is now widely used in business of all sizes, in schools and

Universities, in homes, and of course in radio shacks all over the

world.

Welcome to the world of computing.  You are certainly not alone!

Page 7

Digital Communications

Digital Communication is not new. Other Modes such as Morse Code, RTTY

and AMTOR have been around for years.  But the newer modes, Packet

Radio and PACTOR offer some significant improvements, perhaps the most

significant is Error Free communications.  Let's have a brief look at

these modes.

Morse Code.

Morse Code is the original Digital Mode dating back to the last

Century! Today, while the debate rages over Morse Code licensing

requirements, more and more stations using this mode have replaced

their hand key and their electronic keyer with a Computer keyboard and

display. The technology is good but even today, no machine can beat the

trained human ear for copying hand sent Morse Code under varying

conditions. The keyboard is a more efficient Sending device though,

for most people.

Perhaps the one advantage of Morse over Packet and the others, is that

a human operator can decode the Morse Code transmissions by ear!

RTTY

RTTY or RadioTeletype is a direct machine to machine communications

mode using the Baudot (or Murray) code.

This mode became popular with many amateurs when surplus TTY machines

became available at a reasonable cost after World War II. These

mechanical monsters provided a keyboard for Input and a paper roll for

printed Output. They were also useful to help hold the house down in

times of hurricane winds - they must weigh a ton.  Video displays were

still too exotic and expensive in those days. It was not until the mid

1970s that we began to see the Video Display come into more widespread

use. (By the way, have you ever wondered why early Program Languages

like BASIC use the command PRINT to display their output?)

When transmitting Morse Code, the transmitter is switched on and off to

make the dits and dahs.  When sending Teletype however the transmitter

runs continuously, sending either of two frequencies conventionally

known as Mark and Space (a reference to paper tape reception of

telegraphy).  The early pioneers found on-off keying was not all that

successful for Teletype signals because of interference from static.

They experimented with FSK, or Frequency Shift Keying and found it

performed much better. With FSK, the transmitter is shifted up in

frequency every time a Mark is to be sent, reverting to the lower

frequency for a Space.  The amount of the shift is usually 170 Hz for

Amateur Radio use although many commercial Teletype signals use other

shifts, notably 425 Hz and 850 Hz.

Many systems use AFSK or Audio Frequency Shift Keying.  When this is

used, the transmitting station generates the Mark and Space audio

tones and feeds them into the transmitter's microphone input.  The

result at the receiving end is that the same audio tones are heard and

processed, whether the transmitting station used FSK or AFSK.

Page 8

When listening to a teletype signal off air, you will soon get to

recognise the familiar warble of Mark and Space tones.

In the amateur shack the TTY machine is usually connected to an HF

receiver or transceiver which the operator tunes so that the received

audio is just the right pitch or audio frequency to trigger the

demodulator's Mark and Space resonators.

If the receiver is slightly off the correct frequency the tones vary

and the text becomes garbled or even lost altogether.  To help the

other station tune the receiver correctly, a RTTY operator can send a

string of alternate R and Y characters viz: RYRYRYRYRY.  This pattern

is chosen as it produces the most frequent and almost symmetrical

alternation of Mark and Space tones, giving the receiving operator the

best chance to tune the receiver before the "real" message starts.

However, even if the signal is accurately tuned, the information can

become garbled or completely lost due to interference, fading, or

noise.  Often, it is possible to make sense of the message even with

parts missing, but RTTY is by NO means an error free mode!

I should point out that similar problems exist for other modes

including Packet.  While information can still fail to get through on

the more sophisticated modes the Error Detecting capability of some,

especially Packet and PACTOR, ensure that the operator will receive

either accurate information or nothing at all.  Usually, where "nothing

at all" is received, the information will automatically be

retransmitted when the radio is retuned, or the interference stops,

(etc) and nothing is lost.

The Baudot code is a 5 bit code and those of you who are familiar with

Binary Notation will know that the maximum number of values we can have

with 5 bits is 32.  That means that each unit of transmission, one

keystroke if you like, can contain any one of 32 possible values.  If

you look up a table of Baudot codes you will see there are 32 values

listed, one code for each letter of the alphabet plus a few other codes

for other things such as a space and a Carriage Return.  But, what if

we want to send a number such as "9" or a question mark?  These are not

mentioned in that table because all 32 codes are already used.

The solution is rather similar to the Typewriter or Computer Keyboard

where we have the Shift key to get various additional codes from the

keyboard.  Most keys will produce a different result if we hold down

the Shift key as we type.  Well, one of those original 32 codes is a

special code known as FIGS (for Figures Shift). The convention is that

when we want to send a number or some other special character such as a

punctuation mark, we can do that by firstly transmitting a FIGS code.

Then instead of using that original table of 32 codes, we have a second

table of codes to use, and that second table includes all ten numeric

digits and various punctuation marks.  Provided both sides of the

conversation observe the convention, the sender can send a FIGS and

start using the second table; the receiver will see the FIGS code and

it too will interpret all data that follows from the second table.

Page 9

With just 5 bits of data we then have almost 64 different codes we can

send and receive.  (I say almost because there is some duplication in

the two tables, including a space and a Carriage Return but that is not

important here).  Even that many codes is not enough to handle all 26

letters of the alphabet in both UPPER and lower case, so RTTY systems

always operate in upper case only.

If we wanted to type a big number (say "13579") we don't have to send

FIGS before every digit.  We send that code only once and the receiver

then will take EVERYTHING we type from now as if it belongs in the

second table.   When we want to revert to the normal alphabetic table

of codes we can send another special code, this one called LTRS (for

Letters Shift).  Then everything goes back to normal, using the

original alphabetic table of codes.

Normally we don't have to concern ourselves with these FIGS and LTRS

codes.  Our computing equipment will take care of those things for us.

We just type away and rely on the system to generate and send those

codes when necessary.

As I mentioned earlier, it is quite possible to lose bits here and

there when receiving a RTTY signal, whether it be because of fading,

interference, frequency drift, or whatever.  One of the big problems

with lost data is the possible loss of a FIGS or LTRS code! Say we had

sent "13579" and then typed "HAPPY BIRTHDAY".   Our equipment would

have sent a LTRS code before the first "H" but what if the receiver did

not copy the LTRS code we sent?   Can you imagine what happens?  As far

as the receiver is concerned we are still sending numbers or other

codes from the numeric table!  So our "HAPPY BIRTHDAY" is going to come

out looking something like "#-006 ?845#$-6".  And EVERYTHING we type

from then on is going to look just as strange until we happen to send

another LTRS code later.   It is for this reason that many systems

include an option to "Un-shift on space".  If you have a multi mode TNC

capable of handling RTTY, you will probably have this option in your

TNC.  If that option is ON then your receiving system will imply a LTRS

code every time it receives a space. So if you seem to be copying lots

of funny numbers from a strong, well tuned signal, try setting that

option ON.

We can overcome some of these problems by using ASCII instead of using

the Baudot code.  With ASCII we can have 128 different codes so we do

not need the FIGS/LTRS codes.  All Personal Computers use ASCII as

their native "language" so it would be a reasonable thing to use.

Although not part of the defined ASCII standard, it has become an

almost de-facto standard in the computer world that an additional 128

characters are available, often called Extended ASCII.  But, despite

these benefits, Baudot continues to rule the airwaves for Amateur and

Commercial Teletype transmissions.

Today, RTTY is still a popular mode especially on the HF bands, and the

advent of the "Glass Terminal", firstly the Dumb Terminal and now the

Personal Computer, has brought this mode to even more operators the

world over. Many specialised RTTY systems were developed for the

Amateur enthusiasts but have been superseded now by the Personal

Page 10

Computer with one of the Multi Mode TNCs which handle RTTY and many

other modes besides.

The latest Computerised RTTY equipment generally allows us to use the

mode better, quieter, more efficiently, using less power and occupying

less space than the old TTY machines, but the limitations of the mode

remain.

AMTOR

AMTOR is a specialised form of RTTY. The term is an acronym for AMateur

Teleprinting Over Radio and is derived from the commercial SITOR system

(Simplex Telex Over radio) developed primarily for Maritime use in the

1970s.

In the early 1980's, Peter Martinez, G3PLX, made several minor changes

to the SITOR protocol and called it AMTOR.

AMTOR improves on RTTY by incorporating a simple Error Detection

technique. The system remains relatively uncomplicated but AMTOR

performs well even in poor HF conditions.  While there can still be

many errors in AMTOR data, the Error Detection helps a lot and the

result is quite tolerable for normal text mode conversations because of

the high redundancy in plain language text.  Certainly much better than

RTTY. But for more critical data such as program code, or even some

technical information messages, NO errors can be tolerated.

There are two modes used in Amtor:  ARQ and FEC.

ARQ

This mode is a little different in that it is a Synchronous protocol,

which means both stations are synchronised to each other's signals.

In ARQ mode (Automatic Repeat Query), sometimes called Mode A, data is

sent in groups of 3 characters. Although each character is only 5 bits

(same as for RTTY), two additional control bits make it up to 7 bits

per "character" and they are set so there are always 4 marks and three

spaces in every transmitted character.  If the receiving station gets

some other combination it knows an error has occurred.  The 40 percent

overhead is considered worthwhile to get some error detection. This

technique can identify a lot of errors that might occur but is not as

thorough as the methods used in PACTOR and Packet which we look at

later.

The receiver responds to each 3 character group by sending either an

ACK (ACKnowledge) code (if OK) or a NAK (Negative AcKnowledge).  Each

time the transmitting station gets a NAK, that 3 character group is

sent again.

If you listen around on the HF bands in the recognised Data Segments of

the bands, you might hear a chirp-chirp sound that identifies an ARQ

transmission. Even when there is no data actually being transmitted,

the transmitting station continues to send idle "chirps" to maintain

the link.

Page 11

Your AMTOR equipment probably supports a Listen Mode too and that

allows you to monitor another ARQ session even though you are not

participating in the session with the usual acknowledgements. Of course

that means you don't get the opportunity to say "NAK" if you don't copy

something properly!

FEC

In FEC mode (Forward Error Correcting), sometimes called Mode B, the

sending station sends each character twice so this mode provides a

means of transmitting to several stations at once.  The receiving

station does not acknowledge the data received. If a receiving station

matches both instances of a character, that character will be printed,

otherwise some error symbol is printed. This mode does not provide for

the receiver to ask for the missing data to be retransmitted.  An FEC

transmission sounds more like a Baudot RTTY signal.

The two stations need to keep in phase with each other so each FEC

transmission is started with several sets of "phasing pairs" and these

are sent at regular intervals even while there is no data being

transmitted.

FEC Mode is still better than ordinary RTTY but its error detection is

not as reliable as that in the ARQ Mode.

AMTOR systems are still limited to the technology of the 60s with

limitations such as the character set and the maximum transmission rate

(100 baud) geared to the mechanical teleprinter. The Error Detection

technique provides improved accuracy over the "vanilla" RTTY mode, but

is still not entirely reliable.  It is perhaps better termed Error

Reduction than Error Detection and has limited application for critical

data.

PACTOR

Amateur Packet Radio has been in use since the early 80s and is used

extensively on both HF and VHF/UHF.  But it is better suited to the

VHF/UHF bands. It was found AMTOR would often perform better than

Packet on the HF bands and so despite the limitations of AMTOR, a large

number of HF operators preferred AMTOR to Packet Radio.

As computerised systems became more popular, amateurs began to explore

more sophisticated options for AMTOR.  Various alternatives were tried

but every time, AMTOR's poor error handling prevented any significant

breakthrough.  And as AMTOR did not handle the standard ASCII code, a

new Protocol was called for.

In 1987 two German amateurs, Hans-Peter Helfert (DL6MAA) and Ulrich

Strate (KF4KV) began work on a new Protocol which they called PACTOR.

Although this new system incorporated the basic structure of AMTOR with

its fixed interval data blocks and corresponding acknowledgements,

PACTOR also combines important characteristics of Packet Radio.

Page 12

PACTOR is designed especially for HF operations.  There is no reason

why it could not be used on VHF/UHF, but it does not offer significant

advantages over Packet Radio on those bands.  It is on HF that the new

protocol shines.

The block length of PACTOR is fixed at 1.44 seconds with significantly

lower overheads than those of AMTOR.  This was found to be the most

suitable compromise for HF operations with the relatively short block

having a better chance of survival through difficult conditions and

allowing short break-in times. The protocol provides for automatic

adjustments to the baud rate too, so when the going gets really tough

it will reduce the speed; and when things improve it will increase the

speed to improve the throughput again.

Huffman data compression is used in PACTOR and that adds to the

effective performance.  The Huffman code is based on the assumption

that in plain language text there is a high content of frequently

occurring letters such as E, A, etc. It has been found that a block of

normal ASCII text can often be compressed to half its original size

without any loss of data at all.

PACTOR offers high quality error detection, which overcomes the main

weakness of AMTOR.  PACTOR uses a longer acknowledgement signal as well

as the CRC checksum code used in Packet Radio to detect errors in the

transmission.  I will discuss Error Free Communications with CRC

checksums in the next section on Packet Radio, so we'll have a look at

that in a moment.  The comments there apply to both PACTOR and Packet.

Other features of PACTOR include an improved error correction system

which has become known as "Memory ARQ".  I won't go into the full

detail here, but in summary it improves the performance of the

receiving system in weak or noisy conditions.  For example in any

digital system, a signal is reduced to a series of bits which are

either a binary 1 or 0, but this does not distinguish between an input

signal of a millivolt or 10 volts!  So long as the analog input signal

is above the threshold, the digital output data is the same!

The Memory ARQ system remembers not only the binary 1s and 0s, but also

the incoming analog values, so if the received block reveals errors,

the analog values from the repeated blocks can be combined into a "sum"

block and subjected to another CRC error check.  While this system is

not perfect it can provide error free communications in conditions that

other modes, including Packet, could not handle.

Page 13

Packet Radio (AX.25)

Perhaps the most important feature of Packet Radio is the Error

Detection system it uses to give us Error Free communications.  PACTOR

is the only other mode that offers such data integrity but Packet

offers other advantages that even the new PACTOR does not support.

Packet offers simultaneous frequency sharing by several stations,

multiple repeater operation ("digipeating") for longer distance

communications, and Networking.

Error Free Communications.

The Error Detection (and Correction) is perhaps the major benefit of

Packet Radio over the older modes.  (The newer PACTOR mode shares this

feature).  With Packet Radio using HDLC and the AX.25 protocol, data is

checked and verified before being accepted by the receiver's TNC.

HDLC (High-level Data Link Control) is a procedure defined by ISO, the

International Organisation for Standardisation, for the handling of

Error Free frames of information over a communications link.  AX.25 is a

set of rules defining the format and content of packets and how they

are handled. It is not the only Protocol for data communications but is

the recognised standard in the world of Amateur Packet Radio.

Fortunately we don't have to know the details of either HDLC or AX.25

to use Packet Radio. (I read the AX.25 Protocol Specifications and am

still not too sure what it said!)  The hard work is done in a special

device called a TNC (Terminal Node Controller) which is a small

computer in itself, programmed to perform the HDLC Error Handling and

many AX.25 functions to make it easy for us.

The Error Detection, similar to that used in PACTOR, is much more

sophisticated than that used in AMTOR so the result is significantly

more reliable.   Each block of data (packet) contains various control

information including a CRC checksum.  (CRC is Cyclical Redundancy

Check - true!)  This checksum is a value calculated by the sending

station based on the content of the data contained in this packet.

When the receiving station receives that packet, it performs the same

calculation on the data it received and it compares its result with the

checksum it received.  If there was something wrong with the received

data, the checksums will not match and the receiver knows there is

something wrong with this block of data.  The chance of an undetected

error getting through is less than one in a many millions. So, for all

practical purposes Packet Radio and PACTOR are considered completely

error free. If any data comes through the system to your computer (to

paKet in your case) you can be sure that what you see is exactly what

the other station sent.

Frequency Sharing.

Packet Radio, like the commercial X.25 Packet Switched systems, allows

several simultaneous point to point connections to share the same

frequency.

Page 14

In Packet Radio whenever we send a message to another station, our TNC

builds a Packet, like an envelope, around our message adding details

such as the callsign of the station this message is addressed to. Then,

if the frequency is clear, the Packet is transmitted into the airwaves.

Our TNC listens to the audio coming from the radio receiver and if

there are any other Packet Radio stations using the frequency, it waits

until there is a lull in the transmissions before transmitting our

Packet. At first, you might wonder how long it has to wait, thinking

the other stations might be going for hours! However, in practice, a

Packet might take only a fraction of a second to transmit - a

relatively short burst of time compared to the time it takes us to type

a message. Unless the frequency is really busy, you might not even

notice the delay!

The Packet Radio frequency in use also carries messages for many people

at the same time. When receiving, your TNC looks at every Packet

received, checks the address on each Packet and identifies any Packets

addressed to you.

Any Packets not intended for you may be ignored by your TNC so you see

on your screen only those messages addressed to you. This way you need

not concern yourself with all the other information flowing around on

the frequency, and are free to concentrate on your contact with the

other station.

If you wish, you can ask your TNC to display everything it receives.

This is called Monitoring.  Every Packet contains the AX.25 Header

("envelope") which identifies the station who sent it and the station

it is addressed to.  The Header contains other information too but we

won't get into that just now.  When monitoring, your TNC will send to

the Computer the Headers as well as the contents of the messages so you

can see who sent what to whom. Then you will see how the AX.25 system

is able to handle lots of different conversations on the frequency at

the same time.  The other modes do not offer this capability.

Multiple Repeater Operation.

When we want to establish contact with some station within range we

issue a CONNECT command using that station's callsign, and provided

that other station hears our call, it will acknowledge and we can

simply type messages to each other (or exchange files, etc).

To illustrate, I'll use a callsign of ZZ1FRD for the other station, and

for these illustrations, the "messages" that I put inside brackets are

internally generated, not typed:

Page 15

������Ŀ                    ������Ŀ

� Our  �                    � Fred �

�System� ------------------ �ZZ1FRD�

�      �                    �      �

��������                    ��������

CONN ZZ1FRD

(Connect command)-->

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD)

Hi there Fred.-->

<--(ACKnowledge)

If the station we wish to communicate with is outside radio range, we

may establish the connection via one or more intermediate stations

which act as repeaters.

A station providing this repeater function is commonly called a

Digipeater ("Digital Repeater"). It is part of the AX.25 design that

any TNC may be used as a Digipeater so, unless you deactivate this

facility in your TNC, your system too can be used as a Digipeater.

Say we are trying to establish a connection with our friend Fred who

is out of radio range.

������Ŀ                                         ������Ŀ

� Our  �                                         � Fred �

�System� ----------- (too far away) ------------ �ZZ1FRD�

�      �                                         �      �

��������                                         ��������

CONN ZZ1FRD

(Connect command)-->

(no response)

There might be another station in between who can "hear" both ZZ1FRD

and us.  So, we can call ZZ1FRD "via" the intermediate station. Our

TNC, using the AX.25 logic, will transmit a "Connect" message with both

those callsigns; the intermediate station will see its callsign used as

a Digipeater so it will capture the message, store it in its memory for

the time being, and when the frequency is clear it will retransmit that

same message. This time ZZ1FRD will hear it and will respond, again

"via" the Digipeater station.

Let's have a look at that situation with an illustration:

Page 16

������Ŀ                ������Ŀ                 ������Ŀ

� Our  �                � Digi �                 � Fred �

�System� -------------- �      � --------------- �ZZ1FRD�

�      �                �XY4DIG�                 �      �

��������                ��������                 ��������

CONN ZZ1FRD via XY4DIG

(Connect command)-->

(Connect command)-->

<--(ACKnowledge)

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD)

Hi there Fred.-->

(Hi there Fred.)-->

<--(ACKnowledge)

<--(ACKnowledge)

So, we now have a point to point connection with Fred as if it were a

local contact. It just takes a little longer because each message is

transmitted twice.

The AX.25 Protocol provides for up to eight Digipeaters to be used for

a single point to point connection. So for a more distant destination,

we could call that station via two or more intermediate stations.  Each

of those Digipeaters will store our message and retransmit it as soon

as the frequency is clear.  This allows distances of hundreds, maybe

thousands, of kilometers (even miles!) on VHF or UHF frequencies.

Note the Digipeater does no intelligent processing with the messages -

it simply retransmits whatever it receives from either station.

Networks.

Following on from the previous discussion on Digipeating, I should

mention Networks.  The term "Network" is often used in different ways

but I want to mention just two of those here.

Firstly the term is used to describe an advanced form of Digipeating

where we can make contact with distant stations without necessarily

knowing what intermediate stations are retransmitting our messages. The

idea is like our telephone Network where we make a call by dialling the

number of the person we wish to speak to. We don't know nor care how

the call is routed, whether it goes via cable or via satellite, or how

many telephone exchanges it goes through. We are simply interested in

getting our call through to the other person, no matter how it gets

there!

Networking systems in use in the field of Amateur Packet Radio include

TCP/IP, NET/ROM, and ROSE. Unlike the widespread acceptance of the

AX.25 Protocol, we do not have any one Networking system preferred over

all others in use. There is still lots of healthy debate going on and

developments continuing. It may be that we will have to wait for a new

generation of Networking System before a single, global system is

implemented.

Page 17

TCP/IP

TCP/IP is a set of protocols developed to allow communications across a

network.

TCP (Transmission Control Protocol, but often called Transport Control

Protocol) is that part of the system which is responsible for getting

our data from one place to the next. It will resend anything that did

not get through and it will divide large amounts of data into smaller,

more manageable packets, or "datagrams" and make sure they all arrive

safely.

IP (Internet Protocol)  establishes various rules for formatting,

routing and processing our data.

The various protocols in the TCP/IP family include:

- FTP (File Transfer Protocol).

This is used to exchange files with another FTP user elsewhere on

the Network.  Any kind of data may be transferred using FTP,

including programs and other Binary Files.

paKet uses the standard YAPP (or the advanced pP) protocol for

Binary File transfers.  FTP is not currently supported, but you can

use the TCP/IP Network to exchange files with another station

running paKet or one that supports standard YAPP protocol.

- TELNET.

This facility allows you to conduct a person-to-person chat with

another user on the Network.  It allows Remote Login which means

you can log in on any other computer system on the Network.  While

connected to that station, anything you type will be sent to that

screen and anything the other guy types will appear on your screen.

As far as paKet is concerned, once the connection is established,

it is a one-to-one connection and it does not matter whether the

other station is using TELNET, paKet, or any other Terminal

Emulation program.  This mode is simply for the exchange of plain

text information.

- SMTP (Simple Mail Transfer Protocol).

This allows you to send messages to other TCP/IP users on the

Network. The idea is to leave a message in your own system and the

SMTP will automatically feed it into the Network for Forwarding to

the destination.

Although paKet is not a TCP/IP implementation, it does perform a

similar function using the BBS Network (discussed below).  Leave a

message in paKet's Mailbox and it will be automatically sent to the

BBS Network for onward forwarding to the destination.

Page 18

- POP (Post Office Protocol).

This is a variant of the SMTP mail handling facility where you may

nominate some other system as your "Post Office" so that any mail

addressed to you will be stored there for you.  Then, when you run

POP, your mail will be automatically Forwarded to you from the Post

Office station.

paKet will do this automatically for you, not with a TCP/IP "Post

Office" but with your local BBS.  This is discussed in the section

on paKet's Personal Message System.

- PING (Packet INternet Groper)

A PING allows you to put out a feeler to see if some particular

station is on the air.  If that station is there, you will get a

response, telling you the time it took for the round trip.

There are lots of other protocols used in the TCP/IP family but this

should give you an idea of the complexity and sophistication of this

advanced system.

TCP/IP is widely used on the Internet, a world wide network of

computers.  In fact it is a collection of networks, all of which are

connected to each other.  I heard there were over 20 million people in

125 countries with access to Internet.   Of course I am talking about

the telephone subscribers with their landline modems, not Amateur Radio

operators, but it is a huge system by any standards.   Users can chat,

exchange programs and files, conduct research, explore the world,  play

games, or just engage in some meaningful conversation with another user

on Internet - some have even been seen courting and proposing!  It is

much the same as we can do on Amateur Radio but on a wider scale.

(Must admit though I have not seen much courting going on around here

lately - not on air anyway!)

Each TCP/IP user is allocated a station address which consists of a

four part number.  For example, my TCP/IP address is 44.136.56.3.  This

number is not just a random number but is carefully managed and

allocated by IP Address Coordinators all over the world.  The first

part is 44 which is the number allocated to Amateur radio stations. The

second part, 136 for me, identifies the Country (or a State in the

USA), the third part identifies a region or area and the last part is

one station within that region.

In Amateur Radio the program mostly used to access the TCP/IP system is

NOS developed by Phil Karn, KA9Q although a variant called GRINOS has

been developed by Gerard van der Grinten, PA0GRI.  paKet can be used to

access a TCP/IP system but it does not currently implement the various

TCP/IP protocols mentioned here.

Page 19

NET/ROM

NET/ROM is Networking Protocol that is used to link a number of

stations together and to provide routing services for users connected

to the Network. TCP/IP often uses NET/ROM to move its data from one

place to another.

The NET/ROM system is somewhat automatic because it works out for

itself what other NET/ROM stations are available and so it can

establish its own message routing without human intervention.  It does

this by broadcasting coded messages at regular intervals for the

purpose of announcing itself to other NET/ROM stations within range.

Then all NET/ROM stations, called Nodes, can listen on air to determine

what other Nodes are available. Each one builds and maintains its own

listing of other Nodes.

If we wish to connect to our friend Fred, a distant station, where

there is NET/ROM Network available, we could issue a connect command to

a NET/ROM Node within reach.  Once connected to that Node, we usually

are presented with a menu which includes (among other things) a Node

Command.  If we type "N" that system will send to us a listing of all

the Nodes it currently knows about.  It is also possible to get a list

of the various Routes this system can use to get to another Node in the

list. This includes a a "Quality Factor" which gives us an idea of the

quality and therefore the reliability of the radio links from one place

to the next.

While we are connected to this Node, we can then give a Connect command

to the Node, asking it to make a call to another station or Node it

knows about. Let's illustrate, assuming there is a NET/ROM Node with a

callsign of N0DE within range.

������Ŀ                ������Ŀ                 ������Ŀ

� Our  �                �NETROM�                 � Fred �

�System� -------------- � Node � --------------- �ZZ1FRD�

�      �                � N0DE �                 �      �

��������                ��������                 ��������

CONN N0DE

(Connect command)-->

<--(ACKnowledge)

(*** CONNECTED to N0DE)

CONN 1 ZZ1FRD-->

<--(ACKnowledge)

(Connect command)-->

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD)

(*** CONNECTED to ZZ1FRD)

Hi there Fred.-->

<--(ACKnowledge)

(Hi there Fred.)-->

<--(ACKnowledge)

Page 20

All that just to say Hi to Fred?  It is not so bad really.  Let's have

a look at that.  Firstly we send a connect command to the NET/ROM Node

(N0DE) and when that acknowledges, we have established connection with

that station.  Then we send "CONN 1 ZZ1FRD" to the Node.  Although this

looks similar to the connect command we just issued, that string not

processed here as a command - it is just a string of characters that is

sent to the Node. That string asks the Node to issue a connect to

Fred.  The Node often has more than one port (that is more than one

radio channel) available so we must tell it which port to use. Some

Nodes even have HF ports available for convenient DXing.

Note the usual protocol is for a station to acknowledge anything it

receives correctly.  If not copied accurately, it will ask for it to be

sent again.  In this case the Node is doing more than just plain

Digipeating - it is acknowledging each and every packet before

retransmitting it.

Fred's system hears the Node's call and establishes the connection with

the Node.  The Node then connects us to Fred, just like a telephone

switchboard operator, and from now everything sent to or received from

Fred, will be transmitted via the Node.

This example uses just one Node between Fred and us, but it could be

much more complex than that with several Nodes involved in a multi hop

link.  With this simple example, it is rather similar to the Digipeater

example we saw earlier (using XY4DIG).  Note however, that when we send

anything to the Node it acknowledges each transmission immediately it

is received.  Compare this with the Digipeater connection. There the

acknowledgements were not issued by the digipeater - it simply RELAYS

all messages including Fred's acknowledgement.

Imagine a multi hop link:

������Ŀ           ������Ŀ          ������Ŀ         ������Ŀ

� Our  �           �NETROM�          �NETROM�         � Fred �

�System� --------- � Node � -------- � Node � ------- �ZZ1FRD�

�      �           � N0DE �          � S0FAR�         �      �

��������           ��������          ��������         ��������

Hi there Fred.-->

<--(ACKnowledge)

(Hi there Fred.)-->

<--(ACKnowledge)

(Hi there Fred.)-->

<--(NAK)

(Hi there Fred.)-->

<--(ACKnowledge)

I won't illustrate all the connecting dialog, but just assume we

had connected to N0DE, then asked it to connect to a second Node, say

S0FAR, then asked that second Node to connect to Fred.

Page 21

Now, each message we send to N0DE is sent to S0FAR which in turn sends

it on to ZZ1FRD.  If Fred's system copped some interference and failed

to copy our message correctly, we don't need to know about it because

the last link in the chain (S0FAR - ZZ1FRD) will look after it by

retransmitting the message.

If we were using simple digipeaters here, and we failed to receive

Fred's acknowledgement, we would have to retransmit the message to N0DE

which would have to send it again to S0FAR which would have to send it

again to ZZ1FRD!   The Network method is much better: it is more

reliable and involves less traffic.

In this last example it may be the S0FAR Node is in some far away

place, maybe in some overseas country!  Provided the NET/ROM Network

knows about that Node, we do not concern ourselves with that detail.

Let the Network worry about that.  If it is in the Nodes list we can

simply issue a connect command and wait for it to establish the

connection!

And once we are connected to that far-away Node, we can ask it for ITS

Node list then connect to some other exotic place from there!   It's

exciting stuff.

ROSE

ROSE is an acronym for "RATS Open System Environment".   No, no, RATS

has nothing to do with the Mouse used on the Macintosh - RATS is

another acronym.  RATS is the Radio Amateur Telecommunications Society

of New Jersey. They must work for IBM, with all these acronyms.

Members of this Society include Tom Moulton (W2VY) who is credited with

writing the ROSE code, and Gordon Beattie (N2DSY).  These guys and

others developed the ROSE system and today it is widely accepted and

gaining acceptance all over the world.  Some say it is more advanced

than the well established NET/ROM systems.

ROSE uses Node to Node packet acknowledgements, similar to that used in

NET/ROM, as illustrated above. In fact, there are many similarities

between ROSE and NET/ROM.  The differences that concern us are in the

way we access it.

With ROSE we do not need to know the callsigns on various Nodes we want

(or need) to use.  We simply need to know the callsign of our local

ROSE Node, which is where we enter the Network, and the remote

station's network address, which is where we leave the Network.

The network address is a number, based loosely on the telephone area

code and local dialling prefix.  So to establish contact with a distant

user, let's say it is still our friend Fred who now has access to the

ROSE network.  We need to know the Network Address of Fred's local ROSE

Node.  That's all.  The rest is up to the ROSE Network and the people

who set it up for us.

I think we need another illustration.

Page 22

I have shown below a diagram of a ROSE Network with four Nodes

showing.  Each of these four Nodes has a Network Address which is a 6

digit number.  We are within range of our local Node, shown with a

callsign of AA1NET and a Network Address of 658300.  Fred, ZZ1FRD, can

access the ROSE Network via his Node, shown with a callsign of XY2LOC

and a Network Address of 675200.  The other Nodes do not concern us

here - I just included them to indicate there can be lots of entry

points in the Network.

���������������Ŀ

�               �

������Ŀ     Node  �               �Node        ������Ŀ

� Our  �     658300�     ROSE      �675200      � Fred �

�System� --------O �               � O--------- �ZZ1FRD�

�      �     AA1NET�    Network    �XY2LOC      �      �

��������           �               �            ��������

�               �

�����������������            ������Ŀ

������Ŀ  Node      O              O  Node      �other �

� other�  598550   /                \ 476250    � user �

� user �--------- /                  \ -------- �      �

�      �                                        ��������

��������

If we wish to call Fred, we need to know the callsign of our own Node

and Fred's Network address.  So to make that connection we enter the

following command:

CONNECT ZZ1FRD v AA1NET, 675200

The dialog might go something like this:

OUR SYSTEM                      OUR ROSE NODE (AA1NET)

----------                      ----------------------

CONNECT ZZ1FRD v AA1NET, 675200 -->

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD via AA1NET,675200)

(Call being Setup)

*** delay in here while ROSE make the call through to Fred ***

(Call Complete to ZZ1FRD-0 @ 5050675200)

Hi there Fred.-->

<--(ACKnowledge)

(Hi there Fred.)-->

The first part of the connect command is similar to any other connect

command with the callsign of the station we want to talk to and another

callsign of a station we want to digipeat our call.   However in this

case the AA1NET is not just another packet radio digipeater: this one

is a ROSE Node and it behaves a little differently.  It accepts our

call and responds immediately with a CONNECTED message and tells us our

call is being processed.

Page 23

The ROSE system knows about all the other Nodes in the Network so it

knows how to get to 675200.  We just have to wait while it does the

job.  ROSE can automatically attempt alternative paths to that

destination if one path is not working properly.

It does not matter if Fred is right next door, across the other side of

the Country, or even overseas.  Provided we both have access to the

same ROSE Network, the connect command is the same!  We do not need to

know what happens inside "The Network" - there might be one hop to get

to his Node or there might be a dozen hops.

Once Fred's system acknowledges our call, our ROSE Node tells us the

Call is Complete and connects us "directly" to Fred.  Now we can talk

to Fred as if it were a simple, local contact.

For this connection we do not use our own Network Address (658300) but

we might like to tell Fred what it is because he will need that number

if he wants to call us via the ROSE Network.

The Network routes that are used within the ROSE Network are defined by

various ROSE Network Managers each of whom have accepted responsibility

for their own Nodes in the Network.  So if some path is bad or if some

other new Node is installed, the Network paths can be changed and we,

the users, need never know about it.

NET/ROM systems can accommodate changes in the Network too and they

achieve this automatically by broadcasting those coded messages we

talked about earlier.  That is good, but in some situations people

complain at that added activity on a busy frequency!  (Some people

complain about anything though).

Both ROSE and NET/ROM have advantages and I do not wish to get involved

in the debate as to which might be better.  Maybe one day, someone will

create a new generation of Network system that accommodates the

advantages of both, as well as enhancing functionality?

Page 24

The BBS Network and Mail Forwarding

The other meaning of "Network" I want to mention is the BBS (Bulletin

Board System) Network which provides a Global Electronic Mail Service

for us all.

A BBS, sometimes referred to as a PBBS (Packet-radio BBS), is a

computer based Packet Radio system, probably run by some generous local

enthusiast or the local club, which provides unattended (usually 24

hours per day) access for the purpose of storing and forwarding

messages. They are often used as a central store for data files too so

we can transfer programs and other files to or from the BBS.

Telephone BBS systems have been around for years, before Packet Radio

came into being, so you might be familiar with those. The concept is

the same, but we use radio as the medium instead of the telephone.

The main role of the BBS now is to provide a Mailbox service for local

users.  We could call up the BBS, ask it to display any messages it was

holding, read any messages addressed to us and maybe leave a reply to

someone.  This is very useful for sending messages to friends whether

they are currently in their shack or not. The BBS mailbox allows us to

leave a message addressed to another station and when that other

station connects to the BBS they will be able to read our message.

Why not just send that message directly to that "someone"?  Well, for

one thing, they might not be around at the time.  They might not have

their system switched on.  They might be out of range or it might be

simply more convenient to leave the message for them on the BBS and let

them read it later when they check the BBS.

The Mailbox facility became so popular that most TNCs now offer a

private mailbox.   paKet too has a Mailbox (I called it the Personal

Message System - PMS for short) similar to that in the BBS, so you and

others can send and receive messages without needing the BBS to store

those messages.

So the role of the BBS is changing.  But it still has a significant

role to play as part of a Global "Network" of similar BBS systems.

Page 25

Mail Forwarding

It all started when Hank Oredson (W0RLI) developed an automatic

Forwarding system between PBBS systems. Then we could leave a message

on our local BBS and if that message was addressed to someone in an

adjoining area the BBS would, at regular intervals in the day or night,

automatically send our message to that other BBS.  As the "Network"

developed, our mail would move to our neighbouring system and that

system would then Forward it to its neighbour and so on.

This Automatic Forwarding system has been implemented on all Packet

Radio BBS systems now and, although some have added some enhancements,

they are all based on Hank's original standard.  paKet's PMS conforms

to this standard and can participate in the Forwarding with the BBS

Network.  So you can simply leave a message in your paKet PMS and it

will be automatically sent to any packet destination anywhere in the

world.  And paKet can accept any incoming Mail directly into your PMS

without any action on your part. It will just appear there!

However, I see paKet as a PERSONAL system and as such does not belong

in the BBS Network as a link in the Forwarding chain.

The role of the BBS and where paKet fits in

The BBS has a vital role to play as a part of the BBS Network. It is

the role of your local BBS to participate in the automatic Forwarding

of messages, managing the flow of Mail via the BBS Network to and from

other participating BBS systems.

Each BBS that forms a link in the chain will receive Mail from an

adjoining station, and then will Forward that Mail on to the next link

in the chain.  That way, the Mail can find its way all around the

country and all around the world.  Personal Mail addressed to a

particular BBS will hop from one BBS to the next until it eventually

arrives at its destination.

Every BBS in the Network has the opportunity to store those messages so

all local users of that BBS can access them at their leisure.  Of

course, what is STORED on your BBS is entirely up to the owner of that

system.  Some BBS Sysops like to be selective as to what messages are

kept, so you might find some messages "missing" from your local BBS.

But if the BBS is participating as a link in the Forwarding Network it

should be expected to Forward EVERYTHING on to the next link in the

chain, including any messages that might not be stored for its local

users.

Although paKet supports the standard Mail Forwarding protocol, its

role as a PERSONAL system is to manage YOUR Mail, interfacing only with

your local BBS and not with the Network generally.  So any mail you

want to send through the Network or any mail you receive from the

Network, will be channelled through your local BBS.

Page 26

Bulletins

A Bulletin is a general message that is broadcast to anyone and

everyone rather than a personal message addressed to one particular

station. Bulletins may be "addressed" to some topic such as IBM or

HUMOUR or HELP or MORSE... The topics help us to get some idea of what

the Bulletin is about so we don't have to read every message that

appears on the BBS. Bulletins are usually forwarded to every BBS system

in the area, so if there is a Bulletin around, it will probably,

eventually get to your local BBS for you to see.

Personal Mail is usually addressed to a station @ (AT) some BBS

address. For example the local BBS in this area is run by Arthur

VK2ATM. So if you want to send a message to me you would send it to

VK2DHU @ VK2ATM. There must be millions of Amateur Packet Radio

callsigns in use around the world but there are a limited number of BBS

systems so it is easier to track down VK2ATM than it is to find VK2DHU.

This problem is further reduced with the introduction of Hierarchical

Addressing which is a system of specifying more than just the BBS

callsign, adding State, Country and Continent (or Region) details. For

example, my full Packet address according to today's convention is

VK2DHU @ VK2ATM.NSW.AUS.OC (meaning this station is in the State of New

South Wales, in Australia, in the Oceania Region).

Bulletins may be addressed to one particular BBS (eg to MORSE @ VK2ATM)

but more commonly they would be issued to cover to a wider area (eg to

MORSE @ ASIA). Then the Bulletin would be passed on from one BBS to the

next within that area, and every BBS would have that Bulletin available

for users to read.

Page 27

Other Communications Modes

paKet is a COMMUNICATIONS program designed to communicate with a TNC or

Modem.  It provides communication facilities between the computer and

the TNC but does not perform the decoding of packets - that is done in

the TNC!   Neither will paKet decode any of the other communications

modes such as Morse, RTTY or AMTOR.  These modes too, are decoded by the

TNC.

Not all TNCs handle these other modes so if this is what you want to do

you will need one of the Multi-Mode TNCs available (such as the KAM,

MFJ-1278, PK-232 or one of the DSP models) which decode these other

modes.  If you have one of those TNCs, you can use paKet for modes other

than Packet Radio.

The paKet Documentation.

This documentation is supplied in eleven compressed files to make it

easier to manage when printing. Each of the eleven files or Parts is

named MANnn.DOC where the "nn" is the Part.

These files may be combined into a single DOC file if you have the

space. This would be the preferred solution as you can then have the

entire Manual available in a pop-up window during paKet operations

(refer the <Alt-F1> key later).  The supplied INSTALL program, will

attempt to combine these files into a single file PAKET.DOC if you had

the space available on the output drive at the time of Installation.

Have a look at the Table of Contents for details on what you can expect

in the various Parts.

Starting the paKet program

The INSTALL program and the INSTALL.DOC file will help you get the

program installed and ready to run.

To start the program, change into the paKet directory (or log on to the

diskette where PAKET.EXE is installed).  Type:

PAKET

That will use the standard PAKET.CFG configuration file.  If you have

more than one configuration file, for example if you have one set of

parameters for VHF and another for HF ... and so on... you can specify

a different CFG file on the command line.  Eg:

PAKET HF

PAKET VHF

To create a different CFG file, eg: HF.CFG, copy the standard PAKET.CFG

to HF.CFG using the standard DOS COPY command.  Then start the program

with:  "PAKET HF" and make the desired changes through paKet's Online

Configuration Windows.   All changes will be saved into that CFG file

ready for next time.

Page 28

PART II - paKet's WINDOWS

paKet's Windows - Overview

paKet uses a number of windows for all its operations.

Do not confuse this with the popular Microsoft Windows product. You do

not need any other software to run paKet.

Some of paKet's windows are there permanently, others will pop up on

demand. For example, the Communications Window is always there although

it might be partly hidden by other windows at times.

Some windows do not look like "windows" because they do not have a full

box-style border. For example, the Communications Window uses all 80

characters for its communications data, so there is no room for a border

line down the sides. It does, however, have a border line at the top

known as its Header Line.

paKet uses whatever text screen/video mode is in operation when it is

called.  When using an EGA or VGA display, if you were using 40, 52 or

60 lines and paKet is then called, you will find paKet now has a larger

Communications Window where vast amounts of information can be viewed at

once, and larger type-ahead areas (as detailed below) become practical.

A larger monitor is desirable.

If you are running paKet under Microsoft Windows Enhanced Mode, you can

change your DOS session to say 53 line mode using a public domain

utility or perhaps with the video utilities that came with your system,

before starting paKet. Then if you start paKet in a full screen "Window"

you will see the large display and small "fonts".  However when you

press <Alt-Enter> to reduce it to a Windows' Window, you will see it is

showing only part of the paKet display, even when you enlarge that

little window by dragging the corners.  The trick here is to select a

new Windows font - click the mouse on the box in the top left corner and

change the Font to say 6 x 8 and try again.  Experiment with your system

to find a combination that suits you.

Each of paKet's Windows is described, in alphabetic sequence, in the

following sections. An example of the window is shown and the purpose

and operation of each is discussed.

Page 29

Binary File Transfer Window

Example:

����������������� paKet Protocol File Transfer ����������������ͻ

� Receiving File  :       VIDPARK.ARC                           �

���������������������������������������������������������������Ķ

� File Size       :    3,375        Time to complete :  00:00:04�

� Bytes Received  :    3,036            Elapsed time :  00:00:32�

���������������������������������������������������������������Ķ

� Progress        :       89%       Bytes per second :     94   �

� �����������������������������������������������������>������� �

���������������������������������������������������������������Ķ

� Last Packet Recd:  DT - Data Packet                           �

� Current Status  :  Receiving data                             �

������������� Press Esc to abort the file transfer �������������1/4

Description.

During the transfer of a Binary File, this window is displayed to keep

you informed of progress. It is used for all types of Binary Transfer,

the top line indicating the Protocol or method in use. That could be:

paKet Protocol (as shown in this example),

YAPP,

BayCom (including Digicom) or

RAW Binary (no protocol at all).

In the example above, paKet is receiving the file VIDPARK.ARC from the

other station; the size of that file is 3,375 bytes and the display

shows the transfer is 89% complete with 3,036 bytes received at this

stage. paKet has calculated the bytes per second at 94 since the

transfer started, and as it knows how many bytes still to go, it can

calculate the estimated time to complete (4 seconds in the example).

The arrow under "Progress" is a "strip meter" or histograph which gives

a proportional indication of the percentage complete.

The "Last Packet Recd" field indicates some useful information at the

beginning of the file transfer as you can see how the handshaking is

progressing. During the transfer though, this does not change much.

The "Current Status" field too, should not change much during the

transfer, but it helps to see what is happening at the beginning and

end.

While this window is displayed you should not be using the paKet system

for anything else.  You cannot conduct any communications with another

station at this time because we are restricted to a single binary

transfer at a time.  And while the TNC is in Transparent Mode we cannot

communicate with anyone else in Converse Mode.  So you'd better go find

something else to do.  Of course with a multi tasking system, you are

free to go into another session/Window and run some other task while

this transfer is proceeding, but paKet is considered "occupied" and

should be left until this transfer is complete.

Page 30

If you would like to abort the transfer before it completes, press the

<Esc> key.  paKet will attempt to finish off this transfer in a tidy

way, firstly by sending an appropriate <CANCEL> code to the other

station and waiting for its acknowledgement.  If that fails (maybe we

have lost contact with that station?) then you can press <Esc> a second

time to force an immediate abort and return to normal communications.

Page 31

Communications Windows

Example:

���   50 ���������� Window 1 - Stream A - VK2DHU ����Paused with ScrLock���

cmd:echo off

ECHO     was ON

ECHO     now OFF

cmd:

Description:

These windows, one for each connected stream, contain all

communications information flowing to and from the TNC. All data sent

to the TNC is displayed in a highlight colour or shade, while all data

received from the TNC is displayed in the normal colour or shade.

This window is 80 columns wide, so there is no border as such although

the top line of this window is referred to as the Communications Window

Header Line.

Some TNCs use the term "channel" while others use "stream" but this

Manual will use "stream" to identify the different user connections.

The Communications Window Header Line.

Each Communications Window's Header line identifies the current Stream

and also the callsign of the station you are connected to (or "Not

Connected" as appropriate).

In the example shown above, the Header line indicates you are connected

to VK2DHU on stream A in the first Communications Window. If the

callsign does not appear to be correct, you can verify it with the

<Alt-V> function, which is explained in detail in the "Keyboard

Commands" section.

On the left of this header line you will see a counter displaying the

number of characters waiting in this window's input buffer (50 shown in

the example). Usually the system will process the input characters as

they are received so this part of the display would be blank. However,

it is possible to have characters build up in the buffer if you are

receiving the input characters faster than your computer can process

them, or if processing has been suspended for any reason.

If the data is coming in too quickly for you to read, you can pause the

display by pressing the <ScrollLock> key.  While paused, a reminder

message is displayed to the right of the Header Line, as illustrated in

the example above.  To "un-pause", press <ScrollLock> again. This

action does not stop the data coming in from the TNC - the data is

captured and stored in paKet's input buffers until you are ready to

continue.

Page 32

Multiple Communications Windows.

You may configure paKet with up to 10 separate Communications Windows.

When you are connected to more than one station, a separate

Communications Window is used for each connection. Each Window is

associated with one of the TNC's streams, and when you switch windows

the TNC is automatically switched to the corresponding stream.

To switch from one window to another, press <Shift-Right-Arrow> or

<Shift-Left-Arrow>, that is hold down the Shift key while pressing

<Right-Arrow> or <Left-Arrow>. Another way to select a different window

is to hold the Shift key while pressing a Function key (eg: press

<Shift-F3> for the third window). The Communications Window Header Line

will change to indicate the new window number and the TNC stream used

with that window. Here you can make contact with the other station and

all communications for the associated stream will appear in that Window.

Changing streams will not change the TNC's operating Mode. So, if the

TNC was in Command Mode and you change to another stream, the TNC will

remain in Command Mode, regardless of whether you are connected to a

station on that new stream or not. You will need to enter K or CONV to

switch to Converse Mode.  Have a look at the comments on the NEWMODE

option under the Configuration - Multi User options for more on this.

If any data is received for a Stream other than that currently

displayed, it will be diverted to the appropriate Communications Window

and an indicator will appear in the Status Window under "Windows". If

that happens when you are connected to two stations, you will know the

"other" station has sent you a message and you can switch to that

window to see it.

I personally can work two or three simultaneous connections without

much problem, but any more I find rather challenging. So, it should be

noted that although paKet allows up to 10 Communications Windows, most

of those would not be used.  As buffers etc, have to be allocated for

each window, it would be advisable to configure a lesser number to

conserve memory. I think 3, or at most 4, windows would be more than

adequate for most users.

To avoid confusion, it would be a good idea to set your TNC's USER

parameter to match the number of windows you configure. Then, if you

have three windows configured and you are already connected to three

stations, a fourth station would get a "Busy" message from the TNC

rather than confuse things on the screen.  If your TNC has a PMS on a

separate callsign, you could set the TNC's USER parameter to one more

than the number of windows configured.

paKet is almost a Multi Tasking program in that several sessions can be

processed simultaneously, some of which may be processing in the

background. And paKet's Interrupt Handler works, gathering incoming

data even while other programs are running.  However, I do not consider

paKet to be true Multi Tasking and it is not promoted as such. For

example, the processing for Binary File Transfers is NOT multi tasking

and you may not conduct more than one simultaneous Binary File

Transfer.  There are a number of Multi Tasking systems available now,

Page 33

which support DOS-based tasks.  These include OS/2, Windows Enhanced

Mode, DESQview, Windows NT, etc?  In the testing I have done with some

of these, paKet runs just fine as a background task.  So if you have

a need for Multi Tasking, you can run paKet as one DOS task under one

of these systems.

It is unfortunate the original TAPR TNC (from which all modern TNCs are

derived) did not include a Monitor stream.

Consequently any data that is monitored on the frequency is not

identified by the TNC as belonging to a particular stream - it comes

through on whatever stream is currently selected. So, if you have

selected window 2 and some monitored data is received from the TNC, it

will be displayed in that Window 2.

It would have been so easy if TAPR had included another stream (say

Stream M) and optionally have all monitored data directed into that

stream.  Then we could give paKet a Window for Stream M and we would

have all monitored data separated from our connected sessions. (sigh)

I wonder if TAPR could do that and bring out an update? Then other

manufacturers would soon follow suit.  paKet would already support that

modification.   Oh well... where were we?

Flashback.

Example:

��� 1250 �������������FLASHBACK - (Esc when finished)����������������

cmd:

CONNECT VK2ATM-1

cmd:cmd:12-Feb-94  10:35:47  *** CONNECTED to VK2ATM-1

[FBB-5,15-ABFHM$]

G'day Tony

VK2ATM BBS>

Description:

The Communications Windows will also display Flashback data.

Flashback is a facility that allows you to redisplay earlier

communications activity that has scrolled off the screen.

When you select Flashback the Communications Window Header line will

indicate FLASHBACK as shown in the example above. You can scroll up and

down with:

<Up-Arrow>

<Down-Arrow>                    (one line at a time)

<PageUp>

<PageDown>                      (one page or screenful)

<Ctrl-PageUp>                   (top or

<Ctrl-PageDown>                  bottom of Flashback data)

Page 34

NOTE - the Up and Down Arrow keys may also be used to recall data from

the Type Ahead buffer which is discussed later.  To move up and down

one line in Flashback, you might need to hold down the Shift key while

pressing the arrow keys, but this is configurable.  You can choose the

way you would rather have the arrow keys work.  Check the Configuration

- Miscellaneous options.

The amount of data held in Flashback is also configurable up to 64KB

for each Communications Window.

As you scroll back through the buffer, paKet will display in the Header

Line a different coloured "bar" to give you some idea where you are in

the buffer.  In the example above I have shown that as '��' to the

right of centre.  The size of this "bar" changes with the size of the

buffer, or rather the amount of data in the buffer. If, for example

there are four pages (screen-fulls) of data in the Flashback Buffer,

the bar will occupy a quarter of the Header Line, telling you that at

the moment you are looking at about a quarter of the entire Flashback

Buffer.  If there are 80 or more pages of data in the buffer, the bar

will shrink to only one character space telling you you are looking at

only a small portion of it.

While you are in Flashback, any incoming data is held in the input

buffer until you have finished (1250 characters shown in the example

above) so be sure to return to normal communications mode when you have

finished scanning the Flashback data.

When scrolling forward, paket will detect the end of the Flashback

buffer. If there is some further data waiting to be processed, pressing

<PageDn> will display one more page of data from the buffer and paKet

will stay in Flashback Mode. If you reach the end of the Flashback and

there is no data further waiting in the input buffer, paKet will

automatically return to normal communications mode.

While in Flashback Mode you can press the <Esc> key for an immediate

return to normal communications mode.

If you forget to return to normal communications mode paKet will

automatically do so after 10 minutes of no keyboard activity.

Flashback - Text Find.

paKet has a Text FIND facility which will search backwards through the

Flashback buffer for a user-defined string of characters.

Press <Alt-F> to activate the Find facility.

The Message Window will pop up where paKet will ask you for the string

you wish to search for. It does not matter whether you enter the string

in upper or lower case because the search is not case sensitive - both

upper and lower case will be matched.

Page 35

The search begins with the current line in the Communications Window,

moving back through the Flashback buffer until either a match is found

or until the beginning of the Flashback buffer is reached. If a match

is found, searching stops and the line containing the matching text is

highlighted.

If you want to continue the search, "looking" for another match of the

same string, press <Alt-L> and the search resumes.

Alternatively you can press <Alt-F> again to begin a search for another

string.

The Find facility will automatically put paKet into Flashback mode.

Write Flashback buffer to disk.

Although paKet provides a disk log facility, it is possible you have

received something to want to keep but find the log file was not active

at the time.

To cover this situation, paKet has a facility to write some or all of

the Flashback buffer to disk.

This is done with the <Alt-W> key and that is discussed in the

"Keyboard Commands" section of this Manual.

Page 36

Configuration Windows Overview

As mentioned earlier, paKet is fully configurable and you can set up

various parameters to configure the system to suit your own needs.

With the Online Configuration facility you can call up the

Configuration Windows at any time (with the <Alt-Z> key) and change any

of the settings.

Most changes will take effect immediately although there are some that

will not become effective until the program is next started. Examples

of these are where you change the size of the allocated buffers. These

changes are not implemented immediately because memory is allocated at

the beginning of the run and the buffers are already in use. The new

buffer sizes will become effective when paKet is next started.

The following sections describe the various Configuration Windows and

the parameters you can change. There are two, sometimes three levels of

Configuration Window.

Select the desired option by either:

1. Typing the option number; or

2. Moving the Cursor to the desired option and pressing <Enter>; or

3. (In many cases) typing the first letter of the option.

For example in the following section, where the System Configuration

Window is displayed: you could select REMOTE / PMS options by typing

"6" or "R", or you could move the cursor to that item with the

<Down-Arrow> key.

Press the <Esc> key when finished with a Configuration Window to return

to the previous level.

For each of the Configuration Windows, there is an optional Help

message available.  This is called the Configuration Help and if

enabled, another window will automatically pop up to offer some

explanation and comments on each Configuration item you select.

I recommend you use this additional help window at least for the first

time through the various Configuration options.  It is enabled or

disabled in the Miscellaneous options Configuration and it requires the

file PAKETCFG.HLP in the default PAKET directory.

Page 37

Configuration Windows - System Configuration

Example:

�������System Configuration�����ͻ

� 1. Serial Port...              �

� 2. Multi User options...       �

� 3. Comms Window settings...    �

� 4. Colours...                  �

� 5. KB Macros/Auto commands...  �

� 6. REMOTE / PMS options...     �

� 7. File Transfer options...    �

� 8. Directories and Files...    �

� 9. Miscellaneous options...    �

�Press <Esc> for previous Window�1/4

Description.

This System Configuration window appears when you select the online

Configuration option with the <Alt-Z> key.

This System Configuration Window reveals a menu of various

configuration options.  To better manage the large number of options,

they have been subdivided into the categories shown in the above window.

Each of the options in the System Configuration Window has its own

Configuration Window  which will be dealt with in turn in the following

sections of this Manual.

When you have finished with the Configuration options, press <Esc> to

return to normal communications mode.

Page 38

Configuration Windows - Serial Port

Example:

��������� 1. Serial Port...������ͻ

�COM Port                        1�

�Baud Rate                    2400�

�Parity                       None�

�Data bits                       8�

�Stop Bits                       1�

�Handshaking                     H�

�DCD ON when Connected?          Y�

�TNC cmd: exit               BREAK�

�Command to Initialise TNC RESTART�

�������Press ENTER to change������1/4

Description.

This window appears when you select option 1 from the System

Configuration Window. It contains the parameters paKet uses for the

computer's Serial Port parameters.

The Serial port must be set correctly if paKet is to communicate with

the TNC.  The aim here is to configure paKet to match your TNC settings.

As there are a limited range of valid settings here, paKet does not

expect you to remember the valid values. Instead you just press <Enter>

and it will display the next valid value for that item.

COM Port

So, starting with the COM Port, press <Enter> to toggle from COM1 to

COM2, COM3, COM4 and back again.  Only those COM ports actually

recognised by your system will be offered, so if you do not have a COM3

port installed you will not be given that option!

If you have COM3 and/or COM4 installed but these are not being offered

by paKet as options here, it is likely your computer system is one

(like the original IBM PC) that does not automatically recognise these

ports. Run the supplied COMINIT program and try paKet again. If this

fixes the problem, I suggest you add COMINIT to your AUTOEXEC.BAT file.

Many people do not realise that COM1 and COM3 usually share the SAME

interrupt line - IRQ4  (COM2 and COM4 share another interrupt line -

IRQ3).  This makes it difficult to handle the additional ports and is

why a lot of software does not support more than COM1 and COM2.

Let's say, for example we have a mouse on COM1 and the TNC on COM3. If

a character is received from the TNC, an Interrupt occurs on IRQ4 which

means the system will give paKet a nudge to go process the incoming

character.  The trouble is, paKet gets the same nudge if you move the

mouse because the same IRQ4 Interrupt occurs!  I have catered for this

situation and paKet will determine if the Interrupt was caused by our

Serial Port or the "other one", and will either process it or pass

control over to the other driver for its own processing.

Page 39

There are a couple of considerations here if you want to use these

additional serial ports.  Firstly, load paKet AFTER loading the other

software (drivers, etc). Then paKet will be called first when the

Interrupt occurs. If it is the other way around, we are depending on the

other software to make these decisions and they may not handle the

shared interrupts correctly.  Secondly, I suggest you choose a lower

baud rate (say maximum 2400) for the paKet/TNC link to help reduce

conflicts. I tested it at 9600 baud and it worked satisfactorily, but if

two interrupts occur on the same IRQ line at EXACTLY the same moment in

time, I am not confident the system will process them both correctly.

The COMINIT program developed by Dr. Paul Webster VK2BZC, includes some

technical documentation on the use of the additional COM ports.

(Thanks Paul - where would I be without him?).

When the desired port is displayed, press <Down-Arrow> to move down to

the next item, Baud Rate.

Baud Rate

Pressing <Enter> on the baud rate field will cycle through the valid

values which range from 50 baud to 19200 baud.

Press <Shift-Enter> to cycle through these values in the reverse

direction.  So if you go past the one you wanted, press <Shift-Enter>

to go back again.

This option sets your COMPUTER's serial port baud rate; the TNC also

has a baud rate setting which must match the value chosen here.

Parity and Data Bits

You may set Parity and Data Bits to any of the displayed values but

paKet prefers a setting of No Parity and 8 Data Bits. So unless you have

special reason to change, please set Parity to None. You will notice the

Data Bits parameter will automatically be set to the correct value.

Stop Bits

Stop Bits would normally be set to 1.

Handshaking

Handshaking is either H (for Hardware) or S (for Software).

These options are discussed under "Hardware vs Software Handshaking" in

the Technical Section of this Manual. If you are not sure which to use,

try Hardware mode - paKet will tell you if it cannot see the signals it

needs for Hardware handshaking!

It is important to tell the TNC which mode to use too:

for Hardware Handshaking, set XFLOW OFF;

for Software Handshaking, set XFLOW ON.

Page 40

DCD ON when Connected?

This is either Y(es) or N(o) and should be set to match the TNC's

DCDCON option if such an option exists.

Many TNCs have the DCDCON option which, if used, will switch on the DCD

line in the RS-232 cable when we are connected to another station.

paKet relies to a large extent on the "Connected" message from the TNC

to determine whether we are currently connected to another station. If

you are monitoring the frequency and someone sends a string of

characters that happens to look like a "Connected" message, paKet can

easily get confused and take that data to be a signal from the TNC that

we have just connected to someone!  Fortunately this does not happen

too often but if you use this DCDCON option, the risk of this problem

occurring is greatly reduced.

If you set this option to "Y" paKet will look at the status of the DCD

line whenever it monitors a "Connected" message before deciding to

accept it as a new connection. If the DCD line is still low, it will

display a warning message, ignore the "Connected" message and continue

monitoring.  It will also monitor the DCD line during a Binary File

Transfer and will shut down the transfer if DCD is lost before the

completion of the file transfer.

If your TNC supports the DCDCON option and you are using Hardware

Handshaking, it is recommended you use this feature.

TNC cmd: exit

This parameter has three valid values: <BREAK>, <Ctrl-C> or <Esc>.

Most TNCs support the <BREAK> signal and will return to Command mode

when this signal is received. This is the preferred option so try this

and choose one of the others only if the <BREAK> does not work on your

system.

As a second choice for just about all other TNCs, the "Command" code

(<Ctrl-C>) would be used. So, unless you have a TNC-1 or a PacComm

PacTOR Controller, choose this <Ctrl-C> option if <BREAK> does not work

for you. If this option is selected, paKet will send three <Ctrl-C>s to

the TNC when you press <Alt-B>.  Some TNCs require at least a one

second delay before and after the <Ctrl-C>s.  So paKet will take a

couple of seconds to process this break.  In that case the "BREAK"

indicator is displayed in the Status Window for those couple of seconds.

Finally, if you have a TNC-1 or a PacComm PacTOR modem which needs the

<Esc> code to enter a command, you can set this option to <Esc>.  Then

when you type <Alt-B> paKet will actually send the <Esc> code to the

TNC!

Page 41

Command to Initialise TNC

Most TNCs use the RESTART command to perform a warm start of the TNC.

This is required to activate any changes to serial port settings so

paKet will issue this command to the TNC after you change any of the

Serial Port parameters (assuming you have already changed the TNC).

However, some non-standard TNCs use the RESET command for this so the

option is provided here in case you have one of those TNCs.

Please take care with this, because the RESET command, on most TNCs,

will clear every setting you have made and return the TNC to its

factory settings!   RESTART is the normal command here.

Matching settings with the TNC

Any of these parameters may be altered at any time, but of course you

will also need to change the TNC settings to match. For example if you

want to alter the Data Bits setting, you have to change both paKet (via

this Configuration Window) and change the TNC with a command such as

AWLEN. If you change one and not the other, you will probably get

garbage on the screen and will fail to communicate correctly.

With most TNCs, any change to its serial port settings will become

effective after the TNC is next initialised. So, I suggest the

following procedure to change Serial Port settings:

1. Issue the commands to the TNC to change its settings. (They will

not take effect until you initialise the TNC - usually with a

RESTART command).

2. Call up paKet's Online Configuration and enter the desired Serial

Port parameters to match the new TNC settings.

3. Reply "Y" when paKet asks if you would like it to initialise the

TNC with a RESTART command.

Then both the computer and the TNC will continue with the new settings.

Press <Esc> when you have finished with the Serial Port parameters.

Page 42

Configuration Windows - Multi User options

Example:

������ 2. Multi User options...����ͻ

� Maximum no. of Comms windows   3  �

� STREAMDBL (CHDOUBLE) in TNC ?  ON �

� NEWMODE setting in TNC ?       ON �

� Alpha or Numeric Stream Id?    A  �

������������������������������������1/4

Description.

This window appears when you select option 2 from the System

Configuration Window. It contains the options you have regarding the

use of multiple simultaneous connections with other stations.

Maximum no. of Comms windows

The first item specifies the number of Communications windows you would

like to allocate. paKet uses a separate Communications window for each

of the simultaneous connections. This field may be any value between 1

and 10.

You should keep the number down to a reasonable figure because they all

take up some memory and it would be a waste to configure say, all 10

windows when you might hardly ever use more than 2 or 3.

As the present number of windows have already been allocated, the new

value will not take effect until the next time you start the paKet

program.

To help avoid confusion, it is considered desirable to set your TNC's

USER parameter to the same value used here; or perhaps to one extra if

the TNC allocates a separate callsign to its PMS stream to allow one

user to use that as well.

STREAMDBL (CHDOUBLE) option

A standard TNC can handle up to 10 different Streams and when it

receives some data for some OTHER Stream it will send to the computer a

STREAMSW character followed by a letter or number to identify the new

Stream.  These are discussed more fully in the next section on

Communications Windows, and you could check your TNC manual for

information on Stream Switching.

The vertical bar character ("|") is usually used as the STREAMSW

character but that is also often transmitted over the air as a sort-of

graphic character to draw vertical lines or for drawing boxes. When

paKet receives this character from the TNC it must decide whether the

TNC is indicating a change of stream or whether this is just another

data character somebody sent over the radio.

STREAMDBL (called CHDOUBLE in some TNCs) is a TNC facility to help

avoid confusion where the STREAMSW character is received as part of a

data packet.

Page 43

If you set this STREAMDBL option ON, the TNC will generate TWO of these

characters whenever one is received over the air.  So when paKet gets

just one it knows it is a change of stream;  if it gets two, it knows

it was received over the air and it can proceed to ignore one of them

and treat the other as just another data character.

With the STREAMDBL feature, it KNOWS THE DIFFERENCE!

If you decide to set this option OFF in your TNC, or if your TNC does

not support this feature, then paKet's option must be OFF too otherwise

it is going to be looking for TWO of these things and will treat every

vertical bar (or whatever) as a change of stream, even if it is really

just another data character!

If paKet's option is set OFF, it will assume any STREAMSW character

received during Converse Mode is to be processed as a change of Stream;

if received during a Binary File Transfer it will be included as a data

character in the file.  This will hopefully be correct in most cases

but you need STREAMDBL to be sure.

The important thing is that the STREAMSW character will always be

handled correctly if you have STREAMDBL ON in both the TNC and in this

Configuration Window.

It is STRONGLY RECOMMENDED the facility be used.

But please ensure BOTH your TNC and paKet are configured the same way.

************************** I REPEAT *******************************

*  (because this has been one of the main causes of Binary File   *

*        transfer problems in earlier versions of paKet)          *

*                                                                 *

* Set STREAMDBL ON in BOTH paKet's Configuration AND in your TNC. *

*                                                                 *

*  If this feature is not implemented in your TNC then set this   *

*                     option in paKet OFF.                        *

*******************************************************************

Users of Kantronics TNCs will find their TNC does not support the

STREAMDBL command for some unknown reason, so you will have to set this

option OFF.  Please refer to the discussion on the Kantronics TNCs in

the Technical Section of this Manual.

Page 44

NEWMODE setting in TNC

Have a look at this parameter in your TNC manual to get an idea what it

does.  If it is set ON in your TNC you might like to set it this

Configuration item ON too?  Anyway, read on...

This option is provided as a operational convenience - some prefer to

have it on, some prefer to have it off.  I suspect MOST don't care!

But to satisfy those who are particular about these things, the option

is here.

There are two considerations for using this command with paKet.

1.  If NEWMODE is ON in your TNC, it will switch to Converse Mode

immediately a Connect command is ISSUED.

If OFF, it will wait until the connection is ESTABLISHED before

switching to Converse Mode.

This may be relevant if you want to use the multi-node connect

facility that is discussed under the <Alt-C> command in the

"Keyboard Commands" section.   That facility requires NEWMODE ON

if it is to work successfully.

2.  If the NEWMODE command is ON in your TNC, it will return to

Command Mode when a Disconnect is processed.

If OFF, it remains in Converse Mode.

It does this regardless of any other connection on another

stream.  So, if you happen to be connected more than one other

station and someone disconnects, your TNC will switch into

Command Mode (if you had NEWMODE ON).    Now, you probably do

NOT want that to happen because you are still connected to these

other guys and no doubt will want to continue talking to them.

So, if you often use multiple connects, you might have set this

command OFF in your TNC, so it remains in Converse Mode. You can

still set it to Command Mode anytime you want with <Alt-B>.

But if you rarely use multiple connects, sticking with just one

at a time in most cases, you might prefer to have NEWMODE ON so

your TNC returns to Command Mode each time you disconnect.

Most TNCs seem to have NEWMODE ON as their factory default setting and

as many people have not given this parameter too much thought I suspect

most packet users are running with NEWMODE ON.  It seems the main

disadvantage in having it ON is that you have to manually set it back

to Converse Mode if you get a Disconnect while you have multiple

connections running.

Page 45

So.... with this option ON in paKet's configuration, paKet will check

the other Windows whenever a Disconnect occurs and if someone else is

still connected in another Window, it will send "CONV" to the TNC to

put it back into Converse Mode for you.  This way you get the benefits

of NEWMODE ON without the disadvantages. But make sure you DO have

NEWMODE ON in your TNC otherwise I am going to transmit "CONV" over

your radio!

If you set this configuration item OFF, paKet won't do anything at all

about this NEWMODE stuff.  It would be just as if the option wasn't

there at all!

Alpha or Numeric Stream Id?

Although most TNCs use letters of the alphabet to identify the

different streams, some use numeric channel numbers. This configuration

option allows you to use paKet with either type of TNC.

Check the item  "TNC Stream Identifier" in the Communications Windows

Configuration (see the next page) because these identifiers too should

match this setting:

If you enter A here, they should be set to A,B,C... etc.

If you enter N here, they should be set to 0,1,2... etc.

Page 46

Configuration Windows - Communications Windows

Example:

�������������������������� Communications Windows ������������������������ͻ

�                 Window Number -     1   2   3   4   5   6   7   8   9  10�

� TNC Stream Identifier               A   B   C   D   E   F   G   H   I   J�

� Stream Switch character             |   |   |   |   |   |   |   |   |   |�

� Input buffer size     (0-32K)       1   1   1   0   0   0   0   0   0   0�

� Flashback buffer size (2-64K)      64  16   2   2   2   2   2   2   2   2�

� Word Wrap from                     48  48  48  48  48  48  48  48  48  48�

� Screen width (normally 79)         79  79  79  80  80  80  80  80  80  80�

� Bells (0 - 4 or 250 - 999)        525   1   1   1   1   1   1   1   1   1�

� Telephone style connect bell        Y   Y   Y   Y   Y   Y   Y   Y   Y   Y�

� Process data without viewing?       Y   Y   Y   Y   Y   Y   Y   Y   Y   Y�

� Display Status Window?              Y   Y   Y   Y   Y   Y   Y   Y   Y   Y�

� Display Type Ahead Window?          Y   Y   Y   Y   Y   Y   Y   Y   Y   Y�

� Number of Type Ahead lines          2   2   2   2   2   2   2   2   2   2�

������������� Press Tab key to move to the next Comms Window ��������������1/4

Description.

This window appears when you select option 3 from the System

Configuration Window. It contains parameters relating to each of the

Communications Windows you have configured.

Each column of values relates to the settings for that window number.

Press <Enter> or <Up-Arrow> / <Down-Arrow> to move to different lines

in that column.

Use the <Tab> key to move across to the next column.

Note, paKet will use only those values for the windows you have

configured (refer "Maximum no. of Comms Windows" in the previous

section). So if you have configured only 3 windows, the values for

windows 4 to 10 in the Example window above are ignored by paKet.

TNC Stream Identifier

This is the letter or number used by the TNC to identify the different

streams. You would normally use stream A in window 1, B in 2, and so

on. However, paKet can handle any combination so if, for some reason,

you wanted paKet to use TNC stream B in window 1 and stream A in window

2, just specify these streams in the above Configuration settings.

Some TNCs use a particular stream for their PMS. For example if your

TNC uses stream K for your PMS, you could specify stream K for one of

the configured windows.  Note that a programming blunder in some TNCs'

firmware versions prevents access to stream K in the TNC, though it

cheekily issues connect and disconnect messages on that stream!

Of course, if your TNC uses numbers for its channel identifiers instead

of letters, you would specify 0, 1, 2, etc instead of A, B, C, etc.

Page 47

Stream-Switch character

The Stream-Switch character is the code used by paKet and by the TNC to

signal a change of stream.

Most TNCs use the vertical bar character ("|") as the default but you

could change it to any other character if you wanted to. The important

thing is to have both paKet and the TNC using the same character.

The TNC command to change this value is probably either STREAMSW or

CHSWITCH.

If your TNC supports more than one STREAMSW character, for example if

you have a dual port TNC, you can specify a different STREAMSW code for

different windows.  Then paKet will use "that" window for "that" port.

A typical configuration might be to have "|" for the VHF port in

windows 1 and 2  (streams A and B); and have "~" for the HF port in

window 3 (stream A).  Of course this is just an illustration - you can

have whatever STREAMSW characters you want, in whatever windows you

want.

Input buffer size (0-32K)

Each Communications Window has an input buffer to hold the characters

received for that stream and this option allows you to specify the size

of that buffer.

There is also one common 32KB System Input Buffer which is filled by

paKet's Interrupt Handler as data is received from the TNC.  At regular

intervals in the processing, paKet will take data from that System

Input Buffer and distribute it to the appropriate Communications Window

Input Buffer, according to the stream indicated by the TNC.

A large input buffer size for the Communications window should not be

necessary; in fact you can specify 0 and not have one at all! If it is

possible you will have multiple connections, a 2KB buffer (at least one

screenful) should be allocated, but anything larger is unlikely to

provide any benefit in normal circumstances.

Flashback buffer size (2-64K)

Flashback is a paKet facility that allows you to scroll back to view

Communications Window information that has scrolled off the screen.

This option allows you to specify the amount of memory to allocate to

the Flashback buffer for this Communications Window. Of course a larger

buffer will hold more information, but memory is precious and you

should consider whether you really need too many big Flashback buffers.

paKet will look for EMS memory at startup time and if some is available

it will be used for Flashback buffers (among other things).

If you are not sure how much to allocate, try setting 32KB in the first

Communications window and 16KB in the others. You can easily change

these values later.

Page 48

Word Wrap from and Screen width

paKet has a Word Wrap facility that operates on data entered into the

Type Ahead Window. If a word does not entirely fit in the "Screen

width" specified, the whole word will be moved down to the next line.

There is also a Justify option available so that any text entered into

the Type Ahead Window is not only Word Wrapped, but is also adjusted

with additional spaces so that the right hand margin has a straight

edge.  In this case the "Screen Width" specifies the right hand margin.

To paKet, a "word" is a string of characters separated by a space or

end-of-line. So, if you are sending a line of dashes or some other

graphic string, you might not want that "word" wrapped to the next

line. To help avoid such problems, paKet will wrap a word only if it

begins after a certain column, as specified in "Word Wrap from".

For example, if the values in these two fields are 48 and 79

respectively, paKet will wrap any word that does not fit on the line

provided the word begins after column 48. (That is if the word is not

longer than 32 characters).

A Screen Width of 80 will work quite well to most other stations

including another paKet station, but will cause double-spacing when

received on some lesser systems with bad "wrap" logic. I prefer a

screen width of 79 on my system but you can specify 80 if you want to.

Bells (0 - 4 or 250 - 999)

This option determines the sound paKet makes when a <BEL> character is

received from the TNC, or if paKet issues a Beep to alert the operator.

A value of 0 means no sound, so paKet will effectively ignore any

<BEL>s received, and will not sound any warning bells.

1, 2, 3 and 4 sound predefined tones. It is not easy to describe a

sound in a print document, so change the value and try it!

Values between 250 and 999 specify the audio frequency of the beep.

Morse Code

This bells setting also determines the Morse Code audio frequency.

paKet can optionally identify various Alerts with a unique Morse Code

combination. See the Alerts Configuration Window and the <Alt-A> key in

the "Keyboard Commands" section for more details on the Morse options.

If that Morse option is enabled for Alerts, and an audio frequency is

specified here, the Morse Code will be sent at that frequency.  If you

have specified one of the predefined tones here, the Morse will be sent

at 600 Hz.

The <Alt-J> key allows you to adjust the Morse Code speed from 5 to 99

words per minute.

Page 49

Telephone style connect bell

This is a Y(es) or N(o) option.

If set to 'Y', paKet will issue an Aussie telephone-style "ring" from

the computer's speaker after a Connection is established if no

keystrokes have been detected within 15 seconds. (It will not ring if

you have enabled the Auto-REMOTE option where paKet automatically

switches into REMOTE Mode on connection).

This same telephone-style ring can occur if a REMOTE user enters the

T(alk) command.

This is to attract your attention in case you might have missed the

fact that someone wants to talk to you.

Pressing any key-character will stop the "bell".

Call Waiting

If you are running multiple Communications Windows, it is possible some

data will come into another window while you are not looking and,

although the Status Window will display the other Window number, that

message could go unnoticed. So, if this Telephone option is on, paKet

will issue a "Call Waiting" (double beep) sound every 20 seconds if

some data has some into another Communications Window and you have not

yet selected that window to view it.

Process data without viewing?

This is a Y(es) or N(o) option.

When data is received from the TNC for the currently selected stream,

it will be displayed in the Communications Window.

However, if data is received for some other stream, paKet can either:

- hold it in that other Communication Window's Input buffer until

you are ready to select that window and view the data; or

- process that data whether you have seen it or not. (It will still

be available in that other Window's Flashback buffer).

If you set this option to 'N' paKet will hold the data in the buffer

until you have selected that window to view it. However, I have found

it is often nice to just allow the data to flow into the system - you

can easily switch windows and use Flashback if necessary to view

anything that may have come in.

In either case, paKet will let you know if anything is received for any

other stream as it will display in the Status Window the Communications

Window number for that stream. For example, if something comes in to

Communications Window 4, the Status Window will show "<---4->" under

"Windows". So you will know if anything has been received, regardless

of the setting of this option.

Page 50

Try it and see which method suits you best but if you are not sure, set

it to Y.

Display Status Window?

This is a Y(es) or N(o) option.

The Status Window occupies the top two lines of the screen and is

continuously updated with the time and other information as things

change during normal operations. The details on display are discussed

in the section on Status Window in this Manual.

I recommend this option be set to "Y" at least while you are new to the

system, so the Status Window will appear.

If you change this to "N" the Communications Window will start on the

top line, effectively making that window 2 lines bigger.

Display Type Ahead Window?

This is a Y(es) or N(o) option.

The Type Ahead Window appears below the Communications Window and, if

used, may be configured to display from 1 to 9 lines.  This window

allows you to type messages, commands, etc into paKet's Type Ahead

buffer before they are sent to the TNC.

There are several advantages in using this Window, so I recommend you

set this option to "Y" at least while you are new to the system.

Some advantages include word wrap (including optional justification),

and a Recall facility (similar to the DOSKEY function in DOS), not to

mention the ability to edit your typing before it goes to air!

Number of Type Ahead lines

The Type Ahead Window is optional (refer previous option), but if

selected you can choose the number of lines to display.

Valid values here are 1 to 9.

The Type Ahead buffer capacity is limited to the number of lines

displayed.

There is a little overhead in making the Type Ahead Window larger, but

the biggest disadvantage is the reduction in the size of the

Communications Window.

I think two or three lines should be more than enough for most users.

Page 51

Configuration Windows - Colours

Example:

��������� 4. Colours...�������ͻ

� 1. Message Window...         �

� 2. Communications Window...  �

� 3. Status Window...          �

� 4. Yapp Window...            �

� 5. Help Window...            �

� 6. TNC Help Window...        �

� 7. Type Ahead Window...      �

� 8. Configuration Windows...  �

� 9. Directory Window...       �

� A. Online Manual Window...   �

�������������������������������1/4

Description.

This window appears when you select option 4 from the System

Configuration Window. It is a list of the various paKet windows for

which the colours may be changed.

If you are running a monochrome display, you might think this section

would not apply to you, but you could try different settings to get a

combination of highlighting and contrast that suits you best with your

monochrome display.

Selecting one of the items in this window will bring up yet another

window showing three sets of colours:

Page 52

Configuration Windows - Colours - Individual Window Colours

�������� 1. Message Window...������ͻ

�Normal    Foreground   WHITE       �

�Normal    Background   GREEN       �

�Highlight Foreground   LIGHTGREEN  �

�Highlight Background   BLACK       �

�Border    Foreground   WHITE       �

�Border    Background   GREEN       �

�������Press ENTER to change��������1/4

As with the Serial Port options, you may change any of these settings

by pressing <Enter> or <Shift-Enter> to cycle through the available

options. As you change a colour, the new setting is displayed in the

window. When you are happy with a particular setting, press

<Down-Arrow> to move to the next item.

Note that you have 16 colours available: for the foreground, the first

8 colours are normal intensity and the second 8 are high intensity.

Provided the display was in normal text mode, choosing a high intensity

colour for the background will cause the display to blink.

These colours are the standard CGA-compatible colours so this

arrangement will work with any Industry Standard (i.e. IBM Compatible)

computer system.  If you are using a more modern video system such as

VGA, I am sorry but you are stuck with these few colour options.  You

will not be able to select from the 256 or 32,768 or whatever number of

colours your system can handle.  I chose to retain the simple 16/8

colour system so anyone can use paKet, including those who might be

using an older system.  Perhaps in a later version I will add yet

another option to choose the video system?  (sigh - ANOTHER

Configuration option?)

When you have finished changing colours, press <Esc>. You may then

change colours for another window or by pressing <Esc> again, you are

returned to the System Configuration Window.

Note that in many cases, you will not see the new colours immediately.

For example, if you change the Communications window colours, you may

think it has not worked, but you will see the new colours coming up as

new data is sent or received.

Page 53

Configuration Windows - Keyboard Macros/Auto commands (etc)

Example:

��� 5. KB Macros/Auto commands...�ͻ

�1. Keyboard Macros...             �

�2. Begin Auto Commands...         �

�3. End Auto Commands...           �

�4. Alerts...                      �

�5. Control codes to PASS...       �

�6. KISS Mode OFF sequence...      �

�����������������������������������1/4

Description.

This window appears when you select option 5 from the System

Configuration Window.  The items shown in the above window are grouped

here because they all require strings of data, rather than a simple

parameter option.

The "Keyboard Macros" facility provides up to 10 macro strings which

may be generated and sent to the TNC at a single keystroke, well two

anyway!

"Begin and End Auto Commands" are some optional commands that may be

sent to the TNC when paKet is starting up or proceeding to shut down.

"Alerts" is a facility that (again optionally) may sound the PC "bell"

or even generate a unique Morse Code identifier when a nominated string

appears in the Communications Window. This feature could be used if you

want to know when a particular callsign is on the air or if some

particular topic is being discussed.

"Control codes to PASS" identifies the various control codes (For

example: <Ctrl-X>) that you want transmitted to the other station

rather than having them processed by your TNC as an internal command.

"KISS Mode OFF sequence" is a series of codes that will be sent to your

TNC if you press <Alt-K>.  This is provided with the intention of

making it easier to switch the TNC out of KISS Mode and return to

normal Command Mode.

Selecting each of the 6 options will bring up another window for the

details, which are discussed below.

Page 54

Configuration Windows - Keyboard Macros

�����������������������������1. Keyboard Macros...����������������������ͻ

�1 ~BMAIL ON<Enter>MDC<Enter>l<Enter>                                    �

�2 ~BB<Enter>MAIL OFF<Enter>                                             �

�3 ~BMCON OFF<Enter>C VK2ATM-1<Enter>                                    �

�4 73s from Tony in Port Macquarie<Enter>                                �

�5                                                                       �

�6 ~BXMITOK T<Enter>                                                     �

�7                                                                       �

�8 <~s bbsconn>                                                          �

�9 ~B��D<Enter>                                                          �

�0 ^CD^M                                                                 �

F11!xtgold                                                               �

F12Disconnecting from $u at $t...<Enter>������~BD<Enter>                 �

�������������������������������������������������������������������������1/4

Description.

There are 10 Keyboard Macros available (12 if you have an Enhanced

Keyboard) and these are activated by pressing <Alt-1>, <Alt-2>, etc

through to <Alt-0>. That is, the number keys along the top of the main

keyboard (NOT the numeric keypad). The numbers 1 to 0 showing down the

left side of the Window correspond to the macro key you wish to use.

If you have an Enhanced Keyboard, the additional Function Keys <F11>

and <F12> are also available for Keyboard Macros.

To enter a new Macro or to edit an existing Macro, move the highlighted

cursor bar down to the item you want to change and type your macro on

that line.  If you make a mistake or change your mind, just press <Esc>

and paKet will ignore any changes you have made, restoring the original

data.  When you have finished typing your Macro on this line, press the

<Enter> key to complete the line.

This last <Enter> key you type does not go into the string because

there may be times where you do not want a Carriage Return at the end

of the string.  If you DO want to add an <Enter> to the end of your

Macro, you should add that code as shown in some of the examples

above.   The syntax required for entering this and other codes are

discussed below and are detailed further in the section on Special Key

Codes in this Manual.

Sometimes you might want to simply change something in an existing

Macro and sometimes you might want to replace the entire line with a

new Macro.  If the first key you type on a line is some data character,

paKet will assume you want to type a new Macro to replace whatever what

is already on that line, so it will blank out the line to prepare for

the new entry.

If the first key you type is one of the cursor movement keys  (for

example, <Right-Arrow>, <End>, <Home> etc.) then paKet will retain the

existing string for you to edit.

To delete a macro, move the cursor to that line and type <Ctrl-Del>.

Page 55

Special Syntax

Many of the special keys used in paKet may be coded specifically using

Pointer Brackets (Less-Than/Greater-Than bracket pairs). These are

listed under Special Key Codes in this Manual but as an example here,

you could specify the <Enter> key as "<Enter>" or Function key 2 as

"<F2>". Several examples of the <Enter> key appear in the above example

macros.

You can specify <Ctrl- > codes by entering the '^' character followed

by the ASCII character, for example in Macro 0 above, we see "^CD^M".

The first 2 characters, "^C" will generate a single <Ctrl-C> code. Then

a D is sent to the TNC, followed by <Ctrl-M> which is the same code as

the <Enter> key.

For the sake of illustration, I have also shown in the examples above

as Macro 9, a similar function:  "~BD<Enter>" that effectively does the

same thing, but this one uses the tilde ("~") which is how we tell

paKet to generate one of the <Alt- > codes.  The first two characters,

"~B" generates the <Alt-B> code - this is paKet's BREAK code to return

the TNC to Command Mode.  (This is a better method than using <Ctrl-C>

anyway).  Then the letter D is sent to the TNC, followed by "<Enter>"

which is another way of generating a single <Enter> code.

This macro, by the way, is a quick and easy way to disconnect from

another station. After returning the TNC to Command Mode; the "D"

followed by <Enter> issues a Disconnect command to the TNC.

Oh, I almost forgot to mention the two little "Yawn" characters (�)

in that one.  A "Yawn" is a half second delay.  This is displayed (and

printed in this manual) as one of the Extended ASCII characters and I

know some printers do not handle that character too well in text mode.

I hope your printer printed them correctly, but they are the little

'2', the mathematical symbol that is used to indicate "squared". I

thought that symbol looked a bit like a little Z which is often used to

suggest snoring ... which is uh ... sort of like a yawn ... which I

thought ... er ... well anyway that is the Yawn symbol.

You enter that code with the <Alt-Y> key so if it looks a bit strange

on your printer, just type <Alt-Y> to see what it looks like on the

screen.   Each time that code appears, paKet will wait for half a

second delay.  In that example there are two of them so paKet will

process the <Alt-B> then wait for a full second before sending the "D"

and <Enter>.  Of course that delay should not be necessary there - I

just put them in to show how they can be used.

Scripts

In Macro 8 I have shown an example of running a Script from a Keyboard

Macro.  In that example, when you press <Alt-8>, paKet will load and

run the Script BBSCONN.SCP from the default Scripts directory.

The syntax for this is shown as:

<~s bbsconn>

Page 56

As is often the case in paKet, upper and lower case are treated

identically so it doesn't matter which you use here.

The Pointer Brackets are required and the "~S" is the standard <Alt-S>

command which you would use to run a Script from the Keyboard.  In this

case the name of the Script file is included between the Pointer

Brackets.  I have added a space between the ~S and the filename for

clarity here but that space is optional.

If you would like to pass parameters to the Script, you enter them

between the Pointer Brackets and separate them with a comma.  For

example:

<~S BBSCONN,XY4BBS,20>

In that case the two parameters will be available to the BBSCONN.SCP

file.  The first parameter (XY4BBS) will be in the %1 Script parameter.

The second (20) will be available in the %2 Script parameter. These

things are discussed in the Scripts section of this Manual.

Running a DOS command

In the example for the <F11> key we see how easy it is to run a DOS

program from paKet with a single keystroke.  In that example, I simply

press <F11> to exit to DOS, load and run the XTGOLD program.

The "!" symbol is the code used to tell paKet this is a DOS command.

paKet does not check that DOS command at all - it will:

- exit to DOS,

- load a copy of COMMAND.COM, and

- pass to it whatever you have specified here, including any

parameters.

If it is wrong in some way, DOS will soon tell you!

The usual DOS rules apply here.  The program or command you specify in

the macro must be a valid command or must be a program DOS can find

either in the current directory or within its PATH.   This is DOS

material now, nothing to do with paKet, so check your DOS Manual if it

is not doing what you want.

This command could be an internal DOS command such as COPY; it could be

a program such as CHKDSK; or it could be a BATCH file.

Control returns to paKet when that job is finished.

Don't forget, paKet is still in memory while this DOS command is being

processed so the computer will not have its full complement of memory

available.  See comments under <F9> key in the Keyboard Commands

section of this Manual for more details on this.

Page 57

Substitution parameters

The <F12> example looks a bit daunting, doesn't it?  No, it is quite

simple really.  There are two substitution parameters used there ($u

and $t).  These codes are detailed along with other similar codes, in

the Special Key Codes section of this Manual.

When this macro is used, paKet substitutes certain values in place of

those parameters.  For example, $u will be replaced by the callsign of

the station we are currently connected to, and $t will be replaced by

the current time.

I use that macro as another easy way to perform a Disconnect from

the other station. If I am currently connected to say, VK2BZC and the

time is now 15:50, that Macro will generate the following string and

send it to the TNC:

Disconnecting from VK2BZC at 15:50...

followed by an <Enter> key, so that message will be sent to that other

station. Then after a few seconds delay (hopefully enough time to allow

that message to be sent) it will return the TNC to Command Mode

(<Alt-B>); and finally send a D (Disconnect Command).

Note you can use a Macro to send commands to the TNC or to send

a string of data to another station. If sending commands it would be

prudent to include the "~B" to be sure the TNC is in Command Mode.

When finished with this window press <Esc> to return to the previous

KB Macros/Auto commands... Configuration Window.

Page 58

Configuration Windows - Begin-Auto Commands

���������������������������2. Begin Auto Commands...���������������������ͻ

�echo off                                                                 �

�CTEXT [paKet-6.1-HM$]^V                                                  �

� If you get the Menu, try: ^V                                            �

�   H for Help or^V                                                       �

�   S to "Send" me a message^V                                            �

�      ...Tony                                                            �

�<~S MAILOFF>                                                             �

�<~S MORSE-ON>                                                            �

�                                                                         �

�<Ctrl-Ins> to Insert line; <Ctrl-Del> to Delete line; <Esc> when finished1/4

Description.

When paKet starts, a DAY command is optionally sent to the TNC to

initialise its date and time to match that in your computer. Then, as

part of the initialisation, paKet can send up to 10 additional commands

to the TNC. These additional commands are optional, but if you want to

include any commands to be sent at start up time, they would be

specified here.

Each line contains a single command so an <Enter> is implied and will

be generated by paKet as each line is processed.

In the above example, I have specified a multi line CTEXT command using

the PASS character <Ctrl-V> which will cause the following <Enter>

(generated by paKet) to be taken by the TNC as part of the CTEXT data.

Refer to your TNC Manual for information on the PASS character.

Note, you may enter Special Key Codes or the <Ctrl- > or <Alt- > key

sequences as used in the previous section on Keyboard Macros.  The

section on Special Key Codes in this Manual gives more details.

I have also used two Script files as part of this start up process.

Although paKet will also look for the optional AUTOEXEC.SCP file (and

run it if it is found), I thought I would add these Script entries here

to illustrate the syntax required if you want to run a Script from

these Auto Commands.  The syntax is similar to that required for

running a Script from a Keyboard macro.

Editing facilities here are the same as for Keyboard Macros.  Use

<Ctrl-Del> to delete the highlighted line.  And if you want to insert a

new line between two existing lines, press <Ctrl-Ins> to move the

following lines down, making room for the new entry.

You might prefer to put all of this into the AUTOEXEC.SCP and let paKet

do it all from there, leaving this window empty.  If you did that it

would make it easier for you to run all these startup commands again

during the day.  You just call up AUTOEXEC.SCP with the <Alt-S>

command and paKet will do it all again!   The choice is yours.

When finished with this window press <Esc> to return to the previous

KB Macros/Auto commands... Configuration Window.

Page 59

Configuration Windows - End-Auto Commands

����������������������������3. End Auto Commands...����������������������ͻ

�^S                                                                       �

�CTEXT paKet is not currently running.^V                                  �

�  Please leave a message in the PK-232 Mailbox.^V                        �

�    ...Tony.                                                             �

�MAIL ON                                                                  �

�echo on                                                                  �

�xmitok on                                                                �

�conok on                                                                 �

�                                                                         �

�                                                                         �

�<Ctrl-Ins> to Insert line; <Ctrl-Del> to Delete line; <Esc> when finished1/4

Description.

This item is almost identical to the previous option, but as you might

guess, these commands are sent to the TNC when you exit from the

program with the <Alt-X> or <Alt-F4> command.

Once again, these commands are optional so leave the window blank if

you don't want any commands issued as you exit the program.

If you are keen to monitor everything that is happening on the

frequency while you are "offline", you could send an <XOFF> code which

is usually <Ctrl-S>, as part of these End Commands.  Then your TNC will

hold any monitored data in its buffers until it receives an <XON> code

which is usually <Ctrl-Q>.  paKet always sends a <Ctrl-Q> when it

starts up (just in case) so provided there is enough capacity in your

TNC to hold everything received while you were "away", paKet will

capture all that information when you next start the program.

The syntax for entries in this Window is identical to the Begin Auto

Commands so I won't go into all that detail again.

I mentioned the optional AUTOEXEC.SCP Script file which may be used at

startup time... well there is a corresponding Script, AUTOEND.SCP,

which may be used at shutdown time.  So, if you want to do a lot of

sophisticated things at this time, you might decide to leave this

Window empty and put your clever stuff into that Script.  You can have

both if you want - some commands in this Window AND some in the Script

as well.   Again, the choice is yours.

Page 60

Configuration Windows - Alerts

����������������������������������4. Alerts...���������������������������ͻ

�PAKET                                                                    �

�TONY                                                                     �

�DHU                                                                      �

�#WX#EXPERIMENTAL WEATHER STATION AT PORT MACQUARIE                       �

�BZC                                                                      �

�WAC                                                                      �

�                                                                         �

�                                                                         �

�                                                                         �

�                                                                         �

�<Ctrl-Ins> to Insert line; <Ctrl-Del> to Delete line; <Esc> when finished1/4

Description.

The "Alerts" facility sounds an audio alarm when a nominated string

appears in the Communications Window.   Here in this window, you can

specify those strings.

Up to 10 Alert strings may be recorded in this window.  Case is not

important here as paKet will match both upper and lower case.

There are three Alert Modes:

- ON (standard alert)

An "A" appears to the left of the Time in the Status Window.

When one of the nominated strings is detected, paKet will sound

the standard Bell tone.

- Morse

An "M" appears to the left of the Time in the Status Window.

When one of the nominated strings is detected, paKet will generate

a Morse Code sequence to uniquely identify that string.

- OFF

The "A" or "M" is extinguished in the Status Window.

The Alerts facility is disabled.

Press the <Alt-A> key to switch between the three Modes.

In the sample Window above I have shown 6 strings.  Whenever any of

these strings appears in the Communications Window, the program will

sound the Bell if you have enabled the standard Alert option ("A").

However, if you have enabled the Morse option ("M"), the program will

play a Morse Code sequence through the PC speaker.  If the word "PAKET"

is detected, it will sound the Morse letter "A" (dit dah) because this

is the first line in the Alerts Window.  If it detects the word "TONY"

Page 61

it will sound the Morse letter "B" (dah di'di'dit) because this is the

second item in this Window.  And so on, using different Morse letters

for successive lines in this Window.

Hang on, hang on... it doesn't matter if you don't like Morse Code.

And this is certainly not the place to discuss the relative merits of

the Code!  It is just a way of making unique noises to satisfy those

who have asked for something to identify the different strings.  If you

are anti-Morse, just select the standard Alerts option.  (But it IS a

convenient way to learn a few letters of the Morse alphabet!)

While on this Morse option, you will see a strange syntax on the fourth

line in the above Window.  On that line I have specified #WX# at the

start of the line.  That is a special syntax to tell paKet to look for

the string:

EXPERIMENTAL WEATHER STATION AT PORT MACQUARIE

and if that string is detected, it will play the Morse sequence for

"WX" (Di' dah dah,  Dah di'di'dah) instead of the default Morse letter

for that line.  You can do this on any line, specifying the preferred

Morse letters (surrounded by "#") before the Alert string on that

line.  So if you don't want to use "A" for the first line, "B" for the

second line, etc you can make up your own codes!

I might use #E# or #I# or #T# for some frequently occurring items

because those Morse idents are the briefest!

If you are using standard Alerts Mode, paKet will ignore any special

Morse idents (the #-# parts).

You can adjust the Morse Code speed with <Alt-J>.

If the noises start to annoy you, press <Alt-Q> for Quiet Mode to

disable the sounds emanating from the speaker. The strings specified

here in this Window can stay so when you toggle <Alt-Q> again Alert

Monitoring will recommence using the same strings.

Page 62

Configuration Windows - Control Codes to PASS

��5. Control codes to PASS...ͻ

�  TXZ                        �

������������������������������1/4

Description

This Window contains a series of letters which represent various

<Ctrl- > codes, for example "TXZ" indicates <Ctrl-T>, <Ctrl-X> and

<Ctrl-Z>.

Sometimes we send a Control Code to our TNC to make it perform some

internal function.  An example of this might be a <Ctrl-C>.  In many

cases that <Ctrl-C> code is used to switch the TNC into Command Mode,

so when we type that code, we want the TNC to process it and to respond

by switching Modes.  We normally do NOT want that <Ctrl-C> sent to the

other station!

But suppose we DO want to send one of these codes to the other station?

The TAPR-compatible TNCs cover this situation with the provision of a

<PASS> facility.  Have a look at your TNC Manual for a full description

of this feature but briefly, it allows us send one of these special

Control Codes to the other station.

The technique is to type the <PASS> character (fortunately all TNCs

seem to agree <Ctrl-V> is the standard <PASS> character), followed by

the Control Code we want to send.  So, if we want to send (say) a

<Ctrl-X> to the station we are connected to, we would send:

<Ctrl-V> <Ctrl-X>

If we do not type the <PASS> character there, it is likely the TNC will

take that <Ctrl-X> as a command, and not send it to the other station

at all.  For example in my TNC, that code is used as a CANLINE command

to cancel the current input line.  The CANLINE function is not really

required with paKet but the TNC still uses it for that purpose!

I have found that some systems, notably TCP/IP, use Control Codes for

various things and it becomes a bit of a drag having to type the <PASS>

character every time we want to send one of these Codes.  So I created

this Window where you can enter the corresponding letter for those

Control Codes you normally want sent to the other station.

A good example is the <Ctrl-X> which is used by TCP/IP.  Whenever I

type <Ctrl-X> I want it transmitted to the other station; I do not want

to use the CANLINE facility in my TNC.  So I include the letter X in

this Window.  Then whenever I type a <Ctrl-X>, paKet checks this

Window, sees the X here and automatically sends a <PASS> code to the

TNC followed by my <Ctrl-X>.

Now I know the TNC will always transmit my <Ctrl-X>.

Page 63

If you are new to packet radio, you might find this all a bit

confusing.  You could remove all the codes in this Window, leaving it

empty.  Then paKet will take no special action with your Control

Codes.  If you want to send say, a <Ctrl-X> to the other guy, you can

type the <PASS> code yourself and do it the way other Terminal Programs

do it - manually!

Then as you gain experience you might decide to add just the one code,

say an "X" and see how that works.  And if you find it necessary to

type the <PASS> code for some other regular Control Code you could add

that one too, progressing gently.

On the other hand if you are trying to send some Control Code to your

TNC and it seems to be ignoring you, it just might be that letter is

here in this Window and you are inadvertently telling your TNC to

transmit that Code over the radio instead of processing it internally!

(Take it out.)

Page 64

Configuration Windows - KISS Mode OFF sequence

������������������������6. KISS Mode OFF sequence...��������������������ͻ

�  192 255 192                                                           �

������������� Please use DECIMAL numbers for each byte value ������������1/4

Description

This Window is used to define a sequence of codes that will be sent to

your TNC when you type <Alt-K>.  It is intended this option be used to

restore your TNC to Command Mode after some other program has left it in

KISS Mode.

KISS Mode ("Keep It Simple, Stupid") is a simple TNC mode sometimes used

by various other programs.  It is simple in the sense that the TNC's job

is simpler because most of the work is now done in the computer.   The

main advantage in using this method is that the computer now has more

control over the processing allowing it to overcome some of the

limitations found in standard TNCs.

Many TNCs now support a KISS Mode option, but when paKet was first

created that support was restricted to only a few TNCs. I decided to

write paKet to operate with the standard Command Mode so it would run on

ALL TNCs.  But now that support for KISS Mode is growing, I might have a

look at making a KISS Mode option available in a later version of paKet.

Although paKet does not use KISS Mode, it is possible you have used some

other program which did switch the TNC into KISS Mode and which failed

to reset it to the standard Command Mode on exit.  In that case, paKet

will no longer be able to get any sense out of your TNC.

If your TNC is in KISS Mode, you will probably get the "TNC is not

responding" message when you start paKet.  You can type "I" to Ignore

that problem to allow the program to start, then type <Alt-K> to send

these codes to restore the TNC to normal mode.  It might be a good idea

to then exit paKet and start again, now that we can communicate with the

TNC, this time processing all the Begin Auto Commands, etc.

I conducted a packet radio survey recently, asking everyone to let me

know what codes are required for THEIR particular brand of TNC to exit

KISS Mode.  The almost universal reply was the three byte code:

192 255 192

as shown in the example Window above.  Note they are shown as decimal

numbers in paKet's Window, separated by a space, but in some TNC Manuals

these same values are shown in Hexadecimal notation as follows:

C0  FF  C0

Some users reported their TNC comes out of KISS Mode if they send three

<Ctrl-C>s in quick succession, but I understand those TNCs also respond

to the above three byte code.  So, let's stick with the standard code

sequence.  It's easier that way.   Some MFJ users reported their units

seemed to work better with just a two byte code, using only one of the

framing codes, eg: 192 255.  You might like to check your Manual if you

intend to use this feature of paKet.

Page 65

Configuration Windows - REMOTE / PMS options

�������������������������� 6. REMOTE / PMS options...����������������ͻ

�       -- REMOTE Options --                                          �

� Allow REMOTE access?                Y                               �

� Issue REMOTE Menu on connection?    Y                               �

� SEND files  to  REMOTE station?     Y                               �

� RECV files from REMOTE station?     Y                               �

� REMOTE trigger (decimal value)      29                              �

� -- PMS / Mail Forwarding Options --                                 �

� BBS Callsign (eg XY4BBS-1)          VK2ATM-1                        �

� Allow third party messages in PMS?  Y                               �

� Delete message after Forwarding?    N                               �

� Call hourly? (Mail, Always, No)     M                               �

� When?  (minutes past the hour)      06                              �

� Call BBS once per day?  (M, A, N)   A                               �

� What time of day to call ? (hh:mm)  23:24                           �

� Use Script to make BBS connection?  N                               �

� Connect path/Script file for BBS    VK2ATM-1 v VK2RPM-1             �

����������������������������������������������������������������������1/4

Description:

This window appears when you select option 6 from the System

Configuration Window.

There are two sections to this Window:

REMOTE Options; and

PMS / Mail Forwarding Options.

Each of these sub systems is discussed further in its own section of

this Manual.

REMOTE Mode allows other users access to our system via our REMOTE

Menu.

The PMS (Personal Message System) provides a Mailbox where you and

REMOTE users can store and read Bulletins and Personal Messages. The

PMS facility includes full Mail Forwarding facilities to exchange Mail

with your BBS using the standard Automatic Mail Forwarding protocol.

Configuration Windows - REMOTE Options

Allow REMOTE access?

This is a Y(es) or N(o) parameter.

This would normally be 'Y', but if you do NOT want other stations to

use this REMOTE facility, turn this option off with 'N'.

You may still activate it yourself via the <F3> key if you wish,

temporarily overriding this Configuration option, so YOU still have a

choice.

Page 66

Issue REMOTE Menu on connection?

If you are happy to allow REMOTE operations, you can either wait for

the other station to send us the REMOTE Trigger (discussed below) or

you can choose to have paKet automatically issue the REMOTE Menu when

the other station connects to our system.

If you enter "Y" for this option, paKet will switch into REMOTE Mode

and will issue the menu immediately a connection is established.  The

default setting is 'Y'.

This is intended for those times when you are not present to

communicate with the other station. So if you have pressed a key within

the past 60 seconds, paKet will assume you are at the keyboard and will

NOT automatically switch into REMOTE Mode. The other station can still

send the Trigger to get the REMOTE Menu, or you can do it with the <F3>

key if you wish.

SEND files to REMOTE station?

This is a Y(es) or N(o) parameter.

If you have decided to allow REMOTE operations, you might still want to

prevent other stations downloading files FROM our system in REMOTE mode.

This would normally be 'Y', but if you want to prevent REMOTE Downloads

set this option to 'N'.

RECV files from REMOTE station?

This is a Y(es) or N(o) parameter.

If you have decided to allow REMOTE operations, you might still want to

prevent other stations uploading files TO our system in REMOTE mode.

Preventing REMOTE Uploads should reduce the risk of picking up a virus.

Alternatively, specifying a unique Binary Receive directory should

allow you to be suitably suspicious of whatever turns up in that

directory. Nevertheless, if you are still concerned at this possibility

you may choose to set this option to 'N'.  The default setting is 'Y'.

REMOTE trigger (decimal value)

Another station who has connected to our system might like to switch on

our REMOTE Mode (if it is not already on).

This trigger code is specified here as a decimal ASCII value because

some codes do not display well and trying to display the code itself

could become confusing. Using a decimal value leaves no doubt as to the

value set.

The default value of 29 (or the ASCII GS code) is <Ctrl-]>, that is

hold the Ctrl key while typing right-square-bracket.  So, if the other

station sends us that code, with nothing else on the line, paKet will

activate REMOTE Mode and send the REMOTE Menu.

Page 67

Configuration Windows - PMS Options

Allow third party messages in PMS?

This is a Y(es) or N(o) parameter.

When a REMOTE user leaves a message in your PMS, it would usually be a

message addressed to you.  However, paKet is quite happy to have that

user leave a message (or bulletin) addressed to someone else.

That situation might arise if that REMOTE user wants to use our Mail

Forwarding facilities.  He could leave that message in our PMS and at

the appointed time, paKet will Forward that message to our local BBS

for onward Forwarding through the BBS Network.

However, in some countries, this is not allowed.  That message

addressed to "someone else" is considered a "third party" message. (We

are one party, the REMOTE user is the second party, and the someone

else is the third party).  I don't understand how they can leave a

message on their BBS with this ruling, but....

If your Regulations do not permit Third Party messages, you can set

this option to "N" then paKet will refuse to accept a message from a

REMOTE user if it is addressed to anyone else.

BBS Callsign (eg XY4BBS-1)

This is the callsign of your "Home" BBS, including any SSID it uses.

The SSID is the optional numbers after the callsign, for example the

"-1" in the above Window.

While you are connected to this station, paKet will display "BBS" in

the Status Window to the left of the Time display.

This callsign is recognised by the program so it can perform some more

appropriate processing in the Mail Forwarding routines.

This field must be completed before you can do any Automatic Mail

Forwarding.

Delete message after Forwarding?

This is a Y(es) or N(o) parameter.

If Yes, paKet will mark for deletion, any message that is sent to your

BBS by our Automatic Mail Forwarding system.

This makes it all simpler - you know that once the message has been

acknowledged by the BBS it will be removed from our PMS so there is

nothing else you have to do.   If it is still in there, you know it has

not yet been Forwarded.

On the other hand, you might prefer to set this to "N" and do it

yourself so you have more control over what stays and what is deleted

from your PMS.  You can delete ("Kill") any messages later via the

Sysop PMS Menu which is displayed whenever you type the <Alt-M> key.

Page 68

Call hourly? (Mail, Always, No)

This option relates to Mail Forwarding.   Once every hour paKet can

perform some routine processing for Mail Forwarding.  With this item

you specify one of three processing options:

M - Mail.

Check for any Mail waiting in our PMS to determine whether or

not to call the BBS for Mail Forwarding.

This option tells paKet to check the PMS at the appointed time

every hour (see the next option which determines the actual

time).   If there is any mail in our PMS waiting to be

Forwarded, then paKet will call the BBS and establish a Mail

Forwarding session to send that mail.  While that session in is

progress, the BBS can also send us any incoming mail it has been

holding for us.

If there are no messages in our PMS that are waiting to be

Forwarded, paKet will not call the BBS and will return to

whatever it was doing.  It takes only a second or two for paKet

to check the PMS each hour - you probably won't even notice it

doing it.

This is the preferred setting.

If you want to send some mail to someone, you can do it all

here.  You do not have to connect up to the BBS every time you

want to send a message. Just drop it in the paKet PMS and leave

paKet to worry about it.  Of course if you write several

messages, they will all be sent at the next Forwarding session,

within the hour.

A - Always.

Call the BBS for Mail Forwarding EVERY hour.

Choosing this option ensures the system will collect any

INCOMING mail from the BBS every hour, even if there are no

messages in our PMS waiting to be sent.

Usually you would ask your BBS Sysop to set up his system to

call you at regular intervals for the purpose of Mail Forwarding

so if something is received at the BBS for you, it will be

stored in your paKet PMS when the BBS performs its next regular

Forwarding session.  With this "Always" option you don't have to

wait for the next scheduled BBS Forwarding session.

However this option is not recommended for normal circumstances.

I do not believe it is necessary to make several calls a day for

this purpose.  If you are getting THAT much incoming mail, the

BBS Sysop will probably have set you up for several Forwarding

sessions a day and it will all be in your PMS by now!

Page 69

Choosing "Always" will generate additional activity on the

frequency without achieving too much (unless you are expecting a

whole lot of incoming Mail!).  If your local frequency is

congested, you will keep more friends if you choose the former

Mail option.

If you find, for some reason, the BBS is not calling you for a

regular Forwarding session, you could still leave this one set

to Mail and "Always" call the BBS once per day (see below).  It

should not really be necessary to make a call every hour.

N - No.

Don't call the BBS every hour; don't even bother to check the PMS.

In other words, don't make this hourly call under any

circumstances - just forget the hourly processing altogether.

When?  (minutes past the hour)

If the above option is set to Mail or Always, paKet will perform some

PMS processing once every hour.  This option specifies the actual

minute past the hour when this processing takes place.

For example if this is set to 59, paKet will "wake up" at 59 minutes

past the hour (oh ok, one minute before the hour if you insist) and

will check the PMS and/or call the BBS at that time.

If the program needs to call the BBS but you happen to be connected to

someone else at the time, paKet will wait till that connected session

is completed before making the call to the BBS.  For some, especially

those less experienced with packet radio, it might be confusing if all

this suddenly happened while you are in the middle of a conversation

with someone else!   I figured this mail Forwarding should be a

background function and it can wait.

Call BBS once per day?  (M, A, N)

This option is similar to the option for hourly calls above.  But this

one refers to a call to the BBS just once per day.

The same three options are available here so I won't go into the full

detail again.  However there are some additional considerations here.

This time I recommend the "Always" setting.

To cover those situations where the BBS is not initiating a regular

call for the purpose of Mail Forwarding and where you have not been

writing any outgoing messages, we might need to make that call

ourselves. This will help ensure we still receive any incoming mail

that arrives via the BBS.

If you are satisfied the BBS is calling you at regular intervals, you

can set this to "No", and not make this daily call.

Page 70

What time of day to call ? (hh:mm)

If you do decide to make a call to the BBS once per day, whether the

previous option is set to Mail or Always, this is the time of day that

activity will take place.

The time is recorded here in the format HH:MM using a 24 hour time

format.  Examples:

10:05     five minutes past ten am

12:00     midday

15:59     one minute before four pm

A colon (":") is used as the separator between the hours and the

minutes.

Use Script to make BBS connection?

This is a Y(es) or N(o) parameter.

When paKet attempts to establish a connection with your BBS, it needs

to know how to make that connection.  In most cases it is simply a

matter of issuing a single CONNECT command to the TNC followed by the

callsign and the callsigns of any digipeaters you need to get there.

For example if I was within range of my local BBS, I could type:

CONNECT VK2ATM-1

Or if I need a digipeater in the path, I could type something like:

CONNECT VK2ATM-1 v VK2RPM-1

In the case of a simple CONNECT command like this, say "N" to this PMS

option because we do not want to use a Script here.  And specify in the

next option the callsign and digipeaters your system needs to reach the

BBS.

But sometimes it is not so simple to reach your BBS.  It can get pretty

involved sometimes and just issuing a single CONNECT command is not

enough on its own.

For example, you might have to:

1.  Issue a CONNECT command to a Network Node;

2.  Wait for that connection to be established;

3.  Then send another CONNECT command, this time asking the Node we

are now connected to, to try to connect to our BBS.

This is where paKet Scripts may be used to advantage.  If you have this

type of complex path to reach your BBS, it might be better to use a

Script to make the connection because in a Script we can tell paKet to

issue various commands and to wait for certain responses before issuing

other commands, etc.  It provides much more flexibility.

So if this is your situation, say "Y" here and in the following option

specify the name of a Script file you will use to establish the BBS

connection.  I have included some sample Script files with paKet so

Page 71

have a look at those to get an idea how to do it.  Of course you will

have to modify them to suit your own situation, with different

callsigns etc.  Maybe there will be someone else in your area who has

already developed a Script to do this, so ask around before you spend

too much time on it.

Connect path/Script file for BBS

This entry tells paKet how it can reach and establish contact with your

BBS for a Mail Forwarding session.  It is used whenever the program

decides it is the "appointed time" as entered in earlier options, or if

you type <Alt-F3> to manually initiate a Forwarding session.

There are two formats for this item, the correct format depends on the

choice you made in the previous PMS option.

Standard CONNECT command - No Script

If you said "N" to the previous option, you have chosen to use a

standard CONNECT command to establish contact with your BBS.  So here

you should enter just the callsign, and any digipeater callsigns, that

you use to call your BBS.

For example, if I can connect to my local BBS with a simple connect

command as follows:

CONNECT VK2ATM-1

then I would specify here:

VK2ATM-1

If I needed to use a digipeater as indicated in the second example

above, I would enter here:

VK2ATM-1 v VK2RPM-1

Note, you do NOT type the CONNECT command - that is generated within

the program.  Just the callsigns here, please.

Use a Script file

If you specified "Y" to the previous option, you must specify here the

name of the Script file you will be using.

There is a special syntax required here to properly identify this entry

as a Script file name and not just some other text, such as callsigns.

paKet looks for the pointer brackets if you are specifying a Script

file name so if you have a Script file called BBSCONN.SCP you would

enter here:

<BBSCONN>

You could type the extension too if you wish, but if no extension is

specified paKet will assume .SCP and I recommend you use that extension

for all your Scripts.  There is less confusion that way.  paKet will

look for that Script file in your Scripts Directory.

Page 72

Configuration Windows - File Transfer options

������ 7. File Transfer options...���ͻ

� DWAIT  for normal operations     16 �

� DWAIT  for binary operations     48 �

� PACLEN for normal operations    240 �

� PACLEN for binary operations    252 �

� Use paKet Protocol?               Y �

� Ctrl-Z after sending ASCII file?  N �

��������������������������������������1/4

Description.

This window appears when you select option 7 from the System

Configuration Window.

These options relate to File Transfers only.

DWAIT  for normal operations

DWAIT  for binary operations

DWAIT is a TNC parameter that specifies a delay in the TNC with a view

to minimising collisions with digipeated packets. Some operators

recommend a setting of 48 for DWAIT during File Transfers in order to

give other Conversational Mode contacts a better response on a crowded

frequency. The principle here is that a file transfer should be run as

a sort of "Background" process. So, it is considered responsible to

give a longer DWAIT time, especially if you have to do your file

transfers at a busy time.

There are two values for DWAIT that you can set here: the "DWAIT for

binary" value will be used while sending a binary file; and the TNC

will be reset to the normal DWAIT value when the File Transfer is

completed.

Recommended values are 16 for normal mode and at least 32 for Binary

File Transfers.

PACLEN for normal operations

PACLEN for binary operations

The TNC's PACLEN parameter specifies the maximum size of the packets to

be transmitted. It has been found that in most cases a larger PACLEN

value is more efficient for File Transfers on VHF, but perhaps smaller

on poorer HF paths.  It is generally accepted that a large PACLEN for

normal conversation is less important.  paKet gives you the ability to

specify different PACLEN values for binary file transfer and normal

conversation modes.

When you are performing a binary transfer paKet will set the TNC's

PACLEN parameter to the value you specify for "binary operations" and

when the transfer is completed, it sets it back to the "normal

operations" value.

Page 73

Either value can be as high as 256, or 0 which in some TNCs is

equivalent to 256.  It was found in testing that some TNCs do not

handle PACLEN 0 correctly (the TNC "crashes" during file transfers

after 7 packets!). A value of 255 or less appears to be universally

reliable.  Recommended values are 240 for normal mode and 252 for File

Transfers - certain advantages accrue from these settings.

When using normal Converse Mode, the <Enter> at the end of each line

determines the length of the packets transmitted, unless you are using

CPACTIME. Provided you are using paKet's Type Ahead buffer, the use of

CPACTIME with a one second delay is considered more efficient in many

cases than the older <Enter> method. For normal conversations there is

little difference but if you are transferring an ASCII Text file (refer

<F5> key) the CPACTIME method would use this "normal operations" PACLEN

specified here and would result in more efficient transmissions. (Not

all TNCs support CPACTIME so check your TNC manual for details.  It

will explain CPACTIME operations better than I can.)

Use paKet Protocol?

paKet Protocol (pP for short) is an enhanced Binary Transfer protocol

developed especially for Packet Radio and is fully compatible with

YAPP, the current "Standard". Refer to the section on paKet Protocol in

the Technical section of this Manual for more details if required.

I used the YAPP Protocol Specifications published by Jeff Jacobsen

(WA7MBL), the author of the original YAPP program, when developing this

pP system. Extensive testing has shown pP to be fully compatible with

YAPP so you can use pP to exchange Binary files with any station

equipped to handle the YAPP protocol. The pP protocol automatically

detects the presence of another pP station and will use the enhanced

features as required.

However, just in case there is some other system somewhere that does

not quite conform to the original YAPP specifications and causes some

difficulty with the new pP, I have provided this option to turn off the

advanced protocol, forcing paKet to use the Standard YAPP Protocol for

Binary Transfers.

I expect this option will always be set to "Y".  You do not need to turn

pP off when exchanging files with a station using standard YAPP protocol.

Ctrl-Z after sending ASCII file?

When you send an ASCII Text file with the <F5> key paKet will

optionally generate a <Ctrl-Z> character as an End-of-File indicator,

after the last byte is sent.

Most systems receiving a file or message require the <Ctrl-Z> to close

the file or terminate the message being received. So you might find it

convenient to set this option to "Y" and allow paKet to generate the

code when it detects the end of the file you are sending.

For flexibility, however, the choice is yours. If you prefer to enter

the code manually set this option to "N".

Page 74

Configuration Windows - Directories and Files

��������������������� 8. Directories and Files...���������������ͻ

� Default paKet directory    C:\PAKET\                           �

� Text Recv directory        C:\PAKET\TEXT\UPLOAD\               �

� Text Send directory        C:\PAKET\TEXT\                      �

� Binary Recv directory      C:\PAKET\BIN\UPLOAD\                �

� Binary Send directory      C:\PAKET\BIN\                       �

� Log File directory         C:\PAKET\LOG\                       �

� PMS directory              C:\PAKET\LOG\                       �

� Scripts Directory          C:\PAKET\TEXT\SCRIPTS\              �

� ------------------                                             �

� Online Manual File         C:\PAKET\PAKET.DOC                  �

� TNC Help File              C:\PAKET\PK-232.HLP                 �

� System Editor              D:\EDWIN.COM                        �

� Connect File               C:\PAKET\PAKET.DIR                  �

� Command to list a file     C:\UTILITY\LIST                     �

�����������������������������������������������������������������1/4

Description.

This window appears when you select option 8 from the System

Configuration Window.

The paKet system manages a number of directories and files and in this

window you may specify your preferred defaults.

Note, in the above window the first eight items are DIRECTORY names so

for these items please specify a drive and sub directory name finishing

with a backslash character ("\"). So, if you want to specify the

diskette in drive A: using its root directory, you would specify "A:\".

Default paKet directory

There are a number of miscellaneous files used by paKet and these files

should be included in this default paKet directory.  The easy way to

manage this is to make this entry the same as the directory where you

have the paKet program, PAKET.EXE and its configuration file, PAKET.CFG.

Those miscellaneous files include the REMOTE help and information

files, REMOTE.HLP and REMOTE.INF; and any CTX files you choose to

create.  The Configuration Help file (PAKETCFG.HLP) should also be

stored here in this default directory.   In fact any part of the paKet

system that is not configurable in one of the following options, should

be stored in the default directory.

The REMOTE Help and Information files are sent to a REMOTE user if a

"H" or an "I" command (respectively) is entered.

A CTX file is a text file with a file name that matches the callsign of

someone you expect will connect to this system, and with an extension of

.CTX.  So, if I have a text file named VK2BZC.CTX in this default

directory and Paul (VK2BZC) connects to this system, paKet will send the

contents of that VK2BZC.CTX file immediately the connection is processed.

Page 75

Although the Configuration Help file is a plain text ASCII file, the

format and content have been carefully arranged and it would be a

mistake to er, improve it!  Leave it alone, huh?

Text Recv directory

This is the default directory the system uses when we receive an ASCII

text file.

This would be when you select the Receive File function with the <F6>

key, or it would also be used by a REMOTE user if that station sends

us an ASCII text file.

When you press the <F6> key to receive a file, paKet will pop up the

Directory Window showing this "Text Recv" directory as the default

directory, but you can select any drive/directory/filename you wish.

But a REMOTE user does not get a choice - any files sent by a REMOTE

user will always go into this "Text Recv" directory.

Text Send directory

The first time you press <F5> to send a file, paKet will pop up a

Directory Window displaying this "Text Send directory". If the file you

want to send is in some other drive or directory, you can change the

display to show that other drive and or directory to help you select the

file.

I decided it would be useful if paKet remembered the details of that

file, so next time you press <F5> it will come up with that same

filename highlighted.

So this Text Send directory is used mainly for REMOTE operations.

REMOTE operations are discussed elsewhere but it should be pointed out

here that a REMOTE user may issue a "W" ("What") command to ask paKet

to display a directory of our text files.  paKet will send this "Text

Send directory".  For your protection, this directory and its sub

directories are the only directories a REMOTE user will see.

This directory will also be used by a REMOTE operator if that station

wants to transfer one of our text files.  A REMOTE user may access this

directory and any sub directories below this one, so if you specify the

Root directory here, the REMOTE user will be able to access any part of

the specified drive for downloading.

Binary Recv directory

This is the directory the system uses when we receive a Binary File.

In paKet there are four ways to transfer a Binary file:

paKet Protocol (pP);

YAPP protocol;

BayCom; and

RAW Binary (no protocol).

Page 76

Whichever method is chosen, paKet uses this directory to receive the

transferred file.

You are not asked for the file name when receiving a Binary File with

pP or YAPP: that information is expected to be in the protocol Header.

So if you wanted the Binary File to go into a different directory or to

have a different name, you will have to copy it or Rename/Move it

(refer <Alt-R> key) after the transfer has completed.

Binary Send directory

We can select a file to send via one of the Binary File Transfer

protocols with the <F7>, <Alt-F7> or the <Alt-F5> keys.  As I mentioned

under the Text Send directory above, paKet will remember the last file

sent so when you next select a file to send it will highlight that same

file.  You might not necessarily want to send the same file again, but

my experience is that you will quite likely want to send one from the

same directory as the last one.

So here too, this item is used primarily for REMOTE operations. This is

the default directory the system uses when we send a Binary File to a

REMOTE user.

If the REMOTE user asks for a directory display with the REMOTE "YW"

(or "MW") command, paKet will send the contents of this Binary Send

directory.  Note, the REMOTE user can see and access only this

directory and any sub directory below this one.  Other directories

outside this path are inaccessible.

Log File directory

This is the default directory the system uses for the disk log file.

When you press the <F2> key to activate the log file, you will be asked

to select the file name and, though a default is offered using the

drive and directory specified here, you may specify a different name,

directory or even drive if you wish.

If you have the AUTO log option active, paKet will generate a Log File

name based on the computer's date and will create the file in this

directory. You don't have to do anything!

PMS Directory

paKet has a Personal Message System which operates in a style similar

to many of the BBS systems, and is explained in its own section of the

Manual.

There are two files maintained by the system for the PMS, one to

contain the messages themselves (PAKETPMS.DAT) and the other for an

index into the message system (PAKETPMS.IDX).  The directory specified

here is where these files will reside.

Page 77

Once you specify the directory here, there is nothing else you need to

do. If the PMS files cannot be found, paKet will automatically create

new ones.  Of course if you change this directory entry, next time

paKet tries to access the PMS, the files will probably not be found in

the new directory and it will create new files in this directory after

warning you the old files have been lost.

So if you do want to change this directory name, make sure you copy the

two PMS files to the new directory, to keep the system happy.

Scripts directory

Your Scripts may be stored in any directory because when you press

<Alt-S> to run a paKet Script, a Directory Window will pop up to help

you select the desired Script file, and you can swap and change until

you find the right directory.

However, when a REMOTE user enters a "P" command to Perform a Script or

to display a list of our Scripts, paKet will look in THIS Scripts

directory for those files.  And if you use a Script to establish

connection with your BBS, paKet will expect to find that Script here in

this directory.  Or if you have a Script filename in your Connect

Window, that Script must be here in this directory.

So, I suggest you make it a rule to store ALL your Scripts in this

default Scripts directory, including AUTOEXEC.SCP and AUTOEND.SCP if

you use those.  It would save a lot of trouble, I think.

-------------------------------

The following items are FILE names although you can specify a drive

and/or subdirectory path as well.

Online Manual File

paKet includes an Online version of this Manual. Actually it IS this

Manual!

Here you specify the path and file name so when you press <Alt-F1>

paket knows where to look for the file. If you have sufficient disk

space the entire paKet Manual should be combined into a single file,

PAKET.DOC which would be the name specified here. If you prefer, you

could leave the original DOC files separate and specify here the

desired part, eg MAN01.DOC.

TNC Help File

The TNC Help File is the name of the TNC help file that you want to

use. For example, if you have a PK-232 and you had that help file in a

directory called C:\HELP, you could set this item to: C:\HELP\PK-232.HLP

The TNC Help file contains information on the TNC commands, the meaning

of the various options, etc.   Select the Help File most appropriate to

your TNC for maximum benefit.

Page 78

System Editor

The System Editor is an external text editor you may call up with the

<Alt-E> key.

This item allows you to specify the name (and optionally the path) of

your favourite text editor. Any editor may be used.

This editor will be run with paKet in the background, so there are a

couple of considerations to keep in mind here:

1. paKet will continue to capture any input from the TNC and store

it in the System Input buffer while you are using the editor. So

you should be able to run the editor at any time without losing

any input data.

2. As paKet is still in the background, we will not have the full

amount of memory available for the editor. So you may not be

able to edit a very large file or may not even be able to load

the editor if memory demands are high.

For example if you have specified several Communications Windows

there will be less free memory available; and/or if your favourite

editor happens to be one of the LARGE Word Processing packages it

might not fit into the available memory alongside paKet anyway!

Most of the simpler "Text" or "Programmer's" editors should work

just fine.

Connect File

This is the name of the file you wish to use for paKet's Connect

facility. The Connect file contains the names and connect paths for

our regular contacts and will be used when you select <Alt-C> to

initiate a connection to another station.

It is conceivable we could have more than one Connect file although

one file will hold up to 200 entries which should be more than enough

to satisfy most users.

Refer to the Connect Window section for more on the Connect facility.

Command to list a file

If you have pressed <Alt-D> to display a Disk Directory, you may

highlight any file listed in the directory and press <Enter> to view the

contents of that file. paKet will use THIS command to display the file.

You may specify "TYPE", the internal DOS command, although I prefer to

use the excellent Public Domain utility, LIST. If you want to use an

external program (such as LIST), don't forget DOS will need to be able

to find it, so you might need to specify the Drive and/or directory

name along with the command. Of course this will not be necessary if

the program is it is in the current directory or if it is covered by

the DOS PATH statement.

Page 79

Configuration Windows - Miscellaneous options

������������� 9. Miscellaneous options...�����������ͻ

� Station callsign                  VK2DHU           �

� Operator's name                   Tony             �

� Msg level (All,Warnings,Errors)   A                �

� Message duration (seconds)        3                �

� Display Configuration Help?       N                �

� Eliminate SNOW? (CGA only)        N                �

� AUTO LOG (on while Connected)     Y                �

� Add headers to log file?          Y                �

� Screen blanker delay (seconds)    60               �

� Up/Down Arrows (F'back, Recall)   F                �

� Initialise TNC time at startup?   Y                �

�����������������������������������������������������1/4

Description.

This window appears when you select option 9 from the System

Configuration Window. It contains those configurable variables that did

not seem to fit into any of the other sections.

Station callsign

Operator's name

These items are used in a couple of places for display purposes. For

example some REMOTE menu messages will show your name and/or callsign

in order to make it appear a little more friendly.

The callsign is also used in the PMS. If a REMOTE user leaves a message

without specifying a callsign, paKet will assume it is for you and will

insert your callsign as the addressee.  Similarly when you enter a

message into the PMS, there is no need to specify who it is from.

paKet now knows who you are!

Registering your copy of paKet

If this copy of paKet is not yet registered, paKet will ask you for your

Serial Number when you enter your callsign.  If you have your Serial

Number handy, type it in here and paKet will update its stored

information before issuing a Thank You message.

The registration process will update the PAKET.EXE file to identify

this one as your registered copy.  If you experience a problem in this

process, it is possible the Operating System is unable to write to that

PAKET.EXE file.   The two most common reasons for this are:

1. You have write-protected your copy of PAKET.EXE, perhaps with the

Read-Only file attribute;  or

2. You are denied access to that file by the Operating System.

For example if you are running the Microsoft SHARE program, the

system will not allow us to write to the PAKET.EXE file because

it is currently in use.

Page 80

Of course the @#$&* thing is currently in use - WE are using it!!!

It can come back with a fearful error message like "Disk error on

Drive C: - General Failure".  Just type "I" to ignore the error

message.  Of course the registration detail will not have been

recorded in this case.

I imagine a similar problem could occur if you are running paKet

from a Network drive and you don't have appropriate privileges to

update the file on that drive.

In the first case, you can rectify this by removing the Write

Protection or the change the file attributes. Then enter your callsign

again, or simply register the program by typing <Alt-R> while paKet is

running.

In the second case, if the problem is caused by SHARE you can remove it

temporarily (usually it is in your AUTOEXEC.BAT file so you would have

to edit that and reboot), or if you find SHARE a bit of an

inconvenience anyway, use the NOSHARE program instead of running

SHARE in the first place!

If it is a Network problem, consult your Network Supervisor or

Administrator for help.  Tell them you need to write to the PAKET.EXE

file and ask them to set up the necessary access rights for you. Or get

them to perform the paKet registration (using YOUR callsign of course!)

if they happen to have the necessary rights already.

The alternative to this is to register the program with paKet's INSTALL

program. You will not have to perform the full installation again -

just select the option to Register the program. Have a look at the

INSTALL.DOC file for details on the use of the INSTALL program.

Don't worry about updating your copy of PAKET.EXE here.  It is just

writing your Serial Number, nothing else!  You can still give it to

others to try, even after recording your Serial Number.  If you are

REALLY concerned, keep an unregistered copy as a backup and give them

that.

If you do not yet have your Serial Number, just press <Enter> and the

program will continue without performing the registration.  You will

not be disadvantaged at all.  Some Shareware programs provide a cut

down version of the real thing until you register.  Well, I am happy to

say paKet is different - I believe you should have access to all the

features of the paKet system so you are better able to appreciate its

true value.  The only differences in a registered version of the

program are, your callsign will appear:

- in the top left corner of your screen;  and

- in your REMOTE Menu.

So the difference is minimal and you still get all the benefits of

the paKet system.  It is tempting to not register at all, isn't it?

Page 81

But, let's be reasonable now.  I have put a lot of time and effort into

this, as I am sure you will appreciate.  Australians, as a general

rule, like to be fair to their "mates".  Well, if you think I am being

fair to you, please be fair to me and register your copy of paKet.  If

you think it is too expensive, tell me;  otherwise I would appreciate

your kind consideration.  Have a look at the file REGISTER.DOC for

details on how to get your own Serial Number.

Configuration Windows - Miscellaneous options cont...

Msg level (All, Warning, Error)

There is a complete listing of paKet's messages in its own section of

this Manual. They are listed in alphabetic sequence.

Some messages are considered Information Only. They tell you what is

going on at a particular time but are not really important as no action

is required. Warning messages alert you to something unexpected but in

many cases paKet can cope without your intervention, taking whatever

action it deems necessary!  Error messages are those that occur in

situations that are considered serious and usually require some action

on your part.

This option allows you to specify the level you wish to use.

With level A - All messages will appear;

level W - Warnings and Errors only will appear;

level E - Errors only will appear.

I know the paKet system pretty well because I wrote the program!  And I

prefer to leave this set to All.  So I recommend EVERYONE, regardless

of experience, specify A (All) to start with, maybe changing the level

later. I expect it would be in very unusual circumstances that anyone

would want to hide the Warning messages by specifying level E.

Message duration (seconds)

When displaying a message on the screen, the program pops up a Message

Window and leaves it on the screen for a while to give you time to read

it.  At first those messages were left on the screen for about three

seconds, but I received some complaints (well, let's call them

"suggestions") from paKet users saying either:

"Some of those messages are not left on the screen long enough.

I don't have time to read them!"   or

Page 82

"Some of those messages stay on the screen for too long.

They get in the way!"

So here we have an option where YOU can decide how long they stay on

the screen.  If you specify a longer duration here you can always get

rid of that Message by pressing <Esc> while the Message Window is

displayed.

Display Configuration Help?

This is a Y(es) or N(o) parameter.

One of the most loved features of paKet is its Online Configuration and

the number of options available to set up the system to an individual's

own preferences.

At the same time, this is one of the more difficult parts of the system

because a less experienced paKet user might find all these options a

little daunting.  Until you gain some experience with paKet, even some

of the terminology might seem confusing.

So we have an optional Configuration Help facility which will pop up a

Help message for each of the items in paKet's Online Configuration.

This is somewhat limited because of the available space on your display

screen, but it does serve to offer some helpful information on the

particular option you have selected, and hopefully guide you to the

correct choice every time.

This Manual provides much more information so you should refer to this

for the full details of each of the configuration options.  Check it

out with the Online Manual if the printed copy is not to hand.

The Configuration Help comes from a file PAKETCFG.HLP which should be

in your default paKet directory.  Although this is a plain ASCII text

file the content and the format of this file have been carefully

prepared so please leave this file alone.

Normally this Configuration Help option will be set to "Y" especially

while you are new to the system.  It might slow you down a bit, but try

it both ways and make up your own mind.  If you have a slower system,

and especially if you are running on a machine without a Hard Disk, you

might prefer to set this option to "N" and run without the

Configuration Help.

Eliminate SNOW? (CGA only)

This is a Y(es) or N(o) parameter.

The SNOW parameter is used for antique CGA displays that misbehave when

a program writes directly to the video display's RAM. The effect on

these displays is a 'snow' effect. paKet uses direct memory writes to

enhance performance but it will use a different (slightly slower)

method if you set this parameter to 'Y' to minimise the 'snow'.

Page 83

Note, however, that this is for CGA displays only - it is not necessary

with other types of display adapter and might not work with them

anyway!

AUTO LOG (on while Connected)

This is a Y(es) or N(o) parameter.

paKet provides a disk logging facility which may be activated at any

time with the <F2> key. However, the AUTO LOG feature (if ON) will

automatically activate the disk log file using the default naming

convention when a connection is established with another station and

will turn it off when disconnected.  This allows you to keep a log of

your communications with other stations without clogging the log file

with other data.

Set this field to 'Y' if you want to use the AUTO LOG feature.

Add headers to log file?

This is a Y(es) or N(o) parameter.

The Log file contains a record of all activity that comes and goes

through the Communications Windows.  When you are connected to only one

station it is quite obvious where that information came from, but if

you happen to be connected to more than one station at a time, it all

goes into the same log file and it is sometimes not so obvious later

on, who said what.

There is an optional header that paKet will generate and write into the

log file to help overcome that confusion.  This optional header simply

contains the callsign of the station you are connected to in that

Communications Window.  When something is written to the log from one

of the other Communications Windows, paKet will generate another header

and write it to the file before logging that other information.

It is not a lot of overhead and it does not noticeably affect the

operation of the system, so set this option to "Y" and see it for your

self.

Screen blanker delay (seconds)

The nature of Packet Radio is such that you might leave the system

running even while you are not actually present, especially with the

availability of paKet's REMOTE facilities.  However, prolonged display

of unchanging information on a screen can cause a "burning-in" effect

which may be relieved with a Screen Blanker to switch off the display

when nothing is happening.

Page 84

paKet has its own Screen Blanker which becomes effective after a

specified period of inactivity. This option allows you to specify the

period.

For example, if you specify 60 seconds here, the screen will be blanked

if nothing is received from the TNC and if no keys have been pressed

during the past 60 seconds. If something is received from the TNC or if

you press a key, the 60 second countdown restarts, so in times of

normal activity the screen might not blank at all!

If the screen is blanked, it will be restored for normal communications

immediately a character is received from the TNC or upon any keystroke

from you.   Note, you can simply start typing a message because your

keystroke will be processed by the system even if you could not see the

paKet screen when you pressed the key.

If you simply want to redisplay the screen without actually "typing"

something, I suggest you press Ctrl or Alt (without any other key).

These keys will be detected by paKet and will cause the Screen to be

restored, but they do not actually generate any character on their own.

A value of 0 (zero) here tells the system you do NOT want any screen

blanking at all.

Up/Down Arrows (F'back, Recall)

Here you must enter either "F" or "R".

The <Up-Arrow> and <Down-Arrow> keys may be used for different purposes

in paKet and it is hard to know which is best.  For example, they may

be used to recall lines of data from the Type Ahead buffer.  This is

how it works in paKet 5 and is how they are used in DOS (DOSKEY) for

recalling previously typed lines.  Many people like it this way.

However, in early versions of paKet the arrow keys were used for

scrolling the Flashback data in the Communications Window.   Some long

standing paKet users did not like the change and some didn't even

realise they could have this function with <Shift-up-arrow> in paKet 5!

I happen to use the Flashback more frequently than I recall lines from

Type Ahead so I prefer the old way.  But, rather than impose MY

preferences on you, I added this option so you can do it whichever way

you like.

If you type "F" here, the arrow keys will be used for scrolling the

Flashback buffer.   With this setting you can press <Shift-Up-Arrow> or

<Shift-Down-Arrow> for the Type Ahead Recall.

If you type "R" here, the arrow keys will be used for Recalling lines

from the Type Ahead buffer.  With this setting you can press

<Shift-Up-Arrow> or <Shift-Down-Arrow> to scroll the Flashback Buffer.

Page 85

Initialise TNC time at startup?

This is a Y(es) or N(o) parameter.

Some TNCs have an internal clock and do not need the program to send

the computer's date and time every time the program starts up.  In fact

I am told the TNC clock is often more accurate than the computer's

time-of-day clock and it is better if it is NOT sent!

If you want to use paKet with a telephone modem you will not want the

date/time sent either because those modems do not have a DAYTIME option!

For normal use and for most TNCs, set this option to "Y".

Page 86

Connect Window

��Name         ->� Callsign to CONNECT to (including Digipeaters, etc)�Ŀ

�Call BBS        <BBSCONN>                                              �

�Get BBS Mail    <GETMAIL>                                              �

�Paul            VK2BZC                                                 �

�Geoff           VK2EO v vk2rpm-1                                       �

�Gerard          VK2DAA v VK2RPM 475700                                 �

�Newcastle TCP   VK2EO-1 v VK2RPM 655400;VK2RAP                         �

�David           VK2RND v VK2RPM 496800;VK2KDM                          �

�                                                                       �

� <Enter>:CONNECT; <Ctrl-Ins>:insert new entry; <Ctrl-Del>:delete entry �

Description.

The Connect Window appears when you press <Alt-C>.

It contains a list of names and connect paths for your regular

contacts. This feature is provided to facilitate the connection

process. You may record up to 200 entries in the Connect Window.

At first the window will be blank but you may enter the details of your

regular contacts to more easily establish contact with them next time.

To make a connection, move the highlight bar to the desired entry and

press <Enter>.

In the example Connect Window above you will see several forms of

connection details.

Simple Connection

The simplest form is shown on the third line which I can use to

connect to Paul, VK2BZC.   The left hand side of the Window contains a

name or identifier to help me quickly see who this line is for.  This

column is not used by the paKet system - it is just there to help us

humans.  But to the right, on this line, is the callsign.  So when I

select this line paKet will generate a "CONNECT" command followed by

this callsign. Eg:

CONNECT VK2BZC

Digipeater Connection

For such a simple style of connection it is hardly worthwhile using the

Connect Window.  But where you need to enter more details, for example

where there is a digipeater involved (eg: line 4 to "Geoff") or network

information such as the ROSE numbers you see on line 5 (to "Gerard"),

the Connect Window can save you a lot of typing and remembering.

Network (Multiple) Connection

Some Networks require you to connect to a particular Node, then when

that connection is established, you issue another connect request to

the desired station.  Provided you have NEWMODE ON in your TNC, you

could use something like the line to "Newcastle TCP" where there are

two separate connect paths, separated by a semi-colon (";").  Using

that example, paKet will generate two CONNECT commands as follows:

Page 87

CONNECT VK2EO-1 v VK2RPM 655400

and

CONNECT VK2RAP

You might like to refer to your TNC Manual for details of the NEWMODE

parameter and also to the discussion on NEWMODE in the Configuration

Window for Multi User Options (in this section of the paKet Manual).

With NEWMODE ON, the TNC will change to Converse Mode immediately a

CONNECT command is issued.  So we can take advantage of this situation

by sending two "CONNECT" commands, because we now know the first will

be taken as a command (paKet will always place the TNC in command mode

before sending the first CONNECT command); and the TNC will switch into

Converse Mode so our second CONNECT "command" will be transmitted to

the other station.  In this case, the string "CONNECT VK2RAP" will be

transmitted to the Node (VK2EO-1) and hopefully the Node will process

that string as its own CONNECT command so we can get through to VK2RAP.

Complex Connection

That previous example (the Multiple Connection) is a bit of a worry to

me.  I am not at all confident it will always work as explained.  I

must admit I have never experienced a problem with that method, but I

don't have good vibes when I use it.  Must be something to do with my

hidden childhood memories?

In any case, there might be situations where the path to your desired

connection is more complex or you might want to perform some special

processing such as automatically entering a password or account

number. Or you might want to regularly connect to your BBS and read the

mail or something like that?  In paKet, a Script offers maximum

flexibility.  I won't go into details on Scripts here - that is covered

elsewhere.  But let's say we already have a Script that contains the

various CONNECT commands to establish the connection and perform the

desired processing.

That Script may be entered as shown on line 1, where I use the

"BBSCONN.SCP Script to connect to my local BBS.  A Script file name is

entered in Pointer Brackets as shown. The file extension is not

required because paKet will assume an extension of .SCP.  That Script

file should be available in your default Scripts directory.

So when you select that line in the Connect Window, paKet will recognise

that as a Script file name from the Pointer Brackets and instead of

generating a CONNECT command, it will simply load and run that Script.

Come to think of it, you could use this Connect Window as a launching

pad for any of your Scripts, whether they relate to a new Connection or

not!  I have another one there on line 2 - this is one I wrote to call

the BBS and ask for certain Mail and Bulletins.

To create a new entry, type in the left hand column a name that you can

easily recognise, then press <Tab> to move to the start of the Callsign

section. Here you enter the details of the connect path you wish to use

or Script file name, etc.

Page 88

If you want to insert a new entry between two existing entries, move to

the desired line and press <Ctrl-Ins>.  If you want to remove an

existing entry, move to that entry and press <Ctrl-Del>.

Press <Esc> if you do not want to make a Connection at this time. Any

changes are saved automatically.

Page 89

Disk Directory Window

������������������C:\PAKET\������������������ͻ

�        (Enter file name manually)           �

�        (Switch to another drive)            �

� ..            <Previous directory>          �

� BIN           <DIR>                         �

� BOOK           2577  27-01-94   8:44        �

� CONTACTS      11142  19-11-93  10:04        �

� EXITIN.SSS      290  18-10-93   8:53  R     �

� EXITOUT.SSS     260  18-10-93   7:45  R     �

� FLAGS           531  31-05-91   6:42        �

� FWD           <DIR>                         �

� KISS.DOC      21219  26-11-93  13:54        �

� LOG           <DIR>                         �

� PAKET.ICO       874   8-08-93  19:42        �

� PAKET.LZH    875564  30-11-93   7:41  A     �

� PAKETDOC      <DIR>                         �

� PP.DOC         5701  11-07-93   8:18        �

��������� Directory - 24,363,008 free���������1/4

Description.

This window will appear whenever you request a directory display (with

the <Alt-D> key). It will also appear when paKet asks you to choose a

file name (eg if you want to send a file to the other station).

The directory is displayed in alphabetical filename sequence with

hidden and system files and the volume label displayed in a different

shade.  Also shown are the file date/time and file attributes.

At the top of the window is the drive\path of the displayed directory,

and on the bottom line is the amount of free space on that drive.

If there is another level of subdirectory below this one, that

subdirectory name will be listed in sequence with "<DIR>" instead of a

file size. In the example above, there is a subdirectory named "BIN"

shown. Selecting that item will ask paKet to display that subdirectory

in this Window.

You may scroll up and down this window using the <Up-Arrow> and

<Down-Arrow>, the <PageUp> and <PageDown> keys, or the <Home> and <End>

keys. Or you can use paKet's hot-key typing by entering part or all of

the file name - paKet will highlight the first filename that matches

the characters you have typed so far. Often all you need to type is one

or two characters to move to the file you want.

If you hold the Shift key as you type, paKet will match only Sub

Directory names.  For example, using the window shown above, if we type

"P" the highlight bar moves to PAKET.ICO, the first file that begins

with "P".  But if you hold the Shift key while typing "P", the

highlight bar moves to PAKETDOC, the first Sub Directory name that

begins with "P".

Page 90

An item may be "selected" by moving the cursor (the highlight bar) to

that item and pressing <Enter>.

Sometimes you might prefer to enter the file name yourself rather than

go through a number of display options. This is especially so if you

want to create a file that does not yet exist on the disk (for example

if you are using the System Editor to create a NEW text file). In this

case, press <Home> to jump to the top line and select the option to

"(Enter file name manually)" and paKet's Message Window will pop up for

you to enter the file name.

You may display the directory of a different drive by selecting the

"(Switch to another drive)" option second from the top of the window.

And you can select "<Previous directory>" to go back a level.

(Hint:  type a "." then <Enter> to quickly move to the previous or

parent directory when searching the disk.)

If you used <Alt-D> to Display the directory, selecting a file will

display the cntents of that file. The command used to perform the

display is configurable - refer to the "Configuration Windows -

Directories and Files" section for details.

If you press the <Delete> key, paKet will delete the highlighted file

after asking for your confirmation that you really wish to perform the

deletion.  This is convenient if you are doing some housekeeping such

as cleaning out unwanted log files. You can use the System Editor to

inspect log files on the disk and to edit out unwanted data. Then

delete any unwanted files. All without leaving paKet.  You may delete a

sub directory only if that sub directory is empty.

Alternatively, a file may be renamed or indeed moved from one directory

to another by pressing <Alt-R> whilst that file is highlighted.  You

are then asked to type the full new path and name for the file.  If a

new name is typed without a path, the file will remain in the current

directory.   Note, DOS does not permit a file to be "moved" to another

drive.  You will have to go to DOS and COPY the file to get it on to

another drive.

Page 91

Help Window

������������������������������ paKet 6.1 Help ���������������������������Ŀ

� F1  - Help                          Alt-A  - Turn Alerts OFF            �

� F2  - Turn Log File ON              Alt-B  - Send a BREAK signal to TNC �

� F3  - Turn REMOTE mode ON           Alt-C  - Connect to another station �

� F4  - Word Wrap/Single Line         Alt-D  - Disk Directory             �

� F5  - Send an ASCII text file       Alt-E  - Edit a data file           �

� F6  - Receive ASCII text file       Alt-F  - Find text in Flashback     �

� F7  - Send Binary File using pP     Alt-H  - MHEARD                     �

� F8  - Recv Binary File using pP     Alt-I  - Initialise Comms Windows   �

� F9  - DOS Gateway                   Alt-J  - adJust Morse Code speed    �

� F10 - Help with TNC Commands        Alt-K  - Exit KISS Mode             �

� Alt-F1 - Display Online Manual      Alt-L  - Continue Find in Flashback �

� Alt-F3 - Call BBS for Mail Fwding   Alt-M  - Personal Message System    �

� Alt-F5 - Send RAW Binary file       Alt-P  - Turn Print log ON          �

� Alt-F6 - Recv RAW Binary file       Alt-Q  - Turn QUIET Mode ON         �

� Alt-F7 - Send File  to  BayCom      Alt-R  - Rename/Move data file      �

� Alt-F8 - Recv File from BayCom      Alt-S  - Script processing          �

� Scroll Lock  - Pause                Alt-T  - Set TNC Time and date      �

� Page Up/Down - Flashback            Alt-V  - Verify Connected callsign  �

� - or        - Flashback            Alt-W  - Write Flashback to disk    �

� Shift - or  - Recall Type Ahead    Alt-X  - Exit this program          �

� Shift # or # - Change Comms Window  Alt-Y  - Yawn (half second delay)   �

� Shift <Fn>   - Change Comms Window  Alt-Z  - Configuration              �

������������Enter desired function, or Esc to leave Help Screen������������

Description.

This Help Window is always available at the touch of the <F1> key.

You may select any function by pressing the key shown in the Help

Window - it is not necessary to remove the Window first. If you do not

want to use any of those functions at this time, press <Esc> to remove

the Help Window and return to the previous display.

In many cases this Help display will show slightly different

information.  For example in the above example, we see:

F2  - Turn Log File ON

This suggests the Log File is currently OFF and if you press this <F2>

key, paKet will open the Log File and commence Logging to disk.

But if that Log File was currently ON at the time you call up the Help

Window, it would display:

F2  - Turn Log File OFF

telling you paKet will close the Log File if you press <F2>.

Page 92

Message Window

������������������������������������������������������������������������ͻ

�                            Log File closed                             �

�                                                                        �

�������������������������������������������������������������������������1/4

Description.

This is a small window that will appear from time to time, mostly to

inform you of some condition or problem. Some messages will disappear

after a predefined period, others which are considered more serious will

wait for you to press the <Enter> key before they disappear.

Full details of all messages, together with an explanation of each are

provided in the Messages Section of this Manual.

Some of the routine messages may be suppressed entirely by specifying a

higher "Message Level" in the Configuration.  But, be warned, that is

for experienced users only!

In the Miscellaneous options Configuration Window you may also choose

the message duration.  That is the time in seconds you would like these

messages to stay on the screen.   Sometimes you might like them to stay

on the screen a little longer to give you time to read them, but

whatever the duration, you can always press <Esc> while this Window is

displayed and paKet will immediately clear the Message Window from the

screen.

Page 93

Status Window

VK2DHU �� Windows ��������� paKet 6.1 ��������� Log File: AUTO ��������

2045    <-2->     REMOTE M 07:37:52   960 S  COM2: RTS CTS DSR DCD DTR

Description:

This window is optional (but recommended) and displays some of paKet's

operational settings on the top two lines of your screen.

The Top Line

On the top line, along with the fixed headings, you have the current

version number of the paKet program in the centre.  If you are using a

registered copy of the program your callsign will appear at the left

(VK2DHU shown in the example).

On the right, the status of the Log File is displayed.  That will show:

AUTO       - AUTO is displayed if the Log File is currently

closed and the AUTO configuration option is set to Y.

In this case, paKet will open the Log File when a

new connection is established, and will close it

again when there is no longer anyone connected.

OFF        - OFF is displayed if the Log File is currently closed

and the AUTO configuration option is set to N.

(filename) - When the Log File is open, paKet will display here

the file name of the current Log File.

Buffer

Most status information appears on the second line.  At the left is the

System Input Buffer counter (2045 shown in the example).

All data received from the TNC is stored in this System Input Buffer

temporarily while waiting for paKet to come and see where it belongs.

If we are conducting multiple sessions, with a separate Communications

Window used for each of the stations we are currently connected to,

this incoming data will have to be distributed to the appropriate

Communications Window.  At regular intervals in the processing, paKet

will perform this distribution.

The Status Window shows the number of characters in the buffer that

have yet to be distributed.

The count may build up if data is being received faster than your

system can process it, or if it cannot yet be distributed (eg: because

one of the Window input buffers has already filled).

Page 94

If you have started the System Editor <Alt-E> or taken the DOS Gateway

<F9>, paKet no longer has control of your system and so cannot perform

this data distribution. However, the Interrupt Handler is still

operational so anything received from the TNC during this time will be

captured and stored in the System Input Buffer.  So you will not lose

any data. When you return to paKet you will see the System Input Buffer

drain quickly into the Window input buffer(s) and data will flood into

the Communications Window as paKet catches up. Just keep in mind that

if you take TOO long and the System Input Buffer fills (it is 32KB),

any further data would then be lost.

If the number of held characters reaches 25000, the counter will change

colour to warn you it is filling up and an intermittent audible alarm

will sound (if Bells are enabled).

Windows

<-2->   This item indicates there has been some activity on another

Communications Window. In the example shown, there has been

some activity in Window 2 and you should switch to that window

(with <Shift-Right-Arrow> or <Shift-F2>) to see that data.

This field will reflect the number of available windows. The

example shown indicates three Communications Windows have been

configured but if you had configured six Communications

Windows, this item would have appeared as: "<-2---->".

Indicators

To the right of the "Windows" field is an area that paKet uses to

display various indicators, confirming some particular operational

status.   In the example above, the "REMOTE" indicator is shown.

This indicator field could contain:

DUMP     - When you press <Alt-W> to dump all or part of the

Flashback buffer to disk, this indicator is shown.

During this time you might be moving the Highlight bar up

and down the Flashback buffer, looking for the data you

want to capture.  This might take a while so, until you

complete this operation, you are reminded of paKet's

status - just in case you forget!

INIT    -  During startup, paKet will be sending any startup

commands, and processing any data that the TNC is sending

at this time (sometimes there is a build up of data in

the TNC buffers and it might take a while to empty them).

While all this is happening, you should not be using the

system because at that stage it is not completely set up.

So be patient while "INIT" is displayed, and wait for the

indicator to go out before you begin packet operations.

Page 95

BBS      - If you are connected to your BBS this indicator will

appear.

There are times, especially when performing Mail

Forwarding, that the processing is a little different. So

I thought it might be a good idea to show this indicator.

MAIL     - You might not see this one too often, but it will be

displayed when our system is performing Mail Forwarding

with some station other than your regular BBS.

paKet will not initiate a connection to any other station

for Mail Forwarding - that will be done only to your

regular BBS.   But it is possible some other station, say

some other BBS, could call us and trigger our Mail

Forwarding logic.

paKet will observe the standard RLI/MBL conventions for

Mail Forwarding, so if another station sends the

appropriate codes, paKet will respond accordingly.  In

that case, this MAIL indicator will be displayed to let

you know what is going on.

PMS      - This indicator will appear while you are using your PMS.

When you are accessing the PMS with the Sysop PMS Menu,

paKet will hold any data coming from the TNC. Otherwise

that incoming data will interfere with what you are doing

in the PMS.  This TNC data will be held until you exit

the PMS Menu, so the indicator serves to remind you to

return to normal communications when you have finished

with the PMS.

REMOTE   - This appears when the system is in REMOTE Mode.

SCRIPT   - This indicator is displayed while a Script file is

running.

BREAK    - If you have configured your "Break" code as <Ctrl-C>

there will be a one second delay before and after the

three <Ctrl-C>s are sent. During this couple of seconds

this BREAK indicator is displayed.

(msg)    - If there is one or more unread messages addressed to you

in your PMS, this indicator will appear to alert you.

When you have read all your messages this indicator will

disappear.

Page 96

Beside these indicators, the following single letter indicators could

also appear (M is shown in the example above):

A        - Alerts On - Standard Alerts Mode.

This indicates that paKet is scanning incoming data for

strings that match those specified in the Alerts

Configuration Window. If a match is detected (case is

unimportant - both upper and lower case will be matched)

paKet will sound the standard Bell tone as Configured for

this Communications Window.

M        - Alerts On - Morse Code Mode

The Morse Code option operates in a similar mode to the

Standard Alerts Mode mentioned above, but in this case,

when one of the Alert strings is detected, paKet will

generate a unique Morse Code sequence to identify the

particular string that was detected.

(The <Alt-A> key switches Alerts Mode between ON, Morse, and OFF)

P       - This will appear while the Print Log is active. (refer

<Alt-P> key).

Time

In the centre of the Status Window is the computer's current time. This

will probably be the TNC's time as well because paKet can automatically

initialise the TNC with the computer system's time when the program

starts or whenever you press <Alt-T>.

File Send/Receive

To the right of the time display, paKet will show the status of any

TEXT file operations while they are in progress.

In the example above we see "960 S".   This is indicating we are

currently sending a text file and so far we have sent 960 bytes of that

file.  We might have pressed <F5> to send the file, or if we are in

REMOTE Mode the other station could have triggered this with the REMOTE

"D" command.

If that part of the Status Window were showing say, "11347 R", that

would be telling us we are receiving a text file and at this stage we

have received 11,347 bytes.   We might have pressed <F6> to open a file

for capturing incoming data, or if we are in REMOTE Mode, the other

station could have triggered this with the REMOTE "U" command.

COM Port

This indicates the Computer Serial Port currently used by paKet.

Page 97

Handshaking

RTS   This area of the Status Window is used for Handshaking details.

etc

If you have specified Hardware Handshaking, the status of

significant RS-232 lines is displayed - if the line is high, the

indicator is shown; if the line goes low, that indicator

disappears.  For example, if the CTS (Clear to Send) "light" goes

out you will know the TNC has asked paKet to stop sending for a

while.

You will see "Software Handshaking" instead of these indicators,

if you have selected that mode. However, this will change to

"XOFF" when the TNC sends that code to ask paKet to stop sending

for a while. When the TNC sends the <XON> code, the Status Window

reverts to its original "Software Handshaking" display.

This Status Window is optional.  You could adjust your Communications

Windows Configuration so this Status Window does not appear.  However,

I expect such occasions would be quite rare.

Page 98

TNC Help Window

��COMMAND�����ARG������DEFAULT�����������DESCRIPTION���������������������ͻ

�MTo          'text'    NONE    Select stations for monitoring            �

�MWeight 'n'  5-15      10      Change Morse dot-space ratio.             �

�MYAlias      call      blank   Alternate station identifier.             �

�MYALTcal     'aaaa'    (empty) Alternate SELCAL for AMTOR                �

�MYcall       call      PK-232  Your station callsign.                    �

�MYIdent      'aaaa'    (empty) New 7 character SELCAL for AMTOR          �

�MYSelcal     'aaaa'    (empty) Specify your SELCAL for AMTOR             �

�NAVMsg       'text'    All     Specify which NAVTEX messages to monitor  �

�NAVStn       'text'    ALL     Specify which NAVTEX stations to monitor. �

�NAvtex       Immediate Cmd     Switch the PK-232 into NAVTEX Mode        �

�NEwmode      ON|OFF    ON      Sets mode on disconnect                   �

�NOmode       ON|OFF    OFF     Allow/disallow explicit mode change only. �

�NUcr         ON|OFF    OFF     Select send <NULL>s ($00) after a <CR>.   �

�NULf         ON|OFF    OFF     Select send <NULL>s ($00) after a <LF>.   �

�NULLs 'n'    0-30      0       Number of nulls to send.                  �

�Nums         Immediate Cmd     Force the PK-232 into FIGS case           �

�Opmode       Immediate Cmd     Display PK-232's current operating mode   �

�PAcket       Immediate Cmd     Switch the PK-232 to PACKET mode          �

�PACLen 'n'   0-255     128     Maximum number of Bytes in a data frame.  �

�PACTime E|A  0-250     A10     Transparent Mode packet timeout           �

��Command Reference for the PK-232 (firmware to 10/89) - (PK-232.HLP v1.3)1/4

Description.

The TNC Help Window shows the available commands for your TNC.

It will display each command on a single line with the valid

abbreviation in UPPER CASE, the valid arguments, the usual default, and

a brief description of the command.

For example, there is a "NEWMODE" command shown in the example above.

NE is the valid abbreviation; valid arguments are ON and OFF and the

normal default is ON.  This command "Sets the mode on disconnect"

(among other things).

If you move the highlight bar to any of those commands and press

<Enter>, paKet will display Detailed Help for that TNC command.

There are several ways to move the highlight bar:

1. use the arrow keys to move the cursor up or down - the window

will scroll if necessary; or

2. use the <PageUp>, <PageDown>, <Home>, or <End> keys to move

around the available items more quickly; or

3. use paKet's hot-key typing facility and type the command (or part

of the command) you are interested in, either before or after

selecting this window with the <F10> key.  paKet will try to

match what you have typed with the available commands and will

move the cursor to the first matching item. You do not have to

type the full command.

Page 99

Once the desired item is highlighted, press <Enter> to get the Detailed

Help description of that command, such as:

��COMMAND�����ARG������DEFAULT�����������DESCRIPTION���������������������ͻ

�NEwmode      ON|OFF    ON      Sets mode on disconnect                   �

�Modes:      All                                                          �

�                                                                         �

�Parameters:                                                              �

�     ON  - Switch to data transfer mode with CONNECT command;            �

�           Return to Command mode on disconnect.                         �

�     OFF - Switch to data transfer mode at time of connection;           �

�           Do not return to Command mode on disconnection.               �

�                                                                         �

�If ON the PK-232 will switch to whatever CONMODE is set when the         �

�connection is started without waiting for the connection to be           �

�established. If the connection is terminated, or if the request fails,   �

�the TNC will revert to Command mode.                                     �

�                                                                         �

�If OFF, the PK-232 will remain in Command Mode until the connection is   �

�established. Then it will switch to the mode specified in CONMODE. The   �

�PK-232 remains in that mode when the connection is terminated.           �

�                                                                         �

�This feature is provided mainly for BBS or host systems.                 �

�                                                                         �

��Command Reference for the PK-232 (firmware to 10/89) - (PK-232.HLP v1.3)1/4

The solid vertical bar that appears in the right hand margin is a

scroll bar.  That scroll bar indicates where you are in this section

and also indicates how large this section is.

In the example shown the bar occupies the full right hand margin,

telling you you can see the top AND the bottom of this section; in

other words you are looking at the whole thing!

If that bar occupied say, half the length of this window, you know you

are looking at only half the available information in this section and

that you can scroll up and down to view the rest of it.  As you scroll,

that vertical bar will slide up and down, so you can see where you are

in this section.

The information shown in this Help Window was gathered from TNC manuals

but you will probably be aware that there can be variations between

TNCs and even between different releases of firmware for the same TNC!

So while this feature may prove helpful in most cases, you should refer

to your TNC manual if in doubt or if it does not appear to be working

the way this Help window suggests.

As there are some differences in the commands of various TNCs it will

be necessary to select the appropriate HLP file for your TNC. So check

the supplied HLP files and make sure you have specified the appropriate

file name in "Configuration Windows - Directories and Files".

Page 100

There are several Help files included with the current version of paKet

and more may be added later, usually supplied by some generous paKet

users who might have some other TNC, or perhaps have the time to update

one of the existing Help Files.

If there does not appear to be a HLP file for your TNC, try the

TNC2.HLP file as it covers the basic TAPR-compatible commands that most

TNCs use. If that does not appear to suit, let me know (via Packet to

VK2DHU @ VK2ATM) in case another TNC Help file has been developed since

your copy of paKet was distributed.

Otherwise, I would appreciate it if you could let me know the details,

or (preferably) if you could edit an existing HLP file to suit your TNC

and let me have a copy of the new file so I can arrange to have it

distributed to all users of that TNC.

Page 101

Type Ahead Window

���� Single Line ���������Type Ahead buffer����������� Word Wrap ON ���

echo off

Description.

This window is optional (but recommended) and will appear at the bottom

of your screen. If you choose to have a Type Ahead Window, you may

configure the number of lines to use. Refer to the section:

"Configuration Windows - Communications Windows" for details. This

window is always 80 columns wide.

When enabled, all typed characters are displayed in this window but are

not sent to the TNC until the <Enter> key is pressed.  This allows you

to start typing before the other station has finished, and is sometimes

convenient where you can start answering questions, etc and have the

answers ready to send when your turn comes.  And of course you can

correct any typing mistakes before they go to air.

If the Single Line option is ON the data will be sent to the TNC as each

line is completed.  You don't have to press the <Enter> key in that case.

The capacity of the Type Ahead buffer is limited to the number of lines

displayed. So if you have configured 2 lines for the Type Ahead Window,

the maximum buffer capacity is 160 characters.

When using this Type Ahead facility, Keyboard Macros (<Alt-1> to

<Alt-0>) are sent and most <Alt- > functions are executed without

affecting the partial line in the Type-Ahead.

The following keys are effective when used in the Type Ahead buffer:

<Enter>            - send the contents of the buffer to the TNC

(the entire Type Ahead buffer will be sent

regardless of the cursor position);

<Backspace>        - remove the character before the cursor;

<Delete>           - remove the character under the cursor;

<Left-Arrow>       - move the cursor left one character;

<Right-Arrow>      - move the cursor right one character;

<Ctrl-Left-Arrow>  - move the cursor left one word;

<Ctrl-Right-Arrow> - move the cursor right one word;

<Home>             - move the cursor to the start of the buffer;

<End>              - move the cursor to the end of the buffer;

<Ins>              - toggle Insert mode.

When in Insert Mode the cursor will appear as a

Block character and when in overwrite mode it

will appear as an underscore;

<Tab>              - will be expanded to spaces;

Tab "stops" are assumed to be every 8 positions.

<Ctrl-Del>         - Clear the contents of the Type Ahead buffer

without sending any data to the TNC.

<Up-Arrow>         - Recall a line from the Type Ahead "history"

<Down-Arrow> or      (This will clear anything that was previously

<Shift-Up-Arrow>     in the buffer);

<Shift-Down-Arrow>   Refer to the discussion on this facility below.

Page 102

Type Ahead Recall

Each line sent from the Type Ahead buffer is saved in a Recall buffer.

Lines may be recalled one at a time from this Recall buffer with the

arrow keys. This facility operates in a similar manner to the DOSKEY or

CED utilities so, if you are familiar with those utilities, you know

what to expect here.

In paKet, the <Up-Arrow> and <Down-Arrow> keys may be used for this

Recall function but they may also be used to scroll the Flashback

buffer.  It is a matter of personal preference really.  There is a

Configuration option in the Miscellaneous options that determines how

you want to use these arrow keys.

If you choose to use them for scrolling the Flashback buffer, you can

still do the Type Ahead Recall by pressing <Shift-Up-Arrow> or

<Shift-Down-Arrow>.  In other words, hold down either <Shift> key while

pressing the arrow keys.

A recalled line is placed into the Type Ahead buffer as if you had

typed it again, ready for editing and/or resending.

This could be used so "improved" versions of previously sent text can

be sent after some editing; or the same text may be sent again. I have

often typed a long line of text and sent it only to find the TNC was in

Command Mode and it responds with "What?" or "Eh?" or something equally

intelligent. So, after restoring the TNC to Converse Mode, this Recall

facility allows me to send that line of text again, this time to the

other station, without retyping it all!

Similarly, if I enter a command to the TNC but find it was not in

Command Mode at all but in Converse Mode, my "command" will go to air

for all to see.  No problem: I just press <Alt-B> to reset the TNC to

Command mode, then <Up-Arrow> (or <Shift-Up-Arrow>) and <Enter> to

reissue the command.

The Type Ahead buffers for each Communications Window are preserved

between switching windows, but the Recall buffer is common.  You can

send the same line to many connectees in quick succession!

Text formatting

The <F4> key offers you some options in how your text is handled in the

Type Ahead Window.

Press the <F4> key to toggle Word Wrap, Justified and Single Line

Modes on and off.  There are five combinations here and as you press

the <F4> key, the active modes are displayed in the Type Ahead Window

Header Line.  For example in the example shown above, we have Single

Line and Word Wrap options active.

These options are discussed below.

Page 103

Word Wrap:

paKet has a Word Wrap facility that operates on data entered in the

Type Ahead window. If a word does not entirely fit on the current line,

the whole word will be moved to the next line.

You might want to turn it off (with the <F4> key) where you are typing

a long message such as a CTEXT message and you don't want paKet to

interfere when you reach the end of the line. However I expect most

paKet users will have Word Wrap or Justified ON all the time. It is so

much nicer to be on the receiving end of Word Wrapped text than the

broken words we sometimes get from lesser systems.

Justified

Justified text  is text  which is  aligned with  both the  left and the

right margins as shown in this  paragraph.   The left margin is usually

the left side of the screen,  but the right margin may be configured to

suit yourself.  If you choose the Justified option, we assume Word Wrap

is on too  and as a word  is wrapped to  the next line,  paKet will add

some spaces to  the existing line to  pad it out so the  last character

on that line finishes exactly at the right margin position.

Many advanced word processors using a graphic display can perform this

justification by making very slight adjustments to the spacing between

letters and words so the additional spacing is hardly noticeable.

Unfortunately we can't do this with text mode displays or printouts.

We are stuck with the full "space" character as you see in the previous

paragraph.

Single Line

If Word Wrap or Justified is ON, you also have a Single Line option.

Without Single Line, the data you type will be held in the Type Ahead

buffer until you press the <Enter> key, at which time the buffer will

be transferred to the Communications Window and the data sent to the

TNC.  If Single Line option is ON, the data will still be entered into

the Type Ahead buffer, but when the line is filled (ie when the last

word is wrapped to the next line) the full line will be sent to the TNC.

Why have a Single Line option?  Well, sometimes you might be typing a

lengthy reply to the other station, and with Word Wrap (as in a Word

Processor) you do not need to concern yourself with the end of a line -

you just keep typing. In that case, the buffer could fill, or at best,

the other station is waiting, waiting, ... until eventually you press

<Enter> to send all that text at once!  With the Single Line option ON,

you get the benefits of the Type Ahead Window, with its added text

handling features and the ability to edit or delete some of your text,

but without the disadvantages of filling the Type Ahead buffer or

making the other station wait for the entire message.

Anyway, try it. I was not so excited about it when a paKet user

suggested it, but now I like it and have it ON all the time!

Page 104

PART III - SPECIAL KEY CODES

Overview

Special Key Codes are expressions of keystroke-combinations which, if

required, need to be specially coded in the various "Command Texts"

such as:

Keyboard Macros,

Script commands:

Begin-Auto or End-Auto commands.

An example of a Special Key Code is the <Ctrl-C> combination. When you

are using the keyboard this key-combination can be entered into the

system by typing the letter C while holding the Ctrl key. Trying to

write such a combination into one of the Command Texts requires special

syntax, which is described in this section.

Types of Special Key Codes include:

Ctrl-key combinations such as <Ctrl-V>

Alt-key combinations such as <Alt-B>

Function-keys such as <F2>

Many commonly used command keys such as <Enter>

Macro Substitutions such as $D

In this section, in fact in the entire Manual, I use the Less-Than and

Greater-Than ('< >') characters as brackets to to better identify

special key names.  I call them "Pointer Brackets".

Ctrl-key combinations.

Any of the <Ctrl- > key combinations from <Ctrl-A> to <Ctrl-Z> may be

entered into a Command Text.

The syntax is to enter two characters in the Command Text, the caret

('^') followed by the desired Control Letter. For example:

^H for <Ctrl-H> (which is the ASCII code for Backspace)

^M for <Ctrl-M> (which is the ASCII code for Enter)

So a Keyboard Macro which contains the following 5-character string:

^CD^M

would result in 3 bytes being sent to the TNC. The first 2 characters

of the Macro would be combined into a single <Ctrl-C> character; the

letter 'D' comes next; then the last 2 characters in the macro would be

combined into a single <Ctrl-M> character (same as Enter).

If you want to send a caret ('^') to the TNC, enter 2 of them in the

Command Text. So, for example this Keyboard Macro:

ABC^^XYZ

would result in the following text being sent to the TNC:

ABC^XYZ

Page 105

Alt-key combinations.

Any of the <Alt- > key combinations may be coded into a Command Text

too.

The syntax is similar to that for the <Ctrl- > key combination but here

we use the tilde ('~') as the prefix. For example:

~B for <Alt-B> (which is paKet's BREAK command)

~6 for <Alt-6> (which is Keyboard Macro 6)

So a Keyboard Macro which contains the following 4 character string:

~P~H

would send our MHEARD list to the printer. Assuming the Print Log

function is currently OFF, the first two characters (~P) would be

processed as <Alt-P> to turn ON the Print Log. The next two characters

(~H) are processed as <Alt-H> which sends the MHEARD command to our

TNC.

If you want to send a tilde ('~') to the TNC, enter 2 of them in the

Command Text. So this Command Text:

ABC~~XYZ

would result in the following text being sent to the TNC:

ABC~XYZ

In a keyboard macro you may also perform a Script, which is usually

initiated via the <Alt-S> key.  In the keyboard macro, the script file

name must be specified.  For example to run the Script named

GETMAIL.SCP you would specify the following in your keyboard macro:

<~S GETMAIL>

The space after the "~S" is optional but I think it makes it easier to

read if you put the space in there.  If you have assigned that macro as

the first keyboard macro, you simply press <Alt-1> to perform the

GETMAIL Script.

If you want to send some additional parameters to be processed with

that Script, you enter them inside the Pointer Brackets and separate

them with commas.  For example:

<~S GETMAIL,XY4BBS,PAKET,AMSAT>

Then when that Script is being processed, the Script parameters may be

accessed as follows:

%0 contains GETMAIL (the name of the Script)

%1          XY4BBS

%2          PAKET

%3          AMSAT

Scripts are discussed fully in that section of the Manual.

Page 106

Function Keys.

Any of the Function keys may be specified in a Command Text.

The syntax is to specify the letter 'F' followed by the number of the

Function Key and surround that with Pointer Brackets. For example:

<F2> for Function Key 2 (which paKet uses to toggle the Log File)

<F10> for Function Key 10 (display TNC Help File)

You can also specify an <Alt-Fkey> combination as follows:

~<F8> for <Alt-F8> (for Receiving a Binary File from BayCom)

So a Keyboard Macro which contains the following string:

YD PROG.EXE^M<F8>

would send the string "YD PROG.EXE" to the TNC, followed by a Carriage

Return (the <Ctrl-M>). Then the "<F8>" causes paKet to behave as if you

pressed the <F8> key - to receive a Binary File.

Other commonly used command keys.

The following keys may be coded for use in Command Texts and are

written as spelt here, surrounded by Pointer Brackets:

UP                 HOME              ENTER

DOWN               END               ESC

LEFT               PGUP

RIGHT              PGDN

Upper or lower case (or any combination) is OK.

For example, instead of using the ^M combination as demonstrated above

for a Carriage Return, we could specify <Enter>.  The two codes (^M and

<Enter>) do the same thing - it is a matter of preference. I think

"<Enter>" is more descriptive and should be preferred unless you need

to reduce the number of characters on the line.

So, the example given in Ctrl-key combinations above (^CD^M) could be

written as:

~BD<Enter>

The <Alt-B> is a better method for returning the TNC to command mode

than <Ctrl-C> although for many TNCs it would achieve the same result;

the 'D' is the Disconnect command;  and "<Enter>" generates a Carriage

Return.

Page 107

Macro Substitutions.

A Macro Substitution is a paKet facility where specially coded key

sequences in Command Texts are converted to text as shown in the

following table:

$D  Today's Date (MMM-DD)

$N  Your name as configured

$O  Your own callsign as configured

$S  Stream Id of currently selected stream

$T  Current Time (HH:MM)

$U  User's callsign (in the current stream)

$#  Number of active sessions (connected)

$^  Number of available sessions as configured

$%  List of callsigns currently connected

For example say I had the following text recorded as a keyboard macro:

$O disconnecting from $U on $D at $T.^MBye for now...$N^M~BD^M

When I select that macro, the following text would be sent to the TNC:

VK2DHU disconnecting from VK2BZC on JAN-01 at 21:45.

Bye for now...Tony

The Macro Substitution codes in this keyboard macro are replaced by the

text currently assigned to each of those codes.

For example - the $O and $N codes are taken from the paKet

Configuration;

- $D and $T are taken from the computer's clock at the

moment the macro is processed;

- $U was determined when (s)he connected earlier.

(In the example I have assumed my callsign is VK2DHU and

that I was connected to VK2BZC at the time).

The ~B (i.e. <Alt-B>) puts the TNC into command mode and then that is

followed by the Disconnect command.

You will note I used the "^M" combinations here instead of the more

descriptive "<Enter>" to help ensure I got it all on the one macro

line.

Page 108

PART IV - KEYBOARD COMMANDS

It is intended that the operation of the paKet program should at all

times be intuitive and as easy to use as possible. However, you still

need to know about some of the special keys, details of which are given

below.

F1               Display Help Screen.

Display a help screen showing most of paKet's special keys and

their functions.

You may select any function by pressing the key shown on the Help

Screen - it is not necessary to remove the Help Window first. If

you do not want to use any of those functions at this time, press

<Esc> to return to normal communications.

This Help Screen is intended as a memory jogger. Full details are

included here in this Manual which may be viewed by pressing

<Alt-F1>.

Page 109

F2               Turn Log File on/off.

Pressing this key will switch the Log File on or off.

The Log File has three possible states:

- ON    When the log file is ON, the Status Window will display

the name of the file currently used as a log file and

all Communications Window activity will be written to

that log file.

If the Log File is not ON, its state will be either:

- OFF   The disk log facility is not currently active so

nothing is written to any log file.

or

- AUTO  (This will only occur if you set the AUTO LOG option ON).

In this state the disk log facility is OFF, but paKet

will automatically activate the disk log, using the

default file name, when a connection is established, and

will automatically close it upon disconnection.

The effect of this is the same as if you press <F2> on

connection and then press <F2> again on disconnection.

So, if the Log File is currently OFF (or AUTO), pressing <F2> will

turn it ON.

A Directory window will appear, together with a suggested file

name. The default file name will be today's date in the form

MMMDD.LOG, for example: NOV23.LOG but you may use any name you

like. Using the default name means you can easily keep a separate

log for each day's activities.

If the selected file already exists on the disk, it will be

preserved and incoming data will be appended to that file.

If, when you press <F2>, the Log File is currently ON, it will be

closed and further logging activity will cease. The Log status

will return to either OFF or AUTO, depending on the AUTO LOG

option.

Some operators prefer to use the log file for all captured data

including ASCII file transfers, and use their favourite editor to

extract desired messages, files, etc from the log file later.

Page 110

F3               Turn REMOTE Mode on/off

REMOTE Mode is discussed more fully in its own section in this

Manual, but briefly it is a facility that allows another (remote)

user to operate our paKet system's PMS or File Transfer systems

even if our station is unattended.

There are three ways to get this system into REMOTE Mode:

1. You can configure paKet to automatically switch into REMOTE

Mode when another station connects;

2. The other station could send the REMOTE Trigger code.

This code would normally be the default <Ctrl-]>, but is

configurable via the "Configuration - REMOTE Options"

Window;

3. You can activate our REMOTE Mode by pressing <F3>.

When paKet enters REMOTE Mode it will issue the REMOTE Menu to the

other station.

If this system is already in REMOTE Mode, there are three ways to

switch that Mode OFF:

1. The other station could enter B (Bye) to log off REMOTE Mode

and Disconnect the TNC session; or he may just disconnect.

2. The other station could enter T (Talk) to switch off REMOTE

Mode but retain the connection, continuing in normal

communications.

3. You could press <F3> to switch off REMOTE Mode returning to

normal communications (same effect as if the other station

sent "T").

Page 111

F4               Toggle Word Wrap, Justified and Single Line formats

Word Wrap:

paKet has a Word Wrap facility that operates on data entered in

the Type Ahead window. If a word does not entirely fit on the

current line, the whole word will be moved to the next line.

Word Wrapped text is much tidier for the receiver to read and I

expect this option to be the norm. Perhaps you might want to

turn it off where you are typing a long message such as a CTEXT

message and you don't want paKet to interfere when you reach

the end of the line.

The status of the Word Wrap toggle is shown to the right of the

Type Ahead header line.

Justified:

Justified text  is that  which contains  sufficient  additional

spaces to ensure the last word on each line ends exactly on the

right  hand  margin  column.   The effect  is  an aligned right

margin such  as  you  see  in  this  paragraph.   The technique

works much better with a graphic display and proportional fonts

where the  spacing can  be varied  in very small increments all

across  the line,  but with the text  display used  by paKet it

must be accomplished with space  characters inserted at various

points in the line.

paKet supports justified text in the Type Ahead Buffer so if

you like this format press <F4> until you see "Justified"

displayed in the Type Ahead header line.  This implies Word

Wrap too.

Single Line:

If Word Wrap or Justified is ON, you also have a Single Line

option.

With the Single line option OFF, the data you type will be held

in the Type Ahead buffer until you press the <Enter> key, at

which time the contents of the buffer will be transferred to

the Communications window and the data sent to the TNC.

If Single Line option is ON, the data will still be entered

into the Type Ahead buffer, but when the line is filled (i.e.

when the last word is wrapped to the next line) that full line

will be sent to the TNC.

The <F4> key cycles through the Word Wrap, Justified and Single

Line options if the Type Ahead window is displayed.

My personal preference?   I like to use Single Line mode with Word

Wrap.

Page 112

F5               Send an ASCII text file

With this option you can send the contents of an ASCII text file

to the TNC.

An example of this might be where you want to send a lengthy

message to someone, or if you want to send the same message to

more than one station, you could prepare a disk file with the

desired text before you connect to that station. Then when you are

ready to enter the message, just press <F5> to send the file.

Of course this option is also used to transfer a data file or

source program file to another station, provided the file contains

only ASCII text. While it is POSSIBLE to transmit a binary file

using this command, it is prone to problems and NOT recommended!

You should use one of the Binary File Transfer methods to send a

file that contains non-ASCII data.

When you press <F5>, a Directory Window will appear.  Select the

desired file by moving the highlight bar to the desired file and

pressing <Enter>.  You can enter the filename manually if you

prefer, by selecting that option at the top of the Directory

Window.

While the file is being sent an 'S' will appear in the Status

Window to the right of the Time display, along with a counter

showing the number of bytes actually sent to the TNC.

When the entire file has been transmitted, paKet will optionally

add a <Ctrl-Z> character indicating end-of-file to the other

station. If you do not want this end-of-file code added, you can

disable the option via the Configuration - File Transfer options.

It should be noted that if you have configured more than one

Communications Window, paKet will keep the data separate and will

send only data from THIS file to the station connected on THIS

window.  You can change to another window and press <F5> again to

send ANOTHER file to the TNC from THAT window. There is no limit

to the number of files open at any one time, provided you have

specified enough FILES in your CONFIG.SYS (refer to your DOS

manual for details of the FILES parameter).

Page 113

F6               Receive ASCII text file

When you press <F6>, paKet will ask you for the name of the file

you want to open.

From then, all data received in this Communications Window will be

written to this file.

If the other station sends an end-of-file character, <Ctrl-Z>, the

file will be closed. Or you can close it yourself by pressing <F6>

again.

While the file is open, an 'R' will appear in the Status Window to

the right of the Time display, along with a counter showing the

number of bytes actually received into this file so far.

It should be noted that if you have configured more than one

Communications Window, paKet will keep the data separate and will

write only data for THIS window to this file.  You can change to

another window and press <F6> again to create ANOTHER file for

data received into THAT window. There is no limit to the number of

files open at any one time, provided you have specified enough

FILES in your CONFIG.SYS (refer to your DOS manual for details of

the FILES parameter).

Page 114

F7               Send a Binary File using either pP or YAPP Protocol.

To use this feature the other station must be able to handle

either YAPP protocol or pP (paKet Protocol).

Normally the standard YAPP protocol will be used; the program will

automatically select and use the advanced paKet-Protocol if

necessary.  You do not have to concern yourself with the use of

the more advanced protocol; paKet will take care of that itself.

If you want to send a Binary File to a BayCom or Digicom system,

refer to the <Alt-F7> key. And if you prefer to send a Binary File

to the other station with no protocol at all (you will have to

synchronise it manually), refer to the RAW Binary mode with the

<Alt-F5> key.

When you wish to send (or "upload") a file to the other station,

first of all arrange for that station to be ready to receive the

file.  You do this by sending a "Yapp Upload" command to the other

station.  For example, if you are connected to a BBS which is

waiting for a command you type:

"YU PROG.EXE"

(This is the BBS command to upload the PROG.EXE file using YAPP

protocol.)

If you are connected to another paKet station which is in REMOTE

Mode, you simply type:

"YU"

(the paKet program at the receiving end is smart enough to figure

out the name of the file!)

The other station will respond with something like:

"Ready to Receive with YAPP protocol"

Of course various other systems might require a slightly different

command and might respond with a different prompt, but you get the

idea.  If in doubt, ask the operator of the other station.

Now press <F7> and tell it the name of your file you want to send.

A Directory Window will pop up to help you select the file.

Then paKet will:

prepare the TNC for the Binary Transfer;

display the Binary File Transfer Window;

synchronise with the other station; and

begin the transfer.

If all goes well, you do nothing - just sit back and watch - and

the file will be sent. The program returns to normal

communications when the transfer is completed.

Page 115

Any problems with the file transfer could be:

1. a problem with the other station (not in Binary Receive mode;

not enough room on disk there; etc) in which case you need to

try again when that station is ready to receive the Binary

file transfer; or

2. there could be a problem in the handshaking between your

computer and your TNC. If you are using Hardware Handshaking

(you will see RTS CTS etc in the right hand side of the

Status Window), try using Software Handshaking as there could

be a problem in the cable you are using between the computer

and the TNC.

To change to Software Handshaking, refer to the

"Configuration Windows - Serial Port" section of this

manual.

Check both the TNC and paKet's Serial Port Configuration to

ensure the Handshaking options correspond:

If you have specified Software Handshaking to paKet, set

XFLOW ON in your TNC;

If you have specified Hardware Handshaking to paKet, set

XFLOW OFF in your TNC.

3. There is a mismatch in your settings for STREAMDBL.

paKet works best if STREAMDBL is ON in both the paKet

Configuration AND in the TNC, but the important thing is to

ensure they are BOTH set the same way!

If your TNC does not have a STREAMDBL option, set paKet's

option OFF.

Refer to the section, Configuration Windows - Multi User

options, for a full discussion on the STREAMDBL settings.

4. Some TNCs cannot handle a PACLEN of 256 (or 0 which, to the

TNC, is equivalent to 256).

Reduce PACLEN for binary operations to 252 in the

Configuration - File Transfer options.

5. Check Serial Port settings.  It is essential you have

specified 8 data bits and No Parity for the Serial Port

parameters.  They must be set the same way in the TNC too so

check your TNC settings with the appropriate commands,

usually AWLEN and PARITY.

Page 116

F8               Receive Binary file using either pP or YAPP protocol.

To use this feature the other station must be able to handle

either YAPP Protocol or pP (paKet Protocol).  Normally the

standard YAPP Protocol will be used; the advanced pP will be

activated by the program only when necessary.

If you want to receive a Binary File from a BayCom or Digicom

system, use the <Alt-F8> key. And if you prefer to receive a

Binary File from the other station with no protocol at all (you

will have to synchronise it manually), refer to the RAW Binary

mode with the <Alt-F6> key.

Receiving (or "Downloading") a Binary File is even simpler than

sending one.

To use this feature, prepare the other station for the file

transfer then press <F8>.  That is all!

For example, if you are logged on to a BBS system, or to a paKet

REMOTE system, you could type:

"YD PROG.EXE"    (The command to download a YAPP file)

and it would respond with something like:

"Ready to Send with YAPP protocol"

Then you press <F8> and watch as the file PROG.EXE (or whatever

you have specified) is transferred to your disk.

paKet will:

prepare the TNC for the Binary Transfer;

display the Binary File Transfer Window;

synchronise with the other station; and

begin the transfer.

Once again, I suggest you check with the operator of the other

station if this command does not appear to be working on that

system.

Page 117

F9               DOS Gateway

Pressing this key will provide access to DOS so you can run other

DOS jobs without leaving paKet. A Message Window will pop up

asking you for a DOS command.

There are two ways to perform a DOS command:

1.  You may enter the command into the message window beside

the ">" prompt.

If you do this, the command will be executed (if possible)

and you will be invited to "Press any key to continue"

before paKet resumes where it left off.  The "Press any key

to continue" message appears briefly over the DOS display

to allow you time to peruse the result of the job and to

read any messages etc that may have come from the DOS

command.

2.  Press <Enter> instead of entering a command, and the screen

will be cleared before control is passed over to DOS and

the familiar DOS Prompt.

When you have finished the DOS operations, type EXIT to

return to paKet.  Do NOT type "PAKET" again as the system

will attempt to load a second copy of paKet into memory!

Please keep in mind that paKet is still in memory at this stage so

you will have considerably less free memory in the Base 640K for

your other DOS programs. You may even find some programs will not

run due to insufficient memory.

If you are running paKet under a multi tasking system (for example:

DESQview, OS/2, Windows Enhanced Mode, Windows NT, etc) you might

find it better to start another DOS session instead of using this

Gateway to perform your DOS operations.  That way you will get a

full complement of memory and not be constrained by the memory

occupied by the paKet system.

If you are running plain vanilla DOS and find insufficient memory

to run your DOS job, the only remedy for this I am afraid, is to

exit the paKet program (with the <Alt-X> or <Alt-F4> key) so you

can release the memory occupied by the paKet program.

If using the Gateway and are "away" in DOS, paKet's Interrupt

Handler continues to operate so any input from the TNC during that

time will be captured and stored in paKet's 32KB System Input

Buffer.  Unless your DOS jobs take a looonnngg time, you should

not lose any input data.  Please note though, paKet is not a multi

tasking program and the captured data will not be processed until

you return to the program.

Page 118

F10              Display TNC Help File commands

This feature will load the TNC Help file and will display a window

showing the available commands for your TNC.

The TNC Help file that is displayed is the one specified in the

Configuration - Directories and Files Window. Several TNC Help

Files are supplied with paKet so be sure to specify the file best

suited to your TNC.

The TNC Help File Window will display a list of all the commands

for this TNC and this may be all you need to choose the desired

command. If so, press <Esc> to return to normal communications.

For more detail on any of these commands, highlight the desired

command and press <Enter>. When finished with this Detail display

press <Esc> to return to the main TNC Help file list. Then select

another item to display or press <Esc> to return to normal

communications.

Further details of this display are available in the section on

"TNC Help Window".

Page 119

Alt-F1           Display the Online Manual

Pressing this key will bring up paKet's Online Manual Window which

reveals the entire contents of this Manual.

The Manual is indexed and the index will appear for you to select

a topic. Move the highlight bar with the <Up-Arrow>, <Down-Arrow>,

<PageUp>, <PageDown>, <Home> or <End> keys to the desired topic,

then press <Enter> to select that topic.

Once a particular topic is selected the full text for that section

of the Manual will be displayed and you may scroll up or down as

desired.

When you have finished with that section, press <Esc> to return to

the Index to select another topic. If you have finished with the

Online Manual, press <Esc> to return to normal communications.

Page 120

Alt-F3           Call the BBS for Mail Forwarding

Mail Forwarding is discussed in the PMS section of this Manual.

Usually the Mail Forwarding with your chosen BBS is done at

regular times during the day or night and does not require your

manual intervention.  However, if you wish, you can force the

paKet system to call the BBS now and conduct a Forwarding session

to send and receive any mail that might be waiting.

To do this, just press <Alt-F3> and paKet will do the rest.

The program will initiate a Connect to the BBS using the Connect

Path (or Script file) specified in your PMS Configuration.  If the

BBS has any mail for you, that mail will be sent and stored in

paKet's PMS. And if you have any outgoing mail in there waiting to

be sent, it will be sent to the BBS during this session.

While connected to the BBS, whether for Mail Forwarding or for a

normal session, the symbol "BBS" will be displayed in the Status

Window.

When there is no more mail to be sent or received, the system will

disconnect from the BBS.

Page 121

Alt-F4           Exit the paKet program

This command returns you to the operating system. Any files that

were open will be closed automatically.

To guard against inadvertent exits, paKet will ask for your

confirmation that you really do want to exit.

If you have specified any End-Auto Commands (Refer the

Configuration section for details) they will be sent to the TNC

and if paKet finds an AUTOEND.SCP file in the current directory

that Script will be executed before paKet returns to the operating

system.

The <Alt-X> key achieves the same as <Alt-F4>.

Page 122

Alt-F5           Send a RAW Binary File

This option is similar to the other Binary File Transfer options

in that paKet will put the TNC into Transparent Mode and will

display the Binary File Transfer Window while the file is being

transferred.   The main difference here is that no protocol at all

will be used. If there is any synchronisation required with the

other station, you will have to do that manually.

This facility is provided to cover those situations where you want

to send a data file containing non-ASCII codes to another station

(or non-ASCII commands to the TNC). If the other station cannot

handle one of the Binary Transfer protocols supported by paKet and

you are unable to use the standard ASCII Text file transfer (<F5>)

because of the non-ASCII codes in the file, this might be your

last hope!

paKet will ask you for the name of the file you wish to send. A

Directory Window will pop up to help you select the file.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

begin the transfer.

Remember, with this RAW method, you will have to perform any

synchronisation required and to manually issue any commands to

start and to end the file transfer.  paKet does nothing to help -

this time it is your responsibility!

Page 123

Alt-F6           Receive a RAW Binary File

As with the <Alt-F5> command to Send a RAW Binary File, this

option is similar to the other Binary File Transfer options in

that paKet will put the TNC into Transparent Mode and will display

the Binary File Transfer Window while the file is being

transferred.   The main difference here is that no protocol at all

will be used. If there is any synchronisation required with the

other station, you will have to do that manually.

This facility is provided to cover those situations where you want

to transfer a data file containing non-ASCII codes from another

station which cannot handle one of the Binary Transfer protocols

supported by paKet.

paKet will ask you for the name of the file you wish to open to

hold the captured data.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

open the nominated file;

write any data received into that file.

You will have to close the file manually by pressing <Alt-F6>

again.

Remember, with this RAW method, you will have to perform any

synchronisation required and to manually issue any commands to

start and to end the file transfer.  paKet does nothing to help -

this time it is your responsibility!

Page 124

Alt-F7           Send a Binary File to a BayCom system

The BayCom system supports Binary Transfers via its //WPRG and

//RPRG commands but does not currently support any Binary File

Transfer PROTOCOLS. However, with just one reservation, paKet is

able to effect a reliable Binary File Transfer to a BayCom system.

This paKet facility is provided primarily for Binary Transfers to

a BayCom system although it should work into a Digicom system as

well.

When you press <Alt-F7> the Disk Directory window will appear for

you to select the file to send. Then paKet will ask for the name

(and path if that is required) this file is to have in the BayCom

system.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send BayCom's //WPRG command;

transfer the contents of the file;

send BayCom's <CR>//WPRG OFF<CR> command at end-of-file.

If all goes well, you do nothing - just sit back and watch - and

the file will be sent. The program returns to normal

communications when the transfer is completed.

As the BayCom system does not have a defined File Transfer

Protocol, the only way to indicate the end-of-file is with the

<CR>//WPRG OFF<CR> command. The trouble is, the BayCom system will

add this string of characters to the file BEFORE recognising it as

the command to close the file! So the file now on the BayCom

system will be 12 bytes longer than the one we sent. This is a

flaw in the BayCom system and there is nothing paKet can do about

it.

Fortunately it is not a big problem: I have found it usually does

not matter that the additional characters are there.  Executable

program files and archived (compressed) files seem to ignore the

additional characters.  And if required, the BayCom operator can

easily edit the additional bytes from the end of the file. Small

price to pay for the convenience of having Binary Transfer

capability!

Page 125

Alt-F8           Receive a Binary File from a BayCom system

The BayCom system supports Binary Transfers via its //WPRG and

//RPRG commands but does not currently support any Binary File

Transfer PROTOCOLS. However with some operator input, paKet is

able to effect a reliable Binary File Transfer from a BayCom

system.

This paKet facility is provided primarily for Binary Transfers

from a BayCom system although it should work from a Digicom system

as well.

The main difficulty in transferring a Binary File from a BayCom

system is the absence of any end-of-file indicator. The BayCom

system will send the Binary File in response to its //RPRG command

and will simply stop sending when it has reached its end-of-file.

paKet needs to know when to close the file. There are four ways

this can be done:

1. paKet will count each byte received and, provided it knows

the size of the file, it will close the file when that

number of bytes has been received.

2. If paKet receives BayCom's <CR>//WPRG OFF<CR> string it will

assume that is the end of the file. That string will NOT be

written to our file.

This means the Binary File being transferred cannot have

this string embedded in the file. If the file does happen to

contain this string, I suggest you ask the BayCom operator

to compress the file with LHA or similar and send the

compressed version of the file - this should change the file

so the string is no longer apparent.

If the file being received was previously transferred to the

BayCom system by some other user (or by us for that matter),

that file will probably have the <CR>//WPRG OFF<CR> string

on the end of the file.   paKet would recognise that and

close the file at the right time! (Refer to the comments on

sending a Binary File to a BayCom system, under <Alt-F7>).

The BayCom operator could send us this "command" when that

system has finished sending the file, although if the BayCom

operator is present we should know the file size and will

probably have closed the file by now.

3. You can press <Alt-F8> during the transfer to close the file

manually.

4. paKet will close the file if nothing has been received for a

period of 2 minutes.

Page 126

As the BayCom system will stop sending when it has finished

and will wait for our next command, this option might be

used if the BayCom system is unattended and you do not know

the filesize.

If the radio frequency is not busy, it should be rather

obvious when the other system has stopped sending so you

might close the file manually with <Alt-F8> (option 3 above)

before the 2 minutes expires.

Before attempting the transfer, you should get the details of

the file from the BayCom operator. You will need the filename

of course and if it is not in the BayCom system's current

directory you will need to know its path as well. If possible

get the file size in bytes too.

When you press <Alt-F8> paKet will ask you for the name of the

file in the BayCom system. This name will form part of the

//RPRG command paKet sends, so be sure to specify any path

details if required.   If you are transferring a file from a

Digicom system, it might be an unusual file name because the

Commodore 64 can cope with longer file names including embedded

blanks! You will have to specify the file name as it appears in

that Digicom system. Of course the file name you choose for this

file on our system will have to conform to the DOS requirements.

Then paKet will ask you for the file size in bytes. If you do

not know the file size, enter 0. This means the first of the

above options will not apply and you will probably have to

close the file manually or wait for the 2 minute timeout.

If you are transferring a file from a Digicom system, you will

probably not know the file size in bytes because the Commodore

64 system displays its file sizes in blocks rather than bytes.

In this case, enter -1 for the file size. This will tell paKet

to expect the Digicom responses (which are a little different

from BayCom's) as well as indicating the file is to be closed

manually.

Finally, paKet will ask you for the filename you wish to use

here. As you will most likely use the same name as that on the

BayCom system, paKet offers this name as a default. You can

change the details if you wish or simply press <Enter> to

accept the same name.

Then the transfer begins.  paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send the BayCom //RPRG command;

begin the transfer;

When the file is closed by one of the above methods, paKet

removes the Binary File Transfer window and returns to normal

communications.

Page 127

Alt-A            Turn Alerts on/off

This toggle enables or disables the Alerts facility which can

sound a Bell or a coded sound sequence (using Morse Code) if one

of the predefined strings is detected in the Communications Window.

Refer to the Configuration - Alerts window to see how to specify

the desired search strings.

<Alt-A> is a three-way toggle.  Press this key combination to

toggle from "No Alerts" to "Standard Alert tones" to "Morse Code

Alert tones" to "No Alerts"...etc.

No Alerts

If this is the current setting, paKet will ignore the Alerts

facility and will not even look for a matching Alert string.

Standard tones

When the Alerts facility is enabled for standard tones, an "A"

will appear in the Status Window to the left of the Time display.

If one of the Alert strings is detected, paKet will sound the bell

using the standard Bell tone (or audio frequency) you have

specified in the Configuration Window for Communications Window

Settings.

Morse Code tones

When the Alerts facility is enabled for the coded sounds, an "M"

will appear in the Status Window to the left of the Time display.

If this Morse option is selected, a unique set of tones will be

sent for each of the Alert strings you have defined.  The first

Alert specified in the Alerts Configuration Window will be

reported with the Morse Code signal for the letter 'A';  the

second Alert recorded in that Configuration Window will be

reported with the Morse letter 'B'; etc.

You can override those simple Morse signals with even more

sophisticated Morse sequences.  This is discussed in the

Configuration section where the Alerts Window is discussed.

Use the <Alt-J> key to adjust the Morse Code speed.

Page 128

Alt-B            Send a BREAK signal to TNC - return to Command Mode

This is the preferred method of returning your TNC to Command Mode.

Pressing <Alt-B> will send to the TNC either a BREAK signal, three

<Ctrl-C>s, or an <Esc> depending on the setting you have chosen in

your Serial Port configuration.

Most TNCs support the BREAK signal and will return to Command mode

when this signal is received.

If the BREAK is not supported, the TNC's "Command" character

(usually <Ctrl-C>) must be used.

On a few non-standard TNCs you must send the <Esc> code prior to

your command. For example the old, original TNC-1 works like this.

I have found the PacComm PacTOR Controller also uses the <Esc> code

for Command Mode!  So this option is provided in case your TNC works

this way.

But, regardless of the TNC used and regardless of the way you have

configured this option, you can consider the <Alt-B> function the

correct way to prepare your TNC for a Command.

Page 129

Alt-C            Connect to another station

This feature allows you to more easily make a connection to your

regular contacts, especially where there are long path details to

be entered.

Pressing <Alt-C> will bring up paKet's Connect Window which is

fully described in the section on the Connect Window.

To make a call, simply move the cursor to the desired item and

press <Enter>.

You can change an existing entry by moving the cursor to the

character to be changed, and insert the new data. Pressing the

<Insert> key allows you to toggle Insert Mode so you can choose

between Inserting new data or Overtyping existing data.

Use the <Tab> key (or the <Right-Arrow> key if you prefer) to move

from the Name field to the Callsign field.

To insert a new entry, move the highlight bar to the desired line

and press <Ctrl-Ins>. The entry on that line and on all following

lines will move down making room for your new entry.

To remove an existing entry, move the highlight bar to that entry

and press <Ctrl-Del>.

Nodes and Digipeaters

For those who wish to connect to stations outside normal range you

can specify digipeaters in the path along with the callsign.

For example:

VK2DHU via VK2DIG-1 VK2RPT-4

You might want to connect via a Node (where you must firstly

connect to the Node then, after you are connected, issue a second

connect command to ask the Node to establish contact with the

desired station). In that case you may specify both connect paths

in the Connect line separated by a semi-colon.  For example:

VK2NOD via VK2DIG-1 VK2RPT-4 ; VK2DHU via VK2XXX

When this format is used, paKet will generate a CONNECT command

using the path specified up to the ';' then it will generate a

second CONNECT command using the path specified after the ';'.  So

using the example above, it will generate the following commands:

CONNECT VK2NOD v VK2DIG-1 VK2RPT-4

CONNECT VK2DHU v VK2XXX

Page 130

Please note it will not wait nor perform any intelligent processing

of the first connect command.  It will ensure the TNC is in Command

Mode before the first "CONNECT" is sent and then it simply generates

and sends both these lines to the TNC.

You will need NEWMODE ON in your TNC for this technique to work

because we want the second line to be processed as data, not as a

command for your TNC!   (Have a look in your TNC Manual for more

information on the NEWMODE parameter.  And check paKet's Multi

User Configuration Window to ensure paKet's NEWMODE setting is

configured the same as your TNC's setting).

The first "CONNECT" is processed by the TNC as a CONNECT command

to connect to the Node.  With NEWMODE ON the TNC switches

immediately into Converse Mode, so our second line is treated as

data and is transmitted to the Node.

I think we need an example.  Let's use that same sample we used

above:

- the TNC would initiate a connect to the Node (which is VK2NOD

in our example) via two digipeaters;

- the second line is sent as data by our TNC to that Node;

- that data will be recognised by the Node as a Connect

command; so  the connection to VK2DHU via VK2XXX is then

processed by the Node!

Scripts

If that seems rather complicated or maybe with your TNC it doesn't

work too well, the last resort, and the most flexible of all, is

to use a Script to establish the connection.  You can specify a

Script file name in the Connect Window in place of the callsign

and path details.  Just record the Script file name in Pointer

Brackets.  As an example, say you had a Script file called

BBSCONN.SCP. You could specify:

<BBSCONN>

instead of the callsign and when that item is selected paKet

recognises that entry as a Script and will load and run that

Script file.

The Script can provide all the flexibility you need with multiple

connects and delays etc.  Refer to the Script section of this

Manual for details on how to create or change a Script file.

Page 131

Alt-D            Display a Disk Directory

Pressing <Alt-D> will bring up the Disk Directory Window which is

fully described in the Windows section of this Manual.

You may scroll up and down this window using the <Up-Arrow> or

<Down-Arrow> keys, the <PageUp> or <PageDown> keys, or the <Home>

or <End> keys. Or you can use paKet's hot-key typing by entering

part or all of the file name - paKet will highlight the first

filename that matches the characters you have typed so far. Often

all you need to type is one or two characters to move to the file

you want.

If you press the <Enter> key to select the highlighted entry, the

action taken depends on the type of line highlighted. If the

highlight bar is on:

(Enter file name manually)

The Message Window pops up to ask you for the desired file

name. When you enter the file name the contents of that file

are displayed as if you selected that file with the <Enter>

key (see below).

HINT: Press <Home> to jump quickly to this entry at the top

of the Disk Directory Window.

(Change to another drive)

The Message Window pops up to ask you which drive you wish

to change to. Then the directory display changes to show the

root directory for that drive.

When entering the new drive letter, the colon is optional.

For example, you could enter either:

A  or

a:

to select drive A:. Note upper or lower case is acceptable.

<Previous directory>

The directory display changes to show the contents of the

"Parent" directory, that is the directory above this one.

Hint:

Usually it will be quicker to type a "." to highlight this

entry instead of using the arrows to move the highlight

bar.

<DIR>

If there is an entry with "<DIR>" instead of the file size,

this represents a sub directory below this one. So pressing

<Enter> on this line, causes that subdirectory to be

displayed.

Page 132

(filename)

If you press <Enter> with any file highlighted, paKet will

display the contents of that file. The command used to

create the display is configurable and is defined in

Configuration Windows - Directories and Files.

I prefer something like the LIST program, but you could use

the DOS "TYPE" command if you wish.  You could even specify

your favourite Text Editor there if you prefer!  When the

display program has finished, press any key to return to the

Directory Window. If you don't want to  display any more

files, press <Esc> to return to normal communications.

While you are viewing the file, paKet's Interrupt Handler

continues to operate so any input from the TNC during that

time will be captured and stored in paKet's 32KB System

Input Buffer.

If you press the <Delete> key while a file is highlighted, paKet

will delete that file (after seeking your confirmation).   You may

also delete a sub directory entry, provided that sub directory is

empty.

If you press the <Alt-R> key while a file is highlighted, paKet

will ask you for the new filename then will Rename that file.  You

can specify a new directory with the new filename and paKet will

Move the file to the new directory.

Page 133

Alt-E            Edit a data file with your preferred System Editor

This feature allows you to edit a file using your favourite text

editor.

Details of the System Editor program are defined in Configuration

Windows - Directories and Files and may be any editor or word

processor you choose. Keep in mind however, that paKet with all

its buffers and data is still in memory, so you will probably not

be able to load and run one of the BIG Word Processing packages

using this feature.

When you press <Alt-E>, paKet will display the Disk Directory

Window so you can identify the file you wish to edit. Then it will

load the nominated editor program.

When you have finished the edit and exit from that program, you

will be returned to paKet where you left off.

While you are using the System Editor, paKet's Interrupt Handler

continues to operate so any input from the TNC during that time

will be captured and stored in paKet's 32KB System Input Buffer.

If you are running paKet from one of the multi tasking systems

such as Windows Enhanced Mode, OS/2, DESQview, Windows NT, etc it

might be more convenient to start up another task and do the

editing there, leaving paKet running in its own window.

Page 134

Alt-F/Alt-L      Find text in Flashback/Continue Find in Flashback

Pressing <Alt-F> will commence a search backward through the

Flashback buffer, looking for a particular string of characters.

A Message Window will pop up asking you to enter the desired

search string. You can enter this in either upper or lower case

because the search is not case sensitive.

If found, the line containing the string is highlighted and paKet

remains in Flashback mode.

Pressing <Alt-L> will continue the search, looking for an earlier

occurrence of the same string.

Further details on this option are found in the Communications

Windows - Flashback section.

Page 135

Alt-G            Get last message back again

Sometimes a message disappears before you have had time to digest

it or perhaps you thought you understood it but have forgotten

some important detail ("what was that filename again"?)

You can increase the message duration in the Miscellaneous

Configuration options, but it is possible you might want to

take a little more time with just an occasional message.

This <Alt-G> command will re-display the last message displayed in

the Message Window.  The re-displayed message will remain for the

normal message duration but you can type <Alt-G> as many times as

you like.

Page 136

Alt-I            Initialise Communications Windows

This option would not be used in normal circumstances but has been

provided mainly to "fix" problems that were discovered while

testing with a KAM TNC. Other Kantronics TNCs were not tested but

it is likely the problems will occur with those models as well.

These TNCs do not behave in the same way as other TNCs when

multiple streams are used as they do not support the STREAMDBL

command.

If, when monitoring the frequency, the program receives a STREAMSW

character from the TNC, it has no way of knowing whether it is a

data character that somebody has transmitted or whether it is

really a change of stream.  Other TNCs resolve this issue with the

STREAMDBL facility. If we are going to be able to handle multiple

streams, the program cannot ignore this STREAMSW character - it

must process it as a change of stream!

Suppose we have decided to use the vertical bar ("|") as our

STREAMSW character and while monitoring the frequency, we see one

of these characters come in.  Without the benefit of the STREAMDBL

facility paKet must assume it is a change of Stream and will

change over to another Communications Window!  So any further

monitored data will start appearing in that other Window.

If you are using one of these Kantronics TNCs and find data is

going into some other Communications Window, you will need to

reset the Streams, clear any responses in the other Windows and

get everything back into order.  This is what this <Alt-I> command

will do for you.

If you are using some other brand of TNC this command should not

be required although it will not cause any problems. It will

simply clear any backlog in the other Communications Windows and

reset the TNC to the Stream specified for the first Communications

Window.

Page 137

Alt-J            adJust Morse speed

There is a Morse Code option in paKet's Alerts facility.  This is

provided to make some unique sounds for the different alert strings

that might appear.

When you type <Alt-J> a Message Window pops up showing the current

Morse speed setting in WPM (words per minute).  You can change

this by typing the new WPM value or if you prefer, you can press

the <Up-Arrow> or the <Down-Arrow> keys to adjust the speed up or

down.   Press <Enter> to complete the change.

The speed will automatically be saved and the Morse Alerts will

now come out at this new speed.

Hev a look at the <Alt-A> command and the Alerts Window in the

Configuration section for more details.

Page 138

Alt-M            Activate the Personal Message System (PMS)

When you press this key you get the Sysop PMS menu:

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

While the menu is active all other communications will be held in

the input buffers until you have finished. You don't have to do

anything - paKet will automatically switch <ScrollLock> on and

will switch it off again when you have finished.

The Menu options are used as follows:

B  (Bye)       - End Sysop PMS Processing

C  (Copy)      - Copy a message to a file (eg C 3)

E  (Edit)      - Edit a message header (eg E 3)

H  (Help)      - Display these details

K  (Kill)      - Kill a message (eg K 3)

KM (Kill Mine) - Kill all messages addressed to me

L  (List)      - List available messages

LM (List Mine) - list all unread messages addressed to me

R  (Read)      - Read a message (eg R 3)

RM (Read Mine) - Read all unread messages addressed to me

S  (Send)      - Send a message (eg S VK2XYZ)

?  (Help)      - Display these details

The Personal Message System is discussed fully in its own section

of this Manual.

Page 139

Alt-P            Turn Print Log on/off

This is a toggle, meaning you press <Alt-P> to turn the Print Log

ON and press it again to turn the Print Log OFF again.

While the Print Log is ON, a 'P' will be displayed in the Status

Window to the left of the Windows display and all Communications

data will be echoed to the printer on LPT1.

If the Print Log is OFF, no data will be sent to the Printer.

Page 140

Alt-Q            Quiet Mode on/off

When Quiet Mode is enabled, paKet will make NO sounds at all.  It

doesn't matter where the sounds would normally come from, whether

from a new Connection, or from an Alert, or from another station's

<BEL> character, or whatever.  Quiet Mode will silence them all.

This might be convenient if you want to leave the system running

at night and do not want to be disturbed by the various sounds

that can emanate from this system.

This is a toggle switch, meaning you press <Alt-Q> to activate

Quiet Mode and you press <Alt-Q> again to switch it off.

When Quiet Mode is active, a "Q" will be displayed to the left of

the time in the Status Window.

Page 141

Alt-R            Rename/Move a data file  or  Register this paKet program

When the Disk Directory Window is displayed, <Alt-R> may be used

to Rename a data file or Move it into another directory.

Move the Highlight bar to the file you want to rename or move

before pressing <Alt-R>.

The Message Window will then pop up asking you to enter the new

filename.

If you want to simply change the file's name, leaving the file in

its existing directory, just type the new filename and press

<Enter>. The filename will be changed and the Directory Window

will be re-sorted and re-displayed.

If you want to Move the file into another directory, enter the

full path details into the Message Window (you may also enter a

different filename if you wish) and paKet will transfer the file

to the new directory.  The new Directory Window will then be

displayed, showing that file in the new directory.

When a file is Moved into another directory, the data is not

actually transferred, it is just the directory entry that is

changed. DOS does not permit a "Move" to another drive, so if you

want the file on a different drive, you'll have to COPY the file

yourself.  Refer to the <F9> DOS Command for access to DOS to

perform the COPY.

If you have not yet Registered this copy of paKet, you can do so

by pressing <Alt-R> at any time (except when the Disk Directory

Window is displayed!) and paKet will ask you for your Serial

Number.  If your number is entered correctly, it will be recorded

and your callsign will appear in the top left corner of the

display, indicating your copy of paKet is now registered.

Page 142

Alt-S            Script processing - load and run a Script file

This is the command you use to activate a paKet Script file.

(Script Processing is discussed fully in its own section in this

Manual).

When you select this option, a Disk Directory Window will appear

with the default Script file name already highlighted. If you

decide to enter a file name manually and you type a file name

without an extension, paKet will assume an extension of ".SCP".

When you select the desired Script file, that file will be loaded

and Script processing will commence.   The word "SCRIPT" will be

displayed in the Status Window to the left of the time display

while the Script is running.

When the Script has completed, or is aborted by <Esc>, the

indicator in the Status Window will disappear and you will be

returned to normal communications.

Page 143

Alt-T            Set TNC's Time and date

If for any reason, you need to reset your TNC, you can easily

reset the date and time with this function. paKet will generate a

"DAY" command using the computer system's date and time as it does

during initialisation..

The format of the generated command includes the time in Hours,

Minutes and Seconds. It has been brought to my notice that some

TNCs cannot handle that format and will accept the time as Hours

and Minutes only. So, if you hold the Ctrl key while typing this

<Alt-T> command (i.e. <Ctrl-Alt-T>), paKet will generate the DAY

command without the seconds.

Page 144

Alt-V            Verify the TNC's Connected status

When a Connection is established with another station, paKet will

try to determine the callsign of that station from the TNC's

"Connected" message when it appears in the Communications Window.

paKet will then display that callsign in the Communication

Window's Header line.

Some software packages operating in Host Mode or KISS Mode can

interrogate the TNC to get this information, but these packages

are usually more specific to certain TNCs. paKet, on the other

hand was designed to run on all TAPR-compatible TNCs and so relies

on the data flowing from the TNC rather than on some special TNC

codes.

paKet's method works well most of the time, but there are

occasions where the callsign displayed is incorrect. For example,

if you reload paKet after a TNC connection has already been

established, the newly loaded copy of paKet will not see a

"Connected" message and so will think there is no connection!

This <Alt-V> command will issue a "C" (Connect) command to the TNC

and will look at its reply to determine the connected status. A

Message Window will pop up to advise you of the result and, if

necessary, paKet will change the displayed callsign in the

Communications Window Header.

Page 145

Alt-W            Write contents of the Flashback buffer to disk

If you have received some information which you wanted to save and

is still in the Flashback buffer, but did not have the disk Log

File on at the time, you can select this option to write some or

all of the Flashback buffer to disk.

A Message Window will pop up asking you to:

"Move to the start of the block and press <Enter>"

The program will automatically enter Flashback Mode allowing you

to use the arrow keys, <PageUp>, <PageDown>, etc to move the

highlight bar to the start of the block of data you want to write

to disk.  If you want to capture the entire Flashback buffer,

press <Ctrl-PageUp> to move to the start of the buffer.

Then press <Enter> to identify that as the first line to be

captured.   Another message will pop up:

"Move to the end of the block and press <Enter>"

Using the same technique, move down to the last line you want to

capture (<Ctrl-PageDown> to move quickly to the last line) and

press <Enter> to mark that as the last line to be captured.

Note, the block of data to be written will be highlighted as you

move through the Flashback buffer.

Now that we know what is to be written to disk, paKet will ask you

for the desired file name, via a Disk Directory Window.

When that data is written to the file, processing continues in

normal conversation mode.

Page 146

Alt-X            Exit the paKet program

This command returns you to the operating system. Any files that

were open will be closed automatically.

To guard against inadvertent exits, paKet will ask for your

confirmation that you really do want to exit.

If you have specified any End-Auto Commands (Refer the

Configuration section for details) they will be sent to the TNC

and if paKet finds an AUTOEND.SCP file in the current directory

that Script will be executed before paKet returns to the operating

system.

The <Alt-F4> key achieves the same as <Alt-X>.

Page 147

Alt-Y            Yawn

Typing this key combination inserts a special code into the text

which causes a half second delay.  It may be added to your text as

you type in the Type Ahead buffer and the delay will occur when

the data is sent to the Communications Window and of course to the

TNC.  It may be typed into other strings too such as Keyboard

Macros.

Page 148

Alt-Z            Configuration

This option allows you to configure paKet's many options to set up

an environment especially to suit yourself.

When you press <Alt-Z> the System Configuration Window will appear

to show the major items that may be configured.  When you select

one of those options additional Configuration Windows will appear

to allow you to select the desired options.

Full details of all configurable options are shown and explained

in the section on Configuration Windows in this Manual.  If you

are new to this, start with the Configuration Windows - Overview

and work your way through all the options from there.

There is some additional help available with the Online

Configuration.  When the System Configuration Window appears,

another Window should appear below it offering some additional

comments on the Configuration Window currently being displayed.

This is called the Configuration Help and is optional.  If it is

not being displayed, you can activate it by setting that option to

Y in the Miscellaneous options.

To change that Configuration Help option, after pressing <Alt-Z>

type 9 and you will see in the Miscellaneous options window an

item in there that says "Display Configuration Help ?" and you can

set that to Y or N. Set it to Y and you should now see the

additional Help windows while you are using the Online

Configuration.  You will need the file PAKETCFG.HLP in your

default PAKET directory.

Page 149

Insert           Toggle Insert Mode

The <Insert> key toggles the Insert Mode on and off.

When Insert Mode is on, the cursor will be a Block shape and any newly

typed data characters will be inserted between existing data

characters at the cursor position.

When Insert Mode is off, (i.e. Overwrite Mode) the cursor will be an

Underscore shape and newly typed data will overwrite any data

previously stored at the cursor position.

If the Type Ahead Window is not being used, you will not be able to

"insert" characters (or move the cursor back) because in this mode

each character you type into the Communications Window is sent to the

TNC as you type. It is too late to insert something else!

Delete           Delete a character or delete a file

When entering text the <Delete> key will remove the character under

the cursor.

If a Disk Directory Window is currently displayed, you can delete a

file from the disk by highlighting the unwanted file then pressing the

<Delete> key.

Bksp             Backspace - delete the character before the cursor

When entering text the <Backspace> key will remove the character to

the left of the cursor.

Ctrl-Del         Delete a line

When entering text the <Ctrl-Del> key will remove all text currently in

the Type Ahead Buffer or on the line being entered.

Ctrl-Ins         Insert a blank line

In the Connect Window and in the Configuration Windows for the

Keyboard Macros, Auto Commands and Alerts, you might want to insert an

entry in the Window between two existing entries.

Move the cursor or highlight bar to the line which you want moved down

and press <Ctrl-Ins>.  This will move that line and everything below

it down to leave a blank line where the cursor is.

Page 150

Esc              Return to previous operation

The <Esc> key is used in a number of places in paKet, each time in a

manner of "exiting" or "returning" from the current function. It is

your way of saying "I don't want to do this any more".

For example, if in Flashback Mode, the <Esc> key will return the

system to normal communications.

And if displaying a Disk Directory Window the <Esc> key will return

the system to normal communications.

If there are several "levels" involved in the current function, for

example when using the Configuration Windows you could have two or

even three levels of Configuration Window displayed. The <Esc> key

will take you back to the previous level. If you are already at the

first level, it will take you back to normal communications.

You may note the <Esc> code cannot therefore be sent to the TNC from

the keyboard. If you need to send an <Esc> code to the TNC you could

define it in a Keyboard Macro as <Ctrl-[> (written as "^[" which is

caret and left-square-bracket). <Ctrl-[> generates the same code as

<Esc>. If you enter it this way paKet will process that as data,

sending the code to the TNC. Refer to the Special Key Codes section

for more information on the use of the '^' symbol for generating

<Ctrl- > key sequences.

ScrollLock       Hold all input data in the buffers

The <ScrollLock> key in paKet operates as a Pause key for input data.

Keyboard activity remains unaffected, so you may continue to send to

the TNC.

While the system is "ScrollLock"ed, any input data coming in from the

TNC will be held in paKet's input buffers until the Scroll Lock key is

pressed again to release it.  While ScrollLock is ON, an indicator

appears to the right of the Communications Window Header line.

You may have difficulty finding the <ScrollLock> function on some

machines such as the Tandy 1000 that are not quite compatible with the

Industry Standard.  Refer to your manual!

Left-Arrow       Move the cursor left one position

When entering text this key is used to move the cursor one character

position to the left. The text remains unchanged.

Right-Arrow      Move the cursor right one position

When entering text this key is used to move the cursor one character

position to the right. The text remains unchanged.

Page 151

Up/Shift-Up      Move UP one line or Type Ahead Recall

The function of this key depends on the situation you are in when it

is used.

It is generally used in the sense of "moving up one line".  So if you

have a Disk Directory Window displayed, this key will move the

highlight bar up to the previous line.  Same for the Online Manual,

TNC Help, and just about everything else.

The main exception to this is the Type Ahead Buffer.  If you have

configured multiple lines of Type Ahead, you must use the <Left-Arrow>

or <Ctrl-Left-Arrow> or <Home> to move back through that Type-Ahead

data.  That is, you cannot move up a line in a multi line Type Ahead

buffer.  You get there by moving left through the buffer, wrapping

around at the beginning of the line.

If one of the subsidiary Windows is not currently on display, the

<Up-Arrow> is used either to move back through the Flashback Buffer or

to Recall previous lines from the Type Ahead Buffer.   Whew!   Trouble

is... it can do one or the other but can't do both at once!

The previous version of paKet used this key to Recall from Type Ahead

and many people complained they wanted it for Flashback!  Well, in

this version you can make your own choice.  You can choose whether you

would like to use the <Up-Arrow> for moving up in the Flashback buffer

or whether you would rather use it for recalling lines from the Type

Ahead Buffer.   This selection is made in the Miscellaneous options

Configuration Window.

No matter which way you set that option, the OTHER option is available

with the Shift key.  So if you configure the <Up-Arrow> as a Flashback

key, you can recall Type Ahead lines with <Shift-Up-Arrow>. And vice

versa.

Down/Shift-Down  Move DOWN one line or Type Ahead Recall

The function of this key depends on the situation you are in when it

is used and is similar to the way the <Up-Arrow> is used.

It is generally used in the sense of "moving down one line".  So if

you have a Disk Directory Window displayed, this key will move the

highlight bar down to the next line.  Same for the Online Manual, TNC

Help, and just about everything else.

The main exception to this is the Type Ahead Buffer.  If you have

configured multiple lines of Type Ahead, you must use the

<Right-Arrow> or <Ctrl-Right-Arrow> or <End> to move forward through

that Type-Ahead data.  That is, you cannot move down a line in a multi

line Type Ahead buffer.  You get there by moving forward through the

buffer, wrapping around at the end of the line.

Page 152

If one of the subsidiary Windows is not currently on display, the

<Down-Arrow> is used either to move down through the Flashback Buffer

or to Recall the next line from the Type Ahead Buffer.   Whew!

Trouble is... it can do one or the other but can't do both at once!

The previous version of paKet used this key to Recall from Type Ahead

and many people complained they wanted it for Flashback!  Well, in

this version you can make your own choice.  You can choose whether you

would like to use the <Down-Arrow> for moving down the Flashback

buffer or whether you would rather use it for recalling lines from the

Type Ahead Buffer.   This selection is made in the Miscellaneous

options Configuration Window.

No matter which way you set that option, the OTHER option is available

with the Shift key.  So if you configure <Down-Arrow> as a Flashback

key, you can recall Type Ahead lines with <Shift-Down-Arrow>. And vice

versa.

Home             Move to the start of the text or the window

When entering text the <Home> key moves the cursor to the beginning of

the text. The text remains unchanged.

When viewing a Disk Directory Window, Configuration Window, the Online

Manual or the TNC Help File, the <Home> key is used to move to the

first of the items on display.

End              Move to the end of the text or the window

When entering text the <End> key moves the cursor to the end of the

text. The text remains unchanged.

When viewing a Disk Directory Window, a Configuration Window, the

Online Manual or the TNC Help File, the <End> key is used to move to

the last of the items on display.

PageUp           Move display up one page

When viewing a Disk Directory Window, the Online Manual or the TNC

Help File, the <PageUp> key is used to move up the list of items on

display a page at a time.

At other times this key is used for Flashback operations, moving up

one page at a time.

A "page" here is considered to be the size of the window being

displayed.

Page 153

PageDown         Move display down one page

When viewing a Disk Directory Window, the Online Manual or the TNC

Help File, the <PageDown> key is used to move down the list of items

on display a page at a time.

At other times this key is used for Flashback operations, moving down

one page at a time. If you reach the end of the Flashback Buffer and

there is some Input data waiting to be displayed, paKet will display

one page of that Input data and remain in Flashback mode. If there is

less than a page waiting, it will return to normal communications.

Ctrl-PgUp        Move to beginning of the Flashback buffer

This key is used for Flashback operations, scrolling to the beginning

of the Flashback buffer. While the scrolling is in progress, pressing

any key including <Ctrl-PageUp> again will stop it at that point.

Ctrl-PgDown      Move to end of the Flashback buffer

This key is used for Flashback operations, moving to the end of the

Flashback buffer. While the scrolling is in progress, pressing

any key including <Ctrl-PageDown> again will stop it at that point.

Shift-Left       Change to previous Communications Window

<Shift-Left-Arrow> tells paKet to change to the previous

Communications Window if available.

You might prefer to use the <Shift-Fn> keys to change Communications

Windows (eg: <Shift-F3> to change to Window 3) but whichever method

you choose, paKet will automatically issue the change of stream

commands to the TNC when you change Windows.

Shift-Right      Change to next Communications Window

<Shift-Right-Arrow> tells paKet to change to the next

Communications Window if available.

You might prefer to use the <Shift-Fn> keys to change Communications

Windows (eg: <Shift-F3> to change to Window 3) but whichever method

you choose, paKet will automatically issue the change of stream

commands to the TNC when you change Windows.

Page 154

Shift-Fn         Select another Communications window.

You may select another Communications Window, that is select another

Stream, by pressing Shift and a Function key.

<Shift-F1> is the first Communications Window, <Shift-F2> the

second, etc. For example, you can select Window 4 by pressing

<Shift-F4>.

Another way to change to another Communications Window is to use the

<Shift-Left-Arrow> or <Shift-Right-Arrow> keys to move, one-by-one

to the desired window.

Page 155

(This page intentionally blank)

Page 156

PART V - Messages in the paKet system

Overview

This is a list of messages in the paKet system. They are listed in

alphabetic order.

Most messages will appear in the Message Window. That window will stay

on the screen for a short time before it disappears to allow normal

processing to continue.

However, some messages might require some action on your part; or they

might be messages that I thought could take a little longer to digest.

Those messages will remain on the screen and will appear with a prompt

for you to press the <Enter> key when you are ready to continue.

Some paKet users have asked that the messages stay on the screen for a

bit longer to give them more time to read the message.  Others have asked

that the messages clear more quickly so they can get on with the job!  I

know it is dangerous to try to please everybody, but I have added another

option to allow you to choose your own message duration - check out the

"Message duration" option in the Miscellaneous Configuration Window. You

might like to note that if you have a message on the screen and want to

get rid of it before its "duration" has expired, you can press the <Esc>

key to clear it off the screen.

Many of the messages that appear are routine information messages that

require no action on your part.  They are there to let you know what is

happening in the system.  Other messages are warnings or even serious

errors and are things you really should know about.   As you gain

experience with the paKet system you might find some of the routine

information messages are not really required any more; they might even

become a little tiring.  If you would like to remove those Information

messages, you can adjust the "Msg level" option in the Miscellaneous

Configuration Window.  There are three levels there - the normal setting

is A so All messages are displayed.  You might like to change it to W so

only the Warning and Error level messages appear.   Personally, I prefer

the All option so all messages appear, and sometimes reduce the "Message

duration" rather than remove them altogether with the "Msg level" option.

But the choice is yours.

Some of the messages listed in this section appear in the Communications

Window.  For example, some are sent to a REMOTE operator who might be

accessing this system in your absence.

Page 157

Messages starting with A

A disk error has occurred here.

This message can come from a number of processes within the paKet

program.  Usually it means just what it says.  Sometimes it will be

followed with additional information that might help you to identify

just what is wrong.

For example, if you were saving a message to your Personal Message

System (PMS) it will add: "Your message is not saved."

Usually it will be obvious what operation caused the error.  The

message will appear immediately the operation is attempted so you will

know that (for example) if you just tried to create a new Log File with

the <F2> key and this message appears, then there is something wrong

with that Log File operation.

Typically a disk error is something as simple as a lack of space, a

write protected diskette, lack of privileges to access a network

device, etc.  It can also be caused if a new file is to be opened and

you have insufficient FILE handles available - increase the FILES

parameter in your CONFIG.SYS file if necessary.

If it is a genuine disk error other messages will appear and you will

probably be getting various errors when running other programs.

This message can be issued to a REMOTE user too if some attempted

REMOTE operation fails.

A smaller version of this file exists.

I will attempt a paKet-Protocol Recovery

This message could appear if a Binary File is being transferred to our

system and both stations are equipped to handle the advanced paKet

Protocol.

The situation that causes this message is where the file being

transferred already exists on our system but our file is shorter than

the one about to be sent by the other station.   This could happen if

an earlier file transfer was aborted for some reason and so our file

would be incomplete.

This message informs you that paKet is asking the other station to

check the details to see if a Recovery is possible.

Page 158

A Multiple of 16K allows more efficient use of EMS memory

Eg: 16, 32 48 or 64K

This message might appear after you change the Flashback Buffer size in

the Communications Windows Configuration.

If you have EMS memory available paKet can use some of it for Flashback

buffers.  The program manages pages of EMS memory in multiples of 16KB

so if you specify some other figure, you will not be making best use of

your available memory.

You may ignore the message if you wish, but it is recommended you

reconfigure the Flashback Buffer size to one of the above figures.

Abort, Retry, Ignore <R>

An error has occurred. You have three options: A, R or I...

A(bort) the program. If you select this option, paKet will simply give

up, and return to DOS. As this is a rather serious step, paKet will

seek your confirmation that this is really what you want to do by

prompting you with the following question:

Aborting paKet and returning to DOS...  Are you sure? <N>

If you then reply "Y", everything stops and you will be returned to

the DOS Prompt.  It is nice to have a way out in case of a really

serious problem but I expect this option would be rarely required.

R(etry) the operation that caused the error. This is the default

response so if you simply press <Enter>, paKet will assume a Retry. In

most cases I hope this will get you going again. For example it might

be a diskette problem such as "door not closed", "write protect tab",

etc so you can attend to the problem, select Retry and things should

proceed normally.

I(gnore) the problem.  An error message is like pain: it is there for a

reason and should not be ignored.  But if you are sure you know what

the problem is and choose to ignore it, you can select this option and

paKet will try to continue despite the error.  However, do this at your

own peril and don't call me if you get into trouble!

Aborting at end of block...

Press <Esc> again to force immediate end of the transfer

When you are performing a Binary File Transfer using the YAPP or the pP

protocol, data is sent in blocks of up to 254 bytes each.  The data in

each block may contain any value at all so if we need to send a <CANCEL>

code to the other station in the middle of a transfer, that station will

not be able to determine whether that code is really a <CANCEL> or just

another data byte in the file that happens to have that same value.

Page 159

So if you do decide to Abort a file transfer by pressing the <Esc> key,

paKet will not issue the <CANCEL> code immediately.  It will wait until

the end of the block currently being sent, before transmitting the

<CANCEL> code.  That way, the other station has a better chance of

recognising the <CANCEL> code.

This message is displayed to acknowledge the Abort request and to let

you know what is going on.

If there is some problem with the transfer and it appears to be locked

up, or if the other station is not responding, you can press <Esc> a

second time and force an immediate abort.

Aborting...

Refer to the above message.

When the <CANCEL> code is sent to the other station, this message

appears to let you know it is done and that we are now waiting for the

other station to acknowledge that command before we continue.

Alerts now enabled but will not sound while QUIET mode is on

This message can appear after you press <Alt-A> to enable Alerts Mode.

The Alerts facility in paKet monitors data coming and going in the

Communications Window and sounds the bell when some predefined string

of characters appears.  The predefined strings are specified in the

Configuration under the "KB Macros/Auto commands" section.

However, paKet also has a Quiet Mode option (refer <Alt-Q> key) where

ALL sounds are disabled with a single keystroke.  If you happen to

enable Alerts at a time when Quiet Mode in on, this message appears to

warn you paKet will not be making any sounds even if the specified

Alert string is detected!

... already exists.  Message NOT copied!

Eg:

VK2DHU.123 already exists.  Message NOT copied!

In paKet's PMS there is a Copy function which allows you to copy any

message in the PMS to a data file on disk.

The PMS and the Copy command are discussed more fully in the PMS

Section of this Manual.  But briefly, you can type "C 123" at the Sysop

PMS Menu and paKet will copy the contents of message 123 to disk.  The

Page 160

name of the new disk file will consist of the callsign of the station

the message is addressed to; and the extension of the file will be the

message number.  For example, if message number 123 is addressed to

VK2DHU, the new disk file will be VK2DHU.123.

If you try to do the copy again, paKet will not overwrite that disk

file.  Instead it warns you with the above message.  You should either

delete or rename that existing disk file before trying the Copy

function again.

Another station is currently connected.

Please try again later

Although paKet can handle up to 10 simultaneous sessions with 10

different users, there is a problem when a REMOTE user tries to perform

a Binary File Transfer.

The standard TAPR TNC, on which all modern TNCs are based, can run in

either Converse Mode or Transparent Mode, but not both!  Converse Mode

is used for normal conversation and Transparent Mode is required for

Binary File Transfers.  So if a standard TNC is switched into

Transparent Mode, ALL sessions are switched into Transparent Mode,

including those other sessions currently in Converse Mode!  And vice

versa...   This is a limitation of the standard TNC and as paKet is

written to support the standard TAPR compatible TNC, there is not much

I can do about this problem.

Therefore the program will not allow a REMOTE user to initiate a Binary

File Transfer while there is any other station connected at the time.

The above message is issued if such an attempt is made.

If YOU decide to initiate a Binary File transfer while connected to

someone else in another Window, I can only assume you know what you are

doing!  The program will allow this to continue, but I hope you DO know

what you are doing!

Appending to existing file ..."

This message appears when you are starting the Log file or are writing

the Flashback buffer to disk if paKet finds the specified file is

already on the disk.

paKet will preserve the contents of that file and will write the new

data to the end of this file.

No action required.

Page 161

ASCII file command must be AD, AC or AU

The Script file you have just loaded contains a command beginning with

A but it is not one of the valid ASCII file transfer commands.

The Script has been aborted.  Edit the Script file to correct this

invalid line and run the Script again.

Page 162

Messages starting with B and C

Bad copy of program.  Reinstall paKet from original diskette

Something is wrong with the copy of PAKET.EXE you are running.  It is

hard to know what could have caused the corruption so the safest option

at this stage is to reinstall a fresh copy from the original diskette

or from your backup copy.

BaycoM command must be MW, MD or MU.

A REMOTE user has issued a command beginning with M but paKet has

failed to recognise it as one of the valid commands.  Normally M is

used for the BayCom file transfer commands so paKet issues the above

message to help the REMOTE user with the required syntax.

Before registering, please enter your Call Sign in the

Configuration's Miscellaneous options (<Alt-Z> then 9)

You typed <Alt-R> to register the program but I need to know your

callsign to confirm whose copy this is.  Enter your callsign in the

Miscellaneous Configuration Window and paKet will then ask for your

serial number.

Buffer Search

Please enter the search string  -

When you wish to search backwards through the Flashback buffer for a

particular string of characters, this message will prompt you for the

data to search for.

This is not case sensitive so it doesn't matter if the data is in upper

or lower case, if it is spelt the same it will be matched!

Refer to the section on the <Alt-F> and <Alt-L> keys for further

discussion on this facility.

CFG file error - incorrect version

This message is displayed on the DOS screen immediately prior to the

program's return to the operating system.  If the config file is not

the correct version, paKet does not know what parameters to use so it

aborts before it gets started!

Page 163

The version number appears in the first line of the CFG file and must

be the same as the current version of the program.

If you have no backup of the correct CFG file, run INSTALL again to

rebuild the CFG file.

CFG file error

This error will appear immediately after you attempt to start the

program if there is something wrong with the PAKET.CFG file. It will

not even appear in a pop-up window because those windows will not have

been created yet.

The error message could be followed by further details such as:

"invalid Keyword ignored"

"no keyword"

"invalid colour"

"invalid data"

"too many beginning commands"

"too many ending commands"

"too many alerts"

In any case, the offending line in the config file will be displayed

to help you identify the fault.

You should never see any of these messages because the config file is

managed by the program itself and it can be expected to do the job

correctly.  You should have no need to manually edit the file.  (Er, no

offence, but I suspect the only way the config file will be corrupted

in this way is if someone tries to edit the file with their text editor)

There is a section in this Manual covering the Config File, and

discusses the various parameters.  This was included for completeness

of the program documentation and for those who are interested in the

workings of the system.  It is not included as an invitation to fiddle!

Closing existing file capture

You are running a Script and paKet finds an AD command in that Script.

(That AD command is used to open a file for an ASCII Text Download)

If there is already a Text file download in progress, paKet will close

that existing file before opening the new file specified in the AD

command.

The above message appears to inform you of the action being taken.  You

do not have to do anything.

Page 164

Closing log file (filename)

This is a message to confirm the Log File is now being closed.

This message could appear after you close the Log File with the <F2>

key or if the file is closed automatically by the AUTO log function

when a station is Disconnected.

No action is required unless you want to continue logging, in which

case you should manually reactivate the Log File with the <F2> key.

COMx is not installed. Try another? (Y/N)

When paKet starts, one of the first things it does is to set up the

Computer's Serial port. It tries to initialise the port you have

specified in your Configuration but if that port is unavailable, this

message appears to give you the opportunity to try another one.

This condition should rarely occur but is possible if you have changed

your Computer's hardware or if you have copied a Configuration File

from another system.

Anyway, no harm is done as paKet will cycle through all the Serial

Ports until it finds the one you wish to use.

You should check the rest of the Configuration, especially the Serial

Port details once the system starts up.  If the Port number was wrong,

it is possible other parameters will need adjustment too.

Confirming...Connected to (callsign)

After pressing <Alt-V> to verify the connected status, this message

will appear if the TNC agrees with the callsign already displayed in

the Communications Window Header line.

No action required.

Confirming...Not Connected

After pressing <Alt-V> to verify the connected status, this message

will appear if there is currently no connection and the TNC has simply

confirmed this.

No action required.

Page 165

Corrupted PAKETCFG.HLP file

The PAKETCFG.HLP file contains help information that may (optionally)

be displayed while you are accessing the Online Configuration Windows.

You should not see this error message at all - if it does appear, it

suggests someone has modified the file!

Extract the original file from your original paKet diskette or from

your backup copy.

Page 166

Messages starting with D and E

Deleting (filename) - Are you sure? (Y/N)

If you attempt to delete a file, paKet will ask for confirmation before

it performs the deletion. You have one final chance!

Enter "Y" to carry out the deletion.

Disk error on drive X

Unfortunately this means just what it says.

This error condition is returned by the DOS operating system. The causes

are many and varied so you should perform the usual checks to try to

identify the cause.  Typically the error is related to the lack of space

on a disk or diskette, or perhaps a diskette drive door was opened.

If there really is a fault with the disk you will be getting similar

error messages when you are running other programs. In that case

specialist attention is required.

The error message is usually followed for some additional information

returned by the Operating System, such as:

"Write protected"

"Unknown unit"

"Drive not ready"

"Unknown command"

"Data Error (CRC)"

"Bad Request"

"Seek Error"

"Unknown media type"

"Sector not found"

"Printer out of paper"

"Write fault"

"Read fault"

"General failure"

"Unknown error type"

and you will usually be given the following options:

"Abort, Retry, Ignore <R>"

Check the DOS documentation for further information, and refer to the

description on the "Abort, Retry, Ignore" message for discussion on the

options available.

Page 167

Displayed Callsign changed to match the TNC

After pressing <Alt-V> to verify the connected status, this message

will appear if the TNC indicates we are connected to a station other

than that indicated in the current window's Communications Window

Header.

No action required.

Download commenced

Download successful

Download UNSUCCESSFUL

These advisory messages appear in a System Note which is displayed in

the Communications Window (and in your log if the Log File is active at

the time) when a Binary File Transfer is performed.

The messages are produced mainly to cover those situations where the

file is transferred by a REMOTE operator or from within a Script file.

In other words, at a time where you are not present to see the activity

and I thought it would be helpful to be able to scroll back into the

Flashback buffer or to check the log to determine the success or

otherwise of the transfer.

Enter a DOS command, or press Enter for a DOS window.

>

When you want to exit to DOS to perform some program or DOS command you

can do it in one of two ways:

1.  You may enter the command into the message window beside the

">" prompt.

If you do this, the command will be executed (if possible) and you

will be invited to "Press any key to continue" before paKet

resumes where it left off.  The "Press any key to continue"

message appears briefly over the DOS display to allow you time to

peruse the result of the job and to read any messages etc that may

have come from the DOS command.

2. Press <Enter> instead of entering a command, and the screen will

be cleared before control is passed over to DOS and the familiar

DOS Prompt.

When you have finished the DOS operations, type EXIT to return to

paKet.  Do NOT type PAKET again as the system will attempt to load

a second copy of paKet into memory!

Please keep in mind that paKet is still in memory at this stage,

gathering any incoming data that may be sent from the TNC while you are

Page 168

doing your DOS jobs. So you will have considerably less free memory for

your other DOS programs and may even find some will not run due to

insufficient memory. The only remedy for this, I am afraid, is to

return to paKet and exit the program so you can release the memory

occupied by paKet.

If you are running paKet from a Multi Tasking system such as OS/2, or

perhaps as a DOS task under Windows Enhanced Mode, DESQview, etc it

would be better to simply use the system to start another DOS task and

run your other program from there.  That way, you will have the full

facilities of the system and probably full memory etc.  Check your

system parameters to ensure paKet's task continues to share the CPU

while in the "background" otherwise you could lose any data that might

be sent from the TNC.

Enter any command you want sent to the other station (Esc if none).

There is a RAW Binary File Transfer mode available in paKet.  This mode

is provided to cover those situations where you need to conduct a

Binary File transfer with some other computer system that does not

support pP, YAPP or BayCom file transfer protocols.

With the RAW mode there is no protocol used at all so there is no

handshaking performed by the system - it is up to you to do it all

manually.

One of the things you might need to do after initiating the transfer is

to send some command to the other system to get its end of things

going.  So this message will appear to allow you to enter that other

command and if entered, that data will be sent to the other station

before paKet switches over to Transparent Mode for the File Transfer.

Enter new Path\Filename for (filename)

When you select a file to be Renamed or Moved to another directory

(with the <Alt-R> key), paKet will ask for the new name.

In response to this message, enter the new filename or, if you wish to

move it to another sub directory, enter the full path details. For

example: \PAKET\BINARY\UTILS\USEFUL.COM

If the Rename or Move cannot be performed you might see one of the

following self explanatory messages:

"No such file or directory"

"I can't do that."

"Maybe that file exists?"

"I can't MOVE to a different device!"

Page 169

Enter Path:

When you choose to "Enter file name manually" from the Directory Window

you will be asked to enter the details of the desired file in this

Message Window.

Any valid file name may be entered here, including a drive and/or

directory together with the desired filename.

Error closing the log file

The operating system has detected an error while closing the Log File.

Check the usual things (available space, diskette drive door open, etc)

and be warned that you could have lost some of the log file with this

error.

Processing will continue without the Log File.

Error in Script JUMP command

This message appears when you run a Script but the program finds some

syntax error in your Script file.  In this case the error is in a JUMP

command.

Refer to the section on Scripts in this manual for a discussion on the

correct syntax requirements.

Error saving Configuration. No changes saved.

If you make any changes to paKet's Configuration, the changes are

automatically saved to disk, but if this message occurs, the latest

changes to the Configuration have NOT been saved.

Processing continues, but you should check your system to see why the

error occurred. The possible causes are many and varied but if you are

not having other problems this could be as simple as running out of

space.

Error setting new directory

When the Directory Window is on the screen, you may change directories

by highlighting a sub directory entry (shown as "<DIR>") then pressing

<Enter>. The system will change to that directory and will then display

the contents of that new sub directory.

Page 170

You may also choose another directory by specifying the path manually.

This error message occurs when the system is unable to change to the

new directory. As you would usually be changing by pointing to the

displayed directories, this error should be rare.  It is most likely

you have typed the path manually and have mistyped something.

Error writing to file being Received - now closed

A disk write error has occurred while receiving an ASCII file.

The file is now closed but is probably incomplete. You should check

your computer system to see what might have caused the problem.

Typically this error is caused by a lack of space on the disk or, if a

diskette is being used, check the diskette is not write protected and

the drive door is closed.

When you have corrected the problem, you can try the file transfer

again.

EXIT to DOS?

Press "Y" to Confirm

After pressing <Alt-X> or <Alt-F4> to exit the program, this message

will appear asking for your confirmation that this is what you really

want. It is so easy to press the wrong key (I have done it often).

If you enter "Y" the paKet program will close down and control returns

to the DOS prompt (or to the Menu if you loaded paKet from a Menu).

Type anything else and paKet ignores the Exit request, resuming

whatever it was doing.

Page 171

Messages starting with F, G and H

Failed to make window

There are over 20 different windows used by the paKet system and there

is some memory allocation and initialisation required to set these up.

The most likely problem if this message appears is Insufficient Memory.

File display not available.

A REMOTE user can request a display of the disk directory on our system

with the W, MW or YW commands.   paKet prepares the listing in sorted

sequence as a temporary file on disk, then that temporary file is sent

to the other station.

If some disk error occurs during the creation of this temporary file,

the above message is sent to the other station and the directory

listing is not sent.

Action required is similar to that for other disk errors.

File has no data

You have selected a file to be sent to another station but this file is

empty!

File path is ignored (I will use the file NAME).

A REMOTE user has issued a file upload command to send a file to our

system but has included a drive and/or directory name in addition to

the file name.

For your protection, paket will not allow uploads into anything but the

Upload directory you have specified in the Configuration.

The above message is sent to the other user, but the file transfer

continues using just the file name specified.  That new file will be

written into your Upload (receive) directory.

File write error

While performing a Binary File transfer a disk error has occurred. As

various checks are usually performed before the transfer begins, I am

afraid this message does suggest you have a hardware problem.

Maybe if you are using diskettes the problem is simply a matter of the

drive door being open?

Page 172

... has the telephone bells disabled here, but give it a minute or so..."

Eg:  Tony has the telephone bells disabled here, but ...(etc)

If a REMOTE user selects the T(alk) option to leave REMOTE Mode and

continue in normal conversation mode with you, paKet will usually ring

the bells to attract your attention.

If you have the "telephone bells" option disabled or you are running in

Quiet Mode the program will not ring the bells.  In this case, the

above message is sent to the REMOTE user for information.

Hey, we have got more data than the file size expected!

I can't handle that, so I will abort the transfer

This should not happen because the system will be expecting the exact

file size as indicated in the directory. But just in case something

goes wrong...

Hey, you can't send the CURRENT log file!

Think about it...the file will grow as fast as you can send it!

You wouldn't really try to transmit the current log file, would you?

Close the Log File, then try again.

Housekeeping...tidying up the PMS DataBase...

From time to time, when you are using the Sysop PMS Menu (refer the

<Alt-M> key) paKet will perform some internal housekeeping to minimise

any wastage of space in the PMS sub-system. This message appears

briefly to inform you of this housekeeping.

Page 173

Messages starting with I

I am unable to record your Serial Number

Try recording your Serial Number with the INSTALL program

but if no success, contact your official paKet distributor.

When recording your Serial Number, the program updates itself on disk.

That is, it writes your Serial Number into the PAKET.EXE file.

But these messages will appear if it is unable to do so.

The two most common causes of this error are:

1. The program is write-protected with the Read-Only attribute or

it is on a write-protected diskette;  or

2. You are running the Microsoft SHARE program (usually from

within your system's AUTOEXEC.BAT file) and SHARE will not

allow us to write the new serial number into the PAKET.EXE file

while it is being used!

If 1, remove the write protect status from the PAKET.EXE file and try

again.   Please note there is no other form of modification being done

to the program - it is simply recording the Serial Number.  You may

still give a copy of your program to others, even with your Serial

Number recorded.  In fact I encourage you to do so (although I would

rather you give them a copy of the original distribution diskette just

to help ensure they get the entire package).

If 2, SHARE will not allow us to record the Serial Number while paKet

is running.  You could remove SHARE temporarily, probably by editing

your AUTOEXEC.BAT and rebooting, or you can register the program from

paKet's INSTALL program. See the INSTALL.DOC file for details. The

computer system will be happy to update your copy of the PAKET.EXE

program from the INSTALL program because then you will not be actually

running the PAKET program at that time!

I assume you have ALREADY set the TNC's parameters.

If you want to change some Serial Port settings, the procedure is

usually to issue the commands to the TNC then change the computer's

settings in the Serial Port Configuration Window.

The TNC will note the changes but not actually use the new settings

until a RESTART command is issued (some non standard TNCs use RESET but

RESTART is the most widely used).

If, after making some changes to the Serial Port Configuration, you

ask paKet to issue the RESTART command, the above message appears to

remind you the TNC parameters should have been made already.

Page 174

Messages starting with "I can't..."

I can't append to the file

A disk error has occurred after choosing to append to the existing

file. At this stage, we know the file exists so the most likely cause

would be a lack of space or a genuine hardware error. I hope you are

out of space!

I can't create a new PMS DataBase

I'll try again next time.

While doing some housekeeping on the PMS DataBase, paKet tried to

create a new file to contain the "cleaned up" PMS but got a disk error.

This housekeeping can wait till next time - it is not critical - so

processing continues. You should check it out as soon as possible to

see what the problem is.

I can't create the file

An attempt was made to create a new data file but it failed.

You could check to see there is sufficient space available and if you

are using the Root directory, ensure there are sufficient directory

entries available.

If using diskette, ensure the diskette is not write protected and the

diskette is inserted in the drive with the door closed.

It should be quite obvious which file paKet is trying to create because

of other messages that will be displayed and because it is probably the

result of some command you have given that the file is being created

anyway.

If the new file is one specified in the Configuration, for example the

Connect File, check the details in the Configuration - it may be you

have specified a drive or directory that does not exist.

I can't delete this entry

You may delete a normal FILE from a Directory Window, but you may NOT

delete a Read-Only, Hidden or System File, nor a Volume Label!

I can't do that. Maybe that file exists?

The system returned an error condition when trying to rename a file.

Page 175

Typically this could be because the new name you gave is a file that

already exists in that directory.  The Rename/Move function is not like

a COPY command - it will not overwrite an existing file.

I can't find COMMAND.COM

You are in big trouble!  The system could not find COMMAND.COM and you

will not be able to run other DOS jobs without it!

Usually DOS can find it via the COMSPEC parameter, or maybe in the

current directory or via the PATH statement.

I can't find (filename)

This error message appears after paKet is asked to process this data

file but the file could not be found.

It is most likely to appear after you have manually entered a file name.

Try the operation again, specifying the correct file name, not

forgetting to specify a path (drive and subdirectory) if necessary.

The error could also appear after selecting a file which is specified

incorrectly in the Configuration. For example if the location of the

TNC Help file is not correctly recorded in the Configuration this

message will appear when you press <F10> to see the TNC Help file. In

this case, select the Configuration (<Alt-Z>, then 8) and make the

necessary corrections before trying the operation again.

If you are in REMOTE Mode and the other station requests a file

transfer, this message will be sent to that other station if the

specified file could not be found.

If the file paKet is looking for is a Help File, it will probably

display an additional message such as:

"I will continue but without the Configuration Help windows"

and processing will continue, as this error is considered less serious.

If paKet is unable to find the PAKET.CFG file, this message will be

displayed immediately prior to control returning to DOS. The CFG file

should normally be in the same directory as the paKet program, but you

may specify a different the CFG file, including a path,  on the command

line if you wish.

If you would like to have multiple configurations available, for example

a husband and wife might share the computer for packet operations; then

each can have their own configuration, not just for things like colours

but for Operator Name and callsign, etc. Or you might have a different

setup for VHF and for say HF operations or for PacTOR operations.

Page 176

To specify a different CFG file, specify the name of the desired CFG

file on the command line.  For example to load and run PAKET with a

config file of HF.CFG you type the command line as follows:

PAKET HF

You do not need to specify the .CFG extension.  If no parameter is

specified on the command line, paKet will look for a CFG file with the

same name as the program (i.e. PAKET.CFG) in the same directory as the

program.

I can't find (filename). I'll create a new one.

When you first use the <Alt-C> facility to Connect to another station,

paKet will fail to find a Connect File and will produce this message to

let you know it is creating a new one.

You can enter as many station details as you wish into this window and

it will be automatically saved to disk using the "filename" specified.

Next time that file should be found and the message will no longer

appear.

There is a possibility the filename is incorrect in paKet's

Configuration in which case the message will serve as a warning that

you are not accessing the correct file. Or perhaps the file is on

diskette and the appropriate diskette is not in the drive. If so, you

can delete or ignore the new file just created, and insert the correct

diskette.

I can't find message nn!

You have entered a PMS command but the message number you have

specified does not exist.

Reenter the command with the correct message number. You could perform

a List command ("L") to see the available messages.

I can't find the (filename) Script file.

If a REMOTE user attempts to Perform a Script on our system, paKet will

send back this message if that file cannot be found in our Scripts

directory.

I can't find the (dir) directory.

This message will appear if you have manually entered a file name and

path but the directory could not be found.

Try the operation again with the correct name.

Page 177

I can't find the program to run

The "program" in this case could be a program name you have entered at

the DOS command line provided by paKet's <F9> DOS exit.

Another possibility is the system could not find the program to Edit or

to Display a data file if you have used the <Alt-E> or the <Alt-D>

commands.

If the message occurs after you select <Alt-E>, check paKet's Online

Configuration - Directories and Files and correct the System Editor

entry before trying the operation again.

If the message occurs after you select <Alt-D>, check paKet's Online

Configuration - Directories and Files and correct the "Command to list

a file" entry before trying the operation again.

I can't MOVE to a different device!

When moving a file with the <Alt-R> key, the system will not actually

copy the data, it simply adjusts the directory entry.  As different

drives have different directories, a "move" is not possible between

drives. You will have to copy the file to the new drive then (if

desired) delete the file on the original drive. Use the COPY command

from the DOS Gateway (refer <F9> key).

I can't read the message file.

Ask (your name) to investigate the cause

A REMOTE operator tried to read a message from our PMS but something is

wrong with the file.

Your guess is as good as mine at this stage.  Perhaps you have had some

problems with the system or maybe you have been adjusting the

configuration and the files are no longer in the correct directory?

This message is sent to the other station to let them know they are not

going to be able to see that message they wanted!

I can't Rename/Move this entry

You may Rename or Move a normal FILE in a Directory Window, but you may

NOT Rename or Move a Hidden or System File, a Sub Directory, nor a

Volume Label!

Page 178

I can't see CTS signal on the Serial Port!

Would you like to try Software Handshaking? (Y/N)

You might need to refer to the discussion on Hardware vs Software

Handshaking in the Technical Section of this Manual if you have

difficulty understanding what this is all about.

If you are using Hardware Handshaking the CTS signal should be present

when the program starts but paKet has found that signal "missing".

It is possible the cable used to connect the TNC to the Computer does

not have this CTS line connected. Such cables are quite common and if

you have one of these, you must change to Software Handshaking.

If you reply "Y" paKet will change it's Configuration to Software

Handshaking and processing will continue. Please ensure the TNC's XFLOW

command is set ON if you use Software Handshaking - this is especially

important if you are doing any file transfers.

If you reply "N" paKet will attempt to continue in Hardware Handshaking

mode, although without a CTS signal it will be unable to send anything

to the TNC!   Usually you would select this option after you have been

able to restore the CTS line and then processing will continue normally.

Page 179

Messages starting with I (cont)

I did not get the expected response from the TNC

So, I am not too sure whether we are connected or not!

When you press <Alt-V> to verify the TNC's connected status, paKet

issues a command to the TNC and interrogates its response but in this

case the expected response was not received.

The displayed callsign is not changed.

If this happens more than once, it may be the TNC is not responding for

some reason so you should check that and ensure it is still working for

normal commands, then try the <Alt-V> again.

I don't have room for all the help file

This error message means there are more index entries in the file than

I have allowed for.

It is unlikely you will see this message, but if you do it is because

someone has modified the Help file since I released it.  In that case

modify it back again, or restore an original copy from the original

paKet system.

I have a disk error and cannot record the message.

Ask (your name) to investigate the cause.

This message is sent to a REMOTE user when a message is being left on

our system but paKet is unable to write it to the disk.

I have ignored this CONNECTED message because

we seem to be Connected on this stream already!

A "CONNECTED" message has appeared in the Communications Window but the

program thinks we are already connected on this stream.

Usually this string comes from the TNC when a new connection is

established and paKet detects that particular string of characters to

determine the callsign of the station we are now connected to.

Sometimes someone else could transmit some data that happens to contain

that same string of characters, so when it appears in the Communications

Window paKet can easily get confused and think the TNC is telling us we

now have a new connection!

Page 180

If the program thinks we are already connected on this stream it will

not process this CONNECTED string as a new connection; instead it will

simply issue the above message to let you know it has been ignored.

If you wish, you can press <Alt-V> to ask your TNC to verify the

callsign of the station we are currently connected to.

I need the file name and the file size.

Eg: MU PROG.COM,14392

Enter zero for the filesize if you don't know it's size in bytes.

A REMOTE user (presumably a BayCom user) is attempting to send a

Binary File to us but the MU command was incomplete.

At the time of writing this Manual, the BayCom system does not support

any Binary File Transfer protocol.  Without a formal file transfer

protocol, paKet has no way of knowing when the end of the incoming data

file is reached.

So, I have implemented a requirement that the file SIZE be specified.

If the program knows the size of the file, it can count the data bytes

as they are received and then it will know when to close the file!

To cover the situation where the file size is not known, the program

will allow the transfer to proceed if you specify a 0 for the file

size.  But then you will have to close the file manually.

For full details on this operation, have a look at the description of

the <Alt-F8> key in the Keyboard Commands section of this Manual.

I'll ring the bell. If no response soon, maybe (your name) is not around?

A REMOTE user has typed a "T" command at the menu to leave REMOTE Mode

and to talk to you in normal conversation mode.  The above message is

sent to the other station to acknowledge that command.

paKet will sound the bell to alert you to this and will remain in

Converse Mode for a short time to allow you to come to the keyboard.

If you are not there, the program will return to REMOTE Mode and send

the menu to the other station.

Insufficient memory

There are thousands of places within paKet (well quite a few anyway)

that could issue this dreaded message.

In the current version of paKet I have gone to great lengths to

minimise the chances of getting this message.  paKet will perform some

Page 181

internal overlays to reduce the memory demands but will still require a

system with at least 384KB available.

If you happen to have EMS memory available, paKet will use some of that,

notably for the Flashback buffers which tend to consume lots of precious

memory.   But most of the memory required comes from the Base 640KB.

Try to conserve as much memory as possible because even if paKet runs

quite happily in the available memory, you might still need more if you

want to run some external program from within paKet.  For example,

editing a file will add to the memory demands as the system will have to

load the chosen Editor program then load the data file you want to edit.

Or if you take the DOS exit (refer <F9> key) you might find there is not

enough memory left, after paKet has taken its lot, to run your DOS

commands.

There are some things you can do to overcome this "Insufficient memory"

problem:

1. If you are one of the few remaining diehards who are running an

IBM or compatible system with less than 640KB RAM, then the

solution is easy: "buy some more memory!"

2. Reduce the memory demands from within paKet by:

a. configuring fewer Communications Windows (I find 3 plenty

for my operation). Each of these will consume some memory

overheads and if you don't really need all 10 windows you

can save some memory by reducing the number.

b. configuring smaller buffers for Input and Flashback buffers

in each of the Communication Windows.

Most of the time I use only the first window - it is only

on relatively rare occasions the second window gets used,

the third window hardly ever. So I tend to configure larger

buffers for the first window and reduce the others.

3. Reduce other memory overheads such as TSRs.

Many people do not realise that Menu programs, Shells, and the

like that provide a "launching pad" for their programs, usually

remain in memory while their other program is running. You should

run paKet from the DOS prompt, by all means from a BATch file,

but not from a Shell or Menu.

And any other TSRs such as DOS's Print Spooler, Mouse drivers,

various Pop-Up utilities such as Calendars and Phone Diallers,

all consume some memory even if they are not currently being

used. There are techniques available for some systems now to load

these things in High Memory (eg the latest versions of MS-DOS and

DR-DOS, as well as other products such as QEMM provide this

"load-high" facility). If you can, load as much as possible into

High Memory to relieve the pressure on the Base 640K. Otherwise

try removing any unwanted TSRs and drivers before loading paKet.

Page 182

4. Make some EMS memory available.  If you have more than 640KB of

memory in your system you can usually allocate some of this for

EMS (sometimes called Expanded memory) and then paKet will use

that for some overlays and Flashback buffers.

Refer to your DOS documentation (or to the documentation for the

memory manager program you are using, such as QEMM, etc) for

details on how to set up your system for EMS memory.

If EMS memory is available, paKet will automatically detect it

and use as much as it can for its buffers etc.  Depending on your

configured buffer sizes, paKet might use 200 to 300 KB of your

EMS memory but in any case it will display a message at start up

time, to inform you it is using EMS memory and telling you how

much is used and how much is still free.

Sometimes the lack of memory is not critical.  For example if you are

loading the Online Manual (refer <Alt-F1> key) and paKet has

insufficient memory to load that information, you will get the above

"Insufficient memory" message followed by something like:

"Continuing without Online Manual..."

You will not be able to view that Online Manual but at least paKet

processing can continue (without that facility).

Insufficient space for this file.

When receiving a Binary File, paKet will check the file size (once it

is known) and verify there is sufficient space on the drive to hold the

file.

If the file is not going to fit, this will avoid wasted time by

aborting the transfer before it starts.

Invalid argument

This message will appear if you have specified an invalid DOS command.

Check the command and reenter.

Invalid FLAG command in Script

The Script file you have just loaded contains a command beginning with

F which suggests a Flag command but the syntax is invalid.

Refer to the section on Script Processing for syntax details.

Page 183

Invalid keyword ignored in CFG file

This message will appear only if someone has manually edited the

PAKET.CFG file!  It is best to leave the configuration to paKet's

online configuration facility.  That way, all changes to the

configuration will be checked and validated before they are written to

the file!

If you don't know how to correct the PAKET.CFG file to get rid of this

message, you might have to run the INSTALL program again to rebuild the

file.

Invalid Message Header.

Just enter S to leave a message for (your name), (your callsign)

To leave a message for a particular station, enter 'S (callsign)'

Note the <space> after the 'S'. Eg: S VK2XYZ

These messages are sent to a REMOTE user who has typed a command

beginning with S but it does not conform to the required PMS syntax for

sending a message.

These messages are self explanatory but for more information on the

syntax, check the PMS section of this Manual.

Invalid path.

There is something wrong with the path specified.

For example, this message will be sent to a REMOTE user who has

attempted to access an invalid directory with a W or D command.

I/O Error - Bad Request

I/O Error - Data Error (CRC)

I/O Error - Drive not ready

I/O Error - General failure

I/O Error - Printer out of paper

I/O Error - Read fault

I/O Error - Sector not found

I/O Error - Seek Error

I/O Error - Unknown command

I/O Error - Unknown error type

I/O Error - Unknown media type

I/O Error - Unknown unit

I/O Error - Write fault

I/O Error - Write protected

Page 184

An error has occurred and paKet has attempted to identify the

particular error. These errors are documented in your computer's

Operating System documentation and are reasonably descriptive of the

problem.

In most cases you will know what is causing the error as you will have

just performed some function, for example you will have requested a

directory display from a diskette, or you will have attempted to use

the printer.

Refer to your DOS system documentation for further information on any

action to be taken.

And refer to the description on the "Abort, Retry, Ignore" message for

discussion on the options available.

Page 185

Messages starting with K, L and M

Keystroke ignored.  Press <Esc> to abort the transfer.

While conducting a Binary File Transfer, you should not be typing any

other commands and you can't talk to the other station while the

transfer is in progress, so any keystrokes (except the <Esc> key) are

going to be ignored.

This message will pop up if you do happen to type something during the

Binary File Transfer.   No harm done though.

Leaving REMOTE mode - returning to Normal Communications.

This message is sent to a REMOTE user when paKet is leaving REMOTE Mode

and returning to normal conversation mode.

It will usually appear if the REMOTE operator types the T(alk) command.

Leaving the menu...now communicating with (your name) the paKet operator

If you decide to cancel REMOTE mode with the <F3> key, this message

will be sent to the other station to let them know they are no longer in

REMOTE Mode.

Loading Manual from (filename)

Loading (TNC Help File Name)

paKet maintains an index into the Online Manual (and the TNC Help file)

but if that index is lost of if there is any change to the Manual or

Help File, the program will reload the manual and rebuild the index so

it will be quicker next time.

If the index needs to be reloaded, this message appears to let you know

it is working and it confirms the name of the file it is loading the

information from.

It can take a few seconds, longer if loading from diskette, so you are

reassured the system is still working.

If you get an "Insufficient memory" message here, it will be followed

by a message such as:

"Continuing without Online Manual..."

paKet will then continue with normal communications but, of course, in

that case you will not be able to view the Online Manual or the Help

File as the case may be.

Page 186

Log file now closed because of file error

Further input held with <ScrollLock>

A disk error has occurred while writing to the Log File.

Perhaps the most likely cause is the disk has run out of space. You can

delete any unwanted files from paKet's online Disk Directory Window

(<Alt-D>) ; or you can exit to DOS with the <F9> key and fix the

problem there.

If you were logging to diskette, you can replace that diskette and open

another log file on a new diskette.

In most cases paKet will automatically switch on <ScrollLock> to hold

any input data in the buffer until you get the log file going again.

This way you will not lose anything from the log file.  When you have

rectified the problem, press <F2> to activate the Log file again (if

you want to) then press <ScrollLock> to release the data from the Input

Buffer.

Looks like a Disconnect?

REMOTE mode cancelled.

paKet has detected what looks like a Disconnected message while in

REMOTE Mode..

If the other station disconnects we must shut down REMOTE mode because

if we are going to send the Menu, we must have someone to sent it to!

The normal procedure would be for the REMOTE user to type B(ye) to end

the session then everything is closed down tidily.  But if that other

station simply disconnects or perhaps if there is a problem with the

radio link, paKet will ensure there are no loose ends to cause any

problems.

Maximum 280 lines per 'page'

When displaying the Online Manual or the TNC Help file, each "chapter

or section" is retained in memory for quick and easy scrolling. In

doing this I have imposed a limit of 280 lines for each chapter or

section.

This has proven to be quite ample and works fine with the files as

distributed. So if this message appears, the file is either corrupted

or has been changed by someone (and, come to think of it, isn't that

the same thing?).

Page 187

Message file closed by Disconnect

The other station has disconnected while paKet is receiving a PMS

message.

The Disconnect is processed and the Message File is closed to preserve

whatever part of the message has been received so far.

Move to the start of the block and press <Enter>

Move to the end of the block and press <Enter>

When you press <Alt-W> to write some data from the Flashback Buffer out

to disk, these messages will pop up to guide you in selecting the data

you want to capture.

(For users of earlier versions of paKet, this is new.  Previously you

had the option to write the entire buffer or none at all, but now you

can select any part of the Flashback Buffer you wish).

So, use the arrows to move back through the Flashback buffer to the

beginning of the block of data, then press <Enter>.  The second of

these messages will then appear asking for the end of the block.

Move the highlight bar to the end of the block (note the block is

highlighted as you move the bar) and press <Enter>.

Hint:

If you find the thing beeping at you when you press the arrows it is

because the message window is still on the screen and you should wait

for it to disappear.   You can remove that message window before its

duration has expired by pressing <Esc>.

Must be .......

When making changes to paKet's Online Configuration, there are some

checks performed to ensure the new values entered are valid.  paKet

will issue a message in the above format to advise you if it thinks an

error has been made.

The message will tell you what it expects to see.  For example a

message might be:

"Must be between 0 and 250"

This tells you the value you entered was wrong, and that the correct

values must fall within the range indicated.

Page 188

Messages starting with N

No files

This message will be sent to a remote user if a directory display has

been requested and:

1. there are no files in the specified directory;  or

2. if a file specification has been included in the request

(eg: YW \BIN\*.EXE) this message will be sent if there are

no files matching that specification;  or

3. if the directory name is incorrectly specified.

No further Information available. Sorry.

When the REMOTE user types "I" for Information, paKet will send the

contents of REMOTE.INF file from the default directory.

However, if that file cannot be found, this message will be sent

instead.

No HELP available. Sorry.

When the REMOTE user types "H" for Help, paKet will send the contents

of the REMOTE.HLP file from the default directory.

However, if that file cannot be found, this message will be sent

instead.

No messages

This message will appear in response to a PMS command (eg List) if

there are no messages currently recorded.

No such file or directory

This error message occurs after an attempt to Rename or to Move a file

with the <Alt-R> key.

It would usually appear if you have specified a new sub directory in an

attempt to move a file to another subdirectory, but the specified

subdirectory does not exist on this drive.

Page 189

No, the TNC thinks we are NOT connected!

After pressing <Alt-V> to verify the connected status, this message

will appear if paKet thought there was a connection in the current

window but the TNC disagrees, indicating there is NO connection.

The Communications Window Header is changed to show "Not Connected".

No, there's nobody here.  Here's the REMOTE menu back again...

After the REMOTE user types a T command, paKet will return to normal

converse mode, optionally sound the bell, and wait for you to respond

with some keyboard activity. If you have not responded within say 30

seconds, paKet will return to REMOTE Mode and will send the above

message before re-issuing the menu.

No, you may not leave a message for a third party

In some countries it is still illegal for a message to be addressed to

a third party.  So there is an option in paKet's configuration to

disallow third party messages in your PMS.

If your system is configured this way a REMOTE user tries to leave a

message addressed to a third party, paKet will refuse to accept it and

will send this message to the other station.

Not enough free memory

The external program you are trying to run will not fit into whatever

memory is still available after paKet has taken what it needs.

You might have to return to paKet and exit (<Alt-X> or <Alt-F4)) before

trying the other program again.

Not found

If you have initiated a search through the Flashback buffer for some

text (via the <Alt-F> or the <Alt-L> keys) the matching text will be

highlighted.  This message will appear if there is no (more) matching

data in the buffer.

Page 190

Now I can't find message nn

This is probably a program bug, so hopefully you will never see it. It

means paKet is now unable to find a message it was working on earlier.

If you get this, you'd better let me know!

Now I can't read the PMS DataBase file!

An error has occurred trying to do some housekeeping on the PMS

DataBase file.

As far as the rest of the paKet processing is concerned, this is not a

critical error because normal communications can continue without the

PMS. So normal processing continues.  It is likely you will lose the

messages that were in the PMS but check it out.

Page 191

Messages starting with P

paKet Registration

Please enter your Serial Number

When you type <Alt-R> to register this copy of the program, or when you

change the configured callsign, paKet will ask you for your Serial

Number.  Type the number into the message window that is displayed.

If you do not yet have your Serial Number, just press <Enter> and the

program will continue as an unregistered copy.  I assume you are going

to register so you will always have the full facilities available in

the copy you have.

paKet-Protocol Recovery approved

Issuing approval to the receiver station...

We are sending a Binary File and the other station has requested a

Recovery of a partly completed file transfer.

After checking the details provided by the other station, paKet has

decided it DOES look like the same file and is sending its approval to

the other station.  Now both sides can begin the transfer from the

agreed point in the file.

paKet-Protocol Recovery approved

now appending to file...

We are receiving a Binary File from another station and our system had

requested a paKet Protocol Recovery.

This message will appear if the Recovery is approved by the other

station. The transfer will continue from the point where it aborted

last time. It will be approved provided the other station is using

paKet Protocol and both stations agree it seems to be the same file.

paKet-Protocol Recovery denied

This does NOT look like the same file

We are receiving a Binary File from another station and our system had

requested a paKet Protocol Recovery.

This message appears if the other station has refused our request for a

Recovery because it does not appear to be the same file.  The file

already on our system will be preserved and a new file will be

transferred in its entirety.

Page 192

paKet-Protocol Recovery denied. The receiver's file does NOT match ours.

The entire file will be sent from the beginning

We are sending a Binary File and the other station has requested a

Recovery of a partly completed file transfer.

After checking the details provided by the other station, paKet has

decided it is NOT the same file and is sending a message to the other

station, refusing the Recovery request.

paKet-Protocol Recovery requested by the receiver

Attempting recovery - verifying file contents...

We are sending a Binary File to another station and that station thinks

we may have tried this one before, unsuccessfully, because a shorter

version of the same file already exists on that system.

Some details of that file have been provided by the other station and

paKet is now checking to see if it could be the same file.

Performing AUTOEXEC.SCP

Performing AUTOEND.SCP

The AUTOEXEC and AUTOEND Script files are optional.

If the AUTOEXEC.SCP file is present paKet will run that Script as soon

as it starts up (immediately after any Begin-Auto commands).  The

Begin-Auto commands, which may be specified in the Configuration, are a

simpler way to send any initialisation commands to the TNC but a Script

provides more flexibility should the need arise. You can have both the

Begin-Auto commands and the AUTOEXEC.SCP if you wish.

When you press <Alt-X> or <Alt-F4> to exit the program, paKet will look

for the AUTOEND.SCP file and if present, it will run that Script before

any End-Auto commands that may exist in the Configuration.

The above messages are displayed to inform you these Script files are

being processed.

Please check the BBS Connect Path in the PMS Configuration

This should be a valid Script file name enclosed in pointer brackets.

According to the configuration file, you want to use a Script to

establish connection with the BBS but the syntax is invalid for a

Script File name.  Check the configuration details by typing <Alt-Z>

then 6.

Page 193

If you specify the callsign and connect path to your BBS, you just type

the callsign etc. For example:

VK2ATM-1 v VK2RPM-1

But if you want to specify a Script file name, it should be entered in

that Configuration with "pointer brackets".  For example: if the Script

File is BBSCONN.SCP, it should be specified as

<BBSCONN>

(The SCP extension is optional).

Please close the log file first

You should not edit a file that is already open and being used for

something else. DOS gets horribly confused and upset.

You will get this message if you try to edit the current Log File. If

you really want to edit this Log file, press <F2> to close it first,

then try the Edit command again.

Please enter a Message Number with the command.

Eg: R 4

In paKet's PMS, the messages are identified by a number. When you enter

a command to process a message, the system needs to know which message

you are referring to.

Reenter the command, specifying the message number after the command as

shown in the above example.

Please include the filename with the command.

A REMOTE user will get this message if paKet receives a file command

such as D (download) or U (upload) but the other station has not told

us which file!

Please set the BBS Connect Path in the PMS Configuration

You entered <Alt-F3> to initiate a Mail Forwarding session but have not

yet set the BBS connect path in the PMS configuration.  So I do not

know the BBS callsign nor how to connect to it.

Type <Alt-Z> then 6 for the Configuration Window to record the BBS

details before attempting a Mail Forwarding session.

Page 194

Please take care with this. RESTART is the command mostly used.

On most TNCs, RESET will clear everything including your MYCALL!

This message will appear as a warning when you select the Serial Port

Configuration and change the item specifying the command to initialise

your TNC.

When changing Serial Port parameters, you will usually be changing both

the TNC's parameters and the computer's parameters and need to keep the

two devices synchronised in order to maintain communications. The way

this is done is to send the commands to the TNC first because most TNCs

will accept the changes but will not actually apply those changes until

the TNC is re-initialised.  Next, you change the Computer's Serial Port

parameters.  paKet will then ask you if you would like to initialise

the TNC and (if you reply 'Y') will send the RESTART command to the TNC.

This worked fine until I found the Kantronics range of TNCs are

different to the others and require the RESET command instead of the

RESTART command! So, in order to offer some support for these devices I

added a configurable option to use either RESTART or RESET.

This warning message appears whenever you select RESET because this

command should be selected ONLY if you have a non standard TNC which

uses the RESET command in this way!  On all other TNCs the RESET

command will clear EVERYTHING from it's memory including your callsign!

You will have to enter all your settings again if you (or the program)

send a RESET command to the TNC!

Press any key to return to paKet...

This message will appear briefly after an external program has been run.

If you run a DOS command from the paKet screen (refer <F9> key) or

Display (<Alt-D>) a file, you will often need some time to look at that

screen before returning to normal communications.

Pressing <Alt-F6> again will close the file

This message will appear after you begin a RAW Binary File Transfer

with the <Alt-F6> key because with this RAW Mode there is no protocol

being used and you will be expected to close this file manually by

pressing that same key combination again.

Pressing <F6> again will close the file

This message will appear after you begin an ASCII Text File Receive

operation with the <F6> key. It serves to remind you how to terminate

the file transfer if the other station does not do so.

Page 195

All data received into this Communications Window will be written to

the selected file until the other station sends a <Ctrl-Z> to close the

file and terminate the File Transfer operation.

If the other station does not send the <Ctrl-Z>, or if you wish to

close the file part way through, you can press <F6> at any time and the

file will be closed and the File Transfer terminated.

Printer is not ready!

The Printer status indicates it is not ready to accept any print data.

When the printer is ready, press <Enter> and paKet will continue.

You may see some additional message text, especially if the Printer Log

is active:

"Esc to switch off Printing"

In this case you could press <Esc> to clear this message and to switch

off the Printer Log.

Program error.

This message will never appear because it refers to a program logic

error.  It can never happen!

But er, in previous versions some of these "can never happen" messages

have appeared so I thought it best to include a reference them in this

Manual.

If you do get one of these it indicates something like

"Config Help already loaded."

which is telling us the program tried to load this help file when it is

already loaded.  That can never happen, can it?   But uh, just in case

it does, you had better let me know about it.

Page 196

Messages starting with R

Ready to receive ASCII text file...(Ctrl-Z to end)

A REMOTE user has commenced an ASCII file upload to our system with the

U command.

This message is sent back to acknowledge that command.

Ready to receive binary file using paKet Protocol...

or

Ready to receive binary file using YAPP protocol...

When a REMOTE user initiates a Binary File Transfer to our system with

the YU command, paKet will send back this acknowledgement if it is

ready and able to receive the file.

The content of this message is determined by your configuration setting

as to whether or not you wish to allow the use of paKet-Protocol (pP).

Even if it does indicate the use of pP, the transfer might still be

conducted using standard YAPP protocol if the other station supports

only that method.

Ready to send binary file using paKet Protocol...

or

Ready to send binary file using YAPP protocol...

When a REMOTE user initiates a Binary File Transfer from our system with

the YD command, paKet will send back this acknowledgement if it is

ready and able to send the file.

The content of this message is determined by your configuration setting

as to whether or not you wish to allow the use of paKet-Protocol (pP).

Even if it does indicate the use of pP, the transfer might still be

conducted using standard YAPP protocol if the other station supports

only that method.

Receive file now closed

An EOF (i.e. a <Ctrl-Z> character) was received while receiving ASCII

file.

Normal condition - no action required.

This message can also appear if the other station disconnects without

closing the ASCII file we are receiving.

Page 197

Receiving ASCII file

Script command 'AC' will close the file

This message appears if you are running a Script and there is an AD

command which is processed to open a text file to receive any incoming

data.

The normal process would be for that same Script to close the file with

an AC command somewhere else in that Script file, but the message

alerts you to the fact that the file is open.  You can close it

yourself (with the <F6> key) if necessary.

Receiving Binary file FROM BayCom

Enter the file name in the BayCom system:

When you request a Binary File transfer from a BayCom system, paket

needs to know details of the file so it can generate the required

commands to send to that system. If this command is used with Digicom

(some versions), that system's name for the file may be one containing

blanks or other oddities incompatible with a MS-DOS filename.

If the BayCom system has the desired file in another directory you

should also specify its sub directory name. For example if the file you

want is called MYPROG.EXE and it is stored in the BayCom system's PRG

directory, you would enter:

PRG\MYPROG.EXE

If it responds with a "file not found" error message, check the

directory details with the BayCom operator. A common cause of this

error would be if you inadvertently specify a leading "\", eg:

\PRG\MYPROG.EXE

This message will then be followed by:

What should I call it here?

paKet wants to know what name to give the file here on our system. You

might normally use the same name as that in the BayCom system, but you

can change it if you wish.

And finally, the following message appears to ask for the file size:

Now please enter file SIZE in bytes (-1 for Digicom):

You should enter the size of the file you are transferring because the

BayCom system may not tell us when the transfer is complete. The BayCom

operator should be able to tell you the file size. If you do not know

the file size in BYTES, enter 0 (zero), but in that case you will have

to close the file manually.

If you are trasnferring a file from a Digicom, enter -1 for the file

size to tell paKet it is a Digicom transfer. You will still have to

close the file manually.

Page 198

Receiving RAW Binary file

Enter the file name:

We are about to receive a file using the RAW Binary File Transfer

method.  As this method has no file transfer protocol at all we have to

do everything ourselves, including identifying the name of the file.

REMOTE deactivated by Disconnect

The usual way to log off from REMOTE is to enter B for Bye or T for

Talk.  This message indicates the connection has been severed by a

Disconnect command to the TNC at either end.

No harm done and no action is required.  paKet will close down the

REMOTE system and return to normal mode.

REMOTE is available only while Connected

You have pressed <F3> to place paKet into REMOTE mode and to issue the

Menu to the other station, but paKet thinks we are currently not

connected, so there is no other station to send the Menu to!

Try it again after a connection is established.

Occasionally paKet might fail to detect the connection and although you

are really connected to another station, paKet shows "Not Connected" in

the Communications Window Header Line.

If this has happened you will not be able to enter REMOTE mode. You can

correct this by pressing <Alt-V> to ask paKet to verify the connected

status with the TNC. If this changes paKet from "Not Connected" to show

a callsign in the Communications Window Header, you can then try the

<F3> again.

REMOTE Mode cancelled

At times things can go wrong with a Binary File Transfer.  There are a

multitude of possible causes, but the effect is that one station

continues to send while the other end, the receiving end of the

connection has aborted the transfer and is back in the REMOTE Menu.

When the sending station sends another block of data, the poor guy with

the REMOTE Menu sees all these funny "commands" coming in and sends

back a whole lot of "*** Invalid option" messages!

So if paKet sees several of these "*** Invalid option" messages go out

in quick succession, it will shut down REMOTE Mode, display this

message, and wait for things to settle down.

Page 199

Returning to normal communications...

You pressed B (Bye) after using the Sysop PMS Menu. This will release

the <ScrollLock> condition and the system reverts to normal

communications mode once again.

Running under OS/2

Running under Windows

Running under Windows Enhanced Mode

While testing under these different environments, I displayed these

messages to confirm the program recognised the particular environment

it was running under at the time.  It was my vague intention to see

what enhancements could be added to gain better performance from the

multi tasking systems.  (Needless to say, that didn't get done.)

Maybe in another release, I will find time to enhance the paket system

so it performs better in a multi tasking system.  This version does

work well with the above systems but has not been enhanced to take

advantage of the facilities offered by any particular operating

environment.

I was going to remove these messages after initial testing once I was

satisfied the program correctly detected the situation.  However,

despite the small overhead, I decided to leave them in.

Page 200

Messages starting with S

Saving configuration parameters in PAKET.CFG

If you make any changes to paKet's Configuration, the changes are saved

to disk automatically.

This message confirms the action taking place.

Script file not found

After starting Script Processing with the <Alt-S> key, you have entered

a Script file name, but paKet cannot find that file.

Usually you would select a Script from the Directory Window so this

message would be rare.  However, if you have manually entered the file

name, check the spelling and ensure you have entered any subdirectory

details if necessary.

Script label not found

A Script command specifies a Script label but paKet cannot find that

label in this Script.  Refer to the Script Processing section of this

Manual for details of Script syntax.

Script Performed successfully

This message is sent to a REMOTE user after the successful performance

of a Script via the P (Perform) command.

Note however, a successful Performance means the Script completed

without syntax errors etc.  It does not necessarily mean the Script has

done everything expected of it.  For example if it was to transfer a

file and that file could not be found, paKet might have issued an error

message about the file not found, but as far as the Script processing

is concerned, the Script has now completed successfully because it

performed every one of the commands it was asked to Perform!

It is up to you, or the Script programmer, to ensure adequate messages

are issued from the Script itself to inform the REMOTE user of

progress for that particular job.

Script Processing Aborted

If paKet detects a serious problem in your Script file, the Script will

be aborted and paKet returns to normal communications mode.

Page 201

This message alerts you to the fact that the Script has NOT been

completely executed.

Script UNSUCCESSFUL!  Ask (your name) to check the Script

A REMOTE user has issued the P command to Perform a Script but paKet

has failed to complete the Script successfully.  The reason is possibly

a fault in the Script code itself.

The REMOTE user gets this message but if the problem is in the Script

file, you will have to attend to that yourself.  The REMOTE user cannot

fix that remotely.

Searching for available Script files...

If a REMOTE user enters a P command without specifying the name of the

particular Script File to perform, paKet will send back this message

while it goes searching for all the files with an SCP extension in our

Script directory.

Then it will send the details of those filenames to the REMOTE station.

Sending Binary file TO BayCom

Enter the file name in the BayCom system:

When you request the transfer of a binary file to a BayCom system,

paKet will ask for the name you want to give this file when it is

written to the BayCom system. (You can send to some Digicom versions

also).

Usually it would be the same as the file name in our system but you can

change it if you wish.

Another reason for this message is to allow you to specify a

sub-directory as well as a file name for the BayCom system. For

example, say we are sending the file UTIL.EXE to a BayCom system and we

want to write it to the BayCom user's PROG sub directory. So, in

response to the above message you would enter:

PROG\UTIL.EXE

Of course paKet does not know what sub directories are available on the

BayCom system - that is your job to find out.

Page 202

Setting KISS Mode OFF. Sending these codes to the TNC:

nnn nnn nnn

If you press <Alt-K> to set KISS Mode off, this message appears to show

you the codes being sent to the TNC.

Refer to the  <Alt-K> key in the Keyboard Commands section of this Manual

for further discussion on resetting KISS Mode.

Sorry, but this station does not accept Uploads

This message is sent to a REMOTE user who types a U, MU or YU command

to attempt to upload a file to this system, but you have disallowed

file uploads in your Configuration options.

Sorry, but this station does not allow Downloads

This message is sent to a REMOTE user who types a D, MD or YD command

to attempt to download a file from this system, but you have disallowed

file downloads in your Configuration options.

Syntax error in Script line.

The Script file currently being processed has a syntax error.  Details

of the line are give to help you identify the fault.  For example the

message might include additional information such as:

"(no comma)"

Some Script command require two fields separated by a comma and if that

comma is not found, the Script process will abort with that message.

Refer to the section on Scripts in this Manual for full details of

Script syntax.

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

This is the menu that appears when you press <Alt-M>.

The PMS sub system is discussed in its own section of this Manual.

Page 203

Sysop PMS Menu Help...

B  (Bye)       - End Sysop PMS Processing

C  (Copy)      - Copy a message to a file (eg C 3)

E  (Edit)      - Edit a message header (eg E 3)

H  (Help)      - Display these details

K  (Kill)      - Kill a message (eg K 3)

KM (Kill Mine) - Kill all messages addressed to me

L  (List)      - List available messages

LM (List Mine) - list all unread messages addressed to me

R  (Read)      - Read a message (eg R 3)

RM (Read Mine) - Read all unread messages addressed to me

S  (Send)      - Send a message (eg S VK2XYZ)

?  (Help)      - Display these details

This list of commands is available only from the Sysop PMS Menu. It

appears in response to the H or ? command.

Further details of each of these commands are given in the section on

the Personal Message System in this Manual.

System Note at HH:MM:SS - msg

A System Note is an information message that is a little different to

other information messages in that it appears in the Communications

Window (and optionally in the log file) rather than in the popup

Message Window.

These System Notes are mostly used to log information on the success or

otherwise of a file transfer.  These file transfer operations can be

time consuming and the operator might start the transfer and go do

something else while the file being transferred.  So a brief popup

message might not be seen and it was felt a more permanent record was

required.

Of course no action is required, as this type of message is designed to

cater for those situations where the operator might not be present

anyway.

The HH:MM:SS is the computer's time when the System Note is issued.

"msg" should be self-explanatory. For Example:

"Upload successful - FILENAME.DAT"

Page 204

Messages starting with T

Thank you.

Recording your Serial Number...

This copy of paKet is now Registered.

A note of appreciation to those who do the right thing.

That directory is not available. Sorry.

This message is sent to the REMOTE user who has entered a command to

access one of our directories but that directory name is either invalid

or inaccessible to the REMOTE user.

That's not the correct Serial Number.

When you enter your Serial Number, paKet will check it for validity and

let you know if it does not check out.

No harm done, just try it again.

If you find there must be something wrong with the number you have,

contact me or your official paKet distributor.  In the meantime, you

can continue to operate the paKet system with full facilities

available.

The BBS is not responding

You tried to initiate a Mail Forwarding session with the BBS but the

TNC failed to establish a connection with the BBS within a reasonable

time.

That "reasonable" time is currently set to 60 seconds for a direct

connection and to 4 minutes in the case of a connect path which

includes digipeaters or network nodes.

Those times should be quite sufficient in normal circumstances. In

case of some temporary problem, just press <Alt-F3> when you think

things are back to normal and paKet will try again to connect to the

BBS and conduct a Mail Forwarding session, sending any mail waiting in

your PMS and taking any incoming mail the BBS wants to send.

The EOF has not been acknowledged.

We are sending a Binary File with pP or YAPP Protocol and have sent the

last byte followed by the End-of-File code.  The other station should

Page 205

then acknowledge that code so we know the transfer has successfully

completed.

This message appears if the EOF acknowledgement has still not arrived

after the specified delay has expired. (2 minutes is the time specified

in the YAPP Protocol specifications and is the default paKet setting).

When paKet has sent the last byte and the EOF code, it starts counting.

In most cases 2 minutes is plenty of time for the other station to get

the last of the data and close the file there before sending its

acknowledgement.  However, if your TNC is equipped with a large buffer

your TNC might still have a significant part of the file still in its

buffers waiting to be transmitted.  paKet doesn't know about that and

thinks it finished when it sent the EOF to the TNC!  It could easily

take more than 2 minutes for the TNC's buffer to be cleared.

This message will pop up to warn you we are still waiting for the

acknowledgement.  But, provided paKet is not in REMOTE mode, the

following message will also appear:

Continue waiting for another  nn  minutes.  (Press <Esc> to Abort)

You can choose to wait for the time shown just by pressing <Enter>.

But you can change that time so if you do happen to have a TNC with a

large buffer, you might like to change the default value of 2 minutes

to something larger to suit your particular TNC.  paKet will remember

the new value, so if you change it to (say) 5 minutes, paKet will wait

for 5 minutes next time before telling you the EOF has not been

acknowledged.

The same messages will reappear at the end of that time if the

acknowledgement has still not arrived.

If you press <Esc> the transfer will be aborted in accordance with the

original YAPP specifications.

If our paKet system is performing this Binary File transfer in REMOTE

mode, you may not be around to answer the question. In that case the

following message appears:

"but as I am in REMOTE mode I will continue waiting..."

and paKet will start another timeout period.  It will do this 3

times (i.e if the default delay is 2 minutes, paKet will wait a total

of 8 minutes) before aborting.

The message file was not closed

The other station started to send a message to our PMS but has

disconnected without completing the message.

paKet has closed the message file so the message, or at least that part

that we had received, will be retained.

Page 206

The PMS data file is missing, so all messages are lost!

The PMS index is missing so all messages are lost!

paKet's PMS is stored in two files, a data file which contains the

content of the messages in the system; and an index file which contains

just the message headers and an index into the data file.  The two

files are PAKETPMS.DAT and PAKETPMS.IDX.

If one of these files are missing, or if the Operating System is unable

to open the file, one or both of the above messages will be displayed.

Both files are required for the PMS system to work.  So if one of them

is OK and one is bad or missing, there is not much point in keeping the

good one!  You might as well consider them both bad and start again.

This problem is possibly be due the the files being unavailable for

some reason. Possible reasons include:

1. you have deleted them;

If so, that's tough! You've lost any messages that might have been

in the PMS. If the data file alone is still there, you can get a

fair idea of the messages by reading it though.

You could restore the TWO files from a recent backup?

2. the Configuration file has been changed and paKet can no longer

find the files;

In this case, you should change the Configuration by entering

<Alt-Z> then selecting option 8 (Directories and Files) to

correctly specify the location of the PMS files.

3. The PMS files are on a diskette and that diskette is not in the

drive.

I don't think you need me to tell you what to do here. I'll leave

you to figure this one out on your own.

The other station seems to be in paKet REMOTE mode too.

REMOTE mode cancelled!

The other station has sent what looks like the paKet REMOTE menu while

our system is also in REMOTE mode.

Our REMOTE system expects to get one of the valid menu commands but if

instead of one of the valid commands, it receives the other guy's menu

it will respond with an "*** Invalid option" message followed by

another copy of our Menu.

Page 207

The other guy is going to do the same when he gets our Menu and they

would happily go on all day sending each other these "*** Invalid

option" messages and Menus!

REMOTE mode is now disabled on this system to avoid the loop and the

unnecessary exchange of error messages.

The Serial Port has been adjusted.

Would you like me to send a RESTART to the TNC, now? (Y/N)

I assume you have ALREADY set the TNC's parameters.

These messages appear after you make some change to the computer's

serial port via paKet's Online Configuration Window.

Usually when you change one of these parameters, you should change the

setting in the TNC too because if the two devices are not using the

same settings they will not communicate.  The trick is to get them both

changed at the same time.

The recommended method is to:

1. send the required commands to the TNC to change the TNC settings;

2. then make the change in paKet's Configuration.

Most TNCs will note the requested change but will not apply certain new

settings until a RESTART command is issued.

So, after changing paKet's Configuration, paKet will optionally issue

the RESTART command to the TNC to enable both devices to now continue

with the new settings.

Some TNCs, such as the Kantronics range, use the RESET command instead

of RESTART so paKet has a configurable option (refer Serial Port

options) and will send either RESTART or RESET depending on the option

you choose.

The TNC does not appear to be ready

paKet is trying to send a character from a string of data to the TNC,

but the TNC is unwilling to accept it at this time.

paKet will wait up to 10 seconds without complaint, but any longer

suggests a possible problem, so this message will appear to warn you.

After this message appears, that character will be sent to the TNC

anyway as in most cases the TNC will have an input buffer of 80 free

bytes or more and the character will not be lost.

If the TNC is still locked after another 10 seconds, the next character

will cause the message to be repeated.

Page 208

This message should appear quite rarely as under normal conditions you

will not fill the TNC's buffers. If you are doing file transfers the

buffers could fill but special handshaking logic in that part of the

program will take care of it under those circumstances. If you have

filled the buffers with lots of KB Macros, then slow down a bit!

The only action you need to take here is to ensure the TNC and radio

are functioning normally, and slow down if you are pumping in a lot of

data from a Script or Macros.

Page 209

Messages starting with T (cont)

The TNC is not responding.

Abort, Retry, Ignore  R

When the program first starts up, it will attempt to establish

communications with the TNC by sending the following:

<Ctrl-X>

<XON>

DAYTIME command (if that option is enabled)

any Begin Auto commands

AUTOEXEC.SCP (if it exists)

If it gets no response, the above message will appear to alert you to

the possibility the TNC might not be switched on or the cables might

not be properly connected.

There are three options: A, R or I.

A  is to ABORT the program and return to DOS. In this case, paKet will

simply give up and you will see the DOS prompt reappear (or the menu

if you started paKet from a menu).

You may decide to take this option if you find something wrong with

the TNC or its connections and prefer to do something else with the

computer while you attend to those problems. It is a quick and easy

way out if communications with the TNC cannot be established.

R  is the RETRY option. If you can fix the problem that caused this

message to appear, selecting the RETRY option will allow paKet to

try again and continue as if the problem had not occurred.  Of

course if you have not really fixed the problem you will get the

message again!

I  will IGNORE the problem and paKet's processing will continue.

Normally I recommend against the Ignore option when an error occurs,

however in this case it may be desirable to Ignore the error and let

paKet to continue, despite the lack of communication with the TNC.

An example of this situation is where the problem is caused by a

mismatch in the Serial Port settings. Either the TNC or the Computer

must be changed to allow normal communications to take place. It is

easier to change the Computer (paKet in this case).

Type "I" to Ignore the errors and when the Initialisation is

complete, you can select the Online Configuration's Serial Port

Window to make the necessary changes. If you do this, don't forget

the TNC's Date/Time may not be set and the usual initialisation

commands may not have been processed by the TNC.

Whenever I get into this situation, I make the necessary changes,

then exit the program so I can make a good, fresh start.

Page 210

This "Ignore" option will also allow paKet to be used for serial

communications with something OTHER than a TNC!  (e.g., a telephone

modem)

(The upload directory is on another drive - nnn,nnn bytes free there)

When a REMOTE user asks for a directory display with the W, MW or YW

commands, paKet sends the file details from your Download directory

(that is your Send directory) followed by the space remaining on that

drive.

But if you happen to configure your Send and Receive directories on

different drives, the REMOTE user will not know how much free space you

have on your other (Upload) drive.  That might be relevant because if

he wants to send a file to us, he might want to know if it will fit.

So if that happens to be the way you have configured your directories,

paKet will send this message after the rest of the directory display so

the REMOTE user knows how much free space you have on the other drive

too.

The YU command is sufficient - there is no need to specify a file name.

If a REMOTE user wants to send a Binary File to us using the YAPP or pP

Protocol, there is no need to specify a file name with the YU command

because the Protocol includes details of the file being sent.

So if a YU command is received with a filename, paKet will send back

some friendly advice in the form of this message.

No harm done - paKet continues with the YU command, ignoring anything

else on the command line.

This Connected message is ignored because DCD line is LOW.

(Check the DCDCON option in Configuration/Serial Port window)

A "Connected" message has appeared in the Communications Window but has

been ignored because the DCD line is still low.

paKet uses the TNC's Connected message to determine when and to whom we

are connected.  Occasionally some text is monitored that looks like a

Connected message and paKet gets confused, thinking there is a new

Connection.

If the DCDCON option is ON in the TNC, paKet can check the DCD "light"

to confirm we really ARE connected now.

Page 211

If a Connected message is detected but the DCD line is still low, paKet

will treat that text as plain data and will display this message to

advise you of the decision taken.

Refer to the discussion on the DCDCON option in Configuration - Serial

Port options and refer to the <Alt-V> key for more discussion on this

issue.

This feature is not used if you are using Software Handshaking.

This file already exists.

A new file, (filename), will be created.

When receiving a file, paKet will check to see if the file already

exists and if it does, paKet will issue this warning message.

The original file will not be tampered with; instead paKet will create

a new file, using the same file Name but with a new Extension. It will

try to create a new file with an extension of .000 and if THAT file

exists, it will try .001 etc. When it finds a filename/extension that

does not already exist it will advise you of this filename in the above

message.

For example, if you are about to receive a file "TEXTFILE.DAT" and it

already exists, paKet will create a new file "TEXTFILE.000".

This is also done if you are receiving a Binary File with the pP

protocol and paKet decides a Recovery of an existing file is not

possible.  It will transfer the entire file to our disk using this new

filename.

This looks like a Script File Name!

The previous (Script) option is changed to Y

In the REMOTE/PMS Configuration you may specify the name of your home

BBS as well as details on how to establish connection with that BBS for

the purpose of Mail Forwarding.

In some cases the path to that BBS is complex, involving a number of

Network Nodes so a simple callsign and digipeater is not always

convenient.  So paKet has an option to use a Script file instead of a

"simple" connect path for this BBS connection.

You will see in that part of the Configuration an option to "Use Script

to make BBS connection?" and if that is set to N you should specify the

BBS callsign and optionally any digipeaters your system needs to

connect to that BBS.

But if you specify Y to that configuration option, paKet expects to

find a Script filename instead of the callsigns etc.   The correct

Page 212

syntax for specifying a Script file name is to include the file name

within pointer brackets, for example"

<BBSCONN>

If there is some conflict in the syntax, paKet will try to correct it

and alert you to the problem with the above message.

This new value will take effect next time you run paKet

After making some changes to paKet's Configuration you may see this

message to remind you that the changes you have made will not take

effect immediately.

There are some things that have to be accomplished at the beginning of

the program, for example the allocation of memory for buffers. So if

you want to change these things, you will have to exit and restart the

program for the changes to become effective.

This subdirectory is not empty!

When the Directory Window is displayed you may perform some file

management functions including deleting a file or sub directory.

You will get this message if you attempt to delete a sub directory that

in not empty.  You will have to go into that sub directory and delete

any files in there before we may delete the sub directory name itself.

Too many arguments

This message will appear if you have specified an invalid DOS command.

Check the command and try again.

Too many connect entries

There are too many entries in the Connect File and the program cannot

hold any more. At the time of writing there is provision for 200

connect entries.

Delete any unwanted entries and if you really need more, you could

establish more than one Connect File, specifying the chosen file name

in the Configuration as required.

Page 213

Too many files

This message will appear if you have selected a function that tries to

display the paKet Directory Window, and paKet finds more than 400 files

in this particular directory or subdirectory.

I have reserved space within paKet for handling directories containing

up to 400 files. If you have a directory or sub directory with more

than this number you will not be able to display that directory with

paKet's Directory windows.

I think the limit of 400 files is reasonable as there are certain

inefficiencies in DOS's handling of very large directories.

If you get this message, maybe it is time for a bit of housekeeping?

Too many PMS messages here.

I have allowed for a maximum of 200 current messages in your PMS. It is

supposed to be a PERSONAL System and I think 200 CURRENT messages is

more than ample for a Personal Message System.

Anyway, paKet can't hold any more so you will have to remove some of

these to make room for any further messages.

Too many sections in the Online Manual

This error message means there are more index entries in the

documentation file (the Manual) than I have allowed for.

It is unlikely you will see this message, because I have tested all

sections of the Manual before releasing this version.  If you do it is

because someone has modified the Manual since I released it.

Get a valid copy of the Manual from the original diskette or from your

backup copy.

Too many Traps in Script.

The Script file you have just loaded contains more than 30 Traps.

Script processing will continue with only the first 30 Traps active so

be warned it may not be doing quite what you want!

Refer to the section on Script Processing for details.

Page 214

TRAP in Script ignored.

You must specify 2 fields separated by comma!

The Script file you have just loaded contains a command beginning with

T which suggests a Trap command but the syntax is invalid.

Refer to the section on Script Processing for syntax details.

Type ahead buffer is full.

Press <Enter> to send it or <Ctrl-Del> to clear it

The Type Ahead buffer is limited to the number of lines visible on the

screen.  So if you have configured a two-line Type Ahead Window, you

have a buffer capacity of two lines.

When this buffer capacity is filled, paKet will not accept any further

characters and you must either type <Enter> to send that buffer to the

TNC or type <Ctrl-Del> to clear the buffer (without sending it).

Page 215

Messages starting with U, V and W

Unknown Script command

There is a limited number of commands known to the Script sub system.

They are mostly single character commands and are covered in the

section on Script processing.

You will get this message if paKet finds a line in the Script that does

not begin with one of the valid commands.

Unknown state in YAPP processing

The YAPP specifications provide for a variety of conditions which the

program must cater for but if there is a problem in the logic at some

stage this message could occur. It is really a program bug so I hope

you never see it!

If you do see this message, the current Binary Transfer will most

likely be unsuccessful. Try it again and if it still fails, you'd

better let me know so I can attend to it. If you could arrange a copy

of the file too that would help with my debugging.

Upload Cancelled

During a Binary File Transfer you might decide to abort the transfer by

pressing the <Esc> key.  paKet acknowledges that request with this

message.

Upload commenced

This message is displayed in a System Note at the beginning of a Binary

File transfer.

Upload Successful

Upload UNSUCCESSFUL

At the completion of a Binary File Upload, this message is issued in

a System Note to confirm the results.

If unsuccessful the message will often include a reason for the

failure.

Page 216

Uploaded Script file renamed with .SSS extension

This version of paKet has an option in the REMOTE Menu for a REMOTE

user to Perform a Script on our system.  The Scripts available to a

REMOTE user are those .SCP files in our Scripts directory.

A Script may contain a wide variety of commands including DOS commands,

so it is conceivable a Script could contain commands to do a lot of

damage to your system including Formatting disks etc!  So be careful

just what Scripts you make available to REMOTE users.   If you want to

keep all your Scripts in the same directory, rename the private ones

with an extension of something other than .SCP then the REMOTE users

will not be able to see them nor access them.  You can still run the

Script, regardless of its extension.

I was concerned that someone might want to create a mischievous Script

of their own, upload it to our system and Perform it here.   If we

allowed that to happen, it could do untold damage to our system.  Even

if their intention was good, we might still not want others to do just

anything they want with our computer system - they could accidentally

upset things.

You could arrange your directory structure so that your upload

directory was in another path to your Scripts directory, so any

uploaded files including Scripts would be out of the way and could not

be performed anyway.

But just in case... paKet will change ANY file being uploaded to this

system if it has a file extension of SCP, and modify it to SSS.  That

way, the Perform command will not recognise it and so it cannot be

performed by a REMOTE user until you change its extension back to SCP.

Using EMS Version n.n

(nnn KB used by paKet - nnn KB still available)

As the program starts up, it looks for any available EMS memory and

will use some of it for buffers and program overlays, if it can.

This message appears to let you know what the program is doing.

WAIT in Script needs time and a string separated by ,

The Script file you have just loaded contains a command beginning with

W which suggests a Wait command but the syntax is invalid.

Refer to the section on Script Processing for syntax details.

Page 217

We seem to be in Command mode.

REMOTE mode cancelled.

The TNC should not be in Command mode while paKet is in REMOTE mode.

If paKet is in REMOTE Mode it will be issuing the REMOTE Menu. If the

TNC is in Command mode the Menu will not be transmitted to the other

station, but will be taken by the TNC as some sort of "command" and

will be rejected with "What?" or "Eh?" etc.

REMOTE mode is now disabled so you can perform your TNC commands.

When you have finished with the TNC's Command mode, place the TNC back

into CONVERSE mode and, if you then want to issue the REMOTE Menu to

the other station, press <F3>.

We seem to have lost DCD. REMOTE cancelled.

paKet was in REMOTE mode and the DCD line goes low, indicating the

connection with the other station has been lost.

In normal circumstances paKet detects the "Disconnected" message when

the other station disconnects, and will automatically deactivate REMOTE

mode (if the other station has not logged off normally).

paKet will also monitor the state of the DCD line during REMOTE

operations because if you are using Hardware Handshaking, the DCD line

can be used by the TNC to indicate the other station is still

connected. So, if the DCD line goes low, paKet will disable REMOTE mode

and will issue this message to inform you of the action taken.

This will only occur if you have specified the use of the DCDCON option

in the Configuration - Serial Port options.

We seem to have lost DCD. Transfer cancelled.

During a Binary File Transfer paKet checks the state of the DCD line at

regular intervals to confirm we are still connected to the other

station.

Of course this happens only if you are using Hardware Handshaking and

if you have enabled the DCD option in the Serial Port options.

If the link fails during the transfer, it is possible the program will

not know about it and will continue to send the file to no one in

particular!   This precaution helps to avoid some embarrassment.

The above message is issued to let you know what the transfer has been

cancelled.

Page 218

Which Drive?

This question appears after you select "Change to another drive" from

the Directory Window.

Enter a valid drive letter, it may be either Upper or lower case and

the colon is optional.

Without the filesize I won't know when the transfer is finished

So, when your system has finished sending the file, type the following:

<CR>//WPRG OFF<CR>

(the <CR> is the Enter key).

When I receive that I'll close the file; then I'll send the Menu again.

These messages are sent to a REMOTE (BayCom) user when a Binary File

Transfer to our system is about to commence, if the size of the file

has been entered as 0 (zero).

This is telling him to type the "// WPRG OFF" command on a separate

line when his system has finished sending, because that is the

end-of-file string used by the BayCom system and so that is what paKet

will be looking for to close the file.

Without the filesize I won't know when the transfer is finished

You will have to close the file manually (with <Alt-F8>)

This is very similar to the previous message but this time it is not

the REMOTE user that has initiated the file transfer request.  You

have done it yourself with the <Alt-F8> key to transfer a file from a

BayCom system but you have entered a file size of zero.

This message reminds you it is up to you to close the file when you

think the transfer has finished. It is quite likely the BayCom system

will simply stop sending when the last byte has been transmitted -

there may be no indication from that station that the transfer is

finished.

Write Flashback buffer to disk

The LOG file is open - add to this LOG file? (Y/N)

If the log file is currently open when you select <Alt-W>, paKet will

give you the opportunity to write the extracted Flashback data to the

existing log file so you can keep it all together.

If you wish to do this, reply "Y".   If you reply "N", paKet will ask

you for details of the file that is to contain the contents of the

buffer extract.

Page 219

Messages starting with Y and Z

YAPP command must be YD or YU

This message could appear if you have specified a "Y" command in a

Script but the command is invalid.

Refer to the section on Script Processing for syntax details.

YAPP command must be YW, YD or YU.

If a REMOTE user types a command beginning with Y, paKet expects it to

be one of the "YAPP" (or Binary File Transfer) commands.

However, in this case the second letter was not W, D or U so the

program considers it invalid and sends this message back to the other

station.

You are not allowed access to that message

This message appears when someone tries to Read or Kill a message in

our PMS but that message is not addressed TO that station nor sent BY

that station.  In other words it is not THEIR message!

You can close the file manually with <Alt-F8> if you wish

When receiving a Binary File from a BayCom system, paKet may not know

when to close the file because the BayCom system may not indicate when

the transfer is complete.

If paKet knows the size of the file, it can close the file after that

number of bytes have been received but sometimes the file size is not

known.  In these cases it is up to the operator to close the file

manually by pressing the <Alt-F8> key.

This message pops up to remind you of this option if nothing has been

received from the BayCom system for a while.  I figured that if the

other station has stopped sending maybe it has finished?  However the

decision is yours and yours alone!

You may specify a DIRECTORY if necessary, but a DRIVE is NOT permitted

A REMOTE user may specify a sub directory name when accessing our

system to display the directory or to Download a file.  But for your

protection, I decided to disallow the ability to specify another drive.

Page 220

If a drive is specified in the command entered, this message will be

sent back to the REMOTE station.

You must specify the other station's callsign.

When a REMOTE user enters a message in our PMS, all that is required is

the S command because paKet will assume the message is for you unless

some other callsign is entered.

But when you enter a message into your own PMS, the program has no idea

who it is for so you have to enter a callsign with the S command!

Page 221

(This page intentionally blank)

Page 222

PART VI - REMOTE Mode

Overview

Description.

paKet's REMOTE Mode is a facility that allows other users to access and

use our paKet system, even while our system is unattended. A REMOTE

Menu will be issued to the other station and paKet REMOTE will respond

to any of the valid Menu options.

For your protection, the other station will have access only to the

directory or directories you specify. The rest of your system will be

both invisible and inaccessible to the other station. You may choose to

disallow all REMOTE operations; or you may disallow just file Uploads

or file Downloads. It is your system - you decide what the other guy

can see and do.  Refer to the section on "Configuration Windows -

REMOTE / PMS options" for details on choosing these options.

The operation of REMOTE Mode is somewhat similar to most popular BBS

systems currently available.  paKet has a PMS where the other station

can send and receive messages. Details of the PMS are covered fully in

its own section in this Manual.

While REMOTE Mode is active, the Status Window displays the word

"REMOTE" to the left of the time display.

Page 223

Activating REMOTE Mode.

REMOTE Mode can only be activated while we are connected to another

station.

There are three ways to put our system into REMOTE Mode:

1. Automatic

You can configure the system to automatically switch into REMOTE

Mode when another station connects to our system.  This includes

those times where a BBS connects to this system to conduct a Mail

Forwarding session.

The Automatic option is set in the Configuration Windows -

REMOTE / PMS options.

2. REMOTE Trigger

The other station can send the REMOTE Trigger code. This code

too, is configurable although most stations would use the

default which is the <Ctrl-]> (that is Ctrl key with

right-square-bracket or ASCII value 29).

Why such an unusual combination? Well it should be unusual and

unique otherwise we would find paKet switching into REMOTE Mode

unnecessarily! And most of the other codes were already being

used for something or were commonly used as special TNC command

values. <Ctrl-]> seemed to be about the only one left!

It was found in testing that some Commodore 64 systems cannot

send <Ctrl-]>. A DIGICOM user can send that code by pressing

<Ctrl-=>, that is: Ctrl and the "equals" key!  If you find some

users cannot manage to send the REMOTE Trigger I suggest the best

solution is to enable the Automatic REMOTE mode so they will get

the REMOTE Menu on connection and will not have to worry about

the Trigger code. For some it is easier to get out of REMOTE Mode

than to get into it!

The Trigger code must be sent as the first and only character on

the line if it is to be recognised by paKet as a REMOTE Trigger.

If you want to specify some other code for the REMOTE Trigger,

you can do that in the Configuration Windows - REMOTE / PMS

options.

If you intend to use this Trigger method to provide access to

your REMOTE system, I suggest you put something into the CTEXT

message to advise the other stations to press <Ctrl-]> (or

whatever) to activate REMOTE Mode.  The "Begin" and "End"

auto-commands can be used to alter this message according to

whether paKet is active or not.

3. You can press the <F3> key to switch into REMOTE Mode and send

the first REMOTE Menu to the other station.

Page 224

Leaving REMOTE Mode

There are four ways REMOTE Mode can be cancelled:

1. The other station can enter "B", which is the REMOTE Menu command

for "Bye" indicating the user is finished and wants to disconnect.

paKet will:

disable REMOTE Mode;

send "Logged Off" to confirm acceptance of the Bye command;

then issue the Disconnect command to the TNC.

2. The other station can enter "T", which is the REMOTE Menu command

for "Talk" indicating the user no longer wants to use the REMOTE

Menu, preferring to continue this session communicating with us

in normal communications mode.

paKet will:

send a message to the other station accepting the Talk command;

disable REMOTE Mode.

sound the "telephone" bell (if bells are enabled);

paKet will wait a while (usually about 30 seconds) to give you

time to respond to the other operator's call.  But if you are not

present, the program will advise the other station that you are

not "here" before returning to REMOTE Mode and reissuing the

REMOTE Menu.

3. You could press <F3> to manually switch the system out of REMOTE

Mode. The effect of this is similar to the other station sending

the "Talk" command.

paKet will send a message to the other station advising them they

are no longer in REMOTE Mode.

4. If the other station Disconnects or if the link between the two

stations fails (which causes a Disconnect in our TNC anyway)

paKet will automatically disable REMOTE Mode as part of the

Disconnect process.

Page 225

The REMOTE Menu - A to L commands

paKet REMOTE will issue the following Menu to the other station:

[paKet - VK2DHU] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

Each of these options is explained below.

The callsign shown will be the registered callsign for this copy of

paKet.  If you have not yet registered, the callsign is not shown.

paKet will then wait for one of those options to be entered by the

REMOTE operator.  Each of these options is discussed below.  If

something other than a valid option is received paKet will respond with

"*** Invalid Option".

Sometimes the program might issue lots of "*** Invalid option" messages

even without the other guy typing anything!   This can happen if you

have MCON ON and are monitoring other activity on the frequency.  If

this is the case, our computer might receive something OTHER than the

data sent by our REMOTE operator.  The program cannot know what is

typed by the other guy and what is monitored (etc) and if this happens,

it is inevitable the program will start sending those "*** Invalid

option" messages to the other guy who will be wondering what is going

on! So if you do happen to get (or send) some unwanted "*** Invalid

option" messages, check to see exactly what is coming in from the TNC,

not just what the other guy is typing.

The same situation might arise if you have ECHO ON in your TNC.  Then

when we transmit the Menu, the TNC will echo it back to the computer!

The computer doesn't know what was sent by the other guy or what might

have been generated from within the TNC.  All it knows is that this

string of "data" has just been received from the TNC so it will try to

process that as a REMOTE command.

Menu commands are:

A  - Abort a file transfer

This might be used by the other station if an ASCII text file

transfer is in progress and they decide the rest of it is not

required.  A quantity of data is probably already in the TNC's

buffer so that will be sent anyway.

This command can also be used to abort a long directory listing

(see the W, MW or YW commands below).

B  - Bye (logoff)

This option will disable REMOTE Mode and issue a Disconnect command

to our TNC to disconnect our contact with the other station.

Page 226

D  - Download an ASCII text file

This operation is a Download from the REMOTE operator's point of

view: an ASCII text file is being sent from our system to the

other station.

The REMOTE operator must send the required filename along with the

"D" command, for example:

D TEXTFILE.DAT

The file will be taken from our Text Send directory or one of its

subdirectories. If the desired file is in a subdirectory, the

directory path must be specified as well.  The Text Send directory

is specified in the Directories and Files Configuration.

I think an example is needed here. Suppose our directory tree

looks something like this and you have specified "C:\PAKET\TEXT\"

as our Text Send Directory.

C:\

���DOS

���PAKET

�  ���BIN

�  �  ���GAMES

�  �  ���MISC

�  �  ���SATS

�  �  ���UPLOAD

�  �  ���UTILS

�  ���TEXT               < These are the only directories

�  �  ���BASIC           < the Remote operator can see or

�  �  ���DOCS            < access with the D or W commands

�  �  ���MISC            < if C:\PAKET\TEXT\ is specified

�  �  ���HUMOUR          < as our Text Send directory.

�  �  ���UPLOAD          <

���UTILITY

Then as far as the Remote operator is concerned, our C:\PAKET\TEXT

directory, and its subdirectories BASIC, DOCS, MISC, HUMOUR and

UPLOAD are available. Our other directories such as DOS and

UTILITY are quite invisible and cannot be accessed.

So, if the file TEXTFILE.DAT exists in our C:\PAKET\TEXT

directory, the REMOTE operator can enter the command:

D TEXTFILE.DAT

and paKet will find (and send) that file.

If the Remote operator wants a file, LAFF.DAT, in our

C:\PAKET\TEXT\HUMOUR\ directory, the following command is

required:

D HUMOUR\LAFF.DAT

Page 227

An attempt to access another directory will result in an error

message being sent. For example if the Remote operator sent a

command:

D \DOS\READ.ME

paKet would innocently reply that it can't find that file, even if

that READ.ME file DID exist in our DOS directory!

If you do not want another station to transfer any of our files

you can disable this option - refer to "Configuration Windows -

REMOTE / PMS options" for details. The option would still appear

in the Menu but the REMOTE operator would get an error message if

it were selected.

H  - Help

?  - Help

The contents of the "REMOTE.HLP" file will be sent to the other

station if either of these commands are sent by the REMOTE

operator.

This file must be available in the default paKet directory as

specified in the Directories and Files Configuration.

A sample REMOTE.HLP file is supplied with paKet. This file is

plain ASCII text so you can change it to suit your own system. For

example if you do not wish to allow File Uploads, you might like

to remove that detail from your copy of the Help file.

I  - Information about this system

The contents of the "REMOTE.INF" file will be sent to the other

station.

This file must be available in the default paKet directory as

specified in the Directories and Files Configuration.

A sample REMOTE.INF file is supplied with paKet. This file is

plain ASCII text so you can change it to suit your own

circumstances. For example you might like to add some details of

your computer system, your TNC, your QTH, or any special interests

you have.

K  - Kill a message from paKet's PMS system

KM - Kill all "My" messages

The REMOTE operator may remove any message sent BY that station or

addressed TO that station.

Page 228

If one or more individual messages are to be deleted the command

must include the message number that is to be removed, eg:

K 5

It is permissable to enter more than one message number on the

same command line, separated by a space.  Eg:

K 5 7 22

Another option would be to enter "KM" which would remove all

messages addressed TO the REMOTE operator.

If possible, paKet will delete the message(s) from the PMS

Database and will advise the REMOTE operator of the result of this

operation.

Refer to the discussion on the PMS system in its own section of

this Manual for more details.

L  - List available messages in paKet's PMS Database

LM - List all "My" messages

Upon receipt of this command paKet will send a listing of messages

currently stored in our PMS Database, showing one message per line.

If 'L' is entered on its own, details of all messages in our PMS

will be sent.  Although all message headers are listed here, the

REMOTE operator will not be able to access someone else's Personal

message.

A selective listing may be produced by specifying a FROM or TO

callsign (or part of a callsign). To ask for a listing of messages

FROM a particular callsign, specify the "L<" followed by the

desired callsign.  Eg:

L< XX9ABC

For a listing of messages addressed TO a particular callsign use

"L>".  For example to list all messages addressed to VK9:

L> VK9

If 'LM' is entered, details of only the unread messages addressed

TO the REMOTE operator will be sent.

Refer to the discussion on the PMS system in its own section of

this Manual for more details.

Page 229

The REMOTE Menu - M (Baycom) commands

MD - Download a Binary file to a BayCom system

This operation is a Download from the REMOTE operator's point of

view: a binary file is being sent from our system to the other

station which presumably is using BayCom.

The REMOTE operator must send the required filename along with the

"MD" command, for example:

MD PROGRAM.EXE.

The file will be taken from our Binary Send directory (specified

in Configuration Windows - File/Directories) or one of its sub

directories. If the desired file is in a subdirectory, the

directory path must be specified as well.

For example, if you refer to the sample Directory Tree shown under

the REMOTE D (Download) command above, suppose you have specified

C:\PAKET\BIN\ as our Binary Send Directory and the required file

is USEFUL.COM in that directory.  The command required from the

REMOTE operator would be:

MD USEFUL.COM

If the file is in the C:\PAKET\BIN\UTILS\ sub directory, the

command would be:

MD UTILS\USEFUL.COM

If you were not in REMOTE Mode and were performing this transfer

via the <Alt-F7> key, paKet would ask which filename and directory

should be used on the BayCom system. But as this operation is being

performed by the remote BayCom operator, paKet won't ask that

question, leaving the BayCom operator to make his own decisions.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send the //WPRG command to the BayCom system

send the file

send the //WPRG OFF command to the BayCom system.

Refer to the section on the Binary File Transfer Window for details

of the display and also refer to the Keyboard Commands section of

this Manual, where the discussion on the <Alt-F7> key provides some

more detail on sending a Binary File to a BayCom system.

When the transfer is complete, paKet will return to REMOTE Mode

and the Menu will be re-issued to the other station.

If you do not want another station to transfer any of our files

you can disable this option - refer to "Configuration Windows -

REMOTE / PMS options" for details.

Page 230

MU - Upload a binary file from a BayCom system

This operation is an Upload from the REMOTE operator's point of

view: a Binary File is being sent to our system from the other

station which presumably is a BayCom system.

That file will be stored in our Binary Recv directory (specified

in Configuration Windows - Directories and Files). For your

protection, paKet will not allow the REMOTE operator to specify a

drive or directory.

The REMOTE operator must provide the filename and filesize along

with the "MU" command, for example:

MU PROGRAM.EXE,54257

We need the filesize because the BayCom system does not tell us

when the file transfer has finished. If the filesize is known

paKet can count the bytes received and close the file when they

have all been received.

If the filesize is not known, a 0 may be entered to allow the

transfer to continue but the file will have to be closed manually.

The BayCom operator can do this by entering the BayCom end-of-file

string:

<Enter>//WPRG OFF<Enter>

That is, the "//WPRG OFF" command must be entered on a line by

itself.  If that string is not entered, paKet will close the file

after 2 minutes of inactivity. If you are present, you can close

the file yourself with the <Alt-F8> key.

The preferred method is for the BayCom operator to specify the

filesize.

When the required information has been entered, paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send the BayCom //RPRG command;

begin the transfer;

Refer to the discussion on BayCom Binary File Transfers under

<Alt-F8> in the Keyboard Commands section in this Manual for

more details.

When paKet determines the transfer is complete, the file is closed

and the REMOTE Menu is re-issued to the other station.

If you do not want another station to send any files to our

system, you can disable this Upload option - refer to

"Configuration Windows - REMOTE options" for details.

Page 231

MW - What files are available in the Binary Send Directory

If the REMOTE operator sends a "MW" command, paKet REMOTE will

send a sorted list of filenames found in our Binary Send

directory, including the directory names of any subdirectories

found in that Binary Send directory.

If a list of files in one of those Binary Send subdirectories is

required, the REMOTE operator may add that directory name to the

MW Command. Eg:

MW UTILS

Processing for the MW command is identical to that for the YW

command discussed below. The REMOTE operator could enter the YW

command to get an identical response. However, the BayCom user has

Upload (MU) and Download (MD) commands so this MW command is

provided for consistency.

Page 232

The REMOTE Menu - P to W commands

P  - Perform a Script

This is a new facility introduced in paKet version 6 and allows a

REMOTE operator to run a Script on our system.  The scripts

available to the REMOTE operator are those files with an extension

of .SCP that you choose to include in your Scripts directory.  You

can nominate your preferred Scripts directory in the Directories

and Files Configuration Window.

The Perform command would normally be entered by the REMOTE

operator with the name of the desired Script.  Eg:

P SENDFILE

and paKet would load and run the SENDFILE.SCP Script.

Note it is not necessary for the REMOTE operator to enter the .SCP

extension as the program will automatically add .SCP to the

filename given.  In fact, for your protection no other extension

is permitted!   Why?

Well, a REMOTE Script is no different to any other paKet Script

and may include commands such as a DOS command to format your hard

disk!!!   Therefore, if a REMOTE operator tries to upload a file

with an extension of .SCP, paKet will automatically rename it to

.SSS, and until you decide to rename it to the normal .SCP

extension and place it in your Scripts directory, it cannot be

Performed by a REMOTE operator.  Without this protection, a REMOTE

user with a mischievous bent could upload a nasty Script and

Perform it without your knowledge or permission.

For added security, I recommend you choose a Scripts directory

that is well away from your File Receive directory so any uploaded

Scripts cannot be Performed until you check them out.

If you decide you want to allow someone to upload and Perform a

Script without you having to be present to rename the file, you

could include a Script (let's call it MYCOPY.SCP) in your regular

Scripts directory, which contained a single command such as:

!COPY C:\TEXT\UPLOAD\MYSCRIPT.SSS C:\SCRIPTS\MYSCRIPT.SCP

This is a Script that contains a DOS command to copy the uploaded

Script file (with its .SSS extension) into the regular Scripts

directory with a valid SCP extension.   Then when the REMOTE

operator uploads his new Script into your Text Upload directory,

he could Perform MYCOPY which would then make the new one

available to any REMOTE operators' Perform commands.

If the REMOTE operator enters the P command without a filename,

paKet will respond with a list of available Scripts in the Scripts

directory.

Page 233

R  - Read a message from paKet's PMS

RM - Read "My" messages

The REMOTE operator wants to read one or more messages stored in

our PMS Database.

If one or more individual messages are to be read the command

must include the desired message number.  Eg:

R 4.

It is permissable to enter more than one message number on the

same command line, separated by a space.  Eg:

R 4 5 22.

Another option would be to enter "RM" which would read all unread

messages addressed TO the REMOTE operator.

The REMOTE operator may read any message sent BY that station,

addressed TO that station, or any Bulletin.

Refer to the discussion on the PMS system in its own section of

this Manual for more details.

S  - Send a Personal message to paKet's PMS system

SP - Send a Personal message to paKet's PMS system (same as 'S')

SB - Send a Bulletin to paKet's PMS system

The REMOTE operator wants to leave message in our PMS.  This

message might be for you or for someone else and it could be a

Personal Message or a Bulletin.

If a callsign is provided on the command line, eg: S VK2DHU the

PMS will record the message as being addressed to that station but

if no callsign is given, paKet will assume the message is to be

addressed to you. Your callsign will be that specified in the

Configuration Windows - Miscellaneous options.

Although this command is "Send", the message is not sent anywhere

at this stage - it stays here and is recorded in our PMS

Database.

This version of paKet supports Automatic Mail Forwarding using the

MBL/RLI protocol (which is the universal standard supported by all

BBS systems). So, although the mail is not sent anywhere at this

stage, it may be sent to the BBS later for onward Forwarding.

The REMOTE operator may enter a TO address which includes a

destination as well as the callsign.  For example:

SP VK2DHU @ VK2ATM

or even a more detailed destination, using the well accepted

hierarchical system:

SP VK2DHU @ VK2ATM.NSW.AUS.OC

Page 234

If there is an '@' address specified, paKet will mark this message

as one to be Forwarded to the local BBS.  Once it is in the BBS

Network it may be sent across the world!  So if you (or anyone

else) want to get a message to someone across the other side of

the world all you have to do to is to leave it in your paKet PMS

with a TO address!

If there is no destination address, eg:

S VK2DHU

the message will remain in this PMS until it is deleted.

The message will be recorded as FROM the Remote operator's

callsign which appears in the Communications Header Line while we

are connected.

Refer to the discussion on the PMS system in its own section of

this Manual for more details, including information on Auto

Forwarding.

T  - Talk to the paKet operator

This option will disable REMOTE Mode, ring the telephone-style

connect bell (if those Bells are enabled) and return our paKet

system to normal communications.

If Bells are enabled, the telephone bell will ring up to 10 times.

Your first keystroke will stop the bells ringing.

If you are not present, paKet will tell the REMOTE operator you

are not here and will return to REMOTE Mode, re-issuing the REMOTE

Menu.

U  - Upload an ASCII text file

This operation is an Upload from the REMOTE operator's point of

view: an ASCII text file is being sent to our system from the

other station.

That file will be stored in our Text Recv directory (specified in

Configuration Windows - Directories and Files). For your

protection, paKet will not allow the REMOTE operator to specify a

drive or directory. On my system I use a sub directory called

UPLOAD and anything that is sent to me will appear in that

directory. Of course you may use any directory name you wish,

including the Root directory if that's what you want.

The REMOTE operator must provide the filename along with the "U"

command, for example:

U TEXTFILE.DAT.

Page 235

This file will be closed when paKet receives a <Ctrl-Z>.

If you do not want another station to send any files to our

system, you can disable this Upload option - refer to

"Configuration Windows - REMOTE / PMS options" for details.

V  - Version number of this system

When this option is specified, paKet will send the version number

of this paKet program.

It may be useful for the REMOTE operator to know which version we

are currently using as some versions of the program might behave

differently.

W  - What ASCII text files are available

If the REMOTE operator sends a "W" command, paKet REMOTE will send

a sorted list of filenames found in our Text Send directory,

including the directory names of any subdirectories found in that

Text Send directory.

The available space on that drive will be sent. If our Text Send

and Text Recv directories are on different drives, the available

space on the Text Recv drive will also be sent. This allows the

other operator to see how much space is available before uploading

a file to our system.

If the REMOTE operator wants to see a directory listing of one of

the subdirectories, the desired directory name must be entered

with the W command. For example, using the sample directory tree

shown in the REMOTE D (Download) command above, suppose the REMOTE

operator wanted a listing of our BASIC directory, the following

command would be required:

W BASIC

Page 236

The REMOTE Menu - Y (YAPP or pP) commands

YD - Download a Binary file using pP or YAPP protocol

This operation is a Download from the REMOTE operator's point of

view: a Binary File is being sent from our system to the other

station using either pP (paKet Protocol) or YAPP protocol.

The REMOTE operator must send the required filename along with the

"YD" command, for example:

YD PROGRAM.EXE.

The file will be taken from our Binary Send directory (specified

in Configuration Windows - Directories and Files) or one of its

sub directories. If the desired file is in one of those sub

directories, the directory path must be specified as well.

"Wildcards" or multiple file specifications (eg: *.EXE) may be

used to cause downloading of more than one file in the same

transaction, provided the remote station also supports this

feature.

For example, if you refer to the sample Directory Tree shown under

the REMOTE D (Download) command above, suppose you have specified

C:\PAKET\BIN\ as our Binary Send Directory and the required file

is USEFUL.COM in our C:\PAKET\BIN\ directory. The command required

from the REMOTE operator is:

YD USEFUL.COM

If that file was in our UTILS sub directory, the command required

from the REMOTE operator would be:

YD UTILS\USEFUL.COM

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

synchronise with the other station; then

begin the transfer(s).

Refer to the section on the Binary File Transfer Window for

details of the display and also refer to the "Keyboard Commands"

section of this manual, where the discussion on the <F7> key

provides some more information on sending a Binary File.

When the transfer is complete, paKet will return to REMOTE Mode

and the Menu will be re-issued to the other station.

If you do not want another station to transfer any of your files

you can disable this option - refer to "Configuration Windows -

REMOTE / PMS options" for details.

Page 237

YU - Upload a binary file using pP or YAPP protocol

This operation is an Upload from the REMOTE operator's point of

view: a binary file is being sent to our system from the other

station using either pP (paKet Protocol) or YAPP protocol.

That file will be stored in our Binary Recv directory (specified

in Configuration Windows - Directories and Files). For your

protection, paKet will not allow the REMOTE operator to specify a

drive or directory.

It is not necessary for the REMOTE operator to enter the filename

with the YU command because that information is included in the

Protocol Header.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

synchronise with the other station; then

begin the transfer.

When the transfer is complete, paKet will return to REMOTE Mode

and the Menu will be re-issued to the other station.

Refer to the section on the Binary File Transfer Window for

details of the display and also refer to the "Keyboard Commands"

section of this Manual, where the discussion on the <F8> key

provides some more information on receiving a Binary File.

If you do not want another station to transfer any files to this

system you can disable this option. Refer to "Configuration

Windows - REMOTE / PMS options" for details.

YW - What files are available in the Binary Send Directory

If the REMOTE operator sends a "YW" command, paKet REMOTE will

send a sorted list of filenames found in our Binary Send

directory, including the directory names of any sub directories

found in that Binary Send directory.

The available space on that drive will be sent. If our Binary Send

and Binary Recv directories are on different drives, the available

space on the Binary Recv drive will also be sent.

If the REMOTE operator wants to see a directory listing of one of

the sub directories, the desired directory name must be entered

with the YW command. For example, using the sample directory tree

shown in the REMOTE D (Download) command above, and assuming you

have specified C:\PAKET\BIN\ as the Binary Send Directory, suppose

the REMOTE operator wanted a listing of our GAMES directory. The

following command would be required:

YW GAMES

Page 238

Other REMOTE commands

*

; -  Any "command line" beginning with this asterisk (some call it a

star) or a semi-colon will be treated as a comment.  Although it

will be displayed on our screen, it will not be processed by paKet

REMOTE.

This is useful if you are present at the keyboard while the system

is in REMOTE Mode.

You might want to chat briefly to the other station and this

allows you to receive a message without having to return to normal

communications mode.  If the REMOTE user does not prefix his

message with this '*' or ';', paKet will try to interpret the line

as a valid command and will probably spew out that "*** Invalid

option" message!

The following "commands" are recognised by the REMOTE system but these

commands are used internally and not expected to be sent by a human

operator.

> -  BBS Prompt for Mail Forwarding

This character is used for paKet's Automatic Mail Forwarding. It

is sent by the other station's Auto-Forwarding logic (usually the

"other station" in this case is your local BBS) and triggers a

response from paKet's Mail Handling facilities.  Unless it is a

very experienced operator at the other end, it is unlikely this

code would be deliberately typed in manual mode.

I suggest you leave it alone and let the computer look after it.

[ -  First character of a System Identifier (SID) which is used by

BBS and other compatible systems in establishing Auto Forwarding

protocols.

This is not something that would normally be entered manually by a

REMOTE operator and as suggested above, leave this one alone and

let the computer worry about the details.

When a SID is received while we are in REMOTE Mode, paKet will

enter MAIL mode and will expect the other station to respond with

the standard MBL/RLI mail forwarding protocol.

For your information, our paKet SID is "[paKet-6.1-HM$]" and will

be issued as necessary when Mail Forwarding is initiated.

If you wish to automatically receive mail from the BBS, you should

have this SID text as the first line of your CTEXT in your TNC and

arrange with your BBS Sysop for your callsign to be included in

his Forwarding tables.

Page 239

(This page intentionally blank)

Page 240

PART VII - SCRIPT PROCESSING

Overview

paKet includes a simple Script processing facility. The key word there

is "simple".  Script is a special language that paKet interprets,

allowing it to process a pre-written set of commands.  The concept is

the same as the BATCH facility in DOS where we can store a sequence of

DOS commands in a BAT file.

A Script file contains one or more Script commands and usually has a

.SCP extension.  A Script is written in plain, ASCII text so you can use

any text editor to create or modify a Script file.

It is not a complex language and it is certainly NOT a sophisticated

compiler.  The degree of error checking is not to be even compared to a

formal language compiler or assembler. I am relying on you to check out

your own Scripts and get the syntax correct.  If paKet detects something

strange it usually gives you enough information to help fix the problem

but occasionally it might just abort the Script where the problem is

detected.

In practice it is not as bad as it sounds.   Most Script commands are

rather simple, so any problems are usually pretty obvious when they

occur.  Start out with some simple Scripts, to get the feel of it and

then you can develop more complex Scripts as you gain experience.  Many

paKet users have written Scripts without problems so I am sure you too

can join the ranks of successful paKet Script-writers.

The Script facility should be sufficiently flexible to allow a variety

of operations, given a little imagination.  So if you do develop a

"good" Script file of your own, why not spread it around?  Broadcast it

to other users (I suggest a Bulletin addressed to PAKET rather than to

ALL would attract the attention of paKet users). I would appreciate it

if you could let me have a copy too, in case I miss your Bulletin.

In this section of the Manual, we will be looking at some examples in a

Tutorial format to see what can be done with Scripts and to demonstrate

the syntax required.  You might like to have a look at these and then

with the ideas offered here you can go create some of your own.  Or even

easier, take one that NEARLY does what you want and alter it to suit. It

doesn't matter, you might not even use Scripts at all, but I am sure you

will find it challenging and fun, even creative. At least give it a try.

Various Script commands will be introduced as we examine the sample

Scripts.  And there is a full command reference at the end of this

section of the Manual.

A final word, before we get into the detail... it is probably obvious to

you by now but just in case, let me state quite clearly:  you do NOT

need to use Scripts if you don't want to.  It is NOT necessary to even

read this section of the Manual.  This is one of the more advanced

features of paKet so if you feel a bit threatened or confused by all

this talk of Scripts and Programming, just take a Valium and skip to

another part of the Manual.

Page 241

Running a Script File.

Press <Alt-S> to commence processing a Script you have available.

A Directory Window will appear to help you select the wanted Script.

Highlight the desired Script filename (selecting another directory if

necessary) and press <Enter>. paKet will load and run that Script file.

I recommend you keep all your Scripts in their own directory.  So when

the Directory Window appears, it will be easy to find and select the

Script you want.  It will be easier to manage too if you use the .SCP

extension for all your Scripts as paKet recognises this by default.

The word "SCRIPT" will be displayed in the Status Window to the left of

the time display while the Script is running.   When the Script has

completed, the indicator in the Status Window will disappear and you

will be returned to normal communications.

Page 242

Script Tutorial - the S command

To start with, let's look at a very simple Script:

S MCON OFF

S CONOK ON

In this section of the Manual I will indent the sample Scripts to

help separate the Script from my comments.  But when you are writing

your own Script, begin your Script commands in the first character

position of the line.

This first Script file consists of just two lines, two Send commands.

Yes, the "S" is a Script command to "Send" the rest of that line to the

TNC.

Most Script commands are single letter commands.  In the case of the S

command there is one parameter - that is the string which occupies the

rest of the line, starting from the first non-blank character.  Oh, I

should mention too that the Script commands may be typed in either

upper or lower case, so the Send command can be "S" or "s".

When you run a Script, paKet will always start at the top, the first

line.  In this example, paKet will:

- process that first Send command by sending the text "MCON OFF" to

the TNC.  It will automatically generate and send <Enter> at the

end of each Send line.

- it will drop down to the next line.  This is another Send command

so the text "CONOK ON" will be sent to the TNC, followed by the

<Enter>.

- as there are no more lines in the Script,  paKet will terminate

Script processing and return to normal communications mode.

That's not too hard now, is it?

Review

In this part:

We introduced the S command.

This command:

The S command is used to Send text to the TNC.

The syntax required:

S text

eg:

S CONVERSE

s Hello. My name is Tony.

Page 243

Practical exercise

OK, that's enough theory to get us going.  You already have enough

information to create and run a Script!   So, let's try a practical

exercise.

We are going to create a Script file, then we are going to run it.

Before we start we need to establish a few prerequisites:

I am assuming here that you have a text editor available and that

you know how to use it.

And I am assuming you have configured your paKet system so there is

a Scripts directory somewhere on your disk.  If you have not yet

done that, press <F9> now to get the DOS prompt and Make a new

SCRIPTS directory.  Then as we continue with this Tutorial, the new

Scripts you create can go into that new directory.

Check your Directories and Files Configuration (<Alt-Z> then 8) to

verify you have specified that path as your Scripts directory.

Now, using your text editor, create a new file in your Scripts

directory (use paKet's <Alt-E> command to call up your editor).  Name

it FIRST.SCP and type in the two lines of S commands as shown in the

example above.  Don't forget it is better if you commence typing these

commands in column 1.  Save the file and return to paKet.

You have created your first Script!

To run this Script, press <Alt-S> and you will see paKet's Directory

Window pop up.  If that is not showing your Scripts Directory, change

to that directory before continuing. (Normally paKet will display the

directory you used last time you ran a Script, but if this is the first

time, you might have to go find it yourself).

Now that we have your SCRIPTS directory on the screen you should see

your file FIRST.SCP in that directory.  To run that Script, all you

have to do is select that file.  That is, move the highlight bar to

that file and press <Enter>.

paKet will load and run that Script and you will see the two commands

appear on the screen as they are sent to the TNC and you will probably

see the TNC's responses.

When that is finished you are back in paKet's normal communications

mode.   Easy huh?

Page 244

Script Tutorial - the B and D commands

When you ran that first Script you might have noticed a couple of

problems.

Firstly, we are sending some commands to the TNC, but what if the TNC

was not in Command Mode to start with?  if we were unlucky we could be

sending "MCON OFF" etc to the radio for transmission and the whole

world will be watching our mistake!

So it is a good technique, when planning to send a TNC command, to

ensure our TNC is in Command Mode to start with.  If we were doing this

manually we would type <Alt-B> at the keyboard to send a BREAK code to

the TNC.  In a Script, we can use the "B" command to do the same thing.

So let's add the following line to the beginning of our Script:

B

Just a single character on that line.  That's all we need here because

the B command has no parameters.

You will notice when running that Script, that paKet will send the

first command (MCON OFF) followed almost immediately by the second

command.  This is OK and the TNC will accept our second command even

though it has not yet had time to process the first one.  But we will

see our second command appear on the screen before we see the TNC's

response to the first one.  It does not look nice and tidy on the

screen - it's out of sequence.

With just two Send commands it probably doesn't matter too much... we

could cope with that, even if it doesn't look too good.  But if we had

ten or twenty or more commands to send, I think it would be worth the

trouble to add some delays to our Script.

One way to add a delay is the "D" command.  This command will delay for

a specified number of seconds. So if we add:

D 3

to our Script after each of the commands, paKet would delay for 3

seconds before proceeding to the next line.  That 3 second delay should

be sufficient time for our TNC to process the previous command and send

its acknowledgement to paKet's display.

So our first Script would now look something like:

B

D 3

S MCON OFF

D 3

S CONOK ON

Page 245

Now, our Script will issue a BREAK code to the TNC to ensure we are in

Command Mode and delay for 3 seconds to give the TNC time to do that.

Then after sending that first command it will wait for 3 seconds before

sending the second command.  That is an improvement on our first

attempt, although it takes longer to run.

Review:

In this part:

We introduced the B and D commands.

These commands:

The B command is similar to the <Alt-B> command we can type on the

keyboard.  It ensures the TNC is in Command Mode.

The D command causes a delay in the Script processing for a

specified number of seconds.

The syntax required:

for the B command:

B      (just a single character -

there are no other parameters required)

for the D command:

D seconds

eg:

D 3

D 60

There is another form of the D command we have not used here.  This

form allows you to delay UNTIL a certain time of day.  To do that

you type the D command as follows:

D hh:mm

for example:

D 21:45

which tells paKet to delay until 9:45pm before continuing.

Practical exercise

Modify FIRST.SCP to include these additional Script commands and run

it again by typing <Alt-S>.

Notice the delays and the effect they have on the screen display?

Page 246

Script Tutorial - the W and Q commands

The D command is a fixed delay - it will always wait for that same

period of time.  So if we used that method of separating our commands,

we would find the Script taking more time than is really necessary,

because most commands would not take the full 3 seconds to process.

You could try a shorter delay to speed things up but sometimes we might

need an even longer delay, for example if waiting for a response from

another station.  A fixed delay just does not suit.

There is a better way!   The "W" command will Wait for a period of

time, looking for a particular string of characters.  So if we include

the following line after sending a command:

W 10,cmd:

the Script will wait for up to 10 seconds, and while waiting it will

be looking for the string "cmd:".  If the 10 second period expires

before that string is received, paKet will continue processing from the

next line in the Script.

On the other hand, if that string ("cmd:" in this example) is detected,

paKet will immediately cancel the wait, skip the next line in the

Script and continue processing from the following line.

Note the string you specify in the Wait command does not have to be a

full line as received from the TNC.  paKet can match any part of the

incoming data. So you could have:

W 20,VK2DHU

and the Wait would be satisfied if paKet receives:

*** CONNECTED to VK2DHU

When you write a Wait command, be sure to specify a time delay that is

much longer than the time you would reasonably expect for the string to

appear.  Processing will continue immediately the string is detected so

in normal circumstances you will not have to wait for the entire period.

In the example we are using here, we are waiting for the "cmd:" prompt

to come up after we send a command to the TNC.  So a 10 second delay is

more than enough, in fact we could make that 5 seconds and still be

quite comfortable.  So, if the Script times out after that period,

there must be something wrong.  When that situation arises, I generally

want to abandon the Script and go look to see what caused the problem.

The "Q" command tells paKet to Quit the Script.  This command has no

parameters.

So, let's modify our FIRST.SCP again to replace the 3 second Delay with

a Wait and a Quit command, so it looks as follows:

Page 247

B

W 5,cmd:

Q

S MCON OFF

W 5,cmd:

Q

S CONOK ON

When we run this one, paKet will issue the Break command and wait for

the TNC to respond with its "cmd:" prompt.  If that does not appear

within 5 seconds, processing will drop down to the next line where the

Q command will abort the Script.   If your TNC is one that does not

produce a "cmd:" after the Break, you will wait for the full 5 seconds

before the Script aborts!  In your case, remove the W and Q commands

after the Break and go straight on to the first Send command.

But in normal circumstances, the TNC will respond with its "cmd:"

prompt and, with the Wait satisfied, the Script will immediately skip

the Q command on the next line and continue processing our S command.

Then it will wait again for another "cmd:", and so on...

If you wanted to, you could add another Wait (and Quit) command after

the last Send but that is not really necessary here in this example.

Review:

In this part:

We introduced the W and Q commands.

These commands:

The W command is a selective Wait. There are two parameters here,

separated by a comma.  We must specify a maximum time to wait (in

seconds) as well as specifying the string to wait for.

The Q command is a single letter command to Quit the Script.

The syntax required:

for the W command:

W seconds,text

eg:

W 30,CONNECTED to

W 120,messages waiting

for the Q command:

Q      (just a single character -

there are no other parameters required)

Practical exercise

Change FIRST.SCP again and replace the D command with the W and Q

commands as shown.

Run it again and watch how this Script is performed more quickly

than the previous one.  You can see why the Wait command is to be

preferred over the standard Delay command for this type of

application.

Page 248

Script Tutorial - the G and : commands

In modern Programming Schools, the humble GOTO statement is frowned

upon.  But it is alive and well in paKet Scripts.  (As it is in DOS's

BATCH facility)

Usually paKet will move through the Script from beginning to end.  It

starts with the first line, then moves to the second line, then the

third etc until it reaches the last line of the Script.  A "G" (Goto)

command allows us to vary that sequence, telling paKet we now want it

to proceed from somewhere other than the next line in the Script.

To use a Goto command we need to have a "Label" somewhere in the Script

and in the Goto command we specify the name of that label. The Label is

any line beginning with a colon (":").  Consider the following example:

B

:start

S CONVERSE

S This is my regular 15 minute Beacon message.

B

D 900

G START

In this example we have:

- A Break command so we know we are starting in Command Mode.

- The second line is a label because it begins with a colon.

This line has no effect on the processing, it is just used as a

marker for the G command we will come to shortly.

- It sends a CONVERSE command to the TNC (we were in Command Mode

don't forget) followed by a one-line message, before returning to

Command Mode again with another Break code.

- The Script will delay for 900 seconds (that is 15 minutes).

- Then we come to a Goto command which tells the Script to adjust

itself so its current line is the line where we wrote the ":start"

label.  That means it will send another CONVERSE command etc etc

and do it all again.

This Script will go forever - there is no way out.  It is what

programmers call a "loop".  Of course you can press <Esc> at any time

to cancel a Script so you are still in control here.

Note I used lower case when writing ":start" and upper case when writing

"G START".  I did that deliberately to demonstrate paKet is not fussy

about the case for the label.  If you spell it the same it will be OK.

A label would usually be one word because that is how most programming

languages use labels.  But here a label can be an entire line if you

want.  You just have to ensure the Goto command specifies that full

string if you want it to match.

Page 249

Review:

In this part:

We introduced the G command and Labels (":").

This commands:

The G command is a Goto.  The Goto command must specify the label

which identifies the point in the Script where processing is to

continue.

A Label is any line beginning with a colon (":").

The label is simply used as a marker which may be specified in a

Goto command.   paKet does no processing with a Label.

The syntax required:

for the G command:

G text       (where "text" is a Label in this Script File)

eg:

G START

G We are now connected

for the Label:

: text

eg:

: START

: We are now connected    --- Note the leading blank is ignored so

:WE ARE NOW CONNECTED     --- these labels are considered "equal"

Practical exercise

Create the Script as shown in this section, call it SECOND.SCP and

store it in your Scripts directory.

Feel free to change the "Beacon" text to anything that suits you and

for testing you could reduce the 900 secs to 5 secs, just to make it

quicker to test your Script when you run it.

Watch your TNC switch from Command Mode to Converse Mode and back

again after it sends the Beacon Text, and note the indicator

"SCRIPT" is showing in the Status Window while the Script is running.

Press <Esc> to cancel the Script when you want to stop it.

Page 250

Script Tutorial - the  * and E commands - documenting your Script

You could used a Label as a comment in your Scripts.  As I mentioned

above, a label has no effect on the processing so we can add several of

these "labels" to our Scripts as a form of documentation. For example

we could modify the example Script, SECOND.SCP, we were using here to

look something like:

: Let's start with a BREAK code to ensure we are in Command Mode

B

:--------------------------------------------------------------

: The following line is the START label - the start of our loop

:--------------------------------------------------------------

:start

:--------------------------------------------------------------

: Set CONVERSE Command, send a message, return to Command Mode

S CONVERSE

S This is my regular 15 minute Beacon message.

B

:--------------------------------------------------------------

: Delay for 900 seconds which is 15 minutes

D 900

:--------------------------------------------------------------

: Now go back up to the "start" label to send another beacon

G START

Yes OK, I might have gone a bit overboard here with the comments, but if

I had not explained the various commands or the concept of a loop, this

Script might have seemed confusing to some people.  In Scripts and in

conventional programming, I like to see plenty of comments.  If it is

too difficult to easily explain in a comment, there is something wrong

with your logic!  The addition of comments can make a Script more easily

understood, especially if you send a copy of your Script to someone else.

A paKet Script file is limited to 200 lines.  I decided to impose some

limit to make it easier for paKet to manage the processing.  We can get

around that limit by using the Call command which we will see a little

later.  But I mention it here because each Script command line,

including a Label, is counted as one line.   If you do happen to be

writing a large Script and are approaching the 200 line limit, you

should be aware of this and reduce the number of unnecessary "labels".

* - a real comment.

May I introduce the "*" command?   We didn't have this in earlier

versions of paKet, but this one is a genuine comment - it's not really

a command at all.  Although a Label does not actually DO anything, it

still must be included in case we need it for a Goto command.

But a comment line, that is any line beginning with an asterisk ("*"),

is not counted as part of the Script when it is loaded into memory.  So

you can have as many of these comment lines as you like without

carrying any overhead in the Script.  Obviously this is to be preferred

to using Labels for documenting your Scripts.

Page 251

So, in the previous example it would be better if you replaced the

colon (":") with an asterisk ("*") in every one of the Label/comments -

except of course the REAL label, the ":start" label!

E command

Labels and comments are visible in the Script file when you edit or

type the file, but while the Script is running they are not seen.

The E command is documentation with a difference!   The E command will

Echo to the screen all the text on this line, so you can use this as a

form of documentation which is always seen by the paKet operator.  The

information appears on the screen in the Communications Window as if it

was received from the other station. It will also appear in your log

file if it happens to be open at the time.

This command could be used to:

- inform the paKet operator of progress through various stages of

processing as the Script is performed (especially if it is a

complex Script taking some time to complete), or

- it could be used to display some operator instructions at the

beginning of the Script, or

- it could be used simply to Jazz Up your Script to make it look

more impressive.

Let's have another look at our SECOND.SCP, this time with some

different "documentation"...

*********************************************************

*                      SECOND.SCP                       *

*                                                       *

*  This Script will transmit a "Beacon" message every   *

*  fifteen minutes.  The text of the Beacon is as shown *

*  here in this Script.                                 *

*                                                       *

*  The 15 minute period may be changed by altering the  *

*  D command to a different value.                      *

*********************************************************

*

E Running Script file:  SECOND

B

:start

*****************************************

*   This part is done every 15 minutes  *

*****************************************

S CONVERSE

S This is my regular 15 minute Beacon message.

E

E      +----------------------------------------------------+

E      | Press <Esc> if you want to abort the Beacon Script |

E      +----------------------------------------------------+

E

B

*         Note - 900 seconds is the same as 15 minutes

D 900

G START

Page 252

Here we see the comments using the asterisk now.  This is better than

using Labels for comments because when the Script is loaded into memory

for processing, these comment lines are ignored - they are not even

loaded into memory with the rest of the Script.  Yet they remain in the

Script file on disk so we have the benefit of that documentation

without any overheads or limitations.

The Echo lines will be displayed on the paKet screen as the Script is

running.  I have added a couple of blank lines and a "box" around the

message that is Echoed each time around the loop - just trying to be a

bit fancy!

I think this is sufficient to demonstrate the concepts of commenting

your Scripts, both with the genuine comments and with Echo commands.

It DOES make a difference, I think.

Review:

In this part:

We introduced the E command and Comments ("*").

These commands:

The E command is a command to Echo text on the display while the

Script is running.  The rest of the line is displayed as if it was

received from the other station.

A Comment is any line beginning with an asterisk ("*").

Comment lines are not loaded into memory when running a Script so

you can add as many Comments as you like to the Script File on disk

without affecting the performance of the Script.

The syntax required:

for the E command:

E text

eg:

E We are starting the SECOND Script File ...

E The TNC should be back in Command Mode by now.

for the Comments:

* (text)       (The asterisk is required but text is optional)

eg:

* This Script was created by Tony Lonsdale on March 8th

* The time delay could be increased if the frequency is busy.

Practical exercise

Edit our first Script file, FIRST.SCP, and add some comments plus an

Echo command or two.

Add a MYCALL command to send your callsign to the TNC.

Run that Script again and observe the effect of the Echo commands.

Page 253

Script Tutorial - the C and R commands

I mentioned earlier that a Script file is limited to 200 lines, not

counting Comment lines.

If you have a need for more than 200 lines, or if you have a more

complex "application" you could structure your Script processing into

modules, each of which may be up to 200 lines long.

Then you can use the "C" command to call another Script.  Suppose we

had three Script files, PART1.SCP, PART2.SCP and PART3.SCP and we

wanted to run them all in sequence as one single task.  We could create

another Script, let's call it MAIN.SCP as follows:

E Beginning the 3 part series...

C PART1

E Part 1 completed

C PART2

E Part 2 completed

C PART3

E Part 3 completed

In this example, we run the MAIN Script.  I have added a few Echo

commands, to demonstrate its use in a more complex Script.  When MAIN

is running the first thing it does (after the Echo) is to Call

PART1.SCP.  paKet will load and run the PART1 Script file, then it will

return to MAIN where it left off... this time dropping down to the next

Echo then to another Call command which is the Call to PART2.  That one

is loaded and run, and then back in MAIN, paKet will do PART3 before

the MAIN Script is completed and control is returned to paKet's normal

communications mode.

So you can see we can overcome the 200 line limit by dividing our

really long Scripts into two or more smaller ones and use the Call

command to perform them all.

There is no practical limit to the number of nested Scripts (depends on

the amount of memory you have available).  So in our example, the PART1

Script file could Call another Script, say PART1A. And that PART1A

Script could call yet another one called ummm, PART1B. In every case,

when a Script is completed it "Returns" to the one that called it, and

if we are back to the highest level (or if we had only one level to

start with), control returns to paKet.

Return

We have seen that when a Script is completed, that is when the last

line has been processed and there is no more to do, that Script is

terminated.   Control is returned to the calling Script if any, or to

paKet.

There may be times where we would like to "Return" before we reach the

end of a Script.  The "R" command will do this for us.  Let me

demonstrate with an example:

Page 254

E

E  **********************************************************

E  *                This is the BBS Script                  *

E  *      This Script establishes contact with the BBS.     *

E  *                                                        *

E  *         Once it has established that contact,          *

E  *      it returns to paKet or to the calling Script.     *

E  *                                                        *

E  **********************************************************

* Send a BREAK to the TNC to get into Command Mode

B

* Now, SEND a CONNECT command

*  NOTE  change this Send command to use your own BBS's callsign

S C VK2ATM-1

* ... and wait up to 20 seconds for the TNC's CONNECTED message

W 20,CONNECTED to

G noanswer

* We are now connected, so we can leave this Script now!

R

:noanswer

E *** The BBS did not respond within 20 seconds ***

Q

I have inserted some blank lines in this example.  These, like the

comment lines will be ignored when the Script is loaded into memory so

if you think it makes your Script File easier to read with some

additional spacing, feel free to add them to your Scripts.  It will not

cost you anything, except in very rare circumstances it might take a

little more disk space.

This Script is using commands we have already covered.  So, if you are

still reading this part of the Manual, you should be able to figure out

what the Script is doing by now.  If you are not still reading, then

... uh no, we won't go into that!

This Script might be called from another Script or we can call it up

from the keyboard with the <Alt-S> command.

In the example shown above I have a simple connection to my BBS so the

20 seconds I allowed should be quite adequate there.  If your system

requires a more complex connect path, you might need to make that

somewhat longer.

In this example, if we receive the usual acknowledgement within the 20

second period, the Script will come down to the "R" command and

terminate.   The job is done!

So the R command allows us to return from any point in our Script.

Page 255

Note the difference between a Return command and a Quit command which

we use if the BBS does not respond.   A Quit is usually used when

something is wrong and we just want to get out of here.  It will cause

an error message to be displayed and the entire Script process will be

aborted.  So if you are running in a nested Script, a Quit command will

not only cancel that Script but any others that might have called it.

You are back in paKet.   Unless there is a problem, a Return command is

a tidier way out and is the preferred way of leaving a Script.

Return Code

The Return command can have an optional Return Code.  This could be

used to indicate to the calling Script the results of a particular

operation.  Let's have another look at the above example. Instead of

the Quit at the end, we could issue a Return command with a return

code.  For example:

.

.

.

:noanswer

E *** The BBS did not respond within 20 seconds ***

R 1

The "R 1" will return to the calling Script just like any other Return

command, but this one will also take with it a return code of 1.  If no

return code is specified, paKet assumes a return code of 0.

Then a calling Script could look at this return code and determine

whether we are connected to the BBS or not.  If the code is 0 it knows

everything is OK but if it is 1 it might decide to Quit or take some

other action?

At this stage, just keep in mind that you can add a return code to the

Return command.  When we discuss Variables later, we will see how it is

used in the calling Script.

Review:

In this part:

We introduced the C and R commands.

These commands:

The C command will Call another paKet Script file.

The name of the other Script is given on the Call command line.  If

that Script is not in the normal Scripts directory, the Call command

can specify full path details so the program can find that Script

file.

Page 256

The R command is Return command.

When this command is performed in a Script, that Script is

terminated and control is returned to the calling Script or to

paKet, as appropriate.

The syntax required:

for the C command:

C Script-file-name

eg:

C PART1

C C:\TEMP\ONCEONLY.SSS

for the R command:

R return code     (The return code is optional)

eg:

R

R 55

Practical exercise

Create a new Script, BBS.SCP, from the example given above.  Use

the first example (with the Quit command) because we have not yet

fully covered the handling of Return Codes.

Test it by pressing <Alt-S> and establish contact with your BBS.

Don't forget to change the callsign, and add any other path details

required for your own BBS.  You might need more than 20 seconds too.

This BBS Script will be used in the exercise for the following

section.

Page 257

Script Tutorial - the AD, AU and AC commands

File Transfers

Here for the first time we see some two letter commands.  These are

ASCII file transfer commands.  An ASCII file, by the way, is a file

containing plain, printable text.

"AD" is an ASCII Download command, and we must specify on the command

line the name of the file we want to create.

When this command is performed by the Script processing, paKet will open

the file we nominate.  It works in a similar manner to the <F6> key so

now everything received in this Stream will be written to that file.

That file will be closed if we receive a <Ctrl-Z> code from the other

station, but we can also close it with an "AC" Script command.

If the named file is already present on our disk, paKet will not change

that existing file but will create a new file with a different

extension, starting with .000 but incrementing that number until it

finds a filename that does not already exist!

Once the file is opened and ready to receive data, processing continues

from the next line in our Script.

The "AU" command is an ASCII Upload command, and once again we must

specify on the command line the name of the file we want to send.

When this Script command is processed, paKet will open that file and

send the entire contents of that file to the TNC.  When the last byte

has been sent, the file is closed and paKet continues Script processing

from the next line.

What's this?  You want more examples?

Hang on, I just want to mention the other type of file transfer -

Binary File transfers.

YD and YU commands

------------------

These commands are the Binary File Transfer commands.  The name is

taken from the original Packet Radio file transfer protocol, YAPP.  So

these commands refer to "YAPP Download" and "YAPP Upload".  Of course

paKet can use the advanced pP Protocol too and it will automatically

select the advanced protocol if necessary. You don't have to worry

about that - let paKet take care of it.

A Binary File can be any type of file at all.  You can use these

commands to send and receive plain ASCII text files if you wish, but

they are more commonly used to transfer files which contain non

printable codes; files such as programs (.COM and .EXE files) or any

archived file (eg: files with .LZH or .ZIP extension).

Page 258

The use of these commands is very similar to the ASCII file transfer

commands.  The process is also covered fully in the Windows section of

this Manual - see the part on the Binary File Transfer Window.  Also

have a look at <F7> and <F8> in the Keyboard Commands section of this

Manual.

If you want to send a Binary File you must specify the file name on the

"YU" command line.   When receiving a Binary File, the "YD" command

alone is quite sufficient because the other station will include the

filename in the Protocol Header.

OK, OK... those examples.

Here is a Script to call the BBS and List all unread messages,

capturing those details into a file on our disk. Conveniently, it will

Call the BBS Script we created earlier to establish connection with the

BBS so we won't have to go into all that detail here.

************************************************

*                 BBSLIST.SCP

*

*  This Script will connect to the BBS, List

*  all unread messages and Bulletins, storing

*  all that data in a file named BBSLIST.DAT

*  on our hard disk.

************************************************

C BBS

*

* The BBS Script will Quit if the connection is not established

* so if it comes back here we must be connected now!

*

* Open a file to capture the BBS message list

AD C:\PAKET\BBSLIST.DAT

E The text file is now open and ready to receive data...

* Ask the BBS to "List" the last 10 messages in its Mailbox

S LL 10

*

* OK, that's done, now we can close our data file

*

AC

* and disconnect from the BBS

B

S D

R

That looks easy enough.  But when we run it, we find there are no

message details in our new data file, even though we might have seen

them on the screen as they were received from the BBS!   What went

wrong?   Can you figure it out?  This is called "debugging" and this

aspect of programming can be the most frustrating part of it.

Page 259

Yes, "programming".  Whether you knew it or not, you are now a computer

programmer.  Writing a program means to develop a series of

instructions for the computer to follow.  That is exactly what we are

doing here.

Anyway, now that we have something in common, let's investigate the

problem we have with this program... er, I mean Script.  What is

happening here?

No, come to think of it, I might leave you to have a go at debugging

that one yourself.  I'll come back to it later.  We'll make that part

of your Practical exercise for this section.

Review:

In this part:

We introduced the AD, AC and AU commands.

We also covered the YD and YU commands.

These commands:

The AD command ("ASCII Download") will open a text file on disk and

will write all incoming data to that file until it is closed.

The AC command ("ASCII Close") will close the file we opened with

the AD command.

The AU command ("ASCII Upload") will send the contents of the

specified text file to the TNC.

The YD command is the Binary File Transfer Download command, which

is used to transfer a Binary File FROM the other station.

No parameters are required here because the other station will

include those details with the file header, and paKet will know what

to do with it.  Before issuing the YD command in your Script you

should make sure the other station is ready to transmit in YAPP or

pP mode.  This usually involves sending some sort of command to the

other station, such as the BBS's "YD" command to tell it we want it

to send one of its files to us.

The YU command is the Binary File Transfer Upload command, which is

used to transfer a Binary File TO the other station.

We must tell paKet which file we want to send on the YU command

line.  Again we should ensure we have issued the appropriate

commands to the other station to prepare it to receive our file

before we issue this YU command in our Script.

The syntax required:

for the AD command:

AD New-file-name

eg:

AD MSGS.DAT

AD D:\TEMP

Page 260

for the AC command:

AC      (there are no parameters required)

for the AU command:

AU file-name

eg:

AU CMDS.DAT

AU C:\PAKET\PREPARED.MSG

for the YD command:

YD      (there are no parameters required)

for the YU command:

YU file-name

eg:

YU PAKET.EXE

YU C:\GOODIES\FRED.LZH

Practical exercise

Create the BBSLIST.SCP from the example given above. This calls the

BBS.SCP we discussed in the previous section so it will be easier to

do this exercise if you have done that one and got it working.

Load and run this BBSLIST Script, watching the Screen closely as it

runs.  If you have successfully Called the BBS Script file, you will

see the Connect command appear on your screen.  If it aborts you

will get a message pop up - in that case you will have to try to

find out why it aborted.  Is everything typed correctly, exactly as

shown here?  Have you made any changes - if so they must be checked

carefully.  Maybe the problem is simply the BBS did not have time to

respond within the time you have allowed? (That's easy to fix!)

Assuming you do get a successful connection and the BBS sent back

some message details in response to our List command, it should all

be in the BBSLIST.DAT data file we specified with our AD command.

If it is not there, we have to find out what went wrong.  We have to

go back over the Script commands, examining each one carefully and

try to "play computer", that is to imagine WE are the computer,

religiously and blindly following each and every instruction.

It might take a little experience but I am sure you will soon spot

the problem here.  We have told the system to send a List command to

the BBS.  Fine, it does that.  Then it drops down to the next

command which closes the file!  Already!  Hey, the BBS is still

looking for those messages and we have closed the file already!  We

need some sort of Delay, preferably a Wait in here so we do not

close the file until AFTER we get those message details.

See if you can work out what to do here, but I'll give you a hint.

I would insert a Wait command after sending the LL 10.

Page 261

Script Tutorial - the ! command

Wouldn't it be nice if we could run some DOS commands from within paKet?

Oh, I thought you would never ask...

The "!" command can be used in a Script to do just that.  With this

command paKet will pass over to DOS whatever you place on this Script

line.  So if you have:

! COPY C:\PAKET\BIN\UPLOAD\*.LZH A:\

paKet will send that COPY command to DOS for processing.  There are no

limitations on the type of command you can run here.  If you can run it

at the keyboard from the <F9> DOS Gateway, then you can run it here

from the Script.  The paKet program is still in memory of course so it

is taking up a lot of precious memory.  Hopefully there is enough left

to run your DOS command?  It might be an internal DOS command such as

this COPY command or DIR, etc.  It could be a program such as:

! C:\DOS\MEM /C > D:MEM.LST

although in that case you do not even need to specify the "C:\DOS\"

if DOS can find that MEM program in its PATH.

Or you can use a DOS batch file.  So if you had a batch file called

say, HOUSEKEP.BAT available in the current directory or within the DOS

PATH, you could have:

! HOUSEKEP

If the Script command line contains just the "!" command without any

parameters, paKet will call up DOS and leave you with the DOS prompt -

just like pressing <F9>.

I decided to avoid any pause after this DOS command is performed.  I

could have added some sort of pause where you have to press a key or

something like that to give you time to peruse the results of this DOS

operation.  However, that tends to defeat the purpose of automating the

process in a Script!  So when the DOS command is processed and control

returns to paKet's Script, everything proceeds without any delay.  This

means you will not have time to look at the DOS screen to see what

happened.  If you want to see that screen I suggest you make up a DOS

Batch file with a PAUSE command and specify that Batch file here as

your Script command.

You should keep in mind that a REMOTE user will have access to any

Script file in your Scripts directory, if it has the standard .SCP

extension.  So if you have a Script to do some significant DOS

processing, such as erasing files from your disk, or even to ...

(shudder) ... to format your disk, you might need to keep that Script

out of the Scripts directory or perhaps to rename its extension to

something other than SCP.  That way, you will not risk having some

(perhaps innocent) REMOTE user Perform that Script without your approval.

Page 262

Review:

In this part:

We introduced the ! command.

This command:

The ! command is a Script command to execute a DOS program,

including a BATch file or an internal DOS command.

The syntax required:

! (DOS-program-name/command)    (The parameter is optional)

eg:

! LHA M C:\PAKET\SAVELOG C:\PAKET\*.LOG

! SCANDISK

!

Practical Exercise:

I am sure it won't take too much imagination on your part to come

up with all sorts of things you could do with this command.  But

just for the simple exercise, make up a Script with just one DOS

command: display the contents of your CONFIG.SYS file on the screen.

If you use the DOS "TYPE" command it will appear briefly then

quickly disappear as control returns to paKet.  If you have the LIST

program handy, use that.

Yes, I know this is a trivial example, but it is pretty harmless

too!   Here, let me illustrate:

! LIST C:\CONFIG.SYS

or

! TYPE C:\CONFIG.SYS

------------------------------------------------------------------

At this stage I consider we have covered enough about Scripts for you

to be able to make effective use of the Script system in paKet.

With the commands we have covered I think you could achieve quite a

lot of automation in your daily operations.

The rest of this section of the Manual contains more information on

Scripts and introduces some more sophisticated commands and techniques.

By all means read on, and explore the power and creativity that lies

in the use of these more powerful commands.  But if you are having

trouble keeping up, you might like to get some practice with the

commands we have covered so far, and get into the heavy stuff later.

------------------------------------------------------------------

Page 263

Script Tutorial - the T command

A Wait command will wait for a time, looking for a particular string of

characters.  This is most useful if that is the only string we are

interested in at the time.  A good example of this is the TNC's "cmd:"

prompt.  If we issue a command and want to know when the TNC is ready

to take another one, the "cmd:" is the only string we need to look

for.  Anything else is not really relevant to our needs at the moment,

so we can wait for that string and ignore everything else.

But things are not always so precise and convenient.  Sometimes there

may be several possibilities and we cannot be sure which string will

come first, or even IF a particular string will come in at all!

This is where the Trap command ("T") can be useful.

Trap is a Script facility where we can specify up to 30 different

strings to watch out for, each of which has a Label to identify the

Script code we want to perform if THAT particular string is detected.

Perhaps the best way to illustrate the concept of Traps is to give you

a working example. For your Practical Exercise for this section, make

up a Script as follows:

T The BBS,VK2ATM

T pgm,paKet

T My friend,Andy

E We are starting the Trap Script to look for

E some things while we monitor the frequency.

:top

D 60

E We are still in the Trap Script

G top

* The following Script commands are used only if a Trap occurs

:The BBS

E We have spotted some BBS activity

G top

:pgm

E Somebody is using paKet around here

G top

:My friend

E Oh, there's my mate Andy!

G top

This Script starts with three Trap commands.  I suggest you change the

text following the comma on these commands to something you are likely

to see when monitoring the frequency on your system.

In the example shown above the first Trap is looking for the callsign

of my local BBS (VK2ATM in my example).  If that string is detected,

paKet will perform a Goto to the ":The BBS" Label.

Page 264

The second Trap command is looking for the word "paKet"; the third

command is looking for the string "Andy".

So when I run that Script here, paKet makes a note of these three

Traps, then after a couple of Echoes it begins a 60 second delay.

During this time anything that is received over the air will be

processed and displayed on the screen.  When the 60 second period

expires, we get the message "We are still in the Trap Script" then we

go back for another 60 second delay.  We get that message every 60

seconds just to remind us what we are doing!

If, while we are monitoring and displaying incoming data, we receive

one of the Trap strings (eg: "paKet") the Script will jump down to the

Label specified for that string.  In this example we would go to the

:pgm Label and continue processing the Script from there.

You can see that will Echo a message about paKet the go back for

another turn around the Delay loop.   You can do whatever processing

you like there - I am just Echoing a message to keep it simple. The

important thing here is to see how the Traps operate.

And if the word "Andy" appears anywhere in the monitored data, it will

echo a message about my mate Andy before going back around the delay

loop.

If none of these Trap strings is received, it will just go on repeating

the "We are still in the Trap Script" message every 60 seconds.

Press <Esc> to cancel this Script.

Another (more complicated) example?

Suppose our BBS produces the message "You have mail waiting" if there

happens to be some new mail in its Mailbox that we have not yet read.

We could modify our BBSLIST Script File to include the following Trap

command:

T mail, You have mail waiting

This new command tells paKet to watch out for the "You have mail

waiting" string and if it is detected, the Script processing is to

continue from the ":mail" Label.

Our BBSLIST Script might look something like this:

Page 265

************************************************

*                 BBSLIST.SCP

************************************************

T mail, You have mail waiting

C BBS

AD C:\PAKET\BBSLIST.DAT

E The text file is now open and ready to receive BBS data...

* We know the TNC has connected, but we should wait for

* the normal BBS prompt which includes the "X,Y,Z etc"

:wait

W 60, X,Y,Z,?) >

Q

* When the BBS is ready, THEN send our command

:list

S LL 10

* Then wait for the listing to finish before closing our file

W 120, X,Y,Z,?) >

Q

AC

B

S D

R

:mail

* We come here ONLY IF the Trap detects the Mail Waiting string

* Which would be sent while we were waiting for the first BBS prompt

W 120, X,Y,Z,?) >

Q

S RM

G wait

It is just as well you are now becoming familiar with Scripts because

this one is looking a bit complicated now.  But take it steadily, one

command at a time and I am sure you will cope with this OK.  It is

basically the same as the previous BBSLIST Script with the addition of

the Trap command at the top and the ":mail" routine at the bottom.

I have removed some of the spacing and comments because it is becoming

a bit big to be repeating all that here in this section when you can

easily go back a few pages to see the earlier copy.

Oh... it looks like I have also added those Waits we were talking about

earlier.  Remember that bug we had?   Oh well, you have seen it now so

it's too late to cover it up.  We might as well talk about it then...

I added a Wait for 60 seconds at first just to wait for the BBS prompt.

The other Script we called (BBS.SCP) returns as soon as it sees the

TNC's Connected message.  So I added a Wait to give the BBS time to

come up with its first prompt before sending the "LL 10".

Page 266

Then after sending the "LL 10" another Wait - this time I waited for up

to 2 minutes to get the next prompt.  The BBS will send another prompt

AFTER it has sent the Listing we requested (I hope 2 minutes is

sufficient time).  Then we can close the file.

In case you are wondering about that funny string "X,Y,Z,?) >", I

should explain that is just the end of the usual BBS prompt string.

Yes I know I could have specified the entire prompt string but I

figured that much was rather unique and should be more than enough to

properly identify the string I was looking for.

Anyway, back to the Trap:

When our new Script runs, it processes the Trap command first.  That

tells paKet to record those details and to maintain a watch for that

string.

My local BBS might come up with that Mail Waiting message as soon as I

connect so it would be received before I get the first Menu Prompt.

That means if we are going to get that string at all, it would come in

during our 60 second Wait.

If we do not happen to have any mail waiting, the BBS will not send

that message.  So our Script will perform exactly the same as if we had

not bothered about the Trap at all.  It simply sends the "LL 10", waits

for another prompt, closes the ASCII file and Disconnects.  So, if the

Mail Waiting string does not come in, our new ":mail" routine is not

used at all!

But, suppose we did have some mail waiting and the BBS did send us that

message after we make the connection.... let's follow the logic flow as

we run this Script:

1. The Trap command is recorded.  That's all that is done at this

stage.  paKet simply records both the String we are looking for

and the Label we want to associate with that string.  No other

action is taken.

2. we call BBS.SCP to make the connection.  That will Quit if we

fail to make the connection, so if control returns to our BBSLIST

Script, we know the TNC is now connected.

3. The AD command opens our data file and the Echo command sends a

confirmation to the screen.

4. We begin a 60 second Wait now, looking for the BBS prompt.  As

soon as that is received, we will drop down to the Send command

where "LL 10" is sent to the TNC.

5. But in this particular case, the BBS sends us the "You have mail

waiting" string while we are waiting for its first prompt.

6. The Trap processing detects this and switches control over to the

":mail" label.

Page 267

7. Here (in the :mail routine) we continue to wait for that first

prompt, just to keep things tidy.  When we get the prompt we

issue a BBS command to read our mail (that is what the RM is

for).

Then it can go back up to the ":wait" label because there it will

wait for another BBS prompt.  The BBS will send us another

prompt when it has finished sending our mail.

8. Now that we have our mail, and the BBS has sent its next prompt,

telling us it is waiting for another command, we can drop down to

the List command and we continue our normal processing from

there, eventually closing the data file and disconnecting from

the BBS.

Yes, it does look rather complicated at first glance, but you can see

that if you take each command in turn and follow the logic through,

step by step, it does make sense.  And as you gain more experience with

Scripts you will be able to read that sort of thing as easily as you

read the morning newspaper.   Promise!

Page 268

Script Tutorial - the FC, FS and FT commands

These commands are the Script Flag commands.  There are three of these

commands and each requires a parameter, a number between 0 and 9:

FC n     Flag Clear

FS n     Flag Set

FT n     Flag Test.

A "Flag" is simply an indicator we can set (or clear) during the Script

to remind us of some condition or circumstance, and that flag can be

tested later in that Script.   There are 10 Flags available, 0 to 9.

This facility was created in paKet 5 and is included in this version

for compatibility with existing Scripts.  Now that we have Variables in

paKet 6 (yes, yes I am coming to Variables) you could use them to do

the same job.   Anyway, Flags are here so I'll show you how to use

them.

If we come to some condition we need to know about later we can set a

Flag, for example:

FS 6

This sets Flag number 6.  By the way all Flags are presumed to be Clear

(i.e. NOT Set) before a Script starts.

You can Clear a Flag with the Flag Clear (FC) command.

Then, later in our processing we can test this Flag to see which way we

want to go.   The Flag Test (FT) command will look at the Flag we

nominate here and if the Flag is set, the next Script line is

performed.  But if that Flag is Clear, paKet skips the next line in the

Script and continues from the line after that.  Let's have another

example:

.

.

FT 6

G Gotcha

E The Flag was not set

.

.

.

: Gotcha

E The flag was set...

FC 6

E ... but is now cleared again

.

.

Page 269

Script Tutorial - Variables

I'll bet you thought we would never get to Variables!

What is a Variable anyway?

In its simplest terms, a Variable is a small piece of the computer's

memory that we can use in our Script, to store some temporary

information.  It may contain varying values because we choose what goes

into it!

Each of these Variables is identified by a unique name.  There are 36

Variables altogether, 26 ordinary Variables and 10 special Variables

which I will call "Passed-Parameters".

The 26 ordinary Variables are identified by the letters of the alphabet

and the syntax we must use is to specify a Variable with a Percent sign

followed by a single letter, eg:

%A  (or %a  - same Variable)

%J

%Z

The Passed-Parameters are similar but they use numbers from 0 to 9, eg:

%0

%5

%9

How are variables used?  Perhaps the easiest way to demonstrate is to

introduce a new command:

Script Tutorial - the M command

The Move command moves some data into a Variable.  The syntax required

is:

M variable,data

For example:

M %A,VK2DHU

M %b,Tony

M %K,99

All Variables contain text data so that one with 99 actually contains

two ASCII characters, not a single numeric value of 99!

You may use Variables in any of the Script text strings.  So we could

have, for example:

T exit,%B is not home

E Connecting to %A...

S CONNECT %A

W 10, Connected to %A

Page 270

These commands would be interpreted as follows (assuming the above Move

commands have been performed earlier in this Script):

T exit,Tony is not home

E Connecting to VK2DHU...

S CONNECT VK2DHU

W 10, Connected to VK2DHU

You can choose any of the 26 Variables for any use you wish.  There is

just one thing I need to mention.  The %Z variable is used by the

Script system to carry the Return Code from a "Called" Script.

The Return Code.   Don't tell you've forgotten that already?

Alright, let's have another quick look at the Return command. We might

usually specify just the single letter "R" to return from a Called

Script, but if we wanted to return the result of some operation, we

could give it a Return Code by writing something like "R 50".

If we did that, the %Z variable would now contain a value of 50.

Let's have a look at that again with an example:

E Testing the Return Code

C SUBS

E *** The return code from Subs is %Z ***

So if the SUBS Script finished with:

R 9

then the last line of our Example would Echo on our screen:

*** The return code from Subs is 9 ***

You can use the %Z Variable for any purpose at all, but remember it

might be overwritten by the system if you Call another Script!

Script Tutorial - Passed-Parameters

These are the ten numeric Variables, %0 to %9.

When we Call another Script, we have an option to include on the

Call command line some additional information, which will be passed to

the Called Script. If we have the following line in our MAIN Script:

C SUBS,Tony

Then the SUBS Script knows what parameter was specified because it is

stored in %1 (it is the first parameter on the line).

If the SUBS Script included something like:

E SUBS Script parameter number 1 contains %1

you get no prizes for guessing that will Echo on our screen:

SUBS Script parameter number 1 contains Tony

Page 271

Each of the parameters on the Call command line are allocated to the

Passed-Parameters Variables in the Called Script in the same sequence

they are written.  The first parameter goes into %1, the second into %2

and so on.

You might see why the numeric Variables are called Passed-Parameters.

In the example above I had only one parameter ("Tony") but you can have

up to nine of them, separated by commas.  If we had this Call command:

C SUBS,Tony,VK2DHU,Port Macquarie,Australia

then the Variables %1, %2, %3 and %4 would be filled and accessible

from within the SUBS Script.  For example, %3 would contain the string

"Port Macquarie".

If you are observant, you will have noticed we are unable to include a

comma as part of a parameter because it is used as a delimiter to

separate the parameters.  Yeah, OK you got me there!  Sorry about that,

but it is true.  You can't have a comma in a parameter!  Maybe in paKet

version 97.2?

While I am dealing with you observant ones, why do we have ten

Passed-Parameters and only nine available on the command line?

Time's up!   The first one, %0, always contains the name of the Script

file currently running.  It doesn't matter whether we are using Called

Scripts or not, you can always specify %0 in any Script and it will

contain the name of THIS Script that is currently running.

Page 272

Script Tutorial - Scope of Variables

The ordinary Variables are considered Global in scope which means you

can set a Variable in one Script and use that same variable in another

Script in this run.   For example, if we have a MAIN Script as follows:

M %A,Australia

E We start with %A

C SUBS

E Back again and our variable now contains %A.

.

and our SUBS Script looks like:

E In SUBS now... and A still has %A

M %A,UK

R

When we run the MAIN Script, we will see the following Echoes on our

screen:

We start with Australia

In SUBS now... and A still has Australia

Back again and our variable now contains UK.

The Passed-Parameters are different.  They are considered local and are

not visible to other Scripts.  So %1 in the main Script will probably

contain a different value to the %1 in a called Script.

Page 273

Script Tutorial - a breather...

Whew!

How's it going?

If you have had some programming experience you might be finding this a

bit easier than if you are new to computers.  But don't despair if you

are finding this a bit heavy.  You can get a lot out of Scripts by

using some of the simpler commands we covered early in this section of

the Manual.

The Called Scripts, the Variables, and other complexities can wait...

or you can still make use of some "clever" Scripts written by someone

else.

But if you are still with me... there's more.  Oh yes, we're not

finished with Scripts yet.

Page 274

Script Tutorial - the I command

Wouldn't it be nice if we could get the Script to stop and allow us to

enter some information at the time of RUNNING the Script, instead of

having to edit the Script File every time we want to run it?

Didn't I tell you about this one?  This is the Input command.

There are two parameters to this command.  The syntax is:

I variable,prompt_string

For example:

I %C,Enter the callsign of the station you want to call...

when the Script comes to this command, a Message Window will pop up

displaying our prompt:

Enter the callsign of the station you want to call...

and paKet will stop while you enter something on the input line.

Press <Enter> to complete your data.  Whatever you typed (except the

<Enter> - that is not included) will now be stored in that variable,

%C in our example here.  The Script then continues from the next line.

Script Tutorial - the ? and J commands

The "?" is a Query command and is used to compare two values.

In most formal programming languages, there is an IF statement which

might look something like "IF A > 99 THEN Do-something".  I would have

liked to provide some better way to compare two values, but this "?"

command is the best I could do in the time available.  Hey, this is

just a Communications Program after all!

Anyway, the Query command is used with similar syntax to the other

Script commands:

? value1,value2

For example:

? %A,Port Macquarie

? %Z, %2

? Paul,Tony      (Don't ask me why you would want to do this,

but at least the syntax is valid!)

The comparison is a text comparison and once again it is not sensitive

to the case.  So "Tony" and "tony" are considered equal. And "90" is

higher than "100".   Eh?  90 is higher than 100?   Yep, when doing a

text comparison it finds the first character, "9" higher than the "1"

so it decides 90 is higher than 100!

Page 275

Performing a Query command is only half the job.  We also need some way

of acting on the result of that comparison.  So we have a Jump ("J")

command.  Actually we have six Jump commands:

JE    Jump if the two values are Equal

JNE   Jump if the two values are Not Equal

JL    Jump if the first value is Lower than the second

JNL   Jump if the first value is Not Lower than the second

JG    Jump if the first value is Greater than the second

JNG   Jump if the first value is Not Greater than the second

The Jump commands have one parameter, that is the Label we want to Jump

to if that condition is satisfied.  So we could have:

C SUBS

? %Z,0

JE Success

? %z,5

JG Big Score

JE five

.

.

:Success

E The SUBS went OK because we got a zero return code

.

.

:Big Score

E SUBS returned a value over 5!

.

.

:five

E We got exactly 5 in the SUBS return code

.

.

The Jump commands will act on the LAST Query to be performed.  It would

be the norm to issue a "?" command immediately prior to the Jump.  But

if you forget to do the Query, paKet is quite happy to continue.  It

will use the result of the last Query you did, no matter how long ago

that was.  If there have been no Queries at all in this Script, paKet

will consider the "result" to be Equal!

Page 276

Script Tutorial - the L, P and X commands

I almost forgot these.

But these simple commands are so simple it won't take long to explain

them ALL!

They are all single letter commands - no parameters on any of them.

The L command toggles the Disk Log file on or off.  If it was on, this

command will turn it off;  if it was off, this command will turn it on

(using the default Log File name).

The P command toggles the Printer Log on or off.  If it was on, this

comm... (yes, yes, alright.  I know you know!)

The X command is different and one that I sometimes think you should

not have at all.  But someone asked for it, so it is still here.  This

one will exit the paKet system altogether, returning us to the DOS

Operating System (or to Windows or OS/2 or wherever we came from).  It

is even more drastic than the <Alt-X>  (or the <Alt-F4>) command.  At

least with those, you are asked "Are you sure?".  The X command does

not give you a second chance.  So please use this one only if you are

really sure you need it.

Well, that's it!   Now we have covered all the Script commands.

And now it's over to you to see what you can do with it all.  Try some

simple ones to start with and build on them.  Discuss them with other

paKet users on air and swap those you are satisfied with.  We can

always learn from each other.

I would be interested to hear of any ideas you have and to see the

fruits of your Scripting labours.

Page 277

The Script File syntax.

The rules:-

1. The Script file is plain ASCII text

(Extended ASCII graphic characters > $128 allowed).

2. All lines must be less than 80 characters.

3. Each Script file is limited to a maximum of 200 lines.

(Use the CALL command to CALL other Scripts if you need more).

All spaces will be ignored, except for EMBEDDED spaces in "text strings"

(eg: in the Send command - LEADING spaces in "text" will be ignored). For

example:

s YU PROG.EXE

W 10, Ready to Receive with YAPP protocol

g not ready

y u prog.exe

is identical to:

SYU PROG.EXE

W10,Ready to Receive with YAPP protocol

GNOT READY

YUPROG.EXE

You are the "programmer" here so choose a style that suits you best.

Most people would find the latter example more difficult to read.  The

careful use of spaces makes for more easily understood programming, which

probably means fewer logic problems in your Scripts.

Page 278

Script Command Summary

The following Script commands are valid:

! dos_command      Run a DOS command.

This will load a copy of COMMAND.COM and will pass

'dos_command' to DOS for processing. When that DOS

command/program has finished, processing of the

Script will continue from the next line in the

Script.

The comments I made for the <F9> key and for the

<Alt-E> in the "Keyboard Commands" section of this

Manual apply here too. You will not have the usual

amount of memory available for these DOS commands

because paKet is still running and still occupying a

lot of memory.

*                  This is a comment line if it begins with an "*".

The rest of the line may contain anything at all

because a comment line is not loaded by the Script

processing.  It is used entirely as documentation in

your Script File on disk.

I strongly recommend the generous use of comments

in your Scripts.  If nothing else, writing comments

helps you to focus your mind on your own logic.

: label            No processing is performed on this line.

It is used by the Goto, Trap and Jump commands.

As no processing is performed here, this could also

be used as a comment in your Script file.  But in

this case it IS loaded into memory with your Script.

If you intend this to be a comment only and not a

Label, it would be better to use the "*".

"label" may be any length but the total line length

must be less than 80 chars.

? value1,value2    Query command to compare two values.

This command compares the contents of the first

value with the contents of the second value.

The value fields may be Variables or constants.

This comparison is used together with one or more

Jump commands to provide some decision making

capability in a paKet Script.

Page 279

AC                 Close the Download file.

Presumably this will follow the AD command somewhere

in your Script?

If the file is already closed, this command will be

ignored.

AD filename        ASCII File download

(Same as <F6> but you must put filename on the

Script command line. (Eg: AD RECVFILE.DAT)  Note

that like <F6> if the file already exists, a new one

will be created with a sequential numeric extension

(RECVFILE.000 or RECVFILE.001 etc).

AU filename        ASCII file Upload.

Same as <F5> but again you specify the filename in

the Script command line. (Eg: AU SENDFILE.DAT).

B                  Send a BREAK code to the Serial Port. (Same as

<Alt-B> - return to cmd: mode)

C script,p1,p2,... Call another Script, with optional parameters.

A Script file is limited to a maximum of 200 lines,

but if you find this limiting your style, you can

set up a number of Scripts and Call them as

required.

There is no practical limit to the number of nested

Scripts you may call. Each of the called Scripts

should "Return" to the calling Script, where

processing continues from the line following the

Call command.

Note the facility for Variables, Return Codes and

Flags to return the status of important events while

in the called Script.

If you want to pass some parameters to the called

Script, those parameters may be entered on the

command line.  The parameters are separated by a

comma.  Up to nine parameters may be specified.

D time             Delay for "time" seconds, or delay until hh:mm.

or               "time" may be 1 to 32767 seconds (about 9 hours).

D hh:mm            "hh:mm" is a time of day.

If this time-of-day format is used, paKet will wait

until that time occurs before continuing.  Note, if

this time is earlier than the current time, paKet

will wait until that time TOMORROW before continuing.

Page 280

E text             Echo text to the screen.

This is included mainly as a form of documentation

within the Script. The text is echoed to the screen

as if it was received from the other station, so it

will appear in your log file if it is active.

FC n               Flag Clear

FS n               Flag Set

FT n               Flag Test

There are 10 flags available within the Script

system.

These are Binary flags (i.e either ON or OFF)

numbered 0 through to 9 and may be used by the

Script Programmer (that's YOU).

It was found by a paKet/Script user that there are

times when you need to remember the status of some

condition during the Script process.  By setting one

(or more) of the flags you can test the status of

that flag later in the Script and adjust the

processing as required.

For example you might need to note the fact that a

particular string was received from the TNC (using

the "W" command). When that string was received you

could Set a flag with the Flag Set Command,

(eg) "FS 4".

Later during this Script you could check the status

of that flag with the Flag Test command (eg) "FT 4".

If the flag is set, the next Script command is

performed, but if the tested flag is not Set (or

Clear) the next line in the Script is skipped.

With the introduction of Variables in paKet 6, we

have an alternative.  Instead of using a Flag, we

could Move some value into a Variable and Query that

Variable later.

You are the programmer, you make your choice!

G label            Goto "label" and continue the Script from there.

(Yeah, yeah I know Goto is unfashionable!)

I var,prompt       Input a string into a Variable.

paKet will display the prompt string in a Message

Window while waiting for you to enter some data

which will be stored in the specified Variable.

Page 281

Script Command Summary (cont)

JE  label          These Jump commands will Jump to the Label specified

JNE label          here, depending on the result of the previous Query

JL  label          ("?" command).

JNL label

JG  label

JNG label

L                  Disk Log File On/Off toggle

This uses the default log filename.

M var,text         Move data into a Variable.

The data is always taken as ASCII text data, even if

numeric strings are entered.

P                  Printer Log On/Off toggle.

Q                  Quit.

The Script processing is terminated. This would be

used for an aborted exit.

If we are using nested Scripts and the Quit is

performed in a Called Script, the entire chain is

aborted and control returns directly to paKet.

The Return command is a tidier way to exit a Script.

R (return code)    Return with an optional Return Code.

This command is especially used to return control to

a calling Script. If used in the first level, or

only Script, Script processing is terminated and

control is returned to paKet.

The optional Return Code can be used to send back

results of this Script.

A Return will be assumed at the end of a Script file.

S text             Send "text" to the TNC.

If you want to include a Control character such as

<Ctrl-C>, or an <Alt- > key combination such as

<Alt-A>, refer to the Special Key Codes section of

this Manual for the special syntax. Do not enter

quotes unless you want them sent to the TNC.

Page 282

You can also use substitution parameters such as $t

which will be replaced by the current time.  See the

Special Key Codes section of this Manual for details.

You can use any of the Script Variables here too.

A Carriage Return is generated automatically when

"text" has been sent. Note, if you do not want the

<CR> to be sent, add a caret ('^') as the last

character of the text.  paKet will then NOT generate

the <CR>. (It won't send that final caret either!)

T label,string     Trap -  Goto 'label' if 'string' is received.

If 'string' is received at any time during this

Script, processing will continue from the nominated

'label'.  Note that this happens even if a "Delay"

or "Wait" instruction is in progress.

For example, say your Script includes the following

commands:

T MSG,Message waiting

T DONE,No Messages Available

Then, if the string "Message waiting" is received

from the TNC at any time during the processing of

this Script, paKet will detect the matching Trap

string and will continue processing from the MSG

label.

And if the string "No Messages Available" is

received at any time during the Script, processing

continues from the DONE label.

The Trap remains set even if nested Scripts are

CALLed, so 'label' must be a defined label within

all Scripts if the Trap is to be effective.

If a Trap is no longer required, it may be disabled

by specifying the command without a string. Eg:

T DONE,

(Note the comma is still required here).

Up to 30 Trap strings may be current at any one time.

W time,string      Wait up to "time" seconds for "string" to be

received from the TNC.

The received "string" need not start at the

beginning of the line.

Page 283

If "string" is received within the specified "time"

the next line of the Script file is skipped,

otherwise the next line is executed.

Do not enter quotes in "string" unless you are

expecting them in the received text.

If no string is entered, (eg: W 60, ) paKet will

wait up to the time specified but the Wait is

considered satisfied as soon as ANYTHING is received!

X                  Exit the paKet program.

This should be used with care. It will not only shut

down the Script, it will also shut down the paKet

program and will return to DOS.

YD                 Commence a Binary File download.

This assumes the other station is now waiting to

send a Binary File to us with either pP or YAPP

protocol.

YU filename        Commence a Binary File upload.

This assumes the other station is now waiting to

receive a file with either pP or YAPP protocol.

'filename' must include full path details unless it

is in the current directory.

Page 284

PART VIII - PERSONAL MESSAGE SYSTEM (PMS)

Overview

paKet's PMS system offers message handling facilities for you and your

REMOTE users.

The operation of this PMS is similar to most other PMS systems with

facilities for users to list available messages, leave a message, read a

message, and delete a message.

paKet supports the standard MBL/RLI Mail Forwarding protocol so messages

can be automatically sent to and received from other systems, including

all known packet BBS systems.  There are several convenient options for

you to set up your own Mail Forwarding timetables so you do not even

have to connect to the BBS - paKet can do that for you at regular

intervals during the day or night.

Messages in the system are allocated a message-number when they are

entered. This message-number is retained for the life of the message.

paKet's REMOTE Menu provides access to the PMS for other users. You, the

paKet operator, have full control of the PMS via the Sysop PMS Menu

which may be called up with the <Alt-M> key.

The Sysop PMS Menu

You have access to the PMS system via the Sysop PMS Menu. This menu

appears in the Communications Window when you press <Alt-M>:

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

The <ScrollLock> will automatically be applied to hold any input from

the TNC while you are using this menu. Please do not "release" it

yourself - it will confuse the PMS processing if you do!

The options in the Sysop PMS Menu are as follows:

B  (Bye)       - End Sysop PMS Processing

C  (Copy)      - Copy a message to a file (eg C 3)

E  (Edit)      - Edit a message header (eg E 3)

H  (Help)      - Display these details

K  (Kill)      - Kill a message (eg K 3)

KM (Kill Mine) - Kill all messages addressed to me

L  (List)      - List available messages

LM (List Mine) - list all unread messages addressed to me

R  (Read)      - Read a message (eg R 3)

RM (Read Mine) - Read all unread messages addressed to me

S  (Send)      - Send a message (eg S VK2XYZ)

?  (Help)      - Display these details

All operations on the PMS are performed in the Communications Window so

they will be logged if the Log File is active at the time.

Page 285

The 'H' command and the '?' command do the same thing which is to

display the brief summary of the Sysop PMS Menu commands shown above.

When you have finished with the PMS system, enter the B(ye) command or

press <Esc>. <ScrollLock> will be automatically released and you will

see the message:

Returning to normal communications...

All these commands are discussed in detail in the section, "Accessing

the PMS" which follows shortly.

PMS commands in the REMOTE Menu

It is via the REMOTE Menu that other users have access to our PMS. The

REMOTE menu is discussed more fully in its own section of this Manual;

here in this section we are interested only in those REMOTE commands

relating to the PMS.

The REMOTE Menu commands relating to the PMS system are K, L, R and S.

The "Mine" variants (KM, LM, RM) may also be used by REMOTE users.

A REMOTE user may List all messages in our PMS and may Send a message

to any other station. However they may not Read nor Kill any "third

party" messages, that is Personal messages that were neither sent by

nor addressed to that station.  If third party messages are not

permitted in your Country, paKet will not allow a REMOTE user to Send a

message to a third party either.

When another station first connects to our system, and you have the

Automatic REMOTE Menu option active, paKet will check the

PMS and if there is any unread mail addressed to that station the

following information will be transmitted:

You have mail waiting in my PMS

Msg#     Size To            From     Date   Time  Subject

---- -- ----- ------------- -------- ------ ----- ---------

186 PN    14 VK2DHU        VK2XYZ   MAR-11 13:50 Howdy

In this example, I have connected to another station and it found there

is an unread message addressed to me (VK2DHU) so it gives me these

details before it sends its REMOTE Menu for the first time.

The REMOTE commands to access the PMS are similar to some of the

commands you have in the Sysop PMS Menu.  These are discussed in the

following section, "Accessing the PMS".

Page 286

Accessing the PMS - B, C and E commands

I will cover all the PMS commands in alphabetical sequence.  Some of

these commands are available only to you the paKet operator via the

Sysop PMS Menu, but some are also available to a REMOTE user.   As we

cover each command I will explain which is which.

B  (Bye) - End Sysop PMS Processing

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

This PMS command is available only in the Sysop PMS Menu.

There is a similar Bye command in the REMOTE Menu but that has a

different purpose: that is used by REMOTE users to disconnect from

our system.

Type "B" when you have finished with the PMS for now and want to go

back to normal communications.  The <ScrollLock> is released and any

data that was being held in paKet's input buffers will now flow into

the Communications Window.

If you press the <Esc> key while the PMS Menu is waiting for a

command, it has the same effect as typing the Bye command.

C (Copy) - Copy a message to a file (eg C 3)

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

This PMS command is available only in the Sysop PMS Menu.

paKet keeps all your PMS messages in one file.  The Copy command

provides an easy way to extract one (or more) message from the PMS,

creating a separate disk file with the contents of just that message.

The name of the disk file that is created is composed of the

callsign of the other station; and the extension is the message

number. So, if I had in my PMS a message addressed to XY4BBS and

that was message number 200, a "C 200" command would create a file

in my PMS Directory with a name of XY4BBS.200.

If you enter more than one message number on that command line, all

those messages would be combined into the one file.  The file name is

created from the first of those messages.  For example, if I typed:

C 205 257 450

(and say message 205 was sent to me by VK2BZC) the contents of all

three messages would be written to a file with the name VK2BZC.205.

The new file will contain a reconstructed command line and subject.

You may type the Copy command as CM (Copy Message data only) which

will copy the message(s) to disk without including the two lines of

header and subject information.  Eg:

CM 205

Page 287

E  (Edit) - Edit a message header (eg E 3)

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

This PMS command is available only in the Sysop PMS Menu.

In the PMS, every message has a Header.  The Header is that part you

see when you call up a List of messages.  It includes the message

number, callsigns, title, etc.

You can edit those details with this command.  Just type the E

command followed by the message number of the one you want to

change.  paKet will bring up the following window where you can move

to any line and change its contents.

����������������������Message Number 115������������������Ŀ

�From          VK2DHU                                      �

�To            G1XGP                                       �

�At BBS        @GB7DUG.#32.GBR.EU                          �

�Type          P                                           �

�Status        F                                           �

�Date          MAR-12                                      �

�Time          7:31                                        �

�Bid           $115_VK2DHU                                 �

�Subject       Performance.                                �

��������������������Press <Esc> when finished���������������

This is probably a good time to discuss these Header fields and

explain what they mean.

Msg#    - The Message Number is part of the Header but you can't

change that!  It is shown in the top line of this window.

From    - This is the callsign of the station who lodged the message

in the PMS.   If you sent this one, this will be your

callsign.

This example is showing my callsign (VK2DHU) because I

entered this message into the PMS.

To      - This is the callsign of the station the message is

addressed to.   Note this is the callsign only.  If the

network address is required, see the following field for

that information.

In this example I am sending a message to G1XGP.

Page 288

At BBS  - This field contains optional addressing information for

messages that are to be Forwarded to the BBS (and beyond).

If this field is blank, the message is considered to be

local and will not be Forwarded, remaining here in this

PMS until it is deleted.

But in this example there is an address recorded. When I

entered that message I typed:

SP G1XGP @ GB7DUG.#32.GBR.EU

The address conforms to the Hierarchical Addressing method

which is now widely used by BBS systems worldwide. Here I

am sending a message to Steve, G1XGP whose home BBS is

GB7DUG. The #32 is a local identifier used in Great

Britain to subdivide the Country into zones to help them

manage the large number of BBS systems there. The GBR and

EU, as you might guess, are the codes for Great Britain

and Europe.

If you are not familiar with this system of addressing,

ask your local BBS Sysop for more information.

If you have a message already in your PMS, with a blank

"To BBS" field,  you can add that information by moving

the cursor to this line in the Edit Window and typing your

new information here.   paKet will then send this message

to your local BBS for onward Forwarding during our next

Forwarding session (you might also need to change the

Status to N).

Type      A message can be either:

P - Personal message addressed to a particular station.

Personal messages are entered into the PMS with an

"SP" command.  If you enter "S" without a Type,

paKet assumes "SP".

B - Bulletin, which is a message intended for a wider

audience and usually addressed to some topic, such

as PAKET, AMSAT, etc

Bulletins are entered into the PMS with an "SB"

command.

Status    The status can be:

N - the message has not yet been read nor Forwarded.

Y - the message has been read by its addressee.

F - the message has now been Forwarded to our BBS.

Page 289

H - the message is Held.  paKet changes the status to

Held if the BBS has rejected our attempt to Forward

this one.  The most common reason is Duplicate Bid,

meaning the BBS already has a message with this

BID number.  (You could edit the Status to N,

change the BID to something else and try again?)

K - the message is Killed.  The message has been marked

for deletion but remains in the PMS until paKet

does its housekeeping, usually after leaving the

Sysop PMS Menu.

You can see it is possible to undelete a message

provided you do not exit the PMS before editing

that message header.  While the message is still

there with a K status, you can edit that Header

changing the status to any of the other values.

Then the message will not be removed during

housekeeping.

Date

Time      This is the date and time the message was entered into our

PMS system.

Bid       A "Bid" is a unique identifier for this message.

This field is used (mostly by BBS systems) to determine

whether this message has already been Forwarded.  paKet

uses the same Bid format used by many BBS programs - it

builds the Bid from the current message number and your

callsign.  Actually, it doesn't matter what is in this

field, so long as it is unique.

If you happen to reset your PMS message numbers (maybe you

have deleted your PMS files and started again from message

number 1) there is a risk the BBS will reject some of your

messages that are being Forwarded.  It is possible we have

sent a message with this same Bid number some time ago and

the BBS thinks this new one must be a duplicate. It can do

this even if the message is addressed to a different

callsign!

You could use this Edit facility to change the BID to

something else - then the BBS will accept it next time we

try to Forward this message.

Hint:

If you are getting a lot of Rejects because of duplicate

Bids, (i.e you are reusing the old message numbers) you

could get paKet to use different numbers for all new

messages.   There is a line in the PAKET.CFG file which

Page 290

records the last message number.  This field is "PMSMSG"

and if you edit that line in the CFG file so it reads:

PMSMSG=1000

then your message numbers will start from 1000 and that

should avoid the duplicate Bid problem with the BBS.

Or you could add a new line to the CFG file (after the

first line):

PMSSUFFIX=A

Then paKet will create Bid numbers with that suffix

resulting in a different Bid!.

Subject   The contents of this field is displayed when messages are

Listed.  The subject field is usually limited to 30

characters because most BBS systems will display only that

number when sending its List of messages.  paKet can

handle up to 80 characters in this field.

When sending a new message, especially if it is a

Bulletin, try to provide a brief but descriptive summary

of your message in this Subject field.  People often

decide to read or ignore a Bulletin solely on the basis of

this Subject field.  If you want them to read your

Bulletin, make it sound interesting!

When editing any of these fields, the usual rules apply for editing

in paKet.  Press <Enter>, <Up-Arrow> or <Down-Arrow> when finished

with a line.  If you make a mistake, you can press <Esc> to ignore

the changes you have made to that line.

When there are no more changes press <Esc> to return to the Sysop

PMS Menu.

Page 291

Accessing the PMS - H, K and L commands

H  (Help) - Display details of the Sysop PMS Menu

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

This PMS command is available only in the Sysop PMS Menu.

Typing a "?" is the same as typing "H".  In either case paket

displays the following information in the Communications Window to

jog your memory:

B  (Bye)       - End Sysop PMS Processing

C  (Copy)      - Copy a message to a file (eg C 3)

E  (Edit)      - Edit a message header (eg E 3)

H  (Help)      - Display these details

K  (Kill)      - Kill a message (eg K 3)

KM (Kill Mine) - Kill all messages addressed to me

L  (List)      - List available messages

LM (List Mine) - list all unread messages addressed to me

R  (Read)      - Read a message (eg R 3)

RM (Read Mine) - Read all unread messages addressed to me

S  (Send)      - Send a message (eg S VK2XYZ)

?  (Help)      - Display these details

K  (Kill) - Kill a message (eg K 3)

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

[paKet] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

^

This PMS command is available in both the Sysop PMS Menu and

in the REMOTE Menu.

(I wish other systems had used the term "Delete" or "Remove" rather

than "Kill", but that is the accepted term for removing a message so

I use "Kill" for the sake of conformity).

A remote operator may Kill only a message sent BY that station or

addressed TO that station, but if you are accessing the PMS through

the Sysop PMS Menu, you have full authority to delete any message.

The messages are identified by number so the command must include

the message number that is to be killed, eg:

K 5.

If possible, paKet will delete that message from the PMS Database

and will respond with a message such as:

Message 5 Killed

It is permissable to enter more than one message number on the line.

So you can for example, enter:

K 1065 1108

to kill those two messages.

Page 292

As mentioned earlier under the Edit command, a Killed message is

not removed from the PMS immediately.  It is flagged with a Status

of "K" and left there until paKet does its housekeeping.  So if you

are using the Sysop PMS Menu, you could edit that message Header and

change its Status to something else, effectively undeleting that

message.

KM (Kill Mine) - Kill all messages addressed to me

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

[paKet] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

^

This PMS command is available in both the Sysop PMS Menu and to a

REMOTE user, although it is not shown separately in the REMOTE Menu

(it is full enough already!).

"KM" is almost the same as the "K" command but this one will Kill

all messages addressed to the user if the message Status is Y or F.

So, if you are using the Sysop PMS, KM will Kill all messages

addressed to you provided you have read them.

And REMOTE users who type "KM" will Kill all messages addressed to

them, provided they have read those messages.

L  (List) - List available messages

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

[paKet] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

^

This PMS command is available in both the Sysop PMS Menu and

in the REMOTE Menu.

Upon receipt of a simple "L" command, paKet will produce a listing

of every message in our PMS, showing one message per line.  If we

enter the "L" command from the Sysop PMS Menu, the listing appears

in our current Communications Window.  That means it will also be

written to the log file if it happened to be active at the time.

If the REMOTE user types the "L" command the listing is sent to the

TNC for transmission back to that station.  All messages in the PMS

are listed here, whether the REMOTE user has access to the message

or not.

The PMS Listing will look something like this:

Page 293

Msg#     Size To            From     Date   Time  Subject

---- -- ----- ------------- -------- ------ ----- ---------

1057 PK  1447 VK3DJM@VK3KSD VK2DHU   JAN-31 19:42 paKet and Windows

1065 PY   862 VK2DHU        VK4WAC   FEB-20  8:49 Gidday

1077 BF  1011 PAKET@WW      VK2DHU   FEB-17 21:50 Binary Transfer

problems.

1085 PY   862 VK2DHU        VK4CMC   FEB-20  5:43 paKet 5.1

1105 PF  2820 VK4CMC@VK4DIT VK2DHU   FEB-28 22:29 paKet program

1107 PY  1552 VK2DHU        AI8I     MAR-01  5:44 NEW VERSION??

1108 PN  2176 VK2DHU        ZS0STB   MAR-01  5:45 PAKET 5.1

1110 PF  1179 AI8I@KQ4OK.VA VK2DHU   MAR-01  6:36 New version.

The contents of the Message Headers are shown here, each line

representing one message in our PMS.   These fields were covered in

detail earlier when we discussed the Edit command.  The one thing we

didn't mention earlier is the Size of each message, because you

can't change that with the Edit command.

Screen space is limited so the normal thing is to abbreviate any

long addresses in this Listing.  In this example the messages with a

To BBS address (1057 1077 1105 and 1110) are shortened if necessary

to enable them to fit into the space available.  Of course the full

address is still there.  That will be obvious if you call up that

message with the Edit or Read commands.

If the Subject is too long to fit on the line, paKet will allow that

data to run over to the next line, as shown in message 1077.

You (or the REMOTE user) can select a partial listing with the LM

command which is discussed below.  You can also specify a From or To

callsign in order to get a listing of those messages sent by or

addressed to the specified callsign.  For example, I could enter:

L> VK2DHU

and, using the same PMS shown above, paKet would respond with:

Msg#     Size To            From     Date   Time  Subject

---- -- ----- ------------- -------- ------ ----- ---------

1065 PY   862 VK2DHU        VK4WAC   FEB-20  8:49 Gidday

1085 PY   862 VK2DHU        VK4CMC   FEB-20  5:43 paKet 5.1

1107 PY  1552 VK2DHU        AI8I     MAR-01  5:44 NEW VERSION??

1108 PN  2176 VK2DHU        ZS0STB   MAR-01  5:45 PAKET 5.1

Or, if you are interested in a wider search pattern you can use a

partial callsign.  For example, if I enter:

L< VK4

paket would respond with:

Msg#     Size To            From     Date   Time  Subject

---- -- ----- ------------- -------- ------ ----- ---------

1065 PY   862 VK2DHU        VK4WAC   FEB-20  8:49 Gidday

1085 PY   862 VK2DHU        VK4CMC   FEB-20  5:43 paKet 5.1

which includes all messages from VK4.

Page 294

If the PMS DataBase is empty, paKet will display:

No messages

LM (List Mine) - list all unread messages addressed to me

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

[paKet] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

^

This PMS command is available in both the Sysop PMS Menu and to a

REMOTE user, although it is not shown separately in the REMOTE Menu.

This command is very similar to the "L" command we just covered but

here the user is asking for a selective listing, displaying only

messages addressed to that user that have not yet been read.

Page 295

Accessing the PMS - R and S commands

R  (Read) - Read a message (eg R 3)

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

[paKet] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

^

This PMS command is available in both the Sysop PMS Menu and

in the REMOTE Menu.

The messages are identified by number so the command must include

the message number that is to be read, eg: R 5.   paKet will access

the PMS DataBase and locate the specified message. If the command

came from a REMOTE user, the contents of the message will be sent to

the other station. If the command came from the Sysop PMS Menu, the

contents of the message are displayed in the Communications Window

and also written to the log file if it is active.

When the message has been read, paKet will change its Status to "Y".

It is permissable to enter more than one message number on the line.

So you can for example, enter:

R 1065 1108

to read those two messages.

If the message number specified does not exist in the PMS DataBase,

paKet will respond with:

I can't find that message!

A REMOTE operator may Read any Bulletin but is allowed access to

Personal messages only if sent by or addressed to that station.

If an attempt is made to read another Personal message, paKet will

send:

You are not allowed access to that message

RM (Read Mine) - Read all unread messages addressed to me

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

[paKet] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

^

This PMS command is available in both the Sysop PMS Menu and to a

REMOTE user, although it is not shown separately in the REMOTE Menu.

Instead of Reading just one message at a time, you may type "RM" to

ask paKet to display the contents of ALL your unread messages.

A REMOTE user can do the same thing, asking for all unread messages

addressed to that station.

Page 296

S  (Send) - Send a message (eg S VK2XYZ)

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

[paKet] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

^

This PMS command is available in both the Sysop PMS Menu and

in the REMOTE Menu.

This is the command used to enter a new message into our PMS.

It should be noted that the message is not actually "sent" anywhere

at this stage.  It is recorded in our PMS DataBase.  If this message

is given a "To BBS" address, paKet will Forward the message to our

local BBS during the next Forwarding session, but if there is no To

BBS address, this message will stay here in our PMS until it is

Killed.

The Send command may be entered as "SP" or "S" (which is the same

thing) if entering a Personal message, or "SB" to enter a Bulletin.

paKet needs to know who the message is being "sent" to.  If you are

entering a message at the Sysop PMS Menu, you must enter a callsign

(or Bulletin topic) along with the Send command.  For a REMOTE user

the callsign is optional; if no callsign is entered, paKet will

assume the message is for you.

A common practice is to address a Bulletin with a topic of "ALL".

This is to be discouraged as far as possible because many Sysops and

many packet users like to sort their mail into well defined

categories.  A Bulletin addressed to ALL is a bit vague and is

likely to be ignored by busy people.  If you are going to the

trouble to write a Bulletin you want people to read it, so try to

make a "topic" that might give readers an idea what this one is

about. There are often dozens even hundreds of new messages to read

when accessing the BBS and few people have time to read them all.

So if your message does not have some title or description to grab

the readers' attention, all your effort might be in vain because few

people might read it.

When we ask for the BBS or paKet for a List of messages, it will

display details of each message on one line, which includes the

callsign or Topic, and up to 30 characters for the "Subject" (paKet

can display more but most systems limit the subject display to 30).

So if you feel your Bulletin is of such wide appeal that "ALL" is

appropriate, be sure to include a meaningful Subject with your

message, to try to attract the readers' attention.

Examples of a Send command:

SP VK2BZC                        Personal message addressed

to VK2BZC, may be read by that

station when he connects to this

PMS.

Page 297

SB PAKET                         Bulletin for this PMS only.

Anyone accessing our PMS can

read this message.

SP G1XGP @ GB7DUG.#32.GBR.EU     Personal message to G1XGP, but

this one has a "To BBS"

specified, so it will be

Forwarded to our local BBS later

for onward Forwarding.

SB PAKET @ WW                    Bulletin with a "To BBS"

destination specified.  This too

will be Forwarded to the BBS.

S                                A simple S (or SP) command is

permissable for a REMOTE user.

In this case the message will be

addressed to you and stored here

in your PMS.

When the PMS is ready to take the message it will prompt for the

message header. Eg:

Subject for msg 5 from VK2DHU to G1XGP?

The "Subject" is the text that is shown in the PMS Listing so try to

make it interesting and meaningful.  That could be the only part of

your message some users will see!

Then, finally, the following prompt is issued:

OK, send your message...(Ctrl-Z to end)

Any text that is entered after this will be recorded in the PMS for

this message. A <Ctrl-Z>, or "/EX" on a separate line, will

terminate the message. paKet then saves the message details in the

PMS DataBase and issues the following confirmation that all is well:

Message 5 stored

?  (Help) - Display details of the Sysop PMS Menu

Sysop PMS menu: <B,C,E,H,K,KM,L,LM,R,RM,S,?>

^

This PMS command is available only in the Sysop PMS Menu and is the

same as the Help command discussed earlier.

Page 298

Mail Forwarding

What is Mail Forwarding?

This is a system of exchanging messages via packet radio with any other

packet radio station anywhere in the world.

The system of Mail Forwarding was originally created by Hank Oredson

(W0RLI) where two compatible BBS systems could communicate with each

other using this special handshaking protocol.  Then we could leave a

message on one BBS and it could automatically send our message to the

other one without any manual intervention.

As the idea spread, we found a large number of these BBS systems were

talking to each other, sending messages back and forth.  When the

destination address of a particular message was the "next-door" BBS

that message was Forwarded easily.  But sometimes a message was

addressed to some far distant place which could not be reached by the

radio channels in use.  So the Forwarding system was developed to allow

a message to hop from one BBS to the next until it finally reached its

destination.   Marvellous stuff!

This sounds easy but the poor Sysops had trouble figuring out where to

send a message addressed to some unknown faraway place.  So the concept

of Routing Tables was developed.  Then a Sysop could specify that all

messages addressed to such-and-such would be forwarded to the

neighbouring BBS to the North; and these others addressed to so-and-so

destination would be sent to the neighbouring BBS to the South; and so

on.  With these Tables in place, the system just did it automatically.

It no longer had to know where every BBS was; it only had to know which

neighbouring BBS to send the mail to.

Hierarchical Addressing

The introduction of the Hierarchical Addressing system made this easier

too, because that address includes State, Country and Regional details.

For example, my packet address (using that convention) is:

VK2DHU @ VK2ATM.NSW.AUS.OC

That means my local BBS is VK2ATM and that is in the State of New South

Wales, which is in Australia in the Oceania Region.  A BBS on the other

side of the world trying to Forward a message to me might have to

look only at the OC in that address to know which direction to send the

message on its next hop.  As the message gets closer to home, say

somewhere in Australia, that BBS might look at the NSW part to identify

which direction to send it and finally as it gets somewhere within my

State, the VK2ATM BBS will probably be known.

We certainly don't expect a BBS in Outer Mongolia to know where VK2ATM

is!   The Hierarchical Addressing system makes it easier for everybody.

Some addresses include additional zone or local address information.

Usually these additional parts begin with a "#" and are accepted (in

many cases ignored) by Forwarding mechanisms in various BBS systems.

Page 299

We saw an example of this one earlier when I addressed a message to

G1XGP @ GB7DUG.#32.GBR.EU - the #32 is their local code. The BBS here

in Australia doesn't care about that.  All it is interested in is the

fact that this message has to go to Europe so it directs the message

towards the Gateway station that looks after our overseas mail.

Nowadays, the Routing mechanism is even easier with the development

of automatic Routing systems such as White Pages.

The BBS Network has developed significantly in only a few years, with

UHF, VHF and HF Gateways, even Satellite Gateways to move our mail.  I

hope all packet users appreciate the efforts, the dedication and the

contribution made by BBS Sysops all around the world.  Without these

guys, we would not have a Network like this to look after our mail.

Where paKet fits in to all this.

When I created paKet's Mail Forwarding system, I considered it

essential that it conform to the established standards.  But paKet is

not a BBS - it is a personal tool for a personal user system.  Its role

is to serve the needs of an individual user.

The role of a BBS is to serve the needs of the local packet community

and that is largely satisfied with its link into the Network for Mail

Handling and distribution.  It gathers our incoming mail from the

Network and it can also send into the Network any mail we want sent to

some other destinatino.

The role of the Network is to move our packets from one place to

another.  Mostly it is the various BBS systems around the country that

make up the links in this Network.  Those systems then are serving a

dual role: one role as a local Bulletin Board for its regular users,

and one role as a vital link in the Network for Mail Forwarding.

For paKet's Mail Handling, I decided to make this Mail Forwarding

system an interface to the local BBS only.  paKet is not considered to

be part of the Network as such - the BBS systems are doing a fine job

and introducing too many peers into that Network is asking for trouble,

I think.  And the BBS systems are already handling the complex Routing

information that would probably seem a little daunting to a lot of

paKet users.

You might think of the local BBS as your local Post Office.  All we

have to do is exchange our mail with that BBS, leaving it to worry

about the Routing and distribution details.

I have developed paKet's Mail Forwarding system so it conforms to the

standard RLI/MBL Mail Forwarding protocol which is supported by all

packet BBS systems in use today.  Provided you set up your system with

a few simple options, you too can have your mail automatically

Forwarded to and from any station anywhere in the world.

Page 300

Making Auto Mail Forwarding work for you.

To make it work for you, there are three things you have to do.

1. you have to set up the CTEXT parameter in your TNC; and

2. check the PMS Configuration in paKet's Online Configuration.

3. arrange with your local BBS Sysop to have your mail forwarded to

you at regular intervals.

Let's have a look at these things.

- 1  Set up CTEXT parameter

The CTEXT parameter in your TNC contains some optional text that you

can have sent to another station immediately that station connects to

your TNC.  Usually this might contain a welcome message such as:

Welcome to this paKet system... feel free to use the Menu.

or whatever takes your fancy.  Have a look at your TNC Manual for more

information on this parameter.  You might also ensure it is enabled -

ensure the TNC's CMSG setting is ON.

We have to change the CTEXT because we want the TNC to identify this

system as one that supports Mail Forwarding.  That identification

should be sent as the first line of data sent to a calling station if

we want to be sure it will recognise our SID.

Our SID!  Oh, haven't I explained a SID?

Before I get into an explanation of a SID, I should point out I have

taken much of the information in this section from documents produced

by other people.   Many thanks to Jim G6FCL for providing most of these

documents.  The information seems to be in the Public Domain but I had

trouble getting it all together.  Jim did that and sent it all to me.

I am not sure who the original authors are so I am unable to give

credit where credit is due, but I am sure readers of this Manual will

realise the work is a product of many dedicated amateurs who have left

their mark on our hobby with their valuable contributions.

SID

A SID is a  System IDentifer  which is a special code recognised by all

modern packet Forwarding systems, and is expected to be the FIRST piece

of information sent by a system that supports Forwarding.  So if you

make this the first part of your CTEXT, the TNC will send this

information to the other station before it sends anything else.

For paKet the SID is:

[paKet-6.1-HM$]

You could change your CTEXT by sending the following command to your

TNC:

CTEXT [paKet-6.1-HM$]

Page 301

If you wish, you can ADD that text to the beginning of any other CTEXT

information you want to send.  Do this by typing a <PASS> character

which is usually a <Ctrl-V> before you press the <Enter> key.  Then you

could, for example, type:

CTEXT [paKet-6.1-HM$]^V<Enter>Welcome to paKet.<Enter>

The <PASS> or <Ctrl-V> character tells your TNC the keystroke

immediately following is part of the text and therefore is NOT to be

taken as the usual command termination character.  The CTEXT command is

terminated when you type the second <Enter>.

I have also included an example of setting my CTEXT in the Begin Auto

commands so have a look at that in the Windows - Configuration (Begin

Auto commands) section of this Manual.

For those interested, here is a slightly technical description of a

SID:

The SID consists of three parts separated by a dash and surrounded

by square brackets.

The first part is the author/program identification. In our case

this is "paKet".

The second part is optional but if used, contains author/program

specific data. For our SID this is the paKet version number.

The third part is the most complex part.  This contains various

codes to identify the features supported by this station.  These

codes are as defined as follows:

C - Supports "forwarding" of date and time (obsolete).

H - Supports hierarchical location identifiers.

M - Supports message identifiers.

R - Supports extended forwarding responses.

Y - Supports YAPP binary protocol (unused).

$ - Supports BID. MUST BE LAST CHARACTER in the last part for

downward compatibility.

There are other features in use now including Compressed Forwarding

used by the FBB system.  But that was not included in the documents

I received.   paKet does not support that method anyway.

Our paKet SID contains HM$ in this field.

The existence of a SID implies that the system supports reverse

forwarding and OK/NO message rejection.

You didn't really need to know all that, but now you know anyway.  The

important thing is to add the standard paKet SID to your TNC's CTEXT

setting, or in your Begin Auto commands if there is a CTEXT command in

there.

Now, when another system connects, our TNC will send that SID to let

the other guy know we are able to handle standard Mail Forwarding.

Page 302

- 2  Check the PMS Configuration in paKet's Online Configuration.

The second thing we must do to set up our system for Mail Forwarding is

to check out PMS Configuration to ensure we have entered all the

relevant information there.

These things are covered in the Configuration Windows section of the

Manual so you could refer to that as well, just to confirm you have the

correct settings here.  Some of these things are critical for correct

Forwarding, others are not so critical but it is opportune to mention

them all here while we are looking at this Configuration Window.

Let's go through each of the REMOTE/PMS Configuration options:

- Allow REMOTE access?

- Issue REMOTE Menu on connection?

Both these items  must be Y because we want the BBS to be able to

connect and access our system for Mail Forwarding operations.

- SEND files  to  REMOTE station?

- RECV files from REMOTE station?

- REMOTE trigger

None of these items are relevant to Mail Forwarding operations.

- BBS callsign

We  need to recognise our BBS when it connects to our system.

Here you enter the callsign of your local BBS, including any

SSID.  For example, my local BBS is VK2ATM-1 and I must enter all

that into this field, including the "-1".

- Allow third party messages in PMS?

This is not actually required to be set for Forwarding but now is

as good a time as any to set this to suit your local regulations.

Most users would set this to Y but if third party messages are

illegal in your area then set this item to N.

- Delete message after Forwarding?

You can make up your own mind on this one.  If set to Y, paKet

will flag a Forwarded message with a status of K, so it will be

removed next time paKet does its PMS housekeeping.

If set to N paKet flags a Forwarded message with a status of F,

indicating it has been Forwarded, and that message stays in the

PMS until it is removed manually.

Page 303

- Call hourly (Mail, Always, No)

paKet can check the PMS every hour, at the appointed time, and

will take action depending on the option set here.  There are

three options available here:

M - Mail

This tells paKet to check the PMS every hour and make a

call to the BBS ONLY IF there is mail in there waiting to

be Forwarded.

This is the preferred setting.  There is very little

overhead if there is no new mail waiting, but if you do

enter a new message into the PMS you know it will be sent

within the hour.

A - Always

This tells paKet to call the BBS every hour, whether there

is any mail waiting in our PMS or not.

This one could generate additonal load on the frequency

because in most cases I suspect there will be no more mail

to send or receive.  Not every hour, surely?

But it does mean you will be in contact with the BBS at

lease once per hour, so if you are waiting on some urgent

or important message, this option helps to ensure you get

it within an hour of its arrival at the BBS.

N - No

This tells paKet to forget it.  Don't even check the PMS

every hour and certainly do not make any hourly calls to

the BBS.

- When (minutes past the hour)

This item is ignored if you specify No to the previous option.

But if you specify Mail or Always to the Call hourly option, paKet

needs to know when you want to do this.

Type a value between 0 and 59 here.  This identifies the minute in

each hour that you want paKet to perform its hourly processing.

- Call BBS once per day? (M, A, N)

In addition to the hourly option, paKet has a similar option to do

the same processing once per day.  This separate option is

provided so you can be sure you are in contact with your BBS at

least once per day.

If the BBS is set up to call you at least once per day you can set

this option to N and not bother to have paKet make another call.

Page 304

But if the BBS is not already calling you for a Forwarding

session, you can set this to Always.  This will ensure you are in

contact with the BBS at least once per day to collect any incoming

Mail that might be waiting for you there.

The three options, Mail, Always, or No are the same as described

earlier, although this time we are talking about a call once per

day not once per hour.

You could set this option to Mail, in which case paKet will call

the BBS once per day only if you happen to have any mail in your

PMS at that time waiting to be delivered.

The preferred options here are:

N if the BBS is already calling you; or

A if the BBS is not already calling you for Forwarding.

- What time of day to call ? (hh:mm)

This is the time of day paKet will do its PMS processing if the

previous item is set to M or A.

Enter the time in 24 hour format so if, for example, you want to

do this at 10 minutes past 9am, set this to 09:10 and if you want

it to be done at 2 minutes before midnight set this to 23:58.

- Use Script to make BBS connection?

- Connect path/Script file for BBS

When paKet is to make a call to the BBS to initiate a Mail

Forwarding session, it has to know how to establish that

connection.

It might be a simple CONNECT command is all we need but sometimes

a paKet user needs to make a complex connection via Network Nodes

to reach the desired BBS station.  In that case a paKet Script

will be required to provide the flexibility to handle the multiple

connections.

So we have two options here:

1. If you are able to connect to your BBS with a single CONNECT

command:

Enter "N" to the first of these two options because we do

NOT need to use a Script file to make this connection.

In the second of these two options, enter the callsign and

any digipeaters etc you need to connect to your BBS. For

example in my case I am within line of sight of my BBS so

all I need here is to enter:

VK2ATM-1

Page 305

but if I needed a digipeater to get there, I could enter

something like:

VK2ATM-1 v VK2RPM-1

or if I wanted to connect via the ROSE Network, I could

specify here:

VK2ATM-1 v VK2RPM 658300

The point to keep in mind here is that all of these examples

require just a single CONNECT command, no matter how far

away the BBS is.

2. If you are not so lucky and are unable to make contact with

your BBS with a single CONNECT command, you can use a Script

to make that connection.

For example say you need to establish contact with some

intermediate station, typically a Network Node, then once

you are connected to that station you need to issue another

Connect command to establish contact with another station

further on, and so on until you are connected to your BBS.

This procedure can be automated with a paKet Script.  The

Script can be set up to make the appropriate calls, each

time waiting for the proper response before sending the next

command.

In this case, you should specify "Y" to the "Use Script to

make BBS connection" option.

Then, in the second option you specify the name of the

Script File paKet is to  use to make that connection. Please

note the special syntax required to specify a Script File

name.  The file name must be entered in Pointer Brackets.

For example, if the Script File is BBSCONN.SCP you would

enter that as:

<BBSCONN>

As always, you do not need to enter the extension because

paKet will assume ".SCP" for a Script File.

Details on how to create a Script is covered fully in the

Scripts section of this Manual.

- 3  Make arrangements with your BBS Sysop.

It is provided in all modern BBS systems for Mail to be automatically

Forwarded to one or more other stations.  This feature is included in

the BBS software so it can participate in the Global Forwarding Network.

Page 306

The BBS Sysop maintains some Forwarding Tables in his system to

identify those other stations he wants mail to be Forwarded to. In

those tables, he identifies which messages are to go to which stations.

You could ask him to include your callsign in his regular Forwarding

scheduling and to Forward to your system all messages addressed to

you.  You could also ask if he would include any particular Bulletin

topics you are interested in too, such as Bulletins addressed to AMSAT.

He might need to record your callsign as a "BBS" too, in order to allow

his system to recognise you as one who can participate in a Forwarding

session.  This depends on the particular BBS system.  Your Sysop will

be able to help you here.

If he agrees to this (and don't forget that is his system and it is his

right to refuse) the Forwarding might take place at any time day or

night so it is best done only if you are prepared to leave your system

running 24 hours per day.  It can be frustrating to see the BBS calling

and calling and calling some station that is obviously off the air!

If you leave your system running, the Mail can be sent at a quiet time,

such as the wee small hours of the morning. This relieves congestion on

a busy frequency and also is convenient because your mail will be ready

and waiting in your PMS when you rise next morning.

We should remember though, that paKet is not to be considered part of

the Mail Forwarding Network - this is your private system and paKet

will not normally be Forwarding Mail on to the next link in the

chain!   We do our Forwarding only with our own BBS as a convenience

for more efficient Mail handling.

Forwarding Summary

Right.  Now you will find paKet will do the automatic Mail Forwarding

and you don't have to lift a finger.  Any messages that arrive for you

at the BBS will just appear in your PMS and paKet will display the

"(msg)" signal in the Status Window to alert you to the new arrival.

And any messages you want to send to anyone, are simply left in your

PMS and they will automatically be Forwarded at the appointed time.

(Don't forget to include a To BBS address if you want it Forwarded).

One more thing I would like to mention while we are talking about Mail

Forwarding.   If some OTHER station (other than our BBS) connects and

sends us a SID with a view to Forwarding some mail to us, paKet will

respond with the standard handshaking and will accept into the PMS any

messages the other station wants to send.   However, any outgoing Mail

will be Forwarded only to our own BBS.   While paKet is in Forwarding

Mode with some other station it will display "MAIL" in the Status

Window.

Page 307

(This page intentionally blank)

Page 308

PART IX - paKet PROTOCOL (pP)

What is pP ?

paKet Protocol, or "pP" for short, is a YAPP-compatible Binary File

Transfer protocol designed especially to provide a Recovery capability

after an aborted file transfer. It is basically the YAPP Protocol with

some additional enhancements to provide the automatic restart capability.

Consider the situation where you are sending a Binary File to another

station and the transfer is aborted for some reason, whether it be a bad

link or power failure or any other reason. You simply send the file

again, using the same <F7> command!  With pP, the file transfer will

automatically start from where the previous transfer left off.

pP will mostly be appreciated when you are transferring a large file and

the transfer is aborted when almost finished. For small files, or if a

transfer is aborted near the beginning, you might as well send the

entire file again.

pP may be used to exchange Binary Files with any other station equipped

with either pP or YAPP protocol. The pP protocol automatically detects

the presence of another pP station and will use the enhanced features as

required, otherwise the standard YAPP protocol will be used.

I used the YAPP Protocol Specifications published by Jeff Jacobsen

(WA7MBL), the author of the original YAPP program, when developing this

pP system. Extensive testing has shown pP to be fully compatible with

YAPP Protocol.

At this stage, pP is available only with paKet.  However, the paKet

Protocol details are offered to the Public Domain and are discussed in

some detail below. I will be happy to assist developers of other YAPP

Protocol software with the pP enhancements so they too can offer this

feature in their software.

But please note, I am offering to discuss the pP *ENHANCEMENTS* to

*EXISTING* YAPP Protocol programs. Please don't call me for assistance

with the development/debugging of Standard YAPP Protocol software!

In the sections that follow, I give some examples of the exchange of

dialog between the two stations. For convenience here, I will refer to

the two sides as the Sender and the Receiver. So, if I use the term

"Sender" or "Receiver" I might be referring to the operator or to the

software or to the system generally, but I am sure you will get the idea.

I hope Jeff Jacobsen will forgive me if I suggest his original YAPP

program has now been superseded but his YAPP Protocol lives on as the

current Standard for Binary File transfers in the world of Packet Radio.

In the following sections, when I refer to "YAPP", I am referring to the

YAPP Binary File Transfer Protocol, not to the original YAPP program

that started it all.

Page 309

Using pP

For most Binary File transfers, pP will behave in a similar manner to

YAPP and the performance will be the same.

No special action is required to use pP.  Using pP is the same as using

YAPP - the procedure to send or receive a Binary File is the same.

Refer to the <F7> and <F8> keys in the "Keyboard Commands" section of

this Manual for operational details.

Typical scenario:

Say you are sending a Binary File to another pP station. You send the

file in the usual way, starting with the <F7> key.

The transfer begins just like any other Binary File transfer starting

from the beginning of the file.

Then part way through, let's say after sending half the file, something

goes wrong.  Ummm, for this illustration we'll say the digipeater we

were using goes off the air and we lose the link with the other station.

So, call up using a different path and establish communications with

that station again.  Then start the same file transfer again, using the

same <F7> key again, specifying the same filename again. Come to think

of it, this is what you would do anyway, isn't it? Whether using pP or

not! But with most other protocols, the entire file would be resent from

the beginning. With pP, a Recovery is possible.

When the file transfer is started again, the following activity takes

place (assuming both stations support pP):

1. The exchange of pP/YAPP initialisation codes verifies each station

is ready to start the transfer (standard YAPP Protocol).

2. The Sender sends the Protocol header, which includes the file name

and file size (standard YAPP protocol). The header also includes a

new field to identify this station as a pP station.

3. The Receiver detects that this file already exists on that system

and is smaller than the one about to be received. (The file is

smaller because it is only half there!)

The Receiver now knows that the Sender is a pP station because of

the additional field in the header.

4. The Receiver requests a pP recovery and sends some details of its

(shorter) file.

5. The Sender checks those details against the file about to be sent.

It looks like the same file, so the Sender issues an approval then

starts sending from the agreed point in the file.

Page 310

(If the Sender decides it does not look like the same file, the

Recovery request would be denied and the transfer would start from

the beginning).

6. When the Receiver gets the Recovery approval, that system will

reset the file pointers and will start writing any data received

at the agreed starting point in the file, effectively appending to

the existing file.

7. The transfer then continues from that point using standard YAPP

Protocol.

While all this is going on, you do nothing - just watch the messages

informing you of the exchange of pP dialogue as outlined above. Then

when the Binary File Transfer Window appears you will see the file

transfer is already partly completed! It has picked up almost from where

it left off.

Page 311

The technical details of the paKet Protocol:

For this discussion I will be referring to some of Jeff Jacobsen's YAPP

Specifications. If you are a developer of YAPP compatible software, you

will already know about these things. If you are an interested user and

do not have these specifications handy, I will try to explain as I go.

A warning though... this section does get a bit heavy and is included

for the technically minded. If you find you are having trouble

understanding it, just skip to another section - it is NOT necessary to

understand all this in order to use pP.

Sender   - Sending a pP/YAPP Header

The YAPP Header is listed in the YAPP specifications as follows:

HD    Send_Hdr

<SOH> <len> (Filename) <NUL> (File Size in ASCII) <NUL> (Opt)

(The "< >" combination is used to indicate single byte values).

This header is sent at the beginning of the transfer to identify the

name and size of the file about to be sent. Note there is an optional

field provided, and it is this optional field that pP uses to identify

the Sender as a pP station.

When the Sender sends a file header, the "optional" field will contain

the pP Identifier string, "paKet-Protocol" (without the quotes).

So if we were sending a file "PROG.EXE" and that file was 74692 bytes

long, the pP header might look like:

<SOH><29>PROG.EXE<NUL>74692<NUL>paKet-Protocol

This optional field should be sent with every Header. If the Receiver is

running Standard YAPP Protocol it will not get upset because the

optional field is provided for in the Specifications. Testing with

various YAPP Protocol systems has shown this works without problems,

however should some system somewhere object to this optional field being

used in this way, the paKet user can switch off the pP option in the

Configuration - File Transfer options. In that case the Sender would not

send the optional field and the Receiver will think we are running

Standard YAPP Protocol!  I have yet to find a system that misbehaves so

you would normally leave the pP option ON in your Configuration.

Receiver - Receiving a pP/YAPP Header and requesting Recovery

When the Header is received, the Receiver will check the Header's file

details. If that file does not yet exist it will create a new file, send

an <ACK> to the Sender and continue with the transfer using Standard

YAPP Protocol.

If a file of the same name already exists on the Receiver system it

could be just another version of the same file or maybe it is the result

Page 312

of an earlier transfer attempt that was not completed. If it is a partly

completed copy of an earlier transfer attempt, pP can identify this and

perform a Recovery. If it is not that same file, pP can recognise this

too and revert to Standard YAPP Protocol for a "normal" transfer of the

entire file.

The Receiver will request a pP Recovery if the following conditions are

ALL met:

1. a file already exists with the same file name;

and

2. The size of the existing file is greater than 1000 bytes;

and

3. The file size is smaller than that in the Header just received;

and

4. the Header contains the "paKet-Protocol" identifier;

and

5. the user of this Receiver system has indicated in the

Configuration that pP may be used.

(This last condition is implemented in paKet but is an optional

feature of the pP Protocol).

Let's discuss briefly each of these requirements.

1. If the file does not yet exist, there is no need for Recovery and a

standard YAPP Protocol transfer will take place.

2. You will see later that a Recovery does not actually begin from the

end of this short file, but begins 750 bytes back from the end. If

the Receiver's file is shorter than 1000 bytes, it is hardly worth a

Recovery anyway!  We might just as well start again from the

beginning.

3. If the Receiver's file is already bigger than the one about to be

sent, it is certainly NOT the result of an aborted transfer of this

same file!

4. If the Header does not contain the pP identifier, the Sender is not

running pP. There is no point in asking a non-pP system for a

Recovery!

5. The paKet user may disable the pP facility, so the ultimate control

remains with the user. This option is provided in paKet but is not a

requirement of the Protocol. Another program that supports pP might

not provide this user option.

If all the above conditions are met and the Receiver decides to request

a Recovery, a "NP" record is prepared and sent to the Sender. This NP

record is defined below:

Page 313

NP record.

The YAPP Specifications include an NR record for a negative

acknowledgement ("Not Ready" sometimes referred to as NAK). The standard

NR record provides for the <NAK> code followed by an optional "reason"

in ASCII.

Using this format with the optional field to maintain compatibility

with the Standard YAPP Protocol, I have created a new Protocol record

and I called it NP. This new record is:

NP    pP Recovery request

<NAK><len>paKet-Protocol,(length of file),(first 100 bytes)(last 100 bytes)

- The <NAK> is the ASCII code with a value of 21;

- <len> is the length of the following data;

- The pP identifier string "paKet-Protocol" comes next (14 bytes).

This is the optional "reason" provided for in the YAPP Specifications!;

- The length of the file is actually the length of the file on the

Receiver's system LESS 850 bytes. (Further discussion on this below)

The value is sent as ASCII characters and please note there are commas

before and after these numbers.

So, if the file on the Receiver's system is 53,964 bytes, this field

would contain the following seven ASCII characters:

,53114,

- The first 100 bytes of the short file on the Receiver system;

- The "last" 100 bytes of the short file on the Receiver system.

To get the "last" 100 bytes the Receiver system will move to a position

850 bytes back from the end of the file and read the next 100 bytes

from there. So the last 750 bytes will be ignored and if a Recovery is

successful, those 750 bytes will be overwritten.

This figure of 850 bytes is somewhat arbitrary. I felt it was necessary

to provide SOME overlap to cover the situation where the end of the

short file might be corrupted (it WAS aborted for some reason, don't

forget!). If you are developing your own pP system, feel free to choose

another figure - this affects only the Receiver, so your system can

still perform a Recovery from paKet (or any other pP system). I would

suggest a larger rather than a shorter figure should be chosen if you

wish to change it. You might like to reduce the overlap to save time on

duplicating the data already sent, but keep in mind it takes only a

second or two to send 100 bytes and the Recovery could save many

minutes, maybe an hour! I don't think the user will get upset if it

takes a few seconds to Recover.

Page 314

So, as an example, an NP record might look like:

<NAK><221>paKet-Protocol,53114,........

(the dots represent the 200 data bytes).

Sender   - Processing a Recovery request (NP record) from the Receiver

After sending the Header, the Sender would usually get an RF packet

indicating the other station is now ready to "Receive the File". In that

case the entire file is sent from the beginning in accordance with the

Standard YAPP Protocol.

If the Sender gets the NP packet, it means the other station is

requesting a pP Recovery. It could be they already have part of this

file so it is now up to the Sender to decide whether it is or is not the

same file. Of course the only way to be really sure about this is to

compare the entire file but if we are going to do this we might as well

send the whole thing again!

I decided it is a reasonable risk to compare the first 100 bytes of the

two files, then to compare the last 100 bytes of the files. (The "last"

100 bytes in this case would be somewhere in the middle of the Sender's

file.) If these 200 bytes match - same values at the same locations and

the two files have the same name, then pP will accept they ARE the same

file and the Recovery will be approved.

The NP packet includes the size of the file on the Receiver's system,

the first 100 bytes plus the last 100 bytes of that file. So the Sender

system will access the file about to be sent and will compare the first

100 bytes. If they are all the same, it will move down the file to the

location corresponding to the file size given in the NP record. If the

next 100 bytes in the file also match those given in the NP record, a

Recovery will be approved.

If even one byte does not match, the Recovery will be denied.

So, if the Sender approves the Recovery, it will send the "Approval for

Recovery" packet, the "AP" record, which is a new Protocol record and

consists of two bytes:

<ACK><6>

Then it will position the file pointer to the file size given by the NP

record plus 100 bytes. (Those "last" 100 bytes have been matched so

there's no need to send them again.)  And the file transfer continues

from that point using Standard YAPP Protocol.

If the Sender denies the Recovery, it will send the "Recovery Denied"

packet, the "DN" record. This too, is a new Protocol record and consists

of 16 bytes to distinguish it from an "ordinary" NAK:

<NAK><14>paKet-Protocol

As the Recovery has been denied, the file will be sent in its entirety

and the transfer will proceed using Standard YAPP Protocol, starting

from the beginning of the file.

Page 315

Receiver - Processing Response to Recovery request.

This is the final stage of the Recovery process. The Receiver had

requested a Recovery and now will find out whether the Recovery is

approved or denied. The Receiver will get either an Approval (AP record)

or a Denial (DN record). Details of these records are shown in the

previous section.

If the Recovery is approved, the Receiver will adjust the file pointer

to the file size less 750 bytes.

Yes, that's right 750! We moved back 850 bytes into the file to include

the "last" 100 bytes in the the NP record. Now that we have an approval,

we know those "last" 100 bytes are OK so there's no need to transfer

them again. We will start receiving data from AFTER those "last" 100

bytes, overwriting the last 750 bytes in the file.  (See comments above

under "NP Record" - you may vary this "setback" figure of 850 bytes if

you are developing your own pP software).

If Recovery is denied, the entire file will be sent so the Receiver will

either have to overwrite the existing file or create a new one.

I would like to point out here that if the paKet program is receiving a

file and Recovery is denied, the existing (short) file will be preserved

and a new file with a different extension (eg: .000 or .001 etc) will be

created. This is a feature of the paKet program NOT part of the pP file

transfer Protocol.  pP just manages the Recovery process. So if some

other program uses pP it might handle "Recovery Denied" differently by

simply producing an error message such as "File Exists" or it might

overwrite the existing short file.

Page 316

PART X - TECHNICAL

Hardware vs Software Handshaking.

"Handshaking" is a technical term that refers to a method used to

regulate the flow of data between two devices - in this case between

the Computer and the TNC.

Handshaking is required to prevent loss of data. For example, it is

quite possible to send data to the TNC faster than the radio network

can take it, even at low baud rates. This is OK for a while because the

TNC has a memory buffer and the data will be held there until it can be

transmitted. However, there is a limit to the amount of data that can

be held in the TNC's memory, and if the computer continues to send,

some data will be lost. So, if the TNC's buffer is filling up it will

ask the computer (via its "Handshaking") to stop sending.

The reverse could apply, where the TNC could be sending data to the

computer faster than the computer can process it.  If you think about

it, a baud rate of 9600 means there is only one millisecond between

characters!  So, at that rate the computer cannot do much processing,

especially the slower computer systems. If the computer program was

unable to process the data quickly enough, it could use "Handshaking"

to ask the TNC to stop sending in order to allow the computer system

time to catch up.

There are two forms of Handshaking, both supported by paKet:

Hardware Handshaking and Software Handshaking.

Hardware Handshaking:

Hardware Handshaking is preferred, although it will work only if you

have the correct cable installed. A "normal" RS-232 cable will have 9

(or more) pins connected, some of which carry data but most of which

are used for various forms of hardware level communication. With this

method, the two devices (the Computer and the TNC) switch some of these

lines on and off to tell the other whether it is OK to send more data.

Should you wish to prepare a cable to use the Hardware Handshaking, the

following table shows the connections required (connections shown for

both a 9 pin and a 25 pin connector):

DE-9   DB-25   Signal Description

1       8     DCD Carrier Detect   Needed to enable the CONNECT check

2       3     RXD Receive Data     Essential!

3       2     TXD Transmit Data    Essential!

4      20     DTR Terminal Ready   Optional

5       7     GND Signal Ground    Essential!

6       6     DSR Data Set Ready   Optional

7       4     RTS Ready to Send    Needed for Hardware Handshake

8       5     CTS Clear to Send    Needed for Hardware Handshake

9      22     RI  Ring Indicator   Unnecessary for paKet

So, a cable can be constructed using a length of six-conductor flexible

cable (low-capacitance shielded is desirable but not essential) such as

Page 317

telephone flex with a plug on the modem end and socket on the computer

end, wired pin-for-pin according to the above table (it may also adapt

from DB-25 to DE-9 connector in the process).

If, say, the TNC wants to ask the Computer to stop sending, it will

switch off the CTS signal.  The Computer (in this case, paKet) will

stop sending data while the CTS signal is low (or off). When the TNC is

able to accept data once again, it will switch on the CTS line and

paKet will resume sending from where it left off. When this happens you

might notice the CTS "light" in the Status Window come on and off as

the two machines talk to each other.

Check your TNC Manual for any special connection requirements. Most

TNCs do not use the DTR, DSR or RI functions.

Software Handshaking:

Some so-called RS-232 cables have only 3 pins connected: one line for

Receive Data, one for Transmit Data and a Signal Ground. With these

cables, Hardware Handshaking is not possible and you must specify

Software Handshaking.

With this method, the two systems (the Computer and the TNC) must do

their handshaking via special data codes, conventionally known as

<XOFF> and <XON>.

If, say, the TNC wants to ask the Computer to stop sending, it will

send the <XOFF> code to the Computer.  When it is able to accept data

once again, it will send an <XON> character.   When this happens you

might notice in the Status Window, the words "Software Handshaking"

will be replaced by "XOFF" temporarily, indicating we are waiting for

the TNC to clear its buffers before it can accept some more data.

On a three-wire only cable, RTS and CTS should be connected together on

the TNC plug else the TNC may not talk to you AT ALL!

Setting the chosen Handshaking method in the TNC - XFLOW:

It is important to note that configuring paKet to either Hardware or

Software Handshaking will set up the COMPUTER to use that method, but

the TNC needs to be told too! It may not work unless both devices are

set to use the same method.

The TNC's handshaking is set with its XFLOW command:

for Hardware Handshaking, set XFLOW OFF

for Software Handshaking, set XFLOW ON.

If you encounter problems with File Transfers, my experience is that

the problem is often related to the incorrect setting of XFLOW in the

TNC. With paKet's online configuration facility, it is easy to change

from HW to SW mode and vice versa. To do this without changing XFLOW in

the TNC is to invite problems.

Page 318

In closing this section, it should be pointed out (in case it is not

already obvious) that if you do happen to have a full cable, you may

choose to use either Hardware or Software Handshaking. If you choose

Software Handshaking, the system will not use the additional

connections. The reverse is not the case, of course. If you do not have

a full cable, you don't have a choice!

Page 319

PREFERRED TNC SETTINGS

This list of TNC settings is not a complete set of commands for your

TNC. It is intended to be a guide to some of the preferred settings for

use with paKet.

There are some differences in some TNC commands - a command may be

available in one TNC and not another. Or, it may be known in other TNCs

by a different name. So, if your TNC does not have a command listed

below, just skip that one.

You will see in your TNC manual there are many more commands than I

have included here.  Those mentioned here are either critical for

correct operations with paKet, or perhaps are those whose default

settings should be changed.

8BITCONV ON

We do NOT want the top bit stripped from our data. You will see below

we have AWLEN set to 8 so we are using all 8 data bits. This is

especially useful if some graphics characters are being sent.  It does

not affect Binary transfers.

AWLEN    8

Use 8 data bits so we can send all characters including graphic

characters. You should also set PARITY to "None".

paKet must be configured to the same value - refer to the Configuration

Windows - Serial Port for details.

CHCALL   OFF

This must be OFF so we do not get the callsign of the other station

coming through with a change of stream character. (paKet remembers the

callsigns associated with the different streams).

This, in some TNCs, is the same as the STREAMCA in others.

CHDOUBLE ON

This is optional but STRONGLY RECOMMENDED to be ON.

If ON, you must set the corresponding option ON in paKet's

Configuration Windows - Multi User options.

This, in some TNCs, is the same as the STREAMDBL in others. If your TNC

has neither CHDOUBLE nor STREAMDBL you must set paKet's Configuration

option OFF.

CHSWITCH $7C (|)

This parameter is the same as STREAMSW in other TNCs.

Page 320

Have a look at my comments on the STREAMSW command below, but your case

substitute any mention of STREAMSW with your equivalent which is

CHSWITCH.

CMDTIME  1 second

Please leave this set to the default of 1 second.

CMSG ON

This should be ON so your TNC will send your CTEXT string to the other

station when a connection is established.  See the CTEXT command below

for further discussion.

COMMAND  $03 (Ctrl-C)

This must be left set to its <Ctrl-C> default setting.

CONMODE  CONVERSE

The only time paKet must use Transparent Mode is for Binary File

transfers, and paKet will switch automatically. So leave this set to

CONVERSE, otherwise Stream switching will not work correctly.

CTEXT [paKet-6.1-HM$]

The TNC's CTEXT command defines the text string that will be sent to

the other station when a new connection is established (provided CMSG

is ON).

For paKet's Mail Forwarding to work correctly we should ensure the

first thing we send to our BBS when it connects should be our SID, the

special System IDentifier which is discussed more fully in the PMS

section of this Manual.  The way to do this is to ensure that is

included in our CTEXT message.

You can include any other message after the SID if you want, with the

use of the <PASS> character.  This technique is discussed in the PMS

section of the Manual.

DCDCONN  ON

If you are using Hardware Handshaking, set this ON.

paKet will work either way, but the Status Display can then indicate

with its DCD "light" whether or not you are currently "Connected".

If set to ON, it may also be used by the paKet program to confirm the

"Connected" status of the TNC

paKet's Configuration Windows - Serial Port must also be configured to

the same setting chosen for the TNC.  But if your TNC does not switch

the DCD line, set paKet's option OFF.

Page 321

PREFERRED TNC SETTINGS (cont)

ECHO     OFF

paKet does its own echoing, so this should be OFF.

FLOW     OFF

paKet has its own Flow Control, so this should be OFF.

INTFACE  BBS

Kantronics TNCs have this command. Setting INTFACE to BBS will ensure

we don't get unwanted messages interfering with our processing.

MCON     OFF or 0

You may choose to have this option ON sometimes, however it will

interfere with REMOTE Mode and with Binary File transfers if ON - the

system cannot distinguish the Monitored data from the Binary data!

paKet will turn it off before a Binary File transfer anyway.

The preferred setting is OFF.

MDIGI    OFF

Preferred setting is OFF for the same reason as for MCON.

MYCALL  (your_callsign)

This hardly needs mentioning here because this is something that should

be set whether you are using paKet or something else.  But I mention

it, not only as a reminder to those with a new TNC, but also to remind

you to set your name and callsign in paKet's Miscellaneous options

Configuration Window.

NEWMODE  ON

This could be either way and is a matter of personal preference.

Most users seem to leave this at its default setting which is usually

ON.  That means when someone disconnects, the TNC will revert to

Command Mode.  That is fine where you usually talk to only one other

station at a time.

You may like to have NEWMODE OFF if you often work multiple connections

because then when someone disconnects, the TNC will stay in Converse

Mode.

Whichever way you set this option, you should set paKet's option the

same way.  This option is in the Multi User Configuration Window.  Have

a look at that section for more discussion on the NEWMODE command.

Page 322

NOMODE   OFF

Preferred setting is OFF so the TNC will switch to Converse Mode when

you establish a connection.

PARITY   0 (none)

This is related to the Word Length (see AWLEN above) and should be set

to "No Parity".

paKet must be configured to the same value - refer to the Configuration

Windows - Serial Port for details.

PASS     $16 (Ctrl-V)

SENDPAC  $0D (Ctrl-M)

START    $11 (Ctrl-Q)

STOP     $13 (Ctrl-S)

Please leave all these set to their default values.

STREAMCA OFF

This must be OFF so we do not get the callsign of the other station

coming through with a change of stream character. (paKet remembers the

callsigns associated with the different streams).

This, in some TNCs, is the same as the CHCALL in others.

STREAMEV  ON/OFF

paKet does not really need the stream id with every packet. This is

required only upon change of stream.  However in some TNCs, especially

those which do not support STREAMDBL, you might find it beneficial to

have this option ON in your TNC.  So if you are experiencing Stream

Switching problems, set this ON and see if it makes any difference.

You can have it ON all the time - the result will be some additional

traffic between the computer and the TNC but is unlikely you will

notice the difference.

There is no corresponding paKet configuration item for this.

STREAMDBL ON

This is optional but STRONGLY RECOMMENDED to be ON.

If ON, you must set the corresponding option ON in paKet's

Configuration Windows - Multi User options.

This, in some TNCs, is the same as the CHDOUBLE in others. If your TNC

has neither STREAMDBL nor CHDOUBLE you must set paKet's Configuration

option OFF.

Page 323

STREAMSW $7C (|)

Note, some TNCs (eg: AEA brand) use the CHSWITCH command to do the same

thing as STREAMSW.  So if your TNC is one of those just read "CHSWITCH"

whenever I mention "STREAMSW" here.

You may set this character to any value you wish, although most users

seem to retain the default which is the vertical bar character ("|").

It should not be set to one of the valid Stream Id characters, so do

not set STREAMSW to one of the letters A to J.

paKet must be configured to the same character (whatever character you

choose) and this is recorded in paKet's Configuration Windows -

Communications Windows settings.  Usually you would specify the same

STREAMSW character in each Communications Window.  And of course the

same value in your TNC.

If your TNC supports dual ports you will have two of these STREAMSW

characters - one for each port.  In that case you can choose which of

paKet's Windows you want to use for which port.

Suppose your two STREAMSW characters were set in your TNC to "|" for

the VHF port and "~" for the HF port.   Then you could specify (say)

"|" in Comms Windows 1 and 2, and specify "~" in Comms Window 3.  This

would mean that when you wanted to switch to the HF port, you could

simply press <Shift-F3> to select Comms Window 3!  Anything coming in

on the VHF port for Stream A would go into the first Window; anything on

Stream B to the second Window; and anything received on HF would go

into Window 3.

Of course you can choose any combination of Windows, Streams and

STREAMSW characters you wish.  I am just using these window numbers etc

as an illustration.

The important point here is that the STREAMSW character(s) your TNC

recognises MUST be the same as the character(s) you use in the

Communications Windows Configuration.

TRFLOW   OFF

TXFLOW   OFF

These commands are used for Software Handshaking in Transparent Mode.

You should set them both OFF. paKet will switch them on and off if

required.

USERS    n

Although not essential, it is considered desirable to set this

parameter to the same number of users as you have Communications

Windows. Then you know you can have a separate window for each of the

possible connections.

If your TNC allocates a separate callsign to its PMS I suggest the USER

parameter be set to the number of Communications Windows plus one.

Page 324

XFLOW    ON/OFF

This parameter is important and must be set to reflect the chosen

method of Handshaking that you have specified in paKet's Serial Port

configuration:

For Hardware Handshaking, you must have XFLOW OFF.

For Software Handshaking, you must have XFLOW ON.

Refer to the discussion of Hardware vs Software Handshaking for more

details.

XOFF     $13 (Ctrl-S)

XON      $11 (Ctrl-Q)

Please leave both these set to their default values.

Page 325

Kantronics TNCs

My comments in this section are directed at the KAM specifically but I

understand other models in Kantronics range operate in a similar manner

so in many cases these comments would most likely apply to the other

models as well.

Several KAM users have written to me about problems, mainly in the area

of Binary File transfers, with the previous versions of paKet.

paKet was written to suit the standard TAPR TNC (and compatibles) which

was the first of the popular TNCs used in Amateur Radio. They created a

Standard on which all modern TNCs are based.   paKet is being used on

many thousands of systems around the world with a wide range on TNCs, so

it seems the approach taken has been successful in most cases.

However, because of the differences that exist in Kantronics TNCs,

users of those TNCs should be aware of some potential operational

problems.

In this version of paKet I have made some internal changes in the paKet

processing to better handle these TNCs. I have had limited access to a

KAM for testing and have tried to accommodate the differences in that

TNC.  For example I have provided for dual ports and have improved the

code for the handling of multiple streams.   The big problem I have had

with this TNC is the lack of support for the standard STREAMDBL

command.   Without that facility in the TNC I am unable to guarantee

reliable Binary Transfers. However, it seems to work well both sending

and receiving Binary Files when using a KAM.

All I can say to users of the KAM and other Kantronics TNCs, is to give

it a try.  If a Binary Transfer fails it is probably due to some rare

combination of cimcumstances in that file, but in most cases it does

seem to work OK.

Perhaps the other thing you could do is mention this to Kantronics and

see if they have a fix for it yet.  The Kantronics TNCs I have seen are

first class units, apart from this one anomaly.  If it were not for the

omission of STREAMDBL, I am sure these TNCs would be considered among

the best in the world.

If you are experiencing some unusual and unwanted stream switching, you

might switch on the STREAMEV option in the TNC.  That will generate a

small amount of overhead, probably not enough to notice, but it might

offer some relief, helping to keep things coming into the right window.

Of course there are a considerable number of enhancements in the KAM,

supporting all those other modes such as RTTY, AMTOR, etc. I am not

referring to those changes, but rather to the standard Packet Mode and

the standard commands used by TAPR-compatible TNCs.

Page 326

The following differences have been noted in testing:

1. STREAMDBL command

Because the Kantronics range of TNCs do not support the STREAMDBL

command, you will have to set paKet's STREAMDBL option OFF.

STREAMDBL is a standard TNC facility to help avoid confusion where

the STREAMSW character is received over the air as part of a data

packet.

For example, the vertical bar character ("|") that is usually used as

the STREAMSW character is also often used as a pseudo graphic

character to draw vertical lines or for drawing boxes. When paKet

receives this character from the TNC it must decide whether the TNC

is indicating a change of stream or whether this is just another data

character received over the radio.

TNCs which support STREAMDBL can generate TWO of these vertical bar

characters whenever one is received over the air - then paKet can

tell the difference and there is no confusion.

Without the STREAMDBL feature, you could experience some occasional

(unexpected) changing of streams if you are monitoring the frequency

and happen to receive a STREAMSW character (say a 'vertical bar')

followed by some other character (say 'C').  paKet has no way of

knowing whether the TNC is requesting a change of stream (to stream C

in this example) or if those characters were received over the radio

because the KAM sends the SAME data to the computer in either case!

In order to support multi stream operations, paKet has no choice but

to process it as a change of stream!

With a view to improving reliability in Binary Transfers, I have

chosen to assume a STREAMSW character received during a Binary File

Transfer is actually a data character and not a change of Stream.  In

the testing I have done with the KAM, this has improved the success

rate significantly.  So you should ensure we don't get a genuine

change of stream (including dual port operation) while performing

Binary File Transfers otherwise my assumption could be wrong!

Another anomaly with the KAM is where responses to various commands

were going into a different Communications Window!  It seems in

Command Mode, the KAM might not respond to a command on the same

stream the command was issued.  The symptoms are paKet and the KAM do

not agree on what is the current Stream!

I cannot change the KAM so I cannot prevent these problems from

occurring. However I have added the <Alt-I> command, to help tidy up

your display if you do happen to experience these problems on your

system. The <Alt-I> command will initialise the Communications

Windows, clear any backlog in the other Windows, and reset both the

program and the TNC back to the first Stream in the first

Communications Window.  Refer to the <Alt-I> command in the Keyboard

Commands section of this Manual for details.

Page 327

2. RESET used for soft start

When making changes to the TNC's Communications settings, the usual

procedure is to issue the TNC commands to make the desired changes

then, after making the changes to the computer's serial port the TNC

is initialised to make the new settings effective.

On most TNCs the command to re initialise is RESTART, but on the

Kantronics units the command is RESET.   This is not a problem for

paKet because I have added a configurable option in the Serial Port

options to allow you to specify either RESTART or RESET. So if you

are a Kantronics user, just specify RESET for that option and paKet

will issue that command whenever you make any changes to the Serial

Port parameters.

There is a KAM.HLP file supplied with this version of paKet.  If

you are using a KAM or any of the Kantronics TNCs, you can select this

Help File by specifying this file in your Directories and Files

Configuration Window.   Press <F10> to call up the Help details.

Page 328

PacComm PacTOR Controller (PTC)

This is a strange machine, quite unlike the other units from the

PacComm stable that I have seen.  It is rather obvious this was

developed by a team that was not talking to those who developed other

units.  And I suspect these people have not used a TAPR TNC either -

such is the difference in the style of operation.

But it does work well and I just LOVE all those pretty LEDs on the

front panel!  It handles PACTOR, AMTOR and RTTY Modes which are

intended primarily for HF operation.  I understand there will be an

optional Packet board available for VHF operation.

The commands to drive this unit are unlike any other so you will find

paKet's Keyboard Macros handy for the various codes this unit requires.

For example I think it would be convenient to have a macro for the

AMTOR changeover "+?".  If you use F11 or F12 for this it is all done

with a single keystroke.  That makes your operations more convenient if

you use this combination frequently.

Same goes for switching from say, PACTOR to RTTY modes, etc.  There are

several keystrokes involved in switching from one mode to another

especially if you want to enter Listen Mode when you switch.  Depending

on your own operations, you could set up Keyboard Macros for just those

things you use regularly.

Set paKet's Serial Port Configuration:

- the TNC cmd: exit code should be set to <Esc> because this is the

code required to begin a command!  When you need to enter a

command, press <Alt-B> which is paKet's standard method to return

a standard TNC to command mode.

NOTE- in AMTOR Mode, the Controller remains in Command Mode for

one line only!   So for multiple commands you must press <Alt-B>

before EVERY command.

- the Command to Initialise TNC is RESET.   Yep, in this case the

RESTART is the dangerous one that clears EVERYTHING from the

PacComm Controller!  So you should set this to RESET and ignore

the warnings paKet gives you when you do so.

- The PacTOR Controller's firmware supports only Software

Handshaking so set paKet's Handshaking option in the Serial Port

Configuration to S to ensure paKet too uses Software Handshaking.

The PacTOR Controller supports the PASS character but in a slightly

different way.  It is called the Ctrlchr and when you want to send a

control code (eg: <Ctrl-Y>) you would type <Ctrl-V> and Y.  On other

TNCs you would type <Ctrl-V> <Ctrl-Y>.  So for the PTC, you should

ensure you have nothing specified in the "Control codes to PASS"

configuration window.  (It would have been so much easier if they had

conformed to the standards, but...)

The PTC's date format is different too!  So you will have to disable

the automatic DA command in the Miscellaneous Configuration options.

Page 329

DSP-12

The DSP-12 has proved popular with the Satellite users because of the

sheer flexibility of this unit.   There are other brands of DSP

Controllers available but to date I have had no experience with them so

cannot comment on those.

There are a couple of local DSP users including Carl, VK2JJM.  I asked

Carl for any thoughts on the subject and he produced this little gem.

Thanks Carl.  I did modify it a little but the wisdom contained herein

is essentially Carl's.

Operational Hints for using the DSP-12 with paKet.

(Written by Carl, VK2JJM)

(Slightly edited by Tony, VK2DHU)

The DSP-12 will work successfully with paKet if configured correctly.

The DSP-12 has several extra features not available in a standard TAPR

TNC, but it also lacks some standard TAPR commands.  The following

list will help setup the DSP-12 and paKet for basic operation.

These notes refer to the current release of the DSP-12 firmware at the

time this was written, in early 1994.

1) The DSP-12 does NOT support multiple stream connections in VHF

packet.  You should to set the maximum number of Comms Windows in

paKet to 1, and also set the STREAMDBL option OFF in the Multiple

Users Option menu because, with only a single Stream, this is

irrelevant.

2) The DSP-12 does not appear to support hardware handshaking so, for

reliable handshaking, you should set paKet's Handshaking to SW in

the Serial Port Configuration.

However it seems the DSP-12 does switch the DCD line upon

connection to another station.  If you want to use paKet's DCD

option to verify a connection, you will need to configure paKet for

Hardware Handshaking!

With the large buffer space available in this unit, some users have

found handshaking somewhat unnecessary anyway and prefer to have

the system set to Hardware Handshaking and have configured paKet's

DCD option on.

3) The DSP-12 has a nasty habit of exiting the <VHF PKT> mode and

returning to the Main Menu if the PTT 3 minute timer is enabled in

the Debugging menu. This timer is only used for full-duplex

Satellite Modes, so disable it. This setting is NOT stored in

bbRAM, so you will need to set it each time you enter the <VHF PKT>

mode.

Page 330

4) The DSP-12 has about 500k RAM free when operation from ROM, and

about 300k RAM when operating from RAM.  Most of this memory is

allocated to buffers so when sending a Binary File, paKet can often

send the entire file to the DSP-12 buffer, and then waits for the

other station to acknowledge the End-of-File. It can take quite a

while for the other station to acknowledge the EOF depending on how

much data is still in the DSP-12 buffer.  If the standard 2 minute

timeout expires, paKet will ask you how much longer you want to

wait. Give it say 10 minutes, longer if you wish, and paKet will be

happy to wait for that period before asking you again.  It

remembers that time so from now on it will always wait 10 minutes

(or whatever) before reminding you again.

5) When in the mailbox mode the DSP-12 will still send the connect

message defined in CText straight after the Mailbox user menu.

This gets a bit confusing to anyone connecting to the DSP-12

Mailbox.

The Mailbox in the DSP-12, as in most other TNCs, does not have the

same rich features found in paKet's PMS, so use the PMS whenever

paKet is running!

Set "MAILBOX OFF" and "CMS ON" in paKet's Begin Auto commands.

Do the reverse ("CMS OFF" and "MAILBOX ON") in paKet's End Auto

commands.

6) Some of the DSP-12 default values need changing for ideal VHF

packet operation.  The most obvious are DWait and FRack.  Set these

to 16 and 3 respectively.

Page 331

Running paKet under Microsoft Windows

Although paKet uses a number of windows for its operations and various

displays, these windows are paKet's own.  They are NOT "Windows" in the

now popular sense  - meaning part of the popular Microsoft Windows

product.

paKet is a DOS, text-based program and will run on any IBM compatible

hardware including the original PC with its 8088 processor, although

you will need more than its original complement of 64KB RAM!  It takes

me back a bit, but I remember the first PC I saw had a whopping 128KB

RAM and one single sided 160KB diskette drive!  And that lovely

monochrome display.  No hard disk of course - you wouldn't need THAT on

a PC!  Oh, and it had a cassette tape interface. (Sigh....memories...)

Uh, where was I?

With a modern system running Microsoft Windows Enhanced Mode, you can

run multiple DOS tasks.  Several people have reported paKet runs quite

happily in the background as a DOS task.  I too have tried it and found

it ran perfectly well for a while, but lately it seems to miss a few

characters from time to time.  I suspect it has something to do with

the latest Microsoft Mouse driver (version 9) which I installed

recently, but I have not yet identified the cause for sure. It seems

fine while I am not using the mouse!

If that happens to you try reducing the baud rate between the computer

and the TNC.  That gives the system a little more time to process the

incoming data characters.

When running under Windows (by the way, when I type "Windows" with a

capital W here, I am referring to the Microsoft Windows product, OK?),

paKet is using its own Communications Interrupt drivers and not using

the COMM drivers that are part of the Windows system.  So there might

be some sort of conflict there. Anyway, I find it works pretty well

most of the time and the convenience of multi tasking is worth any

slight problems.  It seems to run a little better under OS/2 than under

Windows and I will talk some more about that later. It means I can have

paKet running all the time, including those times when I want to use

the system to do other things.

The current version of Windows as I write is version 3.1 (or 3.11).  By

the time you read this there might be the new version 4 (Chicago?) or

even later versions available and things could be quite different?

To run paKet as a Windows task, you should use a PIF file.  I have

included a PAKET.PIF file with the original paKet distribution files,

so have a look at that.  Note the Background Execution option is

enabled and I think you will find it works better if the EMS and

Application Memory is locked (if you have enough RAM to do that). Those

options are in the PIF file and you can see them with the PIF Editor

you got with your Windows system. Have a play with the settings and I

am sure you will find it will work fine for you.

I have also included a hastily prepared Icon.  Maybe someone will send

me a better one?

Page 332

Try setting a Shortcut key combination - it makes it so easy to jump

directly into the paKet window, no matter how many other things you

have cluttering up the system.  I used <Ctrl-Alt-P> and that is

set in the File Properties dialog box.  Highlight the paKet icon and

select File then Properties from the Program Manager Menu.

I often have paKet running in a Windows window rather than in Full

Screen Mode.  That allows me to have a small part of paKet on display

in case I want to watch what is going on while I am working on

something else.  You can drag the corners to enlarge it to full size

and still find it occupies less than the full display.  Click on the

little box at the top left of the window and you will see the Fonts

option. Experiment with those and you will find (if your screen is big

enough and good enough) you can have a full, and readable, 80 x 25 text

display in a small font so it occupies just a small part of the Windows

display. There are so many options for presenting the display I will

leave you to experiment and find a combination that you are happy with.

You should also experiment with the Time Slicing options too - I have

set it to 100 Foreground and 60 backgound, but other settings seem to

work well too.  This is fine tuning and the optimum settings may be

different on different systems.

If you switch paKet to a Full Screen (press <Alt-Enter> - note that must

be the LEFT Alt key) you will find paKet will run faster because the

display system can now run in true text mode instead of the pseudo

text/graphic mode that Windows must use to manage the display with the

the smaller fonts.

I prefer Full Screen mode when I am actually USING paKet on air. It

runs most of the time as a Windows window and when I want to do

something, I switch into that paKet window with <Ctrl-Alt-P> then press

<Alt-Enter> to go Full Screen.

By the way, I think you will find the Morse Alerts a boon while paKet is

running in the background - then any particular alert strings you are

looking for will still make their unique sounds even if paKet is not

showing at all.

A Windows version of paKet?

In anticipation of the inevitable enquiries, let me say I am aware of the

proliferation of Windows systems "out there".   Windows demands lots of

RAM and CPU power, but it seems there are a lot of people using Windows

now.   Whether it is because Windows is so much better or whether it is

just good marketing by Microsoft, is not for me to say. I know some

people won't touch Windows at all, while others won't use anything else!

Let's just say I will have a look at making a Windows version available

at some later date.  But I must emphasise LATER.  It may be paKet will

go to a multi threaded system such as OS/2 before it moves to Windows.

I am not too sure yet.  For now it is a DOS program that will run fine

as a task under Windows or OS/2.  I have not done it, but I understand

it is also quite happy running as a task under DESQview.

Page 333

Running paKet under OS/2

At the time of writing this Manual (early 1994) OS/2 is still running

well behind Windows in terms of market share, but it seems to be

gaining acceptance, mainly from the power users who want the multi

tasking, multi threading facilities, 32-bit operating system, etc etc

but not necessarily the nice Graphic Interface offered with Windows

(including NT) with all the overheads associated with that type of

system.

Perhaps this is one of the reasons for Microsoft's success in the

market - their products are very well presented.  If IBM devoted some

more attention to installation procedures and support for a wider range

of standard, popular devices I am sure they will win even more converts

to this system.  (Neither my Panasonic CD-ROM drive nor my Canon Bubble

Jet printer are supported in the distributed copy of OS/2 version 2.1,

and these devices are high volume sellers - they are in this part of

the world anyway).

Of course OS/2 offers a Graphic Interface with its Workplace Shell, but

from what I have seen it does seem to perform better than Windows IF it

is tuned properly!  It offers far more tuning options and flexibility

than Windows, especially for the individual DOS Sessions, and it seems

to be a more rugged and reliable multi tasking system.

Unlike Windows NT with its exorbitant demands for memory and disk space,

OS/2 seems to require not much more than a standard DOS plus Windows

combination.  Maybe NT offers a lot more so I should not be making the

comparison?  I did install it for a client but have not used it myself.

There is a large number of paKet 5 users running OS/2 and they all

praise the wonders of that system.  OS/2, not paKet!        Well, both.

I too have been experimenting with OS/2 and have found that tuning some

DOS Settings can make paKet fit in to the OS/2 environment much more

effectively.

When running paKet in a DOS Session using all the standard defaults, the

Pulse Utility program indicated paKet was going flat out, using all

available CPU time, even when there was "nothing happening".  I can

understand that because paKet is written as a DOS program and in that

single tasking environment it must run around that loop waiting for

something to happen, looking something to process, updating the Status

display, etc.   Of course it is running flat out - it was designed that

way!

But changing a few of the DOS Settings for that session made a big

difference to the system performance.  I changed IDLE SECONDS to 0 and

IDLE SENSITIVITY to 25.  With these settings OS/2 was able to determine

that paKet was in an idle Keyboard loop, even though paKet IS going

flat out around that loop waiting for something to happen.  The Pulse

indicator was now indicating an almost idle system (I had nothing else

running at the time to help avoid confusion in the Pulse readings).

Page 334

It still handles the interrupts OK so there are no lost input

characters.  And the keyboard responds OK too.  But the clock display

in the Status Window was updating once every 2 seconds while there was

no packet activity.  That suggests to me that OS/2 is managing its

resources very effectively - giving paKet all the time it needs when

there is some real work to do, and minimising the resources consumed by

paKet when there was very little to do.

So now paKet can run in its own DOS session, and while it has nothing

to process there is negligible overhead, giving the lion's share of the

system to other tasks!  As soon as something is received from the TNC

or upon any keystrokes, paKet is given its share of the system to

process its data promptly.

I suspect other systems might behave differently with these DOS

settings and some experimenting might be justified for different

processor speeds etc.  It is easy to experiment here - just leave Pulse

running as you change the DOS Settings for paKet's DOS session.  You

will see right before your eyes the effect of the different settings.

You should review all the DOS Settings and tune the system to your

liking.  Some others to look at are DOS_BACKGROUND_EXECUTION ON and

COM_HOLD ON.

My experience with this system is limited so it would be good if those

of you with more OS/2 experience could post some Bulletins to inform

other paKet users of the settings, configuration, tuning tips etc to

help others get the most from this powerful system.

With any multi tasking system, it might be a good technique to run with

a lower baud rate between the computer and the TNC.  Unless you are

running a 9600 baud radio link, there is little real need to run a high

terminal baud rate.  I think it helps the system to share resources and

you are less likely to experience problems if a lower baud rate is used.

Page 335

The PAKET.CFG file

paKet is VERY configurable as you will have noticed when looking at all

the options in the Configuration Windows.   All these settings are

stored in the PAKET.CFG file but you really do not need to know how

they are stored nor what it all means.  It would be only in very rare

circumstances, and then only if you are an experienced operator, that

you would bother to even look at this file.  It is normally managed and

updated by paKet's Online Configuration and I recommend you leave the

program to maintain this file.

However, in the interests of complete documentation I am including a

list of all items in this file.  I have added some comments where

necessary, but for most items I have simply identified which

Configuration Window they refer to so you can get full details by

referring to the Configuration Windows section of this Manual.

There are two main sections to the file:

- the keyword section which contains most items;

- the strings section which contains the optional strings, including

Keyboard Macros, Begin Auto commands, End Auto commands, and

Alerts.

The PAKET.CFG file - Keyword section

You will see there is a keyword for each item in this part of the

file.  It was not my intention that this file be viewed by paKet users,

so some of the keyword names might not seem all that meaningful.  But

most keywords will give some clue as to the configuration item it

relates to.

In any case they are shown here in the same sequence they appear in the

Online Configuration Windows so you can more easily identify which is

which. The first item, VERS, must be the first line in the config file,

but the sequence of all other keyword items is not important. It is

likely they will appear in a different sequence in your config file.

In your file, you should see the same keywords but in many cases the

values to the right of the '=' will be different:

VERS=paKet 6.1           This must be the first line in the file.

These items are from Option 1, the Serial

Port configuration

PORT=2

BAUD=1200

PARITY=N

AWLEN=8

STOPS=1

HSHAKE=HW

DCDCON=Y

BREAKCODE=B

RESTART=RESTART

Page 336

These are from Option 2,the Multi User

options

MAXWINDOWS=3

DOUBLE=ON

NEWMODE=ON

STTYPE=A

These are from the Option 3, Communications

Window configuration.  In that Window there

are 10 columns, one for each of paket's

Communications Windows.

Each of the following items has 10 values

separated by a comma, with each value

corresponding to the 10 columns in the

Configuration Window.

STREAMID=A,B,C,D,E,F,G,H,I,J

STREAMSW=|,|,|,|,|,|,|,|,|,|

INBUF=1,1,1,0,0,0,0,0,0,0

FBBUF=64,48,16,2,2,2,2,2,2,2

WWSTART=48,48,48,48,48,48,48,48,48,48

WWEND=79,79,79,80,80,80,80,80,80,80

BELLS=575,1,1,1,1,1,1,1,1,1

PHONE=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

PROCESS=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

STATUS=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

TA=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

TALINES=2,2,2,2,2,2,2,2,2,2

The following items are not shown in any

Configuration Window but are stored in the

config file so paKet can maintain separate

settings for Single Line, Justified and

Word Wrap modes.  These are set with the

<F4> key.

SINGLE=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

JUSTIFY=N,Y,N,N,N,N,N,N,N,N

WW=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

These are from Option 4, Colours which are

recorded for each of 10 different windows.

These are listed in the same sequence as

shown in the Colours Configuration.

COLMSG=WHITE,GREEN,LIGHTGREEN,BLACK,WHITE,GREEN

COLCOMMS=BLACK,CYAN,WHITE,CYAN,WHITE,BLUE

COLSTATUS=WHITE,BLACK,YELLOW,BLACK,LIGHTGREEN,BLACK

COLBIN=DARKGRAY,BROWN,WHITE,BROWN,WHITE,BROWN

COLHELP=WHITE,BROWN,WHITE,BLACK,WHITE,BROWN

COLTNCHELP=BLACK,LIGHTGRAY,YELLOW,BLACK,LIGHTCYAN,BLUE

COLTA=LIGHTGRAY,BLACK,WHITE,BLACK,WHITE,BLUE

COLCFG=BLACK,LIGHTGRAY,WHITE,LIGHTGRAY,WHITE,RED

#0OLDIR=BLACK,LIGHTGRAY,YELLOW,BLACK,WHITE,BROWN

COLMAN=BLACK,LIGHTGRAY,LIGHTCYAN,BLACK,LIGHTCYAN,BLUE

Page 337

In Option 5 of the Online Configuration we

have the Keyboard Macros, Auto Commands and

Alerts.  But those items are recorded

separately at the end of this config file.

We'll come to those shortly. The other

items in Option 5 are:

PASSCODES=GHITXZ

KISSOFF=192 255 192

These are from Option 6, REMOTE / PMS

options.

REMOTE=Y

AUTOMENU=Y

REMUP=Y

REMDOWN=Y

TRIGGER=29

BBSCALL=VK2ATM-1

THIRDPARTY=Y

DELFWD=N

CALLHOURLY=M

CALLMINS=06

CALLDAILY=A

CALLTIME=23:54

BBSSCRIPT=N

BBSPATH=VK2ATM-1

These are from Option 7 the File Transfer

options

NDW=16

BDW=48

NPACL=240

BPACL=252

PP=Y

ADDCTRLZ=N

These are from Option 8, Directories and

Files.

DEFAULTDIR=C:\PAKET\

TEXTDDIR=C:\PAKET\TEXT\UPLOAD\

TEXTUDIR=C:\PAKET\TEXT\

BINDDIR=C:\PAKET\BIN\UPLOAD\

BINUDIR=C:\PAKET\BIN\

LOGDIR=C:\PAKET\LOG\

PMSDIR=C:\PAKET\LOG\

SCRIPTDIR=C:\PAKET\TEXT\SCRIPTS\

MANUALFILE=C:\PAKET\PAKET.DOC

TNCFILE=C:\PAKET\PK-232.HLP

EDITOR=D:\EDWIN.COM

BUDDYS=C:\PAKET\PAKET.DIR

TYPECMD=C:\UTILITY\LIST

Page 338

The following items are not actually in the

Online Configuration at all!   These are

recorded here in the config file so paKet can

start up next time, remembering some file

details you used last time.  So, for example,

when you press <F5> to send a file paKet

remembers which file you used last time.

Then it can bring up the Directory Window

with the same file already highlighted.

DIRPATH=C:\PAKET\PAKET.CFG

EDITFILE=C:\PAKET\LOG\VK4WAC.072

SCRIPTFILE=C:\PAKET\TEXT\SCRIPTS\GETMAIL.SCP

F5DEFAULT=C:\PAKET\LOG\VK2JJM.267

F7DEFAULT=C:\PAKET\BIN\SCAN.LZH DUMPFILE=E:DMP

These are from the Option 9, the

Miscellaneous options.

MYCALL=VK2DHU

MYNAME=Tony

MSGLEVEL=A

MSGDELAY=3

CFGHELP=Y

CGASNOW=0

AUTOLOG=Y

LOGHDRS=Y

BLANKDELAY=60

ARROWS=F

DAYTIME=Y

These items do not appear in the online

configuration and some might not even

appear in your config file at all!

ALARMS=Y                 You may toggle Alerts with the <Alt-A> key.

CWALARMS=Y               These two items reflect the current

settings for the Alerts option.

DCDDELAY=30              May be used to increase delay for those

TNCs which are slow to switch the DCD

signal when a connection occurs.  Time is

in hundredths of a second so a value of 30

is 3 seconds.

DISPMEM=Y                Displays free memory.  Used for debugging.

(<Ctrl-Alt-M> toggles this display)

PMSMSG=1269              This is the next message number to be used

by the PMS.

PMSSUFFIX=A              When generating a BID for the Forwarding

system, paKet uses the message number and

your callsign to construct the BID.  This

is designed to ensure it uses a unique BID

for every message. Eg: $14_VK2DHU.

Page 339

But if you reset your PMS files and start

using the same message numbers again, you

might find the BBS rejecting some mail

because it finds this BID already exists!

You can add this line to the config file so

paKet will add a suffix to the BID to

ensure the new series of BIDs are still

unique despite using the same message

numbers!  Eg: $14A_VK2DHU

QUIET=N                  Quiet Mode is toggled with <Alt-Q>

WPM=20                   This is the Morse Code speed for the

Alerts.  This is adjusted with <Alt-J>.

YAPPWAIT=5               The default timeout period according to the

standard YAPP File Transfer Protocol is 2

minutes, but if you have a TNC with a BIG

buffer paKet might timeout after the 2

minutes, before that big buffer can be

emptied.  You can tell paKet to wait for

some other timeout period, presumably a

longer time.  That period is stored here in

this variable so next time you won't be

interrupted every 2 minutes!  If this has a

value of 5, paKet will wait for 5 minutes

before reminding you it is still waiting.

The PAKET.CFG file - Strings section

In this section at the end of the config file are 4 areas containing

some optional strings.  The four areas are separated by the mandatory

dividers which are the lines beginning with the "***".  These dividers

must be present in the file even if there is no other detail recorded.

The first area is the Keyboard Macros.  Each line contains the Keyboard

Macro number followed by the contents of that macro.  This corresponds

to the detail in the Keyboard Macros Window  (<Alt-Z>, 5, 1).

*** Start of Keyboard Macros

1 ~BMAIL ON<Enter>MDC<Enter>l<Enter>

2 ~BB<Enter>MAIL OFF<Enter>

3 ~BMCON OFF<Enter>C VK2ATM-1<Enter>

4 73s from Tony in Port Macquarie<Enter>

6 ~BXMITOK T<Enter>

8 <~s bbsconn>

9 ~B��D<Enter>

0 ^CD^M

11 !xtgold

12 Disconnecting from $u at $t...<Enter>������~BD<Enter>

Page 340

The second and third areas are the Begin and End Auto Commands and

these too are as shown in the respective Configuration Windows.

*** Start of Beginning Auto Commands

echo off

xmitok on

CTEXT [paKet-6.1-HM$^V

If you get the Menu, try: ^V

H for Help or^V

S to "Send me a message^V

...Tony

<~S MAILOFF>

<~S MORSE-ON>

*** Start of Ending Auto Commands

^S

CTEXT paKet is not currently running.^V

Please leave a message in the PK-232 Mailbox.^V

...Tony.

MAIL ON

echo on

xmitok on

conok on

The final area is the optional Alerts strings.

*** Start of Alerts

PAKET

TONY

DHU

#WX#EXPERIMENTAL WEATHER STATION AT PORT MACQUARIE

BZC

WAC

Page 341

(This page intentionally blank)

Page 342

PART XI - ADMINISTRATION DETAIL

The cost of paKet.

You don't want to know about this, do you?

paKet is offered to the world as a Shareware product for $25 (Aust) per

copy, or equivalent in your currency. If you would like a diskette copy

mailed to you, an additional $5 covers the cost of a diskette, postage,

and packaging.  These charges have not changed at all in paKet's 5 year

history.

For those who are unfamiliar with Shareware, let me explain briefly: A

Shareware product is one which may be copied freely and distributed to

others for evaluation. However, if anyone decides to keep and use the

program, it must be registered and the prescribed payment paid. It is

not a free Public Domain program.

You and I both know there will be many people who take it and use it

illegally, but with Shareware distribution it is possible to keep the

cost down to a minimum. If I decide to go commercial with paKet it is

going to cost you a lot more, partly because there will be some printed

documentation, but most of the increase will be in the costly

distribution through commercial and retail channels. It is likely to

cost many times its current price then. I think you will agree, its

current price of $25 is not an exorbitant price for the "best Packet

Radio software in the world".

I hope you are honest enough to pay the $A25 if you use paKet.

Authorised distributors

There are authorised distributors in many countries and if this copy of

paKet came via one of those distributors, their details are included in

the READ.ME file.

These people are experienced paKet users and should be your first point

of contact if you experience any problems.

If you have an unregistered copy of paKet, you will probably find it

easier to register through them too, especially if you do not normally

use Australian currency.  You can do the right thing with your local

distributor in your own currency.  Their price is equivalent to $A25

anyway, so take advantage of their service.

Please print REGISTER.DOC to get the Registration Form and send that

too if the distributor does not already have your details. They will

need your address etc so they can send your Serial Number to you.

Include your packet BBS details too. That could be handy if we need to

contact you for anything.

Page 343

Updates

When you register for paKet, you are registered for life!  Your paKet

distributor will be happy to send you any updates as they become

available, provided they can cover their costs. So if you have

registered an earlier version and would like the next update, send the

update fee shown on the REGISTER.DOC to cover the cost of diskette,

mailer box, postage, etc.  Perhaps you should wait until a new version

is announced before sending your money because I am not sure how many

more versions there will be!

Contacting the author

If you don't have a local distributor, or if for any other reason you

prefer to register with me directly, you can charge the fee to your

Visa, Mastercard, or Australian Bankcard.  Or if you prefer you could

send a Money Order or a cheque (that's "check" if you're American).

By all means feel free to contact me for support too but I do get a lot

of mail and it might take a while to get back to you.  I need not

mention that registered users will receive priority attention when

demands on my time are high.  So try your local distributor first and

if you are unable to achieve a satisfactory solution to your problem,

then send a packet message to me at:

VK2DHU @ VK2ATM.NSW.AUS.OC

I answer all packet mail I receive unless it is quite obvious a reply

is not required.  So if you have not had a reply within say a week or

two, you had better send a follow up message.  In the past I have been

told I had not answered some queries, and investigation reveals I did

not receive those queries.  Other times, I have found I had replied by

packet radio but my replies did not get through to the destination.

Packet radio is good, but there can be problems at times.  So, send a

follow up if I do not appear to have acknowledged your packet messages.

I do NOT appreciate telephone calls about paKet!

Try packet radio or the Post Office.  My home address is:

M. A. Lonsdale, VK2DHU,

6 Marsden Cres,

Port Macquarie 2444

Australia

Perhaps a more reliable medium would be to send me an E-Mail message

via the Internet to:

tonyl@portmac.apana.org.au

Page 344

Acknowledgements.

paKet is an evolving product with the number of paKet users growing

every day and with registrations coming in from the far corners of the

globe, including many places I had never heard of.  There has been an

enormous amount of feedback over the years and what you see in paKet 6

is to a large extent the result of the combined support from the paKet

community.

To an even larger extent it is the result of support and enthusiasm

from local paKet users who have continually helped me with testing,

with ideas and suggestions, and just as importantly (maybe more so),

with emotional support to keep it going.

Most people would have absolutely no idea of just how much time and

effort has gone into the development and maintenance of paKet.  There

are times when you glow with the feeling of success, with having

achieved something worthwhile; and there are times when, faced with

difficulties and in the depths of depression, you wonder if it is

worth the trouble to continue and maybe give up to go do something

else.  It is at those low times when the when the support and

encouragement from close friends carry you through to the better times.

There are lots of them I should thank.  They all know who they are and

they all know how much I value their contribution.  I sometimes think

it is a mistake to mention any names (or callsigns) because if I

mention one, then I should also mention that other one.  And then of

course, to be fair I must also add so-and-so to the list... and it goes

on and on.

Despite that risk, I feel it would be remiss of me to allow this

release to be distributed without thanking Paul, VK2BZC.  Paul has

expertise far beyond even my wild aspirations and without his support

paKet would not be anything like the successful product it is today.

Thanks Paul.

Although so many people have contributed so much, I want to emphasise

that any problems you have with paKet are my fault because I did the

programming.

Feedback

Please let me know what you think of paKet and let me know if you have

any ideas for improvement. While I cannot promise to incorporate all

suggestions, your opinions and ideas are welcomed. Of course I would

also like to know if you detect any .. er .. bugs .. in the system.

Please use and enjoy paKet, and if you give a copy to your friends,

please give them all the original files to ensure they get the full

paKet system.

73s

de Tony VK2DHU

Page 345

Dedication

In the paKet 5 documentation I

mentioned in my acknowledgements, the

unselfish devotion from a number of

people including Col, VK2KQX.  It was

only a very short time after that

version was released that I received

that dreadful phone call one morning to

tell me Col had suffered a Heart Attack

the previous evening and did not

survive.

We all have lots of friends in Amateur

Radio. We become friendly with

different ones for various reasons.

With some we have many things in common;

Some just happen to live nearby;

Some are naturally warm, friendly

and generous with their friends;

Some are just plain nice guys.

Col was all of these and while I am not

sure what it achieves, I would like to

dedicate paKet version 6 to the memory of

Col Pryce, VK2KQX.

Page 346

#