PART VI - REMOTE Mode

Overview

Description.

paKet's REMOTE Mode is a facility that allows other users to access and

use our paKet system, even while our system is unattended. A REMOTE

Menu will be issued to the other station and paKet REMOTE will respond

to any of the valid Menu options.

For your protection, the other station will have access only to the

directory or directories you specify. The rest of your system will be

both invisible and inaccessible to the other station. You may choose to

disallow all REMOTE operations; or you may disallow just file Uploads

or file Downloads. It is your system - you decide what the other guy

can see and do.  Refer to the section on "Configuration Windows -

REMOTE / PMS options" for details on choosing these options.

The operation of REMOTE Mode is somewhat similar to most popular BBS

systems currently available.  paKet has a PMS where the other station

can send and receive messages. Details of the PMS are covered fully in

its own section in this Manual.

While REMOTE Mode is active, the Status Window displays the word

"REMOTE" to the left of the time display.

Page 223

Activating REMOTE Mode.

REMOTE Mode can only be activated while we are connected to another

station.

There are three ways to put our system into REMOTE Mode:

1. Automatic

You can configure the system to automatically switch into REMOTE

Mode when another station connects to our system.  This includes

those times where a BBS connects to this system to conduct a Mail

Forwarding session.

The Automatic option is set in the Configuration Windows -

REMOTE / PMS options.

2. REMOTE Trigger

The other station can send the REMOTE Trigger code. This code

too, is configurable although most stations would use the

default which is the <Ctrl-]> (that is Ctrl key with

right-square-bracket or ASCII value 29).

Why such an unusual combination? Well it should be unusual and

unique otherwise we would find paKet switching into REMOTE Mode

unnecessarily! And most of the other codes were already being

used for something or were commonly used as special TNC command

values. <Ctrl-]> seemed to be about the only one left!

It was found in testing that some Commodore 64 systems cannot

send <Ctrl-]>. A DIGICOM user can send that code by pressing

<Ctrl-=>, that is: Ctrl and the "equals" key!  If you find some

users cannot manage to send the REMOTE Trigger I suggest the best

solution is to enable the Automatic REMOTE mode so they will get

the REMOTE Menu on connection and will not have to worry about

the Trigger code. For some it is easier to get out of REMOTE Mode

than to get into it!

The Trigger code must be sent as the first and only character on

the line if it is to be recognised by paKet as a REMOTE Trigger.

If you want to specify some other code for the REMOTE Trigger,

you can do that in the Configuration Windows - REMOTE / PMS

options.

If you intend to use this Trigger method to provide access to

your REMOTE system, I suggest you put something into the CTEXT

message to advise the other stations to press <Ctrl-]> (or

whatever) to activate REMOTE Mode.  The "Begin" and "End"

auto-commands can be used to alter this message according to

whether paKet is active or not.

3. You can press the <F3> key to switch into REMOTE Mode and send

the first REMOTE Menu to the other station.

Page 224

Leaving REMOTE Mode

There are four ways REMOTE Mode can be cancelled:

1. The other station can enter "B", which is the REMOTE Menu command

for "Bye" indicating the user is finished and wants to disconnect.

paKet will:

disable REMOTE Mode;

send "Logged Off" to confirm acceptance of the Bye command;

then issue the Disconnect command to the TNC.

2. The other station can enter "T", which is the REMOTE Menu command

for "Talk" indicating the user no longer wants to use the REMOTE

Menu, preferring to continue this session communicating with us

in normal communications mode.

paKet will:

send a message to the other station accepting the Talk command;

disable REMOTE Mode.

sound the "telephone" bell (if bells are enabled);

paKet will wait a while (usually about 30 seconds) to give you

time to respond to the other operator's call.  But if you are not

present, the program will advise the other station that you are

not "here" before returning to REMOTE Mode and reissuing the

REMOTE Menu.

3. You could press <F3> to manually switch the system out of REMOTE

Mode. The effect of this is similar to the other station sending

the "Talk" command.

paKet will send a message to the other station advising them they

are no longer in REMOTE Mode.

4. If the other station Disconnects or if the link between the two

stations fails (which causes a Disconnect in our TNC anyway)

paKet will automatically disable REMOTE Mode as part of the

Disconnect process.

Page 225

The REMOTE Menu - A to L commands

paKet REMOTE will issue the following Menu to the other station:

[paKet - VK2DHU] (A,B,D,H(elp),I,K,L,MD,MU,MW,P,R,S,T,U,V,W,YD,YU,YW,?) >

Each of these options is explained below.

The callsign shown will be the registered callsign for this copy of

paKet.  If you have not yet registered, the callsign is not shown.

paKet will then wait for one of those options to be entered by the

REMOTE operator.  Each of these options is discussed below.  If

something other than a valid option is received paKet will respond with

"*** Invalid Option".

Sometimes the program might issue lots of "*** Invalid option" messages

even without the other guy typing anything!   This can happen if you

have MCON ON and are monitoring other activity on the frequency.  If

this is the case, our computer might receive something OTHER than the

data sent by our REMOTE operator.  The program cannot know what is

typed by the other guy and what is monitored (etc) and if this happens,

it is inevitable the program will start sending those "*** Invalid

option" messages to the other guy who will be wondering what is going

on! So if you do happen to get (or send) some unwanted "*** Invalid

option" messages, check to see exactly what is coming in from the TNC,

not just what the other guy is typing.

The same situation might arise if you have ECHO ON in your TNC.  Then

when we transmit the Menu, the TNC will echo it back to the computer!

The computer doesn't know what was sent by the other guy or what might

have been generated from within the TNC.  All it knows is that this

string of "data" has just been received from the TNC so it will try to

process that as a REMOTE command.

Menu commands are:

A  - Abort a file transfer

This might be used by the other station if an ASCII text file

transfer is in progress and they decide the rest of it is not

required.  A quantity of data is probably already in the TNC's

buffer so that will be sent anyway.

This command can also be used to abort a long directory listing

(see the W, MW or YW commands below).

B  - Bye (logoff)

This option will disable REMOTE Mode and issue a Disconnect command

to our TNC to disconnect our contact with the other station.

Page 226

D  - Download an ASCII text file

This operation is a Download from the REMOTE operator's point of

view: an ASCII text file is being sent from our system to the

other station.

The REMOTE operator must send the required filename along with the

"D" command, for example:

D TEXTFILE.DAT

The file will be taken from our Text Send directory or one of its

subdirectories. If the desired file is in a subdirectory, the

directory path must be specified as well.  The Text Send directory

is specified in the Directories and Files Configuration.

I think an example is needed here. Suppose our directory tree

looks something like this and you have specified "C:\PAKET\TEXT\"

as our Text Send Directory.

C:\

���DOS

���PAKET

�  ���BIN

�  �  ���GAMES

�  �  ���MISC

�  �  ���SATS

�  �  ���UPLOAD

�  �  ���UTILS

�  ���TEXT               < These are the only directories

�  �  ���BASIC           < the Remote operator can see or

�  �  ���DOCS            < access with the D or W commands

�  �  ���MISC            < if C:\PAKET\TEXT\ is specified

�  �  ���HUMOUR          < as our Text Send directory.

�  �  ���UPLOAD          <

���UTILITY

Then as far as the Remote operator is concerned, our C:\PAKET\TEXT

directory, and its subdirectories BASIC, DOCS, MISC, HUMOUR and

UPLOAD are available. Our other directories such as DOS and

UTILITY are quite invisible and cannot be accessed.

So, if the file TEXTFILE.DAT exists in our C:\PAKET\TEXT

directory, the REMOTE operator can enter the command:

D TEXTFILE.DAT

and paKet will find (and send) that file.

If the Remote operator wants a file, LAFF.DAT, in our

C:\PAKET\TEXT\HUMOUR\ directory, the following command is

required:

D HUMOUR\LAFF.DAT

Page 227

An attempt to access another directory will result in an error

message being sent. For example if the Remote operator sent a

command:

D \DOS\READ.ME

paKet would innocently reply that it can't find that file, even if

that READ.ME file DID exist in our DOS directory!

If you do not want another station to transfer any of our files

you can disable this option - refer to "Configuration Windows -

REMOTE / PMS options" for details. The option would still appear

in the Menu but the REMOTE operator would get an error message if

it were selected.

H  - Help

?  - Help

The contents of the "REMOTE.HLP" file will be sent to the other

station if either of these commands are sent by the REMOTE

operator.

This file must be available in the default paKet directory as

specified in the Directories and Files Configuration.

A sample REMOTE.HLP file is supplied with paKet. This file is

plain ASCII text so you can change it to suit your own system. For

example if you do not wish to allow File Uploads, you might like

to remove that detail from your copy of the Help file.

I  - Information about this system

The contents of the "REMOTE.INF" file will be sent to the other

station.

This file must be available in the default paKet directory as

specified in the Directories and Files Configuration.

A sample REMOTE.INF file is supplied with paKet. This file is

plain ASCII text so you can change it to suit your own

circumstances. For example you might like to add some details of

your computer system, your TNC, your QTH, or any special interests

you have.

K  - Kill a message from paKet's PMS system

KM - Kill all "My" messages

The REMOTE operator may remove any message sent BY that station or

addressed TO that station.

Page 228

If one or more individual messages are to be deleted the command

must include the message number that is to be removed, eg:

K 5

It is permissable to enter more than one message number on the

same command line, separated by a space.  Eg:

K 5 7 22

Another option would be to enter "KM" which would remove all

messages addressed TO the REMOTE operator.

If possible, paKet will delete the message(s) from the PMS

Database and will advise the REMOTE operator of the result of this

operation.

Refer to the discussion on the PMS system in its own section of

this Manual for more details.

L  - List available messages in paKet's PMS Database

LM - List all "My" messages

Upon receipt of this command paKet will send a listing of messages

currently stored in our PMS Database, showing one message per line.

If 'L' is entered on its own, details of all messages in our PMS

will be sent.  Although all message headers are listed here, the

REMOTE operator will not be able to access someone else's Personal

message.

A selective listing may be produced by specifying a FROM or TO

callsign (or part of a callsign). To ask for a listing of messages

FROM a particular callsign, specify the "L<" followed by the

desired callsign.  Eg:

L< XX9ABC

For a listing of messages addressed TO a particular callsign use

"L>".  For example to list all messages addressed to VK9:

L> VK9

If 'LM' is entered, details of only the unread messages addressed

TO the REMOTE operator will be sent.

Refer to the discussion on the PMS system in its own section of

this Manual for more details.

Page 229

The REMOTE Menu - M (Baycom) commands

MD - Download a Binary file to a BayCom system

This operation is a Download from the REMOTE operator's point of

view: a binary file is being sent from our system to the other

station which presumably is using BayCom.

The REMOTE operator must send the required filename along with the

"MD" command, for example:

MD PROGRAM.EXE.

The file will be taken from our Binary Send directory (specified

in Configuration Windows - File/Directories) or one of its sub

directories. If the desired file is in a subdirectory, the

directory path must be specified as well.

For example, if you refer to the sample Directory Tree shown under

the REMOTE D (Download) command above, suppose you have specified

C:\PAKET\BIN\ as our Binary Send Directory and the required file

is USEFUL.COM in that directory.  The command required from the

REMOTE operator would be:

MD USEFUL.COM

If the file is in the C:\PAKET\BIN\UTILS\ sub directory, the

command would be:

MD UTILS\USEFUL.COM

If you were not in REMOTE Mode and were performing this transfer

via the <Alt-F7> key, paKet would ask which filename and directory

should be used on the BayCom system. But as this operation is being

performed by the remote BayCom operator, paKet won't ask that

question, leaving the BayCom operator to make his own decisions.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send the //WPRG command to the BayCom system

send the file

send the //WPRG OFF command to the BayCom system.

Refer to the section on the Binary File Transfer Window for details

of the display and also refer to the Keyboard Commands section of

this Manual, where the discussion on the <Alt-F7> key provides some

more detail on sending a Binary File to a BayCom system.

When the transfer is complete, paKet will return to REMOTE Mode

and the Menu will be re-issued to the other station.

If you do not want another station to transfer any of our files

you can disable this option - refer to "Configuration Windows -

REMOTE / PMS options" for details.

Page 230

MU - Upload a binary file from a BayCom system

This operation is an Upload from the REMOTE operator's point of

view: a Binary File is being sent to our system from the other

station which presumably is a BayCom system.

That file will be stored in our Binary Recv directory (specified

in Configuration Windows - Directories and Files). For your

protection, paKet will not allow the REMOTE operator to specify a

drive or directory.

The REMOTE operator must provide the filename and filesize along

with the "MU" command, for example:

MU PROGRAM.EXE,54257

We need the filesize because the BayCom system does not tell us

when the file transfer has finished. If the filesize is known

paKet can count the bytes received and close the file when they

have all been received.

If the filesize is not known, a 0 may be entered to allow the

transfer to continue but the file will have to be closed manually.

The BayCom operator can do this by entering the BayCom end-of-file

string:

<Enter>//WPRG OFF<Enter>

That is, the "//WPRG OFF" command must be entered on a line by

itself.  If that string is not entered, paKet will close the file

after 2 minutes of inactivity. If you are present, you can close

the file yourself with the <Alt-F8> key.

The preferred method is for the BayCom operator to specify the

filesize.

When the required information has been entered, paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

send the BayCom //RPRG command;

begin the transfer;

Refer to the discussion on BayCom Binary File Transfers under

<Alt-F8> in the Keyboard Commands section in this Manual for

more details.

When paKet determines the transfer is complete, the file is closed

and the REMOTE Menu is re-issued to the other station.

If you do not want another station to send any files to our

system, you can disable this Upload option - refer to

"Configuration Windows - REMOTE options" for details.

Page 231

MW - What files are available in the Binary Send Directory

If the REMOTE operator sends a "MW" command, paKet REMOTE will

send a sorted list of filenames found in our Binary Send

directory, including the directory names of any subdirectories

found in that Binary Send directory.

If a list of files in one of those Binary Send subdirectories is

required, the REMOTE operator may add that directory name to the

MW Command. Eg:

MW UTILS

Processing for the MW command is identical to that for the YW

command discussed below. The REMOTE operator could enter the YW

command to get an identical response. However, the BayCom user has

Upload (MU) and Download (MD) commands so this MW command is

provided for consistency.

Page 232

The REMOTE Menu - P to W commands

P  - Perform a Script

This is a new facility introduced in paKet version 6 and allows a

REMOTE operator to run a Script on our system.  The scripts

available to the REMOTE operator are those files with an extension

of .SCP that you choose to include in your Scripts directory.  You

can nominate your preferred Scripts directory in the Directories

and Files Configuration Window.

The Perform command would normally be entered by the REMOTE

operator with the name of the desired Script.  Eg:

P SENDFILE

and paKet would load and run the SENDFILE.SCP Script.

Note it is not necessary for the REMOTE operator to enter the .SCP

extension as the program will automatically add .SCP to the

filename given.  In fact, for your protection no other extension

is permitted!   Why?

Well, a REMOTE Script is no different to any other paKet Script

and may include commands such as a DOS command to format your hard

disk!!!   Therefore, if a REMOTE operator tries to upload a file

with an extension of .SCP, paKet will automatically rename it to

.SSS, and until you decide to rename it to the normal .SCP

extension and place it in your Scripts directory, it cannot be

Performed by a REMOTE operator.  Without this protection, a REMOTE

user with a mischievous bent could upload a nasty Script and

Perform it without your knowledge or permission.

For added security, I recommend you choose a Scripts directory

that is well away from your File Receive directory so any uploaded

Scripts cannot be Performed until you check them out.

If you decide you want to allow someone to upload and Perform a

Script without you having to be present to rename the file, you

could include a Script (let's call it MYCOPY.SCP) in your regular

Scripts directory, which contained a single command such as:

!COPY C:\TEXT\UPLOAD\MYSCRIPT.SSS C:\SCRIPTS\MYSCRIPT.SCP

This is a Script that contains a DOS command to copy the uploaded

Script file (with its .SSS extension) into the regular Scripts

directory with a valid SCP extension.   Then when the REMOTE

operator uploads his new Script into your Text Upload directory,

he could Perform MYCOPY which would then make the new one

available to any REMOTE operators' Perform commands.

If the REMOTE operator enters the P command without a filename,

paKet will respond with a list of available Scripts in the Scripts

directory.

Page 233

R  - Read a message from paKet's PMS

RM - Read "My" messages

The REMOTE operator wants to read one or more messages stored in

our PMS Database.

If one or more individual messages are to be read the command

must include the desired message number.  Eg:

R 4.

It is permissable to enter more than one message number on the

same command line, separated by a space.  Eg:

R 4 5 22.

Another option would be to enter "RM" which would read all unread

messages addressed TO the REMOTE operator.

The REMOTE operator may read any message sent BY that station,

addressed TO that station, or any Bulletin.

Refer to the discussion on the PMS system in its own section of

this Manual for more details.

S  - Send a Personal message to paKet's PMS system

SP - Send a Personal message to paKet's PMS system (same as 'S')

SB - Send a Bulletin to paKet's PMS system

The REMOTE operator wants to leave message in our PMS.  This

message might be for you or for someone else and it could be a

Personal Message or a Bulletin.

If a callsign is provided on the command line, eg: S VK2DHU the

PMS will record the message as being addressed to that station but

if no callsign is given, paKet will assume the message is to be

addressed to you. Your callsign will be that specified in the

Configuration Windows - Miscellaneous options.

Although this command is "Send", the message is not sent anywhere

at this stage - it stays here and is recorded in our PMS

Database.

This version of paKet supports Automatic Mail Forwarding using the

MBL/RLI protocol (which is the universal standard supported by all

BBS systems). So, although the mail is not sent anywhere at this

stage, it may be sent to the BBS later for onward Forwarding.

The REMOTE operator may enter a TO address which includes a

destination as well as the callsign.  For example:

SP VK2DHU @ VK2ATM

or even a more detailed destination, using the well accepted

hierarchical system:

SP VK2DHU @ VK2ATM.NSW.AUS.OC

Page 234

If there is an '@' address specified, paKet will mark this message

as one to be Forwarded to the local BBS.  Once it is in the BBS

Network it may be sent across the world!  So if you (or anyone

else) want to get a message to someone across the other side of

the world all you have to do to is to leave it in your paKet PMS

with a TO address!

If there is no destination address, eg:

S VK2DHU

the message will remain in this PMS until it is deleted.

The message will be recorded as FROM the Remote operator's

callsign which appears in the Communications Header Line while we

are connected.

Refer to the discussion on the PMS system in its own section of

this Manual for more details, including information on Auto

Forwarding.

T  - Talk to the paKet operator

This option will disable REMOTE Mode, ring the telephone-style

connect bell (if those Bells are enabled) and return our paKet

system to normal communications.

If Bells are enabled, the telephone bell will ring up to 10 times.

Your first keystroke will stop the bells ringing.

If you are not present, paKet will tell the REMOTE operator you

are not here and will return to REMOTE Mode, re-issuing the REMOTE

Menu.

U  - Upload an ASCII text file

This operation is an Upload from the REMOTE operator's point of

view: an ASCII text file is being sent to our system from the

other station.

That file will be stored in our Text Recv directory (specified in

Configuration Windows - Directories and Files). For your

protection, paKet will not allow the REMOTE operator to specify a

drive or directory. On my system I use a sub directory called

UPLOAD and anything that is sent to me will appear in that

directory. Of course you may use any directory name you wish,

including the Root directory if that's what you want.

The REMOTE operator must provide the filename along with the "U"

command, for example:

U TEXTFILE.DAT.

Page 235

This file will be closed when paKet receives a <Ctrl-Z>.

If you do not want another station to send any files to our

system, you can disable this Upload option - refer to

"Configuration Windows - REMOTE / PMS options" for details.

V  - Version number of this system

When this option is specified, paKet will send the version number

of this paKet program.

It may be useful for the REMOTE operator to know which version we

are currently using as some versions of the program might behave

differently.

W  - What ASCII text files are available

If the REMOTE operator sends a "W" command, paKet REMOTE will send

a sorted list of filenames found in our Text Send directory,

including the directory names of any subdirectories found in that

Text Send directory.

The available space on that drive will be sent. If our Text Send

and Text Recv directories are on different drives, the available

space on the Text Recv drive will also be sent. This allows the

other operator to see how much space is available before uploading

a file to our system.

If the REMOTE operator wants to see a directory listing of one of

the subdirectories, the desired directory name must be entered

with the W command. For example, using the sample directory tree

shown in the REMOTE D (Download) command above, suppose the REMOTE

operator wanted a listing of our BASIC directory, the following

command would be required:

W BASIC

Page 236

The REMOTE Menu - Y (YAPP or pP) commands

YD - Download a Binary file using pP or YAPP protocol

This operation is a Download from the REMOTE operator's point of

view: a Binary File is being sent from our system to the other

station using either pP (paKet Protocol) or YAPP protocol.

The REMOTE operator must send the required filename along with the

"YD" command, for example:

YD PROGRAM.EXE.

The file will be taken from our Binary Send directory (specified

in Configuration Windows - Directories and Files) or one of its

sub directories. If the desired file is in one of those sub

directories, the directory path must be specified as well.

"Wildcards" or multiple file specifications (eg: *.EXE) may be

used to cause downloading of more than one file in the same

transaction, provided the remote station also supports this

feature.

For example, if you refer to the sample Directory Tree shown under

the REMOTE D (Download) command above, suppose you have specified

C:\PAKET\BIN\ as our Binary Send Directory and the required file

is USEFUL.COM in our C:\PAKET\BIN\ directory. The command required

from the REMOTE operator is:

YD USEFUL.COM

If that file was in our UTILS sub directory, the command required

from the REMOTE operator would be:

YD UTILS\USEFUL.COM

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

synchronise with the other station; then

begin the transfer(s).

Refer to the section on the Binary File Transfer Window for

details of the display and also refer to the "Keyboard Commands"

section of this manual, where the discussion on the <F7> key

provides some more information on sending a Binary File.

When the transfer is complete, paKet will return to REMOTE Mode

and the Menu will be re-issued to the other station.

If you do not want another station to transfer any of your files

you can disable this option - refer to "Configuration Windows -

REMOTE / PMS options" for details.

Page 237

YU - Upload a binary file using pP or YAPP protocol

This operation is an Upload from the REMOTE operator's point of

view: a binary file is being sent to our system from the other

station using either pP (paKet Protocol) or YAPP protocol.

That file will be stored in our Binary Recv directory (specified

in Configuration Windows - Directories and Files). For your

protection, paKet will not allow the REMOTE operator to specify a

drive or directory.

It is not necessary for the REMOTE operator to enter the filename

with the YU command because that information is included in the

Protocol Header.

paKet will:

set up the necessary parameters in the TNC;

display the Binary File Transfer Window;

synchronise with the other station; then

begin the transfer.

When the transfer is complete, paKet will return to REMOTE Mode

and the Menu will be re-issued to the other station.

Refer to the section on the Binary File Transfer Window for

details of the display and also refer to the "Keyboard Commands"

section of this Manual, where the discussion on the <F8> key

provides some more information on receiving a Binary File.

If you do not want another station to transfer any files to this

system you can disable this option. Refer to "Configuration

Windows - REMOTE / PMS options" for details.

YW - What files are available in the Binary Send Directory

If the REMOTE operator sends a "YW" command, paKet REMOTE will

send a sorted list of filenames found in our Binary Send

directory, including the directory names of any sub directories

found in that Binary Send directory.

The available space on that drive will be sent. If our Binary Send

and Binary Recv directories are on different drives, the available

space on the Binary Recv drive will also be sent.

If the REMOTE operator wants to see a directory listing of one of

the sub directories, the desired directory name must be entered

with the YW command. For example, using the sample directory tree

shown in the REMOTE D (Download) command above, and assuming you

have specified C:\PAKET\BIN\ as the Binary Send Directory, suppose

the REMOTE operator wanted a listing of our GAMES directory. The

following command would be required:

YW GAMES

Page 238

Other REMOTE commands

*

; -  Any "command line" beginning with this asterisk (some call it a

star) or a semi-colon will be treated as a comment.  Although it

will be displayed on our screen, it will not be processed by paKet

REMOTE.

This is useful if you are present at the keyboard while the system

is in REMOTE Mode.

You might want to chat briefly to the other station and this

allows you to receive a message without having to return to normal

communications mode.  If the REMOTE user does not prefix his

message with this '*' or ';', paKet will try to interpret the line

as a valid command and will probably spew out that "*** Invalid

option" message!

The following "commands" are recognised by the REMOTE system but these

commands are used internally and not expected to be sent by a human

operator.

> -  BBS Prompt for Mail Forwarding

This character is used for paKet's Automatic Mail Forwarding. It

is sent by the other station's Auto-Forwarding logic (usually the

"other station" in this case is your local BBS) and triggers a

response from paKet's Mail Handling facilities.  Unless it is a

very experienced operator at the other end, it is unlikely this

code would be deliberately typed in manual mode.

I suggest you leave it alone and let the computer look after it.

[ -  First character of a System Identifier (SID) which is used by

BBS and other compatible systems in establishing Auto Forwarding

protocols.

This is not something that would normally be entered manually by a

REMOTE operator and as suggested above, leave this one alone and

let the computer worry about the details.

When a SID is received while we are in REMOTE Mode, paKet will

enter MAIL mode and will expect the other station to respond with

the standard MBL/RLI mail forwarding protocol.

For your information, our paKet SID is "[paKet-6.1-HM$]" and will

be issued as necessary when Mail Forwarding is initiated.

If you wish to automatically receive mail from the BBS, you should

have this SID text as the first line of your CTEXT in your TNC and

arrange with your BBS Sysop for your callsign to be included in

his Forwarding tables.

Page 239

(This page intentionally blank)

Page 240