������������������������������������������������������������������ͻ

�                               ���  ����                          �

�                               ��� ����            ���            ���

�          �������   ������     ��۱���   ������  �������          ���

�         �����߱��    ��߱��   ������   ����߱��  ߱����          ���

�         ���   ���  ��������   ��۱��   ��������   ���            ���

�         ���   ���  ��   ���   ��� ���  �������    ���            ���

�         ���������  �����ܱ��  ���  ���� ������     ����          ���

�         ��������    ����� ��   ��   ���  �����      ���          ���

�         ���                                                      ���

�        �����                 Version 6                           ���

�         ����                                                     ���

������������������������������������������������������������������ͼ��

�������������������������������������������������������������������

"The best Packet Radio software in the world"

by

Tony Lonsdale

VK2DHU

(This page intentionally blank)

TABLE OF CONTENTS

PART I - INTRODUCTION

Welcome to paKet ..............................................   1

Computers - how it all began ..................................   3

Digital Communications ........................................   8

Morse Code .................................................   8

RTTY .......................................................   8

AMTOR ......................................................  11

PACTOR .....................................................  12

Packet Radio ...............................................  14

Error Free Communications ...............................  14

Frequency Sharing .......................................  14

Multiple Repeater Operation .............................  15

Networks ................................................  17

TCP/IP ...............................................  18

NET/ROM ..............................................  20

ROSE .................................................  22

The BBS Network and Mail Forwarding .....................  25

Mail Forwarding ......................................  26

The role of the BBS and where paKet fits in ..........  26

Bulletins ............................................  27

Other Communications Modes ....................................  28

The paKet Documentation .......................................  28

PART II - paKet's WINDOWS

Overview ......................................................  29

Binary File Transfer Window ...................................  30

Communications Windows ........................................  31

Flashback ..............................  34

Flashback Text Find ....................  35

Write Flashback to disk ................  36

Configuration Windows  Overview ...............................  37

System Configuration ...................  38

Serial Port ............................  39

Multi User options .....................  43

Communications Windows .................  47

Colours ................................  52

Keyboard Macros/Auto commands (etc) ....  54

Keyboard Macros .....................  55

Begin Auto Commands .................  59

End Auto Commands ...................  60

Alerts ..............................  61

Control Codes to PASS ...............  63

KISS Mode OFF sequence ..............  65

REMOTE / PMS options ...................  66

REMOTE options ......................  66

PMS options .........................  68

File Transfer options ..................  73

Directories and Files ..................  75

Miscellaneous options ..................  80

Connect Window ................................................  87

Disk Directory Window .........................................  90

Help Window ...................................................  92

Message Window ................................................  93

Status Window .................................................  94

TNC Help Window ...............................................  99

Type Ahead Window ............................................. 102

PART III - SPECIAL KEY CODES

Overview ...................................................... 105

Ctrl-key combinations ......................................... 105

Alt-key combinations .......................................... 106

Function keys ................................................. 107

Other commonly used command keys .............................. 107

Macro substitutions ........................................... 108

PART IV - KEYBOARD COMMANDS

F1            Display Help Screen ............................. 109

F2            Turn Log File on/off ............................ 110

F3            Turn REMOTE Mode on/off ......................... 111

F4            Toggle Word Wrap and Single Line on/off ......... 112

F5            Send ASCII text file ............................ 113

F6            Receive ASCII text file ......................... 114

F7            Send Binary File using pP or YAPP ............... 115

F8            Receive Binary File using pP or YAPP ............ 117

F9            DOS Gateway ..................................... 118

F10           Display TNC Help File commands .................. 119

Alt-F1        Display Online Manual ........................... 120

Alt-F3        Call the BBS for Mail Forwarding ................ 121

Alt-F4        Exit the paKet program .......................... 122

Alt-F5        Send RAW Binary File ............................ 123

Alt-F6        Receive RAW Binary File ......................... 124

Alt-F7        Send Binary File to BayCom (or Digicom) ......... 125

Alt-F8        Receive Binary File from BayCom (or Digicom) .... 126

Alt-A         Turn Alerts on/off .............................. 128

Alt-B         Send BREAK to the TNC - return to Command Mode .. 129

Alt-C         Connect to another station ...................... 130

Alt-D         Display a Disk Directory ........................ 132

Alt-E         Edit a data file ................................ 134

Alt-F/Alt-L   Find text in Flashback .......................... 135

Alt-G         Get last message back again ..................... 136

Alt-I         Initialise Communications Windows ............... 137

Alt-J         adJust Morse speed .............................. 138

Alt-M         Personal Message System ......................... 139

Alt-P         Turn Print Log on/off ........................... 140

Alt-Q         Turn Quiet mode on/off .......................... 141

Alt-R         Rename/Move a data file (or Register) ........... 142

Alt-S         Script Processing ............................... 143

Alt-T         Set TNC's Time and Date ......................... 144

Alt-V         Verify TNC's Connected status ................... 145

Alt-W         Write contents of Flashback to disk ............. 146

Alt-X         Exit the paKet program .......................... 147

Alt-Y         Yawn (half second delay) ........................ 148

Alt-Z         Configuration ................................... 149

Insert        Toggle Insert Mode on/off ....................... 150

Delete        Delete a character or delete a file ............. 150

Backspace     Delete the character before the cursor .......... 150

Ctrl-Del      Delete a line ................................... 150

Ctrl-Ins      Insert a blank line ............................. 150

Esc           Return to previous operation .................... 151

ScrollLock    Hold all input data (pause) ..................... 151

Left-Arrow    Move the cursor left ............................ 151

Right-Arrow   Move the cursor right ........................... 151

Up/Shift Up   Move cursor up or Type Ahead recall ............. 152

Down/Sh. Down Move cursor down or Type Ahead recall ........... 152

Home          Move to start of text or window ................. 153

End           Move to end of text or window ................... 153

PageUp        Flashback - Move back a page .................... 153

PageDown      Flashback - Move forward a page ................. 154

Ctrl-PageUp   Flashback - Move to beginning of buffer ......... 154

Ctrl-PageDown Flashback - MOve to end of buffer ............... 154

Shift-Left    Change to previous Communications Window ........ 154

Shift-Right   Change to next Communications Window ............ 154

Shift-Fn      Change to another Communications Window ......... 155

PART V - Messages in the paKet system

Overview ...................................................... 157

Messages starting with A ...................................... 158

B and C ................................ 163

D and E ................................ 167

F, G and H ............................. 172

I ...................................... 174

"I can't" .............................. 175

I (cont) ............................... 180

K, L and M ............................. 186

N ...................................... 189

P ...................................... 192

R ...................................... 197

S ...................................... 201

T ...................................... 205

U, V and W ............................. 216

Y and Z ................................ 220

PART VI - REMOTE Mode

Overview ...................................................... 223

Activating REMOTE Mode ........................................ 224

Leaving REMOTE Mode ........................................... 225

The REMOTE Menu Commands A to L commands ...................... 226

M (BayCom commands) .................. 230

P to W commands ...................... 233

Y (YAPP or pP commands) .............. 237

Other REMOTE commands ......................................... 239

PART VII - SCRIPT PROCESSING

Overview ...................................................... 241

Running a Script File ......................................... 242

Script Tutorial - S command ................................... 243

B and D commands ............................ 245

W and Q commands ............................ 247

G and : commands ............................ 249

* and E commands ............................ 251

C and R commands ............................ 254

AD, AU and AC commands ...................... 258

YD and YU commands .......................... 258

! command ................................... 262

T command ................................... 264

FC, FS and FT commands ...................... 269

Variables ................................... 270

M command ................................... 270

Passed Parameters ........................... 271

Scope of Variables .......................... 273

I command ................................... 275

? and J commands ............................ 275

L, P and X commands ......................... 277

The Script File syntax ........................................ 278

Script Command Summary ........................................ 279

PART VIII - PERSONAL MESSAGE SYSTEM (PMS)

Overview ...................................................... 285

The Sysop PMS Menu ............................................ 285

PMS commands in the REMOTE Menu ............................... 286

Accessing the PMS - B, C and E commands ....................... 287

H, K and L commands ....................... 292

R and S commands .......................... 296

Mail Forwarding ............................................... 299

Making Auto Mail Forwarding work for you ...................... 301

PART IX - paKet PROTOCOL (pP)

What is pP ? .................................................. 309

Using pP ...................................................... 310

The Technical details of the paKet Protocol ................... 312

PART X - TECHNICAL

Hardware vs Software Handshaking .............................. 317

Preferred TNC Settings ........................................ 320

Kantronics TNCs ............................................... 326

PacComm PacTOR Controller ..................................... 329

DSP-12 ........................................................ 330

Running paKet under Microsoft Windows ......................... 332

Running paKet under OS/2 ...................................... 334

The PAKET.CFG file ............................................ 336

PART XI - ADMINISTRATION DETAIL

The cost of paKet ............................................. 343

Authorised distributors ....................................... 343

Updates ....................................................... 344

Contacting the author ......................................... 344

Acknowledgements .............................................. 345

Dedication .................................................... 346

(This page intentionally blank)

PART I - INTRODUCTION

Welcome to paKet.

paKet is one of the world's most popular programs for amateur radio

communications and if this is your first look at paKet I hope you will

soon agree it IS "the best Packet Radio software in the world".

paKet is a communications program developed especially for use with

Packet Radio, although it can be used for other modes with the

appropriate equipment.  It is designed to run on any IBM compatible

computer system and to communicate through a standard TAPR-compatible

Terminal Node Controller (TNC).

The program is very easy to use, even for newcomers to this field.

As you gain experience with the system, you can explore some of the

more sophisticated features which are provided to meet the needs of

even the most demanding packet operators.  It is simple if that is what

you want; and it is sophisticated if that is what you want.

The modern trend is towards Graphical Environments with the likes

of Windows, or OS/2.  paKet is written to run as a plain DOS program so

if you have an old XT with a CGA or even a Mono display system

gathering dust in the corner, paKet will run on that just fine. Packet

radio is an excellent application for the older systems that are not

much use for anything else nowadays, so it is quite feasible to

dedicate an older system to running paKet 24 hours per day.  What else

do we do with those older systems now?  We used to give them to the

wife or children to use as a Word Processor, play games, etc but many

modern Word Processors need at least 4MB RAM, 10 to 20 MB of disk,

80386, etc... The good games are not much better.   Thank Goodness for

packet radio!  If you are running Windows, OS/2, DESQview, etc you will

find paKet runs quite happily in the background as a simple DOS task.

In addition to providing the usual Terminal Mode where you can send and

receive data via the TNC, the program provides many features and

facilities to enhance the operation of your Packet Radio system. Such

features include:

Full Personal Message System with automatic Mail Forwarding to the BBS

Flashback feature to review earlier communications

Disk and/or printer logging

ASCII text file transfers

Exchange computer programs and other disk files with:

paKet-Protocol, an advanced Binary File Transfer protocol

YAPP Binary File transfer protocol

BayCom Binary File transfers (Digicom supported too)

Full online TNC help for your TNC commands

Online Manual - this entire Manual is available in a pop-up window

Interrupt driven communications, so you don't lose anything even when

paKet is running in a Multi Tasking environment.

Runs on COM1, COM2, COM3 or COM4

Both Hardware and Software Handshaking supported

Multiple windows for up to 10 simultaneous connections

REMOTE Mode - (unattended access)

Access to DOS, your favourite editor, and file LISTer from within paKet

Automatic Script processing..............     etc etc etc

Page 1

paKet is fully configurable so you can customise a system especially to

suit yourself.   Setting up your system is but a matter of a few

keystrokes - see the Configuration Windows section of this Manual for

details.

paKet uses the standard Command Mode (sometimes called Terminal Mode)

of the TNC.  Some other programs require HOST Mode or KISS Mode and so

they will often run only on specific TNCs.   But paKet will run with

any TAPR-compatible TNC and that includes just about all TNCs on the

market today.

paKet is a sophisticated communications system and it may take you a

while to become familiar with all its features, especially if this is

your first look at paKet, but it is designed to be "friendly" and as

easy to use as possible.  You do not need to use all its features if

you would rather keep it simple to start with.  Help is always

available at the touch of the <F1> key if you forget what key to press.

However, you really should read through the entire Manual to get the

most from the paKet system.

In the remainder of this Introductory section I would like to talk a

little about computers and where they fit into the scene.  And I will

cover packet radio principles generally, especially for those who are

new to this mode of operation.  I will also cover some other digital

modes because we often find the same equipment used for packet can also

be used on some other modes as well.

The remaining chapters of the Manual are a reference for the paKet

program, explaining the various part of the system and helping you to

get the most out of it.

This Manual has been fully revised, so if you are upgrading from an

earlier version of paKet you should scan through the entire Manual.

Some of it was found to be still relevant and has been retained without

change; where necessary, sections have been edited to accommodate

improvements in version 6; and some sections have been significantly

rewritten.

I hope you like paKet and find it adds to your pleasure when operating

your equipment in this exciting new generation of radio communications.

Tony Lonsdale, VK2DHU

Page 2

Computers - how it all began

From the mail I have received over the years, I gather there are many

paKet users who might be experienced with radio and electronics, but

are relatively new to Computers.  So I though it would be worthwhile to

provide a brief introduction to Computing.  In this section I will give

you a bit of background and history so you can see how it all came into

being and how we got to where we are today.

Although computers are commonplace now in Amateur Radio shacks, it is a

relatively new technology and there are a large number of Amateur Radio

operators who are embracing this technology for the first time when

they explore the world of digital communications, such as packet radio.

It was during the 1970s that we saw the dawning of this new Computer

Age.  Until then, computers were the exclusive domain of the Scientific

Community and Large Business or Government organisations.  To have a

computer in the home was unheard of, even in the early 70s. Some

amateurs who also happened to be computer enthusiasts, might have been

involved from the beginning, but for most it is only since the mid 80s

that computers have made their presence felt.

Actually it all started back in the 1940s.  That is, if we don't count

the "calculating machines" designed by people like Leonardo da Vinci

(circa 1500), Blaise Pascal (1653), Gottfried Leibnitz (1673) or

Charles Babbage (1820).  But in was in England in the early 1940s that

a group of scientists and mathematicians developed a series of

special-purpose computers including the "Colossus", to crack the secret

message codes being used by the Germans during the War.

The first ones used lots of electromechanical relays and occupied vast

rooms, generating lots of noise and heat.  They were relatively slow

too.

Then came the development of the Valve or Tube.  Remember the old

radios and TVs that had to warm up before they came on?  (You are

showing your age if you do!)  This technology enabled technicians to

get rid of the slow and noisy relay switches and build a much faster

and more reliable computing machine. This was the real beginning of

Electronic Computing as we know it today.

In 1944 at Harvard University in the USA the first general purpose

PROGRAMMABLE digital computer was produced using both relays and tubes.

It was called the "Mark I".

In 1946 ENIAC, the first fully electronic computer with about 18,000

valves/tubes arrived on the scene.  It weighed around 30 tons, covered

150 square yards of floor space (they didn't use the metric system),

and needed its own power sub-station as it used 150 KILOWATTS!   This

like "Mark I" was programmed by hard wiring.  That is, to change the

program, you had to rewire the computer!  ENIAC is reported to have

cost $600,000 which was a lot of money in 1946.  The first stored-

program computers, EDSDAC and BINAC, arrived a few years later.

Page 3

The first commercially available stored-program computer, the UNIVAC-1,

became available in 1951 and was used to process the voting for the

1952 Presidential elections.  It announced President Eisenhower as the

winner less than an hour after voting closed.

The transistor was discovered in 1947 and its potential was realised as

it began to appear in all types of electronic devices from the portable

radio (the "Trannie") to a new generation of computers.  These "solid

state" computers, known as the Second Generation of Computers, appeared

in the early 1960s and were faster and smaller than their predecessors

and even more reliable.

In 1958 Texas Instruments developed the first Integrated Circuit or

"IC", sometimes called a chip because it is manufactured on a chip of

silicon.  These miniature circuits whose detail can be seen only with

the aid of a microscope, can contain many thousands of transistors and

other components in a circuit smaller than the size of a small finger

nail.  With this technology, more computing power could be built in a

smaller space with not only cost savings but also with lower power

requirements meaning less heat and less cooling requirements.  Things

were really happening now!  These computers were known as the Third

Generation.

In more recent times we have seen the development of the VLSI (Very

Large Scale Integration), and ASIC (Application Specific Integrated

Circuit).  These chips pack even more electronic circuitry into a

smaller space than before and one wonders how far we can go with this

miniaturisation.  It is getting to the stage where we have to build the

components smaller so the electric current doesn't have so far to

travel - the closer they are, the less time the electricity takes to

get there.  In other words, the computer can be held up by the Speed of

Light!!!

Computers - Personal Computers

The computers developed up to the mid sixties were not really

classified into big and small computers.  They were all BIG, some

bigger than others.  But in 1965 Digital Equipment Corporation (DEC)

announced the first Mini-Computer, the PDP-8.  This was a much smaller

computer with less computing power but it sold for less than $20,000.

It became one of the Computer Industry's biggest success stories.  At

last ordinary scientists and engineers could get access to computers,

not just for calculations but for controlling and monitoring lab

experiments.

In 1973 the first Micro Processor was developed.  Most of the parts for

the processor section of a computer were reduced to the size of a

single chip.   Developing technology enabled the cost of these chips to

be reduced from hundreds of dollars to around twenty five dollars

within a few years.  The same technology developments produced higher

density memory chips whose prices were also falling dramatically.

It was now possible to produce a general purpose Micro Computer that

was even more powerful than the first Mini Computers for less than

$1,000.

Page 4

It took a little while for the world to appreciate the potential of

these little Micro Processors.  It was the enthusiasts, the hobbyists,

the amateurs, who started to build their own Micro Computer around the

mid seventies.  Note, they had to BUILD it; there was nowhere to BUY

one of these things at that time.  Of course, in those days there was

no software for these computers so any program had to be fed manually

into memory via the front panel switches.  But these WERE enthusiasts!

Around this time, it started to take off.  There were several events

that I believe triggered the whole Personal Computer Industry.

In the mid seventies IBM were promoting the use of a diskette (8 inches

diameter, single sided, 128KB capacity) in some of their business

machines.  A Mr. Gary Kildall, who worked for Intel Corporation, the

manufacturers of the Micro Processor chips, saw the potential of these

diskette storage devices and developed some software to connect them to

his Micro Processor at Intel.  It was far superior to the Cassette Tape

storage medium used until then.  He asked the company to support him in

this software development.  They didn't agree so he left and formed his

own company to develop and market this software.  That company became

known as Digital Research Inc. and the software, CP/M, was the most

popular operating system for Personal Computers for many years and one

of the early success stories.  Digital Research was a rival to

Microsoft in the early 80s and almost established its domination of the

Personal Computer Operating System market.  If just a couple of phone

calls and hurried meetings with IBM had gone differently, Bill Gates

might not be the wealthy man he is today.  Anyway, in the late 70s and

early 80s, CP/M was the dominant system and, because CP/M was available

on a variety of computer systems, it established a standard that

software developers could use when developing their software.

In 1977 there were some other success stories, the development and

marketing of the Apple Personal Computer and the Tandy TRS-80 Personal

Computer.  Here we saw the first of the consumer-targetted Personal

Computers.  You just bought it, plugged it in, and used it.  There was

still not a lot of software available for these things but at least you

didn't have to build it!  For the first time we saw computers that

didn't have front panel switches, flashing lights or spinning wheels.

They didn't look like the computers Science Fiction fans knew so well.

By now some good software was being developed for these machines and

the better software generated more interest in the computers so the

market grew which meant more software development which meant... etc.

The next big event in the development of the Personal Computer was in

1981 when IBM, the giant of the Computer Industry, introduced their own

version of the Personal Computer.  In fact it was called the "PC" and

that term is now used interchangeably with any Personal Computer.

With IBM involved, the business world took a closer look at what was

often regarded as a toy or games machine.  The potential was once again

realised and as the Personal Computers began to find their way into

Businesses, we saw the development of more sophisticated software.

Everyone has been struggling to keep up ever since.

Page 5

Because of the enormous size of IBM, their established presence in the

Computer world, and the prestige associated with the name, everyone sat

up and took notice of the IBM PC.  The design and the technology they

implemented for their PC was rather conservative, but it was "IBM"!

When the system was released, IBM also released full technical details

of the system, including details of the hardware and the firmware.

Some pundits saw this as a foolish move, thinking IBM failed to

appreciate the significance of this "little home computer". (I am sure

the pundits were correct in that IBM did fail to appreciate that

significance). But perhaps, unwittingly, releasing the technical

information proved to be a very successful move, at least for us, the

computer users and for the industry generally.

As a result of this information being freely available (in the Public

Domain if you like) and with the instant success of the IBM model, we

began to see other manufacturers building computers that were based on

the original IBM design.  They called them "clones" but those of you

who experienced some of those early clones will remember they were not

exactly clones at all - many of them bore little resemblance to the IBM

machine.  But it was the start of an industry within an industry.

Nowadays the clones have taken over to the point where we tend to use

the term "Industry Standard" or "Intel machines" (because Intel have a

stranglehold on the "IBM clone" market).  To many people a "PC" is any

model of any IBM Compatible computer.  Some even refer to these systems

as "DOS" machines because of the most popular operating system running

on these computers.  The degree of compatibility is no longer a problem

and all modern IBM-compatible clones conform to the evolving industry

standards.  In some ways, IBM's own PS/2 range are the only ones not

entirely "IBM Compatible!".

paKet requires an IBM Compatible computer, but I am happy to say it

will run on a IBM PS/2 as well.

There are other types of Personal Computer.  The biggest competitor to

the IBM clone models is the Apple Macintosh.  This is a very nice

machine with a Graphic/Mouse interface that "Mac" users claim is an

easier way to use the computer.  The Mac uses a Motorola processor

which is different to the Intel (or compatible) processor used in the

IBM clones.  So the Mac programs do not run on the IBM and the IBM

programs (including paKet) do not run on the Mac.   It is possible to

add some optional equipment to the Mac to allow it to run IBM software

but if that is your aim, you would probably be better off to get an IBM

Compatible maachine to start with.

Other computers include the Amiga which again has lots of nice features

but unless you buy some added equipment, they too will not run IBM

compatible programs.

There is still a very loyal band of Commodore 64/128 users.  This is

perhaps the cheapest way into packet radio, but these systems are

beginning to show their age as they generally offer fewer facilities

than we get from more powerful systems such as an IBM Compatible

computer running paKet.

Page 6

The quality, availability, performance and value for money of modern

Personal Computer systems are constantly improving so that the humble

PC is now widely used in business of all sizes, in schools and

Universities, in homes, and of course in radio shacks all over the

world.

Welcome to the world of computing.  You are certainly not alone!

Page 7

Digital Communications

Digital Communication is not new. Other Modes such as Morse Code, RTTY

and AMTOR have been around for years.  But the newer modes, Packet

Radio and PACTOR offer some significant improvements, perhaps the most

significant is Error Free communications.  Let's have a brief look at

these modes.

Morse Code.

Morse Code is the original Digital Mode dating back to the last

Century! Today, while the debate rages over Morse Code licensing

requirements, more and more stations using this mode have replaced

their hand key and their electronic keyer with a Computer keyboard and

display. The technology is good but even today, no machine can beat the

trained human ear for copying hand sent Morse Code under varying

conditions. The keyboard is a more efficient Sending device though,

for most people.

Perhaps the one advantage of Morse over Packet and the others, is that

a human operator can decode the Morse Code transmissions by ear!

RTTY

RTTY or RadioTeletype is a direct machine to machine communications

mode using the Baudot (or Murray) code.

This mode became popular with many amateurs when surplus TTY machines

became available at a reasonable cost after World War II. These

mechanical monsters provided a keyboard for Input and a paper roll for

printed Output. They were also useful to help hold the house down in

times of hurricane winds - they must weigh a ton.  Video displays were

still too exotic and expensive in those days. It was not until the mid

1970s that we began to see the Video Display come into more widespread

use. (By the way, have you ever wondered why early Program Languages

like BASIC use the command PRINT to display their output?)

When transmitting Morse Code, the transmitter is switched on and off to

make the dits and dahs.  When sending Teletype however the transmitter

runs continuously, sending either of two frequencies conventionally

known as Mark and Space (a reference to paper tape reception of

telegraphy).  The early pioneers found on-off keying was not all that

successful for Teletype signals because of interference from static.

They experimented with FSK, or Frequency Shift Keying and found it

performed much better. With FSK, the transmitter is shifted up in

frequency every time a Mark is to be sent, reverting to the lower

frequency for a Space.  The amount of the shift is usually 170 Hz for

Amateur Radio use although many commercial Teletype signals use other

shifts, notably 425 Hz and 850 Hz.

Many systems use AFSK or Audio Frequency Shift Keying.  When this is

used, the transmitting station generates the Mark and Space audio

tones and feeds them into the transmitter's microphone input.  The

result at the receiving end is that the same audio tones are heard and

processed, whether the transmitting station used FSK or AFSK.

Page 8

When listening to a teletype signal off air, you will soon get to

recognise the familiar warble of Mark and Space tones.

In the amateur shack the TTY machine is usually connected to an HF

receiver or transceiver which the operator tunes so that the received

audio is just the right pitch or audio frequency to trigger the

demodulator's Mark and Space resonators.

If the receiver is slightly off the correct frequency the tones vary

and the text becomes garbled or even lost altogether.  To help the

other station tune the receiver correctly, a RTTY operator can send a

string of alternate R and Y characters viz: RYRYRYRYRY.  This pattern

is chosen as it produces the most frequent and almost symmetrical

alternation of Mark and Space tones, giving the receiving operator the

best chance to tune the receiver before the "real" message starts.

However, even if the signal is accurately tuned, the information can

become garbled or completely lost due to interference, fading, or

noise.  Often, it is possible to make sense of the message even with

parts missing, but RTTY is by NO means an error free mode!

I should point out that similar problems exist for other modes

including Packet.  While information can still fail to get through on

the more sophisticated modes the Error Detecting capability of some,

especially Packet and PACTOR, ensure that the operator will receive

either accurate information or nothing at all.  Usually, where "nothing

at all" is received, the information will automatically be

retransmitted when the radio is retuned, or the interference stops,

(etc) and nothing is lost.

The Baudot code is a 5 bit code and those of you who are familiar with

Binary Notation will know that the maximum number of values we can have

with 5 bits is 32.  That means that each unit of transmission, one

keystroke if you like, can contain any one of 32 possible values.  If

you look up a table of Baudot codes you will see there are 32 values

listed, one code for each letter of the alphabet plus a few other codes

for other things such as a space and a Carriage Return.  But, what if

we want to send a number such as "9" or a question mark?  These are not

mentioned in that table because all 32 codes are already used.

The solution is rather similar to the Typewriter or Computer Keyboard

where we have the Shift key to get various additional codes from the

keyboard.  Most keys will produce a different result if we hold down

the Shift key as we type.  Well, one of those original 32 codes is a

special code known as FIGS (for Figures Shift). The convention is that

when we want to send a number or some other special character such as a

punctuation mark, we can do that by firstly transmitting a FIGS code.

Then instead of using that original table of 32 codes, we have a second

table of codes to use, and that second table includes all ten numeric

digits and various punctuation marks.  Provided both sides of the

conversation observe the convention, the sender can send a FIGS and

start using the second table; the receiver will see the FIGS code and

it too will interpret all data that follows from the second table.

Page 9

With just 5 bits of data we then have almost 64 different codes we can

send and receive.  (I say almost because there is some duplication in

the two tables, including a space and a Carriage Return but that is not

important here).  Even that many codes is not enough to handle all 26

letters of the alphabet in both UPPER and lower case, so RTTY systems

always operate in upper case only.

If we wanted to type a big number (say "13579") we don't have to send

FIGS before every digit.  We send that code only once and the receiver

then will take EVERYTHING we type from now as if it belongs in the

second table.   When we want to revert to the normal alphabetic table

of codes we can send another special code, this one called LTRS (for

Letters Shift).  Then everything goes back to normal, using the

original alphabetic table of codes.

Normally we don't have to concern ourselves with these FIGS and LTRS

codes.  Our computing equipment will take care of those things for us.

We just type away and rely on the system to generate and send those

codes when necessary.

As I mentioned earlier, it is quite possible to lose bits here and

there when receiving a RTTY signal, whether it be because of fading,

interference, frequency drift, or whatever.  One of the big problems

with lost data is the possible loss of a FIGS or LTRS code! Say we had

sent "13579" and then typed "HAPPY BIRTHDAY".   Our equipment would

have sent a LTRS code before the first "H" but what if the receiver did

not copy the LTRS code we sent?   Can you imagine what happens?  As far

as the receiver is concerned we are still sending numbers or other

codes from the numeric table!  So our "HAPPY BIRTHDAY" is going to come

out looking something like "#-006 ?845#$-6".  And EVERYTHING we type

from then on is going to look just as strange until we happen to send

another LTRS code later.   It is for this reason that many systems

include an option to "Un-shift on space".  If you have a multi mode TNC

capable of handling RTTY, you will probably have this option in your

TNC.  If that option is ON then your receiving system will imply a LTRS

code every time it receives a space. So if you seem to be copying lots

of funny numbers from a strong, well tuned signal, try setting that

option ON.

We can overcome some of these problems by using ASCII instead of using

the Baudot code.  With ASCII we can have 128 different codes so we do

not need the FIGS/LTRS codes.  All Personal Computers use ASCII as

their native "language" so it would be a reasonable thing to use.

Although not part of the defined ASCII standard, it has become an

almost de-facto standard in the computer world that an additional 128

characters are available, often called Extended ASCII.  But, despite

these benefits, Baudot continues to rule the airwaves for Amateur and

Commercial Teletype transmissions.

Today, RTTY is still a popular mode especially on the HF bands, and the

advent of the "Glass Terminal", firstly the Dumb Terminal and now the

Personal Computer, has brought this mode to even more operators the

world over. Many specialised RTTY systems were developed for the

Amateur enthusiasts but have been superseded now by the Personal

Page 10

Computer with one of the Multi Mode TNCs which handle RTTY and many

other modes besides.

The latest Computerised RTTY equipment generally allows us to use the

mode better, quieter, more efficiently, using less power and occupying

less space than the old TTY machines, but the limitations of the mode

remain.

AMTOR

AMTOR is a specialised form of RTTY. The term is an acronym for AMateur

Teleprinting Over Radio and is derived from the commercial SITOR system

(Simplex Telex Over radio) developed primarily for Maritime use in the

1970s.

In the early 1980's, Peter Martinez, G3PLX, made several minor changes

to the SITOR protocol and called it AMTOR.

AMTOR improves on RTTY by incorporating a simple Error Detection

technique. The system remains relatively uncomplicated but AMTOR

performs well even in poor HF conditions.  While there can still be

many errors in AMTOR data, the Error Detection helps a lot and the

result is quite tolerable for normal text mode conversations because of

the high redundancy in plain language text.  Certainly much better than

RTTY. But for more critical data such as program code, or even some

technical information messages, NO errors can be tolerated.

There are two modes used in Amtor:  ARQ and FEC.

ARQ

This mode is a little different in that it is a Synchronous protocol,

which means both stations are synchronised to each other's signals.

In ARQ mode (Automatic Repeat Query), sometimes called Mode A, data is

sent in groups of 3 characters. Although each character is only 5 bits

(same as for RTTY), two additional control bits make it up to 7 bits

per "character" and they are set so there are always 4 marks and three

spaces in every transmitted character.  If the receiving station gets

some other combination it knows an error has occurred.  The 40 percent

overhead is considered worthwhile to get some error detection. This

technique can identify a lot of errors that might occur but is not as

thorough as the methods used in PACTOR and Packet which we look at

later.

The receiver responds to each 3 character group by sending either an

ACK (ACKnowledge) code (if OK) or a NAK (Negative AcKnowledge).  Each

time the transmitting station gets a NAK, that 3 character group is

sent again.

If you listen around on the HF bands in the recognised Data Segments of

the bands, you might hear a chirp-chirp sound that identifies an ARQ

transmission. Even when there is no data actually being transmitted,

the transmitting station continues to send idle "chirps" to maintain

the link.

Page 11

Your AMTOR equipment probably supports a Listen Mode too and that

allows you to monitor another ARQ session even though you are not

participating in the session with the usual acknowledgements. Of course

that means you don't get the opportunity to say "NAK" if you don't copy

something properly!

FEC

In FEC mode (Forward Error Correcting), sometimes called Mode B, the

sending station sends each character twice so this mode provides a

means of transmitting to several stations at once.  The receiving

station does not acknowledge the data received. If a receiving station

matches both instances of a character, that character will be printed,

otherwise some error symbol is printed. This mode does not provide for

the receiver to ask for the missing data to be retransmitted.  An FEC

transmission sounds more like a Baudot RTTY signal.

The two stations need to keep in phase with each other so each FEC

transmission is started with several sets of "phasing pairs" and these

are sent at regular intervals even while there is no data being

transmitted.

FEC Mode is still better than ordinary RTTY but its error detection is

not as reliable as that in the ARQ Mode.

AMTOR systems are still limited to the technology of the 60s with

limitations such as the character set and the maximum transmission rate

(100 baud) geared to the mechanical teleprinter. The Error Detection

technique provides improved accuracy over the "vanilla" RTTY mode, but

is still not entirely reliable.  It is perhaps better termed Error

Reduction than Error Detection and has limited application for critical

data.

PACTOR

Amateur Packet Radio has been in use since the early 80s and is used

extensively on both HF and VHF/UHF.  But it is better suited to the

VHF/UHF bands. It was found AMTOR would often perform better than

Packet on the HF bands and so despite the limitations of AMTOR, a large

number of HF operators preferred AMTOR to Packet Radio.

As computerised systems became more popular, amateurs began to explore

more sophisticated options for AMTOR.  Various alternatives were tried

but every time, AMTOR's poor error handling prevented any significant

breakthrough.  And as AMTOR did not handle the standard ASCII code, a

new Protocol was called for.

In 1987 two German amateurs, Hans-Peter Helfert (DL6MAA) and Ulrich

Strate (KF4KV) began work on a new Protocol which they called PACTOR.

Although this new system incorporated the basic structure of AMTOR with

its fixed interval data blocks and corresponding acknowledgements,

PACTOR also combines important characteristics of Packet Radio.

Page 12

PACTOR is designed especially for HF operations.  There is no reason

why it could not be used on VHF/UHF, but it does not offer significant

advantages over Packet Radio on those bands.  It is on HF that the new

protocol shines.

The block length of PACTOR is fixed at 1.44 seconds with significantly

lower overheads than those of AMTOR.  This was found to be the most

suitable compromise for HF operations with the relatively short block

having a better chance of survival through difficult conditions and

allowing short break-in times. The protocol provides for automatic

adjustments to the baud rate too, so when the going gets really tough

it will reduce the speed; and when things improve it will increase the

speed to improve the throughput again.

Huffman data compression is used in PACTOR and that adds to the

effective performance.  The Huffman code is based on the assumption

that in plain language text there is a high content of frequently

occurring letters such as E, A, etc. It has been found that a block of

normal ASCII text can often be compressed to half its original size

without any loss of data at all.

PACTOR offers high quality error detection, which overcomes the main

weakness of AMTOR.  PACTOR uses a longer acknowledgement signal as well

as the CRC checksum code used in Packet Radio to detect errors in the

transmission.  I will discuss Error Free Communications with CRC

checksums in the next section on Packet Radio, so we'll have a look at

that in a moment.  The comments there apply to both PACTOR and Packet.

Other features of PACTOR include an improved error correction system

which has become known as "Memory ARQ".  I won't go into the full

detail here, but in summary it improves the performance of the

receiving system in weak or noisy conditions.  For example in any

digital system, a signal is reduced to a series of bits which are

either a binary 1 or 0, but this does not distinguish between an input

signal of a millivolt or 10 volts!  So long as the analog input signal

is above the threshold, the digital output data is the same!

The Memory ARQ system remembers not only the binary 1s and 0s, but also

the incoming analog values, so if the received block reveals errors,

the analog values from the repeated blocks can be combined into a "sum"

block and subjected to another CRC error check.  While this system is

not perfect it can provide error free communications in conditions that

other modes, including Packet, could not handle.

Page 13

Packet Radio (AX.25)

Perhaps the most important feature of Packet Radio is the Error

Detection system it uses to give us Error Free communications.  PACTOR

is the only other mode that offers such data integrity but Packet

offers other advantages that even the new PACTOR does not support.

Packet offers simultaneous frequency sharing by several stations,

multiple repeater operation ("digipeating") for longer distance

communications, and Networking.

Error Free Communications.

The Error Detection (and Correction) is perhaps the major benefit of

Packet Radio over the older modes.  (The newer PACTOR mode shares this

feature).  With Packet Radio using HDLC and the AX.25 protocol, data is

checked and verified before being accepted by the receiver's TNC.

HDLC (High-level Data Link Control) is a procedure defined by ISO, the

International Organisation for Standardisation, for the handling of

Error Free frames of information over a communications link.  AX.25 is a

set of rules defining the format and content of packets and how they

are handled. It is not the only Protocol for data communications but is

the recognised standard in the world of Amateur Packet Radio.

Fortunately we don't have to know the details of either HDLC or AX.25

to use Packet Radio. (I read the AX.25 Protocol Specifications and am

still not too sure what it said!)  The hard work is done in a special

device called a TNC (Terminal Node Controller) which is a small

computer in itself, programmed to perform the HDLC Error Handling and

many AX.25 functions to make it easy for us.

The Error Detection, similar to that used in PACTOR, is much more

sophisticated than that used in AMTOR so the result is significantly

more reliable.   Each block of data (packet) contains various control

information including a CRC checksum.  (CRC is Cyclical Redundancy

Check - true!)  This checksum is a value calculated by the sending

station based on the content of the data contained in this packet.

When the receiving station receives that packet, it performs the same

calculation on the data it received and it compares its result with the

checksum it received.  If there was something wrong with the received

data, the checksums will not match and the receiver knows there is

something wrong with this block of data.  The chance of an undetected

error getting through is less than one in a many millions. So, for all

practical purposes Packet Radio and PACTOR are considered completely

error free. If any data comes through the system to your computer (to

paKet in your case) you can be sure that what you see is exactly what

the other station sent.

Frequency Sharing.

Packet Radio, like the commercial X.25 Packet Switched systems, allows

several simultaneous point to point connections to share the same

frequency.

Page 14

In Packet Radio whenever we send a message to another station, our TNC

builds a Packet, like an envelope, around our message adding details

such as the callsign of the station this message is addressed to. Then,

if the frequency is clear, the Packet is transmitted into the airwaves.

Our TNC listens to the audio coming from the radio receiver and if

there are any other Packet Radio stations using the frequency, it waits

until there is a lull in the transmissions before transmitting our

Packet. At first, you might wonder how long it has to wait, thinking

the other stations might be going for hours! However, in practice, a

Packet might take only a fraction of a second to transmit - a

relatively short burst of time compared to the time it takes us to type

a message. Unless the frequency is really busy, you might not even

notice the delay!

The Packet Radio frequency in use also carries messages for many people

at the same time. When receiving, your TNC looks at every Packet

received, checks the address on each Packet and identifies any Packets

addressed to you.

Any Packets not intended for you may be ignored by your TNC so you see

on your screen only those messages addressed to you. This way you need

not concern yourself with all the other information flowing around on

the frequency, and are free to concentrate on your contact with the

other station.

If you wish, you can ask your TNC to display everything it receives.

This is called Monitoring.  Every Packet contains the AX.25 Header

("envelope") which identifies the station who sent it and the station

it is addressed to.  The Header contains other information too but we

won't get into that just now.  When monitoring, your TNC will send to

the Computer the Headers as well as the contents of the messages so you

can see who sent what to whom. Then you will see how the AX.25 system

is able to handle lots of different conversations on the frequency at

the same time.  The other modes do not offer this capability.

Multiple Repeater Operation.

When we want to establish contact with some station within range we

issue a CONNECT command using that station's callsign, and provided

that other station hears our call, it will acknowledge and we can

simply type messages to each other (or exchange files, etc).

To illustrate, I'll use a callsign of ZZ1FRD for the other station, and

for these illustrations, the "messages" that I put inside brackets are

internally generated, not typed:

Page 15

������Ŀ                    ������Ŀ

� Our  �                    � Fred �

�System� ------------------ �ZZ1FRD�

�      �                    �      �

��������                    ��������

CONN ZZ1FRD

(Connect command)-->

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD)

Hi there Fred.-->

<--(ACKnowledge)

If the station we wish to communicate with is outside radio range, we

may establish the connection via one or more intermediate stations

which act as repeaters.

A station providing this repeater function is commonly called a

Digipeater ("Digital Repeater"). It is part of the AX.25 design that

any TNC may be used as a Digipeater so, unless you deactivate this

facility in your TNC, your system too can be used as a Digipeater.

Say we are trying to establish a connection with our friend Fred who

is out of radio range.

������Ŀ                                         ������Ŀ

� Our  �                                         � Fred �

�System� ----------- (too far away) ------------ �ZZ1FRD�

�      �                                         �      �

��������                                         ��������

CONN ZZ1FRD

(Connect command)-->

(no response)

There might be another station in between who can "hear" both ZZ1FRD

and us.  So, we can call ZZ1FRD "via" the intermediate station. Our

TNC, using the AX.25 logic, will transmit a "Connect" message with both

those callsigns; the intermediate station will see its callsign used as

a Digipeater so it will capture the message, store it in its memory for

the time being, and when the frequency is clear it will retransmit that

same message. This time ZZ1FRD will hear it and will respond, again

"via" the Digipeater station.

Let's have a look at that situation with an illustration:

Page 16

������Ŀ                ������Ŀ                 ������Ŀ

� Our  �                � Digi �                 � Fred �

�System� -------------- �      � --------------- �ZZ1FRD�

�      �                �XY4DIG�                 �      �

��������                ��������                 ��������

CONN ZZ1FRD via XY4DIG

(Connect command)-->

(Connect command)-->

<--(ACKnowledge)

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD)

Hi there Fred.-->

(Hi there Fred.)-->

<--(ACKnowledge)

<--(ACKnowledge)

So, we now have a point to point connection with Fred as if it were a

local contact. It just takes a little longer because each message is

transmitted twice.

The AX.25 Protocol provides for up to eight Digipeaters to be used for

a single point to point connection. So for a more distant destination,

we could call that station via two or more intermediate stations.  Each

of those Digipeaters will store our message and retransmit it as soon

as the frequency is clear.  This allows distances of hundreds, maybe

thousands, of kilometers (even miles!) on VHF or UHF frequencies.

Note the Digipeater does no intelligent processing with the messages -

it simply retransmits whatever it receives from either station.

Networks.

Following on from the previous discussion on Digipeating, I should

mention Networks.  The term "Network" is often used in different ways

but I want to mention just two of those here.

Firstly the term is used to describe an advanced form of Digipeating

where we can make contact with distant stations without necessarily

knowing what intermediate stations are retransmitting our messages. The

idea is like our telephone Network where we make a call by dialling the

number of the person we wish to speak to. We don't know nor care how

the call is routed, whether it goes via cable or via satellite, or how

many telephone exchanges it goes through. We are simply interested in

getting our call through to the other person, no matter how it gets

there!

Networking systems in use in the field of Amateur Packet Radio include

TCP/IP, NET/ROM, and ROSE. Unlike the widespread acceptance of the

AX.25 Protocol, we do not have any one Networking system preferred over

all others in use. There is still lots of healthy debate going on and

developments continuing. It may be that we will have to wait for a new

generation of Networking System before a single, global system is

implemented.

Page 17

TCP/IP

TCP/IP is a set of protocols developed to allow communications across a

network.

TCP (Transmission Control Protocol, but often called Transport Control

Protocol) is that part of the system which is responsible for getting

our data from one place to the next. It will resend anything that did

not get through and it will divide large amounts of data into smaller,

more manageable packets, or "datagrams" and make sure they all arrive

safely.

IP (Internet Protocol)  establishes various rules for formatting,

routing and processing our data.

The various protocols in the TCP/IP family include:

- FTP (File Transfer Protocol).

This is used to exchange files with another FTP user elsewhere on

the Network.  Any kind of data may be transferred using FTP,

including programs and other Binary Files.

paKet uses the standard YAPP (or the advanced pP) protocol for

Binary File transfers.  FTP is not currently supported, but you can

use the TCP/IP Network to exchange files with another station

running paKet or one that supports standard YAPP protocol.

- TELNET.

This facility allows you to conduct a person-to-person chat with

another user on the Network.  It allows Remote Login which means

you can log in on any other computer system on the Network.  While

connected to that station, anything you type will be sent to that

screen and anything the other guy types will appear on your screen.

As far as paKet is concerned, once the connection is established,

it is a one-to-one connection and it does not matter whether the

other station is using TELNET, paKet, or any other Terminal

Emulation program.  This mode is simply for the exchange of plain

text information.

- SMTP (Simple Mail Transfer Protocol).

This allows you to send messages to other TCP/IP users on the

Network. The idea is to leave a message in your own system and the

SMTP will automatically feed it into the Network for Forwarding to

the destination.

Although paKet is not a TCP/IP implementation, it does perform a

similar function using the BBS Network (discussed below).  Leave a

message in paKet's Mailbox and it will be automatically sent to the

BBS Network for onward forwarding to the destination.

Page 18

- POP (Post Office Protocol).

This is a variant of the SMTP mail handling facility where you may

nominate some other system as your "Post Office" so that any mail

addressed to you will be stored there for you.  Then, when you run

POP, your mail will be automatically Forwarded to you from the Post

Office station.

paKet will do this automatically for you, not with a TCP/IP "Post

Office" but with your local BBS.  This is discussed in the section

on paKet's Personal Message System.

- PING (Packet INternet Groper)

A PING allows you to put out a feeler to see if some particular

station is on the air.  If that station is there, you will get a

response, telling you the time it took for the round trip.

There are lots of other protocols used in the TCP/IP family but this

should give you an idea of the complexity and sophistication of this

advanced system.

TCP/IP is widely used on the Internet, a world wide network of

computers.  In fact it is a collection of networks, all of which are

connected to each other.  I heard there were over 20 million people in

125 countries with access to Internet.   Of course I am talking about

the telephone subscribers with their landline modems, not Amateur Radio

operators, but it is a huge system by any standards.   Users can chat,

exchange programs and files, conduct research, explore the world,  play

games, or just engage in some meaningful conversation with another user

on Internet - some have even been seen courting and proposing!  It is

much the same as we can do on Amateur Radio but on a wider scale.

(Must admit though I have not seen much courting going on around here

lately - not on air anyway!)

Each TCP/IP user is allocated a station address which consists of a

four part number.  For example, my TCP/IP address is 44.136.56.3.  This

number is not just a random number but is carefully managed and

allocated by IP Address Coordinators all over the world.  The first

part is 44 which is the number allocated to Amateur radio stations. The

second part, 136 for me, identifies the Country (or a State in the

USA), the third part identifies a region or area and the last part is

one station within that region.

In Amateur Radio the program mostly used to access the TCP/IP system is

NOS developed by Phil Karn, KA9Q although a variant called GRINOS has

been developed by Gerard van der Grinten, PA0GRI.  paKet can be used to

access a TCP/IP system but it does not currently implement the various

TCP/IP protocols mentioned here.

Page 19

NET/ROM

NET/ROM is Networking Protocol that is used to link a number of

stations together and to provide routing services for users connected

to the Network. TCP/IP often uses NET/ROM to move its data from one

place to another.

The NET/ROM system is somewhat automatic because it works out for

itself what other NET/ROM stations are available and so it can

establish its own message routing without human intervention.  It does

this by broadcasting coded messages at regular intervals for the

purpose of announcing itself to other NET/ROM stations within range.

Then all NET/ROM stations, called Nodes, can listen on air to determine

what other Nodes are available. Each one builds and maintains its own

listing of other Nodes.

If we wish to connect to our friend Fred, a distant station, where

there is NET/ROM Network available, we could issue a connect command to

a NET/ROM Node within reach.  Once connected to that Node, we usually

are presented with a menu which includes (among other things) a Node

Command.  If we type "N" that system will send to us a listing of all

the Nodes it currently knows about.  It is also possible to get a list

of the various Routes this system can use to get to another Node in the

list. This includes a a "Quality Factor" which gives us an idea of the

quality and therefore the reliability of the radio links from one place

to the next.

While we are connected to this Node, we can then give a Connect command

to the Node, asking it to make a call to another station or Node it

knows about. Let's illustrate, assuming there is a NET/ROM Node with a

callsign of N0DE within range.

������Ŀ                ������Ŀ                 ������Ŀ

� Our  �                �NETROM�                 � Fred �

�System� -------------- � Node � --------------- �ZZ1FRD�

�      �                � N0DE �                 �      �

��������                ��������                 ��������

CONN N0DE

(Connect command)-->

<--(ACKnowledge)

(*** CONNECTED to N0DE)

CONN 1 ZZ1FRD-->

<--(ACKnowledge)

(Connect command)-->

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD)

(*** CONNECTED to ZZ1FRD)

Hi there Fred.-->

<--(ACKnowledge)

(Hi there Fred.)-->

<--(ACKnowledge)

Page 20

All that just to say Hi to Fred?  It is not so bad really.  Let's have

a look at that.  Firstly we send a connect command to the NET/ROM Node

(N0DE) and when that acknowledges, we have established connection with

that station.  Then we send "CONN 1 ZZ1FRD" to the Node.  Although this

looks similar to the connect command we just issued, that string not

processed here as a command - it is just a string of characters that is

sent to the Node. That string asks the Node to issue a connect to

Fred.  The Node often has more than one port (that is more than one

radio channel) available so we must tell it which port to use. Some

Nodes even have HF ports available for convenient DXing.

Note the usual protocol is for a station to acknowledge anything it

receives correctly.  If not copied accurately, it will ask for it to be

sent again.  In this case the Node is doing more than just plain

Digipeating - it is acknowledging each and every packet before

retransmitting it.

Fred's system hears the Node's call and establishes the connection with

the Node.  The Node then connects us to Fred, just like a telephone

switchboard operator, and from now everything sent to or received from

Fred, will be transmitted via the Node.

This example uses just one Node between Fred and us, but it could be

much more complex than that with several Nodes involved in a multi hop

link.  With this simple example, it is rather similar to the Digipeater

example we saw earlier (using XY4DIG).  Note however, that when we send

anything to the Node it acknowledges each transmission immediately it

is received.  Compare this with the Digipeater connection. There the

acknowledgements were not issued by the digipeater - it simply RELAYS

all messages including Fred's acknowledgement.

Imagine a multi hop link:

������Ŀ           ������Ŀ          ������Ŀ         ������Ŀ

� Our  �           �NETROM�          �NETROM�         � Fred �

�System� --------- � Node � -------- � Node � ------- �ZZ1FRD�

�      �           � N0DE �          � S0FAR�         �      �

��������           ��������          ��������         ��������

Hi there Fred.-->

<--(ACKnowledge)

(Hi there Fred.)-->

<--(ACKnowledge)

(Hi there Fred.)-->

<--(NAK)

(Hi there Fred.)-->

<--(ACKnowledge)

I won't illustrate all the connecting dialog, but just assume we

had connected to N0DE, then asked it to connect to a second Node, say

S0FAR, then asked that second Node to connect to Fred.

Page 21

Now, each message we send to N0DE is sent to S0FAR which in turn sends

it on to ZZ1FRD.  If Fred's system copped some interference and failed

to copy our message correctly, we don't need to know about it because

the last link in the chain (S0FAR - ZZ1FRD) will look after it by

retransmitting the message.

If we were using simple digipeaters here, and we failed to receive

Fred's acknowledgement, we would have to retransmit the message to N0DE

which would have to send it again to S0FAR which would have to send it

again to ZZ1FRD!   The Network method is much better: it is more

reliable and involves less traffic.

In this last example it may be the S0FAR Node is in some far away

place, maybe in some overseas country!  Provided the NET/ROM Network

knows about that Node, we do not concern ourselves with that detail.

Let the Network worry about that.  If it is in the Nodes list we can

simply issue a connect command and wait for it to establish the

connection!

And once we are connected to that far-away Node, we can ask it for ITS

Node list then connect to some other exotic place from there!   It's

exciting stuff.

ROSE

ROSE is an acronym for "RATS Open System Environment".   No, no, RATS

has nothing to do with the Mouse used on the Macintosh - RATS is

another acronym.  RATS is the Radio Amateur Telecommunications Society

of New Jersey. They must work for IBM, with all these acronyms.

Members of this Society include Tom Moulton (W2VY) who is credited with

writing the ROSE code, and Gordon Beattie (N2DSY).  These guys and

others developed the ROSE system and today it is widely accepted and

gaining acceptance all over the world.  Some say it is more advanced

than the well established NET/ROM systems.

ROSE uses Node to Node packet acknowledgements, similar to that used in

NET/ROM, as illustrated above. In fact, there are many similarities

between ROSE and NET/ROM.  The differences that concern us are in the

way we access it.

With ROSE we do not need to know the callsigns on various Nodes we want

(or need) to use.  We simply need to know the callsign of our local

ROSE Node, which is where we enter the Network, and the remote

station's network address, which is where we leave the Network.

The network address is a number, based loosely on the telephone area

code and local dialling prefix.  So to establish contact with a distant

user, let's say it is still our friend Fred who now has access to the

ROSE network.  We need to know the Network Address of Fred's local ROSE

Node.  That's all.  The rest is up to the ROSE Network and the people

who set it up for us.

I think we need another illustration.

Page 22

I have shown below a diagram of a ROSE Network with four Nodes

showing.  Each of these four Nodes has a Network Address which is a 6

digit number.  We are within range of our local Node, shown with a

callsign of AA1NET and a Network Address of 658300.  Fred, ZZ1FRD, can

access the ROSE Network via his Node, shown with a callsign of XY2LOC

and a Network Address of 675200.  The other Nodes do not concern us

here - I just included them to indicate there can be lots of entry

points in the Network.

���������������Ŀ

�               �

������Ŀ     Node  �               �Node        ������Ŀ

� Our  �     658300�     ROSE      �675200      � Fred �

�System� --------O �               � O--------- �ZZ1FRD�

�      �     AA1NET�    Network    �XY2LOC      �      �

��������           �               �            ��������

�               �

�����������������            ������Ŀ

������Ŀ  Node      O              O  Node      �other �

� other�  598550   /                \ 476250    � user �

� user �--------- /                  \ -------- �      �

�      �                                        ��������

��������

If we wish to call Fred, we need to know the callsign of our own Node

and Fred's Network address.  So to make that connection we enter the

following command:

CONNECT ZZ1FRD v AA1NET, 675200

The dialog might go something like this:

OUR SYSTEM                      OUR ROSE NODE (AA1NET)

----------                      ----------------------

CONNECT ZZ1FRD v AA1NET, 675200 -->

<--(ACKnowledge)

(*** CONNECTED to ZZ1FRD via AA1NET,675200)

(Call being Setup)

*** delay in here while ROSE make the call through to Fred ***

(Call Complete to ZZ1FRD-0 @ 5050675200)

Hi there Fred.-->

<--(ACKnowledge)

(Hi there Fred.)-->

The first part of the connect command is similar to any other connect

command with the callsign of the station we want to talk to and another

callsign of a station we want to digipeat our call.   However in this

case the AA1NET is not just another packet radio digipeater: this one

is a ROSE Node and it behaves a little differently.  It accepts our

call and responds immediately with a CONNECTED message and tells us our

call is being processed.

Page 23

The ROSE system knows about all the other Nodes in the Network so it

knows how to get to 675200.  We just have to wait while it does the

job.  ROSE can automatically attempt alternative paths to that

destination if one path is not working properly.

It does not matter if Fred is right next door, across the other side of

the Country, or even overseas.  Provided we both have access to the

same ROSE Network, the connect command is the same!  We do not need to

know what happens inside "The Network" - there might be one hop to get

to his Node or there might be a dozen hops.

Once Fred's system acknowledges our call, our ROSE Node tells us the

Call is Complete and connects us "directly" to Fred.  Now we can talk

to Fred as if it were a simple, local contact.

For this connection we do not use our own Network Address (658300) but

we might like to tell Fred what it is because he will need that number

if he wants to call us via the ROSE Network.

The Network routes that are used within the ROSE Network are defined by

various ROSE Network Managers each of whom have accepted responsibility

for their own Nodes in the Network.  So if some path is bad or if some

other new Node is installed, the Network paths can be changed and we,

the users, need never know about it.

NET/ROM systems can accommodate changes in the Network too and they

achieve this automatically by broadcasting those coded messages we

talked about earlier.  That is good, but in some situations people

complain at that added activity on a busy frequency!  (Some people

complain about anything though).

Both ROSE and NET/ROM have advantages and I do not wish to get involved

in the debate as to which might be better.  Maybe one day, someone will

create a new generation of Network system that accommodates the

advantages of both, as well as enhancing functionality?

Page 24

The BBS Network and Mail Forwarding

The other meaning of "Network" I want to mention is the BBS (Bulletin

Board System) Network which provides a Global Electronic Mail Service

for us all.

A BBS, sometimes referred to as a PBBS (Packet-radio BBS), is a

computer based Packet Radio system, probably run by some generous local

enthusiast or the local club, which provides unattended (usually 24

hours per day) access for the purpose of storing and forwarding

messages. They are often used as a central store for data files too so

we can transfer programs and other files to or from the BBS.

Telephone BBS systems have been around for years, before Packet Radio

came into being, so you might be familiar with those. The concept is

the same, but we use radio as the medium instead of the telephone.

The main role of the BBS now is to provide a Mailbox service for local

users.  We could call up the BBS, ask it to display any messages it was

holding, read any messages addressed to us and maybe leave a reply to

someone.  This is very useful for sending messages to friends whether

they are currently in their shack or not. The BBS mailbox allows us to

leave a message addressed to another station and when that other

station connects to the BBS they will be able to read our message.

Why not just send that message directly to that "someone"?  Well, for

one thing, they might not be around at the time.  They might not have

their system switched on.  They might be out of range or it might be

simply more convenient to leave the message for them on the BBS and let

them read it later when they check the BBS.

The Mailbox facility became so popular that most TNCs now offer a

private mailbox.   paKet too has a Mailbox (I called it the Personal

Message System - PMS for short) similar to that in the BBS, so you and

others can send and receive messages without needing the BBS to store

those messages.

So the role of the BBS is changing.  But it still has a significant

role to play as part of a Global "Network" of similar BBS systems.

Page 25

Mail Forwarding

It all started when Hank Oredson (W0RLI) developed an automatic

Forwarding system between PBBS systems. Then we could leave a message

on our local BBS and if that message was addressed to someone in an

adjoining area the BBS would, at regular intervals in the day or night,

automatically send our message to that other BBS.  As the "Network"

developed, our mail would move to our neighbouring system and that

system would then Forward it to its neighbour and so on.

This Automatic Forwarding system has been implemented on all Packet

Radio BBS systems now and, although some have added some enhancements,

they are all based on Hank's original standard.  paKet's PMS conforms

to this standard and can participate in the Forwarding with the BBS

Network.  So you can simply leave a message in your paKet PMS and it

will be automatically sent to any packet destination anywhere in the

world.  And paKet can accept any incoming Mail directly into your PMS

without any action on your part. It will just appear there!

However, I see paKet as a PERSONAL system and as such does not belong

in the BBS Network as a link in the Forwarding chain.

The role of the BBS and where paKet fits in

The BBS has a vital role to play as a part of the BBS Network. It is

the role of your local BBS to participate in the automatic Forwarding

of messages, managing the flow of Mail via the BBS Network to and from

other participating BBS systems.

Each BBS that forms a link in the chain will receive Mail from an

adjoining station, and then will Forward that Mail on to the next link

in the chain.  That way, the Mail can find its way all around the

country and all around the world.  Personal Mail addressed to a

particular BBS will hop from one BBS to the next until it eventually

arrives at its destination.

Every BBS in the Network has the opportunity to store those messages so

all local users of that BBS can access them at their leisure.  Of

course, what is STORED on your BBS is entirely up to the owner of that

system.  Some BBS Sysops like to be selective as to what messages are

kept, so you might find some messages "missing" from your local BBS.

But if the BBS is participating as a link in the Forwarding Network it

should be expected to Forward EVERYTHING on to the next link in the

chain, including any messages that might not be stored for its local

users.

Although paKet supports the standard Mail Forwarding protocol, its

role as a PERSONAL system is to manage YOUR Mail, interfacing only with

your local BBS and not with the Network generally.  So any mail you

want to send through the Network or any mail you receive from the

Network, will be channelled through your local BBS.

Page 26

Bulletins

A Bulletin is a general message that is broadcast to anyone and

everyone rather than a personal message addressed to one particular

station. Bulletins may be "addressed" to some topic such as IBM or

HUMOUR or HELP or MORSE... The topics help us to get some idea of what

the Bulletin is about so we don't have to read every message that

appears on the BBS. Bulletins are usually forwarded to every BBS system

in the area, so if there is a Bulletin around, it will probably,

eventually get to your local BBS for you to see.

Personal Mail is usually addressed to a station @ (AT) some BBS

address. For example the local BBS in this area is run by Arthur

VK2ATM. So if you want to send a message to me you would send it to

VK2DHU @ VK2ATM. There must be millions of Amateur Packet Radio

callsigns in use around the world but there are a limited number of BBS

systems so it is easier to track down VK2ATM than it is to find VK2DHU.

This problem is further reduced with the introduction of Hierarchical

Addressing which is a system of specifying more than just the BBS

callsign, adding State, Country and Continent (or Region) details. For

example, my full Packet address according to today's convention is

VK2DHU @ VK2ATM.NSW.AUS.OC (meaning this station is in the State of New

South Wales, in Australia, in the Oceania Region).

Bulletins may be addressed to one particular BBS (eg to MORSE @ VK2ATM)

but more commonly they would be issued to cover to a wider area (eg to

MORSE @ ASIA). Then the Bulletin would be passed on from one BBS to the

next within that area, and every BBS would have that Bulletin available

for users to read.

Page 27

Other Communications Modes

paKet is a COMMUNICATIONS program designed to communicate with a TNC or

Modem.  It provides communication facilities between the computer and

the TNC but does not perform the decoding of packets - that is done in

the TNC!   Neither will paKet decode any of the other communications

modes such as Morse, RTTY or AMTOR.  These modes too, are decoded by the

TNC.

Not all TNCs handle these other modes so if this is what you want to do

you will need one of the Multi-Mode TNCs available (such as the KAM,

MFJ-1278, PK-232 or one of the DSP models) which decode these other

modes.  If you have one of those TNCs, you can use paKet for modes other

than Packet Radio.

The paKet Documentation.

This documentation is supplied in eleven compressed files to make it

easier to manage when printing. Each of the eleven files or Parts is

named MANnn.DOC where the "nn" is the Part.

These files may be combined into a single DOC file if you have the

space. This would be the preferred solution as you can then have the

entire Manual available in a pop-up window during paKet operations

(refer the <Alt-F1> key later).  The supplied INSTALL program, will

attempt to combine these files into a single file PAKET.DOC if you had

the space available on the output drive at the time of Installation.

Have a look at the Table of Contents for details on what you can expect

in the various Parts.

Starting the paKet program

The INSTALL program and the INSTALL.DOC file will help you get the

program installed and ready to run.

To start the program, change into the paKet directory (or log on to the

diskette where PAKET.EXE is installed).  Type:

PAKET

That will use the standard PAKET.CFG configuration file.  If you have

more than one configuration file, for example if you have one set of

parameters for VHF and another for HF ... and so on... you can specify

a different CFG file on the command line.  Eg:

PAKET HF

PAKET VHF

To create a different CFG file, eg: HF.CFG, copy the standard PAKET.CFG

to HF.CFG using the standard DOS COPY command.  Then start the program

with:  "PAKET HF" and make the desired changes through paKet's Online

Configuration Windows.   All changes will be saved into that CFG file

ready for next time.

Page 28