PART X - TECHNICAL

Hardware vs Software Handshaking.

"Handshaking" is a technical term that refers to a method used to

regulate the flow of data between two devices - in this case between

the Computer and the TNC.

Handshaking is required to prevent loss of data. For example, it is

quite possible to send data to the TNC faster than the radio network

can take it, even at low baud rates. This is OK for a while because the

TNC has a memory buffer and the data will be held there until it can be

transmitted. However, there is a limit to the amount of data that can

be held in the TNC's memory, and if the computer continues to send,

some data will be lost. So, if the TNC's buffer is filling up it will

ask the computer (via its "Handshaking") to stop sending.

The reverse could apply, where the TNC could be sending data to the

computer faster than the computer can process it.  If you think about

it, a baud rate of 9600 means there is only one millisecond between

characters!  So, at that rate the computer cannot do much processing,

especially the slower computer systems. If the computer program was

unable to process the data quickly enough, it could use "Handshaking"

to ask the TNC to stop sending in order to allow the computer system

time to catch up.

There are two forms of Handshaking, both supported by paKet:

Hardware Handshaking and Software Handshaking.

Hardware Handshaking:

Hardware Handshaking is preferred, although it will work only if you

have the correct cable installed. A "normal" RS-232 cable will have 9

(or more) pins connected, some of which carry data but most of which

are used for various forms of hardware level communication. With this

method, the two devices (the Computer and the TNC) switch some of these

lines on and off to tell the other whether it is OK to send more data.

Should you wish to prepare a cable to use the Hardware Handshaking, the

following table shows the connections required (connections shown for

both a 9 pin and a 25 pin connector):

DE-9   DB-25   Signal Description

1       8     DCD Carrier Detect   Needed to enable the CONNECT check

2       3     RXD Receive Data     Essential!

3       2     TXD Transmit Data    Essential!

4      20     DTR Terminal Ready   Optional

5       7     GND Signal Ground    Essential!

6       6     DSR Data Set Ready   Optional

7       4     RTS Ready to Send    Needed for Hardware Handshake

8       5     CTS Clear to Send    Needed for Hardware Handshake

9      22     RI  Ring Indicator   Unnecessary for paKet

So, a cable can be constructed using a length of six-conductor flexible

cable (low-capacitance shielded is desirable but not essential) such as

Page 317

telephone flex with a plug on the modem end and socket on the computer

end, wired pin-for-pin according to the above table (it may also adapt

from DB-25 to DE-9 connector in the process).

If, say, the TNC wants to ask the Computer to stop sending, it will

switch off the CTS signal.  The Computer (in this case, paKet) will

stop sending data while the CTS signal is low (or off). When the TNC is

able to accept data once again, it will switch on the CTS line and

paKet will resume sending from where it left off. When this happens you

might notice the CTS "light" in the Status Window come on and off as

the two machines talk to each other.

Check your TNC Manual for any special connection requirements. Most

TNCs do not use the DTR, DSR or RI functions.

Software Handshaking:

Some so-called RS-232 cables have only 3 pins connected: one line for

Receive Data, one for Transmit Data and a Signal Ground. With these

cables, Hardware Handshaking is not possible and you must specify

Software Handshaking.

With this method, the two systems (the Computer and the TNC) must do

their handshaking via special data codes, conventionally known as

<XOFF> and <XON>.

If, say, the TNC wants to ask the Computer to stop sending, it will

send the <XOFF> code to the Computer.  When it is able to accept data

once again, it will send an <XON> character.   When this happens you

might notice in the Status Window, the words "Software Handshaking"

will be replaced by "XOFF" temporarily, indicating we are waiting for

the TNC to clear its buffers before it can accept some more data.

On a three-wire only cable, RTS and CTS should be connected together on

the TNC plug else the TNC may not talk to you AT ALL!

Setting the chosen Handshaking method in the TNC - XFLOW:

It is important to note that configuring paKet to either Hardware or

Software Handshaking will set up the COMPUTER to use that method, but

the TNC needs to be told too! It may not work unless both devices are

set to use the same method.

The TNC's handshaking is set with its XFLOW command:

for Hardware Handshaking, set XFLOW OFF

for Software Handshaking, set XFLOW ON.

If you encounter problems with File Transfers, my experience is that

the problem is often related to the incorrect setting of XFLOW in the

TNC. With paKet's online configuration facility, it is easy to change

from HW to SW mode and vice versa. To do this without changing XFLOW in

the TNC is to invite problems.

Page 318

In closing this section, it should be pointed out (in case it is not

already obvious) that if you do happen to have a full cable, you may

choose to use either Hardware or Software Handshaking. If you choose

Software Handshaking, the system will not use the additional

connections. The reverse is not the case, of course. If you do not have

a full cable, you don't have a choice!

Page 319

PREFERRED TNC SETTINGS

This list of TNC settings is not a complete set of commands for your

TNC. It is intended to be a guide to some of the preferred settings for

use with paKet.

There are some differences in some TNC commands - a command may be

available in one TNC and not another. Or, it may be known in other TNCs

by a different name. So, if your TNC does not have a command listed

below, just skip that one.

You will see in your TNC manual there are many more commands than I

have included here.  Those mentioned here are either critical for

correct operations with paKet, or perhaps are those whose default

settings should be changed.

8BITCONV ON

We do NOT want the top bit stripped from our data. You will see below

we have AWLEN set to 8 so we are using all 8 data bits. This is

especially useful if some graphics characters are being sent.  It does

not affect Binary transfers.

AWLEN    8

Use 8 data bits so we can send all characters including graphic

characters. You should also set PARITY to "None".

paKet must be configured to the same value - refer to the Configuration

Windows - Serial Port for details.

CHCALL   OFF

This must be OFF so we do not get the callsign of the other station

coming through with a change of stream character. (paKet remembers the

callsigns associated with the different streams).

This, in some TNCs, is the same as the STREAMCA in others.

CHDOUBLE ON

This is optional but STRONGLY RECOMMENDED to be ON.

If ON, you must set the corresponding option ON in paKet's

Configuration Windows - Multi User options.

This, in some TNCs, is the same as the STREAMDBL in others. If your TNC

has neither CHDOUBLE nor STREAMDBL you must set paKet's Configuration

option OFF.

CHSWITCH $7C (|)

This parameter is the same as STREAMSW in other TNCs.

Page 320

Have a look at my comments on the STREAMSW command below, but your case

substitute any mention of STREAMSW with your equivalent which is

CHSWITCH.

CMDTIME  1 second

Please leave this set to the default of 1 second.

CMSG ON

This should be ON so your TNC will send your CTEXT string to the other

station when a connection is established.  See the CTEXT command below

for further discussion.

COMMAND  $03 (Ctrl-C)

This must be left set to its <Ctrl-C> default setting.

CONMODE  CONVERSE

The only time paKet must use Transparent Mode is for Binary File

transfers, and paKet will switch automatically. So leave this set to

CONVERSE, otherwise Stream switching will not work correctly.

CTEXT [paKet-6.1-HM$]

The TNC's CTEXT command defines the text string that will be sent to

the other station when a new connection is established (provided CMSG

is ON).

For paKet's Mail Forwarding to work correctly we should ensure the

first thing we send to our BBS when it connects should be our SID, the

special System IDentifier which is discussed more fully in the PMS

section of this Manual.  The way to do this is to ensure that is

included in our CTEXT message.

You can include any other message after the SID if you want, with the

use of the <PASS> character.  This technique is discussed in the PMS

section of the Manual.

DCDCONN  ON

If you are using Hardware Handshaking, set this ON.

paKet will work either way, but the Status Display can then indicate

with its DCD "light" whether or not you are currently "Connected".

If set to ON, it may also be used by the paKet program to confirm the

"Connected" status of the TNC

paKet's Configuration Windows - Serial Port must also be configured to

the same setting chosen for the TNC.  But if your TNC does not switch

the DCD line, set paKet's option OFF.

Page 321

PREFERRED TNC SETTINGS (cont)

ECHO     OFF

paKet does its own echoing, so this should be OFF.

FLOW     OFF

paKet has its own Flow Control, so this should be OFF.

INTFACE  BBS

Kantronics TNCs have this command. Setting INTFACE to BBS will ensure

we don't get unwanted messages interfering with our processing.

MCON     OFF or 0

You may choose to have this option ON sometimes, however it will

interfere with REMOTE Mode and with Binary File transfers if ON - the

system cannot distinguish the Monitored data from the Binary data!

paKet will turn it off before a Binary File transfer anyway.

The preferred setting is OFF.

MDIGI    OFF

Preferred setting is OFF for the same reason as for MCON.

MYCALL  (your_callsign)

This hardly needs mentioning here because this is something that should

be set whether you are using paKet or something else.  But I mention

it, not only as a reminder to those with a new TNC, but also to remind

you to set your name and callsign in paKet's Miscellaneous options

Configuration Window.

NEWMODE  ON

This could be either way and is a matter of personal preference.

Most users seem to leave this at its default setting which is usually

ON.  That means when someone disconnects, the TNC will revert to

Command Mode.  That is fine where you usually talk to only one other

station at a time.

You may like to have NEWMODE OFF if you often work multiple connections

because then when someone disconnects, the TNC will stay in Converse

Mode.

Whichever way you set this option, you should set paKet's option the

same way.  This option is in the Multi User Configuration Window.  Have

a look at that section for more discussion on the NEWMODE command.

Page 322

NOMODE   OFF

Preferred setting is OFF so the TNC will switch to Converse Mode when

you establish a connection.

PARITY   0 (none)

This is related to the Word Length (see AWLEN above) and should be set

to "No Parity".

paKet must be configured to the same value - refer to the Configuration

Windows - Serial Port for details.

PASS     $16 (Ctrl-V)

SENDPAC  $0D (Ctrl-M)

START    $11 (Ctrl-Q)

STOP     $13 (Ctrl-S)

Please leave all these set to their default values.

STREAMCA OFF

This must be OFF so we do not get the callsign of the other station

coming through with a change of stream character. (paKet remembers the

callsigns associated with the different streams).

This, in some TNCs, is the same as the CHCALL in others.

STREAMEV  ON/OFF

paKet does not really need the stream id with every packet. This is

required only upon change of stream.  However in some TNCs, especially

those which do not support STREAMDBL, you might find it beneficial to

have this option ON in your TNC.  So if you are experiencing Stream

Switching problems, set this ON and see if it makes any difference.

You can have it ON all the time - the result will be some additional

traffic between the computer and the TNC but is unlikely you will

notice the difference.

There is no corresponding paKet configuration item for this.

STREAMDBL ON

This is optional but STRONGLY RECOMMENDED to be ON.

If ON, you must set the corresponding option ON in paKet's

Configuration Windows - Multi User options.

This, in some TNCs, is the same as the CHDOUBLE in others. If your TNC

has neither STREAMDBL nor CHDOUBLE you must set paKet's Configuration

option OFF.

Page 323

STREAMSW $7C (|)

Note, some TNCs (eg: AEA brand) use the CHSWITCH command to do the same

thing as STREAMSW.  So if your TNC is one of those just read "CHSWITCH"

whenever I mention "STREAMSW" here.

You may set this character to any value you wish, although most users

seem to retain the default which is the vertical bar character ("|").

It should not be set to one of the valid Stream Id characters, so do

not set STREAMSW to one of the letters A to J.

paKet must be configured to the same character (whatever character you

choose) and this is recorded in paKet's Configuration Windows -

Communications Windows settings.  Usually you would specify the same

STREAMSW character in each Communications Window.  And of course the

same value in your TNC.

If your TNC supports dual ports you will have two of these STREAMSW

characters - one for each port.  In that case you can choose which of

paKet's Windows you want to use for which port.

Suppose your two STREAMSW characters were set in your TNC to "|" for

the VHF port and "~" for the HF port.   Then you could specify (say)

"|" in Comms Windows 1 and 2, and specify "~" in Comms Window 3.  This

would mean that when you wanted to switch to the HF port, you could

simply press <Shift-F3> to select Comms Window 3!  Anything coming in

on the VHF port for Stream A would go into the first Window; anything on

Stream B to the second Window; and anything received on HF would go

into Window 3.

Of course you can choose any combination of Windows, Streams and

STREAMSW characters you wish.  I am just using these window numbers etc

as an illustration.

The important point here is that the STREAMSW character(s) your TNC

recognises MUST be the same as the character(s) you use in the

Communications Windows Configuration.

TRFLOW   OFF

TXFLOW   OFF

These commands are used for Software Handshaking in Transparent Mode.

You should set them both OFF. paKet will switch them on and off if

required.

USERS    n

Although not essential, it is considered desirable to set this

parameter to the same number of users as you have Communications

Windows. Then you know you can have a separate window for each of the

possible connections.

If your TNC allocates a separate callsign to its PMS I suggest the USER

parameter be set to the number of Communications Windows plus one.

Page 324

XFLOW    ON/OFF

This parameter is important and must be set to reflect the chosen

method of Handshaking that you have specified in paKet's Serial Port

configuration:

For Hardware Handshaking, you must have XFLOW OFF.

For Software Handshaking, you must have XFLOW ON.

Refer to the discussion of Hardware vs Software Handshaking for more

details.

XOFF     $13 (Ctrl-S)

XON      $11 (Ctrl-Q)

Please leave both these set to their default values.

Page 325

Kantronics TNCs

My comments in this section are directed at the KAM specifically but I

understand other models in Kantronics range operate in a similar manner

so in many cases these comments would most likely apply to the other

models as well.

Several KAM users have written to me about problems, mainly in the area

of Binary File transfers, with the previous versions of paKet.

paKet was written to suit the standard TAPR TNC (and compatibles) which

was the first of the popular TNCs used in Amateur Radio. They created a

Standard on which all modern TNCs are based.   paKet is being used on

many thousands of systems around the world with a wide range on TNCs, so

it seems the approach taken has been successful in most cases.

However, because of the differences that exist in Kantronics TNCs,

users of those TNCs should be aware of some potential operational

problems.

In this version of paKet I have made some internal changes in the paKet

processing to better handle these TNCs. I have had limited access to a

KAM for testing and have tried to accommodate the differences in that

TNC.  For example I have provided for dual ports and have improved the

code for the handling of multiple streams.   The big problem I have had

with this TNC is the lack of support for the standard STREAMDBL

command.   Without that facility in the TNC I am unable to guarantee

reliable Binary Transfers. However, it seems to work well both sending

and receiving Binary Files when using a KAM.

All I can say to users of the KAM and other Kantronics TNCs, is to give

it a try.  If a Binary Transfer fails it is probably due to some rare

combination of cimcumstances in that file, but in most cases it does

seem to work OK.

Perhaps the other thing you could do is mention this to Kantronics and

see if they have a fix for it yet.  The Kantronics TNCs I have seen are

first class units, apart from this one anomaly.  If it were not for the

omission of STREAMDBL, I am sure these TNCs would be considered among

the best in the world.

If you are experiencing some unusual and unwanted stream switching, you

might switch on the STREAMEV option in the TNC.  That will generate a

small amount of overhead, probably not enough to notice, but it might

offer some relief, helping to keep things coming into the right window.

Of course there are a considerable number of enhancements in the KAM,

supporting all those other modes such as RTTY, AMTOR, etc. I am not

referring to those changes, but rather to the standard Packet Mode and

the standard commands used by TAPR-compatible TNCs.

Page 326

The following differences have been noted in testing:

1. STREAMDBL command

Because the Kantronics range of TNCs do not support the STREAMDBL

command, you will have to set paKet's STREAMDBL option OFF.

STREAMDBL is a standard TNC facility to help avoid confusion where

the STREAMSW character is received over the air as part of a data

packet.

For example, the vertical bar character ("|") that is usually used as

the STREAMSW character is also often used as a pseudo graphic

character to draw vertical lines or for drawing boxes. When paKet

receives this character from the TNC it must decide whether the TNC

is indicating a change of stream or whether this is just another data

character received over the radio.

TNCs which support STREAMDBL can generate TWO of these vertical bar

characters whenever one is received over the air - then paKet can

tell the difference and there is no confusion.

Without the STREAMDBL feature, you could experience some occasional

(unexpected) changing of streams if you are monitoring the frequency

and happen to receive a STREAMSW character (say a 'vertical bar')

followed by some other character (say 'C').  paKet has no way of

knowing whether the TNC is requesting a change of stream (to stream C

in this example) or if those characters were received over the radio

because the KAM sends the SAME data to the computer in either case!

In order to support multi stream operations, paKet has no choice but

to process it as a change of stream!

With a view to improving reliability in Binary Transfers, I have

chosen to assume a STREAMSW character received during a Binary File

Transfer is actually a data character and not a change of Stream.  In

the testing I have done with the KAM, this has improved the success

rate significantly.  So you should ensure we don't get a genuine

change of stream (including dual port operation) while performing

Binary File Transfers otherwise my assumption could be wrong!

Another anomaly with the KAM is where responses to various commands

were going into a different Communications Window!  It seems in

Command Mode, the KAM might not respond to a command on the same

stream the command was issued.  The symptoms are paKet and the KAM do

not agree on what is the current Stream!

I cannot change the KAM so I cannot prevent these problems from

occurring. However I have added the <Alt-I> command, to help tidy up

your display if you do happen to experience these problems on your

system. The <Alt-I> command will initialise the Communications

Windows, clear any backlog in the other Windows, and reset both the

program and the TNC back to the first Stream in the first

Communications Window.  Refer to the <Alt-I> command in the Keyboard

Commands section of this Manual for details.

Page 327

2. RESET used for soft start

When making changes to the TNC's Communications settings, the usual

procedure is to issue the TNC commands to make the desired changes

then, after making the changes to the computer's serial port the TNC

is initialised to make the new settings effective.

On most TNCs the command to re initialise is RESTART, but on the

Kantronics units the command is RESET.   This is not a problem for

paKet because I have added a configurable option in the Serial Port

options to allow you to specify either RESTART or RESET. So if you

are a Kantronics user, just specify RESET for that option and paKet

will issue that command whenever you make any changes to the Serial

Port parameters.

There is a KAM.HLP file supplied with this version of paKet.  If

you are using a KAM or any of the Kantronics TNCs, you can select this

Help File by specifying this file in your Directories and Files

Configuration Window.   Press <F10> to call up the Help details.

Page 328

PacComm PacTOR Controller (PTC)

This is a strange machine, quite unlike the other units from the

PacComm stable that I have seen.  It is rather obvious this was

developed by a team that was not talking to those who developed other

units.  And I suspect these people have not used a TAPR TNC either -

such is the difference in the style of operation.

But it does work well and I just LOVE all those pretty LEDs on the

front panel!  It handles PACTOR, AMTOR and RTTY Modes which are

intended primarily for HF operation.  I understand there will be an

optional Packet board available for VHF operation.

The commands to drive this unit are unlike any other so you will find

paKet's Keyboard Macros handy for the various codes this unit requires.

For example I think it would be convenient to have a macro for the

AMTOR changeover "+?".  If you use F11 or F12 for this it is all done

with a single keystroke.  That makes your operations more convenient if

you use this combination frequently.

Same goes for switching from say, PACTOR to RTTY modes, etc.  There are

several keystrokes involved in switching from one mode to another

especially if you want to enter Listen Mode when you switch.  Depending

on your own operations, you could set up Keyboard Macros for just those

things you use regularly.

Set paKet's Serial Port Configuration:

- the TNC cmd: exit code should be set to <Esc> because this is the

code required to begin a command!  When you need to enter a

command, press <Alt-B> which is paKet's standard method to return

a standard TNC to command mode.

NOTE- in AMTOR Mode, the Controller remains in Command Mode for

one line only!   So for multiple commands you must press <Alt-B>

before EVERY command.

- the Command to Initialise TNC is RESET.   Yep, in this case the

RESTART is the dangerous one that clears EVERYTHING from the

PacComm Controller!  So you should set this to RESET and ignore

the warnings paKet gives you when you do so.

- The PacTOR Controller's firmware supports only Software

Handshaking so set paKet's Handshaking option in the Serial Port

Configuration to S to ensure paKet too uses Software Handshaking.

The PacTOR Controller supports the PASS character but in a slightly

different way.  It is called the Ctrlchr and when you want to send a

control code (eg: <Ctrl-Y>) you would type <Ctrl-V> and Y.  On other

TNCs you would type <Ctrl-V> <Ctrl-Y>.  So for the PTC, you should

ensure you have nothing specified in the "Control codes to PASS"

configuration window.  (It would have been so much easier if they had

conformed to the standards, but...)

The PTC's date format is different too!  So you will have to disable

the automatic DA command in the Miscellaneous Configuration options.

Page 329

DSP-12

The DSP-12 has proved popular with the Satellite users because of the

sheer flexibility of this unit.   There are other brands of DSP

Controllers available but to date I have had no experience with them so

cannot comment on those.

There are a couple of local DSP users including Carl, VK2JJM.  I asked

Carl for any thoughts on the subject and he produced this little gem.

Thanks Carl.  I did modify it a little but the wisdom contained herein

is essentially Carl's.

Operational Hints for using the DSP-12 with paKet.

(Written by Carl, VK2JJM)

(Slightly edited by Tony, VK2DHU)

The DSP-12 will work successfully with paKet if configured correctly.

The DSP-12 has several extra features not available in a standard TAPR

TNC, but it also lacks some standard TAPR commands.  The following

list will help setup the DSP-12 and paKet for basic operation.

These notes refer to the current release of the DSP-12 firmware at the

time this was written, in early 1994.

1) The DSP-12 does NOT support multiple stream connections in VHF

packet.  You should to set the maximum number of Comms Windows in

paKet to 1, and also set the STREAMDBL option OFF in the Multiple

Users Option menu because, with only a single Stream, this is

irrelevant.

2) The DSP-12 does not appear to support hardware handshaking so, for

reliable handshaking, you should set paKet's Handshaking to SW in

the Serial Port Configuration.

However it seems the DSP-12 does switch the DCD line upon

connection to another station.  If you want to use paKet's DCD

option to verify a connection, you will need to configure paKet for

Hardware Handshaking!

With the large buffer space available in this unit, some users have

found handshaking somewhat unnecessary anyway and prefer to have

the system set to Hardware Handshaking and have configured paKet's

DCD option on.

3) The DSP-12 has a nasty habit of exiting the <VHF PKT> mode and

returning to the Main Menu if the PTT 3 minute timer is enabled in

the Debugging menu. This timer is only used for full-duplex

Satellite Modes, so disable it. This setting is NOT stored in

bbRAM, so you will need to set it each time you enter the <VHF PKT>

mode.

Page 330

4) The DSP-12 has about 500k RAM free when operation from ROM, and

about 300k RAM when operating from RAM.  Most of this memory is

allocated to buffers so when sending a Binary File, paKet can often

send the entire file to the DSP-12 buffer, and then waits for the

other station to acknowledge the End-of-File. It can take quite a

while for the other station to acknowledge the EOF depending on how

much data is still in the DSP-12 buffer.  If the standard 2 minute

timeout expires, paKet will ask you how much longer you want to

wait. Give it say 10 minutes, longer if you wish, and paKet will be

happy to wait for that period before asking you again.  It

remembers that time so from now on it will always wait 10 minutes

(or whatever) before reminding you again.

5) When in the mailbox mode the DSP-12 will still send the connect

message defined in CText straight after the Mailbox user menu.

This gets a bit confusing to anyone connecting to the DSP-12

Mailbox.

The Mailbox in the DSP-12, as in most other TNCs, does not have the

same rich features found in paKet's PMS, so use the PMS whenever

paKet is running!

Set "MAILBOX OFF" and "CMS ON" in paKet's Begin Auto commands.

Do the reverse ("CMS OFF" and "MAILBOX ON") in paKet's End Auto

commands.

6) Some of the DSP-12 default values need changing for ideal VHF

packet operation.  The most obvious are DWait and FRack.  Set these

to 16 and 3 respectively.

Page 331

Running paKet under Microsoft Windows

Although paKet uses a number of windows for its operations and various

displays, these windows are paKet's own.  They are NOT "Windows" in the

now popular sense  - meaning part of the popular Microsoft Windows

product.

paKet is a DOS, text-based program and will run on any IBM compatible

hardware including the original PC with its 8088 processor, although

you will need more than its original complement of 64KB RAM!  It takes

me back a bit, but I remember the first PC I saw had a whopping 128KB

RAM and one single sided 160KB diskette drive!  And that lovely

monochrome display.  No hard disk of course - you wouldn't need THAT on

a PC!  Oh, and it had a cassette tape interface. (Sigh....memories...)

Uh, where was I?

With a modern system running Microsoft Windows Enhanced Mode, you can

run multiple DOS tasks.  Several people have reported paKet runs quite

happily in the background as a DOS task.  I too have tried it and found

it ran perfectly well for a while, but lately it seems to miss a few

characters from time to time.  I suspect it has something to do with

the latest Microsoft Mouse driver (version 9) which I installed

recently, but I have not yet identified the cause for sure. It seems

fine while I am not using the mouse!

If that happens to you try reducing the baud rate between the computer

and the TNC.  That gives the system a little more time to process the

incoming data characters.

When running under Windows (by the way, when I type "Windows" with a

capital W here, I am referring to the Microsoft Windows product, OK?),

paKet is using its own Communications Interrupt drivers and not using

the COMM drivers that are part of the Windows system.  So there might

be some sort of conflict there. Anyway, I find it works pretty well

most of the time and the convenience of multi tasking is worth any

slight problems.  It seems to run a little better under OS/2 than under

Windows and I will talk some more about that later. It means I can have

paKet running all the time, including those times when I want to use

the system to do other things.

The current version of Windows as I write is version 3.1 (or 3.11).  By

the time you read this there might be the new version 4 (Chicago?) or

even later versions available and things could be quite different?

To run paKet as a Windows task, you should use a PIF file.  I have

included a PAKET.PIF file with the original paKet distribution files,

so have a look at that.  Note the Background Execution option is

enabled and I think you will find it works better if the EMS and

Application Memory is locked (if you have enough RAM to do that). Those

options are in the PIF file and you can see them with the PIF Editor

you got with your Windows system. Have a play with the settings and I

am sure you will find it will work fine for you.

I have also included a hastily prepared Icon.  Maybe someone will send

me a better one?

Page 332

Try setting a Shortcut key combination - it makes it so easy to jump

directly into the paKet window, no matter how many other things you

have cluttering up the system.  I used <Ctrl-Alt-P> and that is

set in the File Properties dialog box.  Highlight the paKet icon and

select File then Properties from the Program Manager Menu.

I often have paKet running in a Windows window rather than in Full

Screen Mode.  That allows me to have a small part of paKet on display

in case I want to watch what is going on while I am working on

something else.  You can drag the corners to enlarge it to full size

and still find it occupies less than the full display.  Click on the

little box at the top left of the window and you will see the Fonts

option. Experiment with those and you will find (if your screen is big

enough and good enough) you can have a full, and readable, 80 x 25 text

display in a small font so it occupies just a small part of the Windows

display. There are so many options for presenting the display I will

leave you to experiment and find a combination that you are happy with.

You should also experiment with the Time Slicing options too - I have

set it to 100 Foreground and 60 backgound, but other settings seem to

work well too.  This is fine tuning and the optimum settings may be

different on different systems.

If you switch paKet to a Full Screen (press <Alt-Enter> - note that must

be the LEFT Alt key) you will find paKet will run faster because the

display system can now run in true text mode instead of the pseudo

text/graphic mode that Windows must use to manage the display with the

the smaller fonts.

I prefer Full Screen mode when I am actually USING paKet on air. It

runs most of the time as a Windows window and when I want to do

something, I switch into that paKet window with <Ctrl-Alt-P> then press

<Alt-Enter> to go Full Screen.

By the way, I think you will find the Morse Alerts a boon while paKet is

running in the background - then any particular alert strings you are

looking for will still make their unique sounds even if paKet is not

showing at all.

A Windows version of paKet?

In anticipation of the inevitable enquiries, let me say I am aware of the

proliferation of Windows systems "out there".   Windows demands lots of

RAM and CPU power, but it seems there are a lot of people using Windows

now.   Whether it is because Windows is so much better or whether it is

just good marketing by Microsoft, is not for me to say. I know some

people won't touch Windows at all, while others won't use anything else!

Let's just say I will have a look at making a Windows version available

at some later date.  But I must emphasise LATER.  It may be paKet will

go to a multi threaded system such as OS/2 before it moves to Windows.

I am not too sure yet.  For now it is a DOS program that will run fine

as a task under Windows or OS/2.  I have not done it, but I understand

it is also quite happy running as a task under DESQview.

Page 333

Running paKet under OS/2

At the time of writing this Manual (early 1994) OS/2 is still running

well behind Windows in terms of market share, but it seems to be

gaining acceptance, mainly from the power users who want the multi

tasking, multi threading facilities, 32-bit operating system, etc etc

but not necessarily the nice Graphic Interface offered with Windows

(including NT) with all the overheads associated with that type of

system.

Perhaps this is one of the reasons for Microsoft's success in the

market - their products are very well presented.  If IBM devoted some

more attention to installation procedures and support for a wider range

of standard, popular devices I am sure they will win even more converts

to this system.  (Neither my Panasonic CD-ROM drive nor my Canon Bubble

Jet printer are supported in the distributed copy of OS/2 version 2.1,

and these devices are high volume sellers - they are in this part of

the world anyway).

Of course OS/2 offers a Graphic Interface with its Workplace Shell, but

from what I have seen it does seem to perform better than Windows IF it

is tuned properly!  It offers far more tuning options and flexibility

than Windows, especially for the individual DOS Sessions, and it seems

to be a more rugged and reliable multi tasking system.

Unlike Windows NT with its exorbitant demands for memory and disk space,

OS/2 seems to require not much more than a standard DOS plus Windows

combination.  Maybe NT offers a lot more so I should not be making the

comparison?  I did install it for a client but have not used it myself.

There is a large number of paKet 5 users running OS/2 and they all

praise the wonders of that system.  OS/2, not paKet!        Well, both.

I too have been experimenting with OS/2 and have found that tuning some

DOS Settings can make paKet fit in to the OS/2 environment much more

effectively.

When running paKet in a DOS Session using all the standard defaults, the

Pulse Utility program indicated paKet was going flat out, using all

available CPU time, even when there was "nothing happening".  I can

understand that because paKet is written as a DOS program and in that

single tasking environment it must run around that loop waiting for

something to happen, looking something to process, updating the Status

display, etc.   Of course it is running flat out - it was designed that

way!

But changing a few of the DOS Settings for that session made a big

difference to the system performance.  I changed IDLE SECONDS to 0 and

IDLE SENSITIVITY to 25.  With these settings OS/2 was able to determine

that paKet was in an idle Keyboard loop, even though paKet IS going

flat out around that loop waiting for something to happen.  The Pulse

indicator was now indicating an almost idle system (I had nothing else

running at the time to help avoid confusion in the Pulse readings).

Page 334

It still handles the interrupts OK so there are no lost input

characters.  And the keyboard responds OK too.  But the clock display

in the Status Window was updating once every 2 seconds while there was

no packet activity.  That suggests to me that OS/2 is managing its

resources very effectively - giving paKet all the time it needs when

there is some real work to do, and minimising the resources consumed by

paKet when there was very little to do.

So now paKet can run in its own DOS session, and while it has nothing

to process there is negligible overhead, giving the lion's share of the

system to other tasks!  As soon as something is received from the TNC

or upon any keystrokes, paKet is given its share of the system to

process its data promptly.

I suspect other systems might behave differently with these DOS

settings and some experimenting might be justified for different

processor speeds etc.  It is easy to experiment here - just leave Pulse

running as you change the DOS Settings for paKet's DOS session.  You

will see right before your eyes the effect of the different settings.

You should review all the DOS Settings and tune the system to your

liking.  Some others to look at are DOS_BACKGROUND_EXECUTION ON and

COM_HOLD ON.

My experience with this system is limited so it would be good if those

of you with more OS/2 experience could post some Bulletins to inform

other paKet users of the settings, configuration, tuning tips etc to

help others get the most from this powerful system.

With any multi tasking system, it might be a good technique to run with

a lower baud rate between the computer and the TNC.  Unless you are

running a 9600 baud radio link, there is little real need to run a high

terminal baud rate.  I think it helps the system to share resources and

you are less likely to experience problems if a lower baud rate is used.

Page 335

The PAKET.CFG file

paKet is VERY configurable as you will have noticed when looking at all

the options in the Configuration Windows.   All these settings are

stored in the PAKET.CFG file but you really do not need to know how

they are stored nor what it all means.  It would be only in very rare

circumstances, and then only if you are an experienced operator, that

you would bother to even look at this file.  It is normally managed and

updated by paKet's Online Configuration and I recommend you leave the

program to maintain this file.

However, in the interests of complete documentation I am including a

list of all items in this file.  I have added some comments where

necessary, but for most items I have simply identified which

Configuration Window they refer to so you can get full details by

referring to the Configuration Windows section of this Manual.

There are two main sections to the file:

- the keyword section which contains most items;

- the strings section which contains the optional strings, including

Keyboard Macros, Begin Auto commands, End Auto commands, and

Alerts.

The PAKET.CFG file - Keyword section

You will see there is a keyword for each item in this part of the

file.  It was not my intention that this file be viewed by paKet users,

so some of the keyword names might not seem all that meaningful.  But

most keywords will give some clue as to the configuration item it

relates to.

In any case they are shown here in the same sequence they appear in the

Online Configuration Windows so you can more easily identify which is

which. The first item, VERS, must be the first line in the config file,

but the sequence of all other keyword items is not important. It is

likely they will appear in a different sequence in your config file.

In your file, you should see the same keywords but in many cases the

values to the right of the '=' will be different:

VERS=paKet 6.1           This must be the first line in the file.

These items are from Option 1, the Serial

Port configuration

PORT=2

BAUD=1200

PARITY=N

AWLEN=8

STOPS=1

HSHAKE=HW

DCDCON=Y

BREAKCODE=B

RESTART=RESTART

Page 336

These are from Option 2,the Multi User

options

MAXWINDOWS=3

DOUBLE=ON

NEWMODE=ON

STTYPE=A

These are from the Option 3, Communications

Window configuration.  In that Window there

are 10 columns, one for each of paket's

Communications Windows.

Each of the following items has 10 values

separated by a comma, with each value

corresponding to the 10 columns in the

Configuration Window.

STREAMID=A,B,C,D,E,F,G,H,I,J

STREAMSW=|,|,|,|,|,|,|,|,|,|

INBUF=1,1,1,0,0,0,0,0,0,0

FBBUF=64,48,16,2,2,2,2,2,2,2

WWSTART=48,48,48,48,48,48,48,48,48,48

WWEND=79,79,79,80,80,80,80,80,80,80

BELLS=575,1,1,1,1,1,1,1,1,1

PHONE=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

PROCESS=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

STATUS=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

TA=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

TALINES=2,2,2,2,2,2,2,2,2,2

The following items are not shown in any

Configuration Window but are stored in the

config file so paKet can maintain separate

settings for Single Line, Justified and

Word Wrap modes.  These are set with the

<F4> key.

SINGLE=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

JUSTIFY=N,Y,N,N,N,N,N,N,N,N

WW=Y,Y,Y,Y,Y,Y,Y,Y,Y,Y

These are from Option 4, Colours which are

recorded for each of 10 different windows.

These are listed in the same sequence as

shown in the Colours Configuration.

COLMSG=WHITE,GREEN,LIGHTGREEN,BLACK,WHITE,GREEN

COLCOMMS=BLACK,CYAN,WHITE,CYAN,WHITE,BLUE

COLSTATUS=WHITE,BLACK,YELLOW,BLACK,LIGHTGREEN,BLACK

COLBIN=DARKGRAY,BROWN,WHITE,BROWN,WHITE,BROWN

COLHELP=WHITE,BROWN,WHITE,BLACK,WHITE,BROWN

COLTNCHELP=BLACK,LIGHTGRAY,YELLOW,BLACK,LIGHTCYAN,BLUE

COLTA=LIGHTGRAY,BLACK,WHITE,BLACK,WHITE,BLUE

COLCFG=BLACK,LIGHTGRAY,WHITE,LIGHTGRAY,WHITE,RED

COLDIR=BLACK,LIGHTGRAY,YELLOW,BLACK,WHITE,BROWN

COLMAN=BLACK,LIGHTGRAY,LIGHTCYAN,BLACK,LIGHTCYAN,BLUE

Page 337

In Option 5 of the Online Configuration we

have the Keyboard Macros, Auto Commands and

Alerts.  But those items are recorded

separately at the end of this config file.

We'll come to those shortly. The other

items in Option 5 are:

PASSCODES=GHITXZ

KISSOFF=192 255 192

These are from Option 6, REMOTE / PMS

options.

REMOTE=Y

AUTOMENU=Y

REMUP=Y

REMDOWN=Y

TRIGGER=29

BBSCALL=VK2ATM-1

THIRDPARTY=Y

DELFWD=N

CALLHOURLY=M

CALLMINS=06

CALLDAILY=A

CALLTIME=23:54

BBSSCRIPT=N

BBSPATH=VK2ATM-1

These are from Option 7 the File Transfer

options

NDW=16

BDW=48

NPACL=240

BPACL=252

PP=Y

ADDCTRLZ=N

These are from Option 8, Directories and

Files.

DEFAULTDIR=C:\PAKET\

TEXTDDIR=C:\PAKET\TEXT\UPLOAD\

TEXTUDIR=C:\PAKET\TEXT\

BINDDIR=C:\PAKET\BIN\UPLOAD\

BINUDIR=C:\PAKET\BIN\

LOGDIR=C:\PAKET\LOG\

PMSDIR=C:\PAKET\LOG\

SCRIPTDIR=C:\PAKET\TEXT\SCRIPTS\

MANUALFILE=C:\PAKET\PAKET.DOC

TNCFILE=C:\PAKET\PK-232.HLP

EDITOR=D:\EDWIN.COM

BUDDYS=C:\PAKET\PAKET.DIR

TYPECMD=C:\UTILITY\LIST

Page 338

The following items are not actually in the

Online Configuration at all!   These are

recorded here in the config file so paKet can

start up next time, remembering some file

details you used last time.  So, for example,

when you press <F5> to send a file paKet

remembers which file you used last time.

Then it can bring up the Directory Window

with the same file already highlighted.

DIRPATH=C:\PAKET\PAKET.CFG

EDITFILE=C:\PAKET\LOG\VK4WAC.072

SCRIPTFILE=C:\PAKET\TEXT\SCRIPTS\GETMAIL.SCP

F5DEFAULT=C:\PAKET\LOG\VK2JJM.267

F7DEFAULT=C:\PAKET\BIN\SCAN.LZH DUMPFILE=E:DMP

These are from the Option 9, the

Miscellaneous options.

MYCALL=VK2DHU

MYNAME=Tony

MSGLEVEL=A

MSGDELAY=3

CFGHELP=Y

CGASNOW=0

AUTOLOG=Y

LOGHDRS=Y

BLANKDELAY=60

ARROWS=F

DAYTIME=Y

These items do not appear in the online

configuration and some might not even

appear in your config file at all!

ALARMS=Y                 You may toggle Alerts with the <Alt-A> key.

CWALARMS=Y               These two items reflect the current

settings for the Alerts option.

DCDDELAY=30              May be used to increase delay for those

TNCs which are slow to switch the DCD

signal when a connection occurs.  Time is

in hundredths of a second so a value of 30

is 3 seconds.

DISPMEM=Y                Displays free memory.  Used for debugging.

(<Ctrl-Alt-M> toggles this display)

PMSMSG=1269              This is the next message number to be used

by the PMS.

PMSSUFFIX=A              When generating a BID for the Forwarding

system, paKet uses the message number and

your callsign to construct the BID.  This

is designed to ensure it uses a unique BID

for every message. Eg: $14_VK2DHU.

Page 339

But if you reset your PMS files and start

using the same message numbers again, you

might find the BBS rejecting some mail

because it finds this BID already exists!

You can add this line to the config file so

paKet will add a suffix to the BID to

ensure the new series of BIDs are still

unique despite using the same message

numbers!  Eg: $14A_VK2DHU

QUIET=N                  Quiet Mode is toggled with <Alt-Q>

WPM=20                   This is the Morse Code speed for the

Alerts.  This is adjusted with <Alt-J>.

YAPPWAIT=5               The default timeout period according to the

standard YAPP File Transfer Protocol is 2

minutes, but if you have a TNC with a BIG

buffer paKet might timeout after the 2

minutes, before that big buffer can be

emptied.  You can tell paKet to wait for

some other timeout period, presumably a

longer time.  That period is stored here in

this variable so next time you won't be

interrupted every 2 minutes!  If this has a

value of 5, paKet will wait for 5 minutes

before reminding you it is still waiting.

The PAKET.CFG file - Strings section

In this section at the end of the config file are 4 areas containing

some optional strings.  The four areas are separated by the mandatory

dividers which are the lines beginning with the "***".  These dividers

must be present in the file even if there is no other detail recorded.

The first area is the Keyboard Macros.  Each line contains the Keyboard

Macro number followed by the contents of that macro.  This corresponds

to the detail in the Keyboard Macros Window  (<Alt-Z>, 5, 1).

*** Start of Keyboard Macros

1 ~BMAIL ON<Enter>MDC<Enter>l<Enter>

2 ~BB<Enter>MAIL OFF<Enter>

3 ~BMCON OFF<Enter>C VK2ATM-1<Enter>

4 73s from Tony in Port Macquarie<Enter>

6 ~BXMITOK T<Enter>

8 <~s bbsconn>

9 ~BD<Enter>

0 ^CD^M

11 !xtgold

12 Disconnecting from $u at $t...<Enter>~BD<Enter>

Page 340

The second and third areas are the Begin and End Auto Commands and

these too are as shown in the respective Configuration Windows.

*** Start of Beginning Auto Commands

echo off

xmitok on

CTEXT [paKet-6.1-HM$^V

If you get the Menu, try: ^V

H for Help or^V

S to "Send me a message^V

...Tony

<~S MAILOFF>

<~S MORSE-ON>

*** Start of Ending Auto Commands

^S

CTEXT paKet is not currently running.^V

Please leave a message in the PK-232 Mailbox.^V

...Tony.

MAIL ON

echo on

xmitok on

conok on

The final area is the optional Alerts strings.

*** Start of Alerts

PAKET

TONY

DHU

#WX#EXPERIMENTAL WEATHER STATION AT PORT MACQUARIE

BZC

WAC

Page 341

(This page intentionally blank)

Page 342