PART III - SPECIAL KEY CODES

Overview

Special Key Codes are expressions of keystroke-combinations which, if

required, need to be specially coded in the various "Command Texts"

such as:

Keyboard Macros,

Script commands:

Begin-Auto or End-Auto commands.

An example of a Special Key Code is the <Ctrl-C> combination. When you

are using the keyboard this key-combination can be entered into the

system by typing the letter C while holding the Ctrl key. Trying to

write such a combination into one of the Command Texts requires special

syntax, which is described in this section.

Types of Special Key Codes include:

Ctrl-key combinations such as <Ctrl-V>

Alt-key combinations such as <Alt-B>

Function-keys such as <F2>

Many commonly used command keys such as <Enter>

Macro Substitutions such as $D

In this section, in fact in the entire Manual, I use the Less-Than and

Greater-Than ('< >') characters as brackets to to better identify

special key names.  I call them "Pointer Brackets".

Ctrl-key combinations.

Any of the <Ctrl- > key combinations from <Ctrl-A> to <Ctrl-Z> may be

entered into a Command Text.

The syntax is to enter two characters in the Command Text, the caret

('^') followed by the desired Control Letter. For example:

^H for <Ctrl-H> (which is the ASCII code for Backspace)

^M for <Ctrl-M> (which is the ASCII code for Enter)

So a Keyboard Macro which contains the following 5-character string:

^CD^M

would result in 3 bytes being sent to the TNC. The first 2 characters

of the Macro would be combined into a single <Ctrl-C> character; the

letter 'D' comes next; then the last 2 characters in the macro would be

combined into a single <Ctrl-M> character (same as Enter).

If you want to send a caret ('^') to the TNC, enter 2 of them in the

Command Text. So, for example this Keyboard Macro:

ABC^^XYZ

would result in the following text being sent to the TNC:

ABC^XYZ

Page 105

Alt-key combinations.

Any of the <Alt- > key combinations may be coded into a Command Text

too.

The syntax is similar to that for the <Ctrl- > key combination but here

we use the tilde ('~') as the prefix. For example:

~B for <Alt-B> (which is paKet's BREAK command)

~6 for <Alt-6> (which is Keyboard Macro 6)

So a Keyboard Macro which contains the following 4 character string:

~P~H

would send our MHEARD list to the printer. Assuming the Print Log

function is currently OFF, the first two characters (~P) would be

processed as <Alt-P> to turn ON the Print Log. The next two characters

(~H) are processed as <Alt-H> which sends the MHEARD command to our

TNC.

If you want to send a tilde ('~') to the TNC, enter 2 of them in the

Command Text. So this Command Text:

ABC~~XYZ

would result in the following text being sent to the TNC:

ABC~XYZ

In a keyboard macro you may also perform a Script, which is usually

initiated via the <Alt-S> key.  In the keyboard macro, the script file

name must be specified.  For example to run the Script named

GETMAIL.SCP you would specify the following in your keyboard macro:

<~S GETMAIL>

The space after the "~S" is optional but I think it makes it easier to

read if you put the space in there.  If you have assigned that macro as

the first keyboard macro, you simply press <Alt-1> to perform the

GETMAIL Script.

If you want to send some additional parameters to be processed with

that Script, you enter them inside the Pointer Brackets and separate

them with commas.  For example:

<~S GETMAIL,XY4BBS,PAKET,AMSAT>

Then when that Script is being processed, the Script parameters may be

accessed as follows:

%0 contains GETMAIL (the name of the Script)

%1          XY4BBS

%2          PAKET

%3          AMSAT

Scripts are discussed fully in that section of the Manual.

Page 106

Function Keys.

Any of the Function keys may be specified in a Command Text.

The syntax is to specify the letter 'F' followed by the number of the

Function Key and surround that with Pointer Brackets. For example:

<F2> for Function Key 2 (which paKet uses to toggle the Log File)

<F10> for Function Key 10 (display TNC Help File)

You can also specify an <Alt-Fkey> combination as follows:

~<F8> for <Alt-F8> (for Receiving a Binary File from BayCom)

So a Keyboard Macro which contains the following string:

YD PROG.EXE^M<F8>

would send the string "YD PROG.EXE" to the TNC, followed by a Carriage

Return (the <Ctrl-M>). Then the "<F8>" causes paKet to behave as if you

pressed the <F8> key - to receive a Binary File.

Other commonly used command keys.

The following keys may be coded for use in Command Texts and are

written as spelt here, surrounded by Pointer Brackets:

UP                 HOME              ENTER

DOWN               END               ESC

LEFT               PGUP

RIGHT              PGDN

Upper or lower case (or any combination) is OK.

For example, instead of using the ^M combination as demonstrated above

for a Carriage Return, we could specify <Enter>.  The two codes (^M and

<Enter>) do the same thing - it is a matter of preference. I think

"<Enter>" is more descriptive and should be preferred unless you need

to reduce the number of characters on the line.

So, the example given in Ctrl-key combinations above (^CD^M) could be

written as:

~BD<Enter>

The <Alt-B> is a better method for returning the TNC to command mode

than <Ctrl-C> although for many TNCs it would achieve the same result;

the 'D' is the Disconnect command;  and "<Enter>" generates a Carriage

Return.

Page 107

Macro Substitutions.

A Macro Substitution is a paKet facility where specially coded key

sequences in Command Texts are converted to text as shown in the

following table:

$D  Today's Date (MMM-DD)

$N  Your name as configured

$O  Your own callsign as configured

$S  Stream Id of currently selected stream

$T  Current Time (HH:MM)

$U  User's callsign (in the current stream)

$#  Number of active sessions (connected)

$^  Number of available sessions as configured

$%  List of callsigns currently connected

For example say I had the following text recorded as a keyboard macro:

$O disconnecting from $U on $D at $T.^MBye for now...$N^M~BD^M

When I select that macro, the following text would be sent to the TNC:

VK2DHU disconnecting from VK2BZC on JAN-01 at 21:45.

Bye for now...Tony

The Macro Substitution codes in this keyboard macro are replaced by the

text currently assigned to each of those codes.

For example - the $O and $N codes are taken from the paKet

Configuration;

- $D and $T are taken from the computer's clock at the

moment the macro is processed;

- $U was determined when (s)he connected earlier.

(In the example I have assumed my callsign is VK2DHU and

that I was connected to VK2BZC at the time).

The ~B (i.e. <Alt-B>) puts the TNC into command mode and then that is

followed by the Disconnect command.

You will note I used the "^M" combinations here instead of the more

descriptive "<Enter>" to help ensure I got it all on the one macro

line.

Page 108