����  �   �    �����  ����  �� ��  ����

�     ����� �   �  �  ���   � � �  �  �

���   � �     ����   ����  �   �   ��

IMPORTANT NOTE

~~~~~~~~~~~~~~

While CW-DEMO starts immediately, exiting can be another matter.

Either 'Esc', 'Q' or 'Alt-X' may be used to exit, however in the

case of unattended use at public demonstrations there is a

configurable exit password which can be set in the CW-DEMO.INI

file.  This is to discourage the public from accidentally or

deliberately exiting and fooling around with the computer when

your back is turned.  So if this is the first time you are

running CW-DEMO and especially if you didn't get it directly

from the author, you might like to see if a previous user has

set an exit password.

Introduction

~~~~~~~~~~~~

CW-DEMO is a Morse program which reads a text file, sounding out

the individual characters and displaying them on the screen in

large 40 column mode.  There is also morse output on a serial

port which may be used to drive external lamps, buzzers, etc.

(Tip: mount a LED on a model ship or a picture of a ship.)

Target Use

~~~~~~~~~~

It is specifically intended to be used at public demonstrations

of Amateur Radio as a lure to passers-by to attract them, and

also to give a group of people an exposure to Morse code.  While

not aimed at use for individual Morse practice, it may perhaps

be of use where trouble is experienced when attempting to read

conventional sized text.

Requirements

~~~~~~~~~~~~

Display: Must be capable of 40 column mode, which seems to

include virtually all types except MDA.

Sound:   Uses normal internal beeper, does not require sound card.

Best results will be obtained with a loud beeper, of

course.  Perhaps an external speaker might replace the

computer's internal speaker for greater volume.

Serial port outputs:

9 pin: pins 4 and 7 are outputs, 5 is earth.

25 pin: pins 4 and 20 are outputs, 7 is earth.

Skills:  The program is designed to be very simple to use.

Probably the program can be started up and just left

running unattended while the hams at the display do

other things like operate rigs or talk to the public.

Files

~~~~~

CW-DEMO.INI Configures colours and the default speed and

frequency, and the title and text seen in lower half

of the screen. You can also specify the password

used to exit from CW-DEMO for added security when

running unattended at public demonstrations.

CW-DEMO.COM The actual program itself.

CW-DEMO.DOC This file.

CW-DEMO.TXT The text file read by CW-DEMO.COM

The CW-DEMO.TXT file may contain whatever you want.  To attract

an audience it seems that the text displayed should attract and

hold attention, so a sample CW-DEMO.TXT file is supplied, derived

from a packet radio bulletin sent by Frank KB7LYM and which

should hold the attention of any audience.

Background

~~~~~~~~~~

Our club demonstrated Amateur Radio at an A&P Show some years

ago, and it was observed that when my computer was running my

CWIT Morse software, a small crowd began to gather.  It struck

me that minor modifications to CWIT would produce a 40 column

mode which would be more readable by a group, as well as being

more readily readable by people idly wandering by.  ZL1HV's "A

Call To Arms" article on page 9 of the June 1997 issue of

Break-In prompted me to complete the documentation and release

CW40 to be available for others to use, and a further public

display in mid-2000 suggested modifications which have resulted

in CW-DEMO being produced.

Comments

~~~~~~~~

Comments are very welcome.  Currently this program is under

development and has been released for general use.  One area

that I would like to optimise is the text in the lower half of

the screen, so feel free to offer suggestions bearing in mind

that the intended viewers of the program are going to be members

of the public.

Changes

~~~~~~~

20 October 2001: Exit password now optional.  Exits on 'Esc',

'Q' or 'Alt-X'.  Faster startup.  CW-DEMO5.ZIP

13 August 2001: Added serial port output at ZL3TPA's request.

CW-DEMO4.ZIP

23 June 2001: Added password exit and definable limits for speed

and frequency, again a ZL3TPA suggestion. CW-DEMO3.ZIP

7 April 2001: At Paul ZL3TPA's suggestion, the program now

re-reads the text file when it gets to the end of it, thus

making it suitable for running for long periods unattended.

Distributed as CW-DEMO2.ZIP.

David Fraser, ZL3AI.

20 October 2001.

Packet mail: ZL3AI@ZL3VML.#80.NZL.OC

E-mail: zl3ai@nzart.org.nz