DBCONV7.EXE   Information and Help   Copyright (c) R.G.Davison 2005-2006

There are NO warranties of any kind for this programme. If something goes

wrong or breaks that is tough, you get to keep the pieces. Use at your own

risk. Although I believe that the numbers shown are correct, I make no claim

that they are. The programme has in-built checking that tests the EXE file.

You CANNOT change the programme name. The minimum system is a VGA monitor,

DOS 4.0 and a 486 PC (needs a maths co-processor). The programme has been

tried with DOS 4.0 to 8.0, W2k and Wxp. In windoze (95-xp), it should be run

from a short cut icon in "Full screen mode". Right click the DBCONV7R.EXE

icon and then select "Properties". Click "Screen" then select "Full screen".

Then click on "Program" and select "Close on exit", then click "OK".

Programme description

This programme does a number of different conversions or calculations where

one of the terms is expressed in decibels, or a unit incorporating a decibel.

In the main menu you can select the type of conversion by moving the bar

with the arrow keys, then pressing <Enter>. Alternatively, press keys 1 to C.

After doing this a window will open below, showing the possible data inputs.

You may change the values of input data by moving the cursor with the arrow

keys, then typing in the new digits. Use the usual keys for editing the line.

The cursor is an underscore for overwrite mode, or a block for insert mode.

Press <Enter> or Up/Down arrow keys to recalculate. All digits on the line

are then stored as the new value although the display is limited to a few

decimal places. Invalid or out of limit entries result only in a beep.

Menu choices 3 to 5, make use of a unit after the voltage or power value.

Units are pV pW for pico (1E-12), nV nW for nano (1E-9), �V �W for micro

(1E-6), mV mW for milli (1E-3), kV kW for kilo (1E3), MV MW for Mega (1E6).

For Volts/Watts the multiplier =1. Letters of the units are case sensitive.

The two columns of numbers on the left have minimum and maximum limits.

Some of the limits are in scientific notation; e.g. 1E-6 or 5E4. The number

after the E tells you how many places to move the decimal point. For 1E-6 the

decimal point moves left 6 places to become 0.000001, and 5E4 becomes 50000.

In some cases the conversions have commas to make the number more readable.

You may add commas to an input number if you wish. Commas are removed before

making the calculation. N.B. a comma is NOT a substitute for a decimal point.

Units and terms used in the programme.

Decibel

A unit expressing a base 10 logarithmic ratio of a reference level and

another voltage,current, power or sound level. 1 dB = 1/10 Bel

It is not an absolute unit such as a Volt or a Watt.

Neper

A unit expressing a natural logarithmic ratio of a reference level and

another voltage,current or power level. 1 Neper = 8.686 decibels.

Return loss

This is a term expressing the ratio of the forward power and the reverse

power on a transmission line or load. The LOWER the number, the worse the

mismatch.  14 dB return loss = 1.5:1 VSWR.

VSWR

Abbreviation of Voltage Standing Wave Ratio. This is the ratio of maximum

and minimum voltages on a transmission line. A value of 1 applies to a line

terminated in its characteristic impedance,  i.e. a matched line. The HIGHER

the number, the worse the mismatch.  A VSWR of 2:1 = 9.54 dB return loss.

Mismatch loss

This is the reduction in power available from a generator due to a load

mismatch. Not to be confused with the additional loss in a transmission line

due to the mismatch. A 6:1 VSWR causes a 3 dB mismatch loss. For a 2:1 VSWR

the mismatch loss is around 0.5 dB. All available power from the generator

is absorbed by the load and transmission line loss, no matter what the load

VSWR is. If a matching device (ATU) is inserted between the generator (or

transmitter) and a mismatched load, the generator will deliver its maximum

power to the load but there will still be an extra loss in any transmission

line due to the mismatch. See the next paragraph.

Extra line loss due to VSWR

A VSWR greater than 1:1 increases the average power on a transmission line

so increasing power lost in the line. The effect is small when the matched

loss is reasonably low. For transmission lines with high loss the extra loss

will be no more than the mismatch loss for the same VSWR. e.g A 2:1 load VSWR

raises a matched loss of 2 dB to 2.315 dB. e.g. For a line losses of more

than 10 dB, the extra loss is 0.51 dB for a 2.0:1 VSWR.

Rho

Rho is the Greek letter used to denote the reflection coefficient. In print

it looks a bit like p in italics. Rho is the square root of the reverse power

divided by the forward power in a transmission line or load. (Rho = � Pr/Pf)

dBm

A power level referenced to 1 milliwatt. e.g 20 dBm is a level 20 dB higher

than 1 milliwatt. So 20 dBm = 100 milliwatts. -20 dBm is 1/100 of a milliwatt

or 10 microwatts. dBW is a power level with one watt as the reference level.

dBm mV

A voltage level referenced to 1 millivolt, usually in a specified impedance

such as 50 or 75 ohms. e.g 20 dB mV is a level 20 dB higher than 1 millivolt.

So 20 dB mV = 10 millivolts. -20 dB mv is  1/10 of a millivolt.

Zo

This term is used for the characteristic impedance of a transmission line

or system, and is expressed in ohms. For a transmission line it is a function

of the square root of the inductance divided by the capacitance, per unit

length, of a transmission line. (Zo = � L/C)   It cannot be measured with a

multimeter!  Co-axial cables are commonly said to be 50 ohm or 75 ohm types.

Noise factor

Is a dimensionless power ratio of the output noise from an amplifier to the

output noise from noise free amplifier of the same gain, when both amplifiers

have an input termination of Zo, at a temperature of 17 �C or 290 Kelvins.

The input termination will generate noise equal to KBT. Where K is Boltzmanns

constant of 1.38 E-23, B is bandwidth in Hz and T is temperature in Kelvins.

At 17 �C, the noise power from any resistance is -174 dBm per Hz bandwidth.

A noise free amplifier amplifies the noise of the input termination, which

may be an antenna, but does not add any noise of its own to the output noise.

The equivalent temperature of an antenna can be taken as the temperature of

what the antenna "looks" at. An antenna used for terrestrial communication

will have an effective noise temperature of 290 Kelvins, while a dish antenna

pointed at a satellite could have an equivalent temperature of under

30 Kelvins depending on spillover and side lobes from its feed horn.

Noise figure

This term is the decibel equivalent of Noise factor. It can be thought of

as the reduction in the signal to noise ratio in dB, of a signal as it passes

through an amplifier with a source temperature of 290 Kelvins or 17 Celsius.

Kelvins

Kelvins are temperature units equal to degrees Celsius - 273.15. Noise

performance of a Low Noise Amplifier (LNA) can be rated in Kelvins instead of

noise figure in dB. You can think of it this way. LNA1 has a 50 Kelvin noise

temperature with an input termination of Zo, at a physical temperature of

zero Kelvins. LNA2 is a noise free amplifier, of the same gain as LNA1, with

an input termination of Zo, at a physical temperature of 50 Kelvins; both

LNAs have the same output noise. Kelvins add, so for a system with an

antenna temperature of 30 K, a LNA with a temperature of 50 K and a second

stage noise contribution of 4 k, the System temperature will be 84 Kelvins.

Kelvins are also divided by gain of an amplifier. If the first stage of a

LNA has a gain of 400 or 26 dB, it will divide the noise temperature of the

second stage by 400. e.g. A second stage noise figure of 8.14 dB or 1600 K

will be reduced to a 4 Kelvin contribution in the System temperature.

Cascaded noise figures

This function calculates the overall noise figure of a two stage device as

described above. Any loss ahead of amplifier 1 should be added to its noise

figure. Likewise, any loss between amplifier 1 and amplifier 2, should be

added to the amplifier 2 noise figure. If the second stage is a diode ring

mixer with a loss, the mixer loss is not added to the mixer noise figure.

The loss of a diode ring mixer is virtually its noise figure.

Path loss

This calculation is for free space, line of sight conditions, with

isotropic antennas. An isotropic antenna is a hypothetical antenna that

radiates uniformly in all directions. Most practical antennas will have

some gain relative to an isotropic antenna. Gain relative to an isotropic

antenna is stated as dBi. A dipole antenna has a gain of 2.1 dBi.

To calculate the dB loss between a transmitter and a receiver, antenna

gains are subtracted from the path loss, and other losses, such a co-axial

cable loss, are added. Minimum distance is two wavelengths but in practice

the distance must be greater than the near field distances of the antennas

for a valid result.  N field = 2 * D^2/lambda  D = largest antenna dimension.

Yagi antenna gain

This is calculated from an algorithm using the boom length. More precisely,

the distance between the reflector and the last director in metres, is the

basis of the calculation. Gain for boom lengths of 0.25 lambda to 25 lambda

can be calculated. The maximum boom length is 100 metres for frequencies

below 75 MHz. The gain figure should be within 0.2 dB of the true peak gain,

assuming that the antenna is properly designed and constructed, and is in

good condition. The number is in close agreement with published graphs.

Parabolic dish antenna gain

This calculation uses the standard formulas that assume an illumination

efficiency of 55%, i.e. -10 dB power at the rim. The gain is a function of

antenna diameter in metres, and the operating frequency. No allowance is made

for errors in the dish surface. Antenna beamwidth is also calculated.

Distribution etc

The programme should be distributed in PKZIP format along with the info file.

DBCONV7R.EXE is freeware for Radio Amateurs only. It is NOT for commercial

use and it is NOT Public Domain. I retain the copyright. It is NOT to be sold

or packaged with other software if a charge is made for the other software.

If you've paid money for this programme, then your supplier has breached my

copyright, and ripped you off! If you have downloaded this programme and are

not a licensed Radio Amateur, you should feel obliged to make a donation of

$15 Aus or 10 Euro to your local Amateur Radio Club or organisation.

RGD 04/05/2006   All mail to VK2ZRG@VK2WI.#SYD.NSW.AUS.OC