KENWOOD TH-215A handheld transceiver out-of-band modification:

1.  Remove two screws from belt clip

2.  Remove 4 screws from back of radio

3.  Slide front assembly off

4.  Locate set of 4 jumpers:

(top front inside)

+--------------------------------------+

|                                      |

|                                      |

|         +--------+                   |

|         |        |                   |

|         |        |                   |

|         +--------+                   |

|                                      |

|                      +--------+      |

|                      |        |      |

J4: intact        |           J4 o---o   |        |      |

J3: cut           |           J3 o) (o   +--------+      |

J2: cut           |           J2 o) (o                   |

J1: cut           |           J1 o) (o                   |

|                                      |

|                                      |

|                                      |

|                      SPKR            |

|                                      |

|                                      |

|             mic                      |

|                                      |

+--------------------------------------+

5.  Reassemble radio.

6.  Reset microprocessor:  Turn radio on while simultaneously pressing

both _F_ key and _ENTER_ key.

--

DISCLAIMER:  I do not own a TH-215A and have not verified the above

information.  Hopefully, the modified frequency range includes 162 MHz

weather-broadcast freqs.  Remember, it is ILLEGAL to transmit outside

the ham band with non-FCC-type-accepted equipment, even if you are

licensed to use such frequencies.

--

Frank     W9MKV

reid@gold.bacs.indiana.edu

THE ULTIMATE DISASTER:  Big tornado sucks-up Dayton Hamvention

fleamarket, survivors try to save the day by using ham radio.  (All

that Hewlett-Packard test equipment falling out of the sky would be

the deadliest threat!)

##########################################################################################