TH25AT.7

If the

radio is positioned on its back, with the top folded over so that the

touch tone pad is also facing down, the test point you want (TP1) is on

the bottom half of the radio, near the center (left to right), and close

to the battery; the tuning pot you want (TC1) is on the bottom, and

closest to the PTT switch.  Under no circumstances change the tuning of

TC51.  This is used to calibrate the output of the radio with the

display the radio is giving; you don't want to mess with it.  Once

again, the Service Manual makes it very clear where these points are, if

you are having trouble with my descriptions.  On with retuning the PLL.

With the radio on, and receiving, monitor the voltage and the waveform

on test point TC1.  Tune the radio DOWNWARDS in frequency until the PLL

unlocks.  Note that the radio will beep when this happens, and the

waveform on TP1 will change.

CONTINUED IN FILE TH25AT.12