TH77.1

Kenwood TH-77 Extended frequency coverage and dual-band repeater

------------------------------

relayed by n2ixl, from bitnet infohams digest,v90n433

Kenwood has made this radio compatible with the new ICOM S-Series radios

from what I can tell.  It has DTSS (Dual Tone Squelch System) which means

that your radio will not break squelch until it hears a particular

sequence of three DTMF tones (ICOM has this also and they call it

Coded Squelch which also use three tones).  The TH77A also has a paging

function that works *exactly* like the ICOM paging.  You program the

radio with a personal 3 digit DTMF code and someone pages you by

transmitting YOUR 3 digit code, followed by a * character and then

their own 3 digit code.  The ICOM will start to ring to let you know

that you have been paged and display the 3 digit code of the person

that has paged you.  The TH77A will beep once and open squelch as

long as the person that paged you does not drop carrier (I prefer

the ICOM method for this part, but there is a work around that I

will mention in a bit).  One thing the TH77A does at this point is

bring up a timer so you know how long its been since you've been

paged.  Maximum time is 99 hours and 99 minutes.

continued in th77.2