TH78

Mods for Kenwood TH-78A HT:

There are three (known to me) mods for the TH-78A HT.

1) Cross-band repeat

2) MARS

3) Beyond MARS

I have tried all three on my TH-78A and they work for me.  That doesn't

necessarily mean they will work for you.  Working with surface-mount

components without the proper equipment is hazardous at best.

All three mods require removal of the back cover and CPU shield.  This is

the procedure:

A) Remove three long screws from back cover.  You don't have to remove the

belt clip.

B) Remove one short (machine) screw from below the antenna connector on the

PTT switch side of the radio.

C) Make sure the battery is ejected and poke your finger up through the

battery compartment and push the back cover away from the radio.  Tilt

the bottom of the back cover away and up to remove it from the

chassis.  CAUTION: There is a ribbon cable connecting the chassis to

the back cover.  Also, there are two very small o-rings around the

LEDs on the top of the chassis that need to be kept track of.

D) Lay the chassis face down and the back cover component side up on something

soft.  The two will still remain connected by the ribbon cable.  Orient

the chassis so the PTT switch is toward you.

E) On the chassis locate a brass shield.  It is the RF shield for the CPU.

Desolder the shield on all four corners.  CAUTION: The shield is double-

back taped to the CPU also.  Remove the shield.

The following mods require removal of one or more chip diodes.  The diodes

are located below the CPU (toward you if you oriented the chassis as

instructed earlier) in a horizontal row.  There are six diodes and we'll

label them D1 through D6 going from RIGHT to LEFT.  Again, RIGHT to LEFT.

The leftmost diode is D6 while the rightmost diode is D1.  The only reason

we are labelling them this way is because that's what Kenwood does.

Mod 1 - Cross-band repeat:

Remove D4.  Reassemble.  Reset the CPU by holding [M] and power on.  To

activate cross-band repeat press [F] for one second and press [0].  Repeat

to deactivate.  The active band becomes the transmit band.  You can tell the

rig is in cross-band mode by blinking decimal points on both bands.

CONTINUED IN TH78.1