TH78A.9

Mod 1 - Cross-band repeat:

Remove D4.  Reassemble.  Reset the CPU by holding [M] and power on.  To

activate cross-band repeat press [F] for one second and press [0].  Repeat

to deactivate.  The active band becomes the transmit band.  You can tell the

rig is in cross-band mode by blinking decimal points on both bands.

Mod 2 - MARS

(Note: Skip this mod if you are going to perform the Beyond Mars mod.)

Remove D6.  Reassemble.  Reset the CPU by holding [M] and power on.  Range

on 2m is Rx 118.000 to 173.995, Tx 142.000 to 151.995 and on 70cm Rx 400.000

to 511.990, Tx 420.000 to 449.990.

Mod 3 - Beyond MARS

(Note: For Beyond Mars to work D6 MUST be left on the board.)

Remove D5.  Reassemble.  Reset the CPU by holding [M] and power on.  This

gives you a 300MHz band and a 800MHz band.  No transmit is possible on these

two new bands.  To activate them press [F] for one second and press [BAND].

The 2m band will switch to 300MHZ and the 70cm band will switch to 800MHz.

300MHz Band Rx range: 300.000 to 399.990

800MHz Band Rx range: 800.000 to 949.990

Note: All ranges depend upon the setting of your step size.  Not much

difference one way or the other, just how close to the edge you can get.

These mods are NOT FCC type-accepted.  They are for experimental use only.

Transmission in the MARS, CAP or any other non-amateur sub-band requires

proper licensing.

(Note: My TH-78A began to operate erratically with my first modification

which included removing D5 AND D6.  I verified that for Beyond Mars D6 must

remain on the board.  Performing the Beyond Mars mod includes the TX/RX

range of the Mars mod.)

-Mike   km6px@kg6xx.#nocal.ca.usa.na