TH78.4

On UHF all public safety frequencies up to about 500 MHz seem to

come in well but sensivity greatly drops from there (we really

can't ask for more). Frequencies can be programed in up to the

920MHz ham band but I have no way of measuring sensivity.  800MHz

works but the signals are very weak, you must be near the

transmitter for reception.  Assuming you are in the town or city

where the transmissions originate it should work.

Transmit is enabled up to and incl. 500MHz but after testing this

on a frequency counter I find that a signal is only generated to

about 490 MHz, even though the trans. LED shows output in the

higher frequencies.

Crossband repeat seems to work fine but the audio is unacceptable

for use, BE AWARE the radio get's HOT!  Prolonged use or use on a

busy frequency would not be recommended.  Also remember this is a

dual band HT please use a proper antenna while in this mode to

avoid a high SWR, we should all know better... Right?

After first booting up the CPU in the mod I found that the message

screen showed "Cloning" so it seems that this radio now has cloning

capabilities. After searching I have found that holding the "0" key

and powering up the radio will display the clone feature, see below

for further explination.This leads me to believe that this HT may

have some more "Hidden" features that I am trying to find, some may

be useful.

Thanks to Gary KC8UD who sent me the following via packet .....

CLONING:

The TH-78 can be cloned without cloning cables or special equipment. It is done

entirely with RF, and, in fact, can be transmitted over the air, and even via

repeaters. This may be extremely useful for those users who do not have

the patience to program their own radios themselves. This application would

also be useful for clubs and user groups. (However, this can take as long as

50 minutes with the ME-1 expansion module. It is recommended that a dummy

load be used to prevent unwanted QRM.)

(1) Both radios must be on the same frequency.

(2) Activate both radios by pressing the "0" key while turning the power on.

The radios will display CLONE.

(3) Now, click the PTT of the "master" radio. The radio will transmit in the

conomy low power mode. This may take about 4 minutes for fifty channels.

hen the data has been transferred, both radios will revert back to the

riginal frequency.

(4) Turn both radios off and then on again. They will now operate normaly

while the slave radio has the same memory contents as the master radio.

** I have not tried the following modification myself, try at your own risk **

*** Rich N2CZF ***