If you remove W1 which is a green wire located on the "Control Unit"

board which resides in the front of the rig, you will get transmit from

142-152 MHz and from 420-450 MHz.

To go one step further, lift (or remove) D4 which is located near W1.

This will let the radio be capable of transmitting anywhere it can hear.

(136-174 & 335-512)  The rig is capable of transmitting, it however is

limited by the VCO's.  Which is to say it will transmit through just

about the entire 2m. range and about a 50 MHz spread on UHF.  The VCO's

are adjustable (transmit & receive).  If you are interested in receiving

(or transmitting) out of band (on UHF especially) I recomend you get a

service manual.  It is well worth the price.

73,

Ray   WA6TKV @ N6IIU