From: Don_Lewine@SDD.CEO.DG.COM

Newsgroups: rec.ham-radio

Subject: Command protocol to TS-440

Message-ID: <1.004502@adam.DG.COM>

Date: 31 Dec 87 18:30:00 GMT

Date-Received: 1 Jan 88 07:37:05 GMT

Sender: daemon@ucbvax.BERKELEY.EDU

Reply-To: <Don_Lewine%SDD.CEO.DG.COM@adam.DG.COM>

Organization: The ARPA Internet

Lines: 176

Command Descriptions for Kenwood TS-440

Auto Information

AIn;                The radio will send the status info automatically when

the operator changes any control covered in the IF;

command.

n = 0 for Auto Info OFF

n = 1 for Auto Info ON

See IF; for response

Display Memory

DMnnnn;             This is a factory diagnostic and is of no practical

use.

nnnn = MPU address in hex

Response is:

DMnnnn-aabbccddeeffgghhiijjkk;

nnnn = MPU address in hex

aa~kk = 16 bytes of data in hex

Down

DN;                 The frequency or memory channel will go down by one

step.

Read Frequency

FA;                 Read VFO A.

FB;                 Read VFO B.

Response is:

Fxggmmmkkkhhh;

x = A or B

gg  = GHz.

mmm = MHz.

kkk = KHz.

hhh =  Hz.

Load VFO register

Fxggmmmkkkhhh;      Loads a frequency into a VFO:

x = A or B (selects VFO)

gg  = GHz. (May be 00 or spaces)

mmm = Mhz.

kkk = KHz.

hhh =  Hz.

Function Select

FNn;                Select VFO or memory

n = 0 for VFO A

n = 1 for VFO B

n = 2 for memory

Identify Model Request

ID;                 The radio will respond with:

ID001; for a TS-940

ID004; for a TS-440

The radio may respond with:

ID002; for a TS-811

ID003; for a TS-711

Read Information Request

IF;                 The radio will respond with:

IFggmmmkkkhhhsnnnzrxyytdfcp;

ggmmmkkkhhh is the frequency (See Fx;)

s   = "+" or "-" value of RIT/XIT

nnn = value of RIT/XIT

z   = "0" (I am not sure what this is)

r (see RT;)

x (see XT;)

yy  = memory channel number

t   = 0 for Receive

= 1 for Transmit

d (see MD;)

f (see FN;)

c (see SC;)

p (see SP;)

Lock Knob

LKn;                Enable or disable manual frequency control:

n = 0 for LOCK OFF (manual control enabled)

n = 1 for LOCK ON (manual control disabled)

Memory Channel Select

MCmmm;              Select a memory channel:

mmm = channel number (000 to max for radio)

Mode Select

MDn;                Select mode:

n = 1 for LSB

n = 2 for USB

n = 3 for CW

n = 4 for FM

n = 5 for AM

n = 6 for FSK

Memory Read

MRnrrr;             Read contents of memory channel:

n = 0 for RX VFO

n = 1 for TX VFO (split channels only)

rrr = channel number (000 to max for radio)

The radio will respond with:

MRnrrrggmmmkkkhhhdxxxxx;

n = 0 for RX VFO

n = 1 for TX VFO

rrr = channel number

ggmmmkkkhhh is freq. (See FA;)

d = mode (See MD;)

xxxxx = not used (seem to always be "0    ")

Memory Write

MWnrrrggmmmkkkhhhdxxxxx;

Write a memory. See MR; for format

RIT/XIT Clear

RC;                 The RIT/XIT register will be set to 0.00 KHz.

RIT/XIT Down

RD;                 Decrement the RIT/XIT register by 0.01

RIT/XIT Up

RU;                 Increment the RIT/XIT register by 0.01

RIT Enable

RTn;                Enable or diable the RIT function

n = 0 for RIT OFF

n = 1 for RIT ON

Receive

RX;                 Enter receive mode.

Scan Enable

SCn;                Turns scan on and off

n = 0 for Scan off

n = 1 for scan on

Transmit

TX;                 Enter transmit mode.

UP

UP;                 The frequency or memory channel will increment one step

XIT Enable/Disable

XTn;                Turns XIT on and off

n = 0 for XIT off

n = 1 for XIT on