APRSdigi for DOS

Frequently Asked Questions

Current Release Version: APRSdigi v11.5 beta dated 1 FEB 2000

Original Author: Wes Johnston, KD4RDB

Contributions by: Ian Wade, G3NRW and Tj Johnston, N4UYQ

When did you start writing APRSdigi?

APRSdigi was started during the fall of 1998.

Why did you do it?

There was a discussion on the APRS SIG about making packets shorter by

compressing them.  The MIC-Encoders were becoming more and more popular, and the need for a digipeater that could route a compressed MIC-E packet into the normal APRS network.

Was it modeled to be a software version of the KPC3+ or something else?

No.  Most TNCs have KISS mode built in.  APRSdigi was designed to work with any KISS compatible TNC.  Two of the main reasons for putting the brains of the digipeater in a PC were cheap software updates, and making the digipeater easier to program.

What happens in an emergency when the power at the digipeater site fails?

Since APRSdigi runs on a PC, is subject to the availability of 120v power to run the PC. However APRSdigi can be run on either a laptop or a desktop PC with a 12v or 24v direct DC power supply. These are often available for around $120. For the do-it-yourselfer MECI www.meci.com sells a 43w $13 PC power supply that will accept from 7.5v to 18v DC and provide all the voltages necessary to run a desktop.

What is the minimum PC that APRSdigi can run on?

APRSdigi was written on a 286/12 with 640k ram and does not need a HDD to run.  As a matter of fact, it only needs a floppy to boot, and never accesses the disk drive after that.

What EXACTLY do the UIFLOOD and UITRACE commands in the INI file do? and

what do they affect? What are the options?

UIFLOOD and UITRACE commands tell the digipeater which callsigns to use for WIDEn-n and TRACEn-n packets.  These are typically WIDE and TRACE, but in other applications or in private networks, other names may be used.

*UIFLOOD CALLSIGN TO USE WITH WIDEn PROTOCOL *MUST* HAVE A NUMBER FOR n

This means that APRSdigi can differentiate between the alias of WIDE-x and a WIDEn-n path.  The WIDE-x will only provide one additional hop for the packet, while the WIDEn-n can provide as many as 7 additional hops.

*THIS WILL SUBSTITUTE THE *PRECEEDING* DIGI'S CALL WITH mycall

*THIS WILL *NOT* CAUSE THE PACKET TO GROW IN LENGTH

UIFLOOD WIDE

This is a compromise between a TRACE packet (which grows in length) and a WIDE packet which gives no information about where a packet came from.  During the course of digipeating a WIDEn type packet, the callsign preceding the WIDEn statement in the packet's path is altered to be the callsign of the digipeater that handled the packet just before the APRSdigi.  In a sense, this is TRACEn-n with only one "place" to show the path a packet has taken.

*UITRACE WILL ACT JUST LIKE WIDEn, EXCEPT THAT IT WILL CAUSE

*THE PACKET LENGTH TO GROW.  THINK OF THIS AS A TRACEn UITRACE TRACE

This statement defines TRACE as the alias that will cause a packet to travel the APRS network growing in length as it moves.  Each digipeater that handles the packet will add it's callsign to the path and pass the packet along.

The web page mentioned adding support for weather… in what way???

Since APRSdigi has the ability to listen to two comports at once, and the

ability to manipulate a HSP adapter, it can listen to serial data from a

weather station and transmit that data periodically.  There is presently no interface for this because the author does not own a weather station.

I understand the CONV option... but what are YYY and CST????

APRSdigi has the ability to convert packets it hears to a compressed packet. This can compress a 85 character NMEA sentence to 32 bytes. This conversion will only happen if the APRSdigi is the first digipeater to hear the packet.  CST is Bob Bruninga's new base 91 compression scheme.  This compression method, unlike MIC-E compression method, can transverse Internet gateways.  To my knowledge, only DOS APRS can decode the CST packets, whereas all versions of APRS can decode the other.  Until more versions of APRS can decode the CST type of compressed packet, this feature is disabled.

On the WIDEn-8 through WIDEn-15, I assume that the MIC-e sends these??? and then

APRSdigi changes it to the path specified in the INI file??

In order for the MIC-E to send a short packet burst, it is desirable to send a packet with only one callsign in the path.  If you use WIDE 8 through 15 in the first callsign in the path, APRS DIGI will substitute a pre-selected path for you.  For example using the TOCALL of WIDE8-8, APRSdigi will select a path to the north.  The operator of the APRSdigi will pre-select the best path in each direction.

How do I use the second serial port to feed another computer?

Connect the second computer, running one of the many APRS clients, to the APRSdigi computer with a null modem cable. Which ever port on the APRSdigi computer you connected to, make sure the APRSDIGI.INI file indicates TNC mode.

How do I use the second serial port with another TNC?

Connect the second KISS mode TNC to the available serial port on the APRSdigi computer, using a standard serial cable. Which ever port on the APRSdigi computer you connected to, make sure the APRSDIGI.INI file indicates KISS mode.

Will APRSdigi run under Windows?

Yes, it will run under windows, but not recommended. When APRSdigi is not the active window, it may skip sending its beacon and may lose some inbound packets. Some of the timed events will also be affected. Do not run APRSdigi under Windows.

Will APRSdigi run under Linux?

Rumor has it, that APRSdigi will run quite well under DOSemu, and possibly VMware. This is still being studied, and your comments on its success would be welcome.