APRSdigi for DOS

Current Release Version: APRSdigi v11.5 beta dated 1 FEB 2000

Original Author: Wes Johnston, KD4RDB

Contributions by: Ian Wade, G3NRW and Tj Johnston, N4UYQ

In its simplest implementation, APRSdigi will run on a small DOS based computer with 640 kilobytes of random access memory, and one serial port. When connected to a transceiver, APRSdigi, will receive inbound packets, process those packets, and retransmit those packets if needed.

Minimum Requirements

PC compatible 286 or higher

Ms-DOS 3.3 or higher

640kB of RAM or higher

1 serial port

TNC capable of KISS operation

Transceiver

Installation

The most recent version of APRSdigi is available for download at http://www.qsl.net/n4uyq/aprsdigi.htm

Create a directory for APRSdigi (i.e.: C:\APRSDIGI)

Unzip the DIGIxxxx.zip into the directory you created.

The following files will be extracted:

APRSDIGI.INI - Example initialization file

DIGI.DOC     - The main documentation file

DIGI-FAQ.DOC - Frequently asked questions

DIGIxxxx.EXE - The executable program

DIGI.TXT     - Text version of DIGI.DOC

DIGI-FAQ.TXT - Text version of DIGI-FAQ.DOC

FUTURE.TXT   - Future APRSdigi features

README.TXT   - Last minute changes and notes

REVISION.TXT - Revision history file

Check the README.TXT and REVISION.TXT files for any changes that may affect installation or operation of the current version of APRSdigi.

Modifying the APRSDIGI.INI file

Each line of the APRSDIGI.INI file contains an instruction or a remark. Remarks are preceded by an asterisks ‘*’ and contain information about the command that follows. This file contains basic descriptions of each command, but I will attempt to expound on them there.

Commands

MYCALL		Your digipeater’s callsign. This is used in WIDEn-N and

TRACEn-N substitution and status packets.

MYCALL N4UYQ-10

MYALIAS	Standard alias used primarily by mobile APRS stations.

MYALIAS RELAY

MYALIAS1-4	Standard alias’s used by most digipeaters.

MYALIAS-1 WIDE

MYALIAS-2 TRACE

Some localities use a tactical callsign here also.

MYALIAS-3 HOPEWELL

If you do not use one or more of these aliases, do not

include them in the file.

UIFLOOD	UIFLOOD callsign to use with the WIDEn protocol

*MUST* have a number for n. This will substitute the preceding DIGI’s callsign with MYCALL and will not cause the path to grow in length.

UIFLOOD WIDE

UITRACE	This will act just like WIDEn, except that the path will grow in

length. Think of this as a TRACEn.

WIDEn-8 +	WIDEn-8 through WIDEn-15 are used by MIC-E systems to specify a

default path. In order for the MIC-E to send a short packet burst, it is desirable to send a packet with only one callsign in the path.  If you use WIDE 8 through 15 in the first callsign in the path, APRSdigi will substitute a pre-selected path for you.  For example using the path of WIDE8-8, APRSdigi will select a path to the north.  The operator of the APRSdigi will pre-select the best path in each direction.

WIDEn-8 is used for NORTH

WIDEn-9 is used for SOUTH

WIDEn-10 is used for EAST

WIDEn-11 is used for WEST

WIDEN-12 is used for NORTH + WIDE

WIDEN-13 is used for SOUTH + WIDE

WIDEN-14 is used for EAST + WIDE

WIDEN-15 is used for WEST + WIDE

UINORTH W4RAT-10,WD4HDL-1

UISOUTH KD4ZB-3,WIDE

UIEAST KF4HJW-10,WIDE

UIWEST WA4BWB-3,WB4QOJ-5

UINORTHW W4RAT-10,WD4HDL-1,WIDE

UISOUTHW KD4ZB-3,WIDE,WIDE

UIEASTW KF4HJW-10,WIDE,WIDE

UIWESTW WA4BWB-3,WB4QOJ-5,WIDE