PTC-II and PTC-IIe Help File

Quick Start

--------------

1. Rename the default MACRO.TNC in the Logger folder to MACRO1.TNC.

2. Move the new MACRO.TNC to the Logger folder.

3. Open Logger.

4. Click the Data button at the top of the screen to load the MACRO.TNC file.

5. Click on Config at the top of the Data window to open the Data Port Config window.

6. Set the options. Below are my settings.

Baud Rate = 9600 (or what ever you have set in the Plusterm.ini file)

Com Port = 1

Flow Control = None

Parity = None

Data Bits = 8

Echo = Off

Stop Bits = 1 bit

7. Click the RED check mark to save the settings.

8. Check the OPEN/CLOSE button. If it says OPEN - then click to open the port.

9. Click the button on your right at the bottom labeled 'PTC'.

Ignore the error messages.

10. Click the magenta colored macro button marked 'PTC Init'.

This will set internally the following:

ESCchr to decimal 1 (Control-A).

BKchr to decimal 2 (Control-B). The break-in character.

Term to 1 (Terminal mode with delayed echo).

The PTC should echo the changes and all macros should now function.

11. Click the PSK31 macro button or the mode of your choice.

12. Edit the macros as needed. To do this Right-Click on a macro and answer Yes.

Additional Information

----------------------------

The TRX macro on the right is for the PTC-II and rig control for my TenTec OMNI-VI. These control codes should also work for an Icom. In the past I used COM4: for this. Then I bought a new PC and it named the two serial ports A and B. I did not change A (COM1:) and use it for the PTC. Logger did not like me using B as COM4: for some reason as I used this with DOS and it worked fine. I used the device manager and changed and configured it for COM2: Logger was happy with this. You will also need to edit all TRX macros to conform to your radio control codes.

Any macro button that says 'Poll Off' means that internally the PTC-II will not poll the radio each time Logger does. This removes the echo from the PTC back to Logger.

The Pactor Macro has Mailbox on/off. You will need to change the mailbox message in this macro for your station.

RCKRtty – Alpha - NcWinPtc users

------------------

All the above set the PTC to the factory defaults of ESCchr(27) and BKchr(25). You can use the above method to change them to 1 and 2.

XPwin users

------------------

When XPwin runs it sets the ESCchr to 3. You can write a shutdown file for XPwin. In this file you can add the following commands:

ESCchr 1

BKchr 2

Or just exit in the normal way and then open Logger and use the above Quick Start.

Note: If you DO NOT turn the PTC off and plan on using XPwin or RCKRtty, return the PTC to the Pactor mode, then exit Logger.

New PTC-II or PTC-IIe users

----------------------------------

After you install Plusterm you will need to run it or PTpro out of a window. The first time you open a DOS program while in Windows the system will make a .pif file (Program Information File), but not correctly. So you edit/write this .pif file the correct way.

Use Windows Explorer and navagate to the plusterm folder and right-click on Plusterm.exe. Choose properties.

From the PROGRAM tab:

Type in a descriptive name like: SCS-PTC

Cmd Line = c:\plusterm\plusterm.exe

Working = c:\plusterm

Batch file = leave blank

Run = Maximize (use down arrow and scroll)

Check mark 'close on exit'

From the SCREEN tab:

Usage:

Check mark Window

Initial size = default

Check mark all the remaining boxes

Press OK and we are done.

Note: You can also do this with PTpro which I prefer.

If still in Windows Explorer drag on this new Plusterm icon (it will have DOS icon by default) to your desktop for a temporary shortcut. Then exit Explorer. Double-click on the icon to run Plusterm or PTpro. You can now set your personal preferences. All this is in the Plusterm manual. You should set your PSKAmp and FSKAmp values before operating PSK and Pactor modes. Instructions are in the PTC manual.

To check on a default press the Esc key. The command window will open. Type in ‘ESCchr’  <Enter> or ‘BKchr’ <Enter>. The value will be echoed to the screen. See Quick Start above.

To change them type in your new value:

ESCchr 1 <Enter>

BKchr 2 <Enter>

If satisfied then exit the program (ALT-X).

After you change the values and exit Plusterm you MUST change the values in the Plusterm.ini file. On any future openings using Plusterm no changes will be necessary and Logger will work like a champ. Plus you will have 27 instant macros for each mode.

Remember this rule: Change the ESCchr FIRST internally from within Plusterm or PTpro THEN the ESCchr value in the Plusterm.ini file.

Plusterm or Ptpro WILL BE NEEDED for any future firmware updates and to backup your mailbox. Or, this is important, to be able to restore your complete mailbox backup after an upgrade. A upgrade will erase everything in the unit and all defaults will be restored. None of this is available from within Logger.

Any macro button that says 'Poll Off' means that internally the PTC-II will not poll the radio each time Logger does. This removes the echo from the PTC back to Logger.

The Pactor Macro has Mailbox on/off. You will need to change the mailbox message in this macro for your station.

Note: This all started when I had a PSK qso with Hans, DJ1IJ. He put me on to using Logger, the data terminal, and the macros. And Hans received the idea from Miguel, EA8AU, who was the brains at the start. Then I did a lot of editing and added a few things. You can do the same. GL.

de Joey - jfmasur@iag.net - PSK on 28.120 and 10.142 +/- Have Fun!