copyright © 1995 Bob Rushby

A TNC Control Program for the MFJ 1278 Family of Multimode Controllers

Version 1.4B

SkyRider Systems

RR 21

Cambridge, Ontario

Canada

N3C 2V3

Contents

1. Overview

1.1 General

1.2 Registration

2. Installing SkyRider

3. Running SkyRider

3.0 Software Control of the 1278:

What to do if it doesn't cooperate!

3.1 Running From Program Manager

3.2 Running From File Manager

3.3 The SkyRider Main Window

3.3.1 General Description

3.3.2 Upper Tool Bar Description

3.3.2.1 Data Capture Check Boxes

3.3.3 Lower Tool Bar Description

3.3.3.1 Screen Functions

3.3.3.2 Time Display

3.3.3.3 Call Sign Transmission

3.3.3.4 Send / Receive Control

3.3.4 The Basic Information Form

4. Selecting Modes of Operation

4.1 General

4.2 "Mode Button" method

4.3 "Menu" Method

4.4 Notes

5. Operating

5.1 Pactor

5.1.1 Pactor Options

5.1.2 Pactor QSO's

5.1.2.1 Pactor FEC Mode Operation

5.1.2.2 Pactor ARQ Mode Operation

5.2 Amtor

5.2.1 Amtor Options

5.2.2 Amtor QSO's

5.2.2.1 Amtor FEC Mode Operation

5.2.2.2 Amtor ARQ Mode Operation

5.3 VHF and HF Packet

5.3.1 VHF and HF Packet Choices

5.3.2 VHF and HF Packet Operation

5.3.2.1 General

5.3.2.2 Establishing and Breaking Connections

5.3.2.3 BBS and DX Packet Cluster Menus

5.3.3 VHF Packet Defaults

5.3.4 HF Packet Defaults

5.4 CW

5.4.1 CW Options

5.4.2 CW QSO's

5.5 RTTY / ASCII

5.5.1 RTTY / ASCII Options

5.5.2 RTTY QSO's

6. Calling CQ in CW, RTTY / ASCII, Amtor and Pactor

6.1 Editing Messages and Calling CQ Once

6.2 Calling CQ Repeatedly

7. Time Saving Tips

7.1 Sending an Identification Message

7.2 Sending the Other Station's Callsign

7.3 Sending Your Callsign

7.4 Using Text ("Brag") Files

8. Printing

9. File Capture

10. Acknowledgments

1.  Overview

1.1 General

SkyRider is a software tool that simplifies the use of your 1278 multimode controller. Instead of requiring you to memorize a large number of commands, all you need to do is "point and click" to easy-to-use icons and buttons on the screen. SkyRider does all the rest for you. If you need to enter more information, SkyRider presents "forms" for you to enter it easily. SkyRider lets you control your 1278 in the way that's easiest for you: "point and click", or keystrokes. There's no need to use a mouse if you don't want to, but you can take full advantage of the Windows interface if you want.

SkyRider supports the following modes on your 1278:

Pactor  (FEC and ARQ modes)

Amtor (FEC and ARQ modes)

Packet (HF and VHF)

RTTY and ASCII

CW

Navtex

SkyRider does not currently support the following features of the 1278:

SSTV

FAX

Packet mailbox

(Packet mailbox mode can be used by directly entering 1278 commands if you wish)

1.2 Registration

SkyRider is not free software. If, after trying it for a

short while, you decide to continue using it, then you should

register your copy. The benefits of registration are:

-you will receive a printed manual

-you will receive the latest version on 3-1/2 inch floppy

-you will get a free copy of SkyRider's 'World Time' program

-you will be entitled to a free future update

-the author (me!) will be able to make continuous improvements

-you will get the deep, deep, gratitude of the author!

-no more annoying registration messages at the bottom of the

screen!

We also encourage you to make copies of SkyRider for other hams

to try. You must provide them with the original 'ZIP' file when

you do this so they will be able to register.

To register your copy of SkyRider, please fill out and send the

form at the bottom of this text file to:

Bob Rushby, VE3GLA

SkyRider Systems

RR 21

Cambridge Ontario

CANADA  N3C 2V3

We'll send you back your registration code. This code will disable the

"Please Register..."

reminder at the bottom of the screen.   We'll give you

instructions on how to easily enter the code into your computer. You can

use as many copies of SkyRider in your own station as you want, as long

as you only use your own call sign.

If you change call signs, let us know. We'll give you a new

registration  code, free of charge.

Registration Form

Name:__________________   Call Sign:_______________ (enter'SWL'if not a

ham)

Address: _______________________________________  (apartment)

____________________________________________ (street)

___________________________________________   (city)

__________________________________________   (Province or State)

__________________________________________   (Country)

_________________________________________    (Postal Code / Zip Code)

Commercial E-mail address (Optional): ________________________________

Enclosed is a money order / check for  (check appropriate box):

__  $25.00  U.S. Currency per copy

__  $34.50  Canadian Currency per copy (Ontario residents,

includes PST and GST)

__  $32.10  Canadian Currency per copy (Non-Ontario residents,

includes GST)

2. Installing SkyRider

To install SkyRider, simply do the following:

1. Put the SkyRider diskette in a floppy drive.

2. Make sure either the Program Manager or File Manager window is visible....

...select "File"  from the menu bar at the top of the window

...select "Run" from the File menu

...enter:   a:setup      (or b:setup   if you put the diskette in drive b:)

...press the "Enter" key

3. The "Setup" program will run. Simply follow the instructions on the screen. You can quit at any time while Setup is running if you wish.

(Note that you must run Setup from Windows. You can't run it from DOS)

3. Running SkyRider

You can run SkyRider from Program Manager (the most common way), or from File Manager.

3.0  Software Control of the 1278: What to do if it doesn't cooperate!

The 1278 uses "open loop" control. That means that when the 1278 receives a command (from SkyRider, or from you), SkyRider has no way of knowing if the 1278 actually responded correctly. It also means that SkyRider can't tell if you sent a 1278 control command directly from the keyboard (you could have gone so far as changing modes!).

If you find that SkyRider does not respond to your commands, it may have gotten "out of sync" with the 1278. This does not happen often, but it is possible. If it does happen, you should:

-enter   "CONTROL-C"   from the keyboard

-enter the "ENTER" key

-select the desired mode from the Mode menu in the menu bar

at the top of the window

This normally restores control to SkyRider.

3.1 Running From Program Manager

Double-click the SkyRider Icon from the SkyRider program group.

(Note that you can move the SkyRider Icon to another group if you wish. Consult your Windows manual for more information.)

3.2 Running From File Manager

Select the directory containing SkyRider (this is possibly C:\SKYRIDER, but you may have specified a different directory when you installed SkyRider). Double-click "SKYRIDER.EXE".

3.3 The SkyRider Main Window

3.3.1 General Description

When you run SkyRider, you will see the "main window". Because you are using Windows, you will be able to switch to other Windows programs, re-size the SkyRider window, shrink the SkyRider window to an icon, and do anything else you can do in Windows. The SkyRider window has several important parts, specifically:

the "Data From TNC" subwindow

This window show the data received from you 1278. That data includes:

- information received "off the air"

- command responses from the 1278 (i.e. what it sends back in response to commands sent to it by SkyRider)

The Data From TNC window stores the last 300 lines (approximately) of data received. You can scroll back by clicking on the scroll bar at the right side of the window. When new data is received, the Data From TNC window scrolls back to the last line. You can prevent this by clicking on the "Pause Screen" button on the lower left of the SkyRider window (see "Lower Tool Bar"). Data from the 1278 will not cause scrolling when the Pause Screen button is checked.

the "Keyboard Input" subwindow.

This window echoes your keystrokes, and keeps a record of what you entered. It also echoes any data sent when calling CQ automatically, or when text ("brag") files are sent.

the "Upper Tool Bar"

The Upper Tool Bar is located just above the Data From TNC window. It is used primarily to select operating modes, establish and break links (Pactor, Packet, Amtor), manage text "brag" files, and for data (printing and file) capture.

the "Lower Tool Bar"

The Lower Tool Bar is located just below the Keyboard Input window. It has a variety of functions: control of the screen, time display, entry and sending of callsigns, as well as functions that depend on which mode you are in.

3.3.2 Upper Tool Bar Description

The functions of the Upper Tool Bar are described below:

3.3.2.1 "Data Capture" Check Boxes:

These check boxes control capturing received data to the printer and/or the disk. Clicking on the appropriate box causes an 'x' to be displayed. That means the function is enabled. Clicking again on the box causes the function to be disabled, and the 'x' will disappear.

3.3.2.2 Operating Mode Buttons:

The operating mode buttons are used to select the communication mode, and to enter the "setup" mode.

The mode you are currently using is shown in two ways:

-by the text in the main Title Bar at the top of the SkyRider window, and,

-by the "mode" buttons in the tool bar at the top of the SkyRider window. The caption for the current mode is shown in light grey.

You can easily change from one mode to another by pressing the mode buttons or from menus.

Selecting modes: Mode Button method

Press the button for the desired new mode from the tool bar at the top of the SkyRider window.

Selecting modes: Menu method

Either:

-press:    ALT-M

press the letter for you desired mode, as shown in the mode menu

(e.g. "R" for RTTY, "V" for vhf packet, etc.).

Or:

-click the "Mode" menu item from the menu bar at the top of the window

-click the desired mode

3.3.2.3 Connect / Disconnect Buttons

These are only displayed in Pactor, Packet and Amtor modes.

When you are not connected (linked) to another station, the "Connect" button is displayed. Click on this button to try to connect to another station. A menu will appear to prompt you for more information.

When you a connected, or attempting to connect, to another station, the "Disconnect" button will replace the Connect button on the toolbar. Click on the Disconnect button to disconnect ("drop") the connection or to abandon an attempted connection.

3.3.2.4 Text "Brag file" Buttons:

The Text buttons are used to enter and send files of text that you create.

3.3.3 Lower Tool Bar Description

The functions of the Lower Tool Bar are described below.

3.3.3.1 Screen functions

"Layout" button

Click on this button to re-arrange the Keyboard Entry and Data from TNC windows. Each click will change to a different format: either overlapped, vertical tiled, or horizontal tiled (the default, startup, format). Click until you find the format that you prefer.

"Clear" button

Click on this button to clear the screen. SkyRider will ask you to confirm this just to make sure you don't clear the screen by accident.

"Color" button

You can choose the colors of the Keyboard Input and Data from TNC subwindows windows by clicking the "color" button (alternately, you can use the "Screen" menu, and click on "color".  SkyRider will show the following form:

Simply click your color choice for each window. Select OK to enable your choices, or Cancel if you do not wish to use your choices.

"Pause Screen" check box

Click on this box to  temporarily stop new data from being displayed in the Data From TNC window. An 'x' will appear in the box when the screen is paused. Click again to display all characters received while the screen was paused and all new characters (the 'x' will disappear).Use this function when you want to scroll through the Data From TNC window and don't want the display to bounce around with incoming data.

3.3.3.2 Time Display

Local and UTC (GMT) times and dates are displayed. Sometimes data received from another station can change the color of the display. This can happen in Pactor or Packet modes when the other station sends "ANSI graphics" data. Click on this button to revert to the default screen colors for any later data that is received.

3.3.3.3 Call Sign Transmission

Call Sign Buttons

The "de (callsign)" button will send "de " followed by your callsign when clicked.

The "Enter Call" button  will prompt you to enter a call in the button immediately to the right. You don't normally need to do this  in Pactor or Packet modes, since SkyRider can automatically determine the call of the station you are connecting or connected to.

The  callsign button will send the callsign that is displayed on it. If no callsign is displayed, nothing will be sent. If necessary, use the "Enter Call" button to enter and display a call sign.

3.3.3.4 Send / Receive Control

Transmit, Receive, "Over"  buttons

In some modes, a buttons will appear to alternated between transmitting and receiving. In Pactor, an "Over" button will appear which allows you to switch from sending to receiving. It will also allow you to "seize" the link from the other station if you wish.

3.3.3 Menu Bar Description

The menu bar is at the top of the SkyRider window. It allows you to perform almost all SkyRider functions with keystrokes instead of mouse clicks. Experienced users may find this a faster way to interact with SkyRider. To select a Menu bar item, either:

- click on it with the mouse,

or:

- hit the "ALT" key

- enter the underlined letter of the desired menu. The desired menu will be displayed.

You can choose items from the menu by clicking on them or by using the ALT key / underlined character method described above.

3.3.4 Basic Information Form

SkyRider needs some basic information for all modes of operation. Make sure you fill out the Basic Information form before using SkyRider. You can change the basic information any time you want later. You select the Basic Information Form by selecting the "Setup" mode (either by clicking the "Setup" button or by selecting it from the Mode menu. This will cause the Basic Information form to be displayed. The Basic Information form is shown below. Each entry is described.

Call Sign:

This field is mandatory.

Make sure you include your portable designator, if applicable (e.g.    VE3GLA/4).

Do NOT include a packet "SSID" (e.g. don't use VE3GLA-15). That can be added with the packet forms.

Name, City, State/Province/Region, Country, Grid Locator, latitude and longitude.

These are optional fields. You don't have to enter them.

Preferred time:

SkyRider displays the time in both Local and UTC (GMT) modes. Click "Local" if you want local time to be shown in bold characters. Click" UTC" if you want UTC to be bold.   In order to display UTC time correctly, SkyRider needs to know how many hours your local time differs from UTC. Enter the data here. For example, Eastern Standard Time is 5 hours after UTC.

Preferred Startup Mode:

You can specify which mode you will automatically enter after starting SkyRider. Simply click on the mode that you prefer. After SkyRider has started, you can easily change to other modes.   Note that the FAX and  SSTV modes are not supported in this version of SkyRider.

Radio Ports:

Your HF and VHF radios should each be connected to a different port at the rear of your 1278. Click on the port number for either your HF or VHF radio ( notice that the other radio will automatically be set for the other port). If you only have one radio, just click on the port number you are using for that radio.

Computer Ports

Your 1278 is connected to your computer via a serial cable. You must specify what serial ("COM") port that you are using in your computer. COM1 and COM2 are used most often.

PC to TNC Baud Rate

Your 1278 must communicate at the correct speed with your PC. Select the baud rate that

your 1278 is set for. You will have to click “OK”, exit SkyRider, then restart SKyRider to

ensure reliable communications between SKyRider and your 1278.

Use/Cancel/Help buttons

Click on Use if you wish the form settings to be used.

Click on Cancel if you want to use the previous settings to be used (i.e. cancel the changes you have made on the form).

4.0 Selecting Modes of Operation

4.1 General

The mode you are currently using is shown in two ways:

-by the text in the main Title Bar at the top of the SkyRider window,

and,

-by the "mode" buttons in the tool bar at the top of the SkyRider window.

The caption for the current mode is shown in light grey. You can easily change from one mode to another by pressing the mode buttons or from menus.

4.2 Selecting modes: Mode Button method

Press the button for the desired new mode from the tool bar at the top of the SkyRider window.

4.3 Selecting modes: Menu method

Either:

-press:    ALT-M

-press the letter for you desired mode, as shown in the mode menu

(e.g. "R" for RTTY, "V" for vhf packet, etc.).

Or:

-click the "Mode" menu item from the menu bar at the top of the window

-click the desired mode

4.4 NOTE:

If you are "connected", or "linked" to another station, or otherwise in QSO, then the QSO will be automatically terminated when you select a new mode. It would be very impolite to do this, without ending the QSO normally first. Be sure to end your QSO before changing modes!

Connect / Disconnect Buttons

These are only displayed in Pactor, Packet and Amtor modes. When you are not connected (linked) to another station, the "Connect" button is displayed. Click on this button to try to connect to another station. A menu will appear to prompt you for more information. When you a connected, or attempting to connect, to another station, the "Disconnect" button will replace the Connect button on the toolbar. Click on the Disconnect button to disconnect ("drop") the connection or to abandon an attempted connection.

Text "Brag file" Buttons

The Text buttons are used to enter and send files of text that you create.

5.0 Operating

5.1 Pactor Operation

5.1.1 Pactor Options

Enter data in the following form to control Pactor operation in SkyRider.

This form may be accessed while in Pactor mode either by:

-clicking the Options button at the top of the SkyRider window, or,

-selecting Options from the menu at the top of the SkyRider window.

FEC speed

-100 baud is slower, but may have fewer errors.

-200 baud is faster and is most useful under good signal conditions.

Reliability

-choose "normal" under average to good conditions

-choose "reliable" under poorer conditions. Transmission  speed will be slower,

but the other station will see fewer errors.

-choose "very reliable" under very poor conditions. Transmission speed

will be much slower, but there is a better chance of being received with fewer errors.

Link Speed

Click on Automatic 100/200 normally.

Click on 100 only when conditions are poor. This will reduce time wasted futilely switching to 200 baud.

Memory ARQ

Memory ARQ is used by Pactor to combat errors induced by QRM and QRN by combining error packets to see if a valid one can be created. The parameters in this part of the form control how this is accomplished.

"Go to 200 baud.."

This number determines how many error-free 100 baud packets need to be received before going to 200 baud. If too small a number is entered, you may waste time going to 200 baud under poor conditions that may not support 200 baud. I suggest a number of 6 be used initially.

"Go to 100 baud.."

This number determines how many erroneous 200 baud packets need to be received before going to 100 baud. If too small a number is entered, you may waste time going to 100 baud when occasional brief QRM or QRN occurs. If too large a number is used, you may spend too long a 200 baud when conditions may not support 200 baud. I suggest a number of 6 be used initially here, also.

"Use up to..."

Pactor can combine a series of error packets to see if a good packet can be constructed. This can be very effective in getting data through in poor conditions. The number you enter here determines the maximum number of sequential error packets are used in this way. Too low a number will mean you don't get full benefit of this feature. Too high a number can also be bad under certain conditions. A starting value of 10 is suggested.

Connects

Select Normal in almost all operating situations.  Select Long Path only when communicating with very distant (over 10,000 km) stations, and only when needed.

Use / Cancel / Help  Click on Use to enable the changes you have made. Click on Cancel to ignore the change you have made.

5.1.2 Pactor QSO's

There are two different Pactor methods for QSO's. They are defined below, and are discussed in more detail in separate topics:

FEC mode

FEC stands for Forward Error Correcting mode. Despite it's name, it is more prone to errors than ARQ mode. FEC is useful for short contacts, and for calling CQ in Pactor mode.

Pactor FEC mode has different degrees of error correction available. With a high degree of correction, transmissions are slower, but the transmissions will have fewer errors. You can specify the amount of correction you want by clicking on the "Options"  button when you are in Pactor mode.

ARQ mode:

ARQ stands for Automatic Retry reQuest mode. In this mode, the MFJ1278 checks each group of characters received, and if an error is detected, the MFJ1278 automatically asks the characters to be re-sent to you until they are received correctly. Only then are the characters displayed on your screen. This mode will detect virtually 100% of errors.

Most Pactor QSO's use the ARQ mode.

5.1.2.1 Pactor FEC Mode Operation

You can listen to, and operate in both ARQ or FEC while in Pactor mode (this is different from the 1278's Amtor mode, which require a separate 'Listen' mode for copying ARQ transmissions).

The actual speed of transmission is set in the Pactor Options form.

To transmit in FEC mode, either click on the "send" button  at the lower right of the screen, or type  CONTROL-T. Your  transceiver will transmit, and the "send" button will change into a "receive" button .  Everything that you type will be transmitted. Characters will be echoed on the Data from TNC screen at the same speed as they are actually transmitted on the air.

When you are done transmitting, either click on the "receive" button  or type CONTROL-R. The 1278 will finish sending any data that has yet to be sent, then it will switch to receive, and the" receive" button will change to "transmit".

5.1.2.2 Pactor ARQ Mode Operation

You can listen to, and operate in both ARQ or FEC while in Pactor mode (this is different from the 1278's Amtor mode, which require a separate 'Listen' mode for copying ARQ transmissions).

Another station can call you, and the 1278 will automatically establish the link ('connection'). SkyRider will display the other station's call in a 'button' at the bottom of the screen.

Alternatively, you can call another station to try to establish a link ('connection').To do this, simply click on the connect button  near the top of the screen (or select 'Connect' from the menu bar).  A form will appear, and you merely enter the call sign of the station that you want to connect to. The 1278 will try to establish the connection. The message "Connection Attempted" will be shown at the bottom of the window. If successful, the message "Connected" will be shown at the bottom of the window. If you are not successful, simply click on the 'Disconnect'   button at the top of the screen (it replaced the 'Connect' button).

Only one station can transmit at a time. You can 'type ahead' while the other station is sending to you, and what you have entered will be transmitted when it is your turn to send. After you have finished your transmission, you turn it over to the other station by doing either:

-clicking on the "over" button at the lower right of the window,

or

-entering  "CONTROL-R"  from the keyboard.

When you have finished your QSO, either station can "break", or "drop" the link. If you wish to break the link, simply click on the "Disconnect" button at the top of the window, or select the "Disconnect" menu item.

5.2 Amtor Operation

5.2.1 Amtor Options

Enter data in the following form to control Amtor operation in SkyRider.

This form may be accessed while in Amtor FEC or Listen mode either by:

-clicking the Options button  at the top of the SkyRider window,

or,

-selecting Options from the menu at the top of the SkyRider window.

The form allows you to set the following features:

Tones

Usually you will use Normal tones. Occasionally the station you wish to contact will use Inverted tones, and you will need to select "Inverted".

SelCal

Normally you will use the standard SelCal that SkyRider automatically computes for you from your callsign.

Occasionally you may want to use a different SelCal. If so, enter a 4 letter (no numbers or other characters) SelCal in the Optional box, and click on the button beside it.

TX Delay

TX Delay determines how long the 1278 waits before starting to transmit. The delay primarily depends on how fast your rig changes from receive to transmit.  A value of 30 is usually a good starting point. Experiment with other values if you are having difficulty with Amtor QSO's.

Answer-back string

You can leave this blank.   However, and Answer-back string is often useful when contacting APLink or similar bulletin board stations in Amtor. Some of those stations send a special command to your 1278 (you are not normally aware of this occurring) which causes your 1278 to automatically transmit the  Answer-back string. The proper Answerback string can simplify logging on to the bulletin board. For example, a string such as:    QRA. VE3GLA would eliminate the need for VE3GLA to go through the callsign dialog on most APLink bbs's.

Use / Cancel / Help

Click on Use to enable the changes you have made.

Click on Cancel to ignore the change you have made.

5.2.2 Amtor QSO's

There are two different AMTOR methods for QSO's. They are defined below, and are discussed in more detail in separate topics:

FEC mode

FEC stands for Forward Error Correcting mode. Despite it's name, it is more prone to errors than ARQ mode. FEC is useful for short contacts, and for calling CQ in AMTOR mode.

Click on the green "FEC mode" label for more information on how to establish a FEC QSO.

You cannot listen to Amtor ARQ transmissions while in FEC mode in the 1278.

ARQ mode

ARQ stands for Automatic Retry reQuest mode. In this mode, the MFJ1278 checks each group of characters received, and if an error is detected, the MFJ1278 automatically asks the characters to be re-sent to you until they are received correctly. Only then are the characters displayed on your screen. This mode will detect most but not 100% of errors.

Most AMTOR QSO's use the ARQ mode.

The 1278 has a "listen" mode that allows you to monitor other station's ARQ QSO's. While you are monitoring other stations QSO's, you will often  see some errors. Error correction only works in ARQ mode when YOU are engaged (linked or "connected") to another station.

5.2.2.1 Amtor FEC Mode Operation

Make sure you are in Amtor FEC mode. If you are not, then either:

-click the Amtor FEC button on the toolbar

or

-select the Mode menu

-select Amtor FEC from the mode menu

To send, either:

-click the "send" button at the lower right of the screen. It will change to

"send"

or

-enter  "Control-t" from the keyboard

The green "PTT" light on the 1278 will turn on, and your transceiver should start transmitting.

Simply type the whatever you wish to send. The white screen will echo the characters as they are sent, perhaps at a different speed than you key them in.

To receive, either:

-click the "receive" button at the lower right of the screen. It will change to "send"

or

-enter "Control-r" from the keyboard.

The green "PTT" light on the 1278 will turn off, and your transceiver should stop transmitting AFTER all data has been sent (if you are a fast typist, this may take a moment or two to occur).

5.2.2.2 Amtor ARQ Mode Operation

Make sure you are in either Amtor Listen mode or Amtor FEC mode. If you are not, either select one of the modes from the toolbar or from the Mode menu.

To "link" or "connect" to another Amtor station, either:

-click the "Connect" button   on the toolbar

or

-select "Connect" from the menu bar.

The "Connect" button will change to a "Disconnect" button, and the "Connect" menu item will be disabled (turns gray).

The following form will be displayed:

Enter the call sign of the station you wish to link to. If the station uses a standard SelCal (almost all stations do), SkyRider will compute the SelCal for you.  Simply press Enter, or click  "OK".

If the station uses a different SelCal, type it in the Other SelCal box and click the Other SelCal button. Then click "OK".

The 1278 will then attempt to establish the link. Your transmitter should begin sending and receiving rapidly to see if the other station responds. If it succeeds, you may simply begin typing. If you cannot link to the other station, click the "Disconnect"   button on the toolbar or select the "Disconnect" menu item.

When you wish to receive, simply click the "OVER" button at the lower right of the screen, or type "Control-r".

When you wish to end the QSO, simply click the "Disconnect" button  on the toolbar, or select the Disconnect menu.

5.3 VHF and HF Packet Operation

5.3.1 VHF and HF Packet Choices

There are separate menus for making your HF and VHF packet choices. The format of the forms is identical. The VHF packet form is shown below. The description of the choices that follows applies to both the HF and VHF packet forms.

SSID

Sometimes you may wish to add a SSID (e.g. '-7') after your call. If you wish, select a SSID number  (0 to 15). Otherwise, select None.

TX Delay

To enter the delay required for your rig to switch from receive to transmit, simply click on the appropriate value.

Connect Text

Click 'Yes' and enter a message if you want your TNC to send a message automatically when someone connects with you.

Beacon Text

Click "Yes" if you want to send a beacon, and enter the message to be sent. Click on the interval between beacon messages. PLEASE don't send beacons unless you have a *really good* reason to!

Allow Digipeating

Click "Yes" if you want to allow others to digipeat through your station. PLEASE don't allow digipeating on HF packet! It is inefficient on that mode, and only contributes to extra QRM.

5.3.2. VHF and HF Packet Operation

5.3.2.1 General

VHF and HF Packet operation with SkyRider is essentially identical. First you must have set up your 'choices' for the packet mode (VHF or HF) that you are using. You only need to do that once, but you can change your choices at any time if you want.

5.3.3.2 Establishing and Breaking Connections

(Only use this procedure to make connections directly to a station or a node. Do Not use this method to connect from one node to another. Use the keyboard to enter the commands that the node needs)

Establishing a Connection

Either (your choice):

-click on the Connect button  on the upper tool bar,

or..

-select Connect from the Menu bar.

A menu will appear, asking for the call of the station you wish to connect to.

Enter the call, and SkyRider will attempt to make the connection. If you are unsuccessful in making a connection, follow the "Breaking a Connection" procedure.

Breaking a Connection

Either (your choice):

-click on the Disconnect button  on the upper tool bar,

or..

-select Disconnect from the Menu bar.

5.3.2.3 BBS and DX PacketCluster Menus

SkyRider provides two menus that you can optionally display to help you use basic BBS and PacketCluster functions.

To enable a menu, simply click on the appropriate button in the lower toolbar.

To turn a menu off, click on the button again, and the menu will disappear.

These menus are useful primarily to new or infrequent users and will reduce the need to memorize BBS commands. Because there is a wide variety of BBS's in use, these menus may not work in all cases.

5.3.3 VHF Packet Defaults

SkyRider sets the following defaults for VHF Packet. In general, these follow the suggestions in the 1278 manual. You can temporarily change these defaults by doing the following   -enter CONTROL-C from the keyboard   -enter the parameter you wish in the format described in the 1278 manual. The author welcomes suggestions for other defaults.

Defaults:

NULLS 0

NEWMODE ON

CONMODE C

MONITOR ON

DELETE OFF

CR ON

PACTIME OFF

PACLEN 128

DWAIT 16

UNPROTO CQ

NUCR OFF

NULF OFF

AUTOLF ON

AX25L2V2 ON

RESPTIME 0

ACKPRIOR ON

DEADTIME 33

MAXFRAME 4

FRACK 3

ACKTIME 13

SLOTS 4

RETRY 0

DAYUSA ON

5.3.4 HF Packet Defaults

SkyRider sets the following defaults for HF Packet. In general, these follow the suggestions in the 1278 manual. You can temporarily change these defaults by doing the following

-enter CONTROL-C from the keyboard

-enter the parameter you wish in the format described in the 1278 manual.

The author welcomes suggestions for other defaults.

Defaults:

NULLS 0

MONITOR ON

DELETE OFF

CR ON

CPACTIME OFF

PACLEN 32

DWAIT 0

UNPROTO CQ

NUCR OFF

NULF OFF

AUTOLF ON

AX25L2V2 OFF

RESPTIME 0

ACKPRIOR ON

DEADTIME 33

MAXFRAME 1

FRACK 8

ACKTIME 13

SLOTS 4

CHECK 0

RETRY 0

DAYUSA ON

5.4 CW Operation

5.4.1 CW Options

Enter data in the following form to control CW operation in SkyRider.

This form may be accessed while you are in CW mode. You access the form by:

-clicking the Options button  at the top of the SkyRider window,

or,

-selecting Options from the menu at the top of the SkyRider window.

Speed

The red bar graph shows the current speed setting graphically and the option box show the speed in words per minute. You can specify your CW sending speed upon entering CW mode in one of several ways:

-type in a value in the option box,

or,

-click on a speed from the list below the option box,

or,

-drag the box in the bar below the speed graph to set the speed you want.

Keying Weight

This value affects the relative ratio of "dash" times to "dot" times. Heavy weighting makes the dashes slightly longer, and light weighting makes them slightly shorter. Most often "normal" would be selected, but some transmitters will sound better to the station you are QSO'ing with when you use "light" or "heavy" weight.

Speed Style

"Proportional" means that the lengths of dots and dashes and the spaces between characters change with the sending speed. For very slow speeds, this means very long dots and dashes which some operators believe is difficult to copy.

"Farnsworth" uses proportional style ABOVE 15 wpm. At speeds below 15 wpm, the characters are sent at 15 wpm, and only the spacing between the characters is adjusted. Some operators believe that this is a better way for low speed stations to learn to copy CW.

Use / Cancel / Help

Click on Use to enable the changes you have made.

Click on Cancel to ignore the change you have made.

5.4.2 CW QSO's

Conducting a CW QSO is easy. The transmission speed is set to the value that you specified in the Options form. You can increase or decrease the speed by using the drag bar at the bottom of the screen.

The speed in the button below will change as you drag the bar. When you see the speed you wish, click the button and the 1278 will adjust speed to the selected value.

Tune in the station you wish to copy, as described in the MFJ manual.

Text will be sent as sent as soon as you enter data on the keyboard. You will be able to receive during pauses in your sending.

This button at the bottom of the SkyRider window causes the 1278 to analyze the speed of the signal being received.

Clicking on this button causes you to be prompted for a call sign. The call is placed in the button immediately to the right of this button. After doing this, you can click on that button and the call will  be transmitted for you, saving keystrokes.

(your callsign will be shown instead of "YourCall").Clicking on this button will cause "de " followed by your call sign to be transmitted for you, saving keystrokes.

5.5 RTTY / ASCII Operation

5.5.1 RTTY / ASCII Options

You need to make a number of choices before you use RTTY or ASCII for the first time, or if you wish to change your choices later. .Only one form is used for both modes, since RTTY is used much more than ASCII. The form is shown below. You access this form by entering the RTTY mode, then either clicking on the "Options" button  on the toolbar at the top of the screen, or  selecting the "Options" menu.

Speed

Click on the speed (baud rate) that you wish to use when you select RTTY or ASCII operation.

Un-Shift

"Un-shift" means that the TNC reverts to "LETTERS" mode when a space or 'new line' is transmitted. Normally you should consider setting this choice to "On" to avoid a garbled character causing a long string of numbers to be erroneously displayed on the screen.

Tones

You should set this choice to 'normal' usually. Occasionally, the other station may send inverted tones; in that case you should set the choice to 'inverted'.

Shift

Set this choice to 'narrow' if you are operating on HF. Set it to 'wide' if you are operating on VHF.

RTTY or ASCII

Set this to your preferred mode. Note that RTTY is used *much* more frequently than ASCII on the HF bands.

Use and Save: Click on this button after you have made your choices and wish to use them while operating.

Cancel

Click on this button if you wish to discard the choices that you have made. Your prior choices (the ones made before you accessed this form) will be the ones that you continue to use.

RTTY or ASCII QSO's

You must be in RTTY/ASCII mode to start an RTTY or ASCII QSO. You might want to call CQ, or you might want to tune in another station to call. When tuning in another station, make sure the 'threshold' control is set so the yellow LED is on while the other station is transmitting. Refer to the later section "Calling CQ" to learn how to call CQ in RTTY or ASCII modes

.When you are ready to send, either:

(a) click on the Send button   at the lower right of the screen,

or

(b) type  Control-T  from the keyboard then type whatever you wish.

Your transmitted text will be ON THE AIR!

When you are ready to receive after sending, either:

(a) click on the receive button   at the lower right of the screen,

or

(b) type Control-R  from the keyboard.

The TNC will finish sending whatever characters have not yet been transmitted, then will switch to receive mode.

SkyRider also makes it easy to send your callsign, send stored files, send the other station's callsign, or send an identification message.

6.0 Calling CQ in CW, RTTY/ASCII, Amtor and Pactor

You can call CQ once, or send a repeated CQ message. You can create a different CQ message for each mode.

6.1 To call CQ a single time, or to Edit the CQ message:

-Click on this button,

or

-select 'CQ' from the menu bar at the top of the window,

then

-select 'CQ Once' from the CQ menu.

This will cause the following form to be displayed:

If you have already entered a CQ message, it will be displayed.

Click 'Send' to transmit the CQ message.

If you want to enter a CQ message. press Edit.

6.2 To Call CQ repeatedly:

-Click on this button,

or

-select 'CQ' from the menu bar at the top of the window,

then

-select 'CQ Once' from the CQ menu.

This will cause the following form to be displayed:

Enter the number of times you want CQ to be sent, and the time (in seconds) between calls. Note that the time interval is only approximate, so you may need to adjust it later.

Calling CQ will stop automatically in Pactor mode when a connection is made. In the other modes, click the "Stop CQ" button.

7.0 Time Saving Tips

7.1 Sending an Identification Message

An identification message is in the form of:

hiscall  de yourcall

For example, if the other station is K1XYZ and your call is W9ABC, then the identification message would be:

K1XYZ de W9ABC

You can send the identification from the keyboard by manually typing all the keystrokes. However, you can save keystrokes by using some built-in features of SkyRider.

The other station's call must be displayed in the button beside the "Enter Call" button. That happens automatically when a Pactor or Packet connect occurs. In other modes, you should click on the "Enter Call" button to enter the other station's call into the appropriate button.

You can send an identification message in either of two ways:

Mouse method

Click on:

(1)  the other stations call sign button in the lower right of the screen

(2)  the  "de yourcall" button in the lower right of the screen.

Keyboard method

From the File menu, select the Send Id item.

The shortcut way to do so is to enter:    ALT-F  then   I

7.2 Sending the other station's Callsign

You can type the other station's call, of course, from the keyboard. You can save keystrokes, though, by using some built-in features of SkyRider.

The other station's call must be displayed in the button beside the "Enter Call" button. That happens automatically when a Pactor or Packet connect occurs. In other modes, you should click on the "Enter Call" button to enter the other station's call into the appropriate button.

You can send the other station's callsign in one of two ways:

Mouse Method

Click on the other stations call sign button in the lower right of the screen.

Keyboard Method

From the File menu, select the Send Callsign item.

The shortcut way to do so is to enter:    ALT-F  then   C

7.3 Sending Your Callsign

You can type your callsign, of course, from the keyboard. You can save keystrokes, though, by using some built-in features of SkyRider. Your callsign is displayed in a  button near the bottom right corner of the screen.. You can send "de your callsign" in one of two ways:

Mouse method

Click on  your call sign button in the lower right of the screen

Keyboard method

From the File menu, select the Send Callsign item.

The shortcut way to do so is to enter:    ALT-F  then   C

7.4 Using Text ("Brag") Files

Each operating mode can have 4 different files (sometimes called 'brag" files) that the user can edit and transmit whenever needed. SkyRider has a simple editor to help create those files. This editor can even handle graphics characters for operating modes that can use graphics characters (Pactor and Packet).In each operating mode, you will see 4 "file folder" buttons at the top of the screen, numbered 1 through 4.

Underneath each folder, is a title that you can specify. There are different file folders for each different operating mode (except the Amtor Listen mode).You need to know two things to use text files:

(1) how to edit them, and

(2) how to transmit them.

Click on the folder for the file you want to edit, or select the file number from the "Edit" menu. This will cause the following form to be displayed:

If there is already data in the file, it will be displayed in the window.

Click on "Edit" to  edit the file, or "Send" to transmit the file.

If you decide to do nothing, then click on  "Cancel" or use the Escape key.

Editing Text Files

SkyRider will display a form with a large edit area at the top.

You can type in data just like any text editor.

You can cut and paste to other text files using the Edit menu

Click on the "Clear" button to clear the screen if you wish.

At the bottom of the form, you will see an array of graphics characters. To insert a graphics character into your text file, simply:

-click to the point in the file where you want the character to go,

then..

-click on the graphics characters(s) you want to go into the file.

To make it easy to remember what is in your file, enter a "caption", or title, in the box at the bottom right of the form. This caption will be displayed under the appropriate file folder icon on the main SkyRider screen.

When you are done, click on the "Use and Save" button.

If you decide to discard what you have entered, simply click on the "Cancel" button.

Transmitting Text Files

The "Send" button will cause the file to be transmitted. You will see the characters in the file appear in the Keyboard Input window, just as if you typed them from the keyboard.

8.0 Printing

You can have SkyRider print everything that is received from the TNC. Use either of the following methods to turn the printer On or Off:

Mouse method

Click on the "Print"  check box near the upper left of the SkyRider window. This will start capturing printer data. Repeating this will cause capture of printer data to stop.

Keyboard method

Select the file menu by entering  "ALT-F”. Then, select the Capture to Printer function by typing  "P".(You can also use the mouse to select these menu items) This will start capturing printer Data. Repeating this will cause capture of printer data to stop.

The following form will appear:

If you choose "Print via Windows", all data from the TNC is then captured for printing. SkyRider uses the printing capabilities of Windows. All print lines will be saved, and are *not * printed immediately. To have your printer actually print the captured information, you can either:

..stop capture to printer,

or,

..click on the "Feed" button near the "Print" check box.

If you choose "Print Directly to Printer", SkyRider will send data directly to the parallel printer port (only LPT1 is supported). This will only work if your printer can directly print ASCII characters. PostScript printers will not work in this mode.

9.0 Capture to Disk

You save data from the TNC in a disk file for later use. To do this, use either of the following methods:

Mouse Method

Click on the "File capture" check box near the upper left part of the screen.(Repeating the above will cause capturing to disk to stop.)

Keyboard Method

Select the "File" menu by entering "ALT-F".

Select the "Capture to File" item by entering "F".(Repeating the above will cause capturing to disk to stop.)

A form will appear which will allow you to select a name for the file to be saved (the file names will be different on your computer):

SkyRider will suggest the following name:

SRyymmdd.SKY

where:

yy    is the  year  (e.g.95)

mm   is the month (e.g. 01 for January)

dd     is the day  (e.g.  25)

For example, if the date was 25 January 1995, SkyRider would suggest the name:

SR950125.SKY

You can choose a different name, or you can choose to add the captured data to a file already on your disk. Click OK or Cancel when you have chosen a name.

The following form will then be displayed:

Click  "Plain text only" to save "printable" text (no control codes will be saved).Click "Binary" to save everything received from the TNC.

If you choose Plain text only, the file is in "text" format, and can be read by any text editor, including Windows Notepad. Rows of "VVVVVVV"  in the file separate data saved at different times on the same day.

10.0 Acknowledgments

MFJ and 1278 are trademarks of MFJ Corporation

Windows is a trademark of Microsoft Corporation

Special thanks to the patient testing and constructive comments of Joe, KW1K, and other testers. This product could not have been completed without their help. This product will never be finished thanks to their constant suggestions for improvement!

Extra soecial thanks to my family for letting me have the time to work on SkyRider. Without their love and support, SkyRider would not exist.