.  NCTERM.EXE is a Terminal program for the PK-232 / PK-900 and handles PACTOR,

AMTOR and RTTY.It is an upgrade of the program PTCT232.EXE

.  Now Transmitting and Receiving of BINARY files is posssible with the

MT protocol.

.  The program makes full use of function keys F1 to F10 and combinations of

keys to give all the commands to the PK as: AM ,Pt ,AL &c.

.  When the program is running there are 2 windows on the screen

The upper window is the receive screen and the lower window is the

command and transmit screen.

.  A status line gives information about the status of the PK.

.  Packet is not implemented in this program. There are a lot of

good programs with packet.

.  Starting and Interrupting the program during a connect.

Description of the function keys F1 .. F10.

===========================================

F1   Help

Here all the function keys and the combination keys are shown.

F2   Switch to Pactor standby. When connected in pactor to a station

this key disconnects. F2 also stops calling a station.

F3   Switch to Amtor standby.When connected in amtor to a station

this key disconnects. F3 also stops calling a station.

F4   Switch to RTTY.

F5   Call a station in Pactor or Amtor.

There is a extra window on the screen for the call of the station

you want to connect to.

F6   Toggles listen mode for pactor and amtor.

F7   Toggles Unproto(100 Baud) for Pactor

and Fec for Amtor and Transmit for Rtty.

F8   Calling CQ in Pactor, Amtor and Rtty.

F9   The Mailbox Check function of the PK.

F10   Switch to dumb terminal.

Use this if you want to change the PK's parameters.

The ESCAPE key

==============

The Esc can be used to escape from a subwindow.

The PgUp key

============

The PgUp key shows the Scrollback buffer.

Description of Combination keys.

================================

CTRL-F2 Hard disconnect for PACTOR. If you loose the link

with your QSO partner you can force STB PACTOR

with this combination. Don't use it if the

link is good, for your QSO partner sends REQ's

till his 'timeout' time.

(F2 is the normal disconnect key for PACTOR)

CTRL-F3 Hard disconnect for AMTOR. If you loose the link

with your QSO partner you can force STB AMTOR

with this combination. Don't use it if the

link is good, for your QSO partner sends REQ's

till his 'timeout' time.

(F3 is the normal disconnect key for AMTOR)

ALT-F5  Call a listed station in Pactor or Amtor.

There is an extra window on the screen with the calls that are

in the file NCTERM.CAL. The maximum number of calls in this file

is 9. If there are more the remainder are ignored.

You must restart the program if you create a new list.

Don't forget the <return> after the last call in the list.

ALT-F7  Toggles Unproto 200 baud for pactor.

ALT-F9  Toggles REMOTE. When REMOTE is ON, the file NCTERM.MSG

is transmitted at the start of link if the other station connects.

ALT-F9  Toggles the MAILDROP.

ALT-A  Transmit an ASCII file and store on disk at the other side.

(If the other side uses the same protocol)

ALT-B  Transmit a BINARY file with a special protocol and store on

disk at the other side.

(If the other side uses the same protocol)

ALT-C  Configuration Screen.

Description follows.

ALT-D  Delete a file from disk.

ALT-E  Use a text editor. The editor must first be installed

with ALT-C(see below).

ALT-F  Write all received data to a file.

ALT-I  To write additional information (i.e Name and QTH) in the

disklog when disklog is ON and when you are connected.

It is also possible to add information to the logfile (NCTERM.LOG)

(after a disconnect) with your own editor (ALT-E).

ALT-K 1.Make the call of your QSO partner known to the program.

This is ONLY necessary in AMTOR (or if you started NCTERM

with the parameter INIOFF), if you want the CTRL-END

combination (see below).

2.With Alt-K you can give the call of a station already

during his CQ calling and then connect with F5.

ALT-L  Display the disklog on screen.

ALT-O  Go to DOS, and return to NCTERM with EXIT.

ALT-P  Switch the Printer On and Off.

ALT-R  Clear the receive screen.

ALT-S  Transmit (SEND) an Ascii file.(No saving at the other side)

ALT-T  Clear the transmit screen and the transmit buffer.

ALT-V  View a diskfile.

ALT-W  Toggles Wordout. With 'Wordout on' you can correct a typing

error before the word is transmitted. The backspace Char is

not transmitted when Wordout is ON.

ALT-X  Ends the program. ALWAYS end the program in this way to

enable correct initialisation of the PK the next time.

ALT-Y   Clears the Scrollback buffer.

ALT-1 to ALT-9  With this key combination you can transmit a text

wich is stored in the files NCTERM.TX1....NCTERM.TX9.

The maximum length of these files is 255 characters.

When NCTERM is started the files are stored in memory.

You can add a CHO or a Disconnect at the end of the file.

See the 2 sample files NCTERM.TX1 and NCTERM.TX5.

Alt-1..9 is only active during a connect.

CHO = ascii 25 and disc = ascii 4.

End or DEL

Change the transmit direction of the link.

CTRL+END  Transmits the sentence  ' yourcall de mycall '

If for some reason a total RESET for the PK is needed you can use

the combination SHIFT-TAB.

Installing new parameters.

==========================

Below is the window as it appears when ALT-C is pressed.

�����������������������������������������������������������������������������ͻ�

�  �             NCTERM V 4.05  Installation of new parameters                ��

�  �                                                                          ��

�  �  COMx-Hardware   Serial parameters      Screen         Delay times       ��

�  �                                                                          ��

�  �  COM  Adr  IRQ  * Baudrate  :9600      Change          Display/ms:2000   ��

�  �                 * Wordlength:8         COLORS          Blank /min:0      ��

�  � * 1  $3F8   4   * Parity    :n                         MaxIdleTime:5     ��

�  � * 2  $2F8   3   * Stopbits  :1                                           ��

�  � * 3  $3E8   4                          Line len:67                       ��

�  � * 4  $2E8   3     Scrollbackbuffer     TxWindow:5      Warning Signals   ��

�  � * V24-COMx: 1     Bytes :16384         VgaMode :n      Bell/Morse:m      ��

�  �                                                        Frequency :440    ��

�  �   DiskLog         Editor                Save                             ��

�  �   ON              Q.EXE                NCTERM.INI      Station calls     ��

�  �                                                        Call      :PA3CZL ��

�  �   AAB              Remote               Path           Selcall   :PANC   ��

��ͺ   OFF               OFF                USERS                             ��

�  � * = change in NCTERM.INI                                                 ��

�  � Choose one of the blinking characters.                                   ��

�  �                                     Stop with  ESC.                      ��

�  � Your choice :                                                            ��

�  �                                                                          ��

�����������������������������������������������������������������������������ͼ�

F1=Help  PACTOR    STB                         WO Off 9600Bd COM1� NCTERM 4.01

The parameters marked with an * cannot be changed in the ALT-C screen.

Use a text editor instead to change NCTERM.INI.

D     is for installing the disklog feature. See below.

E     is for installing an editor. See below.

A     is for Automatic Answer Back (AAB) in Amtor. See below.

R     If Remote must be On at start of the program.

P     The path of the Remote or Users directory.

O     is for user defined colors. Try it. WARNING! THE COLORS FOR RX AND

TX SCREEN MUST BE DIFFERENT.

I     is the linelength in the TX screen for the linewrap feature.

If transmitting a file, the linelength is 78.

The linelength is shown by up and down arrows.

W     Is for changing the height of the TX window.

V     is for changing the screen to VGA mode.

S     is for saving the new parameter file.

Y     is for the display time of some warning windows.

K     is the blanking time for the 'built-in' screenblanker. This only has

effect if you use a EGA or VGA screen. The value is in minutes.

A  '0'  for the time disables the screenblanker.

X     is the maximum Idle time in Minutes. If the Idle time exceeds

this value the program will disconnect.

A  '0'  for the MaxIdleTime disables this function.

B     the reserved bytes for the Scroll back buffer.

M     When switching to different modes with the F keys you can hear

a morse sign(m), single beep(b) or nothing(n).

F     is for choosing the frequency of the warning signals.

C     is your call for pactor.

L     is your selcall for amtor.

Changing the comport parameters.

=================================

The parameters for the serial port cannot be changed in the ALT-C screen.

They must be changed with an editor in the NCTERM.INI file.

Line 1 : The serial port number.

Line 2 : The baudrate.

Line 3 : The wordlength (always 8).

Line 4 : The parity (always n).

Line 5 : The number of stopbits (always 1).

Line 6 : The hex value of the adress of serial port 1. (don't forget the $)

The values below need not be changed with a 'normal' PC.

Change them only if you know exactly what you are doing !

Line 7 : The IRQ number of serial port 1.

Line 8 : The hex value of the adress of serial port 2.

Line 9 : The IRQ number of serial port 2.

Line 10: The hex value of the adress of serial port 3.

Line 11: The IRQ number of serial port 3.

Line 12: The hex value of the adress of serial port 4.

Line 13: The IRQ number of serial port 4.

The other values in the NCTERM.INI file need NOT to be changed with

an editor. They are changed if you use the ALT-C combination.

Below the text line '***** below this line . . . '

you can type some commands for your PK. eg. some tranceivers must

have RXRev on. You must RESTART the program before these extra

parameters have effect.

Calling CQ

==========

When calling CQ with FEC or Unproto you can transmit a CQ text with

the F8 key. There must be 3 CQ files in the same directory as

NCTERM.EXE,  named  NCTERM.CQA for Amtor, NCTERM.CQP for Pactor and

NCTERM.CQB for rtty.

Sample files are in this package. Always start these files with a blank

line.

PLACE YOUR CALLSIGN IN THESE FILES !

Using The personal Mailbox of the PK.

=====================================

If you want to use (local) the mailbox system of the PK switch to it

with  F9. You can use now the commands which are shown on the screen.

All with the restrictions described in the manual of the PK.

If you want to write a file in the memory of the mailbox do as follows:

First write a textfile with your own editor( ALT-A).

When back in the program:

Strike F9 .

After the prompt type:

S 'call'  ('call' is the addressee).

type:  ALT-S   (Make a choice).

The textfile now goes from disk to the MB memory.

If you want to write on disk a file which is in the MB memory do as follows:

Strike ALT-Y to clear the scrollback buffer.

Strike F9.

Type L : See what files are in the box.

After the prompt  Type: R (number) (number of the MB memory file).

After the reading strike F9 again.

Strike ALT-B (scroll back buffer).

Strike ALT-F (save the scroll back buffer on disk).

Strike ALT-B or ESC for going back to the program.

Now you can edit the diskfile with ALT-A.

The editor.

===========

With ALT-A you can use your favourite texteditor.

The name of the editor must be installed with the ALT-C command

The editor must be in the same directory as NCTERM.EXE is,

or the full path and filename must be given with the ALT-C ,

or the path to the editor must be given in the autoexec.bat file.

If you don't want to use an editor answer  NONE  in the ALT-C screen.

The first character of the editorname must not be a  N !

The AAB feature.

================

There is a automatic AAB function in this software and

only works in AMTOR.

If NCTERM receives a $ (dollar sign) then it sends

QRA mycall myselcall. Both your call and selcall come from

the PK. Some stations and most aplink stations send

a dollar sign as a 'Who Are You' sign (WRU).

Your PK then answers with your call and selcall.

If you don't need this feature switch it OFF with ALT-C and

the l option.

WARNING !  SWITCH AAB to OFF WHEN RECEIVING  7PLUS FILES !

The WORDOUT and LINEWRAP features.

==================================

On PACTOR the wordout is automaticaly switched off while most pactor

stations do accept the backspace character. So if you typed a mistake,

type the backspace key and then the right key.

On AMTOR , UNPROTO and FEC the wordout is automatically switched

to on. In all modes you can change wordout with ALT-W.

If you are typing on the keyboard it is not neccessary to type a hard

return at the end of a line. After a number of characters there is

an automatic return and linefeed at the end of a word. The wraplength

is defined in the ALT-C screen.

A return is still always possible at a shorter line.

The logbook option.

========================

This program has the possibility to write on disk the

contacts you made in PACTOR and in AMTOR. If disklog in

the ALT-C screen is set to OFF there is no logbook activity.

If you set the disklog to ON the logbook for the contacts is

automaticaly saved on disk.

There is no end of the logfile (perhaps your diskcapacity ?).

So it is better to rename after some time the file NCTERM.LOG and

to start with a new one.

Only when you have a link with another station can you give some

additional information (name and QTH) to the logfile NCTERM.LOG. Use ALT-I.

Use it when you are receiving. There is no loss of information of

receiving text during the time you are using ALT-I.

The maximum number of characters you can add is 35. Don't pass the End of

the ALT-I window.

If you have used ALT-L and the first part of the disklog is on the screen

you can search for a call or another item with ALT-F.

Tranmitting files with a bad or broken link.

============================================

If you are transmitting a file and you have a bad link or the link

is broken by your QSO partner after a predefined time your

transmitter goes back to Stand-by and the transmitted file is closed.

This timeout time is derived from the ARQTime of the pk232. The timeout

time is 10 lower as the ARQTime is,but the minimum timeout time is 30 Sec.

Starting the program when already CONNECTED. (NCTERM INIOFF)

============================================================

When you start the program with the parameter INIOFF then it will start

the correct way when there is a connect. If you want that other stations

can connect your MAILBOX without NCTERM running your maildrop must be

switchted ON (Shift-F9) before leaving the program.

The program can be started as NCTERM INIOFF and you can read on your

screen what is going on. In this case the call of the connected

station is not known to te program. Use Alt-K to type the call.

Transmitting and receiving BINARY and ASCII files in the mode PACTOR

(and Store).

=====================================================================

A new feature in NCTERM is the possibility of transmitting and receiving

BINARY and ASCII files. For the Binary files a special protocol is used.

It is the same protocol as used in the program MT* (for the PTC) so it is

now possible to exchange binary files between the PK and the PTC.

The protocol makes from the bytes in the binary file so-called nibbles

and these nibbles are translated to 16 different characters and

transmitted. The receiver works the other way around. Temporary files

are written on disk while receiving and transmitting BINARY files.

They are deleted if everything works fine.

The resulting BINARY file is written on the disk of the receiver.

The command for transmitting BINARY files is ALT-B and for ASCII files

is ALT-A. Files transmitted with ALT-S are not stored on Disk.

REMOTE must be ON (ON BOTH SIDES), for Transmitting and Storing files.

If you don't have a hard disk it costs more time to translate the

file to the nibbles file. Use a RAM disk if possible.

The USERS directory.

====================

In the .INI file you can give a directory name for the users.

If you leave NCTERM.INI unchanged a directory named USERS will be created

automatically by the program the first time you start the program.

If the name of the user directory starts with a *, no user directory will

be created.

The REMOTE Commands

===================

The REMOTE commands all start with + +  or / / and end with a CR.

These commands are given WITHOUT a CHO (END or DEL key) but WITH

a <RETURN> at the end of the command line.

The Remote commands are ++HELP,++MAIL,++DIR,++READ and ++RPRG.

This command are given by the connected station.

++HELP <Return>

---------------

The File NCTERM.HLP is transmitted.

You may change this file.

++MAIL <Return>

---------------

This command switches the maildrop of the PK ON and remote OFF.

The station disconnects and you have to connect again witin 1 minute.

If there is not a new connect within 1 minute, Maildrop is switched OFF

and remote is switched ON again.

++DIR <Return>

--------------

A list of the files in the user directory is transmitted to you.

The user directory is the directory that is marked in the NCTERM.INI

file. The connected user can only read from or write to this directory.

He can't delete any file in this directory.

Deleting must be done bye the sysop.

++READ Filename.ext  <Return>

-----------------------------

With this command you can read text files from the user directory.

If you want to save them on your disk you must open a receive file

with ALT-F at the receiving side, or save the text afterwards when

reading the scroll back buffer.There must be a space between read and

filename.

++RPRG Filename.Ext <Return>

-----------------------------

This is the same command as the use of ALT-Q at the other side.

A binary file or even a text file is translated to nibbles and

transmitted to you. The resulting file will be stored in your user

directory, so a right directory must be given in the NCTERM.INI.

The Remote function at your side must also be ON (Shift-F9) otherwise

the word #BIN# is not recognized. Here also a space between the command

and the filename.

Summary of Remote Commands.

===========================

You type:                          The Other Station answers with

(If he uses: NCTERM)

++HELP                                His NCTERM.HLP.

++DIR                                 Files in his User Directory.

++READ filename                       Transmitting of filename (from his user

(No saving at your side)              directory.)

++MAIL                                Switching on MAILDROP and disconnecting.

(If he is using a PK232 or PK900)

++RPRG filename                       Transmitting filename with special

(saved in your USER directory)        protocol.

ALt-A                                 The received file is saved in

Transmitting an Ascii file            his USER directory.

ALT-B                                 The received file is saved in his user

Transmitting a Binary file            directory. Works also with the

program MT and a PTC.

ALT-S

Transmitting an Ascii file            No saving or with ALT-F, yes.

WARNING !

=========

When you receive or transmit BINARY files there will be temporary files

on the hard disk. The Extension of these files are .NCT or .NCR. If you

end the programm in the normal way with ALT-X the files are deleted.

BUT ALL OTHER FILES WITH THE EXTENSION .NCT AND .NCR ARE DELETED IN

THE SAME DIRECTORY AS NCTERM IS !

Thanks to DL2FAK,DL3MFH,EA5FIN,PA0ANT,PA0HLT,PA0RVR,PA3BVK,PA3DBA,

PA3FDT,PE1KCJ,PE1MHO,PI8DAZ for

Protocol, Testing, Suggestions,Corrections and Uploading.

I hope you enjoy this program.

Best 73 from Roel, PA0NC @ PI8DAZ.NLD.EU