|  | Eimac Power Grid Tube Reference Data |  |
| --- | --- | --- |

| Back to Power Grid Tube Catalog Index Additional Catalogs: Klystrode® IOT ¦ Remanufactured Tube |  |
| --- | --- |

# 4CX250B/7203

The 4CX250B/7203 has a maximum plate dissipation rating of 250 watts and a maximum input power rating of 500 watts. The 4CX250B/7203 is designed to operate with a heater voltage of 6.0 volts.

## Characteristics

| Plate Dissipation (Max.); 250 Watts. Screen Dissipation (Max.); 12 Watts. Grid Dissipation (Max.); 2 Watts. Frequency for Max. rating (CW); 500 MHz. Amplification Factor; 5. Filament/Cathode; Oxide Coated. Voltage Current; 6.0 Volts 2.6 Amps. Capacitance; Grounded Cathode. Input Output Feedthrough; 15.7 pf 4.5 pf .04 pf. Capacitance; Grounded Grid. Input Output Feedthrough; 13.0 pf 4.5 pf .01 pf. Cooling; Forced Air. Base; 9 Pin Special. Air Socket; SK-600A. Air Chimney; SK-606. Boiler; ---. Length; 2.46 in; 62.50 mm. Diameter; 1.64 in; 41.70 mm. Weight; 4 oz; 113 gm. |  |
| --- | --- |

|  |  | Maximum Ratings |  | Typical Operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class of Operation | Type of Service | Plate Voltage (Volts) | Plate current (Amps) | Plate Voltage (Volts) | Screen Voltage (Volts) | Plate Current (Amps) | Drive Power (Watts) | Output Power (kiloWatts) |
| C | RF Amplifier up to 175 MHz | 2,000 | 0.25 | 2,000 | 300 | .025 | 2.9 | 0.390 |
| C | RF Amplifier Plate Modulated up to 175 MHz | 1,500 | 0.20 | 1,500 | 250 | 0.20 | 1.7 | 0.235 |
| AB1 | RF Linear Amplifier up to 175 MHz | 2,000 | 0.25 | 2,000 | 350 | 0.25 | --- | 0.300 |
| AB1 | RF Linear Amplifier(AM Service) up to 175 MHz | 2,000 | 0.25 | 2,000 | 350 | 0.15 | --- | 0.65 |
| AB1 | Af Amlifier or Modulator | 2,000 | 0.25 | 2,000 | 350 | 0.50 | --- | 0.600 |

Characteristics and Operating values are based on performance tests. Figures may change without notice as the result of additional data or product refinement. CPI Eimac Division should be consulted before using this information for final equipment design.

Top of Page

Last page update 3/21/97 5:00:57 PM 
Eimac Web Pages © copyright 1996 CPI, Eimac Division