# Ilmari-2003 Technical Data

70 cm: TX 437.850 MHz, APRS data 300 mW, ~-3 dBi omni, vertical F3E, GPS data + telemetry

70 cm: TX 437.347...437.385 MHz translator output, 1 W PEP output,  ~-1 dBi omni, horisonal, linear

70 cm: TX 436.912 MHz A1A telemetry beacon 1 W PEP,  ~-1 dBi omni horisontal

2 m: RX 144.306...144.344 MHz translator input, NF ~3dB, ~-1 dBi omni, horizontal, spectrum inverting

2 m: RX frequency confidential, command receiver

23 cm: TX 1282 MHz ATV, 1 W, ~-1 dBi omni antenna, horizontal, FM +- 8 MHz + sound channel

80 m:TX 3.5795 MHz, beacon, 1 W out, ~2 dBi omni antenna, A1A, GPS and telemetry data

- the linear 70 cm/2 m transponder, inverting, i.e. LSB uplinking results in USB downlink signals

- please print out the Ilmari-2003 bandplan for easier search of your downlink

- please FIRST check peak level of 436.912 MHz beacon noting peak S meter reading in SSB mode

- adjust your uplink power to be 6 dB peak (one S unit) BELOW the CW level

- the ATV TX may be off occasionally, depending on onboard battery status

- there are two cameras on board, CAM1 is a nadir camera and CAM2 is a horizon camera

- there are two sound subcarriers on the ATV TX: SC1 in on 6.0 MHz and SC2 is on 6.5 MHz

- SC1 carries a pitch modulated audio giving relative illumination data (TBD)

- SC2 carries return data from the command decoder (TBD)

- Ilmari-2003 carries a mission abort device

- batteries: 4 * 3.4 V, 12 Ah, 400 gr total mass, shelf life > 10 years

- balloon manufactured by Totex Corp., Japan, type TX1200, lift 3.5 kg, operating temp to -70°C

- balloon fill diameter 1.94 m, burst diameter 7.87 m, burst altitude typ. 29.8km