- Motorola M-Plus PC Card

- Part No. CDLN4321A

- User Guide

- Version 4A

Revision History

| Revision No. | Date | Changed By | Changes |
| --- | --- | --- | --- |
| 0.9 | 4/6/99 | T Robinson | Initial Draft |
| 1.0 | 2/8/99 | T Robinson | First Release |
| 3E | 30/3/00 | T Robinson | Updated to Mplus Release 3E |
| 4F | 08/05/00 | M Maplethorpe | Updated to Mplus Release 4A |

Table  of Contents

1.	Introduction	5

1.0.	Motorola Mobile Phone Support	5

2.	Kit Contents	6

3.	Getting Started	7

3.0.	Windows 95/98 and 2000 Installation	7

3.0.	Windows NT 4.0 Installation	7

3.0.	Establishing contact between PC and MS	8

3.0.	Configuring the Mobile	8

3.0.0.	Setting Logger Bytes in MS	8

3.0.0.	Non-GPRS Phones	8

3.0.0.	GPRS phones	9

4.	System Block Diagram	10

5.	Motorola M-Plus Test Mode	11

5.0.	M-PLUS Command Summary	11

5.0.	M-PLUS Basic Commands	12

5.0.0.	Timestamping of messages	13

5.0.0.	Idle Mode Report	13

5.0.0.	Path Loss Report	14

5.0.0.	Dedicated Mode Report	15

5.0.0.	Layer 1 / Layer 3 Messages	16

5.0.0.	GPRS RLC/MAC	17

5.0.0.	GPRS LLC	18

5.0.0.	SACCH Reports	18

5.0.0.	Channel Request Report/Immediate Assignment  Report	19

5.0.0.	BCCH   Report	19

5.0.0.	Paging Reports	20

5.0.0.	Idle Channel Description	21

5.0.0.	Dedicated Channel Description	22

5.0.0.	Cell Information	23

5.0.0.	Complete SACCh Reports	23

5.0.0.	Measurement Reports	24

5.0.0.	Phase 2 Path Loss Report	24

5.0.0.	EFEM Reports	25

5.0.	M-PLUS Remote MMI Facilities	26

5.0.0.	Remote Dial Number	26

5.0.0.	Remote Last Number Redial	26

5.0.0.	Remote Call Answer	26

5.0.0.	Remote Call End	26

5.0.0.	MS Power On/Off	27

5.0.0.	PIN Entry	27

5.0.0.	Additional user commands	27

5.0.0.	Flush Buffer	28

5.0.0.	Dial Number	28

5.0.0.	Mobile Registration Status	28

6.	Appendix A: EFEM_Periodic data message structure	29

6.0.	Pdpcontext	32

6.0.	Adjcellparms	32

6.0.	Cellbools	33

6.0.	Urlcthrput / Drlcthrput / Ullcthrput / Dllcthrput	33

6.0.	meas_report	33

6.0.	rlcmodes	33

6.0.	Definitions	34

# Introduction

M-Plus Test Mode is an ASCII-based format via serial port with support for Dual Band, Phase 2 GSM and GPRS (SMG29) additional features.

This format contains most Layer 1, 2 and 3 network information.  Layer 2 and 3 messages are presented as hex strings, and require a 3rd party application to decode the messages into an easily readable format.

## Motorola Mobile Phone Support

The Mplus card Version A4 of the card now supports the following Motorola mobiles:

Timeport L-Series GPRS release AF.29.22 or higher (SMG27)

Timeport L-Series GPRS release AF.2A.14 or higher (SMG29)

Timeport L-Series GPRS release AF.7A.14 or higher (SMG29)

- Timeport L-Series non-GPRS version BA.07.55 or higher

CD920/930

V.

- STAR-TAC 130

The firmware in the card presently supports a subset of  ‘passive’ features, i.e. monitoring the status of the MS.  Any commands that are unsupported will be ignored.

# Kit Contents

- M-Plus Kit - CD1590A

- Poly Insert		56C09104R01

- M-Plus Card		CDLN4321A

- Cable			SKN4821B

# Getting Started

## Windows 95/98 and 2000 Installation

- Switch PC on and wait for it to boot.

- Insert M-Plus card into spare PCMCIA slot.

- When prompted, insert floppy disk supplied, and Windows will automatically load the appropriate driver.  If prompted, select either Win9x or Win 2000 driver file.

- Once the driver has been loaded, you should check in Control Panel/System/Device Manager/Ports that the M-Plus card appears correctly.  Here you will also note the COM port that Windows has assigned to the card.

- It is essential that you note this as your application will need to be configured to talk to the correct COM port!

## Windows NT 4.0 Installation

Native NT 4.0 does not support Plug and Play operation.  The M-Plus card must be inserted and the PC should be rebooted.

The card will automatically be assigned a free COM port, which can be identified by looking in Control Panel/PCMCIA cards.  Clicking on the M-Plus card entry will bring up a window indicating which COM port has been assigned.

## Establishing contact between PC and MS

The following sequence of events can be followed to ensure that the MS talks to the PC:

- Boot PC and ensure that M-Plus card is active

- Connect MS to M-Plus card

- Power on MS

- Run HyperTerminal, and connect ‘direct to COM n’ where n is the COM port assigned to Mplus.

- Typing :I will return the firmware version of the Mplus card.

## Configuring the Mobile

Once communication has been established with the Mplus card, each mobile must be configured to send data to the card.  This is a one-off operation that stores data into the phone.

### Setting Logger Bytes in MS

In order to enable the MS to send out data the Mplus card needs to generate its ASCII output, the MS needs to be configured according to the following table:

### Non-GPRS Phones

| Logger Byte | Value |
| --- | --- |
| 0 | D8 |
| 1 | F6 |
| 2 | 02 |
| 3 | A2 |
| 4 | 80 |

### GPRS phones

| Logger Byte (GPRS phones) | Value |
| --- | --- |
| 0 | D8 |
| 1 | FE |
| 2 | 16 |
| 3 | A2 |
| 4 | D8 |

These values, once set, are stored permanently in EEROM in the phone and only need to be done once.

In Hyperterminal, the above data should be written to the phone by executing the :W command below for each Logger Byte.

:W0,y,z	Write a logger byte

Response

LOGGER BYTES:-STORE SUCCESS

y - Index of byte within array (0 – 4)

z - Value to write / value read (0 - FF)

Note y and z are HEX values

Logger byte settings can be read back as follows:

:R0,y		Read logger byte y

Response

LOGGER BYTE 0,y:-z

# System Block Diagram

Phone cable

3rd Party Windows Applications

PCMCIA Card

Serial Com Port

Laptop PC

- Figure 1 - System Block Diagram

# Motorola M-Plus Test Mode

M-Plus test mode is designed to interface to a human operator as well as PC applications, and so produces ASCII output suitable for displaying on a standard terminal application such as HyperTerminal.

## M-PLUS Command Summary

The M-PLUS commend mode codes controlling message dumps and modes of operation are as follows:

Motorola-specific commands that begin with a ‘+’ and ‘:’ have been added in addition to the standard feature set.  These are all two digit codes i.e. +n where n is the command.

Command	Test Function

1		Enable Idle Mode Reports

2		Enable Dedicated Mode Reports

3		Enable Layer 3 Messages

4		Enable Layer 1 Messages

6		Disable Idle Mode Reports

7		Disable Dedicated Mode Reports

8		Disable Layer 3 Messages

9		Disable layer 1 Messages

A		Enable SACCH Reports

B		Disable SACCH Reports

C		Enable Channel Request

D		Disable Channel Request

E		Enable BCCH Reports

F		Disable BCCH Reports

I		Idle Channel Description

J		Dedicated Channel Description

P		Enable Paging Reports

Q		Disable Paging Reports

Y		Mobile registration status

\1		Enable Path loss Reports

\6 		Disable Path Loss Reports

\A		Enable Complete SACCH Reports

\B		Disable Complete SACCH Reports

\C		Display Cell Information

\D		Remote Dial Number

\E		Remote Call End

\I		Display Idle Channel Description on change

\J		Display Dedicated Channel Description on change

\L		Remote Last Number Redial

\M		Enable Uplink Measurement Reports

\N		Disable Uplink Measurement Reports

\P		PIN entry

\S		Remote Answer Incoming Call

+N		Power on MS

+F		Power off MS

+1		Enable Phase 2 Path Loss Report

+6		Disable Phase 2 Path Loss Report

+G		Enable EFEM messages

+H		Disable EFEM messages

+R		Enable RLC/MAC messages

+Q		Disable RLC/MAC messages

+C		Enable LLC messages

+P		Disable LLC messages

+D<number><CR>	Dial number

+T		Enables Timestamping

+U		disables timestamping

:U		Enable all commands

:V		Disable all commands

:E		Toggles Echo (initially off)

:I		Displays card information

:S		Displays summary of enabled commands

:R		Read logger byte settings

:W		Write logger byte settings

:A		Displays the status of all commands

:P		Displays Phone information

:F		Flush output buffer

## M-PLUS Basic Commands

Test messages may be enabled or disabled as required throughout the logging period.  To control the logging/display of the following reports the user must send a specified command code to the serial port as indicated.

Test messages may be enabled or disabled as required throughout the logging period.  To control the logging/display of the following reports the user must send a specified command code to the serial port as indicated.

### Timestamping of messages

The timestamp values are taken from the phone itself and represent the time in milliseconds since the radio powered on.

+T starts timestamping

+U stops timestamping

- Timestamping is optionally prepended to each message.

- Default: Disabled

Output format:

- TTTTTTTTTT:

### Idle Mode Report

- To enable send '1'; to disable send '6'.

This reports the channel number, signal strength of the serving cell and the six strongest surrounding cells while in Idle Mode. Each report consists of a string of data as follows:

- Idle_Mode_Rpt: ssf ssr, nf1 nr1 nb1,...,nf6 nr6 nb6

Where:

ssf		Serving Cell BCCH Channel Number

ssr		Serving Cell RxLev

nf1.-.nf6	Non Serving Cell BCCH Channel Number

nr1 - nr6	Non Serving Cell RxLev

nb1 - nb6	Non Serving Cell BSIC

- Surplus fields will have the Channel Number and Rx Signal Strength set to 0.

The BSIC’s are reported with the NCC as the most significant digit and the BCC as the least.  If BSIC information is not available then 99 is sent in these fields.

### Path Loss Report

- To enable send '\1', to disable send '\6'.

This reports the channel number, Path Loss criterion value (C1) of the serving cell and the six strongest surrounding cells while in Idle Mode.  Each report consists of a string of data as follows:

- Path_Loss_Rpt:	ssf ssi, nf1 ni1 nb1 , .... nf6 ni6 nb6

Where:

ssf		Serving Cell ARFCN

ssi		Serving Cell Path Loss (C1)

nf1 - nf6	Non Serving Cell BCCH Channel Number

ni1 - ni6	Non Serving Cell Path Loss (C1)

nb1 - nb6	Non Serving Cell BSIC

- Surplus fields will have the channel number end path loss set to 0.

The BSICs are reported with the NCC as the most significant digit and the BCC as the least.  If BSIC information is not available then 99 is sent in these fields.

### Dedicated Mode Report

- To enable send '2'; to disable send '7'.

After being enabled these reports are displayed when the MS is in

Dedicated Mode.  This report shows the RxLev/RxQual readings the

Tx Power setting and the Timing Advance for the dedicated channel.

It also gives details of the six strongest surrounding cells.

Each report consists of a string of data as follows:

Dedicated_Rpt:	ta pwr rlf rqf rls rqs, nf1 nr1 nb1, ..., nf6 nr6 nb6

Where:

ta		Timing Advance (actual)

pwr		Tx Power (actual, decimal)

rlf		RxLev Full

rqf		RxQual Full

rls		Rxlev Sub

rqs		RxQual Sub

nf1 - nf6	Neighbour Cell BCCH Channel Number

nr1 - nr6	Neighbour Cell Rx Signal Strength

nb1 - nb6	Neighbour Cell BSIC

Note:	If there are less than 6 surrounding cells then the channel number and signal strength fields are set to 0. If a BSIC is not known then this field is set to 99.

### Layer 1 / Layer 3 Messages

- To enable Layer 1 messages send '4'; to disable Layer 1 messages send '9'.

- To enable Layer 3 messages send '3', to disable Layer 3 messages send '8'.

These reports show the Layer 1 and Layer 3 frames transmitted and received on the main signalling link (i.e. FACCH or SDCCH) except for the Handover Access and Physical Information messages sent during the handover procedure.  These messages use a common format as follows:

L3_Downlink	lll:	xx xx xx xx xx xx xx xx	xx xx

L3_Uplink	     lll:	xx xx xx xx xx xx xx xx	xx xx

L1_Downlink	lll:	xx xx xx xx xx xx xx xx	xx xx

L1_Uplink	     lll:	xx xx xx xx xx xx xx xx	xx xx

Where:

Ill		Length of message (decimal)

xx 		Octet of date in message (hex)

### GPRS RLC/MAC

- To enable send ‘+R’, to disable send ‘+Q’.

The RLC/MAC output format is specified as follows:

- RLC_UL_DATA [FFFFFFFFFF ]: MM DD DD DD

- RLC_DL_DATA [FFFFFFFFFF ]: MM DD DD DD

- RLC_UL_CTRL [FFFFFFFFFF ]: MM DD DD DD

- RLC_DL_CTRL [FFFFFFFFFF ]: MM DD DD DD

Where:

FFFFFFFFFF is the Absolute Frame Number

MM is the MAC header.

DD is an RLC data byte

- RLC Access burst format

- RLC_UL_ACC [FFFFFFFF] : MM DD DD

Where RLC_UL_ACC indicates an uplink access burst, either on the PACCH or the PRACH

MM specifes the message type as an unsigned byte, values are as follows:

00:  PACKET CONTROL ACK (8 bit format)

01:  PACKET CONTROL ACK (11 bit format)

10:  PACKET CHANNEL REQ (8 bit format)

11: PACKET CHANNEL REQ (11 bit format)

- DD DD

For 11-bit format:

8765 4321        8765   4321

[0] 0000 0XXX [1]  XXXX XXXX

For 8 bit format:

8765  4321        8765   4321

[0] 0000 0000 [1]  XXXX XXXX

### GPRS LLC

- To enable send ‘+C’, to disable send ‘+P’.

- LLC_UL LL : xx xx

- LLC_DL LL : xx xx

Where:

LL		length of message (decimal)

xx 		octet(s)  of message (hex)

### SACCH Reports

- To enable send ‘A’, to disable send ‘B’.

This report shows the reception of SACCH blocks and displays the layer 3 header of the SYSINFO 5 or 6 message.  Each report consists of a string of data as follows:

- Sacch_Report: mrlt crtl pd mt

Where:

mrlt		Max Radio link Timeout counter

crtl		Current Radio Link Timeout counter

pd		Layer 3 Message PD/TI (hex)

mt		Layer 3 Message Type (hex)

If the MS fails to decode the SACCH block then the lost two fields are set to 'ff'.

### Channel Request Report/Immediate Assignment  Report

- To enable send 'C'; to disable send 'D'.

This report shows the data sent by the MS when requesting a channel and the immediate assign messages received in response.  It also indicates if the MS will respond to the IA messages.

Each report consists of two strings of data as follows:

- Chan_Req_Report:	data frame

Agch_Report ll:	Respond xx xx xx xx ... xx xx

Ignore xx xx xx xx ... xx xx

Where:

Data	Data sent in Channel request message (hex)

frame	Frame number of Channel Request

ll	Length of Layer 3 message (decimal)

xx	Octet of data in Layer 3 message (hex)

### BCCH   Report

To enable send 'E'; to disable send 'F'.

Each report consists of a string of data as follows:

- Bcch_Report ll:fff xx xx xx xx ... xx xx

Where:

ll		Length of Layer 3 message (decimal)

fff		Channel Number from which the BCCH block was read

xx		Octet of data in layer 3 message (hex)

### Paging Reports

- To enable send 'P', to disable send 'Q'.

These reports show the messages received on the paging subchannel and from the Extended sub-channel when in extended paging mode.  It will not report all the messages read in Re-organise mode.  Each report consists of a string of data as follows:

- Page_Report ll:mdsc cdsc xx xx xx xx ... xx xx

Where:

ll		Length of Layer 3 message (decimal)

mdsc		Max value of Downlink Signalling counter (decimal)

cdsc		Current value of Downlink Signalling counter (decimal)

xx		Octet of data in Layer 3 message (hex)

Note: Current Downlink Signalling Counter is not available from the MS in current releases and is set to be the same as Max Downlink Signalling Counter.

### Idle Channel Description

This report can be produced as a one-off by sending 'l' or may be caused to appear on any relevant change in configuration by sending '\l'.  The ‘\l’ command has a toggle action for enabling and disabling this feature.

This report shows the current control channel configuration and the MS's paging sub-channel.  The channel is displayed as a single string of data as follows:

Idle_Channel: fff bs, Comb/NonC CchC=d Mfrm=d AGRes=d, CchG=d Pmfrm=d PBI=D

Where:

fff		Serving Cell BCCH Frequency

bs		Serving Cell BSIC

Comb/NonC	Combined or Non-Combined CCCH

CchC		CCCH Channel Count

Mfrm		Multiframes between pages

AGRes	Access Grant Reserved blocks

CchG		Mobile Paging Subgroup - Cch Group

PMfrm		Mobile Paging Subgroup - Paging Multiframe

PBI		Mobile Paging Subgroup- Paging Block Index

### Dedicated Channel Description

This report can be produced as a one-off by entering 'J' or may be caused to appear on any relevant change in configuration by entering ‘\J'.  The ‘\J' command has a toggle action for enabling and disabling this feature.

For a non-hopping channel the report is displayed as a single string:

Dedicated_Chan : fff bs ChanType TS=d Sub=d Tsc=d NonHopping BA=0 Freq=ddd

For a hopping channel the report is displayed as two strings:

- Dedicated_Chan : fff bs ChanType TS=d Sub=d Tsc=d Hopping

- MAIO=dd HSN=dd BA=0 ChanCount=dd, frq frq ... frq

Where:

fff		Serving Cell BCCH Frequency

bs		Serving Cell BSIC

ChanType	TchF/TchH/Sdcch4/Sdcch8

TS		Timeslot Number

Sub		Sub-channel Number

Tsc		Training Sequence Code

Freq		Dedicated Channel Number

MAIO		Mobile Allocation Index Offset

HSN		Hopping Sequence Number

ChanCount	Number of frequencies in hopping sequence

frq		Frequency in hopping sequence

The BA field is not used but is retained in the string to give compatibility with earlier Test Mobiles.

### Cell Information

- To get this report send ‘\C'.  This is a one-off report.

This report shows the cell information of the current serving cell.  The message is displayed as a single string of data as follows:

Cell_ID: Cl	= iiii, LAC=llll MNC=nnn MCC=ccc

Where:

Iiii		Cell Identity (in hex)

ccc		Mobile Country Code

nnn		Mobile Network Code

llll		location Area Code (in hex)

### Complete SACCh Reports

- To enable send '\A', to disable send '\B'.

This report shows the reception of SACCh blocks and displays the layer 2 frame, received at layer 1. Each report consists of a string of data as follows:

- Sacch_Data ll : mrlt crtl xx xx xx xx ... xx xx

Where:

ll		length of Layer 2 frame (decimal)

mrlt		Max Radio link Timeout counter

crti		Current Radio Link Timeout counter

xx		Octet of data in Layer 2 frame

### Measurement Reports

- To enable send ‘\M', to disable send '\N'.

This report shows the transmission of Layer 3 Measurement Reports on the SACCH.  Each report consists of a string of data as follows:

- Meas_Rpt : xx xx xx xx ... xx xx

Where:

Xx		Octet of data in layer 3 message

### Phase 2 Path Loss Report

- To enable send '+1', to disable send '+6'.

This reports the channel number, Path Loss criterion values (C1 and C2) of the serving cell and the six strongest surrounding cells while in Idle Mode.  Each report consists of a string of data as follows:

Ph2_Path_Loss_Rpt:	ssf ssi ssk, nf1 ni1 nk1 nb1 , .... nf6 ni6 nk6 nb6

- Where

ssf		Serving Cell ARFCN

ssi		Serving Cell Path Loss (C1)

ssk		Serving Cell Path Loss (C2)

nf1 - nf6	Non Serving Cell BCCH Channel Number

ni1 - ni6	Non Serving Cell Path Loss (C1)

nk1 - nk6	Non Serving Cell Path Loss (C2)

nb1 - nb6	Non Serving Cell BSIC

- Surplus fields will have the channel number and path loss set to 0.

The BSICs are reported with the NCC as the most significant digit and the BCC as the least.  If BSIC information is not available then 99 is sent in these fields.

### EFEM Reports

- To enable send ‘+G’, to disable send ‘+H’.

- EFEM_Periodic LL : xx xx xx xx xx ... xx xx

Where:

LL		length of message (decimal)

xx 		octet(s)  of message (hex)

- The format of the message contents are defined in appendix A.

- Note:	This message is only available when using a GPRS mobile.

## M-PLUS Remote MMI Facilities

### Remote Dial Number

- To enable send '\D'.

- On entry of the command code the prompt

Dial ?

is printed to the serial interface.  The user may then enter the required number(s) for dialling followed by a carriage return character which will initiate the call.  All message output is suspended while the number is entered.

Accessing this function will flush the contents of the notepad buffer before replacing it with the number entered for dialling.

### Remote Last Number Redial

- To enable send ‘\l'.

- On entry of this command the last number is redialled.

### Remote Call Answer

- To enable send ‘\S’.

On entry of this command any incoming call is answered as if from the keypad.

### Remote Call End

- To enable send '\E'.

- This is the equivalent of operating the END key on the keypad.

### MS Power On/Off

This command enables the PC to control power to the MS.

To turn MS on, send ‘+N’.

To turn MS off, send ‘+F’.

### PIN Entry

- To enable send ‘\P’.

- On entry of the command code the prompt

PIN ?

is printed to the serial interface.  The user may then enter the required PIN (4 – 8 digits) followed by a carriage return character. On success one of the following

- PIN:-SUCCESS

- PIN:-INCORRECT PIN

PIN:-ILLEGAL STATE FOR OPERATION

PIN:-UNDEFINED FAILURE

is printed to the serial interface.

- All message output is suspended while the number is entered.

### Additional user commands

The following commands are useful as diagnostics and to assist in reporting problems, but may not generate consistent data across firmware builds – treat with caution if using these in PC applications.

:U	Enable all commands

:V	Disable all commands

:E	Toggles Echo (initially off)

:I	Displays card information

:S	Displays summary of enabled commands

:A	Displays the status of all commands

:P	Displays Phone information

### Flush Buffer

This command flushes data in the output buffer of the Mplus card.  This command operates both in ASCII and binary mode.

- To flush output buffer, send ‘:F’.

### Dial Number

+D<phone-number><CR>{<LF>}

dials phone number immediately.

### Mobile Registration Status

- To query, send Y

Response: Service_state    :x<CR><LF>

with x  representing

0 : no service

1 : emergency service

2 : normal service

3 : not updated (intermediate or unknown state)

4 : roaming (preferred system)

5 : roaming (non-preferred system)

# Appendix A: EFEM_Periodic data message structure

| Index | Size | Type | Name | Description |
| --- | --- | --- | --- | --- |
| 0 | 12 | Structure (see 6.1) | Pdpcontext[0] | PDP context |
| 12 | 12 | Structure (see 6.1) | Pdpcontext[1] |  |
| 24 | 12 | Structure (see 6.1) | Pdpcontext[2] |  |
| 36 | 12 | Structure (see 6.1) | Pdpcontext[3] |  |
| 48 | 4 | BYTE | Pdptype | Type of PDP context |
| 52 | 20 | Structure (see 6.2) | Adjcellparms[0] | Adjacent cells parameters |
| 72 | 20 | Structure (see 6.2) | Adjcellparms[1] |  |
| 92 | 20 | Structure (see 6.2) | Adjcellparms[2] |  |
| 112 | 20 | Structure (see 6.2) | Adjcellparms[3] |  |
| 132 | 20 | Structure (see 6.2) | Adjcellparms[4] |  |
| 152 | 20 | Structure (see 6.2) | Adjcellparms[5] |  |
| 172 | 2 | Structure (see 6.3) | Cellbools[0] | Bit-fields arrays for adjacent cells + serving cell |
| 174 | 2 | Structure (see 6.3) | Cellbools[1] |  |
| 176 | 2 | Structure (see 6.3) | Cellbools[2] |  |
| 178 | 2 | Structure (see 6.3) | Cellbools[3] |  |
| 180 | 2 | Structure (see 6.3) | Cellbools[4] |  |
| 182 | 2 | Structure (see 6.3) | Cellbools[5] |  |
| 184 | 2 | Structure (see 6.3) | Cellbools[6] |  |
| 186 | 2 | WORD | P_bccharfcn | Broadcast channel |
| 188 | 18 | WORD | Pd_tcharfcn | Traffic Channel |
| 206 | 2 | Structure (see 6.4) | Urlcthrput | RLC Uplink Throughput |
| 208 | 2 | WORD | Urlcnumframe | RLC Uplink total number of frame for time period |
| 210 | 2 | WORD | Urlcnumrepeat | RLC Uplink number of repeated frame for time period |
| 212 | 2 | Structure (see 6.4) | Drlcthrput | RLC Downlink Throughput |
| 214 | 2 | WORD | Drlcnumframe | RLC Downlink total number of frame for time period |
| 216 | 2 | WORD | Drlcnumrepeat | RLC Downlink number of repeated frame for time period |
| 218 | 2 | Structure (see 6.4) | Ullcthrput | LLC Uplink Throughput |
| 220 | 2 | WORD | Ullcnumframe | LLC Uplink total number of frame for time period |
| 222 | 2 | WORD | Ullcnumrepeat | LLC Uplink number of repeated frame for time period |
| 224 | 2 | Structure (see 6.4) | Dllcthrput | LLC Downlink Throughput |
| 226 | 2 | WORD | Dllcnumframe | LLC Downlink total number of frame for time period |
| 228 | 2 | WORD | Dllcnumrepeat | LLC Downlink number of repeated frame for time period |
| 230 | 2 | WORD | Fer | Frame Erasure Rate |
| 232 | 2 | WORD | Bler | Block Error Rate |
| 234 | 2 | WORD | Accessclass | Access Class (as broadcasted by the network) |
| 236 | 2 | WORD | Cellid | Cell Identity |
| 238 | 2 | WORD | Lac | Location Area Code |
| 240 | 2 | WORD | Mcc | Mobile Country Code |
| 242 | 2 | WORD | Mnc | Mobile Network Code |
| 244 | 2 | WORD | Splitpgcycle | SPLIT_PG_CYCLE |
| 246 | 2 | INT16 | Freqerror | Frequency error for serving cell |
| 248 | 2 | INT16 | c1 | C1 PassLoth criterion for serving cell |
| 250 | 2 | INT16 | c2 | C2 Reselection criterion for serving cell |
| 252 | 2 | INT16 | c31 | C31 Reselection criterion for serving cell |
| 254 | 2 | INT16 | c32 | C32 Reselection criterion for serving cell |
| 256 | 1 | BYTE | Bsic | Base Station Identification Code |
| 257 | 1 | BYTE | l1mode | L1 Mode, kind of meta state |
| 258 | 1 | BYTE | Chantype | Channel Type |
| 259 | 1 | BYTE | Attachstatus | Attach Status |
| 260 | 1 | BYTE | Rxlevam | RXlev Access Min |
| 261 | 5 | Structure (see 6.5) | meas_report | Some measurement report information |
| 266 | 1 | BYTE | Syncstatus | Synchronisation Status |
| 267 | 1 | BYTE | Mstxpowermax | Max TX Power |
| 268 | 1 | BYTE | Txpower | TX Power |
| 269 | 1 | BYTE | Rlnktimeout | Radio Link Timeout |
| 270 | 1 | BYTE | num_pd_tcharfcn | Number of traffic Channel |
| 271 | 1 | BYTE | Maio | Mobile Allocation Index Offset |
| 272 | 1 | BYTE | Hsn | Hopping Sequence Number |
| 273 | 1 | BYTE | Ciphermode | Ciphering Mode |
| 274 | 1 | BYTE | Bspamfrms | BS_PA_MFRMS |
| 275 | 1 | BYTE | Bsagblkres | BS_AG_BLK_RES |
| 276 | 1 | BYTE | Maxretrans | max_retrans |
| 277 | 1 | BYTE | Txinteger | TX Integer |
| 278 | 1 | BYTE | Creshys | Cell Reselect Hysteresis |
| 279 | 1 | BYTE | Cresoff | Cell Reselection Offset |
| 280 | 1 | BYTE | Tempoff | Temporary Offset |
| 281 | 1 | BYTE | Powroff | Power Offset |
| 282 | 1 | BYTE | Pentime | Penalty Time |
| 283 | 1 | BYTE | t3212 | t3212, LU timer |
| 284 | 1 | BYTE | t3314 | t3314, Ready Timer |
| 285 | 1 | BYTE | Racolour | RA Colour |
| 286 | 1 | BYTE | Nwopermode | Network Operation Mode |
| 287 | 1 | BYTE | Nwctrlorder | Network Control Order |
| 288 | 1 | BYTE | Priaccessthr | Priority Access Threshold |
| 289 | 1 | BYTE | Dtx | DTX |
| 290 | 1 | BYTE | Vocoder | Vocoder |
| 291 | 1 | BYTE | last0408clr | Last 4.08 Clearing Cause |
| 292 | 1 | BYTE | Ultsallocation | Uplink TS allocation |
| 293 | 1 | BYTE | Dltsallocation | Downlink TS allocation |
| 294 | 1 | BYTE | Ulcodingscheme | Uplink Coding Scheme |
| 295 | 1 | BYTE | Dlcodingscheme | Downlink Coding Scheme |
| 296 | 1 | BYTE | Timingadv | Timing Advance |
| 297 | 1 | BYTE | Numpdp | Number of active PDP |
| 298 | 1 | BYTE | Numadjcell | Number of Adjacent cell |
| 299 | 1 | Structure (see 6.6) | rlcmodes | RLC up-downlink mode, ack/nonack |

## Pdpcontext

| Index | Size | Type | Name | Description |
| --- | --- | --- | --- | --- |
| 0 | 4 | DWORD | ipaddr | IP address |
| 4 | 2 | WORD | pkthrput | Given Peak Throughput |
| 6 | 2 | WORD | mnthrput | Given Mean |
| 8 | 1 | BYTE | delayclass | Given delay class |
| 9 | 1 | BYTE | reliabclass | Given reliability class |
| 10 | 1 | BYTE | precedclass | Given precedence class |
| 11 | 1 | BYTE | radiopriority | Radio priority |

## Adjcellparms

| Index | Size | Type | Name | Description |
| --- | --- | --- | --- | --- |
| 0 | 2 | WORD | arfcn | ARFCN |
| 2 | 2 | INT16 | freqerror | Frequency error |
| 4 | 2 | INT16 | c1 | C1 PassLoth criterion |
| 6 | 2 | INT16 | c2 | C2 Reselection criterion |
| 8 | 2 | INT16 | c31 | C31 Reselection criterion |
| 10 | 2 | INT16 | c32 | C32 Reselection criterion |
| 12 | 1 | INT8 | cresoff | Cell reselection offset |
| 13 | 1 | INT8 | tempoff | temporary offset |
| 14 | 1 | INT8 | rxlev | rxlev |
| 15 | 1 | INT8 | rxlevam | rxlev access min |
| 16 | 1 | INT8 | mstxpowermax | MS max TX power |
| 17 | 1 | BYTE | bsic | Base Station Identification Code |
| 18 | 1 | BYTE | racolour | RA_COLOUR |
| 19 | 1 | BYTE | syncstatus | synchronisation status of cell |

## Cellbools

| Index | Size | Type | Name | Description |
| --- | --- | --- | --- | --- |
| 0. | .1 | BIT | cba | CELL_BAR_ACCESS, adjacent and serving cells |
| 0. | .1 | BIT | cbq | CELL_BAR_QUALIFY, adjacent and serving cells |
| 0. | .1 | BIT | callreestab | Call reestablishment allowed, adjacent and serving cells |
| 0. | .1 | BIT | ecsc | Early ClassMark Sending Control, serving cell only |
| 0. | .1 | BIT | att | ATT flag, serving cell only |
| 0. | .1 | BIT | gprsind | GPRS_INDICATOR, adjacent and serving cells |
| 0. | .1 | BIT | si_2_5_bis | Sysinfo 2/5 bis decoded, adjacent and serving cells |
| 0. | .1 | BIT | si_2_5_ter | Sysinfo 2/5 ter decoded, adjacent and serving cells |
| 1. | .1 | BIT | pcmeaschan | PC_MEAS_CHAN, serving cell only |
| 1. | .1 | BIT | is_meas_report_valid | Tells if the measurement report was valid when the structure was filled in; serving cell only |
| 1. | .6 | BIT | padding |  |

## Urlcthrput / Drlcthrput / Ullcthrput / Dllcthrput

| Index | Size | Type | Name | Description |
| --- | --- | --- | --- | --- |
| 0. | .9 | BIT | kbit_per_seconds | up to 511 Kbit/s |
| 1. | .6 | BIT | decabit_per_seconds | up to 99 Decabit/s |

## meas_report

| Index | Size | Type | Name | Description |
| --- | --- | --- | --- | --- |
| 0 | 1 | INT8 | rxlev | RXlev |
| 1 | 1 | BYTE | rxlevfull | RXlev Full |
| 2 | 1 | BYTE | rxlevsub | RXlev Sub |
| 3 | 1 | BYTE | rxqualfull | RXqual Full |
| 4 | 1 | BYTE | rxqualsub | RXqual Sub |

## rlcmodes

| Index | Size | Type | Name | Description |
| --- | --- | --- | --- | --- |
| 0. | .4 | BIT | rlculmode | rlc uplink mode ack-unack |
| 0. | .4 | BIT | rlcdlmode | rlc downlink mode ack-unack |

## Definitions

DWORD	Unsigned 32 bit data.

WORD	Unsigned 16 bit data.

INT16		Signed 16 bit data.

BYTE		Unsigned 8 bit data.

INT8		Signed 8 bit data.

BIT		Individual bits.

Note: All data is in big endian format (i.e. most significant byte first).