UIDIGI

Version 1.8 BETA 6  (20001228) TNC2

Copyright 2000 Marco Savegnago IW3FQG

English Version

What is UIDIGI?

UIDIGI is firmware to use a TNC2 (or TNC2 clone) as an APRS packet digipeater with advanced functions.

(APRS is a register trademark of Bob Bruninga WB4APR)

Why UIDIGI?

Several reasons.  But the bottom line is cost, availability, and performance !!!  Cost because the TNC2/clone package is the cheapest (when time to assemble is included in calculations), they are widely available, and most importantly, UIDIGI brings features to the TNC2/clone that make it efficient for use in a complex APRS network.

## What's new

Please read the file changes.txt to see the changes from version 1.7

## The characteristics of UIDIGI

- The firmware may be installed in a TNC2 or 100% compatible clone with CPU clocks of 2.4576Mhz, 4.9152Mhz and 9,8304Mhz.  TNC2/clone must have at least 32K of RAM.

- Support AX.25 release versions 1 and 2.

- Full control of AX.25 link layer parameters (Timers, SlotTime, Persistence) also in remote mode via sysop interface

- Repeat only AX.25 UI frame with PID 0xf0 (or any other user

- Support the digipeating of UI AX.25 frame addressed to the digipeater callsign or alias.

- Support the digipeating of UI AX.25 frame addressed to user definable (up to 8 also via remote sysop interface) generic callsign, with call substitution.

- Support the digipeating of UI AX.25 frame addressed a generic call (eg: WIDE) using flooding algorithm.

- Support the digipeating of UI AX.25 frame addressed a generic call (eg: TRACE) using trace+flooding algorithm.

- Ignore duplicated frame sent in a defined interval.

- Ignore the digipeating of digipeater originated frame (beacon), frame already digipeated with call substitution or generic callsign.

- Support for handling the different SSID meaning on destination call of the UI Frame (Directional Beacon)

- Support for preempted digipeating

- Remote management protected by password.

- Digipeater heard station list.

- Up to 3 user definable Beacon Text.

- Reply to the APRS query.

## The release of UIDIGI

If in the program version you see the labels ALPHA or BETA,  this means that the version you are using is not considered to be a stable version.

## Why still use a TNC2?

- Because TNC2 (and clones) are easy to get - both new and used.

- Because it’s better than a computer if you have to mount a digi on top of a mountain!

## How to use UIDIGI

UIDIGI EPROM image is distributed in binary form.

You must configure it before making an EPROM, to do this you change the configuration settings

in the file UIDIGI.TXT and then use the UIDGCFG program to make the binary image.

Once you have the binary image you can burn the EPROM.

## Handling of the UI FRAME

The digipeater is capable of repeating only the AX.25 UI frame type (this insures that the digipeater may not be used as a normal AX.25 digipeater).  As a UI frame digipeater, it only digipeats frames addressed to the callsign, the alias, to the generic addresses (RELAY WIDE or TRACE) or to the special APRS addresses WIDEn-n TRACE n-n.

Method 1 (Specified Path):

A frame like this:

IW3FQG>APRS v UIDIGI, WIDE

will be repeated from the digi:

IW3FQG>APRS v UIDIGI*, WIDE

Method 2 (Generic Path):

A frame like this:

IW3FQG>APRS v RELAY, WIDE

will be repeated from the digi:

IW3FQG>APRS v UIDIGI*, WIDE

Method 3 (WIDEn-n Path):

A frame like this:

IW3FQG>APRS v RELAY,WIDE1-1

will be repeated from the first digi:

IW3FQG>APRS v RELAY*, WIDE1-1

and then from the second digi:

IW3FQG>APRS v RELAY*, WIDE1-0

then the third make the call substitution:

IW3FQG>APRS v RELAY, WIDE1-0*

Method 4 (TRACEn-n Path):

A frame like this:

IW3FQG>APRS v RELAY,TRACE1-1

will be repeated from the first digi:

IW3FQG>APRS v RELAY*, TRACE1-1

and then from the second digi:

IW3FQG>APRS v RELAY, DIGI1*, TRACE1-0

and then from the third digi:

IW3FQG>APRS v RELAY, DIGI1, DIGI2*

Method 5 (SSID Path): (experimental)

This method follows the implementation that I've observed in the TM-D700

I've observed that when UISSID is ON the digipeater repeats all UI frames addressed to ANY CALL with SSID > 0 (not only the mycall or other generic call) and digipeats as follows:

From SSID > 0 and SSID < 8

The frame will be repeated with the SSID of destination decremented by one and the call of the digipeater will be put in the via list.

Example:

A frame like this (MYCALL is IW3FQG and UNPROTO is APRS-7):

IW3FQG>APRS-7:

will be repeated to:

IW3FQG>APRS-6, DIGI*:

With SSID > 8 the translation rules is different because there is the direction rule.

If SSID is equal to 8 (North direction)

If the digipeater hears an UI frame addressed to ANY call with a ssid = 8 it repeat the frame, put the SSID of destination to zero,  put the digipeater callsign in the via list and attach to this if  available the path for North direction (NPATH parameter in D700).

Example:

A frame like this (MYCALL is IW3FQG and UNPROTO is APRS-8, NPATH in the digi is set to NORTH):

IW3FQG>APRS-8:

will be repeated to:

IW3FQG>APRS,DIGI*,NORTH:

If SSID is equal to 12 (North direction)

If the digipeater listen an UI frame addressed to ANY call with ssid = 12 it repeat the frame, put the SSID of destination the same as received, put the digipeater callsign in the via list and attach to this if available the path for North direction (NPATH parameter in D700).

Example:

A frame like this (MYCALL is IW3FQG and UNPROTO is APRS-12, NPATH in the digi is set to NORTH):

IW3FQG>APRS-12:

will be repeated to:

IW3FQG>APRS-12,DIGI*,NORTH:

The same rule is valid for South (SSID 9,13) East (10, 14) and West (11, 15).

I've observed that in all cases if the frame is sent with a digipeater to the via path (ex: APRS-8 VIA DIGI) the frame is not repeated.

IW3FQG>APRS-8,DIGI:

Is not repeated by the digipeater.

Method 6 (Preempt digipeating): (experimental)

The preemption process is a kind of frame preprocessing that remove unused digipeater in the path.

When the digipeater heard a frame like this:

STATION1>APRS, PATH1, PATH2, PATH3

and has PREEMPT 1, PREEMPTCALLS PATH3, PREEPTADD (none)

before processing the frame with the standard digipeating rules changes the frame in:

STATION1>APRS, PATH3

then process the frame with the standard APRS digipeating rules.

With the parameter PREEPTADD NEW1

the frame will be STATION1>APRS, PATH3, NEW1

Please remember that the preemption process is a preprocessing so if after the processing the frame is not to be digipeated UIDIGI don't send any frame.

Now consider the following example:

A station (IW3FQG) send a frame like this:

IW3FQG>APRS,IR3VIF*, IR3VIG, IR3FAL

IR3FAL is a digipeater that heard this frame. Without preemption it don't

repeat this frame because first must be digipeated by IR3VIG.

With preempt ON, and IR3FAL in PREEMPTCALLS, IR3FAL digipeater pre process

this frame like this:

IW3FQG>APRS,IR3VIF*,IR3FAL

and then repeat the frame:

IW3FQG>APRS,IR3VIF,IR3FAL*

Optionally with PREEMPTADD the digipeater may add new path so the same frame with PREEMPTADD PATH1, PATH2 become:

IW3FQG>APRS,IR3VIF,IR3FAL*,PATH1,PATH2

## The Local Sysop interface of UIDIGI

To logon locally, connect the TNC to a computer via serial port.

Check the speed in the hardware jumper of TNC2, then using a terminal program open the com port with the following  communication parameter: <TNC speed > 8N1, local ECHO off.

To enter in the sysop interface use the follow command:

[ESC] C [ENTER]

Now you can send any of the following  command to the digipeater and get response.

To exit use QUIT command.

The sysop can connect directly the digipeater from the radio port or via the serial port.

In the first case (radio port) the sysop must send the password using the SYSOP command (using the

standard NETROM algorithm meaning password request will come as a random number representing the

number requested from the password).

Example: password: MYPASSWORD after the sysop command is issued via remote you will be prompted for something like 2 5 7 9. Your response would be YSWR

From the local port the comm. Port is N81(speed in jumper setting in TNC)

Press [ESC] c [Enter] to logon locally

The commands available to the SYSOP follow:

AUXDEST [address]

- Set the destionation address for the beacon with aux input data

AUXPATH [path]

- Set the path for the beacon with aux input data

AUXRATE [n]

- Set the interval  for the beacon with aux input data

BDL [bnum] [path]

- Set the beacon path (bnum = 1~3, path = AX.25 path)

BEACON [bnum] [n] (default 1-600 2-1800 3-3600)

- Set the beacon interval (bnum = 1 ~ 3, n = 0 ~ 65535 sec)

BEACONOFFSET [bnum] [n] (default 1-0 2-0 3-0)

- Set the beacon offset (bnum = 1 ~ 3, n = 0 ~ 65535 sec)

BTEXT [bnum] [text] (Up to 70 characters)

- Set the beacon text (bnum = 1~3, text = beacon text)

BUDLIST [callsigns]

- Set the callsign that will be ignored by digipeater (all SSID’s of base call)

CHECK [n] (1-65535msec) (default 18,000)

- Set the LinkCheck parameter

- Defines the SysOp No-Activity Period

CLOCK [hh mm ss]

- Set the internal clock of digipeater

CONNECT [call]

- Connect another station

DIGIALIAS [call]

- Set the digipeater alias

- Up to 6 characters

DIGICALL [call]

- Set the digipeater callsign

- Up to 6 characters + SSID

DIGIPEAT [n] (n=0,1) (default 1)

- Global Enable Flag for digipeater

- Enables (1) or Disables (0) Digipeating

EPATH [path]

- Set the path for East destination (SSID 10|14)

- Up to 8 hops

FRACK [n] (n=1-15sec) (default 5)

- Set the FRACK

- Frame Acknowledgement Timeout

HIGH [n] (n=0,1)

- Set ON the TNC2 Led (CON 0 o STA 1)

INFO [text]

- Show the digipeater info

- Up to 80 characters (text shown at initial beacon and on screen)

LED

- Show the state of the TNC2 Led’s

LOW  [n] (n=0,1)

- Set OFF the TNC2 Led (CON 0 o STA 1)

MAXFRAME [n] (n=1-7) (default 4)

- Set the MAXFRAME

- Limit of Unacknowledged Frames

MHEARD

- Show the last heard station list

- With MH _ the sysop can clear the heard list

NPATH [path]

- Set the path for North destination (SSID 8|12)

- Up to 8 hops

PERSISTANCE [n] (n=1-255) (default 64)

- Set the PPersistance

- Parameter for channel access timing

PREEMPT [n]

- Enables(1) or Disables(0) Preemted Digipeating

PREEMPTADD [calls]

- List of Callsigns (all SSIDs of base call) for Digi to add after preempt

PREEMPTCALLS [calls]

- List of Callsigns (all SSIDs of base call) for Digi to search for preempt

QUIT

- Disconnect from digipeater

RESET

- Reset the digipeater

RESPTIME [n] (n=0-6,000ms) (default 100)

- Set RESPTIME

- Mininimum delay imposed on acknowledgement packets

RESTART

- Restart the digipeater

- restart using parameters from PROM

RETRY [n] (n=1-127) (default 10)

- Set the RETRY

- Maximum number of retry attempts

SLOTTIME [n] (n=1-255 x100ms) (default 10)

- Set the SLOTTIME

- Selects Length of delay between channel access attempts

SPATH [path]

- Set the path for South destination (SSID 9|13)

- Up to 8 hops

SYSOP

- Start the authentication process to access to the sysop interface via radio link

With ? the digipeater show the result of sysop autentication

TEST

- Start the TEST of digipeater

TXDELAY [n] (n=1-255 x10ms) (default 10)

- Set TxDelay

UICHECK [n] (n=0-250sec) (default 28)

- Set number of seconds that digipeater use to ignore duplicate

- Minimum period between between digpeating of duplicate packets

UIDIGI [calls] (default relay, wide, trace)

- Set the generic calls (up to 8) that digipeater use to digipeat UI frame

UIDCSB [n] (default 1)

- Enable the call substitution in the frame addressed to callsigns defined in UIDIGI command

- Enables (1)/Disables(0) Callsign substitution for UIDIGI Calls

UIFLDFL [n] (default 0)

Se the UIFLOOD Algorithm flags:

bit 0 make call substitution after the last WIDEn-n digied

bit 1 insert callsign before WIDEn-n

UIFLOOD [call]

- Set the generic call that will be used in the flooding algorithm

- Up to 6 characters

UILOOP [n] (n=0-3) (default 3)

- Set the Loop Packet Suppression Interval Mask

0x01 = Do not repeat frame with source address equal to digipeater call or alias

0x02 = Do not repeat frame with the digipeater call in the already digipeated via list

UIPID [n] (n=1-255) (default 240)

- Enable frame filtering algorithm based on PID of frame (0 disables this feature)

UIQUERY [n] (n=0,1) (default 1)

- Set if digipeater reply to query (with beacon 1).

- Enables (1) or Disables (0) TNC response to ?APRS? query

UISSID [n] (n=0,1) (default 1)

- Set if digipeater process the incoming UI frame using destination SSID

- Enables (1) or Disables (0) SSID Digipeating mode

UITRACE [call]

- Set the generic call that will be used in the TRACE algorithm

- Up to 6 characters

UITRFL [n] (n=0,1) (default 0)

- Enables (1) or Disables (0) Callsign substitution after last TRACEn-n digipeated

UNPROTO [call]

- Set the beacon destination

- up to 6 characters + SSID (AX.25 destination address for UI beacons)

WPATH [path]

- Set the path for West destination (SSID 11|15)

- Up to 8 hops

Note: on all path or callsign command (BDL, BTEXT, BUDLIST, DIGIALIAS,  DIGICALL, EPATH, MHEARD, NPATH, PREEMPTADD, PREEPTCALL,  SPATH, UIDIGI, UIFLOOD, UITRACE, UNPROTO, WPATH) to delete all the strings stored use the character '_'

Eg: NPATH _ <enter> delete the contents of NPATH.

Special case MHEARD where, MHEARD _ clear the heard list

## The Remote Sysop interface of UIDIGI

In order to logon remotely the sysop must fiest connect to the digipeater.

After the connection is established  the sysop must send  the SYSOP command and get the response from the digipeater.

UIDIGI use standard NETROM algorithm for authenticate a user as SYSOP.

This means that  password request will come as a random number representing the number requested from the password).

Example:

if the password stored in the eprom is MYPASSWORD after the sysop command is issued via remote you will be prompted for something like 2 5 7 9. Your response would be YSWR

## The User interface of UIDIGI

A user can connect the digipeater to get some information about digipeater (location power or other info that the sysop or get heard list.

In order to logon remotely the sysop must fiest connect to the digipeater.

After the connection is established  the user can send Info or Mheard command to see the information or the heard list.

## The heard station list

UIDIGI store up to 100 station callsign heard in the radio port. If a station was heard via digipeater the callsign is followed by an asterisk. Each callsign is followed by the time when was heard, the time value is taken from the internal clock value of the digiperater. If the UIDIGI clock was not set by the sysop the all time values probably is wrong.

The sysop may use the command MHEARD _ to cleard

## Frame duplication check

The digipeater on every digipeated frame store the hash value for that frame in a circular buffer with 128 slot.

It never repeat an equal frame within the period defined in the parameter UICHECK

Since the digipeater perform call substitution on generic call (RELAY, WIDE, TRACE ... up to 8 and sysop definable) the digipeater do not repeat frame with source address equal to digipeater call or alias (its beacon) and

also do not repeat frame with the digipeater call in the already digipeated via list.

## The beacon timers handling

With:

Beacon1Interval=300, Beacon2Interval=600, Beacon3Interval=900

Beacon1Offset=0, Beacon2Offset=0, Beacon3Offset=0

At  00:05:00  (Beacon1Interval+Beacon1Offset expire) Beacon 1 will be sent

At  00:10:00  (Beacon1Interval+Beacon1Offset expire and

Beacon2Interval+Beacon2Offset expire and )

Beacon 1 and Beacon 2 will be sent

At  00:15:00  (Beacon1Interval+Beacon1Offset expire and

Beacon3Interval+Beacon3Offset expire and )

Beacon 1 and Beacon 3 will be sent

At  00:20:00  (Beacon1Interval+Beacon1Offset expire and

Beacon2Interval+Beacon2Offset expire and )

Beacon 1 and Beacon 2 will be sent

At  00:25:00  (Beacon1Interval+Beacon1Offset expire) Beacon 1 will be sent

At  00:30:00  (Beacon1Interval+Beacon1Offset expire and

Beacon3Interval+Beacon3Offset expire and )

Beacon 1 and Beacon 3 will be sent

With:

Beacon1Interval=1200, Beacon2Interval=1200, Beacon3Interval=1200

Beacon1Offset=0, Beacon2Offset=300, Beacon3Offset=600

At  00:20:00 (Beacon1Interval+Beacon1Offset expire) Beacon 1 will be sent

At  00:25:00 (Beacon2Interval+Beacon2Offset expire) Beacon 2 will be sent

At  00:30:00 (Beacon3Interval+Beacon3Offset expire) Beacon 3 will be sent

At  00:40:00 (Beacon1Interval+Beacon1Offset expire) Beacon 1 will be sent

At  00:45:00 (Beacon2Interval+Beacon2Offset expire) Beacon 2 will be sent

At  00:50:00 (Beacon3Interval+Beacon3Offset expire) Beacon 3 will be sent

## The configuration of UIDIGI

The UIDGCFG program must be used to change the values of the default parameter stored in the in the image of the EPROM.

To change the parameter first you need to edit the UIDIGI.TXT file with a simple text editor.

Then you must process it with the UIDGCFG.EXE program that stores the configuration in the

destination binary image.

Proceed as follow.

In the file of distribution there is the following files:

UIDIGI.BIN

UIDIGI.TXT

UIDGCFG.EXE

Copy these 3 lines in any directory of your computer disk.

Rename the UIDIGI.BIN and UIDIGI.TXT with the name of the digipeater that you are

setting up (e.g.: IW3FQG).

Now the files have the names:

IW3FQG.BIN

IW3FQG.TXT

UIDGCFG.EXE

With a text editor changed line of the IW3FQG.TXT line following the indication

written in the file.

You compile the file with the command:

UIDGCFG IW3FQG.BIN IW3FQG.TXT

If there are no problems the program won't give any warnings, if there are problems it will show it.

Now you can program the EPROM with the binary image stored in the IW3FQG.BIN.

Install the EPROM in the TNC2 and turn it on.

If everything is ok, the digipeater emit a beacon.

Try it.

## The UIDIGI author

This text is not written with the intent of furnish a complete and detailed manual  for the inexperienced user or sysop.

The program comes furnished without no warranty and the author can decide to alter it any time or drop the free distribution at any moment.

If anybody finds any error or has any good idea or hint (not personal adaptations please!) send me a letter or WRITTEN message, NO SPOKEN Messages (radio, telephone, intercom.).

Please before write me directly read this manual, the FAQ’s, read the messages or  post a message in the UIDIGI mailing list: http://www.egroups.com/group/uidigi. Thanks.

My address is:

Marco Savegnago

Stradella Ospedale 87

36100 Vicenza

Italy

In Packet-Radio:

IW3FQG@I3KUH.IVEN.ITA.EU

In Internet:

iw3fqg@amsat.org

Coordinate:

Latitude: 	45 33’ 24” N

Longitude: 	11 32’ 34” E

## Where to find the software

UIDIGI archives may be downloaded following the link that you can find at

the following URL:

http://space.tin.it/computer/msavegna/uidigi.htm (best)

or in the files section of UIDIGI mailing list

## UIDIGI mailing list

If you want to be informed about news and talk with other UIDIGI sysops subscribe to

the UIDIGI mailing list at:

http://www.egroups.com/group/uidigi

## Legal trademark

APRS is a register trademark of Bob Bruninga WB4APR

## Copyright of UIDIGI

The UIDIGI program and its components are freely copyable and dispensable in any form = but only for amateur radio purposes and only where no profits or financial gain are involved.

Is also forbidden to use this program for demonstrations in commercial private and public business, institutional, towns, regional, state and other governments.

Any type of commercial exploitation not expressly authorized from the author is considered forbidden!

No responsibility is attributable to the author if the program during its execution causes the loss of data or malfunctions of the computer.

Copyright (c) 2000 Savegnago Marco. All right reserved.

## Free contribution

This is a free (but copyrighted!) software project. However if you find it useful and you want to encourage the author in future enhancement a free contribution is always accepted!!

# Special thanks

Thanks:

Greg Noneman WB6ZSU for the support in debugging, testing and for the original UIDIGI command spreadsheet

Allan Sadowski AH6LS for checking and correcting my original version of this text

Mark Cheavens KC5EVE for the correction and changes on the manual

All other who report bug, suggestion.

## Fonts and bibliographic references

-      AX.25 Link Access Protocol for Amateur Radio version 2.2 november 11 1997 TAPR

- Implementation of AX25 in the NOS program of KA9Q

- Implementation of AX25 of the TheFirmware program of the group German NORTH><LINK

- Tanenbaum, Andrew S., "Networks Computer"  1st ed Prentice-Hall 1981.

- Tanenbaum, Andrew S., "Computer  Networks" 3st edPrentice-Hall  1996.

- X.25 MADE EASY Nicolas M.Thrope, Derek Ross Prentice-Hall 1992.