NAME

mem_alloc – allocate a block of memory

SYNOPSYS

success/memory = mem_alloc(size)

C    DPTR	   =           ACC

FUNCTION

This function is used to allocate blocks of memory in XDATA RAM.

The function internally keeps a list of allocated blocks. On call, it searches through the list and tries to fit the requested blocksize between each successively allocated area.

INPUTS

size – number of 256 byte blocks to allocate

RESULTS

success – when CARRY flag is SET, the allocation failed and memory contains useless data.

memory – pointer to the allocated address space.

BEWARE

mem_alloc() relies on the presence of the full 64kbyte of XDATA space on hardware.

mem_alloc() is NOT reentrant, therefore avoid use in both application level and interrupt level at the same time. To solve this problem, see mem_qalloc() and mem_qfill().

SEE ALSO

mem_init, mem_free, mem_qalloc

NAME

mem_init – initializes before use of alloc routines

SYNOPSIS

mem_init()

FUNCTION

This function must be called before use of any other mem_xxxx function.

This function allocates and initializes the space for its own bookkeeping. This takes three 256 bytes blocks of XDATA memory.

INPUTS

-

RESULTS

-

SEE ALSO

mem_alloc, mem_free, mem_qalloc, mem_qfill, mem_resize

NAME

mem_free – deallocate memory

SYNOPSIS

success = mem_free(memory)

C			 DPTR

FUNCTION

Free a block of memory, returning it to the pool from which it came.

mem_free() automatically remembers the number of blocks (size) that was requested in mem_alloc().

INPUTS

memory – pointer to the memory block to free

RESULT

success – when the CARRY flag is set, mem_free() failed. Probably because you tried to free allocs resources instead of your own.

NOTES

If a block is freed twice, or a wrong pointer is passed to mem_free(), the alloc routines will probably go insane and do unpredicable things to the list of allocated memoryblocks. It was either this or a very sloooow mem_free() routine. I chose the former solution.

SEE ALSO

mem_alloc()

NAME

mem_resize – resize an allocated block of memory

SYNOPSIS

success = mem_resize(memory,size)

C                    DPTR   ACC

FUNCTION

Resize a memoryblock that was already allocated.

It is both possible to enlarge and make it smaller, within the limits of where the next allocated block has internally been addressed.

INPUT

memory – pointer to the memory block to resize

size – new, requested number of blocks to be allocated.

RESULT

success – when the CARRY flag is set, mem_resize() failed.

NOTES

when a block is resized to zero blocks, that memoryblock is automatically freed (see mem_free())

SEE ALSO

mem_alloc(), mem_free()

NAME

mem_qalloc – quick allocate a block of memory

SYNOPSIS

success/memory = mem_qalloc()

C    	  DPTR

FUNCTION

mem_qalloc() fetches a fixed sized memoryblock from a pool, pre-allocated by mem_qfill().

The advantage is that there is no search for free memory space during time critical points like interrupts when memory is needed instantly.

The disadvantage is that the memory size is fixed to a size, defined by MEM_QALLOCSIZE during assembly, however, mem_resize() solves this problem.

INPUT

-

RESULT

success – when the CARRY flag is set, mem_qalloc() failed.

memory – pointer to the allocated address space.

NOTE

mem_qalloc() cannot be used until mem_qfill is used at least once.

mem_qalloc() is reentrant.

SEE ALSO

mem_qfill(), mem_qalloc()

NAME

mem_qfill – pre-allocate a stack of memory blocks

SYNOPSIS

success = mem_qfill()

C

FUNCTION

When mem_qfill() is polled during run-time, it ensures that a stack of memoryblocks stays available.

Between two mem_qalloc() polls, mem_qalloc() can very quickly service up to eight caller routines, in a sequential or/and reentrant order.

mem_qfill() can take some time when the list of allocated memoryblocks is growing. Therefore it is adviced to implement it in less time-critical portions of your code.

INPUT

-

RESULT

success = set when memory full

NOTE

the size of the memoryblocks to pre-allocate is defined in the MEM_QALLOCSIZE constant

SEE ALSO

mem_qalloc()