NAME

gsc_init() – initialize the use of gsc device

SYNOPSYS

gsc_init()

VOID gsc_init(VOID);

FUNCTION

initializes the gsc open counter.

NOTE

This routine must only be called once during initialization if the device is used.

BEWARE

If gsc_init() is called after the device is opened, all frames in the receive/transmit list will be lost but will remain allocated in XDATA memory.

NAME

gsc_open – opens the gsc device for usage

SYNOPSIS

success = gsc_open()

C

BOOL = gsc_open(VOID);

FUNCTION

this function opens the gsc device driver and initializes the necessary structures for operating this device.

NOTES

all calls to gsc_open() should have matching calls to gsc_close()!

INTERNAL

The created structures are a set of lists from which one defines the list of received frames and the other one defines the list of frames to be transmitted.

every frame will be attached to the list using a NODE structure.

INPUTS

-

RESULTS

success = set on failures

SEE ALSO

gsc_close(), lists.inc

NAME

gsc_close – concludes access to the gsc device

SYNOPSIS

success = gsc_close()

C

BOOL = gsc_close(VOID);

FUNCTION

this function informs the device that access to the gsc device, previously opened, has been concluded. The device may perform some cleaning operations.

NOTES

all calls to gsc_open() should have matching calls to gsc_close()!

INTERNAL

gsc_close() releases the transmit and receive list headers from allocation.

At this moment, the entire list of frames, connected to this header will be lost without freeing.  This means that they will unnecessarily take up XDATA memory.

INPUTS

-

RESULTS

success = set on failure

SEE ALSO

gsc_open(), lists.inc

NAME

gsc_ioctl/IO_ALLOCBUF

allocate a frame for gsc buffering

SYNOPSIS

memory/success = gsc_ioctl(IO_ALLOCBUF)

DPTR   C                   ACC

APTR/BOOL = gsc_ioctl(UBYTE)

FUNCTION

this function reserves a block of memory to store data.  The reserved memoryblock equals the GSC_MAXFRAMESIZE constant, which is defined during compilation.

INPUT

IO_ALLOCBUF - defines this function.

RESULT

success = set on failure

memory = pointer to memoryblock.

SEE ALSO

gsc_ioctl/IO_FREEBUF, gsc.inc

NAME

gsc_ioctl/IO_FREEBUF

free a frame from gsc buffer

SYNOPSIS

success = gsc_ioctl(IO_FREEBUF,memory)

C                   ACC        DPTR

BOOL = gsc_ioctl(UBYTE,APTR)

FUNCTION

this function frees a block of XDATA memory, that was formerly attached to the list of frames in the RX or TX buffer.

INPUT

IO_FREEBUF - defines this function.

memory – pointer to the memoryblock to be freed.

RESULT

success = set on failure

SEE ALSO

gsc_ioctl/IO_ALLOCBUF, gsc.inc

NAME

gsc_ioctl/IO_RESET - reset gsc hardware

SYNOPSIS

success = gsc_ioctl(IO_RESET)

C                   ACC

BOOL = gsc_ioctl(UBYTE)

FUNCTION

reset the gsc hardware. The baudrate used comes from the GSC_SPEED constant, defined during compilation.

a GSC_SPEED = NULL will result in external clocking for baudrate.

INPUT

IO_RESET - defines this function.

RESULT

success = set on failure

SEE ALSO

gsc_ioctl/IO_RX, gsc_ioctl/IO_TX

NAME

gsc_ioctl/IO_RX – setup gsc for reception

SYNOPSIS

success = gsc_ioctl(IO_RX)

C                   ACC

BOOL = gsc_ioctl(UBYTE)

FUNCTION

Setup the gsc hardware for reception. Internally the gsc driver works with DMA and GSC interrupts. These are prepared by this function.

INPUT

IO_RX - defines this function.

RESULT

success = set on failure

SEE ALSO

gsc_ioctl/IO_TX

NAME

gsc_ioctl/IO_TX – setup gsc for transmission

SYNOPSIS

success = gsc_ioctl(IO_TX)

C                   ACC

BOOL = gsc_ioctl(UBYTE)

FUNCTION

Setup the gsc hardware for transmission. Internally the gsc driver works with DMA and GSC interrupts. These are prepared by this function.

INPUT

IO_TX - defines this function.

RESULT

success = set on failure

SEE ALSO

gsc_ioctl/IO_RX

NAME

gsc_read – read frame from receive buffer

SYNOPSIS

memory/success = gsc_read()

DPTR   C

APTR/BOOL = gsc_read(VOID)

FUNCTION

gsc_read() will check if any node is appended on the gsc_rx_list.  If a frame is found, it is detached from the receive buffer and passed to the calling routine.

RESULT

success - set on failure

memory – frame from receive buffer

NOTE

if a frame is returned, it has to be released when finished processing. This has to be done with the gsc_ioctl/IO_FREEBUF functioncall.

SEE ALSO

gsc_write, gsc_ioctl/IO_FREEBUF, gsc_ioctl/IO_RX