serial port device driver

- - - - - - - - - - - - -

This serial port device driver – or LSC device driver – is specifically written for the 8Xc152 microcontroller and uses the following resources:

- DMA 0 + interrupt

- Serial interrupt

- Timer 1

This driver can operate at either a standard baudrate or at high speed, each with a dedicated clockfrequency in mind.

The standard speed with autobaud detection relies on a clockfrequency of 11.0592 Mhz.  This enables reliable communication on 1200, 2400, 4800, 9600, 14400, 19200, 28800 and 57600 baud with 1 startbit and 1 stopbit.

For high speed communication, a clockrate of 14.7456 Mhz is necessary. A baudrate of 230400 or 460800 is possible here.

When selecting one of these, datatrafic is configured with 1 startbit and 2 stopbits.

With this clockfrequency, the effective baudrate of the standard speed will be 1/3 times higher. (e.g. 57600 baud becomes 57600 * (1 + 1/3) = 76800 baud)

The driver does not use handshaking.

NAME

lsc_open() – open the lsc device for use

SYNOPSYS

success = lsc_open()

C

FUNCTION

Open the local serial channel as device. The serial communication is handled on an interrupt basis.

lsc_open() allocates the necessary buffer, defined in the constant LSC_BUFSIZE during assembly, as well for receive as for transmit.  The size is defined in number of 256 byte blocks.

after lsc_open(), the device is ready to use.

To stop using the GSC, make a gsc_close() call.

INPUT

-

RESULT

success – the CARRY flag is SET on failure.

SEE ALSO

lsc_close()

NAME

lsc_close – close the lsc device for use

SYNOPSIS

success = lsc_close()

C

FUNCTION

Close the local serial channel device driver, disable its interrupts, remove all allocated buffers.

receive and transmit buffers will be discarded. All unhandled data will be lost.

INPUTS

-

RESULTS

success – the CARRY flag is SET on failure

SEE ALSO

lsc_open()

NAME

lsc_ioctl/IO_RESET - reset lsc hardware

SYNOPSIS

success = lsc_ioctl(IO_RESET)

C                   ACC

BOOL = lsc_ioctl(UBYTE)

FUNCTION

Reset the lsc hardware. The baudrate used comes from the LSC_SPEED constant, defined during assembly.

a LSC_SPEED = NULL will result in an automatic baudrate detection.

This function does clears all buffers.

INPUT

IO_RESET - defines this function.

RESULT

success = set on failure

SEE ALSO

-

NAME

gsc_ioctl/IO_START – restart a paused dataflow

SYNOPSIS

success = lsc_ioctl(IO_START)

C                   ACC

BOOL = lsc_ioctl(UBYTE)

FUNCTION

start reception and transmission after pausing.

the internal buffers are not cleared.

INPUT

IO_START - defines this function.

RESULT

success = set on failure

SEE ALSO

gsc_ioctl/IO_STOP

NAME

lsc_read – read char from receive buffer

SYNOPSIS

char/success = lsc_read()

ACC  C

FUNCTION

lsc_read() checks the receive buffer for data.

The character read is returned per byte.

In case of buffer empty, the call will fail.

RESULT

success - set on failure

char – character read from receive buffer

SEE ALSO

lsc_write

NAME

lsc_write – write char to transmit buffer

SYNOPSIS

success = lsc_write(char)

C                   ACC

FUNCTION

lsc_write() writes a character for output to the transmit buffer.  The character is written per byte. In case of buffer full, the call will fail.

RESULT

success - set on failure

char – character to be written to transmit buffer

SEE ALSO

lsc_read