HARDWARE

1

DEVICES

ACQUAINTANCE

2

Communication hardware and software are programmed to be accessed as device drivers.  Using these drivers is similar to programming UNIX device drivers, but adjustments have been made to be able to provide a fast and flexible microcontroller alternative to this filosophy.

All devices can be driven as character devices.  Character devices offer a flow of data that must be read or written in order.  Once performed, the read or write is irreversible. For the users point of view, programming a character device is easiest because the driver does autonomous management of all buffers and queues.  These are basically the only function calls involved to drive a character device:

dev_open()

dev_close()

dev_read()

dev_write()

Additionally, devices can be driven as a block device if the hardware or the concerning communication protocol allows this.  Block devices expect data not per character but as a whole block at once.  This approach enables a flexible – but more difficult management of the data before writing to or after reading from the device driver.  Data blocks used as output need to be pre-allocated by the user.  Data blocks read from the device need to be freed by the user.  These functions are needed:

devb_write()

devb_read()

devb_process()

Some devices need controls to perform device specific tasks.  This must not be done directly to the hardware by the user, but by making use of this call:

dev_ioctl()

Beside the device specific ones, dev_ioctl() supports these standard controls:

CMD_RESET, CMD_UPDATE, CMD_CLEAR, CMD_STOP, CMD_START, CMD_FLUSH

NAME

dev_open – opens the device

SYNOPSIS

result = dev_open()

PSW

BOOL = dev_open(VOID)

FUNCTION

This function opens the device driver for use by allocating memory, initialising structures and queues for internal use. The device will be put in a default state, most common for standard use.

NOTE

Device functions, other than dev_open() and dev_close() *WILL NOT* check the device on its status and rely on a constructive and well-considered programming style. Using a device without opening it first will give unpredicable behaviour of the microcontroller.

INPUTS

-

RESULTS

result(C) is set on failures

SEE ALSO

dev_close()

NAME

dev_close – closes the device

SYNOPSIS

result = dev_close()

PSW

BOOL = dev_close(VOID)

FUNCTION

This function closes the device driver for use by deallocating memory and removing internal queues. The device hardware and interrupts will be disabled if possible.

INPUTS

-

RESULTS

result(C) is set on failures

SEE ALSO

dev_open()

NAME

dev_read – read character input from device

SYNOPSIS

char/result = dev_read()

ACC  PSW

UBYTE/BOOL = dev_read(VOID)

FUNCTION

This function causes one character from the incoming stream to be read.

INPUTS

-

RESULTS

result(C) is set on failures

result(F0) is set if stream closed

result(F1) is set if new stream initiated

SEE ALSO

dev_write()

NAME

dev_write – write character output to device

SYNOPSIS

result = dev_write(char,flags)

PSW                ACC  PSW

BOOL = dev_read(UBYTE,BOOL)

FUNCTION

This function writes one byte to output stream.

INPUTS

char is the character to be output

flags(F0) is set to indicate a block close

flags(F1) is set to indicate a new block

RESULTS

result(C) is set on failures

SEE ALSO

dev_read()

NAME

devb_read – read block from device

SYNOPSIS

block/size/result = dev_read()

DPH  R0-R1 PSW

APTR/UWORD/BOOL = dev_read(VOID)

FUNCTION

This function causes a block from the incoming stream to be read. The block always defines a unit, clearly marked out by the underlying hardware or protocol.

NOTE

All blocks, read are internally pre-allocated by the device driver during operation.  It is up to the user to free this memory using the mem_free() function call.

INPUTS

-

RESULTS

result(C) is set on failure

block is the high-byte of the pointer to a block

size returns a bytesized count of the block

SEE ALSO

devb_write(), devb_process(), mem_free()

NAME

devb_write – write block to device

SYNOPSIS

result = dev_write(block,size)

PSW                ACC   R0-R1

BOOL = dev_read(APTR,UWORD)

FUNCTION

This function writes one block of bytes to the output stream of a device.  By the device, the block is concidered as being one unit and will be passed this way to the underlying hardware or protocol.

NOTE

The memory containing the block must be pre-allocated by the user with either a mem_alloc() or a mem_qalloc() function call. the device driver will free this memoryblock autonomously after being processed.

INPUTS

block is the high-byte of a pointer to the block

size is a bytesized count of the block

RESULTS

result(C) is set on failures

SEE ALSO

devb_read(), devb_process(), mem_alloc(), mem_qalloc()

NAME

devb_process – processes queues and drives hardware

SYNOPSIS

devb_process()

VOID = devb_process(VOID)

FUNCTION

This routine must be called to process blocks and block queues during the execution of the user application.  It is a low-priority task that is best performed during the main() loop.  Depending on the device implementation, the devb_process() routine can process a single character, a whole block or an entire queue. Most implementations will perform a per block process only.

INFORMATION

During development of the devices strategy, devb_process() was chosen to be the least time consuming solution.  The only good alternative was to implement a multitasking environment.  There’s no need to tell what the consequences are for such solutions.

INPUTS

-

RESULTS

-

SEE ALSO

-