Mini ARDF transmitter for 144MHz

Rik Strobbe  ON7YD

Villadreef 14   B-3128 Baal   BELGIUM

e-mail : rik.strobbe@fys.kuleuven.ac.be

General

This is a easy to build and low cost 144MHz mini transmitter that produces a remarkable 'clean' 144MHz signal up to 15 mW. It can also be used as exciter to drive a PA. In combination with a good UHF transitor a 2 transitor 200mW TX can be build or in combination with one of those power modules that are used in handies several Watts can be produced. With a low cost 24MHz xtal the oscillator runs on the 3rd overtone (72MHz). The collector circuit acts as a doubler. In this type of oscillator the xtal oscillates well above it's natural frequency, resulting in a end frequency of +/- 144.070MHz for a 24MHz xtal. Any other 72MHz xtal can be used, but a lot of these xtals are 5th overtone. In this case L1 should be reduced to 0.47µH to avoid that the xtal will run on the 3rd overtone. The oscillator is modulated with a mixture of AM and FM, so both AM or FM receivers can be used.

Technical

The oscillator is built arround T1, with a 24 MHz xtal used on the 3rd overtone (72 MHz). The collector circuit acts as a frequency doubler and the critical coupled double band pass filter (L2, L3, C3, C4) makes sure that all (sub)harmonics are at least 40 dB below the carrier. C3 and C4 have to be adjusted for maximum output power. With R2 the output power of the oscillator can be adjusted between 0.1 and 15 mW. The 2 Schmitt-Trigger NAND gates of the 4093 produce a intermitted LF tone that is used to modulate the oscillator. The remaining gates are disactivated by pulling their inputs to plus. With  R6 the modulation level can be adjusted.

Partlist

IC1 : 4093 (CMOS)	T1 : 2N918 (*1)		T2 : BC557c		X1 : 24MHz

L1 : 1µH		L2/3 :  5 wind. (*2)	R1 : 39k			R2 : 1M

R3 : 10k			R4 : 330k		R5 : 1M			C1 : 47p

C2 : 15p			C3/4 : 3-20p		C5 : 1n			C6 : 47n

C7 : 470n		C8 : 100n

*1 : if you have problems finding the 2N918 you can try any other NPN transistor with a Hfe > 20 and a transit frequency of at least 400MHz. A quick check in a databook gave following transistors that fullfill these requirements and are also pin to pin compatible with a 2N918 :

BF155, BF166, BF255, BF271, BF272, BF290, BFJ77, BFJ78, BFJ79, BFW30, BFW41, BFW70, BFX18, BFX19, BFX20, BFX21, BFX47, BFX59, BFX73, BFX89, BFY66, BFY79, BFY90

Be aware that I just compared this types in a databook. They will most likely do the job, but no guarantee.

*2 : L2 and L3 = 5 wind. of 1mm CuAg wire, diameter 5mm, tapped at 1 winding from the cold end.

Specifications (Output power versus supply voltage and current)

| supply voltage | supply current | output power |
| --- | --- | --- |
| 15 V | 14 mA | 15 mW |
| 12 V | 10 mA | 10 mW |
| 9 V | 9 mA | 6 mW |
| 9 V | 4.5 mA | 3 mW |
| 9 V | 2.5 mA | 1 mW |

The above frequency spectrum shows the output signal measured with a Rohde&Schwarz FSEA for a supply voltage of 9 V and a current of 9 mA. The PCB (mirror !) and component layout are given on a 1/1 scale. The dimensions of the PCB are 50.8 x 38.1 mm (2.0 x 1.5 inch).

Publication

The publication of this article and/or parts of it (eventually as part of another article) in radioamateur magazines is allowed under conditions that my name and callsign are mentioned and that one copy of the magazine is send to :

Rik Strobbe   ON7YD   Villadreef 14   B-3128 Baal   BELGIUM

73, Rik  ON7YD