ARDF TIMER [ON7YD, 04.96]

1

PartList

IC1 : PIC16C54XT	IC2 : 78L05

Q1 : 3.2768MHz		T1/2 : BC547c

C1/2 : 22p		C3/4 : 100n

C5/6 : 100p		C7/8 : 100n

C9 : 10F/10V		R1 : 470

R2/3 : 4k7		R4/5 : 3k3

R6/7 : 1k8		R8 : 8x47k

S1 : DIP sw. 8p.	D1/2 : LED 3mm

DIP switch selections (S1)

Identification				1 2 3

MOE (1. min. of 5 min. cycle)		0 0 0

MOI (2. min. of 5 min. cycle)		1 0 0

MOS (3. min. of 5 min. cycle)		0 1 0

MOH (4. min. of 5 min. cycle)		1 1 0

MO5 (5. min. of 5 min. cycle)		0 0 1

MO  (continue)				1 0 1

MOE (1. min. of 2 min. cycle)		0 1 1

MOI (2. min. of 2 min. cycle)		1 1 1

Delay (value of each switch, add values)

4 : 30 min.	5 : 1 hour	6 : 2 hours

7 : 4 hours	8 : 8 hours

Functional discription

This timer controls ARDF transmitters in accordance to the international ARDF rules. It can be used with 3.5MHz and 144MHz transmitters.

Although it is small (PCB is 48x36mm), low cost (450BF = 22DM = 15USD) and has a low power consumption (less than 5mA at 9 to 15V) it is a complete ARDF timer.

Identification (S1-1/3)

In addition to MOE-MO5 in a 5 minute cycle and MO (continious), a MOE and MOI in a 2 minute cycle is available (for training purposes).

Delay (S1-4/8)

A delay time up to 15.5 hours is programable. The delay starts after synchronisation and during this delay the timer will not drive the TX. This allows the organiser to hide the transmitters up to 15.5 hours prior to the first start, keeping the TX positions hidden for the competitors and saving the batteries.

RESET

After reset (pull RESET to ground) the timer will start with the active minute of the programmed TX, the delay-time is ignored.

Switching the power on will have the same effect as reset. The processor reads the DIP-switches after reset (or power on), later changes of the switches will NOT AFFECT the programming !

SYNC

This input can de used to synchronise multiple timers. Pulling the SYNC input to ground for at least 1 second will 'alert' the synchronisation. The timers are synchronised when the ground connection is interrupted !

After synchronisation the SYNC input is disabled, so in order to permorm another synchronisation the timer has to be reset (or switched off).

TX

This output is pulled to ground when the TX should be active, otherwise it is 'open'. The voltages applied to the TX or KEY output should not exceed 30V, the current should not exceed 50mA.

KEY

This output is pulled to ground when the TX should be 'key-down' (3.5MHz) or modulated (144MHz), otherwise it is 'open'.

D1

Diode 1 has a double function. During synchronisation it will light as soon as SYNC is pulled for at least 1 second (the timer is ready for synchronisation). During the 'delay time' D1 will light during the first second of the active minute, this allows a control of the synchonisation.

D2

When the TX is active D2 will light in the rythm of the modulation. If the voltage (from the TX) applied to KEY exceeds 15V a diode (eg.1N4148) in series with D2 is recommended.

+12V

The timer needs a supply voltage between 9 and 15V, the power consumption is less than 5mA (no LEDs lighting).