TUNE UP

1. Installation test procedure sequence. 
2. Choose a test frequency close by 2182 KHz, and notes the values on a form.
3. Put the antenna switch into position TEST, and the transmitter into treated power test frequency.

4. As a starting punt use the largest value of tuning capacitor C101 ( the upper position ).

5. Push on the TUNE button on the transmitter and check the transmitter's antenna current, tuning can be set to max. aerial current. If it is not possible to set into max. antenna current, try a smaller value of tuning capacitor C101 (a lower position) until max. antenna current is achieved.
6. When the transmitter can be tuned in the dummy antenna, and a strong tone is heard from the speaker, refresh mode is executed. The procedure is refit, and antenna switch is in position TRANSMIT.LuisterenFonetisch lezen
 

Woordenboek - Gedetailleerd woordenboek weergeven
7. As this "SAILOR" send blocked, so that 2 tone alarm signal can only be used on 2182KHz, the obstruction must remove by shorting two contacts on the push button marked ALARM.Luisteren

Fonetisch lezen

 

Woordenboek - Gedetailleerd woordenboek weergeven
