# TECHNICAL DATA

## The exciter S1303/04 delivers USB signals an the displayed frequency.

As option a USB / LSB change-over unit can be built into the exiter S1303/04.

This unit changes yhe transmitted sideband from the upper to the lower or opposite.

Frequency ranges :	MF: 1.6 – 4.0 MHz

HF: 4.0 – 8.5MHz

## The maritime bands 12, 16 , 22 and 25MHz

As option the frequency range can be extended wby 2 extra frequncy bands in the frequency range 8.5 – 10.0MHz and 11.5 – 28MHz.

Frequency transmitted :	Any frequency inside the frequncy range1.6 – 8.5MHz and the maritime bands 12, 16, 22 and 25MHz. (resolution 100Hz).

The exciter S1304 operates only on the 256 programmed frequencies.

Frequency stability :

Temperature range 0ºC to 40ºC :	Less than +/-0.1% (+/- 25Hz)

Long term stability :	Less than +/-0.1% (+/- 25Hz) per year

Short term stability :	Less than +/- 2Hz

A better frequency stability can be obtained as option

Mode operation :	A3H (H3E), A3A (R3E), A3J (J3A), A2H (H2A), A1 (A1A)  and TELEX (F1B and F1C).

Distress call :	Automatic A3H (H3E) on 2182KHz

Two- tone- alarm: 1300 and 2200Hz with a duration of 45 sec.

Output power :	50mW PEP / 50ohm.

Output power reduction :	Four 5 dB steps.

Modulation 8W :	350 – 2700Hz with compressor.

Operation temperature range :	-15ºC to 55ºC.