AH201-01  X20-Connector                                  Stand: 23.10.95

------------------------------------------------------------------------

X20-1   out  TxD/Disp Data, 4k8, 8 Data/1 Stop

X20-2   out  RX PLL Lock, org. Data/Conv

X20-3   out  org. Ring Tone

X20-4   in   via Multiplexer BCD Switch MSB, org. LS-Mode

X20-5   in   via Multiplexer BCD Switch LSB, org. HFS

X20-6   in   PTT

X20-7   in   RxD/Key Data, 4k8, 8 Data/1 Stop

X20-8   out  Multiplex Signal LSB, org. Hook free

X20-9   in   Test

X20-10  in   On/Off, Gnd = On

X20-11  in   Zndung

X20-12  out  +10 V

X20-13       Gnd, connected Pins 13 + 31 ???

X20-14       Gnd, connected Pins 14, 15, 16, 32, 33

X20-15       Gnd, connected Pins 14, 15, 16, 32, 33

X20-16       Gnd, connected Pins 14, 15, 16, 32, 33

X20-17       + 12 V, connected Pins 17, 18, 34, 35, 36

X20-18       + 12 V, connected Pins 17, 18, 34, 35, 36

X20-19  out  TX PLL is Lock, org. Ring Status

X20-20  out  Multiplex Signal MSB, org. HF/HS invers

X20/21       Audio down

X20/22       Audio up

X20/23       Gnd Gehuse

X20/24       Gnd Gehuse

X20/25       n. c.

X20/26       n. c.

X20/27       on/off hook indication

X20/28  in   + 12 V, connected Pins 28 + 30, zum Betrieb erforderlich,

schaltet Relais, wird spter ber Software gesteuert

X20/29       n. c.

X20/30  in   + 12 V, connected Pins 28 + 30, zum Betrieb erforderlich,

schaltet Relais, wird spter ber Software gesteuert

X20/31       Gnd, connected Pins 13 + 31 ???

X20-32       Gnd, connected PINS 14, 15, 16, 32, 33

X20-33       Gnd, connected PINS 14, 15, 16, 32, 33

X20-34       + 12 V, connected Pins 17, 18, 34, 35, 36

X20-35       + 12 V, connected Pins 17, 18, 34, 35, 36

X20-36       + 12 V, connected Pins 17, 18, 34, 35, 36

* END OF FILE *