In single voltage EPROMs two of the pins are power pins; one is +5V and the other ground. Single voltage EPROMs only require +5 to read them, but when programming they require a second voltage (Vpp). The old three-supply EPROMs require +5, +12 & -5Vdc just to read them. Yet a fourth voltage is required to program them! You no longer see three supply EPROMs in new products, but they are still out there in older equipment and are somewhat available for replacements. A good example is the three supply 2716 made by Texas Instruments, TI never made a 2716 single supply. There are also EPROMs that are pin compatible but do not have the same number such as the 2716 and the 2516 (T.I.'s version of a single supply 2716). But for those of you who program a lot of 2716's remember that T.I.'s 2716 are not the same as other manufactures.

Back to pins, we look at the OE (Output Enable) and CE (Chip Enable). The CE pin enables and disables the data output. When disabled most of the chip is in a low power sleep mode. The access time of a chip is given from the time CE becomes active until data appears.

The access time using the OE pin is a lot shorter than CE because the OE pin disables only the data output pins, but not the rest of the chip. This allows data to be accessed at a much higher speed at the cost of increased power consumption.

When programming, we use the Vpp line to supply the programming voltage to the EPROM. Other times, Vpp is kept at +5.

| Pin. 2716 Pin CPU Name Description 1 13 D3 Data 3 2 14 D4 Data 4 3 15 D5 Data 5 4 16 D6 Data 6 5 17 D7 Data 7 6 * A12 Address 12 7 19 A10 Address 10 8 n/c A11 Address 11 9 22 A9 Address 9 10 23 A8 Address 8 11 24 +5V +5 VDC 12 12 SG.ND Shield Ground | Pin 2716 Pin CPU Name Description 1 1 A7 Address 7 2 2 A6 Address 6 3 3 A5 Address 5 4 4 A4 Address 4 5 5 A3 Address 3 6 6 A2 Address 2 7 7 A1 Address 1 8 8 A0 Address 0 9 9 D0 Data 0 10 10 D1 Data 1 11 11 D2 Data 2 12 n/c GND Ground |
| --- | --- |

* to inverter and back to 18 for chip select

Top

D3   D4   D5   D6   D7   A12  A10  A11  A9   A8  +5V   SGND

--1- --2- --3- --4- --5- --6- --7- --8- --9- -10- -11- -12-

GND  D2   D1   D0   A0   A1   A2   A3   A4   A5   A6   A7

Bottom