When connected to the node, most commands will be familiar to folks who used packet in the past.
Node commands (can be uppercase or lowercase):
C -  Connect to a node or destination.
H – for a longer and more detailed list of commands.
N - list Nodes (NetRom nodes).
D - list Destinations (Flexnet nodes).
Q - quit or leave a Node or Destination.
M – stands for “Mail” which will connect you to the BBS.
B – stands for “Bye” to leave the BBS.
CHAT – enter the chatroom.  Any number of hams can be connected for a group chat or conference.
The BBS has its own commands which you can view by sending “H” (upper or lowercase, no quotes).
Chat also has its own commands which uses a / preceding the command.  To view all of the commands after connecting to Chat, send “/H” (upper or lowercase, no quotes).
The AA2YG-1 node combines two different protocols:  NetRom and Flexnet.   The node-control software being used is (X)Net and BPQ.  The majority of the node is controlled by (X)Net and “speaks” both NetRom and Flexnet.  BPQ is used mainly for it’s internal BBS.
There’s two types of node; NetRom nodes which are called simply “Nodes”, and Flexnet nodes which are called “Destinations”.  The majority of nodes will be in the Destination list.  You can connect to any node in the Node or Destination lists.
A word about nodes in general… manual node-hopping was useful and fun back in the day, for example to node-hop a couple nodes away and connect directly to a friend’s hardware TNC and leave mail for them.  But with so many nodes being linked via internet today, it’s not really necessary or practical.  (X)Net will automatically find a route and connect to any node that is in the network, so manual node-hopping isn’t needed.  Also, most folks are using BBS’s on their local node for mail instead of their own TNC’s mailbox in their house, so manually node-hopping to them isn’t necessary.  This node, and the KP3FT node it is linked to, are strictly RF with no internet links, but some of the other nodes in the network we are linked to do use internet links, so the Destination list may be long at times.
