KPG-28D Help File Version 1.00

File name: KPG-28D.HLP

06-17-1996

Y57-3220-20

Modification History:

Version     Release Date     Description

1.00	06/17/'96	KPG-28D First Version

1General Help

*** [ General Help ] ***

Along the top border of the main program screen (Channel Information) are

the "drop down" menus. Press [ALT] and use the arrow keys --> <-- to select

program section. Help is available for each menu by pressing the F1 key.

Pressing F1 will load it's Help screen. To get familiar with the

computer key functions used in the KPG-28D, press [ALT] now; select Keyboard

Help; press [ENTER].  Within a Help screen use the cursor up/down keys to

scroll, F5 to print a hard copy, and [ESC] or [ALT] to escape the screen.

Refer to the KPG-28D Instruction Manual for additional help.

**For Basic Instructions (use F1 for Help in each program section below)**

A. [CREATING PROGRAMMING DATA FOR A RADIO]

1. Open the "Model" menu;

Set the appropriate model type.

2. Open the "Edit-Panel Key Assignment" menu;

Assign functions to each key on the panel.

3. Open the "Edit-Mic Key Assignment" menu;

Assign functions to each key on the DTMF mic.

4. Open the "EDIT-Scan Information" menu;

Set the SCAN features & parameters.

5. Open the "EDIT-Optional Features" menu;

Set the general built-in features.

6. Open the "Edit-System Parameters" menu;

Set the System features & parameters.

7. Open the "Edit-Advanced Options-Decode Formats" menu;

Set the Decode Format features & parameters.

8. Open the "Edit-Advanced Options-Encode Formats" menu;

Set the Encode Format feature & parameters.

9. Open the "Edit-Advanced Options-Status Message" menu;

Set the Status Message feature & parameters.

10. Open the "Edit-Advanced Options-Queue Message" menu;

Set the Queue Message feature & parameters.

11. Open the "Edit-Advanced Options-Kenwood Toneset" menu;

Set the Kenwood Tone feature & parameters.

12. Hit ESC. the cursor will move to the main screen, "Channel Information

Screen"

B. [Test Mode]

Open the "Program-Test Mode" menu;

Connect the radio correctly to measurement equipment.

Set specific radio adjustments.

C. [PROGRAM A RADIO]

1. Open the "Program" menu.

2. Select "Write to Radio".

D. [SAVE PROGRAMMING DATA TO DISK FILE]

1. Open "File" menu.

2. Select "Save" for quick save to the "default" or "current" filename,

or "Save As" to name or rename the file.

*****Organizational chart of the KPG-28D*****

[ MAIN SCREEN ]

+- CHANNEL -+- Receive & Transmit Frequency

|           +- QT/DQT Decode & Encode

|           +- Channel Name (Alphanumeric Display Information)

|           +- Power Select

|           +- Frequency Shift

|           +- Local/Distance

|           +- Wide/Narrow

|           +- Auxiliary

|           +- Scan Include List

|           +- System Reference

|           +- Default TX Address

|           +- Primary RX Address

|           +- Secondary RX Address

|           +- Third RX Address

|           +- Voice Scrambler (Option)

|

+- FILE ----+- New

|           +- Load

|           +- Save  (=F4)

|           +- Save As

|           +- Delete

|           +- Print (=F5)

|           +- Go to Dos

|           +- Exit  (=F3)

|

+- MODEL ---+- Personality -+- Model Name

|                           +- Wide & Narrow

|                           +- Option Unit

|                           +- Scrambler Unit (Option)

|

+- EDIT ----+- Panel Key Assignment

|           |   (press,hold,shift,release)

|           +- Mic Key Assignment

|           |   (press,hold,shift,release)

|           +- Scan --------+- Priority Channel

|           |   Information +- Preference Channel

|           |               +- Look Back Time A

|           |               +- Look Back Time B

|           |               +- User Programming

|           |

|           +- Optional ----+- Time Out Timer (TOT)

|           |    Features   +- TOT Pre-Alert

|           |               +- TOT Rekey Time

|           |               +- TOT Reset Time

|           |               +- Control Tone

|           |               +- Warning Tone

|           |               +- Normal Display Format

|           |               +- Off Hook Decode

|           |               +- On Hook PTT

|           |               +- Off Hook Scan

|           |               +- Ignition Sense

|           |               +- Ext.Alert/Transfer

|           |               +- Lock Code

|           |               +- Keypad Operation

|           |               +- Number Entry Timeout

|           |               +- Overlay Text Channel

|           |               +- Overlay Text Selcall

|           |               +- Overlay Test Status

|           |               +- Mode -----------+- Main

|           |               |                  +- Self

|           |               |                  +- Panel Tuning

|           |               |

|           |               +- Start up -------+- Scan

|           |               |                  +- Monitor

|           |               |                  +- Encode Def.

|           |               |                  +- Channel

|           |               |

|           |               +- Opt MDT --------+- Data PTT with QT/DQT

|           |               |                  +- Data PTT with Mic Mute

|           |               |                  +- TX Sense

|           |               |

|           |               +- Rts Listen Period

|           |               +- Rts Active Period

|           |               +- Hold Delay

|           |               +- Start Del. Required

|           |               +- Digit Entry Method

|           |               +- Copy from Default TX

|           |

|           +- System ------+- System Name

|           |   Parameters  +- Selcall Toneset Standard

|           |               +- Receive Gap Period

|           |               +- Selectable Selcall Digits

|           |               +- Selectable Status Digits

|           |               +- Busy Symbol Function

|           |               +- Return-to-Standby Mode

|           |               +- Automatic Close

|           |               +- Monitor Function

|           |               +- Dropout Delay Time

|           |               +- TX Dwell Time

|           |               +- Scan Dwell Time

|           |               +- Scan Configuration

|           |               +- Group Digit Selectable

|           |               +- System Formats -+- Decode Format

|           |                                  +- Encode Button #1

|           |                                  +- Encode Button #2

|           |                                  +- Encode Button #3

|           |                                  +- Encode Button #4

|           |                                  +- Encode Button #5

|           |                                  +- Encode Button #6

|           |                                  +- Encode PTT-press

|           |                                  +- Encode PTT-release

|           |                                  +- Encode DTMF Overdial

|           |                                  +- Encode DTMF Direct

|           |                                  +- Encode Channel Select

|           |                                  +- Encode On Hook

|           |                                  +- Encode Off Hook

|           |

|           +- Advanced ----+- Decode Formats

|           |    Options    +- Encode Formats

|           |               +- Status Message

|           |               +- Queue Message

|           |               +- Kenwood Toneset

|           |

|           +- Embedded Message

|               (64 characters max. electronic serial # etc.)

|

+- PROGRAM -+- Read From Radio

|           +- Write To Radio

|           +- Test Mode ---+- Test Data ------+- Test Data Read

|                           |                  +- Test Data Write

|                           |                  +- Test Data Load

|                           |                  +- Test Data Save As

|                           |                  +- Test Frequency Edit

|                           |

|                           +- Tuning ---------+- Test Channel

|                                              +- High Power

|                                              +- Low Power

|                                              +- Max Deviation

|                                              +- DQT Balance

|                                              +- QT Deviation

|                                              +- DQT Deviation

|                                              +- Sensitivity

|                                              +- Squelch level

|                                              +- 5TONE Deviation

|                                              +- DTMF Deviation

|                                              +- DQT Polarity

|

+- SETUP ---+- Communication Port

|           +- Display -----+- Display Back Color

|           |     Color     +- Input Data Color

|           |               +- Feature Highlight Color

|           |               +- Screen Border Information Color

|           |               +- Windows Menu Color

|           |               +- General Message Color

|           |               +- Windows Color

|           |               +- Help Window Color

|           +- Help Path

+- HELP ----+- General Help

+- Keyboard Help (Computer Keyboard)

4Keyboard Help

*** [ Computer Keyboard Help ] ***

Available key functions are displayed at the bottom of the screen.

ESC         : Returns to the main screen; called the Channel Information

Screen. (exits menus & sub-screens).

ALT         : "Go to menu" or "Go back" to previous screen.

Arrow Keys  : Moves cursor within a data input field; scrolls help screens.

PgUp & PgDn : Pages up/down General or Keyboard Help screens.

F1          : Opens a Help screen.

F2          : Toggles Yes/No feature selections or increases selection in

data fields. (same as [SPACE BAR])

Opens QT/DQT list when cursor is in QT/DQT field.

TX and RX control while in the Test and Tuning Mode.

F3          : Exits from this program and return to the DOS prompt.

F4          : Saves the current program data with current file name.

F5          : Prints the current data, Tuning data or Help screen.

F7          : Next page (same as page up).

F8          : Previous page (same as page down).

F9          : Channel Insert (inserts a blank channel row and moves the

current channel data & all subsequent channels to the next

channel row).

F10         : Opens Channel Edit when cursor is in Channel Information

field.

Shift+F2    : Opposite function as the [F2].

Shift+F3    : Copies an entire channel to another channel.

Shift+F4    : Copies an RX frequency column entry to a TX frequency column.

Shift+F5    : Copies the data of QT/DQT from a QT/DQT decode column to the

encode column.

Shift+F6    : Changes the IF mixer input Frequency to the upper or lower

mixer Frequency.

(Needs hardware modifications, see S/M for details)

Shift+F9    : Channel Delete (deletes the current channel data row and

go back all subsequent channel rows by one).

HOME        : Jumps to channel 1 of the Channel screen, or to page 1 of

a help screen.

END         : Jumps to the last channel(Ch 2 or Ch 250) of the Channel

screen. Or to the last page of a help screen.

ENTER       : Enters data into a program field, or opens a selected window.

SPACE BAR   : Toggles Yes/No feature selections, increments selection in

data files, or clears data. (same as [F2])

TX and RX control while in the Test Mode.

Shift+SPACE BAR : Opposite function to the [SPACE BAR].

5New File Help (CLEAR FILE)

The New function clears any currently loaded, read or created data

file.

All program data screens will be set to defaults.

5Load Help

The Load function is used to retrieve a stored data file from

a hard disk or floppy disk to be modified, viewed, written

(program a radio) or printed out.

A directory of stored files, sorted alphanumerically, will be

displayed for selection.

5Save Help

Saves the current program data with current file name.

5Save as Help

Saves with the option of assigning a "new" file name, or renaming

an existing file.

5Delete Help

The Delete function is used to remove any stored data files from

the directory of stored files.

5Print Help

Prints all radio programming data currently loaded into the program:

- Model Type

- Panel Key Assignment

- Mic Key Assignment

- Scan Information

- Optional Features

- System parameters

- Decode Formats

- Encode Formats

- Status Message

- Queue Message

- Kenwood Toneset

- Embedded Message

- Channel

6Go to Dos Help

Go back to the DOS prompt temporarily.

Type EXIT and press [ENTER] to quit the DOS prompt and return to the

KPG-28D program.

Note: The KPG-28D program can support this command only if your

computer has sufficient memory.

6Exit Help

Quits this program and return to the DOS Prompt.

6Model name Help

This parameter selects the model number of the radio to be

programmed.

Press the F2 key or SPACE BAR to alternate between selections.

6Wide & Narrow Help

Selects whether it is possible to switch between Narrow,

Wide5k & Narrow, Wide4k & Narrow or Wide5k & Wide4k.

However, note that even if this is set, switching is not possible

for item for which the transceiver does not correspond or for item

which are not remodeled.

When this setting has been switched, adjust the Deviation and

Balance for each setting.

Press the F2 key or SPACE BAR to alternate between selections.

6Option unit Help

Sets whether it is possible to attach option units such as the

mobile data terminal.

If set to "Yes", refer "Opt MDT" in Optional feature screen.

Press the F2 key or SPACE BAR to alternate between selections.

6Scrambler unit (option) Help

Select "SC-A" or SC-B".

If a Voice Scrambler unit is installed in the radio.

If one is not installed, select "No".

(See S/M for details)

Press the F2 key or SPACE BAR to alternate between selections.

6Panel/Mic Key Assignment Help

The key definition window allows assignment of key functions to each

of the key actions. Per button four key actions can be defined.

These key actions are press (executed immediately after pressing

the key), hold (executed after holding down the key),

shift (executed when the key is pressed after activation of the

shift function) and release (executed immediately after releasing

the key).

Press SPACE BAR or the F2 key to select.

Disable      : not assigned

0            : enter digit 0

1            : enter digit 1

2            : enter digit 2

3            : enter digit 3

4            : enter digit 4

5            : enter digit 5

6            : enter digit 6

7            : enter digit 7

8            : enter digit 8

9            : enter digit 9

AUX          : auxiliary on/off

AUX2         : auxiliary 2 on/off

Channel ent. : select channel entry mode  (*1)

Channel temp : temporary channel number display  (*1)

Clear all    : clear full dial buffer  (*1)

Clear digit  : erase one digit  (*1)

Code lock    : lock operation of radio by means of 4-digit code

Control tone : control tone on/off

Digit 1x     : 5th digit up  (*1)

Digit 10x    : 4th digit up  (*1)

Digit shift  : select next digit  (*1)

Digit up     : select digit up  (*1)

Display roll : queue buffer redisplay  (*1)

DTMF entry   : select DTMF entry mode  (*1)

Encode #1    : execute call format #1 (normal call)

Encode #2    : execute call format #2 (free-dialling using selcall)

Encode #3    : execute call format #3 (free-dialling using DTMF)

Encode #4    : execute call format #4 (long tone call)

Encode #5    : execute call format #5 (status call)

Encode #6    : execute call format #6 (emergency call)

Ext/Transfer : switch on/off external alarm and/or transfer function

Group digit  : enter digit A  (*1)

Hold delay   : change hold delay

Key lock     : lock keys for mic keys and channel knob (*3)

Message      : queue and status number/message  (*1)

Mode reset   : reset to default entry mode  (*1)

Mode select  : toggle between channel and selcall entry  (*1)

Monitor      : selcall+QT/DQT monitor on/off

Mon-QT/DQT   : QT/DQT monitor on/off

Mon-selcall  : selcall open/close

Mon-mute     : squelch mute

Mon-unmute   : squelch unmute

Mon-toggle   : squelch mute/unmute

Pri on       : start priority scanning  (*1)

Pri on/off   : toggle priority scan on/off  (*1)

Public ad.   : public address on/off

If you use Public Address function in 2ch model,

assign it to "2" key (either "press", "hold" or

"release").

"1" key and "3" key are automatically assinged as

Public Address volume Up and Down function.

Queue mode   : select queue mode  (*1)

Queue msg    : queue/stqtus  (*1)

Queue next   : select next queue entry  (*1)

Queue select : select next queue entry / default operation mode (*1)

Scan on      : start scanning  (*1)

Scan on/off  : toggle scan on/off  (*1)

Scan+pri     : toggle priority + scan on/off  (*1)

Scan+pri on  : start both scan and priority scan  (*1)

Scan prog    : go to scan programming mode  (*1)

Scrambler    : scramble select

SC/A confirm : beep tones indicate the status of the scrambler A

as follows:

Selcall ent. : select status entry mode  (*1)

Shift on/off : shift mode on/off

Status ent.  : select status entry mode  (*1)

Talk around  : talk around on/off  (*2)

Warning tone : warning tone on/off

(*1)This selection is not available for 2-channel models.

(*2)This selection is not available for Multi-channel models.

(*3)This selection is not available for Mic Key Assignment.

8Priority Channel Help

The 'priority' parameters selects the channel witch is watched

during operation of priority scanning.

Enter None when no priority channel is required.

Press the F2 key or SPACE BAR to alternate between selections.

8Pref. Channel Help

The 'preference' parameters selects the default channel used for

speech transmission by pressing the PTT during operation of scanning.

Press the F2 key or SPACE BAR to alternate between selections.

8Look Back Time Help

Priority channel checking intervals while receiving on a

non-priority channel.

The radios CPU chooses either Lookback time A or B depending on the

priority channel activity --Program both times.

Press the F2 ey or SPACE BAR to alternate between selections.

[Lookback A]

Priority channel is not busy (no carrier detected).

This time is typically set shorter than Lookback B, since the

priority ch. should be checked more often to prevent missing a call.

Range   : 0.3 - 5.0s in 0.1s steps

Default : 0.5s

[Lookback B]

The priority channel is busy, but tone squelched.

This time is typically set long because the priority ch. is busy

with another caller using a different tone.

Range   : 0.5 - 5.0s in 0.5s steps

Default : 2.0s

9User Programming Help

Determines whether the user is allowed program the scan list and the

priority channel for the current scan group(Enable) or not(Disable).

Press the F2 key or SPACE BAR to alternate between selections.

9Time Out Timer Help

The 'time out timer (TOT)' parameter selects the maximum period

of time the user is allowed to transmit continuously by pressing

the PTT button.

After this period of time the transmitter is automatically

turned off.

Range   : Off / 30 - 300s in 30s steps

Default : 60s

Press the F2 key or SPACE BAR to alternate between selections.

9TOT Pre-Alert Help

A separate timer used to emit a warning beep before the transmit TOT period

expires.  For example, if the TOT is set to 200 seconds and the TOT pre-alert

is set to 190 seconds, a warning beep will sound 10 seconds before the TOT

expires.  If you enter a value that is higher than the transmit time-out

value, no transmit warning will be generated.

Range   : Off / 10 - 250s in 1s steps

Default : Off

Press the F2 key or SPACE BAR to alternate between selections.

9TOT Rekey Help

The 'TOT rekey time' parameter selects the period of time the PTT is

inactive after expiring the transmit timeout.

This facility is useful to avoid continuous use of the radio channel

by one person only.

Range   : Off / 1 - 60s in 1s steps

Default : Off

Press the F2 key or SPACE BAR to alternate between selections.

9TOT Reset Help

Determines the period of time for the timeout timer (TOT) to reset

after the PTT button is released.

Range   : Off / 1 - 15s in 1s steps

Default : Off

Press the F2 key or SPACE BAR to alternate between selections.

9Control Tone Help

Beep tone at radio power up.

Keys and selector knob echo-back beeps.

Press the F2 key or SPACE BAR to alternate between selections.

9Warning Tone Help

Time-out timer & Busy ch. lockout alert tones.

Press the F2 key or SPACE BAR to alternate between selections.

10Normal Display Format Help

Determines the way in which the selcall information and channel

number is displayed.

If the display format is set to selcall, the transceiver will show

the selected selcall address in its default state.

If set to channel, the channel number will be shown instead.

When set to Both, the transceiver will show the channel number

as well as the selected selcall address.

Press the F2 key or SPACE BAR to alternate between selections.

10Off Hook Decode Help

The radio is still tone squelched even though the microphone is in

the off hook condition. (valid for QT and DQT decode)

Yes: Decode signaling is active even the microphone is in the off

hook condition.

No : Decode signaling is disabled during off hook, squelch works

as carrier squelch.

Press the F2 key or SPACE BAR to alternate between selections.

10On Hook PTT Help

Sets whether to transmit in an on-hook state.

Press the F2 key or SPACE BAR to alternate between selections.

10Off Hook Scan Help

If enabled-"Yes", the mobile is able to scan event though the mic is

off-hook.

Press the F2 key or SPACE BAR to alternate between selections.

10Ignition Sense Help

Sets ignition sense.

Off        : Disable

Power ctrl : The radio's power is controlled by ignition sense.

See the radio's service manual for details.

Press the F2 key or SPACE BAR to alternate between selections.

10Ext.alert/Transfer Help

The 'ext. alert / transfer' parameter selects if the external alert

output and/or call transfer (diversion) is used.

When activated, it is possible to select one or both of these modes

by the button assigned to it.

Press the F2 key or SPACE BAR to alternate between selections.

10Lock Code Help

The radio lock code is entered in this field.

The lock code is a four digit numerical code, which is used to

temporary lock operation of the radio by the user.

11Keypad Operation Help

The 'keypad operation' parameter selects the default use of the

numerical field of the keypad.

When 'DTMF' is selected, the required mode must be selected first.

When 'selcall', 'channel' or 'status' is selected, the entry mode is

switched automatically to that mode when pressing the first digit.

Press the F2 key or SPACE BAR to alternate between selections.

11Number Entry Timeout Help

The 'number entry timeout' parameter selects the period of time the

radio returns to the default operating mode after selection of one

of the available entry modes (i.e. channel entry mode).

Range   : 0 - 255s in 1s steps

Default : 10s

Press the F2 key or SPACE BAR to alternate between selections.

11Overlay Text Channel Help

The 'overlay text channel' parameter defines the overlay text for

the numerical channel readout mode.

The eight character string will be displayed in a 13-segment form,

which approaches the layout of the characters used as much as

possible.

11Overlay Text Selcall Help

The 'overlay text selcall' parameter defines the overlay text for

selcall address selection.

The eight character string will be displayed in a 13-segment form,

which approaches the layout of the characters used as much as

possible.

11Overlay Text Status Help

The 'overlay text channel' parameter defines the overlay text for

status number selection.

The eight character string will be displayed in a 13-segment form,

which approaches the layout of the characters used as much as

possible.

11Rts Listen Period Help

The RtS (Return-to-Standby) listen period selects the period of time

the user can observe channel activity before placing a call.

After this period of time, the radio closes itself again.

Only active when RtS mode in the system parameters screen is set to

'Enable'.

Range   : Off / 1 - 255s in 1s steps

Default : Off

Press the F2 key or SPACE BAR to alternate between selections.

11Rts Active Period Help

The RtS (Return-to-Standby) active period selects the period of time

the user can listen and/or respond to a conversation before the

radio closes itself.

Only active when RtS mode in the systems parameter screen is set to

'Enable'.

Range   : Off / 1 - 255s in 1s steps

Default : Off

Press the F2 key or SPACE BAR to alternate between selections.

12Hold Delay Help

The 'hold delay' parameter determines the period of time the

emergency switch or button must be pressed before the function

(ex. emergency procedure) is activated.

Suggested value: 1s.

Enter value Off to disable this delay.

Range   : Off / 10 - 2550ms in 10ms steps

Default : Off

Press the F2 key or SPACE BAR to alternate between selections.

12Start Del. Required Help

This parameter selects whether a valid sequential call needs

a preceding period of silence or not.

The minimum period of silence is equal to the receive gap length,

programmed per system group.

Press the F2 key or SPACE BAR to alternate between selections.

12Digit Entry Method Help

The 'digit entry method' selects if entry of digits for selcall

transmit overwrites the previous entered address (shift from right

to left), or is replaced first with the default transmit address

before overwrite.

Press the F2 key or SPACE BAR to alternate between selections.

12Copy From Default Tx Help

The 'copy from default tx' parameter selects if the keypad

selectable selcall address is overwritten each time when changing

channels (Enable) or not (Disable).

Refer "Default TX address" in channel edit screen.

Press the F2 key or SPACE BAR to alternate between selections.

12Startup Scan Help

Determines whether scanning is activated at transceiver switch-on or

not.

0 - 9  : Activated scan group at transceiver is power-on.

Press the F2 key or SPACE BAR to alternate between selections.

12Startup Monitor Help

The 'startup monitor' parameter selects if the radio is initially

open (Enable) or closed (Disable) for selective call operation.

Press the F2 key or SPACE BAR to alternate between selections.

12Startup Encode Def. Help

The 'startup encode definition' selects the encode format executed

when switching on the radio.

Press SPACE BAR  or the F2 key to select.

13Startup Channel Help

The 'startup channel' selects the channel selected when switching on

the radio.

Channel number : Startup channel.

Disable        : The channel at transceiver switch-off appears

again.

Press the F2 key or SPACE BAR to alternate between selections.

13Main Programming Help

Enable or disable the Main Programming function.

This data is for factory use only.

Select Disable before delivering the radio to the end user to avoid

unintentional changes of the RADIO's OPERATING SYSTEM.

(see S/M for details)

Press the F2 key or SPACE BAR to alternate between selections.

13Self Programming Help

Enable or disable the Self Programming function.

(see S/M for details)

Press the F2 key or SPACE BAR to alternate between selections.

13Panel Tuning Help

Enable or disable the Panel Tuning mode function (factory use only).

(see S/M for details)

Press the F2 key or SPACE BAR to alternate between selections.

13Data PTT with QT/DQT Help

This feature is useful for Data Communication, etc.

Select the radio's behavior when accessory PTT goes low.

Select "No" to avoid sounds data note on other parties radio for

data communication.

On  : Transmission with programmed QT/DQT when accessory PTT

changes to low.

Off : Transmission without QT/DQT when accessory PTT changes to

low.

Press the F2 key or SPACE BAR to alternate between selections.

13Data PTT with Mic Mute Help

Select Mic Mute On or Off when accessary PTT is activated for data

communication.

On  : Mic line is muted when accessary PTT is activated.

Off : Mic line is MDT muted.

Press the F2 key or SPACE BAR to alternate between selections.

13TX Sense Help

Select one of the following three output functions for data

communication.

Mic PTT   : This indicates the state of the microphone PTT.

Mic PTT on  : Low

Mic PTT off : High

PTT input : This indicates the state of PTT input.

TX line   : This indicates the actual transmitter activity.

TX on  : Low

TX off : High

Press the F2 key or SPACE BAR to alternate between selections.

14Selcall Toneset Standard Help

The 'selcall toneset standard' parameter determines which selcall

toneset standard is used for decoding selective call messages.

Note that the selcall standard that is used for encoding is defined

within the encode formats.

Press SPACE BAR to alternate between selections.

Press SHIFT key + F2 key to open the tone list.

14Receive Gap Period Help

The 'receive gap period' defines the period of time the selcall

decode algorithm requires a 'no-tone' condition after the last valid

tone has been received.

The recommended value for the receive gap period is 1.5 times the

duration per tone, for example, 150 ms for CCIR 100 ms.

Range   : 0 - 640ms in 5ms steps

Default : 1.5 times the duration per tone

Press SPACE BAR to alternate between selections.

14Selectable Selcall Digits Help

The parameter 'selectable selcall digits' is used to define the

number of digits of the keypad selectable address that may be

changed by the user.

These keypad selectable digits are normally included in the transmit

selective call formats.

Press SPACE BAR to alternate between selections.

14Selectable Status Digits Help

The parameter 'selectable status digits' is used to define the

number of digits of the status number that may be changed by the

user.

Press SPACE BAR to alternate between selections.

14Busy Symbol Function Help

The parameter 'busy symbol function' determines the function of the

busy symbol on the display.

If set to 'Carrier', it will indicate presence of a carrier on the

selected channel.

If set to 'QT/DQT', it will indicate a carrier without or wrong

QT/DQT.

Press SPACE BAR to alternate between selections.

14Return-To-Standby Mode Help

The 'Return-to-Standby mode' parameter selects whether or not the

RtS facility is used.

If set to 'Disable', RtS does not operate.

If set to 'TX only', the RtS timer is retriggered while transmitting

in the active mode.

If set to 'TX/RX', the RtS timer is retriggered while transmitting

as well as receiving (carrier detect of QT/DQT operated) in the

active mode.

Press SPACE BAR to alternate between selections.

15Automatic Close Help

The 'automatic close' parameter determines the number of digits that

have to match the primary receive address in order to close the

radio.

When 'None' is selected, the automatic close facility is disabled.

It is recommended to enter the highest number of digits possible for

operation of automatic close, for example, 'Three' for a 5-tone

decode format with the first three digits fixed and two variable

digits.

Press SPACE BAR to alternate between selections.

15Monitor Function Help

The 'monitor function' parameter determines the function of the

monitor key.

If set to 'Selcall', the monitor key operates as selcall

open / close.

If set to 'QT/DQT', the monitor key operates as QT/DQT monitor.

If set to 'Both', the monitor key operates as combined selcall

and QT/DQT monitor.

If set to 'Reset', the monitor key resets the selcall mute.

If set to 'Disable', the monitor key has no function.

If set to 'Enable', the monitor key operates as QT/DQT monitor.

Press SPACE BAR to alternate between selections.

15Drop Out Delay Time Help

Drop out delay time interval from the end of a received call to the

time scan automatically resumes.

Range   : 0.5 - 15.0s in 0.5s steps

Default : 3.0s

Press SPACE BAR to alternate between selections.

15Tx Dwell Time Help

TX dwell time interval from the end of the transmit to the time scan

automatically resumes.

Range   : 0.5 - 15.0s in 0.5s steps

Default : 3.0s

Press SPACE BAR to alternate between selections.

15Scan Dwell Time Help

The 'scan dwell time' determines the period of time after which

scanning is resumed while continuously detecting activity on the

channel.

If set to Off, the scan dwell facility is disabled.

Range   : Off / 1 - 255s in 1s steps

Default : Off

Press SPACE BAR to alternate between selections.

16Scan Configuration Help

The 'scan configuration' parameter determines the operating mode of

scanning defined for the current system group.

Available selections are: carrier, QT/DQT, selcall, both,

selcall-st, both-st and no carrier.

Using the /st selections, scanning will stop after the unit has been

called until a button is pressed.

Press SPACE BAR to alternate between selections.

16Group Digit Selectable Help

The 'group digit selectable' parameter determines if group digit

selection is possible using the Digit 1x, Digit 10x and Digit Up

keys (Enable) or if group digit selection is not allowed (Disable).

Press SPACE BAR to alternate between selections.

16Decode Format Help

The 'decode format' parameter selects the format used to decode

incoming selective call messages.

Note that the decode format itself can include links to other decode

formats to allow decoding of multiple formats.

Press SPACE BAR or the F2 key to select.

16Encode Button #1 Help

The encode format for 'encode button #1' is selected in this field.

The selected encode format will be executed if the key or button

(linked to this encode button function within the key definition

menu) is activated.

Suggested use: normal call.

Press SPACE BAR or the F2 key to select.

16Encode Button #2 Help

The encode format for 'encode button #2' is selected in this field.

The selected encode format will be executed if the key or button

(linked to this encode button function within the key definition

menu) is activated.

Suggested use: free-dial selcall.

Press SPACE BAR or the F2 key to select.

16Encode Button #3 Help

The encode format for 'encode button #3' is selected in this field.

The selected encode format will be executed if the key or button

(linked to this encode button function within the key definition

menu) is activated.

Suggested use: free-dial DTMF.

Press SPACE BAR or the F2 key  to select.

17Encode Button #4 Help

The encode format for 'encode button #4' is selected in this field.

The selected encode format will be executed if the key or button

(linked to this encode button function within the key definition

menu) is activated.

Suggested use: single-tone transmission.

Press SPACE BAR or the F2 key to select.

17Encode Button #5 Help

The encode format for 'encode button #5' is selected in this field.

The selected encode format will be executed if the key or button

(linked to this encode button function within the key definition

menu) is activated.

Suggested use: status call.

Press SPACE BAR or the F2 key to select.

17Encode Button #6 Help

The encode format for 'encode button #6' is selected in this field.

The selected encode format will be executed if the key or button

(linked to this encode button function within the key definition

menu) is activated.

Suggested use: emergency call.

Press SPACE BAR or the F2 key to select.

17Encode PTT-press Help

The encode format selected here is executed when the PTT-button is

pressed.

This encode format can be used to send an ANI message every time the

PTT button is pressed.

When "Encode PTT-press" is set to disable, the transceiver acts

normal PTT operation.

Press SPACE BAR or the F2 key to select.

17Encode PTT-release Help

The encode format selected here is executed when the PTT-button is

released.

This encode format can be used to send an ANI message every time the

PTT button is released.

Press SPACE BAR or the F2 key to select.

17Encode DTMF Overdial Help

This encode format is executed every time a key is pressed while the

PTT button is held down for speech conversation.

Using a suitable encode format definition this facility can be used

to provide DTMF overdialling.

Press SPACE BAR or the F2 key to select.

17Encode DTMF Direct Help

The encode format selected here is executed every time another

channel is selected by DTMF keypad, also while scanning.

The encode function can be used to customized some channel

characteristics.

Press SPACE BAR or the F2 key to select.

18Encode Channel Select Help

The encode format selected here is executed every time another

channel is selected by channel selector, also while scanning.

This encode function can be used to customize some channel

characteristics.

Press SPACE BAR or the F2 key to select.

18Encode On Hook Help

The encode format selected here is executed when the microphone goes

on hook.

Press SPACE BAR or the F2 key to select.

18Encode Off Hook Help

The encode format selected here is executed when the microphone goes

off hook.

Press SPACE BAR or the F2 key to select.

18Decode Formats Help

The decode format editor allows entry, editing and compiling of

decode formats.

Decode formats consists of a series of instructions, separated by a

space character; these instructions are:

CMP  DECI ENCI DECIT BEEPI QUEUE GRP0  SSTO

STO  DECG ENCG DECGT BEEPG DISPQ GRPA  EMGOF

ACK  DECA ENCA DECAT BEEP  DISPM GRPN  CKSAT

MONO DECF QUE3 QUE6  RSTUN REPOF GRP0A

MONF QUE1 QUE4 QUE7  RVIVE EXTAL ERASE

MONC QUE2 QUE5 QUE8  RKILL XPASS CLOSE

See programming manual for detail.

** Nemonic **   ** Description **

ACK            : show ACK sign

BEEP num       : sound alert for individual and group calls

BEEPG num      : sound alert for group calls only

BEEPI num      : sound alert for individual calls only

CKSAT          : check for QT/DQT decode

CLOSE          : check is decode triggered by Return-to-Standby

CMP seq        : compare to sequence; immediate or indirect

DECA dec       : execute decode format if individual or group call

DECAT time dec : trigger decode format for time ms if individual or

group call

DECF dec       : execute decode format if call fail

DECG dec       : execute decode format if group call

DECGT time dec : trigger decode format for time ms if group call

DECI dec       : execute decode format if individual call

DECIT time dec : trigger decode format for time ms if individual

call

DISPM          : select queue message mode

DISPQ          : select queue mode, show first entry

EMGOF          : emergency mode off

ENCA enc       : execute encode format if individual or group call

ENCG enc       : execute encode format if group call

ENCI enc       : execute encode format if individual call

ERASE time     : erases contents temporary buffer after period of

time

EXTAL alertseq : controls external alert output

GRP0           : decode 0-tone as group digit

GRP0A          : decode both A-tone and 0-tone as group digit

GRPA           : decode A-tone as group digit

GRPN           : no group digit decode

MONC           : monitor close

MONF           : monitor open, flash(blink) LED

MONO           : monitor open

QUEUE          : queue contents of temporary buffer with channel no.

QUE1           : queue buffer, with channel, duplicate allowed, FILO

QUE2           : queue buffer, w/o channel, duplicate allowed, FILO

QUE3           : queue buffer, with channel, no duplicate allowed,

FILO

QUE4           : queue buffer, w/o channel, no duplicate allowed,

FILO

QUE5           : queue buffer, with channel, duplicate allowed, FIFO

QUE6           : queue buffer, w/o channel, duplicate allowed, FIFO

QUE7           : queue buffer, with channel, no duplicate allowed,

FIFO

QUE8           : queue buffer, w/o channel, no duplicate allowed,

FIFO

REPOF          : repeat sequence off

RKILL          : remote kill

RSTUN          : remote stun

RVIVE          : remote revive

SSTO           : status store

STO index      : store digits for queuing

XPASS          : exit if individual or group call match

The parameters mentioned are:

seq      : Sequence of immediate and/or indirect digits:

#dig      immediate digit

^Dptr     default transmit address

^Pptr     primary receive address

^Sptr     secondary receive address

^Tptr     third receive address

^Qptr     temporary queue entry

^Uptr     selectable status address

^Mptr     miscellaneous digits

^Kptr     selectable selcall address

dig      : Digit: 0 .. 9 and A .. F

ptr      : Pointer: 1 .. 5

index    : Index: 1 .. 5

num      : Alert beep number: 0 .. 46

enc      : Encode format

dec      : Decode format

time     : Standard time: 0 .. 2550 ms in 10 ms steps

alertseq : External alert sequence: 00000000 .. 11111111

19Encode Formats Help

The encode format editor allows entry, editing and compiling of

encode formats.

Valid instructions are:

TXS RX  TONE SENDC SEND XCLOS SELT SELCH CLOSE OPEN

TXD SCT SCTP SDIAL SLEN XOPEN SELD SELSC TXTOF TEXT

TXM SCR SCRP SCOFF SCHK XSCAN SELS SELMC STOPS STRTS

TXP LEN SCPH DTAIL RSMS XCARR SELM PREFA STOPP STRTP

TXF DEL TSET ERRX0 RSML XCTCN SELC PREFS DIFIX SBUSY

RXP TPE RIGN ERRX1 RTSO XCTCW XSAT SPREV CLACK

RXF TPD RACK WAITR RTSL XIDEL XPRI RSTDL ENTTM

REP ENC BEEP EMGON RTSA XLIST XNOT XSENS

See programming manual for detail.

** Mnemonic **   ** Description **

BEEP num       : sound alert tone sequence

CLACK          : clear ACK sign

CLOSE          : select selcall mute close

DEL xtime      : execute delay

DIFIX          : display fix

DTAIL          : DQT tail tone transmission

EMGON          : activate emergency mode

ENC enc        : execute encode format

ENTTM ltime    : assign entry timeout

ERRX0          : error alert on exit off

ERRX1          : error alert on exit on

LEN xtime      : select duration per tone

OPEN           : select selcall mute open

PREFA          : select preference channel when in scan mode

PREFS          : select preference channel when in scan mode and

scanning

RACK stime dec : trigger acknowledge receive during period of time

REP rep        : repeat sequence for rep times

RIGN stime     : receive ignore during period of time

RSML ltime     : resume after long period of time (1 .. 255 s)

RSMS stime     : resume after short period of time (10 .. 2550 ms)

RSTDL          : restart encode format if dial digit available

RTSA           : reload RtS with active tome-out

RTSL           : reload RtS with listen or active time-out

RTSO           : reload RtS only when in active mode

RX             : switch to receive mode

RXF rf-freq    : switch to receive using in-line frequency

RXP            : switch to receive using programmed frequency

SBUSY          : skip encode send digits if channel busy

SCHK           : send checksum digit

SCOFF          : QT/DQT transmit off

SCPH           : QT phase 180 degrees

SCR st-tone    : QT/DQT receive using in-line QT or DQT code

SCRP           : QT/DQT receive using programmed frequency

SCT st-tone    : QT/DQT transmit using in-line QT or DQT code

SCTP           : QT/DQT transmit using programmed frequency

SDIAL          : send free-dialled string of digits

SELC           : select queue message

SELCH channel  : select radio channel

SELD           : select free dial mode

SELM           : select status message

SELMC          : select queue message and status message

SELS           : select status entry mode

SELSC scangr   : select scan group

SELT           : select tone mode

SEND seq       : send sequence of digits, immediate or indirect

SENDC          : send queue

SENDS          : send status

SLEN ofs       : send length indicator plus offset

SPREV          : select previously selected channel when in scan

mode

STOPP          : stop priority scan

STOPS          : stop scanning

STRTP          : start priority scan

STRTS          : start scanning

TEXT 'text'    : output text on display

TONE af-freq   : send single tone

TPD            : transparency disable

TPE            : transparency enable

TSET toneset   : select toneset

TXD            : switch to transmit, DTMF path

TXF rf-freq    : switch to transmit using in-line frequency

TXM            : switch to transmit, microphone path

TXP            : switch to transmit using programmed frequency

TXS            : switch to transmit, selcall path

TXTOF          : restore normal display operation

WAITR          : wait for key release

XCARR          : exit if carrier detected

XCLOS          : exit if selcall mute closed

XCTCN          : exit if no or wrong QT/DQT detected

XCTCW          : exit if no carrier, no or wrong QT/DQT detected

XIDLE          : exit if Return-to-Standby idle mode

XLIST          : exit if Return-to-Standby idle or listen mode

XNOT           : exit if not in transfer mode

XOPEN          : exit if selcall mute open

XPRI           : exit if in priority scan mode

XSAT st-tone   : exit if wrong QT/DQT in-line

XSCAN          : exit if in scan mode

XSENS          : exit if scanning, but not stopped on channel

The parameters mentioned are:

seq     : Sequence of immediate and/or indirect digits:

#dig   immediate digit

^Dptr  default transmit address

^Pptr  primary receive address

^Sptr  secondary receive address

^Tptr  third receive address

^Qptr  temporary queue entry

^Uptr  selectable status address

^Mptr  miscellaneous digits

^Kptr  selectable selcall address

dig     : Digit: 0 .. 9 and A .. F

ptr     : Pointer: 1 .. 5

ofs     : Offset: 0 .. 9

dec     : Decode format

enc     : Encode format

re-freq : Transceiver frequency: dependent on selected range

af-freq : Audio frequency: 300 .. 3000 Hz

st-tone : QT frequency (67.0 to 250.3) or DQT code (N or I)

toneset : ZVEI, CCIR, EEA, PZVEI, DZVEI, PCCIR, PDZVEI, ZVEI-2, EIA,

Natel, AP-369, Kenwood

xtime   : Extended time: 1 .. 7000 ms in 1 ms units

stime   : Standard time: 0 .. 2550 ms in 10 ms units

ltime   : Long time: 0 .. 255 seconds in 1 second units

num     : Alert beep number: 0 .. 46

rep     : Repeat factor: 0 .. 255

text    : String of characters

channel : Channel number: 1 .. 250

scangr  : Scan group number or direction: 0 .. 9, UP, DOWN

22Status Message Help

The status message is entered in this field.

The status message is a maximum of 16 characters.

22Queue Message Help

The queue number and who has called in this field.

The queue number is a maximum of 5 digit code.

The who has called is a maximum of 16 characters.

22Kenwood Toneset Help

Type KENWOOD tone directly.

You can have original toneset by this feature.

tone   : 300 - 3000Hz in 1Hz steps

period : 20 - 150ms in 1ms steps

Press the F2 key or SPACE BAR to alternate between selections.

22Embedded Message Help

Up to 64 alphanumeric characters can be programmed into the radio.

This can be used for an electric serial number, program file name

notation, property ID., etc..

It will appear as part of the header statement on a program data

printout along with the model type and filename.

Type required message directly.

22RX/TX Frequency Help

Type a Receive and Transmit frequency directly.

Receive-only channels are allowed.

Refer radio's service manual for spec ex. frequency range, step.

22QT/DQT Help

Type tone or code directly.

QT  : 67.0 to 250.3Hz in 0.1Hz steps

DQT : codes 000 to 777

Enter D###N  for  (Normal)

D###I  for (Inverted)

(codes default to Normal if ( I ) is not entered)

-or-

Press F2 key, open QT/DQT chart,

QT  : 67.0 to 250.3Hz

DQT : codes 023 to 754

then select.

22Channel Name Help

Enter a maximum of 9 characters.

The RX frequency must be entered before the channel name.

23Power Select Help

Select High or Low Power.

Press the F2 key or SPACE BAR to alternate between selections.

23Clock Frequency Shift Help

Shifts the microprocessor system clock frequency per channel to

prevent interference.

Press the F2 key or SPACE BAR to alternate between selections.

23Local/Distance Help

Local/Distance Help

The receive attenuator will active when "Local".

The receive attenuator will not active when "Distance".

Press the F2 key or SPACE BAR to alternate between selections.

23Wide/Narrow Help

Selects either Wide or Narrow.

However, note that this parameter can be selected only when

Wide & Narrow under Personality is set to Wide 5k/Narrow,

Wide 4k/Narrow or Wide 5k/Wide 4k.

Press the F2 key or SPACE BAR to alternate between selections.

23Auxiliary Help

Sets the auxiliary default logic per channel.

Refer AUX 2 key.

Press the F2 key or SPACE BAR to alternate between selections.

23Voice Scrambler Help

Sets the scrambler code.

Set to Off when not using the scrambler.

Select scrambler B in model name screen.

See radio's S/M for hardware information.

Press the F2 key or SPACE BAR to alternate between selections.

23Default Tx Address Help

The 'default tx address' defines the default contents of the

selectable part of the transmit selcall address.

This address is used at radio turn-on, when the channel is selected

while the parameter 'Copy from default tx' is set to 'Enable', or

when the contents of the keypad buffer is replaced on entry of

another selcall address.

23Primary Rx Address Help

The 'primary rx address' defines the primary own identity of the

radio.

In most applications, this parameter is used to define the unit's own

5-tone address.

23Secondary Rx Address Help

The 'secondary rx address' defines the secondary own identity of the

radio.

In most applications, this parameter is used to define the unit's

second 5-tone address.

24Third Rx Address Help

The 'third rx address' defines the third own identity of the radio.

In most applications, this parameter is used to define the unit's

third 5-tone address.

24Scan Include List Help

The 'scan include list' parameter

defines the scan groups to which the

currently selected channel must

belong.

If a digit is shown, the channel is

included in that particular scan

group.

If the channel does not belong to a defined scan group, a dash

symbol will be shown at that position.

Editing of this parameter is possible by entering the scan group

number in one string; for example, enter 156 to include the current

channel into scan groups 1, 5 and 6.

24System Reference Help

The 'system reference' parameter selects the system group for the

currently selected channel.

The parameters of the referred system group will by used when that

channel is selected on the radio.

Press SPACE BAR or the F2 key to select.

24Channel Copy Help

Copies the entire channel data to another channel(s).

Use SHIFT + F3 to copy a channel.

24Frequency Copy Help

Copies a channels RX frequency to the TX column.

Use Shift + F4 to copy a RX frequency.

24QT/DQT Copy Help

Copies the data of QT/DQT from a QT/DQT decode column to the encode

column.

Use Shift + F5 to copy decode tone.

24IF & Heterodyne Help

Do not change IF & Heterodyne in normal use to avoid DAMAGE of DATA.

This data is for factory use only.

Press the F2 key or SPACE BAR to alternate between selections.

24Read From Radio Help

Read program data from the radio --

-- select & hit enter.

24Write To Radio Help

Writes currently loaded program data into the radio's memory.

Before writing to the radio, Verify the model type selected in your

current data file matches the  model of target radio.

-- Then hit Enter to program the radio.

25Test Data Read Help

Read tuning data from the radio --

-- select & hit enter.

25Test Data Write Help

Writes currently loaded tuning data into the radio's memory.

Before writing to the radio, Verify the model type selected in your

current data file matches the  model of target radio.

-- Then hit Enter to program the radio.

25Test Data Load Help

The Load function is used to retrieve a stored tuning data file from

a hard disk or floppy disk to be modified, viewed, written

(program a radio) or printed out.

A directory of stored files, sorted alphabetically, will be

displayed for selection.

25Test Data Save As Help

Saves the current tuning data with the option of assigning a "new"

file name, or renaming an existing file.

25Test Frequency Edit Help

Type a Receive and Transmit frequency directly.

Receive-only channels are allowed.

Refer radio's service manual for spec ex. frequency range, step.

CH1 : center frequency

CH2 : low edge frequency

CH3 : high edge frequency

Type tone or code directly.

QT  : 67.0 to 250.3Hz in 0.1Hz steps

DQT : codes 000 to 777

Enter D###N  for  (Normal)

D###I  for (Inverted)

(codes default to Normal if ( I ) is not entered)

-or-

Press F2 key, open QT/DQT chart,

QT  : 67.0 to 250.3

DQT : codes 023 to 754

then select.

25Test Channel Help

Select the desired frequency from

the Test Frequency table.

Press the F2 key or SPACE BAR to alternate between selections.

25RF High Power Help

Preset RF Power High level adjustment.

Press <- or -> to increment selection.

25RF Low Power Help

Preset RF Power Low level adjustment.

Press <- or -> to increment selection.

26Max Deviation Help

Preset Maximum Deviation level adjustment.

Press <- or -> to increment selection.

26DQT Balance Help

Preset DQT encode balance adjustment.

Press <- or -> to increment selection.

26QT Deviation Help

Preset QT encode level adjustment.

Press <- or -> to increment selection.

26DQT Deviation Help

Preset DQT encode level adjustment.

Press <- or -> to increment selection.

26Sensitivity Help

Preset Sensitivity level adjustment.

Press <- or -> to increment selection.

26Squelch Help

Preset Squelch level adjustment.

Press <- or -> to increment selection.

265TONE Deviation Help

Preset 5TONE encode level adjustment.

Press <- or -> to increment selection.

26DTMF Deviation Help

Preset DTMF encode level adjustment.

Press <- or -> to increment selection.

26DQT Polarity Help

Preset DQT polarity.

Press the F2 key or SPACE BAR to alternate between selections.

26Tuning Data Print Help

Print out currently memorized Tuning Data.

Use F5 to print out.

26Communication Port Help

Select the COM port of the computer's serial communication port for

programming.

These are two choices, COM1 or COM2.

COM port setting's in your computer may be dependent on internal

hardware and/or setup software settings; check your computer manuals

for details.

27Black & White Color Help

Monochrome display mode.

27Display Back Color Help

Main window's background color.

Press the F2 key or SPACE BAR to alternate between selections.

27Input Data Color Help

Input data cursor color.

Press the F2 key or SPACE BAR to alternate between selections.

27Feature Highlight Color Help

Sub window's selected feature cursor background color.

Press the F2 key or SPACE BAR to alternate between selections.

27Screen Border Info. Color Help

Outside of main window's border information background color.

Press the F2 key or SPACE BAR to alternate between selections.

27Windows Menu Color Help

Main menu bar sub windows background color.

Press the F2 key or SPACE BAR to alternate between selections.

27General Message Color Help Important information window's

background color.

Press the F2 key or SPACE BAR to alternate between selections.

27Windows Color Help

Sub menu bar sub window's background color.

Press the F2 key or SPACE BAR to alternate between selections.

27Help Window Color Help

Help window's background color.

Press the F2 key or SPACE BAR to alternate between selections.

27Help Path Help

Help message files are contained in this program under

drive:\KPG28D\KPG28D.HLP.

The drive and path statement that this program uses to access these

messages is initially set to A:\KPG28D.

Type the required drive and path.

27List Help

Move the cursor using arrow key and press enter key to determine.

27End Of Help File