PROGRAMMING SOFTWARE

KPG-28D

__________________________________________________________

INSTRUCTION MANUAL Ver. 1.00

KENWOOD CORPORATION

- 1 -

SOFTWARE LICENSE NOTICE

1.  The copyright for this software is owned by KENWOOD CORPORATION.

2.  The software and manual, or any part thereof, must not be used nor copied

except as authorized in writing by KENWOOD CORPORATION.

3.  One set of software must be purchased for each computer on which the

software will be used.

4.  The contents of the manual are subject to change without notice.

5.  The software and manual must only be used within the terms of the license

agreement.

6.  KENWOOD CORPORATION accepts no responsibility for the effects caused by

the use of the software and manual.

7.  The specifications of the software are subject to change without notice.

8.  The product has been carefully designed; however, after reviewing the

supplied documentation, if you have questions, encounter mistakes, or

discover that necessary information is absent from the manual, please contact

KENWOOD CORPORATION.

IBM is a trademark registered by International Machines Corporation

HANDLING OF FLOPPY DISKS

Observe the following points when handling the programming software disks:

* Never touch the magnetic surface of the floppy disks.

* Keep magnets and magnetic objects away from the floppy disks.

* Do not bend the floppy disks.

* Do not place the floppy disks in areas that are unusually hot, humid, cold,

or dry.

* Always insert the 3.5" floppy disk back into its case after use.

- 2 -

CONTENTS

==================================================================

1   INTRODUCTION

FUNCTIONAL SUMMARY

SUPPLIED ITEMS

SUPPORTED TRANSCEIVERS

HARDWARE/ SOFTWARE REQUIREMENTS

COMPUTER

OPERATING SYSTEM

COMMUNICATION PORT

COMMUNICATION INTERFACE

2   CONNECTING THE EQUIPMENT

3   INSTALLING THE PROGRAM

INSTALLING ONTO A HARD DISK

INSTALLING ONTO A FLOPPY DISK

4   PROGRAM OVERVIEW

SYSTEM FLOW CHART

PROGRAMMING METHODOLOGY

CREATING AND WRITING NEW DATA TO A TRANSCEIVER

MODIFYING DATA PREVIOUSLY WRITTEN TO A TRANSCEIVER

MODIFYING DISK DATA, THEN WRITING TO A TRANSCEIVER

Add

5   BASIC OPERATIONS

STARTING THE PROGRAM

INITIAL SCREEN

MENU BAR

CHANNEL INFORMATION WINDOW

KEY SUMMARY

SELECTING AND ENTERING DATA

USING ARROW KEYS

USING QUICK KEYS

USING THE KEYBOARD

USING THE DIRECTORY WINDOW

EXITING THE PROGRAM

6   USING THE MENU

MENU OVERVIEW

OPENING MENU WINDOWS

MENU FUNCTIONS

FILE

New

Load

Save

Save As

Delete

Print

Go to DOS

Exit

MODEL

Personality

EDIT

Panel Key Assignment

Mic Key Assignment

Scan Information

Optional Features

System Parameters

Decode Formats

Encode Formats

Status Message

Queue Message

- 3 -

Kenwood Toneset

Embedded Message

Channel

PROGRAM

Read from Radio

Write to Radio

Test Mode

SETUP

British English

Communication Port

Display Color Colour

Help Path

F1=HELP

Help Hot Key (F1)

General Help

Keyboard Help

Keys Active During Help

- 4 -

1   INTRODUCTION

==================================================================

<< FUNCTIONAL SUMMARY >>

KPG-28D software is a tool for programming the TK-759, TK-859, TK-752, or TK-

852 transceiver.  A 3.5" distribution disk is provided.  The software allows

Delete

a user to enter programming data into an appropriate computer, and then

transfer this data to a transceiver.  The entered data can also be printed

out to provide a permanent record.  Additionally, data that is already

programmed in a TK-759, TK-859, TK-752, or TK-852 transceiver can be

retrieved and edited using a computer that is running the KPG-28D software.

<< SUPPLIED ITEMS >>

Your programming software package includes the following:

* 2 floppy disks: 3.5", 1.44 MB

1  "KPG-28D  PROGRAMMING DISK"  containing:

* KPG-28D program files

* Instruction manual file

2  "PROGRAMMING MANUAL"  containing:

* Programming manual file

* Installation manual B62-0694-XX

<< SUPPORTED TRANSCEIVERS >>

This programming software supports the following transceivers:

* TK-759 (250 channels)

* TK-859 (250 channels)

* TK-752 (2 channels)

* TK-852 (2 channels)

<< HARDWARE/SOFTWARE REQUIREMENTS >>

< COMPUTER >

The software was designed for use on IBM PC, XT, AT, PS/2 and compatible

computers with RAM (memory) capacity of at least 460 kB.

< OPERATING SYSTEM >

The operating system must be IBM DOS Version 3.1 (or later) or equivalent.

< COMMUNICATION PORT >

Data is transferred between the computer and transceiver via a serial

communication port on the computer.  The port must be compatible with the IBM

asynchronous communications adapter.  Either the COM1 or COM2 port on the

computer may be used.

< COMMUNICATION INTERFACE >

A KPG-4 cable interfaces the computer with the transceiver when transferring

data.

- 5 -

2   CONNECTING THE EQUIPMENT

==================================================================

Install the KPG-4 cable between the transceiver and the computer before

attempting to use the KPG-28D programming software.

1)   Switch OFF the power to the computer.

Refer to the computer operating manual for set-up details.

2)   Switch OFF the power to the transceiver.

3)   Connect the 25-pin connector on the KPG-4 cable to the COM1 or COM2

connector on the computer.

If the computer has a 9-pin connector for COM1 or COM2, use a

commercially available 9-pin to 25-pin conversion plug or cable.

4)   Connect the other end of the KPG-4 cable to the microphone connector on

the transceiver.

5)   Switch ON the power to the computer.

6)   Switch ON the power to the transceiver.

6 7)   Install and start the KPG-28D program.

Refer to "INSTALLING THE PROGRAM" and "STARTING THE PROGRAM" for

details.

- 6 -

3   INSTALLING THE PROGRAM

==================================================================

After completing all physical connections, you are ready to install the

programming software.  The first procedure describes installation on a hard

disk while the second procedure describes installation on a floppy disk.

<< INSTALLING ONTO A HARD DISK >>

This procedure assumes that the hard disk is installed as drive C.

1)   Insert the KPG-28D floppy disk (source disk) into drive A.

2)   Type A: then press [Enter].

"A:\>" appears on the computer display.

3)   Type INSTALL C: then press [Enter].

The installation program copies the necessary files from the source disk

in drive A to the hard disk.

<< INSTALLING ONTO A FLOPPY DISK >>

1)   Insert the KPG-28D floppy disk (source disk) into drive A.

2)   Insert the floppy disk that will receive the software (destination disk)

into drive B.

3)   Type A: then press [Enter].

"A:\>" appears on the computer display.

4)   Type INSTALL B: then press [Enter].

The installation program copies the necessary files from the source disk

in drive A to the destination disk in drive B.

- 7 -

4   PROGRAM OVERVIEW

==================================================================

<< SYSTEM FLOW CHART >>

This chart provides an overview of all functions offered in the menus of the

programming software.

+-Channel----Edit-----+-RX Frequency

|                     +-TX Frequency

|                     +-QT/DQT Decode

|                     +-QT/DQT Encode

|                     +-Channel Name

|                     +-Power Select

|                     +-Frequency Shift

|                     +-Local/Distance

|                     +-Wide/Narrow

|                     +-Auxiliary

|                     +-Scan Include List

|                     +-System Reference

|                     +-Default TX Address

|                     +-Primary RX Address

|                     +-Secondary RX Address

|                     +-Third RX Address

|                     +-Voice Scrambler (Option)

|

+-Menu-+-File----+-New

|         +-Load

|         +-Save

|         +-Save As

|         +-Delete

|         +-Print

|         +-Go To Dos

|         +-Exit

|

+-Model----Personality---------+-Model Name

|                              +-Wide & Narrow

|                              +-Option Unit

|                              +-Scrambler Unit (Option)

|

+-Edit----+-Panel Key Assignment

|         +-Mic Key Assignment

|         +-Scan Information--+-Priority Channel

|         |                   +-Preference Channel

|         |                   +-Look Back Time A

|         |                   +-Look Back Time B

|         |                   +-User Programming

|         |

|         +-Optional Features-+-Time Out Timer (TOT)

|         |                   +-TOT Pre-Alert

|         |                   +-TOT Rekey Time

|         |                   +-TOT Reset Time

|         |                   +-Control Tone

|         |                   +-Warning Tone

|         |                   +-Normal Display Format

|         |                   +-Off Hook Decode

|         |                   +-On Hook PTT

|         |                   +-Off Hook Scan

|         |                   +-Ignition Sense

|         |                   +-Ext. Alert/Transfer

|         |                   +-Lock Code

|         |                   +-Keypad Operation

|         |                   +-Number Entry Timeout

|         |                   +-Overlay Text Channel

- 8 -

|         |                   +-Overlay Text Selcall

|         |                   +-Overlay Text Status

|         |                   +-Mode------+-Main

|         |                   |           +-Self

|         |                   |           +-Panel Tuning

|         |                   |

|         |                   +-Start up--+-Scan

|         |                   |           +-Monitor

|         |                   |           +-Encode Def.

|         |                   |           +-Channel

|         |                   |

|         |                   +-Opt MDT---+-Data PTT with QT/DQT

|         |                   |           +-Data PTT with Mic Mute

|         |                   |           +-TX Sense

|         |                   |

|         |                   +-Rts Listen Period

|         |                   +-Rts Active Period

|         |                   +-Hold Delay

|         |                   +-Start Delimiter Req'd

|         |                   +-Digit Entry Method

|         |                   +-Copy from Default TX

|         |

|         +-System Parameters-+-Selcall Toneset Standard

|         |                   +-Receive Gap Period

|         |                   +-Selectable Selcall Digits

|         |                   +-Selectable Status Digits

|         |                   +-Busy Symbol Function

|         |                   +-Return-to-Standby Mode

|         |                   +-Automatic Close

|         |                   +-Monitor Function

|         |                   +-Drop out Delay Time

|         |                   +-TX Dwell Time

|         |                   +-Scan Dwell Time

|         |                   +-Scan Configuration

|         |                   +-Group Digit Selectable

|         |                   +-System Formats-+-Decode Format

|         |                                    +-Encode Button #1

|         |                                    +-Encode Button #2

|         |                                    +-Encode Button #3

|         |                                    +-Encode Button #4

|         |                                    +-Encode Button #5

|         |                                    +-Encode Button #6

|         |                                    +-Encode PTT-press

|         |                                    +-Encode PTT-release

|         |                                    +-Encode DTMF Overdial

|         |                                    +-Encode DTMF Direct

|         |                                    +-Encode Channel Select

|         |                                    +-Encode On hook

|         |                                    +-Encode Off hook

|         |

|         +-Advanced Options--+-Decode Formats

|         |                   +-Encode Formats

|         |                   +-Status Message

|         |                   +-Queue Message

|         |                   +-KENWOOD Toneset

|         |

|         +-Embedded Message

|

+-Program-+-Read from Radio

|         +-Write to Radio

|         +-Test Mode---------+-Test Data------+-Test Data Read

|                             |                +-Test Data Write

|                             |                +-Test Data Load

|                             |                +-Test Data Save As

- 9 -

|                             |                +-Test Frequency Edit

|                             |

|                             +-Tuning---------+-Test Channel

|                                              +-High Power

|                                              +-Low Power

|                                              +-Max Deviation

|                                              +-DQT Balance

|                                              +-QT Deviation

|                                              +-DQT Deviation

|                                              +-Sensitivity

|                                              +-Squelch Level

|                                              +-5TONE Deviation

|                                              +-DTMF Deviation

|                                              +-DQT Polarity

|

+-Setup---+-Communication Port

|         +-Display Color

|         +-Help Path

|

+-Help----+-General Help

+-Keyboard Help

<< PROGRAMMING METHODOLOGY >>

This section describes the logical process for three typical programming

operations.

< CREATING AND WRITING NEW DATA TO A TRANSCEIVER >

1)   Specify the transceiver model. (Model)

2)   Input the data. (Edit/Channel Edit)

3)   Write the data to the transceiver. (Program/Write to Radio)

4)   Save the data to a disk. (File/Save As)

< MODIFYING DATA PREVIOUSLY WRITTEN TO A TRANSCEIVER >

1)   Read the data from the transceiver. (Program/Read from Radio)

2)   Modify the data. (Edit/Channel Edit)

3)   Write the data to the transceiver. (Program/Write to Radio)

Add

4)   Save the data to a disk. (File/Save As)

< MODIFYING DISK DATA, THEN WRITING TO A TRANSCEIVER >

1)   Read the data from a disk. (File/Load)

2)   Modify the data. (Edit/Channel Edit)

3)   Write the data to the transceiver. (Program/Write to Radio)

4)   Save the data to a disk. (File/Save As)

- 10 -

5   BASIC OPERATIONS

==================================================================

<< STARTING THE PROGRAM >>

With the software correctly installed, you can now proceed to running the

program and learning the necessary techniques for programming transceivers.

1)   Make the directory containing the installed programming software the

current directory.

If the software was installed onto a floppy disk, first insert the

floppy disk into a drive, then make that drive the current drive.

2)   Type KPG28D then press [Enter].

The program starts and the introductory screen appears.

<< INITIAL SCREEN >>

The primary display is referred to as the Initial Screen.  The Initial Screen

is the first screen to appear after the introductory screen.  After starting

the program, press any key to clear the introductory screen and open the

Initial Screen.

The Initial Screen consists of three parts:

* The upper part of the screen contains the Menu Bar.

* The central part contains the Channel Information window.

* The bottom of the screen contains a Key Summary.

Brief introductions to each of the three parts of the Initial Screen follow;

however, more detailed information is provided later in this manual.

< MENU BAR >

The Menu Bar contains the titles of the various menus available.  The term

"menu" refers to a title under which items are grouped that are required to

edit and process both data and screens.

< CHANNEL INFORMATION WINDOW >

The Channel Information window is used to specify frequencies and signaling

information in specific channels of transceivers supported by this

programming software.

< KEY SUMMARY >

Keys frequently used to access the screens and functions presented by the

programming software are listed below:

Arrow

Moves the cursor to different items for editing or to select an item.

[Enter] or [Return]

Starts the program and inputs data entries or selections.

[Alt]

Opens and closes menu windows or selects the previous screen.

- 11 -

Help ([F1])

Displays help messages.  Pressing [F1] displays the help message

corresponding to the open window.

[F2]

Selects the functions assigned to this key as displayed in the windows and

screens, and as defined in this manual.

[Shift]+[F2]

Selects the functions assigned to this key combination as defined in this

manual.

[Page Up] or [Page Dn]

Moves the help message forward or backward one page at a time.

[Esc]

Moves the cursor to the Channel Information window.

<< SELECTING AND ENTERING DATA >>

There are four methods of selecting or inputting data.  These methods are

explained in this section using typical examples.  Review the examples

carefully before attempting to program a transceiver.

< USING ARROW KEYS >

When a display similar to the Read from radio window appears, use the left-

arrow or right-arrow key to select either "[Ok]" or "[Cancel]".

1)   Use the left-arrow or right-arrow key to move the cursor to the item you

wish to select.

2)   Press [Enter].

< USING QUICK KEYS >

If you are using a color display, Quick Key characters appear in a different

color from other characters.  Quick Key characters may not appear different

from other characters if you are not using a color display.  In this case,

one of the other methods of selection will be easier to use.

When a display similar to the Read from radio window appears, press a Quick

Key corresponding to the item that you wish to select.

* Press [O] to select "[Ok]".

* Press [C] to select "[Cancel]".

< USING THE KEYBOARD >

When a display similar to the Drive & Path window appears, enter the drive

and path using the keyboard.  The drive and path shown are the default names

provided in the programming software.

1)   Press the left-arrow key to move the cursor to the left side of the

entry field.

2)   Press [Del] to delete the present characters.

3)   Type the new drive name and path name in the correct sequence starting

- 12 -

from the left using the alphanumeric keys on the keyboard.

4)   Press [Enter].

When a display similar to the Save File Name window appears, enter the

required characters using the keyboard.

1)   Using the alphanumeric keys, type the desired characters.

2)   Press [Enter].

When the cursor is in the Channel Information window, enter the frequency and

signaling data using the keyboard.

1)   Using the arrow keys, move the cursor to the position that you wish to

change.  Enter the desired data.

2)   Press [Enter].

< USING THE DIRECTORY WINDOW >

When the Directory window appears, press the up- or down-arrow key to select

the desired file name from among those displayed.

1)   Press the up- or down-arrow key to move the cursor to the file name that

you wish to select.

If there are more file names than will fit in the window, use the down-

arrow key to scroll down the list of files.

2)   Press [Enter].

have

<< EXITING THE PROGRAM >>

When you are finished programming, the program should be closed correctly

rather than simply turning off the computer.

1)   Press [Alt] to open the File window.

Press the left- or right-arrow key if a different menu window opens.

2)   Move the cursor to "eXit" by using the up- or down-arrow key then press

[Enter].  Alternatively, press the "X" Quick Key to select Exit.

3)   Select [Ok] with the left- or right-arrow key then press [Enter].

Alternatively, use a Quick key if you prefer.

The program closes and the DOS prompt appears.

4)   If running the program from a floppy disk, remove the disk from its

drive.  Otherwise continue to Step 5.

5)   Switch OFF the power to the computer.

6)   Switch OFF the power to the transceiver.

- 13 -

6   USING THE MENU

==================================================================

<< MENU OVERVIEW >>

The Menu Bar is at the top of the Initial Screen above the Channel

Information window.  There are six menu titles on the Menu Bar.

delete

* File

The File menu allows you to create, open, save, delete, and print data files.

This menu also provides two methods to exit the program.

* Model

The Model menu allows you to enter the model of transceiver that will be

programmed.

* Edit

The Edit menu allows you to assign functions to specific keys on the

transceiver, and to configure scan and other transceiver functions that are

not programmed from the Channel Information window.

* Program

The Program menu allows you to read data from and write data to Flash memory

in the transceiver.  This menu also accesses the Test mode.

* Setup

The Setup menu allows you to select the communication port on the computer

that is used for communicating with the transceiver via the KPG-4 interface

cable.  Other configurable items include the color scheme for the screen as

well as the drive and path name for access to the Help file.

* F1=Help

The [F1] Help key provides help messages pertaining to the highlighted data

field of the open window.  The Help menu offers additional general and

keyboard help.

<< OPENING MENU WINDOWS >>

This section describes the keyboard operations required to display the

"frames" of the various menus that contain lists of commands for setting and

selecting data.  These frames are referred to as "windows", and displaying

one of these frames is "opening a menu window".  Making a frame disappear is

"closing a menu window".

1)   Press [Alt] while the Channel Information window is visible.

If any windows other than a menu window are open, press [Esc] to exit

from those windows first.

A menu window opens.

Pressing [Alt] again, closes the window.  Repeatedly pressing [Alt] will

open and close the same window.

2)   Pressing the right- or left-arrow key closes the currently open window

then opens a window for the adjacent menu topic.

Only one menu can be open at a time.  All menus are shown below for

reference.

File

New

Load

Save (=F4)

save As

Delete

Print (=F5)

- 14 -

Go to dos

eXit (=F3)

Model

Personality

Edit

Panel key assignment

Mic key assignment

Scan information

Optional features

sYstem parameters

Advanced options

Decode formats

Encode formats

Status message

Queue message

Kenwood toneset

Embedded message

Program

Read from radio

Write to radio

Test mode

Setup

Communication port

Display color

Help path

F1=Help

General help

Keyboard help

<< MENU FUNCTIONS >>

< FILE >

The File Menu allows you to create, open, save, delete, and print data files.

This menu also provides two methods to exit the program.

New

The New function clears the data file already created then allows you to

create a new file.  This function resets all data to initial values.

1)   Open the File window.

2)   Select "New" then press [Enter].

A warning window opens.

3)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key, then

press [Enter].  Alternatively, use a Quick Key if you prefer.

Selecting "[Ok]" resets all data to initial values then re-displays the

Channel Information window.

Selecting "[Cancel]" aborts the New function then re-displays the

previous screen.

Load

- 15 -

The Load function reads transceiver data files previously saved on a hard

disk or floppy disk, then writes them into the buffer memory of your

computer.

1)   Open the File window.

2)   Select "Load" then press [Enter].

The Drive & Path window opens.

The window displays the drive and path name of the file currently

loaded.

3)   Type the drive and path name of the desired file from the keyboard then

press [Enter].

The Directory window opens.

The window displays the name(s) of the file(s) on that path.

4)   Select the desired file name then press [Enter].

The Load File Name check window opens.

5)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key then press

[Enter].  Alternatively, use a Quick Key if you prefer.

Selecting "[Ok]" loads the file selected in Step 4 then re-displays the

Channel Information window.

Selecting "[Cancel]" re-displays the previous screen.

File name format:

********.2DT (2 channels)

********.FDT (250 channels)

Save

The Save function saves the file that you have created to the file name at

the current drive and path name.  This method permits you to omit specifying

the drive name, the path name, and file name.

1)   Open the File window or press [F4].

If [F4] is pressed, skip Step 2.

2)   Select "Save" then press [Enter].

The warning window opens.  The window shows the name of the currently

loaded file.

3)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key then press

[Enter].  Alternatively, use a Quick Key if you prefer.

Selecting "[Ok]" saves the file then re-displays the Channel Information

window.

Selecting "[Cancel]" aborts the Save function then re-displays the

previous screen.

Save As

The Save As function saves the file that you have created to a file name that

you must specify.  You must also specify the drive and path.  If the drive

and path are not specified, an error occurs.  To prevent this, create in

advance a drive and path name to a file.

1)   Open the File window.

2)   Select "save As" then press [Enter].

The Drive & Path window opens.  The drive and path name displayed are

the defaults.

3)   Type the desired drive and path name from the keyboard then press

- 16 -

[Enter].

The Save File Name window opens.

4)   Type the desired file name from the keyboard then press [Enter].

If you prefer, you can select the file name from the Directory window.

Press [F9] to open the Directory window.  Make a selection then press [Enter]

two times.

The Save File Name check window opens.

5)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key then press

[Enter].  Alternatively, use a Quick Key if you prefer.

Selecting "[Ok]" saves the file specified in Step 5 then re-displays the

Channel Information window.

Selecting "[Cancel]" re-displays the previous screen.

Delete

The Delete function deletes an existing file.  This allows you to delete

unneeded files from a specified disk thereby freeing up additional disk

space.

1)   Open the File window.

2)   Select "Delete" then press [Enter].

The Drive & Path window opens.  The drive and path name displayed are

the defaults.

3)   Type the desired drive and path name from the keyboard then press

[Enter].

The Directory window opens.

4)   Select a file name then press [Enter].

The Delete File Name check window opens.

5)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key then press

[Enter].  Alternatively, use a Quick Key if you prefer.

6)   Repeat Steps 4 and 5 if you want to delete additional files or press

[Esc] to re-display the Channel Information window.

Selecting "[Ok]" deletes the file then re-displays the previous screen.

Selecting "[Cancel]" re-displays the previous screen.

Print

The Print function directs data to the printer.  The data may have been

created, loaded from a file, or read from the transceiver.  The information

that can be printed includes the following:

* File name

* Model

* Panel key assignment

* Mic key assignment

* Scan information

* Optional feature

* System parameters

* Decode formats

* Encode formats

* Status message

* Queue message

* Kenwood toneset

* Embedded message

* Channel

- 17 -

1)   Open the File window or press [F5].

If [F5] is pressed, skip Step 2.

2)   Select "Print" then press [Enter].

The Print window opens.

3)   Move the cursor to the item to be printed using the up- or down-arrow

key, and press the Space bar or the [F2] key to select [Yes].

Pressing the Space bar or the [F2] key toggles [No] and [Yes] for the

item.

4)   Select an item to be printed and press [F5].

The Print check window opens.

5)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key then press

[Enter].  Alternatively, press a Quick Key if you prefer.

Selecting "[Ok]" prints the file then re-displays the Channel

Information window.

Selecting "[Cancel]" aborts the Print function then re-displays the

previous screen.

Go to DOS

The Go to DOS function is a temporary exit used when you want to enter DOS

commands at the DOS prompt while the program is still running in the

background.

1)   Open the File window.

2)   Select "Go to dos" then press [Enter].

The message shown below appears and DOS commands can be entered.

3)   To resume using the KPG-28D program, type "exit" then press [Enter].

Exit

The Exit function stops the program completely and the DOS prompt appears.

1)   Open the File window or press [F3].

If [F3] is pressed, skip Step 2.

2)   Select "eXit" then press [Enter].

The Return to DOS window opens.

3)   Select "[Ok]" or "[Cancel]" by using the left- or right-arrow key then

press [Enter].  Alternatively, use a Quick Key if you prefer.

Selecting "[Ok]" closes the program and the DOS prompt appears.

Selecting "[Cancel]" aborts the Exit function then re-displays the

Channel Information window.

< MODEL >

The Model menu is used to specify the model of the transceiver to be

programmed.  Use this menu first before beginning to create a new Data File

since the items that must be defined via the Edit menu differ depending on

the transceiver being programmed.

Personality

1)   Open the Model window.

- 18 -

2)   Press [Enter].

The Personality window opens.

3)   Change the settings by pressing the Space bar or the [F2] key.

[Shift]+[F2] or [Shift]+Space bar cycles through the available choices in the

opposite direction.

4)   Press [Alt] to close the Personality window and re-display the previous

screen.

5)   Press [Alt] again or [Esc] to re-display the Channel Information window.

< EDIT >

The Edit Menu is used to enter or edit data needed by the transceiver for

operating.

Panel Key Assignment

Panel Key Assignment programs the five transceiver keys for Press, Hold,

Shift, or Release.

1)   Open the Edit window.

2)   Select "Panel key assignment" then press [Enter].

The Panel Key Assignment window opens.

3)   Change the settings by pressing the Space bar.  Or, press [F2] to select

"List".

The Key List window opens.

Select from the list using the arrow keys.  Press [Enter].

4)   Press [Alt] to close the Panel Key Assignment window and re-display the

previous screen.

5)   Press [Alt] again or [Esc] to re-display the Channel Information window.

Mic Key Assignment

Mic Key Assignment programs the microphone keys for Press, Hold, Shift, or

Release.  Change the settings using the Space bar.  Or, press [F2] to open

the Key List window, and select from the list.  Pressing [Shift]+Space bar

cycles through available choices in the opposite direction.

Scan Information

Scan Information allows configuration of functions relating to Scan groups.

This includes assigning the Priority channel and the Preference channel.

Change the settings by pressing the Space bar or the [F2] key.  Pressing

[Shift]+[F2] or [Shift]+Space bar cycles through available choices in the

opposite direction.

For the 2-channel models, this selection is not available.

Optional Features

Optional Features allows configuration of optional features on the

transceiver.  Change the settings by pressing the Space bar or the [F2] key.

Pressing [Enter] while one of the "Mode", "Start up", or "Opt MDT" functions

- 19 -

in the above window are highlighted causes one of the following windows to

open.

When "Disable" is selected for Option Unit in Personality, the "Opt MDT"

items are not displayed.

* Mode

Change the settings by pressing the Space bar or the [F2] key.

* Start up

Change the settings by pressing the Space bar or the [F2] key.

* Opt MDT

Change the settings by pressing the Space bar or the [F2] key.

System Parameters

System Parameters allows various settings to be made for the system.

* System Name

Enter a system name using the keyboard, and press [Enter].  The System

Parameters window opens.

* System Parameters

Change the settings by pressing the Space bar or the [F2] key.

To open the System Formats window, move the cursor to "System formats" and

press [Enter].

Decode Formats

This function sets decoding formats.

1)   Open the Edit window.

2)   Select "Advanced options" then press [Enter].

3)   Select "Decode formats" then press [Enter].

* Decode Format Name

Enter a decode format name via the keyboard, and press [Enter].  The Decode

Formats window opens.

* Decode Formats

Enter the decode format via the keyboard.  Press [F10] to use the format

compiling function.

Encode Formats

This function sets encoding formats.

* Encode Format Name

Enter the encode format name via the keyboard, and press [Enter].  The Encode

Formats window opens.

* Encode Formats

Enter the encode format via the keyboard.  Press [F10] to use the format

compiling function.

Status Message

- 20 -

Enter the status message via the keyboard.

Queue Message

Enter the queue number and who has called via the keyboard.

Kenwood Toneset

Enter the Tone via the keyboard.  Press the [F2] key to open the 5 Tone List

window and set a tone.

Embedded Message

Embedded Message does not affect the transceiver operation.  Data such as a

file name or serial number can be entered from the keyboard.  This data is

visible only while using the KPG-28D program.

Channel

Enter RX/TX frequencies, QT/DQT codes, and a channel name for each channel.

Press [F10] to open the Channel Edit window.

* Channel Edit

Sets data that is not programmed via the Channel window.  Change the settings

by pressing the Space bar or the [F2] key.  Enter via the keyboard the

following parameters:

CH, Channel Name, RX/TX Frequency, QT/DQT codes, Scan Include List, Default

TX Address, Primary/Secondary/Third RX Address.  When "Disable" is selected

for Scrambler Unit (Option) in Personality, Voice Scrambler (Option) is not

displayed.

< PROGRAM >

The Program Menu allows you to read data from a transceiver, write data to a

transceiver, and perform various adjustments in the Test mode.

Read from Radio

1)   Open the Program window.

2)   Select "Read from radio" then press [Enter].

The Read From Radio window opens.

3)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key then press

[Enter].  Alternatively, press a Quick Key if you prefer.

Selecting "[Ok]" reads data from the transceiver.  When completed, a

window opens to report the completed status.

Selecting "[Cancel]" aborts the Read function then re-displays the

Initial Screen.

Note:

If communication with the transceiver is not possible for any reason, the

Check Connection Error window opens.  Confirm that the transceiver is

connected to the computer correctly via a KPG-4 cable then begin from Step 1

above.

Write to Radio

- 21 -

1)   Open the Program window.

2)   Select "Write to radio" then press [Enter].

The Write To Radio window opens.

3)   Select "[Ok]" or "[Cancel]" with the left- or right-arrow key then press

[Enter].  Alternatively, press a Quick Key if you prefer.

Selecting "[Ok]" writes data into the transceiver.  When completed, a

window opens to report the completed status.

Selecting "[Cancel]" aborts the Write function then re-displays the

Initial Screen.

Note:

If communication with the transceiver is not possible for any reason, the

Check Connection Error window opens.  Confirm that the transceiver is

connected to the computer correctly via a KPG-4 cable then begin from Step 1

above.

Test Mode

1)   Open the Program window.

2)   Select "Test mode" then press [Enter].

The Test Menu window opens.

3)   Select the item of interest from the Test Menu window then press

[Enter].

* Test Data

Select the item of interest from the Test Data window then press [Enter].  If

"test frequency Edit" is selected, the Test Frequency Edit window opens.

Enter the receive/transmit frequency and QT/DQT data from the keyboard for

each channel.

* Tuning

Select the item of interest from the Tuning Item window then press [Enter].

If "High power" is selected, the Tuning window opens.  Enter data for "Value"

or adjust power by changing the scroll bar with the left-arrow or right-arrow

key.

CAUTION:

Always connect a 50 ½ dummy load prior to transmission.

W or Ohm

< SETUP >

Setup allows you to set the communication port, the color scheme for the

screen, and the Help path.

Communication Port

Select either COM1 or COM2 as the port for communicating with a transceiver.

1)   Open the Setup window.

2)   Select "Communication port" then press [Enter].

The Communication Port window opens.

Note:   If there is only a COM1 port on your computer, a window opens to

report this fact.  In this case, the  communication port cannot be changed.

3)   Select either "[COM1]" or "[COM2]" with the left- or right-arrow key

- 22 -

then press [Enter].  Alternatively, press a Quick Key if you prefer.

4)   Press [Esc] to re-display the Channel Information window.

Display Color Colour

Color             Colour

Display Color Colour allows you to select the Color Colour mode or Black & White mode.

Color mode permits changing colors colours of the Menu Bar, windows, messages, etc.

1)   Open the Setup window.

2)   Select "Display color colour " then press [Enter].

The Display Color colour Mode window opens.

3)   Select either "[Color]" or "[Black & White]" with the left- or right-

arrow key then press [Enter].  Alternatively, use a Quick Key if you prefer.

Selecting "[Black & White]" configures the program to function with a

non-color display.

Selecting "[Color]" opens the window shown below.

4)   Move the cursor by using the up- or down-arrow key to the item that you

wish to configure.  Alternatively, use a Quick Key if you prefer.

5)   Select the desired color by pressing the Space bar, the [F2] key, or the

right-arrow key.  Pressing the left-arrow key, [Shift]+[F2] or [Shift]+Space

bar cycles through available choices in the opposite direction.

If you want to return all colors to their initial settings, press

[F10].

6)   Repeat Steps 4 and 5 to set the remaining colors.

7)   Press [Esc] to re-display the Channel Information window.

Help Path

Use Help Path to select the drive and path name of the Help file.

1)   Open the Setup window.

2)   Select "Help path" then press [Enter].

The Help Drive & Path window opens.

3)   Type the drive and path name from the keyboard then press [Enter].

The window closes then the Channel Information window is re-displayed.

Note:

* If the entered drive does not contain a disk, the Error window shown below

opens.  Press any key, insert the correct disk in the specified drive, then

press [Enter].

* If the entered drive or path does not exist, the Error window shown below

opens.  Press any key, correct the entered drive or path name, then press

[Enter].

* If there is no Help file at the location indicated by the drive and path

name, the Error window shown below opens.  Press any key, correct the

specified drive and path information, then press [Enter].

< F1=HELP >

- 23 -

Help Hot Key (F1)

The KPG-28D program offers field specific help that consists of brief

explanations pertaining to the data field or menu item where the cursor is

currently located.  For example, if the cursor is in the Receive data field,

pressing [F1] will automatically retrieve an explanation for "Frequency

Help".  Pressing [F1] again calls up an index of related topics for which you

can also request help.  Pressing [F1] still one more time calls up a higher

level index.  Select topics from these indexes by using the up- and down-

arrow keys and the [Enter] key.  When using the KPG-28D program, press the

[F1] Hot Key for assistance anytime you are unsure what to do next.

Additionally, there is a General Help function that summarizes information

relating to screen organization.  Plus, there is a Keyboard Help function

that explains how to use the keyboard.

General Help

1)   Open the F1=Help window.

2)   Select "General help" then press [Enter].

The General Help window opens.

Pressing [Alt] closes the window and displays the F1=Help menu.

Keyboard Help

1)   Open the F1=Help window.

2)   Select "Keyboard help" then press [Enter].

The Keyboard Help window opens.

Pressing [Alt] closes the window and displays the F1=Help menu.

Keys Active During Help

The following keys can be used while accessing Help screens:

[Alt]

Exits Help; re-displays the previous screen.

[Esc]

Returns to Channel Information window.

Up-arrow key

Scrolls the screen up.

Down-arrow key

Scrolls the screen down.

[Page Up]

Displays the previous page.

[Page Dn]

Displays the next page.

[F5]

Prints help information.

- 24 -