N4PY Pegasus Control Program Version 1.40

This documentation assumes the user is already familar with

the operation of the Ten-Tec version of the Pegasus Control

program.  As such, new keys and functions not found in the

Ten-Tec version will be the topic of discussion.

Control Panel Buttons

ManMD

This button controls whether or not filter and step information

automatically change when a new mode is selected.  With this

button off, when a new mode is selected, a new filter and step

will automatically change accorinding to the mode.  With this

button on, only the mode will change when a new mode is selected.

SAM

The mode "SAM" is for synchronous AM.  It is not really sync AM,

but the next best thing.  This mode simply turns on the BFO and

sets the step size to 1 Hz.  You must carefully zero beat the AM

carrier by slowly tuning for this mode to be effective.  This

mode is much more tolerant to AM signal fade.

RTTY

This mode is used for any digital mode you may want to run.  Its

default setup is for RTTY.  It sets a frequency offset of 2100 hertz

and turns on the accessory input for transmit audio, and uses

LSB mode.

You can change the button name, frequency offset, and transmit mode

through pagasus.ini parameters defined in the parameter section.

See the pegasus.ini parameter section for more information.

Store SP

This button stores the frequency, mode, and filter in a temporary

memory for later retrieval.

Recall SP

This button retrieves the values of frequency, mode, and filter,

last saved with the "Store SP" button.

FAST, MED, SLOW

These buttons set the AGC speed.

Step Buttons

These buttons set the frequency step size according to their value.

They are user definable.  See the pegasus.ini parameter section.

Band Buttons

These buttons are band register buttons.  They return you to the

last frequency and mode you were using on the appropriate band

section.  The column labelled "CW" is for the normal "CW" section

of that band.  Clicking these buttons will return you to the last

frequency and mode you were using when in this section of the band.

The column labelled "SSB" will return you to the last frequency

and mode you were using when in this section of the band.

The "SWL" column will return you to the last frequency and mode

you were using when in this band.  The "GEN" band button is a general

band register that gets used if the frequency is not in any other

band.

ACC Inp

This button when clicked on, will change the transmit audio

source to the accessory input.  When clicked off, the transmit

audio source will be the mike input.  Note that the separate

audio gain values are remembered for mike input and accessory

input.  Additionally, the power output setting for each input

source is also remembered.

AMTN

The AMTN (AMP TUNE) button is to make tuning an amplifier easier.

When you click the "AMTN" button, several steps happen

automatically.  They are

1) The current mode and frequency is save in the scratch pad memory.

2) The frequency is moved up 5 Khz.

3) CW mode is set.

4) A wide filter is selected.

5) The Step rate is set to 1 Khz.

6) The remote tuning knob is set to ALC to control power.

7) The Pegasus ALC (power) level is set to the previous saved value

for this band and mode.

While the "AMP TN" button is "RED", you should first make sure

the frequency is clear.  If it is not, tune to a clear frequency

with the mouse by clicking the radio panel knob.  Once a clear

frequency is found, key your rig with your PTT or CW key and tune

your amplifier.  Use the remote tuning knob to set the pegasus output

power to correct drive level.  When the AMP is tuned, click the

"AMP TN" to exit this mode.  The frequency, mode, and step will be

restored to original values.  The Pegasus power output value will be

saved and used anytime "AMP TN" is again clicked.  This value

is kept separate by band and mode.  Try this function out.  It is a

real time saver.  The more you use it, the more you will like it.

TUNE

The power when the "TUNE" button is clicked is remembered.  When

you click "TUNE", set the power to what you want the tune power

to be with the Alc slider.  When you click "TUNE" off, the power

will be restored to the previous value.  The next time you click

"TUNE", it will remember the tune power value and set it to this

value.  Try it, it may save your finals!  This option may be

overridden with the "NO TUNE SAVE" option described in the pegasus.ini

section.

ID

The ID button is to activate a 10 minute ID timer to assist in

meeting the 10 minute FCC ID requirement. Clicking this button causes

it to turn blue and start a 9 minute timer (The FCC requirement

is 10 minutes, this gives you 1 minute to ID).  When this timer

expires, a sound is emitted and the button turns red.  The next

time you transmit, the button goes back to blue.

To turn the timer off, click the button again and it will go black,

indicating the timer is now off.

Tuning Selections

When setting an "F" key on the remote pod to tuning selections,

it will toggle between VFO, PBT, RIT, ALC, and VOL.  VOL is for

volume.  Also, each key can be defined to expressly set one of

the 5 tuning knob functions.  In addition to the functions that

were already there you can directly define each function key to

set the remote tuning knob to go to VFO, PBT, RIT, ALC, or VOL.

This is done on the remote tuning pod configuration box.

There are sliders on the main control panel for Transmit Power (TX),

and Keyer Speed (KS).

The upper of the two front panel offset drag control is for PBT.

The tuning rate of both these drag control may be user configured.

The lower of the two front panel offset drag controls is for RIT

and XIT.  This controls a single offset in hertz that may be

applied to RIT or XIT depending on which is turned on. This is

similar to the implementation of the offset function in the OMNI VI.

This allows you while listening to a DX station, to tune in the

station he is working with the RIT control, and then switch to XIT

to actually work the DX station.

Transmit Bandpass Low Frequency Cutoff

This parameter allows you to set the transmit low frequency cutoff

for SSB operation.  The parameter is on the SSB settings screen and

is called "Low freq cutoff".  It can be anywhere between 0 and 1000

hertz.

The default for this value is 200.

For example, if you are using a transmit filter of 2400 hertz and

specify a low frequency cutoff of 200 hertz, your transmit passband

will then be 200 hertz to 2600 hertz.

Pegasus.ini file Parameters for the [OPERATING STATUS] section

The value for each parameter below is set to the default value.

This is the value you get when the parameter is not defined. The

text will indicate what other values are legal.

DX SPLIT=0

There are 2 ways to define usage for the numeric key pad

on the remote pod.  The default is for direct frequency entry.

This occurs if either this parameter is not in the file of it

is defined as 0.

If it is set to 1, it will cause the numeric pod buttons to act

very different.  Pressing any numeric digit will result in setting

your transmit frequency "UP" by this amount.  The "." button places

you at the bottom the band you are on.  The "E" button is a

"reverse" button. In this mode, for direct frequency entry, the

numeric keys on your keyboard will work fine.

PC_UTC=0

Set to 0, this parameter assumes time on you PC is kept in local

time.  Set to 1, it assumes time in your PC is kept in UTC.  It will

then use your time offset value to calculate local time.

REMOVE EXTRA DECIMAL=0

The second decimal point in the displayed frequency can be removed.

This is useful is you are doing a lot of cut and paste operations.

Set the above parameter to one and only one decimal point will appear

in the displayed frequency.

NO TUNE SAVE=0

Setting this option to one will cause the tune power to always be

whatever the ALC power slider is set to.

The "step buttons" are user configurable to any step

between 1 and 10000.  They are configured as follows:

STEP1=1

STEP2=10

STEP3=100

STEP4=1000

STEP5=5000

STEP6=10000

"STEP1" refers to the left top step button.

"STEP2" refers to the center top step button.

"STEP3" refers to the right top step button.

"STEP4" refers to the left bottom step button.

"STEP5" refers to the center bottom step button.

"STEP6" refers to the right bottom step button.

The default values for the operating modes can be

configured to any step between 1 and 10000.

They are configured as follows:

AMSTEP=5000

SSBSTEP=10

CWSTEP=10

FMSTEP=1000

RTTYSTEP=10

SAMSTEP=1

left-right tuning knob=0

This option allows the selection of the tuning method for the main

tuning knob.  The default method uses the left mouse button to tune

up or down depending on where you are on the knob.  The new method

is from VE1ADH and when set, the left mouse button tunes down and

the right mouse button tunes up.

To select the left/right tuning method, add the following line

in the [OPERATING STATUS] section of pegasus.ini:

left-right tuning knob=1

The default for this is 0.

Default Filter Definition

The default filter positions are defined with numbers from

1 to 12.  Number 1 refers to the top left filter button, number 2

refers to the top center button, and so on to number 12 for the bottom

right filter button.  In the [OPERATING STATUS] section of the

pegasus.ini file add the following to define the default filter buttons

for each mode:

AM FILTER POSITION=2

CW FILTER POSITION=12

SSB FILTER POSITION=8

RTTY FILTER POSITION=12

This example would make the AM default filter the top center button,

the CW default filter the bottom right button, the SSB default filter

the third row center button, and the RTTY default filter the bottom

right button.

Special Right Click on the Filter Bandwidth Window

If you right click on the right side of the filter bandwidth

window, you will go to the next narrower filter.  If you right

click on the left side of the filter bandwidth window, you will

go to the next wider filter.  In this way, you can get access

to all 34 filters without having to configure them into a

filter button.  This is useful for SSB reception with heavy

QRM to select the optimum filter.

VARIABLE FREQUENCY SCALE=0

This parameter causes the analog frequency scale to vary with the

step size.  For steps less than 100 hertz, the scale is multiplied

by 100.  For steps between 100 and 1000 hertz, the scale is multiplied

by 10.  For steps above 1000 hertz, the scale is unchanged. This

scale change is primarily for people that do not have the remote tuning

pod to aid in dragging the frequency. To activate this feature add

the following line:

VARIABLE FREQUENCY SCALE=1

CALLSIGN=N4PY

Your call sign can be displayed on the control panel.

The version of the program can be checked by displaying

the "about" section.

PBT_SCALE=8

RIT_SCALE=8

The fine tuning rate of RIT/XIT and PBT can be configured.

PBT_SCALE and RIT_SCALE can set the fine tuning rates to 1 of 6

legal values.  The legal values are 1, 2, 4, 8, 16, and 32.  Any

other value will cause these rates to be set at the default of 8.

RTTY LABEL=RTTY

RTTY MODE=LSB

RTTY OFFSET=2100

The RTTY button is configurable.  The label for the button can

be specified, the actual operating mode can be set to USB or LSB,

and the frequency offset in hertz can be specified.  For example,

you can define this button for use for PSK31 with 1000 hertz tones

as follows:

To define the label for the button, add the following line:

RTTY LABEL=PSK31

RTTY MODE=USB

RTTY OFFSET=1000

MUTE SPEAKER ONLY=0

If you want "MUTE" to only mute the speaker and not mute the line out,

add the following line to pegasus.ini in the [OPERATING STATUS] section:

MUTE SPEAKER ONLY=1

BUTTON TEXT COLOR=BLACK

BUTTON SELECTED TEXT COLOR=BLUE

SPECIAL BUTTON SELECTED TEXT COLOR=RED

COLOR1 RED=0

COLOR1 GREEN=0

COLOR1 BLUE=0

COLOR2 RED=0

COLOR2 GREEN=0

COLOR2 BLUE=0

In order to help out hams with sight difficulties, the color of

the button text can be configured.  There are three parameters

that can be placed in the [RADIO WINDOW] section of pegasus.ini to

control the text colors of the buttons.  There are 8 standard colors that

can be selected.  They are BLACK, WHITE, RED, GREEN, BLUE, YELLOW,

MAGENTA, and CYAN.  There are also 2 user definable colors that

can be selected with the names COLOR1 and COLOR2.

To set the non-selected text color for all the buttons, add the

following line:

BUTTON TEXT COLOR=BLACK

This example would set the text color to black.

To set the selected text color for all the normal buttons, add the

following line:

BUTTON SELECTED TEXT COLOR=BLUE

This example would set the selected text color to blue.

To set the selected text color for the three special buttons,

(TUNE, AMPTN, and SPLIT) add the following line:

SPECIAL BUTTON SELECTED TEXT COLOR=RED

This example would set the special button selected text color to red.

To define the user color called COLOR1, three lines are needed to

specify the amount of red, green, and blue to make up the color.

The range for the amount of each color is 0 to 255.

It is defined as follows:

COLOR1 RED=255

COLOR1 GREEN=128

COLOR1 BLUE=0

The above example would make COLOR1 be orange.

To define the user color called COLOR2, three lines are needed to

specify the amount of red, green, and blue to make up the color.

The range for the amount of each color is 0 to 255.

It is defined as follows:

COLOR2 RED=255

COLOR2 GREEN=0

COLOR2 BLUE=128

The above example would make COLOR2 be pink.

FREQUENCY CORRECTION=0

There is a frequency correction factor to account for errors

in the displayed frequency.  This factor must be the error in

hertz at 10 Mhz. This value can be expressed as + or -.  To

say that the displayed frequency of WWV at 10 Mhz is 5 hertz

low, add the following line:

FREQUENCY CORRECTION=-5

SCALEX=1

SCALEY=1

There are scale factors for the main radio window.  You can

add lines in the [RADIO WINDOW] section for horizontal

and vertical sizing scale factors.  For horizontal scaling use

SCALEX=1

For vertical scaling use

SCALEY=1

The scale factors are used as multipliers of all coordinates in the

program.  With these values set to 1, the size of the main radio

window will be unchanged.  To make the window 10% larger in both

axis use a value of 1.1 for each scale factor.  To make the window

10% smaller, use 0.9 for each scale factor.  This is being added for

hams that have visual problems seeing the screen and not recommended

if you have normal vision.  If you make the window scale smaller, some

characters may not display properly.  Try this at your own risk.

RX320 TRANSFER FILE=c:\rx320\ftran.dat

Dual operation with the RX-320 is now possible.  You must download

my version 1.08 or later of my RX-320 software.  There are

instructions in it for adding an AUX receiver output to the Pegasus

and a mute circuit for the RX-320.  You must add the above line to

tell the Pegasus where the transfer file is for the RX-320. Once this

is specified, a new button will appear on the Pegasus control panel.

This button is labelled "DUAL".  Clicking on it will cause the RX-320

to follow VFO B on the Pegasus. The RX-320 has a "To Pegasus" button

to send its frequency to the Pegasus.  When you clcik the "DUAL"

button off, it will mute the RX-320.

SIX METER CRYSTAL CORRECTION=0

SIX METER POWER=13

If you use the Ten-Tec 6 meter transverter, you can get the display

to show the frequency very accurately by specifying the amount

of error in the transverter crystal frequency.  Specify this

error in hertz with a plus or minus sign as

SIX METER CRYSTAL CORRECTION=520

The above example indicates the crystal is off 520 hertz.

Also the 6 meter transverter ALC power level can be specified

with:

SIX METER POWER=13

The above example sets the 6 meter transverter ALC power level

to 13 which is the default setting.  This value sets the Pegasus

output to about 5 watts.

TWO METER CRYSTAL CORRECTION=0

TWO METER POWER=13

If you use the Ten-Tec 2 meter transverter, you can get the display

to show the frequency very accurately by specifying the amount

of error in the transverter crystal frequency.  Specify this

error in hertz with a plus or minus sign as

TWO METER CRYSTAL CORRECTION=520

The above example indicates the crystal is off 520 hertz.

Also the 2 meter transverter ALC power level can be specified

with:

TWO METER POWER=13

The above example sets the 2 meter transverter ALC power level

to 13 which is the default setting.  This value sets the Pegasus

output to about 5 watts.

HIGH SPEED TUNING FACTOR=1

This parameter causes the tuning rate when the tuning knob is

turned at a high rate to be multiplied by this factor's amount.

Legal values for this factor are 1 to 10.  Use the value of 1

to disable this feature.

GENERAL COVERAGE TRANSMIT=0

When this parameter is set to 1, the Pegasus will transmit anywhere

it can receive.  This is very useful with the 6 and 2 meter

transverters to allow full transmit coverage on 6 and 2 meters.

Importing and Exporting Files

The file menu has 2 new functions on it.  They are Import and Export.

These functions allow the ability to import and export files that are

text files. This allows you to export the current memory files as a

text file and then add more stations to it.  You may get these files

from any source and simply combine them by following the file format.

Later, you may import this file back into memory an assign an RMY file

name to it.

The format of the file is straight forward.  An entry simply occupies

a single line.  Each line has ascii text for the station name, station

frequency in mHz, station mode, filter in hertz, country, language and

optionally split frequency.  Each value is separated by a comma with

no extra spaces.  Inspection of an existing file will show the format

clearly.

N4PY File Interface

For users that want to experiment with the file interface, I am

describing the interface.

This software allows control of the pegasus from a disk file.

The interface file is a binary file that the with default location

in the directory "c:\my\ham\".  The default file name is

called "ftran.dat".

Make a directory such as "c:\my\ham".  Copy the file "ftran.dat" into

this directory.

This transfer file can be named as follows:

TRANSFER FILE=c:\n4py\transfer.dat

This would locate the transfer file in the c:\n4py directory called

"transfer.dat".  If you do not specify this line, it will use

the default of c:\my\ham\ftran.dat, which is the value I use.

You can select this file interface or the Ten-Tec file interface

with a line in [OPERATING STATUS] called "Ten-Tec File Interface=".

Setting this to zero, the default, selects the N4PY file interface.

Setting this to one, selects the Ten-Tec file interface.  For

explanation of the Ten-Tec file interface see the Ten-Tec

documentation on their web site.

The format of the N4PY file interface file as expressed in "C" is as follows:

struct ftran

{

char flag;

char rcvfreq[7];

char rcvmode[4];

char rcvBW[7];

char xmtfreq[7];

char xmtmode[4];

char xmtBW[7];

}

offset 0 - Flag to indicate new data from an external program.

If this has the value 0x44, no new data is available.

If it is anything else, the values in the file will

cause the Pegasus to go to them.

offset 1 to 7 - ASCII value of the current receive frequency.

For example, 14.267000 Mhz would be a "1426700" without

the quotes.  Special Note - When the Ten-Tec 2 meter

transverter is used, any frequency above 100 Mhz is

expressed as that frequency minus 100 Mhz.  Thus for

a frequency of 144.150000 Mhz would be "4415000".

offset 8 to 11 - The current receive mode in ASCII.

For example, USB would be "USB " without the quotes.

offset 12 to 18 - The current receive bandwidth.  Not yet implemented.

offset 19 to 25 - ASCII value of the current transmit frequency

in tens of hertz.

For example, 14.267000 Mhz would be a "1426700" without

the quotes.  Special Note - When the Ten-Tec 2 meter

transverter is used, any frequency above 100 Mhz is

expressed as that frequency minus 100 Mhz.  Thus for

a frequency of 144.150000 Mhz would be "4415000".

offset 26 to 29 - The current transmit mode in ASCII.

For example, USB would be "USB " without the quotes.

offset 30 to 36 - The current transmit bandwidth.  Not yet implemented.

You can monitor the Pegasus frequency by typing the ftran.dat file.

To set the frequency of the Pegasus, open the file, put the desired

frequency in offset 1 to 7, put a 0x01 in offset 0, and close the

file.

Hot Keys on the keyboard

<Up-Arrow>           Increase step size

<Down-Arrow>         Decrease step size

<Left-Arrow>         Tune Down

<Right-Arrow>        Tune Up

<Ctl-Left-Arrow>     Decrease Speaker Volume

<Ctl-Right-Arrow>    Increase Speaker Volume

<Alt-M>              Mute Audio

<Alt-A>              AM Mode

<Alt-U>              USB Mode

<Alt-L>              LSB Mode

<Alt-C>              CW Mode

<Alt-Q>              Quit

<Alt-space>          Accessory Input Toggle

0 - 9                Begin Entering Frequency

For comments and suggestions email to

n4py@earthlink.net

Carl Moreschi N4PY

173 Cody's Way

Franklinton, NC 27525

Please do not email or call Ten-Tec regarding this software.