Syllabus: The Examination shall consist of the following two parts:

PART I – Written Test

It shall comprise of one paper containing two sections as under:

Section A: Radio Theory and Practice

Note- Applicants holding degree in telecommunication, or electronics and electrical communications, or a degree recognized by the Central Government, as equivalent to the above degree shall be exempted from appearing in Section I of the test.

Section B: National and International Regulations applicable to the operation of amateur station and those relating to the working of station generally.

PART II – Morse

( i) Receiving, and (ii) Sending.

Detailed syllabus : Amateur Station Operator’s Grade II Examination

Part I – Written Test

Section A: Radio Theory and Practice:

Elementary Electricity and Magnetism: Elementary theory of electricity, conductors and insulators, units, Ohm’s Law, resistance in series and Parallel conductance, power and energy, permanent magnets and electromagnets and their use in radio Work; self and mutual inductance; types of inductors used in receiving and transmitting circuits, Capacitance; construction of various types of capacitors and their arrangements in series and/or parallel.

Elementary Theory of Alternating Currents :Sinusoidal alternating quantities – peak, instantaneous, R.M.S. average values, phase; reactance, Impedance; series and parallel circuits containing resistance, inductance, capacitance; power factor, Resonance in series and parallel circuits; coupled circuits; transformers for audio and radio frequencies;

Thermeonic Valves : Construction of valves; thermionic emission, characteristic curves, diodes, triodes and multi-electrode Valves; use of valves as rectifier, oscillators, amplifiers, detectors and frequency changers, power packs, stabilisation and smoothing, elementary theory and construction of semiconductor devices – diodes and transistors.

Radio Receivers : Principles and operation of T.R.F. and super heterodyne receivers, CW reception; receiver characteristics-  sensitivity, selectivity, fidelity; adjacent channel and image interference; A.V.C. and squelch/circuits; signal to noise ratio.

Transmitter: Principles and operation of low power transmitter; crystal oscillators, stability of oscillators.

Radio propagation: Wave length, frequency, nature and propagations of radio waves; ground and sky waves; skip distance; Fading.

Aerials: Common types of transmitting and receiving aerials.

Frequency Measurement: Measurement of frequency and use of simple frequency met.

Section B: National & International Radio Regulations:

(a) Knowledge of

(i) The Indian Wireless Telegraph Rules, 1973.

(ii) The Indian Wireless Telegraphs (Amateur Service) Rules, 1978.

(b) Knowledge of International Radio Regulations as relating to the operation of amateur stations with particular emphasis on the following :

Item                                                                              Provision of Radio Regulation

Designation of Emission ...	104-110

Nomenclature of the Frequency & Wavelength ... 	112

Frequency Allocation of Amateur Services ...	Article 5

Measures against interference ...        	667-677

Interference and Tests ... 	693-703

Identification of Stations ...	735-737 743, 772-773

Distress and Urgency Transmissions ... 	1389-1396, 1477-1478, 1481, 1483

Amateur Station ...	 1560-1567

Phonetic Alphabets and figure code ...  	Appendix 16

[c] Standard Frequency and Time Signals Services in the World.

(d) The following `Q' codes and abbreviations will shall have the same meaning as assigned to them in   the Convention.: QRA, QRG, QRH, QRI, QRK, QRL, QRM, QRN, QRQ, QRS, QRT, QRU, QRV, QRW, QRX, QRZ, QSA, QSB, QSL, QSO, QSU, QSV, QSW, QSX, QSY, QSZ, QTC, QTH, QTR, and QUM.

Abbreviations : AA, AB, AR, AS, C, CFM, CL, CQ, DE, K, NIL, OK, R, TU, VA, WA, WB.

The above written test is of one-hour duration. The maximum number of marks is 100 and candidate  must secure at least 40% in each section and 50% in aggregate for a pass.

Part II – Morse

Section 1 : Morse Receiving : (Speed : 5words per minute)

The test piece will consist of a plain language passage of 125 letters, five letters counting as one word. Candidates are required to receive for five consecutive minutes at the speed of 5 words per minute from a double headgear headphone receiver, international Morse signals from an audio oscillator keyed either manually or automatically. A short practice piece may be sent at the prescribed speed before the start of the actual test. Candidates will not be allowed more than one attempt in each test. The test may be written in ink or pencil but must be legible. Bad handwriting and over-writing will render a candidate liable to disqualification. More than 5 errors will disqualify a candidate.

Section 2 : Morse Sending (Speed : 5words per minute)

The test piece will consist of a plain language passage of 125 letters, 5 letters counting as one word. Candidates are required to send on an ordinary key for five consecutive minutes at the minimum speed of five words per minute. A short practice piece may be allowed before the actual test. Candidates will not be allowed more than one attempt in the test. Efforts should be made to correct all errors. However, more than 5 uncorrected errors will disqualify a candidate. The accuracy of signaling, correct formation of characters and the correctness of spacing shall be taken into account.

Note –A candidate is required to pass both in Part I and Part II. In the case of candidates qualifying in Part I only, the license shall be restricted to radiophone operations only.

Amateur Station Operators’ Grade I Examination

Part I – Written Test :

Same syllabus as for the Amateur Station Operators Grade II examination. The test is of 2 hours duration. The maximum number of marks is 100 and candidates must secure at least 50% in each section and 55% in aggregate for a pass.

Part II-Morse :

(a) Section 1 : Morse Receiving (Speed 12 words per minute)

The test piece will consist of a plain language passage of 300 characters, which may comprise of letters, figures and punctuations (Punctuations are indicated below). The average words shall contain five characters and each figure and punctuation will be counted as two characters. Candidates are required to receive for five consecutive minutes at a speed of 12 words per minute. Other conditions are the same as applicable to amateur Station Operator’s Grade Ii examination.

Note – Test piece may contain only the following punctuations Full stop; comma; Semi-colon; Break Sign;

Hyphen and question mark.

(b) Section 2 : Morse Sending (Speed 12 words per minute)

The test piece will be similar to Morse receiving test. Candidates are required to send for five consecutive minutes at speed not less than 12 words per minute. Other conditions are the same as applicable to Amateur Station Operators’ Grade II examination.

Note –A candidate is required to pass both in Part I and Part II simultaneously.

~~0~~

Morse code

Radio telegraphy by Morse code is named after its inventor SAMUAL FINLEY BREESE MORSE, 1791-1872, an American artist and promoter of the telegraph.

Hints on learning Morse code

The basic unit of time in Morse code is the length of the dot. The duration of the dash is three times that of the dot. The term element is used to include both dots and dashes. The space between two elements forming the same character is also the length of the dot. The space between letters is equal to three dots and the space between words is equal to five dots.

There is no magic, mystery or formula connected to this code. Remember you are learning the language of "SOUND".

Learn to listen to the sound of the letters and avoid memorizing dots and dashes.

Write down each letter immediately as it is received and ignore doubtful ones.

Do not guess but leave a blank space and concentrate on the next letter.

Avoid thinking back, you may lose more letters.

Practice Morse half an hour a day and think of it rest of the day.

Start writing the code as you learn and do not use capitals. This helps in picking up higher speed.

Concentrate on writing what you hear and read it later. Try and copy with eyes closed or the paper covered.

Practice Morse code with a friend together and send to one another.

Simple practice and more practice and reflex actions build. Soon you shall write automatically.

| ALPHABET | PHONETIC | Morse | ALPHABET | PHONETIC | Morse |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K L M | Alfa Bravo Charlie Delta Echo Foxtrot Golf Hotel India Juliet Kilo Lima Mike | . -… -.-. -.. . ..-. --. .... .. .--- -.- .-.. -- | N O P Q R S T U V W X Y Z | November Oscar Papa Quebec Romeo Sierra Tango Uniform Victor Whisky Xray Yankee Zulu | -. .--. --.- .-. … - ..- …- .— -..- -.— --.. |

AR	 End of message

AS	 Stand by, wait

DE	 From; this is (preceding identification)

CT	 Starting signal

HH	 Error in sending. Transmission continues with last word correctly sent

K	 Go ahead; over, reply expected. (Invitation to transmit)

NR	Fraction Bar

R	Received message

SK or VA	End of communication

Section B: National and International Regulations applicable to the operation of amateur station and those relating to the working of station generally

Amateur Service

`Amateur service' means a service of self training, inter-communication and technical investigations carried on by amateurs that is, by persons duly authorised under these rules interested in radio technique solely with a personal aim and without pecuniary interest; `amateur station', and `station' shall have the meaning respectively assigned to them in the Convention;

Radio communications may be exchanged with other stations similarly authorised.  The amateur stations are forbidden to communicate with amateur stations of countries whose administrations have notified the International Telecommunication Union of their objection to such radio communications.

Transmissions shall be made in plain language and limited to messages of a technical nature relating to tests and to remarks of personal character (excluding business affairs or transactions) in which the licensee, or the person with whom he is in communication, are directly concerned and for which, by reason of their unimportance, recourse to the public telecommunication service is not justified.

Special recordings for reproduction sinusoidal tone or tones within the audio frequency spectrum which may be either constant or steadily changing in frequency may be used

Operation of licensed amateur station:

No person other than the licensee shall be permitted to operate the licensed amateur station :

Provided that –

(a) in the presence of the licensee himself, the station may be operated by an other person holding a valid licence of comparable or higher category.  The licensee, however, shall be personally responsible for the observance of these rules as if the station is operated by him.

(b) in case of a licence issued to an authorised official of an amateur radio society or club or a school, college, or an institution or a University in India, the station may also be operated by a person who holds a licence of equivalent or higher category with the prior permission of the Central Government in writing; if the licensee keeps personal surveillance over the operation of the station.  The licensee shall be responsible for the observance of these rules.

Conditions under which third party messages are permitted

In case of failure of normal telecommunication facilities, amateurs are permitted to handle third party messages, relating to natural calamities like earthquakes, floods, cyclones, widespread fires etc.  The messages must originate from and must be addressed to District Collectors, District Magistrates, Deputy Commissioners or any other officials authorised by them. We have to inform our licensing authority regarding it.

Secrecy of Correspondence

If any message which the licensee is not entitled to receive is, nevertheless received, the licensee shall not make known or allow to be made known its contents, its origin or destination, its existence or the fact of its receipt to any person (Other than duly authorised officer of the Central Government or a competent legal tribunal) and shall not reproduce in writing, copy or make any use of such message or allow the same to be reproduced in writing, copied or made use of.

Who are eligible for amateur licences in India?

a.  The Amateur licence will be issued to Citizens of India who passes the ASOL exams with age limitations as follows:

12 years to 14 years  :  only Grade II or Restricted

14 years to 18 years  :  only Grade I, II or Restricted

Above 18 years        :  All grades

b.  Holders of Certificate of Proficiency like Radio Communication Operators'  General or Radio Telegraph Operators First or Second Class Certificate are  eligible  for  Grade II Amateur licence  without writing any exams.

c. Applicants having a Degree in Telecommunication or Electronics & Electrical    Communications or a degree recognised by the Govt. as equivalent to the above  degree is exempted from appearing in Part A of the test.

d.  Licences are also given to Radio Club, society, school, college or any  institution interested in this hobby, provided that the office bearer is    having Grade I or Advanced Licence.

e.  Foreign nationals are eligible for reciprocal licence under the following  conditions:

i. 	The country of which the applicant is a citizen should grant reciprocal licence to Indian nationals. It may also not apply if the Govt. thinks otherwise.

ii.  The applicant must be above 18 years

iii. The applicant must be  holder of appropriate category of licence

iv. The licence shall be initially granted for one year or for the validity of the visa, whichever is earlier.

MOBILE/PORTABLE STATION

Permission for operating a station as mobile from a vehicle or as a portable station from another location for demonstrations or for experiments can be obtained by Grade I or Advanced Grade operators by paying Rs.10. The application also must include the area of operation and purpose. It shall be issued upto 90 days initially and for another 90 days upon request and by paying Rs.10. The suffix "MO" must be added to the callsign followed by the location of the station.

The licencee's amateur station at the fixed location and the mobile station must not communicate with each other.

AMATEUR STATION ON BOARD SHIP

Permission can be obtained for operating the amateur station from ships registered in India. Applications for such authorisation must be accompanied by a written approval from the Master or owner of the ship. It shall be operated while the ship is in International waters or in Indian territorial waters. Its operation from another country's territorial waters shall be according to the rules of that country.  It shall not be operated while the ship is in any harbour in India.

The callsign allotted shall have suffix "MS" followed by the callsign of the ships in case of Morse Code or the name of the ship in case of voice communication. It shall be independent of the ship's radio communication, radio navigation and other safety services. It shall be operated in such a way not to cause any interference to the services of the ship. It shall have independent power supply and must discontinue operations on request from Central Govt., the Master or Radio Officer of the ship or any land station.

CALLSIGN

It is an identification of a station issued by a competent authority. The first part of the callsign is known as prefix  which indicates the country from which the operator is transmitting and last part is the suffix which normally consits of upto three alphabets which indicates the individual operator. While contacting or monitoring  stations, it is necessary to identify the station. This information is immediately known on hearing the callsign. In some cases one can also know from which part of the a country the operator is working. The callsigns of amateurs are listed in a directory known as callbook and we can refer in it to know the name and address of the amateurs. For Amateur stations, the callsigns consist of one or two letters and a single digit followed by a group of not more than three letters. Q codes, distress and other signals which may cause confusions are not issued as suffix. The callsigns has to be given at the starting and ending of each period of communication and if it exceeds 10 minutes, the callsign must be repeated. Callsign blocks are issued to all countries by ITU. Eg. Indian amateur callsigns: VU3LBS VU2LFA, VU2ONC

Standard time and frequency stations

There are some stations that broadcast only time on some standard frequencies. They are called Standard Time and Frequency stations. These stations send time pips and time announcements usually on standard frequencies like 5.000, 10.000 and 15.000 MHz. We can know the time, frequency and propagation conditions by monitoring these stations.

The callsign of the Indian station is ATA located at New Delhi, which is operated by National Physical Laboratory (N.P.L.).

Categories of licence

There shall be five categories of licences, namely :

(i)   Advanced Amateur Wireless Telegraph Station Licence;

(ii)  Amateur Wireless Telegraph Station Licence, Grade-I;

(iii) Amateur Wireless Telegraph Station Licence, Grade-II;

(iv) Restricted Amateur Wireless Telegraph Station Licence;

(v)  Short Wave Listeners' Amateur Wireless Telegraph Station Licence.

The licensee is forbidden to transmit

(a)  messages like the reproduction of broadcast programmes or tape recordings or transmissions of entertainment value or music;

(b) false or misleading calls, or signals, news, advertisements, communications of business, statements on topics of political or industrial controversy;

(c)  superfluous signals or any matter which is indecent or of obscene character or of a seditious tendency or which is grossly offensive or such as is likely to arouse racial, religious, or communal animosity;

(d)  messages for pecuniary reward or any messages for, or on behalf of third parties;

Non-Interference

(1) The amateur station shall be so designed, constructed, erected, maintained and worked as not to cause interference with any wireless telegraph service functioning, within or without India, in accordance with the provisions of the Convention or the wireless signaling between any fixed, land or mobile stations of Indian Land, Naval or Air Force or between such stations and any station abroad :

Provided that in the event of interference being caused by the station the licensee shall discontinue or restrict transmissions, pending adjustment of the equipment, on request from,-

(a) the Central Government; or (b) any land station.

(2) The licensee shall deploy all necessary means to ensure that the radiated frequency is free from harmonics, key clicks, hum and other forms of spurious emissions.

(3) The licensee shall ensure that the transmitter is not over modulated.

(4) The use of class B emissions (damped waves) is forbidden.

Logbook (Dairy of the radio service)

(1) A chronological record of all transmissions emanating from or received at the amateur station shall be kept in bound book (not loose-leaf) showing the following :

(a) Date and time of each transmission;

(b) a summary of the communications exchanged;

(c) a brief description of the experiments and tests undertaken;

(d) the callsign of station or stations with which messages have been exchanged, times and type of emission employed in each case;

(e) time of opening and closing down the amateur station;

(f) in case of portable or mobile amateur station the particulars of temporary location.

(2) All times in the log shall be stated in the Indian Standard Time.

(3) No gaps shall be left between entries in the log and they shall be made and initalled at the time of receiving and transmitting.

(4) In case the station is operated by a person other than the licensee (see rule 23), the licensee shall ensure that log is signed by that person indicating his name, callsign and licence number.

(5) Licensee shall preserve the log for a period of one year from the date of last entry therein before it is destroyed: Provided that no log shall be destroyed for such further period as the Central Government may direct.

(6) The form of log is shown in the Table attached to these conditions.

Wireless Telegraphy Apparatus

(1) The amateur station shall be equipped for reception as well as transmission except in the case of Short Wave Listeners Amateur Wireless Telegraph Station Licence when it shall be equipped for the former only.

(2) The Wireless telegraphy apparatus and other accessory equipment used or intended to be used by the licensee shall be so arranged as not to endanger the safety of licensee or other persons.

(3) The wireless telegraph apparatus shall be kept in a safe condition and housed in such manner as to preclude access to unauthorised persons.

(4) The transmitter shall be of a type that has a frequency stability comparable to that of a crystal control.

(5) Meters of standard accuracy shall be installed to measure the D.C. power input to the anode circuit of the final radio frequency stage of the transmitter.

(6) The aerial used or intended to be used shall be so erected, fixed, or placed as not to cross above or fall on to any power, telegraph or telephone line.

General Radiotelegraph and Radiotelephone Procedure

(1)(a) Before transmitting, the station shall take precautions to ensure that its emissions will not interfere with transmissions already in progress.  If such interference is likely the transmission shall not commence till there is an appropriate break in the communications is progress.

(b)The callsign endorsed in the licence shall be sent for identification at the beginning and at the end of each period of transmission. When the period of transmission exceeds 10 minutes the call sign shall be repeated.  Licensee shall not make transmission without identification or with false identification.

(c) Prolonged calls and transmissions shall be avoided.

(d) When it is necessary to spell out call sign, certain expressions, difficult words, abbreviations, figures  etc., the phonetic alphabet and figure code given in the Convention shall be used.

(2) Call and Reply Procedure

(a) The call shall consist of the call sign of the station called not more than three times;

the word DE (in case of radiotelegraphy) and the words "This is" (in case of radio telephony). the call sign of the calling station, not more than three times.

(b) The reply to call shall consist of - the call sign of the callsing station, not more than three times ;the word DE (in case of a radiotelegraphy)and the words "This us" (in case of radio telephony). the call sign of the station called, not more than three times.

(c) The call may be sent three times at intervals of two minutes; thereafter it shallnot be repeated until an  interval of 10 minutes during which the operator shall listen in the frequency band is which the call has been made.

(d) In case of general call to all stations the signal `CQ' (in case of radiotelegraphy' and the words `Hello  all stations' or the signal `CQ' (in case of radiotelephony) shall replace the call sign of the station called in the calling procedure.

General Radiotelegraph and Radiotelephone Procedure

(3) End of Transmission and Work :

(a) Transmission of a message shall be terminated by the signal AR (in case of radiotelegraphy) and the word `Over' (in case of radiotelephony).

(b)The end of work between two stations shall be indicated by each of them by means of signal VA (in case of radiotelegraph) and by the word `OUT' (or VA spoken as Victor Alfa) in case of radiotelephony.

(4) Tests :

(a) When it is necessary to make test signals either for the adjustment of a transmitter or a receiver or for any experiment, such signals shall not be continued for more than 30 seconds and shall be composed of series of VVV followed by the callsign of the station emitting the test signals. In case of radiotelephony series of VVV shall be replaces by the figures 1,2,3,4...... spoken in the figure code.

(b) For tests exceeding 30 seconds an artificial aerial shall be used.

(c) Emission of carrier wave is forbidden unless such wave is subjected to intelligible modulation.

Callsign blocks allotted to India:  ATA-AWZ, VTA-VWZ, 8TA-8YZ

Documents to be displayed at an amateur wireless station:

a. Original Licence with renewal cards if any

b. Log Book for the period of one year from the date of last entry

c. Copy of Indian Wireless (Amateur Service) rules

d. Register for Wireless equipments

EMISSIONS: (Modes of communication)

Classes of emissions are designated by groups of a minimum  of three characters.

First symbol  	: Type of modulation of main carrier

Second  "     	: Nature of signals modulating the main carrier

Third   "      	: Type of information

Here are the emissions that are usually used by amateurs in India:

SSB  Speech,	Single side band, suppressed carrier     	 J3E

FM      " 	Frequency Modulation                                 	F3E

AM      "	Amplitude Modulation                                 	A3E

CW   Morse	Hand sent, on/off keying of the carrier  	A1A

ABBREVIATIONS: As per ASOL examination syllabus:

AA 	= All After

AC 	= Alternating Current

CT	= Starting Signal

DX	= Distant Station

OK 	= All Correct

TU 	= Thank You

BN	= Between

AS  	=  Please wait.

CL   	=  Closing down

73 	= Best Regards

YL 	= Young Lady

K   	=  Go Ahead.

AB 	= All Before

CQ 	= General Call for all stations

NIL 	= I have nothing for you

IMI 	= Question Mark

R 	= Roger (Received fully)

WA 	= Word After

CALLSIGN FORMATION FOR INDIAN AMATEURS

Restricted/Grade II  	: VU2 prefix followed by 3 letter suffix ending in 				X,Y or Z

: VU3   "      "      "  "    "      "  .

Eg. VU2PPY  VU2XYZ VU3LBS

Grade I/Advanced   	: VU2 prefix with 2 letter suffix

:  "     "     "  3   "      "     not ending in X,Y or Z

Eg: VU2MY, VU2LFA

Procedure by which an amateur station establishes contact with another   station:

The General call shall consist of the word CQ three times followed by the word DE in Morse Code and THIS IS in voice, followed by the stations callsign three times and the ending signals AR K.

Eg. Morse: CQ CQ CQ DE VU3LBS VU3LBS VU3LBS AR K

Voice: CQ CQ CQ THIS IS VU3LBS VU3LBS VU3LBS CALLING CQ AND STANDING BY

The call may be sent three times at intervals of two minutes and then it shall not be repeated for another ten minutes. The reply to a call shall consist the callsign of the calling station not more than three times, the words DE in Morse Code and the words THIS IS in voice and the callsign of the station called, not more than three times. Transmission of a message shall be terminated by the signal AR in Morse Code and by the word OVER in voice. The end of work between two stations shall be indicated by the signal VA in Morse code and the word OUT in voice.

Precautions to be taken before establishing contact with an amateur station

Before transmitting, the station shall take precautions to ensure that its signals will not interfere with transmissions already in progress. If such interference is likely, the transmission shall commence only when there is an appropriate break in communications.

DIFFERENT TYPES OF SIGNALS

DISTRESS SIGNAL:

It indicates that a ship, aircraft or any other vehicle  is threatened by grave and imminent danger and  requests immediate assistance. The distress call in Morse Code consists of the letters SOS sent three times, the word DE and the callsign of the station in distress three times. In voice it is MAYDAY pronounced as the French expression "m'aider", spoken three times, the words THIS IS and the callsign of the station three times. The distress message must consist of the callsign, exact location, type of distress, type of assistance required and any other information helpful for the rescue. The distress call shall have priority over all other transmissions. All stations which hear it shall immediately stop any transmission capable of interfering with it and shall continue to listen on frequency and render any help if possible.

The distress frequency in Morse Code is 500 kHz while on voice it is 2182 kHz.

URGENCY SIGNAL:

It indicates that the calling station has a very urgent message regarding the safety of a ship, aircraft or any other vehicle, or the safety of a person. In Morse Code it consists of three repetitions of the group XXX an in voice the words PAN PAN pronounced as the French word "panne", followed by the urgency message. The urgency signal shall have priority over other communications except distress. Stations which hear it shall take  care not to interfere with that transmission.

SAFETY SIGNAL:

It is sent for giving weather warnings. In Morse Code it consists of three repetitions of the letters 'TTT' and in voice the words 'SECURTIE' followed by the safety message. It has priority  over other communications except distress and urgency signals.

TEST SIGNAL:

Test signals are sent either for the adjustment of a transmitter or a receiver. It shall be sent not more than 30 seconds. In Morse Code it shall consist of a series of VVV and in voice the figures 1,2,3,4,5...followed by the callsign of the station. The Q code for sending test signal is QSV.

Q Codes:

Q Codes are used primarily in CW. They provide an abbreviated way of asking a question or making a statement. A Q code followed by a question mark (?) asks a question. A Q signal without the ? answers the question or makes the statement.

The following are Q signals commonly used

QRA? 	What is the name of your station?

QRA VU3LBS:	The name of station is VU3LBS.

QRG? 	What's my exact frequency?

QRG 14160:	My exact frequency is 14160 KHz..

QRH? 	Does my frequency vary?

QRI? 	How is my tone (1-3)?

QRK? 	What is my signal intelligibility (1-5)?

QRL? 	Are you busy?

QRM? 	Is my transmission being interfered with….?

QRN? 	Are you troubled by static?

QRO? 	Shall I increase transmitter power?

QRP? 	Shall I decrease transmitter power?

QRQ?	Shall I send faster?

QRS? 	Shall I send slower?

QRT? 	Shall I stop sending?

QRU? 	Have you anything for me (message)?

QRV? 	Are you ready?

QRW? 	Shall I tell ______ you're calling him?

QRX? 	When will you call again?

QRZ? 	Who is calling me?

QSA? 	What is my signal strength (1-5)?

QSB? 	Are my signals fading?

QSG? 	Shall I send ______ messages at a time?

QSL? 	Can you acknowledge receipt?

QSO? 	Can you communicate with ______ direct?

QSV? 	Shall I send a series of V's ?

QSW? 	Will you transmit on ______ ?

QSX? 	Will you listen for ______ on ______ ?

QSY? 	Shall I change frequency?

QSZ? 	Shall I send each word/group more than once (Answer, send twice or ______)?

QTC? 	How many messages have you to send?

QTH? 	What is your location?

QTR? 	What is your time?

QUM? 	Is the Distress traffic over?

QSL CARDS:

QSL card is a written confirmation of a contact, exchanged between radio amateurs. These are of post card size. It must have your callsign, name and address, the particulars of the contact like date, time, frequency, RST, mode and callsign of the station contacted, power used, type of transmitter and antenna etc. Collecting these cards is also part of this hobby. For sending and receiving the cards economically, QSL Bureaus of radio clubs are used. One has to send the cards to the bureaus which will despatch it to other foreign bureaus and vice versa.

VU3LBS To Radio. Date; Time; Freq; Mode; RST; Remarks. Tnx for nice QSO, hope to cuagn, 73 de Ashok QTH: L.B.S.N.A.A., Mussoorie, Email

THE RST SYSTEM:  The RST system is used for giving reports of the signals received. R stands for Readability and has a range from 1 to 5. S stands for Signal Strength and has a range from 1 to 9. T stands for Tone and has a range from 1 to 9. In voice only R and S report is to be given and on Morse Code, Tone report also must be given. Eg. For a perfectly readable and extremely strong voice signal, we can give 59 report. Given below are the meanings of the figures used in the RST system.

| READABILITY | SIGNAL STRENGTH |
| --- | --- |
| 1 Unreadable | 1 Faint signal barely audible |
| 2 Barely readable, only some words distinguishable | 2 Very weak signals |
| 3 Readable with considerable difficulty | 3 Weak signals |
| 4 Readable with practically no difficulty | 4 Fair signals |
| 5 Perfectly readable | 5 Fairly good signals |
|  | 6 Good signals |
|  | 7 Moderately strong signals |
|  | 8 Strong signals |
| TONE | 9 Extremely strong signals |
| 1 Very rough and broad AC |  |
| 2 Very hard and broad AC |  |
| 3 Rough AC Tone, rectified but not filtered |  |
| 4 Rough tone, some traces of filtering |  |
| 5 Filtered rectified AC but strongly ripple modulated |  |
| 6 Filtered tone, definite trace of ripple modulation |  |
| 7 Near pure tone, trace of ripple modulation |  |
| 8 Near perfect tone, slight trace of modulation |  |
| 9 Perfect tone, no traces of ripple or modulation of any kind |  |

Try yourself

Q1.a.	Amateur Frequency range in 7 Mhz band is from ---------- to --------- kHz

b.	Maximum power permitted for Grade II Amateur on HF band is ----------- watts.

c.	Distress signal in R/T is -------------------.

d.	Q Code used for “Stop Transmission” is ----------------.

e.	Q Code QTH stands for ----------------------------------.

f.	Minimum age to become an Amateur Licence in India is  ----------------.

g.	Amateur Licence is issued for -------------- years.

h.	Amateur Radio Licence is issued by -----------------.

i.	VHF range starts from ---------- MHz.

j.	IMI stands for ---------------.

2. a.	What are the freq. Bands authorized to Grade II Amateurs in India and their power restrictions. ?

b.	What are the types of messages prohibited by Amateurs?

3. a.	What is the use of a Call-sign and what are the Call-sign blocks allotted to Indian Amateurs?

b.	What are phonetic alphabets and why are they necessary?

4. a.	Give the specimen of a Log Book to be maintained by Amateurs with two entries.

b.	Give the specimen of the equipment register.

5. a. 	What is an Amateur Station?

b.	What are the rules regarding the secrecy of correspondence?

6.	Give the meaning of the following ‘Q’ Codes:

QRI, QRN, QRS, QRT, QRW, QSO, QSV, QSY, QTR, QRG

7.	Write Short notes on any Three of the following:

a. Standard Frequency & Time Station,	b. QSL Cards, c. Distress Signal, d. General Call.