Suggested Compton DCS Installations

Three positions in the Fleet Office and one in the Metal Container if the building should be evacuated.

| Fleet Office |  |
| --- | --- |
| Position One |  |
| ICOM IC-2200H | 2m 65watt (should be in storage at Compton) [15 amps] |
| Diamond X-50A | 2m/440 gain vertical antenna (UHF connector) |
| StationMaster Collinear | 145 MHz antenna (should be available in the CFMB inventory) |
| Position Two |  |
| Alinco DR 235 MkIII | 220 25 watt (should be available at Compton) [8 amps] |
| Cushcraft ARX-220B | 220 gain antenna |
| StationMaster Collinear | 220 MHz antenna (should be available in the CFMB inventory) |
| Astron RS35A 12v power supply for positions one and two [25 amp cont.] |  |
| Position Three |  |
| Kenwood TS-2000 | HF/10m/6m/2m/440 (should be in storage at Compton) Includes built-in HF antenna switch [20.5 amps] |
| Comet-NCG CHA-250B | 80m through 6m vertical antenna |
| HyGain DX-88 | 80m – 10m Trap Vertical (should be available in the EOB inventory) |
| HyGain RRK-88 | Roof Radial System for DX 88 Vertical Antenna |
| Yaesu YA-30 | 1.8 – 30 MHz Terminated Folded Dipole, horizontally polarized for NVIS |
| B&W BWDS-90N | 1.8 – 30 MHz Terminated Folded Dipole, horizontally polarized for NVIS |
| Diamond X-50NA | 2m/440 gain vertical antenna (N connector) |
| Comet CF-416C duplexer | Couples antenna to the 2m and 440 TS-2000 ports |
| Astron RS35A 12v power supply for the TS-2000 by itself [25 amp cont.] |  |

| Metal Container |  |
| --- | --- |
| One Position |  |
| Yaesu FT 8900 | Quad band 2m/440/6m/10m (should be available in the EOB inventory) [8.5 amps] |
| Diamond Antenna CR8900A | Quad band mobile vertical antenna (UHF connector) |
| Permanent PL259 mount | Required for a good ground to the container structure. Exact mount is dependant on how it would be attached to the container. Could be a commercial mobile mount or as simple as an L bracket and bulk head barrel UHF connector. Do not recommend mag mount. |
| Astron RS20A 12v power supply [16 amps cont.] |  |