Yes.    7-E-1 is an obsolete hangover from the mid 1970s when packet was

born.  At that time, before personal computers, the dumb terminals and

Teletype machines widely used with minicomputers (and also with and

early TNCs) almost universally used 7 bits even parity.   This was

before personal computing and binary files with 8-bit bytes.  [Mass

storage was something you did with punched paper tape with 7 holes

across the width of the tape.]  Somehow this configuration persists to

this day as the default setting in TNCs.    For operation on APRS, you

want 8-bit transparency.   For most TNC2 devices and similar, the TNC

init file needs to issue the commands

8BIT OFF   (meaning don't mask incoming 8-bit bytes off to 7 bits)

AWLEN  8   (meaning use 8-bit long full bytes)

I have modified the final release of the official TAPR firmware (Ver

1.1.9) to power up in 8-N-1 instead of the original 7-E-1 mode which

makes it much more convenient for modern computer applications.    A

binary file of this modified firmware, which can be used to program an

EPROM, is on my website at:

< http://wa8lmf.net/miscinfo/index.htm >

On this download page of miscellaneous files and reference sheets, look

for the file

TNC2-119b-8N1Mod.zip

As a added bonus, this release of the TNC2 firmware includes KISS mode,

and alsd the first (and last) explicit support for APRS in the form of a

dumb APRS tracker mode.  From an attached terminal, you command the TNC

into the tracker mode, disconnect the terminal and then connect a

NMEA-format GPS to the serial port.  The TNC will then transmit raw NMEA

strings over the air in a manner similar to the dumb tracker mode on a

KPC3.

Just above this item in my MISCINFO list  is a quick hack I threw

together years ago in compiled Quickbasic (i.e. standalone .EXE) to send

the command sequence to get a TNC out of KISS mode.  Look for

KISS_OFF.EXE

Rich Schiller wrote:

>

>

> Don;t know if this helps or not, but the serial number for the TNC is

> 27446.  There are also labels, handwritten, on some of the e-proms,

> they are as follows:

>

> State 1.09

> WA4FLR-8 FAY

> TheNet 2.11 st

>

TheNet is a replacement EPROM for the standard TNC2-style firmware, to

use the TNC as a dedicated packet (not APRS!) digipeater.  The TNC is

never going to function as a normal end-user unit with this firmware in

it.    Remove this EPROM, erase it and reprogram it with the firmware I

describe above.

tim_cunningham at mindspring.com wrote:

> Rich,

>

> You must replace the TheNet EPROM with a TAPR  EPROM, a Paccomm EPROM,

> or a KISS (JKISS) EPROM. The Paccomm EPROM v5.0 is still available

> from Pac-Comm at http://paccomm.com/

>

> The KISS EPROM would work best if you plan to use the TNC with any of

> the APRS software programs.

>

>

KISS mode will work with UI-View, APRSplus and Xastir.      It WON'T

work with the original DOSaprs, APRSpoint or WinAPRS.  (None of these

programs support KISS-mode operation.)

--

Stephen H. Smith    wa8lmf (at) aol.com

EchoLink Node:      14400    [Think bottom of the 2M band]

Home Page:          http://wa8lmf.com  --OR--   http://wa8lmf.net

NEW!   TNC Test CD

http://wa8lmf.net/TNCtest

JavAPRS Filter Port 14580 Guide

http://wa8lmf.net/aprs/JAVaprsFilters.htm

"APRS 101"  Explanation of APRS Path Selection & Digipeating

http://wa8lmf.net/DigiPaths

Updated "Rev G" APRS            http://wa8lmf.net/aprs

Symbols Set for UI-View,

UIpoint and APRSplus: