From ata-owner  Thu Jan 13 14:40:32 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA16220; Thu, 13 Jan 94 14:40:32 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA16213; Thu, 13 Jan 94 14:40:15 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id OAA20913; Thu, 13 Jan 1994 14:34:29 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 13 Jan 1994 14:24:57 EST

Date:      Thu Jan 13 14:24:57 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d35a00a.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   what gives here? multiple slave drives?

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 01-13-94 02:08:47 PM

_To: ata@dt.wdc.com@internet

_To: sff_reflector@Sun.COM@internet

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: what gives here? multiple slave drives?

I would not normally post something like this to the reflectors, but, I have

now received three inquires about this in the last week.  These inquiries claim

that there is a "specification" floating around that tells how to have more

than one slave device on the ATA interface (single master, multiple slaves).  I

am not aware of such a specification and I don't think it is possible to do

such a thing without an unreasonable amount of extra hardware in the host

adapter, however, I admit that this would not prevent someone from publishing

such a "specification".  Anyone know anything about this?

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Thu Jan 13 16:25:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17513; Thu, 13 Jan 94 16:25:16 PST

Received: from rutgers.edu by dt.wdc.com (4.1/SMI-4.1)

id AA17506; Thu, 13 Jan 94 16:25:13 PST

Received: from cbmvax.cbm.commodore.com by rutgers.edu (5.59/SMI4.0/RU1.5/3.08)

id AA13970; Thu, 13 Jan 94 19:25:08 EST

Received: by cbmvax.cbm.commodore.com

(5.65/UUCP-Project/Commodore - Ultrix 4.3 Ver 07/12/93)

id AA23469; Thu, 13 Jan 94 19:26:13 -0500

Date: Thu, 13 Jan 94 19:26:13 -0500

From: jesup@cbmvax.cbm.commodore.com (Randell Jesup)

Message-Id: <9401140026.AA23469@cbmvax.cbm.commodore.com>

To: ata@dt.wdc.com

Subject: Multiple slaves

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Randy Hilton (who used to work in product qualification here) used to

talk about some spec he was writing for some such thing (he used to work for

WD in the early days of ATA I think).  He was laid off earlier this year.  I

don't know who he works for now.  Perhaps this is his spec and he's working

for one of the drive venders (I suspect he is).

Randell Jesup

GNU Emacs is a LISP operating system disguised as a word processor.

- Doug Mohney, in comp.arch

Randell Jesup, Jack-of-quite-a-few-trades, Commodore Engineering.

jesup@cbmvax.commodore.com or rutgers!cbmvax!jesup	BIX: rjesup

Disclaimer: Nothing I say is anything other than my personal opinion.

From ata-owner  Mon Jan 17 10:17:39 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17227; Mon, 17 Jan 94 10:17:39 PST

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA17212; Mon, 17 Jan 94 10:17:29 PST

Received: from hp427u.tus.ssi1.com by relay2.UU.NET with SMTP

(5.61/UUNET-internet-primary) id AAvzgp02902; Mon, 17 Jan 94 12:58:47 -0500

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id JAA25209; Mon, 17 Jan 1994 09:53:05 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 17 Jan 94 09:46:39 PST

Date: 17 Jan 94 09:29:48 PST

From: "Steve Finch" <uunet!vax2t.tus.ssi1.com!TFINCHS%A1.VAX2T.mrouter>

Subject: ATA Project Proposals

To: "ata@dt.wdc.com" <hp427u.tus.ssi1.com!ata@dt.wdc.com>,

"atapi@dt.wdc.com" <hp427u.tus.ssi1.com!atapi@dt.wdc.com>,

"scsi@wichitaks.ncr.com" <hp427u.tus.ssi1.com!scsi@wichitaks.ncr.com>,

"sff_reflector@sun.com" <hp427u.tus.ssi1.com!sff_reflector@sun.com>

Message-Id: <"84927171104991/224076@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To: 	X3T10 Membership

From:	    Steve Finch

Silicon Systems, Inc.

Date:	    January 14. 1994

Subject:    Proposal for Proposed Project -- ATA3-PH

_________________________________________________________________________

Per the agreement of the ATA Working Group meeting on January 10, 1994, I

am submitting this project proposal as one of four new projects which will

constitute the work on ATA-3 standards.  I request that the discusion and,

hopefully, forwarding of these project proposals be accomplished at the

March X3T10 plenary meeting.

Respectfully,

Steve Finch

Chair, ATA Working Group

1. Identification of Proposed Project

1.1 Title:  ATA-3 Physical Interface (ATA3-PH)

1.2 Proposer

This project is proposed by the X3T10 Technical Committee.  For additional

information, please contact John B. Lohmeyer, X3T10 Chairman, at NCR

Corporation, 3718 N. Rock Rd., Wichita, KS 67226 (Phone: 316-636-8703 FAX:

316-636-8889) or Dal Allan, X3T10 Vice-Chairman at ENDL, 14426 Black Walnut

Court, Saratoga, CA 95070 (Phone: 408-867-6630 Fax: 408-867-2115).

1.3 Date Submitted:   January 14, 1994

1.4 Project Type:   Development

2. Justification of Proposed Standard or Technical Report

2.1 Needs

The draft standard for AT Attachment has been completed and the draft for

the ATA Extensions (ATA-2) is near completion, but as the popularity of the

interface has increased, its application area has grown outside the

originally intended purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of

the ATA interface and to take advantage of the huge installed BIOS (Basic

Input/Output System) base, and software.  Support is needed for traditional

HDD devices plus additional devices such as CD-ROM and tape devices.  It is

expected that the SCSI-3 command sets can be used for these new device

types. In addition, support is also needed for removable media devices and

expanded number of cabling options.  The resulting increase in complexity

has made it desirable to divide the ATA standard into a hiarchical set of

standards, of which the ATA-3 Physical Interface is one.

+--------+---=+

| CSI-3            |   |    ATA-3   Multimedia  | |  |   |

| Command Set | | Command Sets|   | Comt |

+--- +------+------+   +======+======+

|

+-------+-------+                 |

|

+======+===           +======+======+

-3 P        |   ATA-3     ||        ======+======+           +=

|

+------------+------        |

****

*   AT  ter          *************

2.2 Recommended Scope of Standard or t

The proposed standard shall main degreecompatibil whtes:

a)   Provide a means to support thAT Attachment protovar

s such as thaly-emeng requiresrbilities which fit ntended to require

changes to presentlled devices or existing software.  It iat this proposed

standard woused to pnal capabi

2.3 Existing Practice in f Proposed Standard or Toposed project involves

evolutionary expansion of the draft AT

Attachment stdards to provide additional capabilities.

2.4 Expected SliStandard Th

Respect to Current and Potential Technological Advance

The nature of the proposed projechat the AT Attachighly copatible growth

path.  This will insurthat current

iments in AT Attachment are provided with more stability in the face

of tl devel3. Description of Proposed Project

3.1 Type of Doct (Standard or Tel Report):   StaDefinition of Concepts and

Special Terms (if any):   n3 Expectedonship with proved Xls (e.g., DBMS,

OSI)

The ATA-3 Physical Interface standard is fo closed systems.

3.4 Recommended ograoe following progm of work is planned for tA-3 Physical

Interface andard:

o Solicit continuing participasent AT Attachmentnts rough X3T10 procw

particts through presrelte comments by vite proposals

from organizations that may hav con ATA-3 Physical

Interface standard.

o Estabional requirements for ATA- Physical Interface

functional additions along with downward compatibility requirements.

o Prepare a draft standard based on proposals submitted and otinformation

gathereitial investon.

o Consider the results of ATAhyce testing as m

available to the committee through the voluntary efforts of the various

participants in X3T9 and its assigned task group.

o Sroposed stanfor further processing.

3.5 Resources - Individuals and Organization Competent in Subject Matter

The current membership of X3T10 includes representatives from all parts of

hip manufacturers to large

mainframe systecturers as well as Government agencies.  The members

of X3T10 have expressed their desire to participate and cooperate in the

development of this proposed stand.

There are sufficient resources to complete the development of this standar

without delaying work on other stans.

3.6 Recommended X3 Development Techn)

It is rd that the development work be dtask group X3T10

which is responsible for developing the draft ATA Extensions standard.

3.7 Anticiequency and Duration of Meetings

Task group X3T10 mehly.  Specific task ad hoc groups

are called as may be required for one to three days between the

regulainults are not binding.

3.8 Target Date for dpANS to X3:   December 1995

3.9 Estimated Useful Life of Standard or TechnicalReport

It is anticipated that this standard will have a life oover 10 years.

4. Implementampaon Existing User  and Investments

The proposed ATA-3 Physical Interface standard will provide an upward

growth path complementary to the existing practices and investments.  It is

likely that any isod negative impacts would occur inon-staardon or

revolution.

4.2 Impact on Splier Products and Support

The proposed ATA-3 Physical Interface standard will provide an upward

growth path complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case through

non-standard evolution or revolution.

4.3 Techniques and Costs for Compliance Verification

The committee will conr the results of ATA-3 Physical Interface testing

as may be available to the committee through the voluntary efforts of the

various participants  X3 and its assigned task group.  Wtthod

all cozations of the various participants and

have for the most part been mainly an adjunct of their normal development

costs.

4.4 Legaonsideratns

No new legdcted thaottenT Attachment and in accordance with accepted X3

patent policies.

5. Closely Related Standactivities

058 Attachment Intce

X33 X3/SPARC Study Groups:   none

5.4 OtherRelated Domestic orts:   none

5.5 ISO Standards Developis anticipated that this rd will be proposed 6

Otherional Stopment Projects:   none

5.7 Recommendations for Coordinating Liaison:   none

5.8 Recommendations for Close Liaison:   none

To: 	X3T10 Membership

From:	    Steve Finch

Silicon Systems, Inan1994

Subject:    Proposal for Proposed Project -- ATA3-XPT

________________________________________________________________r the

agreement of the ATA Working p meeting on January 10, m submitting thi four

new projects which will

constitute the work on ATndards.  I reqest that tdfully, forwarding of

these project proposals be accomplished at the

poser

This project is proposed by the X3T10 Tecr additional

information, please contact John B. Lohmeyer, X3T10 ChairCorporation, 3718

N. Rock Rd., Wichita, KS 67226 (Phone: 316-636-8703 FAX:

316-6889) or Dal Allan, X3T10 Vice-Chairman at ENDL, 14426 Black Walnut

Court, Saratoga, CA 95070 (Phone: 408-867-6630 Fax: 408-867-2115).

1.3 Date Submitted:   January 14, 1993

1.4 Project Type:   Development

2. Justification of Proposed Standard or Technical Report

2.1 Needs

The draft standard for AT Attachment has been completed and the draft for

the ATA Exteions (ATAetion, but as the popularity of the

interface has increased, its application area has grown outside the

originally intended purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of

the ATA interface and to take advantage of the huge installed BIOS (Basic

Input/Output System) base, aditionces plus additional devices such as

CD-ROM and tape devices.  It is

expectets can be used for these new device

types.  In addition, support is alor removable media devices and

expanded number of cabling options.  The resulting incase complexity

has made it desirable to divide the ATA standard into a hierarchical set of

standards, of which the ATA-3 Transport Protocol is one.

+-----------------------+   +=============+

| SI-3      | | Other       |   |    ATA-3    |

| Multimedia  | | SI-3      |   |     HDD     |

| Command Set | | Command Sets|   | Command Set |

+------+------+ +------+------+   +======+======+

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           ***************

| ATA-3 Packet|           *   ATA-3     * <- Proposed

| Interface   | - - - - ->* Transport   *    Project

| Command Set |           * Protocol    *

+======+======+           ***************

|                         |

--+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

+=====+=====+

2.2 Recommended Scope of Standard or Technical Report

The proposed ATA-3 Transport Protocol standard shall maintain a high

degreeatibility with the Attachmeproviding documentation for

new capabilities including the following candidates:

a)   Provide a means to support the AT Attachment protocol on a variety of

physical interfaces such as thfor the newly-emes to support a variety of

device types on the AT

Attachment Intece.

c)   Other c fiton a broader rications.

This proposed standard is not intended to rposed Standard or Technical

Report

The propproject involves evolutionary expanf the draft AT

Attachment standards to provideabilities.

2.4 Expected Stability of Proposed Standard or Technical Report with

Respect to Current and Potental Advan of the proposed project is to insure

that AT Attachment has

ompatible growth path.  This will insure that current

investments in AT Attachment are provided with more stability in t

of technological developments.

3. Description of Proposed Project

3.1 Type of Documentndard or hnical rt):   Standard

3.2 Definition of Concepts and Special Terms (if any):ne

3.3 Expected Relationship with Approved X3 Refe Models (e.g., DBMS,

OSI)

The ATAort Protocol stanor use in closed sycommended ProgramThe following

program of work is pla3 Traocol

sSol participatit AT Attachment

participants thgh X3 new through press

releases.  Invite commenbyons and invis

from organizations that may hution to a viable AT

Protocol ard.

o Establish functional requirements for ATA-tional additions along with

downward compatibiirements.

o Prepare a draft standardubmitted and othegathered during the inital

investigationider the results of ATA3-Transport Protocol testingilable to

the committee through the voluntary efforts of the various

participants in9 anded t o Submit the draft proposed  to X3 for further

processing.

3.5 Resou - Indnt in Subject Matter

The current member3T10 atives from all parts of

the computmainframe system manufacturers as well as Government agencies.

standard

without delaying work on other standards.

3Recelopmentechnical Committees (Exisng or New)

It is recommendelopment one in task group X3T10

which is respor developing the draft ATA Extensions standard.

3cipated Frequency and Duration of Meetings

Task group X3T10 meets for bimonthlySpecific task ad hoc groups

are called as mayequired for one to three daywill have a life of over 10

years.

4. Implementation Impacts

4.1 Impices and Investments

The proposed ATA-3 Transport Protocol staide an upward

growth path complementary to the existing practices an.  It is

likely that any isolated negative impacts would occur i through

non-stion tion.4.2 Impact on SuProductSupphe proposed ATA-3 Transpootocol

standard will pard

growth path complementary to the existing pnts.  It is

likely thay isolated negative impacts would ocvolution or revolution.

4.3 Techniques and Costs for CompliaVer committee will ults of port Pe

as may be available to the voluntary efforts of thv X3T9 and its assigned

task group.th this mall costs are borne by the organizations of the various

participants and

have for the most part been mainly an of their normal

developcConsiderations

rations aretre not alreattT Attachmeand in accordance with acceptnt posely

Rlated Sds Activities

5.1 Edards:   none

5.2 X3 Standards Development Projects

X3T9.2/90-Attachmeerface

X3T94onsudy Grone

5r Related Domestic Standards Efforts:   noordinating Liaison:   none

5.8 Recommendations foon:   none

To

From:	    Steve Finch

Silicon Systems, In

Date:nuary_____________________________________________________

Per ttof the ATA Working Group meeting on January 10, 1994, I

mitting this prroposal as one of four new projects which will

constitute the work on ATA-3 standards.  I request that the discusion

and,hopefulct proposals be accomplished at the

March X3T10 plenary meeting.

Rely,

Steve Finch

C1. Identification of Proposed Project

1.1 Title:  ATA-3 Packet Inface (ATA3-PI)

1.2 Proser

This project is proposed by the X3T10 Technical Committee.  For additional

informationcontact John B. LohX3T10 Chaat NCR

Corporation, 3718 N. Rock Richita, KS 67226 (Phone: 316-636-8703 FAX:

316-636-8889) or Dal Allan, X3T10 Vice-Chairman at EFax: 408-867-21151.3

Dated:   December 14, 1993

1.4 Project Type:   Development

2. Justification of Proposed Standard or Technical Report

2.1 Needs

The draft standard for AT Attachment has beente for

the ATA Extensions (ATA-2) i near completion, but as the popularity of

rface has increased, its application area has grown outside the

originald purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of

and to take advantage of the huge installed BIOS (Basic

Input/Output System) base, and software.  Support is needed for traditional

HDD devices plus additional devices such as CD-ROM and tape devices.  It is

expected that the SCSI-3 command sets can ees.  In addition, support is

also neede for reable

expanded number of cabling options.  The resulting increase in complexity

has made it esirable to divideA standard into ical set of

ich the ATA-3 Packet Interface is one.

+------CSI-3      | | Other       |   |    ATA-3    |

|ia  | | SCSI-3      |     |

| Command Set | | Command Sets|                   |

+-------+-------+                 |

|                     *****           +======+======+

Propose ATA-3 Packet*          ATA-3     |

Project * Interface   * - - - - ->   |

Command Set *           | Protocol    |

***********           +======+======+

|

+-----------+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

| Interface |

+===========+

2.2 Recommended Scope of Standard or Technical Report

The proposed ATA-3 Packet Interface standard shall maintaegree

of compatibility with the AT Attachment while providing documentation for

new capabilities including the following candidates:

a)   Provide a means to support a variety of device tutilizing a

packetized command block in a device independe

b)   Other bilitwhich fit within the general scope of implementing

the AT Attachment Interfapplications.

This proposed standard is not intended to requirechanges to presently

installed devices or existing software.  It is intended that this proposed

would be used to provide capabilities.

2.3 Existing Practice in Area of Proposed Standard or Technical Report

The proposed project involves evolutionary expansion of the draft AT

Atchment standards to provide addith

Respect to Current and Potential Technological Advance

The nature of the proposed project is to insure that the AT Attachment has

an upward, highly compatvestments in AT Atachment are prith more stability

in the face

of technological dents.

3. Description of Proposed Project

3.1 Type of Document (Standard or Techandard

3.2 Definipecial Terms (if any):   none

3.3 Expected Relationship witpproved X3 Reference Models (e.g., DBMS,

OSI)he Aet Interface standard isin closed systems.

3.4 Recommended Program of Worwng program of work is planned for the ATA-3

Packet Interstandard:

o Solicit continuing participation by the present AT Attachment

participaprocedures and new participants through press

releases.  Invite comments by end-user organizations and invite proposals

from organizations that may have a contribution to a viable ATA-3 Packet

terface standard.

o Establish functioal requirements for ATA-3 Packet Interface functional

additions along with downward compatibility requirements.

o Prepare a drat standard based on proposals submitted and other

information gathered during the initial investigation.

o Consider the results of ATA3-Pay be

available to the committee through the voluntary efforts of the various

participants in X3T9 and its assigned task group.

o Submit the draft proposed standard to X3 for further processing.

3.5 Resources - Individuals and Organizations Competent in Subject Matter

The current membership of X3T10 includes representatives from all parts of

the computer industry from semiconductomanufacturers to large

mainframe system manufacturers as well as Government agencies.  The members

of X3T10 have expressed their desire toate and coopeevelopment of this

proposed standard.

There are sufficient resources to complete the development of this standard

without delaying work on other standards.

3.6 Recommended X3 Development Technical Committees (Existing or New)

It is recommended that the development work be done in task group X3T10

which is responsible for developing the draft ATA Extensard.

3.7 Anticipated Frequency and Dur of Meetings

Task group X0 meetsbimonthly.  Specific task ad hoc groups

are called as may be required for one to three days between the regular

meetings but their results are n3.8 Target Date for dpANS to X3 (Milestone

10):   Deceer 1995

3.9 Estimated Useful Life of Standard or Technical Reportnticipated that

this standard will have a life of over 1ing practices and investments.  It

is likely

that any isolated negative impacts would occur in any se through non-

standard evolution or revolution.

4.2 Impact on Suppl proposed ATA-3 Packet Interfacandard will provide an

upward growth

path complementary to the existing practices and investments. It is likely

that any isolated tive impacts would occ any case through non-

standard evolution or revol.

4.3 Techniques and Costs for Compliance Verification

The committee will consider the results of ATA-3 Packet Interface testing

as may be availammittee through the voluntary efforts of the

various participants in X3T9 and its assigned task group.  With this method

all costs are borne by the organizations of the various participants and

have for the most pan adjunct of their normal development

costs.

4.4 Legal Considerations

No new legal considerations are expected that are not already attendant

with AT Attachment and in accordance with accepted X3 patees.

5. Closely Related Standards Activities

5.1 Existing Standards:   none

5.2 X3 Standards Development Projects

X3T9.2/90-058 Attachment Interface

X3T9.2/91-104 ATA Extensions

5.3 X3/SPARC Study Groups:   none

5.4 O Efforts:   none

5.5 ISO Standards Development Projects

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6 Other Related International Standards Development Projects:   none

5.7 Recommendations for Coordinating Liaison:   none

5.8 Recommendations for Close Liaison:   none

To: 	X3T10 Membership

From:	    Steve Finch

Silicon Systems, Inc.

Date:	    January 14. 1994

Subject:    Proposal for Proposed Project -- ATA3-BC

________________________________________________________________________

Per the agreement of the ATA Working Group meeting on January 10, 1994, I

am submitting this project proposal as one of four new projects which will

constitute the work on ATA-3 standards.  I request that the discusion and,

hopefully, forwarding of these project proposals be accomplished at the

March X3T10 plenary meeting.

Respectfully,

Steve Finch

Chair, ATA Working Group

1. Identification of Proposed Project

1.1 Title:  ATA-3 Block Command Set (ATA3-BC)

1.2 Proposer

This project is proposed by the X3T10 Technical Committee.  For additional

information, please contact John B. Lohmeyer, X3T10 Chairman, at NCR

Corporation, 3718 N. Rock Rd., Wichita, KS 67226 (Phone: 316-636-8703 FAX:

316-636-8889) or Dal Allan, X3T10 Vice-Chairman at ENDL, 14426 Black Wa,a,

CA 95070 (Phone: 408-867-6630 Fax: 408-867-2115).

1.3 Date Submitted:   December 14, 1993

1.4 Project Type:   Development

2. Justification of Proposed Technical Report

2.1 Needs

The draft standard for AT Attachment has been completed and the draft for

the ATA Extensions (ATA-2) is near completion, but as the popularity of the

interface has increased, its application area has grown outside the

originally intended purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to en the scope and applicatin area of

the ATA interface and to take advantage of theBIOS (Basic

Input/Output System) base, and or traditional

HDD devices plus additional devices such as CD-ROM and tape device

expected that the SCSI-3 command sets can be used for these n

types.  In addition, support is also needed for removable media devices and

expanded number of cabling options.  The resulting increase in complexity

has mable to divide the ATA standard into a hierarchical set of

standards, of which the ATA-3 Block Command Set is one.

+-------------+ +-------------+   ***************

| SCSI-3      | | Other       |   *    ATA-3    *

| Multimedia  | | SCSI-3      |   *     HDD     *

| Command Set | | Command Sets|   * Command Set+------+ +------+------+

***************

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           +======+======+

| ATA-3 Packet|           |   ATA-3     |

| Interface   | - - - - ->| Transport   |

| Command Set |           | Protoc

+======+======+           +======+======+

|                         |

+------------+------------+

|

|

+=====+====                     |   ATA-3   |

| Physical  |

| Interface |

+===========+

2.2 Recommended Scope of Standard or Technical Report

The proposed ATA-3 Block Command Set standard shall maintain a high degree

of compatibility with the AT Attachment while providing documentation for

new capabilities including the following candidates:

a)   Provide a means to support traditional hard disk drives and removable

media disk drives.

b)   Other capabilities which fit within the general scope of implementing

the AT Attachment Interface on a broader range of applns.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this proposed

standrd would be used to providabilities.

2.3 Existing Practice in Area of Pinvolves evolutionary expansion of the

draft AT

Attachment standards to provide additional capabilities.

2.4 Expected Stability of Proposed Standard or Technical Report with

Respect to Current and Potential Technological Advance

The nature of the proposed project is ure that the AT Attachment has

an upward, highly compatible growth path.  This will insure that current

investments in AT Attachment are provided with moity in the face

of technological developments.

3. Description of Proposed Project

3.1 Type of Document (Standard or Technical Report):   Standard

3.2 Definiton of Concepts and Special Terms (if any):   none

3.3lationship with ApproRefere., DBMS,

OSI)

The ATA-3 Block Command Set standard is for use in closed amogram of work

is planned for the ATA-3 Block Command Set

standard:

o Solicit continuing participation by the present AT Attachment

participants through X3T10 procedurw participants through press

releases.  Invite comments by end-user organizations and invite proposals

from organizations that may have a contribution to a viable ATA-3 Block

Command Set standard.

o Estanctional requirements for ATA-3 Block Command Set functional

additions along with downward compatibility requirements.

o Prepare a draft standard based on proposals submitted and other

information gatheed during the initial investigation.

o Consider esults of ATA3-Block Command Set testing as may be

available to the committee through ths of the various

participants in X3T9 and its assigned task group.

o Submitthe draft proposed standard to X3 for further processing.

3.5 Resources - Individuals and Organizations Competent in Subject Matter

The current membership of X3T10 includes representatives from all parts of

the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The members

of X3T10 have expressed their desire to participate and cooperate in the

development of this proposed standard.

There are sufficient resources to complete the development of this standard

without delaying work on other standards.

3.6 Recommended X3 Development Technical Committees (Existing or New)

It is recommended that the development work be done in task group X3T10

which is responsible for developing the draft ATA Extensions standard.

3.7 Anticipated Frequency and Duration of Meetings

Task group X3T10 meets for two days bimonthly.  Specific task ad hoc groups

are called as may be required for one to three days between the regular

meetings but their results are not binding.

3.8 Target Date for dpANS to X3 (Milestone 10):   December 1995

3.9 Estimated Useful Life of Standard or Technical Report

It is anticipated that this standard will have a life of over 10 years.

4. Implementation Impacts

4.1 Impact on Existing User Practices and Investments

The proposed ATA-3 Block Command Set standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negaive impacts would occur in any case through non-

standard evolution or revolution.

4.2 Impact on Supplier Products and SuThe proposed ATA-3 Block Command Set

standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.3 Techniques and Costs for Compliance Verification

The committee will consider the results of ATA-3 Block Command Set testing

as may be available to mmittee through the voluntary efforts of the

various participants in X3T9 and its assigned task group.  With this method

all costs are borne by the organizations of the various participants and

have for the most part been mainly an adjunct of their normal development

costs.

4.4 Legal Considerations

No new legal considerations are expected that are not already attendant

with AT Attachment and in accordance with accepted X3 patent policies.

5. Closely Related Standards Activities

5.1 Existing Standards:   none

5.2 X3 Standards Development Projects

X3T9.2/90-058 Attachment Interface

X3T9.2/91-104 ATA Extensions

5.3 X3/SPARC Study Groups:   none

5.4 Other Related Domestic Standards Efforts:   none

5.5 ISO Standards Development Projects

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6 Other Related International Standards Development Projects:   none

5.7 Recommendations for Coordinating Liaison:   none

5.8 Recommendations for Close Liaison:   none

===============================================================

Steve Finch, Silicon Systems, Inc.

PH:  714 573-6808,  FAX:  714 573-6914

email:  5723283@mcimail.com

===============================================================

From ata-owner  Mon Jan 17 17:53:41 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23787; Mon, 17 Jan 94 17:53:41 PST

Received: from cirrus.com by dt.wdc.com (4.1/SMI-4.1)

id AA23779; Mon, 17 Jan 94 17:53:36 PST

Received: from sunstorm.corp.cirrus.com (sunstorm1) by cirrus.com with SMTP id AA23889

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Mon, 17 Jan 1994 17:53:32 -0800

Received: from ss306.corp.cirrus.com by sunstorm.corp.cirrus.com with SMTP id AA05564

(5.65c/IDA-1.4.4); Mon, 17 Jan 1994 17:53:31 -0800

Received: by ss306.corp.cirrus.com (4.1-Corp/1.00)

id AA00419; Mon, 17 Jan 94 17:53:30 PST

Date: Mon, 17 Jan 94 17:53:30 PST

From: johng@cirrus.com (John Geldman)

Message-Id: <9401180153.AA00419@ss306.corp.cirrus.com>

To: ata@dt.wdc.com, scsi@wichitaks.ncr.com, sff_reflector@sun.com

Subject: ATA Meeting Notice

Sender: ata-owner@dt.wdc.com

Precedence: bulk

*************************************************************************

**************   ATA     M E E T I N G   N O T I C E   ******************

*************************************************************************

SUBJECT:	ANSI ATA SWIG MEETING

DATE:		January 28 (Friday)

TIME:		9:30AM - 4PM

PLACE:          CROWN STERLING SUITES HOTEL

ADDRESS:        901 Calaveras, Milpitas

PHONE:          408-942-0400, Ask for Cirrus Logic for special corp rate

at $89.00 with Breakfast

City:		Milpitas, Ca.

San Jose International is

the closest airport.

Direction:      Crown Sterling Suites is located on Calaveras and Hwy 680.

If you travel from San Jose Airport, you can take 880

north to Calaveras/237 exit.  Turn east at Calaveras and

drive 2 miles before it hits Hwy 680.  Crown Sterling

Suites Hotel is located to your left.  If you have any

question please contact me.

---------------------------------------------------------------------

Cirrus Logic, Inc.

Joseph Chen

Ph.  (510) 226-2101,  Fax. (510) 226-2170

E-Mail chen@cirrus.com

---------------------------------------------------------------------

From ata-owner  Tue Jan 18 08:08:18 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04447; Tue, 18 Jan 94 08:08:18 PST

Received: from MCIGATEWAY.MCIMail.com by dt.wdc.com (4.1/SMI-4.1)

id AA04440; Tue, 18 Jan 94 08:08:08 PST

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id bk21090;

18 Jan 94 14:56 GMT

Date: Tue, 18 Jan 94 09:59 EST

From: STEVE ANDERSON <STEVE_ANDERSON+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ata <ata@dt.wdc.com>

To: "SFF_REFLECTOREMS: INTERNETMBX:" <SFF+_REFLECTOR+aSUN+pCOM%CONNER@mcimail.com>

Subject:

Message-Id: <21940118145912/0003946488NA5EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Message origin:    Steve Anderson@DEVELOPMENT@COLORADO

Forwarded to:      VBRIDGE@B4north@servers[ata

EMS:INTERNET

MBX:ata@dt.wdc.com]

cc:

Comments by:       Steve Anderson@DEVELOPMENT@COLORADO

-------------------------- [Original Message] -------------------------

After reviewing the timing presented in Tom Hanan's document

of changing mode 4 from 20 to 16.6 Mb/s and assuming that this

will be the base document that the committee will work with on

January 28 I would like to make the following changes to the timing

tables.  I do note that the document was only presented by Tom

to aid the committee and does not necessary reflects WD's view but

it seems like the best place to start.

Table 6-1 Multiword DMA Timing Parameters

+---------------------------------------------------+

| Multiword DMA                       |   Mode 2    |

| timing parameters                   |    nsec     |

|                                     | Min  | Max  |

+-----+-------------------------------------+------+------|

<c>| t0  | Cycle time                     *3   | 120  |      |

| tC  | DMACK to DMREQ delay                |      | ---  |

<c>| tD  | DIOR-/DIOW-          16-bit         |  70  |      |

<c>| tE  | DIOR- data access                   |      |  50  |

| tF  | DIOR- data hold                     |   5  |      |

| tF  | DIOR- data hold                *1   |      | n/a  |

| tG  | DIOW- data setup                    |  25  |      |

| tH  | DIOW- data hold                     |  10  |      |

| tI  | DMACK to DIOR-/DIOW- setup          |   0  |      |

| tJ  | DIOR-/DIOW- to DMACK hold           |   5  |      |

<c>| tKr | DIOR- negated pulse width           |  25  |      |

<c>| tKw | DIOW- negated pulse width           |  25  |      |

| tLr | DIOR- to DMREQ delay                |      |  35  |

| tLw | DIOW- to DMREQ delay                |      |  35  |

| tZ  | DMACK- to tristate            *2    |      |  25  |

+-----+-------------------------------------+------+------+

<a> *3 This value is varible and is returned in the ID command

of each drive that supports mode 2 timing.  This is a bottom

limit.

Table 6-2 PIO timing parameters.

+-------------------------------------+------+------+

| PIO                                 |   Mode 4    |

| timing parameters                   |    nsec     |

|                                     | Min  | Max  |

+-----+-------------------------------------+------+------|

<c>| t0  | Cycle time                    *4*5  | 120  |      |

| t1  | Address valid to DIOR-/DIOW- setup  |  25  |      |

<c>| t2  | DIOR-/DIOW-      16-bit       *4    |  70  |      |

<c>|     |   Pulse width     8-bit       *4    |  70  |      |

<c>| t2i | DIOR-/DIOW- recovery time     *4    |  25  |      |

| t3  | DIOW- data setup                    |  25  |      |

| t4  | DIOW- data hold                     |   5  |      |

| t5  | DIOR- data setup                    |  20  |      |

| t6  | DIOR- data hold                     |   5  |      |

| t6  | DIOR- data hold               *1    |      |  n/a |

| t6z | DIOR- data tristate           *2    |      |  30  |

| t7  | Addr valid to IOCS16- assertion     |      |  30  |

| t8  | Addr invalid to IOCS16- negation    |      |  25  |

| t9  | DIOR-/DIOW- to address valid hold   |   5  |      |

| tA  | IORDY setup time              *3    |  25  |      |

| tB  | IORDY Pulse width                   |      | 1250 |

| tRD | Read data Valid to IORDY active     |   0  |      |

|     |   (if IORDY initially low after tA) |      |      |

+-----+-------------------------------------+------+------+

<a>*5 This value is varible and is returned in the ID command of

each drive that supports mode 4 timing.  This is a bottom

limit.

I believe that these timing changes would not affect silcon that

is in development but will allow other machines such as 50 mhz based

machines ability to meet the timing requirements.  The added note

I think will help clarify the meaning at the table level of t0 since

in talking with customer they only looked at the table not the ID descriptions

and were caught slightly off guard to it new meaning.

Thanks,

Steve

From ata-owner  Tue Jan 18 10:17:57 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06313; Tue, 18 Jan 94 10:17:57 PST

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA06302; Tue, 18 Jan 94 10:17:49 PST

Received: from hp427u.tus.ssi1.com by relay2.UU.NET with SMTP

(5.61/UUNET-internet-primary) id AAvzkb26645; Tue, 18 Jan 94 11:21:00 -0500

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id IAA29273; Tue, 18 Jan 1994 08:15:20 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 18 Jan 94 08:01:13 PST

Date: 18 Jan 94 07:49:06 PST

From: "Steve Finch" <uunet!vax2t.tus.ssi1.com!TFINCHS%A1.VAX2T.mrouter>

Subject: Comment ATA ProjProposal

To: "ata@dt.wdc.com" <hp427u.tus.ssi1.com!ata@dt.wdc.com>,

"scsi@wichitaks.ncr.com" <hp427u.tus.ssi1.com!scsi@wichitaks.ncr.com>,

"SFF_Reflector@sun.com" <hp427u.tus.ssi1.com!SFF_Reflector@sun.com>,

"atapi@dt.wdc.com" <hp427u.tus.ssi1.com!atapi@dt.wdc.com>

Message-Id: <"60945181104991/224480@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

John Lohmeyer has informed me of some problems with the project proposals:

1.  The address information on him is WAY OUT OF DATE.  His address is

NCR Corp., 1635 Aeroplaza Dr, Colorado Springs, CO 80916, PH: 719-573-3362,

FX 719-597-8225, EM: John.Lohmeyer@FtCollinsCO.NCR.COM.

2.  The format of the project proposal is "out of date".  While this is

true, the content and intent are still the same.  We will have to work this

out, but it should slow us down!  (Should it?)

You probably received the proposals at least twice and possibly as many as

eight times.  The first time the information was garbaged by my machine

because a stupid operator ASSUMED a functionality that was not there.

Sorry!  The second time I sent it, all was well.  I did send it to all

pertinent reflectors (ATA, SCSI, ATAPI, and SFF).  I will soon stop doing

this and only send things to the ATA reflector.  Those not signed up to the

NEW ATA reflector please read the following which is paraphrased from a WD

document passed out at the last working group:

To sign up to receive ATA reflector messages send a message to:

Majordomo@dt.wdc.com

and include, as a single line, the following text

subscribe ata <your-email-address>

If you wish also to subscribe to the ATAPI and/or SSA reflectors which WD

is also providing, add one or two additional lines to your message:

subscribe ssa <your-email-address>

subscribe atapi <your-email-address>

To send a message to the ATA reflector, send the message to:

ata@dt.wdc.com

PLEASE NOTE:  IT IS NECESSARY AND SUFFICIENT TO SEND ATA RELATED MESSAGES

TO ONLY ONE REFLECTOR:  THE ATA REFLECTOR.  Continue use of the SCSI

reflector for ATA subjects is discouraged.  Since ATA and ATAPI are so

related, there will have to be messages sent to both reflectors when those

messages are relavent to both topics.  I hope that in the near future that

ATAPI will be forwarded to the ATA Working Group and, at that time, the

ATAPI reflector can be cycled down.

=======================================================================

Steve Finch, Silicon Systems, Inc.

PH:  714 573-6808, FX: 714 573-6914, EMAIL:  5723283@mcimail.com

=======================================================================

From ata-owner  Tue Jan 18 10:18:01 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06320; Tue, 18 Jan 94 10:18:01 PST

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA06309; Tue, 18 Jan 94 10:17:53 PST

Received: from hp427u.tus.ssi1.com by relay2.UU.NET with SMTP

(5.61/UUNET-internet-primary) id AAvzkc03536; Tue, 18 Jan 94 11:42:23 -0500

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id IAA29346; Tue, 18 Jan 1994 08:36:56 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 18 Jan 94 08:23:09 PST

Date: 18 Jan 94 08:19:33 PST

From: "Steve Finch" <uunet!vax2t.tus.ssi1.com!TFINCHS%A1.VAX2T.mrouter>

Subject: Payen Joel's question on ATA and 4 drives

To: "SFF_Reflector@sun.com" <hp427u.tus.ssi1.com!SFF_Reflector@sun.com>,

"scsi@wichitaks.ncr.com" <hp427u.tus.ssi1.com!scsi@wichitaks.ncr.com>,

"ata@dt.wdc.com" <hp427u.tus.ssi1.com!ata@dt.wdc.com>

Message-Id: <"33916181104991/224494@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Joel Payne wrote

"The capability of ATA to support 4 drives is described in Western Digital

documents "Enhanced IDE implementation guide" (august 93) and "Enhanced

IDE 1993/1994".  So "Enhanced IDE" is meaningful and "Enhanced ATA" seems

like a new topic...  If anyone is aware of a given standard on this topic

let us know."

I have always found Western Digital's approach to the ATA/IDE market

interesting and confusing.  The WD documents I have seen merge together

several interesting and important concepts:  more than two drives, higher

speed transfers and CD-ROM device support.  The three are described

together and one could get the impression that they must be adopted

together.  They have succeeded in not only confusing the customers but also

the vendors as well.

The method of putting 4 drives on ATA, is really putting two ATA interfaces

on a machine and placing 2 drives on each interface.  Nothing new here, as

this has been around for a long time.  There is an annex in the dpANS ATA-2

document that lists the addresses used for some PC implementations (be

careful, as these address are believed to be incorrect and should be

updated in the very near future (by March?)).

Higher speed transfers (to 11.1 megabytes/sec PIO, 13.3 megabytes/sec DMA)

are also in the dpANS ATA-2.

The support for CD-ROM is currently an SFF activity, which is nearing

completion.  It is expected that SFF will submit the CD-ROM proposal (named

the ATA Packet Interface or ATAPI) to X3T10 for standardization very soon.

I expect that this information will be incorporated into ATA-3 activities.

I don't know how many times I've had to answer similar questions on the

phone....   If only WD wouldn't make its statements and make them seem like

they reflect the beliefs of the entire industry...  But it is often hard to

slow down the outflow from a broken dam.

The above comments are mine alone, and not a reflection of my company.

(I'll wait for the shot's fired by WD for my point of view.  Ain't it great

to live in America?)

======================================================================

Steve Finch, Silicon Systems, Inc.

PH:  714 573-6808, FX: 714 573-6914, Email:  5723283@mcimail.com

======================================================================

From ata-owner  Tue Jan 18 10:18:06 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06329; Tue, 18 Jan 94 10:18:06 PST

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA06318; Tue, 18 Jan 94 10:17:59 PST

Received: from hp427u.tus.ssi1.com by relay2.UU.NET with SMTP

(5.61/UUNET-internet-primary) id AAvzkc03559; Tue, 18 Jan 94 11:42:28 -0500

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id IAA29348; Tue, 18 Jan 1994 08:37:02 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 18 Jan 94 08:23:05 PST

Date: 18 Jan 94 08:19:27 PST

From: "Steve Finch" <uunet!vax2t.tus.ssi1.com!TFINCHS%A1.VAX2T.mrouter>

Subject: The World of ATA connectors...

To: "SFF_Reflector@sun.com" <hp427u.tus.ssi1.com!SFF_Reflector@sun.com>,

"scsi@wichitaks.ncr.com" <hp427u.tus.ssi1.com!scsi@wichitaks.ncr.com>,

"ata@dt.wdc.com" <hp427u.tus.ssi1.com!ata@dt.wdc.com>

Message-Id: <"72916181104991/224493@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

There has been many objections to having specific manufacturer's part

numbers in the proposed ATA standards.  There are two approaches to solve

this:  find the connectors defined in other documents and reference them,

or create the necessary information in the ATA document.  Personally (as

editor) I like the first approach, but I don't know where to find such

references.  Thus the reason for this message:

ATTENTION ALL CONNECTOR TYPES:

Can we find references in EIA, IEEE, ANSI, ISO or other standards for any

of the following connectors (or their equivalent)?

AMP      1-480424-0              4 pin power connector (plus pins...)

MOLEX    5484  39-27-0032        3 pin power connector

AMP      61664-1 and 62137-2     Ground connector

3M       3417-7000 (and mate)    40 pin connector

Dupont   86451 and 86455         50 (44) 2 mm connector

??       PCMCIA connector        68 pin connector

All of these are referenced in the proposed ATA standards either by part

number or reference to PCMCIA.

YOUR HELP IS REQUESTED:

1)  Can you find a standard that contains any of these?

2)  Are you willing to provide or help provide the necessary drawings and

dimensions for the same?

As an IC vendor, I really didn't want to send this message as I'll never

need them, but it will benefit the standards to solve the problem.

Thanks in advance....

Steve Finch, SSI

(Please respond to the SCSI reflector.  Thanks)

From ata-owner  Tue Jan 18 10:18:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06346; Tue, 18 Jan 94 10:18:16 PST

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA06327; Tue, 18 Jan 94 10:18:04 PST

Received: from hp427u.tus.ssi1.com by relay1.UU.NET with SMTP

(5.61/UUNET-internet-primary) id AAvzkc16966; Tue, 18 Jan 94 11:42:26 -0500

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id IAA29344; Tue, 18 Jan 1994 08:36:49 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 18 Jan 94 08:23:13 PST

Date: 18 Jan 94 08:19:42 PST

From: "Steve Finch" <uunet!vax2t.tus.ssi1.com!TFINCHS%A1.VAX2T.mrouter>

Subject: Dan Pinvidic's ATA and memory refresh message

To: "scsi@wichitaks.ncr.com" <hp427u.tus.ssi1.com!scsi@wichitaks.ncr.com>,

"ata@dt.wdc.com" <hp427u.tus.ssi1.com!ata@dt.wdc.com>

Message-Id: <"24916181104991/224495@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The ATA interface doesn't know what refreshes are and should never care!

If a host is doing DMA and needs to momentarily stop what it is doing to

perform a refresh, there is nothing in the ATA interface standards that

prevents him from doing so...

Since ATA can be used in a wide variety of implementations, not just PCs

and not just machines which require refreshes, and since the DMA protocol

does not prevent these systems from solving their refresh issues, I don't

see that this area is of concern to the ATA interface.

The original PC's which implemented DMA did so using a DMA controller which

also performed the refresh function.  The refresh DMA channel had a higher

proirity than any of the data transfer channels, so it would stop the other

transfers, do the refresh, and then continue the transfer.  I expect that

current systems would do similar things.

I really don't think this is an ATA interface issue...

Please send comments to the ATA reflector.  Thanks.

======================================================================

Steve Finch, Silicon Systems, Inc.

PH:  714 573-6808, FX: 714 573-6914, Email:  5723283@mcimail.com

======================================================================

From ata-owner  Tue Jan 18 10:18:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06380; Tue, 18 Jan 94 10:18:42 PST

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA06344; Tue, 18 Jan 94 10:18:15 PST

Received: from hp427u.tus.ssi1.com by relay2.UU.NET with SMTP

(5.61/UUNET-internet-primary) id AAvzjz18913; Tue, 18 Jan 94 10:59:25 -0500

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id HAA29220; Tue, 18 Jan 1994 07:53:43 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 18 Jan 94 07:39:48 PST

Date: 18 Jan 94 07:33:44 PST

From: "Steve Finch" <uunet!vax2t.tus.ssi1.com!TFINCHS%A1.VAX2T.mrouter>

Subject: AGAIN! ATA Proj. Proposals

To: "SFF_Reflector@sun.com" <hp427u.tus.ssi1.com!SFF_Reflector@sun.com>,

"scsi@wichitaks.ncr.com" <hp427u.tus.ssi1.com!scsi@wichitaks.ncr.com>,

"ata@dt.wdc.com" <hp427u.tus.ssi1.com!ata@dt.wdc.com>,

"atapi@dt.wdc.com" <hp427u.tus.ssi1.com!atapi@dt.wdc.com>

Message-Id: <"44335181104991/224477@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

My last attempt really messed up some text.  Someday I'll learn how to send

my files through my system(s) and across internet and not screw it up.

Here is another try.  Sorry for the inconvenience...

=====================================================================

To: 	X3T10 Membership

From:	    Steve Finch

Silicon Systems, Inc.

Date:	    January 14. 1994

Subject:    Proposal for Proposed Project -- ATA3-PH

_______________________________________________________________________

Per the agreement of the ATA Working Group meeting on January 10, 1994,

I am submitting this project proposal as one of four new projects which

will constitute the work on ATA-3 standards.  I request that the

discusion and, hopefully, forwarding of these project proposals be

accomplished at the March X3T10 plenary meeting.

Respectfully,

Steve Finch

Chair, ATA Working Group

_______________________________________________________________________

1. Identification of Proposed Project

1.1 Title:  ATA-3 Physical Interface (ATA3-PH)

1.2 Proposer

This project is proposed by the X3T10 Technical Committee.  For

additional information, please contact John B. Lohmeyer, X3T10 Chairman,

at NCR Corporation, 3718 N. Rock Rd., Wichita, KS 67226 (Phone: 316-636-

8703 FAX: 316-636-8889) or Dal Allan, X3T10 Vice-Chairman at ENDL, 14426

Black Walnut Court, Saratoga, CA 95070 (Phone: 408-867-6630 Fax: 408-

867-2115).

1.3 Date Submitted:   January 14, 1994

1.4 Project Type:   Development

2. Justification of Proposed Standard or Technical Report

2.1 Needs

The draft standard for AT Attachment has been completed and the draft

for the ATA Extensions (ATA-2) is near completion, but as the popularity

of the interface has increased, its application area has grown outside

the originally intended purpose of direct attachment of small disk

drives to desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of

the ATA interface and to take advantage of the huge installed BIOS

(Basic Input/Output System) base, and software.  Support is needed for

traditional HDD devices plus additional devices such as CD-ROM and tape

devices.  It is expected that the SCSI-3 command sets can be used for

these new device types. In addition, support is also needed for

removable media devices and expanded number of cabling options.  The

resulting increase in complexity has made it desirable to divide the ATA

standard into a hierarchical set of standards, of which the ATA-3

Physical Interface is one.

+-------------+ +-------------+   +=============+

| SCSI-3      | | Other       |   |    ATA-3    |

| Multimedia  | | SCSI-3      |   |     HDD     |

| Command Set | | Command Sets|   | Command Set |

+------+------+ +------+------+   +======+======+

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           +======+======+

| ATA-3 Packet|           |   ATA-3     |

| Interface   | - - - - ->| Transport   |

| Command Set |           | Protocol    |

+======+======+           +======+======+

|                         |

+------------+------------+

|

|

*************

*   ATA-3   *

Proposed -->  * Physical  *

Project       * Interface *

*************

2.2 Recommended Scope of Standard or Technical Report

The proposed ATA-3 Physical Interface standard shall maintain a high

degree of compatibility with the AT Attachment while providing

documentation for new capabilities including the following candidates:

a)   Provide a means to support the AT Attachment protocol on a variety

of physical interfaces such as that for the newly-emerging requirements

for memory cards.

b)   Other capabilities which fit within the general scope of

implementing the AT Attachment Interface on a broader range of

applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this

proposed standard would be used to provide additional capabilities.

2.3 Existing Practice in Area of Proposed Standard or Technical Report

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4 Expected Stability of Proposed Standard or Technical Report with

Respect to Current and Potential Technological Advance

The nature of the proposed project is to insure that the AT Attachment

has an upward, highly compatible growth path.  This will insure that

current investments in AT Attachment are provided with more stability in

the face of technological developments.

3. Description of Proposed Project

3.1 Type of Document (Standard or Technical Report):   Standard

3.2 Definition of Concepts and Special Terms (if any):   none

3.3 Expected Relationship with Approved X3 Reference Models (e.g., DBMS,

OSI)

The ATA-3 Physical Interface standard is for use in closed systems.

3.4 Recommended Program of Work

The following program of work is planned for the ATA-3 Physical

Interface standard:

o Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through press

releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Physical Interface standard.

o Establish functional requirements for ATA-3 Physical Interface

functional additions along with downward compatibility requirements.

o Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

o Consider the results of ATA3-Physical Interface testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T9 and its assigned task group.

o Submit the draft proposed standard to X3 for further processing.

3.5 Resources - Individuals and Organizations Competent in Subject

Matter

The current membership of X3T10 includes representatives from all parts

of the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The

members of X3T10 have expressed their desire to participate and

cooperate in the development of this proposed standard.

There are sufficient resources to complete the development of this

standard without delaying work on other standards.

3.6 Recommended X3 Development Technical Committees (Existing or New)

It is recommended that the development work be done in task group X3T10

which is responsible for developing the draft ATA Extensions standard.

3.7 Anticipated Frequency and Duration of Meetings

Task group X3T10 meets for two days bimonthly.  Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8 Target Date for dpANS to X3 (Milestone 10):   December 1995

3.9 Estimated Useful Life of Standard or Technical Report

It is anticipated that this standard will have a life of over 10 years.

4. Implementation Impacts

4.1 Impact on Existing User Practices and Investments

The proposed ATA-3 Physical Interface standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.2 Impact on Supplier Products and Support

The proposed ATA-3 Physical Interface standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.3 Techniques and Costs for Compliance Verification

The committee will consider the results of ATA-3 Physical Interface

testing as may be available to the committee through the voluntary

efforts of the various participants in X3T9 and its assigned task group.

With this method all costs are borne by the organizations of the various

participants and have for the most part been mainly an adjunct of their

normal development costs.

4.4 Legal Considerations

No new legal considerations are expected that are not already attendant

with AT Attachment and in accordance with accepted X3 patent policies.

5. Closely Related Standards Activities

5.1 Existing Standards:   none

5.2 X3 Standards Development Projects

X3T9.2/90-058 Attachment Interface

X3T9.2/91-104 ATA Extensions

5.3 X3/SPARC Study Groups:   none

5.4 Other Related Domestic Standards Efforts:   none

5.5 ISO Standards Development Projects

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6 Other Related International Standards Development Projects:   none

5.7 Recommendations for Coordinating Liaison:   none

5.8 Recommendations for Close Liaison:   none

==========================================================================T

o:  	X3T10 Membership

From:	    Steve Finch

Silicon Systems, Inc.

Date:	    January 14. 1994

Subject:    Proposal for Proposed Project -- ATA3-XPT

_______________________________________________________________________

Per the agreement of the ATA Working Group meeting on January 10, 1994,

I am submitting this project proposal as one of four new projects which

will constitute the work on ATA-3 standards.  I request that the

discusion and, hopefully, forwarding of these project proposals be

accomplished at the March X3T10 plenary meeting.

Respectfully,

Steve Finch

Chair, ATA Working Group

_______________________________________________________________________

1. Identification of Proposed Project

1.1 Title:  ATA-3 Transport Protocol (ATA3-XPT)

1.2 Proposer

This project is proposed by the X3T10 Technical Committee.  For

additional information, please contact John B. Lohmeyer, X3T10 Chairman,

at NCR Corporation, 3718 N. Rock Rd., Wichita, KS 67226 (Phone: 316-636-

8703 FAX: 316-636-8889) or Dal Allan, X3T10 Vice-Chairman at ENDL, 14426

Black Walnut Court, Saratoga, CA 95070 (Phone: 408-867-6630 Fax: 408-

867-2115).

1.3 Date Submitted:   January 14, 1993

1.4 Project Type:   Development

2. Justification of Proposed Standard or Technical Report

2.1 Needs

The draft standard for AT Attachment has been completed and the draft

for the ATA Extensions (ATA-2) is near completion, but as the popularity

of the interface has increased, its application area has grown outside

the originally intended purpose of direct attachment of small disk

drives to desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of

the ATA interface and to take advantage of the huge installed BIOS

(Basic Input/Output System) base, and software.  Support is needed for

traditional HDD devices plus additional devices such as CD-ROM and tape

devices.  It is expected that the SCSI-3 command sets can be used for

these new device types.  In addition, support is also needed for

removable media devices and expanded number of cabling options.  The

resulting increase in complexity has made it desirable to divide the ATA

standard into a hierarchical set of standards, of which the ATA-3

Transport Protocol is one.

+-------------+ +-------------+   +=============+

| SCSI-3      | | Other       |   |    ATA-3    |

| Multimedia  | | SCSI-3      |   |     HDD     |

| Command Set | | Command Sets|   | Command Set |

+------+------+ +------+------+   +======+======+

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           ***************

| ATA-3 Packet|           *   ATA-3     * <- Proposed

| Interface   | - - - - ->* Transport   *    Project

| Command Set |           * Protocol    *

+======+======+           ***************

|                         |

+------------+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

| Interface |

+=====+=====+

2.2 Recommended Scope of Standard or Technical Report

The proposed ATA-3 Transport Protocol standard shall maintain a high

degree of compatibility with the AT Attachment while providing

documentation for new capabilities including the following candidates:

a)   Provide a means to support the AT Attachment protocol on a variety

of physical interfaces such as that for the newly-emerging requirements

for memory cards.

b)   Provide a means to support a variety of device types on the AT

Attachment Interface.

c)   Other capabilities which fit within the general scope of

implementing the AT Attachment Interface on a broader range of

applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this

proposed standard would be used to provide additional capabilities.

2.3 Existing Practice in Area of Proposed Standard or Technical Report

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4 Expected Stability of Proposed Standard or Technical Report with

Respect to Current and Potential Technological Advance

The nature of the proposed project is to insure that the AT Attachment

has an upward, highly compatible growth path.  This will insure that

current investments in AT Attachment are provided with more stability in

the face of technological developments.

3. Description of Proposed Project

3.1 Type of Document (Standard or Technical Report):   Standard

3.2 Definition of Concepts and Special Terms (if any):   none

3.3 Expected Relationship with Approved X3 Reference Models (e.g., DBMS,

OSI)

The ATA-3 Transport Protocol standard is for use in closed systems.

3.4 Recommended Program of Work

The following program of work is planned for the ATA-3 Transport

Protocol standard:

o Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through press

releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Transport Protocol standard.

o Establish functional requirements for ATA-3 Transport Protocol

functional additions along with downward compatibility requirements.

o Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

o Consider the results of ATA3-Transport Protocol testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T9 and its assigned task group.

o Submit the draft proposed standard to X3 for further processing.

3.5 Resources - Individuals and Organizations Competent in Subject

Matter

The current membership of X3T10 includes representatives from all parts

of the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The

members of X3T10 have expressed their desire to participate and

cooperate in the development of this proposed standard.

There are sufficient resources to complete the development of this

standard without delaying work on other standards.

3.6 Recommended X3 Development Technical Committees (Existing or New)

It is recommended that the development work be done in task group X3T10

which is responsible for developing the draft ATA Extensions standard.

3.7 Anticipated Frequency and Duration of Meetings

Task group X3T10 meets for two days bimonthly.  Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8 Target Date for dpANS to X3 (Milestone 10):   December 1995

3.9 Estimated Useful Life of Standard or Technical Report

It is anticipated that this standard will have a life of over 10 years.

4. Implementation Impacts

4.1 Impact on Existing User Practices and Investments

The proposed ATA-3 Transport Protocol standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.2 Impact on Supplier Products and Support

The proposed ATA-3 Transport Protocol standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.3 Techniques and Costs for Compliance Verification

The committee will consider the results of ATA-3 Transport Protocol

testing as may be available to the committee through the voluntary

efforts of the various participants in X3T9 and its assigned task group.

With this method all costs are borne by the organizations of the various

participants and have for the most part been mainly an adjunct of their

normal development costs.

4.4 Legal Considerations

No new legal considerations are expected that are not already attendant

with AT Attachment and in accordance with accepted X3 patent policies.

5. Closely Related Standards Activities

5.1 Existing Standards:   none

5.2 X3 Standards Development Projects

X3T9.2/90-058 Attachment Interface

X3T9.2/91-104 ATA Extensions

5.3 X3/SPARC Study Groups:   none

5.4 Other Related Domestic Standards Efforts:   none

5.5 ISO Standards Development Projects

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6 Other Related International Standards Development Projects:   none

5.7 Recommendations for Coordinating Liaison:   none

5.8 Recommendations for Close Liaison:   none

==========================================================================T

o:  	X3T10 Membership

From:	    Steve Finch

Silicon Systems, Inc.

Date:	    January 14. 1994

Subject:    Proposal for Proposed Project -- ATA3-PI

________________________________________________________________________

Per the agreement of the ATA Working Group meeting on January 10, 1994,

I am submitting this project proposal as one of four new projects which

will constitute the work on ATA-3 standards.  I request that the

discusion and, hopefully, forwarding of these project proposals be

accomplished at the March X3T10 plenary meeting.

Respectfully,

Steve Finch

Chair, ATA Working Group

________________________________________________________________________

1. Identification of Proposed Project

1.1 Title:  ATA-3 Packet Interface (ATA3-PI)

1.2 Proposer

This project is proposed by the X3T10 Technical Committee.  For

additional information, please contact John B. Lohmeyer, X3T10 Chairman,

at NCR Corporation, 3718 N. Rock Rd., Wichita, KS 67226 (Phone: 316-636-

8703 FAX: 316-636-8889) or Dal Allan, X3T10 Vice-Chairman at ENDL, 14426

Black Walnut Court, Saratoga, CA 95070 (Phone: 408-867-6630 Fax: 408-

867-2115).

1.3 Date Submitted:   December 14, 1993

1.4 Project Type:   Development

2. Justification of Proposed Standard or Technical Report

2.1 Needs

The draft standard for AT Attachment has been completed and the draft

for the ATA Extensions (ATA-2) is near completion, but as the popularity

of the interface has increased, its application area has grown outside

the originally intended purpose of direct attachment of small disk

drives to desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of

the ATA interface and to take advantage of the huge installed BIOS

(Basic Input/Output System) base, and software.  Support is needed for

traditional HDD devices plus additional devices such as CD-ROM and tape

devices.  It is expected that the SCSI-3 command sets can be used for

these new device types.  In addition, support is also needed for

removable media devices and expanded number of cabling options.  The

resulting increase in complexity has made it desirable to divide the ATA

standard into a hierarchical set of standards, of which the ATA-3 Packet

Interface is one.

+-------------+ +-------------+   +=============+

| SCSI-3      | | Other       |   |    ATA-3    |

| Multimedia  | | SCSI-3      |   |     HDD     |

| Command Set | | Command Sets|   | Command Set |

+------+------+ +------+------+   +======+======+

|               |                 |

+-------+-------+                 |

|                         |

***************           +======+======+

Proposed -> * ATA-3 Packet*           |   ATA-3     |

Project     * Interface   * - - - - ->| Transport   |

* Command Set *           | Protocol    |

***************           +======+======+

|                         |

+------------+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

| Interface |

+===========+

2.2 Recommended Scope of Standard or Technical Report

The proposed ATA-3 Packet Interface standard shall maintain a high

degree of compatibility with the AT Attachment while providing

documentation for new capabilities including the following candidates:

a)   Provide a means to support a variety of device types utilizing a

packetized command block in a device independent way.

b)   Other capabilities which fit within the general scope of

implementing the AT Attachment Interface on a broader range of

applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this

proposed standard would be used to provide additional capabilities.

2.3 Existing Practice in Area of Proposed Standard or Technical Report

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4 Expected Stability of Proposed Standard or Technical Report with

Respect to Current and Potential Technological Advance

The nature of the proposed project is to insure that the AT Attachment

has an upward, highly compatible growth path.  This will insure that

current investments in AT Attachment are provided with more stability in

the face of technological developments.

3. Description of Proposed Project

3.1 Type of Document (Standard or Technical Report):   Standard

3.2 Definition of Concepts and Special Terms (if any):   none

3.3 Expected Relationship with Approved X3 Reference Models (e.g., DBMS,

OSI)

The ATA-3 Packet Interface standard is for use in closed systems.

3.4 Recommended Program of Work

The following program of work is planned for the ATA-3 Packet Interface

standard:

o Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through press

releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Packet Interface standard.

o Establish functional requirements for ATA-3 Packet Interface

functional additions along with downward compatibility requirements.

o Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

o Consider the results of ATA3-Packet Interface testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T9 and its assigned task group.

o Submit the draft proposed standard to X3 for further processing.

3.5 Resources - Individuals and Organizations Competent in Subject

Matter

The current membership of X3T10 includes representatives from all parts

of the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The

members of X3T10 have expressed their desire to participate and

cooperate in the development of this proposed standard.

There are sufficient resources to complete the development of this

standard without delaying work on other standards.

3.6 Recommended X3 Development Technical Committees (Existing or New)

It is recommended that the development work be done in task group X3T10

which is responsible for developing the draft ATA Extensions standard.

3.7 Anticipated Frequency and Duration of Meetings

Task group X3T10 meets for two days bimonthly.  Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8 Target Date for dpANS to X3 (Milestone 10):   December 1995

3.9 Estimated Useful Life of Standard or Technical Report

It is anticipated that this standard will have a life of over 10 years.

4. Implementation Impacts

4.1 Impact on Existing User Practices and Investments

The proposed ATA-3 Packet Interface standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.2 Impact on Supplier Products and Support

The proposed ATA-3 Packet Interface standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.3 Techniques and Costs for Compliance Verification

The committee will consider the results of ATA-3 Packet Interface

testing as may be available to the committee through the voluntary

efforts of the various participants in X3T9 and its assigned task group.

With this method all costs are borne by the organizations of the various

participants and have for the most part been mainly an adjunct of their

normal development costs.

4.4 Legal Considerations

No new legal considerations are expected that are not already attendant

with AT Attachment and in accordance with accepted X3 patent policies.

5. Closely Related Standards Activities

5.1 Existing Standards:   none

5.2 X3 Standards Development Projects

X3T9.2/90-058 Attachment Interface

X3T9.2/91-104 ATA Extensions

5.3 X3/SPARC Study Groups:   none

5.4 Other Related Domestic Standards Efforts:   none

5.5 ISO Standards Development Projects

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6 Other Related International Standards Development Projects:   none

5.7 Recommendations for Coordinating Liaison:   none

5.8 Recommendations for Close Liaison:   none

=======================================================================

To: 	X3T10 Membership

From:	    Steve Finch

Silicon Systems, Inc.

Date:	    January 14. 1994

Subject:    Proposal for Proposed Project -- ATA3-BC

_______________________________________________________________________

Per the agreement of the ATA Working Group meeting on January 10, 1994,

I am submitting this project proposal as one of four new projects which

will constitute the work on ATA-3 standards.  I request that the

discusion and, hopefully, forwarding of these project proposals be

accomplished at the March X3T10 plenary meeting.

Respectfully,

Steve Finch

Chair, ATA Working Group

_______________________________________________________________________

1. Identification of Proposed Project

1.1 Title:  ATA-3 Block Command Set (ATA3-BC)

1.2 Proposer

This project is proposed by the X3T10 Technical Committee.  For

additional information, please contact John B. Lohmeyer, X3T10 Chairman,

at NCR Corporation, 3718 N. Rock Rd., Wichita, KS 67226 (Phone: 316-636-

8703 FAX: 316-636-8889) or Dal Allan, X3T10 Vice-Chairman at ENDL, 14426

Black Walnut Court, Saratoga, CA 95070 (Phone: 408-867-6630 Fax: 408-

867-2115).

1.3 Date Submitted:   December 14, 1993

1.4 Project Type:   Development

2. Justification of Proposed Standard or Technical Report

2.1 Needs

The draft standard for AT Attachment has been completed and the draft

for the ATA Extensions (ATA-2) is near completion, but as the popularity

of the interface has increased, its application area has grown outside

the originally intended purpose of direct attachment of small disk

drives to desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of

the ATA interface and to take advantage of the huge installed BIOS

(Basic Input/Output System) base, and software.  Support is needed for

traditional HDD devices plus additional devices such as CD-ROM and tape

devices.  It is expected that the SCSI-3 command sets can be used for

these new device types.  In addition, support is also needed for

removable media devices and expanded number of cabling options.  The

resulting increase in complexity has made it desirable to divide the ATA

standard into a hierarchical set of standards, of which the ATA-3 Block

Command Set is one.

+-------------+ +-------------+   ***************

| SCSI-3      | | Other       |   *    ATA-3    *

| Multimedia  | | SCSI-3      |   *     HDD     *

| Command Set | | Command Sets|   * Command Set *

+------+------+ +------+------+   ***************

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           +======+======+

| ATA-3 Packet|           |   ATA-3     |

| Interface   | - - - - ->| Transport   |

| Command Set |           | Protocol    |

+======+======+           +======+======+

|                         |

+------------+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

| Interface |

+===========+

2.2 Recommended Scope of Standard or Technical Report

The proposed ATA-3 Block Command Set standard shall maintain a high

degree of compatibility with the AT Attachment while providing

documentation for new capabilities including the following candidates:

a)   Provide a means to support traditional hard disk drives and

removable media disk drives.

b)   Other capabilities which fit within the general scope of

implementing the AT Attachment Interface on a broader range of

applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this

proposed standard would be used to provide additional capabilities.

2.3 Existing Practice in Area of Proposed Standard or Technical Report

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4 Expected Stability of Proposed Standard or Technical Report with

Respect to Current and Potential Technological Advance

The nature of the proposed project is to insure that the AT Attachment

has an upward, highly compatible growth path.  This will insure that

current investments in AT Attachment are provided with more stability in

the face of technological developments.

3. Description of Proposed Project

3.1 Type of Document (Standard or Technical Report):   Standard

3.2 Definition of Concepts and Special Terms (if any):   none

3.3 Expected Relationship with Approved X3 Reference Models (e.g., DBMS,

OSI)

The ATA-3 Block Command Set standard is for use in closed systems.

3.4 Recommended Program of Work

The following program of work is planned for the ATA-3 Block Command Set

standard:

o Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through press

releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Block Command Set standard.

o Establish functional requirements for ATA-3 Block Command Set

functional additions along with downward compatibility requirements.

o Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

o Consider the results of ATA3-Block Command Set testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T9 and its assigned task group.

o Submit the draft proposed standard to X3 for further processing.

3.5 Resources - Individuals and Organizations Competent in Subject

Matter

The current membership of X3T10 includes representatives from all parts

of the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The

members of X3T10 have expressed their desire to participate and

cooperate in the development of this proposed standard.

There are sufficient resources to complete the development of this

standard without delaying work on other standards.

3.6 Recommended X3 Development Technical Committees (Existing or New)

It is recommended that the development work be done in task group X3T10

which is responsible for developing the draft ATA Extensions standard.

3.7 Anticipated Frequency and Duration of Meetings

Task group X3T10 meets for two days bimonthly.  Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8 Target Date for dpANS to X3 (Milestone 10):   December 1995

3.9 Estimated Useful Life of Standard or Technical Report

It is anticipated that this standard will have a life of over 10 years.

4. Implementation Impacts

4.1 Impact on Existing User Practices and Investments

The proposed ATA-3 Block Command Set standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.2 Impact on Supplier Products and Support

The proposed ATA-3 Block Command Set standard will provide an upward

growth path complementary to the existing practices and investments.  It

is likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

4.3 Techniques and Costs for Compliance Verification

The committee will consider the results of ATA-3 Block Command Set

testing as may be available to the committee through the voluntary

efforts of the various participants in X3T9 and its assigned task group.

With this method all costs are borne by the organizations of the various

participants and have for the most part been mainly an adjunct of their

normal development costs.

4.4 Legal Considerations

No new legal considerations are expected that are not already attendant

with AT Attachment and in accordance with accepted X3 patent policies.

5. Closely Related Standards Activities

5.1 Existing Standards:   none

5.2 X3 Standards Development Projects

X3T9.2/90-058 Attachment Interface

X3T9.2/91-104 ATA Extensions

5.3 X3/SPARC Study Groups:   none

5.4 Other Related Domestic Standards Efforts:   none

5.5 ISO Standards Development Projects

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6 Other Related International Standards Development Projects:   none

5.7 Recommendations for Coordinating Liaison:   none

5.8 Recommendations for Close Liaison:   none

======================================================================

Steve Finch, Silicon Systems, Inc.

PH:  714 573-6808, FX:  714 573-6914, EMAIL:  5723283@mcimail.com

======================================================================

From ata-owner  Wed Jan 19 22:38:32 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06450; Wed, 19 Jan 94 22:38:32 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA06443; Wed, 19 Jan 94 22:38:21 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA17130; Wed, 19 Jan 1994 12:31:43 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Wed, 19 Jan 1994 11:30:47 EST

Date:      Wed Jan 19 11:30:46 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d3d6037.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Steve Finch's comment on Enhanced IDE

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 01-19-94 11:23:33 AM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_To: scsi@wichitaks.ncr.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: Steve Finch's comment on Enhanced IDE

First, the following are my opinions and not the opinions or position of

Seagate.

Steve Finch wrote:

>I have always found Western Digital's approach to the ATA/IDE market

>interesting and confusing.  The WD documents I have seen merge together

>several interesting and important concepts:  more than two drives, higher

>speed transfers and CD-ROM device support.  The three are described

>together and one could get the impression that they must be adopted

>together.  They have succeeded in not only confusing the customers but also

>the vendors as well.

Yep, I agree with Steve's comments.  I'm getting all kinds of calls from some

very confused

people.  It's one thing to distribute FUD to give your company a marketing

advantage but

when it is not done in the proper context, it can have the effect of damaging

an entire industry

and this is what I think is happening here.

Steve wrote:

>I don't know how many times I've had to answer similar questions on the

>phone....   If only WD wouldn't make its statements and make them seem like

>they reflect the beliefs of the entire industry...  But it is often hard to

>slow down the outflow from a broken dam.

Yep again.  I see press releases all the time from WD that imply that they are

speaking

for the entire industry or that the entire industry has "signed up" to do

something the WD way.

I have been very surprised at times by press releases that imply they were

issued by the SFF

committee or an X3T9 committee and therefore represent an industry position

when in

reality, the press release was from WD and the only people quoted in it are WD

people.

I notice that WD is first up at the ATA porition of the Tech Forum next Monday

talking about

Enhanced IDE.  Gee, I thought the Tech Forum was not an appropiate place to make

marketing presentations?  As far as I'm concerned, Enhanced IDE is a WD

marketing

thing.

Steve wrote:

>The above comments are mine alone, and not a reflection of my company.

Same for me.

Steve wrote:

>(I'll wait for the shot's fired by WD for my point of view.  Ain't it great

>to live in America?)

Sure is, but don't overlook the fact that a lot of people don't agree with

the 1st admemdment any more.

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Thu Jan 20 09:31:34 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14573; Thu, 20 Jan 94 09:31:34 PST

Received: from MCIGATEWAY.MCIMail.com by dt.wdc.com (4.1/SMI-4.1)

id AA14564; Thu, 20 Jan 94 09:31:25 PST

Received: by MCIGATEWAY.MCIMail.com id aa27657; 20 Jan 94 15:40 GMT

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id ad23406;

20 Jan 94 14:40 GMT

Date: Thu, 20 Jan 94 09:43 EST

From: STEVE ANDERSON <STEVE_ANDERSON+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ata <ata@dt.wdc.com>

To: sff_reflector <sff_reflector@sun.com>

To: "SFF_REFLECTOREMS: INTERNETMBX:" <SFF+_REFLECTOR+aSUN+pCOM%CONNER@mcimail.com>

Subject:

Message-Id: <80940120144308/0003946488NA3EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Message origin:    Steve Anderson@DEVELOPMENT@COLORADO

Forwarded to:      VBRIDGE@B4north@servers[ata

EMS:INTERNET

MBX:ata@dt.wdc.com]

VBRIDGE@B4north@Servers[sff_reflector

EMS: INTERNET

MBX: sff_reflector@Sun.COM]

cc:

Comments by:       Steve Anderson@DEVELOPMENT@COLORADO

-------------------------- [Original Message] -------------------------

After reviewing the timing presented in Tom Hanan's document

of changing mode 4 from 20 to 16.6 Mb/s and assuming that this

will be the base document that the committee will work with on

January 28 I would like to make the following changes to the timing

tables.  I do note that the document was only presented by Tom

to aid the committee and does not necessary reflects WD's view but

it seems like the best place to start.

Table 6-1 Multiword DMA Timing Parameters

+---------------------------------------------------+

| Multiword DMA                       |   Mode 2    |

| timing parameters                   |    nsec     |

|                                     | Min  | Max  |

+-----+-------------------------------------+------+------|

<c>| t0  | Cycle time                     *3   | 120  |      |

| tC  | DMACK to DMREQ delay                |      | ---  |

<c>| tD  | DIOR-/DIOW-          16-bit         |  70  |      |

<c>| tE  | DIOR- data access                   |      |  50  |

| tF  | DIOR- data hold                     |   5  |      |

| tF  | DIOR- data hold                *1   |      | n/a  |

| tG  | DIOW- data setup                    |  25  |      |

| tH  | DIOW- data hold                     |  10  |      |

| tI  | DMACK to DIOR-/DIOW- setup          |   0  |      |

| tJ  | DIOR-/DIOW- to DMACK hold           |   5  |      |

<c>| tKr | DIOR- negated pulse width           |  25  |      |

<c>| tKw | DIOW- negated pulse width           |  25  |      |

| tLr | DIOR- to DMREQ delay                |      |  35  |

| tLw | DIOW- to DMREQ delay                |      |  35  |

| tZ  | DMACK- to tristate            *2    |      |  25  |

+-----+-------------------------------------+------+------+

<a> *3 This value is varible and is returned in the ID command

of each drive that supports mode 2 timing.  This is a bottom

limit.

Table 6-2 PIO timing parameters.

+-------------------------------------+------+------+

| PIO                                 |   Mode 4    |

| timing parameters                   |    nsec     |

|                                     | Min  | Max  |

+-----+-------------------------------------+------+------|

<c>| t0  | Cycle time                    *4*5  | 120  |      |

| t1  | Address valid to DIOR-/DIOW- setup  |  25  |      |

<c>| t2  | DIOR-/DIOW-      16-bit       *4    |  70  |      |

<c>|     |   Pulse width     8-bit       *4    |  70  |      |

<c>| t2i | DIOR-/DIOW- recovery time     *4    |  25  |      |

| t3  | DIOW- data setup                    |  25  |      |

| t4  | DIOW- data hold                     |   5  |      |

| t5  | DIOR- data setup                    |  20  |      |

| t6  | DIOR- data hold                     |   5  |      |

| t6  | DIOR- data hold               *1    |      |  n/a |

| t6z | DIOR- data tristate           *2    |      |  30  |

| t7  | Addr valid to IOCS16- assertion     |      |  30  |

| t8  | Addr invalid to IOCS16- negation    |      |  25  |

| t9  | DIOR-/DIOW- to address valid hold   |   5  |      |

| tA  | IORDY setup time              *3    |  25  |      |

| tB  | IORDY Pulse width                   |      | 1250 |

| tRD | Read data Valid to IORDY active     |   0  |      |

|     |   (if IORDY initially low after tA) |      |      |

+-----+-------------------------------------+------+------+

<a>*5 This value is varible and is returned in the ID command of

each drive that supports mode 4 timing.  This is a bottom

limit.

I believe that these timing changes would not affect silcon that

is in development but will allow other machines such as 50 mhz based

machines ability to meet the timing requirements.  The added note

I think will help clarify the meaning at the table level of t0 since

in talking with customer they only looked at the table not the ID descriptions

and were caught slightly off guard to it new meaning.

Thanks,

Steve

From ata-owner  Thu Jan 20 10:14:58 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15167; Thu, 20 Jan 94 10:14:58 PST

Received: from bolero.rahul.net by dt.wdc.com (4.1/SMI-4.1)

id AA15160; Thu, 20 Jan 94 10:14:44 PST

Received: by bolero.rahul.net id AA15351

(5.67a8/IDA-1.5 for ATA Reflector <ata@dt.wdc.com>); Thu, 20 Jan 1994 10:14:39 -0800

Date: Thu, 20 Jan 1994 10:11:17 -0800 (PST)

From: Richard Kalish <rkalish@rahul.net>

Subject: Proposal for the ATA Local bus meeting

To: ATA Reflector <ata@dt.wdc.com>

Message-Id: <Pine.3.07.9401201017.C15067-e101000@bolero>

Mime-Version: 1.0

Content-Type: MULTIPART/MIXED; BOUNDARY="1435140434-1070521716-759089679:#15067"

Sender: ata-owner@dt.wdc.com

Precedence: bulk

--1435140434-1070521716-759089679:#15067

Content-Type: TEXT/PLAIN; charset=US-ASCII

Most people will probably get this two times, but that is better than not

at all.

Rick

--1435140434-1070521716-759089679:#15067

Content-Type: APPLICATION/octet-stream; name="ata-crc.doc"

Content-Transfer-Encoding: BASE64

Content-ID: <Pine.3.07.9401201039.D15067@bolero>

Content-Description:

AAthdGEtY3JjLmRvYwAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAABXREJOTVNXRAAAAAAAAAAAAAAAACIAAAAA

AKlj7uqpY+7rAAAAAAAAAAAAAAAAAAAAAAAAAAAAAACBgb1GAAD+NwAjAAAA

AAAABAAAGQAAAAAAAAAAAQAAABbyAAAgMAAAAAAAABXyAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAeAABnAAAeAABnAAAeZwAAAAAeZwAAAAAeZwAOAAAe

dQAuAAAeowAAAAAeowAAAAAeowAAAAAeowAKAAAerQAQAAAevQAAAAAevQBm

AAAfIwB4AAAeowAAAAAfmwAgAAAfuwAYAAAgBgAqAAAgMAAAAAAeZwAAAAEA

AgAAIAYAAAAAH9MAMwAAIAYAAAAAIAYAAAAAIDAAAAAAIAYAAAAAIAYAAAAA

IAYAAAAAIAYAAAAAIAYAAAAAIAYAAAAAVE86CUFUQSBMb2NhbCBCdXMgV29y

a2luZyBHcm91cA0NRlJPTToJUi4gS2FsaXNoIAk0MDgtOTU3LTcxNjkNCUEu

IFBoYW0JCTQwOC05NTctNDgwNA0NREFURToJSmFudWFyeSAyMCwgMTk5NA0N

RG9jLiAjOglYM1QxMC85NC0wMjdyMQ0NVGhpcyBpbmZvcm1hdGlvbiB3YXMg

cHJlc2VudGVkIGF0IHRoZSBsYXN0IEFUQSBMb2NhbCBCdXMgd29ya2luZyBn

cm91cCBtZWV0aW5nIGhlbGQgaW4gTWlscGl0YXMgb24gRGVjZW1iZXIgOCwg

MTk5My4gIEFsdGhvdWdoIHRoZXJlIHdhcyBvbmx5IGEgdmVyeSBsaW1pdGVk

IHRpbWUgc3BlbnQgZGlzY3Vzc2luZyBpdCwgdGhlIG1haW4gb2JqZWN0aW9u

IHZvaWNlZCB3YXMgdGhhdCB0aGlzIHdhcyB1bi1uZWNlc3NhcnkgYW5kIGNv

dWxkIGJlIGFjaGlldmVkIGN1cnJlbnRseSB1c2luZyBhIHJlYWQtbG9uZyBj

b21tYW5kLiAgSG9wZWZ1bGx5IG1vcmUgdGltZSBjYW4gYmUgc3BlbnQgZHVy

aW5nIHRoZSB1cGNvbWluZyBtZWV0aW5nIG5leHQgd2VlayBvbiB0aGlzIHN1

YmplY3QgYW5kIHdlIGNhbiBkaXNjdXNzIHdoeSB0aGlzIGNhbiBub3QgYmUg

ZG9uZSB3aXRoIHRoZSByZWFkLWxvbmcgY29tbWFuZCBhbmQgd2h5IHRoaXMg

c2NoZW1lIHdpbGwgaGVscCBpbmNyZWFzZSBkYXRhIHRyYW5zZmVyIGludGVn

cml0eS4gIFdlIGRvIG5vdCBiZWxpZXZlIHRoYXQgdGhlIGV4aXN0aW5nIEFU

QSBwcm90b2NvbCB3aWxsIGJlIHZpb2xhdGVkIG9yIHJlcXVpcmVkIHRvIGNo

YW5nZSBiZWNhdXNlIGV2ZW4gdGhvdWdoIHRoZSBzdGF0dXMgaXMgcmVhZCBi

ZWZvcmUgdGhlIGRhdGEgaXMgdHJhbnNmZXJyZWQgb24gYSBob3N0IHJlYWQs

IHdoZW4gdGhlIGhvc3QgZGV0ZXJtaW5lcyB0aGF0IHRoZSBkYXRhIGl0IHJl

Y2VpdmVkIGlzIGNvcnJ1cHRlZCwgdGhlIGhvc3Qgc2hvdWxkIGp1c3QgZXhl

Y3V0ZSBhbm90aGVyIHJlYWQgY29tbWFuZC4gIFRoZSBkcml2ZSBtYXkgdmVy

eSB3ZWxsIGhhdmUgc2VudCBnb29kIGRhdGEsIGJ1dCB0aGUgZGF0YSB3YXMg

Y29ycnVwdGVkIGR1cmluZyB0aGUgdHJhbnNmZXIuDQ09PT09PT09PT09PT09

PT09PT09PT09PT09PT09PT09PT09PT09PT09PT09PT09PT09PT09PT09PT09

PT0NICAgICAgICAgICBQcm9wb3NhbCBmb3IgRGF0YSBDUkMgYWNyb3NzIHRo

ZSBDYWJsZQ0NQVRBIFBJTyBNb2RlIDQgYW4gTXVsdGktd29yZCBETUEgTW9k

ZSAyIG5lZWQgYSByZWxpYWJsZSBtZXRob2QgdG8gdmVyaWZ5IHRoZSBkYXRh

IHRyYW5zZmVycmVkIGFjcm9zcyB0aGUgY2FibGUgd2FzIHJlY2VpdmVkIHBy

b3Blcmx5IGFuZCB3YXMgbm90IGNvcnJ1cHRlZC4gIFRoaXMgZmVhdHVyZSBo

YXMgbG9uZyBiZWVuIGEgcGFydCBvZiB0aGUgU0NTSSBpbnRlcmZhY2Ugd2l0

aCB0aGUgcGFyaXR5IGJpdCBlbmFibGluZyB0aGUgdmVyaWZpY2F0aW9uIG9m

IGVhY2ggYnl0ZSBvZiBkYXRhLiAgU2luY2UgdGhlcmUgaXMgbm8gcm9vbSBv

biB0aGUgY2FibGUgZm9yIHBhcml0eSBvbiBlYWNoIGJ5dGUgb2YgYSBkYXRh

IHRyYW5zZmVycmVkLCBBbCBhbmQgSSB3b3VsZCBsaWtlIHRvIHByb3Bvc2Ug

YSBtZXRob2QgZm9yIGRhdGEgaW50ZWdyaXR5IHZlcmlmaWNhdGlvbiBhY3Jv

c3MgdGhlIGNhYmxlLiAgVGhpcyB3b3VsZCBtZWFuICBhZGRpbmcgNCBuZXcg

Y29tbWFuZHMgZm9yIFJlYWQvV3JpdGUtTXVsdGlwbGUgKFcvQ1JDKSBhbmQg

UmVhZC9Xcml0ZSBETUEgKFcvQ1JDKSBhbmQgb25seSBiZSBmb3IgdGhlIGhp

Z2ggc3BlZWQgbW9kZXMgKDIwIE1ieXRlcy9zZWMgdHJhbnNmZXIgcmF0ZXMg

LSBvciB3aGF0ZXZlciBzcGVlZCBQSU8gTW9kZSA0IGJlY29tZXMpLiBTaW5j

ZSB0aGUgY3VycmVudCBSZWFkL1dyaXRlLU11bHRpcGxlIGFuZCBETUEgY29t

bWFuZHMgYXJlIG9wdGlvbmFsLCB0aGVzZSBuZXcgY29tbWFuZHMgd291bGQg

YWxzbyBiZSBvcHRpb25hbC4gIA0NV2UgYXJlIHByb3Bvc2luZyB0aGUgZm9s

bG93aW5nOg0NMS4JUkVBRCBNdWx0aXBsZSBXL0NSQyAtIENvbW1hbmQgQ29k

ZSBDQ2gNVGhlIG5ldyBjb21tYW5kIGZvciBSZWFkIE11bHRpcGxlIFcvQ1JD

IHdpbGwgdHJhbnNmZXIgNTE2IGJ5dGVzIG9mIGRhdGEgZnJvbSB0aGUgZHJp

dmUgdG8gdGhlIGhvc3QuICBUaGUgaG9zdCBzeXN0ZW0gIG1pY3JvcHJvY2Vz

c29yIGlzIHJlcXVpcmVkIHRvIHNjYW4gdGhlIGRhdGEgYW5kIHZlcmlmeSB0

aGUgQ1JDIHRvIGluc3VyZSB0aGUgdHJhbnNmZXIgd2FzIG5vdCBjb3JydXB0

ZWQuDQ0yLglXUklURSBNdWx0aXBsZSBXL0NSQyAtIENvbW1hbmQgQ29kZSBD

RGgNVGhlIG5ldyBjb21tYW5kIGZvciBXcml0ZSBNdWx0aXBsZSBXL0NSQyB3

aWxsIHRyYW5zZmVyIDUxNiBieXRlcyBvZiBkYXRhIGZyb20gdGhlIGhvc3Qg

dG8gdGhlIGRyaXZlLiAgVGhlIGhvc3Qgc3lzdGVtICBtaWNyb3Byb2Nlc3Nv

ciBpcyByZXF1aXJlZCB0byBnZW5lcmF0ZSBhIENSQyBhcHBlbmRlZCB0byB0

aGUgNTEyIGJ5dGVzIG9mIGRhdGEgYW5kIHRoZSBkcml2ZSBtdXN0IHZlcmlm

eSB0aGUgQ1JDIHRvIGluc3VyZSB0aGUgdHJhbnNmZXIgd2FzIG5vdCBjb3Jy

dXB0ZWQuIEEgYmFkIHRyYW5zZmVyIHdpbGwgcmVzdWx0IGluIGFuIGVycm9y

IGZsYWcgaW4gYm90aCBzdGF0dXMgcmVnaXN0ZXJzIGZvciB0aGUgc3RhdHVz

IG9mIHRoZSBvcGVyYXRpb24uDQ0zLglSRUFEIE11bHRpcGxlIERNQSBXL0NS

QyAtIENvbW1hbmQgQ29kZSBDRWgNVGhlIG5ldyBjb21tYW5kIGZvciBSZWFk

IE11bHRpcGxlIERNQSBXL0NSQyB3aWxsIHRyYW5zZmVyIDUxNiBieXRlcyBv

ZiBkYXRhIGZyb20gdGhlIGRyaXZlIHRvIHRoZSBob3N0LiAgVGhlIGhvc3Qg

c3lzdGVtICBtaWNyb3Byb2Nlc3NvciBpcyByZXF1aXJlZCB0byBzY2FuIHRo

ZSBkYXRhIGFuZCB2ZXJpZnkgdGhlIENSQyB0byBpbnN1cmUgdGhlIHRyYW5z

ZmVyIHdhcyBub3QgY29ycnVwdGVkLiBUaGlzIGNvbW1hbmQgd2lsbCBhYm9y

dCBpZiB0aGUgZHJpdmUgaXMgbm90IHNldCB1cCBmb3IgTXVsdGlwbGUgTW9k

ZSBETUEsDQ00LglXUklURSBNdWx0aXBsZSBETUEgVy9DUkMgLSBDb21tYW5k

IENvZGUgQ0ZoDVRoZSBuZXcgY29tbWFuZCBmb3IgV3JpdGUgTXVsdGlwbGUg

RE1BIFcvQ1JDIHdpbGwgdHJhbnNmZXIgNTE2IGJ5dGVzIG9mIGRhdGEgZnJv

bSB0aGUgaG9zdCB0byB0aGUgZHJpdmUuICBUaGUgaG9zdCBzeXN0ZW0gIG1p

Y3JvcHJvY2Vzc29yIGlzIHJlcXVpcmVkIHRvIGdlbmVyYXRlIGEgQ1JDIGFw

cGVuZGVkIHRvIHRoZSA1MTIgYnl0ZXMgb2YgZGF0YSBhbmQgdGhlIGRyaXZl

IG11c3QgdmVyaWZ5IHRoZSBDUkMgdG8gaW5zdXJlIHRoZSB0cmFuc2ZlciB3

YXMgbm90IGNvcnJ1cHRlZC4gQSBiYWQgdHJhbnNmZXIgd2lsbCByZXN1bHQg

aW4gYW4gZXJyb3IgZmxhZyBpbiBib3RoIHN0YXR1cyByZWdpc3RlcnMgZm9y

IHRoZSBzdGF0dXMgb2YgdGhlIHdyaXRlIG9wZXJhdGlvbi4gVGhpcyBjb21t

YW5kIHdpbGwgYWJvcnQgaWYgdGhlIGRyaXZlIGlzIG5vdCBzZXQgdXAgZm9y

IE11bHRpcGxlIE1vZGUgRE1BLg0NNS4gIAlDUkMgUG9seW5vbWlhbA1UaGUg

NCBieXRlIENSQyBwb2x5bm9taWFsIGlzIHRoZSBzYW1lIGFzIHVzZWQgZm9y

IFAxMzk0IChhbmQgbWFueSBvdGhlciBzcGVjaWZpY2F0aW9ucykgd2hpY2gg

c2hvdWxkIGJlIHVuaXZlcnNhbGx5IGFjY2VwdGFibGUgKFdlIHdpbGwgbm90

IGRpc2N1c3MgdGhlIG1lcml0cyBvZiB0aGlzIG51bWJlciBzaW5jZSB0aGVy

ZSBhcmUgb25seSBhIHZlcnkgZmV3IHBlb3BsZSB3aG8gZnVsbHkgdW5kZXJz

dGFuZCB0aGUgcHJvcyBhbmQgY29ucyBvZiB0aGlzIHBvbHlub21pYWwgdmVy

c3VzIGFub3RoZXIsIGFuZCB3ZSBhcmUgbm90IGluIHRoYXQgZ3JvdXApLiBU

aGUgQ1JDIGhhcyB0aGUgZm9sbG93aW5nIHByb3BlcnRpZXM6DQ2lCUdlbmVy

YXRvciBQb2x5bm9taWFsOg0JRyh4KSA9ICB4XjMyICsgeF4yNiArIHheMjMg

KyB4XjIyICsgeF4xNiArIHheMTIgKyB4XjExICsgeF4xMCArIHheOCArIHhe

NyArIHheNSArIHheNCArIHheMiArIHggKyAxDQ2lCUNSQyBSZWdpc3RlciBp

cyBwcmVzZXQgdG8gYWxsIDHVcyBiZWZvcmUgZ2VuZXJhdGlvbiBhbmQgY2hl

Y2tpbmcuDQ2lCUdlbmVyYXRlZCBDUkMgUmVkdW5kYW5jeSBQb2x5bm9taWFs

Og0JUiQoeCkgPSBvbmXVcyBjb21wbGVtZW50IG9mIHRoZSByZW1haW5kZXIg

IHBvbHlub21pYWwgb2YgZGVncmVlIGxlc3MgdGhhbiAzMiwgaS5lLiB0aGUg

cmVtYWluZGVyIGlzIGludmVydGVkIGJlZm9yZSBiZWluZyBzaGlmdGVkIG91

dC4NDaUJQ2hlY2sgUG9seW5vbWlhbCBwcm9kdWNlZCBieSB0aGUgcmVjZWl2

ZXIgdXBvbiByZWNlaXZpbmcgYW4gZXJyb3ItZnJlZSBzZXF1ZW5jZSAoIGRh

dGEgYW5kIENSQyByZWR1bmRhbmN5ICk6DQlDKHgpID0gIHheMzEgKyB4XjMw

ICsgeF4yNiArIHheMjUgKyB4XjI0ICsgeF4xOCArIHheMTUgKyB4XjE0ICsg

eF4xMiArIHheMTEgKyB4XjEwICsgeF44ICsgeF42ICsgeF41ICsgeF40ICsg

eF4zICsgeCArIDENDTYuCUdlbmVyYWwgUmVxdWlyZW1lbnRzDVRoZSBkcml2

ZSBtdXN0IGJlIHByZXZpb3VzbHkgc2V0IGluIHRoZSBtdWx0aXBsZSBtb2Rl

IG9yIGFsbCBvZiB0aGUgYWJvdmUgY29tbWFuZHMgd2lsbCBhYm9ydC4gIFRo

ZXNlIGNvbW1hbmRzIHdpbGwgYmUgdXNlZCBvbiBhIHNlY3RvciBieSBzZWN0

b3IgYmxvY2sgb2YgZGF0YSBhbmQgdmVyaWZpZWQgYnkgdGhlIGhvc3QgbWlj

cm9wcm9jZXNzb3IuICBUaGlzIHdpbGwgZWxpbWluYXRlIHRoZSBleHRyYSBj

aXJjdWl0cnkgcmVxdWlyZW1lbnQgb2YgYSBob3N0IGFkYXB0ZXIgdG8gaGFu

ZGxlIHRoZSBDUkMgb24gdGhlIGRhdGEuIFNpbmNlIHRoZSBidXJkZW4gZm9y

IGNoZWNraW5nIHRoZSBkYXRhIGlzIHVwb24gdGhlIGhvc3QsIHRoaXMgd2ls

bCByZXF1aXJlIGEgQklPUyBvciBkcml2ZXIgd2hpY2gga25vd3MgYWJvdXQg

dGhlIG5ldyBjb21tYW5kcyBhbmQgYWxzbyBhIElkZW50aWZ5IERyaXZlIGZl

YXR1cmUgdG8gbGV0IHRoZSBob3N0IGtub3cgdGhhdCB0aGUgZHJpdmUgY2Fu

IHN1cHBvcnQgdGhlIG5ldyB0cmFuc2ZlciBtb2Rlcy4NDTcuCURyaXZlIFJl

cXVpcmVtZW50cw1UaGUgZHJpdmUgd2lsbCBuZWVkIHR3byBiaXRzIGluIHRo

ZSBJZGVudGlmeSBEcml2ZSBQYXJhbWV0ZXIgSW5mb3JtYXRpb24gdG8gc3Bl

Y2lmeSB0aGF0IHRoZSBkcml2ZSBjYW4gc3VwcG9ydCB0aGUgUmVhZC9Xcml0

ZSBNdWx0aXBsZSBXL0NSQyBhbmQgdGhlIFJlYWQvV3JpdGUgTXVsdGlwbGUg

RE1BIFcvQ1JDIGNvbW1hbmRzLiAgQSBsaWtlbHkgcGxhY2UgaXMgaW4gV29y

ZCA0OSBiaXRzIDEzIGFuZCAxNCBzaW5jZSB0aGlzIHdvcmQgaXMgc3BlY2lm

eWluZyBkcml2ZSBjYXBhYmlsaXRpZXMsIGJ1dCB0aGVyZSBpcyBubyBoYXJk

IHJlYXNvbiBmb3Igbm90IHB1dHRpbmcgdGhpcyBpbmZvcm1hdGlvbiBlbHNl

d2hlcmUuICBXZSBhcmUgYXNzdW1pbmcgdGhhdCBpZiBhIGRyaXZlIHdpbGwg

c3VwcG9ydCBhIFJlYWQgVy9DUkMgdGhhdCBpdCBtdXN0IHN1cHBvcnQgdGhl

IFdyaXRlIFcvQ1JDIGFsc28sIGJ1dCB0aGUgZHJpdmUgZG9lcyBub3QgaGF2

ZSB0byBzdXBwb3J0IGJvdGggdGhlIFBJTyBhbmQgRE1BIG1vZGVzIGlmIGl0

IHN1cHBvcnRzIG9uZS4gDQ0CdQAho0oMoOGLSWujg0mrjBYjjLOmkpiAGdl0

UfGKLLm/S2KGCRoMoImOtJEkXUfMKLsBBF0UB6ANNH8bilTTFOZtEl7A3S8x

SLniWDGb9SEMIpSRHdmjYShMwSOqCriNYj4tSdgGDMUjAUMRCQ9hiIaPjEQg

GxEMBpHUeUOTQirqkrGQMtFCV5gEsZ6BjGexFOL1ISZsEA2glYoyKUIQC7yS

grCGPQAzppEFtbE9QcyktGloeWHrCUDmhpn0YQkEpWZATo2ZBGmtcAAp08As

hyQwDEMYUFjDGIYhjDV8QxZGxhjEOaQGIccD+JGZlRgJQUKgyEQyJQQDxODA

oCQWCAQBoFAwGZqlHoSzfskAAAEAAAAW8gAAFvX99wAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAoAQAACGAAAAAAYAAQAEAAWAgAAAQAAAAEgAAAB

IQAAAT8AAAFWAAABVwAAAW4AAAFvAAABhgAAAYcAAAUSAAAFEwAABVAAAAWC

AAAFgwAACIsAAAiMAAAIrAAACK0AAAjXAAAJtgAACbcAAAniAAALWgAAC1sA

AAuJAAAMtgAADLcAAAzmAAAOsgAADrMAAA7HAAAQKwAAECwAABBEAAAQqQAA

EKoAABDuAAAQ7wAAERYAABGeAAARnwAAEg4AABKIAAASiQAAEqH4+Pj4+Pj4

+PPt8+P4893X19fRycHRucHRsdHRqcHRodeYj5iPmJiGmIaGfdEAAAAAAAAA

AAAAAAAAAAAAAAAAAAAIAAABJJAADBAC0BEBaBP+mAAIAAADJJAADBAC0BED

hBP+mAAIAAACJJAADBAC0BEDhBP+mAAIAAABJJAADBAC0BEDhBP+mAAHAAAH

JJAADBAC0BECHAAABwAACSSQAAwQAtARAtAAAAcAAAYkkAAMEALQEQLQAAAH

AAAHJJAADBAC0BEC0AAABwAAASSQAAwQAtARAtAAAAcAAAQkkAAMEALQEQLQ

AAAF/QABJJAADBAC0AAFAAABJJAADBAC0AAFAAAOJJAADBAC0AAJAAABJJAA

DBAC0A8FAAEFoAAAAAUAABAkkAAMEALQAAQAAAEkkAAMAAAHAAABJJAADA8F

AAEFoAAtAAASoQAAFLkAABS6AAAU0AAAFvEAABby9erk3NYAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAFAAABJJAADBAC0AAHAAAK

JJAADBAC0BECHAAABf0AASSQAAwQAtAACgAAASSQAAwQAtARAhwPBQABAhwA

AAoAAAokkAAMEALQEQIcDwUAAQIcAAUADAAPAP//////////AP8AAAAT////

////////AP//BAAQB9gAJ////////////xH9AAAAAAAAFgB4DwUAAQIcAP//

BwAAAAAAAAAADfQAAAD2AAAAAAAAAAAAAAAAAAAAAAAAAADeAAAAAAAAFfIA

BAAAFvIAAAAAAAAJuwAAEn8AABXyEAD//wAAAAEAnwAA//8AAAACAToQAP//

AAAAAwAAAAABAAAAFvUADAAAAQAAABKhAAAW8gANAA4AAAACAAMABAAFAAYA

BwAIAAsADAANAA4ADwAQABIAFAAVABYAFwAYACAAIQBAAIAAogDIAMkAygDb

AOIA5wDzAPsBAAEBB9AH0QfSB9MH1AfVB9YH1wfYB9oH2wnTOOA44TjiOOMA

DAAAAEgASAAAAAAC+gJA//H/7gMJAlIqAgUoA/wBAAAAASwBLAAAAAAMZwlg

AAEAAABkAAABAQABAAAAAAABJw8AAQAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAA/8L/tQylCasAAQEBAAUAAQAAAAAAAAAAAAA94C/QA2AFoANgBaAAAALQ

QgAAAQABAAFEAAAAAAAASAAYDy06TFcgU2VsZWN0IDMwMAAKAAAAAAADAAAA

FQlIZWx2ZXRpY2EAAAAWB0NvdXJpZXIAAAfYEkNlbnR1cnkgU2Nob29sYm9v

a4ABAAAAAACFAAAAhQAGgACAAAAAAIUAAAAAAEUAAgHVAiAARQACAQgCIAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA

AAAAAAAAAAAAAAAA

--1435140434-1070521716-759089679:#15067--

From ata-owner  Thu Jan 20 10:53:00 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15929; Thu, 20 Jan 94 10:53:00 PST

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA15922; Thu, 20 Jan 94 10:52:56 PST

Received: from mail.qntm.com by worf.qntm.com with SMTP

(16.6/16.2) id AA29700; Thu, 20 Jan 94 10:50:44 -0800

Received: from wt400 by mail with SMTP

(16.6/16.2) id AA14456; Thu, 20 Jan 94 10:53:15 -0800

Received: by wt400 (5.65/1.3)

id AA00337; Thu, 20 Jan 94 11:40:10 -0800

Date: Thu, 20 Jan 94 11:40:10 -0800

From: Jim McGrath  </G=Jim/S=McGrath/O=QMAILGW/PRMD=QUANTUM/ADMD=MCI/C=US/@qntm.com>

To: a  <scsi@WichitaKS.NCR.COM>, ATA@dt.wdc.com, Hale_Landis@notes.seagate.com

Subject: Spreading False SFF Informa

Message-Id: <Macintosh*/PRMD=QUANTUM/ADMD=MCI/C=US/@wt400>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   Spreading False SFF Information

I AM speaking on behalf of Quantum.  A major PC OEM just informed me

that the SFF committee has endorsed using LBA mode.  This is an

obvious reference to Hale's project proposal that was approved by the

SFF committee this month, with the understanding that the approval by

SFF was to approval work on a proposal on the topic of > 528 MB

extensions, not on any specific details.  Indeed, the proposals Hale

has issued have a number of technical issues outstanding, including

LBA mode requirement (I believe our verbal agreement at the meeting

was to drop any mention of LBA mode as being a requirement in the

next draft).

I know which company is spreading this (mis)information,

and it disturbs me.  It creates confusion in the minds of our

customers, and can possibly lead them to stating internal positions

and direct internal work that will later embarass them and possibly

cause schedule slips.  Neither is in the interest of my customers, so

please - report what ACTUALLY HAPPENED at a meeting and report

YOUR DESIRES AS TO WHAT IS TO HAPPEN separately, do not treat

the later as the former.

Jim

From ata-owner  Thu Jan 20 12:46:59 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17873; Thu, 20 Jan 94 12:46:59 PST

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA17866; Thu, 20 Jan 94 12:46:51 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA08978; Thu, 20 Jan 1994 12:30:44 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 20 Jan 1994 12:18:33 EST

Date:      Thu Jan 20 12:18:32 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d3ebcea.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Re: Spreading False SFF Information

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 01-20-94 11:54:05 AM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: Re: Spreading False SFF Information

I agree with Jim McGrath of Quantum:  The SFF-8019 project was voted on at the

last SFF meeting and the vote was to continue work on the project.  The vote

was not a vote to approve the SFF-8019 document.

I also agree that there are several outstanding issues with the SFF-8019

document and I am waiting for comments from the software people (Phoenix

Technologies) before I produce SFF-8019 Rev 0.4.  SFF-8019 Rev 0.4 could be the

final draft (if we get lucky!).

It appears that the next revision of SFF-8019 will 1) not say anything about

LBA or 2) not go beyond the ATA-1/ATA-2 requirements for LBA.  Again, I'm

waiting for input from the software people.  If they come back and say that LBA

is a software requirement, then I'm sure this will generate lots of discussion

in both the ATA-2 and SFF committees.

I hope that the company spreading the incorrect LBA information is not my

company but, I must admit, Seagate is a big company and not everyone in the

company understands the workings of committees like SFF and therefore may be

spreading misleading information.  If you think someone at Seagate is doing

that, please contact me and I'll try to fix the problem.

Seagate has said that all Seagate ATA drives over 528MB will support LBA.

Seagate has not changed that position and that position should not be in

conflict with ATA-1, ATA-2 or SFF-8019.

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Thu Jan 20 14:37:37 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA19642; Thu, 20 Jan 94 14:37:37 PST

Received: from  by dt.wdc.com (4.1/SMI-4.1)

id AB19631; Thu, 20 Jan 94 14:37:26 PST

Received: from ncr-mpd by ncrhub1.NCR.COM id ax09987; 20 Jan 94 17:01 EST

Subject: Re: Comment ATA ProjProposal

To: Steve Finch <5723283@mcimail.com>

Date: Wed, 19 Jan 94 13:29:09 MST

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

Cc: ata@dt.wdc.com

In-Reply-To: <"60945181104991/224480@VAX2T"@vax2t.tus.ssi1.com>; from "Steve Finch" at Jan 18, 94 7:49 am

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9401201701.ax09987@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Steve,

In keeping with your later advice, I am only copying the ata reflector

on this message (I am tired of getting 5 copies of everything!).

"Steve Finch wrote:"

> 2.  The format of the project proposal is "out of date".  While this is

> true, the content and intent are still the same.  We will have to work this

> out, but it should slow us down!  (Should it?)

Not really.  Please make sure you get a document number for your draft

project proposal from Larry and make sure he includes it in the mailing.

Between now and the March meeting we can fix the minor errors.  At the March

meeting we can forward the revised project proposal to X3.  (It won't be

significantly different as the format changes are fairly minor.)

John

--

John Lohmeyer            E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Corp.                 Voice:  719-573-3362

1635 Aeroplaza Dr.          Fax:  719-597-8225

Colo Spgs, CO 80916    SCSI BBS:  719-574-0424 300--14400 baud (16800 HST)

From ata-owner  Fri Jan 21 09:48:13 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04640; Fri, 21 Jan 94 09:48:13 PST

Received: from maxtor.com (zeus.maxtor.com) by dt.wdc.com (4.1/SMI-4.1)

id AA04633; Fri, 21 Jan 94 09:48:09 PST

Received: from sanjose3.maxtor.com ([134.6.34.200]) by maxtor.com (4.1/SMI-4.1)

id AA26301; Fri, 21 Jan 94 09:49:53 PST

Message-Id: <9401211749.AA26301@maxtor.com>

Received: by sanjose3.maxtor.com with VINES ; Fri, 21 Jan 94 09:49:34 PST

Date: Fri, 21 Jan 94 09:49:15 PST

From: Larry=Lamers%ENG7%MCA@sanjose3.maxtor.com

Subject: Minutes ATA Working Group

To: ata@dt.wdc.com

Cc: scsi@wichitaks.ncr.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The size of the Message Text is  6336(> 6K), So it is kept in the First Attachmen

*****  ATTACHMENT: mailman.txt *****

Gentlemen - Included below are the prelimary minutes of the ATA working group

held in San Diego.

4

Accredited Standards Committee

X3, Information Processing Systems

Doc. No.: X3T10/94-012R0

Date: January 21, 1994

Project:

Ref. Doc.:

Reply to: S. Finch

To:    Membership of X3T9.2

From:  Lamers/Finch

Subject:  Minutes of X3T9.2 ATA Disk Drive Working Group November 8,1993

Agenda

1.Openning Remarks                                         2

2.Attendance and Membership, Introductions                 2

3.Approval of Agenda                                       2

4.Document Distribution                                    2

5.Report on X3T9.2 ATA Working Group Meeting,  December 8, 1993

2

6.Old Business                                             2

6.1  ATA DMA Issues (Landis)                          2

6.2  Download of Microcode (McGrath)                  2

6.3  20 MB/s Issues                                   2

6.4  16 MB/s Issues                                   2

7.New Business                                             3

7.1  ATA-3 Project Proposals                          3

7.2  ATA Reflector                                    3

7.3  Master Slave Hardware Issues                     3

7.4  Modeless ATA timings                             3

8.Review of X3T9.2/948D (ATA-2)                            3

8.1  Activites centered on 528 Mb Limitations         3

9.Meeting Schedule                                         3

10.Adjournment                                             4

Results of Meeting

1.Openning Remarks

Steve Finch convened the meeting at 1:00 pm.  Steve thanked  Rich

Bailly of Vitro for hosting the meeting.

As  is customary, the people attending introduced themselves.   A

copy  of  the  attendance list was circulated for attendance  and

corrections.

It  was stated that the meeting had been authorized by X3T9.2 and

would  be conducted under the X3 rules.  Ad hoc meetings take  no

final  actions, but prepare recommendations for approval  by  the

X3T9.2 task group.  The voting rules for the meeting are those of

the  parent  committee, X3T9.2.  These rules are:  one  vote  per

company; and any participating company member may vote.

The  minutes of this meeting will be posted to the SCSI  BBS  and

the  SCSI  Reflector and will be included in the  next  committee

mailing.

2.   Attendance and Membership, Introductions

Attendance at working group meetings does not count toward

minimum attendance requirements for X3T9.2 membership.  Working

group meetings are open to any person or company to attend and to

express their opinion on the subjects being discussed.

The following people attended the meeting.

NOTE:  The attendance list seems to have disappeared so there is

not a list included.

3.   Approval of Agenda

The agenda was approved as proposed.

4.   Document Distribution

Several pieces of new paper were handed out but none of the

authors had requested document numbers.

5.   Report on X3T9.2 ATA Working Group Meeting,  December 8,

1993

Not supplied.

6.   Old Business

6.1  ATA DMA Issues (Landis)

Hale will update the document and distribute it.

6.2  Download of Microcode (McGrath)

Jim McGrath not present; deferred.

6.3  20 MB/s Issues

Not discussed.

6.4  16 MB/s Issues

This issue was brought up by John Masciewicz at the last meeting.

It will be revised and presented at the January 28 meeting to

determine the level of consensus.  An SFF information bulletin is

being considered.

7.   New Business

7.1  ATA-3 Project Proposals

Steve Finch presented five project proposals for ATA standards.

7.2  ATA Reflector

How to identify issues and reach consensus on resolution via

reflector.

Information regarding subscribing to the reflector can be had by

contacting Tom Hanan at Western Digital.  hanan_t@a1.wdc.com.

Tom stated that sequential numbering is not yet a feature of the

Majordomo software.

Subscribe by sending a message to "majordom@dt.wdc.com".  The

message should contain a single line as follows:

subscribe ata your.email.address.

Larry Lamers requested that signature files be used by everyone.

The address of the reflector is "ata@dt.wdc.com".

7.3  Master Slave Hardware Issues

Hale talked about the master-slave issues.  Hale will repost the

proposad changes to Annex A and B.  These annexes can be

misinterpreted by folks that do not have a background in PC

architecure.  These changes attempt to clarify when a drive is

ready to accept a command.

7.4  Modeless ATA timings

Tom Hanan presented a paper summarizing the reasons why modeless

ATA timings are not an acceptable idea.

8.   Review of X3T9.2/948D (ATA-2)

Steve Finch stated that the group should focus on completing ATA-

2 in an expeditious fashion so that the group could focus on ATA-

3.

Richard Kalish moved and Larry Lamers seconded that a

recommendation be made to X3T10 that revision 2 be accepted as

the working draft for project 948D.  The motion failed 4:7.

8.1  Activites centered on 528 Mb Limitations

Seagate kicked off an industy wide effort at Fall Comdex to

resolve the addressing and capacity issues.  There were a lot of

folks in attendance at that meeting and a good start was made.

However the issue  affects a broader constituency than just the

membership of ATA/SFF.  Further work is expected to fine tune a

proposal.

9.   Meeting Schedule

January 28, 1994 -  Crown Sterling Suites in Milpitas, CA.

February 23, 1994 - TBD.

There was a discussion on finding a more appropriate meeting time

for ATA Working Group during the X3T10 plenary week.  The group

settled on two approaches:  keep the meeting where it is or move

it to  Thursday AM for ATA and Friday for SFF.

10.  Adjournment

The meeting adjourned at approximately 7:00 pm.

From ata-owner  Fri Jan 21 10:23:10 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA05327; Fri, 21 Jan 94 10:23:10 PST

Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA05286; Fri, 21 Jan 94 10:22:53 PST

Received: with SMTP-MR; Fri, 21 Jan 1994 10:18:15 PDT

Mr-Received: by mta WDUS04; Relayed; Fri, 21 Jan 1994 10:18:15 -0700 (PDT)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Fri, 21 Jan 1994 18:14:00 PDT

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Reflector etiquette

To: ATAPI@dt.wdc.com, ATA@dt.wdc.com

Message-Id: <01H7XVVF9CPUAC3JKJ@wdus03.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Fri, 21 Jan 1994 18:15:00 PDT

Importance: normal

Priority: normal

X400-Mts-Identifier: [;51810112104991/211935@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

We are rapidly eliminating the advantages of the technical reflectors we

have placed into service by saturating them with the very politics which

prevent a large number of technical contributors from attending our

meetings.

Recently, I have received several phone calls regarding the use of the

reflector for political banter. Several of our reflector-only contributors

have indicated that they do not have time to read and delete what they

consider committee political nonsense. They only wish to contribute to the

technical discussions.

As a result I have drafted some suggested etiquette for technical

reflectors such as ATA and ATAPI. Recognizing that some individuals find

reflectors convenient for the purpose of committee political discussions I

am also volunteering to provide a separate reflector to support their

needs. "committee@dt.wdc.com"

1) Please limit the content of e-mail submitted to technical reflectors to

technical or meeting coordination issues. Avoid combining political

opinions with technical information.

2) Please keep your e-mail to professional reflectors, professional.

Don't use the reflector to Flame. E-mail is a somewhat impersonal

communications medium which makes it very easy to insult people.

The fact that reflectors have such a wide distribution makes it

very easy to insult and or infuriate a very large number of people.

3) If you feel you have been flamed by someone, just let it go, don't

respond. Reflectors make it far too easy to drag hundreds of innocent

people into a personal or corporate dispute.

4) If you feel you must express an opinion on a technical reflector do not

mix roles. Avoid including a personal opinion with a response as a

corporate or committee representative. Send a separate e-mail for each

and clearly identify in what capacity you are representing yourself.

5) Please use direct e-mail where appropriate. If you have an opinion to

share with someone, contact them directly. Avoid sending e-mail to the

reflector that you believe will be useful to less than 20% of the list

members. Avoid using the reflectors as a convenient distribution list

for a small number of people.

The ata and atapi reflectors allow you to ask for a copy of the list so

that you can contact other members directly when necessary. Just send

the following commands to "majordomo@dt.wdc.com".

who ata

who atapi

6) Please be careful with what you send to the reflectors. The purpose of

a reflector is to increase the productivity of it's members. Too many

trivial or repeated messages, all of which must be read and deleted,

can make the reflectors as much of a burden to their members as a help.

Take a few extra moments to review what you are about to send so that

you can avoid sending another oop's copy the same day. If you are having

problems with your e-mail service, send yourself a copy to check it

before you send it to the reflector. Remember, several hundred people

will see your mistakes.

7) Please send e-mail to reflectors in human readable ASCII text. If you

need to distribute a non ASCII Text file containing a proposal, place it

on the appropriate FTP site designated for that reflector and send an

e-mail telling the members the file name and it's location.

ATA   = fission.dt.wdc.com      /pub/ata

ATAPI = fission.dt.wdc.com      /pub/atapi

8) Please limit the length of your e-mail to less than 10K. Many members

use e-mail systems which have trouble with large e-mail. If the document

is very large please place it on the appropriate FTP site.

9) Avoid sending e-mail to multiple reflectors on an FYI basis. If you

are not sure an e-mail is appropriate for at least 20% of a reflectors

(unique) membership, send the e-mail to the reflectors manager and let

them decide.

If you have comments or suggestions regarding this e-mail send them

directly to me at hanan_t@a1.wdc.com.

------------------------------------------------------------------------

Tom Hanan

ata reflector manager

ph 	714 932-7472

e-mail  hanan_t@a1.wdc.com

------------------------------------------------------------------------

From ata-owner  Fri Jan 21 15:19:10 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA09524; Fri, 21 Jan 94 15:19:10 PST

Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA09511; Fri, 21 Jan 94 15:19:03 PST

Received: with SMTP-MR; Fri, 21 Jan 1994 15:03:11 PDT

Mr-Received: by mta WDUS04; Relayed; Fri, 21 Jan 1994 15:03:11 -0700 (PDT)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Fri, 21 Jan 1994 22:58:00 PDT

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Rev 2 Mode 4 16MB/s Timing

To: SFF_REFLECTOR@SUN.COM, ATA@dt.wdc.com

Cc: WDCATA@dt.wdc.com

Message-Id: <01H7Y67ZT61OAC3I38@wdus03.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Fri, 21 Jan 1994 23:00:00 PDT

Importance: normal

Priority: normal

X400-Mts-Identifier: [;11305112104991/212487@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

________________________________________________________________________

To:		SFF Membership

From:		Tom Hanan, Western Digital

Date:		1/21/94

Subject:	Proposed Change of Mode 4 from 20MB/s to 16.6MB/s.

________________________________________________________________________

This is the second version of this proposal. The primary differences

between this version and the last are several timing changes and a new

timing parameter (tGr). These changes were made to address issues

identified by Seagate, Conner and Western Digital. If you are aware of

additional timing issues not addressed by this second proposal please

place them on the ATA and SFF reflectors.

------------------------------------------------------------------------

The following document contains the changes necessary to create an new

16MB/s SFF80?? draft specification from the existing SFF8011 specification.

Only those sections changed are included.

The intent of this proposal is to provide members of SFF with a summary

of the changes to SFF8011 necessary to create a new 16MB/s specification

which Dal could simply cut and past together for the next SFF mailing.

Western Digital is not proposing that this cut and pasted document be

approved, Instead we are simply attempting to make it easier for SFF

members to identify the issues associated with the proposal to facilitate

future discussion.

It is important to note that 120ns timings will require that we address

some cable integrity issues; however; our present evaluation indicates

that these should be minimal when compared to 100ns and 20MB/s.

The timings shown in this document should be considered preliminary

and subject to change by the SFF membership. Implimentors are forewarned

against use of this information until formally approved by the SFF

membership.

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

5. ATA Compatibility

5.1  New Signals

No new signals are defined by this Specification.

5.2  Signal Timing Changes

Signal timing changes are restricted to PIO and Multiword DMA timing.

These timing changes are only implemented if the host places the device in

one of the advanced transfer timing modes specified by this specification.

- If the host does not select an advanced timing mode, a device remains

compatible with ATA.

- Drives compatible with this specification can maintain compatibility with

the existing ATA.

- No changes have been made to the signals, signal names, registers,

register names or existing commands. The usage of these names is

compatible with ATA, and have the same meaning.

The four new timing parameters which are additions to or clarifications of

those in ATA, do not conflict with ATA.

- tZ in Multiword DMA

- t2i, t6Z, and tR in PIO

5.3  Command and Parameter Changes

Command changes are restricted to the setting of Transfer Mode with the Set

Features command.

- The usage of existing commands and parameter names is compatible with

ATA.

- Additional parameters defined to existing commands do not obsolete any

existing parameters.

- With the exception of Set Features, no parameter names or parameter

meanings have been changed.

The functionality of Set Features is the same, but the capability of this

command has been expanded and the definition in ATA has been clarified.

6. New Timing Modes

Two new timing modes, one for Multiword DMA and one for PIO data transfers

have been defined.

6.1  Cycle Time For New Timing Modes

For PIO Modes 0, 1, 2, 3, and Multiword DMA Mode 0, 1, the timing parameters

defined the operating characteristics the device was required to provide if

it supported the associated mode e.g. if the device supported PIO Mode 3,

it was required to support a minimum cycle time of 180 nanosecond (t0 for

PIO Mode 3).

The use of IORDY is optional in ATA. ATA devices reported support of various

PIO modes only if they could support a mode without the use of IORDY. This was

necessary to prevent problems between a device that could support a timing

mode only if IORDY is used and a host which supported the same timing mode

without IORDY. This meant that ATA devices reported lower transfer rates than

they could support if IORDY was used.

The investigation of methods to increase the ATA transfer rate learned that a

number of new rates were desirable. PIO Mode 2 had a transfer rate of 8.33

megabytes per second and host systems indicated a desire for several granular

increments. There was an even larger set of transfer rates desired for DMA.

Since each transfer rate represented a new mode, this would have caused a

significant increase in the number of modes defined.

The alternative adopted by this Specification is to change the usage of the

cycle time parameter t0, and include additional information in the Identify

Drive parameters. This makes it possible to support a wide range of speed

increments, identify them to the host, and reflect the maximum transfer

capabilities of a particular implementation.

In the new PIO Mode 4 and Multiword DMA Mode 2, Cycle Time t0 has been changed

to the minimum cycle supported by the mode. The definition of other timing

parameters associated with the new modes are not changed. If a device supports

one of the new modes it is required to meet the timing requirements for all

timing specifications associated with that mode, other than the possible

exception of Cycle Time. NOTE: PIO Modes 0, 1, 2, and Multiword DMA Mode 0

remain compatible with ATA.

The minimum cycle time supported by the device in PIO Mode 3, 4 and Multiword

DMA Mode 1, 2 respectively shall always be greater than or equal to the minimum

cycle time defined by the associated Mode e.g. a drive supporting PIO Mode 4

timing shall not report a value less than 120 nanoseconds, the minimum cycle

time defined for Mode 4 PIO Timings.

6.2  New Multiword DMA Transfer Mode

The new Multiword DMA Transfer Mode is Mode 2.

6.2.1  Timing Parameters

The timing parameters associated with Multiword DMA Transfer Mode 2 are

defined in Table 6-1. Peripherals reporting support for Multiword DMA Transfer

Mode 2 shall also support Multiword DMA Transfer Mode 0 and 1.

TABLE 6-1  MULTIWORD DMA TIMING PARAMETERS

+----------------------------------------+

| Multiword DMA              |   Mode 2  |

| timing parameters          |    nsec   |

|                            | Min | Max |

+-----+----------------------------+-----+-----|

| t0  | Cycle time                 | 120 |     |

| tC  | DMACK to DMREQ delay       |     | --- |

| tD  | DIOR-/DIOW-      16-bit    |  75 |     |

| tE  | DIOR- data access          |     | --- |

| tF  | DIOR- data hold            |   5 |     |

| tF  | DIOR- data hold         *1 |     | n/a |

| tGr | DIOR- data setup           |  20 |     |

| tGw | DIOW- data setup           |  20 |     |

| tH  | DIOW- data hold            |  10 |     |

| tI  | DMACK to DIOR-/DIOW- setup |   0 |     |

| tJ  | DIOR-/DIOW- to DMACK hold  |   5 |     |

| tKr | DIOR- negated pulse width  |  20 |     |

| tKw | DIOW- negated pulse width  |  20 |     |

| tLr | DIOR- to DMREQ delay       |     |  35 |

| tLw | DIOW- to DMREQ delay       |     |  35 |

| tZ  | DMACK- to tristate      *2 |     |  25 |

+----------------------------------+-----+-----+

*1 The meaning of this parameter in ATA was not clear. The parameter is not

applicable to this Specification.

*2 This parameter specifies the time from the negation edge of DMACK- to the

time the data bus is no longer driven by the device (tristate). The tZ

parameter applies only at the end of a Multiword DMA cycle, i.e. when

DMACK is negated. The device may actively drive the data bus or may

tristate the data bus while DMACK- is active from the first time that

DIOR- is asserted until DMACK- is deasserted as long as tE and tF

requirements are met.

6.2.2  Timings

The timings associated with Multiword DMA Transfers are defined in Figure 6-1,

and include the new timing parameter tZ. The minimum value of t0 is specified

by word 65 in the Identify Drive parameter list (see 7.1.6).

|<------------- t0 ------------>|

_____________________________________________            ________

DMARQ  ___/                                             \__________/

|                               |<--->|

|                               |  tL

___________________________________________________          _

DMACK- ______/                                                   \________/

|<-->|         |                     |         |

tI |<-- tD ->|<------- tK -------->|         |<-->|

|         |                     |         | tJ |

DIOR-             |_________|                     |_________     |

DIOW-  ___________/         \_____________________/         \_______________

|         |                                    |

|<-->|    |                                 -->|  |<-tZ

READ              | tE  _________ _ _ _ _ _ _ _ _ _ _ _  ___________

DD0-15 ----------------<_________X_X_X_X_X_X_X_X_X_X_X_X<_______X_X_>-------

|    |    |

|<-->|<-->|

| tGr| tF

WRITE                 ____________                    ____________

DD0-15 --------------<____________>------------------<____________>---------

|<---->|<--->|

tGw    tH

FIGURE 6-1  MULTIWORD DMA TIMING DIAGRAM

6.3  New PIO Transfer Mode

The new PIO Transfer Mode is Mode 4.

6.3.1  Timing Parameters

The timing parameters associated with PIO Transfer Mode 3 are defined in Table

6-2. Peripherals reporting support for PIO Transfer Mode 4 shall power up in a

PIO Transfer Mode compatible with ATA.

TABLE 6-2  PIO TIMING PARAMETERS

+------------------------------------------------------+

| PIO                                      |  Mode 4   |

| timing parameters                        |   nsec    |

|                                          | Min | Max |

+----+------------------------------------------+-----+-----+

| t0 | Cycle time                            *4 | 120 |     |

| t1 | Address valid to DIOR-/DIOW- setup       |  25 |     |

| t2 | DIOR-/DIOW-      16-bit               *4 |  75 |     |

|    |   Pulse width     8-bit               *4 |  75 |     |

| t2i| DIOR-/DIOW- recovery time             *4 |  20 |     |

| t3 | DIOW- data setup                         |  20 |     |

| t4 | DIOW- data hold                          |   5 |     |

| t5 | DIOR- data setup                         |  20 |     |

| t6 | DIOR- data hold                          |   5 |     |

| t6 | DIOR- data hold                       *1 |     | n/a |

| t6Z| DIOR- data tristate                   *2 |     |  30 |

| t7 | Addr valid to IOCS16- assertion          |     |  30 |

| t8 | Addr invalid to IOCS16- negation         |     |  25 |

| t9 | DIOR-/DIOW- to address valid hold        |   5 |     |

| tA | IORDY  Setup time                     *3 |  25 |     |

| tB | IORDY  Pulse Width                       |     |1,250|

| tRD| Read Data Valid to IORDY active          |   0 |     |

|    |  (if IORDY initially low after tA)       |     |     |

+-----------------------------------------------------------+

*1 The meaning of this parameter in ATA was not clear. The parameter is not

applicable to this Specification.

*2 This parameter specifies the time from the negation edge of DIOR- to the

time that the data bus is no longer driven by the device (tristate).

*3 The delay from DIOR- or DIOW- until the state of IORDY is first sampled.

If IORDY is inactive then the host shall wait until IORDY is active before

the PIO cycle can be completed. If the device is not driving IORDY negated

at the time tA after the activation of DIOR- or DIOW-, then t5 shall be

met and tRD is not applicable. If the device is driving IORDY negated at

the time tA after the activation of DIOR- or DIOW-, then tRD shall be met

and t5 is not applicable.

*4 t0 is the minimum total cycle time, t2 is the minimum command active

time, and t2i is the minimum command recovery time or command inactive

time. The actual cycle time equals the sum of the actual command active

time and the actual command inactive time. The three timing requirements

of t0, t2, t2i shall be met. The minimum total cycle time requirement, t0,

is greater than the sum of t2 and t2i. This means a host implementation

can lengthen either or both t2 or t2i to ensure that t0 is met. A device

implementation shall support any legal host implementation.

6.3.2  Timings

The timings associated with PIO Mode 4 transfers are defined in Figure 6-2,

and include the new timing parameters t2i, t6Z and tR. The minimum value of t0

is specified by word 68 in the Identify Drive parameter list (see 7.1.8).

It is mandatory that IORDY be supported when PIO Mode 3 or 4 are the current

mode of operation.

|<------------ t0 ------------------------->|

___________________________________________       |

Address Valid *1 _____/                                           \_________

|<-t1->|                            ->| t9  |<-

|      |<----------- t2 ------------->|   ->|t8|<-

|      |______________________________|<---t2i---->|__

DIOR-/DIOW-      ____________/                              \____________/

|      |                              |        |

WRITE                 |      |                    _____________      |

DD0-15           *2------------------------------<_____________>------------

|      |                    |<---t3-->|  |     |

|      |                            ->|t4|<-   |

READ                  |      |                     _____________ ____|

DD0-15           *2-------------------------------<_____________X____>------

|      |                    |<---t5-->|   |    |

->|t7|<- |                    |       ->|t6 |<-  |

|  | ->| tA |<-             |         |<-t6Z-->|

|  |___________________________________________

IOCS16-          ________/        |               |                  \______

|             ->|tRD|<-

________________|_________________________________________

IORDY            XXXXXXXXXXXXXXXXX___________________//

FIGURE 6-2  PIO TIMING DIAGRAM

7. Command and Parameter Changes

Changes have been made to the SFF8011 Identify Drive and Set Feature commands.

7.1.4  Word 64: Flow Control PIO Transfer Modes Supported

Word 64 Bits 7-0 is a bit significant field. Any number of bits may be set

in this field by the device to indicate which Advanced PIO Modes it is

capable of supporting. Bits 7-2 are reserved for future use.

- If Bit 0=1, the device supports PIO Mode 3.

- If Bit 1=1, the device supports PIO Mode 4.

NOTE: For backwards compatibility with BIOSs written before Word 64 was

defined for advanced modes, a device reports in Word 51 the highest original

PIO mode it can support.

7.1.5  Word 65: Minimum Multiword DMA Transfer Cycle Time Per Word

Word 65 defines the minimum cycle time that the device can support when

performing Multiword DMA transfers on a per word basis.

If this field is supported, Word 53 Bit 1=1 shall be set.

Any device which supports Multiword DMA Mode 1 or above shall support this

field, and the value in word 65 shall not be less than the minimum cycle

time reported by the fastest DMA mode supported by the device.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other than

this field, the device shall return a value of zero in this field.

7.1.7  Word 67: Minimum PIO Transfer Cycle Time Without Flow Control

Word 67 defines the minimum cycle time that the device guarantees shall not

cause data integrity degradation during a transfer which does not utilize flow

control.

Any device may support this field, and if this field is supported, bit 1 of

word 53 shall be set.

Any device which supports PIO Mode 3 or above shall support this field, and

the value in Word 67 shall not be less than the value reproted in word 68.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other than

this field, the device shall return a value of zero in this field.

7.1.8  Word 68: Minimum PIO Transfer Cycle Time With IORDY Flow Control

Word 68 defines the minimum cycle time that the device can support when

performing data transfers while utilizing IORDY flow control.

Any device may support this field, and if this field is supported, bit 1 of

word 53 shall be set.

Any device which supports PIO Mode 3 or above shall support this field, and

the value in word 68 shall not be less than the fastest PIO mode reported

by the device.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other than

this field, the device shall return a value of zero in this field.

From ata-owner  Mon Jan 24 12:35:23 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27802; Mon, 24 Jan 94 12:35:23 PST

Received: from ncrhub1.NCR.COM (h192-127-251-3.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA27794; Mon, 24 Jan 94 12:34:52 PST

Received: from ncrcolo by ncrhub1.NCR.COM id ab12855; 24 Jan 94 13:29 EST

Received: by ncrcolo.FtCollinsCO.NCR.com; 24 Jan 94 11:23:30 MST

Subject: March 14-18, 1994 Meeting List and Announcement

To: SCSI Reflector List <scsi@WichitaKS.NCR.COM>,

ATA Reflector <ata@dt.wdc.com>, SSA Reflector <ssa@dt.wdc.com>,

P1394 Reflector <p1394@sun.com>,

Small Form Factor Reflector <sff_reflector@sun.com>

Date: Mon, 24 Jan 94 11:20:24 MST

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9401241329.ab12855@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I just made my room reservations with the Hyatt Newporter.  They had a

lot of difficulty finding our group in their computer. They have it under

"American National Standards Institute", not ANSI or X3T**.  So, save yourself

some time and tell them you are with the American National Standards

Institute (even though this is technically incorrect).

John

PS: The reservations deadline is coming up fast: Feb 1, 1994.

------------------------------------------------------------------------------

Meeting Map -- March 14-18, 1994 -- Newport Beach, CA

(1/13/94 Version -- Does not include X3T11 meetings)

Sunday 3/13        9   10  11  12  1   2   3   4   5

+=============+

|Gen Edit Sess|              (10)

+=============+

Monday 3/14        9   10  11  12  1   2   3   4   5   6   7   8

+-------------------+=======================+

| Small Form Factor | ATA Working Group     |  (40)

+-------------------+=======================+

| SCSI CAM Working Group                    |  (25)

+===========================================+

Tuesday 3/15       9   10  11  12  1   2   3   4   5   6   7   8

+---------------+===============+

| SSA-UIG       | X3T10.1 (SSA) |              (30)

+===============+===============+

| SBP LB Res    | P1394 Plenary |              (30)

+===============+---------------+

| RAID SG / RAB                 |              (40)

+===============================+

| P1394 Connector Task Group    |              (15)

+-------------------------------+

Wednesday 3/16     9   10  11  12  1   2   3   4   5   6   7   8

+===========================================+

| SCSI-3 WG                                 |  (55)

+===============================+===========+

| P1394 Plenary Meeting         |              (30)

+-------------------------------+

Thursday 3/17      9   10  11  12  1   2   3   4   5   6   7   8

+============+  +===========================+

| SSP WG     |  | X3T10 Plenary             |  (30/80)

+============+--+===========================+

Friday 3/18    8   9   10  11  12  1   2

+=====================+

| X3T10 Plenary       |                          (80)

+=====================+

Meetings marked with == are X3T10-authorized meetings.

Meetings marked with -- are authorized by other groups and may change

without notice.

Abbreviation Key:

ATA:              AT Attachment (a.k.a. IDE)

CAM:              Common Access Method

Gen Edit Sess:    General Editing Session

P1394:            IEEE Project on High Performance Serial Bus

Plenary:          Full Committee Meeting

RAB:              RAID Advisory Board

RAID:             Redundant Array of Independent Devices

SG:               Study Group

SSA:              Serial Storage Interface

SSP:              Serial Storage Protocol (Protocol for SSA)

UIG:              User Interest Group

WG:               Working Group (same as Ad Hoc Group)

X3T10:            Lower Level Interfaces Technical Committee (was X3T9.2)

X3T10.1:          SSA Task Group of X3T10 (was X3T9.7)

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, X3T11, and IEEE P1394)

MARCH 14-18, 1994

HOSTED BY:     QLogic Corporation

LOCATION:      Hyatt Newporter

1107 Jamboree Road

Newport Beach, California  92660

Phone 800-233-1234 or 714-729-1234

ROOM RATE:     $100.00 per night (includes tax and parking)

$150.00 for a suite

GROUP NAME:    American National Standards Institute

CUT-OFF DATE:  February 1, 1993

DRIVING DIRECTIONS:

1. When leaving Orange County, John Wayne airport, follow

signs to Interstate South 405 (left out of airport onto

MacAurther Drive).

2. Get in right lane and turn right onto 405-South freeway.

3. Exit the 405 freeway at the next exit, Jamboree Blvd.

4. Make a right onto Jamboree Blvd. and continue for about 5

miles (toward ocean).

5. Pass UC Irvine, MacAurther, Highway 73, University Ave.,

Bison, Ford Rd.

6. Just past San Joaquin Rd., about 0.75 miles, look to the

right for the Hyatt Newporter Hotel entrance.

HOST CONTACT:  Skip Jones, QLogic Corporation (714-668-5058)

internet: SK_JONES@QLC.COM

From ata-owner  Mon Jan 31 16:15:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA05106; Mon, 31 Jan 94 16:15:16 PST

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA05079; Mon, 31 Jan 94 16:13:50 PST

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id QAA26000; Mon, 31 Jan 1994 16:08:16 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 31 Jan 94 16:13:44 PST

Date: 31 Jan 94 15:55:18 PST

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: ATA-2 Editors Meeting

To: "scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"81553213104991/228823@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Date:     Monday Jan 31, 1994

From:     Steve Finch/Tom Hanan

Subject:  ATA-2 editors meeting

***********************************************************************

********  ATA-2  E D I T O R S    M E E T I N G   N O T I C E  ********

***********************************************************************

SUBJECT:    ATA-2 Editors MEETING

DATE:	    Feb. 17 (Thursday)

TIME:	    9:30AM - 4PM

PLACE:      Western Digital (Plaza)

Address:    8105 Irvine Center Drive

PHONE:      714-932-7472 (Tom Hanan)

City	    Irvine CA 92718

Orange County (SNA) "John Wayne" International is

the closest airport.

PLEASE RSVP ASAP TO TOM HANAN.  We expect this meeting to be small, but

if we are wrong we need to know ASAP so that we can make arrangements

for a larger meeting area.   THANK YOU!

------------------------------------------------------------------------

Directions:

------------------------------------------------------------------------

From LAX take the 405 fwy south, toward San Diego, approximently 45

miles.  Orange County International airport will be on your right just

past the 405 / 55 interchange. From this point continue south using the

directions from Orange County International airport.

From Orange County International airport take the 405 fwy south, toward

San Diego, about 6 miles and exit at Irvine Center Drive. When you exit

the fwy take a left onto Irvine Center Drive and go to the second signal

(Pacifica) and take a right onto Pacifica. Pacifica dead ends into

Fortune where you will take a right. By this time you will have noticed

that you are circling a brown 15 story high rise which is visible from

the 405 fwy. Once you are on Fortune take a right into the first

driveway to enter Western Digital's parking lot. Visitor parking will be

to your left against the street.

The meeting will be in the cafeteria which is to the left as you enter

the building. There is no need to sign in, the cafeteria is a public

area.

------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END -----------------------------------

========================================================================

Steve Finch, Silicon Systems, Inc.

Ph.  714 573-6808, FX: 714 573-6914

Email:  5723283@mcimail.com

========================================================================

From ata-owner  Wed Feb  2 12:31:37 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA09981; Wed, 2 Feb 94 12:31:37 PST

Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

id AA09954; Wed, 2 Feb 94 12:31:18 PST

Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.4/8.6.4-1.13) with ESMTP id MAA09434; Wed, 2 Feb 1994 12:31:59 -0800

Received: from zebra.unix.portal.com  by portal.unix.portal.com (2.31)

id MAA24440; Wed, 2 Feb 1994 12:31:58 -0800

Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id MAA15083; Wed, 2 Feb 1994 12:31:56 -0800

Received:  from endl by netmail.com (UUPC/extended 1.12e) with UUCP

for multiple addressees; Wed, 02 Feb 1994 12:05:10 -0800

Received:  by endl.com (UUPC/extended 1.12b);

Wed, 02 Feb 1994 11:56:42 pst

Message-Id: <2d50057a.endl@endl.com>

Date:      Wed, 02 Feb 1994 11:56:42 pst

From: "Dal Allan" <dal@endl.com>

To: ata@dt.wdc.com, sff_reflector@sun.com

Subject:   Innovation_Award

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:  ATA Reflector

SFF Reflector

This is a notice of general interest to both reflectors.

Let me quote from a fax which came this morning from Manfred Barth of PC

Professionell (the German version of PC Magazine).

Subject:  Enhanced IDE/ATA

At the CeBIT-Fair in Hannover (March 16-23) we will promote

something similar to the Techex-Award of PC Magazine that

is promoted at Comdex/Fall. We call it "Innovationen des

Jahres".

We would like to invite you and the relevant people of the

SFF Committee to the CeBIT event on March 16th in Hannover,

because the SFF Proposal/Specification is nominated in the

category of Hardware/Technology.

Please tell us, if someone and who will be in Hannover at

CeBIT. It would be a great pleasure for us, if someone from

the SFF Committee will join our event.

Unfortunately, March 16 coincides with X3T10 week. Our regular meeting is on

March 14 so it is possible that some of you will be flying to Hannover after

Newport Beach.

Could all those who out there in reflector-land who are going to Hannover

please email me their plans?

We have one voting member in Germany who can accept an award on behalf of

the committee, but the more the merrier.....

Dal

NOTE: Internet is only available in my office so to reach me at all

times do not use the REPLY TO: address. Instead, respond to:

250-1752@mcimail.com  or dal_allan@mcimail.com

From ata-owner  Wed Feb  2 14:01:37 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA11148; Wed, 2 Feb 94 14:01:37 PST

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA11128; Wed, 2 Feb 94 14:01:25 PST

Received: from mail.qntm.com by worf.qntm.com with SMTP

(16.6/16.2) id AA23019; Wed, 2 Feb 94 13:59:17 -0800

Received: from wt400 by mail with SMTP

(16.6/16.2) id AA12141; Wed, 2 Feb 94 14:01:28 -0800

Received: by wt400 (5.65/1.3)

id AA03161; Wed, 2 Feb 94 14:48:18 -0800

Date: Wed, 2 Feb 94 14:48:18 -0800

From: Jim McGrath  </G=Jim/S=McGrath/O=QMAILGW/PRMD=QUANTUM/ADMD=MCI/C=US/@qntm.com>

To: a  <atapi@dt.wdc.com>, ATA@dt.wdc.com, Worrell  <worrell@dt.wdc.com>

Subject: RE>Packet Command timing

Message-Id: <Macintosh*/PRMD=QUANTUM/ADMD=MCI/C=US/@wt400>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>Packet Command timing

The problem with the 100 us timeout is that it cannot be met 100% of the time

(the meaning of a timeout) in firmware if the microcontroller is doing anything

other than interface related matters.  In particular, unusual electromechanical

operations (e.g. servo error recovery) may, for a brief period of time, require

close to 100% of the microcontroller.  Only overspecifying the microcontroller

or providing this function in an ASIC would solve the problem.  One would cost

extra money, the other would limit time to market of initial products.

While past AT BIOSes did have requirements of this sort on DATA OUT type

commands (e.g. write), we have been trying to wean the industry away from this

practice.  The ATA standard does NOT endorse these types of timeouts, and I

expect (based on past discussions) any standard version of ATAPI will not

include this timeout.

As to the points raised, driver timeout is a real issue, but any finite timeout

value will do to allow software writers to provide closure with a timeout.  As

to overhead concerns, it is not appropriate to manage this with a timeout value.

Implementors will try to implement functions as fast as possible with the

available technology at the market price.  The reason why SCSI initially had

high overheads was a general lack of available technology at a reasonable cost

at that time, not a lack of understandaing or appreciation of overhead issues.

If I were a system manufacturer, I would not appreciate paying another $5 for

this function or reducing the number of component vendors (with the supply and

price issues that entails) until everyone turns their ASICs to add the function.

To provide a timeout value for drive writers, I would suggest something on the

order of several ms.

Jim

Original message included below for ATA audiance.

--------------------------------------

Date: 2/2/94 12:42 PM

To: Jim McGrath

From: Devon Worrell (Worrell) x7042

Precedence: bulk

X-Sequence: atapi-14

All ATAPI SSWG members,

A question regarding the 100 microseconds (Maximum) time between the A0

comand being issued and the point at which the CD-ROM peripheral is

ready for the actual Packet Data, in the specification has been asked?

This should be VERY CLEAR:

1) The reason for the 100 microsecond maximum is so that the Host

driver can timeout the wait for DRQ.

2) To ensure that the CD-ROM implementations do NOT add extra overhead

in the ATAPI protocol. Note that this is one of the problems with

SCSI, in that it take a long time for the "Phases" to be processed.

3) The 100 microseconds is measured from the point where the A0 command

has been written to the Command Register, until all of the following:

a) BSY has been DEASSERTED and;

b) CoD is set and;

c) IO is cleared and;

d) DRQ is ASSERTED

4) The order that the signals change adhear to the following conditions:

a) BSY must be set IMMEDIATLY after the A0 is written

to the Command Register (within 400 nanoseconds)

b) DRQ must not be set until CoD(1) and IO(0) are valid for

Command acceptance

c) DRQ may be set before or after BSY has been deasserted

d) BSY must be cleared and DRQ set within 100 microseconds

I hope that this makes the requirements clear,

Devon,

"Every truth passes through three stages before it is recognized.

In the first it is ridiculed, in the second it is opposed, in the

third it is regarded as self-evident!"

_/_/_/_/    _/_/_/_/    _/      _/ | Devon B. Worrell <worrell@dt.wdc.com>

_/      _/  _/      _/  _/      _/  | Senior Principal Engineer

_/      _/  _/_/_/_/    _/  _/  _/   | Western Digital Corporation

_/      _/  _/       _/ _/_/_/_/_/    | 8105 Irvine Center Drive

_/_/_/_/    _/_/_/_/_/  _/      _/     | Irvine, CA 92718

| Host Interface & Central Firmware

| Technology Staging Group

| Phone: (714) 932-7042

| Fax:   (714) 932-7796

"Not speaking in an official capacity for my employer"

"Void where prohibited by law"

"No refunds--all sales are final"

"Does not include CA Redemption Value"

From ata-owner  Thu Feb  3 03:01:45 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA21534; Thu, 3 Feb 94 03:01:45 PST

Received: from nova.unix.portal.com ([156.151.254.3]) by dt.wdc.com (4.1/SMI-4.1)

id AA21521; Thu, 3 Feb 94 03:01:39 PST

Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.4/8.6.4-1.13) with ESMTP id DAA07226 for <ata@dt.wdc.com>; Thu, 3 Feb 1994 03:02:23 -0800

Received: from zebra.unix.portal.com  by portal.unix.portal.com (2.31)

id DAA04036; Thu, 3 Feb 1994 03:02:21 -0800

Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id DAA22061 for ata@dt.wdc.com; Thu, 3 Feb 1994 03:02:20 -0800

Received:  from endl by netmail.com (UUPC/extended 1.12e) with UUCP

for ata@dt.wdc.com; Thu, 03 Feb 1994 02:05:14 -0800

Received:  by endl.com (UUPC/extended 1.12b);

Wed, 02 Feb 1994 23:55:42 pst

Message-Id: <2d50adff.endl@endl.com>

Date:      Wed, 02 Feb 1994 23:55:43 pst

From: "Dal Allan" <dal@endl.com>

To: ata@dt.wdc.com

Subject:   94-95_Calendars

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:  ata@dt.wdc.com

atapi@dt.wdc.com

fibre-channel-ext@think.com

hippi-ext@think.com

scsi@wichitaks.ncr.com

sff_reflector@sun.com

ssa@dt.wdc.com

Date:  2/2/94

Subject:  Meeting Calendar

May I freeze in Hell might well be the thoughts if you receive multiple

copies of the meeting calendar for the rest of 1994 and for 1995.

For those of you who did not already know, the host for Toronto in September

was Hewlett Packard Canada Network Operations which is involved with Fibre

Channel. The separation into two technical committees and 2-monthly cycle

meant that an X3T11 attender was hosting an X3T10 meeting.

The call for an alternative site, hosted by an X3T10 participant, was

answered by Compaq.

Therefore, the site of the September meeting of X3T10 will be Houston.

Our collective thanks to Compaq for volunteering to host on such short

notice.

The summary calendar for 1994 follows.

February       15 16 17          Austin            X3T11  IBM

March       14 15 16 17 18       Newport Beach     X3T10  Q Logic

April          19 20 21          Seattle           X3T11  Boeing

May         16 17 18 19 20       Harrisburg        X3T10  AMP

June           21 22 23          Minneapolis       X3T11  StanaTek

July        18 19 29 21 22       Manchester        X3T10  DEC

August         16 17 18          Keystone          X3T11  Methode

September   12 13 14 15 16       Houston           X3T10  Compaq

October        11 12 13          St Petersburg     X3T11  AMP

November     7  8  9 10 11       Irvine            X3T10  Western Digital

December        6  7  8          San Diego         X3T11

Here is the latest set of dates and information as I understand it through

year end 1995. I have included the full year so that you can seen how it all

fits together because many people tell me it helps to see the full year as

it lets them see how the meetings fit in with holidays and family events

like Mothers Day and Fathers Day. If you prefer the summary, let me know, I

will go with the squeakiest wheel....

1995 CALENDAR

January   *h *h  3  4  5  6  7                           New Years

8  x  x  x  x  x *h   Irvine           X3T10  Silicon Systems

*h *h 17 18 19 20 21                           Martin Luther King

22 23 24 25 26 27 28

February  29 30 31  1  2  3  4

5  6  x  x  x  x *h   tbd              X3T11

*h *h 14 15 16 17 18                           President's Day

19 20 21 22 23 24 25

March     26 27 28  1  2  3  4

5  x  x  x  x  x 11   Newport Beach    X3T10  QLogic

12 13 14 15 16 17 18

19 20 21 22 23 24 25

April     26 27 28 29 30 31  1

2  3  x  x  x  x  8   Monterey CA      X3T11  National Semiconductor

9 10 11 12 13 14 EE                           Easter

EE 17 18 19 20 21 22

23 24 25 26 27 28 29

May       30  1  2  3  4  5  6

7  x  x  x  x  x 13   Harrisburg PA    X3T10  AMP

mm 15 16 17 18 19 20                           Mother's Day

21 22 23 24 25 26 *h                           Memorial Day

June      *h *h 30 31  1  2  3

4  5  x  x  x  x 10   Rochester MN     X3T11  IBM

11 12 13 14 15 16 17

ff 19 20 21 22 23 24                           Father's Day

July      25 26 27 28 29 30 *h

*h *h *h  5  6  7  8                           July 4

9  x  x  x  x  x 15   Colorado Springs X3T10  NCR

16 17 18 19 20 21 22

23 24 25 26 27 28 29

August    30 31  1  2  3  4  5

6  7  x  x  x  x 12   tbd              X3T11

13 14 15 16 17 18 19

20 21 22 23 24 25 26

September 27 28 29 30 31  1 *h

*h *h  5  6  7  8  9                           Labor Day

10  x  x  x  x  x 16   Manchester NH    X3T10  Digital Equipment

17 18 19 20 21 22 23

24 25 26 27 28 29 30

October    1  2  3  4  5  6  7

8  9  x  x  x  x 14   tbd              X3T11

15 16 17 18 19 20 21

22 23 24 25 26 27 28

November  29 30 31  1  2  3  4

5  x  x  x  x  x 11   Irvine           X3T10  Western Digital

12 13 14 15 16 17 18

19 20 21 22 *h *h *h                           Thanksgiving

December  *h 27 28 29 30  1  2

3  4  x  x  x  x  9   San Diego        X3T11  * in conj w/Supercom 95

10 11 12 13 14 15 16

17 18 19 20 21 22 23

24 *h 26 27 28 29 30                           Christmas

31

* The ACM is holding SuperComputing 1995 in San Diego and has offered

to host our meeting. More to follow as the arrangements are worked

on by ACM. I assume X3T11 members think it is a good idea, because

I accepted the offer on your behalf.

From ata-owner  Tue Feb  8 11:23:44 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23360; Tue, 8 Feb 94 11:23:44 PST

Received: from qlc.qlc.com by dt.wdc.com (4.1/SMI-4.1)

id AA23353; Tue, 8 Feb 94 11:23:36 PST

Received: from triton.qlc.com by qlc.qlc.com with SMTP id AA24393

(5.64+/IDA-1.3.4 for ata@dt.wdc.com); Tue, 8 Feb 94 11:22:48 -0800

Received: by triton.qlc.com (4.1/SMI-4.0)

id AA19339; Tue, 8 Feb 94 11:22:54 PST

Date: Tue, 8 Feb 94 11:22:54 PST

From: j_schaffner@qlc.com (Jim Schaffner)

Message-Id: <9402081922.AA19339@triton.qlc.com>

To: ata@dt.wdc.com, sff_reflector@sun.com

Subject: Question obout:   Device Control Reg. bits 0 and 3

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Does anyone out there in "ATA_land" have any insight into

bits 0 and 3 of the Device Control Reg. ?

the ATA spec. currenty specified these as:

bit 0 = '0'

bit 3 = '1'

My questions would be:

1)  Why are these specified at fixed values ?  (any relevant history ?)

2)  Are either of these bits ever used for anything else ?

3)  Has bit 0 any existing (or prior) definiton for controlling DMA ?

Any information would be appreciated.  Thanks.

Yogi Schaffner -->  yogi@qlc.com   or   via reflector

From ata-owner  Wed Feb  9 08:16:40 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10059; Wed, 9 Feb 94 08:16:40 PST

Received: from mailgate.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA10048; Wed, 9 Feb 94 08:16:30 PST

Received: by mailgate.prod.aol.net

(1.37.109.4/16.2) id AA14647; Wed, 9 Feb 94 10:26:54 -0500

From: ljlamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9402091026.tn04090@aol.com>

To: ata@dt.wdc.com

Date: Wed, 09 Feb 94 10:26:26 EST

Subject: CC:

Sender: ata-owner@dt.wdc.com

Precedence: bulk

SUBJ: New Address

Larry Lamers has changed employers - he now works for Adaptec.  The

pertinent information is contained in the signature below.

However, since nothing in life is ever as straightforward as it should be

note that if you are sending packages via any route other that the

U.S. Mail, these packages should be sent to

Lawrence J. Lamers

698 Gibraltar Court

Milpitas, CA   95035

Regards

Lawrence J. Lamers

Adaptec                    TEL: 408-945-8600 ext 3214

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@ADAPTEC.COM

#

From ata-owner  Thu Feb 10 10:43:31 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01185; Thu, 10 Feb 94 10:43:31 PST

Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

id AA01178; Thu, 10 Feb 94 10:43:28 PST

Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.4/8.6.4-1.13) with ESMTP id KAA03537 for <ata@dt.wdc.com>; Thu, 10 Feb 1994 10:44:19 -0800

Received: from jmcbride.UUCP  by portal.unix.portal.com (2.31)

id KAA25095; Thu, 10 Feb 1994 10:44:17 -0800

Received:  from endl by netmail.com (UUPC/extended 1.12b) with UUCP;

Thu, 10 Feb 1994 10:10:05 pst

Received:  by endl.com (UUPC/extended 1.12b);

Thu, 10 Feb 1994 10:07:48 pst

Message-Id: <2d5a77f4.endl@endl.com>

Date:      Thu, 10 Feb 1994 10:07:48 pst

From: "Dal Allan" <dal@endl.com>

To: sff_reflector@sun.com, ata@dt.wdc.com

Subject:   Hannover_news

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:  PC Professionell Award

To:  ATA Reflector

SFF Reflector

CeBIT is in Hannover March 16-23, just after our next meeting, and Tom Hanan

will be at the show.

Tom led the SSWG which developed SFF-8011 and it is the ATA Timing Extension

Specification which is the basis for the award.

I have asked Tom to represent the SFF Committee and accept the award (if

any) on our behalf.

No doubt there will have to be an acceptance speech, so I have asked Tom if

he could prepare some remarks before he leaves and distribute them on the

reflector. That way, everyone will know what he plans to say and companies

which want to publicize the nomination can quote from Tom's remarks.

Is anyone else likely to be there? If there is going to be press conference

then the more companies that are members of SFF the better.

Dal

NOTE: Internet is only available in my office so to reach me at all

times do not use the REPLY TO: address. Instead, respond to:

250-1752@mcimail.com  or dal_allan@mcimail.com

From ata-owner  Fri Feb 11 07:59:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA13845; Fri, 11 Feb 94 07:59:16 PST

Received: from ncrhub1.NCR.COM (h192-127-251-3.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA13820; Fri, 11 Feb 94 07:52:46 PST

Received: from [153.72.254.250] by ncrhub1.NCR.COM id aa03870;

10 Feb 94 15:02 EST

Received: by ncrcolo.FtCollinsCO.NCR.com; 10 Feb 94 12:42:21 MST

Received: by ncrcolo.FtCollinsCO.NCR.com; 10 Feb 94 09:27:30 MST

Received: by ncrcolo.FtCollinsCO.NCR.com; 10 Feb 94 08:53:19 MST

Subject: X3T10 Minutes (1/13/94)

To: SCSI Reflector List <scsi@WichitaKS.NCR.COM>,

ATA Reflector <ata@dt.wdc.com>

Date: Mon, 7 Feb 94 10:55:54 MST

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9402101502.aa03870@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Accredited Standards Committee

Doc. No.: X3T10/94-016r2

Date: January 25, 1994

Project:

Ref. Doc.:

Reply to: John Lohmeyer

To:         Membership of X3T10

From:       Lawrence J. Lamers, Acting Secretary X3T10

John B. Lohmeyer, Acting Chair X3T10

Subject:    Minutes of X3T10 Plenary Meeting #1

San Diego, CA -- January 13, 1994

Agenda

1.    Opening Remarks

2.    Approval of Agenda

3.    Attendance and Membership

4.    X3 Tutorial

5.    Approval of Minutes - November Meeting Colo Spgs, CO (X3T9.2/93-

176r1)

6.    Document Distribution

7.    Call for Patents

8.    Review of Old Action Items

9.    Subgroup and/or Project Status Reports

9.1   Project Status Summary [Lohmeyer]

9.2   SCSI-2 Status [Lamers]

9.3   SCSI Common Access Method (CAM) [Dallas]

9.4   AT Attachment Status (ATA) [Lamers]

9.5   ATA Extensions (ATA-2) [Finch]

9.6   SCSI-3 Architecture Model (SAM) Status [Monia]

9.7   SCSI-3 Parallel Interface (SPI) Status [Lamers]

9.8   SCSI-3 Interlocked Protocol (SIP) Status [Lamers]

9.9   SCSI-3 Generic Packetized Protocol (GPP) Status [Stephens]

9.10  SCSI-3 Serial Bus Protocol (SBP) Status [Marazas]

9.11  SCSI-3 Fiber Channel Protocol (FCP) Status [Snively]

9.12  SCSI-3 Primary Commands (SPC) Status [Weber]

9.13  SCSI-3 Block Commands (SBC) [open]

9.14  SCSI-3 Stream Commands (SSC) [open]

9.15  SCSI-3 Graphics Commands (SGC) [open]

9.16  SCSI-3 Medium Changer Commands (SMC) [open]

9.17  Directly-Addressable Device Interface (DADI) []

9.18  SCSI Common Access Method -2 (CAM-2) (93-082) [Dallas]

9.19  SCSI-3 Serial Storage Protocol (SSP) [Scheible]

9.20  SCSI-3 Controller Commands (SCC) (93-130) [Hagerman]

9.21  Replacement of ANSI X3.80-1988, Interfaces Between Flexible Disk

Cartridges and Their Host Controllers, with ISO 9315:1989.

9.22  RAID Study Group [Penokie]

10.   Old Business

10.1  Technical Information Bulletins

10.2  GPP: Standard or Technical Report? [Milligan] ()

11.   New Business

11.1  X3T10 Subscription Mailing Program

11.2  Review of new documents

11.3  Task Set Basis Selection

11.4  Delivery Mechanism Sequentiality

11.5  SCC Project Proposal

11.6  MMC Project Proposal

12.   ISO [Milligan]

13.   Liaison Reports

13.1  X3T10.1 Task Group Report [Scheible]

13.2  X3T9.3 Liaison Report [Cummings]

13.3  Small Form Factor Industry Group [Allan]

13.4  IEEE P1394 [Marazas]

13.5  IT8 [Lohmeyer]

13.6  IEEE P1285 Status [Pak]

13.7  PCMCIA Report

13.8  AIIM C21 [Podio]

14.   Review of Action Items

15.   Meeting Schedule

15.1  Authorization of Working Group Meetings

16.   Adjournment

Results of Meeting

1.    Opening Remarks

John Lohmeyer, the Acting Chair, called the meeting to order at 1:00 p.m.,

Thursday January 13, 1994.  He thanked Rich Bailly of Vitro Corp. for

arranging and hosting the first meeting of X3T10.  John said that he had

copies of the call for volunteers to Chair X3T10 for anyone who wishes to

apply.

John said that X3 Subgroup Membership forms and X3T10 Mailing Subscription

form were available on the document table.  He said that people should use

the X3 forms to insure the X3 Secretariat has the latest membership

information.  The X3 Membership forms and the X3T10 Mailing Subscription

forms should be mailed to the X3 Secretariat.

He also announced that any eligible organization may join X3T10 at the

meeting by giving John a letter stating their interest in X3T10 and naming

the principal and alternate members.  Form letters were available for those

who forgot to bring a letter with them.

As is customary, the people attending introduced themselves and a copy of the

attendance list was circulated.  Copies of the draft agenda and general

information on X3T10 were made available to those attending.

2.    Approval of Agenda

The draft agenda was approved with the following additions:

11.3    Task Set Basis Selection

11.4    Delivery Mechanism Sequentiality

11.5    SCC Project Proposal

11.5    MMC Project Proposal

3.    Attendance and Membership

The membership requirements were reviewed.  X3 rules permit one vote per

organization.  An individual from a new organization must attend two out of

three consecutive plenary meetings.  He/she may apply for voting membership

the second meeting via a letter to the Chair of X3T10.  The individual may

vote at the second plenary meeting.  Working group meetings do not count

toward attendance requirements.  Since the meeting was the first meeting of

X3T10, it was a special case and those present were permitted to join without

prior attendance.

Organizations must also pay the X3 Service Fee ($300) to maintain membership.

This year, the International Participation fee ($300) will be billed

separately directly from ANSI.

The X3 Secretariat will be invoicing all former X3T9.2 members for X3T10

membership.  Those that do not wish to join the new committee should return

their invoice marked "cancel".  New members of X3T10 who were not on the

X3T9.2 roster will receive an invoice eventually.

The initial X3T10 membership was 60 organizations.  Enclosure (1) is the list

of the people present at the meeting and enclosure (2) is the current X3T10

membership list.

4.    X3 Tutorial

Per the requirements in 4.6.2 of the Organization, Rules and Procedures of

X3 (SD-2), "Each newly formed TC shall establish time on its first agenda

prior to conducting any technical business for presentation of an overview

on X3 and its procedure."  John Hill of OMC was present to give the overview

(see X3T10/94-034).

5.    Approval of Minutes - November Meeting Colo Spgs, CO (X3T9.2/93-176r1)

The minutes were approved as written except that the motion regarding

postponement of consideration of the motion on changing GPP to a technical

report should have noted that the motion was postponed until January 13,

1994.  The minutes will not be re-distributed.

6.    Document Distribution

Members who have any problems with their mailing subscription should deal

directly with Katrina Gray at (202) 626-5741.

Larry Lamers requested that all documents for the next mailing must be sent

so as to be received by noon on Friday, January 21, 1994.  Documents received

after this date will go into a subsequent mailing.

Larry Lamers reminded the committee of the requirements for document

submission to be included in the committee mailing.  Since X3T10 meets its

document archiving requirements by including the X3 Secretariat on the

mailing list, documents included in the mailing must be suitable for

microfilming.  This means that high-quality, single-sided originals with

adequate margins and proper identification must be submitted.  Document

X3T9.2/92-84 R1 contains the list of requirements for submissions.

As usual, minutes are also available on the SCSI BBS (719-574-0424) and will

be distributed to the SCSI Reflector E-mail list.  To be added to the SCSI

Reflector, send an electronic message to: scsiadm@WichitaKS.NCR.COM.

The current document register is enclosure (3).

7.    Call for Patents

This is a regular agenda item to identify any potential patent issues with

developing standards.

A document is available from ANSI, "Procedures for the Development and

Coordination of American National Standards", at no charge.  The last page

contains the ANSI patent policy.

X3 has recently approved two patent statements, one for the case where no

patents have been identified and another for the case where patent(s) have

been identified and the patent holder(s) have agreed to comply with the ANSI

patent policy.

Gene Milligan asked about a statement in the proposed SCSI-to-SSA mapping.

John Scheible stated that the statement was only applicable to SSA physical

and will be removed from the proposed draft in the next revision.

8.    Review of Old Action Items

1)    John Lohmeyer will contact Marilyn Courtot regarding AIIM activity.

Carry Over.

2)    John Lohmeyer will contact Fernando Podia regarding liaison reports.

Carry Over.

John Lohmeyer stated that for X3T10 action items would be numbered

sequentially without reusing previous numbers.

9.    Subgroup and/or Project Status Reports

9.1   Project Status Summary [Lohmeyer]

John Lohmeyer presented the current X3T10 project status summary.

9.2   SCSI-2 Status [Lamers]

John Lohmeyer reported that SCSI-2 was sent to the ANSI Board of Standards

Review and should be approved, but he had not yet seen the approval letter.

9.3   SCSI Common Access Method (CAM) [Dallas]

Ralph Weber reported.  Rev 5 has been prepared to address public review

comments.  There were significant editorial comments that need to be

reviewed.  The next working group will have expanded hours to deal with the

further editorial comments.

Gene Milligan moved and Ralph Weber seconded that revision 5 be accepted as

the current working draft document for CAM.  The motion passed 35:0.

9.4   AT Attachment Status (ATA) [Lamers]

John Lohmeyer reported that ATA had passed its second public review and is

now ready for the X3 letter ballot.  The X3 Secretariat held up the ballot

waiting for X3T10 to incorporate the correct ANSI patent statement.  Since

no patents have been identified to date, the statement saying no patents have

been identified will be included by the technical editor and the document

will be released for X3 letter ballot.

9.5   ATA Extensions (ATA-2) [Finch]

The ATA extensions project is being driven to conclusion so that it can be

forwarded.

Project proposals for ATA-3 standards are being drafted.  ATA-3 will included

a layered document structure with faster transfer rates and added device type

support.

The first set of projects will deal with ATA-PH, ATA-XPT, ATA-BC, ATA-PI.

A reflector for ATA has been established.  Send an email to

majordomo@dt.wdc.com.  The email should contain a line in the body of the

message that states "subscribe ata yourinternetaddress".  Postings to the ATA

reflector should be sent to ata@dt.wdc.com.

9.6   SCSI-3 Architecture Model (SAM) Status [Monia]

Charles Monia reported responses have been prepared to the letter ballot

comments.  All responses will be or have been posted to the SCSI reflector.

There are two issues that need plenary action (see agenda items 11.3 and

11.4).

The next revision of SAM will be available prior to the next plenary.

Charles will mail a copy of the document to anyone requesting it.  It most

likely will not be finished in time for the next mailing.

9.7   SCSI-3 Parallel Interface (SPI) Status [Lamers]

The SPI first public review is October 29, 1993 to February 26, 1994.  At its

September '93 meeting, X3T9.2 authorized its chairman to submit a public

review comment requesting that SCAM be incorporated into SPI as a normative

annex.

9.8   SCSI-3 Interlocked Protocol (SIP) Status [Lamers]

An editors meeting was held in December.  More work needs to be done.

Another working group meeting will be scheduled.

9.9   SCSI-3 Generic Packetized Protocol (GPP) Status [Stephens]

Gary Stephens reported.  Several changes have been made to accommodate

comments received.  A revision is planned to be available at the next

meeting.

9.10  SCSI-3 Serial Bus Protocol (SBP) Status [Marazas]

Scott Smyers reported.  A working group meeting is scheduled for January 26,

1994 to resolve the letter ballot comments.

9.11  SCSI-3 Fiber Channel Protocol (FCP) Status [Snively]

Robert Snively reported.  Revision 7a is available and addresses all comments

received from the letter ballot.  Several items came to light in the review

at the last working group.  A revision 8 will be in the next mailing.

Bob requested a letter ballot be conducted to forward FCP in conjunction with

the next mailing.  The chair stated he would include a letter ballot if FCP

Rev 8 is available at the time of the mailing.

9.12  SCSI-3 Primary Commands (SPC) Status [Weber]

Ralph Weber reported.  An outline for the SPC document was in the mailing.

A first draft will be available at the March meeting.  This topic will be on

the agenda for the next working group.

9.13  SCSI-3 Block Commands (SBC) [open]

A project editor is being sought.

9.14  SCSI-3 Stream Commands (SSC) [open]

Edward Lappin indicated that he is willing to act as project editor for the

SSC project.

9.15  SCSI-3 Graphics Commands (SGC) [open]

A project editor is being sought.

9.16  SCSI-3 Medium Changer Commands (SMC) [open]

A project editor is being sought.

9.17  Directly-Addressable Device Interface (DADI) []

No report.

9.18  SCSI Common Access Method -2 (CAM-2) (93-082) [Dallas]

This project is on hold until CAM-1 is ready to be returned to X3 for second

public review.

9.19  SCSI-3 Serial Storage Protocol (SSP) [Scheible]

John Scheible reported.  The project proposal is still in process.  A

proposal for an eventual working draft was in the last mailing.

9.20  SCSI-3 Controller Commands (SCC) (93-130) [Hagerman]

John Lohmeyer reported that while X3T9 did approve the SCC project proposal

in December, they failed to send it OMC for its meeting last week.  The

failure was due to misplacing the project proposal file.  Agenda item 11.5

was added to the agenda to re-forward the SCC project proposal to X3.

9.21  Replacement of ANSI X3.80-1988, Interfaces Between Flexible Disk

Cartridges and Their Host Controllers, with ISO 9315:1989.

In April '93, X3T9.2 voted to recommend that simultaneous withdrawal of

X3.80-1988 and adoption of ISO 9315:1989 as an ANSI standard.  X3T9 accepted

this recommendation and voted to make the same recommendation to X3.  The

public review period began October 15, 1993 and closes February 13, 1994.

9.22  RAID Study Group [Penokie]

George Penokie reported.  Two meetings have occurred since the last plenary.

The RAID model document has been revised.  An extended addressing proposal

has been developed and is stable.  It will be incorporated into the RAID

model.

10.   Old Business

10.1  Technical Information Bulletins

Gary Stephens had prepared a TIB on the tape partition issue.  Folks are

advised to review it.  It is anticipated that a forwarding motion will be

taken as soon as SCSI-2 is approved.

10.2  GPP: Standard or Technical Report? [Milligan] ()

A motion on whether GPP should be a standard or technical report was

postponed until this meeting:

Whereas having two I/O protocols for each SCSI physical interconnect is

a problem for the industry although there may be some niche applications

where GPP is useful,  Gene Milligan moved and Ed Gardner seconded that

X3T9.2 take the appropriate steps to convert it from a dpANS and to

publish it as a technical report.

Neil Wanamaker moved and Jim McGrath seconded that the question be called.

The motion passed: 22:8.

The main motion passed in a roll call vote:

Voting Organization       Name                        S Yes No Abs NP

------------------------- --------------------------- - --- -- --- --

3M Company                Mr. Alan R. Olson           P  1

AMP, Inc.                                             P             1

AT&T                      Mr. John Lohmeyer           P      1

Adaptec, Inc.             Mr. Norm Harris             P  1

Advanced Micro Devices    Mr. David Skinner           P  1

Amdahl Corp.              Mr. Neil T. Wanamaker       P      1

Ancot Corp.               Mr. Gary Porter             A      1

Apple Computer            Mr. Scott Smyers            P  1

BusLogic                  Mr. Clifford E. Strang Jr.  P  1

CMD Technology            Mr. Edward Haske            P  1

Ciprico Inc.              Mr. Gerry Johnsen           P      1

Circuit Assembly Corp.    Mr. Ian Morrell             A  1

Cirrus Logic Inc.         Mr. Dave Weber              A  1

Compaq Computer Corp.     Mr. Bill Galloway           P  1

Conner Peripherals        Mr. Joseph Wach             A  1

Dallas Semiconductor      Mr. Louis Grantham          P  1

Digital Equipment Corp.   Mr. Edward A. Gardner       A  1

Distributed Processing Te Mr. Tom Treadway            P  1

ENDL                      Mr. I. Dal Allan            P      1

Exabyte Corp.             Mr. Edward Lappin           P      1

FSI Consulting Services   Mr. Gary R. Stephens        P      1

Foxconn                                               P             1

Fujitsu                   Mr. Robert Liu              P  1

Future Domain Corp.       Mr. Mark Woithe             P  1

Harbor Electronics        Mr. Ray Kallio              P  1

Harting Electronik, Inc.                              P             1

Hewlett Packard Co.       Mr. Jeffrey L. Williams     P  1

Hitachi                   Dr. Sam Karunanithi         P  1

Honda Connectors                                      P             1

IBM Corp.                 Mr. George Penokie          P  1

Interphase Corp.          Mr. Varouj Der-Hacopian     A  1

JAE                       Mr. Guy R. Addis            A  1

Maxoptix Corp.            Ms. Donna Pope              P  1

Maxtor Corp.              Mr. Lawrence J. Lamers      A      1

Methode Electronics, Inc.                             P             1

Molex Inc.                Mr. Joe Dambach             P  1

NEC                       Mr. Bruce Anderson          P  1

National Semiconductor                                P             1

Oak Technology, Inc.                                  P             1

P. E. Logic Corp.                                     P             1

Pacific Electro Data Inc.                             P             1

Panasonic (Matsushita)    Mr. Charles I. Yang         A  1

Quantum Corp.             Mr. James McGrath           P  1

Rancho Technology, Inc.   Mr. Anuj Garg               A  1

Ricoh Corporation         Mr. Satwinder S. Mangat     A      1

Samsung Semiconductor     Mr. S. Jauher Zaidi         P  1

Seagate Technology        Mr. Gene Milligan           P  1

Silicon Systems, Inc.     Mr. Stephen G. Finch        P  1

Sony Electronics, Inc.    Mr. Mike Yokoyama           P  1

Storage Technology Corp.  Mr. Erich Oetting           P  1

Sun Microsystems, Inc.    Mr. Robert N. Snively       P  1

Tandem Computers          Mr. John Moy                P      1

Thomas & Betts                                        P             1

U.S. Design                                           P             1

UNISYS Corporation        Mr. Peter Dougherty         P      1

Unitrode                  Mr. Paul D. Aloisi          P  1

Western Digital Corporati Mr. Jeff Stai               P  1

Woven Electronics         Mr. Doug Piper              P  1

Totals:                       36  11  0  11  = 58

Note: The above tally is slightly different from the results tentatively

announced during the meeting.  Upon re-examining the membership list, it was

discovered that one observer organization erroneously was called during the

roll call (who voted in the affirmative).  This error was due to the inherent

difficulties in establishing the initial X3T10 membership and does not affect

the outcome of the vote.  Two organizations joined X3T10 after the roll call

vote was taken.

The chair requested a volunteer to revise the project proposal for GPP.  Gene

Milligan volunteered.

11.   New Business

11.1  X3T10 Subscription Mailing Program

Bob Snively moved and Jeff Williams seconded that X3T10 request the X3

Secretariat to initiate a Document Distribution Program for X3T10 and to

transfer the remaining funds from the X3T9.2 Document Distribution Program

to X3T10's fund.  The motion passed unanimously.

Steve Finch requested that consideration be given to combining the X3T10.1

task group mailing in the X3T10 mailing and allowing members of either

subgroup to subscribe to the combined mailing.  The chair stated he will

investigate the possibility.

11.2  Review of new documents

None.

11.3  Task Set Basis Selection

This item was added to the agenda at the request of the working group to

attempt resolution of this recurring topic.

Ralph Weber moved and Jeff Williams seconded that alternative task set

definitions (Annex A) be removed from SAM.  The motion carried 34:2.

11.4  Delivery Mechanism Sequentiality

Bob Snively moved and Scott Smyers seconded that in-order delivery not be

required by SAM.  The motion carried 32:4.

11.5  SCC Project Proposal

Bob Snively moved and Steve Finch seconded that X3T10 forward the SCSI-3

Controller Commands project proposal (X3T10/94-035r0) to X3 for further

processing.  The motion carried 47:0:0:5.

11.6  MMC Project Proposal

Steve Finch moved and Jeff Stai seconded that X3T10 forward the SCSI-3 Multi-

Media Commands project proposal (X3T10/94-036r0) to X3 for further

processing.  The motion carried 47:0:0:5.

12.   ISO [Milligan]

Gene Milligan, the acting X3T10 International Representative, gave an oral

report.  The electronic report was as follows:

ISO Report

January 1994

The following is extracted from a letter sent by the IR to Walter

v. Pattay, Siemens Nixdorf:

I have been trying to reach the SC 25 Secretariat for a while to

find out why nothing seems to have progressed since the June

meeting. When I called Barth the phone just rings without answer

and without voice mail. I was going to switch my enquiry to Prof.

Popovic this week to find out why we have heard nothing since the

meeting.

Last Thursday I heard a rumor that Hr. Barth had left Siemens and

that you had taken over  the Secretariat. So that added another

question to ask Prof. Popovic. I presume from your fax that the

rumor may be factual. So I will ask you the following questions,

instead of Popovic,  most of the questions  relate to the WG 4

report:

1) Would you please confirm what is the Status and Makeup of the SC

25 Secretariat. (Depending on the answer to this item please

address or ignore the following items.)

2) The FDDI HRC (Project 25.13.01.06) was submitted for ISO/IEC

publication. What is the publication status?

3) The ESDI  10 222 (Project 25.13.10.12) was submitted for ISO/IEC

publication. What is the publication status? This was a specific

resolved action item for the SC 25 Secretariat to check on from the

June meeting.

4) SCSI-2 9316-1 (was previously 10 288) (Project 25.13.10.12) was

to be issued as a DIS letter ballot. What is the status of the DIS

ballot?

5) IPI-6  (Project 25.13.10.14) was to be issued as 9318-6 CD

letter ballot. What is the status of the CD ballot?

6) HIPPI-PH (Project 25.13.12.01) was to be issued as 11 518-1 DIS

letter ballot. What is the status of the DIS ballot?

7) HIPPI-FP (Project 25.13.12.02) was to be issued as 11 518-1 CD

letter ballot. What is the status of the CD ballot?

8) HIPPI-LE (Project 25.13.12.03) was to be issued as 11 518-1 CD

letter ballot. What is the status of the CD ballot?

9) FC-PH (Project 25.13.13.01) (WG 4 Den Haag 5) was to be issued

as a CD letter ballot. What is the status of the CD ballot? (Note:

If this CD has still not issued and if it would not cause much

further delay I could provide the latest X3T9 revision for the CD.)

Many of the questions have not yet been answered and it is not

known if Barth is still with Siemens. But Walter v. Pattay is the

new secretariat. He is now working down the backlog. As a first

step SCSI-2 has been processed and a DIS should now be forthcoming.

It seems reasonable to expect faster processing in the future.

13.   Liaison Reports

13.1  X3T10.1 Task Group Report [Scheible]

John Scheible reported orally.  See document 94-017 for details.

13.2  X3T9.3 Liaison Report [Cummings]

Roger Cummings provided the following report:

91 people attended the December X3T9.3 Plenary.  Final membership

in X3T9.3 was 45.

FC-PH Revision 4.2 is in Second Public Review, closing 2/15/94.

FC-FP Revision 2.1 is in First Public Review, closing May 21,1994.

FC-AL Revision 3.4 is in X3T9 Letter Ballot, closing 1/21/94.

HIPPI-ATM, FC-ATM, HIPPI-FP Amendement project proposals have beeen

approved.  FC-SB draft expected at 2/94 X3T11 Plenary for Letter

Ballot for First Public Review.

Activity starting in 2/94 on:

Optical variants faster than 1062.5 megabaud

Additional copper variants (e.g. STP-5)

New features for FC-EP (fractional bandwidth etc.)

13.3  Small Form Factor Industry Group [Allan]

Dal Allan reported on the recent SFF activities.

The January meeting was held on Monday 1/10 at the Catamaran Hotel.

All documents being balloted passed, and the projects for >528MB and ATAPI

CD-ROM were approved.

The suggestion that the Unitized Parallel Connector (3-in-1) for Rack

Applications be dropped was opposed.  As a result of the discussion, it was

agreed that it is important to define the 3 1/2" from factor with the same

degree of precision as the 2 1/2".

The call for a leader was met by Jay Neer, who volunteered to lead a SSWG

which will be held in the first week of February to consider the following:

- Form factor of 3 1/2" drives

- Extended form factor of 3 1/2" drives with Unitized Serial Connector

-     "        "    "   " 2 1/2"    "     "     "      "       "

-     "        "    "   " 5 1/4"    "     "     "      "       "

- Placement of Serial Unitized Connector on 2 1/2" drives

-     "        "    "   "      "       " 3 1/2" drives

-     "        "    "   "      "       " 5 1/4" drives

An updated revision of the >528MB draft was presented by Hale Landis.

Bill Ham extended the description of the various wiring schemes (22 of 26 are

completed) between unlike connectors.

ATAPI CD-ROM was distributed shortly before Christmas, and a SSWG will be

held in late January to address the remaining issues.

13.4  IEEE P1394 [Marazas]

Scott Smyers reported.  The editing is on going - forwarding is expected in

March '94.  Sections 7 and 8 still need work.

13.5  IT8 [Lohmeyer]

No report.

13.6  IEEE P1285 Status [Pak]

Scott Smyers said that Dennis Pak had given him a report, which Scott had

forgotten to bring with him.  The report is in document 94-037.

13.7  PCMCIA Report

No report.

13.8  AIIM C21 [Podio]

No report.

14.   Review of Action Items

1)    John Lohmeyer will contact AIIM regarding AIIM activities.

2)    John Lohmeyer will contact Fernando Podia regarding liaison reports.

3)    Larry Lamers will incorporate the correct patent statement in the dpANS

AT Attachment and will send a copy to the X3 Secretariat.

4)    On behalf of X3T10, John Lohmeyer will submit a public review comment

against SPI requesting that SCAM be incorporated.

5)    John Lohmeyer to progress the SCC project proposal to X3.

6)    John Lohmeyer to progress the MMC project proposal to X3.

7)    John Lohmeyer to discuss combined TC/TG mailing with X3 Secretariat.

8)    Gene Milligan to revise project proposal on GPP.

9)    John Lohmeyer to contact the X3 Secretariat regarding electronic SDs.

15.   Meeting Schedule

The next plenary meeting of X3T10 will be March 17-18, 1994, in Newport

Beach, CA, at the Hyatt Newporter Hotel (714-729-1234), hosted by QLogic

Corporation.  The meeting will begin at 1:00 pm on Thursday and at 8:30 am

on Friday (if necessary).

The meeting list for the next plenary week is:

Meeting Map -- March 14-18, 1994 -- Newport Beach, CA

(1/13/94 Version)

Sunday 3/13        9   10  11  12  1   2   3   4   5

+=============+

|Gen Edit Sess|              (10)

+=============+

Monday 3/14        9   10  11  12  1   2   3   4   5   6   7   8

+-------------------+=======================+

| Small Form Factor | ATA Working Group     |  (40)

+-------------------+=======================+

| SCSI CAM Working Group                    |  (25)

+===========================================+

Tuesday 3/15       9   10  11  12  1   2   3   4   5   6   7   8

+---------------+===============+

| SSA-UIG       | X3T10.1 (SSA) |              (30)

+===============+===============+

| SBP LB Res    | P1394 Plenary |              (30)

+===============+---------------+

| RAID SG / RAB                 |              (40)

+===============================+

| P1394 Connector Task Group    |              (15)

+-------------------------------+

Wednesday 3/16     9   10  11  12  1   2   3   4   5   6   7   8

+===========================================+

| SCSI-3 WG                                 |  (55)

+===============================+===========+

| P1394 Plenary Meeting         |              (30)

+-------------------------------+

Thursday 3/17      9   10  11  12  1   2   3   4   5   6   7   8

+============+  +===========================+

| SSP WG     |  | X3T10 Plenary             |  (30/80)

+============+--+===========================+

Friday 3/18    8   9   10  11  12  1   2

+=====================+

| X3T10 Plenary       |                          (80)

+=====================+

Meetings marked with == are X3T10-authorized meetings.

Meetings marked with -- are authorized by other groups and may change

without notice.

Several other X3T10 events are also planned:

Event                           Day/Date  Time      Location/contact

------------------------------- --------- --------- -----------------------

ATA Working Group               Fri  1/28 9a - 5p   Milpitas/Finch

ATA Working Group               Wed  2/23 9a - 5p   Bay Area/Finch

ATA Working Group               Wed  3/30 9a - 5p   TBD/Finch

ATA-2 Editors Meeting           TBD                 TBD/Finch

MMC Study Group                 ~ 2/22 or 2/24      Bay Area/Harris

RAID Study Group                Tue  2/15 9a - 5p   Austin/Penokie

For more information about X3T10 activities, please contact the following

people:

Activity                     Contact        Telephone/Email

---------------------------- -------------- ------------------------------

CAM Working Group            Bill Dallas    (603) 881-2508

dallas@wasted.enet.dec.com

ATA Working Group            Steve Finch    (714) 573-6808

5723283@mcimail.com

GPP Working Group            Gary Stephens  (602) 321-1725

6363897@mcimail.com

MMC Study Group              Norm Harris    (408) 945-8600 x2230

nharris@adaptec.com

RAID Study Group             George Penokie (507) 253-5208

gop@rchvmp3.vnet.ibm.com

SAM Working Group            Charles Monia  (508) 841-6757

monia@starch.enet.dec.com

SBP Working Group            Jerry Marazas  (407) 982-4423

marazas@bcrvmpc2.vnet.ibm.com

SPI Contact                  Larry Lamers   (408) 432-4776

71540.2756@compuserve.com

SIP Contact                  Larry Lamers   (408) 432-4776

71540.2756@compuserve.com

SCAM Contact                 Ed Gardner     (719) 548-2247

gardner@ssag.enet.dec.com

Other Activities             John Lohmeyer  (719) 573-3362

john.lohmeyer@ftcollinsco.ncr.com

The long-term X3T10 meeting week schedule is:

Date             Location                  Host/Comments

---------------  ------------------------  --------------------------

Mar 14-18, 1994  Newport Beach, CA         Emulex

May 16-20, 1994  Harrisburg, PA            AMP

Jul 18-22, 1994  Manchester, NH            DEC

Sep 12-16, 1994  Houston, TX               Compaq

Nov 7-11,  1994  Southern California       Western Digital

John Lohmeyer presented a foil showing a proposal for future plenary weeks

(May '94 and beyond) that would move the ATA working group to Thursday

morning.  The idea is to make it easier for those interested in ATA to attend

the X3T10 meeting:

Proposed Future Meeting Map (after March '94)

Monday             9   10  11  12  1   2   3   4   5   6   7   8

+===========================================+

| SCSI CAM Working Group                    |  (25)

+=================+=============+===========+

|Gen Edit Sess|              (10)

+=============+

Tuesday            9   10  11  12  1   2   3   4   5   6   7   8

+---------------+===============+

| SSA-UIG       | X3T10.1       |              (30)

+===============+===============+

| RAID SG / RAB                 |              (40)

+===============================+

| IEEE P1394 Plenary            |              (30)

+-------------------------------+

| P1394 Connector Task Group (?)|              (15)

+-------------------------------+

Wednesday          9   10  11  12  1   2   3   4   5   6   7   8

+===========================================+

| SCSI-3 WG                                 |  (45)

+===============================+===========+

| P1394 Plenary Meeting         |              (30)

+-------------------------------+

Thursday           9   10  11  12  1   2   3   4   5   6   7   8

+===============+===========================+

| ATA WG        | X3T10 Plenary             |  (40/80)

+===============+===========================+

| SSP WG (?)    |                              (30)

+===============+

Friday         8   9   10  11  12  1   2

+=====================+

| X3T10 (if needed)   |                          (80)

+=====================+

| Small Form Factor   |                          (40)

+---------------------+

Several variations of the above proposal were also discussed.  A decision on

the meeting schedule is planned for the March plenary meeting.

15.1  Authorization of Working Group Meetings

John noted that the working group meetings are required to operate under the

same voting procedures as does X3T10 (that is, one vote per organization) and

the results of a working group are not binding on the parent committee.  This

later requirement is why we have traditionally only used non-binding straw

polls in working group meetings.  Any issue where the voting rules would make

a difference is automatically deferred to the plenary group.

Jeff Stai moved and Ed Gardner seconded that X3T10 authorize the X3T10-

related meetings and events in the list above.  The motion passed

unanimously.

16.   Adjournment

The meeting was adjourned at 8:00 p.m. on Thursday January 13, 1994.

Meeting Attenders -- Enclosure (1)

Name          S       Organization         Electronic Mail Address

---------------------- - ------------------------- -------------------------

Mr. Alan R. Olson      P 3M Company                usmmm5cd@ibmmail.com

Mr. Norm Harris        P Adaptec, Inc.             nharris@adaptec.com

Mr. David Skinner      P Advanced Micro Devices    David.Skinner@amd.com

Mr. Neil T. Wanamaker  P Amdahl Corp.              ntw20@amail.amdahl.com

Mr. Jerrie L. Allen    O Amdahl Corp.              Jerrie.Allen@cccmvs.amdah

l.com

Mr. Charles Brill      P AMP, Inc.                 cbrill@cup.portal.com

Mr. Ken Scherzinger    V Amphenol Spectra Strip

Mr. Gary Porter        A Ancot Corp.               garyp@ancot.com

Mr. Scott Smyers       P Apple Computer            smyers.s@applelink.apple.

com

Mr. Christian Mollard  O BULL S.A.

Mr. Clifford E. Strang P BusLogic

Jr.

Mr. Gerry Johnsen      P Ciprico Inc.              gerry@cipric.mn.org

Mr. Dennis Lang        P Circuit Assembly Corp.

Mr. Ian Morrell        A Circuit Assembly Corp.

Mr. Dave Weber         A Cirrus Logic Inc.         davew@cirrus.com

Mr. Edward Haske       P CMD Technology

Mr. Bill Galloway      P Compaq Computer Corp.     billg@bangate.compaq.com

Mr. Joseph Wach        A Conner Peripherals        Joe_Wach+aDevelopment+aCo

lorado%conner@mcimail.com

Mr. Louis Grantham     P Dallas Semiconductor      grantham@dalsemi.com

Mr. Charles Monia      P Digital Equipment Corp.   monia@starch.enet.dec.com

Dr. William Ham        A Digital Equipment Corp.   ham@subsys.enet.dec.com

Mr. Ralph Weber        A Digital Equipment Corp.   weber@star.enet.dec.com

Mr. Douglas Hagerman   A Digital Equipment Corp.   hagerman@starch.enet.dec.

com

Mr. Edward A. Gardner  A Digital Equipment Corp.   gardner@ssag.enet.dec.com

Mr. Tom Treadway       P Distributed Processing    Treadway@dpt.com

Tech.

Mr. I. Dal Allan       P ENDL                      2501752@mcimail.com

Mr. Kenneth J. Hallam  A ENDL                      khallam@endlas.com

Mr. Edward Lappin      P Exabyte Corp.             tedl@exabyte.com

Mr. Nick Ladas         P Foxconn

Mr. Gary R. Stephens   P FSI Consulting Services   6363897@mcimail.com

Mr. Robert Liu         P Fujitsu Computer

Products,Am

Mr. Kevin R. Pokorney  A Fujitsu Computer          pokorney@intellistor.com

Products,Am

Mr. Joel Urban         A Fujitsu

Microelectronics/ECD

Mr. Mark Woithe        P Future Domain Corp.       markw@fdc.mhs.compuserve.

com

Mr. Ray Kallio         P Harbor Electronics

Ms. Sherry L. Johnson  P Harting Electronik, Inc.

Mr. Jeffrey L.         P Hewlett Packard Co.       jlw@hpdmd48.boi.hp.com

Williams

Mr. Paul Boulay        A Hitachi Computer Products p_boulay@hitachi.com

Dr. Sam Karunanithi    P Hitachi Micro Systems,    skarunan@hmsi.hitachi.com

Inc.

Mr. Thomas J. Kulesza  A Honda Connectors

Mr. George Penokie     P IBM Corp.                 gop@rchvmp3.vnet.ibm.com

Mr. John P. Scheible   A IBM Corp.                 Scheible@vnet.ibm.com

Mr. Robert Kembel      O IBM Corp.                 rkembel@vnet.ibm.com

Mr. Varouj             A Interphase Corp.          varouj@iphase.com

Der-Hacopian

Mr. Guy R. Addis       A JAE Electronics

Mr. Dean Wallace       P Linfinity Micro

Meeting Attenders -- Enclosure (1)

Name          S       Organization         Electronic Mail Address

---------------------- - ------------------------- -------------------------

Mr. Robert Bellino     P Madison Cable Corp.

Ms. Donna Pope         P Maxoptix Corp.            6471084@mcimail.com

Mr. Ron Roberts        P Maxtor Corp.              ron_roberts@maxtor.com

Mr. Lawrence J. Lamers A Maxtor Corp.              larry_lamers@maxtor.com

Mr. Bob Masterson      P Methode Electronics, Inc.

Mr. Joe Dambach        P Molex Inc.

Mr. Robbie Shergill    P National Semiconductor    rss@berlioz.nsc.com

Mr. John Lohmeyer      P NCR Corp.                 john.lohmeyer@ftcollinsco

.ncr.com

Mr. Charles Binford      NCR Corp.                 Charles.Binford@WichitaKS

.NCR.COM

Mr. Bruce Anderson     P NEC Technologies Inc.     banderson@nectech.com

Mr. Cliff Carlson      V NRAD                      wcarlson@nosc.mil

Mr. Peter Brown        P Oak Technology, Inc.      brown@oaktech.com

Mr. Earl J. Perera     P P. E. Logic Corp.

Mr. Thomas Steury      P Pacific Electro Data Inc.

Mr. Charles I. Yang    A Panasonic Industrial Co.

Mr. James McGrath      P Quantum Corp.             JMCGRATH@QNTM.COM

Mr. Anuj Garg          A Rancho Technology, Inc.

Mr. Satwinder S.       A Ricoh Corporation         mangat@str.ricoh.com

Mangat

Mr. S. Jauher Zaidi    P Samsung Semiconductor     syed@sam.com

Mr. Gene Milligan      P Seagate Technology        Gene_Milligan@notes.seaga

te.com

Mr. Gerald Houlder     A Seagate Technology        Gerry_Houlder@notes.seaga

te.com

Mr. Stephen G. Finch   P Silicon Systems, Inc.     5723283@mcimail.com

Mr. Mike Yokoyama      P Sony Electronics, Inc.    masayuki@cppc.sel.sony.co

m

Mr. Erich Oetting      P Storage Technology Corp.  Erich_Oetting@Stortek.com

Mr. Roger Cummings     A Storage Technology Corp.  Roger_Cummings@Stortek.co

m

Mr. Robert N. Snively  P Sun Microsystems, Inc.    Bob.Snively@sun.com

Mr. John Moy           P Tandem Computers          moy_john@tandem.com

Mr. Bill Boyd          O Texas Instruments

Mr. Harvey Waltersdorf P Thomas & Betts

Mr. Chuck Duquette     P U.S. Design               dennis@usdc.com

Mr. Peter Dougherty    P UNISYS Corporation        peter.dougherty@mv-oc.com

Mr. Paul D. Aloisi     P Unitrode Integrated       Aloisi@uicc.com

Circuits

Mr. Jeff Stai          P Western Digital           stai@dt.wdc.com

Corporation

Mr. Tak Asami          A Western Digital           asami@dt.wdc.com

Corporation

Mr. Devon Worrell      A Western Digital           worrell@dt.wdc.com

Corporation

Mr. Doug Piper         P Woven Electronics         WOVEN ELECTRONICS / MCI

ID: 549-9900

82 People Present

Status Key:  P   -  Principal

A   -  Alternate

O   -  Observer

L   -  Liaison

S,V -  Visitor

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Sat Feb 12 21:08:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04329; Sat, 12 Feb 94 21:08:49 PST

Received: from ncrhub1.NCR.COM (h192-127-251-3.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA04258; Sat, 12 Feb 94 21:02:39 PST

Received: from ncrcolo by ncrhub1.NCR.COM id cs28789; 11 Feb 94 14:14 EST

Received: by ncrcolo.FtCollinsCO.NCR.com; 11 Feb 94 11:26:42 MST

Subject: ATA 3 Project Proposals

To: ATA Reflector <ata@dt.wdc.com>

Date: Fri, 11 Feb 94 11:19:50 MST

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9402111414.cs28789@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Attached at the four ATA3 project proposals, recently distributed by

Steve Finch, edited into the format the X3 Operational Management

Committee (OMC) prefers.  I have made minor editing changes as

follows:

1. The 4.4 LEGAL CONSIDERATIONS section was changed to:

"There are no known legal considerations.  A Call for Patents

will be made under Milestone 7 of the SD-2."  This is consistent

with the X3T9 standard phrase.

2. The 5.2 X3 STANDARDS DEVELOPMENT PROJECTS section was changed

to reference the ATA and ATA-2 project numbers, not X3T9.2

draft document numbers.

3. Global changes were made to remove references to X3T9 and to

fix references to X3T10 as a Task Group to be a Technical

Committee.

4. The projects roadmap figures in section 2.1 were changed from

'HDD Command Set' to 'Block Command Set' to match the name of

the ATA3 Block Command Set project proposal.

ZIP'd Word Perfect versions of these project proposals are available

for downloading from the SCSI BBS (files 94-019r1.wpz -- 94-022r1.wpz).

John Lohmeyer

^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v

X3T10/94-019r1

Project Proposal

For a New

X3 Standard

-----------------------------------------------------------------------------

ATA-3 Physical Interface

(ATA3-PH)

-----------------------------------------------------------------------------

March 17, 1994

1  IDENTIFICATION OF PROPOSED PROJECT

1.1   TITLE:  ATA-3 Physical Interface (ATA3-PH)

1.2   PROPOSER:  X3T10

1.3   DATE SUBMITTED:  March 17, 1994  X3T10 forwarding date.

1.4   PROJECT TYPE:  D - Development of standards within X3 TC.

2  JUSTIFICATION OF PROPOSED STANDARD

2.1   NEEDS:

The draft standard for AT Attachment has been completed and the draft for the

ATA Extensions (ATA-2) is near completion, but as the popularity of the

interface has increased, its application area has grown outside the

originally intended purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of the

ATA interface and to take advantage of the huge installed BIOS (Basic

Input/Output System) base, and software.  Support is needed for traditional

HDD devices plus additional devices such as CD-ROM and tape devices.  It is

expected that the SCSI-3 command sets can be used for these new device types.

In addition, support is also needed for removable media devices and expanded

number of cabling options.  The resulting increase in complexity has made it

desirable to divide the ATA standard into a hierarchical set of standards,

of which the ATA-3 Physical Interface is one.

+-------------+ +-------------+   +=============+

| SCSI-3      | | Other       |   |    ATA-3    |

| Multimedia  | | SCSI-3      |   |    Block    |

| Command Set | | Command Sets|   | Command Set |

+------+------+ +------+------+   +======+======+

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           +======+======+

| ATA-3 Packet|           |   ATA-3     |

| Interface   | - - - - ->| Transport   |

| Command Set |           | Protocol    |

+======+======+           +======+======+

|                         |

+------------+------------+

|

|

*************

*   ATA-3   *

Proposed -->  * Physical  *

Project       * Interface *

*************

2.2   RECOMMENDED SCOPE OF STANDARD:

The proposed ATA-3 Physical Interface standard shall maintain a high degree

of compatibility with the AT Attachment while providing documentation for new

capabilities including the following candidates:

a)    Provide a means to support the AT Attachment protocol on a variety of

physical interfaces such as that for the newly-emerging requirements

for memory cards.

b)    Other capabilities which fit within the general scope of implementing

the AT Attachment Interface on a broader range of applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this proposed

standard would be used to provide additional capabilities.

2.3   EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4   EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO CURRENT AND

POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to insure that the AT Attachment has

an upward, highly compatible growth path.  This will insure that current

investments in AT Attachment are provided with more stability in the face of

technological developments.

3  DESCRIPTION OF PROPOSED PROJECT:

3.1   TYPE OF DOCUMENT:  Standard.

3.2   DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

3.3   EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATA-3 Physical Interface standard is for use in closed systems.

3.4   RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATA-3 Physical Interface

standard:

(1)   Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through

press releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Physical Interface standard.

(2)   Establish functional requirements for ATA-3 Physical Interface

functional additions along with downward compatibility requirements.

(3)   Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

(4)   Consider the results of ATA3-Physical Interface testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T10.

(5)   Submit the draft proposed standard to X3 for further processing.

3.5   RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN SUBJECT MATTER:

The current membership of X3T10 includes representatives from all parts of

the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The members

of X3T10 have expressed their desire to participate and cooperate in the

development of this proposed standard.

There are sufficient resources to complete the development of this standard

without delaying work on other standards.

3.6   RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical Committee

X3T10 which is responsible for developing the draft ATA Extensions standard.

3.7   ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets for two days bi-monthly. Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8   TARGET DATE FOR DPANS TO X3 (MILESTONE 10):  December 1995

3.9   ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 10 years.

4  IMPLEMENTATION IMPACTS

4.1   IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATA-3 Physical Interface standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.2   IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATA-3 Physical Interface standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.3   TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATA-3 Physical Interface testing

as may be available to the committee through the voluntary efforts of the

various participants in X3T10.  With this method all costs are borne by the

organizations of the various participants and have for the most part been

mainly an adjunct of their normal development costs.

4.4   LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will be made

under Milestone 7 of the SD-2.

5  CLOSELY RELATED STANDARDS ACTIVITIES

5.1   EXISTING STANDARDS:  None.

5.2   X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface -- Project 791-D

ATA Extensions -- Project 948-D

5.3   X3 STUDY GROUPS:  None.

5.4   OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

5.5   ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6   OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:  None.

5.7   RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

5.8   RECOMMENDATIONS FOR CLOSE LIAISON:  None.

^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v

X3T10/94-020r1

Project Proposal

For a New

X3 Standard

-----------------------------------------------------------------------------

ATA-3 Transport Protocol

(ATA3-XPT)

-----------------------------------------------------------------------------

March 17, 1994

1  IDENTIFICATION OF PROPOSED PROJECT

1.1   TITLE:  ATA-3 Transport Protocol (ATA3-XPT)

1.2   PROPOSER:  X3T10

1.3   DATE SUBMITTED:  March 17, 1994  X3T10 forwarding date.

1.4   PROJECT TYPE:  D - Development of standards within X3 TC.

2  JUSTIFICATION OF PROPOSED STANDARD

2.1   NEEDS:

The draft standard for AT Attachment has been completed and the draft for the

ATA Extensions (ATA-2) is near completion, but as the popularity of the

interface has increased, its application area has grown outside the

originally intended purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of the

ATA interface and to take advantage of the huge installed BIOS (Basic

Input/Output System) base, and software.  Support is needed for traditional

HDD devices plus additional devices such as CD-ROM and tape devices.  It is

expected that the SCSI-3 command sets can be used for these new device types.

In addition, support is also needed for removable media devices and expanded

number of cabling options.  The resulting increase in complexity has made it

desirable to divide the ATA standard into a hierarchical set of standards,

of which the ATA-3 Transport Protocol is one.

+-------------+ +-------------+   +=============+

| SCSI-3      | | Other       |   |    ATA-3    |

| Multimedia  | | SCSI-3      |   |    Block    |

| Command Set | | Command Sets|   | Command Set |

+------+------+ +------+------+   +======+======+

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           ***************

| ATA-3 Packet|           *   ATA-3     * <- Proposed

| Interface   | - - - - ->* Transport   *    Project

| Command Set |           * Protocol    *

+======+======+           ***************

|                         |

+------------+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

| Interface |

+===========+

2.2   RECOMMENDED SCOPE OF STANDARD:

The proposed ATA-3 Transport Protocol standard shall maintain a high degree

of compatibility with the AT Attachment while providing documentation for new

capabilities including the following candidates:

a)    Provide a means to support the AT Attachment protocol on a variety of

physical interfaces such as that for the newly-emerging requirements

for memory cards.

b)    Provide a means to support a variety of device types on the AT

Attachment Interface.

c)    Other capabilities which fit within the general scope of implementing

the AT Attachment Interface on a broader range of applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this proposed

standard would be used to provide additional capabilities.

2.3   EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4   EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO CURRENT AND

POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to insure that the AT Attachment has

an upward, highly compatible growth path.  This will insure that current

investments in AT Attachment are provided with more stability in the face of

technological developments.

3  DESCRIPTION OF PROPOSED PROJECT:

3.1   TYPE OF DOCUMENT:  Standard.

3.2   DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

3.3   EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATA-3 Transport Protocol standard is for use in closed systems.

3.4   RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATA-3 Transport Protocol

standard:

(1)   Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through

press releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Transport Protocol standard.

(2)   Establish functional requirements for ATA-3 Transport Protocol

functional additions along with downward compatibility requirements.

(3)   Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

(4)   Consider the results of ATA3-Transport Protocol testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T10.

(5)   Submit the draft proposed standard to X3 for further processing.

3.5   RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN SUBJECT MATTER:

The current membership of X3T10 includes representatives from all parts of

the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The members

of X3T10 have expressed their desire to participate and cooperate in the

development of this proposed standard.

There are sufficient resources to complete the development of this standard

without delaying work on other standards.

3.6   RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical Committee

X3T10 which is responsible for developing the draft ATA Extensions standard.

3.7   ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets for two days bi-monthly. Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8   TARGET DATE FOR DPANS TO X3 (MILESTONE 10):  December 1995

3.9   ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 10 years.

4  IMPLEMENTATION IMPACTS

4.1   IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATA-3 Transport Protocol standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.2   IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATA-3 Transport Protocol standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.3   TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATA-3 Transport Protocol testing

as may be available to the committee through the voluntary efforts of the

various participants in X3T10.  With this method all costs are borne by the

organizations of the various participants and have for the most part been

mainly an adjunct of their normal development costs.

4.4   LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will be made

under Milestone 7 of the SD-2.

5  CLOSELY RELATED STANDARDS ACTIVITIES

5.1   EXISTING STANDARDS:  None.

5.2   X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface -- Project 791-D

ATA Extensions -- Project 948-D

5.3   X3 STUDY GROUPS:  None.

5.4   OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

5.5   ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6   OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:  None.

5.7   RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

5.8   RECOMMENDATIONS FOR CLOSE LIAISON:  None.

^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v

X3T10/94-021r1

Project Proposal

For a New

X3 Standard

-----------------------------------------------------------------------------

ATA-3 Packet Interface

(ATA3-PI)

-----------------------------------------------------------------------------

March 17, 1994

1  IDENTIFICATION OF PROPOSED PROJECT

1.1   TITLE:  ATA-3 Packet Interface (ATA3-PI)

1.2   PROPOSER:  X3T10

1.3   DATE SUBMITTED:  March 17, 1994  X3T10 forwarding date.

1.4   PROJECT TYPE:  D - Development of standards within X3 TC.

2  JUSTIFICATION OF PROPOSED STANDARD

2.1   NEEDS:

The draft standard for AT Attachment has been completed and the draft for the

ATA Extensions (ATA-2) is near completion, but as the popularity of the

interface has increased, its application area has grown outside the

originally intended purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of the

ATA interface and to take advantage of the huge installed BIOS (Basic

Input/Output System) base, and software.  Support is needed for traditional

HDD devices plus additional devices such as CD-ROM and tape devices.  It is

expected that the SCSI-3 command sets can be used for these new device types.

In addition, support is also needed for removable media devices and expanded

number of cabling options.  The resulting increase in complexity has made it

desirable to divide the ATA standard into a hierarchical set of standards,

of which the ATA-3 Packet Interface is one.

+-------------+ +-------------+   +=============+

| SCSI-3      | | Other       |   |    ATA-3    |

| Multimedia  | | SCSI-3      |   |    Block    |

| Command Set | | Command Sets|   | Command Set |

+------+------+ +------+------+   +======+======+

|               |                 |

+-------+-------+                 |

|                         |

***************           +======+======+

Proposed -> * ATA-3 Packet*           |   ATA-3     |

Project     * Interface   * - - - - ->| Transport   |

* Command Set *           | Protocol    |

***************           +======+======+

|                         |

+------------+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

| Interface |

+===========+

2.2   RECOMMENDED SCOPE OF STANDARD:

The proposed ATA-3 Packet Interface standard shall maintain a high degree of

compatibility with the AT Attachment while providing documentation for new

capabilities including the following candidates:

a)    Provide a means to support a variety of device types utilizing a

packetized command block in a device independent way.

b)    Other capabilities which fit within the general scope of implementing

the AT Attachment Interface on a broader range of applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this proposed

standard would be used to provide additional capabilities.

2.3   EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4   EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO CURRENT AND

POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to insure that the AT Attachment has

an upward, highly compatible growth path.  This will insure that current

investments in AT Attachment are provided with more stability in the face of

technological developments.

3  DESCRIPTION OF PROPOSED PROJECT:

3.1   TYPE OF DOCUMENT:  Standard.

3.2   DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

3.3   EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATA-3 Packet Interface standard is for use in closed systems.

3.4   RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATA-3 Packet Interface

standard:

(1)   Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through

press releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Packet Interface standard.

(2)   Establish functional requirements for ATA-3 Packet Interface functional

additions along with downward compatibility requirements.

(3)   Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

(4)   Consider the results of ATA3-Packet Interface testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T10.

(5)   Submit the draft proposed standard to X3 for further processing.

3.5   RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN SUBJECT MATTER:

The current membership of X3T10 includes representatives from all parts of

the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The members

of X3T10 have expressed their desire to participate and cooperate in the

development of this proposed standard.

There are sufficient resources to complete the development of this standard

without delaying work on other standards.

3.6   RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical Committee

X3T10 which is responsible for developing the draft ATA Extensions standard.

3.7   ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets for two days bi-monthly. Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8   TARGET DATE FOR DPANS TO X3 (MILESTONE 10):  December 1995

3.9   ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 10 years.

4  IMPLEMENTATION IMPACTS

4.1   IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATA-3 Packet Interface standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.2   IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATA-3 Packet Interface standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.3   TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATA-3 Packet Interface testing as

may be available to the committee through the voluntary efforts of the

various participants in X3T10.  With this method all costs are borne by the

organizations of the various participants and have for the most part been

mainly an adjunct of their normal development costs.

4.4   LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will be made

under Milestone 7 of the SD-2.

5  CLOSELY RELATED STANDARDS ACTIVITIES

5.1   EXISTING STANDARDS:  None.

5.2   X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface -- Project 791-D

ATA Extensions -- Project 948-D

5.3   X3 STUDY GROUPS:  None.

5.4   OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

5.5   ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6   OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:  None.

5.7   RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

5.8   RECOMMENDATIONS FOR CLOSE LIAISON:  None.

^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v

X3T10/94-022r1

Project Proposal

For a New

X3 Standard

-----------------------------------------------------------------------------

ATA-3 Block Command Set

(ATA3-BC)

-----------------------------------------------------------------------------

March 17, 1994

1  IDENTIFICATION OF PROPOSED PROJECT

1.1   TITLE:  ATA-3 Block Command Set (ATA3-BC)

1.2   PROPOSER:  X3T10

1.3   DATE SUBMITTED:  March 17, 1994  X3T10 forwarding date.

1.4   PROJECT TYPE:  D - Development of standards within X3 TC.

2  JUSTIFICATION OF PROPOSED STANDARD

2.1   NEEDS:

The draft standard for AT Attachment has been completed and the draft for the

ATA Extensions (ATA-2) is near completion, but as the popularity of the

interface has increased, its application area has grown outside the

originally intended purpose of direct attachment of small disk drives to

desktop computers.

ATA-3 is needed to continue to broaden the scope and application area of the

ATA interface and to take advantage of the huge installed BIOS (Basic

Input/Output System) base, and software.  Support is needed for traditional

HDD devices plus additional devices such as CD-ROM and tape devices.  It is

expected that the SCSI-3 command sets can be used for these new device types.

In addition, support is also needed for removable media devices and expanded

number of cabling options.  The resulting increase in complexity has made it

desirable to divide the ATA standard into a hierarchical set of standards,

of which the ATA-3 Block Command Set is one.

+-------------+ +-------------+   ***************

| SCSI-3      | | Other       |   *    ATA-3    *

| Multimedia  | | SCSI-3      |   *    Block    * <-- Proposed

| Command Set | | Command Sets|   * Command Set *     Project

+------+------+ +------+------+   ***************

|               |                 |

+-------+-------+                 |

|                         |

+======+======+           +======+======+

| ATA-3 Packet|           |   ATA-3     |

| Interface   | - - - - ->| Transport   |

| Command Set |           | Protocol    |

+======+======+           +======+======+

|                         |

+------------+------------+

|

|

+=====+=====+

|   ATA-3   |

| Physical  |

| Interface |

+===========+

2.2   RECOMMENDED SCOPE OF STANDARD:

The proposed ATA-3 Block Command Set standard shall maintain a high degree

of compatibility with the AT Attachment while providing documentation for new

capabilities including the following candidates:

a)    Provide a means to support traditional hard disk drives and removable

media disk drives.

b)    Other capabilities which fit within the general scope of implementing

the AT Attachment Interface on a broader range of applications.

This proposed standard is not intended to require changes to presently

installed devices or existing software.  It is intended that this proposed

standard would be used to provide additional capabilities.

2.3   EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft AT

Attachment standards to provide additional capabilities.

2.4   EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO CURRENT AND

POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to insure that the AT Attachment has

an upward, highly compatible growth path.  This will insure that current

investments in AT Attachment are provided with more stability in the face of

technological developments.

3  DESCRIPTION OF PROPOSED PROJECT:

3.1   TYPE OF DOCUMENT:  Standard.

3.2   DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

3.3   EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATA-3 Block Command Set standard is for use in closed systems.

3.4   RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATA-3 Block Command Set

standard:

(1)   Solicit continuing participation by the present AT Attachment

participants through X3T10 procedures and new participants through

press releases.  Invite comments by end-user organizations and invite

proposals from organizations that may have a contribution to a viable

ATA-3 Block Command Set standard.

(2)   Establish functional requirements for ATA-3 Block Command Set

functional additions along with downward compatibility requirements.

(3)   Prepare a draft standard based on proposals submitted and other

information gathered during the initial investigation.

(4)   Consider the results of ATA3-Block Command Set testing as may be

available to the committee through the voluntary efforts of the various

participants in X3T10.

(5)   Submit the draft proposed standard to X3 for further processing.

3.5   RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN SUBJECT MATTER:

The current membership of X3T10 includes representatives from all parts of

the computer industry from semiconductor chip manufacturers to large

mainframe system manufacturers as well as Government agencies.  The members

of X3T10 have expressed their desire to participate and cooperate in the

development of this proposed standard.

There are sufficient resources to complete the development of this standard

without delaying work on other standards.

3.6   RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical Committee

X3T10 which is responsible for developing the draft ATA Extensions standard.

3.7   ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets for two days bi-monthly. Specific task ad hoc

groups are called as may be required for one to three days between the

regular meetings but their results are not binding.

3.8   TARGET DATE FOR DPANS TO X3 (MILESTONE 10):  December 1995

3.9   ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 10 years.

4  IMPLEMENTATION IMPACTS

4.1   IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATA-3 Block Command Set standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.2   IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATA-3 Block Command Set standard will provide an upward growth

path complementary to the existing practices and investments.  It is likely

that any isolated negative impacts would occur in any case through non-

standard evolution or revolution.

4.3   TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATA-3 Block Command Set testing

as may be available to the committee through the voluntary efforts of the

various participants in X3T10.  With this method all costs are borne by the

organizations of the various participants and have for the most part been

mainly an adjunct of their normal development costs.

4.4   LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will be made

under Milestone 7 of the SD-2.

5  CLOSELY RELATED STANDARDS ACTIVITIES

5.1   EXISTING STANDARDS:  None.

5.2   X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface -- Project 791-D

ATA Extensions -- Project 948-D

5.3   X3 STUDY GROUPS:  None.

5.4   OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

5.5   ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

It is anticipated that this standard will be proposed to JTC1/SC25/WG4.

5.6   OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:  None.

5.7   RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

5.8   RECOMMENDATIONS FOR CLOSE LIAISON:  None.

^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v^v

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Mon Feb 14 12:15:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA25747; Mon, 14 Feb 94 12:15:26 PST

Received: from nexus.dt.wdc.com ([192.40.1.32]) by dt.wdc.com (4.1/SMI-4.1)

id AA25735; Mon, 14 Feb 94 12:15:18 PST

Received: from st9.wdc.com by nexus.dt.wdc.com (4.1/SMI-4.1)

id AA05015; Mon, 14 Feb 94 12:15:16 PST

Date: Mon, 14 Feb 94 12:15:16 PST

From: worrell@nexus (Devon Worrell (Worrell) x7042)

Message-Id: <9402142015.AA05015@nexus.dt.wdc.com>

To: ata@dt.wdc.com

Subject: ATA-3 Project Proposal

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Fellow ATA-ites,

I believe that the definition of the six (6) documents should

be slightly different. I don't understand the ATA-3 packet Interface

Command Set. There is no specific "Command" set for the Packet

Interface. I would like to see the documents defined as follows:

+---------------+ +---------------+   +===============+

| SCSI-3        | |  Other        |   |     ATA-3     |

| Multimedia    | |  SCSI-3       |   |     Block     |

| Command Set   | |  Command Sets |   |  Command Set  |

+-------+-------+ +-------+-------+   +=======+=======+

|                 |                   |

+-------+---------+                   |

|                             |

+=======+=======+             +=======+=======+

|  ATA-3 Packet |             |     ATA-3     |

|   Transport   | - - - - - ->|   Transport   |

|   Protocol    |             |   Protocol    |

+=======+=======+             +=======+=======+

|                             |

+------------+----------------+

|

|

+======+======+

|   ATA-3     |

|  Physical   |

|  Interface  |

+=============+

Devon,

"Every truth passes through three stages before it is recognized.

In the first it is ridiculed, in the second it is opposed, in the

third it is regarded as self-evident!"

_/_/_/_/    _/_/_/_/    _/      _/ | Devon B. Worrell <worrell@dt.wdc.com>

_/      _/  _/      _/  _/      _/  | Senior Principal Engineer

_/      _/  _/_/_/_/    _/  _/  _/   | Western Digital Corporation

_/      _/  _/       _/ _/_/_/_/_/    | 8105 Irvine Center Drive

_/_/_/_/    _/_/_/_/_/  _/      _/     | Irvine, CA 92718

| Host Interface & Central Firmware

| Technology Staging Group

| Phone: (714) 932-7042

| Fax:   (714) 932-7796

"Not speaking in an official capacity for my employer"

"Void where prohibited by law"

"No refunds--all sales are final"

"Does not include CA Redemption Value"

From ata-owner  Tue Feb 15 13:29:57 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15585; Tue, 15 Feb 94 13:29:57 PST

Received: from ncrhub1.NCR.COM (h192-127-251-3.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA15578; Tue, 15 Feb 94 13:28:53 PST

Received: from ncrcolo by ncrhub1.NCR.COM id ae26360; 15 Feb 94 13:51 EST

Received: by ncrcolo.FtCollinsCO.NCR.com; 15 Feb 94 11:44:58 MST

Subject: X3T10 1994 Hotel Information

To: SCSI Reflector List <scsi@WichitaKS.NCR.COM>,

ATA Reflector <ata@dt.wdc.com>, SSA Reflector <ssa@dt.wdc.com>,

P1394 Reflector <p1394@sun.com>,

Fibre Channel Reflector <fibre-channel-ext@think.com>

Date: Tue, 15 Feb 94 11:44:39 MST

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9402151351.ae26360@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This file contains the meeting hotel information for the 1994

X3T10 meetings.  Please note that the X3T11/X3T12 meeting

information is not included here -- please contact Roger Cummings

(Roger_Cummings@Stortek.com) or Gene Milligan

(Gene_Milligan@notes.seagate.com) for X3T11 or X3T12 information.

Blanks and "?" will be filled in as the information becomes

available from the hosts.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, X3T11, and IEEE P1394)

January 10-15, 1994

HOSTED BY:     Vitro NCR Corporation

LOCATION:      Catamaran Resort Hotel

3999 Mission Blvd.

San Diego, CA 92109

Phone: (619) 488-1081 or (800) 228-0770

Fax:   (619) 488-1619

ROOM RATE:     $99.00 per night (includes tax and parking)

GROUP NAME:    X3T9

CUT-OFF DATE:  December 16, 1993

HOST CONTACT:  Rich Bailly, Vitro Corp. (619) 569-1991

internet: rbailly@nosc.mil

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, X3T11, and IEEE P1394)

MARCH 14-18, 1994

HOSTED BY:     QLogic Corporation

LOCATION:      Hyatt Newporter

1107 Jamboree Road

Newport Beach, California  92660

Phone 800-233-1234 or 714-729-1234

ROOM RATE:     $100.00 per night (includes tax and parking)

$150.00 for a suite

GROUP NAME:    American National Standards Institute

CUT-OFF DATE:  February 1, 1993

DRIVING DIRECTIONS:

1. When leaving Orange County, John Wayne airport, follow

signs to Interstate South 405 (left out of airport onto

MacAurther Drive).

2. Get in right lane and turn right onto 405-South freeway.

3. Exit the 405 freeway at the next exit, Jamboree Blvd.

4. Make a right onto Jamboree Blvd. and continue for about 5

miles (toward ocean).

5. Pass UC Irvine, MacAurther, Highway 73, University Ave.,

Bison, Ford Rd.

6. Just past San Joaquin Rd., about 0.75 miles, look to the

right for the Hyatt Newporter Hotel entrance.

HOST CONTACT:  Skip Jones, QLogic Corporation (714-668-5058)

internet: SK_JONES@QLC.COM

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, X3T11, and IEEE P1394)

MAY 16-20, 1994

HOSTED BY:     AMP, Inc.

LOCATION:      Sheraton Inn Harrisburg

800 E. Park Drive

Harrisburg, PA 17111

Ph:  (717) 561-2800 or (800) 325-3535

Fax: (717) 561-8398

ROOM RATE:     $69.00 + 6% tax

GROUP NAME:    AMP/ANSI

CUT-OFF DATE:  April 26, 1994

DRIVING DIRECTIONS:

From the Harrisburg/Middletown Airport:

Follow I-283 North and it will merge with I-83 North.  Exit on

Union Deposit Road (exit 29).  Go East to East Park Drive and

turn right.  CAUTION: I-283 will branch three ways to

Harrisburg--Lewistown--Hershey.  Stay to the left on I-283 and

follow signs "North to I-83 -- I-81 U.S. Lewistown:.  DO NOT

follow signs to Harrisburg.

HOST CONTACT:  Chuck Brill, AMP Inc. (717-561-6198)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, and IEEE P1394)

JULY 18-22, 1994

HOSTED BY:     Digital Corp.

LOCATION:      Manchester, NH

ROOM RATE:     ?

GROUP NAME:    ?

CUT-OFF DATE:  ?

DRIVING DIRECTIONS:

HOST CONTACT:  Ralph Weber (603-881-1497)

internet: weber@star.enet.dec.com

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, and IEEE P1394)

SEPTEMBER 12-16, 1994

HOSTED BY:     Compaq Computer

LOCATION:      Houston, TX         *** Location has changed ***

ROOM RATE:     ?

GROUP NAME:    ?

CUT-OFF DATE:  ?

DRIVING DIRECTIONS:

HOST CONTACT:  Bill Galloway

internet: billg@bangate.compaq.com

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, and IEEE P1394)

NOVEMBER 7-11, 1994

HOSTED BY:     Western Digital Corp.

LOCATION:      Southern CA

ROOM RATE:     ?

GROUP NAME:    ?

CUT-OFF DATE:  ?

DRIVING DIRECTIONS:

HOST CONTACT:  Jeff Stai (714-932-7644)

internet: stai@dt.wdc.com

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Wed Feb 16 06:42:18 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27078; Wed, 16 Feb 94 06:42:18 PST

Received: from IBMH1.ORL.MMC.COM by dt.wdc.com (4.1/SMI-4.1)

id AA27057; Wed, 16 Feb 94 06:42:06 PST

Received: from ODC by IBMH1.ORL.MMC.COM (IBM VM SMTP V2R2) with BSMTP id 1481;

Wed, 16 Feb 94 09:41:01 EST

Message-Id:  <19940216.094100.OTCE009@ODC>

Date: 16 Feb 94 09:40:59 EST

From: "Belland, Frank"       <OTCE009%ODC.MMDSH@IBMH1.ORL.MMC.COM>

To: John Lohmeyer          <jlohmeye@ncr-mpd.ftcollinsco.ncr.com>

Cc: ATA Reflector          <ata@dt.wdc.com>,

Fibre Channel Reflector <fibre-channel-ext@think.com>,

P1394 Reflector        <p1394@sun.com>,

SCSI Reflector List    <scsi@wichitaks.ncr.com>,

SSA Reflector          <ssa@dt.wdc.com>

Subject: Addr: X3T10 1994 Hotel Information

In-Reply-To: note of Tue, 15 Feb 94 11:44:39 MST from John Lohmeyer

<jlohmeye@ncr-mpd.ftcollinsco.ncr.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

From: Frank Belland - Program Management

please change your addr book for frank belland to a new email

addr of "Frank Belland" <fbelland@dmewkr1.orl.mmc.com>

Thanks Frank

Please respond, if any problems. I can be reached via MARCALL

(407) 826-7299.  Thank you for your time and cooperation in

this matter.                             Frank Belland

From ata-owner  Wed Feb 16 12:30:44 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02202; Wed, 16 Feb 94 12:30:44 PST

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA02193; Wed, 16 Feb 94 12:30:33 PST

Received: from mail.qntm.com by worf.qntm.com with SMTP

(16.6/16.2) id AA23798; Wed, 16 Feb 94 12:28:21 -0800

Received: from wt400 by mail with SMTP

(1.37.109.4/16.2) id AA05218; Wed, 16 Feb 94 12:30:20 -0800

Received: by wt400 (5.65/1.3)

id AA00911; Wed, 16 Feb 94 13:17:01 -0800

Date: Wed, 16 Feb 94 13:17:01 -0800

From: Jim McGrath  </G=Jim/S=McGrath/O=QMAILGW/PRMD=QUANTUM/ADMD=MCI/C=US/@qntm.com>

To: ATA@dt.wdc.com

Subject: Next ATA working group meet

Message-Id: <Macintosh*/PRMD=QUANTUM/ADMD=MCI/C=US/@wt400>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  12:18 PM

OFFICE MEMO         Next ATA working group meeting        Date:  2/16/94

ATA Working Group

DATE:           Wednesday, February 23, 9:30 am to 4 or so

PLACE:         CROWN STERLING SUITES HOTEL

ADDRESS:     901 Calaveras, Milpitas, CA

PHONE:        408-942-0400

AIRPORT:     San Jose International is the closest airport.

DIRECTION:  Crown Sterling Suites is located on Calaveras and Hwy

680.  If you travel from San Jose Airport, you can take

880 north to Calaveras/237 exit.  Turn east at Calaveras

and drive 2 miles before it hits Hwy 680.  Crown

Sterling Suites Hotel is located to your left.

Note: there is a group before us having breakfast in the same room, so we may

start a few minutes late.

Please contact Steve Finch for details of the agenda.

From ata-owner  Fri Feb 18 10:08:40 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10784; Fri, 18 Feb 94 10:08:40 PST

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA10776; Fri, 18 Feb 94 10:08:31 PST

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id KAA14773; Fri, 18 Feb 1994 10:02:42 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 18 Feb 94 09:38:51 PST

Date: 18 Feb 94 09:25:55 PST

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: Agenda for ATA WG, Feb 23, 1994

To: "hall@peyoti.adv.shr.dec.com" <hall@peyoti.adv.shr.dec.com@hp427u.tus.ssi1.com>,

"ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"55527181204991/234850@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The next meeting of the ATA WG, as announced by Jim McGrath, is Wednesday,

February 23, 9:30 to 4:00 (probably ending closer to 3:00 unless we hit a

big snag).

Proposed Agenda:

1.  Opening Remarks, Introductions and Welcomes

2.  Approval of Agenda

3.  Attendance and Memberships

4.  Time and place of next meeting

5.  Old Business

5a.  16.6 MB/s timing

6.  New business

6a.  Issues raised by ATA Editor's Meeting

Please note that these are not the only issues being discussed at the ATA

WG meetings.  These are the issues that we plan to address.  In the past,

other issues have been raised during the meetings and we typically have

discussions about them and decide when to "do real work" on them in the

future.

Anyone out there who has additional items to put on the agenda, please send

them on the reflector and bring related info/presentation material to the

meeting.

The next two ATA WG meetings are planned for:

March 14, 2 P.M. to 8 P.M. at the Hyatt Newporter, Newport Beach, CA.

and will be hosted by Q-Logic (Emulex).

March 30, 9:30 A.M. to 4 P.M. at the Crown Sterling Suites, Milpitas, CA.

and will be hosted by Adaptec.

I appologize for the lateness of the agenda and formal meeting notice.  The

meeting was documented, but details were not given, in the last X3T10

minutes.

=======================================================================

Steve Finch

Silicon Systems

Ph:  714 573-6808

FX:  714 573-6916

email:  5723283@mcimail.com

=======================================================================

From ata-owner  Fri Feb 18 18:43:23 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17661; Fri, 18 Feb 94 18:43:23 PST

Received: from cirrus.com by dt.wdc.com (4.1/SMI-4.1)

id AA17653; Fri, 18 Feb 94 18:43:17 PST

Received: from sunstorm.corp.cirrus.com (sunstorm1) by cirrus.com with SMTP id AA10434

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Fri, 18 Feb 1994 18:43:08 -0800

Received: from sundrive.corp.cirrus.com by sunstorm.corp.cirrus.com with SMTP id AA22651

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Fri, 18 Feb 1994 18:43:07 -0800

Received: from lx101.corp.cirrus.com by sundrive.corp.cirrus.com (4.1-Cirrus/S)

id AA14084; Fri, 18 Feb 94 18:43:06 PST

Date: Fri, 18 Feb 94 18:43:06 PST

From: chen@cirrus.com (Joe Chen)

Message-Id: <9402190243.AA14084@sundrive.corp.cirrus.com>

To: ata@dt.wdc.com

Subject: ATA Working Group Meeting

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To: Steve Finch

There appears to be some confusion as to whether there is an ATA working group meeting on Wednesday, Feb 23, 1994.  The January ATA meeting minute indicated that there is a meeting on 2/23.  However, I have not seen any meeting annoucement.  Can you please clarify this?  Thank you.

Joe Chen

Cirrus Logic, Inc.

From ata-owner  Mon Feb 21 16:42:31 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29858; Mon, 21 Feb 94 16:42:31 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA29851; Mon, 21 Feb 94 16:42:19 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id QAA04710; Mon, 21 Feb 1994 16:18:53 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Mon, 21 Feb 1994 16:10:19 EST

Date:      Mon Feb 21 16:10:11 199 PST

From: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

Message-Id: <2d69233c.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Comments on Proposed Working Draft 948D, dated Jan 1, 1994

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 02-21-94 04:00:17 PM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: John Masiewicz@SEAGATE

_Subject: Comments on Proposed Working Draft 948D, dated Jan 1, 1994

TO:         X3T9 Membership

FROM:       John Masiewicz, Seagate Technology

Date:       February 21, 1994

Subject:    Comments/Recommendations on ATA-2 Working Draft

X3T9.2 Working Draft 948D

Revision 2

Jan 1, 1994

The following document contains Seagate's recommendations for electrical

changes and corrections to the ATA-2 proposed working draft.

**********************************************************************

Page 19 Section 4.2.2

STATES:

"A drive designed for 3.3V applications may be plugged into a receptacle

designed to accept a drive designed for 5V applications.

RECOMMENDATION:

"A drive designed for 3.3V applications can be physically plugged into a

receptacle designed to accept a drive designed for 5V applications.

COMMENT:  As stated implies that we are saying we endorse.

**********************************************************************

Page 21 Section 4.6

The following is in the Draft:

+-------------------------------------------+------+--------+

| Description                               |  Min |   Max  |

+------+-------------------------------------------+------+--------+

|  IoH | Output HIGH current @ 2.4V                |      | -400uA |

SHOULD BE:

+-------------------------------------------+------+--------+

| Description                               |  Min |   Max  |

+------+-------------------------------------------+------+--------+

|  IoH | Output HIGH current @ 2.4V                |-400uA|        |

THE CORRECTED TABLE:

+-------------------------------------------+------+--------+

| Description                               |  Min |   Max  |

+------+-------------------------------------------+------+--------+

|  IoL | Output LOW current, 5V operation, @ 0,5V  | 12mA |        |

|  IoL | Output LOW current, 3,3V operation @ 0,5V |  8mA |        |

|  IoH | Output HIGH current @ 2.4V                |-400uA|        |

|  Vih | Voltage Input High                        | 2,0 V|        |

|  Vil | Voltage Input Low                         |      |  0,8 V |

|  Voh | Voltage Output High (Ioh = -400 u A)      | 2,4 V|        |

|  Vol | Voltage Output Low (5V, Iol = 12ma)       |      |  0,5 V |

|  Vol | Voltage Output Low (3,3V, Iol = 8ma)      |      |  0,5 V |

|  Cin | Input Capacitance (of Device)             |      |  25 pf |

|  Cout| Output Capacitance (of Device)            |      |  25 pf |

|tRISE | Rise time for any signal on AT interface  |  5ns |        |

|      | (1)                                       |      |        |

|tFALL | Rise time for any signal on AT interface  |  5ns |        |

|      | (1)                                       |      |        |

+------+-------------------------------------------+------+--------+

(1): tRISE and tFALL are measured from 0.5V to 2.8V with a resistive load of 1K

Ohm and a capacitive load of 50 pF.

********************************************************************

Page 22  X.X.X  Driver Types and Required Pull-ups

AS STATED:

+==============+========+========+=========+===========+===========+

|              |        | Driver | Pull-up | Pull-up   |           |

| Signal       | Source | Type   | at Host | at each   |  Notes    |

|              |        |   (1)  |   (2)   | Device (2)|           |

+--------------+--------+--------+---------+-----------+-----------+

| PDIAG-       | Device |  TP    |         |           |           |

SHOULD BE:

+==============+========+========+=========+===========+===========+

|              |        | Driver | Pull-up | Pull-up   |           |

| Signal       | Source | Type   | at Host | at each   |  Notes    |

|              |        |   (1)  |   (2)   | Device (2)|           |

+--------------+--------+--------+---------+-----------+-----------+

| PDIAG-       | Device |  TS    |         |   10K     |           |

CORRECTED TABLE:

X.X.X  ATA-2 Driver Types and Required Pull-ups

+==============+========+========+=========+===========+===========+

|              |        | Driver | Pull-up | Pull-up   |           |

| Signal       | Source | Type   | at Host | at each   |  Notes    |

|              |        |   (1)  |   (2)   | Device (2)|           |

+--------------+--------+--------+---------+-----------+-----------+

| Reset        | Host   |  TP    |         |           |           |

| DD 0:15      | Bidir. |  TS    |         |           |           |

| DMARQ        | Device |  TS    | 5.6K PD |           |    (3)    |

| DIOR-  DIOW- | Host   |  TS    |         |           |           |

| IORDY        | Device |  TS    |  1.0K   |           |    (4)    |

| SPSYNC/CSEL  |        |        |         |           |    (5)    |

|    CSEL      | Host   |        | Ground  |   10K     |    (6)    |

|    SPSYNC    | Device |  TS/OC |         |   VU      |    (7)    |

| DMACK-       | Host   |  TP    |         |           |           |

| INTRQ        | Device |  TS    |         |           |           |

| IOCS16-      | Device |  OC    |  1.0K   |           |           |

| DA0:2        | Host   |  TP    |         |           |           |

| PDIAG-       | Device |  TS    |         |   10K     |           |

| CS0- CS1-    | Host   |  TP    |         |           |           |

| DASP-        | Device |  OC    |         |   10K     |    (5)    |

+==============+========+========+=========+===========+===========+

Table 3 - ATA-2 Driver Types and Required Pull-ups

(1) TS=Tri-State; OC=Open Collector; TP=Totem-Pole: PU=Pull-up; PD=Pull-Down

(2) All resistor values are minimum (lowest) allowed.

(3) ATA-2 defines this line to be tri-stated whenever the device is not

selected or is not executing a DMA data transfer.  When enabled by DMA

transfer, it will be driven high and low by the device.

(4) This signal should only be enabled during DIOR/DIOW cycles to the selected

device.

(5) See signal descriptions for information on dual use of this signal.

(6) When used as CSEL, Line is grounded at Host and 10K Pull-up is required at

both devices.

(7) When Used as SPSYNC, application is vendor unique.

*********************************************************************

*  ALTERNATIVE TO PAGES 21-22                                       *

*  I Propose the following alternative definition for Electrical    *

*  Requirements be adopted as previously discussed.                 *

*********************************************************************

X.X.X  ATA-1 Driver Types and Allowed Pull-ups

The table below represents the generally accepted driver/receiver and

terminations used for ATA-1.  Most of these were not specified by the ATA-1

specification, but have been adopted de-facto by the industry.  New I/O

transfer modes added by ATA-2 may not work with the following ATA-1 drivers and

loading.  See tables 3 and 4 for ATA-2 driver types and terminations.

+==============+========+========+=========+===========+===========+

|              |        | Driver | Pull-up | Pull-up   |           |

| Signal       | Source | Type   | at Host | at Each   |  Notes    |

|              |        |  (1)   |   (2)   | Device (2)|           |

+--------------+--------+--------+---------+-----------+-----------+

| Reset        | Host   |  TP    |         |           |           |

| DD 0:15      | Bidir. |  TS    | 8.2-10K |           | Note 3    |

| DMARQ        | Device |  TP    | 5.6K PD |           | Note 3,4  |

| DIOR-  DIOW- | Host   |  TS    |4.7-8.2K |           | Note 3    |

| IORDY        | Device |  OC/TS |   1.0K  |           | Note 3,5  |

| SPSYNC/CSEL  |        |        |         |           | Note 6    |

|    CSEL      | Host   |        |  Ground |   10K     | Note 4,7  |

|    SPSYNC    | Device |  TS/OC |         |   VU      | Note 8    |

| DMACK-       | Host   |  TP    |         |           |           |

| INTRQ        | Device |  TS    |2.2K-8.2K|           | Note 3    |

| IOCS16-      | Device |  OC    |  300    |           | Note 3    |

| DA0:2        | Host   |  TP    |         |           |           |

| PDIAG-       | Device |  TS    |         |   10K     |           |

| CS0- CS1-    | Host   |  TP    |         |           |           |

| DASP-        | Device |  OC    |         |   10K     | Note 6    |

+==============+========+========+=========+===========+===========+

Table 1 - ATA-1 Driver Type and Allowed Pull-ups

(1) TS=Tri-State; OC=Open Collector; TP=Totem-Pole: PU=Pull-up; PD=Pull-Down

(2) All resistor values are minimum (lowest) allowed.

(3): Pull-up resistors normally associated with ISA/EISA busses may be seen by

ATA devices in unbuffered implementations, These pull-ups reside only on the

Host, not on the devices. The use of these pull-ups were not defined by ATA-1

and this summary is provided for informational purposes.

(4) Some host systems use 5.6K pull-down on this line. ATA-1 specified a

pull-down but did not specify value or location. EISA specified 5.6K pull-down

at Host. ATA-1 permitted use of totem-pole drivers.  If the system cannot

tolerate totem-pole driver due to non-ATA signal-sharing applications,

appropriate buffering or series protection resistor (up to 47 ohm) should be

employed at the host or ATA adapter.

(5)  If TS is used, should be "sink only". If actively asserted, this signal

should only be enabled during DIOR/DIOW cycles to the selected device.

(6) See signal descriptions for information on dual use of this signal.

(7) When used as CSEL, Line is grounded at Host and 10K Pull-up is required at

both devices.

(8) When Used as SPSYNC, application is vendor unique.

X.X.X   ATA-1 Electrical Characteristics

The following table represents the generally accepted driver and receiver

specifications used for ATA-1.  The source and sink specifications in ATA-1

were widely ignored by the industry, and the threshold voltages listed below

are by general agreement. New I/O transfer modes added by ATA-2 may not work

with the following ATA-1 drivers and loading.  See Tables 3 and 4 for ATA-2

compatible electrical characteristics and loading.

+-------------------------------------------+------+--------+

| Description                               |  Min |   Max  |

+------+-------------------------------------------+------+--------+

|  IoL | Output LOW current, 5V operation, @ 0.5V  | 12mA |        |

|      |   (Table 1 loading)(Note 1)               |      |        |

|  IoL | Output LOW current, 3.3V operation @ 0.5V |  8mA |        |

|      |   (Table 1 Loading)(Note 1 )              |      |        |

|  IoH | Output HIGH current @ 2.4V                |-400uA|        |

|      |   (Table 1 Loading)                       |      |        |

|  Vih | Voltage Input High                        | 2.0 V|        |

|  Vil | Voltage Input Low                         |      |  0.8 V |

|  Voh | Voltage Output High (Ioh = -400 u A)      | 2.4 V|        |

|  Vol | Voltage Output Low (5V, Iol = 12ma)       |      |  0.5 V |

|      | Cable Capacitive Loading                  |      |  200pF |

+------+-------------------------------------------+------+--------+

Table 2 - ATA-1 Electrical Characteristics

Note 1: ISA/EISA systems use pull-up resistors on IOCS16 as low as 300 Ohms.

In these applications, IoL of 12ma may not be sufficient to guarantee VoL.

X.X.X  ATA-2 Driver Types and Required Pull-ups

+==============+========+========+=========+===========+===========+

|              |        | Driver | Pull-up | Pull-up   |           |

| Signal       | Source | Type   | at Host | at Each   |  Notes    |

|              |        |  (1)   |   (2)   | Device (2)|           |

+--------------+--------+--------+---------+-----------+-----------+

|              |        |        |         |           |           |

| Reset        | Host   |  TP    |         |           |           |

| DD 0:15      | Bidir. |  TS    |         |           |           |

| DMARQ        | Device |  TS    | 5.6K PD |           | Note 3    |

| DIOR-  DIOW- | Host   |  TS    |         |           |           |

| IORDY        | Device |  TS    |  1.0K   |           | Note 4    |

| SPSYNC/CSEL  |        |        |         |           | Note 5    |

|    CSEL      | Host   |        | Ground  |   10K     | Note 6    |

|    SPSYNC    | Device |  TS/OC |         |   VU      | Note 7    |

| DMACK-       | Host   |  TP    |         |           |           |

| INTRQ        | Device |  TS    |         |           |           |

| IOCS16-      | Device |  OC    |  1.0K   |           |           |

| DA0:2        | Host   |  TP    |         |           |           |

| PDIAG-       | Device |  TS    |         |   10K     |           |

| CS0- CS1-    | Host   |  TP    |         |           |           |

| DASP-        | Device |  OC    |         |   10K     | Note 5    |

|              |        |        |         |           |           |

+==============+========+========+=========+===========+===========+

Table 3 - ATA-2 Driver Types and Required Pull-ups

(1) TS=Tri-State; OC=Open Collector; TP=Totem-Pole: PU=Pull-up; PD=Pull-Down

(2) All resistor values are minimum (lowest) allowed.

(3) ATA-2 defines this line to be tri-stated whenever the device is not

selected or is not executing a DMA data transfer.  When enabled by DMA

transfer, it will be driven high and low by the device.

(4) This signal should only be enabled during DIOR/DIOW cycles to the selected

device.

(5) See signal descriptions for information on dual use of this signal.

(6) When used as CSEL, Line is grounded at Host and 10K Pull-up is required at

both devices.

(7) When Used as SPSYNC, application is vendor unique.

X.X.X   ATA-2 Electrical Characteristics

The following table should be used for ATA-2 applications which use PIO Mode 3

or greater and DMA Mode 1 or greater where table 3 loading is used.

+-------------------------------------------+------+--------+

| Description                               |  Min |   Max  |

+------+-------------------------------------------+------+--------+

|  IoL | Output LOW current, 5V operation, @ 0.5 V |  4mA |        |

|      |   with Thevinin Load Rt > 700 Ohms        |      |        |

|      |   at equiv Voltage of 3.0 V               |      |        |

|      |   Load Capacitance CL < 50pf              |      |        |

|  IoH | Output HIGH current @ 2.4V                | -4ma |        |

|      |   with Thevinin Load Rt > 700 Ohms        |      |        |

|      |   at equiv Voltage of 3.0 V               |      |        |

|      |   Load Cap CL < 50pf                      |      |        |

|  Vih | Voltage Input High                        | 2.0 V|        |

|  Vil | Voltage Input Low                         |      |  0.8 V |

|  Voh | Voltage Output High                       | 2.4 V|        |

|  Vol | Voltage Output Low                        |      |  0.5 V |

|  Vol | Voltage Output Low                        |      |  0.5 V |

|  Cin | Input Capacitance (of Device)             |      |  25 pf |

|  Cout| Output Capacitance (of Device)            |      |  25 pf |

|tRISE | Rise time for any signal on AT interface  |  5ns |        |

|      |   (1)                                     |      |        |

|tFALL | Rise time for any signal on AT interface  |  5ns |        |

|      |   (1)                                     |      |        |

+------+-------------------------------------------+------+--------+

Table 4 - ATA-2 Electrical Characteristics

(1) tRISE and tFALL are measured from 0.5V to 2.8V with a resistive load of 1K

Ohm and a capacitive load of 50 pF.

******************************************************************

******************************************************************

Page 21 Section 4.6  AC Electrical Requirements - See my Tables 1-3

Several issues here ...

> These are really DC Characteristics, not AC Requirements, but most

specifications just refer to them as "Electrical Characteristics"

*********************************************************************

Page 26 Section 5.2.6 DIOR- (Drive I/O Read)

STATES:

"The rising edge of DIOR- latches the data at the host."

COMMENT:  Incorrect?  Not always true on DMA and some PIO implementations.

RECOMMENDATION:  Delete Statement

*********************************************************************

Page 26 Section 5.2.7 DIOW- (Drive I/O write)

STATES:

"The rising edge of DIOW- clocks data from the device data signals, ...."

COMMENT:  Incorrect?   Not always true on DMA and some PIO implementations.

RECOMMENDATION:  Delete Statement

*******************************************************************

Page 26 Section 5.2.8 DMACK- (DMA Acknowledge)

COMMENT:  Add statement allow host deassertion of DMACK.  As agreed in WG,

this is allowed.

RECOMMENDATION add statement:

"This signal may be deasserted by the Host at any time to suspend the DMA

transfer in process.  The Device may deassert DMARQ once DMACK- is deasserted

and reassert it again or the Device may leave DMARQ asserted and wait for the

Host to reassert DMACK-, at which time the transfer resumes."

*********************************************************************

Page 27 Section 5.2.9 DMARQ (DMA Request)

COMMENT:  The "NOTE" does not adequately explain the problem and is confusingin

what is expected and who is expected to to it.

RECOMMENDATION:  Go back to note as proposed:

"In ATA-1 devices, this signal was either totem-pole or tri-state in

different implementations.  The use of a 5.6K pull-down in EISA was to cause a

logic low on undriven lines.  ATA-2 defines this line to be in high-impedance

mode except when DMA transfer is active from the selected drive. In systems

which may use mixed devices where totem-pole drivers are used, and the system

shares this line with other non-ATA devices, the ATA host or ATA adapter shall

ensure that appropriate protection is employed to protect ATA device DMARQ

drivers from damage. If series resistors are used, they shall reside on the

host side of the cable and not exceed 47 Ohms."

REASON:

There are several problems with DMARQ. First of all, there was never a

requirement in ATA-1 that DMARQ be placed in high-impedance state at any time,

i.e. it was only defined as "TTL compatible", Subsequently, there are many

applications which handle the signaldifferently (Totem-pole, Tri-state,

source-only).

If the Host shares DMARQ with other non-ATA devices in an unbuffered

situation, and a totem-pole driver is used on any device, a bus fight

may occur, possibly causing indeterminate results or I/O buffer damage.  For

example, if two drives (master/slave) are connected and DMARQ is enabled by

'selected', the DMARQ line will always be driven by someone. If the Host elects

to share this DMARQ line with other (non-ATA) devices, a bus fight will occur.

The EISA specification (sec 2.11.2) requires that if totem-pole drivers are

used, a series protection resistor should be placed on all DRQ outputs.

Recent Working group discussions have come to some agreement that this signal

should only be driven when the drive is selected and it is currently in a DMA

transfer operation. It is also generally agreed that the line needs to be

driven high and low to maximize system performance, and eliminate the

"sometimes resistor pull-down" problem.

RECOMMENDATION:

Add Note to Say:

"This line will be released (tri-stated) whenever the device is not

selected or is not executing a DMA transfer.  When enabled by DMA

transfer, it will be driven high and low by the device."

********************************************************************

Page 27 Section 5.2.10 INTRQ

COMMENT:

There is some question about the type of driver used here. It seems that most

devices use tri-state drivers so that they can actively negate the signal.

There is typically a pull-up on this line, but there is no ATA requirement for

it. The ATA spec says that this signal must be placed in the 'high impedance

state' if nIEN=1 or if the drive is not selected. This is not a problem if the

ISA/EISA system does not share this line with other non-ATA devices.

> RECOMMENDATION:  Add a note to say:

If the system shares this line with non-ATA devices, the ATA host or ATA

adapter shall ensure that appropriate protection is employed to protect ATA

device INTRQ drivers from damage. If series resistors are used, they shall

reside on the host side of the cable and not exceed 47 Ohms.

***********************************************************************

Page 27 Section 5.2.12  IORDY

STATES:

"When IORDY is not negated, IORDY shall be in the high impedance state."

COMMENT:

This line is frequently tri-state and sometimes open collector. Many devices

are using actively negated buffers for timing reasons.  This is especially true

at higher transfer modes where timing is very tight. This is OK if the IORDY

driver is only enabled when the drive is selected AND the IORD/IOWR is to the

ATA device.

RECOMMENDATION: Add statement:

"If actively negated, this signal shall only be enabled during DIOR-/DIOW-

cycles to the selected drive. If open collector, when IORDY is not negated, it

shall be in the high-impedance (undriven) state."

**********************************************************************

John Masiewicz  (408) 439-2152  (Masiewicz@notes.seagate.com)

--

John Masiewicz (John_Masiewicz@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Tue Feb 22 15:26:57 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17503; Tue, 22 Feb 94 15:26:57 PST

Received: from bolero.rahul.net by dt.wdc.com (4.1/SMI-4.1)

id AA17487; Tue, 22 Feb 94 15:26:15 PST

Received: by bolero.rahul.net id AA14778

(5.67a8/IDA-1.5 for ATA Reflector <ata@dt.wdc.com>); Tue, 22 Feb 1994 15:25:29 -0800

Date: Tue, 22 Feb 1994 15:22:55 -0800 (PST)

From: Richard Kalish <rkalish@rahul.net>

Subject: CRC on the DATA for high speed operation

To: ATA Reflector <ata@dt.wdc.com>, AL Pham <alp@adaptec.com>

Message-Id: <Pine.3.07.9402221555.A14596-d100000@bolero>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hopfully this time I will be able to get my message out to the reflector

in a form that others can finally read.

FROM:   R. Kalish     408-957-7169	ADAPTEC, INC.

A. Pham       408-957-4804	ADAPTEC, INC.

DATE:   February 16, 1994

Doc. #:  X3T10/94-027r2

This information was presented at the December ATA Local Bus

working group meeting. Although there was only a very limited

time spent discussing it, the main objection voiced was that this

was un-necessary and could be achieved currently using a read-long

command. Hopefully more time can be spent during the next

meeting, and we can discuss why this can not be done with the

read-long command and why this scheme will increase data transfer

integrity. We do not believe that the existing ATA protocol will

be violated or required to change because even though the status

is read before the data is transferred on a host read, when the

host determines that the data it received is corrupted, the host

should just execute another read command. The drive may very well

have sent good data, but the data was corrupted during the

transfer.

Proposal for Data CRC across the Cable

ATA PIO Mode 4 and Multi-word DMA Mode 2 need a reliable method to

verify the data transferred across the cable was received properly

and was not corrupted. This feature has long been a part of the

SCSI interface with the parity bit enabling the verification of

each byte of data. Since there is no room on the cable for parity

on each data transfer, Al and I would like to propose a method for

data integrity verification across the cable. This would mean

adding 4 new commands for Read/Write-Multiple W/CRC and Read/Write

DMA W/CRC and only be for the high speed modes (20 Mbytes/sec

transfer rates - or whatever speed PIO Mode 4 becomes). Since the

current Read/Write-Multiple and DMA commands are optional, these

new commands would also be optional.

We are proposing the following new commands:

1.  READ Multiple W/CRC - Command Code CCh

The new command for Read Multiple W/CRC will transfer 516 bytes of

data per sector from the drive to the host.

2.  WRITE Multiple W/CRC - Command Code CDh

The new command for Write Multiple W/CRC will transfer 516 bytes

of data per sector from the host to the drive. A bad transfer

will result in an error flag in both status registers for the

status of the operation.

3.  READ Multiple DMA W/CRC - Command Code CEh

The new command for Read Multiple DMA W/CRC will transfer 516

bytes of data per sector from the drive to the host. This command

will abort if the drive is not set up for Multiple Mode DMA.

4.  WRITE Multiple DMA W/CRC - Command Code CFh

The new command for Write Multiple DMA W/CRC will transfer 516

bytes of data per sector from the host to the drive. A bad

transfer will result in an error flag in both status registers for

the status of the write operation. This command will abort if the

drive is not set up for Multiple Mode DMA.

5.  CRC Polynomial

The 4 byte CRC polynomial is the same as used for P1394 (and many

other specifications) which should be universally acceptable (We

will not discuss the merits of this number since there are only a

very few people who fully understand the pros and cons of this

polynomial versus another, and we are not in that group). The CRC

has the following properties:

--  Generator Polynomial:

G(x) = x^32 + x^26 + x^23 + x^22 + x^16 + x^12 + x^11 + x^10

+ x^8 + x^7 + x^5 + x^4 + x^2 + x + 1

--   CRC Register is preset to all 1's before generation and

checking.

--   Generated CRC Redundancy Polynomial:

R$(x) = one's complement of the remainder polynomial of

degree less than 32, i.e. the remainder is inverted before

being shifted out.

--  Check Polynomial produced by the receiver upon receiving an

error-free sequence ( data and CRC redundancy ):

C(x) = x^31 + x^30 + x^26 + x^25 + x^24 + x^18 + x^15 + x^14

+ x^12 + x^11 + x^10 + x^8 + x^6 + x^5 + x^4 + x^3 + x

+ 1

6.  General Requirements

The drive must be previously set in the multiple mode or all of

the above commands will abort. These commands will be used on a

sector by sector block of data. The process of generating and

checking the CRC would most likely be implemented with logic in a

bridge adapter appending the CRC on a disk write and check the CRC

on a disk read. If there is a CRC error when reading data from

the disk, the host bridge logic will be required to notify the

host of the error. We can propose the exact method for this, or

leave the implementation up to the bridge designer since this will

require BIOS modification or Driver support anyway. Our earlier

proposal of letting the host microprocessor generate and check the

CRC would probably cause a major time and resource burden to the

system.

7.  Drive Requirements

The drive will need two bits in the Identify Drive Parameter

Information to specify that the drive can support the Read/Write

Multiple W/CRC and the Read/Write Multiple DMA W/CRC commands. A

likely place is in Word 49 bits 13 and 14 since this word is

specifying drive capabilities, but there is no hard reason for not

putting this information elsewhere. We are assuming that if a

drive will support a Read W/CRC that it must support the Write

W/CRC also, but the drive does not have to support both the PIO

and DMA modes if it supports one.

From ata-owner  Tue Feb 22 15:43:57 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17840; Tue, 22 Feb 94 15:43:57 PST

Received: from bolero.rahul.net by dt.wdc.com (4.1/SMI-4.1)

id AA17832; Tue, 22 Feb 94 15:43:53 PST

Received: by bolero.rahul.net id AA15969

(5.67a8/IDA-1.5 for ATA Reflector <ata@dt.wdc.com>); Tue, 22 Feb 1994 15:43:51 -0800

Date: Tue, 22 Feb 1994 15:25:32 -0800 (PST)

From: Richard Kalish <rkalish@rahul.net>

Subject: Host reset Minimum for drive to respond

To: ATA Reflector <ata@dt.wdc.com>

Message-Id: <Pine.3.07.9402221532.B14596-b100000@bolero>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Sender: ata-owner@dt.wdc.com

Precedence: bulk

From:	R. Kalish, Adaptec

TO:	ATA Committee and interested parties

An issue has come up that I feel needs clarification in the ATA-2

document.  The standard states that the host must hold RESET for a minimum

of 25 microseconds.  This was great when the standard was first created

and was probably necessary for the microprocessor on the disk drive to

have plenty of time to see the RESET, but this needs clarification for new

controllers on the drives.  Should a drive respond to a RESET if the

signal is only 10 microseconds long?? How about a 10 nanosecond signal??

Many controllers on drives today will respond to very short length signals

and don't need the 25 microseconds that the standard lists.  Host system

makers say that a drive should not respond to a 10 nanosecond "glitch",

but in todays fast systems, 10 nS is not always considered a "glitch".

I propose the following:

A drive must not see a strobe of "X" or less as a RESET from the host.

The exact time of "X" should be good for about a 2-3 hour discussion at

the  meeting on 23 Feb. 1994 if this subject is as controversial as some of

the previous topics we have discussed in the last few months.  While I

understand how the standard shows this signal in such an unusual way, I

feel that we now have a chance to correct a potential problem area.

Rick Kalish

Adaptec

rkalish@rahul.net

From ata-owner  Tue Feb 22 15:47:52 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17894; Tue, 22 Feb 94 15:47:52 PST

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA17887; Tue, 22 Feb 94 15:47:47 PST

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA20071 for ata@dt.wdc.com; Tue, 22 Feb 94 15:47:46 -0800

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA04260 for ata@dt.wdc.com; Tue, 22 Feb 94 15:47:45 -0800

Received: from bach.nsc.com by berlioz.nsc.com (4.1/SMI-4.1)

id AA01154; Tue, 22 Feb 94 15:47:44 PST

Date: Tue, 22 Feb 94 15:47:44 PST

From: rss@berlioz.nsc.com (Ravinder S. Shergill)

Message-Id: <9402222347.AA01154@berlioz.nsc.com>

To: ata@dt.wdc.com, sff_reflector@sun.com

Subject: ATA Extensions SSWG Minutes - Jan 94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Doc No: X3T10/94-047r0

Minutes of the ATA-Extensions Special Working Group Meeting

Held January 28, 1994, in Milpitas, CA, at Crown Sterling Suites Hotel.

Chaired by:          Steve Finch, SSI.

Hosted by:           Joe Chen, Cirrus Logic.

Minutes recorded by: Robbie Shergill, National Semiconductor.

Following documents were distributed in this meeting:

- 94-027r1: Proposal for Data CRC across the Cable (R. Kalish, Adaptec)

- ________: 16MB/s Timing Numbers (Steve Anderson, Conner)

- ________: Minutes of the December SSWG meeting (R. Shergill, National)

- X3T9.2/0948D rev 2: ATA-2 Proposed Working Draft (Steve Finch, SSI)

1. Steve Finch opened the meeting at 9:30AM by thanking the host and stating

the purpose of the meeting and some pertinent ANSI mechanisms. He asked each

attendee to introduce themselves and also sign the attendees list.

2. Agenda for this meeting was circulated by Steve Finch. It was modified

and accepted.

1. Introductions

2. Approval of Agenda

3. Attendance and Membership (Group Introductions)

4. Time and Place for Next Meeting

5. Spec Documents

5.1  ATA-2 Status

5.2  ATA-2 Editors' Meeting

5.3  ATA Reflector

5.4  ATA-3 Project Proposals

6. High Speed Transfers

6.1  Data CRC

6.2  16MB/s Timings

7. Adjournment.

3. Attendance List

NAME                 COMPANY               PHONE

Al Pham              Adaptec              408-945-2560

Joe Chen             Cirrus Logic         510-226-2101

Ken Bush             Compaq

Steve Anderson       Conner Peripherals

Dan Colegrove        IBM

Larry Lamers         Maxtor               408-432-3889

Robbie Shergill      National Semi.       408-721-7959

Jim McGrath          Quantum              408-894-4504

Farbod Falakfarsa    Quantum

John Brooks          Quantum

Clyde Nagakura       Quantum

Jim Wilshire         Quantum              408-894-5245

Mike Carpenter       Q-Logic

Peter van Beckum     Samsung

Hank Davenport       Samsung

John Masiewicz       Seagate              408-439-2152

Gene Milligan        Seagate

Steve Finch          Silicon Systems      714-573-6808

Tom Hanan            Western Digital      714-832-7472

4. Time and Place for the Next Meeting:

Feb 23: Milpitas. (hosted by Quantum; at Crown Sterling Suites hotel)

Mar 14: Newport Beach (as part of X3T9.10)

Mar 30: TBD  (possibly Western Digital, Irvine)

It was conveyed by Steve Finch that, in response to this group's desires,

John Lohmeyer is suggesting that ATA working group be held on Thursday

mornings of the Plenary week and SFF on Friday AM. This was discussed at

last plenary and will be decided at the next plenary in March.

5. Spec Documents.

5.1  Status:

Steve Finch circulated rev 2 of ATA-2 draft. The change bars are relative to

rev 1, not ATA 4.0a. If there is quick agreement on 16MB/s then we will

include this mode in ATA-2 also; but we will not delay the spec for this

reason.

Tom Hanan said Western Digital is compiling a table of "shalls" and

"shoulds" from the spec. He asked if the group would like for it to be

included in the spec. Steve Finch opposed this idea on the basis that this

will deemphasize the "is's" and some of these are quite necessary to

implement. John Masiewicz suggested that we should just highlight the

"shalls" and "shoulds" in the actual spec - all agreed on this. The group

agreed that this issue can be revisited once we have seen Tom's list.

5.2  Editors' Meeting:

Steve will be setting up the next editors' meeting by next Monday and put it

on reflector. He intends to hold it in Southern California. The editors will

go through the document in detail.

5.3  Reflector:

Tom Hanan reported that sequence numbers are being attached to each item

now. In addition, Western Digital is in the process of putting in the

ability for people to see a register of postings with sequence numbers. Tom

is working with Larry Lamers to add the doc numbers to this same list, and

at a later time, the ability to obtain doc numbers from it.

Tom asked if people want a separate political reflector. There was very

little desire for such a reflector. The sense of the group was that

political discussions should simply be limited by way of self-discipline.

5.4 ATA-3 PROJECT PROPOSALS:

Steve Finch described the four proposals he is working on.

Tom Hanan asked how one can become chair for any of these. Gene Milligan

explained that there will be only one chairperson; and then  an editor per

document. Steve Finch further explained that there will be a primary editor

for each document and a secondary editor for coordinating between the four

editors/documents. Steve is prepared to serve the function of secondary

editor.

6. High Speed Transfers.

6.1 DATA CRC:

Al Pham (Adaptec) presented the proposal. In answer to a question, he agreed

that a possibility would be to do Set Features instead of new commands. Tom

Hanan said that he doesn't like Set Features method.

John Masiewicz said that the first question is - do we need it at all?

Seagate doesn't want any additional commands. All ATA commands end up

becoming mandatory. Additionally, if this is really necessary, then the OS

can append CRC to files.

Steve Finch objected that this is not a complete solution because the non-

data transfers are not protected. Tom Hanan pointed out that WD recommends

that non-data transfers should be done at Mode 0 - this proposal should be

covered.

Joe Chen asked what was adaptec's reason for bringing this proposal. Al Pham

answered that some of Adaptec's customers are asking for it. Joe Chen and

John Masiewicz asserted that that depends on who the customer is - major

OEMs have to buy into it. Ken Bush (Compaq) said that we don't need a new

feature. We do need integrity, but the primary place to do it is in the

definition of the high speed timing. Compaq doesn't see the need for to

evangelize this issue.

Steve Finch suggested that Adaptec ask for a data integrity SSWG so the

whole issue can be discussed.Tom Hanan offered to lead such an effort. John

Masiewicz and Robbie Shergill agreed with this approach, but asserted that

ATA-2 work shouldn't be impacted because of this. Gene Milligan further

added that we should not lose sight of the fact that we want to keep ATA

cheap and simple. Steve said that that SSWG will have to work outside of

here. If that SSWG feels that 16MB/s needs CRC then they can bring it to us.

In conclusion, it was agreed that the issue of data integrity should start

as a reflector discussion to be followed with a possible SSWG; and that we

don't want to impact the current ATA-2 work.

6.2 16MB/s Timings:

Steve Anderson (Conner) opened this discussion by presenting his proposal.

He asked if we are trying to define this mode under SFF or ATA?

Steve Finch clarified that the intention is to do it in ATA and then ask SFF

to do an informational document. John Masiewicz would prefer doing it as

part of ATA-2 document. Tom Hanan will continue to edit the current proposal

document; eventhough the info has come from many sources. Document number is

94-045.

A long discussion ensued on the 16mb/s timing numbers. Although certain

timing values have to be carefully chosen because of their impact on various

disk controller designs, the key reason for the complexity of this issue is

that bridge controllers have to work at various system clock frequencies.

One thing that became clear was that in the current spec t0 does not cover

the other edge-reference possibility and this should be clarified. That is,

the cycle time has to be observed from deassertion-edge to deassertion-edge

also. Tom Hanan pointed out that there is a real possibility that a bridge

adapter can violate this spec when system frequencies are switched.

The other thing that became clear was that access time in terms of (tk + tE)

needs to be specified. For the purpose of further discussion this time will

be referred to as (tu).

Each individual spec was also discussed. Keeping in mind that (td + tk) is

important, we can reduce tD to 70, but then tK has to be 25.

Three options were offered and a vote was taken:

a       b       c

td     70      75      70

te     50      55      50

tk     25      20      20

vote:  10       3       0

So the group went with option A. The group was much more agreeable to the

rest of the timings.

Steve Anderson also suggested that a note be added to both PIO and DMA

tables that clarifies that the cycle time a drive supports is returned in ID

data. This was agreed to.

Steve Finch asked if 16MB/s can be done with the existing cable plant.

John Masiewicz answered yes - with the qualification that the cable may have

to be shorter than 18" and slew rate has to be controlled. Tom Hanan further

suggested that the bridge should tie IOCS16 low and that series resistors

should be used (on the host) on the strobe signals. Each of these items have

been covered in this group before and there has been general agreement on

these.

John Masiewicz brought up the issue of mixing of slow and fast drives on a

cable. Tom Hanan agreed to write an implementor's note on this subject.

7. The meeting was adjourned at 3:00 PM.

From ata-owner  Wed Feb 23 10:46:51 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02795; Wed, 23 Feb 94 10:46:51 PST

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA02788; Wed, 23 Feb 94 10:46:43 PST

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id KAA06919; Wed, 23 Feb 1994 10:38:29 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 23 Feb 94 10:43:40 PST

Date: 22 Feb 94 16:37:08 PST

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: X3T10 ATA-2 Editor's Meeting

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>,

"scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>

Message-Id: <"80730032204991/235724@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

There will be an ATA Editor's meeting on Sunday, March 13, at the Hyatt

Newporter, in Newport Beach.  This meeting will be part of the General

Editor's session scheduled for that day.  Anyone interested in partipating

in the reviewing the ATA-2 Rev. 2 working draft is welcome to attend.

For general information on the location of the hotel, etc., please see the

meeting notice for the X3T10 Working Group and Plenary meetings previously

published or contact me directly.

See you there!

========================================================================

Steve Finch

Silicon Systems, Inc.

PH:  714 573-6808

FX:  714 573-6916

email:  5723283@mcimail.com

========================================================================

From ata-owner  Fri Feb 25 12:19:14 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA12356; Fri, 25 Feb 94 12:19:14 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA12340; Fri, 25 Feb 94 12:17:46 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA07093; Fri, 25 Feb 1994 12:03:26 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 25 Feb 1994 11:20:25 EST

Date:      Fri Feb 25 11:20:22 199 PST

From: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

Message-Id: <2d6e254a.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Word 51,52, 64, 66 Revisions for comments.

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 02-25-94 10:56:59 AM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: John Masiewicz@SEAGATE

_Subject: Word 51,52, 64, 66 Revisions for comments.

The following are comments are on X3T9.2, document 0948D Revision 2 dated Jan

1, 1994.

I believe all of the comments are editorial, and do not present new

information, but clarify agreed-upon intent.  Comments are welcome.

**************************************************************************

Page 50 Section 8.10.14 STATES:

8.10.14 Word 51: PIO data transfer cycle timing mode

The PIO transfer timing for each ATA device falls into categories which have

unique parametric timing specifications.  To determine the proper device timing

category, compare the Cycle Time specified in figure 6 with the contents of

this field.  The value returned in Bits 15-8 should fall into one of the

categories specified in figure 6, and if it does not, then Mode 0 shall be used

to serve as the default timing.

COMMENT:  The figure referred to (figure 6) is incorrect.  It should refer to

figure 5 (PIO data transfer).  Also, figure 5 now includes mode 3.  The

agreement for word 51 was that it would only be used for the original PIO

modes, i.e. modes 0,1, and 2.  Higher modes would not be allowed in word 51 for

ATA-1 compatibility.  This is referred to in a note in word 64 (page 51). The

note should also be clarified to refer to PIO modes 0-2 instead of "original

PIO modes". I believe this is an editiorial issue and I am not recommending

anything new other than clarification.

RECOMMENDATION:  Correct paragraph 8.10.14 and add corrected note below:

8.10.14 Word 51: PIO data transfer cycle timing mode

The PIO transfer timing for each ATA device falls into categories which have

unique parametric timing specifications.  To determine the proper device timing

category, compare the Cycle Time specified in figure 5 with the contents of

this field.  The value returned in Bits 15-8 should fall into one of the mode 0

through mode 2 categories specified in figure 5, and if it does not, then Mode

0 shall be used to serve as the default timing.

Note:  For backwards compatibility with BIOSs written before Word 64 was

defined for advanced modes, a device reports in Word 51 the highest original

PIO mode (i.e. PIO mode 0, 1, or 2) it can support.

****************************************************************************

Page 50 Section 8.10.15 States:

8.10.15 Word 52: DMA data transfer cycle timing mode

The DMA transfer timing for each ATA device falls into categories which have

unique parametric timing specifications.  To determine the proper device timing

category, compare the Cycle Time specified in figure 6 with the contents of

this field.  The value returned in Bits 15-8 should fall into one of the

categories specified in figure 6, and if it does not, then Mode 0 shall be used

to serve as the default timing.

The contents of this word shall be ignored if Words 62 or 63 are supported.

COMMENT:  Figure 6 is Single Word DMA, and the Description is not very specific

that this word refers only to SINGLE-WORD DMA transfer timing mode.

RECOMMENDATION: Clarify wording to state Single word DMA, and add clarification

parenthesis to paragraph as shown below as shown below:

8.10.15 Word 52: Single Word DMA data transfer cycle timing mode

The DMA transfer timing for each ATA device falls into categories which have

unique parametric timing specifications.  To determine the proper device timing

category, compare the Cycle Time specified in figure 6 with the contents of

this field.  The value returned in Bits 15-8 should fall into one of the

categories specified in figure 6 (i.e. 0,1, or 2), and if it does not, then

Mode 0 shall be used to serve as the default timing.

The contents of this word shall be ignored if Words 62 or 63 are supported.

***********************************************************************

Page 51 Section 8.10.25 STATES:

8.10.25 Word 64: Flow Control PIO Transfer Modes Supported

Bits 7 through 0 of word 64 of the Identify Drive parameter information is

defined as the Advanced PIO Data Transfer Supported Field.  This field is bit

significant.  Any number of bits may be set in this field by the device to

indicate which Advanced PIO Modes that it is capable of supporting.

Of these bits, bits 7 through 1 are Reserved for future Advanced PIO Modes.

Bit 0, if set, indicates that the device supports PIO Mode 3.

Note:  For backwards compatibility with BIOSs written before Word 64 was

defined for advanced modes, a device reports in Word 51 the highest original

PIO mode it can support.

COMMENT: Reference in Note to "highest original PIO mode" is unclear.

RECOMMENDATION:  Revise note to say:

Note:  For backwards compatibility with BIOSs written before Word 64 was

defined for advanced modes, a device reports in Word 51 the highest original

PIO mode (i.e. PIO mode 0, 1, or 2) it can support.

************************************************************************

Page 52 Section 8.10.27 STATES:

8.10.27 Word 66: Manufacturer's Recommended Multiword DMA Cycle Time

Word 66 of the parameter information of the IDENTIFY DRIVE command is defined

as the Manufacturer's Recommended Multiword DMA Transfer Cycle Time.  This

field defines, in nanoseconds, the minimum cycle time per word during a single

sector host transfer while performing a multiple sector READ DMA or WRITE DMA

commands over all locations on the media under nominal conditions.

If a host attempts to run at a faster cycle rate by operating at a cycle time

of less than this value, the device may be forced to negate DMARQ to prevent

data corruption.  The rate at which DMARQ is negated could result in reduced

throughput despite the faster cycled rate.

If this field is supported, bit 1 of word 53 must be set.

Any device which supports Multiword DMA Mode 1 or above shall support this

field, and the value in word 66 shall not be less than the value in word 65.

If bit 1 of word 53 is set because a device supports a field in Words 64-70

other than this field and the device does not support this field, the device

shall return a value of zero in this field.

COMMENT:

The second paragraph is confusing.  Marketing types object to any reference of

data corruption and say it suggests that anything faster than Word 66 cycle

time WILL result in corruption.  Some other people say that Word 66 is the rate

at which DMARQ will NEVER be deasserted.  They compare it to running PIO

without IORDY support.  Neither of these arguments is correct, but the

paragraph could be interpreted in many ways.

RECOMMENDATION:  Rewrite paragraph to say:

If a host runs at a faster cycle rate by operating at a cycle time of less than

this value, the device may negate DMARQ for flow control.  The rate at which

DMARQ is negated could result in reduced throughput despite the faster cycled

rate.  Transfer at this rate does not ensure that flow control will not be

used, but implies that higher performance MAY result.

--

John Masiewicz (John_Masiewicz@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Fri Feb 25 14:34:23 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA13947; Fri, 25 Feb 94 14:34:23 PST

Received: from bolero.rahul.net by dt.wdc.com (4.1/SMI-4.1)

id AA13940; Fri, 25 Feb 94 14:34:18 PST

Received: by bolero.rahul.net id AA19060

(5.67a8/IDA-1.5 for ATA Reflector <ata@dt.wdc.com>); Fri, 25 Feb 1994 14:34:09 -0800

Date: Fri, 25 Feb 1994 14:32:11 -0800 (PST)

From: Richard Kalish <rkalish@rahul.net>

Subject: Reset on the ATA cable

To: ATA Reflector <ata@dt.wdc.com>

Message-Id: <Pine.3.07.9402251411.A18745-c100000@bolero>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Doc#  X3T10/94-053r1

TO:       ATA Reflector

From:     R. Kalish - Adaptec, Inc.

RE:       RESET on the Bus

From time to  time an issue has come up that I think we

finally need to address in the ATA-2 Document.  The problem

of how short a RESET signal on the cable should or should

not be seen as a RESET by an ATA device needs to be covered

in the standard.  The standard currently states that the

host shall hold the RESET for 25 micro-seconds, but there is

no specification for how the drive should respond to a

signal shorter than this.  Should a drive see a 10 micro-

second signal on the RESET signal as a RESET? How about a

pulse of only 10 nano-seconds??

I propose that we add a statement like the following:

All drives must not respond to a signal shorter than

"X"  on the RESET- signal ( pin 1 ).

The exact value of "X" should be jointly agreed upon by the

committee but I think it should be somewhere in the 25

nsecond to 100 nsecond range.

--------------------------------------------------------

The above document is the official version I presented at

the ATA Local bus meeting on 23 February.  The wording is

slightly different from the last one I put on the reflector

because the reflector version was generated on-the-fly in

the mail editor. The outcome of the discussion at the meting

was basically to keep this as a reflector discussion until

the 30 March meeting.

As a discussion point, I would like concerned and interested

parties to please provide the following:

1)   The value for "X", where "X" is the minimum time that

your controller on the drive will NOT respond to a low

going strobe on the RESET- ( pin 1 ) line.

2)   The value for "Y", where "Y" is the minimum time where

your controller WILL respond to a low going strobe on  the

RESET- ( pin 1 ) line ( This value will naturally be

greater than "X" and will generally be much lower than

the ATA specification of 25 micro-seconds ).

By stating these two values, there is an implied gray area

where a controller may or may not see a low going strobe on

pin 1 as a RESET.

I understand the concerns by some that by adding this

specification to the standard, older drives may now not meet

the ATA standard.  I am not sure if this is really a

industry problem, or a marketing problem.  I believe that by

specifying this time, we can all benefit and help make

future ATA devices better.

Naturally, all comments and concerns are appreciated

regarding these issues. Please direct them to me directly,

and/or to the ATA Reflector.

Richard Kalish

Adaptec, Inc

Phone     408-957-7169

FAX:      408-957-7165

E-Mail:   rkalish@rahul.net

From ata-owner  Sat Feb 26 23:17:20 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27401; Sat, 26 Feb 94 23:17:20 PST

Received: from ncrhub1.NCR.COM (h192-127-251-3.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA27394; Sat, 26 Feb 94 23:17:07 PST

Received: from ncrcolo by ncrhub1.NCR.COM id aa01722; 27 Feb 94 1:05 EST

Received: by ncrcolo.FtCollinsCO.NCR.com; 25 Feb 94 16:55:22 MST

Received: from tailspin.noname by airport.noname (4.1/SMI-4.1)

id AA15349; Fri, 25 Feb 94 16:58:18 MST

Date: Fri, 25 Feb 94 16:58:18 MST

From: Dave Isenberg <davei@airport>

Mmdf-Warning:  Unable to confirm address in preceding line at ncrhub1.NCR.COM

Message-Id: <9402252358.AA15349@airport.noname>

Received: by tailspin.noname (4.1/SMI-4.1)

id AA05016; Fri, 25 Feb 94 16:58:19 MST

To: ata@dt.wdc.com

Subject: Comments on Proposed Working Draft 948D, dated Jan 1, 1994

Reply-To: David.Isenberg@FtCollinsCO.NCR.COM

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To   : ATA Reflector

From : Dave Isenberg - AT+T Global Information Solutions

RE   : Comments on Proposed Working Draft 948D, dated Jan 1, 1994

In a recent post, a recommendation was made to add the following:

>*******************************************************************

>Page 26 Section 5.2.8 DMACK- (DMA Acknowledge)

>

>RECOMMENDATION add statement:

>"This signal may be deasserted by the Host at any time to suspend the DMA

>transfer in process.  The Device may deassert DMARQ once DMACK- is deasserted

>and reassert it again or the Device may leave DMARQ asserted and wait for the

>Host to reassert DMACK-, at which time the transfer resumes."

In reviewing the existing spec. however, the device may only deassert DMARQ

when DMACK- is asserted.  Should the DRQ remain active after the deassertion edge

of DMACK- for even a short time, and then be deasserted it could cause the host

interface to start another DMA cycle, just as the DRQ is being deasserted, as

follows:

__________________

DMARQ       ___/                  \________

_________       _______

DMACK-      _______/         \_____/

DIOW-               _______         _______

DIOR-       _______/       \_______/

Generally speaking there is no way of knowing how host interfaces will handle

this situation.  Since this mode of operation appears to violate tL

(DIOR- to DMREQ delay) for Multi-word DMA, and tC (DMACK to DMREQ delay) for

Single word DMA, Hopefully drive interfaces won't do this.

NCR (now known as: AT+T Global Information Solutions) would recommend either

that this statement be removed from the additions made to the 2.0 spec, since the

timing diagrams which already exist explain the needed behaviors of DRQ and DACK-,

or that it be re-written to something like the following:

"This signal may be deasserted by the Host at any time to suspend the DMA

transfer in process.  For Multi-Word DMA transfers, the Device may deassert

DMARQ within the tL specified time (refer to Figure 7 pg. 71) once DMACK- is

asserted and reassert it again at a later time to resume the DMA operation.

Alternatively, if the device is able to continue the transfer of data, the

Device may leave DMARQ asserted and wait for the Host to reassert DMACK-."

The reference to tL, Figure 7 of page 71 will draw attention to the dashed

line on the DMARQ in the diagram which is what this recommendation seems

intended to explain.

-----------------------------------------------------------------------

Dave Isenberg

AT+T Global Information Solutions (Formerly NCR)

Ph. (303) 226-9577     Fax. (303) 226-9654

E-Mail David.Isenberg@FtCollinsCO.NCR.COM

-----------------------------------------------------------------------

From ata-owner  Wed Mar  2 11:12:27 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02348; Wed, 2 Mar 94 11:12:27 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA02341; Wed, 2 Mar 94 11:12:20 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id LAA05607; Wed, 2 Mar 1994 11:03:26 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Wed, 02 Mar 1994 10:52:02 EST

Date:      Wed Mar 02 10:52:01 199 PST

From: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

Message-Id: <2d74b623.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Endorse NCR (Isenberg) wording change to 94-051r0 with minor mod.

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-02-94 10:38:08 AM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: John Masiewicz@SEAGATE

_Subject: Endorse NCR (Isenberg) wording change to 94-051r0 with minor mod.

My original proposal dated Jan 21 to the reflector (assigned doc X3T10/94-051r0

at last WG) generated a comment from David Isenberg at NCR.

My original proposal read as follows:

*******************************************************************

Page 26 Section 5.2.8 DMACK- (DMA Acknowledge)

RECOMMENDATION add statement:

"This signal may be deasserted by the Host at any time to suspend the DMA

transfer in process.  The Device may deassert DMARQ once DMACK- is deasserted

and reassert it again or the Device may leave DMARQ asserted and wait for the

Host to reassert DMACK-, at which time the transfer resumes."

*******************************************************************

This, unfortunately, was poorly worded and did not convey the intent.  See

David's posting for further discussion.

David proposed the following wording in a response to the reflector:

"This signal may be deasserted by the Host at any time to suspend the DMA

transfer in process.  For Multi-Word DMA transfers, the Device may deassert

DMARQ within the tL specified time (refer to Figure 7 pg. 71) once DMACK- is

asserted and reassert it again at a later time to resume the DMA operation.

Alternatively, if the device is able to continue the transfer of data, the

Device may leave DMARQ asserted and wait for the Host to reassert DMACK-."

This wording is much improved over the original proposal.  I did not like

the words "at any time", however.  I spoke with David, and he agreed that

those words could be removed.  With David's agreement, I endorse his

proposed modified wording to the note to read:

"This signal may be deasserted by the Host to suspend the DMA

transfer in process.  For Multi-Word DMA transfers, the Device may deassert

DMARQ within the tL specified time (refer to Figure 7 pg. 71) once DMACK- is

asserted and reassert it again at a later time to resume the DMA operation.

Alternatively, if the device is able to continue the transfer of data, the

Device may leave DMARQ asserted and wait for the Host to reassert DMACK-."

Unless I receive negative comment, I will modify my 94-051r0 document (along

with other changes discussed at the meeting) with the above note, and re-post

as revision r1.

I will leave the reference to page and figure numbers to the Editors of the

final document.

--

John Masiewicz (John_Masiewicz@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Thu Mar  3 10:47:40 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22327; Thu, 3 Mar 94 10:47:40 PST

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA22320; Thu, 3 Mar 94 10:47:29 PST

Received: from ncrcolo by ncrhub1.NCR.COM id ae07136; 3 Mar 94 13:33 EST

Received: by ncrcolo.FtCollinsCO.NCR.com; 3 Mar 94 11:27:05 MST

Subject: X3T10 Meeting List and Agenda

To: SCSI Reflector List <scsi@WichitaKS.NCR.COM>,

ATA Reflector <ata@dt.wdc.com>, SSA Reflector <ssa@dt.wdc.com>

Date: Thu, 3 Mar 94 11:25:58 MST

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

Reply-To: John Lohmeyer <John.Lohmeyer@FtCollinsCO.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9403031333.ae07136@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Attached are the March '94 X3T10 / X3T10.1 / P1394 / SFF meeting

map and Meeting Announcement (unchanged from the mailing)

plus the revised X3T10 agenda (added items 10.5 -- 10.7).

Please let me know if there are any other topics that need

to be added to the agenda.

Also, please note that there are several items of Old Business,

New Business, ISO Business that will require 2/3rds votes.

Please plan to be available for these votes if you are a

voting member of X3T10!

John Lohmeyer

------------------------------------------------------------------------------

Meeting Map -- March 14-18, 1994 -- Newport Beach, CA

(1/13/94 Version -- Does not include X3T11 meetings)

Sunday 3/13        9   10  11  12  1   2   3   4   5

+=============+

|Gen Edit Sess|              (10)

+=============+

Monday 3/14        9   10  11  12  1   2   3   4   5   6   7   8

+-------------------+=======================+

| Small Form Factor | ATA Working Group     |  (40)

+-------------------+=======================+

| SCSI CAM Working Group                    |  (25)

+===========================================+

Tuesday 3/15       9   10  11  12  1   2   3   4   5   6   7   8

+---------------+===============+

| SSA-UIG       | X3T10.1 (SSA) |              (30)

+===============+===============+

| SBP LB Res    | P1394 Plenary |              (30)

+===============+---------------+

| RAID SG / RAB                 |              (40)

+===============================+

| P1394 Connector Task Group    |              (15)

+-------------------------------+

Wednesday 3/16     9   10  11  12  1   2   3   4   5   6   7   8

+===========================================+

| SCSI-3 WG                                 |  (55)

+===============================+===========+

| P1394 Plenary Meeting         |              (30)

+-------------------------------+

Thursday 3/17      9   10  11  12  1   2   3   4   5   6   7   8

+============+  +===========================+

| SSP WG     |  | X3T10 Plenary             |  (30/80)

+============+--+===========================+

Friday 3/18    8   9   10  11  12  1   2

+=====================+

| X3T10 Plenary       |                          (80)

+=====================+

Meetings marked with == are X3T10-authorized meetings.

Meetings marked with -- are authorized by other groups and may change

without notice.

Abbreviation Key:

ATA:              AT Attachment (a.k.a. IDE)

CAM:              Common Access Method

Gen Edit Sess:    General Editing Session

P1394:            IEEE Project on High Performance Serial Bus

Plenary:          Full Committee Meeting

RAB:              RAID Advisory Board

RAID:             Redundant Array of Independent Devices

SG:               Study Group

SSA:              Serial Storage Interface

SSP:              Serial Storage Protocol (Protocol for SSA)

UIG:              User Interest Group

WG:               Working Group (same as Ad Hoc Group)

X3T10:            Lower Level Interfaces Technical Committee (was X3T9.2)

X3T10.1:          SSA Task Group of X3T10 (was X3T9.7)

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, X3T11, and IEEE P1394)

MARCH 14-18, 1994

HOSTED BY:     QLogic Corporation

LOCATION:      Hyatt Newporter

1107 Jamboree Road

Newport Beach, California  92660

Phone 800-233-1234 or 714-729-1234

ROOM RATE:     $100.00 per night (includes tax and parking)

$150.00 for a suite

GROUP NAME:    American National Standards Institute

CUT-OFF DATE:  February 1, 1993

DRIVING DIRECTIONS:

1. When leaving Orange County, John Wayne airport, follow

signs to Interstate South 405 (left out of airport onto

MacAurther Drive).

2. Get in right lane and turn right onto 405-South freeway.

3. Exit the 405 freeway at the next exit, Jamboree Blvd.

4. Make a right onto Jamboree Blvd. and continue for about 5

miles (toward ocean).

5. Pass UC Irvine, MacAurther, Highway 73, University Ave.,

Bison, Ford Rd.

6. Just past San Joaquin Rd., about 0.75 miles, look to the

right for the Hyatt Newporter Hotel entrance.

HOST CONTACT:  Skip Jones, QLogic Corporation (714-668-5058)

internet: SK_JONES@QLC.COM

------------------------------------------------------------------------------

X3T10/94-002r1

March 3, 1994

Draft Agenda -- X3T10 Meeting #2 March 17-18, 1994 -- Newport Beach, CA

1.    Opening Remarks

2.    Approval of Agenda

3.    Attendance and Membership

4.    Approval of Minutes - January Meeting San Diego, CO (X3T10/94-016r1)

5.    Document Distribution

6.    Call for Patents

7.    Review of Old Action Items

8.    Subgroup and/or Project Status Reports

8.1   Project Status Summary [Lohmeyer]

8.2   X3T10.1 Task Group Report [Scheible]

8.3   SCSI-2 Status [Lamers]

8.4   SCSI Common Access Method (CAM) [Dallas]

8.5   AT Attachment Status (ATA) [Lamers]

8.6   ATA Extensions (ATA-2) [Finch]

8.7   SCSI-3 Architecture Model (SAM) Status [Monia]

8.8   SCSI-3 Parallel Interface (SPI) Status [Lamers]

8.9   SCSI-3 Interlocked Protocol (SIP) Status [Lamers]

8.10  SCSI-3 Generic Packetized Protocol (GPP) Status [Stephens]

8.11  SCSI-3 Serial Bus Protocol (SBP) Status [Marazas]

8.12  SCSI-3 Fiber Channel Protocol (FCP) Status [Snively]

8.13  SCSI-3 Primary Commands (SPC) Status [Weber]

8.14  SCSI-3 Block Commands (SBC) [open]

8.15  SCSI-3 Stream Commands (SSC) [Lappin]

8.16  SCSI-3 Graphics Commands (SGC) [open]

8.17  SCSI-3 Medium Changer Commands (SMC) [open]

8.18  Directly-Addressable Device Interface (DADI) []

8.19  SCSI Common Access Method -2 (CAM-2) (93-082) [Dallas]

8.20  SCSI-3 Serial Storage Protocol (SSP) [Scheible]

8.21  SCSI-3 Controller Commands (SCC) (93-130) [Hagerman]

8.22  Multimedia Command Set (MMC) [Harris]

8.23  Replacement of ANSI X3.80-1988, Interfaces Between Flexible Disk

Cartridges and Their Host Controllers, with ISO 9315:1989.

8.24  RAID Study Group [Penokie]

9.    Old Business

9.1   Technical Information Bulletins

9.2   Revised GPP Project Proposal [Milligan]

10.   New Business

10.1  Results of SPI Public Review

10.2  Results of Flexible Disk Public Review

10.3  Results of FCP Letter Ballot

10.4  Review of new documents

|     10.5  Withdrawl or Reaffirmation of ANSI X3.170-1989, Enhanced Small

|           Device Interface (ESDI)

|     10.6  Forwarding motions on ATA-3 Project Proposals (ATA3-PH, ATA3-XPT,

|           ATA3-PI, and ATA3-BC)

|     10.7  SPI-2 Project Proposal and Ultra SCSI Status Report [Ham]

11.   ISO [Milligan]

12.   Liaison Reports

12.1  X3T11 Liaison Report [Cummings]

12.2  Small Form Factor Industry Group [Allan]

12.3  IEEE P1394 [Marazas]

12.4  IT8 [Lohmeyer]

12.5  IEEE P1285 Status [Pak]

12.6  PCMCIA Report

12.7  AIIM C21 [Podio]

13.   Review of Action Items

14.   Meeting Schedule

14.1  Authorization of Working Group Meetings

15.   Adjournment

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Tue Mar  8 21:31:52 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10087; Tue, 8 Mar 94 21:31:52 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA10080; Tue, 8 Mar 94 21:31:39 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id VAA21209; Tue, 8 Mar 1994 21:22:06 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Tue, 08 Mar 1994 21:10:23 EST

Date:      Tue Mar 08 21:10:17 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d7d300f.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   SFF-8019 rev 0.4

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-08-94 08:13:41 PM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: SFF-8019 rev 0.4

Here is SFF-8019 rev 0.4.  Sorry it has taken so long to get this out.

====================

NOTE:  Changes since the original versions, rev 0.1 and 0.2, are

marked by "|" at the left margin.

** Submitted Proposal **                       SFF-8019 Rev 0.4

SFF Committee documentation may be purchased (see p4).

SFF Committee documents are available by FaxAccess at 408-741-1600

Small Form Factor Committee

Specification for

Identify Drive Data for ATA Disks Over 528 MB

Rev 0.4   March 4, 1994

Secretariat:  Small Form Factor Committee

Abstract:  This document defines the presentation of the Identify

Drive data for ATA drives over 528 MB.

This document provides a common specification for systems

manufacturers, system integrators, and suppliers of magnetic disk

drives.  This is an internal working document of the Small Form

Factor Committee, an industry ad hoc group.

This document is made available for public review, and written

comments are solicited from readers.  Comments received by the

members will be considered for inclusion in future revisions of

this document.

Support:  This document is supported by the identified member

companies of the Small Form Factor Committee.

POINTS OF CONTACT:

Hale Landis                           I. Dal Allan

Seagate Technology                    Chairman SFF Committee

920 Disc Dr B-3                       ENDL

Scotts Valley                         14426 Black Walnut Court

CA 95066                              Saratoga  CA 95070

Ph: 408-439-2443                      Ph: 408-867-6630

Fx: 408-438-4190                      Fx: 408-867-2115

hale_landis@notes.seagate.com         250-1752@mcimail.com

Page 1

** Submitted Proposal **                       SFF-8019 Rev 0.4

EXPRESSION OF SUPPORT BY MANUFACTURERS

The following member companies of the Small Form Factor Committee

voted in favor of this industry specification.

<TBD>

The following member companies of the Small Form Factor Committee

voted against this industry specification.

<TBD>

The following member companies of the Small Form Factor Committee

voted to forward this document to an accredited standards body.

<TBD>

To save space for SFF Specifications being reviewed, the

information on the principles of the SFF Committee and how to

join has not been printed.

Identify Drive Data for ATA Disks Over 528 MB

Page 2

** Submitted Proposal **                       SFF-8019 Rev 0.4

TABLE OF CONTENTS

1.          Scope                                          3

1.1         Description of Clauses                         3

2.          References                                     3

2.1         Industry Documents                             3

2.2         SFF Specifications                             3

2.3         Sources                                        4

3.          General Description                            5

4.          Definitions and Conventions                    5

4.1         Definitions                                    5

4.2         Conventions                                    6

5.          Identify Drive Data Requirements               6

5.1         Background Information                         6

5.2         Cylinder, Head and Sector Addressing           7

5.2.1       Word 1                                         7

5.2.2       Word 3                                         7

5.2.3       Word 6                                         7

5.2.4       Use of words 53 through 58                     7

5.2.5       Word 53                                        7

5.2.6       Word 54                                        7

5.2.7       Word 55                                        7

5.2.8       Word 56                                        8

5.2.9       Words 57-58                                    8

5.3         Logical Block Addressing                       8

5.3.1       Words 60-61                                    8

5.3.2       Orphan Sectors                                 8

FIGURES

TABLES

Identify Drive Data for ATA Disks Over 528 MB

Page 3

** Submitted Proposal **                       SFF-8019 Rev 0.4

Small Form Factor Committee --

Identify Drive Data for ATA Disks Over 528 MB

1. Scope

This document defines the manner in which the Identify Drive data

supplied by an ATA drive will be presented by drives of over 528

MB in capacity.

The purpose of this SFF Specification is to define a common

| manner in which all manufacturers of drives of over 528 MB can

present Identify Drive data, so that system implementors, BIOS

(Basic Input/Output System) writers, device driver programmers,

and other integrators of ATA drives can use the contents in a

consistent manner.

In an effort to broaden the applications for disk drives, an ad

hoc industry group of companies representing system integrators,

peripheral suppliers, and component suppliers decided to address

the issues involved.

The Small Form Factor Committee was formed in August, 1990 and

the first working document was introduced in January, 1991.

1.1  Description of Clauses

Clause 1 contains the Scope and Purpose.

Clause 2 contains Referenced and Related Standards and SFF

Specifications.

Clause 3 contains the General Description.

Clause 4 contains the Glossary.

Clause 5 defines the Identify Drive Data Requirements.

2. References

The Small Form Factor Committee activities support the

requirement of the disk drive industry, and it is involved with

several standards.

2.1  Industry Documents

The following interface standards are relevant to disk drives.

- X3.131R-199x     SCSI-2 Small Computer System Interface

- X3.221-199x      ATA (AT Attachment) Interface

- X3T9.2/0855D     SPI (SCSI-3 Parallel Interface)

- PCMCIA 2.01      Personal Computer Card Physical Specification

- PCMCIA           PC Card-ATA Specification

2.2  SFF Specifications

There are several projects active within the Small Form Factor

Committee.  At the date of printing document numbers had been

assigned to the following projects.  The status of Specifications

is dependent on committee activities.

Identify Drive Data for ATA Disks Over 528 MB

Page 4

** Submitted Proposal **                       SFF-8019 Rev 0.4

F = Forwarded The document has been approved by a majority of the

members for forwarding to a formal standards body.

P = Published A Specification recognized by members as complete.

It is in the the format of an SFF Specification and is no

longer a development project.

A = Approved A draft Specification voted by members to be

technically complete.  It may not be in the final format of an

SFF Specification.

D = Development A project is under way, and a draft Specification

exists or one is in process.

E = Expired The document had been published as an SFF

Specification, and a majority of the members agreed not to

re-publish it when it came up for annual review.

s = submitted The document is a proposal to the members for

consideration to become an SFF Specification.

Spec #   Rev  List of Titles as of November 5, 1993

--------  ---  ------------------------------------------------

SFF-8001  1.0  44-pin ATA (AT Attachment) Pinouts for SFF Drive

SFF-8002  2.0  68-pin ATA (AT Attachment) for SFF Drives

SFF-8003  1.1  SCSI Pinouts for SFF Drives

SFF-8004  1.0  Small Form Factor 2.5" Drives

SFF-8005  2.3  Small Form Factor 1.8" Drives

SFF-8006  2.0  Small Form Factor 1.3" Drives

SFF-8007  0.3  SFF Connector Alternatives

SFF-8008  2.2  68-pin Embedded Interface for SFF Drives

SFF-8009  2.1  Unitized Connector for Cabled Drives

SFF-8010  1.0  Small Form Factor 15mm 1.8" Drives

SFF-8011  1.1  ATA Timing Extensions for Local Bus

SFF-8012  1.0  Power Connector Pin Dimensions

SFF-8013  0.3  ATA Download Microcode Command

SFF-8014       Unitized Connector for Rack Mounted Drives

SFF-8015       Single Connector for Rack Mounted Drives

SFF-8017  0.3  A-Cable SCSI on a 68-pin Connector

SFF-8018  0.3  ATA Low Power Modes

SFF-8019  0.4  Identification of ATA Drives >528 MB

2.3  Sources

Copies of ANSI standards or proposed ANSI standards may be

purchased from Global Engineering.

15 Inverness Way East    800-854-7179 or 303-792-2181

Englewood                303-792-2192Fx

CO 80112-5704

Copies of PCMCIA specifications may be purchased from PCMCIA.

1030G E Duane Ave        408-720-0107

Sunnyvale                408-720-9416Fx

CA 94086

Identify Drive Data for ATA Disks Over 528 MB

Page 5

** Submitted Proposal **                       SFF-8019 Rev 0.4

Copies of SFF Specifications are available by FaxAccess or by

joining the SFF Committee as an Observer or Member.

14426 Black Walnut Ct    408-867-6630x303

Saratoga                 408-867-2115Fx

CA 95070                 FaxAccess: 408-741-1600

FaxAccess is a computer-operated service capable of faxing copies

of documents selected from a menu.  Anyone ordering documents

over FaxAccess must be using the handset of a fax machine, as the

documents are transmitted over the same line as the caller dialed

in on to make the selection(s).

3.  General Description

The application environment for this SFF Specification is any

computer which implements a device using the ATA interface.

The purpose of the SFF Specification is to define the contents of

the Identify Drive data for use disk drives of grater than 528 MB

in capacity.

4. Definitions and Conventions

4.1  Definitions

For the purpose of SFF Specifications, the following definitions

apply:

4.1.1 ATA (AT Attachment):  This term defines the signal and

logical protocol described in X3.221 for IDE (Integrated Drive

Electronics) peripherals.

4.1.2 DMA (Direct Memory Access):  A means of data transfer

between peripheral and host memory without processor

intervention.

4.1.3 IDE (Integrated Drive Electronics):  IDE describes a device

with built in ATA protocol electronics.

4.1.4 Optional:  This term describes features which are not

required by the SFF Specification.  However, if any feature

defined by the SFF Specification is implemented, it shall be done

in the same way as defined by the Specification.  Describing a

feature as optional in the text is done to assist the reader.  If

there is a conflict between text and tables on a feature

described as optional, the table shall be accepted as being

correct.

4.1.5 PCMCIA:  This is the acronym for the Personal Computer

Memory Card International Association Person, a trade association

responsible for the promotion of removable device interfaces for

a variety of products including memory, modems, disks, etc.

4.1.6 PCMCIA 2.01:  This is the Specification published for

removable devices based upon a 68-pin connector and associated

form factors.

4.1.7 PC Card-ATA:  This term describes an application

specification for the implementation of ATA-like devices

compatible with host systems implementing PCMCIA Type III slots.

4.1.8 Reserved:  Where this term is used for bits, bytes, fields

and code values; the bits, bytes, fields and code values are set

aside for future standardization.  The default value shall be

zero.  The originator is required

Identify Drive Data for ATA Disks Over 528 MB

Page 6

** Submitted Proposal **                       SFF-8019 Rev 0.4

to define a Reserved field or bit as zero, but the receiver

should not check Reserved fields or bits for zero.

4.1.9 VU (Vendor Unique):  This term is used to describe bits,

bytes, fields, pins, signals, code values and features which are

not described in this SFF Specification, and may be used in a way

that varies between vendors.

4.1.10 VU Mode:  A mode of execution by the drive in which its

use is not defined by this SFF Specification.  The means by which

a vendor invokes vendor unique operations within a drive is

defined by this SFF Specification.

4.2  Conventions

Certain terms used herein are the proper names of signals.  These

are printed in uppercase to avoid possible confusion with other

uses of the same words; e.g., ATTENTION.  Any lower-case uses of

these words have the normal American- English meaning.

A number of conditions, commands, sequence parameters, events,

English text, states or similar terms are printed with the first

letter of each word in uppercase and the rest lower-case; e.g.,

In, Out, Request Status.  Any lower- case uses of these words

have the normal American-English meaning.

The American convention of numbering is used i.e., the thousands

and higher multiples are separated by a comma and a period is

used as the decimal point.  This is equivalent to the ISO

convention of a space and comma.

American:         0.6                      ISO:         0,6

1,000                                  1 000

1,323,462.9                            1 323 462,9

5. Identify Drive Data Requirements

5.1 Background Information

The original IBM PC BIOS (Basic Input/Output System) imposed

several restrictions on the support of disk drives, and these

have been incorporated into many higher level software products.

One such restriction limits the capacity of a hard disk drive.

Most BIOS software cannot support a disk drive with more than

1,024 cylinders, 16 heads and 63 sectors per track.  The maximum

addressable capacity of an ATA disk drive under this scheme is

1,032,192 sectors or 528,482,304 bytes or 528 MB.

There is growing support of auto-configuration for disk drives on

PC systems.  The auto-configuration capability usually resides in

the BIOS and uses the Identify Drive command data to configure an

ATA disk drive.

| This specification defines rules for ATA disk drives over 528 MB

| and allows BIOS support of ATA drives up to 16,515,072 sectors or

| 8,455,716,864 bytes or 8 GB using Cylinder/Head/Sector (CHS)

| addressing.

| It is unlikely that BIOS architecture based on CHS addressing can

| be extented beyond 8 GB.

This specification defines information that newer BIOSs and

system software can use to determine the true size of a disk

drive and access the full capacity of the drive.

Identify Drive Data for ATA Disks Over 528 MB

Page 7

** Submitted Proposal **                       SFF-8019 Rev 0.4

5.2 Cylinder, Head and Sector Addressing

BIOSs and other software that operate an ATA disk drive in CHS

(Cylinder, Head and Sector) addressing mode use Identify Drive

data words 1, 3, 6 and words 53-58 to ascertain the appropriate

translation mode to use and determine the capacity of an ATA disk

drive.

| Maximum compatibility is achieved if the following rules are

| obeyed.  These rules limit the values placed into words 1, 3, 6,

| and 53-58.  The rules specified here for CHS addressing apply

| to drives up to 8 GB in size.

5.2.1 Word 1

| For drives 528 MB or less, Identify Data word 1 (Default Cylinders)

| shall not specify a value greater than 1,024.  If a drive is over

| 528 MB this word may contain a value greater than 1,024 but not

| greater than 16,383. The contents of this word shall not change.

5.2.2 Word 3

Identify Data word 3 (Default Heads) shall not specify a value

greater than 16.  If a drive is over 528 MB this word shall

contain a value of 16.  The contents of this word shall not

change.

5.2.3 Word 6

| Identify Data word 6 (Default Sectors) shall not specify a value

| greater than 63.  The content of this word shall not change.

5.2.4 Use of words 53 through 58

ATA drives that are over 528 MB shall implement words 53-58.

Drives not over 528 MB may also implement these words.  These

words define the addressing for all sectors accessible in CHS

mode.

5.2.5 Word 53

| Identify Data word 53 bit 0 shall be 1 at all times that the drive

| is in a valid translation mode.  Some drives may have translation

| modes that cannot be supported.  An attempt to put a drive into

| one of these unsupported modes shall cause word 53 bit 0 to be 0

| with words 54-58 cleared to zero until a valid translation mode

| is established.

5.2.6 Word 54

| Identify Data word 54 (Current Cylinders) shall specify the

| number of full logical cylinders that can be accessed in the

| current translation mode.  This value may be greater than 1,024

| but not greater than 16,383.  This word is the integer result of

| dividing the total number of user sectors (this value may be in

| words 60-61) by the number of sectors per logical cylinder (

| [word55] x [word56] ). If this result is greater than 16,383,

| word 54 shall contain a value of 16,383.  If an Initialize Drive

| Parameters command has not been executed, the contents of this

| word shall be the same as word 1.

5.2.7 Word 55

Identify Data word 55 (Current Heads) is the number of heads

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 16.  If an Initialize

Drive Parameters command has not been

Identify Drive Data for ATA Disks Over 528 MB

Page 8

** Submitted Proposal **                       SFF-8019 Rev 0.4

executed, the contents of this word shall be the same as word 3.

5.2.8 Word 56

Identify Data word 56 (Current Sectors) is the number of sectors

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 255.  If an Initialize

Drive Parameters command has not been executed, the contents of

this word shall be the same as word 6.

5.2.9 Words 57-58

| Identify Data words 57-58 contain a 32-bit value that shall be

| equal to [word54] x [word55] x [word56].  If words 60-61, LBA

| sectors, are not zero, words 57-58 shall be less than or equal to

| the value in words 60-61 at all times.

|

| The 32-bit value is placed into these words as follows:

|

| data bits  7 -  0 in word 57 bits 15 - 8,

| data bits 15 -  8 in word 57 bits  7 - 0,

| data bits 23 - 16 in word 58 bits 15 - 8,

| data bits 31 - 24 in word 58 bits  7 - 0.

5.3 Logical Block Addressing

| It is recommended that ATA drives over 528 MB support LBA

| (Logical Block Addressing).

5.3.1 Words 60-61

Identify Data words 60-61 shall specify the total number of user

sectors available in LBA mode at all times.  This value shall be

equal to or greater than the value in words 57-58 at all times.

The contents of these words shall not change.

| The 32-bit value is placed into these words as follows:

|

| data bits  7 -  0 in word 60 bits 15 - 8,

| data bits 15 -  8 in word 60 bits  7 - 0,

| data bits 23 - 16 in word 61 bits 15 - 8,

| data bits 31 - 24 in word 61 bits  7 - 0.

5.3.2 Orphan Sectors

The sectors, if any, between the last sector addressable in CHS

mode and the last sector addressable in LBA mode are known as

"orphan" sectors.  A drive may or may not allow access to these

| sectors in CHS addressing mode.

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Wed Mar  9 11:18:38 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17147; Wed, 9 Mar 94 11:18:38 PST

Received: from rutgers.edu by dt.wdc.com (4.1/SMI-4.1)

id AA17140; Wed, 9 Mar 94 11:18:33 PST

Received: from cbmvax.cbm.commodore.com by rutgers.edu (5.59/SMI4.0/RU1.5/3.08)

id AA28996; Wed, 9 Mar 94 14:18:07 EST

Received: by cbmvax.cbm.commodore.com

(5.65/UUCP-Project/Commodore - Ultrix 4.3 Ver 07/12/93)

id AA25510; Wed, 9 Mar 94 14:19:21 -0500

Date: Wed, 9 Mar 94 14:19:21 -0500

From: jesup@cbmvax.cbm.commodore.com (Randell Jesup)

Message-Id: <9403091919.AA25510@cbmvax.cbm.commodore.com>

To: ata@dt.wdc.com

Subject: Re: SFF-8019 rev 0.4

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Since we never get to any group/committee meetings, I just thought

I'd voice my support of the >528MB proposal.

Our ATA drivers already support these, since we allow up to 32K

cylinders.

Randell Jesup, OS Group Head, Commodore

jesup@commodore.com

From ata-owner  Wed Mar  9 12:26:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17963; Wed, 9 Mar 94 12:26:47 PST

Received: from ptltd.com (ns.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA17956; Wed, 9 Mar 94 12:26:39 PST

Received: from camb1.ptltd.com by ptltd.com (4.1/ (Hub configuration V0.2-2/28/92))

id AA11132; Wed, 9 Mar 94 15:26:36 EST

Message-Id: <9403092026.AA11132@ptltd.com>

Received: by camb1.ptltd.com with VINES ; Wed,  9 Mar 94 15:20:25 EST

Date: Wed,  9 Mar 94 14:53:57 EST

From: Curtis=Stevens%Eng.corp%PTLIRVINE@ptltd.com

Subject: SFF 8019

To: ATA@dt.wdc.com

Cc:

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Phoenix Technologies would like to voice their support for SFF-8019.  This

specification will ensure that drive ID data is consistent across all drive

manufacturers as we run into the 8GB wall.  Phoenix Technologies also

supports the limits on drive geometries presented in the ID drive data

because various MSDOS artifacts such as the INT 13 interface and the Fixed

Disk Parameter Tables have intrinsic geometric limitations which can not be

violated.  There is really only one solution to the 528 barrier, but there

are several different implementations.  An implementation which is code

intensive (too big), or time intensive (too slow) is not a reasonable

solution.  The geometry limits imposed by SFF-8019 allow for simple, quick

and consistent solutions.

Curtis E. Stevens

From ata-owner  Thu Mar 10 22:56:18 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08228; Thu, 10 Mar 94 22:56:18 PST

Received: from vnet.IBM.COM by dt.wdc.com (4.1/SMI-4.1)

id AA08221; Thu, 10 Mar 94 22:56:09 PST

Message-Id: <9403110656.AA08221@dt.wdc.com>

Received: from BCRVM1.VNET.IBM.COM by vnet.IBM.COM (IBM VM SMTP V2R2)

with BSMTP id 4759; Fri, 11 Mar 94 01:53:00 EST

Date: Fri, 11 Mar 94 01:52:59 EST

From: russb@vnet.IBM.COM

To: ata@dt.wdc.com

Subject: (U)  SFF-8019 - Comments

Sender: ata-owner@dt.wdc.com

Precedence: bulk

From: Russ Berkoff

Subject: (U)  SFF-8019 - Comments

Our experience with the 'Current Translation' fields in the

ATA Identify data have been FAR from pleasant.

The implementation of these fields by drive manufacturers

has been far from uniform. Unfortunately older ATA drives

tend to use this area for VENDOR unique data.

The validation fields suggested in the ATA spec are INADEQUATE.

Unfortunately, ATA Identify data does not contain a VALID LENGTH

which would tell us where the standardized fields end and the

the vendor unique fields begin.

This was a VERY unfortunate architecture oversight!

The remedy applied in our OS/2 drivers is to avoid usage

of these fields!

This requires us to ask translating BIOS suppliers to leave

the drives which require BIOS translation at their DEFAULT Identify

settings.

It would help us greatly if BIOS providers adhered to this

guideline.

Thanks

Russ Berkoff

OS/2 Development

IBM Corp.

From ata-owner  Fri Mar 11 00:02:52 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08852; Fri, 11 Mar 94 00:02:52 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08847; Fri, 11 Mar 94 00:02:44 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id XAA22861; Thu, 10 Mar 1994 23:39:35 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 10 Mar 1994 23:30:21 EST

Date:      Thu Mar 10 23:30:18 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d7ff3dd.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   DMA Command Description Cleanup = 93-110r5

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-10-94 10:58:34 PM

_To: ata@dt.wdc.com@internet

_Cc: ata review

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: DMA Command Description Cleanup = 93-110r5

DMA Command Description Cleanup

Document 93-110r5

--- --- --- NOTE NOTE NOTE --- --- ---

All of the items discussed in this document are included in

John Masiewicz's document 94-051r1.  This will be the last

revision of this document.  This document is superceeded by

document 94-051r1.

This document was prepared for the 14 March 94 ATA Extension

WG meeting in Newport Beach, CA.

--- The Round #5 Updated Issues List ---

Here is the final/last round of the DMA Definition discussion.

Please recall that round #4 was discussed at the X3T9.2 meetings

in Colorado Springs in November 1993.

For rounds #1 - #4 I used the ATA-1 Rev 4 section numbers.  For

round #5 I have switched to the ATA-2 (X3T9.2 948D rev 2) section

numbers.

Here is my update list of issues by ATA-2 section number:

* 5.2.9 DMARQ -- [See John Masiewicz's document 94-051r1.]

* 8 Command descriptions -- The following paragraphs should be

moved to section 9:

"If a new command is issued ... could have been completed."

"There shall be no indication ... New Command was issued."

* 8.10.15 Word 52 -- [See John Masiewicz's document 94-051r1 and

the SFF-8011 document.]

* 8.16 Read DMA -- I recommend that the second paragraph of this

section be replaced with the following text:

| During the execution of a Read DMA command, the drive shall

| provide status of BSY or DRQ until the command is completed.

|

| At command completion, the Command Block Registers contain the

| sector address of the last sector transferred.

|

| An unrecoverable error encountered during the execution of a

| Read DMA command results in the termination of the command.

| At command completion, the Command Block Registers contain the

| sector address of the sector where the first error occurred.

* 8.23 Set Features -- [SFF-8011 has addressed this issue.]

* 8.29 Write DMA -- I recommend that the second paragraph of this

section be replaced with the following text:

| During the execution of a Write DMA command, the drive shall

| provide status of BSY or DRQ until the command is completed.

|

| At command completion, the Command Block Registers contain the

| sector address of the last sector transferred.

|

| An unrecoverable error encountered during the execution of a

| Write DMA command results in the termination of the command.

| At command completion, the Command Block Registers contain the

| sector address of the sector where the first error occurred.

* 9 Protocol --

This section describes what happens when one command is

interrupted by another command.

The following paragraphs should be moved here from section 8:

| If a new command is issued ... could have been completed.

|

| There shall be no indication ... New Command was issued.

It is not possible to interrupt a Read DMA or Write DMA

command with another command.  The only way to interrupt a DMA

command is with a reset.  I recommend the following be added

to this section:

| The only way to interrupt a Read DMA or Write DMA command is

| with a reset.  The result of interrupting a Read DMA or Write

| DMA command by writing a new command into the Command register

| is unpredictable.

* 9.5 DMA Data Transfer Command -- I recommend the that the text

of this section be replaced by the following text:

| 10.5 DMA data transfer commands (optional)

|

| This class comprises:

|

| - Read DMA

| - Write DMA

|

| Data transfers using DMA commands differ in two ways from PIO

| transfers:

|

| - data transfers are performed using the slave-DMA channel

| - a single interrupt is issued at the completion of the command

|

| Initiation of the DMA transfer commands is identical to the

| Read Sector or Write Sector commands except that the host

| initializes the slave-DMA channel prior to issuing the

| command.

|

| The interrupt handler for DMA transfer is different in that no

| intermediate sector interrupts are issued on multi-sector

| commands.

|

| a) The host writes any required parameters to the Features,

|    Sector Count, Sector Number, Cylinder and Drive/Head

|    registers.

| b) The host initializes the slave-DMA channel.

| c) The host writes the command code to the Command register.

| d) The drive sets BSY.

| e) When the drive is ready to transfer data, the drive

|    asserts DMARQ.  The DMA data transfer may be split

|    into several partial transfers at the discretion

|    of the drive or slave-DMA channel.  The drive shall

|    assert either BSY or DRQ during the entire DMA data

|    transfer phase.

| f) When the drive has completed processing, it clears both

|    BSY and DRQ and asserts INTRQ.  The host reads the

|    Status register.

| g) The host resets the slave-DMA channel.

* 9.5.1 Normal DMA Transfer -- I recommend that the diagram be

replaced with the following diagram:

| +-a)---+-b)-------+-c)----+-d)---+-e)-----+-f)---+-g)--+

| |Select|Initialize|Issue  |      |DMA data|Status|Reset|

| |Drive |DMA       |Command|      |transfer|      |DMA  |

| +------+----------+-------+------+--------+------+-----+

| |      |BSY=0     |BSY=0  |BSY=1 |BSY=1   |BSY=0 |     |

| |      |          |       |      |or      |DRQ=0 |     |

| |      |          |       |      |DRQ=1   |Assert|     |

| |      |          |       |      |        |INTRQ |     |

* 9.5.2 Aborted DMA transfer -- I recommend that this section be

replaced by:

| 9.5.2 DMA transfer terminated by an error during data

|       transfer.

| +-a)---+-b)-------+-c)----+-d)---+-e)-----+-f)---+-g)--+

| |Setup |Initialize|Issue  |      |DMA data|Status|Reset|

| |      |DMA       |Command|      |transfer|      |DMA  |

| +------+----------+-------+------+--------+------+-----+

| |      |BSY=0     |BSY=0  |BSY=1 |BSY=1   |BSY=0 |     |

| |      |          |       |      |or      |DRQ=0 |     |

| |      |          |       |      |DRQ=1   |Assert|     |

| |      |          |       |      |        |INTRQ |     |

* 9.5.3 Aborted DMA command -- I recommend that this section be

replaced by:

| 9.5.3 DMA command terminated by an error before any data is

|       transferred.

| +-a)---+-b)-------+-c)----+-d)---+-f)---+-g)--+

| |Setup |Initialize|Issue  |      |Status|Reset|

| |      |DMA       |Command|      |      |DMA  |

| +------+----------+-------+------+------+-----+

| |      |BSY=0     |BSY=0  |BSY=1 |BSY=0 |     |

| |      |          |       |      |DRQ=0 |     |

| |      |          |       |      |Assert|     |

| |      |          |       |      |INTRQ |     |

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Fri Mar 11 00:03:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08899; Fri, 11 Mar 94 00:03:16 PST

Received: from uucp2-b.netcom.com by dt.wdc.com (4.1/SMI-4.1)

id AB08847; Fri, 11 Mar 94 00:03:03 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id XAA22866; Thu, 10 Mar 1994 23:39:53 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 10 Mar 1994 23:30:45 EST

Date:      Thu Mar 10 23:30:40 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d7ff3f5.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Power Management Description Cleanup = 93-188r1

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-10-94 11:03:20 PM

_To: ata@dt.wdc.com@internet

_Cc: ata review

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: Power Management Description Cleanup = 93-188r1

Power Management Description Cleanup

Document 93-188r1

At the 29 Sep 93 SFF / ATA WG meetings in Milpitas, CA, I was

talked into rewriting the Power Management command sections of

the ATA specification.  Later I joined the group of people

helping Steve Finch edit the ATA-2 document.  I prepared the

following command descriptions in the proposed ATA-2 command

description format for the Nov 93 ATA WG meeting in Colorado

Springs.  Here is revision 1 of the command descriptions prepared

for the 14 Mar 94 ATA Extensions WG meeting in Newport Beach, CA.

============================================================

What I present here are the many sections of the ATA-2 document

that describe the Power Management Feature Set.  A Feature Set is

a set of commands or functions that comprise an optional ATA

feature (another Feature Set is Read/Set/Write Multiple).

First, here is the ATA-2 section that describes the command

description format.

============================================================

z.z Command Description Format

Each command description in this document includes the following

information:  Type, Protocol, Inputs, Outputs, Prerequisites,

Description, and Errors.

TYPE

Type defines command category this command belongs to.

PROTOCOL

Type defines the command protocol this command uses.

INPUTS

Inputs define the Command Block registers used to pass command

parameters to the drive.

OUTPUTS

Outputs define the information returned by this command in the

Command Block registers.

PREREQUISITES

Prerequisites define any required prerequisite commands or state

that must exist before this command can be executed.

DESCRIPTION

Description is a detailed description of the command's processing.

ERRORS

Errors define the error conditions that may be signaled by

this command.

============================================================

Second, here are the sections that introduce the Power

Management Feature Set.

============================================================

x.x. Power Management Feature Set

The Power Management Feature Set permits a host to modify the

behavior of a drive in a manner which reduces the power required

to operate.  The Power Management Feature Set provides a set of

commands and a timer that enable a drive to implement low power

consumption modes.  A drive that implements the Power Management

feature shall implement the following minimum set functions:

1) A Standby timer

2) Idle command

3) Idle Immediate command

4) Sleep command

5) Standby command

6) Standby Immediate command

Additional vendor unique commands and functions are allowed.

x.x.1 Power modes

The lowest power consumption when the drive is powered on occurs

in Sleep Mode.  When in Sleep Mode, the drive requires a reset to

be activated.  The time to respond could be as long as 30

seconds.

In Standby Mode the drive interface is capable of accepting

commands, but as the media is not immediately accessible, it

could take the drive as long as 30 seconds to respond.

In Idle Mode the drive is capable of responding immediately to

media access requests.  A drive in Idle Mode may take longer to

complete the execution of a command because it may have to

activate some circuitry.

In Active mode the drive is capable of responding immediately to

media access requests, and commands complete execution in the

shortest possible time.  During the execution of a media access

command a drive shall be in Active mode.

x.x.2 Commands

The Check Power Mode command allows a host to determine if a

drive is currently in, going to or leaving Standby mode.

The Idle and Idle Immediate commands move a drive to Idle mode

immediately from the Active or Standby modes.  The Idle command

also sets the Standby Timer count and enables or disables the

Standby Timer.

The Sleep command moves a drive to Sleep mode.  The drive's

interface becomes inactive at the completion of the Sleep

command.  A reset is required to move a drive out of Sleep mode.

When a drive exits Sleep mode it may enter Active, Idle or

Standby mode.  The mode selected by the drive is based on the

type of reset received and on vendor unique implementation.

The Standby and Standby Immediate commands move a drive to

Standby mode immediately from the Active or Idle modes.

x.x.3 Standby timer

The Standby timer provides a method for the drive to

automatically enter Standby mode from either Active or Idle mode

following a host programmed period of inactivity.  If the Standby

timer is enabled and if the drive is in the Active or Idle mode,

the drive waits for the specified time period and if no command

is received, the drive automatically enters the Standby mode.

If the Standby Timer is disabled, the drive will not

automatically enter Standby mode.

x.x.4 Active to Idle mode transition

The transition from Active to Idle mode is vendor specified.  A

drive may immediately enter Idle mode at the completion of a

media access command or there may be a vendor specified delay

before Idle mode is entered.  It is possible that the Standby

timer may be enable for a time period that is less than the

normal automatic Active to Idle mode time period.  In this case,

the drive may move directly from Active to Standby mode.

x.x.5 Status

In the Active, Idle and Standby modes, the drive shall have Ready

status and shall be ready to accept any command.

In Sleep mode, the drive's interface is not active.  A host shall

not attempt to read the drive's status or issue commands to the

drive.

x.x.6 Power Mode transitions

Figure ? shows the minimum set of mode transitions that shall be

implemented.

----------------------+

|

+--------1-------- ACTIVE -------3---------+    |

|                                          |    |

| +------4------->        <------4-------+ |    |

| |                                      | |    |

| |                                      | |    |

| |                                      | |    |

| |                                      | |    |

| |                                      | |    |

| |                                      | |    |

V |                                      | V    5

|

IDLE <----------------2--------------- STANDBY   |

|

|  -----------------3-------------->    |      |

|                                       |      |

|                                       |      |

|                                       |      |

|                                       |      |

|                                       |      |

|                                       |      |

+-------5--------> SLEEP <------5-------+      |

|

|   <---------------------+

|

Resets ------6-----+   6

|   |

|   |

V   V

(see Path 6)

Path 1: An Idle command, Idle Immediate command or vendor unique

implementation moves the drive to Idle mode.

Path 2: An Idle command or Idle Immediate command moves the

drive to Idle mode.

Path 3: A Standby command, Standby Immediate or Standby timer

expiration moves the drive to Standby mode.

Path 4: A Media access command moves the drive to Active mode.

Path 5: A Sleep command moves the drive to Sleep mode.

Path 6: A reset, either hardware or software, moves the

drive to Active, Idle or Standby as specify by

the drive vendor.

Figure ?.  Power Management Modes

============================================================

Third, here are the command description sections.

============================================================

8.4. CHECK POWER MODE

TYPE

Optional - Power Management Feature Set.

PROTOCOL

Non-data command.

INPUTS

None.

OUTPUTS

The Sector Count register is set to 0 (00h) if the drive is going

to, in or leaving Standby mode.  The Sector Count register is set

to 255 (FFh) if the drive is in Active or Idle mode.

PREREQUISITES

None.

DESCRIPTION

If the drive is in, going to, or recovering from the Standby Mode

the drive shall set BSY, set the Sector Count register to 0 (00h),

clear BSY, and assert INTRQ.

If the drive is in Active or Idle Mode, the drive shall set BSY,

set the Sector Count register to 255 (FFh), clear BSY, and assert

INTRQ.

ERRORS

Command Abort - The drive does not support the Power Management

command set.

------------------------------------------------------------

8.11. IDLE

TYPE

Optional - Power Management Feature Set.

PROTOCOL

Non-data command.

INPUTS

The value in the Sector Count register when the IDLE command is

issued shall determine the time period programmed into the

Standby Timer.  See Table ??.

+---------------------+------------------------------+

| Sector Count        | Corresponding                |

| Register contents   | Timeout Period               |

+---------------------+------------------------------+

|   0       (00h)     | Timeout Disabled             |

|   1 - 240 (01h-F0h) | (value * 5) seconds          |

| 241 - 251 (F1h-FBh) | ((value - 240) * 30) minutes |

| 252       (FCh)     | 21 minutes                   |

| 253       (FDh)     | Vendor unique period between |

|                     | 8 and 12 hours               |

| 254       (FEh)     | Reserved                     |

| 255       (FFh)     | 21 minutes 15 seconds        |

+---------------------+------------------------------+

Table ?? - Automatic Standby Timer Periods

OUTPUTS

None.

PREREQUISITES

None.

DESCRIPTION

This command causes the drive to set BSY, enter the Idle Mode,

clear BSY, and assert INTRQ.  INTRQ is asserted even though the

drive may not have fully transitioned to Idle Mode.

If the Sector Count register is non-zero then the Standby Timer

shall be enabled.  The value in the Sector Count register shall

be used to determine the time programmed into the Standby Timer.

If the Sector Count register is zero then the Standby Timer is

disabled.

ERRORS

Command Abort - The drive does not support the Power Management

command set.

------------------------------------------------------------

8.12. IDLE IMMEDIATE

TYPE

Optional - Power Management Feature Set.

PROTOCOL

Non-data command.

INPUTS

None.

OUTPUTS

None.

PREREQUISITES

None.

DESCRIPTION

This command causes the drive to set BSY, enter the Idle Mode,

clear BSY, and assert INTRQ.  INTRQ is asserted even though the

drive may not have fully transitioned to Idle Mode.

ERRORS

Command Abort - The drive does not support the Power Management

command set.

------------------------------------------------------------

8.25. SLEEP

TYPE

Optional - Power Management Feature Set.

PROTOCOL

Non-data command.

INPUTS

None.

OUTPUTS

None.

PREREQUISITES

None.

DESCRIPTION

This command is the only way to cause the drive to enter Sleep

Mode.

This command causes the drive to set BSY, prepare to enter Sleep

mode, clear BSY and assert INTRQ.  The host shall read the Status

register in order to clear the interrupt and allow the drive to

enter Sleep mode.  In Sleep mode the interface becomes inactive.

The host shall not attempt to access the Command Block registers

while the drive is in Sleep mode.

Because some host systems may not read the Status register

and clear the interrupt, a drive may automatically deassert INTRQ

and enter Sleep mode after a vendor specified time period of not

less than 2 seconds.

The only way to recover from Sleep Mode is with a software reset

or a hardware reset.

A drive shall not power on in Sleep Mode nor remain in Sleep Mode

following a reset sequence.

ERRORS

Command Abort - The drive does not support the Power Management

command set.

------------------------------------------------------------

8.26. STANDBY

TYPE

Optional - Power Management Feature Set.

PROTOCOL

Non-data command.

INPUTS

The value in the Sector Count register when the STANDBY command

is issued shall determine the time period programmed into the

Standby Timer.  See Table ??.

+---------------------+------------------------------+

| Sector Count        | Corresponding                |

| Register contents   | Timeout Period               |

+---------------------+------------------------------+

|   0       (00h)     | Timeout Disabled             |

|   1 - 240 (01h-F0h) | (value * 5) seconds          |

| 241 - 251 (F1h-FBh) | ((value - 240) * 30) minutes |

| 252       (FCh)     | 21 minutes                   |

| 253       (FDh)     | Vendor unique period between |

|                     | 8 and 12 hours               |

| 254       (FEh)     | Reserved                     |

| 255       (FFh)     | 21 minutes 15 seconds        |

+---------------------+------------------------------+

Table ?? - Automatic Standby Timer Periods

OUTPUTS

None.

PREREQUISITES

None.

DESCRIPTION

This command causes the drive to set BSY, enter the Standby Mode,

clear BSY, and assert INTRQ.  INTRQ is asserted even though the

drive may not have fully transitioned to Standby Mode.

If the Sector Count register is non-zero then the Standby Timer

shall be enabled.  The value in the Sector Count register shall

be used to determine the time programmed into the Standby Timer.

If the Sector Count register is zero then the Standby Timer is

disabled.

ERRORS

Command Abort - The drive does not support the Power Management

command set.

------------------------------------------------------------

8.27. STANDBY IMMEDIATE

TYPE

Optional - Power Management Feature Set.

PROTOCOL

Non-data command.

INPUTS

None.

OUTPUTS

None.

PREREQUISITES

None.

DESCRIPTION

This command causes the drive to set BSY, enter the Standby Mode,

clear BSY, and assert INTRQ.  INTRQ is asserted even though the

drive may not have fully transitioned to Standby Mode.

ERRORS

Command Abort - The drive does not support the Power Management

command set.

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Fri Mar 11 00:12:12 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08963; Fri, 11 Mar 94 00:12:12 PST

Received: from uucp2-b.netcom.com by dt.wdc.com (4.1/SMI-4.1)

id AB08847; Fri, 11 Mar 94 00:03:25 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id XAA22863; Thu, 10 Mar 1994 23:39:46 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 10 Mar 1994 23:30:32 EST

Date:      Thu Mar 10 23:30:26 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d7ff3e9.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Master/Slave Handshaking = (no doc number)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-10-94 11:00:36 PM

_To: ata@dt.wdc.com@internet

_Cc: ata review

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: Master/Slave Handshaking = (no doc number)

Master/Slave Handshaking

Document ??-???r1

[I requested but never received a document number for this

document.  This is a repeat of the original posting of this

document.]

On the subject of master/slave handshaking...

One of the problems with ATA-1 is the unclear way that

master/slave handshaking is described and how the results of this

activity affect following operations.  This is really unfortunate

since there are more pages of the ATA-1 document devoted to this

topic than to any other part of the ATA interface.

Lets review the background...  The only reasons that this

handshaking is needed is to make ATA devices "look" like a single

device controller with one or two devices, or in other words, to

"look" like an MFM controller with one or two drives.  Why do

this?  Only one reason:  software compatibility -- making two

controllers "look" like one controller.

There are three events that invoke master/slave handshaking

(master is also known as drive 0, slave is also known as drive

1):

- after power on or hardware reset, drive 0's controller sets

BSY=0 when the reset function is completed by both

controllers.

- after software reset, drive 0's controller sets BSY=0 when the

reset function is completed by both controllers.

- after a Diagnostic command, drive 0's controller sets BSY=0

when the diagnostic command is completed by both controllers.

The master/slave handshaking is uni-directional:  the slave sends

signals to the master so that the master can determine when the

slave has completed one of these three functions.

Another "feature" of the MFM type controller was that if there

was only one drive, a master, and the host selected the

non-existant slave drive, all of the drive status bits would be

zero (because there was no drive to assert any of the these

bits).  In the ATA world, this means that the master must respond

with status of 00H if the non-existant slave drive is selected.

Note that DRDY is a DRIVE status bit, not a controller status

bit, and just because the controller is "not busy" does not imply

anything about the "ready" status of a drive.

Now to the root to the problem...  Following a hard reset, a soft

reset or a Diagnostic command, when is it illegal to issue a

command to the master or slave drive?

The answer is not simple and depends on several things:  which

drive and which command are the most important.  For all three

events, the sequence of events can be summarized as follows...

Drive 0 sets BSY=1, does the master/slave handshake, which could

take up to 31 seconds, and then drive 0 may set BSY=0.  Then

drive 0 can complete spinup, etc, and set DRDY=1.  The ATA has no

specification for when DRDY=1 should happen.  Please note that

there is NO reference to the DRDY bit of the Status register in

Annex A or B. What is in Annex A and B is the statement

"Drive 0 clears BSY when ready to accept commands (within 31

seconds)."

I think a lot of people read this sentence as if it said

"Drive 0 clears BSY and sets DRDY when ready to accept

commands (within 31 seconds)."

If people do interpret this sentence in this way, they are wrong.

Don't forget that traditionally there are two commands that were

legal when BSY=0 and DRDY=0:  Execute Diagnostics and Initialize

Drive Parameters.  These are "controller commands" and may not

require media access.  Media access commands are not legal until

BSY=0 AND DRDY=1.

So how can we clear this up?  Well...  ATA-1 Annex A and Annex B

both describe master/slave handshaking, Annex A from the drive

standpoint and Annex B from the host standpoint.  For ATA-2, I

assmue that Annex A will be move into the body of the document

and that Annex B will be discarded.  Therefore, I recommend the

following changes to the Annex A text.  My changes are marked by

|| at the left margin.  I do realize that my suggested changes

may not be acceptable to everyone for several reasons.

Also note the sentence marked by "->".  I would bet that a lot of

people fail to "see" this sentence when they read Annex A!

----- begin Hale's marked up Annex A -----

Annex A (informative)

Diagnostic and reset considerations

This annex describes the following timing relationships during:

a) Power on and hardware resets

- One drive

- Two drives

b) Software reset

- One drive

- Two drives

c) Diagnostic command execution

- One drive

- Two drives

- Two drives - drive 1 failed

The timing assumes the following:

- DASP- is asserted by Drive 1 and received by Drive 0 at power-on or

hardware reset to indicate the presence of Drive 1.  At all other

times it is asserted by Drive 0 and Drive 1 to indicate when a drive

is active.

- PDIAG- is asserted by Drive 1 and detected by Drive 0.  It is used

by Drive 1 to indicate to Drive 0 that it has completed diagnostics

and is ready to accept commands from the Host (BSY bit is cleared).

->         This does not indicate that the drive is ready, only that it can

->         accept commands.  This line may remain asserted until the next reset

occurs or an Execute Diagnostic command is received.

- Unless indicated otherwise, all times are relative to the event that

triggers the operation (RESET-, SRST=1, Execute Diagnostic Command).

A.1 Power on and hardware resets

A.1.1 Power on and hardware resets - one drive

- Host asserts RESET- for a minimum of 25 usec.

- Drive 0 sets BSY within 400 nsecs after RESET- is negated.

- Drive 0 negates DASP- within 1 msec after RESET- negated.

- Drive 0 performs hardware initialization

- Drive 0 may revert to its default condition

- Drive 0 waits 1 msec then samples for at least 450 msec for DASP- to

be asserted from Drive 1.

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (optional, within 31 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.1.2 Power on and hardware resets - two drives

- Host asserts RESET- for a minimum of 25 usec.

- Drive 0 and Drive 1 set BSY within 400 nsec after RESET- negated.

- DASP- is negated within 1 msec after RESET- is negated.

A.1.2.1 Drive 1

- Drive 1 negates PDIAG- before asserting DASP-.

- Drive 1 asserts DASP- within 400 msecs after RESET- (to show

presence).

- Drive 1 performs hardware initialization and executes its internal

diagnostics.

- Drive 1 may revert to its default condition

- Drive 1 posts diagnostic results to the Error Register

||       - Drive 1 clears BSY when ready to accept non-media access commands

||         (optional, within 30 seconds from RESET-).

- Drive 1 asserts PDIAG- to indicate that it is ready to accept

commands (within 30 seconds from RESET-).

||       - Drive 1 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

- Drive 1 negates DASP- after the first command is received or negates

DASP-if no command is received within 31 seconds after RESET-.

A.1.2.2 Drive 0

- Drive 0 performs hardware initialization and executes its internal

diagnostics.

- Drive 0 may revert to its default condition

- Drive 0 posts diagnostic results to the Error Register

- After 1 msec, Drive 0 waits at least 450 msec for DASP- to be

asserted (from Drive 1).  If DASP- is not asserted, no Drive 1 is

present (see POWER-ON RESET - One Drive operation).

- Drive 0 waits up to 31 seconds for Drive 1 to assert PDIAG-.  If

PDIAG- is not asserted, Drive 0 sets Bit 7=1 in the Error Register.

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (optional, within 31 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any commands.

A.2 Software reset

A.2. A.2.1  Software reset - one drive

- Host sets SRST=1 in the Device Control Register.

- Drive 0 sets BSY within 400 nsec after detecting that SRST=1.

- Drive 0 performs hardware initialization and executes its internal

diagnostics.

- Drive 0 may revert to its default condition.

- Drive 0 posts diagnostic results to the Error Register.

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (optional, within 31 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.2.2 Software reset - two drives

- Host sets SRST=1 in the Device Control Register.

- Drive 0 and Drive 1 set BSY within 400 nsec after detecting that

SRST=1.

- Drive 0 and Drive 1 perform hardware initialization.

- Drive 0 and Drive 1 may revert to their default condition.

A.2.2.1 Drive 1

- Drive 1 negates PDIAG- within 1 msec.

||       - Drive 1 clears BSY when ready to accept non-media access commands

||         (within 30 seconds).

- Drive 1 asserts PDIAG- to indicate that it is ready to accept

commands (within 30 seconds).

||       - Drive 1 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.2.2.2 Drive 0

- Drive 0 waits up to 31 seconds for Drive 1 to assert PDIAG-.

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (within 31 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.3 Diagnostic Command Execution

A.3.1 Diagnostic command execution - one drive (passed)

- Drive 0 sets BSY within 400 nsec after the Execute Diagnostic command

was received.

- Drive 0 performs hardware initialization and internal diagnostics.

- Drive 0 resets Command Block registers to default condition.

- Drive 0 posts diagnostic results to the Error Register

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (within 6 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.3.2 Diagnostic command - two drives (passed)

- Drive 0 and Drive 1 set BSY within 400 nsec after the Execute

Diagnostic command was received.

A.3.2.1 Drive 1

- Drive 1 negates PDIAG- within 1 msec after command received.

- Drive 1 performs hardware initialization and internal diagnostics.

- Drive 1 resets the Command Block registers to their default

condition.

- Drive 1 posts diagnostic results to the Error Register

||       - Drive 1 clears BSY when ready to accept non-media access commands.

- Drive 1 asserts PDIAG- to indicate that it is ready to accept

commands (within 5 seconds).

||       - Drive 1 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.3.2.2 Drive 0

- Drive 0 performs hardware initialization and internal diagnostics.

- Drive 0 resets the Command Block registers to their default

condition.

- Drive 0 waits up to 6 seconds for Drive 1 to assert PDIAG-.

- Drive 0 posts diagnostic results to the Error Register

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (within 6 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.3.3  Diagnostic command execution - one drive (failed)

- Drive 0 sets BSY within 400 nsec after Diagnostic command received.

- Drive 0 performs hardware initialization and internal diagnostics.

- Drive 0 resets Command Block registers to default condition.

- Drive 0 posts a Diagnostic Code to the Error Register indicating a

failure.

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (within 6 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.3.4 Diagnostic command execution - two drives (drive 1 failed)

- Drive 0 and Drive 1 set BSY within 400 nsec after Diagnostic command

received.

A.3.4.1 Drive 1

- Drive 1 negates PDIAG- within 1 msec after command received.

- Drive 1 performs hardware initialization and internal diagnostics.

- Drive 1 resets the Command Block registers to their default

condition.

- Drive 1 posts a Diagnostic Code to the Error Register indicating

failure.

||       - Drive 1 clears BSY when ready to accept non-media access commands.

- Drive 1 does not assert PDIAG-, indicating that it failed

diagnostics.

||       - Drive 1 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

A.3.4.2 Drive 0

- Drive 0 performs hardware initialization and internal diagnostics.

- Drive 0 resets the Command Block registers to their default

condition.

- Drive 0 waits 6 seconds for Drive 1 to assert PDIAG- but PDIAG- is

not asserted by Drive 1.

- Drive 0 posts a Diagnostic Code to the Error Register setting Bit 7=1

to indicate that Drive 1 failed diagnostics.

||       - Drive 0 clears BSY when ready to accept non-media access commands

||         (within 6 seconds).

||       - Drive 0 clears BSY (if not previously done) and sets DRDY when

||         ready to accept any command.

NOTE 1 The 6 seconds referenced above is a host-oriented value.

----- end Hale's marked up Annex A -----

Now lets look at all of this from a host software standpoint...

There are three different methods implemented in drives to

handle the BSY and DRDY bits following a hard reset, a soft reset

or a Diagnostic command.  Host software MUST be designed to

handle all three methods since drives have been built using all

three methods.  Note that all three methods are allowed by the

ATA.

"m/s" denotes when the master/slave handshaking takes place (up

to 31 seconds!) and "spin-up" denotes the time required to become

ready to accept media access commands (no time limit!  REPEAT -

NO TIME LIMIT IS SPECIFIED BY THE ATA FOR THIS ACTIVITY!).

Method 1 -- The traditional method.

Most drives implementing Method 1 will reject a media access

command received during the spin-up time with Error and Command

Abort.  Don't forget about the commands that are legal during the

spin-up time when a drive's status is 00H:  Initialize Drive

Parameters and Execute Diagnostics.

____

BSY  ___/    \_________________________________________

___________

DRDY ______________________________________/

|m/s.|......spin-up................|

Method 2 -- Very popular these days.

Method 2 is very popular these days probably because it is the

most simple to implement in drive firmware.  Time to DRDY=1/BSY=0

must not exceed 31 seconds to be ATA compatible.

___________________________________

BSY  ___/                                   \__________

___________

DRDY ______________________________________/

|m/s.|......spin-up................|

Method 3 -- Makes for complex drive firmware.

Method 3 requires that the drive "accept and queue" a media

access command that is received during the spin-up time and this

way cause a command time-out on some host systems.  This is

tricky... the time from BSY=0 until the actual start of the

"queued" command's execution should not exceed 31 seconds.

____

BSY  ___/    \_________________________________________

___________________________________________

DRDY ______/

|m/s.|......spin-up................|

Remember, host software MUST be implemented such that all three

methods are acceptable.  And also remember that the master may

implement a different method than the slave.

--- And now, one final comment ---

If I could change the way host software is designed, I would do

the following (and I would definitely do this in a notebook

computer if there was no possibility that a second drive could be

added to the computer's ATA host adapter):

Support only one drive per ATA host adapter with that drive

configured as slave (drive 1).  This would be great...  No

master/slave handshaking to worry about...  No multi-vendor

compatibility problems...  No problems with a Drive 0 that

"hangs" waiting for a Drive 1 to do something...  No problems

with a Drive 0 that thinks there is no Drive 1 when there really

is a Drive 1...  No problems with a Drive 0 that thinks there is

a Drive 1 when there really is no Drive 1...

Ahhh...  Life would be wonderful!  But then there is nothing more

difficult in the entire universe than getting software people to

change the way they do things.

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Fri Mar 11 12:38:41 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15629; Fri, 11 Mar 94 12:38:41 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA15622; Fri, 11 Mar 94 12:38:32 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA02704; Fri, 11 Mar 1994 12:09:05 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 11 Mar 1994 12:00:38 EST

Date:      Fri Mar 11 12:00:36 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d80a3b6.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Russ Berkoff's comments on ATA ID data

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-11-94 11:24:12 AM

_To: ata@dt.wdc.com@internet

_To: sff_reflector@Sun.COM@internet

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: Russ Berkoff's comments on ATA ID data

Russ Berkoff (OS/2 Development, IBM Corp.) posted a message concerning ATA

Identify

data  I would like to follow up on his comments...

RB>>>Our experience with the 'Current Translation' fields in the

ATA Identify data have been FAR from pleasant.

The implementation of these fields by drive manufacturers

has been far from uniform. Unfortunately older ATA drives

tend to use this area for VENDOR unique data.

HL:  How true, but the reserved for future use and reserved for vendor use has

been clearly

defined in ATA as far back as I can remember.  Vendors that use the reserved

for future use

words as vendor unique are probably not reading the ATA document and do not

realize the

amount of trouble they are causing for the rest of the world.  Not much we can

do about that

now.

RB>>>The validation fields suggested in the ATA spec are INADEQUATE.

HL:  This has been said about almost every ID word.  I recall a long discussion

about word

47 and how to tell if a drive really supported R/W Multiple correctly.  One

result of this

discussion was the addition of word 59.  But the problem of correct

implementation of the

R/W Multiple commands exists even on drives that have both word 47 and word 59

set correctly.  This is a quality issue -- vendors, including my company, must

insure that

the drive really does do what the ID says it does -- I don't think the ATA

committee can do

much more.

RB>>>Unfortunately, ATA Identify data does not contain a VALID LENGTH

which would tell us where the standardized fields end and the

the vendor unique fields begin.  This was a VERY unfortunate architecture

oversight!

HL: I just looked at ATA rev 3.0, November 1991, and it clearly defines which

words are

Vendor Unique.  True, it may not have said in plain English that words 53-127

were reserved

(for future use and should be zero) but I think it was pretty clear that this

is what the committee

intended.

RB>>>The remedy applied in our OS/2 drivers is to avoid usage

of these fields!

HL:  I think there are some simple tests that can be made on many of the ID

words to see if they are valid. For example a word that is FFFFh is probably

invalid or if word

55 is greater than 16 it is probably invalid.  I don't think the software

people of the world will

ever be able to use any of the ID data without doing some kind of validity

checking.  To say

that just because vendor X put the wrong information into word Y, we will never

use word Y

again is unfortunate.  However, if this is going to become accepted practice,

then lets eliminate

the ID command from the ATA.  Why have something that no one uses and why waste

hours

of committee time creating the ID data definitions?

RB>>>This requires us to ask translating BIOS suppliers to leave

the drives which require BIOS translation at their DEFAULT Identify

settings.

HL: But given the way words 1, 3, 6 and 53-58 have been implemented to date,

how can

a BIOS tell what the drive's real DEFAULT settings is?  I don't think words 1,

3 and 6 were

ever supposed to change for the life of a drive.  However, several vendors have

for years

produced drives that reflect the current setting back in these words instead of

in words

53-58.  To make matter worse, some drives remember the last setting through

power off/on

so that the original factory default SETTING is lost forever!  Yep, this is a

mess -- that's why

I initiated the documents at Seagate that have resulted in SFF-8019 rev 0.4.

RB>>>It would help us greatly if BIOS providers adhered to this guideline.

HL: One of the reasons Seagate/WD held the High Capacity Storage Conference at

Comdex

was to get the software people to start talking to each other.  The disc drive

industry talks

all the time and works out specifications and standards for all to use.  We did

not see this

happening amoung the software people for the issue of larger ATA drives.  While

the

conference was somewhat of a

success, I don't think it fostered any large amount of public discussion of the

software

issues around large drives.  The result is that we now have a specification

from Phoenix

that was not publicly discussed that will probably become the defacto software

standard.

I don't know what to do about this -- all I know is that large ATA drives are

here and the

Seagate expects those drives to be supported by BIOS, O.S. and other software.

My request of all the software people is to get this issue resolved quickly.

Seagate does not

want to see a vast array of incompatible solutions to this problem.  If this

software issue is

not solved immediately, a vast array of incompatible solutions is exactly what

will see.

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Fri Mar 11 13:03:52 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15920; Fri, 11 Mar 94 13:03:52 PST

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA15913; Fri, 11 Mar 94 13:03:41 PST

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA25756 for ata@dt.wdc.com; Fri, 11 Mar 94 13:03:39 -0800

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA28997 for ata@dt.wdc.com; Fri, 11 Mar 94 13:03:38 -0800

Received: from bach.nsc.com by berlioz.nsc.com (4.1/SMI-4.1)

id AA17763; Fri, 11 Mar 94 13:03:36 PST

Date: Fri, 11 Mar 94 13:03:36 PST

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9403112103.AA17763@berlioz.nsc.com>

To: ata@dt.wdc.com, sff_reflector@sun.com

Subject: ATA SSWG Minutes - Feb 94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:     ATA Reflector

SFF Reflector

From:   Robbie Shergill

National Semiconductor

Subj:   ATA Extensions SSWG Minutes - Feb 94

Date:   March 11, 1994

*****************************************************************************

Doc No: X3T10/94-055r0

Minutes of the ATA-Extensions Special Working Group Meeting

Held February 23, 1994, in Milpitas, CA, at Crown Sterling Suites Hotel.

Chaired by:          Steve Finch, SSI.

Hosted by:           Jim McGrath, Quantum Corp.

Minutes recorded by: Robbie Shergill, National Semiconductor.

Following documents were distributed in this meeting:

- 94-052r0: Proposed change of Mode 4 T0... (T. Hanan, WD)

- 94-051r0: Comments on ATA-2 rev 2 (J. Masiewicz, Seagate)

- 94-047r0: Minutes of the January SSWG meeting (R. Shergill, National)

- 94-027r1: Data CRC (R. Kalish, Adaptec)

- ________: Low Power Modes (T. Hanan, WD)

1. Steve Finch opened the meeting at 9:30AM by thanking the host and stating

the purpose of the meeting and some pertinent ANSI mechanisms. He asked each

attendee to introduce themselves and also sign the attendees list.

2. Agenda for this meeting was circulated by Steve Finch. It was modified

and accepted.

1. Opening remarks, introductions and welcome

2. Approval of agenda

3. Attendance and membership

4. Time and place of next meeting

5. Old Business

5.1 16.6 MB/s

5.2 Data CRC

6. New Business

6.1 Seagate Comments and Driver Specs (Masiewicz)

6.2 Rise/fall times

6.3 Issues from ATA-2 editors meeting

6.4 Minimum reset pulse width

6.5 DRQ deassertion after the last byte is transfered

6.6 Miscellaneous

7. Adjournment

3. Attendance List

NAME                 COMPANY               PHONE

Al Pham              Adaptec              408-945-2560

Richard Kalish       Adaptec              408-957-7169

John Geldman         Cirrus Logic         510-226-2101

Dave Weber           Cirrus Logic         510-226-2367

Mark Gurkowski       Conner Peripherals   303-682-8317

Dan Colegrove        IBM                  507-289-0922

Larry Lamers         Adaptec              408-945-8600

Robbie Shergill      National Semi.       408-721-7959

Charles Yang         Panasonic            408-262-2200

Jim McGrath          Quantum              408-894-4504

Jim Wilshire         Quantum              408-894-5245

Mike Carpenter       Q-Logic              714-668-5366

Hank Davenport       Samsung              408-434-5427

John Masiewicz       Seagate              408-439-2152

Steve Finch          Silicon Systems      714-573-6808

Tom Hanan            Western Digital      714-832-7472

4. Time and Place for the Next Meeting:

Mar 14: Newport Beach (as part of X3T9.10, 2-8 PM)

Mar 30: TBD  (possibly Western Digital, Irvine)

(Next Editors' Meeting is on March 13 at Newport Beach).

5. Old Business

5.1  16.6 MB/s:

Tom Hanan presented revised Mode 4/Mode 2 timings that are contained in his

document. The group reviewed the proposed timing values. Although Tom is

pointing out the system design difficulty with 25ns recovery time, he is not

recommending that this time should be reduced to 20ns.

The group decided to change t4 back to 10ns so that it would be the same in

PIO and DMA modes.

The note concerning IOCS16- was debated. John Geldman's suggestion of

removing the note and putting in a recommendation was agreed to.

John Masiewicz suggested that DIOR- setup time be reduced by 5ns. This was

not agreed to.

John Geldman suggested that IOCS16- "negation" should be changed to

"release". Agreed.

Mark Gurkowski asked again that Note 4 of the table make it clear that the

minimum t0 time supported is returned in the ID data. This was agreed to.

Robbie Shergill raised the point that t0 now doesn't apply to assert edge-

to-assert edge cycle. After a brief discussion it was decided that this is

true and it is allowed.

It was decided that the DMA diagram will be changed back to show that data

lines can be kept enabled during burst transfers.

John Geldman pointed out that the "xx"s on read data have been dropped -

these should begin when DIOR- is asserted. Tom Hanan wanted the "xx"s to

start from DMACK- assertion. Significant discussion took place, a vote was

taken and Tom's proposal was denied.

Dan Colegrove asked if a drive is required to set mode 3 bit also if mode 4

bit is set. It was clarified that you don't have to; you only have to always

meet mode 0. However, the low-level timings guarantee that if you can make

mode 4, you can make mode3 also.

5.2 DATA CRC:

Since Richard Kalish was not at the last meeting, he wanted to revisit this

issue. It was clarified that at the last meeting it was decided that this

issue should be debated on the reflector under the broader heading of data

integrity. If necessary, a separate SSWG can be started on this topic. No

further discussion took place.

6. NEW BUSINESS:

6.1 SEAGATE COMMENTS ON ATA-2:

John Masiewicz went over his document and each item was discussed in order.

The issue of 3v and 5v drive power was discussed. John Masiewicz just

doesn't want to endorse  the plugging of 3v drives into 5v systems.

Steve Finch proposed that we eliminate support the 3v option from the power

connector. After further discussion, a vote was taken on this proposal and

it passed unanimously. Steve will take this recomendation to X3T10 next

month.

Tom Hanan propsed that since we don't adequately address 3v operation, we

should take out all references to 3v operation. This was also agreed to

unanimously.

John Masiewicz feels that his proposed electrical specs have been put into

ATA-2 in such a way that the true intent is lost - thus the new proposal.

John wanted to know if we want to include the ATA-1 info in ATA-2 spec?  Tom

answered yes, because ATA-2 replaces ATA-1.

Jim McGrath brought up the issue of mixing ATA-2 and ATA-1 drives/systems.

Tom felt that we have to have a way to alter the driver characteristics

based on the mode we're running in. McGrath said that this would be

difficult because most systems don't even want to do a Set Features.

Robbie Shergill said that it sounds like we need to do mode selection in

hardware - such as using the IOCS16- (grounded means ATA-2) for this

purpose. Tom Hanan agreed with the desirability of hardware selection, but

not at the cost of an interface signal.

As far as electrical specifications are concerned, John Masiewicz asserted

that his proposal of using thevenin equivalent loads covers both ATA-1 and

ATA-2 for all signals except IOCS16-. He will look at this issue more and

revise the load circuit if needed.

Based on Masiewicz's comments, a number of other corrections were also

agreed to. John will revise his document and Steve Finch will edit the ATA-2

document appropriately.

Al Pham (Adaptec) suggested that INTRQ signal's enabling/disabling times

should be specified. Concern is that the signal may drift up to a logical-

high level when one drive is deselected and another is selected. The group

agreed that something like 50ns max time should be spec'd from drive

selection to INTRQ enabled. Tom Hanan will write a proposal.

6.2 RISE AND FALL TIMES:

The issue of rise and fall time specification was debated extensively. Jim

Wilshire (Quantum) wanted to change to 10 and 90 percent points, but John

Masiewicz said that there is difficulty with this method in determining what

the full amplitude should be. Tom Hanan and Jim Wilshire said that their

driver design slews at different rates during different portions of the

transition; but Mark Gurkowski asserted that a spec shouldn't tell people

how to implement. Finally, there was some agreement that perhaps specifying

dv/dt for a given load is the best way to go.

No final resolution was reached. It was agreed that we all will bring our

specific preference to the next meeting in Newport Beach.

6.3 ISSUES FROM EDITORS' MEETING:

Steve Finch asked if we should drop the alternate connector descriptions

from the annexes?

Tom Hanan wants to either do a complete job or drop it altogether.

Steve will leave them in for now. Further discussion defered to the editors'

meeting.

6.4 RESET MIN TIME:

Adaptec raised this issue again: at what point does a reset pulse become

narrow enough to be called a glitch and, therefore, is ignored?

John Masiewicz doesn't want a new spec that will obselete existing hardware.

He also believes that the safest thing for the system is that the drive goes

ahead and resets anytime it sees a reset pulse of any duration. Tom Hanan

doesn't agree with this philosophy.

Steve Finch stated that as it stands today, under 255s the drive's behavior

is undefined as per ATA spec. The drive spec may say that below n ns it will

definitely ignore, but the ATA spec doesn't have to address it.

Jim McGrath said that in reality a few nano-seconds wide pulse has to be

filtered. It was decided that we should take this discussion to the

Reflector. Everyone should give their input on x and y as defined below and

as their silicon does it today:

0            x                y             255s

|------------|----------------|-------------|---------

ignore        maybe          recognize

6.5 DRQ CLEARING AFTER LAST TRANSFER:

Tom Hanan reported that one of his customers, who is a system manufacturer,

is saying that they have a fast BIOS that is having a problem where, on more

than one drive, they see that just after the last data word transfer the DRQ

bit is still set and the BUSY bit is still not set. The BIOS then thinks

that the drive wants to transfer the next sector and it just proceeds with

that. Tom's customer is not blaming the drive for doing this, but in order

to avoid this he wants the ATA-2 spec to add a required minimum time delay

between the data phase and the status phase that the BIOS wil have to

observe. They recommended 55s, but the group thinks that 55s is too long.

All agreed to put a limit of 1 5s. Tom will write a proposal.

6.6 MISCELLANEOUS:

Steve Finch informed everyone that the ATA-3 project proposals will be voted

upon in the March Plenary. So, make sure your rep is ready to vote.

John Masiewicz asked if we can safely say now that 16MB/s mode will be

included in ATA-2. Steve Finch said that he intends to show 16MB/s to the

Plenary this March and then give them two months so in May they can vote on

it.

7. The meeting was adjourned at 3:00 PM.

***************************************************************************

From ata-owner  Fri Mar 11 14:29:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA16754; Fri, 11 Mar 94 14:29:49 PST

Received: from rutgers.edu ([128.6.7.10]) by dt.wdc.com (4.1/SMI-4.1)

id AA16744; Fri, 11 Mar 94 14:29:42 PST

Received: from cbmvax.cbm.commodore.com by rutgers.edu (5.59/SMI4.0/RU1.5/3.08)

id AA10170; Fri, 11 Mar 94 17:29:21 EST

Received: by cbmvax.cbm.commodore.com

(5.65/UUCP-Project/Commodore - Ultrix 4.3 Ver 07/12/93)

id AA27776; Fri, 11 Mar 94 17:30:51 -0500

Date: Fri, 11 Mar 94 17:30:51 -0500

From: jesup@cbmvax.cbm.commodore.com (Randell Jesup)

Message-Id: <9403112230.AA27776@cbmvax.cbm.commodore.com>

To: ata@dt.wdc.com

Subject: re:  Russ Berkoff's comments on ATA ID data

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hale Landis writes:

>>Our experience with the 'Current Translation' fields in the

>>ATA Identify data have been FAR from pleasant.

>>The implementation of these fields by drive manufacturers

>>has been far from uniform. Unfortunately older ATA drives

>>tend to use this area for VENDOR unique data.

>>The validation fields suggested in the ATA spec are INADEQUATE.

>

>HL:  This has been said about almost every ID word.  I recall a long discussion

>about word

>47 and how to tell if a drive really supported R/W Multiple correctly.  One

>result of this

>discussion was the addition of word 59.  But the problem of correct

>implementation of the

>R/W Multiple commands exists even on drives that have both word 47 and word 59

>set correctly.  This is a quality issue -- vendors, including my company, must

>insure that

>the drive really does do what the ID says it does -- I don't think the ATA

>committee can do

>much more.

First, according to rev 4a of ATA word 47 isn't defined, so how can it

be set correctly?

Second, our drivers USE word 47!  Since it's no longer defined in the spec

does that mean that future drives might not set it?  And without word 47, how

can a driver know what size to use for SetMultiple?  Does it have to try large

sizes and see what shows up in word 59?

>>The remedy applied in our OS/2 drivers is to avoid usage

>>of these fields!

>

>HL:  I think there are some simple tests that can be made on many of the ID

>words to see if they are valid. For example a word that is FFFFh is probably

>invalid or if word

>55 is greater than 16 it is probably invalid.  I don't think the software

>people of the world will

>ever be able to use any of the ID data without doing some kind of validity

>checking.  To say

>that just because vendor X put the wrong information into word Y, we will never

>use word Y

>again is unfortunate.

If it's so critical to to validate fields in the identify data, why

isn't this even hinted at in the spec?  Those of us who came to the spec from

SCSI-land (and non-MSDOS-land) don't know the "history" of all these parameters

or what workarounds you have to do to be compatible with even a reasonable

fraction of the drives on the market.  We all end up learning the hard way

that what the spec says will work probably won't, even in the cases where

the spec is clear.  (A large part of the spec is less-than-clear unless you've

been working with PC disk controllers for years.)

Maybe ATA needs an appendix labelled "Here's how ATA _actually_ works",

or "All the incompatible ways ATA has been implemented", or "These drives

are broken in these ways".  (That last isn't really meant as a joke... it would

be very useful.  We'll never see it, since drive manufacturers wouldn't want to

publicly air their dirty laundry.)

A random example: ATA (the spec) supports 256 block transfers.  ATA

(the reality) doesn't - dozens of drives work for 255, and fail for 256 (or

fail for 256 when in R/W multiple mode).  Yes, most manufacturers have cleaned

up their act, but without a way to identify these drives the driver can't

allow 256 block transfers.

Without a real spec, and without the quality control to FOLLOW that

spec from all the manufacturers, we software people are forced to write to

the lowest common denominator (which is pretty darn low).

Oh well, frustration vented.

Randell Jesup, Commodore Engineering

From ata-owner  Fri Mar 11 16:05:59 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17828; Fri, 11 Mar 94 16:05:59 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA17821; Fri, 11 Mar 94 16:05:49 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id PAA19357; Fri, 11 Mar 1994 15:45:15 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 11 Mar 1994 15:35:08 EST

Date:      Fri Mar 11 15:35:01 199 PST

From: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

Message-Id: <2d80d5fd.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   X3T10/94-051r1 (ATA-2 Draft Comments revision 1)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-11-94 03:13:23 PM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: John Masiewicz@SEAGATE

_Subject: X3T10/94-051r1 (ATA-2 Draft Comments revision 1)

This document contains the recommended changes from my original documents:

X3T10/94-051r0 dated 2/18/94 (Subject: Comments / Recommendations on ATA-2

Working Draft)

X3T10/94-058r0 dated 2/25/94.(Subject Word 51,52,64,66 Revisions for Comments)

This revision also contains previously agreed-upon changes from Hale Landis'

document:

X3T9/92-110r5 dated 3/10/94 (Subject:  DMA Command Description Cleanup)

I have included each affected paragraph or section with the changes edited in.

At the next meeting I will distribute a hard copy which highlights the

differences with strike-outs & additions.

All comments are against the X3T9.2/0948D revision 2 document.

4.2.2 3-pin power

The pin assignments are shown in table 2.  Recommended part numbers for the

mating connector to 18 AWG cable are shown below, but equivalent parts may be

used.

Connector (3 pin) Molex 5484 39-27-0032 or equivalent.

[Editor's Note:  Is this the device side connectors or the cable side

connectors?]

+========================+============+

| Power Line Designation | Pin Number |

+===========+============+============+

|  +5 Volts |            |     1      |

+-----------+------------+------------+

| +12 Volts |            |     2      |

+-----------+------------+------------+

|   Ground  |            |     3      |

+-----------+------------+------------+

Table 2 - DC Interface Using 3 Pin Power Connector

[Editor's Note:  How do we specify the pin one orientation?]

4.6  DC Characteristics

+--------------------------------------------+------+--------+

| Description                                |  Min |   Max  |

+--------+--------------------------------------------+------+--------+

|  IoL   | Driver sink current for 5V operation       | 12mA |        |

|  IoH   | Driver source current                      |-400uA|        |

|  Vih   | Voltage Input High                         | 2,0 V|        |

|  Vil   | Voltage Input Low                          |      |  0,8 V |

|  Voh   | Voltage Output High (Ioh = -400 u A)       | 2,4 V|        |

|  Vol   | Voltage Output Low (5V, Iol = 12ma)        |      |  0,5 V |

|  Cin   | Input Capacitance (of Device)              |      |  25 pf |

|  Cout  | Output Capacitance (of Device)             |      |  25 pf |

+--------+--------------------------------------------+------+--------+

Table 3A -  DC Characteristics

+--------------------------------------------+------+--------+

| Description                                |  Min |   Max  |

+--------+--------------------------------------------+------+--------+

|  tRISE | Rise time for any signal on AT interface(1)|  5ns |        |

|  tFALL | Rise time for any signal on AT interface(1)|  5ns |        |

+--------+--------------------------------------------+------+--------+

(1)  tRISE and tFALL are measured from 10-90% of full signal alplitude with a

total capacitive load of 100 pF.

Table 3B - AC Characteristics

[Editor's note:  IoH value at 400 uA is insufficient at least in the case of

DMARQ which is typically pulled low by a 5.6k resistor.]

X.X.X  ATA-2 Driver Types and Required Pull-ups

+==============+========+========+=========+==========+========+

|              |        | Driver | Pull-up | Pull-up  |        |

| Signal       | Source | Type   | at Host | at each  | Notes  |

|              |        |  (1)   |   (2)   | Device(2)|        |

+--------------+--------+--------+---------+----------+--------+

|              |        |        |         |          |        |

| Reset        | Host   |  TP    |         |          |        |

| DD 0:15      | Bidir. |  TS    |         |          |        |

| DMARQ        | Device |  TS    | 5.6K PD |          | (3)    |

| DIOR-  DIOW- | Host   |  TS    |         |          |        |

| IORDY        | Device |  TS    |  1.0K   |          | (4)    |

| SPSYNC/CSEL  |        |        |         |          | (5)    |

|    CSEL      | Host   |        | Ground  |   10K    | (6)    |

|    SPSYNC    | Device |  TS/OC |         |   VU     | (7)    |

| DMACK-       | Host   |  TP    |         |          |        |

| INTRQ        | Device |  TS    |         |          |        |

| IOCS16-      | Device |  OC    |  1.0K   |          |        |

| DA0:2        | Host   |  TP    |         |          |        |

| PDIAG-       | Device |  TS    |         |   10K    |        |

| CS0- CS1-    | Host   |  TP    |         |          |        |

| DASP-        | Device |  OC    |         |   10K    | (5)    |

|              |        |        |         |          |        |

+==============+========+========+=========+==========+========+

(1) TS=Tri-State; OC=Open Collector; TP=Totem-Pole; PU=Pull-up; PD=Pull-down;

VU=Vendor Unique

(2) All resistor values are minimum (lowest) allowed.

(3) ATA-2 defines this line to be tri-stated whenever the device is not

selected or is not executing a DMA data transfer.  When enabled by DMA

transfer, it will be driven high and low by the device.

(4) This signal should only be enabled during DIOR/DIOW cycles to the selected

device.

(5) See signal descriptions for information on dual use of this signal.

(6) When used as CSEL, Line is grounded at Host and 10K Pull-up is required at

both devices.

(7) When Used as SPSYNC, application is vendor unique.

Table x - ATA-2 Driver Types and Required Pull-ups

5.2.8 DMACK- (DMA acknowledge) (Optional)

This signal shall be used by the host in response to DMARQ to either

acknowledge that data has been accepted, or that data is available.

NOTE:  This signal may be deasserted by the Host to suspend the DMA

transfer in process.  For Multi-Word DMA transfers, the Device may deassert

DMARQ within the tL specified time (refer to Figure 7 pg. 71) once DMACK- is

asserted and reassert it again at a later time to resume the DMA operation.

Alternatively, if the device is able to continue the transfer of data, the

Device may leave DMARQ asserted and wait for the Host to reassert DMACK-.

5.2.9 DMARQ (DMA request) (Optional)

This signal, used for DMA data transfers between host and drive, shall be

asserted by the drive when it is ready to transfer data to or from the host.

The direction of data transfer is controlled by DIOR- and DIOW-.  This signal

is used in a handshake manner with DMACK- i.e.  the drive shall wait until the

host asserts DMACK- before negating DMARQ, and re-asserting DMARQ if there is

more data to transfer.

This line will be released (high impedance state) whenever the device is not

selected or is selected and no DMA command is in progress.  When enabled by DMA

transfer, it will be driven high and low by the device.

When a DMA operation is enabled, IOCS16-, CS0- and CS1- shall not be asserted

and transfers shall be 16-bits wide.

NOTE:  In ATA-1 devices, this signal was either totem-pole or tri-state in

different implementations.  In EISA systems, 5.6K pull-down is used to cause a

logic low on undriven lines.  ATA-2 defines this line to be in high-impedance

mode except when DMA transfer is active from the selected drive.

In systems which may use mixed devices where totem-pole drivers are used, and

the system shares this line with other non-ATA devices, the ATA host or ATA

adapter shall ensure that appropriate protection is employed to protect ATA

device DMARQ drivers from damage.

5.2.10 INTRQ (Drive interrupt)

This signal is used to interrupt the host system.  INTRQ is asserted only when

the drive has a pending interrupt, the drive is selected, and the host has

cleared nIEN in the Device Control register.  If nIEN=1, or the drive is not

selected, this output is in a high impedance state, regardless of the presence

or absence of a pending interrupt.

INTRQ shall be negated by:

-   assertion of RESET- or

-   the setting of SRST of the Device Control register, or

-   the host writing the Command register or

-   the host reading the Status register

On PIO transfers, INTRQ is asserted at the beginning of each data block to be

transferred.  A data block is typically a single sector, except when declared

otherwise by use of the SET MULTIPLE MODE command.  An exception occurs on

FORMAT TRACK, WRITE SECTOR(S), WRITE BUFFER and WRITE LONG commands - INTRQ

shall not be asserted at the beginning of the first data block to be

transferred.

On DMA transfers, INTRQ is asserted only once, after the command has completed.

If the system shares this line with non-ATA devices, the ATA host or ATA

adapter shall ensure that appropriate protection is employed to protect ATA

device INTRQ drivers from damage.

5.2.12 IORDY (I/O channel ready) (Optional)

This signal is negated to extend the host transfer cycle of any host register

access (Read or Write) when the drive is not ready to respond to a data

transfer request.

If actively asserted, this signal shall only be enabled during DIOR-/DIOW-

cycles to the selected drive. If open collector, when IORDY is not negated, it

shall be in the high-impedance (undriven) state.

8. Command Descriptions

Commands are issued to the drive by loading the pertinent registers in the

command block with the needed parameters, and then writing the command code to

the Command register.

The manner in which a command is accepted varies.  There are three classes (see

table 9) of command acceptance, all predicated on the fact that to receive a

command, BSY=0:

Upon receipt of a Class 1 command, the drive sets BSY within 400 nsec.

Upon receipt of a Class 2 command, the drive sets BSY within 400 nsec, sets

up the sector buffer for a write operation, sets DRQ within 700 usec, and

clears BSY within 400 nsec of setting DRQ.

Upon receipt of a Class 3 command, the drive sets BSY within 400 nsec,  sets

up the sector buffer for a write operation, sets DRQ within 20 msec, and clears

BSY within 400 nsec of setting DRQ.

NOTE:  DRQ may be set so quickly on Class 2 and Class 3 that the BSY transition

is too short for BSY=1 to be recognized.

The drive shall implement all mandatory commands as identified by an M, and may

implement the optional commands identified by an O, in table 9.  V indicates a

Vendor Specific command code.

+=====-======================================-Command-=====Parameters Used====+

|Class|                                      | code  | FR | SC | SN | CY | DH |

|-----+--------------------------------------+-------+----+----+----+----+----|

|  1  | ACKNOWLEDGE MEDIA CHANGE         | O |  DBh  |    |    |    |    |  D |

|  1  | BOOT - POST-BOOT                 | O |  DCh  |    |    |    |    |  D |

|  1  | BOOT - PRE-BOOT                  | O |  DDh  |    |    |    |    |  D |

|  1  | CHECK POWER MODE                 | O |98h E5h|    |  y |    |    |  D |

|  1  | DOOR LOCK                        | O |  DEh  |    |    |    |    |  D |

|  1  | DOOR UNLOCK                      | O |  DFh  |    |    |    |    |  D |

|  3  | DOWNLOAD MICROCODE               | O |  92h  |  y |  y |  y |  y |  D |

|  1  | EXECUTE DRIVE DIAGNOSTIC         | M |  90h  |    |    |    |    |  D*|

|  2  | FORMAT TRACK                     | M |  50h  |  * |  y |    |  y |  y |

|  1  | IDENTIFY DRIVE                   | O |  ECh  |    |    |    |    |  D |

|-----+-----------------------------------------------------------------------|

|     | CY = Cylinder registers         SC = Sector Count register            |

|     | DH = Drive/Head register        SN = Sector Number register           |

|     | FR = Features register (see command descriptions for use)             |

|     | y - the register contains a valid parameter for this command.         |

|     |     For the Drive/Head register, y means both the drive and           |

|     |     head parameters are used.                                         |

|     | D - only the drive parameter is valid and not the head parameter.     |

|     | D* - Addressed to drive 0 but both drives execute it.                 |

|     | * - Maintained for compatibility (see 7.2.10)                         |

+=============================================================================+

Table 9 - Command Codes and Parameters (Part 1 of 2)

8.10.14 Word 51: PIO data transfer cycle timing mode

The PIO transfer timing for each ATA device falls into categories which have

unique parametric timing specifications.  To determine the proper device timing

category, compare the Cycle Time specified in figure 5 with the contents of

this field.  The value returned in Bits 15-8 should fall into one of the mode

0  through mode 2 categories specified in figure 5, and if it does not, then

Mode 0 shall be used to serve as the default timing.

Note:  For backwards compatibility with BIOSs written before Word 64 was

defined for advanced modes, a device reports in Word 51 the highest original

PIO mode (i.e. PIO mode 0, 1, or 2) it can support.

Word 52: Single Word DMA data transfer cycle timing mode

The DMA transfer timing for each ATA device falls into categories which have

unique parametric timing specifications.  To determine the proper device timing

category, compare the Cycle Time specified in figure 6 with the contents of

this field.  The value returned in Bits 15-8 should fall into one of the

categories specified in figure 6 (i.e. 0, 1, or 2), and if it does not, then

Mode 0 shall be used to serve as the default timing.

The contents of this word shall be ignored if Words 62 or 63 are supported.

8.10.25 Word 64: Flow Control PIO Transfer Modes Supported

Bits 7 through 0 of word 64 of the Identify Drive parameter information is

defined as the Advanced PIO Data Transfer Supported Field.  This field is bit

significant.  Any number of bits may be set in this field by the device to

indicate which Advanced PIO Modes that it is capable of supporting.

Of these bits, bits 7 through 1 are Reserved for future Advanced PIO Modes.

Bit 0, if set, indicates that the device supports PIO Mode 3.

Note:  For backwards compatibility with BIOSs written before Word 64 was

defined for advanced modes, a device reports in Word 51 the highest original

PIO mode (i.e. PIO mode 0, 1, or 2) it can support.

8.10.27 Word 66: Manufacturer's Recommended Multiword DMA Cycle Time

Word 66 of the parameter information of the IDENTIFY DRIVE command is defined

as the Manufacturer's Recommended Multiword DMA Transfer Cycle Time.  This

field defines, in nanoseconds, the minimum cycle time per word during a single

sector host transfer while performing a multiple sector READ DMA or WRITE DMA

commands over all locations on the media under nominal conditions.

If a host runs at a faster cycle rate by operating at a cycle time of less than

this value, the device may  negate DMARQ for flow control.   The rate at which

DMARQ is negated could result in reduced throughput despite the faster cycled

rate. Transfer at this rate does not ensure that flow control will not be used,

but implies that higher performance MAY result.

If this field is supported, bit 1 of word 53 must be set.

Any device which supports Multiword DMA Mode 1 or above shall support this

field, and the value in word 66 shall not be less than the value in word 65.

If bit 1 of word 53 is set because a device supports a field in Words 64-70

other than this field and the device does not support this field, the device

shall return a value of zero in this field.

8.16. READ DMA

This command executes in a similar manner to the READ SECTOR(S) command except

for the following:

the host initializes a slave-DMA channel prior to issuing the command

data transfers are qualified by DMARQ and are performed by the slave-DMA

channel

the drive issues only one interrupt per command to indicate that data transfer

has terminated and status is available.

During the DMA transfer phase of a Read DMA command, the drive shall provide

status of BSY or DRQ until the command is completed.

At command completion, the Command Block Registers contain the sector address

of the last sector transferred.

An unrecoverable error encountered during execution of a READ DMA command

results in the termination of the command.  At command completion, the Command

Block Registers contain the sector address of the sector where the first error

occurred.

8.29 WRITE DMA

This command executes in a similar manner to WRITE SECTOR(S) except for the

following:

the host initializes a slave-DMA channel prior to issuing the command

data transfers are qualified by DMARQ and are performed by the slave-DMA

channel

the drive issues only one interrupt per command to indicate that data transfer

has terminated and status is available.

During the execution of a Write DMA command, the drive shall provide status of

BSY or DRQ until the command is completed.

At command completion, the Command Block Registers contain the sector address

of the last sector transferred.

An unrecoverable error encountered during the  execution of a Write DMA command

results in the termination of the command. At command completion, the Command

Block Registers contain the sector address of the sector address of the sector

where the first error occurred.

9. Protocol

Commands can be grouped into different classes according to the protocols

followed for command execution.  The command classes with their associated

protocols are defined below.

For all commands, the host first checks if BSY=1, and should proceed no further

unless and until BSY=0.  For most commands, the host will also wait for DRDY=1

before proceeding.  Those commands shown with DRDY=x can be executed when

DRDY=0.

Data transfers may be accomplished in more ways than are described below, but

these sequences should work with all known implementations of ATA drives.

If a new command is issued to a drive which has an uncompleted command

(subsequently referred to as Old Command) in progress, the drive shall

immediately respond to the new command (Subsequently referred to as New

Command), even if execution of the Old Command could have been completed.

There shall be no indication given to the system as to the status of the Old

Command which was being executed at the time the New Command was issued.

The only way to interrupt a Read DMA or Write DMA command is with a reset.  The

result of interrupting a Read DMA or Write DMA command by writing a new command

into the Command Register is unpredictable.

9.5 DMA data transfer commands (optional)

This class comprises:

READ DMA

WRITE DMA

Data transfers using DMA commands differ in two ways from PIO transfers:

data transfers are performed using the slave-DMA channel

A Single interrupt is issued at the completion of the command.

Initiation of the DMA transfer commands is identical to the READ SECTOR(S) or

WRITE SECTOR(S) commands except that the host initializes the slave-DMA channel

prior to issuing the command.

The interrupt handler for DMA transfers is different in that no intermediate

sector interrupts are issued on multi-sector commands

a) The host writes any required parameters to the Features, Sector Count,

Sector Number, Cylinder and Drive/Head registers.

b) The Host initializes the slave-DMA channel

c) The host writes the command code to the Command register.

d) The drive sets BSY.

e) When the drive is ready to transfer data, the drive asserts DMARQ.  The DMA

data transfer may be split into several partial transfers at the discretion of

the drive or slave-DMA channel.  The drive shall assert either BSY or DRQ

during the entire DMA data transfer phase.

f) When the drive has completed processing, it clears both BSY and DRQ and

asserts INTRQ.  The host reads the Status Register.g) Host resets the slave-DMA

channel

9.5.1 Normal DMA transfer

+-a)---+-b)-------+-c)----+-d)---+-e)-----+-f)---+-g)--+

|Setup |Initialize|Issue  |      |DMA data|Status|Reset|

|      |DMA       |Command|      |transfer|      |DMA  |

+------+----------+-------+------+--------+------+-----+

|      |BSY=0     |BSY=0  |BSY=1 |BSY=1   |BSY=0 |     |

|      |          |       |      |or      |DRQ=0 |     |

|      |          |       |      |DRQ=1   |Assert|     |

|      |          |       |      |        |INTRQ |     |

9.5.2  DMA Transfer terminated by an error during data transfer.

+-a)---+-b)-------+-c)----+-d)---+-e)-----+-f)---+-g)--+

|Setup |Initialize|Issue  |      |DMA data|Status|Reset|

|      |DMA       |Command|      |transfer|      |DMA  |

+------+----------+-------+------+--------+------+-----+

|      |BSY=0     |BSY=0  |BSY=1 |BSY=1   |BSY=0 |     |

|      |          |       |      |or      |DRQ=0 |     |

|      |          |       |      |DRQ=1   |Assert|     |

|      |          |       |      |        |INTRQ |     |

9.5.3  DMA Command terminated by an error before any data is transferred.

+-a)---+-b)-------+-c)----+-d)---+-f)---+-g)--+

|Setup |Initialize|Issue  |      |Status|Reset|

|      |DMA       |Command|      |      |DMA  |

+------+----------+-------+------+------+-----+

|      |BSY=0     |BSY=0  |BSY=1 |BSY=0 |     |

|      |          |       |      |DRQ=0 |     |

|      |          |       |      |Assert|     |

|      |          |       |      |INTRQ |     |

--

John Masiewicz (John_Masiewicz@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Fri Mar 11 17:08:40 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18578; Fri, 11 Mar 94 17:08:40 PST

Received: from vnet.IBM.COM by dt.wdc.com (4.1/SMI-4.1)

id AA18571; Fri, 11 Mar 94 17:08:37 PST

Message-Id: <9403120108.AA18571@dt.wdc.com>

Received: from BCRVM1.VNET.IBM.COM by vnet.IBM.COM (IBM VM SMTP V2R2)

with BSMTP id 2501; Fri, 11 Mar 94 20:05:32 EST

Date: Fri, 11 Mar 94 20:05:31 EST

From: russb@vnet.IBM.COM

To: ata@dt.wdc.com

Subject: (U) SFF-8019.........

Sender: ata-owner@dt.wdc.com

Precedence: bulk

From: Russ Berkoff

Subject: (U) SFF-8019.........

*** Forwarding note from RUSSB   --BCRVM1   03/11/94 20:06 ***

To: SMTP    --IINUS1

From: Russ Berkoff

Subject: (U) SFF-8019.........

First let me say it is far from trivial for software to do

validation of ATA Identify data. It greatly (and unnecessarily)

complicates the design / implementation / maintenance of our

drivers. It also compromises their reliability when installed

on new hardware!

We as a purveyor of an operating system intended to be used

on equipment from multiple vendors, we do not have the luxury of

saying a particular vendor's equipment is BROKEN since they

did not meet spec. This is especially true when a major

(non-IBM) system supplier or drive supplier is involved!

And yes....Determining whether a drive implements read-write

multiple properly is a headache for us!

We currently check Word 47 Bit 15 = 1 as well as a non-zero

block count before the driver will allow multiple mode to

be switched on. This did not 'end-up' in the ATA spec but

turns out to be a common practice among drive suppliers.

The net is that the incomplete standardization among drives

causes to program to the 'lowest common denominator'. This

puts the onus back to the END USER, to KNOW that the drive

he purchased supports Multiple Mode or DMA  or Fast PIO

properly and to figure out how to switch these functions

on in the operating system or during system setup!

Hardly a satisfactory situation. Some of these problems could

have been avoided with some foresight and two bytes of length

data at the beginning of the Identify or a Spec Compliance

level as in SCSI Inquiry data!

Hindsight is wonderful. (It makes one always right!). My only objective

here is to try to sensitize the audience to some of the problems

one has in writing / maintaining a driver intended to work

and perform well across a wide range of drives!

Regards,

Russ Berkoff

OS/2 Development

IBM Corp

From ata-owner  Fri Mar 11 19:33:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA19903; Fri, 11 Mar 94 19:33:42 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA19896; Fri, 11 Mar 94 19:33:33 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id TAA06264; Fri, 11 Mar 1994 19:21:14 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 11 Mar 1994 19:12:24 EST

Date:      Fri Mar 11 19:12:22 199 PST

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2d8108e8.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   Comments on Jesup's comments

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-11-94 06:17:17 PM

_To: sff_reflector@Sun.COM@internet

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: Hale Landis@SEAGATE

_Subject: Comments on Jesup's comments

Randell Jesup of Commodore Engineering posted a response to my response to Russ

Berkoff's (IBM) message about ID data.

RJ>>>First, according to rev 4a of ATA word 47 isn't defined, so how can it

be set correctly?

HL: I'm not sure I understand this comment.  Here is the definition of ID Word

47 right out of

ATA rev 4a...

|   47  | 15-8  Vendor unique                                              |

|       |  7-0  00h = Read/write multiple commands not implemented         |

|       |       xxh = Maximum number of sectors that can be transferred    |

|       |             per interrupt on read and write multiple commands    |

RJ>>>If it's so critical to to validate fields in the identify data, why

isn't this even hinted at in the spec?  Those of us who came to the spec from

SCSI-land (and non-MSDOS-land) don't know the "history" of all these parameters

or what workarounds you have to do to be compatible with even a reasonable

fraction of the drives on the market.  We all end up learning the hard way

that what the spec says will work probably won't, even in the cases where

the spec is clear.  (A large part of the spec is less-than-clear unless you've

been working with PC disk controllers for years.)

RJ>>>Maybe ATA needs an appendix labelled "Here's how ATA _actually_ works",

or "All the incompatible ways ATA has been implemented", or "These drives

are broken in these ways".  (That last isn't really meant as a joke... it would

be very useful.  We'll never see it, since drive manufacturers wouldn't want to

publicly air their dirty laundry.)

HL: I think the ATA-1 specification is probably the worst written "standard"

document I

have even seen.  Yes, I know a lot of people worked on it for many years but

I'm sorry,

I agree with Mr. Jesup here, you can't implement devices or software using the

ATA-1 specification.

HL: Lucky for us, ATA-2 is here and many people are taking the time to produce

a much

better document.  I'm sure it still will not cover all the things that some of

us would like to see

but I'm sure it will be many orders of manigtude better than ATA-1.  But here's

a good one to

think about:

An "implementation guide for PIO Mode 3 or DMA MW Mode 1" would be

20-30 pages of detailed information on which commands to implement and in

which order to use those commands and how to interpret the ID data for PIO Mode

3

or DMA MW Mode 1, etc.  I know, I'm trying to write such a document.  I have 10

pages now and I'm only about 1/3 done!

--

Hale Landis (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Sun Mar 13 12:26:01 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA09477; Sun, 13 Mar 94 12:26:01 PST

Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA09470; Sun, 13 Mar 94 12:25:56 PST

Received: with SMTP-MR; Sun, 13 Mar 1994 12:19:09 PST

Mr-Received: by mta WDUS04; Relayed; Sun, 13 Mar 1994 12:19:09 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Sun, 13 Mar 1994 20:14:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Updated Mode 4 16.6MB/s timing

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com, SFF_REFLECTOR@SUN.COM

Message-Id: <01H9X92CNG44934XA7@wdus03.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Sun, 13 Mar 1994 20:15:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;90912131304991/280228@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

________________________________________________________________________

To:		SFF/ATA Memberships

From:		Tom Hanan, Western Digital

Date:		3/12/94

Subject:	Proposed Change of Mode 4.

Per our original agreement, I am continuing to distribute this timing

information to both memberships. The intent is to solicit as many inputs

from as many industry groups as possible. No one group has complete coverage

so this makes the best technical sense.

Hopefully we will have enough of the technical issues addressed at the next

ATA meeting so that the group can make a final decision about the inclusion

of these timings. At present the group has said that they will consider the

timings once they have seen the changes.

SFF-8033 is a place holder for this work should the ATA group determine that

the mode 4 timings are not mature enough for inclusion into the ATA-2 document.

If the ATA group votes to publish these timings in ATA-2 then SFF-8033 can be

the number used by SFF to publish the information document containing the

timings from the ATA group.

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

The minimum cycle time supported by the device in PIO Mode 3, 4 and Multiword

DMA Mode 1, 2 respectively shall always be greater than or equal to the minimum

cycle time defined by the associated Mode e.g. a drive supporting PIO Mode 4

timing shall not report a value less than 120 nanoseconds, the minimum cycle

time defined for Mode 4 PIO Timings.

6.2  New Multiword DMA Transfer Mode

The new Multiword DMA Transfer Mode is Mode 2.

6.2.1  Timing Parameters

The timing parameters associated with Multiword DMA Transfer Mode 2 are

defined in Table 6-1. Peripherals reporting support for Multiword DMA Transfer

Mode 2 shall also support Multiword DMA Transfer Mode 0 and 1.

TABLE 6-1  MULTIWORD DMA TIMING PARAMETERS

+----------------------------------------+

| Multiword DMA              |   Mode 2  |

| timing parameters          |    nsec   |

|                            | Min | Max |

+-----+----------------------------+-----+-----|

| t0  | Cycle time              *3 | 120 |     |

| tC  | DMACK to DMREQ delay       |     | --- |

| tD  | DIOR-/DIOW-      16-bit *3 |  70 |     |

| tE  | DIOR- data access          |     | --- |

| tF  | DIOR- data hold            |   5 |     |

| tF  | DIOR- data hold         *1 |     | n/a |

| tG  | DIOR-/DIOW- data setup     |  20 |     |

| tH  | DIOW- data hold            |  10 |     |

| tI  | DMACK to DIOR-/DIOW- setup |   0 |     |

| tJ  | DIOR-/DIOW- to DMACK hold  |   5 |     |

| tK  | DIOR-/DIOW- negated width*3|  25 |     |

| tLr | DIOR- to DMREQ delay       |     |  35 |

| tLw | DIOW- to DMREQ delay       |     |  35 |

| tZ  | DMACK- to tristate      *2 |     |  25 |

+----------------------------------+-----+-----+

*1 The meaning of this parameter in ATA was not clear. The parameter is not

applicable to this Specification.

*2 This parameter specifies the time from the negation edge of DMACK- to the

time the data bus is no longer driven by the device (tristate). The tZ

parameter applies only at the end of a Multiword DMA cycle, i.e. when

DMACK is negated. The device may actively drive the data bus or may

tristate the data bus while DMACK- is active from the first time that

DIOR- is asserted until DMACK- is deasserted as long as tE and tF

requirements are met.

*3 t0 is the minimum total cycle time, tD is the minimum command active

time, and tK is the minimum command recovery time or command inactive

time. The actual cycle time equals the sum of the actual command active

time and the actual command inactive time. The three timing requirements

of t0, tD, tK shall be met. The minimum total cycle time requirement, t0,

is greater than the sum of tD and tK. This means a host implementation

can lengthen either or both tD or tK to ensure that t0 is met. A device

implementation shall support any legal host implementation.

6.2.2  Timings

The timings associated with Multiword DMA Transfers are defined in Figure 6-1,

and include the new timing parameter tZ. The minimum value of t0 is specified

by word 65 in the Identify Drive parameter list (see 7.1.6). Note that the edge

used to measure T0 was changed from asserted to de-asserted in order to make

the timing diagrams more closly reflect real VLSI and system timing issues.

|<------------- t0 ------------>|

_____________________________________________            ________

DMARQ  ___/                                             \__________/

|                     |<--->|

|                     |  tL

___________________________________________________          _

DMACK- ______/                                                   \________/

|<-->|         |                     |         |

tI |<-- tD ->|<------- tK -------->|         |<-->|

|         |                     |         | tJ |

DIOR-             |_________|                     |_________     |

DIOW-  ___________/         \_____________________/         \_______________

|         |                                    |

|<-->|    |                                 -->|  |<-tZ

READ              | tE  _________                         __________

DD0-15 ------------<XXXX_________XXXXXXXXXXXXXXXXXXXXXXXXX_______x_X>-------

|    |    |

|<-->|<-->|

| tG | tF

WRITE                   __________                      ____________

DD0-15 XXXXXXXXXXXXXXXXX__________XXXXXXXXXXXXXXXXXXXXXX____________XXXXXXXX

|<-->|<--->|

tG   tH

FIGURE 6-1  MULTIWORD DMA TIMING DIAGRAM

6.3  New PIO Transfer Mode

The new PIO Transfer Mode is Mode 4.

6.3.1  Timing Parameters

The timing parameters associated with PIO Transfer Mode 4 are defined in Table

6-2. Peripherals reporting support for PIO Transfer Mode 4 shall power up in a

PIO Transfer Mode compatible with ATA.

TABLE 6-2  PIO TIMING PARAMETERS

+------------------------------------------------------+

| PIO                                      |  Mode 4   |

| timing parameters                        |   nsec    |

|                                          | Min | Max |

+----+------------------------------------------+-----+-----+

| t0 | Cycle time                            *4 | 120 |     |

| t1 | Address valid to DIOR-/DIOW- setup       |  25 |     |

| t2 | DIOR-/DIOW-      16-bit               *4 |  70 |     |

|    |   Pulse width     8-bit               *4 |  70 |     |

| t2i| DIOR-/DIOW- recovery time             *4 |  25 |     |

| t3 | DIOW- data setup                         |  20 |     |

| t4 | DIOW- data hold                          |  10 |     |

| t5 | DIOR- data setup                         |  20 |     |

| t6 | DIOR- data hold                          |   5 |     |

| t6 | DIOR- data hold                       *1 |     | n/a |

| t6Z| DIOR- data tristate                   *2 |     |  30 |

| t7 | Addr valid to IOCS16- assertion          |     |  30 |

| t8 | Addr invalid to IOCS16- Release          |     |  25 |

| t9 | DIOR-/DIOW- to address valid hold        |   5 |     |

| tA | IORDY  Setup time                     *3 |  25 |     |

| tB | IORDY  Pulse Width                       |     |1,250|

| tRD| Read Data Valid to IORDY active          |   0 |     |

|    |  (if IORDY initially low after tA)       |     |     |

+-----------------------------------------------------------+

*1 The meaning of this parameter in ATA was not clear. The parameter is not

applicable to this Specification.

*2 This parameter specifies the time from the negation edge of DIOR- to the

time that the data bus is no longer driven by the device (tristate).

*3 The delay from DIOR- or DIOW- until the state of IORDY is first sampled.

If IORDY is inactive then the host shall wait until IORDY is active before

the PIO cycle can be completed. If the device is not driving IORDY negated

at the time tA after the activation of DIOR- or DIOW-, then t5 shall be

met and tRD is not applicable. If the device is driving IORDY negated at

the time tA after the activation of DIOR- or DIOW-, then tRD shall be met

and t5 is not applicable.

*4 t0 is the minimum total cycle time, t2 is the minimum command active

time, and t2i is the minimum command recovery time or command inactive

time. The actual cycle time equals the sum of the actual command active

time and the actual command inactive time. The three timing requirements

of t0, t2, t2i shall be met. The minimum total cycle time requirement, t0,

is greater than the sum of t2 and t2i. This means a host implementation

can lengthen either or both t2 or t2i to ensure that t0 is equal to the

value reported in the devices identify drive data. A device implementation

shall support any legal host implementation.

6.3.2  Timings

The timings associated with PIO Mode 4 transfers are defined in Figure 6-2,

and include the new timing parameters t2i, t6Z and tR. The minimum value of t0

is specified by word 68 in the Identify Drive parameter list (see 7.1.8).

Note that the edge used to measure T0 was changed from asserted to

de-asserted in order to make the timing diagrams more closly reflect real

VLSI and system timing issues.

It is mandatory that IORDY be supported when PIO Mode 3 or 4 are the current

mode of operation.

|<--------- t0 ----------------------->|

|    ________________________________________

Addr Valid *1 ______/                                        \_________

|   |<-t1->|                         ->| t9  |<-

|   |      |<-------- t2 ------------->|   ->|t8|<-

__    |      |___________________________|<---t2i---->|__

DIOR-/DIOW-     \__________/                           \____________/

|      |                           |        |

WRITE               |      |                 _____________      |

DD0-15         *2---------------------------<_____________>------------

|      |                 |<---t3-->|  |     |

|      |                         ->|t4|<-   |

READ                |      |                  _____________ ____|

DD0-15         *2----------------------------<_____________X____>------

|      |                 |<---t5-->|   |    |

->|t7|<- |                 |       ->|t6 |<-  |

|  | ->| tA |<-          |         |<-t6Z-->|

|  |________________________________________

IOCS16-         _______/        |            |                  \______

|          ->|tRD|<-

________________|______________________________________

IORDY          XXXXXXXXXXXXXXXXX________________//

FIGURE 6-2  PIO TIMING DIAGRAM

7. Command and Parameter Changes

Changes have been made to the SFF8011 Identify Drive and Set Feature commands.

7.1.4  Word 64: Flow Control PIO Transfer Modes Supported

Word 64 Bits 7-0 is a bit significant field. Any number of bits may be set

in this field by the device to indicate which Advanced PIO Modes it is

capable of supporting. Bits 7-2 are reserved for future use.

- If Bit 0=1, the device supports PIO Mode 3.

- If Bit 1=1, the device supports PIO Mode 4.

NOTE: For backwards compatibility with BIOSs written before Word 64 was

defined for advanced modes, a device reports in Word 51 the highest original

PIO mode it can support.

7.1.5  Word 65: Minimum Multiword DMA Transfer Cycle Time Per Word

Word 65 defines the minimum cycle time that the device can support when

performing Multiword DMA transfers on a per word basis.

If this field is supported, Word 53 Bit 1=1 shall be set.

Any device which supports Multiword DMA Mode 1 or above shall support this

field, and the value in word 65 shall not be less than the minimum cycle

time reported by the fastest DMA mode supported by the device.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other than

this field, the device shall return a value of zero in this field.

7.1.7  Word 67: Minimum PIO Transfer Cycle Time Without Flow Control

Word 67 defines the minimum cycle time that the device guarantees shall not

cause data integrity degradation during a transfer which does not utilize flow

control.

Any device may support this field, and if this field is supported, bit 1 of

word 53 shall be set.

Any device which supports PIO Mode 3 or above shall support this field, and

the value in Word 67 shall not be less than the value reproted in word 68.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other than

this field, the device shall return a value of zero in this field.

7.1.8  Word 68: Minimum PIO Transfer Cycle Time With IORDY Flow Control

Word 68 defines the minimum cycle time that the device can support when

performing data transfers while utilizing IORDY flow control.

Any device may support this field, and if this field is supported, bit 1 of

word 53 shall be set.

Any device which supports PIO Mode 3 or above shall support this field, and

the value in word 68 shall not be less than the fastest PIO mode reported

by the device.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other than

this field, the device shall return a value of zero in this field.

From ata-owner  Mon Mar 14 16:24:20 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23711; Mon, 14 Mar 94 16:24:20 PST

Received: from ptltd.com (ns.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA23704; Mon, 14 Mar 94 16:24:12 PST

Received: from camb1.ptltd.com by ptltd.com (4.1/ (Hub configuration V0.2-2/28/92))

id AA06339; Mon, 14 Mar 94 19:24:08 EST

Message-Id: <9403150024.AA06339@ptltd.com>

Received: by camb1.ptltd.com with VINES ; Mon, 14 Mar 94 19:25:01 EST

Date: Mon, 14 Mar 94 19:21:14 EST

From: Curtis=Stevens%Eng.corp%PTLIRVINE@ptltd.com

Subject: Phoenix Enhanced Disk Drive (EDD) Specification

To: ATA@dt.wdc.com, sff_reflector@sun.com

Cc:

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Here is a copy of the Phoenix Enhanced Disk Drive Specification in ASCII

form.  This is the method which Phoenix has chosen to accomplish the

following goals:

1. Solve the 1024 Cylinder Problem

2. Allow for expansion beyond 4 drives

3. Communicate BIOS configuration information to OS/Application

software.

Please feel free to E-MAIL Curtis_Stevens@PTLTD.COM if you have any

questions.  There is a Word for Windows 6.0/2.0 source copy of this document

on the Western Digital "Fission.DT.WDC.COM" FTP site under /PUB/ATA/E-IDE.

Curtis E. Stevens [curtis_stevens@ptltd.com] (714) 440-8330

*****  ATTACHMENT: disk1.asc *****

Enhanced Disk Drive Specification

Version 1.0

January 25, 1994

Phoenix Technologies Ltd.

THIS SPECIFICATION IS MADE AVAILABLE WITHOUT CHARGE FOR USE IN

DEVELOPING COMPUTER SYSTEMS AND DISK DRIVES.  PHOENIX MAKES NO

REPRESENTATION OR WARRANTY REGARDING THIS SPECIFICATION OR ANY

ITEM DEVELOPED BASED ON THIS SPECIFICATION, AND PHOENIX DISCLAIMS

ALL EXPRESS AND IMPLIED WARRANTIES, INCLUDING BUT NOT LIMITED TO

THE IMPLIED WARRANTIES OF MERCHANTABILITY, FITNESS FOR A

PARTICULAR PURPOSE AND FREEDOM FROM INFRINGEMENT.  WITHOUT

LIMITING THE GENERALITY OF THE FOREGOING, PHOENIX MAKES NO

WARRANTY OF ANY KIND THAT ANY ITEM DEVELOPED BASED ON THIS

SPECIFICATION WILL NOT INFRINGE ANY COPYRIGHT, PATENT, TRADE

SECRET OR OTHER INTELLECTUAL PROPERTY RIGHT OF ANY PERSON OR

ENTITY IN ANY COUNTRY.  USE OF THIS SPECIFICATION FOR ANY PURPOSE IS

AT THE RISK OF THE PERSON OR ENTITY USING IT.

version 1.0  Copyright * 1994 Phoenix Technologies Ltd.  All Rights

Reserved.

--------------------------------------------------------------------------

1.0 	Overview

The Phoenix Technologies Enhanced Disk Drive Support Specification is a

comprehensive solution to BIOS issues that result from recent advances in

the disk-drive industry.  The purpose of this specification is to provide a

downward compatible method for expanding the current Fixed Disk Parameter

Tables, as well as a new method of accessing these tables.  This new

specification not only provides for the unlimited expansion of systems

beyond two drives, but it also provides information regarding

IDE-compatible features enabled by the BIOS for OS and application

software.

1.1 	Scope

This paper assumes that the reader is familiar with the BIOS INT 13

interface, the usage of the Fixed Disk Parameter table, and the basic

operation of IDE devices.  This document describes in detail extensions to

the BIOS and extensions to the data maintained by the BIOS.

1.2 	Introduction

The disk-drive industry has increased the capacity and functionality of the

IDE-compatible disk drive, finally surpassing the capability of the BIOS to

service these new capabilities.  At present we must solve the following

BIOS specific problems:

1. The BIOS must now support drives with a capacity greater than 528MB.  At

the present time

the INT 13 interface has a limit of 1024 cylinders.

2. The INT 13 interface allows more than two drives to be attached to a

system but has no

consistent method for storing the additional parameters.

3. New, CHS-independent methods for accessing the drives have now been

defined.  These are

drive-geometry independent and require a different method of data

representation and operation.

4. New methods of data transfer are now supported.  Drives now support

additional DMA modes

as well as multi-sector data transfers and Fast PIO.  None of this

information is currently

available to OS or application software.

5. Systems require more than two disk drives, and with this requirement

comes the requirement to

assign the order in which the drives are to be accessed.

This document is divided into three sections: Parameter Tables, BIOS

Extensions, and compatibility Issues.  Parameter Tables describes the

information in the Extended Parameter Table area.  BIOS Extensions

describes the new functions required for accessing the parameter tables and

the extensions.  Compatibility Issues discusses some of the topics related

to implementing the structures in this document.

2.0 	Parameter Tables

This section provides a comprehensive description of the data which the

BIOS makes available to OS and application software.  The current

industry-standard INT 13 interface is limited to 1024 cylinders, resulting

in the limitation of OS and application software as well.  The INT 13

interface operates directly from the Fixed Disk Parameter Table and

therefore limits the size of the cylinder field in this table.  There is a

compatible version of this table which has been embraced by some OS

developers, that allows the BIOS to change the way the geometry is

represented and then to translate the changed ("logical") geometry back to

actual ("physical") geometry when a disk access is performed.  This table

format is identified by a signature byte (A0) which flags the system that

some form of translation is taking place.  Phoenix has chosen to use this

table format for representing drives with more than 1024 cylinders.

2.1 	Parameter Table Extensions

It has become necessary for the BIOS to store information representing the

type of translation currently in operation, as well as any information

relating to the current operation of the drive.  One purpose of this

specification is to define a standard format for this extension area and to

document its fields and functions so that other OS/Application software can

easily use this information.  This new information, called the "Fixed Disk

Parameter Table Extension" is 16 bytes long and is accessed via the

extended INT 13 functions described in this document.  Please see table 1

for a layout of the Fixed Disk Parameter Table with its Extension.  The

remainder of this section describes the fields in this extension.

Table 1 - Fixed Disk Parameter

Standard Table

Byte	Type	Description

-----------------------------------------------------------------

0-1	Word	Physical Number of Cylinders

2	Byte	Physical Number of Heads

3	Byte	Reserved

4	Byte	Reserved

5-6	Word	Pre Compensation

7	Byte	Reserved

8	Byte	Drive Control Byte

9-10	Word	Reserved

11	Byte	Reserved

12-13	Word	Landing Zone

14	Byte	Physical Sectors per Track

15	Byte	Reserved

Translated Table

Byte	Type	Description

-----------------------------------------------------------------

0-1	Word	Logical Number of Cylinders (Limit 1024)

2	Byte	Logical Number of Heads

3	Byte	A0 signature indicating translated table

4	Byte	Physical Sectors per Track

5-6	Word	Pre compensation

7	Byte	Reserved

8	Byte	Drive Control Byte

9-10	Word	Physical Number of Cylinders

11	Byte	Physical Number of Heads

12-13	Word	Landing Zone

14	Byte	Logical Sectors per Track

15	Byte	Checksum

Fixed Disk Parameter Table Extension

Byte	Type	Description

-----------------------------------------------------------------

0-1	Word	I/O port base Address

2-3	Word	Control port Address

4	Bit 0-3	Reserved, must be 0

Bit 4	Master/Slave Bit (0=Master)

Bit 5	Reserved, must be 1

Bit 6	LBA Enable (1 = Enabled)

Bit 7	Reserved, must be 1

5	Bits 0-3	Phoenix Proprietary

Bits 4-7	Reserved, must be 0

6	Bits 0-3	IRQ for this drive

Bits 4-7	Reserved, must be 0

7	Byte	Sector count for multi-sector transfers

8	Bits 0-3	DMA Channel

Bits 4-7	DMA Type

9	Bits 0-3	PIO Type

Bits 4-7	Reserved, must be 0

10-11	Bit 0	Fast PIO accessing enabled

Bit 1	DMA accessing enabled

Bit 2	Block PIO accessing enabled

Bit 3	CHS translation enabled

Bit 4	LBA translation enabled

Bit 5	Removable Media

Bit 6	CD ROM

Bit 7	32 bit transfer mode

Bits 8-15	Reserved, must be 0

12-13	Byte	Reserved, must be 0

14	Byte	10h, Current revision level of this extension

15	Byte	Checksum, 2's complement of the sum of bytes 0-14

2.2 	Bytes 0-1	I/O Port Base

This word is the physical base address used for accessing the drive.  It is

assumed that any application which provides a proprietary interface to the disk drive will

use this base address for addressing the drive.

2.3 	Bytes 2-3	Control Port Base

This word is the physical address of the disk-drive control port.  It is

assumed that applications which provide a proprietary interface to the disk will use

this address.

2.4 	Byte 4	Head Prefix

This is the upper nibble of the byte used for selecting the head.  This

byte is logically ORed with the head each time the disk is addressed.  The Master/Slave bit

and the LBA addressing bit are preset, which makes the use of these functions

transparent to any software using this extension.

2.5 	Byte 5	Phoenix Proprietary Nibble

This nibble is used by the BIOS and should be ignored.

2.6 	Byte 6	IRQ

Each IDE channel requires an interrupt.  This enables applications to

access the interrupts for additional IDE channels.

2.7 	Byte 7	Sector Count

If the BIOS has configured the drive for multi-sector transfers, this field

contains the block size of the transfer, in sectors, used by the BIOS.

2.8 	Byte 8	DMA Channel/DMA Type

If the BIOS has configured the system to perform multi-word DMA data

transfers in place of the normal PIO transfers, this field specifies the DMA mode in the lower

nibble (currently 0 or 1), as per the SFF definition, and the DMA Channel in the

upper nibble.  Note that the DMA Type field does not follow the format of the data

returned by the drive.

2.9 	Byte 9	PIO Type

If the BIOS has configured the system to perform Fast PIO data transfers in

place of normal PIO data transfers, this field specifies the PIO mode as

per the SFF definition (currently 1, 2 or 3).

2.10 	Byte 10-11	Disk Drive Options

This byte has a bit for each of the currently defined drive enhancements.

If this byte is 0, the drive is a "standard" drive with less than 1024

cylinders using normal PIO-type data transfers.

2.10.1 	Bit 0	Fast PIO

If the system is configured for Fast PIO, this bit is set to 1 and

byte 9 (PIO Type) should be used to configure the system before

each transfer request.  If this bit is 0, the PIO-Type field should

be ignored, regardless of any data it may contain.

2.10.2 	Bit 1	Fast DMA

If the system is configured for Fast DMA, this bit is set to 1 and

byte 8 (DMA Channel/DMA Type) should be used to configure

the system before each transfer request.  If this bit is 0, the DMA

Channel/DMA Type field should be ignored, regardless of any

data it may contain.

2.10.3 	Bit 2	Block PIO

If the system is configured for multi-sector transfers, this bit is set

to 1 and byte 7 (Sector Count) will specify the number of sectors

used for each data transfer.  If Block PIO is disabled, the Sector

Count should be ignored, regardless of the data it may contain.

2.10.4 	Bit 3	CHS Translation

If the disk-drive has more than 1024 cylinders, this bit is set to 1.

CHS translation translates from "Logical" to "Physical" drive

geometry.

2.10.5 	Bit 4	LBA Translation

If the system is configured for LBA type addressing, this bit is set

to 1.  When LBA translation is on, the INT 13 interface will

convert all CHS-type accessing to LBA-type accessing.  LBA-

type addressing is available on drives with less than 1024

cylinders, and therefore it should not be assumed that bit 3 (CHS

translation) will always be enabled whenever this bit is enabled.

2.10.6	Bit 5	Removable Media

If the disk drive supports removable media, this bit is set to 1.

2.10.7 	Bit 6	CD-ROM

If this IDE device is a CD-ROM, this bit is set to 1.

2.10.8 	Bit 7	32 Bit Transfer Mode

If the BIOS has configured the system to perform 32 bit wide data

transfers, this bit is set to 1.

2.11 	Byte 14	Extension Revision

This is set to 10 and represents the revision level of this extension as a

BCD number where the first digit is the major revision number and the second digit is the

minor revision number.  In the event that more information needs to be added to the

Extension, this revision number should be changed.

2.12 	Byte 15	Checksum

This is the two's complement of the sum of bytes 0 through 14.  Adding

bytes 0 through 15 should in all cases produce an 8 bit result of 0.

3.0 	BIOS Extensions

If compatibility with current and future software is to be maintained, the

BIOS must provide several extended INT 13 functions to deal with the changes in system architecture

that allow for more than two drives to be supported as well as making the

Fixed Disk Parameter Table Extension available to application and OS software.  The

major purpose of these INT 13 extensions is to remove the current requirement of using

INT pointers to point at the Parameter Table information and to give the BIOS better

control over how this data is manipulated.  Phoenix has chosen to integrate this information with

the INT 13 Extensions adopted by both IBM and Microsoft.  The revision of these

extensions at the time of this writing is 1.1, only the changes to these functions are

documented in this

section.

3.1 	Check Extensions Present

INT 13h, Function 41h

Registers at call:

AH = 41h

BX = 55AAh

DL = Drive (80h - FFh)

Registers on return:

CF - Set if function not available

AH - Major version of extensions (20h)

AL - Internal Use Only

BX - AA55h

CX - API subset support bitmap

DH - Extension Version

The Major Version is upgraded to 2.0 which allows applications to know that

the Extended Drive Parameter Tables are available.  Bit 2 of CX indicates that

the Extended Drive Parameter Table for the specified drive is valid.  Bits 3-15 are

reserved and should

be zero.

3.2 	Get Drive Parameters INT 13h, Function 48h

Registers at call:

AH = 48h

DL = Drive

DS:SI = Result Buffer Address

Registers on return:

CF - Set if function not available or error

AH = Error Code

CF - Clear if success

AH = 0

DS:SI = Following Structure

info_size	dw 30	; Size of this buffer

flags		dw ?	; Information Flags

cylinders	dd ?	; Number of cylinders

heads		dd ?	; Number of heads

sec_per_track	dd ?	; Number of sectors per track

sectors		dq ?	; Number of sectors on the disk

sector_size	dw ?	; Bytes per sector

extended_table	dd ?	;Pointer to configuration parameters

The extended_table is a DWORD pointer of the form SEG:OFFSET which points

to the Fixed Disk Parameter Table Extension as described in section 2 (diagrammed

in table 1) of this document.  If this field is FFFF:FFFF then the address is

considered to be invalid.  All other fields operate as described in revision 1.1 of the extensions.

4.0 	Compatibility Issues

Adding new features to the BIOS, or changing the functionality of an

existing BIOS, will always impact software performance.  The purpose of this section is to

discuss how some of the anticipated issues are to be handled.

4.1	INT 41/46

INT 41 and INT 46 are platform specific issues.  When a platform supports

INT 41/46, the BIOS will use INT 41 to point to drive 80, and INT 46 to point to drive 81.

The BIOS maintains pointers to drives greater than 81 internally.  If the platform

does not support INT 41/46, the BIOS maintains all pointers internally.  In either case, INT

13 Function 48 always returns the specified drive information.

4.2 	Disk Drive Mapping

When a system has more than 1 drive, the person configuring the system

needs a way to set the relationship between the BIOS references (80, 81, 82, ...) and the

physical drive.  The Fixed Disk Parameter Table Extension easily allows this capability because

it stores the physical port addresses and control bits used for setting

Master/Slave as well as LBA accessing.  One side effect of this implementation is that any drive can be

the boot drive, and slave drives may be at any location.  The following are legal

configurations:

Standard Configuration:

80 - Primary Master		[INT 41]

81 - Primary Slave		[INT 46]

Alternate Configurations:

80 - Primary Master		[INT 41]

81 - Secondary Master		[INT 46]

82 - Secondary Slave

80 - Secondary Master		[INT 41]

81 - Primary Master		[INT 46]

82 - CD (Secondary Slave)

80 - Secondary Slave		[INT 41]

81 - Secondary Master		[INT 46]

82 - Primary Master

From ata-owner  Tue Mar 15 09:13:05 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02524; Tue, 15 Mar 94 09:13:05 PST

Received: from gatekeeper.mcimail.com by dt.wdc.com (4.1/SMI-4.1)

id AA02495; Tue, 15 Mar 94 09:12:53 PST

Received: by gatekeeper.mcimail.com (5.65/fma-120691);

id AA25775; Tue, 15 Mar 94 11:13:03 -0600

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id ar09657;

15 Mar 94 17:05 GMT

Date: Tue, 15 Mar 94 12:09 EST

From: STEVE ANDERSON <STEVE_ANDERSON+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ata <ata@dt.wdc.com>

Subject: SFF-8033 cycle timing notes

Message-Id: <05940315170950/0003946488NA5EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Where are are the cycle timing notes to refer the reader

to the ID command to get the real value of t0 that have been

agreed to in the last two meeting and keep getting

omitted from the document.

Could we please have a retransmit of the document with those

timing notes.

Thanks

Steve

From ata-owner  Wed Mar 16 07:31:35 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15040; Wed, 16 Mar 94 07:31:35 PST

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA15033; Wed, 16 Mar 94 07:31:25 PST

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.4/8.6.4) with SMTP id HAA24326; Wed, 16 Mar 1994 07:25:47 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 16 Mar 94 07:23:29 PST

Date: 16 Mar 94 07:12:46 PST

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: Meeting Announcement:  X3T10 ATA Working Group

To: "scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"64215161304991/243802@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

There will be an X3T10 ATA Working Group meeting on Wednesday, March 30th.

The meeting will start at 9:30 AM and end before 4:00 PM.

The meeting will be hosted by Adaptec at its facilities (building 4).  The

address is 500 Yosemite Drive, Milpitas.

To get there from the Crown Sterling Suites (where many of our ATA WG

meetings have been held), go West on Calavaras (HWY 237) to S. Milpitas

Blvd. and turn left (South).  If you are coming East on Calavaras (HWY

237), turn right on S. Milpitas Blvd, which is located just after you go

over the railroad overpass bridge.

Once on S. Milpitas Blvd., proceed to the second light, which is Yosemite

Drive.  The build is on the far right corner of the intersection.  Turn

right on Yosemite and left into the parking lot.

==========================================================

Steve Finch, Chair ATA WG.

Silicon Systems

Ph:  714 573-6808, Fx:  714 573-6916

email: 5723283@mcimail.com

==========================================================

From ata-owner  Thu Mar 17 07:41:54 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28546; Thu, 17 Mar 94 07:41:54 PST

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA28539; Thu, 17 Mar 94 07:41:42 PST

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.200.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id HAA29223; Thu, 17 Mar 1994 07:35:31 -0800

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 17 Mar 94 07:28:43 PST

Date: 17 Mar 94 07:28:10 PST

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: 16.6 MB timing, updated document

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>,

"atapi@dt.wdc.com" <atapi@dt.wdc.com@hp427u.tus.ssi1.com>,

"SFF_Reflector@sun.com" <SFF_Reflector@sun.com@hp427u.tus.ssi1.com>

Message-Id: <"01825171304991/244413@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Date:     March 16, 1994

From:     Tom Hanan, Western Digital

Updated by: Steve Finch for presentation to X3T10 Plenary

TO:       X3T10 ATA Working Group and SFF ATAPI Project Group

Subject:  Updated Mode 4 16.6 MB/s timing for ATA-2

________________________________________________________________________

Per our original agreement, I am continuing to distribute this timing

information to both memberships. The intent is to solicit as many inputs

from as many industry groups as possible.

This information was updated at the ATA Working Group meeting on Monday,

March 14.  The ATA Working Group feels this information is technically

complete, and should be included in the proposed ATA-2 standard.

The timing information contained in this document has been stable for

approximately two months, and has been agreed to by the ATA Working

Group.  Minor changes have been made to the text and timing charts for

clarification.

------------------------------------------------------------------------

---

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END -----------------------------------

---

============================================

Steve Finch, Chair ATA Working Group

Silicon Systems, Inc.

Ph: 714 573-6808, Fx: 714 573-6916

email:  5723283@mcimail.com

============================================

The minimum cycle time supported by the device in PIO Mode 3, 4 and

Multiword DMA Mode 1, 2 respectively shall always be greater than or

equal to the minimum cycle time defined by the associated Mode e.g. a

drive supporting PIO Mode 4 timing shall not report a value less than

120 nanoseconds, the minimum cycle time defined for Mode 4 PIO Timings.

x.1  New Multiword DMA Transfer Mode

The new Multiword DMA Transfer Mode is Mode 2.

x.1.1  Timing Parameters

The timing parameters associated with Multiword DMA Transfer Mode 2 are

defined in Table x-1. Peripherals reporting support for Multiword DMA

Transfer Mode 2 shall also support Multiword DMA Transfer Mode 0 and 1.

TABLE x-1  MULTIWORD DMA TIMING PARAMETERS

+----------------------------------------+

| Multiword DMA              |   Mode 2  |

| timing parameters          |    nsec   |

|                            | Min | Max |

+-----+----------------------------+-----+-----|

| t0  | Cycle time              *3 | 120 |     |

| tC  | DMACK to DMREQ delay       |     | --- |

| tD  | DIOR-/DIOW-      16-bit *3 |  70 |     |

| tE  | DIOR- data access          |     | --- |

| tF  | DIOR- data hold            |   5 |     |

| tF  | DIOR- data hold         *1 |     | n/a |

| tGr | DIOR- data setup           |  20 |     |

| tGw | DIOW- data setup           |  20 |     |

| tH  | DIOW- data hold            |  10 |     |

| tI  | DMACK to DIOR-/DIOW- setup |   0 |     |

| tJ  | DIOR-/DIOW- to DMACK hold  |   5 |     |

| tK  | DIOR-/DIOW- negated width*3|  25 |     |

| tLr | DIOR- to DMREQ delay       |     |  35 |

| tLw | DIOW- to DMREQ delay       |     |  35 |

| tZ  | DMACK- to tristate      *2 |     |  25 |

+----------------------------------+-----+-----+

*1 The meaning of this parameter in ATA was not clear. The parameter is

not applicable to this Specification.

*2 This parameter specifies the time from the negation edge of DMACK-

to the time the data bus is no longer driven by the device (tristate).

The tZ parameter applies only at the end of a Multiword DMA cycle, i.e.

when DMACK is negated. The device may actively drive the data bus or may

tristate the data bus while DMACK- is active from the first time that

DIOR- is asserted until DMACK- is deasserted as long as tE and tF

requirements are met.

*3 t0 is the minimum total cycle time, tD is the minimum command active

time, and tK is the minimum command recovery time or command inactive

time. The actual cycle time equals the sum of the actual command active

time and the actual command inactive time. The three timing requirements

of t0, tD, tK shall be met. The minimum total cycle time requirement,

t0, is greater than the sum of tD and tK. This means a host

implementation can lengthen either or both tD or tK to ensure that t0 is

met. A device implementation shall support any legal host

implementation.

x.1.2  Timings

The timings associated with Multiword DMA Transfers are defined in

Figure x-1, and include the new timing parameter tZ. The minimum value

of t0 is specified by word 65 in the Identify Drive parameter list. Note

that the edge used to measure T0 was changed from asserted to de-

asserted in order to make the timing diagrams more closely reflect real

VLSI and system timing issues.

|<------------- t0 ------------>|

_____________________________________________    |

________

DMARQ  ___/                                             \__________/

|                     |<--->|   |

|                     |  tL     |

___________________________________________________

_

DMACK- ______/

\________/

|<-->|         |                     |         |

tI |<-- tD ->|<------- tK -------->|         |<-->|

|         |                     |         | tJ |

DIOR-             |_________|                     |_________|    |

DIOW-  ___________/         \_____________________/

\_______________

|                                    |

|                                 -->|  |<-

tZ

READ                    _________                         _______

DD0-15 ------------<XXXX_________XXXXXXXXXXXXXXXXXXXXXXXXX_______XXX>--

-----

|    |    |

|<-->|<-->|

|tGr | tF

WRITE                 ____________                      ____________

DD0-15

XXXXXXXXXXXXXXX____________XXXXXXXXXXXXXXXXXXXXXX____________XXXXXXXX

|<---->|<--->|

tGw    tH

FIGURE x-1  MULTIWORD DMA TIMING DIAGRAM

x.2  New PIO Transfer Mode

The new PIO Transfer Mode is Mode 4.

x.2.1  Timing Parameters

The timing parameters associated with PIO Transfer Mode 4 are defined in

Table   x-2.  Peripherals reporting support for PIO Transfer Mode 4

shall power up in a PIO Transfer Mode compatible with ATA.

TABLE x-2  PIO TIMING PARAMETERS

+------------------------------------------------------+

| PIO                                      |  Mode 4   |

| timing parameters                        |   nsec    |

|                                          | Min | Max |

+----+------------------------------------------+-----+-----+

| t0 | Cycle time                            *4 | 120 |     |

| t1 | Address valid to DIOR-/DIOW- setup       |  25 |     |

| t2 | DIOR-/DIOW-      16-bit               *4 |  70 |     |

|    |   Pulse width     8-bit               *4 |  70 |     |

| t2i| DIOR-/DIOW- recovery time             *4 |  25 |     |

| t3 | DIOW- data setup                         |  20 |     |

| t4 | DIOW- data hold                          |  10 |     |

| t5 | DIOR- data setup                         |  20 |     |

| t6 | DIOR- data hold                          |   5 |     |

| t6 | DIOR- data hold                       *1 |     | n/a |

| t6Z| DIOR- data tristate                   *2 |     |  30 |

| t7 | Addr valid to IOCS16- assertion          |     |  30 |

| t8 | Addr invalid to IOCS16- Release          |     |  25 |

| t9 | DIOR-/DIOW- to address valid hold        |   5 |     |

| tA | IORDY  Setup time                     *3 |  25 |     |

| tB | IORDY  Pulse Width                       |     |1,250|

| tRD| Read Data Valid to IORDY active          |   0 |     |

|    |  (if IORDY initially low after tA)       |     |     |

+-----------------------------------------------------------+

*1 The meaning of this parameter in ATA was not clear. The parameter is

not applicable to this Specification.

*2 This parameter specifies the time from the negation edge of DIOR- to

the time that the data bus is no longer driven by the device (tristate).

*3 The delay from DIOR- or DIOW- until the state of IORDY is first

sampled. If IORDY is inactive then the host shall wait until IORDY is

active before the PIO cycle can be completed. If the device is not

driving IORDY negated at the time tA after the activation of DIOR- or

DIOW-, then t5 shall be met and tRD is not applicable. If the device is

driving IORDY negated at the time tA after the activation of DIOR- or

DIOW-, then tRD shall be met and t5 is not applicable.

*4 t0 is the minimum total cycle time, t2 is the minimum command active

time, and t2i is the minimum command recovery time or command inactive

time. The actual cycle time equals the sum of the actual command active

time and the actual command inactive time. The three timing requirements

of t0, t2, t2i shall be met. The minimum total cycle time requirement,

t0, is greater than the sum of t2 and t2i. This means a host

implementation can lengthen either or both t2 or t2i to ensure that t0

is equal to the value reported in the devices identify drive data. A

device implementation shall support any legal host implementation.

x.2.2  Timings

The timings associated with PIO Mode 4 transfers are defined in Figure

x-2, and include the new timing parameters t2i, t6Z and tR. The minimum

value of t0 is specified by word 68 in the Identify Drive parameter list

(see 7.1.8).  Note that the edge used to measure T0 was changed from

asserted to de-asserted in order to make the timing diagrams more

closely reflect real VLSI and system timing issues.

It is mandatory that IORDY be supported when PIO Mode 3 or 4 are the

current mode of operation.

|<--------- t0 ----------------------->|

|    ________________________________________

Addr Valid *1 ______/                                        \_________

|   |<-t1->|                         ->| t9  |<-

|   |      |<-------- t2 ------------->|   ->|t8|<-

__    |      |___________________________|<---t2i---->|__

DIOR-/DIOW-     \__________/                           \____________/

|      |                           |        |

WRITE               |      |                 _____________      |

DD0-15         *2---------------------------<_____________>------------

|      |                 |<---t3-->|  |     |

|      |                         ->|t4|<-   |

READ                |      |                  ______________    |

DD0-15         *2----------------------------<______________XXXX>------

|      |                 |<---t5-->|    |   |

->|t7|<- |                 |       ->| t6 |<- |

|  |   |                 |         |<-t6Z-->|

|  |________________________________________

IOCS16-         _______/   |                 |                  \______

|                 |

___________|_________________|_________________________

IORDY (1-1)                |                 |

|<--tA-->|        |

___________|________|________|_________________________

IORDY (1-2)                 \\\___//         |

|        |      ->|tRD|<-

___________|________|________|___|_____________________

IORDY (1-3)                 \\\__________________/

FIGURE x-2  PIO TIMING DIAGRAM

Notes:

(1)  The deassertion of IORDY by the device is used to extend the PIO

cycle.  The determination of whether the cycle is to be extended is made

by the host after tA from the assertion of DIOR- or DIOW-.  The

assertion and deassertion of IORDY are described in the following three

cases:

(1-1)  Device never deasserts IORDY: no wait is generated.

(1-2)  Device starts to drive IORDY low before tA, but causes

IORDY to be asserted before tA: no wait generated.

(1-3)  Device drives IORDY low before tA:  wait generated.  The

cycle completes after IORDY is reasserted.  For cycles where a

wait is generated and DIOR- is asserted, device must place read

data on DD0-15 for tRD before causing IORDY to be asserted.

y. Command and Parameter Changes

Changes to the Identify Drive Command

y.1  Word 64: Flow Control PIO Transfer Modes Supported

Word 64 Bits 7-0 is a bit significant field. Any number of bits may be set

in this

field by the device to indicate which Advanced PIO Modes it is capable of

supporting. Bits 7-2 are reserved for future use.

- If Bit 0=1, the device supports PIO Mode 3.

- If Bit 1=1, the device supports PIO Mode 4.

NOTE: For backwards compatibility with BIOSs written before Word 64 was

defined

for advanced modes, a device reports in Word 51 the highest original PIO

mode it

can support.

y.2  Word 65: Minimum Multiword DMA Transfer Cycle Time Per Word

Word 65 defines the minimum cycle time that the device can support when

performing

Multiword DMA transfers on a per word basis.

If this field is supported, Word 53 Bit 1=1 shall be set.

Any device which supports Multiword DMA Mode 1 or above shall support this

field,

and the value in word 65 shall not be less than the minimum cycle time

reported by

the fastest DMA mode supported by the device.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other

than this

field, the device shall return a value of zero in this field.

y.3  Word 67: Minimum PIO Transfer Cycle Time Without Flow Control

Word 67 defines the minimum cycle time that the device guarantees shall not

cause

data integrity degradation during a transfer which does not utilize flow

control.

Any device may support this field, and if this field is supported, bit 1 of

word

53 shall be set.

Any device which supports PIO Mode 3 or above shall support this field, and

the

value in Word 67 shall not be less than the value reported in word 68.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other

than this

field, the device shall return a value of zero in this field.

y.4  Word 68: Minimum PIO Transfer Cycle Time With IORDY Flow Control

Word 68 defines the minimum cycle time that the device can support when

performing

data transfers while utilizing IORDY flow control.

Any device may support this field, and if this field is supported, bit 1 of

word

53 shall be set.

Any device which supports PIO Mode 3 or above shall support this field, and

the

value in word 68 shall not be less than the fastest PIO mode reported by

the

device.

If Word 53 Bit 1=1 because a drive supports a field in Words 64-70 other

than this

field, the device shall return a value of zero in this field.

From ata-owner  Fri Mar 18 12:03:06 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14853; Fri, 18 Mar 94 12:03:06 PST

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA14825; Fri, 18 Mar 94 12:02:57 PST

Received: from mail.qntm.com by worf.qntm.com with SMTP

(16.6/16.2) id AA22893; Fri, 18 Mar 94 12:02:51 -0800

Received: by mail

(1.37.109.8/16.2) id AA09811; Fri, 18 Mar 1994 12:01:50 -0800

From: jmcgrath@qntm.com

Received: by wt400. via Worldtalk with X400 (2.3.0/1.38.1.3)

id WT24586.396; Fri, 18 Mar 1994 12:01:49 PST

Date: 18 Mar 94 12:01:34 -0800

To: ATA@dt.wdc.com, SCSI@WICHITAKS.NCR.COM

Subject: ATA Project Proposals

Ua-Content-Id: ATA Project Prop

P1-Recipient: ATA@dt.wdc.com, SCSI@WICHITAKS.NCR.COM

P1-Message-Id: US*MCI*QUANTUM;q\qmailgw\940318120134b

Original-Encoded-Information-Types: IA5-Text

X400-Trace: US*MCI*QUANTUM; arrival 940318120134-0800 deferred 940318120134-0800 action Relayed

Message-Id: <"Macintosh */PRMD=QUANTUM/ADMD=MCI/C=US/"@MHS>

P1-Content-Type: P2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  11:01 AM

OFFICE MEMO         ATA Project Proposals                 Date:  3/18/94

This is a follow up to the conversation we had in Newport Beach concerning the

current drafts of the ATA project proposal(s).  I am very concerned about the

relative lack of specific constraints on the scope, especially in light of our

recent problems in approving a GPP standard.  I am also convinced that a broad

scope allows us to prolong the development of a standard by incrementally adding

new stuff over time.  I would rather limit the scope of the projects, and

authorize either new projects of modify the scope of an exisiting project at a

later time than run the risk of delaying the completion of the standard.

Some suggestions and comments:

Physical

"Provide a means to support the AT Attachment protocol on a variety of physical

interfaces such as that for the newly-emerging requirements for memory cards."

The only specific item mentioned is support for ATA on memory cards.  I am not

even sure this is needed - do we really want to define physical characteristics

of ATA memory cards independent of those defined by PCMCIA?  And what is meant

by PHYSICAL anyway?  Connectors?  Electrical requirements?

If we just want to separate out the physical elements of the ATA-2 document,

then why not say this?  If we want to define physical requirements for new

interconnects (e.g. PCI), then say that.  I  am against incorporaing PCI

material into any of these SD3s, since I think there is a lot of work needed in

that area which would surely keep the standards in development for a long time

to come.

Transport

"Provide a means to support the AT Attachment protocol on a variety of physical

interfaces such as that for the newly-emerging requirements for memory cards."

This is the same scope as the Physical SD3.  In reading this, the Physical SD3

appears to be a subset of the Transport SD3.  I suggest eliminating this element

of the scope - physical stuff belongs in the physical SD3 only.

"Provide a means to support a variety of device types on the AT Attachement

interface"

This comes about through the strange dual use of this standard by the Block

Command and Packet Interface standards.  I have no idea how this operates in

practice.  Once again, what is transport?

Block and Packet Interface

I have the same concerns about the lack of specifics in the scope for these

SD3s, but I also have 2 global issues.  First, no where does it state whether

Packet Interface is to be SAM compliant.  I would argue for making it SAM

compliant (the whole point of SAM is to allow the device command sets to be used

on alternate physical interconnects without requiring software rewrites - a

highly desirable goal for any market, but especially for the PC market).  But at

least the issue of compliance should be addressed.

Second, there is an inherent conflict between the Block and Packet interface

standards regarding disk drives.  It may be unavoidable to continue independent

development of capabilities for disk drives in two different standards - but I

am not convinced of this.  I do not want to support two different efforts over

time in a market which has always been intolerant of multiple solutions to a

single problem.  We should  limit our scope (e.g. Packet is not to be used for

disks, or Block is to be limited to existing functions with no more

improvements) in order to make our work manageable.

I realize that many of these issues have been raised in the ATA working group

(which I have not been regularly attending due to scheduling conflicts - that

should be resolved with the new meeting time).  But I feel they have been

deferred rather than resolved.  Since many of them are issues of policy

direction or simple definition, we should (and in the interest of smooth

operations must) resolve them now.  For these reasons I would have to vote NO on

the proposed letter ballot for the current revisions of all 4 SD3s.

Jim McGrath

Quantum

I

From ata-owner  Thu Mar 24 11:28:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01985; Thu, 24 Mar 94 11:28:42 PST

Received: from bolero.rahul.net by dt.wdc.com (4.1/SMI-4.1)

id AA01978; Thu, 24 Mar 94 11:28:32 PST

Received: by bolero.rahul.net id AA18328

(5.67a8/IDA-1.5 for ATA Reflector <ata@dt.wdc.com>); Thu, 24 Mar 1994 11:28:28 -0800

Date: Thu, 24 Mar 1994 11:23:17 -0800 (PST)

From: Richard Kalish <rkalish@rahul.net>

Subject: March 30 Local Bus Meeting Location

To: ATA Reflector <ata@dt.wdc.com>

Cc: SFF Reflector <sff_reflector@sun.COM>

Message-Id: <Pine.3.07.9403241117.A17981-a100000@bolero>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This is to inform all ATA Local Bus committee members that the meeting on

March 30 which was to be at the Adaptec facility has been moved to the

Crown Sterling Hotel ( where most of the meetings have be for the past few

months ) in Milpitas.  It seems that our meeting room here in my building

has another function that needs the room which can not be moved.

The meeting time is still 9:30 starting with a continental breakfast, and

we have the room until 5PM.

Hope this does not inconvience anyone.

Richard Kalish

Adaptec - 408-957-7169

From ata-owner  Thu Mar 24 11:31:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02063; Thu, 24 Mar 94 11:31:49 PST

Received: from bolero.rahul.net by dt.wdc.com (4.1/SMI-4.1)

id AA02053; Thu, 24 Mar 94 11:31:31 PST

Received: by bolero.rahul.net id AA18594

(5.67a8/IDA-1.5 for ATA Reflector <ata@dt.wdc.com>); Thu, 24 Mar 1994 11:31:24 -0800

Date: Thu, 24 Mar 1994 11:28:29 -0800 (PST)

From: Richard Kalish <rkalish@rahul.net>

Subject: Reset timing for the bus

To: ATA Reflector <ata@dt.wdc.com>

Cc: SFF Reflector <sff_reflector@sun.COM>

Message-Id: <Pine.3.07.9403241129.B17981-9100000@bolero>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To date I have had no comments on either the SFF or the ATA reflector

about how a drive should respond to a host reset signal smaller than the

specified 25 microseconds.  Does this mean that there is no interest?

I still want to talk about this at the meeting on March 30th.

Hope to hear about this issue soon.

Richard Kalish

Adaptec - 408-957-7169

From ata-owner  Fri Mar 25 10:08:04 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14369; Fri, 25 Mar 94 10:08:04 PST

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA14362; Fri, 25 Mar 94 10:07:50 PST

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id JAA05064; Fri, 25 Mar 1994 09:44:24 -0800

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 25 Mar 1994 08:57:58 EST

Date:      Fri Mar 25 08:57:57 199 PST

From: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

Message-Id: <2d92ede6.seagate@notes.seagate.com>

Organization: SEAGATE

Reply-To: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

To: ata@dt.wdc.com

Subject:   r.e. Reset on the Bus

Sender: ata-owner@dt.wdc.com

Precedence: bulk

_Date: 03-25-94 08:34:06 AM

_To: ata@dt.wdc.com@internet

_Recipient: ata@dt.wdc.com@internet

_From: John Masiewicz@SEAGATE

_Subject: r.e. Reset on the Bus

I would like to the the originator of this (or any) proposal to provide

proposed wording changes to the ATA-2 specification.

It is very difficult to comment on proposals which are undefined.  Discussions

at the working group go much faster and cleaner if we are discussing specific

proposals.  The "idea" of revising the reset timing specification already had

preliminary discussion, and enough interest was generated for the originator

(or anyone else) to make a specific proposal.  I believe that was what was

agreed at the time.

As for my comments, I think that a change is unnecessary.  If a host violates

the existing specification, operation is undefined.  This is true for all

control signals, not just RESET  Many of us already include various glitch

filtering schemes on RESET as well as the other control signals.  This is

normal design practice and is not violating any specification.  I don't see

that it is necessary to 'define' what should be done here.

However, if there is sufficient support, I would not object to a proposal which

includes glitch filtering which is less than 20 nS on RESET.  I pick this

number because anything greater than this is more than noise since it is of an

order of magnitude of a valid signal on ATA-2.

I also would not want to obsolete anyone's current design.  This should be a

primary concern.  If just one vendor is made obsolete by this proposal, I would

vote with them against it.

--

John Masiewicz (John_Masiewicz@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

From ata-owner  Tue Mar 29 12:21:30 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA03176; Tue, 29 Mar 94 12:21:30 PST

Received: from qlc.qlc.com by dt.wdc.com (4.1/SMI-4.1)

id AA03169; Tue, 29 Mar 94 12:21:26 PST

Received: from triton.qlc.com by qlc.qlc.com with SMTP id AA21006

(5.64+/IDA-1.3.4 for ata@dt.wdc.com); Tue, 29 Mar 94 12:19:45 -0800

Received: by triton.qlc.com (4.1/SMI-4.0)

id AA02167; Tue, 29 Mar 94 12:20:23 PST

Date: Tue, 29 Mar 94 12:20:23 PST

From: j_schaffner@qlc.com (Jim Schaffner)

Message-Id: <9403292020.AA02167@triton.qlc.com>

To: ata@dt.wdc.com, sff_reflector@Sun.COM

Subject: Question on sector sizes in existing products...

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Is anyone aware of a drive/BIOS combination

that can perform Read or Write data transfers

with a sector size less than 512 bytes

without an abort or any other error situation ?

This would possibly be either a direct violation of the

ATA specification with a Read or Write Sector(s) command

(for ??? purpose) or, more possibly, a vendor unique command ?

Would appreciate any konwledge.

Thanks,

"Yogi" Schaffner  QLogic

From ata-owner  Sun Apr  3 23:40:23 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04523; Sun, 3 Apr 94 23:40:23 PDT

Received: from mailgate.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA04513; Sun, 3 Apr 94 23:40:16 PDT

Received: by mailgate.prod.aol.net

(1.37.109.4/16.2) id AA20999; Mon, 4 Apr 94 02:38:18 -0400

From: ljlamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9404040238.tn393877@aol.com>

To: ata@dt.wdc.com

Date: Mon, 04 Apr 94 02:38:16 EDT

Subject: -

Sender: ata-owner@dt.wdc.com

Precedence: bulk

ATA WORKING GROUP MINUTES

Start of Message:

Accredited Standards Committee**Operating under the procedures of

The American National Standards Institute.

X3 Secretariat, Computer and Business Equipment Manufacturers

Association (CBEMA)

1250 Eye Street NW, Suite 200, Washington, DC 20005-3922

Telephone: 202-737-8888 (Press 1 twice) FAX: 202-638-4922 or 202-

628-2829

X3, Information Processing Systems

Doc.  No.  X3T10/94-066R0

Date:     March  15,1994

Project:

Ref. Doc.:

Reply to: S. Finch

To:    Membership of X3T10

From:  Lamers/Finch

Subject:   Minutes of X3T9.2 Disk Drive Working Group  March  14,1994

Agenda

1.Opening Remarks                                          2

2.Attendance and Membership, Introductions                 2

3.Approval of Agenda                                       2

4.Document Distribution                                    3

5.Old Business                                             3

5.1  ATA DMA Issues (Landis)                          3

5.2  Power Management Issues (Landis)                 3

5.3  Electrical Definitions (Masiewicz)               3

5.4  16 MB/s Issues (Hanan)                           3

6.New Business                                             3

6.1  ATA-3 Project Proposals                          3

6.2  Master Slave Hardware Issues                     4

7.Meeting Schedule                                         4

8.Adjournment                                              4

Results of Meeting

1.Opening Remarks

Steve Finch convened the meeting at 2:00 PM.  Steve thanked  Skip

Jones of Qlogic for hosting the meeting.

As  is customary, the people attending introduced themselves.   A

copy  of  the  attendance list was circulated for attendance  and

corrections.

It  was stated that the meeting had been authorized by X3T10  and

would  be conducted under the X3 rules.  Ad hoc meetings take  no

final  actions, but prepare recommendations for approval  by  the

X3T10 task group.  The voting rules for the meeting are those  of

the  parent  committee, X3T10.  These rules are:   one  vote  per

company; and any participating company member may vote.

The  minutes of this meeting will be posted to the SCSI  BBS  and

the  SCSI  Reflector and will be included in the  next  committee

mailing.

2.   Attendance and Membership, Introductions

Attendance at working group meetings does not count toward

minimum attendance requirements for X3T10 membership.  Working

group meetings are open to any person or company to attend and to

express their opinion on the subjects being discussed.

The following people attended the meeting.

Richard Kalish Adaptec rkalish@rahul.net

Lawrence J. Lamers Adaptec ljlamers@adaptec.com

Carlos Orosco AST carlos.orosco@ast.com

John Geldman Cirrus Logic johng@cirrus.com

Joseph Chen Cirrus Logic chen@cirrus.com

Lane Lee Conner lane.lee+adevelopment+acolorado% conner@mci.well

Dal Allan ENDL 250-752@mcimail.com

Robert Liu Fujitsu Computer Products

Jeff Epstein Future Domain jeffe@fdc.mhs.compuserve.com

Nancy Cheng Hitachi Computer Products n.chen@hitachi.com

George Penokie IBM gop@rchvmp3.vnet.ibm.com

Dan Colegrove IBM colegrove@vnet.ibm.com

Dean Wallace Infinity

Ron Roberts Maxtor ron_roberts@maxtor.com

David Isenberg NCR david.isenberg@fcollinsco.ncr.com

Peter Brown Oak Technology brown@oaktech.com

Mike Carpenter Qlogic m_carpenter@qlc.com

John Masiewicz Seagate Technology masiewicz@notes.seagate.com

Hale Landis Seagate Technology landis@notes.seagate.com

Stephen Finch Silicon Systems 5723283@mcimail.com

Devon Worrell Western Digital worrell@dt.wdc.com

Tom Hanan Western Digital t_hanan@dt.wdc.com

3.   Approval of Agenda

The agenda was approved as proposed.

4.   Document Distribution

Copies of the project proposals for ATA-3 were distributed.

5.   Old Business

5.1  16.6 Mb/s [94-045] (Hanan)

Tom Hanan presented the current proposal for 16.6 Mb/sec

transfers.   There was some debate on the format of the timing.

There is an implication that tE and tG have a common point, when

they in fact do not.  The tE timing can be removed since tG has

been applied to reads as well as writes.  Steve Finch noted that

in the older modes the tG value was not the same for reads and

writes.  The timing will be split to have a tGr and a tGw.

The IORDY timing has questions.  IORDY is driven inactive,

normally true.  The timing showing the "thinking" transition will

be incorporated since this is likely to be the normal case with

higher transfer rates.

Steve Finch plans to show the proposal to the X3T10 plenary and

request a vote on it at the next meeting.

5.2  ATA DMA Issues [94-063] (Landis)

Hale Landis presented the proposed changes regarding DMA

transfers.  These proposed changes are incorporated in the latest

revision of 94-051.

The consensus of the working group is to incorporate these

changes into the ATA-2 working draft.

5.3  Power Management [94-064] (Landis)

Hale Landis presented the latest proposal on the command

description format, illustrated with the power management.

5.4  Electrical Definitions [94-051] (Masiewicz)

Steve Finch stated a need for a definition of full amplitude

signal.  John Geldman accepted the task of coming up with one.

Another debate over the meaning of word 66.  The wording was

changed to indicate that it does not indicate that flow control

will not be used.

5.5  Reset Width [94-053] (Landis)

This will be dealt with at the next working group.

6.   New Business

6.1  Project Proposals  [94-019,94-020,94-021,94-022] (Finch)

Steve Finch stated that the project proposals had previously been

distributed and that the copies being distributed today contained

only slight changes to put them in the current style for project

proposals.  Steve also stated that he expects the project

proposals will be discussed and voted on in the X3T10 Plenary

meeting later in the week.  There was no further discussion.

6.2  Master/Slave Handshaking [94-065] (Landis)

Hale Landis presented a proposal for dealing with master/slave

handshaking and when a drive is ready.  This constitutes an

update to annex A.  What is a non-media access command?

Hale showed the three methods used to deal with BSY and DRDY..

7.   Meeting Schedule

March 30, 1994 at Adaptec in Milpitas, CA.

April 27, 1994, at the Crown Sterling Suites, Milpitas, CA,

hosted by Cirrus Logic

Mary 19, 1994 at Sheraton Inn in Harrisburg, PA.

June 1, 1994 at Adaptec in Milpitas, CA.

8.   Adjournment

The meeting adjourned at approximately 5:00 PM.

Lawrence J. Lamers

Adaptec                    TEL: 408-945-8600 ext 3214

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@ADAPTEC.COM

From ata-owner  Fri Apr  8 09:56:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08178; Fri, 8 Apr 94 09:56:47 PDT

Received: from cirrus.com by dt.wdc.com (4.1/SMI-4.1)

id AA08171; Fri, 8 Apr 94 09:56:39 PDT

Received: from sunstorm.corp.cirrus.com (sunstorm1) by cirrus.com with SMTP id AA13292

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Fri, 8 Apr 1994 09:55:09 -0700

Received: from sundrive.corp.cirrus.com by sunstorm.corp.cirrus.com with SMTP id AA00930

(5.65c/IDA-1.4.4); Fri, 8 Apr 1994 09:55:08 -0700

Received: from novmailpc.cirrus.com by sundrive.corp.cirrus.com (4.1-Cirrus/S)

id AA03438; Fri, 8 Apr 94 09:54:55 PDT

Received: from cc:Mail by novmailpc.cirrus.com

id AA765820349 Fri, 08 Apr 94 08:52:29 PST

Date: Fri, 08 Apr 94 08:52:29 PST

From: chen@corp.cirrus.com

Message-Id: <9403087658.AA765820349@novmailpc.cirrus.com>

To: TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com, ata@dt.wdc.com,

nicos@corp.cirrus.com, johng@corp.cirrus.com

Subject: April 27 ATA SWIG Meeting Notice

Sender: ata-owner@dt.wdc.com

Precedence: bulk

********************************************************************

*********   ATA     M E E T I N G   N O T I C E   ******************

********************************************************************

SUBJECT:        ANSI ATA SWIG MEETING

DATE:           April 27 (Wednesday)

TIME:           9:30AM - 4PM

PLACE:          CROWN STERLING SUITES HOTEL ADDRESS:

901 Calaveras, Milpitas

PHONE:          408-942-0400, Ask for Cirrus Logic for special

corp rate at $89.00 with Breakfast

HOST:           Cirrus Logic

City:           Milpitas, Ca.

San Jose International is the closest airport.

Direction:      Crown Sterling Suites is located on Calaveras and

Hwy 680. If you travel from San Jose Airport, you

can take 880 north to Calaveras/237 exit.  Turn

east at Calaveras and drive 2 miles before it hits

Hwy 680.  Crown Sterling Suites Hotel is located

to your left.  If you have any question please

contact me.

---------------------------------------------------------------------

Cirrus Logic, Inc.

Joseph Chen

Ph.  (510) 226-2101,  Fax. (510) 226-2170 E-Mail chen@cirrus.com

---------------------------------------------------------------------

From ata-owner  Mon Apr 11 15:55:36 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01931; Mon, 11 Apr 94 15:55:36 PDT

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA01924; Mon, 11 Apr 94 15:55:34 PDT

Received: from uucp4.uu.net by relay1.UU.NET with SMTP

(5.61/UUNET-internet-primary) id AAwlfl19546; Mon, 11 Apr 94 18:26:50 -0400

Received: from adaptec.UUCP by uucp4.uu.net with UUCP/RMAIL

; Mon, 11 Apr 1994 18:26:50 -0400

Received: from ppo9.adaptec by adaptec.com (4.1/SMI-4.1)

id AA20555; Mon, 11 Apr 94 15:15:36 PDT

Date: Mon, 11 Apr 94 15:15:35 PDT

From: uunet!adaptec.com!alp (Al Pham x4804)

Message-Id: <9404112215.AA20555@adaptec.com>

To: sff_reflector@sun.com

Subject: Re: DMA Mode - Identify data and Set Feature operation relations - Response

Cc: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Totani-san:

Being a member of the ATA Working Group and a "developer" of ATA/IDE BIOS & O/S drivers, I think I can answer your question.

AL Pham	- Adaptec, Inc.

>    Subject: DMA Mode - Identify data and Set Feature operation relations

>

> Regarding X3T9.2/791D revision 4a (ATA) & SFF-8011 Identify and Set

> Features specification, I am thrown into confusion.

> Can anyone answer following question I have?

>

> When supporting Single word DMA transfer, there are 3 modes 0,1 and 2.

> How should a drive supporting mode 2 act when mode 0 or mode 1 is

> requested by Set Feature command.

>

> 1. Should post an Aborted Command error and the Identify Word 62 should

>  still indicate mode 2 i.e. 00000100 00000100.

>

If a drive supports mode 2, it "automatically" supports modes 0,1 & 2 and word 62 should have bits 0,1 & 2 set to "1".  Therefore, it must not abort a set feature for mode 0 or 1.

> 2. Recieve Identify command and change the DMA mode to requested value

>  which means reduce the performance of the DMA channel to mode 0,1

>  as requested.(I think this is meaningless, ATA Single word DMA timing

>  parameters defines transfer modes by host control signals only. tC has

>  max. value, not min. value.)

>  Also Identify word 62 should show the current DMA mode. i.e. if mode 1

>  is requested word 62 should be 00000010 00000111.

>

Yes, it is meaningless trying reduce performance of DMA channel (timing is controlled by host, not drive).  Besides, there is no need for disk drive to change any timing at all.  Because mode 2 timings meet all timings of modes 0,1 & 2.

By the way, tC must be a max, a min number would not make any sense.  The drive must de-assert DMARQ within this time so that host will not erronously start another transfer.

>

> 3. Recieve Identify command and stay to mode 2 but Identify word 62

>  should show the requested DMA mode. i.e. if mode 1 is requested word 62

>  should be 00000010 00000111.

>

EXACTLY !!!  Because as explained above it is meaningless trying to change timing.

> 4. Recieve Identify command but will not change the DMA mode.

>  Identify word 62 will stay stable. 0000100 00000111

>

Drive should update word 62 to reflect requested mode.

> from the spec.

> >X3T9.2/791D revision 4a (ATA) -- Identify Word 62: Single word DMA tran

> sfer

> > The low order byte identifies by bit all of the Modes which are suppor

> ted

> > e.g. if Mode 0 is supported,bit 0 is set. The high order byte contains

>  a

> > single bit set to indicate which mode is active.

> >

> >X3T9.2/791D revision 4a (ATA) & SFF-8011 -- Set features

> > A host can choose the transfer mechanism by Set Transfer Mode and

> > specifying a value in the Sector Count Register. The upper 5 bits

> > define the type of transfer and the low order 3 bits encode the mode

> > value.

> >  PIO Default Transfer Mode             00000 00n

> >  PIO Flow Control Transfer Mode X      00001 nnn

> >  Single Word DMA Mode X                00010 nnn

> >  - - - - - -

> > If a device does not suport the mode specified, the device posts an Ab

> orted

> > Command error.

>

> Regards;

>

>  Tokuyuki Totani

>  DISK SYSTEMS DESIGN SECT , MAGNETIC DISK DESIGN ENGINEERING DEPT

>  TOSHIBA CORPORATION OME WORKS , 2-9 SUEHIRO-CHO OME, TOKYO JAPAN

>  ZIP 198 PHONE 81-428-33-1292 000092050009@tg-mail.toshiba.co.jp

>

>

>

From ata-owner  Sun Apr 17 11:18:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15360; Sun, 17 Apr 94 11:18:47 PDT

Received: from mailgate.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA15353; Sun, 17 Apr 94 11:18:44 PDT

Received: by mailgate.prod.aol.net

(1.37.109.4/16.2) id AA25142; Sun, 17 Apr 94 14:16:15 -0400

From: ljlamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9404171416.tn16971@aol.com>

To: ata@dt.wdc.com

Date: Sun, 17 Apr 94 14:16:13 EDT

Subject: Error Detection on ATA

Sender: ata-owner@dt.wdc.com

Precedence: bulk

4/16/94

FR:    Larry Lamers/Rick Kalish/Al Pham

RE:    Error Detection for ATA-3

DATE:  April 15, 1994

This proposal is to add a one-bit parity to the 16-bit ATA word

for detecting data errors during reads and writes. This proposal

is based on Mode 4 timing multi-word DMA timing.  The method of

enabling parity is needs to be determined.

The Parity- signal uses the existing IOCS16- signal.  Data parity

is odd.  A low active state indicates a parity bit of 1.

The Error- signal use the PDIAG- signal.  If a parity error is

detected the receiving device would indicate this by asserting

this signal (low active).

If a Error- signal is asserted the transmitting device shall

resend the same word of data on the next cycle.  The disk drive

is expected to retry parity errors indefinitly.  The host may

terminate the retry of parity errors by reset.

On a read operation the disk drive may negate IORDY to extend the

cycle time allowing it to setup for retransitting the data word.

The host may also extend the cycle time by delaying DIOR-.

On a write operation the host may extend the cycle time by by

delaying DIOW- allowing it to setup for retransmitting the data

word.  The disk drive may also negate IOCHRDY if additional time

is needed to setup to receive the retransmission.

The basic timing considerations for generating and detecting an

error are given in figures 1 and 2.  Note that all times are

measured at the receiver.

If the time allowed for parity generation is not satisfactory

parity could be generated in the data buffer and not when data is

gated to the bus.  This would require 17-bit wide buffers.

tGr  20 ns          data setup time read

tGw  20 ns          data setup time write

tX1  10 ns max parity setup time

tX2  30 ns max parity error detected to Error- signal asserted

tA   25 ns max maximum time to assert IORDY following IOR-

_________________        __________....______        _____

DIOR-  ____/                 \______/                    \______/

|--|<--tGr

______                    ______

DATA   ------------------<______>------------------<______>----------

|-|<--tX1

______                    ______

Parity- --------------------<______>------------------<______>-------

|-----|<--tA

_____________________________                         ________

Error-                        \\\\\\_________....___________/

|-----|<--tX3

____________________________________        ___________________

IORDY                                      \______/

Figure 1 - Read with Parity Error

If a parity error is detected on a read the drive asserts IORDY

to allow time to setup for retransmitting the last data word.

_________________        _________...._____       _______

DIOW-  ____/                 \______/                  \_____/

|--|<--tGw

______                    ______

DATA   ------------------<______>------------------<______>----------

|-|<--tX1

______                    ______

Parity- --------------------<______>------------------<______>-------

|-----|<--tX2

_____________________________                          _______

Error-                              \_________....___________/

Figure 2 - Write with Parity Error

If a parity error is detected on a write the host extends the

cycle time as needed to setup for retransmitting the last word of

data.

Lawrence J. Lamers

Adaptec                    TEL: 408-945-8600 ext 3214

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@ADAPTEC.COM

From ata-owner  Mon Apr 18 09:23:54 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10302; Mon, 18 Apr 94 09:23:54 PDT

Received: from maxtor.com (zeus.maxtor.com) by dt.wdc.com (4.1/SMI-4.1)

id AA10295; Mon, 18 Apr 94 09:23:47 PDT

Received: from sanjose3.maxtor.com ([134.6.34.200]) by maxtor.com (4.1/SMI-4.1)

id AA09674; Mon, 18 Apr 94 09:26:51 PDT

Message-Id: <9404181626.AA09674@maxtor.com>

Received: by sanjose3.maxtor.com with VINES ; Mon, 18 Apr 94 09:03:56 PDT

Date: Mon, 18 Apr 94 09:01:48 PDT

From: Pete=McLean%ENG12%MCO@sanjose3.maxtor.com

Subject: Security proposal

To: ata@dt.wdc.com

Cc:

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Attached is the security proposal I presented at the 30 March meeting in

ASCII text format.

Regards, Pete

*****  ATTACHMENT: c:\winword\secure\ansisec.txt *****

X3T10/94-087R0

Proposal for adding Secure Mode to ATA Specification

30 March 1994

Pete McLean

Maxtor Corporation

2190 Miller Drive

Longmont, Colorado 80501

(303) 678-2149

1.0 Background

With the introduction of small, removable hard-disk drives, particularly PCMCIA drives, users now have

a convenient, removable data storage medium. This medium provides relatively high data capacity in a

size and weight that conveniently fits in one's shirt pocket. This medium is easily transportable from one

host to another. Users are now free to travel taking their data with them, creating or updating it on mobile

systems or systems other than their own.

This new freedom comes with a problem. When the media containing users valuable data is not removable

from their system, the system's security protection prevents access by others. With a small removable

medium, the user's data is now subject to loss or theft.

This specification describes a method for limiting access to data stored on small form factor, removable

hard disk drives to only authorized users or host systems. To accomplish this, a drive is placed in "secure

mode" by a user. Having done so, whenever the drive is placed in a host, it must be "unlocked" before it

will execute data transfer commands. "Unlocking" is accomplished by providing the drive with a valid

"password".

2.0 Operational Overview

Whether a drive supports secure mode and, if so,  the secure mode state can be determined by issuing an

ATA command (SECURE_STATE).

The drive may be set into either Secure Mode Read Only where the drive may be read but not written, or

Secure Mode Read/Write where no data transfers can be executed.

The drive is set into "secure mode" by an ATA command (SET_SECURE_RO or SET_SECURE_RW).

The user may set up to four "passwords" each up to 512 bytes in length and the host system will create

an "emergency password". The user may also specify that one or more of the "passwords" must be

matched to "unlock" the drive. The number of matches required to unlock the drive is placed in the

Features register before issuing the SET_SECURE_XX command. The SET_SECURE_XX command

resembles a write command in that the sector count is valid and the command includes the transfer of n

sectors of data to the drive. Each of the n sectors represents a unique "password". If the user defined

password contains fewer than 512 bytes, the password will be zero filled to complete a full sector, so that

a password sent when the drive is installed on one system will be exactly the same as a password sent

when installed on another system.

When in "secure mode" and "locked", non-data transfer commands will be executed normally, however,

all data transfer commands, i.e., commands that read or modify user data on the disk, will be rejected

with error if in secure mode RW and write commands, i.e., commands that modify user data on the disk,

will be rejected if in secure mode RO. When "unlocked", all commands will execute normally.

An ATA command will allow the drive to be taken out of "secure mode" (DISABLE_SECURE),  If in

"secure mode" and "unlocked", a drive will accept the DISABLE_SECURE command and go out of

"secure mode" and delete all passwords. If the drive is in "secure mode" and has not been "unlocked", it

will reject the command with errors set.

When inserted/powered up, the drive will go through the standard PCMCIA startup and ATA startup and

the state of "secure mode" will be noted. If in "Secure Mode RW" and "locked", the drive will respond to

non-data transfer commands but will reject transfer commands until an UNLOCK command is received.

If in "Secure Mode RO" and "locked, the drive will reject all write commands until an UNLOCK

command is received. The UNLOCK command is another ATA command and again resembles a write

command. It must have sector count set at the number of sector matches required to unlock the drive and

will include the transfer of one sector of data to the memory card for each required match.  When data is

received, it will not be written, instead it will be compared to the valid "passwords" stored. If the required

matches are found, the drive will "unlock" and function normally.  If no match is found, errors will be

reported in response to the command and the drive will remain "locked". An unlock command with only

one password, the emergency password will always unlock the drive.

When the drive is unlocked, a flag tells the drive when to relock. It may be set such that the drive will

automatically lock when powered down or it may require a LOCK command to lock the drive. The flag to

disable locking at power down is provided for systems that frequently remove power from the drive. The

passwords remain valid, and normal secure mode locking at power down can be re-enabled by issuing

another UNLOCK command.

When the UNLOCK command is issued and the drive is already in the unlocked state, the command is

executed and if the match count and passwords are not valid, i.e., the command would not have unlocked

the drive an error is returned. Thus when in unlocked state, passwords can be verified using the

UNLOCK command. The flag for locking the drive at power down must be valid when verifying

passwords.

The LOCK command locks the drive immediately on receipt.

The ADD_PASSWORDS, REMOVE_PASSWORDS, and MODIFY_MATCH commands allow the user

to modify the set of passwords and number of matches required to unlock the drive.

3.0 Secure Mode Commands

The following ATA commands are used in the implementation of Secure Mode. The Secure Mode state of

the drive resulting from the execution of these commands must be maintained by the drive when

powered-down. This state includes whether or not the drive is in secure mode, if it is locked, the number

of password matches required to unlock and the existing set of valid passwords.

Class       Command                  Command                                Parameters Used

Code                                FR      SC     SN         CY       DH

1        SECURE_STATE                  EAh                               y                  80            y           D

3      SET_SECURE_RW                 EBh                               y         y       81            y           D

3      SET_SECURE_RO                  EBh                               y         y       82            y           D

3   ADD_PASSWORDS                 EBh                               y         y       83            y           D

3   REMOVE_PASSWORDS         EBh                               y         y       84            y           D

1      MODIFY_MATCH                 EAh                              y                   85           y           D

1    DISABLE_SECURE                  EAh                                                    86          y            D

3           UNLOCK                             EBh                               y         y        87          y            D

1             LOCK                                 EAh                                                    88          y            D

The host will set the Cylinder High Register with the ASCII character S (53h) and the Cylinder Low

Register with the ASCII character M (4Dh) prior to command assertion. The drive will validate these

contents before executing the command and will invert the contents of the Cylinder Registers. All secure

mode commands Abort if the cylinder registers do not contain these values. Thus upon completion of the

command, the host will know that the drive supports secure mode even if the command fails due to an

error.

3.1 SECURE_STATE Command

This command allows the host to determine the current secure mode state of the drive. It will be accepted

and executed at any time. Flow of this command is shown in figure 1.

Upon completion of this command, the contents of the drive registers will be as follows:

Cylinder High - ACh

Cylinder Low - B2h

Features - Version number of secure mode implemented by the drive

Sector count -

7                  6              5              4                 3              2               1                0

Secure State            LockFlag          0                   Match Count

Bits7:6 - Secure State

Bit 7 - Secure Set R/W- If set indicates the drive has been set in Secure Mode

Read/Write.

Bit 6 - Secure Set RO - If set indicates the drive has been set in Secure Mode Read Only.

Bit 5 - Unlocked - If set indicates the drive  has been Unlocked so that data transfer 			commands will be executed.

Bit 4 - Lock Flag - this bit is only valid if the Secure State is 011 or 101.

0 - indicates drive will assume the Locked State when powered down

1 - indicates drive can only be locked by issuing the LOCK command

Bit 3 - Zero

Bit  2:0 - Number of password matches required to unlock drive

Valid Secure State (bits 7:5) are:

000	Indicates the drive is not in secure mode

100	Indicates the drive is set into Secure mode R/W, but is locked. Data transfer commands,

i.e., commands that read or modify user data on the disk, will be rejected.

010	Indicates the drive is set into Secure mode RO, but is locked. Write data transfer

commands, i.e., commands that modify user data on the disk, will be rejected.

101	Indicates the drive  is set into Secure Mode R/W, and is unlocked. All data transfer

commands will execute.

011	Indicates the drive  is set into Secure Mode RO, and is unlocked. All data transfer

commands will execute.

3.2 SET_SECURE_RW Command

This command is used to set a drive into Secure Mode Read/Write, define the valid set of passwords and

the number of password matches required to unlock a drive. Flow of this command is shown in figure 2.

If the drive is not in Secure Mode when the command is received,  the value set in the Features register

indicates the number of matches that will be required to unlock the drive. The value set in the Sector

Count register indicates the number of 512 byte passwords that will be passed with this command

including the Emergency password. If either the Features register low order four bits or the Sector Count

register contain a value less than 1 or greater than 5, the command is not executed and Abort error is

returned. If the number in the Features register low order four bits is greater than the number in the

Sector Count register, the command is not executed and Abort error is returned.

Use of the Emergency password is optional. If the most significant bit of the Features register is set, the

first password received by the drive is considered to be the Emergency password. The number of matches

required and the passwords received with the command become the valid passwords and match count

values. Upon successful completion of this command, the secure state will reflect Secure Mode RW set,

unlocked, value 101. The contents of the Cylinder Registers will be ACh and B2h.

The Emergency password shall be created by the host by asking the user a series of questions. This

password will consist of 18 twenty byte fields. Each field will contain the ASCII response to one question.

If the user response is longer than twenty bytes, the field will contain the first twenty bytes of the response

(i.e., response will be truncated). If the response is less than twenty bytes the end of the field will be zero

filled. The last 152 bytes of the password will be zero filled.  The Emergency password may only be

modified by removing the drive from secure mode, then setting into secure mode again. The questions in

order will be:

Question                                                                                             Field                         Password bytes

Last Name?                                                                                              0                                  0-19

First Name?                                                                                             1                                 20-39

Home address - street?                                                                         2                                 40-59

Home address - city?                                                                            3                                  60-79

Home address - state?                                                                          4                                  80-99

Home Address - country?                                                                    5                               100-119

Home phone number?                                                                           6                               120-139

Employer's name?                                                                                  7                               140-159

Work address - street?                                                                          8                               160-179

Work address - city?                                                                             9                               180-199

Work address - state?                                                                         10                               200-219

Work address - country?                                                                    11                               220-239

Work phone number?                                                                          12                               240-259

Place of birth - city?                                                                             13                               260-279

Place of birth - state?                                                                           14                               280-299

Place of birth - country?                                                                      15                               300-319

Date of birth?                                                                                        16                               320-339

Mother's maiden name?                                                                       17                              340-359

Zero fill                                                                                                                                      360-511

If the drive is in Secure Mode when this command is received, the command will not be executed and an

Abort error is returned.

3.3 SET_SECURE_RO Command

This command is used to set a drive into Secure Mode Read Only, define the valid set of passwords and

the number of password matches required to unlock a drive. Flow of this command is shown in figure 2.

If the drive is not in Secure Mode when the command is received,  the value set in the Features register

indicates the number of matches that will be required to unlock the drive. The value set in the Sector

Count register indicates the number of 512 byte passwords that will be passed with this command

including the Emergency password. If either the Features register low order four bits or the Sector Count

register contain a value less than 1 or greater than 5, the command is not executed and Abort error is

returned. If the number in the Features register low order four bits is greater than the number in the

Sector Count register, the command is not executed and Abort error is returned.

Use of the Emergency password is optional. If the most significant bit of the Features register is set, the

first password received by the drive is considered to be the Emergency password. The number of matches

required and the passwords received with the command become the valid passwords and match count

values. Upon successful completion of this command, the secure state will reflect Secure Mode RO set,

unlocked, value 011. The contents of the Cylinder Registers will be ACh and B2h.

The Emergency password shall be created by the host by asking the user a series of questions. This

password will consist of 18 twenty byte fields. Each field will contain the ASCII response to one question

. If the user response is longer than twenty bytes, the field will contain the first twenty bytes of the

response (i.e., response will be truncated). If the response is less than twenty bytes the end of the field

will be zero filled. The last 152 bytes of the password will be zero filled.  The Emergency password may

only be modified by removing the drive from secure mode, then setting into secure mode again. The

questions in order will be as defined in the table shown in the SET_SECURE_RW command.

If the drive is in Secure Mode when this command is received, the command will not be executed and an

Abort error is returned.

3.4 ADD_PASSWORDS Command

When the drive is in Secure Mode XX, unlocked, state 101 or 011, with an existing match count and set

of valid passwords, this command is used to add passwords to the existing set of valid passwords and

reset the match count. Flow of this command is shown in figure 4.

The value in the Features register will become the new match count value. The value in the Sector Count

register indicates the number of passwords to be added. If the value in the Sector Count register is zero,

or if the value in the Sector Count register plus the existing number of valid passwords is greater than 5,

the command will be rejected and an Abort error returned. If the value in the Features register is less than

1 or greater than the value in the Sector Count register plus the number of existing valid passwords, the

command will be rejected and an Abort error returned.

Upon successful completion of this command, the passwords received with this command will be added to

the set of existing passwords, the match count value will be the new match count value, and the drive will

be in Secure Mode XX, unlocked state, 101 or 011.

If this command is received when not in Secure Mode XX, unlocked state, 101 or 011, the command will

be rejected and an Abort error returned.

3.5 REMOVE_PASSWORDS Command

When the drive is in Secure Mode XX, unlocked, state 101 or 011, with an existing match count and set

of valid passwords, this command is used to remove passwords from the existing set of valid passwords

and reset the match count. Flow of this command is shown in figure 5.

The value in the Features register will become the new match count value. The value in the Sector Count

register indicates the number of passwords to be removed. If the existing number of valid passwords

minus the value in the Sector Count register is less than 1, the command will be rejected and an Abort

error returned. If the value in the Features register is less than 1 or greater than the number of existing

valid passwords minus the value in the Sector Count register, the command will be rejected and an Abort

error returned. The emergency password may not be removed and the command will be rejected with an

Abort error if this is attempted.

Upon successful completion of this command, the passwords received with this command will be

compared with the set of existing passwords and each that matches with a different password will be

removed from the valid set of passwords. Passwords that do not match any valid passwords do not affect

the set of valid passwords. The match count value will be the new match count value, and the drive will

be in Secure Mode XX, unlocked state, 101 or 011.

If this command is received when not in Secure Mode XX, unlocked state, 101 or 011, the command will

be rejected and an Abort error returned.

3.6 MODIFY_MATCH Command

When the drive is in Secure Mode XX, unlocked, state 101 or 011, with an existing match count and set

of valid passwords, this command is used to reset only the match count. Flow of this command is shown

in figure 6.

The value in the Features register will become the new match count value. If the value in the Features

register is less than 1 or greater than the number of existing valid passwords, the command will be

rejected and an Abort error returned.

Upon successful completion of this command, the match count value will be the new match count value,

and the drive will be in Secure Mode XX, unlocked state, 101 or 011.

If this command is received when not in Secure Mode XX, unlocked state, 101 or 011, the command will

be rejected and an Abort error returned.

3.7 DISABLE_SECURE Command

When the drive is in Secure Mode XX, unlocked, state 101 or 011, with an existing match count and set

of valid passwords, this command is used to remove the drive from Secure Mode. Flow of this command

is shown in figure 7.

Upon successful completion of this command, the drive will be in state 000, not in Secure Mode. All

passwords are deleted.

If this command is received when not in Secure Mode XX, unlocked state, 101 or 011, the command will

be rejected and an Abort error returned.

3.8 UNLOCK Command

This command unlocks a drive in the Secure Mode, locked state to allow data transfers. Flow of this

command is shown in figure 8.

If the user has forgotten the user defined passwords, the host may recreate the emergency password by

asking the questions described in the SET_SECURE_XX command.

If the sector count register contains the value 1 and the password received is the emergency password the

command will execute and the drive will unlock. Otherwise, the sector count register will contain the

number of 512 byte passwords to be passed to the drive and matched. If this number is not equal to the

number of matches specified when the drive was placed in Secure Mode, the command will be rejected

and the Abort error returned.

The drive will match the password(s) received with this command with the existing  set of valid

passwords. If each unlock password matches with a different password in the established set of passwords,

the drive will unlock.

For example, if the number of matches is specified as one and the set of passwords consists of four

passwords, the single password received with the UNLOCK command will be compared to each of the

four possibilities until a match is found.

If the number of matches is specified as two and the set of passwords consists of three passwords, the first

password received with the UNLOCK command will be compared to each of the three possibilities until a

match is found, and then the second password received with the UNLOCK command will be compared to

the remaining two possibilities until a match is found.

If the UNLOCK was successful, the Features register indicates the required action to relock the drive. If

the Features register contains the value 00h, the drive will assume the lock state when powered-down. If

the Features register contains the value FFh, the drive will only assume the locked state when the LOCK

command is received, that is, the drive may be powered-down and back up without assuming the locked

state. Passwords remain valid.

If the value in the Features register is other than 00h or FFh, or if the required number of passwords

received with the command do not match existing valid passwords, the command is rejected, an Abort

error is returned, and the drive will remain in secure mode XX set, locked state, secure state 100 or 010.

If eight  UNLOCK commands are rejected with error, the drive will cease response to any command until

powered-down and re powered-up.

If this command is received when the drive is in Secure Mode XX, unlocked state, 101 or 011, the

command will be executed and if the match count or passwords do not match, an Abort error will be

returned but the drive will remain unlocked. Thus when in the unlocked state, this command can be used

to verify match count and passwords. The Features register is used as described above to set or clear the

Lock Flag.

Upon successful completion, the secure state will reflect 101 or 011, Secure Mode XX set and unlocked.

Having been unlocked, the drive will now accept and execute all data transfer commands.

If this command is received when not in Secure Mode , state 000, the command will be rejected and an

Abort error returned.

3.9 LOCK Command

This command will lock a drive any time the drive is in secure mode XX, unlocked, state 101 or 011. If

the drive was unlocked with the Features register value FFh, this is the only means of locking the drive. If

the drive was unlocked with the Features register value 00h, either this command or powering-down the

drive will cause the drive to assume the locked state. Flow of this command is shown in figure 9.

Upon successful completion of this command the drive will be in secure mode XX, locked, state 100 or

010.

If this command is received when the drive is not in Secure Mode, state 000, or in Secure Mode XX,

locked, state 100 or 010, the command will be rejected and an Abort error returned.

From ata-owner  Mon Apr 18 11:51:40 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA13432; Mon, 18 Apr 94 11:51:40 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA13425; Mon, 18 Apr 94 11:51:35 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id LAA02375 for <ata@dt.wdc.com@relay>; Mon, 18 Apr 1994 11:45:24 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 18 Apr 94 11:50:11 PDT

Date: 18 Apr 94 12:39:05 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: Proposed Agenda for April 27

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"50939181404991/257785@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The following is the proposed agenda for the April 27th ATA Working Group

meeting to be held in Milpitas.  (Meeting notice was previously sent, but

if you didn't get it contact me for additional info.)

Proposed Agenda

X3T10 ATA Working Group

9:30 A.M. - 4:00 P.M., April 27, 1994

Milpitas, California

1.  Opening Remarks

2.  Attendance and Membership, Introductions

3.  Approval of Agenda

4.  Document Distribution

5.  Old Business

5.1.	16.6 MB/s (94-045r2, Hanan/Finch) (waiting plenary review)

5.2.	Master/Slave Handshaking (94-065,Landis)

5.3.	Identify Drive for Over 528 MB(______,Landis)

6.  New Business

6.1.	Parity Proposal (94-_____,Lamers)

7.   Meeting Schedule (All except March 30 are proposed)

April 27, 1994, Milpitas, Host:  Cirrus Logic

May 19, 1994, Harrisburg, PA (Note:  This is a Thursday!)

June 1, 1994, Milpitas, Host: Adaptec

June 29, 1994, Irvine, Western Digital

8.  Adjournment

____________________________________________________________________

The ATA Working Group has identified the following items which are to be

addressed in ATA-3.  These items are not on the agenda for this meeting.

1.  Data reliability.  (Discussion initiated by Adaptec CRC proposal)

2.  Division of ATA into multiple documents

3.  Major clean-up/re-write of document.

4.  Inclusion of ATAPI and CD-ROM support

5.  20 MB/s transfer rates

6.  Reset Pulse Width

7.  Secure Mode (for removeable devices)

8.  Format Issues

==============================================================

If you have additional items to be placed on the agenda, please send me a

note so I can include them.  If a new item for the agenda needs supporting

documentation (very little get done without it), I recommend that you

obtain a document number from Larry Lamers (at Adaptec), bring hardcopy of

your material to the meeting and be prepared to present it.  Things move

faster when this method is used.

Regards....

==============================================================

Steve Finch, Chair, ATA Working Group

Silicon Systems, Inc.

PH:  714 573-6808,  FAX:  714 573-6916

email:  steve.finch@tus.ssi1.com

(You can still use my old email too!  5723283@mcimail.com)

=============================================================

From ata-owner  Tue Apr 19 08:46:58 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08127; Tue, 19 Apr 94 08:46:58 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA07901; Tue, 19 Apr 94 08:40:01 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id IAA06429; Tue, 19 Apr 1994 08:32:19 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 19 Apr 94 08:38:11 PDT

Date: 19 Apr 94 09:26:18 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: ATA-2 Proposal Rev 2b available

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>,

"scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>

Message-Id: <"81626191404991/258261@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I have just completed uploading the proposal for ATA-2 Working Draft.  This

is revision 2b.  It is available on the SCSI BBS, in Area 9, Upload Area.

I have placed 3 versions:

0948DR2b.ps     Postscript output (I have never tested this file)

0948DR2b.txt    Flat text file

0948DR2b.doc    Word for Windows (2.0)

Those interested can download these files.

I will be bringing hard copy to the ATA Working Group meeting on April 27.

This meeting will be held at the Crown Sterling Suites in Milpitas from

9:30 AM to 4:00 PM.

======================================

Steve Finch, Chair ATA WG

Silicon Systems, Inc.

PH:  714 573-6808   FX:  714 573-6916

Email:  steve.finch@tus.ssi1.com

======================================

From ata-owner  Tue Apr 19 11:48:27 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA11998; Tue, 19 Apr 94 11:48:27 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA11991; Tue, 19 Apr 94 11:48:17 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id ad01270; 19 Apr 94 13:48 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 19 Apr 94 11:46:40 MDT

Subject: X3T10 Letter Ballots

To: ATA Reflector <ata@dt.wdc.com>,

SCSI Reflector List <scsi@WichitaKS.NCR.COM>

Date: Tue, 19 Apr 94 11:39:35 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9404191348.ad01270@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

There are seven (7) X3T10 letter ballots due to close May 12th, which

have been mailed directly to the X3T10 Principal members.  Copies of

these letter ballots are enclosed in the April '94 X3T10 mailing.  The

documents being balloted are in the X3T10 mailing:

Document Number                        Description

--------------------- ---------------------------------------------------

X3T10/94-090R0        Letter  Ballot for SAM Forwarding

X3T10/94-091R0        Letter  Ballot for SBP Forwarding

X3T10/94-092R0        Letter  Ballot for ATA-3 PH Project Proposal

X3T10/94-093R0        Letter  Ballot for ATA-3 XPT Project Proposal

X3T10/94-094R0        Letter  Ballot for ATA-3 PI Project Proposal

X3T10/94-095R0        Letter  Ballot for ATA-3 BC Project Proposal

X3T10/94-096R0        Letter  Ballot for Fast-20 Project Proposal

Apparently due to the large size of the X3T10 mailing (and/or the

trucking strike), the U.S. Postal Service has been rather slow in

delivering the mailing to some X3T10 members.  The mailing went out on

April 12th (technically in time for 30-day ballots ending May 12th).

I received my copy of the mailing on Friday April 15th, but some

members still haven't received it.

I encourage X3T10 Principals (or their designated Alternates) to

complete their letter ballots by May 12th and fax their responses to

me (at 719-597-8225), however I will extend the deadline for those who

need the extra time.

If you require extra time, please hand carry your ballots (and comments,

if any) to the Harrisburg meetings and give them to me no later than

Tuesday morning May 17th.  If you need the extension and are not

attending the Harrisburg meetings, please fax your responses to me at the

hotel (Fax 717-561-8398) and note on the Fax cover page that I am

a guest arriving 5/16/94.  In order to permit timely resolution of

comments, I plan to disclose the letter ballot comments on May 17th.

In all cases, I will need an electronic copy of any comments you have

on any of the ballots.

P L E A S E  use the extension only if you really need it.

John

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Thu Apr 21 12:33:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA03556; Thu, 21 Apr 94 12:33:26 PDT

Received: from gatekeeper.mcimail.com by dt.wdc.com (4.1/SMI-4.1)

id AA03549; Thu, 21 Apr 94 12:33:24 PDT

Received: by gatekeeper.mcimail.com (5.65/fma-120691);

id AA19758; Thu, 21 Apr 94 14:34:05 -0500

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id aa29428;

21 Apr 94 19:25 GMT

Date: Thu, 21 Apr 94 13:31 EST

From: STEVE ANDERSON <STEVE_ANDERSON+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ata <ata@dt.wdc.com>

Subject:

Message-Id: <52940421183125/0003946488NA3EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This message is for Tom Newman as a test to

establish a link.

From ata-owner  Sat Apr 23 19:37:33 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10007; Sat, 23 Apr 94 19:37:33 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA10000; Sat, 23 Apr 94 19:37:25 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id TAA27951; Sat, 23 Apr 1994 19:36:27 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Sat, 23 Apr 1994 19:31:14 EDT

Date:         Sat Apr 23 19:31:13 1994

From: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

Message-Id: <2db9afc3.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Vendor Unique Set Feature Subcommands

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This is a proposal to reserve two sets of Set Features subcommand

codes as vendor unique.  Each set would contain 16 subcommand

codes with one set in the 00H-7FH range and the other set in the

80H-FFH range.  This is in keeping with the current subcommand

definitions allowing which use a pair of codes, one from 00H-7FH

range to disable a feature and one from the 80H-FFH range to

enable a feature.

I proposed that the following be added to Table 13 in section

8.23 of the ATA-2 document:

1xh   Vendor Unique

Exh   Vendor Unique

plus add the following note:

All other codes are reserved for future use.

--

Hale Landis (Notes)  (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 7 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\723.MSG

### Org Date:    04-23-94 06:37:06 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Vendor Unique Set Feature Subcommands

### Attachments: none

From ata-owner  Sun Apr 24 02:15:06 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17450; Sun, 24 Apr 94 02:15:06 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA17443; Sun, 24 Apr 94 02:14:58 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id CAA15821; Sun, 24 Apr 1994 02:03:36 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Sat, 23 Apr 1994 23:54:27 EDT

Date:         Sat Apr 23 23:54:26 1994

From: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

Message-Id: <2db9ed74.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Format Track / Format Sector Commands

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This is a proposal to "fix" the problems with the Format Track

command.  This proposal includes the following:

1) The definition of a new optional command:  Format Sector.

2) Conversion of the Format Track command to optional and vendor

unique.

First, the Format Sector command.  I propose that the following

new command be added to the ATA-2 specification.  I leave the

command code to the ATA WG to select.

============================================================

8.x. FORMAT SECTOR

TYPE

Optional.

PROTOCOL

Non-data command.

INPUTS

The Feature register contains one of the following action codes:

00h - Format sector as good

20h - Unassign the alternate location for this sector

40h - Assign this sector to an alternate location

80h - Format sector as bad

Fxh - Vendor Unique

xxh - All other codes are reserved for future use.

A drive may support none, one or more of these codes.

The Cylinder High, Cylinder Low, Sector Number and bits 3-0 of

the Drive Head register specify the CHS or LBA address of the

sector to be acted on by this command.

OUTPUTS

None for Feature Register codes 00h, 20h, 40h and 80h.  Output

values for codes Fxh are vender unique.

PREREQUISITES

None.

DESCRIPTION

This command changes the status of a single sector address on a

drive such that it points to a a sector with the Bad Block flag

not set, a sector with the Bad Block flag set, an alternate

sector or to the original sector location.  Operations performed

by the drive to carry out these actions is vendor unique.  The

actions performed by this command may not be reversable.

This command does not affect the total number of addressable

sectors on a drive and it does not affect the way any sector on

the drive is addressed.

If the requested action is supported by the drive, the drive

shall set BSY, perform the requested action, clear BSY, and

assert INTRQ.

If the requested action is not supported by the drive or if there

is an error performing the requested action, the drive shall set

BSY, indicate the appropriate error in the Error register and

Error status in the Status register, clear BSY, and assert INTRQ.

ERRORS

ABRT - The drive does not support the command or does not

support the action requested.

IDNF - The specified sector could not be found.

BBK  - No spare sectors are available to perform an alternate

sector assignment.

============================================================

Second, I propose that the Format Track command be convert to an

optional command with vendor unique as the description and a

recommendation that if implemented, the command be implemented in

a manner that is compatible with the ATA-1 specification:

============================================================

8.9. FORMAT TRACK

TYPE

Optional.

PROTOCOL

Vendor unique.

INPUTS

Vendor unique.

OUTPUTS

Vendor unique.

PREREQUISITES

Vendor unique.

DESCRIPTION

Vendor unique, however, it is recommended that if this command is

implemented, that implementation should be compatible with the

ATA-1 implementation of this command.

ERRORS

ABRT - The drive does not support the command.

All other errors are vendor unique.

============================================================

--

Hale Landis (Notes)  (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 7 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\739.MSG

### Org Date:    04-23-94 10:16:36 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Format Track / Format Sector Commands

### Attachments: none

From ata-owner  Sun Apr 24 13:00:31 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28975; Sun, 24 Apr 94 13:00:31 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA28968; Sun, 24 Apr 94 13:00:23 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id NAA02898; Sun, 24 Apr 1994 13:00:33 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Sun, 24 Apr 1994 12:57:09 EDT

Date:         Sun Apr 24 12:57:08 1994

From: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

Message-Id: <2dbaa4e6.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Identify Drive Command Cleanup

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This is a proposal to "cleanup" the Identify Drive command

description.

This proposal includes the following:

1) Convert the Identify Drive command from optional to mandatory

status.

2) Redefine several Identify Drive data bits and words to vendor

unique status.

3) Add a text description of all words that are not reserved or

vendor unique.

First, it is time to convert the Identify Drive command to a

mandatory command since many ATA-2 basic features require that

the host use the Identify Drive command to configure those

features.

Second, it is time to convert several obsolete Identify Drive

data bits and words to vendor unique as they have no real meaning

in the ATA-2 world.  The following bits or entire words should be

redefined as vendor unique:

Word 0 bits 15, 14, 13, 12, 11, 10, 9, 8, 4, 3, 2, and 1.

Word 4 (unformatted bytes/track).

Word 5 (unformatted bytes/sector).

Word 20 (buffer type).

Third, a text description should be added after Table 11 for each

bit or word that is not reserved or vendor unique.  Many people

are confused by the fact that there is not text definition for a

bit or word that appears in the table.  The ATA committee

method of not providing text to define a bit or word that appears

to not need additional description beyond what is in Table 11 is

too confusing for many people.  In many cases the text

description need only repeat what is in the table.

--

Hale Landis (Notes)  (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 7 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\839.MSG

### Org Date:    04-24-94 11:39:50 AM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Identify Drive Command Cleanup

### Attachments: none

From ata-owner  Sun Apr 24 15:23:17 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01852; Sun, 24 Apr 94 15:23:17 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA01844; Sun, 24 Apr 94 15:23:11 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id PAA02524; Sun, 24 Apr 1994 15:20:10 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Sun, 24 Apr 1994 15:09:40 EDT

Date:         Sun Apr 24 15:09:39 1994

From: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

Message-Id: <2dbac3f4.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Drive Address register

Sender: ata-owner@dt.wdc.com

Precedence: bulk

It is time to "do away" with the Drive Address register.  The

PCMCIA has already done this and now the ATA WG should too.  The

register has no use in the ATA-2 world and it "shares" a data bit

with another device -- a situtation that is very difficult to

handle at the hardware level.

In section 6.3, Table 6 should be updated as follows:

|------+------+-----+-----+-----+------------------------------------------|

|                               |        Control block registers           |

|-------------------------------+------------------------------------------|

|  N   |  A   |  0  |  x  |  x  | Data bus high imped | Not used           |

|  N   |  A   |  1  |  0  |  x  | Data bus high imped | Not used           |

|  N   |  A   |  1  |  1  |  0  | ALTERNATE STATUS    | DEVICE CONTROL     |

|  N   |  A   |  1  |  1  |  1  | Data bus high imped | Not used (see note)|

|-------------------------------+------------------------------------------|

Note:  This is the address of the ATA-1 DRIVE ADDRESS register.

This register is obsolete.  If implemented, the use and contents

are vendor unique.

In section 6.3.7, the Drive Address register should be described

as follows:

This register is obsolete and it is recommend that it not be

implemented.  An ATA-2 device should not respond to this

address.  If it is implemented, the contents and use are

vendor unique.

--

Hale Landis (Notes)  (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 7 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\861.MSG

### Org Date:    04-24-94 01:29:19 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Drive Address register

### Attachments: none

From ata-owner  Sun Apr 24 15:24:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01866; Sun, 24 Apr 94 15:24:16 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA01853; Sun, 24 Apr 94 15:24:06 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id PAA02592; Sun, 24 Apr 1994 15:20:36 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Sun, 24 Apr 1994 15:09:47 EDT

Date:         Sun Apr 24 15:09:46 1994

From: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

Message-Id: <2dbac3fc.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Interrupting commands

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This is a proposal to "fix" the "interrupting a command with a

command" problem.  As ATA devices become more complex and more

command protocol functions are taken over by hardware sequencers,

the current "interrrupting a command with a command"

specification looks more and more silly if not impossible to

implement.

I also propose to add some additional wording to make it clear

that a device must not allow both BSY and DRQ to be zero during

the execution of a command.  This is necessary to insure that a

host system can operate without requiring the host to use the

INTRQ signal.

In section 9, the following paragraphs should be replaced:

If a new command is issued to a device which has an

uncompleted command (subsequently referred to as Old Command)

in progress, the device shall immediately respond to the new

command (Subsequently referred to as New Command), even if

execution of the Old Command could have been completed.

There shall be no indication given to the system as to the

status of the Old Command which was being executed at the time

the New Command was issued.

The only way to interrupt a Read DMA or Write DMA command is

with a reset.  The result of interrupting a Read DMA or Write

DMA command by writing a new command into the Command Register

is unpredictable.

with the following paragraphs:

A device must maintain either BSY=1 or DRQ=1 at all times

until the command is completed.  The INTRQ signal is used by

the device to signal most, but not all, times when the BSY bit

is changed from 1 to 0 during command execution.

A command may be interrupted with a reset.  The result of

interrupting a command by writing a new command into the

Command Register is unpredictable and may result in data

corruption.  A command should only be interrupted by a reset

at times when the host thinks there may be a problem, such as

a device that is no longer responding.  Host programmers are

warned against setting unrealistically short command timeout

periods since this may impact a device's ability to perform

device level retry and data recovery activities.

--

Hale Landis (Notes)  (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 7 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\863.MSG

### Org Date:    04-24-94 02:12:11 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Interrupting commands

### Attachments: none

From ata-owner  Sun Apr 24 17:43:34 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04504; Sun, 24 Apr 94 17:43:34 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA04438; Sun, 24 Apr 94 17:43:23 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA28571; Sun, 24 Apr 1994 17:34:53 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Sun, 24 Apr 1994 17:28:07 EDT

Date:         Sun Apr 24 17:28:06 1994

From: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

Message-Id: <2dbae467.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      SFF-8019 rev 0.5 aka X3T10 doc 94-089r0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

SFF-8019 rev 0.5 aka X3T10 document 94-089r0

24 April 94

by Hale Landis, Seagate

I'm sorry I'm so slow, but my work schedule lately has simply not

allowed me time to update this document until today.

Because I have been so slow in updating the SFF-8019 document,

Dal Allan prepared a new rev, rev 1.0.  It does not appear that

Dal distributed his rev 1.0 document to anyone but me, but I'm

not sure.

And unfortunately, it appears that Dal based his rev 1.0 on my

rev 0.3 and there were significant changes between my rev 0.3 and

rev 0.4.  It is unclear to me which rev the SFF committee voted

on at the Newport Beach meeting, but rev 0.3 was the version

distributed in the SFF mailing prior to that meeting.  However, I

presented rev 0.4 at the meeting and I was asked to make some

specific, but minor, changes which would result in rev 0.5.

To add to the confusion, Larry Lamars has assigned a X3T10

document number to this proposal:  94-089r0.

Having said all that, and I hope you are not too confused by all

this seemingly random document updating, here is what I call rev

0.5.  Note that the X3T10 document 94-089r0 consists of only

section 5.x of the SFF-8019 document.

This is based on my rev 0.4 as presented at the Newport Beach

meeting.  Too save paper and electrons, I present only paragraphs

from sections 1.x - 4.x of SFF-8019 that are changed.  The entire

section 5.x of SFF-8019 is presented here.

Changes from rev 0.4 have a vertical bar (|) at the left margin.

----------

| Abstract:  This document defines the presentation of the Identify

| Drive data for ATA drives less than or equal to 8 GB.

----------

1. Scope

| This document defines the manner in which the Identify Drive data

| supplied by an ATA drive will be presented by drives of less than

| or equal to 8 GB capacity.

| The purpose of this SFF Specification is to define a common

| manner in which all manufacturers of large capacity drives can

| present Identify Drive data, so that system implementors, BIOS

| (Basic Input/Output System) writers, device driver programmers,

| and other integrators of ATA drives can use the contents in a

| consistent manner.

In an effort to broaden the applications for disk drives, an ad

hoc industry group of companies representing system integrators,

peripheral suppliers, and component suppliers decided to address

the issues involved.

The Small Form Factor Committee was formed in August, 1990 and

the first working document was introduced in January, 1991.

----------

5. Identify Drive Data Requirements

5.1 Background Information

The original IBM PC BIOS (Basic Input/Output System) imposed

several restrictions on the support of disk drives, and these

have been incorporated into many higher level software products.

One such restriction limits the capacity of a hard disk drive.

Most BIOS software cannot support a disk drive with more than

1,024 cylinders, 16 heads and 63 sectors per track.  The maximum

addressable capacity of an ATA disk drive under this scheme is

1,032,192 sectors or 528,482,304 bytes or 528 MB.

There is growing support of auto-configuration for disk drives on

PC systems.  The auto-configuration capability usually resides in

the BIOS and uses the Identify Drive command data to configure an

ATA disk drive.

| This specification defines rules for the Identify Drive data of

| large capacity ATA disk drives and allows BIOS support of ATA

| drives up to 16,515,072 sectors or 8,455,716,864 bytes or 8 GB

| using Cylinder/Head/Sector (CHS) addressing.

It is unlikely that BIOS architecture based on CHS addressing can

be extented beyond 8 GB.

This specification defines information that newer BIOSs and

system software can use to determine the true size of a disk

drive and access the full capacity of the drive.

5.2 Cylinder, Head and Sector Addressing

BIOSs and other software that operate an ATA disk drive in CHS

(Cylinder, Head and Sector) addressing mode use Identify Drive

data words 1, 3, 6 and words 53-58 to ascertain the appropriate

translation mode to use and determine the capacity of an ATA disk

drive.

Maximum compatibility is achieved if the following rules are

obeyed.  These rules limit the values placed into words 1, 3, 6,

and 53-58.  The rules specified here for CHS addressing apply

to drives up to 8 GB in size.

5.2.1 Word 1

| For drives less than or equal to 528 MB, Identify Data word 1

| (Default Cylinders) shall not specify a value greater than 1,024.

| If a drive is greater than 528 MB but less than or equal to 8 GB,

| this word shall not specify a value greater than 16,383.  There

| is no restriction on the value of this word for drives over 8 GB.

| The contents of this word shall not change.

5.2.2 Word 3

Identify Data word 3 (Default Heads) shall not specify a value

greater than 16.  If a drive is greater than 528 MB this word

shall contain a value of 16.  The contents of this word shall not

change.

5.2.3 Word 6

| For drives of 8 GB or less, Identify Data word 6 (Default Sectors)

| shall not specify a value greater than 63.  For drives over 8 GB,

| there is no restriction on the value of this word except that it

| shall not exceed 255.  The content of this word shall not change.

5.2.4 Use of words 53 through 58

ATA drives that are over 528 MB shall implement words 53-58.

Drives not over 528 MB may also implement these words.  These

words define the addressing for all sectors accessible in CHS

mode.

5.2.5 Word 53

Identify Data word 53 bit 0 shall be 1 at all times the drive is

in a valid translation mode.  Some drives may have translation

modes that cannot be supported.  An attempt to put a drive into

one of these unsupported modes shall cause word 53 bit 0 to be 0

with words 54-58 cleared to zero until a valid translation mode

is established.

5.2.6 Word 54

| Identify Data word 54 (Current Cylinders) shall specify the

| number of full logical cylinders that can be accessed in the

| current translation mode.  If an Initialize Drive Parameters

| command has not been executed, the contents of this word shall be

| the same as word 1. If an Initialize Drive Parameters command has

| been executed, this word is the integer result of dividing the

| total number of user sectors (this value may be in words 60-61)

| by the number of sectors per logical cylinder ( [word55] x

| [word56] ).

5.2.7 Word 55

Identify Data word 55 (Current Heads) is the number of heads

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 16.  If an Initialize

Drive Parameters command has not been executed, the contents of

this word shall be the same as word 3.

5.2.8 Word 56

Identify Data word 56 (Current Sectors) is the number of sectors

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 255.  If an Initialize

Drive Parameters command has not been executed, the contents of

this word shall be the same as word 6.

5.2.9 Words 57-58

Identify Data words 57-58 contain a 32-bit value that shall be

equal to [word54] x [word55] x [word56].  If words 60-61 (LBA

sectors) are not zero, words 57-58 shall be less than or equal to

the value in words 60-61 at all times.

The 32-bit value is placed into these words as follows:

data bits  7 -  0 in word 57 bits 15 - 8,

data bits 15 -  8 in word 57 bits  7 - 0,

data bits 23 - 16 in word 58 bits 15 - 8,

data bits 31 - 24 in word 58 bits  7 - 0.

5.3 Logical Block Addressing

It is recommended that ATA drives over 528 MB support Logical

Block Addressing (LBA).

5.3.1 Words 60-61

Identify Data words 60-61 shall specify the total number of user

sectors available in LBA mode at all times.  This value shall be

equal to or greater than the value in words 57-58 at all times.

The contents of these words shall not change.

The 32-bit value is placed into these words as follows:

data bits  7 -  0 in word 60 bits 15 - 8,

data bits 15 -  8 in word 60 bits  7 - 0,

data bits 23 - 16 in word 61 bits 15 - 8,

data bits 31 - 24 in word 61 bits  7 - 0.

5.3.2 Orphan Sectors

The sectors, if any, between the last sector addressable in CHS

mode and the last sector addressable in LBA mode are known as

"orphan" sectors.  A drive may or may not allow access to these

sectors in CHS addressing mode.

| The values in words 1, 3 and 6 should be selected such that the

| number of orphan sectors is minimized.  Normally, the number of

| orphan sectors should not exceed ( [word55] x [word56] - 1 ).

| However, the host system can create conditions where there are a

| larger number of orphans sectors by issuing the Initialize Drive

| Parameters command with values other than the values in words 3

| and 6.

--

Hale Landis (Notes)  (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 7 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\877.MSG

### Org Date:    04-24-94 04:23:14 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     SFF-8019 rev 0.5 aka X3T10 doc 94-089r0

### Attachments: none

From ata-owner  Sun Apr 24 22:45:44 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10055; Sun, 24 Apr 94 22:45:44 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA09840; Sun, 24 Apr 94 22:30:37 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id WAA02663; Sun, 24 Apr 1994 22:27:31 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Sun, 24 Apr 1994 22:06:11 EDT

Date:         Sun Apr 24 22:06:10 1994

From: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

Message-Id: <2dbb2594.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis (Notes)" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Master/Slave Handshaking -- 94065r2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Master/Slave Handshaking

Document 94065r2

24 April 94

by Hale Landis, Seagate

This document proposes changes to Annex A and B that add

references to the DRDY status bit and clear up some other

misleading information.  Please refer to my previous posting on

this subject for additional background information.

Note that Annex B in the ATA-2 rev 2b document is formatted

strangely -- looks like a word processor's automatic outline mode

was used but it did work right.  I have added outline style

numbering to all paragraphs of my proposal.  This gives us a way

to refer to an individual paragraph by a unique section number.

I recommend the following changes to the Annex A and Annex B

text.  My changes are marked by a vertical bar (|) at the left

margin.

I suggest that you review this carefully -- it is relatively

complex and I could have make a mistake (yep, it can happen).

----- Annex A -----

Annex A Diagnostic and reset considerations

(informative)

This annex describes the following timing relationships during:

a) Power on and hardware resets

- One drive

- Two drives

b) Software reset

- One drive

- Two drives

c) Diagnostic command execution

- One drive

- Two drives

- Two drives - drive 1 failed

The timing assumes the following:

a) DASP- is asserted by Drive 1 and received by Drive 0 at

power-on or hardware reset to indicate the presence of Drive

1. At all other times it is asserted by Drive 0 and Drive 1 to

indicate when a drive is active.

b) PDIAG- is asserted by Drive 1 and detected by Drive 0. It is

used by Drive 1 to indicate to Drive 0 that it has completed

diagnostics and is ready to accept commands from the Host (BSY

bit is cleared).  This does not indicate that the drive is

ready, only that it can accept commands.  This line may remain

asserted until the next reset occurs or an Execute Diagnostic

command is received.

c) Unless indicated otherwise, all times are relative to the event

that triggers the operation (RESET-, SRST=1, Execute

Diagnostic Command).

A.1 Power on and hardware resets

A.1.1 Power on and hardware resets - one drive

a) Host asserts RESET- for a minimum of 25 usec.

b) Drive 0 sets BSY within 400 nsecs after RESET- is negated.

c) Drive 0 negates DASP- within 1 msec after RESET- negated.

d) Drive 0 performs hardware initialization

e) Drive 0 may revert to its default condition

f) Drive 0 waits 1 msec then samples for at least 450 msec for

DASP- to be asserted from Drive 1.

|  g) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 31 seconds from the

|     time that RESET- is asserted.

|  h) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps g) and h) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 9.

A.1.2 Power on and hardware resets - two drives

a) Host asserts RESET- for a minimum of 25 usec.

b) Drive 0 and Drive 1 set BSY within 400 nsec after RESET-

negated.

c) DASP- is negated within 1 msec after RESET- is negated.

A.1.2.1 Drive 1

a) Drive 1 negates PDIAG- before asserting DASP-.

b) Drive 1 asserts DASP- within 400 msecs after RESET- (to show

presence).

c) Drive 1 performs hardware initialization and executes its

internal diagnostics.

d) Drive 1 may revert to its default condition

e) Drive 1 posts diagnostic results to the Error Register

|  f) Drive 1 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|  g) Drive 1 asserts PDIAG- to indicate that it is ready to accept

|     commands.

|     NOTE: Drive 1 must clear BSY and assert PDIAG- within 30

|     seconds from the time RESET- is asserted.

|  h) Drive 1 sets DRDY when ready to accept any command.

|     NOTE: Steps f), g) and h) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 9.

|  i) Drive 1 negates DASP- after the first command is received or

|     negates DASP- if no command is received within 31 seconds after

|     RESET- is asserted.

A.1.2.2 Drive 0

a) Drive 0 performs hardware initialization and executes its

internal diagnostics.

b) Drive 0 may revert to its default condition

c) Drive 0 posts diagnostic results to the Error Register

d) After 1 msec, Drive 0 waits at least 450 msec for DASP- to be

asserted (from Drive 1).  If DASP- is not asserted, no Drive 1

is present (see section A.1.1).

e) Drive 0 waits up to 31 seconds for Drive 1 to assert PDIAG-.

If PDIAG- is not asserted, Drive 0 sets Bit 7=1 in the Error

Register.

|  f) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 31 seconds from the

|     time when RESET- is asserted.

|  g) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps f) and g) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 9.

__

RESET- ___/  \_________________________________________

____________________

BSY    ____/                    \______________________

___

DRDY   ____________________________________________/

|<---------------->|<---------------->|

Max for drive 0    Minimun time is

is 31 seconds.     0 seconds.

Max for drive 1    No maximum time

is 30 seconds.     is specified.

Figure 9- BSY and DRDY timing for power on and hardware resets

A.2 Software reset

A.2.1  Software reset - one drive

a) Host sets SRST=1 in the Device Control register.

b) Drive 0 sets BSY within 400 nsec after detecting that SRST=1.

c) Drive 0 performs hardware initialization and executes its

internal diagnostics.

d) Drive 0 may revert to its default condition.

e) Drive 0 posts diagnostic results to the Error Register.

|  f) Drive 0 waits for the host to set SRST=0.

|  g) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 31 seconds from the

|     time that the host sets SRST=0.

|  h) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps g) and h) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 10.

A.2.2 Software reset - two drives

a) Host sets SRST=1 in the Device Control register.

b) Drive 0 and Drive 1 set BSY within 400 nsec after detecting

that SRST=1.

c) Drive 0 and Drive 1 perform hardware initialization.

d) Drive 0 and Drive 1 may revert to their default condition.

A.2.2.1 Drive 1

a) Drive 1 negates PDIAG- within 1 msec.

|  b) Drive 1 waits for the host to set SRST=0.

|  c) Drive 1 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|  d) Drive 1 asserts PDIAG- to indicate that it is ready to accept

|     commands.

|     NOTE: Drive 1 must clear BSY and assert PDIAG- within 30

|     seconds from the time the host sets SRST=0.

|  e) Drive 1 sets DRDY when ready to accept any command.

|     NOTE: Steps b), c) and d) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 10.

A.2.2.2 Drive 0

a) Drive 0 waits up to 31 seconds for Drive 1 to assert PDIAG-.

|  b) Drive 0 waits for the host to set SRST=0.

|  c) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 31 seconds from the

|     time when the host sets SRST=0.

|  d) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps c) and d) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 10.

__

SRST ___/  \_________________________________________

_____________________

BSY  ___/                     \______________________

___

DRDY ____________________________________________/

|<---------------->|<---------------->|

Max for drive 0    Minimun time is

is 31 seconds.     0 seconds.

Max for drive 1    No maximum time

is 30 seconds.     is specified.

Figure 10- BSY and DRDY timing for software reset

A.3 Diagnostic Command Execution

A.3.1 Diagnostic command execution - one drive (passed)

a) Drive 0 sets BSY within 400 nsec after the Execute Diagnostic

command was received.

b) Drive 0 performs hardware initialization and internal

diagnostics.

c) Drive 0 resets Command Block registers to default condition.

d) Drive 0 posts diagnostic results to the Error Register

|  e) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 6 seconds from the

|     time that RESET- is asserted.

|  f) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps e) and f) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 11.

A.3.2 Diagnostic command - two drives (passed)

a) Drive 0 and Drive 1 set BSY within 400 nsec after the Execute

Diagnostic command was received.

A.3.2.1 Drive 1

a) Drive 1 negates PDIAG- within 1 msec after command received.

b) Drive 1 performs hardware initialization and internal

diagnostics.

c) Drive 1 resets the Command Block registers to their default

condition.

d) Drive 1 posts diagnostic results to the Error Register

|  e) Drive 1 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|  f) Drive 1 asserts PDIAG- to indicate that it is ready to accept

|     commands.

|     NOTE: Drive 1 must clear BSY and assert PDIAG- within 5

|     seconds from the time RESET- is asserted.

|  g) Drive 1 sets DRDY when ready to accept any command.

|     NOTE: Steps e), f) and g) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 11.

A.3.2.2 Drive 0

a) Drive 0 performs hardware initialization and internal

diagnostics.

b) Drive 0 resets the Command Block registers to their default

condition.

c) Drive 0 waits up to 6 seconds for Drive 1 to assert PDIAG-.

d) Drive 0 posts diagnostic results to the Error Register

|  f) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 6 seconds from the

|     time when RESET- is asserted.

|  g) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps f) and g) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 11.

A.3.3  Diagnostic command execution - one drive (failed)

a) Drive 0 sets BSY within 400 nsec after Diagnostic command

received.

b) Drive 0 performs hardware initialization and internal

diagnostics.

c) Drive 0 resets Command Block registers to default condition.

d) Drive 0 posts a Diagnostic Code to the Error Register

indicating a failure.

|  e) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 6 seconds from the

|     time when RESET- is asserted.

|  f) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps e) and f) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 11.

A.3.4 Diagnostic command execution - two drives (drive 1 failed)

a) Drive 0 and Drive 1 set BSY within 400 nsec after Diagnostic

command received.

A.3.4.1 Drive 1

a) Drive 1 negates PDIAG- within 1 msec after command received.

b) Drive 1 performs hardware initialization and internal

diagnostics.

c) Drive 1 resets the Command Block registers to their default

condition.

d) Drive 1 posts a Diagnostic Code to the Error Register

indicating failure.

|  e) Drive 1 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|  f) Drive 1 does not assert PDIAG-, indicating that it failed

|     diagnostics.

|     NOTE: Drive 1 must clear BSY within 5 seconds from the

|     time RESET- is asserted.

|  g) Drive 1 sets DRDY when ready to accept any command.

|     NOTE: Steps e), f) and g) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 11.

A.3.4.2 Drive 0

a) Drive 0 performs hardware initialization and internal

diagnostics.

b) Drive 0 resets the Command Block registers to their default

condition.

c) Drive 0 waits 6 seconds for Drive 1 to assert PDIAG- but PDIAG-

is not asserted by Drive 1.

d) Drive 0 posts a Diagnostic Code to the Error Register setting

Bit 7=1 to indicate that Drive 1 failed diagnostics.

|  e) Drive 0 clears BSY when ready to accept commands that do not

|     require DRDY=1.

|     NOTE: Drive 0 must clear BSY within 6 seconds from the

|     time when RESET- is asserted.

|  f) Drive 0 sets DRDY when ready to accept any command.

|     NOTE: Steps e) and f) may occur at the same time.  No maximum

|     time limit is specified for DRDY=1 to occur.  See Figure 11.

__________________

BSY  ___/                  \______________________________

___________

DRDY _________________________________________/

|<---------------->|<---------------->|

Max for drive 0    Minimun time is

is  6 seconds.     0 seconds.

Max for drive 1    No maximum time

is  5 seconds.     is specified.

Figure 11- BSY and DRDY timing for Diagnostic command

----- end Annex A -----

----- Annex B -----

Annex B. Diagnostic and reset considerations

(informative)

B.1. Power on and hardware reset (RESET-)

DASP- is read by Device 0 to determine if Device 1 is present.

If Device 1 is present Device 0 will read PDIAG- to determine

when it is valid to clear BSY and whether Device 1 has powered on

or reset without error, otherwise Device 0 clears BSY whenever it

is ready to accept commands.  Device 0 may assert DASP- to

indicate device activity.

B.2. Software reset

If Device 1 is present Device 0 will read PDIAG- to determine

when it is valid to clear BSY and whether Device 1 has reset

without any errors, otherwise Device 0 will simply reset and

clear BSY.  DASP- is asserted by Device 0 (and Device 1 if it is

present) in order to indicate device active.

B.3. Device diagnostic command

If Device 1 is present, Device 0 will read PDIAG- to determine

when it is valid to clear BSY and if Device 1 passed or failed

the EXECUTE DEVICE DIAGNOSTIC command, otherwise Device 0 will

simply execute its diagnostics and then clear BSY.  DASP- is

asserted by Device 0 (and Device 1 if it is present) in order to

indicate the device is active.

B.4. Truth table

In all the above cases:  Power on, RESET-, software reset, and

the EXECUTE DEVICE DIAGNOSTIC command the Device 0 Error register

is calculated as follows:

+-----------+------------+-----------+-----------+

|  Device 1 |    PDIAG-  |  Device 0 |  Error    |

|  Present? |  Asserted? |  Passed   |  Register |

+-----------+------------+-----------+-----------+

|    Yes    |    Yes     |    Yes    |    01h    |

|    Yes    |    Yes     |    No     |    0xh    |

|    Yes    |    No      |    Yes    |    81h    |

|    Yes    |    No      |    No     |    8xh    |

|    No     | (not read) |    Yes    |    01h    |

|    No     | (not read) |    No     |    0xh    |

+-----------+------------+-----------+-----------+

Where x indicates the appropriate Diagnostic Code for the

Power on, RESET-, software reset, or device diagnostics error.

Table 14 - Reset Error register Values

B.5. Power on or hardware reset algorithm

1) Power on or hardware reset

2) The hardware should automatically do the following:

a) Set up the hardware to post both Device 0 and Device 1 status

b) Set the Device 0 Status register to 80h (set BSY and clear all

the other status bits)

c) Set the Device 1 Status register to 80h (set BSY and clear all

the other status bits)

| 3) A drive must determine if it is Device 0 or Device 1.  This can

|    be done at least two different ways: by jumper or by using the

|    CSEL signal.

4) Perform any remaining time critical hardware initialization

including starting the spin up of the device if needed

5) If Device 1

a) Negate the PDIAG- signal

b) Set up PDIAG- as an output

c) Assert the DASP- output

d) Set up DASP- as an output if necessary

e) Set up the hardware so it posts Device 1 status only and

continue to post 80h for Device 1 status

NOTE:  All of this must happen within 400 msec after power on

or RESET-

If Device 0

a) Set up PDIAG- as an input

b) Release DASP- and set up DASP- as an input

c) Test DASP- for 450 msec or until DASP- is asserted by Device 1

|     d) If DASP- is asserted within 450 msec,

|           note that Device 1 is present and

|           set up the hardware so it posts Device 0 status only

|        else

|        If DASP- is not asserted within 450 msec,

|           note that Device 1 is not present and

|           set up the hardware so it posts Device 0 and 1 status

e) Assert DASP- to indicate device activity

6) Complete all the hardware initialization needed to get the

device ready, including:

a) Set the Sector Count register to 01h

b) Set the Sector Number register to 01h

c) Set the Cylinder Low register to 00h

d) Set the Cylinder High register to 00h

e) Set the Device/Head register to 00h

7) If Device 1 and power on, or RESET- is valid

a) Set the Error register to Diagnostic Code 01h

b) Set the Device 1 Status register to 00h

c) Assert PDIAG-

NOTE:  All of this must happen within 30 seconds of power on

or the negation of RESET-

If Device 1 and power on or RESET- bad

a) Set the Error register to the appropriate Diagnostic Code

b) Set the Device 1 Status register to 00h

NOTE:  All of this must happen within 30 seconds of power on

or the negation of RESET-

If Device 0, power on or RESET- valid, and a Device 1 is

present

a) Test PDIAG- for 31 seconds or until PDIAG- is asserted by

Device 1

|     b) If PDIAG- is asserted within 31 seconds,

|           set the Error register to Diagnostic Code 01h

|        else

|        If PDIAG- is not asserted within 31 seconds,

|           set the Error register to 81h

c) Set the Device 0 Status register to 00h

If Device 0, power on or RESET- bad, and a Device 1 is present

a) Test PDIAG- for 31 seconds or until PDIAG- is asserted by

Device 1

|     b) If PDIAG- is asserted within 31 seconds,

|           set the Error register to the appropriate Diagnostic Code

|        else

|        If PDIAG- is not asserted within 31 seconds,

|           set the Error register to 80h plus the appropriate code

c) Set the Device 0 Status register to 00h

If Device 0, power on or RESET- valid, and no Device 1 is

present

a) Set the Error register to Diagnostic Code 01h

b) Set the Device 1 Status register to 00h

c) Set the Device 0 Status register to 00h

If Device 0, power on or RESET- bad, and no Device 1 is

present

a) Set the Error register to the appropriate Diagnostic Code

b) Set the Device 1 Status register to 00h

c) Set the Device 0 Status register to 00h

8) Finish spin up if needed

9) If Device 1

a) Set the Device 1 Status register to 50h

b) Negate DASP- if a command is not received within 31 seconds

If Device 0 and a Device 1 is present

a) Set the Device 0 Status register to 50h

b) Negate DASP-

If Device 0 and no Device 1 is present

a) Leave the Device 1 Status register 00h

b) Set the Device 0 Status register to 50h

c) Negate DASP-

B.6 Software Reset Algorithm

1) The software reset bit is set

2) If Device 1

|     a) The hardware should set 80h in the Device 1 Status register

b) Negate the PDIAG- signal

NOTE:  This must happen within 1 msec of the software reset

If Device 0 and Device 1 is present

|     a) The hardware should set 80h in the Device 0 Status register

If Device 0 and there is no Device 1 the hardware should:

|     a) Set 80h in the Device 0 Status register

b) Set the Device 1 Status register to 80h

3) Assert DASP-

4) Finish all the hardware initialization needed to place the

device in reset

5) Wait for the software reset bit to clear

6) Finish all hardware initialization needed to get the device

ready to receive any type of command from the host including:

a) Set the Sector Count register to 01h

b) Set the Sector Number register to 01h

c) Set the Cylinder Low register to 00h

d) Set the Cylinder High register to 00h

e) Set the Device/Head register to 00h

7) If Device 1 and reset valid

a) Set the Error register to Diagnostic Code 01h

b) Set the Device 1 Status register to 50h

c) Assert PDIAG-

NOTE:  All of this must happen within 30 seconds of the

clearing of the software reset bit

If Device 1 and reset bad

a) Set the Error register to the appropriate Diagnostic Code

b) Set the Device 1 Status register to 50h

NOTE:  All of this must happen within 30 seconds of the

clearing of the software reset bit

If Device 0, reset valid, and a Device 1 is present

a) Test PDIAG- for 31 seconds or until PDIAG- is asserted by

Device 1

|     b) If PDIAG- is asserted within 31 seconds,

|           set the Error register to Diagnostic Code 01h

|        else

|        If PDIAG- is not asserted within 31 seconds,

|           set the Error register to 81h

c) Set the Device 0 Status register to 50h

If Device 0, reset bad, and a Device 1 is present

a) Test PDIAG- for 31 seconds or until PDIAG- is asserted by

Device 1

|     b) If PDIAG- is asserted within 31 seconds,

|           set the Error register to the appropriate Diagnostic Code

|        else

|        If PDIAG- is not asserted within 31 seconds,

|           set the Error register to 80h plus the appropriate code

c) Set the Device 0 Status register to 50h

If Device 0, reset valid, and no Device 1 is present

a) Set the Error register to Diagnostic Code 01h

b) Set the Device 1 Status register to 00h

c) Set the Device 0 Status register to 50h

If Device 0, reset bad, and no Device 1 is present

a) Set the Error register to the appropriate Diagnostic Code

b) Set the Device 1 Status register to 00h

c) Set the Device 0 Status register to 50h

B.7.  Diagnostic Command Algorithm

1) The diagnostics command is received

2) If Device 1

|     a) The hardware should set 80h in the Device 1 Status register

|     b) Negate the PDIAG- signal within 1 msec after command acceptance

If Device 0 and Device 1 is present

|     a) The hardware should set 80h in the Device 0 Status register

If Device 0 and there is no Device 1 the hardware should

|     a) Set 80h in the Device 0 Status register

|     b) Set 80h in the Device 1 Status register

3) Assert DASP-

4) Perform all the device diagnostics and note their results

5) Finish all the hardware initialization needed to get the

device ready to receive any type of command from the host

including:

a) Set the Sector Count register to 01h

b) Set the Sector Number register to 01h

c) Set the Cylinder Low register to 00h

d) Set the Cylinder High register to 00h

e) Set the Device/Head register to 00h

6) If Device 1 and passed

a) Set the Error register to Diagnostic Code 01h

b) Set the Device 1 status to 50h

c) Assert PDIAG-

NOTE:  All of this must happen within 5 seconds of the

acceptance of the diagnostic command

If Device 1 and did not pass

a) Set the Error register to the appropriate Diagnostic Code

b) Set the Device 1 status to 50h

NOTE:  All of this must happen within 5 seconds of the

acceptance of the diagnostic command

If Device 0, passed, and a Device 1 is present

a) Test PDIAG- for 6 seconds or until PDIAG- is asserted by

Device 1

|     b) If PDIAG- is asserted within 6 seconds,

|           set the Error register to Diagnostic Code 01h

|        else

|        If PDIAG- is not asserted within 6 seconds,

|           set the Error register to 81h

c) Set the Device 0 status to 50h

d) Issue interrupt to the host

If Device 0, did not pass, and a Device 1 is present

a) Test PDIAG- for 6 seconds or until PDIAG- is asserted by

Device 1

|     b) If PDIAG- is asserted within 6 seconds,

|           set the Error register to the appropriate Diagnostic Code

|        else

|        If PDIAG- is not asserted within 6 seconds,

|           set the Error register to 80h plus the appropriate code

c) Set the Device 0 Status register to 50h

d) Issue interrupt to the host

If Device 0, passed, and no Device 1 is present

a) Set the Error register to Diagnostic Code 01h

b) Set the Device 1 Status register to 00h

c) Set the Device 0 Status register to 50h

d) Issue interrupt to the host

If Device 0, did not pass, and no Device 1 is present

a) Set the Error register to the appropriate Diagnostic Code

b) Set the Device 1 Status register to 00h

c) Set the Device 0 Status register to 50h

d) Issue interrupt to the host

----- end Annex B -----

--

Hale Landis (Notes)  (Hale_Landis@notes.seagate.com)

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 7 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\927.MSG

### Org Date:    04-24-94 09:25:57 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Master/Slave Handshaking -- 94065r2

### Attachments: none

From ata-owner  Mon Apr 25 10:41:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23992; Mon, 25 Apr 94 10:41:42 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA23982; Mon, 25 Apr 94 10:41:27 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id KAA16353; Mon, 25 Apr 1994 10:37:03 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Mon, 25 Apr 1994 09:38:34 EDT

Date:         Mon Apr 25 09:38:34 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2dbbc7db.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Sorry Dal...

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Oppps, I was wrong... Dal Allan's rev 1.0 of SFF-8019 WAS based on my SFF-8019

rev 0.4.  I'm sorry... It would appear that I'm just adding to the confusion.

I did receive a message from Dal this morning pointing out that the changes I

made between my rev 0.4 and 0.5 may not agree with the minutes of the Newport

Beach SFF meeting.  I have not seen those minutes but if I did not update the

document as discussed at that meeting, then I'm really adding to the

confusion.  I'll investigate today and let you know what the status is.

We must now be aware that it is now possible that SFF-8019 may not stay in sync

with X3T10 94-089rx since two different commitees are "working" on the same

thing.  I don't think any of us want that to happen and I'll do my best to

prevent it in this case.

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\1031.MSG

### Org Date:    04-25-94 09:21:03 AM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Sorry Dal...

### Attachments: none

From ata-owner  Mon Apr 25 14:29:30 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AB28975; Mon, 25 Apr 94 14:29:30 PDT

Received: from rutgers.edu by dt.wdc.com (4.1/SMI-4.1)

id AA28968; Mon, 25 Apr 94 14:29:25 PDT

Received: from cbmvax.cbm.commodore.com by rutgers.edu (5.59/SMI4.0/RU1.5/3.08)

id AA16831; Mon, 25 Apr 94 17:29:22 EDT

Received: by cbmvax.cbm.commodore.com

(5.65/UUCP-Project/Commodore - Ultrix 4.3 Ver 07/12/93)

id AA15328; Mon, 25 Apr 94 17:31:04 -0400

Date: Mon, 25 Apr 94 17:31:04 -0400

From: jesup@cbmvax.cbm.commodore.com (Randell Jesup)

Message-Id: <9404252131.AA15328@cbmvax.cbm.commodore.com>

To: ata@dt.wdc.com

Subject: Identify Drive

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Re: redefining Word 0 bits as Vendor Unique

Some of the word 0 bits used to have some sort of definition, I think.

If they're made vendor-unique, then some note of their former status might

be in order, like "xxx in ATA-1, superceded by bit N in word Y in ATA-2".

Also, defining them as "obsolete" would be better than "vendor-unique".  This

reduces the likelyhood that some "clever" engineer will break backwards-

compatibility by using them for something different.  Again, this would only

apply to bits that actually had written definitions in ATA-1.

Never forget that people using this spec to write drivers might not

be experts on ATA-1 (or ATA-2), and any driver will have to deal with

older drives (and newer firmware will have to deal with older drivers).

--

Disclaimer: I probably won't have a job next week anyways.

Randell Jesup, OS Group Head, Commodore Engineering

jesup@commodore.com or rjesup@bix.com (more likely to work next week)

(610) 269-7683 (h)

From ata-owner  Mon Apr 25 17:34:53 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA03180; Mon, 25 Apr 94 17:34:53 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA03129; Mon, 25 Apr 94 17:34:39 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA18130; Mon, 25 Apr 1994 17:06:49 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Mon, 25 Apr 1994 16:57:52 EDT

Date:         Mon Apr 25 16:57:51 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2dbc2ed1.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      I think we have it now (?)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

First, you should ignore my posting of SFF-8019 rev 0.5 and my posting of X3T10

document 94-089r0.  I need to re-edit and re-post.  When re-posted I guess it

should be called SFF-8019 rev 1.1 (I'll confirm with Dal first) and 94-089r1.

I will be offsite Tuesday (26 Apr) and the ATA WG meeting is Wednesday (27

Apr), so I'm not sure if I'm going to have time to repost until  later in the

week (could be another long weekend at the keyboard).

Sorry for all the confusion.

Second, now to the technical stuff...

Curtis Stevens of Phoenix sent me the following description of how the cylinder

value in

ID words 1 or 54 can be greater than 16,383.  I will work this into the

SFF-8019 rev 1.1 document.

---- begin Curtis' description ----

Word 1 is limited to 16 bits, it will not exceed the decimal value

65535.  Word 3 is limited to 5 bits and will not exceed the decimal value 16.

The product of (Word 1) and (Word 3 - 1) must be less than or equal to 18

bits.  The number of bits used by word 6 must not exceed 6.

--- end description ---

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\54.MSG

### Org Date:    04-25-94 04:15:01 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     I think we have it now (?)

### Attachments: none

From ata-owner  Tue Apr 26 09:37:10 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA21032; Tue, 26 Apr 94 09:37:10 PDT

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA21025; Tue, 26 Apr 94 09:37:06 PDT

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA27168 for ata@dt.wdc.com; Tue, 26 Apr 94 09:37:02 -0700

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA19078 for ata@dt.wdc.com; Tue, 26 Apr 94 09:37:01 -0700

Received: from bach.nsc.com by berlioz.nsc.com (4.1/SMI-4.1)

id AA27004; Tue, 26 Apr 94 09:36:59 PDT

Date: Tue, 26 Apr 94 09:36:59 PDT

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9404261636.AA27004@berlioz.nsc.com>

To: ata@dt.wdc.com, sff_reflector@sun.com

Subject: ATA Extension SSWG Minutes - 03/94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Doc No: X3T10/94-____r0

Minutes of the ATA-Extensions Special Working Group Meeting

Held March 30, 1994, in Milpitas, CA, at Crown Sterling Suites Hotel.

Chaired by:          Steve Finch, Silicon Systems.

Hosted by:           Richard Kalish, Adaptec.

Minutes recorded by: Robbie Shergill, National Semiconductor.

Following documents were distributed in this meeting:

- 94-045r2: Mode 4  16.6MB/s Timings (T. Hanan, WD)

- 94-___r0: Proposal for Secure Mode (P. McLean, Maxtor)

1. Steve Finch opened the meeting at 9:30AM by thanking the host, stating

the purpose of the meeting and some pertinent ANSI mechanisms. He asked each

attendee to introduce themselves and also sign the attendees' list.

2. Attendance List

NAME                 COMPANY               PHONE

Al Pham              Adaptec              408-945-2560

Richard Kalish       Adaptec              408-957-7169

Larry Lamers         Adaptec              408-945-8600

John Geldman         Cirrus Logic         510-249-4953

Joe Chen             Cirrus Logic         chen@cirrus.com

Steve Anderson       Conner Peripherals   303-682-8320

Jeff Epstein         Future Domain        714-253-0546

Dan Colegrove        IBM                  507-289-0922

Pete McLean          Maxtor               303-678-2149

Ron Roberts          Maxtor               408-432-4322

Tom Newman           Mission Peak Designs 510-790-0230

Robbie Shergill      National Semi.       408-721-7959

Jim McGrath          Quantum              408-894-4504

Farbod Falakfarsa    Quantum              408-894-4066

Jim "Yogi" Schaffner Q-Logic              714-668-5365

Hank Davenport       Samsung              408-434-5427

Hale Landis          Seagate              408-439-2443

John Masiewicz       Seagate              408-439-2152

Steve Finch          Silicon Systems      714-573-6808

Tom Hanan            Western Digital      714-832-7472

Duncan Penman        Zadian               408-293-0800

3. Agenda for this meeting was circulated by Steve Finch. It was modified

and accepted as follows.

1. Opening Remarks

2. Attendance and Membership, Introductions

3. Approval of Agenda

4. Document Distribution

5. Old Business

5.1 16.6 MB/s

5.2 Power Management

5.3 Reset Width

5.4 Master/Slave Handshaking

5.5 Identify Drive for over 528MB

6. New Business

6.1 Secure Mode

7. Status on Remaining Work (ATA-2 Document)

8. Time and place of next meeting

9. Adjournment

5. Old Business

5.1  16.6 MB/s:

The rev. 2 document brought to this meeting still needs to have the cycle

time clarification note added. Steve Finch will correct and provide rev. 3

to Larry Lamers in time for the mailing. There were no other comments.

5.2  Power Management:

Hale said that he is getting no feedback. No one in the meeting had any

technical objections.

5.3 Reset Width:

Richard Kalish suggested the following wording:

..shall not recognize signals shorter than 20nsec...

John Geldman said that Cirrus prefers 5ns. Richard said that their customers

want at least 10ns. Tom Hanan prefers the rejection time to be more than

50ns due to ringing effects.

Steve Anderson expressed concern that with this requirement an ATA-2 drive will

not work well with an ATA-1 drive in the area of master/slave handshaking.

Larry Lamers asserted that this is a device spec, not an interface spec. So far

the ATA specifies what the system should do (25us) but doesn't specify what

the device should do. This proposal tries to remove this problem.

Steve Finch said that this still doesn't solve the existing problem -

because two existing devices will behave differently. John Masiewicz said

that this is not an industry problem, Seagate people haven't seen it in

their labs. Only thing John will agree to is a statement that a device may

respond to a pulse less than 25us. Richard Kalish said that they haven't

seen this as an industry problem either - just trying to fill a hole.

At this point Steve Finch asked for a vote:

Option 1: don't need a noise filter specified at all: 4

Option 2: need it and want it specified it in ATA-2: 5

Option 3: need it and want it specified it in ATA-3: 5

This eliminated first option, voted again:

Option 1: do it in ATA-2: 5

Option 2: do it in ATA-3: 8

It was decided that this issue will be covered under ATA-3.

5.4 Master/Slave Handshaking:

The meaning of "accepting a command" was discussed. Tom Newman said that the

Annex A language assumes that returning an abort is also accepting a

command. Steve Anderson agreed, but Hanan differed. Larry likes Hale's

Method 2 because if your code is on the media you can't handle commands

until drive has spun up anyway. Hanan stated that ATAPI prefers method

1 and it should be pushed in the market because it helps create systems that

are the "sanest".

The group finally agreed that we should make clear that "a command issued

when BUSY is set or DRDY is not set will not be accepted" and that method 1

diagram should be added. Agreed also that Annex B should be retained, same

corrections made and the title changed to say that this annex specifies what

the host should do.

5.5 Identify Drive for over 528MB:

Hale will fix up the document as per changes agreed to in the last SFF

meeting, Hanan will help, and the edited document will be agreed to in the

next meeting.

6. New Business

6.1 Secure Mode

Pete McLean (Maxtor) presented his proposal. Pete said that parts of this

scheme are covered by an approved patent of Maxtor. Also, Maxtor knows that

IBM has filed a patent. To the best of Pete's knowledge, Maxtor has already

written the release letter to ANSI and believes that IBM has also written a

similar letter.

Steve Finch wound up the discussion by commenting that by the next meeting

people will have digested this material and will have useful feedback. So he

suggested to for Pete to come back in a month and also get on the reflector

in the meanwhile.

7. Status on Remaining Work (ATA-2 Document):

The group, in general, felt that the document has taken too long to

stabilize. There are two choices - send out ATA-2 draft asap for review, or

do it correctly and then send it out.

Larry prefers to hurry it up. John Masiewicz agreed - ATA-2 draft is the

best ATA document in existence today. Other agreed with this also.

Tom Hanan wanted a list of items that have changed in the ATA-2 to be added

to the draft.

Robbie Shergill asked if the Seagate (Hale) documents are ready to include

into ATA-2? Steve Finch said that he hasn't included these into the draft

because such determination was never made by this group. Steve has been

proceeding with the intention of presenting ATA-2 to the plenary in May.

Tom Hanan suggested that a list should be made of outstanding items with

ATA-2. The rest of the group agreed and the following list was produced:

List of things that could be included in ata-2:

already in:

- mode 3/mode 1 timings

- seagate comments on draft (electrical specs, etc.)

- byte ordering

done; awaiting review before inclusion:

- 16.6MB/s

in works:

1 power management  (Hale)

2 master/slave handshake  (Hale)

3 ID drive for >528MB/<8GB  (Hale)

4 Format track proposal by Seagate (Hale)

5 Secure Mode (McLean)

6 DRQ Delay   (Hanan)

List of Items items relegated to ATA-3:

- 20MB/s

- ATAPI

- Split-up doc and rewrite

- Data reliability (data CRC, etc.)

- Reset signal filtering

Richard Kalish moved that in-works items 1-3 should be included into ATA-2

and items 4-6 should be moved to ATA-3. Pete McLean said that if this helps

ATA-2 get out faster, then he doesn't mind moving Secure Mode to ATA-3; but

when will ATA-2 go out?

Steve Finch said that he can't make the next mailing, but the document can

definitely be done by the May plenary. John Lohmeyer clarified that this

group doesn't have to vote to forward the document to plenary and ask for a

letter ballot; but it would be good for this group to really agree on it.

Richard changed his motion and reduced it to just including the first three

items and it was agreed to unanimously. Then items 4-6 were debated

individually.

Format track: Tom Hanan stated that currently Format Track doesn't work in

LBA mode. Question is does it need to be fixed because DOS Format command

doesn't use Format Track. Hale Landis said that it is needed for

diagnostics. Dan Colegrove said that you need it for flaw management also.

Duncan and Geldman observed that it's obvious that this item will be

complex. Geldman suggested to add a comment that Format Track doesn't work

in LBA mode - this was agreed to.

Secure mode: McLean asked to move it to ATA-3. Agreed.

DRQ bit: Tom will post the proposal on reflector before the next sswg

meeting and we'll decide next time.

8. Time and Place for the Next Meeting:

Apr 27:  Milpitas, CA; hosted by Cirrus.

May 19:  Harrisburg, PA; part of the X3T10 Plenary Week (Thursday)

June 1:  Milpitas, CA; hosted by Adaptec.

June 29: Irvine, CA; hosted by Western Digital.

Southern California people expressed the desire to hold some meetings in their

locale in future.

9. The meeting was adjourned at 2:50 PM.

From ata-owner  Thu Apr 28 23:30:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06371; Thu, 28 Apr 94 23:30:47 PDT

Received: from gatekeeper.mcimail.com by dt.wdc.com (4.1/SMI-4.1)

id AA06364; Thu, 28 Apr 94 23:30:43 PDT

Received: by gatekeeper.mcimail.com (5.65/fma-120691);

id AA15164; Fri, 29 Apr 94 01:31:30 -0500

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id bt19072;

29 Apr 94 6:22 GMT

Date: Thu, 28 Apr 94 19:46 EST

From: LANE LEE <LANE_LEE+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ata <ata@dt.wdc.com>

To: SFF_reflector <SFF_reflector@sun.com>

Subject: ATAPI/ATA Compatibility

Message-Id: <95940429004659/0003946488NA4EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

There are some issues which require clarification in SFF-8020 pertaining to

attaching an ATAPI device with an existing ATA disk drive on the bus. These

focus on the requirements placed upon the ATAPI device driver, which are

not well defined in the current (rev 1.2) specification. Specifically:

Section 5.14.4 ATAPI aware BIOS and Drive Considerations

This section makes no mention of any requirement for the ATAPI driver to

wait for an attached ATA drive to be in an idle state (i.e. busy and DRQ

cleared) before changing the drive select register and issuing a command to

the ATAPI device. If the ATAPI driver does not wait for this condition, the

action of selecting the ATAPI device and/or issuing an ATAPI command may

cause the ATA drive to abort or corrupt its currently active command. This

line of reasoning mirrors that of having the ATAPI device ignore an ATA

Soft Reset because it might interrupt a currently executing ATAPI command.

This is extremely likely in the case of an immediate ATAPI command which

would allow an ATA driver to issue a command while the ATAPI driver waits

for DSC. When the ATAPI driver regains control, the drive bit will have

changed and the DSC of the ATAPI device is unavailable without changing the

drive select bit. Deselecting the ATA device at this time may cause

unpredictable results; the only safe time to do this drive switch is when

the ATA device has completed the command (Busy and DRQ = zero).

Section 6.2 ATAPI Soft Reset Command and Protocol

Again, this command must not be issued to an ATAPI device unless the

attached ATA drive is in an idle state (Busy and DRQ = 0). Additionally,

why is the ATAPI device asserting the PDIAG* signal if no other devices on

the bus are going to respond to this command?

Of a more editorial nature:

Section 5.6 Overlapped Command Operation

An ATAPI device will abort any ATA command which is interrupting a

currently active command. This differs from both ATA-1 and ATA-2 and should

be stated as such.

Section 1.5 Scope

This document should only prevail with respect to ATAPI devices. It should

NOT prevail with respect to ATA or SCSI-CDROM devices, as is implied in

this section.

Lane Lee

Conner Peripherals

Lane_Lee+aDEVELOPMENT+aCOLORADO%conner@mcimail.com

From ata-owner  Fri Apr 29 07:58:59 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15536; Fri, 29 Apr 94 07:58:59 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA15529; Fri, 29 Apr 94 07:58:50 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id HAA22839; Fri, 29 Apr 1994 07:52:37 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 29 Apr 94 07:40:11 PDT

Date: 29 Apr 94 08:28:17 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: ATA-2 Proposed Draft, Rev 2c Available

To: "scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"71825192404991/262741@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Yesterday, I posted 0948DR2C.DOC to the SCSI BBS.  This document contains

all of the changes agreed to at the April 27, ATA Working Group meeting.

Those interested should download a copy for reveiw.  The May 26 ATA Working

Group meeting, in Harrisburg, will concentrate on reviewing this document.

The goal of the ATA Working Group is to  complete review before the July

meetings so that we can forward the document shortly thereafter.

The document can be found on the SCSI BBS in the ATA file area.  The

document is titled:  0948DR2C.DOC.  This file is a Word for Windows

document created on version 2.0.  The SCSI BBS telephone number is:

719-574-0424.

I will bring hard copies to Harrisburg.

If you find problems or issues with the ATA-2 document, please post those

to the ATA reflector as soon as possible and I strongly suggest that you

bring hardcopy (and overhead view graphs) of those comments with suggested

corrections to the Harrisburg meeting, as we can make progress much more

quickly that way.

===================================================

Steve Finch, Chair ATA Working Group

Silicon Systems, Inc.

14351 Myford Rd.

Tustin, CA 92680-7022

PH:  714-573-6808

FX:  714-573-6916

EM:  steve.finch@tus.ssi1.com

===================================================

From ata-owner  Thu May  5 12:03:11 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28030; Thu, 5 May 94 12:03:11 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA28011; Thu, 5 May 94 12:03:08 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id LAA13295; Thu, 5 May 1994 11:47:43 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 05 May 1994 08:20:39 EDT

Date:         Thu May 05 08:20:38 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2dc8e497.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: Error Detection on ATA

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Without commenting on the merits of the proposal in general, I have a specific

question:

Larry wrote:

>If a parity error is detected on a write the host extends the

>cycle time as needed to setup for retransmitting the last word of

>data.

If parity errors are occuring is it reliable to assume the count is OK? Why the

last word as opposed to the whole sector?

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\82.MSG

### Org Date:    05-05-94 08:03:12 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: Error Detection on ATA

### Attachments: none

From ata-owner  Fri May  6 08:04:43 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22577; Fri, 6 May 94 08:04:43 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA22570; Fri, 6 May 94 08:04:39 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id HAA24435 for <ata@dt.wdc.com@relay>; Fri, 6 May 1994 07:58:19 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 6 May 94 08:01:30 PDT

Date: 6 May 94 08:56:27 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: Problem with Identifying the host

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"72655160504991/265398@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hello, SSA Land!  I haven't seen much on the reflector lately, is everyone

asleep, or what?

Well, let's liven things up a bit with a comment/question.

-------------------------------------------------------

When a device receives a Command message to start a new command, is the

return path information validated?  During the configuration process the

device built an Initiator Table with all known paths to all known

Initiators.  Should the device verify that the return path of the Command

Message is equal to one of the known paths to a known Initiators?  Should

the "should" in the previous sentence be "shall", "recommanded", "not

necessary", or "shouldn't"?

What happens when a Quiesce message is received?  How do we know if

commands associated with the "unknown Initiator" should be aborted?

If the device doesn't check the validity of the return path, are there any

error conditions that might occur that not knowing the exact Initiator

could result in "weird" attempts by the protocol to recover and really ball

things up?

To prevent possible problems, I must assume that checking of the return

path is a SHALL.

But this leads me to another problem:  it takes time to search throught a

list of "at least 64 entries" in the Initator table.  (The "at least" is in

the proposed standard.)  This takes a lot of time and, for best speed

results, requires re-organizing the Initiator Table into interestingly

complex data structures with lots of pointers pointing back and forth.

Making the table in this form takes time, and makes the Quiesce message

processing a minor nightmare.

I think a new approach should be considered.

Why not have the Target device return an Initiator ID in the

Query_Node_Reply message.  Then the Initiator would use this number when

sending a message which requires a return path.  The Target can allocate

these number as it sees fit with the only requirement be that the number is

unique for each Initiator/path combination that it places into the it's

Initiator table.  The device can create numbers which are easy to validate

when a message is received.  Since the return path for each Query_Node

message was put into the Initiator Table, it is easily found when the

Target needs to generate the return path for the physical level routing.

What we have accomplished is saving a significant amount of firmware time

searching tables EACH TIME A COMMAND IS RECEIVED, and possibly at other

times as well.  As currently defined, the Initiator must place the return

path information into the message packet.  It wouldn't take the Initiator

any longer to place the value received in the Query_Node_Reply message into

this field instead of the return path value, so it wouldn't cost anything

in Initator performance.

I would like to see a discussion of this idea on the reflector.  If there

is no objection to the concept, I'll come up with proposed wording changes

to the proposed standards.

Also, I'll take a lack of discussion as implicit support of this idea....

(I hope this statement will cause more responses.)

From ata-owner  Fri May  6 08:15:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22792; Fri, 6 May 94 08:15:16 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA22785; Fri, 6 May 94 08:15:14 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA22336; Fri, 6 May 1994 11:15:07 -0400

From: rkalish@aol.com

X-Mailer: America Online Mailer

Message-Id: <9405061115.tn219997@aol.com>

To: ata@dt.wdc.com

Date: Fri, 06 May 94 11:15:06 EDT

Subject: Re:      Re: Error Detection on ATA

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Gene missed the point of the parity protection as far as I can see when he

asked about the possibility of the whole sector being bad.

Assuming that the parity generation and checking circuitry is functioning

properly ( which might be a big assumption, but we have to start somewhere

and believe something ) the parity error is just caused by noise and

transmission problems in the cable for the word in question.  The point of

the parity is to protect against a transient problem which should be overcome

and not repeat when the word is transfered a second ( maybe a 3rd or 4th

re-try??? ) time.  The fact that there were no parity errors for previous

word transfers should be sufficient proof that the transfers for these words

were successful.

I agree that the parity is not the best protection scheme around, and that if

there are multiple bit errors the parity could not show a bad xfer, bit it is

better than the scheme we currently have.

Rick K.

Adaptec

rkalish@aol.com

From ata-owner  Tue May 10 22:53:01 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02020; Tue, 10 May 94 22:53:01 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA02009; Tue, 10 May 94 22:52:55 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id WAA03904; Tue, 10 May 1994 22:41:44 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Tue, 10 May 1994 22:31:51 EDT

Date:         Tue May 10 22:31:51 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2dd04398.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      One more time with feeling... Please read!

Sender: ata-owner@dt.wdc.com

Precedence: bulk

PLEASE READ THIS!

This is the latest revision of the now famous SFF-8019, aka X3T10

94-089r? document.  While the technical content has not really

changed from the previous revision, much of the text has been

modified and I have not included change bars at the left margin.

I have retained the original SFF-8019 section numbering.

The major change is that I have modified the definition of word 1

such that there should be no misunderstanding about what values

may be placed into the word.

At the 27 April 94 meeting of the ATA Working Group, it was

requested that the SFF Committee "kill" the SFF-8019 project and

convert SFF-8019 to an "informational document".  We can't have

two committees working on the same document!

I will present this at the SFF meeting if requested, but I

request that any discussion of the technical content be conducted

at the ATA Working Group.

Also, the previous versions of this proposal have carried various

titles, all of which are inappropriate.  This proposal applies to

ATA drives of ALL sizes.  Therefore, I now call it "Identify

Drive Data Requirements".

Please, lets get this behind us so that it can be put into the

ATA-2 document!

----------------------------------------------------------------

Identify Drive Data Requirements

SFF-8019 rev 1.1 or X3T10 94-089r1

17 May 94

5. Identify Drive Data Requirements

5.1 Definitions and Background Information

The following abbreviations are used in this proposal:

- 512 MB is used to describe a drive that has 1,032,192 sectors

or 528,482,304 bytes.

- 8 GB is used to describe a drive that has 16,515,072 sectors

or 8,455,716,864 bytes.

The original IBM PC BIOS (Basic Input/Output System) imposed

several restrictions on the support of disk drives, and these

have been incorporated into many higher level software products.

One such restriction limits the capacity of a hard disk drive.

Most BIOS software cannot support a disk drive with more than

1,024 cylinders, 16 heads and 63 sectors per track.  The maximum

addressable capacity of an ATA disk drive under this scheme is

528 MB.

There is growing support of auto-configuration for disk drives on

PC systems.  The auto-configuration capability usually resides in

the BIOS and uses the Identify Drive command data to configure an

ATA disk drive.

This specification defines rules for the Identify Drive data of

all capacity ATA disk drives and allows BIOS support of ATA

drives up to 8 GB using Cylinder/Head/Sector (CHS) addressing.

It is unlikely that BIOS architecture based on CHS addressing can

be extented beyond 8 GB.

This specification defines information that newer BIOSs and

system software can use to determine the true size of a disk

drive and access the full capacity of the drive.

5.2 Cylinder, Head and Sector Addressing

BIOSs and other software that operate an ATA disk drive in CHS

(Cylinder, Head and Sector) addressing mode use Identify Drive

data words 1, 3, 6 and words 53-58 to ascertain the appropriate

translation mode to use and determine the capacity of an ATA disk

drive.

Maximum compatibility is achieved if the following rules are

obeyed.  These rules limit the values placed into words 1, 3, 6,

and 53-58.  The rules specified here for CHS addressing apply to

drives up to 8 GB.

5.2.1 Word 1

For drives less than or equal to 528 MB, Identify Data word 1

(Default Cylinders) shall not specify a value greater than 1,024.

If a drive is greater than 528 MB but less than or equal to 8 GB,

the maximum value that shall be placed into this word is determined

by the value in Word 3 as shown in the following table.

Value in Word 3      Maximum value in Word 1

1  1h               65,535 FFFFh

2  2h               65,535 FFFFh

3  3h               65,535 FFFFh

4  4h               65,535 FFFFh

5  5h               52,428 CCCCh

6  6h               43,690 AAAAh

7  7h               37,449 9249h

8  8h               32,767 7FFFh

9  9h               29,127 71C7h

10  Ah               26,214 6666h

11  Bh               23,831 5D17h

12  Ch               21,845 5555h

13  Dh               20,164 4EC4h

14  Eh               18,724 4924h

15  Fh               17,476 4444h

16 10h               16,383 3FFFh

There is no restriction on the value of Word 1 for drives over 8

GB.

The value in this word shall not change.

5.2.2 Word 3

Identify Data word 3 (Default Heads) shall not specify a value

greater than 16.  If a drive is greater than 528 MB this word

shall contain a value of 16.

The value in this word shall not change.

5.2.3 Word 6

For drives of 8 GB or less, Identify Data word 6 (Default

Sectors) shall not specify a value greater than 63.

For drives over 8 GB, there is no restriction on the value of

this word except that it shall not exceed 255.

The value in this word shall not change.

5.2.4 Use of words 53 through 58

ATA drives that are over 528 MB shall implement words 53-58.

Drives not over 528 MB may also implement these words.  These

words define the addressing for all sectors accessible in CHS

mode.

5.2.5 Word 53

Identify Data word 53 bit 0 shall be set to 1 at all times that

the drive is in a valid translation mode.  Some drives may have

translation modes that cannot be supported.  An attempt to put a

drive into one of these unsupported modes shall cause word 53 bit

0 to be set to 0 with words 54-58 cleared to zero until a valid

translation mode is established.

5.2.6 Word 54

Identify Data word 54 (Current Cylinders) shall specify the

number of full logical cylinders that can be accessed in the

current translation mode.  If an Initialize Drive Parameters

command has not been executed, the contents of this word shall be

the same as word 1. If an Initialize Drive Parameters command has

been executed, this word is the integer result of dividing the

total number of user sectors (this value may be in words 60-61)

by the number of sectors per logical cylinder ( [word55] x

[word56] ), but shall not be a value greater than 65,535.

5.2.7 Word 55

Identify Data word 55 (Current Heads) is the number of heads

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 16.  If an Initialize

Drive Parameters command has not been executed, the contents of

this word shall be the same as word 3.

5.2.8 Word 56

Identify Data word 56 (Current Sectors) is the number of sectors

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 255.  If an Initialize

Drive Parameters command has not been executed, the contents of

this word shall be the same as word 6.

5.2.9 Words 57-58

Identify Data words 57-58 contain a 32-bit value that shall be

equal to [word54] x [word55] x [word56].  If words 60-61, LBA

sectors, are not zero, words 57-58 shall be less than or equal to

the value in words 60-61 at all times.

The 32-bit value is placed into these words as:

- data bits  7- 0 in word 57 bits 15-8

- data bits 15- 8 in word 57 bits  7-0

- data bits 23-16 in word 58 bits 15-8

- data bits 31-24 in word 58 bits  7-0

5.3 Logical Block Addressing

It is recommended that ATA drives over 528 MB support Logical

Block Addressing (LBA).

5.3.1 Words 60-61

Identify Data words 60-61 shall specify the total number of user

sectors available in LBA mode at all times.  This value shall be

equal to or greater than the value in words 57-58 at all times.

The contents of these words shall not change.

The 32-bit value is placed into these words as:

- data bits  7- 0 in word 60 bits 15-8

- data bits 15- 8 in word 60 bits  7-0

- data bits 23-16 in word 61 bits 15-8

- data bits 31-24 in word 61 bits  7-0

5.3.2 Orphan Sectors

The sectors, if any, between the last sector addressable in CHS

mode and the last sector addressable in LBA mode are known as

"orphan" sectors.  A drive may or may not allow access to these

sectors in CHS addressing mode.

The values in words 1, 3 and 6 should be selected such that the

number of orphan sectors is minimized.  Normally, the number of

orphan sectors should not exceed ( [word55] x [word56] - 1 ).

However, the host system can create conditions where there are a

larger number of orphans sectors by issuing the Initialize Drive

Parameters command with values other than the values in words 3

and 6.

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\825.MSG

### Org Date:    05-10-94 10:06:00 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     One more time with feeling... Please read!

### Attachments: none

From ata-owner  Wed May 11 08:37:41 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA12628; Wed, 11 May 94 08:37:41 PDT

Received: from gatekeeper.mcimail.com by dt.wdc.com (4.1/SMI-4.1)

id AA12621; Wed, 11 May 94 08:37:38 PDT

Received: by gatekeeper.mcimail.com (5.65/fma-120691);

id AA19288; Wed, 11 May 94 10:38:36 -0500

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id aa23047;

11 May 94 15:28 GMT

Date: Wed, 11 May 94 10:35 EST

From: LANE LEE <LANE_LEE+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ata <ata@dt.wdc.com>

To: SFF_reflector <SFF_reflector@sun.com>

Cc: "Mark Gurkowski@DEVELOPMENT@COLORADO" <Mark_Gurkowski+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

Cc: "Don Clay@DEVELOPMENT@COLORADO" <Don_Clay+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

Cc: "Steve Anderson@DEVELOPMENT@COLORADO" <Steve_Anderson+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

Subject: re: SFF8019 rev1.1

Message-Id: <50940511153505/0003946488NA1EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Regarding SFF-8019 rev 1.1 (X3T10 94-089r1):

The description for Words 1 and 3 continue to cause problems. The table shown

for word 1 has no meaning because Word 3 specifies that the number of heads

must be 16 for drives over 528MB. Additionally, it does not address the issue

which lead to the adoption of this heads limitation in the first place.

Here is my attempt to make sense of the situation. In the March 14 SFF

meeting, it was brought to our attention by Curtis Stevens that the

limitation in BIOS has two parts:

1) The product of the number of heads and the number of cylinders must fit

within 18 bits.

2) The number of heads must be a power of two, so that BIOSs may use shifts

and rotates to perform their multiplication.

Given these two restrictions, word 1 and word 3 can be described as follows:

5.2.1 Word 1

For drives less than or equal to 528 MB, Identify Data word 1

(Default Cylinders) shall not specify a value greater than 1,024.

If a drive is greater than 528 MB but less than or equal to 8 GB,

the maximum value that shall be placed into this word is determined

by the value in Word 3 as shown in the following table.

Value in Word 3      Maximum value in Word 1

1  1h               65,535 FFFFh

2  2h               65,535 FFFFh

4  4h               65,535 FFFFh

8  8h               32,767 7FFFh

16 10h               16,383 3FFFh

There is no restriction on the value of Word 1 for drives over 8

GB.

The value in this word shall not change.

5.2.2 Word 3

Identify Data word 3 (Default Heads) shall not specify a value

greater than 16.  If a drive is greater than 528 MB this word

shall contain a value of 1, 2, 4, 8, or 16.

The value in this word shall not change.

Lane Lee

Conner Peripherals

From ata-owner  Wed May 11 14:58:38 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA20801; Wed, 11 May 94 14:58:38 PDT

Received: from inet-gw-2.pa.dec.com by dt.wdc.com (4.1/SMI-4.1)

id AA20794; Wed, 11 May 94 14:58:31 PDT

Received: from enet-gw.pa.dec.com by inet-gw-2.pa.dec.com (5.65/21Mar94)

id AA24746; Wed, 11 May 94 14:53:20 -0700

Received: from advlsi.enet by enet-gw.pa.dec.com (5.65/rmc-22feb94)

id AA11894; Wed, 11 May 94 14:23:16 -0700

From: rinne@advlsi.enet.dec.com

Message-Id: <9405112123.AA11894@enet-gw.pa.dec.com>

Received: from advlsi.enet; by decwrl.enet; Wed, 11 May 94 14:52:06 PDT

Date: Wed, 11 May 94 14:52:06 PDT

To: ata@advlsi.enet.dec.com

Cc: me@advlsi.enet.dec.com

Apparently-To: ata@dt.wdc.com

Subject: Question regarding ATA-2 X3T9.2 948D 1/1/94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hello.

I'm Floyd Rinne, working at Digital in Shrewsbury, MA, and I've got

a question about the ATA-2 X3T9.2 948D Revision 2 dated 1/1/94.

In the document, several facts are given:

1. (section 10.4) "t7" Addr valid to IOCS16- assertion (mode 3) 30nS

2. (section 10.4) "t8" Addr valid to IOCS16- negation (mode 3) 30nS

3. (section 4.6) IOCS16- is an OC (open-collector) driver with a

1K (min) pullup at the host.

Assuming two devices, plus host plus cable, typical load capacitance

on a line will be ~100pF. Device cap is given (section 4.6) as 25pF,

cable cap (section 4.5) is 35pF. Host cap unspecified, but I feel

that 100 pF is reasonable for the system.

The total turn-around time from address valid on the bus, through

receivers, decoder and bus driver must be 30 nS in mode 3 (t7 or t8).

How is this possible in the case of t8, where only a 1K resistor is

pulling the load cap high ? Logic delays aside, the rise time alone

will exceed 30nS.

Or is there an assumption that the delay times given in section 10.4

are to be measured without the cable, host or second drive present.

Thanks in advance for any assistance you can give me; I've already sent

a subscription request to the reflector.

Email will reach me at: rinne@growl.adv.shr.dec.com

floydrinne@aol.com

Telephone at (508) 841-2576

Fax at (508) 841-2604

From ata-owner  Wed May 11 16:06:15 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22450; Wed, 11 May 94 16:06:15 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA22443; Wed, 11 May 94 16:06:08 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id PAA11868; Wed, 11 May 1994 15:53:58 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Wed, 11 May 1994 15:43:25 EDT

Date:         Wed May 11 15:43:25 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2dd1355e.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      ID Data -- Wrong Again!

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I'm sure this is the last time you will want me to edit an SFF

or ATA document -- I continue to get things wrong (well,... I

get some things wrong)!

I just talked to Curtis Stevens of Phoenix Technologies at

length about the document and here my current understanding...

1) The table I show in section 5.2.1, the description of Word 1,

is not correct.  The following table should be correct:

Value in Word 3      Maximum value in Word 1

1  1h                 65,535 FFFFh

2  2h                 65,535 FFFFh

3  3h                 65,535 FFFFh

4  4h                 65,535 FFFFh

5  5h                 32,767 7FFFh

6  6h                 32,767 7FFFh

7  7h                 32,767 7FFFh

8  8h                 32,767 7FFFh

9  9h                 16,383 3FFFh

10  Ah                 16,383 3FFFh

11  Bh                 16,383 3FFFh

12  Ch                 16,383 3FFFh

13  Dh                 16,383 3FFFh

14  Eh                 16,383 3FFFh

15  Fh                 16,383 3FFFh

16 10h                 16,383 3FFFh

2) There is NO requirement that the number of heads in Word 3 be

a power of 2.  Hopefully, by including the table in the

description of Word 1, this is made clear.

3) The following sentence in section 5.2.2, the description of

Word 3, should be deleted:

"If a drive is greater than 528 MB this word shall contain a

value of 16".

4) Please note that over time this proposal has aquired wording

such that it applies to all ATA drives of ALL capacities.  Curtis

Stevens believes that this proposal should be limited to drives

less than or equal to 8GB.  Curtis believes a new proposal with

different Identify Data requirements needs to be prepared for

drives over 8GB.  Neither Curtis or I want to further delay this

proposal by trying to include a new proposal at this time.  Also,

it is unlikely that ATA drives over 8GB will be a problem this

year.  That should give Curtis a few months to put together his

over 8GB Identify Data proposal.  Please speak up if you have a

problem or question about this.

5) I will wait another 24 hours for more comments and then I will

repost SFF-8019 rev 1.1A and X3T10 94-089r1A with the corrections

listed here plus any other relevant comments.  I will mark those

sentences/paragraphs having to do with drives over 8GB so that

if we decided to, we can quickly remove them from this proposal.

And, yes, I will date the revised document "18 May 94".

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\1013.MSG

### Org Date:    05-11-94 03:37:20 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     ID Data -- Wrong Again!

### Attachments: none

From ata-owner  Thu May 12 08:19:43 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA11155; Thu, 12 May 94 08:19:43 PDT

Received: from gatekeeper.mcimail.com by dt.wdc.com (4.1/SMI-4.1)

id AA11148; Thu, 12 May 94 08:19:40 PDT

Received: by gatekeeper.mcimail.com (5.65/fma-120691);

id AA06436; Thu, 12 May 94 10:20:36 -0500

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id ay00788;

12 May 94 15:10 GMT

Date: Thu, 12 May 94 10:14 EST

From: MARK GURKOWSKI <MARK_GURKOWSKI+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ata <ata@dt.wdc.com>

Cc: "Steve Anderson@DEVELOPMENT@COLORADO" <Steve_Anderson+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

Cc: "Don Clay@DEVELOPMENT@COLORADO" <Don_Clay+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

Cc: "Lane Lee@DEVELOPMENT@COLORADO" <Lane_Lee+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

Subject:

Message-Id: <91940512151419/0003946488NA4EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:      ATA Working Group

From:    Mark Gurkowski, Conner Peripherals

Date:    May 12, 1994

Subject: Proposal For ID Command Support of Extended Power Command Time-outs

I would like to propose adding a bit in the Identify Drive Information that

allows hosts to determine if the drive supports the new Extended Power

Command Time-out Periods.  I would like to define Word 49 Bit 13 for this

purpose.

The following is the proposed change to the Identify Drive Information:

Word

49     Capabilities

|                  15-14  0=Reserved

|                  13     1=Extended Power Command Time-outs supported

|                         0=Extended Power Command Time-outs may be supported

12     1=Reserved (for advanced PIO mode support)

This will allow new host systems to determine if the drive supports the new

time-outs, while not excluding drives who supported the new time-outs before

the addition of this proposal.

Mark

From ata-owner  Thu May 12 16:51:51 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22454; Thu, 12 May 94 16:51:51 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA22443; Thu, 12 May 94 16:50:28 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id QAA26774; Thu, 12 May 1994 16:36:06 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 12 May 1994 16:05:48 EDT

Date:         Thu May 12 16:05:47 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2dd28c1c.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: Question regarding ATA-2 X3T9.2 948D 1/1/94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

For the capacity spec the host is a device.

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\102.MSG

### Org Date:    05-12-94 03:53:34 PM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: Question regarding ATA-2 X3T9.2 948D 1/1/94

### Attachments: none

From ata-owner  Fri May 13 14:09:33 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17169; Fri, 13 May 94 14:09:33 PDT

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA17162; Fri, 13 May 94 14:09:17 PDT

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA15705 for ata@dt.wdc.com; Fri, 13 May 94 14:09:13 -0700

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA02901 for ata@dt.wdc.com; Fri, 13 May 94 14:09:11 -0700

Received: from bach.nsc.com by berlioz.nsc.com (4.1/SMI-4.1)

id AA01329; Fri, 13 May 94 14:09:10 PDT

Date: Fri, 13 May 94 14:09:10 PDT

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9405132109.AA01329@berlioz.nsc.com>

To: ata@dt.wdc.com, sff_reflector@sun.com

Subject: ATA Ad-hoc Minutes - April 94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Doc No: X3T10/94-102r0

Minutes of the ATA-Extensions Ad-Hoc Meeting

Held April 27, 1994, in Milpitas, CA, at Crown Sterling Suites Hotel.

Chaired by:          Steve Finch, Silicon Systems.

Hosted by:           Joe Chen, Cirrus Logic.

Minutes recorded by: Robbie Shergill, National Semiconductor.

1. Steve Finch opened the meeting at 9:30AM by thanking the host, stating

the purpose of the meeting and some pertinent ANSI mechanisms. He asked each

attendee to introduce themselves and also sign the attendance list.

2. Attendance List

NAME                 COMPANY               PHONE

Richard Kalish       Adaptec              408-957-7169

Larry Lamers         Adaptec              408-945-8600

Nicos Syrimis        Cirrus Logic         510-226-2153

Joseph Chen          Cirrus Logic         510-226-2101

Steve Anderson       Conner Peripherals   303-682-8320

Robert Liu           Fujitsu              408-894-3790

Dan Colegrove        IBM                  507-289-0922

Ron Roberts          Maxtor               408-432-4322

Ray Heineman         Maxtor               303-678-2591

Robbie Shergill      National Semi.       408-721-7959

Charles Yang         Panasonic            408-945-5641

Jim McGrath          Quantum              408-894-4504

Farbod Falakfarsa    Quantum              408-894-4066

Hank Davenport       Samsung              408-434-5427

Hale Landis          Seagate              408-439-2443

Steve Finch          Silicon Systems      714-573-6808

Tom Hanan            Western Digital      714-832-7472

3. Agenda for this meeting was circulated by Steve Finch. It was modified

and accepted as follows.

1. Opening Remarks

2. Attendance and membership, introductions

3. Approval of agenda

4. Document distribution

5. Meeting schedule

6. Old business

6.1  16.6 MB/s

6.2  Master/Slave handshaking

6.3  Identify drive over 528MB

7. New business

7.1  Parity Proposal

7.2  DRQ Bit

7.3  Vendor Unique Set Feature Subcommands

7.4  Format Sector/Track

7.5  Drive Address register

7.6  Interrupting commands

7.7  Identify Drive cleanup

8. Adjournment

4. Following documents were distributed in this meeting:

94-      Error detection for ATA-3 (Lamers, Adaptec)

94-      Drive Address register (Landis, Seagate)

94-      Identify Drive cleanup (Landis, Seagate)

94-      Vendor unique Set Feature subcommands (Landis, Seagate)

94-      Interrupting commands (Landis, Seagate)

94-      Format Track (Landis, Seagate)

94-065r2 Master/slave handshaking (Landis, Seagate)

948D     Proposed working draft for ATA-2 (Finch, SSI)

94-101r0 Minutes of the 3/30/94 Meeting (Shergill, National)

5. Meeting Schedule:

Upcoming, confirmed meetings are as follows:

5/19/94:   Thursday morning in the plenary week at Harrisburg, PA.

6/1/94:    Milpitas, hosted by Adaptec

6/29/94:   Irvine, hosted by Western Digital

Steve Finch stated that he expects to have the X3T10 plenary approve ATA-2

draft in the July meeting. Question is when should the ATA-3 work begin?

Hale Landis proposed and Steve Finch seconded that there should be no ad-hoc

meetings in July and December. There were no objections to this. Larry

Lamers suggested that October meeting should also be moved to an earlier

time because of conflict with PCMCIA meetings in the last week of that

month.

At this point, the meeting schedule for the rest of the year looks like the

following:

July   - no meeting

Aug 31 -  ad-hoc meeting

Sep 15 -   plenary in Houston

Sep 28 -  ad-hoc meeting

Oct 19 -  ad-hoc meeting

Nov 10 -   plenary in Palm Springs

Nov 30 -  ad-hoc meeting

Dec    - no meeting

Jan 95 -   plenary in Lake Tahoe

Tom Hanan announced that the ATA reflector is getting too many bad

addresses. It is getting too much to handle and he will start sending

official notices to offending addresses. Clean up your address within 60

days or you will be purged.

Larry Lamers asked if Steve Finch will indeed chair each of the ATA-3 sub-

groups as well as being the overall editor. Steve is open either way -

doesn't mind chairing each of the meetings because he expects them to be

held in succession on the same day. Landis said that we can deal with this

when we start ATA-3 work - Hanan and Steve agreed.

Joe Chen commented that four ATA-3 projects are causing confusion already -

each document should be kept focused. Finch and Lamers asked him to cover

this by commenting on the project proposals.

6. OLD BUSINESS:

6.1 16.6mb/s:

Steve Finch said that this appears to be a done deal. He will roll this spec

into the ATA-2 draft. No one had any comments to the contrary.

6.2 Master/slave handshaking:

Hale Landis went over his proposal - he has added the timing diagarms. He

knows that there are a few typo errors in there, but he has received no

feedback.

Steve Anderson commented that there should be something specified for max

time for DRDY to be asserted - after which the drive should be considered

dead. Tom Hanan said that 30 sec. time should be applied here - that was the

intention with ATA-1. Anderson said that it should be more like 2 minutes.

Landis will have to check, but felt it has to be more than 30 sec. Hanan,

Chen and Lamers felt that the user will not wait for 2 minutes. Hanan said

that we should enquire of the host people as to what they can do. Hale will

put out the question on the reflector.

Robbie Shergill asked to change the titles of the two annexes so they can be

differentiated. This was agreed to. Tom Hanan and Steve Finch asked if we

should include this in the ATA-2? Went around the room - everyone wanted to

include it.

6.3 Identify drive for 528MB:

Because of the confusion that existed over the rev numbers, Hale had

promised Dal Allen that this issue will not be discussed today. At this

point, Hale's document only has a problem with the description of word 1.

Hale suggested that this document should be titled, "ID data for capacity

determination". Everyone liked this name, but Jim McGrath felt that this

removes the clarity of the fact that this document allows drives larger than

528MB.

Hale's intention is to complete this document under ATA and Steve intends to

include it in ATA-2 document. Larry said that the SFF-8019 should be killed

in that case - everyone agreed with this. Hale will call Dal and ask him to

have the 8019 reference the appropriate ATA document to remove confusion.

Joe Chen asked for clarification on the relationship between ATA-2 and

Enhanced IDE. Steve said that Enhanced IDE is one vendor's spec - that

vendor has to comment on its relationship to ATA-2. Tom Hanan commented that

Western Digital has no intention of treating Enhanced IDE as an exclusive

document.

7. NEW BUSINESS:

7.1  Parity Proposal

Question was raised whether this should even be considered for ATA-2. There

was consensus that this issue belongs with the data integrity topic that has

already been deferred to ATA-3.

7.2 DRQ Bit

The group was of the opinion that there is no need for specific handling of

this issue.

7.3 Vendor Unique Set Features:

Tom Hanan didn't like the proposal on the basis that Vendor Unique commands

should be used to do vendor unique functionality. Jim also agreed - Quantum

does this type of things with a vendor unique command. Steve asked why Hale

has to use Set Features instead of a vendor unique command. Hale responded

that the real problem is that currently the Set Features command description

doesn't say that the unspecified codes are reserved. All agreed to just do

this much.

7.4 Format track/sector:

Steve Finch has alrady edited the ATA-2 document to say that Format Track

works only in the CHS mode. But he said that the response to Format Track

command in LBA mode is vendor unique. The group wanted it to be illegal -

Steve will fix the document.

Tom Hanan was of the opinion that we shouldn't cover this - we had already

decided that we don't want to work on this. Jim McGrath said that he doesn't

like giving the host the abillity to do flaw management. But Dan Colegrove

said that the IBM-Boca people want to use ft to do flaw management.

Hale said that he wants the command to be made optional so he doesn't have

to implement it. Except for some very old software, no one uses this

command. But, Tom Hanan pointed out that we had made a statement that ATA-2

will not eliminate any existing ATA commands - there is some software out

there that will break if the Format Track command aborts.

After lunch, the group decided to remove most of the body of this command

from the document and just leave the first sentence with a second sentence

to the effect: "host systems are recommended not to utilize this command".

Dan Colegrove expressed concern about this change being too close to the

time when we try to have it approved. But once it was clarified that the

vote is four months away, he agreed to go along.

7.5 Drive Address register:

Hale asked if anyone knows of any useful purpose for this register. No one

offered a purpose, but Robbie Shergill and Joe Chen said that a change here

should not obselete any existing silicon. Shergill said that if the silicon

is required by this change to not drive the bus then the existing silicon

will have to change.

The group decided to not change the table with "high imped" for the Drive

Address register, but to cover it in the note only with the recommendation

that Hale had put together; with additional text that says that if it is

implemented then it should be imlemented as ... (ATA-1 description).

7.6 Interrupting commands:

Hale Landis presented his proposal. Larry Lamers pointed out that ATAPI

allows the current command to be interrupted with a new command even while

BSY bit is set. This caused a long discussion. (Tom Hanan was not present

during this discussion).

In the end, the group accepted Hale's recommendation with the first sentence

of his last paragraph changed to "a command shall only be interrupted with a

software or hardware reset". Second sentence also was changed to " the

result of writing the command register while BSY=1 or DRQ=1 is unpredictable

..."

7.7 Identify Drive cleanup:

The group had no objection to making this command mandatory or on making

several of the words vendor unique. Hale is going to rewrite this section

for ATA-2. He promised to have this done before next plenary.

8. Meeting was adjourned at 3:00 PM

From ata-owner  Tue May 17 09:27:56 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29943; Tue, 17 May 94 09:27:56 PDT

Received: from cirrus.com by dt.wdc.com (4.1/SMI-4.1)

id AA29933; Tue, 17 May 94 09:27:40 PDT

Received: from sunstorm.corp.cirrus.com (sunstorm1) by cirrus.com with SMTP id AA05599

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Tue, 17 May 1994 09:27:34 -0700

Received: from ss306.corp.cirrus.com by sunstorm.corp.cirrus.com with SMTP id AA27893

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Tue, 17 May 1994 09:27:33 -0700

Received: by ss306.corp.cirrus.com (4.1-Corp/1.00)

id AA08675; Tue, 17 May 94 09:27:32 PDT

Date: Tue, 17 May 94 09:27:32 PDT

From: johng@corp.cirrus.com (John Geldman)

Message-Id: <9405171627.AA08675@ss306.corp.cirrus.com>

To: ata@dt.wdc.com

Subject: comments on 0948dr1.doc - uuencoded WFW

Sender: ata-owner@dt.wdc.com

Precedence: bulk

begin 644 comments.doc

MT,\1X*&Q&N$                     .P # /[_"0 &               !

M     0          $    @    $   #^____          #_____________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M_______________________]____'@   /[___\?    !0    8    '

M"     D    *    "P    P    -    #@    \    0    $0   !(    3

M    %    !4    6    %P   !@    9    &@   !L    <    '0   "

M  #^_____O___R$    B    (P   "0    E    )@   "<    H    _O__

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M_____________________________________________U( ;P!O '0 ( !%

M &X = !R 'D

M           6  4 __________\#      D"      #         1@

M         (8-N+@Q!;D! P   $ #         0!# &\ ;0!P $\ 8@!J

M

M !(  @'_______________\

M                8@        !7 &\ <@!D $0 ;P!C '4 ;0!E &X =

M                                                    &@ " ?__

M__\$    _____P

M  0   !V1        $\ 8@!J &4 8P!T %  ;P!O &P

M                                           6  $! 0    (   #_

M____                          "&#;BX,06Y 88-N+@Q!;D!

M         0   /[___\#    !     4    &    !P    @    )    "@

M  L    ,    _O______________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M______________________________\! /[_ PH  /____\ "0(      ,

M      !&'    $UI8W)O<V]F="!7;W)D(#8N,"!$;V-U;65N=  *    35-7

M;W)D1&]C !    !7;W)D+D1O8W5M96YT+C8      #L  P#^_PD !@

M         0    $      /[_   #"@                        $   #@

MA9_R^4]H$*N1"  K)[/9,    (P"   -    !P   )@    $    W     @

M     0  #    "0!   +    2 $   T   !L 0  #P   ) !   0    M $

M  H   #8 0  $@   /P!   .    ( (   D   !$ @  $P   &@"  #_____

M________________________________________________'@   "8   !#

M.EQ54T]&5%Q724Y73U)$-EQ414U03$%415Q.3U)-04PN1$]4

M                   >    #0   $IO:&X@1V5L9&UA;@

M       >    #0   $IO:&X@1V5L9&UA;@                    !

MADF7,2<#N0$                               !          -RE90 S

MP D$   ( &4                #  #9*@  =D0

M  #9)P                                              0   U@

M  !   #6    UD        #60        -9         UD        #60

M%    &!!        8$$       !@00       &!!        8$$       !@

M00  "@   &I!   B    8$$       "L0P  2@   (Q!   B    KD$

M  "N00       *Y!        KD$       "N00       *Y!        KD$

M      "E0@   @   *="        IT(       "G0@  )P   ,Y"  !@

M+D,  &    ".0P  '@   /9#  !4    2D0  "P   "L0P

M              #60        *Y!           : !L  0 % *Y!

MKD$                             KD$       "N00       *Q#

M    KD$       #60        -9         KD$

M        C$$       "N00       *Y!        KD$       "N00

M -9         KD$       #60        *Y!        I4(

M                ZD   "P    600  2@   -9         UD        #6

M0        -9         KD$       "E0@       *Y!  #W    KD$

M

M                                                          T3

M14U"140@4$)R=7-H(%QS("!<*B!M97)G969O<FUA=!0!%4-I<G)U<R!,;V=I

M8R -,S$P,"!797-T(%=A<G)E;B!!=F5N=64L($9R96UO;G0L($-!(" Y-#4S

M."P@5&5L+B H-3$P*2 R-#DM-#DU,RP@1F%X("@U,3 I(#(T.2TT.30P#0U$

M051%.@DU+3$W+3DT#0U43SH)"5@S5#$P($%402!W;W)K:6YG(&=R;W5P#0U&

M4D]-.@E*;VAN($=E;&1M86X-#5-50DI%0U0Z"3 Y-#AD<C(N9&]C(&-O;6UE

M;G1S#0D)"0D)"0D)"0D)"0T-5&AE<V4@87)E('1H92!C;VUM96YT<R!)(&AA

M=F4@<V\@9F%R(&9R;VT@;7D@<F5V:65W(&]F('1H:7,@9&]C=6UE;G0N( U3

M964@>6]U(&EN($AA<G)I<V)U<F<N#0T-*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*@T-4V5C=&EO;B S+C$N,2P@

M<&%G92 Q,PT)5VET:"!T=V\@861D:71I;VYA;"!C;VYN96-T;W)S(&EN('1H

M92!A;FYE>"P@=&AE('1Y<&4@;V8@8V]N;F5C=&]R('-H;W5L9"!N;W0@8F4@

M96UP:&%S:7IE9"X-#5=A<SH-051!(&1E9FEN97,@82!R96=I<W1E<B!S970L

M(&$@-# M<&EN(&-O;FYE8W1O<B!A;F0@=&AE(&%S<V]C:6%T960@<VEG;F%L

M<R!O;B!T:&4@8V]N;F5C=&]R+@T-4W5G9V5S=&5D.@U!5$$@9&5F:6YE<R!A

M(')E9VES=&5R+"!C;VUM86YD+"!A;F0@<')O=&]C;VP@<V5T+"!A;&]N9R!W

M:71H(&-O;FYE8W1O<B!A;F0@<VEG;F%L(&1E9FEN:71I;VYS+@T-*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*@T-

M4V5C=&EO;B S+C(N-2P@<&%G92 Q-@T)5&AE(&)Y=&4@;W)D97(@9&5S8W)I

M8F5D(&AE<F4@:7,@;F]T(&5S<V5N=&EA;"P@87,@86QL(&1R:79E<R!T96YD

M('1O(&1E;&EV97(@9&%T82!B86-K(&EN('1H92!O<F1E<B!I="!W87,@<F5C

M96EV960N($AO=V5V97(L('1H92!T>7!I8V%L($EN=&5L(&]R:65N=&%T:6]N

M('1H870@051!('-T87)T960@=VET:"!W;W5L9"!H879E(&AA9"!T:&4@:&EG

M:"!B>71E(&]F('=O<F0@*'@I+@T-#%=A<SH-1$0@1$0@1$0@1$0@1$0@1$0@

M1$0@1$0@("!$1"!$1"!$1"!$1"!$1"!$1"!$1"!$1 T@(" @(" @(" @(" Q

M-2 Q-" Q,R Q,B Q,2 Q," P.2 P." @(# W(# V(# U(# T(# S(# R(# Q

M(# P#5=O<F0H>"D@(" @("TM+2TM+2TM0GET92@P*2TM+2TM+2TM(" @+2TM

M+2TM+2U">71E*#$I+2TM+2TM+2T-5V]R9"AX*S$I(" @+2TM+2TM+2U">71E

M*#(I+2TM+2TM+2T@(" M+2TM+2TM+4)Y=&4H,RDM+2TM+2TM+0T-4W5G9V5S

M=&5D.@U$1"!$1"!$1"!$1"!$1"!$1"!$1"!$1" @($1$($1$($1$($1$($1$

M($1$($1$($1$#2 @(" @(" @(" @(#$U(#$T(#$S(#$R(#$Q(#$P(# Y(# X

M(" @,#<@,#8@,#4@,#0@,#,@,#(@,#$@,# -5V]R9"AX*2 @(" @+2TM+2TM

M+2U">71E*#$I+2TM+2TM+2T@(" M+2TM+2TM+4)Y=&4H,"DM+2TM+2TM+0U7

M;W)D*'@K,2D@(" M+2TM+2TM+4)Y=&4H,RDM+2TM+2TM+2 @("TM+2TM+2TM

M0GET92@R*2TM+2TM+2TM#0TJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ#0U396-T:6]N(#0L('!A9V4@,3<Z#0E4

M:&4@961I=&]R(&%S:V5D(&9O<B!S=6=G97-T:6]N<R!O;B!H;W<@=&\@9&5F

M:6YE(&QE9V%L(&-O;FYE8W1I;VYS(&%N9" -8V%B;&EN9R!C;VYF:6=U<F%T

M:6]N<RX@3VYE('-U9V=E<W1I;VX@:7,@=&\@86QL;W<@;VYL>2!O;F4@<&EE

M8V4@86YD('1Y<&4@;V8@8V%B;&4@*'1O(&%V;VED(%-#4TDM='EP92!I;7!E

M9&%N8V4@;6ES+6UA=&-H(')E9FQE8W1I;VYS*2P@:&]W979E<B!A;GD@8V]N

M;F5C=&]R+6)A<V5D(&-O=7!L97(@*#8X('!I;B!D<FEV92!T;R T-"!P:6X@

M8V%B;&4I('=O=6QD(&)E(&QE9V%L+@T-*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*@T-4V5C=&EO;B T+C(N,BP@

M<&%G92 Q.2!396-T:6]N(#0N-BP@<&%G92 R,0T)37D@;F]T97,@<VAO=R!T

M:&%T(&]N($9E8G)U87)Y(#(S<F0L('1H92!7;W)K:6YG($=R;W5P(&%G<F5E

M9"!T;R!R96UO=F4@86QL(')E9F5R96YC97,@=&\@,RXS5B!S>7-T96US(&%S

M('=E(&AA9"!N;W0@<')O=FED960@82!C;VUP;&5T92!S<&5C:69I8V%T:6]N

M(&9O<B!O<&5R871I;F<@870@=&AI<R!P;W=E<B!L979E;"X-5V%S.@T@(" @

M(" @(" @(" @(" @(" @(" @*ST]/3T]/3T]/3T]/3T]/3T]/3T]/3T]/2L]

M/3T]/3T]/3T]/3TK#2 @(" @(" @(" @(" @(" @(" @("!\(%!O=V5R($QI

M;F4@1&5S:6=N871I;VX@?"!0:6X@3G5M8F5R('P-(" @(" @(" @(" @(" @

M(" @(" @("L]/3T]/3T]/3T]/2L]/3T]/3T]/3T]/3TK/3T]/3T]/3T]/3T]

M*PT@(" @(" @(" @(" @(" @(" @(" @?" @(" @(" @*S4@5F]L=',@?" K

M,RXS(%9O;'1S('P@(" @(#$@(" @("!\#2 @(" @(" @(" @(" @(" @(" @

M(" K+2TM+2TM+2TM+2TK+2TM+2TM+2TM+2TM*RTM+2TM+2TM+2TM+2L-(" @

M(" @(" @(" @(" @(" @(" @('P@*S$R(%9O;'1S('P@("LU(%9O;'1S("!\

M(" @(" R(" @(" @? T@(" @(" @(" @(" @(" @(" @(" @*RTM+2TM+2TM

M+2TM*RTM+2TM+2TM+2TM+2LM+2TM+2TM+2TM+2TK#2 @(" @(" @(" @(" @

M(" @(" @("!\(" @1W)O=6YD("!\(" @1W)O=6YD(" @?" @(" @,R @(" @

M('P-(" @(" @(" @(" @(" @(" @(" @("LM+2TM+2TM+2TM+2LM+2TM+2TM

M+2TM+2TK+2TM+2TM+2TM+2TM*PT-5&%B;&4@,B M($1#($EN=&5R9F%C92!5

M<VEN9R S(%!I;B!0;W=E<B!#;VYN96-T;W(-$U1A8FQE(#(@+2!$0R!);G1E

M<F9A8V4@57-I;F<@,R!0:6X@4&]W97(@0V]N;F5C=&]R%0T-6T5D:71O<B=S

M($YO=&4Z("!(;W<@9&\@=V4@<W!E8VEF>2!T:&4@<&EN(&]N92!O<FEE;G1A

M=&EO;C]=#0U!(&1R:79E(&1E<VEG;F5D(&9O<B S+#-6(&%P<&QI8V%T:6]N

M<R!M87D@8F4@<&QU9V=E9"!I;G1O(&$@<F5C97!T86-L92!D97-I9VYE9"!T

M;R!A8V-E<'0@82!D<FEV92!D97-I9VYE9"!F;W(@-58@87!P;&EC871I;VYS

M+"!W:71H(#$R5B!L:6YE<R!F;W(@861D:71I;VYA;"!P;W=E<BX@($ET(&ES

M(&YO="!R97%U:7)E9"!T:&%T('1H92!D<FEV92!O<&5R871E+"!B=70@:70@

M:7,@<F5C;VUM96YD960@=&AA="!P<F5C875T:6]N<R!B92!T86ME;B!T;R!P

M<F5V96YT(&1A;6%G92!T;R!T:&4@9')I=F4N#0U!(&1R:79E(&1E<VEG;F5D

M(&9O<B U5B!A<'!L:6-A=&EO;G,@;6%Y(&)E('!L=6=G960@:6YT;R!A(')E

M8V5P=&%C;&4@9&5S:6=N960@=&\@86-C97!T(&$@9')I=F4@9&5S:6=N960@

M9F]R(#,L,U8@87!P;&EC871I;VYS+"!W:71H(#56(&QI;F5S(&9O<B!A9&1I

M=&EO;F%L('!O=V5R+B @270@:7,@;F]T(&%N=&EC:7!A=&5D('1H870@9&%M

M86=E(&-O=6QD(&]C8W5R('1O('1H92!D<FEV92P@8G5T(&ET(&ES(&QI:V5L

M>2!T;R!F86EL(&EN(&%N('5N9&5T97)M:6YE9"!M86YN97(N#0U3=6=G97-T

M960Z#2 @(" @(" @(" @(" @(" @(" @("L]/3T]/3T]/3T]/3T]/3T]/3TK

M/3T]/3T]/3T]*PT@(" @(" @(" @(" @(" @(" @(" @?"!0;W=E<B!,:6YE

M($1E<VEG;F%T:6]N('P@4&EN($YU;6)E<B!\#2 @(" @(" @(" @(" @(" @

M(" @("L]/3T]/3T]/3T]/3T]/3T]/3TK/3T]/3T]/3T]*PT@(" @(" @(" @

M(" @(" @(" @(" @?" @(" @(" @("LU(%9O;'1S(" @(" @(" @(" @(" @

M(" @?" @(" @(" @(#$@(" @(" @(" @? T@(" @(" @(" @(" @(" @(" @

M(" K+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM*RTM+2TM+2TM+2TM

M+2TM+2L-(" @(" @(" @(" @(" @(" @(" @('P@(" @(" @*S$R(%9O;'1S

M(" @(" @(" @(" @(" @(" @?" @(" @(" @(" R(" @(" @(" @? T@(" @

M(" @(" @(" @(" @(" @(" K+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM

M+2TM*RTM+2TM+2TM+2TM+2TM+2L-(" @(" @(" @(" @(" @(" @(" @('P@

M(" @(" @(" @1W)O=6YD(" @(" @(" @(" @(" @(" @('P@(" @(" @(" S

M(" @(" @(" @? T@(" @(" @(" @(" @(" @(" @(" K+2TM+2TM+2TM+2TM

M+2TM+2TM+2TM+2TM+2TM+2TM*RTM+2TM+2TM+2TM+2TM+2L-#51A8FQE(#(@

M+2!$0R!);G1E<F9A8V4@57-I;F<@,R!0:6X@4&]W97(@0V]N;F5C=&]R#0U;

M161I=&]R)W,@3F]T93H@($AO=R!D;R!W92!S<&5C:69Y('1H92!P:6X@;VYE

M(&]R:65N=&%T:6]N/UT-#2HJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BH-#5-E8W1I;VX@-"XV+"!P86=E(#(Q#0E&

M:7)S="P@<VEM:6QA<B!T;R!T:&4@<')E=FEO=7,@8V]M;65N="P@=&AE('=O

M<FMI;F<@9W)O=7 @:&%S(&%G<F5E9"!T;R!R96UO=F4@<F5F97)E;F-E<R!T

M;R S+C-6+B!4:&ES('=O=6QD(&EN8VQU9&4@=&AE(&5N=')I97,@<W!E8VEF

M:6,@=&\@,RXS5B!A;F0@=&AE(&-O;6UE;G1S(&1I9F9E<F5N=&EA=&EN9R!T

M:&4@-2XP5B!S<&5C:69I8V%T:6]N<RX-#5-E8V]N9"P@82!S=6=G97-T:6]N

M(&9O<B!R:7-E(&%N9"!F86QL("AW:&EC:"!)(&AA9"!B965N(&%S:V5D('1O

M('=R:71E*2X@5&AE(&ME>2!C;VYC97)N('=A<R!W:&5N(&1I<V-U<W-I;F<@

M82!P97)C96YT86=E(&]F(&9U;&P@86UP;&ET=61E+"!J=7-T('=H870@:7,@

M9G5L;"!A;7!L:71U9&4N(%1H92!S=6=G97-T960@=&5X="!R969E<G,@=&\@

M82!R96QA=&EV96QY('-L;W<@<VEG;F%L(&EN=&\@82!W96QL+6)E:&%V960@

M=&5S="!C:7)C=6ET+B!4:&5R92!S:&]U;&0@;F]T(&)E(&%N>2!S:6=N:69I

M8V%N="!O=F5R<VAO;W0@;W(@=6YD97)S:&]O="!P<F]B;&5M<RX-#0Q787,Z

M#7P@('1225-%('P@4FES92!T:6UE(&9O<B!A;GD@<VEG;F%L(&]N($%4(&EN

M=&5R9F%C92@Q*7P@(#5N<R!\(" @(" @("!\#7P@('1&04Q,('P@4FES92!T

M:6UE(&9O<B!A;GD@<VEG;F%L(&]N($%4(&EN=&5R9F%C92@Q*7P@(#5N<R!\

M(" @(" @("!\#2LM+2TM+2TM+2LM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM

M+2TM+2TM+2TM+2TM+2TM+2TM+2LM+2TM+2TK+2TM+2TM+2TK#2@Q*2 @=%))

M4T4@86YD('1&04Q,(&%R92!M96%S=7)E9"!F<F]M(# N-58@=&\@,BXX5B!W

M:71H(&$@<F5S:7-T:79E(&QO860@;V8@,6L@;VAM(&%N9"!A(&-A<&%C:71I

M=F4@;&]A9"!O9B U,"!P1BX-#5-U9V=E<W1E9#H-?" @=%))4T4@?"!2:7-E

M('1I;64@9F]R(&%N>2!S:6=N86P@:6YT;R!T97-T(&-I<F-U:70H,2E\(" U

M;G,@?" @(" @(" @? U\("!T1D%,3"!\(%)I<V4@=&EM92!F;W(@86YY('-I

M9VYA;"!I;G1O('1E<W0@8VER8W5I="@Q*7P@(#5N<R!\(" @(" @("!\#2LM

M+2TM+2TM+2LM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM+2TM

M+2TM+2TM+2LM+2TM+2TK+2TM+2TM+2TK#2@Q*2 @=%))4T4@86YD('1&04Q,

M(')E9F5R('1O(&$@=F]L=&%G92!C:&%N9V4@,3 M.3 E(&]F('1H92!P96%K

M('1O('!E86L@86UP;&ET=61E(&EN=&\@82!R97-I<W1I=F4@;&]A9"!O9B Q

M:R!O:&T@86YD(&$@8V%P86-I=&EV92!L;V%D(&]F(#4P('!&+@T-(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @(" @("!?7U\@(" @(" @-2XP5@T@

M(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @('P-(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @("!<#2 @(" @(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @(" @+PT@(" @(" @(" @(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(%P@(" @(" @(" Q2V]H;0T@(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @("\-(" @(" @(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @("!\#2 @(" @(" @(" @(" @(" @

M(" @(" @(" @(" @(" @(" @(" K+2TM+2 @<VEG;F%L('5N9&5R('1E<W0-

M(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @("!\#2 @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @? T@(" @(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @("!?7U\-(" @(" @(" @(" @(" @

M(" @(" @(" @(" @(" @(" @(" @7U]?(" @(" @(#4P('!&#2 @(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @(" @(" @? T@(" @(" @(" @(" @

M(" @(" @(" @(" @(" @(" @(" @(" @('P-(" @(" @(" @(" @(" @(" @

M(" @(" @(" @(" @(" @("!?*U\-(" @(" @(" @(" @(" @(" @(" @(" @

M(" @(" @(" @("!<(" @("\-(" @(" @(" @(" @(" @(" @(" @(" @(" @

M(" @(" @(" @(%PO#0TJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ#0U396-T:6]N(#0N-BP@<&%G92 R,@T-"4EN

M('1H:7,@;&]A9&EN9R!T86)L92P@1$E/4BT@86YD($1)3U<M(&%R92!D97-C

M<FEB960@87,@=&AR964M<W1A=&4@<VEG;F%L<RX@5&AE<F4@<VAO=6QD(&)E

M('-O;64@<F5F97)E;F-E('-U8V@@=&AA="!T:&4@:&]S="!I<R!R97-P;VYS

M:6)L92!T:&%T('1H97-E('-I9VYA;',@87)E(&%L=V%Y<R!D<FEV96X@;W(@

M<'5L;&5D('1O(&$@;&5G86P@=F]L=&%G92X@1FQO871I;F<@<VEG;F%L<R!O

M;B!T:&5S92!C<FET:6-A;"!I;G!U=',@;6%Y('!R;V1U8V4@=6YP<F5D:6-T

M86)L92!R97-U;'1S+@T-#"HJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BH-#5-E8W1I;VX@-2XQ+"!P86=E(#(T#0T)

M26X@5&%B;&4@-2P@=&AE(&=R;W5N9"!E;G1R>2!I<R!N;W0@8V%P:71A;&EZ

M960N($%S(&%N(&%B<V]L=71E(&YI="P@:70@;6EG:'0@;&]O:R!B971T97(@

M:68@4U!364Y#.B!A;F0@0U-%3"!W97)E(&]N('1H92!S86UE(&QI;F4N#0TJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ#0U396-T:6]N(#4N,BXT+"!P86=E(#(U#0T)5&AE($%402!S<&5C:69I

M8V%T:6]N('-T87)T960@;W5T(&%S(&$@9&5S8W)I<'1I;VX@;V8@=&AE(&1R

M:79E+B!)="!H87,@='5R;F5D(&EN=&\@=&AE(&1E<V-R:7!T:6]N(&]F('1H

M92!I;G1E<F9A8V4N($EN('1H:7,@8V]N=&5X="P@=&AE($1-04-++2!S:6=N

M86P@:7,@9&5S8W)I8F5D(&%S(&]P=&EO;F%L+B!&;W(@=&AE('-I9VYA;"!I

M;G!U=" H=&AE(&1R:79E*2P@=&AE('-I9VYA;"!C86X@8F4@;W!T:6]N86P@

M=VET:"!N;R!I;&PM969F96-T<RX@1F]R('1H92!S:6=N86P@;W5T<'5T("AT

M:&4@:&]S="DL(&EF('1H92!S:6=N86P@:7,@9FQO871I;F<L(&%N(&]P=&EO

M;F%L(&-R:71I8V%L(&EN<'5T(&UA>2!B92!F;&]A=&EN9RP@86=A:6X@<')O

M9'5C:6YG('5N<')E9&EC=&%B;&4@<F5S=6QT<RX@1F]R('1H92!O=71P=70L

M('1H:7,@=&]T96TM<&]L92!S:6=N86P@<VAO=6QD(&)E('-P96-I9FEE9"!A

M<R!D<FEV96X@;W(@<'5L;&5D(&AI9V@N#0TJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ#0U396-T:6]N(#4N,BXQ

M,"P@<&%G92 R-@T)5&AI<R!S96-T:6]N(')E9F5R<R!T;R!T:&4@24Y44E$@

M<VEG;F%L(&%N9"!T:&4@8V]N=F5N=&EO;G,@9F]R(&YE9V%T:6YG(&ET+B!4

M:&4@9F]L;&]W:6YG('1E>'0@<V5E;7,@=&\@8F4@<F5M;W9E9"X@3VYC92!U

M<&]N(&$@=&EM92P@=&AE(&1R:79E<R!W97)E('-U<'!O<V5D('1O(&ME97 @

M24Y44E$@87-S97)T960@;VX@86X@97)R;W(@;W(@=&AE(&QA<W0@<V5C=&]R

M(&]F(&$@<F5A9"!O<&5R871I;VXN($ES('1H92!C;VYS96YS=7,@;V8@=&AE

M(%=O<FMI;F<@1W)O=7 @=&\@<F5M;W9E('1H:7,@;F]T93\-#4]L9"!T97AT

M.@U.3U1%(#,@4V]M92!D<FEV97,@;6%Y(&YE9V%T92!)3E1242!O;B!A(%!)

M3R!D871A('1R86YS9F5R(&-O;7!L971I;VXL(&5X8V5P=" -;VX@82!S:6YG

M;&4@<V5C=&]R(')E860@;W(@;VX@=&AE(&QA<W0@<V5C=&]R(&]F(&$@;75L

M=&DM<V5C=&]R(')E860N#0TJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ#0U396-T:6]N(#4N,BXQ,2P@<&%G92 R

M-@T)5V%S;I)T('1H97)E(&$@8V]N<V5N<W5S('1H870@24]#4S$V+2!W87,@

M;F]T('1O(&)E('5S960@8GD@=&AE(&AO<W0@:6X@4$E/($UO9&4@,R!A;F0@

M:&EG:&5R('-P965D(%!)3R!-;V1E<S\-#0PJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ#0U396-T:6]N(#4N,BXQ

M,BP@<&%G92 R-@T)26X@=&AI<R!P87)A9W)A<&@L($E/4D19(&ES(&1E<V-R

M:6)E9"!A<R!F;&]A=&EN9R!W:&5N(&ET(&ES(&YO="!B96EN9R!N96=A=&5D

M+B!);B!T:&4@;&]A9&EN9R!T86)L92!I;B!396-T:6]N(#0N-BP@:70@:7,@

M9&5S8W)I8F5D(&%S(&$@=&AR964M<W1A=&4@;W5T<'5T+B!4:&ES('!A<F%G

M<F%P:"!N965D<R!T;R!B92!E>'1E;F1E9"X-#5=A<SH-5VAE;B!)3U)$62!I

M<R!N;W0@;F5G871E9"P@24]21%D@<VAA;&P@8F4@:6X@82!H:6=H(&EM<&5D

M86YC92!S=&%T92X-#5-U9V=E<W1E9#H-5VAE;B!T:&4@9')I=F4@:7,@;F]T

M(&EN(&$@=F%L:60@<F5G:7-T97(@86-C97-S(&]R(&EF(&YE:71H97(@1$E/

M4BT@;W(@1$E/5RT@:7,@87-S97)T960L($E/4D19('-H86QL(&)E(&EN(&$@

M:&EG:"!I;7!E9&%N8V4@<W1A=&4N#0TJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ#0U396-T:6]N(#8N,RP@<&%G

M92 S,"P@86YD(%-E8W1I;VX@-BXS+C<L('!A9V4@,S(-"51H97-E('-E8W1I

M;VYS(')E9F5R('1O('1H92!$4DE612!!1$1215-3(')E9VES=&5R+B!)('1H

M;W5G:'0@=&AE(%=O<FMI;F<@1W)O=7 @8V]N<V5N<W5S('=A<R!T;R!R96UO

M=F4@=&AI<R!R96=I<W1E<BP@;&5A=FEN9R!A(&YO=&4@86)O=70@<')E=FEO

M=7,@:6UP;&5M96YT871I;VYS(&AA=FEN9R!U<V5D('1H:7,@861D<F5S<RX-

M#2HJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*BH-#5-E8W1I;VX@."P@=F%R:6]U<R!P86=E<PT)06YO=&AE<B!E9&ET

M;W)I86P@8V]M;65N="P@=&AE('1E<FT@5F5N9&]R(%-P96-I9FEC(&ES('5S

M960@:6X@<V5V97)A;"!P;&%C97,@:6X@=&AI<R!C:&%P=&5R+"!W:&EL92!T

M:&4@=&5R;2!696YD;W(@56YI<75E(&ES(&1E9FEN960@:6X@=&AE($=L;W-S

M87)Y+B!3:&]U;&0@=&AE('1E<FT@5F5N9&]R(%5N:7%U92!B92!U<V5D+"!O

M<B!S:&]U;&0@5F5N9&]R(%-P96-I9FEC(&)E(&%D9&5D('1O('1H92!';&]S

M<V%R>2P@;W(@<VAO=6QD('=E(&QE879E('=E;&P@96YO=6=H(&%L;VYE/PT-

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ

M*BHJ*@T-4V5C=&EO;B X+C$P+CDL(&%N9"!396-T:6]N(#@N,3 N,3 L('!A

M9V4@-#<-"51H97-E('!A<F%G<F%P:',@8V%L;"!O=70@=&AE(')E9VES=&5R

M(&]R9&5R:6YG(&9R;VT@;&]W('1O(&AI9V@L(&DN92X@,C,M,C0M,C4M,C8M

M,C@L(&%N9"!T:&5N('-U9V=E<W0@=&AA="!T:&4@9&%T82!I;B!T:&ES(&9I

M96QD('-H;W5L9"!B92!L969T(&IU<W1I9FEE9"X@26X@=&AI<R!C;VYT97AT

M+"!L969T+"!D;V5S(&YO="!M96%N('1H92!S86UE('1H:6YG('1O(&%L;"!P

M96]P;&4N($D@<W5G9V5S="!T:&4@9F]L;&]W:6YG('1E>'0N("A"5%<L('1H

M92!A;B!W87,@82!M:6YO<B!M:7-T86ME(&EN('1H92!O<FEG:6YA;"D-#5=A

M<SH-5&AE(&-O;G1E;G1S(&]F('1H:7,@9FEE;&0@87)E(&QE9G0@:G5S=&EF

M:65D(&%N('!A9&1E9"!W:71H('-P86-E<R H,C!H*2X-#5-U9V=E<W1E9#H-

M5&AE(&-O;G1E;G1S(&]F('1H:7,@9FEE;&0@87)E(&IU<W1I9FEE9"!T;R!T

M:&4@;&]W(&%D9')E<W,@;&]C871I;VYS+"!A;F0@86YY('5N=7-E9"!L;V-A

M=&EO;G,@87)E('!A9&1E9"!W:71H('-P86-E<R H,C!H*2X-#2HJ*BHJ*BHJ

M*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BHJ*BH-#5!)

M3R!-;V1E(#0-"51H:7,@;6]D92!W87,@;F]T(&EN8VQU9&5D(&EN('1H:7,@

M<F5V:7-I;VXN(%=A<R!T:&4@:6YT96YT:6]N('1O(&=E="!0;&5N87)Y('!E

M<FUI<W-I;VX@9FER<W0L(&]R('-H;W5L9"!T:&4@:6YT96YT:6]N(&)E('1O

M('!R;V1U8V4@=&AE(&1O8W5M96YT(&%S('1H92!7;W)K:6YG($=R;W5P('=I

M<VAE<R!I="!A;F0@869T97(@9&5S8W)I8FEN9R!T:&4@<VEG;FEF:6-A;G0@

M8VAA;F=E<RP@87-K(&9O<B!F;W)W87)D:6YG/PT# *AP"#(@^@(  #H " !@

M";<%]B0                  %$%/@/H ^@#

M      $ "0   U !   " ,$       4    + @     %    # (G $  00

M /<   ,> /___P      ____      #Z^OH "0D) /+R\@ 2$A( XN+B "$A

M(0#0T-  ,C(R ,+"P@! 0$  L+"P %)24@"?GY\ 86%A (*"@@!(2$@ KJZN

M %%140"EI:4 6EI: )R<G !C8V, DY.3 &QL; "*BHH =75U  0    T @

M P   #4 P0   $$+( #, "< 0       )P!        H    0    "<    !

M  $                                      /___P#_____________

M_____________________@    #____P     #_____\    '_______P  /

M_____________        ____X     !______P   #____________P

M    /__^       ?____\     ___________\         '__P       ?_

M___@    !___________P         ?         !__\       '____X

M  ?__________^         /__\      !_____X    '___________^

M     #___\      ?_____\   #_____________P      '_____P   !__

M______\ __________________________________\M    ]P   Q0

M (      @   @(      @ "  (   ("  ,# P # W,  ILKP /_[\ "@H*0

M@("  /\     _P  __\     _P#_ /\  /__ /___P $    - (!  ,

M /__     /\ _P#_  #__P#___\ ________________/3V>!   "  #  $%

M   "    !P   %!"<G5S:       "@   # @," V-" S.0"  0  0DUV 0

M     #X    H    0    "<    !  $      #@!

M         /___P#__________________________________@    #____P

M     #_____\    '_______P  /_____________        ____X     !

M______P   #____________P        /__^       ?____\     ______

M_____\         '__P       ?____@    !___________P         ?

M        !__\       '____X     ?__________^         /__\

M !_____X    '___________^        #___\      ?_____\   #_____

M________P      '_____P   !________\ ________________________

M__________\              04   4    -    3454049)3$5024-4 & )

M  !)^O__R (   @ 8 FW!>8) 0 )   #4 $   ( P0      !0    L"

M  4    , B< 0 !!    ]P   QX ____      #___\      /KZ^@ )"0D

M\O+R !(2$@#BXN( (2$A -#0T  R,C( PL+" $! 0 "PL+  4E)2 )^?GP!A

M86$ @H*" $A(2 "NKJX 45%1 *6EI0!:6EH G)R< &-C8P"3DY, ;&QL (J*

MB@!U=74 !    #0"   #    -0#!    00L@ ,P )P!        G $

M "@   !     )P    $  0

M____ /_________________________________^     /____      /___

M__P    ?_______   _____________\       #____@     '______

M /____________         ___X      !_____P    #___________P

M      ?__       !____^     '___________         !\         '

M__P       ?____@    !___________X         ___P      '_____@

M   ?___________X        /___P     !______P   /_____________

M      ?_____    '________P#_________________________________

M_RT   #W   #%       @     "   " @     "  (  @   @(  P,#  ,#<

MP "FRO  __OP *"@I " @(  _P    #_  #__P    #_ /\ _P  __\ ____

M  0    T @$  P      __\     _P#_ /\  /__ /___P#_____________

M___0SQ'@H;$:X0                     [  , _O\)  8

M  $    !           0   "     0   /[___\          /__________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M__________________________________________\% %, =0!M &T 80!R

M 'D 20!N &8 ;P!R &T 80!T &D ;P!N

M        *  " /_______________P

M                  (   "\ @

M

M    ________________

M

M                                                      #_____

M__________\

M

M                                             /______________

M_P

M                                          !     AO%IJC$%N0$

M                               #    *

M                   #    R0

M      !      (Z-LA,                                    >

M$P   $UI8W)O<V]F="!7;W)D(#8N,              #     0

M                               >     @   #@

M                   #

M            ________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________  ,   $#   " P  (0,  "(#   C P

M) ,  # #   R P  AP,  (T#  "7 P  F@,  +4#  "Z P  R0,  -$#  #G

M P  ] ,  %4$  #6#0  UPT   D.   *#@  V2H  'HR    ^??LYOGAW-C5

MT]73U=/5T]#3 ,L RP#)

M

M

M

M

M

M                         G4!  AU 40$       %70, 7@$#70, !56!

M70, !ET# &,2   (5H%= @!C(   "%:!70  8R    MU 40$X"H  %T  !5U

M 40$W"T  %T  &$ !&( 8Q0 =@$#70  "W4!1 0     70   !D  P   0,

M #(#  "' P  B ,  )8#  "7 P  M ,  +4#  #( P  R0,  .<#  #T P

M]0,  #P$  !3!   5 0  %4$  "'!   B 0  )\$  #]!   _@0   ,%  !?

M!0  8 4  &L%  #+!0  S 4  /X%  #_!0  %@8  /0&  #U!@  ^P8  "T'

M  !K!P  J0<  .<'  #H!P  \P<  "4(  !C"   H0@  -\(  #^  ' (18!

M_@ !P"&+ _X  < AW0#\  ' (2 !_  !P"$@ ?P  < A%@'\  ' (2 !_  !

MP"$6 ?P  < A( '\  ' (18!_  !P"$@ ?P  < A%@'^  ' (18!_@ !P"$6

M ?X  < A%@'^  ' (18!_@ !P"$6 ?H  < A%@'Z  ' (18!^@ !P"$6 ?H

M L A%@'Z  ' (18!^@ !P"$6 ?@  L A%@'Z  ' (18!^@ !P"$6 ?@  L A

M%@'Z  ' (18!^@ !P"$6 ?H  < A%@'Z  ' (18!^@ #P"$6 ?H  < A%@'Z

M        ^  !P"$6 ?@  < A%@'X  ' (18!^  !P"$6 ?H  < A%@'Z  '

M(18!^  !P"$6 ?@  < A%@'X  ' (18!^  !P"$6 0           1$   $2

M   !$    0  +-\(  #@"   $@D  !,)   G"0  <0D  $X*  !/"@  @0H

M ((*  "N"@  :0L  &X+  "L"P  Z@L  "@,  !L#   J@P  .@,   F#0

M9 T  *(-  "C#0  U@T   L.   ,#@  20X  $H.  !L#P  ;0\  (,0  "$

M$   CQ   ,,0   !$0  -1$  (81  #,$0  '!(  &(2  "R$@  ^!(  /D2

M   L$P  +1,  &H3  #^  ' (18!_@ !P"$6 ?X  < A%@'^  ' (18!_@ !

MP"$6 ?X  \ A%@'^  ' (18!_@ !P"$6 ?X  < A%@'^  ' (18!_@ #P"$6

M ?X  < A%@'\  ' (18!_  !P"$6 ?P  < A%@'\  ' (18!_  !P"$6 ?P

M < A%@'\  ' (18!_  !P"$6 ?P  < A%@'\  ' (18!_  !P"$6 ?P  < A

M%@'\  ' (18!_  !P"$6 ?P  < A%@'\  3 (18!_  !P"$6 ?P !, A%@'\

M  ' (18!_@ !P"$6 ?P  < A%@'\  ' (18!_  !P"$6 ?P  < A%@'\  '

M(18!_  !P"$6 ?P  < A%@'\  ' (18!_  !P"$6 ?P  < A%@'\  ' (18!

M_  !P"$6 ?P  < A%@$       $1   !$@ M:A,  &L3  "=$P  GA,  +,3

M  "!%   @A0  ,L5  #,%0  TA4  !H6  !B%@  JA8  !X7   ?%P  *A<

M '07  "^%P  !A@  )H8  ";&   S1@  /48   =&0  11D  'L9  "C&0

MRQD   D:   Q&@  61H  ($:  "U&@  W1H   4;   L&P  5AL  'X;  !_

M&P  L1L  +(;  #'&P  R!L  .<<  #H'   &QT  /X  < A%@'^  ' (18!

M_@ !P"$6 ?X  < A%@'^  / (18!_@ !P"$6 ?X !< A%@'^  ' (18!_@

M     /P  < A%@'\  ' (18!_  !P"$6 ?P  L A%@'^  ' (18!_@ !P"$6

M ?P  < A%@'\  ' (18!_  !P"$6 ?P  L A%@'\  ' (18!_  !P"$6 ?P

M < A%@'\  ' (18!_  !P"$6 ?P  < A%@'\  ' (18!_  !P"$6 ?P  < A

M%@'\  ' (18!_  !P"$6 ?P  < A%@'\  ' (18!_  !P"$6 ?P  < A%@'\

M  ' (18!_  !P"$6 ?P  < A%@'\  ' (18!_@ !P"$6 ?X  < A%@'^  '

M(18!_@ !P"$6 ?X !, A%@'^  ' (18!_@               1$   $2 "T;

M'0  '!T  #$=   R'0  N!T  +D=  #K'0  [!T   ,>   $'@  [Q\  / ?

M   B(   (R   #L@  !B(0  8R$  &TA  "\(0   B(   ,B   U(@  -B(

M $XB  "^(@  OR(  /(B  #S(@  "R,  ,\C  #0(P  U2,  !HD   ;)

M)B0  *LD  "L)   WB0  -\D   0)0  T"4  -$E   #)@  !"8  !TF   W

M)P  _@ !P"$6 ?X  < A%@'^  ' (18!_@ "P"$6 ?X  < A%@'^  ' (18!

M_@ !P"$6 ?X  < A%@'^  ' (18!_@ &P"$6 ?X  < A%@'^  ' (18!_@ !

MP"$6 ?X  < A%@'^  3 (18!_@ !P"$6 ?X  < A%@'\  ' (18!_  !P"$6

M ?X  < A%@'^  ' (18!_@ !P"$6 ?X  < A%@'^  + (18!_@ !P"$6 ?X

M      #^  ' (18!_@ !P"$6 ?X  \ A%@'^  ' (18!_@ !P"$6 ?P  < A

M%@'^  ' (18!_@ !P"$6 ?P  L A%@'^  ' (18!_@ !P"$6 ?X  < A%@'^

M  ' (18!_@ #P"$6 ?X  < A%@'^  ' (18!_@ !P"$6 ?X  < A%@'^  3

M(18!       !$0   1( +3<G   X)P  :B<  &LG  "8)P  RB@  ,LH  #0

M*   &RD  !PI   G*0  I2D  *8I  #8*0  V2D  .0I  #9*@  _@ !P"$6

M ?X  < A%@'^  ' (18!_@ !P"$6 ?X !, A%@'^  ' (18!_@ !P"$6 ?P

M < A%@'^  ' (18!_@ !P"$6 ?P  L A%@'^  ' (18!_@ !P"$6 ?X  < A

M%@'^  ' (18!_@ $P"$6 0

M

M

M

M

M

M

M                                  $    !$@ 0#@ 3  @  0!+  \

M   $ "(  $#Q_P( (@ &3F]R;6%L  (    + %T$ &$)!&(!8Q@

M                 "( 04#R_Z$ (@ 61&5F875L="!087)A9W)A<&@@1F]N

M=               & #^3P$ \@ 8  A4:6UE<R Q,@ "  \    6 /Y/ 0 "

M 18 !E1I;65S*P " !     4 /Y/ 0 2 10  T1O8P   @ 1    &@#^3P$

M(@$:  9,971T97(  @ 2  , 70,       #9)P  " #9*@  !P#_____!P $

M(?__ 0  (?__ @ $(?__ P  (?__!  $(?__!0  (?__!@ $(?__!P

M]@,  $H+  #-$@  Z1D  , ?  "E)@  V2<     !0    $ (@$   ( !0

M  , ,@    0 ,@    4  0    8        #  !Z,@  &@   P  WP@  &H3

M   ;'0  -R<  -DJ   ; !P '0 > !\  0   "$    C    U@H   D+  #9

M)P  $SH4_Q6$$P(5!/< #$IO:&X@1V5L9&UA;B=#.EQ&24Q%4UQ35$%.1$%2

M1%Q!3E-)7$%405Q#3TU-14Y44RY$3T,,2F]H;B!'96QD;6%N(D,Z7$9)3$53

M7%=/4DM<051!4%)/4%Q#3TU-14Y44RY$3T,,2F]H;B!'96QD;6%N(D,Z7$9)

M3$537%=/4DM<051!4%)/4%Q#3TU-14Y44RY$3T,,2F]H;B!'96QD;6%N(D,Z

M7$9)3$537%=/4DM<051!4%)/4%Q#3TU-14Y44RY$3T,,2F]H;B!'96QD;6%N

M(D,Z7$9)3$537%=/4DM<051!4%)/4%Q#3TU-14Y44RY$3T/_0$A0($1E<VM*

M970@-34P0R!0<FEN=&5R $Q05#$Z $1%4TM*151# $A0($1E<VM*970@-34P

M0R!0<FEN=&5R            "@, !$0 '  #%@   0 !          $  0 L

M 0(  0 L 0$ 0 %? 9H!  !\ 0  N &X =<!!  #         $A0($1E<VM*

M970@-34P0R!0<FEN=&5R            "@, !$0 '  #%@   0 !

M  $  0 L 0(  0 L 0$ 0 %? 9H!  !\ 0  N &X =<!!  #          &

M 0 \ 0  / $   <  (  @#P!        .P$  $H %1:0 0  5&EM97,@3F5W

M(%)O;6%N  P6D $" %-Y;6)O;  +)I !  !!<FEA;  *(I !  !(96QV  T2

MD $  %1M<R!2;6X (@ $  ,#"1A6#- "           N<^7%QHCE10     (

M (T    H    R0    $       0 @P

M     "0C+          ,2F]H;B!'96QD;6%N#$IO:&X@1V5L9&UA;@

M      #0SQ'@H;$:X0                     [  , _O\)  8

M      $    !           0   "     0   /[___\          /______

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

M____________________________________________________________

G____________________________________________________

end

From ata-owner  Tue May 17 09:30:10 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00101; Tue, 17 May 94 09:30:10 PDT

Received: from cirrus.com by dt.wdc.com (4.1/SMI-4.1)

id AA29952; Tue, 17 May 94 09:29:58 PDT

Received: from sunstorm.corp.cirrus.com (sunstorm1) by cirrus.com with SMTP id AA05682

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Tue, 17 May 1994 09:29:46 -0700

Received: from ss306.corp.cirrus.com by sunstorm.corp.cirrus.com with SMTP id AA28032

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Tue, 17 May 1994 09:29:45 -0700

Received: by ss306.corp.cirrus.com (4.1-Corp/1.00)

id AA08681; Tue, 17 May 94 09:29:45 PDT

Date: Tue, 17 May 94 09:29:45 PDT

From: johng@corp.cirrus.com (John Geldman)

Message-Id: <9405171629.AA08681@ss306.corp.cirrus.com>

To: ata@dt.wdc.com

Subject: comments on 0948dr1.doc - text only

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Cirrus Logic 3100 West Warren Avenue, Fremont, CA  94538,

Tel. (510) 249-4953, Fax (510) 249-4940

DATE:		5-17-94

TO:		X3T10 ATA working group

FROM:		John Geldman

SUBJECT:	0948dr2.doc comments

*************************************************

These are the comments I have so far from my review of this document.

See you in Harrisburg.

*************************************************

Section 3.1.1, page 13

With two additional connectors in the annex, the type of

connector should not be emphasized.

Was:

ATA defines a register set, a 40-pin connector and the associated

signals on the connector.

Suggested:

ATA defines a register, command, and protocol set, along with connector

and signal definitions.

*************************************************

Section 3.2.5, page 16

The byte order described here is not essential, as all drives

tend to deliver data back in the order it was received. However, the

typical Intel orientation that ATA started with would have had the high

byte of word (x).

Was:

DD DD DD DD DD DD DD DD   DD DD DD DD DD DD DD DD

15 14 13 12 11 10 09 08   07 06 05 04 03 02 01 00

Word(x)     --------Byte(0)--------   --------Byte(1)--------

Word(x+1)   --------Byte(2)--------   --------Byte(3)--------

Suggested:

DD DD DD DD DD DD DD DD   DD DD DD DD DD DD DD DD

15 14 13 12 11 10 09 08   07 06 05 04 03 02 01 00

Word(x)     --------Byte(1)--------   --------Byte(0)--------

Word(x+1)   --------Byte(3)--------   --------Byte(2)--------

*************************************************

Section 4, page 17:

The editor asked for suggestions on how to define legal

connections and cabling configurations. One suggestion is to allow only

one piece and type of cable (to avoid SCSI-type impedance mis-match

reflections), however any connector-based coupler (68 pin drive to 44

pin cable) would be legal.

*************************************************

Section 4.2.2, page 19 Section 4.6, page 21

My notes show that on February 23rd, the Working Group agreed to

remove all references to 3.3V systems as we had not provided a complete

specification for operating at this power level.

Was:

+========================+============+

| Power Line Designation | Pin Number |

+===========+============+============+

|  +5 Volts | +3.3 Volts |     1      |

+-----------+------------+------------+

| +12 Volts |  +5 Volts  |     2      |

+-----------+------------+------------+

|   Ground  |   Ground   |     3      |

+-----------+------------+------------+

Table 2 - DC Interface Using 3 Pin Power Connector

[Editor's Note:  How do we specify the pin one orientation?]

A drive designed for 3,3V applications may be plugged into a receptacle

designed to accept a drive designed for 5V applications, with 12V lines

for additional power.  It is not required that the drive operate, but it

is recommended that precautions be taken to prevent damage to the drive.

A drive designed for 5V applications may be plugged into a receptacle

designed to accept a drive designed for 3,3V applications, with 5V lines

for additional power.  It is not anticipated that damage could occur to

the drive, but it is likely to fail in an undetermined manner.

Suggested:

+=========================+============+

| Power Line Designation | Pin Number |

+=========================+============+

|         +5 Volts       |     1      |

+-------------------------+------------+

|       +12 Volts        |     2      |

+-------------------------+------------+

|          Ground        |     3      |

+-------------------------+------------+

Table 2 - DC Interface Using 3 Pin Power Connector

[Editor's Note:  How do we specify the pin one orientation?]

*************************************************

Section 4.6, page 21

First, similar to the previous comment, the working group has

agreed to remove references to 3.3V. This would include the entries

specific to 3.3V and the comments differentiating the 5.0V

specifications.

Second, a suggestion for rise and fall (which I had been asked to

write). The key concern was when discussing a percentage of full

amplitude, just what is full amplitude. The suggested text refers to a

relatively slow signal into a well-behaved test circuit. There should

not be any significant overshoot or undershoot problems.

Was:

|  tRISE | Rise time for any signal on AT interface(1)|  5ns |        |

|  tFALL | Rise time for any signal on AT interface(1)|  5ns |        |

+--------+--------------------------------------------+------+--------+

(1)  tRISE and tFALL are measured from 0.5V to 2.8V with a resistive

load of 1k ohm and a capacitive load of 50 pF.

Suggested:

|  tRISE | Rise time for any signal into test circuit(1)|  5ns |        |

|  tFALL | Rise time for any signal into test circuit(1)|  5ns |        |

+--------+--------------------------------------------+------+--------+

(1)  tRISE and tFALL refer to a voltage change 10-90% of the peak to

peak amplitude into a resistive load of 1k ohm and a capacitive load of

50 pF.

___       5.0V

|

\

/

\         1Kohm

/

|

+----  signal under test

|

|

___

___       50 pF

|

|

_+_

\   /

\ /

*************************************************

Section 4.6, page 22

In this loading table, DIOR- and DIOW- are described as

three-state signals. There should be some reference such that the host

is responsible that these signals are always driven or pulled to a legal

voltage. Floating signals on these critical inputs may produce

unpredictable results.

*************************************************

Section 5.1, page 24

In Table 5, the ground entry is not capitalized. As an absolute

nit, it might look better if SPSYNC: and CSEL were on the same line.

*************************************************

Section 5.2.4, page 25

The ATA specification started out as a description of the drive.

It has turned into the description of the interface. In this context,

the DMACK- signal is described as optional. For the signal input (the

drive), the signal can be optional with no ill-effects. For the signal

output (the host), if the signal is floating, an optional critical input

may be floating, again producing unpredictable results. For the output,

this totem-pole signal should be specified as driven or pulled high.

*************************************************

Section 5.2.10, page 26

This section refers to the INTRQ signal and the conventions for

negating it. The following text seems to be removed. Once upon a time,

the drives were supposed to keep INTRQ asserted on an error or the last

sector of a read operation. Is the consensus of the Working Group to

remove this note?

Old text:

NOTE 3 Some drives may negate INTRQ on a PIO data transfer completion,

except  on a single sector read or on the last sector of a multi-sector

read.

*************************************************

Section 5.2.11, page 26

Wasn't there a consensus that IOCS16- was not to be used by the

host in PIO Mode 3 and higher speed PIO Modes?

*************************************************

Section 5.2.12, page 26

In this paragraph, IORDY is described as floating when it is not

being negated. In the loading table in Section 4.6, it is described as a

three-state output. This paragraph needs to be extended.

Was:

When IORDY is not negated, IORDY shall be in a high impedance state.

Suggested:

When the drive is not in a valid register access or if neither DIOR- or

DIOW- is asserted, IORDY shall be in a high impedance state.

*************************************************

Section 6.3, page 30, and Section 6.3.7, page 32

These sections refer to the DRIVE ADDRESS register. I thought

the Working Group consensus was to remove this register, leaving a note

about previous implementations having used this address.

*************************************************

Section 8, various pages

Another editorial comment, the term Vendor Specific is used in

several places in this chapter, while the term Vendor Unique is defined

in the Glossary. Should the term Vendor Unique be used, or should Vendor

Specific be added to the Glossary, or should we leave well enough alone?

*************************************************

Section 8.10.9, and Section 8.10.10, page 47

These paragraphs call out the register ordering from low to

high, i.e. 23-24-25-26-28, and then suggest that the data in this field

should be left justified. In this context, left, does not mean the same

thing to all people. I suggest the following text. (BTW, the an was a

minor mistake in the original)

Was:

The contents of this field are left justified an padded with spaces

(20h).

Suggested:

The contents of this field are justified to the low address locations,

and any unused locations are padded with spaces (20h).

*************************************************

PIO Mode 4

This mode was not included in this revision. Was the intention

to get Plenary permission first, or should the intention be to produce

the document as the Working Group wishes it and after describing the

significant changes, ask for forwarding?

From ata-owner  Tue May 17 18:56:00 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA12137; Tue, 17 May 94 18:56:00 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA12130; Tue, 17 May 94 18:55:50 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id SAA19292; Tue, 17 May 1994 18:39:15 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Tue, 17 May 1994 17:37:04 EDT

Date:         Tue May 17 17:37:04 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2dd93901.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      SFF-8019 rev 1.1A -- X3T10 94-089r2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Notes for Rev 1.1A...

Here is the revised SFF-8019, aka X3T10 94-089r? document.

This revision includes the corrected table for Word 1, other

corrections as noted in my previous message, and has all references

to "over 8GB" marked with a ">" at the left margin.

---------------------------------------------------------------

Notes for Rev 1.1...

This is the latest revision of the now famous SFF-8019, aka X3T10

94-089r? document.  While the technical content has not really

changed from the previous revision, much of the text has been

modified and I have not included change bars at the left margin.

I have retained the original SFF-8019 section numbering.

The major change is that I have modified the definition of word 1

such that there should be no misunderstanding about what values

may be placed into the word.

At the 27 April 94 meeting of the ATA Working Group, it was

requested that the SFF Committee "kill" the SFF-8019 project and

convert SFF-8019 to an "informational document".  We can't have

two committees working on the same document!

I will present this at the SFF meeting if requested, but I

request that any discussion of the technical content be conducted

at the ATA Working Group.

Also, the previous versions of this proposal have carried various

titles, all of which are inappropriate.  This proposal applies to

ATA drives of ALL sizes.  Therefore, I now call it "Identify

Drive Data Requirements".

Please, lets get this behind us so that it can be put into the

ATA-2 document!

----------------------------------------------------------------

Identify Drive Data Requirements

SFF-8019 rev 1.1A or X3T10 94-089r2

18 May 94

5. Identify Drive Data Requirements

5.1 Definitions and Background Information

The following abbreviations are used in this proposal:

- 512 MB is used to describe a drive that has 1,032,192 sectors

or 528,482,304 bytes.

- 8 GB is used to describe a drive that has 16,515,072 sectors

or 8,455,716,864 bytes.

The original IBM PC BIOS (Basic Input/Output System) imposed

several restrictions on the support of disk drives, and these

have been incorporated into many higher level software products.

One such restriction limits the capacity of a hard disk drive.

Most BIOS software cannot support a disk drive with more than

1,024 cylinders, 16 heads and 63 sectors per track.  The maximum

addressable capacity of an ATA disk drive under this scheme is

528 MB.

There is growing support of auto-configuration for disk drives on

PC systems.  The auto-configuration capability usually resides in

the BIOS and uses the Identify Drive command data to configure an

ATA disk drive.

This specification defines rules for the Identify Drive data of

all capacity ATA disk drives and allows BIOS support of ATA

drives up to 8 GB using Cylinder/Head/Sector (CHS) addressing.

> It is unlikely that BIOS architecture based on CHS addressing can

> be extented beyond 8 GB.

This specification defines information that newer BIOSs and

system software can use to determine the true size of a disk

drive and access the full capacity of the drive.

5.2 Cylinder, Head and Sector Addressing

BIOSs and other software that operate an ATA disk drive in CHS

(Cylinder, Head and Sector) addressing mode use Identify Drive

data words 1, 3, 6 and words 53-58 to ascertain the appropriate

translation mode to use and determine the capacity of an ATA disk

drive.

Maximum compatibility is achieved if the following rules are

obeyed.  These rules limit the values placed into words 1, 3, 6,

and 53-58.  The rules specified here for CHS addressing apply to

drives up to 8 GB.

5.2.1 Word 1

For drives less than or equal to 528 MB, Identify Data word 1

(Default Cylinders) shall not specify a value greater than 1,024.

If a drive is greater than 528 MB but less than or equal to 8 GB,

the maximum value that shall be placed into this word is determined

by the value in Word 3 as shown in the following table.

Value in Word 3      Maximum value in Word 1

1  1h                 65,535 FFFFh

2  2h                 65,535 FFFFh

3  3h                 65,535 FFFFh

4  4h                 65,535 FFFFh

5  5h                 32,767 7FFFh

6  6h                 32,767 7FFFh

7  7h                 32,767 7FFFh

8  8h                 32,767 7FFFh

9  9h                 16,383 3FFFh

10  Ah                 16,383 3FFFh

11  Bh                 16,383 3FFFh

12  Ch                 16,383 3FFFh

13  Dh                 16,383 3FFFh

14  Eh                 16,383 3FFFh

15  Fh                 16,383 3FFFh

16 10h                 16,383 3FFFh

> There is no restriction on the value of Word 1 for drives over 8

> GB.

The value in this word shall not change.

5.2.2 Word 3

Identify Data word 3 (Default Heads) shall not specify a value

greater than 16.

The value in this word shall not change.

5.2.3 Word 6

For drives of 8 GB or less, Identify Data word 6 (Default

Sectors) shall not specify a value greater than 63.

> For drives over 8 GB, there is no restriction on the value of

> this word except that it shall not exceed 255.

The value in this word shall not change.

5.2.4 Use of words 53 through 58

ATA drives that are over 528 MB shall implement words 53-58.

Drives not over 528 MB may also implement these words.  These

words define the addressing for all sectors accessible in CHS

mode.

5.2.5 Word 53

Identify Data word 53 bit 0 shall be set to 1 at all times that

the drive is in a valid translation mode.  Some drives may have

translation modes that cannot be supported.  An attempt to put a

drive into one of these unsupported modes shall cause word 53 bit

0 to be set to 0 with words 54-58 cleared to zero until a valid

translation mode is established.

5.2.6 Word 54

Identify Data word 54 (Current Cylinders) shall specify the

number of full logical cylinders that can be accessed in the

current translation mode.  If an Initialize Drive Parameters

command has not been executed, the contents of this word shall be

the same as word 1. If an Initialize Drive Parameters command has

been executed, this word is the integer result of dividing the

total number of user sectors (this value may be in words 60-61)

by the number of sectors per logical cylinder ( [word55] x

[word56] ), but shall not be a value greater than 65,535.

5.2.7 Word 55

Identify Data word 55 (Current Heads) is the number of heads

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 16.  If an Initialize

Drive Parameters command has not been executed, the contents of

this word shall be the same as word 3.

5.2.8 Word 56

Identify Data word 56 (Current Sectors) is the number of sectors

specified by the last Initialize Drive Parameters command.  This

word may contain a value of between 1 and 255.  If an Initialize

Drive Parameters command has not been executed, the contents of

this word shall be the same as word 6.

5.2.9 Words 57-58

Identify Data words 57-58 contain a 32-bit value that shall be

equal to [word54] x [word55] x [word56].  If words 60-61, LBA

sectors, are not zero, words 57-58 shall be less than or equal to

the value in words 60-61 at all times.

The 32-bit value is placed into these words as:

- data bits  7- 0 in word 57 bits 15-8

- data bits 15- 8 in word 57 bits  7-0

- data bits 23-16 in word 58 bits 15-8

- data bits 31-24 in word 58 bits  7-0

5.3 Logical Block Addressing

It is recommended that ATA drives over 528 MB support Logical

Block Addressing (LBA).

5.3.1 Words 60-61

Identify Data words 60-61 shall specify the total number of user

sectors available in LBA mode at all times.  This value shall be

equal to or greater than the value in words 57-58 at all times.

The contents of these words shall not change.

The 32-bit value is placed into these words as:

- data bits  7- 0 in word 60 bits 15-8

- data bits 15- 8 in word 60 bits  7-0

- data bits 23-16 in word 61 bits 15-8

- data bits 31-24 in word 61 bits  7-0

5.3.2 Orphan Sectors

The sectors, if any, between the last sector addressable in CHS

mode and the last sector addressable in LBA mode are known as

"orphan" sectors.  A drive may or may not allow access to these

sectors in CHS addressing mode.

The values in words 1, 3 and 6 should be selected such that the

number of orphan sectors is minimized.  Normally, the number of

orphan sectors should not exceed ( [word55] x [word56] - 1 ).

However, the host system can create conditions where there are a

larger number of orphans sectors by issuing the Initialize Drive

Parameters command with values other than the values in words 3

and 6.

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\19.MSG

### Org Date:    05-17-94 04:49:41 PM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     SFF-8019 rev 1.1A -- X3T10 94-089r2

### Attachments: none

From ata-owner  Mon May 23 11:34:31 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18987; Mon, 23 May 94 11:34:31 PDT

Received: from nexus.dt.wdc.com ([192.40.1.32]) by dt.wdc.com (4.1/SMI-4.1)

id AA18980; Mon, 23 May 94 11:34:29 PDT

Received: from st9.wdc.com by nexus.dt.wdc.com (4.1/SMI-4.1)

id AA08040; Mon, 23 May 94 11:30:21 PDT

Date: Mon, 23 May 94 11:30:21 PDT

From: worrell@nexus (Devon Worrell (Worrell) x7042)

Message-Id: <9405231830.AA08040@nexus.dt.wdc.com>

To: atapi@dt.wdc.com, ata@dt.wdc.com

Subject: New Reflector established for QIC IDE Tape activity

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hi fellow ATAPItes,

This message to to notify as many people working on the ATA Standards

and the ATAPI specification as possible, that there is now a reflector

for the new Enhanced IDE Streaming Tape specification effort. This work

is being done under the auspices of the QIC Interface/Format

Subcommittee. Please use the idetape reflector for questions pertaining

ATAPI/Tape.

Subscribing To The Reflector:

The reflector is managed by the automated software package Majordomo which

is capable of automatically responding to a wide range of user requests

such as adding yourself or deleting yourself from a reflector list.

subscribe <list> [<your internet address>]

unsubscribe <list> [<your internet address>]

Examples:

subscribe idetape real_person@host.com

unsubscribe idetape real_person@host.com

info idetape	{retrieve this message}

help		{retrieve list of Majordomo commands}

To obtain a list of the commands Majordomo understands, send an E-mail to

"Majordomo@dt.wdc.com" with the word "help" in the body of the E-Mail.

Majordomo does not scan the subject line so make sure you place your

commands in the body of the e-mail.

Since Majordomo handles several different lists make sure you include the

<list> when you subscribe and unsubscribe.

Sending Mail to the Reflector:

The e-mail addresses for the public reflectors supported by Western Digital

are:

ata@dt.wdc.com

atapi@dt.wdc.com

idetape@dt.wdc.com		*** This is the new reflector **

ssa@dt.wdc.com

You do not need to subscribe to send a message via these reflectors;

however, non-member e-mail may need to be approved before it is forwarded.

FTP Availability:

Each of the supported reflector lists has an anonymous FTP directory which

is available at "fission.dt.wdc.com". Presently the following directories

are available.

fission.dt.wdc.com

/pub/ata/X3T9791D	{X3T9 791D ATA 4a ASCII & rtf files}

/pub/idetape/Rev_*	{QIC 94-46 postscript files}

/pub/sff/SFF8020r1.2	{SFF 8020 r1.2 ATAPI eps & rtf files}

/pub/sff/SFF8011r1.2	{SFF 8011 r1.2 Local bus timing text files}

Note that the above list of FTP directories is only an example since the

actual directory structure will change as new documents are made available.

Devon,

"Every truth passes through three stages before it is recognized.

In the first it is ridiculed, in the second it is opposed, in the

third it is regarded as self-evident!"

_/_/_/_/    _/_/_/_/    _/      _/ | Devon B. Worrell <worrell@dt.wdc.com>

_/      _/  _/      _/  _/      _/  | Senior Principal Engineer

_/      _/  _/_/_/_/    _/  _/  _/   | Western Digital Corporation

_/      _/  _/       _/ _/_/_/_/_/    | 8105 Irvine Center Drive

_/_/_/_/    _/_/_/_/_/  _/      _/     | Irvine, CA 92718

| Host Interface & Central Firmware

| Technology Staging Group

| Phone: (714) 932-7042

| Fax:   (714) 932-7796

"Not speaking in an official capacity for my employer"

"Void where prohibited by law"

"No refunds--all sales are final"

"Does not include CA Redemption Value"

From ata-owner  Mon May 23 11:49:34 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA19257; Mon, 23 May 94 11:49:34 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA19250; Mon, 23 May 94 11:49:32 PDT

Received: with SMTP-MR; Mon, 23 May 1994 11:44:05 PST

Mr-Received: by mta WDUS04; Relayed; Mon, 23 May 1994 11:44:05 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Mon, 23 May 1994 19:28:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Inclusion of Identify drive data in ata-2

To: SFF_REFLECTOR@SUN.COM, ATA@dt.wdc.com

Message-Id: <01HCOEH15LGK8Y5C66@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Mon, 23 May 1994 19:43:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;50441132504991/376759@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

After careful review Western Digital has identified one major issue with

the inclusion of the "Identify Drive for over 528MB drives" into the ata-2

draft standard proposal.

The issue is simple, as an SFF specification the document meets the need of

helping designers to address BIOS compatibility and performance issues by

artificially limiting the capacity of ata drives to less than 8GB.

This is fine as a short term fix however this limitation has no place in a

standard which will see 8GB IDE array products before 1995!

It is therefore Western Digitals opinion that the 8GB limiting statements

be placed within an addendum to the ata-2 standard and marked as

"Informational".

This is essentially the agreement that I believe we reached at the X3T10

ata working group meeting held last week.

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Mon May 23 14:45:18 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23072; Mon, 23 May 94 14:45:18 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA23065; Mon, 23 May 94 14:45:16 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA13684; Mon, 23 May 1994 17:45:15 -0400

From: Rkalish@aol.com

X-Mailer: America Online Mailer

Message-Id: <9405231743.tn404181@aol.com>

To: ata@dt.wdc.com

Date: Mon, 23 May 94 17:43:44 EDT

Subject: 6/1/94 ATA Local Bus Working Group Meeting

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The next Local Bus Working Group will be meeting at the Adaptec Building 4 on

Wednesday, 1 June 1994.  The address is 500 Yosemite Drive, Milpitas

California.  The building is within 1 mile of the Crown Sterling Hotel where

the meetings are usually held.

To get to the Adaptec building from Calavaras Blvd ( Hwy. 237 East ) turn

South on S. Milpitas Blvd. ( Right if coming from I-880, and left if coming

from the Crown Sterling ).  About 3/4 of a mile down the road, there is a

stop light on the corner of S. Milpitas and Yosemite.  Turn right on Yosemite

and then into the first driveway on the left (There will be flags directly in

front of the building).  Let security know you are here for the ATA meeting

and they will direct you to the Sales Demo meeting room.

The meeting will begin at 9:30 as usual, and usually ends around 3:30 PM to

allow for time to get to the airport for late afternoon flights.

The bulk of this next meeting will be for reviewing the ATA-2 Document.  The

948D rev. 2c was distributed at the ATA meeting in Harrisburg PA meeting on

19 May, and there will most likely be a new revision on the SCSI BBS before

this meeting.

Note that is you received a copy of the rev. 2C document a few weeks ago off

the SCSI BBS, you may have a rev. 2b document labeled 2c.  You should get a

new 2c version to verify that you really have a rev. 2c document.

The tentative agenda is as follows:

1.     Opening Remarks

2.     Attendance and Membership introductions

3.     Approval of Agenda

4.     Document Distribution

5.     Meeting Schedule

6.     Old Business

6.1  16.6 MB/s Status

6.2  ATA-3 Specification Status

6.3  ATA-2 Review

7.     New Business

8.     Adjournment

From ata-owner  Tue May 24 06:57:46 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA11988; Tue, 24 May 94 06:57:46 PDT

Received: from inet-gw-2.pa.dec.com by dt.wdc.com (4.1/SMI-4.1)

id AA11981; Tue, 24 May 94 06:57:44 PDT

Received: from enet-gw.pa.dec.com by inet-gw-2.pa.dec.com (5.65/21Mar94)

id AA22738; Tue, 24 May 94 06:52:36 -0700

Received: from advlsi.enet by enet-gw.pa.dec.com (5.65/rmc-22feb94)

id AA14231; Tue, 24 May 94 06:46:58 -0700

From: rinne@advlsi.enet.dec.com

Message-Id: <9405241346.AA14231@enet-gw.pa.dec.com>

Received: from advlsi.enet; by decwrl.enet; Tue, 24 May 94 06:51:08 PDT

Date: Tue, 24 May 94 06:51:08 PDT

To: ata@advlsi.enet.dec.com

Cc: me@advlsi.enet.dec.com

Apparently-To: ata@dt.wdc.com

Subject: Still a question

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I sent the following questions in a couple of weeks ago, and though

I've received some responses, I'm still looking for a definite answer to the

question of loading conditions at which the timing specs should be measured,

>I'm Floyd Rinne, working at Digital in Shrewsbury, MA, and I've got

>a question about the ATA-2 X3T9.2 948D Revision 2 dated 1/1/94.

>In the document, several facts are given:

>	1. (section 10.4) "t7" Addr valid to IOCS16- assertion (mode 3) 30nS

>	2. (section 10.4) "t8" Addr valid to IOCS16- negation (mode 3) 30nS

>	3. (section 4.6) IOCS16- is an OC (open-collector) driver with a

>	   1K (min) pullup at the host.

>	Assuming two devices, plus host plus cable, typical load capacitance

>	on a line will be ~100pF. Device cap is given (section 4.6) as 25pF,

>	cable cap (section 4.5) is 35pF. Host cap unspecified, but I feel

>	that 100 pF is reasonable for the system.

>	The total turn-around time from address valid on the bus, through

>	receivers, decoder and bus driver must be 30 nS in mode 3 (t7 or t8).

>	How is this possible in the case of t8, where only a 1K resistor is

>	pulling the load cap high ? Logic delays aside, the rise time alone

>	will exceed 30nS.

>	Or is there an assumption that the delay times given in section 10.4

>	are to be measured without the cable, host or second drive present.

>Thanks in advance for any assistance you can give me; I've already sent

>a subscription request to the reflector.

>Email will reach me at: rinne@growl.adv.shr.dec.com

>                        floydrinne@aol.com

>Telephone at (508) 841-2576

>Fax at (508) 841-2604

From ata-owner  Tue May 24 18:23:30 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27198; Tue, 24 May 94 18:23:30 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA27158; Tue, 24 May 94 18:23:00 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id ad29007; 24 May 94 19:21 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 24 May 94 17:15:49 MDT

Subject: ATA-3 Project Proposals Letter Ballot Results

To: ATA Reflector <ata@dt.wdc.com>

Date: Tue, 24 May 94 17:07:39 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9405241921.ad29007@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

These are the results of the X3T10 letter ballots on the four ATA-3

Project Proposals.  While these ballots have all passed, we are under

obligation to attempt to resolve the negative comments.

John Lohmeyer

*****************************************************************************

ATA3-PH Letter Ballot Results: 44:6:0:9 = 59

No: Cirrus Logic, Hewlett Packard, IBM, Quantum, Milligan (Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor Electronics,

Maxtor, P.E. Logic, Samsung

------------------------------------------------------------------------------

Cirrus Logic comment:

This is to explain the reason why I voted against some of the ATA-3 proposals.

I approve the ATA-3 project as a whole, but do not agree this project been

broken down into many projects as it stated in the project proposals.  The

reasons for this disagreement are list below:

1. This many project proposals for a single ATA-3 standard create confusion

internally and externally.  It is hard to explain to internal engineers which

project is which spec and it become very difficult to trace and communicate

the progress of each project.  It also not easy to explain to customers about

which level of spec and standard we are and will endorsing.

2. The diagram shown in the project proposal of the ATA-3 BC, ATA-3 XPT, and

ATA-3 PH are integrated piece of standard. It was contained in one standard

in ATA-2 and I do not see a clear reason to break it down into three projects.

As in the initial ATA project discussion, my understanding was these three

blocks represent the way to organize the sections of ATA-X document, rather

than to break it down into projects.

3. ATA-X had a tradition of keep it simple approach and this approach has

proven its effectiveness.  I don not to see a confusion been created by this

many projects.

4. I approve the ATAPI to be its own project in the command definition.

Please feel free to contact me if you have any question.

------------------------------------------------------------------------------

Hewlett Packard comment:

I intend to vote in the negative for all four ATA proposals.  The reasons

for voting in this way are:

1)  SCSI-3 was segmented into multiple documents which

have resulted in tremendous confusion in the industry.

I don't think we should follow that plan.

2)  ATA is simple, cheap and should stay that way.  Adding

packetized interfaces and SCSI commands to it will only

increase the complexity and cost.

3)  Breaking it into multiple documents implies a conformance

to SAM (SCSI-3 Architecture Model) which is not feasible.

There is no reason to stop enhancing the protocol, but this vast overhaul

of the document structure and feature set is not appropriate.  Items such

as ATAPI can be added to the existing protocol document if it is deemed

needed.  It can then be reviewed as a normal process of improving the

existing protocol.

------------------------------------------------------------------------------

IBM comment:

AT Project Proposal Comments

IBM votes no on this proposal for the following reasons:

1) The AT interface cannot comply to SAM and therefore should

not be included under the family of protocols that SAM

defines.

2)Splitting up the AT standard into multiple documents is a

bad idea.  It is causing nothing but problems in SCSI-3.

ATA-3 should be entirely contained within a single document.

------------------------------------------------------------------------------

Quantum comment:

This is a follow up to the conversation we had in Newport Beach concerning the

current drafts of the ATA project proposal(s).  I am very concerned about the

relative lack of specific constraints on the scope, especially in light of our

recent problems in approving a GPP standard.  I am also convinced that a broad

scope allows us to prolong the development of a standard by incrementally

adding new stuff over time.  I would rather limit the scope of the projects,

and authorize either new projects of modify the scope of an exisiting project

at a later time than run the risk of delaying the completion of the standard.

Some suggestions and comments:

Physical

"Provide a means to support the AT Attachment protocol on a variety of

physical interfaces such as that for the newly-emerging requirements for

memory cards."

The only specific item mentioned is support for ATA on memory cards.  I am not

even sure this is needed - do we really want to define physical

characteristics of ATA memory cards independent of those defined by PCMCIA?

And what is meant by PHYSICAL anyway?  Connectors?  Electrical requirements?

If we just want to separate out the physical elements of the ATA-2 document,

then why not say this?  If we want to define physical requirements for new

interconnects (e.g. PCI), then say that.  I  am against incorporaing PCI

material into any of these SD3s, since I think there is a lot of work needed

in that area which would surely keep the standards in development for a long

time to come.

Transport

"Provide a means to support the AT Attachment protocol on a variety of

physical interfaces such as that for the newly-emerging requirements for

memory cards."

This is the same scope as the Physical SD3.  In reading this, the Physical SD3

appears to be a subset of the Transport SD3.  I suggest eliminating this

element of the scope - physical stuff belongs in the physical SD3 only.

"Provide a means to support a variety of device types on the AT Attachement

interface"

This comes about through the strange dual use of this standard by the Block

Command and Packet Interface standards.  I have no idea how this operates in

practice.  Once again, what is transport?

Block and Packet Interface

I have the same concerns about the lack of specifics in the scope for these

SD3s, but I also have 2 global issues.  First, no where does it state whether

Packet Interface is to be SAM compliant.  I would argue for making it SAM

compliant (the whole point of SAM is to allow the device command sets to be

used on alternate physical interconnects without requiring software rewrites

- a highly desirable goal for any market, but especially for the PC market).

But at least the issue of compliance should be addressed.

Second, there is an inherent conflict between the Block and Packet interface

standards regarding disk drives.  It may be unavoidable to continue

independent development of capabilities for disk drives in two different

standards - but I am not convinced of this.  I do not want to support two

different efforts over time in a market which has always been intolerant of

multiple solutions to a single problem.  We should  limit our scope (e.g.

Packet is not to be used for disks, or Block is to be limited to existing

functions with no more improvements) in order to make our work manageable.

I realize that many of these issues have been raised in the ATA working group

(which I have not been regularly attending due to scheduling conflicts - that

should be resolved with the new meeting time).  But I feel they have been

deferred rather than resolved.  Since many of them are issues of policy

direction or simple definition, we should (and in the interest of smooth

operations must) resolve them now.  For these reasons I would have to vote NO

on the proposed letter ballot for the current revisions of all 4 SD3s.

------------------------------------------------------------------------------

Milligan (Seagate) comment:

Comments Accompanying ATA-3 PH No Ballot

I am in favor of the concept of moving to ATA-3 layered standards but have

specific problems with the proposals as they are currently cast. Appropriate

revisions to address the items outlined below would allow the vote to be

changed to Yes.

The following changes should be made to the project proposal:

1) The title page and elsewhere should have the acronym corrected from

(ATA3-PH) to (ATA-3 PH).

2) The Needs do not clearly address the reason for the layering. The need for

layering and consequently ATA-3 PH is twofold. One is to make it easier to

evolve physical interface alternatives. The second is to allow alternative

transport layers. An increase in complexity is not a need and should be avoided

with the ATA since the simplicity is what differentiates it from SCSI.

3) The figure incorrectly identifies the packet transport layer as a command

set. I think it is desirable to keep the ATAPI accronym so I suggest using the

designation ATA-3 Packet Interface Transport Protocol and deleting "command

Set".

4) The SCSI-3 Multimedia Command Set and Other SCSI-3 Command Sets should be

changed to SCSI Like ... Command Set(s). Considerable additional work is

required to determine if it really could be SCSI-3, which I doubt, or if it

should be ATA-3 Command Sets which may be very close to the basic elements of

SCSI.

5) In 2.2 (c) change "on" to "of".

6) In In 2.2 (d) change "Higher transfers" to "Higher transfer rates" and "in"

to "with".

7) Since ATA-3 PH is a specific physical environment, it is not clear what "a

broader range of physical environemnts" is and, if can be determined to have

some meaning in terms of the scope, "on" should be changed to "with".

8) The last paragraph in 2.2 should be moved to 4.2.

9) I question that X3T10 does have sufficient resources. The work relating to

the PH has so far suffered from sufficient modeling experts willing to

participate in the working group meetings. I suggest changing the last sentence

of 3.5 to "Press releases will be utilized in an attempt to augment the X3T10

resources with additional experts in the scope of the ATA-3 PH."

10) 3.7 is currently a misstatement of fact. Delete "for two days".

11) Based upon the frequency of revising the interface standard it seems

incredulous to claim in 3.9 a life of over ten years. I suggest over three

years.

12) Regarding 5.5, ISO/IEC JTC 1 declined to accept the ATA project. X3T10, or

X3T9.2, voted to not pursue an international standard. I suggest changing 5.5

to "In view of the fact that ISO/IEC JTC 1 did not support the prior ATA

project proposal, the X3T10 International representative will be requested to

address the desirability of ATA projects at future JTC 1/SC 25/WG 4 meetings.

------------------------------------------------------------------------------

Unitrode comment:

1. The Proposed standard is to add tape drives and CD-ROMs to the ATA bus,

mapping the SCSI-3 commands on to ATA.  There use to be a need for a bootable

bus to support all the peripherals.  SCSI - Plug and Play has worked with the

BIOS manufactures to include SCSI in the basic BIOS.  SCSI devices already

exist for these applications.

A. SCSI can be booted directly with the new BIOS

B. SCSI cost has been reduced and is only a minor penalty. The cost of

supporting 2 busses fo each device is more than the cost advantage

of ATA.

C. Multimedia requires more than just the CD and Tape, Scanners and

Cameras are being attached to SCSI, these would have to be considered

far ATA.

Note: The original premise for ATA3 is being mmet by SCSI, the vendors are

already complaining about supporting 3 SCSI busses, SPI, FCP, and P1394. Do

we really want another standard to support these devices?

******************************************************************************

ATA3-XPT Letter Ballot Results: 44:6:0:9 = 59

No: Cirrus Logic, Hewlett Packard, IBM, Quantum, Milligan (Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor Electronics,

Maxtor, P.E. Logic, Samsung

------------------------------------------------------------------------------

Cirrus Logic comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Hewlett Packard comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

IBM comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Quantum comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Milligan (Seagate) comment:

Comments Accompanying ATA-3 XPT No Ballot

I am in favor of the concept of moving to ATA-3 layered standards but have

specific problems with the proposals as they are currently cast. Appropriate

revisions to address the items outlined below would allow the vote to be

changed to Yes.

The following changes should be made to the project proposal:

1) The title page and elsewhere should have the acronym corrected from

(ATA3-XPT) to (ATA-3 XPT).

2) The Needs do not clearly address the reason for the layering. The need for

layering and consequently ATA-3 XPT is twofold. One is to make it easier to

evolve physical interface alternatives. The second is to allow alternative

transport layers. An increase in complexity is not a need and should be avoided

with the ATA since the simplicity is what differentiates it from SCSI.

3) The figure incorrectly identifies the packet transport layer as a command

set. I think it is desirable to keep the ATAPI acronym so I suggest using the

designation ATA-3 Packet Interface Transport Protocol and deleting "command

Set".

4) The SCSI-3 Multimedia Command Set and Other SCSI-3 Command Sets should be

changed to SCSI Like ... Command Set(s). Considerable additional work is

required to determine if it really could be SCSI-3, which I doubt, or if it

should be ATA-3 Command Sets which may be very close to the basic elements of

SCSI.

5) The Scope does not make it clear what is covered by the Transport Protocol.

The general outline of what it is to cover should be included. A convenient way

to do this probably would be to indicate that it roughly corresponds to

sections ??? of the ATA standard.

6) Section 2.2 (b) is incorrect. The scope should not be to support a variety

of device types, it should be to support ATA-3 Block Devices. This item seems

to be confused with the ATAPI Transport Protocol.

7) Are there any viable examples of 2.2 (c) "broader range of applications" for

ATA-3 XPT? If there are some, change "on" to "with".

8) The last paragraph in 2.2 should be moved to 4.2.

9) I think there are special terms. Since the ATA is not being defined in

accordance with the Open Systems Reference Model, I presume that Transport does

not correspond to the the OSI model. Therefore I think "Transport" or

"Transport Protocol" should be defined.

10) 3.7 is currently a misstatement of fact. Delete "for two days".

11) Based upon the frequency of revising the interface standard it seems

incredulous to claim in 3.9 a life of over ten years. I suggest over three

years.

12) Regarding 5.5, ISO/IEC JTC 1 declined to accept the ATA project. X3T10, or

X3T9.2, voted to not pursue an international standard. I suggest changing 5.5

to "In view of the fact that ISO/IEC JTC 1 did not support the prior ATA

project proposal, the X3T10 International representative will be requested to

address the desirability of ATA projects at future JTC 1/SC 25/WG 4 meetings.

------------------------------------------------------------------------------

Unitrode comment: (Same comment as for ATA3-PH)

******************************************************************************

ATA3-PI Letter Ballot Results: 45:5:0:9 = 59

No: Hewlett Packard, IBM, Quantum, Milligan (Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor Electronics,

Maxtor, P.E. Logic, Samsung

------------------------------------------------------------------------------

Hewlett Packard comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

IBM comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Quantum comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Milligan (Seagate) comment:

Comments Accompanying ATA-3 PI No Ballot

I am in favor of the concept of moving to ATA-3 layered standards and

especially of accommodating the ATAPI to extend the ATA architecture to CDROM

and low end tape but have specific problems with the proposals as they are

currently cast. Appropriate revisions to address the items outlined below would

allow the vote to be changed to Yes.

The following changes should be made to the project proposal:

1) The title page and elsewhere should have the acronym corrected from

(ATA3-PI) to (ATA-3 PI).

2) In 2.1 change "It is expected to that the SCSI-3 command set sets can be

used for these new device types." to "It is expected that the basic elements of

the non-block SCSI command sets can be used for these additional device types.

If this turns out to not be the case, additional ATA-3 Command Set projects

would be proposed to adapt appropriate SCSI command sets.

3) The figure incorrectly identifies the packet transport layer as a command

set. I think it is desirable to keep the ATAPI acronym so I suggest using the

designation ATA-3 Packet Interface Transport Protocol and deleting "command

Set". (It is conceivable that the diagram is misleading and that the real

intention is to use the ATA-3 Transport Protocol below through the ATAPI Packet

Command with the CDROM or Tape Command Set above.)

4) In this and other ATA-3 project proposals replace references such as in 2.3

to draft ATA standards with references to ATA standards. Likewise in each I

think in 2.2 "will" should be used rather than "shall".

5) The SCSI-3 Multimedia Command Set and Other SCSI-3 Command Sets should be

changed to SCSI Like ... Command Set(s). Considerable additional work is

required to determine if it really could be SCSI-3, which I doubt, or if it

should be ATA-3 Command Sets which may be very close to the basic elements of

SCSI.

6) The Scope does not make it clear what is covered by the PI. The general

outline of what it is to cover should be included.

7) In Section 2.2 (a) is it only the command block that is packetized?

8) In Section 2.2 (a) change "e.g. SCSI command sets" to "i.e. SCSI like

command sets". However some additional work is needed because a command block

is not equal to a command set.

9) In Section 2.2 (b) change  "on" to "with".

10) The last paragraph in 2.2 should be moved to 4.1 and/or 4.2.

11) I think there are special terms. "Packet Interface" should be defined.

12) 3.7 is currently a misstatement of fact. Delete "for two days".

13) Based upon the frequency of revising the interface standard it seems

incredulous to claim in 3.9 a life of over ten years. I suggest over three

years.

14) In 5.1 add SCSI-2 and in 5.2 add SCSI-3.

15) Regarding 5.5, ISO/IEC JTC 1 declined to accept the ATA project. X3T10, or

X3T9.2, voted to not pursue an international standard. I suggest changing 5.5

to "In view of the fact that ISO/IEC JTC 1 did not support the prior ATA

project proposal, the X3T10 International representative will be requested to

address the desirability of ATA projects at future JTC 1/SC 25/WG 4 meetings.

------------------------------------------------------------------------------

Unitrode comment: (Same comment as for ATA3-PH)

******************************************************************************

ATA3-BC Letter Ballot Results: 44:6:0:9 = 59

No: Cirrus Logic, Hewlett Packard, IBM, Quantum, Milligan (Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor Electronics,

Maxtor, P.E. Logic, Samsung

------------------------------------------------------------------------------

Cirrus Logic comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Hewlett Packard comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

IBM comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Quantum comment: (Same comment as for ATA3-PH)

------------------------------------------------------------------------------

Milligan (Seagate) comment:

Comments Accompanying ATA-3 BC No Ballot

I am in favor of the concept of moving to ATA-3 layered standards but have

specific problems with the proposals as they are currently cast. Appropriate

revisions to address the items outlined below would allow the vote to be

changed to Yes.

The following changes should be made to the project proposal:

1) The title page and elsewhere should have the acronym corrected from

(ATA3-BC) to (ATA-3 BC).

2) The Needs do not clearly address the reason for the layering. The need for

layering and consequently ATA-3 BC is twofold. One is to make it easier to

evolve physical interface alternatives. The second is to allow alternative

transport layers. An increase in complexity is not a need and should be avoided

with the ATA since the simplicity is what differentiates it from SCSI.

3) The figure incorrectly identifies the packet transport layer as a command

set. I think it is desirable to keep the ATAPI acronym so I suggest using the

designation ATA-3 Packet Interface Transport Protocol and deleting "command

Set".

4) The SCSI-3 Multimedia Command Set and Other SCSI-3 Command Sets should be

changed to SCSI Like ... Command Set(s). Considerable additional work is

required to determine if it really could be SCSI-3, which I doubt, or if it

should be ATA-3 Command Sets which may be very close to the basic elements of

SCSI.

5) Change Section 2.2 (a) to "Provide the ATA hard disk commands in a separate

layered standard as a complement to the ATA-3 XPT and ATA-3 PH standards."

Note: I presume this statement includes hard disc removable media drives and

excludes floppy disc drives which utilize a different standard.

6) The last paragraph in 2.2 should be moved to 4.1 or 4.2.

7) 3.7 is currently a misstatement of fact. Delete "for two days".

8) Based upon the frequency of revising the interface standard it seems

incredulous to claim in 3.9 a life of over ten years. I suggest over three

years. However the command set may have a longer life than the other ATA-3

standards.

9) Regarding 5.5, ISO/IEC JTC 1 declined to accept the ATA project. X3T10, or

X3T9.2, voted to not pursue an international standard. I suggest changing 5.5

to "In view of the fact that ISO/IEC JTC 1 did not support the prior ATA

project proposal, the X3T10 International representative will be requested to

address the desirability of ATA projects at future JTC 1/SC 25/WG 4 meetings.

------------------------------------------------------------------------------

Unitrode comment: (Same comment as for ATA3-PH)

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Wed May 25 10:00:23 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14704; Wed, 25 May 94 10:00:23 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA14697; Wed, 25 May 94 10:00:18 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA01214; Wed, 25 May 1994 13:00:17 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9405251259.tn473208@aol.com>

To: ata@dt.wdc.com

Date: Wed, 25 May 94 12:59:32 EDT

Subject: Minutes ATA WG 5/19/94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

May 25, 1994

-----------------------------------------------------------------------------

Minutes of ATA Working Group - May 19, 1994        X3T10/94-114r0

4

Accredited Standards Committee*

X3, Information Processing Systems

Doc.             No.

X3T10/94-114R0

Date:            May

19, 1994

Project:

Ref. Doc.:

Reply to: S. Finch

To:    Membership of X3T10

From:  Lamers/Finch

Subject:  Minutes of X3T10 ATA  Working Group May 19, 1994

Agenda

1. Opening Remarks                                         2

2. Attendance and Membership, Introductions                2

3. Approval of Agenda                                      3

4. Document Distribution                                   3

5. Old Business                                            3

5.2 16.6 MB/s Issues [94-045r3] (Hanan/Finch)         3

5.1 Identify Drive for Over 528 MB  [94-089r2] (Landis)

3

6. New Business                                            3

6.1 Project Proposals Comment Resolution [94-019,94-020,94-

021,94-022] (Finch)                                   3

6.1 Comments on ATA-2 [94-115] (Geldman)              4

7. Meeting Schedule                                        4

8. Adjournment                                             4

Results of Meeting

1.   Opening Remarks

Steve  Finch  convened  the meeting at 9:00  am.   Steve  thanked

Charles Brill of AMP for hosting the meeting.

As  is customary, the people attending introduced themselves.   A

copy  of  the  attendance list was circulated for attendance  and

corrections.

It  was stated that the meeting had been authorized by X3T10  and

would  be conducted under the X3 rules.  Ad hoc meetings take  no

final  actions, but prepare recommendations for approval  by  the

X3T10 task group.  The voting rules for the meeting are those  of

the  parent  committee, X3T10.  These rules are:   one  vote  per

company; and any participating company member may vote.

The  minutes of this meeting will be posted to the SCSI  BBS  and

the  SCSI  Reflector and will be included in the  next  committee

mailing.

2.   Attendance and Membership, Introductions

Attendance at working group meetings does not count toward

minimum attendance requirements for X3T10 membership.  Working

group meetings are open to any person or company to attend and to

express their opinion on the subjects being discussed.

The following people attended the meeting.

Alan Olson          3M Company          usmmm5cd@ibmmail.com

Richard Kalish      Adaptec             rkalish@rahul.net

Lawrence J. Lamers  Adaptec             ljlamers@adaptec.com

Norm Harris         Adaptec             nharris@adaptec.com

John Lohmeyer       AT&T/NCR            john.lohmeyer@ftcollins

Microelectronics    co.ncr.com

John Geldman        Cirrus Logic        johng@cirrus.com

Joseph Chen         Cirrus Logic        chen@cirrus.com

Ken Hallam          ENDL                khallam@endlas.com

Dal Allan           ENDL                250-752@mcimail.com

Edward Hu           Future Domain

Rene Macare         Future Domain       renem@fdc.mhs.compuserv

e.com

Nancy Cheng         Hitachi Computer    n.chen@hitachi.com

Products

Dan Colegrove       IBM                 colegrove@vnet.ibm.com

Dean Wallace        Linfinity

Pete McClean        Maxtor              pete_mcclean@maxtor.com

Ray Heineman        Maxtor              ray_heineman@maxtor.com

Jay Neer            Molex

Bruce Anderson      NEC Technologies    banderson@nectech.com

Skip Jones          Qlogic              sk_jones@qlc.com

Mike Carpenter      Qlogic              m_carpenter@qlc.com

Jim McGrath         Quantum Corp        JMCGRATH@QNTM.COM

John Masiewicz      Seagate Technology  masiewicz@notes.seagate

.com

Hale Landis         Seagate Technology  landis@notes.seagate.co

m

Gene Milligan       Seagate Technology  Gene_Milligan@notes.sea

gate.com

Stephen Finch       Silicon Systems     5723283@mcimail.com

Paul Aloisi         Unitrode            Aloisi@uicc.com

Integrated Circuit

Tom Hanan           Western Digital     t_hanan@dt.wdc.com

3.   Approval of Agenda

The agenda was approved as proposed.

4.   Document Distribution

Copies of  ATA-2 Proposed Working Draft revision 2c was

distributed.

X3T10/94-115r0 "Comments on ATA-2" was distributed.

5.   Old Business

5.2  16.6 MB/s Issues [94-045r3] (Hanan/Finch)

Jim McGrath stated that his understanding of the PIO timing

diagram was that a device cannot begin deassertion of IORDY until

DIOR/DIOW is asserted instead of after the address is stable.  He

stated that he believed that this was not the intent of the

group.

The conclusion was to put `xxx' from Addr Valid to DIOR/DIOW

asserted for IORDY (1-2) and (1-3).

Jim McGrath stated that his understanding was that a device does

not want to validate use of IOCS16- in higher speed modes.

Tom wants a * next to IOCS16 and t7 removed and the note stating

the signal is optional and shown for reference only.  John

Geldman stated this should be for both mode 3 and mode 4.  Jim

McGrath stated that he wants it to be a do not connect line.

Larry Lamers pointed out that the wording should be constructed

with a view to the future use of this line for error detection.

The following note will be added `IOCS16- is shown for modes

0,1,2.  For other modes this signal is not valid.  The timings t7

and t8 only apply to modes 0,1,2.'

Dal Allan requested that the t8 timing indicating IOCS16 negation

be extended so that it is not confused with data transitions.

Agreed.  Add note on t7 and t8 to table on PIO timing parameters

as note 5.  The values should be delete also for Mode 4 table.

5.1  Identify Drive for Over 528 MB  [94-089r2] (Landis)

A question of what the disposition of SFF-8019 should be vis-a-

vis the X3T10 document.  The problem stems from having two

editors.

Technically this is still an SFF document.  Hale withdrew the

proposal from ATA until the SFF completes its review.

6.   New Business

6.1  Project Proposals Comment Resolution [94-019,94-020,94-

021,94-022] (Finch)

John Lohmeyer presented the results of the letter ballots on the

ATA-3 project proposals.

ATA3-PH Letter Ballot Results: 44:6:0:9 = 59

No: Cirrus Logic, Hewlett Packard, IBM, Quantum, Milligan

(Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor

Electronics,

Maxtor, P.E. Logic, Samsung

ATA3-XPT Letter Ballot Results: 44:6:0:9 = 59

No: Cirrus Logic, Hewlett Packard, IBM, Quantum, Milligan

(Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor

Electronics,

Maxtor, P.E. Logic, Samsung

ATA3-PI Letter Ballot Results: 45:5:0:9 = 59

No: Hewlett Packard, IBM, Quantum, Milligan (Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor

Electronics,

Maxtor, P.E. Logic, Samsung

ATA3-BC Letter Ballot Results: 44:6:0:9 = 59

No: Cirrus Logic, Hewlett Packard, IBM, Quantum, Milligan

(Seagate), Unitrode

Did not respond: AMD, Apple, Compaq, DPT, Future Domain, Harbor

Electronics,

Maxtor, P.E. Logic, Samsung

The basic Issues raised include:

Is it 4, 2, or 1 project.

Relationship to SCSI-3; SAM compliance; use of SCSI-3 commands.

Clarity of division; define transport more clearly; define packet

interface;

Should the ATA-PI specifically state that SCSI Block Commands are

not to be used?

6.1  Comments on ATA-2 [94-115] (Geldman)

Defered to next meeting.

7.   Meeting Schedule

June 1, 1994 at Adaptec in Milpitas, CA.

June 29, 1994 at Western Digital in Irvine, CA.

July 21, 1994 at Sheraton in Manchester, NH.

8.   Adjournment

The meeting adjourned at approximately 1:00 PM.

_______________________________

*Operating under the procedures of The American National

Standards Institute.

X3 Secretariat, Computer and Business Equipment Manufacturers

Association (CBEMA)

1250 Eye Street NW, Suite 200, Washington, DC 20005-3922

Telephone: 202-737-8888 (Press 1 twice) FAX: 202-638-4922 or 202-

628-2829

From ata-owner  Wed May 25 10:00:31 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14712; Wed, 25 May 94 10:00:31 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA14705; Wed, 25 May 94 10:00:23 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA01289; Wed, 25 May 1994 13:00:22 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9405251300.tn473222@aol.com>

To: ata@dt.wdc.com

Date: Wed, 25 May 94 13:00:03 EDT

Subject: ATA3 LB Comments

Sender: ata-owner@dt.wdc.com

Precedence: bulk

May 25, 1994

-----------------------------------------------------------------------------

X3T10/94-120r0

7

Accredited Standards Committee*

X3, Information Processing Systems

Doc.             No.

X3T10/94-120R0

Date:            May

19, 1994

Project:

Ref. Doc.:

Reply     to:     J.

Lohmeyer

To:    Membership of X3T10

From:  L. Lamers

Subject:  ATA-3 Project Proposals Letter Ballot Comments

ATA3-PH Letter Ballot Results:      44:6:0:9 = 59

No:            Cirrus Logic, Hewlett Packard, IBM, Quantum,

Milligan (Seagate), Unitrode

Did not respond:    AMD, Apple, Compaq, DPT, Future Domain,

Harbor Electronics, Maxtor, P.E. Logic, Samsung

=================================================================

================

Cirrus Logic comment:

This is to explain the reason why I voted against some of the ATA-

3 proposals.  I approve the ATA-3 project as a whole, but do not

agree this project been broken down into many projects as it

stated in the project proposals.  The reasons for this

disagreement are list below:

1.This many project proposals for a single ATA-3 standard create

confusion internally and externally.  It is hard to explain to

internal engineers which project is which spec and it become

very difficult to trace and communicate the progress of each

project.  It also not easy to explain to customers about which

level of spec and standard we are and will endorsing.

2.The diagram shown in the project proposal of the ATA-3 BC, ATA-

3 XPT, and ATA-3 PH are integrated piece of standard. It was

contained in one standard in ATA-2 and I do not see a clear

reason to break it down into three projects.  As in the

initial ATA project discussion, my understanding was these

three blocks represent the way to organize the sections of ATA-

X document, rather than to break it down into projects.

3.ATA-X had a tradition of keep it simple approach and this

approach has proven its effectiveness.  I don not to see a

confusion been created by this many projects.

4.I approve the ATAPI to be its own project in the command

definition.

Please feel free to contact me if you have any question.

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

Hewlett Packard comment:

I intend to vote in the negative for all four ATA proposals.  The

reasons for voting in this way are:

1)SCSI-3 was segmented into multiple documents which have

resulted in tremendous confusion in the industry.  I don't

think we should follow that plan.

2)ATA is simple, cheap and should stay that way.  Adding

packetized interfaces and SCSI commands to it will only

increase the complexity and cost.

3)Breaking it into multiple documents implies a conformance to

SAM (SCSI-3 Architecture Model) which is not feasible.

There is no reason to stop enhancing the protocol, but this vast

overhaul of the document structure and feature set is not

appropriate.  Items such as ATAPI can be added to the existing

protocol document if it is deemed needed.  It can then be

reviewed as a normal process of improving the existing protocol.

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

IBM comment:

AT Project Proposal Comments

IBM votes no on this proposal for the following reasons:

1)   The AT interface cannot comply to SAM and therefore should

not be included under the family of protocols that SAM defines.

2)   Splitting up the AT standard into multiple documents is a

bad idea.  It is causing nothing but problems in SCSI-3.  ATA-3

should be entirely contained within a single document.

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

Quantum comment:

This is a follow up to the conversation we had in Newport Beach

concerning the current drafts of the ATA project proposal(s).  I

am very concerned about the relative lack of specific constraints

on the scope, especially in light of our recent problems in

approving a GPP standard.  I am also convinced that a broad scope

allows us to prolong the development of a standard by

incrementally adding new stuff over time.  I would rather limit

the scope of the projects, and authorize either new projects of

modify the scope of an exisiting project at a later time than run

the risk of delaying the completion of the standard.

Some suggestions and comments:

Physical

"Provide a means to support the AT Attachment protocol on a

variety of physical interfaces such as that for the newly-

emerging requirements for memory cards."

The only specific item mentioned is support for ATA on memory

cards.  I am not even sure this is needed - do we really want to

define physical characteristics of ATA memory cards independent

of those defined by PCMCIA?  And what is meant by PHYSICAL

anyway?  Connectors?  Electrical requirements?

If we just want to separate out the physical elements of the ATA-

2 document, then why not say this?  If we want to define physical

requirements for new interconnects (e.g. PCI), then say that.  I

am against incorporaing PCI material into any of these SD3s,

since I think there is a lot of work needed in that area which

would surely keep the standards in development for a long time to

come.

Transport

"Provide a means to support the AT Attachment protocol on a

variety of physical interfaces such as that for the newly-

emerging requirements for memory cards."

This is the same scope as the Physical SD3.  In reading this, the

Physical SD3 appears to be a subset of the Transport SD3.  I

suggest eliminating this element of the scope - physical stuff

belongs in the physical SD3 only.

"Provide a means to support a variety of device types on the AT

Attachement interface"

This comes about through the strange dual use of this standard by

the Block Command and Packet Interface standards.  I have no idea

how this operates in practice.  Once again, what is transport?

Block and Packet Interface

I have the same concerns about the lack of specifics in the scope

for these SD3s, but I also have 2 global issues.  First, no where

does it state whether Packet Interface is to be SAM compliant.  I

would argue for making it SAM compliant (the whole point of SAM

is to allow the device command sets to be used on alternate

physical interconnects without requiring software rewrites - a

highly desirable goal for any market, but especially for the PC

market).  But at least the issue of compliance should be

addressed.

Second, there is an inherent conflict between the Block and

Packet interface standards regarding disk drives.  It may be

unavoidable to continue independent development of capabilities

for disk drives in two different standards - but I am not

convinced of this.  I do not want to support two different

efforts over time in a market which has always been intolerant of

multiple solutions to a single problem.  We should  limit our

scope (e.g.  Packet is not to be used for disks, or Block is to

be limited to existing functions with no more improvements) in

order to make our work manageable.

I realize that many of these issues have been raised in the ATA

working group (which I have not been regularly attending due to

scheduling conflicts - that should be resolved with the new

meeting time).  But I feel they have been deferred rather than

resolved.  Since many of them are issues of policy direction or

simple definition, we should (and in the interest of smooth

operations must) resolve them now.  For these reasons I would

have to vote NO on the proposed letter ballot for the current

revisions of all 4 SD3s.

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

Milligan (Seagate) comment:

Comments Accompanying ATA-3 PH No Ballot

I am in favor of the concept of moving to ATA-3 layered standards

but have specific problems with the proposals as they are

currently cast. Appropriate revisions to address the items

outlined below would allow the vote to be changed to Yes.

The following changes should be made to the project proposal:

1)The title page and elsewhere should have the acronym corrected

from (ATA3-PH) to (ATA-3 PH).

2)The Needs do not clearly address the reason for the layering.

The need for layering and consequently ATA-3 PH is twofold.

One is to make it easier to evolve physical interface

alternatives. The second is to allow alternative transport

layers. An increase in complexity is not a need and should be

avoided with the ATA since the simplicity is what

differentiates it from SCSI.

3)The figure incorrectly identifies the packet transport layer

as a command set. I think it is desirable to keep the ATAPI

accronym so I suggest using the designation ATA-3 Packet

Interface Transport Protocol and deleting "command Set".

4)The SCSI-3 Multimedia Command Set and Other SCSI-3 Command

Sets should be changed to SCSI Like ... Command Set(s).

Considerable additional work is required to determine if it

really could be SCSI-3, which I doubt, or if it should be ATA-

3 Command Sets which may be very close to the basic elements

of SCSI.

5)In 2.2 c change "on" to "of".

6)In In 2.2 (d) change "Higher transfers" to "Higher transfer

rates" and "in" to "with".

7)Since ATA-3 PH is a specific physical environment, it is not

clear what "a broader range of physical environemnts" is and,

if can be determined to have some meaning in terms of the

scope, "on" should be changed to "with".

8)The last paragraph in 2.2 should be moved to 4.2.

9)I question that X3T10 does have sufficient resources. The work

relating to the PH has so far suffered from sufficient

modeling experts willing to participate in the working group

meetings. I suggest changing the last sentence of 3.5 to

"Press releases will be utilized in an attempt to augment the

X3T10 resources with additional experts in the scope of the

ATA-3 PH."

10)    3.7 is currently a misstatement of fact. Delete "for two

days".

11)    Based upon the frequency of revising the interface

standard it seems incredulous to claim in 3.9 a life of over

ten years. I suggest over three years.

12)    Regarding 5.5, ISO/IEC JTC 1 declined to accept the ATA

project. X3T10, or X3T9.2, voted to not pursue an

international standard. I suggest changing 5.5 to "In view of

the fact that ISO/IEC JTC 1 did not support the prior ATA

project proposal, the X3T10 International representative will

be requested to address the desirability of ATA projects at

future JTC 1/SC 25/WG 4 meetings.

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

Unitrode comment:

1.The Proposed standard is to add tape drives and CD-ROMs to the

ATA bus, mapping the SCSI-3 commands on to ATA.  There use to

be a need for a bootable bus to support all the peripherals.

SCSI - Plug and Play has worked with the BIOS manufactures to

include SCSI in the basic BIOS.  SCSI devices already exist

for these applications.

A.SCSI can be booted directly with the new BIOS

B.SCSI cost has been reduced and is only a minor penalty. The

cost of supporting 2 busses fo each device is more than the

cost advantage of ATA.

C.Multimedia requires more than just the CD and Tape, Scanners

and

Cameras are being attached to SCSI, these would have to be

considered far ATA.

Note: The original premise for ATA3 is being mmet by SCSI, the

vendors are already complaining about supporting 3 SCSI busses,

SPI, FCP, and P1394. Do we really want another standard to

support these devices?

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

++++++++++++++++

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

++++++++++++++++

ATA3-XPT Letter Ballot Results:    44:6:0:9 = 59

No:            Cirrus Logic, Hewlett Packard, IBM, Quantum,

Milligan (Seagate), Unitrode

Did not respond:    AMD, Apple, Compaq, DPT, Future Domain,

Harbor Electronics,

Maxtor, P.E. Logic, Samsung

=================================================================

================

Cirrus Logic comment: (Same comment as for ATA3-PH)

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

Hewlett Packard comment: (Same comment as for ATA3-PH)

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

IBM comment: (Same comment as for ATA3-PH)

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

Quantum comment: (Same comment as for ATA3-PH)

-----------------------------------------------------------------

-----------------------------------------------------------------

---------

Milligan (Seagate) comment:

Comments Accompanying ATA-3 XPT No Ballot

I am in favor of the concept of moving to ATA-3 layered standards

but have specific problems with the proposals as they are

currently cast. Appropriate revisions to address the items

outlined below would allow the vote to be changed to Yes.

The following changes should be made to the project proposal:

1)The title page and elsewhere should have the acronym corrected

from (ATA3-XPT) to (ATA-3 XPT).

2)The Needs do not clearly address the reason for the layering.

The need for layering and consequently ATA-3 XPT is twofold.

One is to make it easier to evolve physical interface

alternatives. The second is to allow alternative transport

layers. An increase in complexity is not a need and should be

avoided with the ATA since the simplicity is what

differentiates it from SCSI.

3)The figure incorrectly identifies the packet transport layer

as a command set. I think it is desirable to keep the ATAPI

acronym so I suggest using the designation ATA-3 Packet

Interface Transport Protocol and deleting "command Set".

4)The SCSI-3 Multimedia Command Set and Other SCSI-3 Command

Sets should be changed to SCSI Like ... Command Set(s).

Considerable additional work is required to determine if it

really could be SCSI-3, which I doubt, or if it should be ATA-

3 Command Sets which may be very close to the basic elements

of SCSI.

5)The Scope does not make it clear what is covered by the

Transport Protocol.

The general outline of what it is to cover should be included. A

convenient way to do this probably would be to indicate that

it roughly corresponds to sections ??? of the ATA standard.

6)Section 2.2 (b) is incorrect. The scope should not be to

support a variety of device types, it should be to support ATA-

3 Block Devices. This item seems to be confused with the ATAPI

Transport Protocol.

7)Are there any viable examples of 2.2 c "broader range of

applications" for ATA-3 XPT? If there are some, change "on" to

"with".

8)The last paragraph in 2.2 should be moved to 4.2.

9)I think there are special terms. Since the ATA is not being

defined in accordance with the Open Systems Reference Model, I

presume that Transport does not correspond to the the OSI

model. Therefore I think "Transport" or "Transport Protocol"

should be defined.

10)    3.7 is currently a misstatement of fact. Delete "for two

days".

11)    Based upon the frequency of revising the interface

standard it seems incredulous to claim in 3.9 a life

From ata-owner  Thu Jun  2 06:42:22 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06039; Thu, 2 Jun 94 06:42:22 PDT

Received: from mailserver.conner.com by dt.wdc.com (4.1/SMI-4.1)

id AA06032; Thu, 2 Jun 94 06:42:20 PDT

Received: from BanyanMail.conner.com by mailserver.conner.com with smtp

(Smail3.1.28.1 #1) id m0q9Ct9-000AUbC; Thu, 2 Jun 94 06:32 PDT

Received: by BanyanMail.conner.com; Thu, 2 Jun 94 6:42:46 PDT

Date: Thu, 2 Jun 94 6:35:24 PDT

Message-Id: <9+h8+e6Svha@BanyanMail.conner.com>

From: Steve.Anderson@conner.com

To: ata@dt.wdc.com, sff_reflector@Sun.COM

Subject: Internet ALIAS

X-Incognito-Sn: 379

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

My company has provided and Internet Alias for me so that

you the world do not have to spend a day typing my address.

My new address is Steve.Anderson@conner.com

Thank you.

Steve

From ata-owner  Thu Jun  2 16:58:51 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA19726; Thu, 2 Jun 94 16:58:51 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA19719; Thu, 2 Jun 94 16:58:49 PDT

Received: with SMTP-MR; Thu, 2 Jun 1994 16:56:15 PST

Mr-Received: by mta WDUS04; Relayed; Thu, 02 Jun 1994 16:56:15 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Fri, 3 Jun 1994 00:53:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: June 29th ATA-2 MTG. NOTICE

To: ATA@dt.wdc.com

Message-Id: <01HD2O7RQYI68Y6O9E@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Fri, 3 Jun 1994 00:54:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;51656120604991/391860@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

*************************************************************************

**************   ATA-2   M E E T I N G   N O T I C E   ******************

*************************************************************************

SUBJECT:	ATA-2 Ad-Hoc REVIEW MEETING

DATE:		June 29 (Wed.)

TIME:		9:30AM - 4PM

PLACE:          Western Digital

ADDRESS:        8105 irvine Center drive

Irvine CA. 92718-2902

Contact:        Tom Hann, 714 932-7472, hanan_t@a1.wdc.com

*************************************************************************

Orange County International is

the closest airport.

Direction:     	Follow the signs from Orange County International leading to

the 405 FWY (SOUTH). Take the 405 FWY south 7 miles to

Irvine Center Drive. Exit the FWY and take a left onto Irvine

Center Drive, which should take you back over the 405 FWY.

After you cross over the 405 FWY you should see the tall brown

High Rise Western Digital building on the right hand side.

Go just past the building and take a right on PACIFICA.

Take PACIFICA to the next signal and take a right onto FORTUNE.

Take the first right hand entrance after turning onto FORTUNE

to enter Western Digitals parking lot.

Visitor parking is on your left as you enter the parking lot.

You will not need a badge, so just ask the guard for directions

to the multi purpose room in the first floor cafeteria.

Good Luck!!!

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Wed Jun  8 05:56:21 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23262; Wed, 8 Jun 94 05:56:21 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA23255; Wed, 8 Jun 94 05:56:19 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA11046; Wed, 8 Jun 1994 08:56:17 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9406080856.tn957867@aol.com>

To: ata@dt.wdc.com

Date: Wed, 08 Jun 94 08:56:16 EDT

Subject: New Phone Number

Sender: ata-owner@dt.wdc.com

Precedence: bulk

6/7/94

-------------------------------------------------------------------------

ATA Folks - Please note that I have a new telephone number.

(408) 957-7817   It is a direct dial line - so it will be easier

to leave voicemail.  The old extension will not work anymore

as some have already discovered.

Lawrence J. Lamers

Adaptec                    TEL: 408-957-7817

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@AOL.COM

From ata-owner  Wed Jun  8 16:08:28 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06326; Wed, 8 Jun 94 16:08:28 PDT

Received: from ns.Novell.COM by dt.wdc.com (4.1/SMI-4.1)

id AA06316; Wed, 8 Jun 94 16:08:25 PDT

Received: from  by ns.Novell.COM (4.1/SMI-4.1)

id AC00443; Wed, 8 Jun 94 17:07:59 MDT

X-Nvlenv-01Date-Transferred:  8-Jun-1994 15:09:49 -0400; at novell

X-Nvlenv-01Date-Posted: 08-Jun-1994 15:17:59 -0400; at

Prv-Mail-Corp.Novell

To: ata%dt.wdc.com@ngmhs.IS.Provo.Novell.Com

Message-Id: <D116F62D01610370@-SMF->

Subject: ATA Issues

From: Blair_Merrell@Novell.COM (Merrell, Blair)

Date: 08 Jun 94 15:08:48 EDT

References: <B0D3F46D02610370@-SMF->

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Part of my responsibilities at Novell it to define test

specifications and perform certification testing for third-party

storage devices and device drivers in a NetWare server

environment.  One of the issues I deal with and attempt to quantify

is that of fault tolerance.  I would like some feedback from you the

experts on ATA devices.  Are ATA devices fault tolerant, and if

not is there any movement to push the interface in that direction?

Currently as part of our fault tolerance testing, we power

down one of two devices on a channel and watch to see how the

other device is affected.  For all but one ATA manufacture I've

dealt with, this has an adverse effect on a ATA master/slave

configuration.  If the master dies the slave ceases to communicate

and vise versa.  I know that an 18 inch cable length almost requires

that ATA devices be housed internal to a system and so also be

connected to system power.  Is this the only configuration in which

ATA devices are used, or are there others as the ATA interface

spec allows?  If not is the power down test valid? (What about

future needs such as hot-replaceable drives, or brown out

conditions where one drive resets and the other doesn't.)

In the case of the one manufacture whose drives don't die

during the power down test, if power is restored to the master

drive, the slave will stop communicating when the master

completes its reset.  Why? because the slave is unaware of the reset

and doesn't give the proper handshake to make its presence known.

Moreover, it appears to me that once the drives are in this state

there is no way in software to recover other than by rebooting the

machine.  The software reset doesn't seem to execute the

master/slave handshake, and the hardware reset is usually tied

directly to the system bus reset line.  Is this desirable or

acceptable?  Would it be possible to redefine the software reset, or

define a port address to individually hard reset each drive?

On to an unrelated topic.  I believe support for LBA mode

should be a requirement for all drives that support the ATA-2 spec.

This would help eliminate the perpetuation of the need for CHS

addressing.

Blair A. Merrell

Novell,Inc.   MS PF-11-1

122 East 1700 South

Provo, UT  84606

(801) 429-5781  FAX: (801) 429-2665

INTERNET: bmerrell@novell.com

From ata-owner  Thu Jun  9 10:33:58 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27778; Thu, 9 Jun 94 10:33:58 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA27771; Thu, 9 Jun 94 10:33:51 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id KAA14094; Thu, 9 Jun 1994 10:18:53 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 09 Jun 1994 10:10:51 EDT

Date:         Thu Jun 09 10:10:51 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2df722ec.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      re: ATA Issues

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I am responding to the questions from Blair Merrell of Novell...

>Are ATA devices fault tolerant, and if not is there any movement to

>push the interface in that direction?

There have been no specific discussions about "fault tolerance" at any

meeting I have been to.  My only comment is that if this adds cost to

an ATA device (read as ATA disk device), then it probably will not happen.

>I know that an 18 inch cable length almost requires

>that ATA devices be housed internal to a system and so also be

>connected to system power.  Is this the only configuration in which

>ATA devices are used, or are there others as the ATA interface

>spec allows?  If not is the power down test valid? (What about

>future needs such as hot-replaceable drives, or brown out

>conditions where one drive resets and the other doesn't.)

ATA is a device interface that is not system dependent (or it is suppose be

a device interface that is not system dependent!).  To date, most ATA devices

are used only in "IBM" PC type systems, but may appear in other types systems.

The traditional "IBM" PC configuration for ATA does not take into account that

one of the two drives can be powered down and the ATA specification also does

not allow this.  I don't want to say your test is invalid, but it does test a

condition that

will probably never happen in the typical (and traditional) "IBM" PC system.

>The software reset doesn't seem to execute the

>master/slave handshake, and the hardware reset is usually tied

>directly to the system bus reset line.  Is this desirable or

>acceptable?

>Would it be possible to redefine the software reset, or

>define a port address to individually hard reset each drive?

Changing something as basic as the master/slave handshaking at this time is

probably

not possible.   How the RESET signal is connected to the host is up to the

designer of

the ATA host adapter and is not part of the ATA specification.  Changing the

master/slave

protocol at this late date is probably not possible.

We are just now getting around to cleaning up Annex A and B in the ATA-2

document which describe this handshaking, but I have heard of no attempts to

redefine

how it works.  I have always wanted to get rid of the master/slave handshaking

(along with

the DASP and PDIAG signals) but I realized along time ago this would not be

possible.

However, there is a configuration that most people don't consider that is much

more

flexible with respect to this -- please see below.

>On to an unrelated topic.  I believe support for LBA mode

>should be a requirement for all drives that support the ATA-2 spec.

>This would help eliminate the perpetuation of the need for CHS

>addressing.

The SFF-8019 document, which is about to be incorporated into the ATA-2

document,

recommends that all drives over 528MB support LBA addressing.  Stronger wording

that would have made LBA mandatory was rejected many months ago.  From a

marketing

standpoint, I don't think it will be possible to sell drives over 528MB without

LBA support

(even though that LBA support may never be used) so I expect LBA to become a

"standard fixture" in all ATA drives soon.

------

A configuration that many people never consider is the following:  A single

drive per host

adapter with the drive configured as drive 1 (slave).

Note that this is a valid configuration -- there is nothing in the ATA that

really prohibits this

and we just got the ATA-2 document to be more clear on this.  There is an

unresolved

question as to whether we want to modify Annex A and B to explicitly include

this

configuration. Please note that a slave does not receive any signals from the

master and

therefore does not know if there is a master drive present.

This configuration has many advantages:

1) It eliminates all master/slave handshaking problems.  There is no

possibility that a

master drive will detect a slave that doesn't exist or fail to detect a slave

that does exist.

2) This would solve your issues with powering a drive on/off and could allow

"hot swapping"

-- assuming a good host adapter design.

3) The software modifications required to support this configuration are very

small and in

fact should probably be included in any well written ATA BIOS or device driver

today (why

shouldn't I be able to access the data on the drive 1 when the drive 0 is

dead?).

If I were designing a disk array using ATA drives or using an ATA drive in

some embedded (non-PC) application, I would use this configuration.

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\358.MSG

### Org Date:    06-09-94 09:58:52 AM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     re: ATA Issues

### Attachments: none

From ata-owner  Thu Jun  9 22:04:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA12221; Thu, 9 Jun 94 22:04:47 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA12214; Thu, 9 Jun 94 22:04:45 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA21417; Fri, 10 Jun 1994 01:04:44 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9406100102.tn1037475@aol.com>

To: ata@dt.wdc.com

Date: Fri, 10 Jun 94 01:02:07 EDT

Subject: Missing Documents

Sender: ata-owner@dt.wdc.com

Precedence: bulk

June 9, 1994

---------------------------------------------------------------------------

Folks - The following document numbers have been requested by the

the listed authors - the authors have failed to provide those

documents.  These documents are needed for the mailing immediately.

X3T10/94-048R0  Data Integrity Proposal

T. Hanan

X3T10/94-052R0  Proposed Change to Mode 4 T0 definition and diagrams

T. Hanan

X3T10/94-056R0  DRQ Timing on Last Data Transfer

T. Hanan

X3T10/94-065R2  Master/Slave Handshaking

H. Landis

X3T10/94-073R0  Minutes of CAM Working Group- 3/14/94

B. Dallas

X3T10/94-086R0  Minutes of SSP Study Group - 3/94

J. Scheible

X3T10/94-089R0  Handling High Capacity Drives

H. Landis

Regards,

Lawrence J. Lamers

Adaptec                    TEL: 408-945-8600 ext 3214

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@AOL.COM

From ata-owner  Mon Jun 20 16:23:11 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10665; Mon, 20 Jun 94 16:23:11 PDT

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA10625; Mon, 20 Jun 94 16:23:04 PDT

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA28737 for ata@dt.wdc.com; Mon, 20 Jun 94 16:22:57 -0700

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA01888 for ata@dt.wdc.com; Mon, 20 Jun 94 16:22:54 -0700

Received: from bach.nsc.com by berlioz.nsc.com (4.1/SMI-4.1)

id AA08512; Mon, 20 Jun 94 16:22:53 PDT

Date: Mon, 20 Jun 94 16:22:53 PDT

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9406202322.AA08512@berlioz.nsc.com>

To: ata@dt.wdc.com, sff_reflector@sun.com

Subject: ATA-X SSWG Minutes - June '94

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Doc No: X3T10/94-_____r0

Minutes of the ATA-Extensions Special Subject Working Group (SSWG) Meeting

Held June 1, 1994, in Milpitas, CA, at Adaptec.

Chaired by:          Steve Finch, Silicon Systems.

Hosted by:           Richard Kalish, Adaptec.

Minutes recorded by: Robbie Shergill, National Semi;

and Larry Lamers, Adaptec.

1. Steve Finch opened the meeting at 9:30AM by thanking the host, stating

the purpose of the meeting and some pertinent ANSI mechanisms. He asked

each attendee to introduce themselves and also sign the attendance list.

2. Attendance List

NAME                COMPANY

----                -------

Larry Lamers        Adaptec

Ricahrd Kalish      Adaptec

Joseph Chen         Cirrus Logic

Dan Colegrove       IBM

Ray Heineman        Maxtor

Tom Newman          Mission Peak Designs

Robbie Shergill     National Semiconductor

Robert Griffith     National Semiconductor

Charles Yang        Panasonic

Jim McGrath         Quantum

Hank Davenport      Samsung

Hale Landis         Seagate

John Masiewicz      Seagate Technology

Steve Finch         Silicon Systems

Paul Aloisi         Unitrode

Tom Hanan           Western Digital

3. Agenda for this meeting was circulated by Steve Finch. It was accepted

in the following form:

1. Opening Remarks

2. Attendance & Introductions

3. Approval of Agenda

4. Document Distribution

5. Next Meeting Schedule

6. Old Business

6.1 Project Proposals - time limit 1 hour.

6.2 16.6 MB/s status

6.3 ATA-2 Detailed Review

7. New Business

8. Adjournment

4. Documents distributed in this meeting:

X3T10/94-102r0  Minutes of the April 27, 1994, ATA-Ext. SSWG Meeting

5. Next Meetings:

June 29  Irvine, CA

in-depth review of ATA-2;

one hour on scope of ATA-3.

July 21  Manchester, NH

in-depth review of ATA-2; motion to accept as working draft.

ATA-3: start planning meetings etc.

Tom Hanan asked that everything that needs to be added to ATA-2 draft be

added before the 6/29 meeting so that the meeting is only editorial. Steve

will do his best - there is question about when hard copies will be

available.

6. Old Business

6.1 Project Proposals:

Tom offered to formulate responses to comments and put them on the

Reflector. Steve Finch and Larry Lamers didn't think that this was

necessary.

Larry took floor and asserted that the issue is too many documents. Larry

wants two documents - one for Phy + protocol and one for commands. He said

that discussion at Harrisburg airport had shown consensus on this. Larry

would like to side-step the SAM compliance and CDB issues because they are

not relevant to project proposal discussions. He felt, however, that ATA

doesn't have to be compliant to SAM - if the market wants to ship SCSI-3

over ATA then people will do it.

Tom Hanan thought that it was more important to give ATA-3 a good

foundation. Clarity issues can be dealt with education. ATA-3 should do a

good job of laying the foundation and do things differently from before if

that's what's needed.

Steve Finch did an informal poll - one person voted for 4 documents;

several for 2; couple for 1; but lots of people had no strong opinion.

Steve said that he is afraid of SCSI-3 experience where, today, some

documents are done while others are not. For this reason, he thinks two

documents is a good compromise. This is his personal opinion, not as chair.

Larry said that two is just a start, we can go to more documents if

necessary.

Tom said that WD has also asked around, including the Press. Feedback was

that the compatibility model should be maintained - ie, one coherent

portion that's Phy + Reg Set + ATA-2 Cmds. In this set, only the Phy level

is seen as having to change. PI extension is not confusing to people - a

need is seen for disk command set over PI, in fact.

John Masiewicz said that it is this type of mixup that has caused all the

negative comments.

Jim McGrath said that the only reason to do multi-layer standards is to be

able to do some portions at different times. Is it an asset or a liability?

ATA doesn't have SCSI's size problem so the asset is really not that great.

But the liability of confusing the market - specially when our customer

base is less sophisticated than SCSI - is quite substantial. So, reduce the

number of project proposals to even one.

At this point, Steve went around the room to obtain other comments:

Shergill - One or two documents to prevent confusion; but include better

explanation of the layering needed for wide applications and manage this

layering within the unified document.

Griffith - One document is much better to work with.

Hanan - OK to unify into one while working on it as long as internal

structure is the way he wants it; but then wants it broken up into 1 or 3

when shipped.

McGrath - One.

Masiewicz - Unify into one now and then we can break it into more later if

needed.

Newman - 1 or 2. The second one can be a section or separate that specifies

the differences between ATA-3 and the original ATA.

Heineman - prefers one.

Kalish - 1 or 2.

Chen - Expressed concern that number of documents can grow from the many

command sets thru PI. Tom clarified that the PI cmd-set extensions are

outside the scope of ATA. Joe voted for one.

Aloisi - Don't want to fragment the effort and momentum. Should be simple

to take care of the disk drive. Other things should be accomodated through

extensions to it.

Davenport - One document.

Colegrove - From the standpoint of a user he wants one document.

Hale - ATA-3 should be a logical extension of ATA-2. It should address only

the disk drive. ATAPI should remain a separate project.

Lamers - one or two.

Yang - one.

Conclusion - one document. Now need to resolve the scope issue. We will

devote one hour to it on the 29th.

6.2 16.6 MB/s status

16.6Mb/s timing mode was approved to be included in the last meeting.

Also, SFF approved the ID Drive document to be forwarded to ATA-2. Question

remains about how much of this should be informative, however.

6.3 ATA-2 Detailed Review (Rev 2C)

The group decided to review sections 1-6 today.

Clause 3.1.11 Reserved: Lengthy discussion. Hale said OK to check a

reserved field for zeroes. Tom didn't agree. Jim didn't want the drive

firmware to have to check reserved fields. Hale said if the recipient

doesn't check for zero, then how will you define that bit in the future?

Decided to defer it 'till ID Drive command is discussed in the next

meeting.

Clause 3.1.13 vendor unique - use VU throughout the document and remove

vendor specific.

Clause 3.1.7 LBA - remove the word "mode" and remove the max value.

Clause 3.1.3 CHS - remove the word "mode" and remove the max value.

Clause 3.2.5 byte ordering - Hale wanted it deleted; Jim wanted it kept but

better defined. Tom said that the real way to fix it is to rewrite ID Drive

in a interface-specific way. The actual description is incorrect also -

Steve will correct it when rewriting it for ID Drive section.

Clause 4.1: Configuration

Tom raised the issue that with a given CSEL cable, swapping dev0 and dev1

will cause problems. So, don't say "order of dev0 ...."

Larry brought up a bigger problem - first diagram in each figure is not

good for one device operation. Tom agreed - but for Mode 4 only; Problem is

that the first diagram is the way it is generally done. Hale pointed out

that another way to do it would be to stay with diagram 1 and use only a

slave drive - this also eliminates all master/slave issues. In that case,

Jim suggested that the language should be just not to leave a stub hanging

at the end of the cable. Along with this note, Steve will add a third

diagram.

Tom Newman also wanted the switch and jumper options to be combined.

-- Lunch --

Clause 4.2.1 connector pin one location should be addressed

change wording to compatible from equivalent

add 'devices should utilize connectors that work with

these cables & connectors'

use SFF-8012 illustrations

Clause 4.2.2 - delete

Clause 4.3 - delete in absence of objection from IBM

Connector Specification - the device connector is not specified in ATA.

add sections for 3.5, 2.5 and 1.8 inch drives with signal definitions

and pin assignments.

Clause 4.6 - Wording added to explain what this clause is.

Cin reduced to 20 pf for mode 4.

add host to 'of device'.

slew rate specification does not recognize the series resistors used on

most ATA drives.  Tom Hanan to post proposal on reflector.

Clause 5.2.4 new paragraph added to DASP requiring VOH and VOL to remain

compliant even if LED is attached.

Clause 5.2.10 INTRQ questions raised by Joe Chen.  He will post his concern

to reflector.

Clause 5.2.11 Add information on mode 3 & 4 using IOCS16.

Clause 5.2.13 PDIAG is a no connect at host.

Clause 5.2.15.1 - This section is vendor unique.

Write Caching - how to handle - raised by Ray Heineman.

7. New Business:

Rick Kalish - DEC issue with 30ns rise time.  Deal with on 6/29/94

8. The meeting was adjourned at 3:00 PM.

From ata-owner  Tue Jun 21 11:51:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA03376; Tue, 21 Jun 94 11:51:16 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA03369; Tue, 21 Jun 94 11:51:14 PDT

Received: with SMTP-MR; Tue, 21 Jun 1994 11:45:59 PST

Mr-Received: by mta WDUS04; Relayed; Tue, 21 Jun 1994 11:45:59 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Tue, 21 Jun 1994 19:00:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Cleaning Up The Mail Lists

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com

Message-Id: <01HDSWZ8ZG3M8Y9JYY@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Tue, 21 Jun 1994 19:46:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;95541112604991/423266@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

***************************************************************

In a effort to clean up the e-mail reflector list to which you belong, we

are asking each of you to reply to this e-mail with the following

information:

If your system can not reply to this e-mail then send the information to:

hanan_t@a1.wdc.com

Please use the following format: (One item per line, <CR>)

Format				Example

----------------------------	------------------------------

****************************	******************************

Name				Tom Hanan

Company (or "self")		Western Digital

Address1			8105 Irvine Drive

Address2			Irvine, CA.  92718-2902

Phone #				714 932-7472

Fax #				714 932-7312

Your E-mail Address		hanan_t@a1.wdc.com

Reflectors			ATA, ATAPI, SFF, SCSI, SSA

****************************	******************************

Using the above format will allow your silent servants to use automated

scripts to contact you when your e-mail bounces. It will also be used to

audit the existing lists for bit buckets.

Please, ONLY ONE PERSON PER E-MAIL REPLY!!!!

More than one person per e-mail will cause the entire e-mail to be

ignored!!! This is not a new way to subscribe to the reflectors!!!

Failure to reply to this request may result in your accidental removal from

this list if your e-mail occasionally bounces!!!!

P.S. You might not know that your e-mail bounces since we typically

try for up to 3 days.

Regards,

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Fri Jun 24 10:50:15 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29088; Fri, 24 Jun 94 10:50:15 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA29080; Fri, 24 Jun 94 10:50:08 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id KAA10296; Fri, 24 Jun 1994 10:37:05 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 24 Jun 1994 09:06:17 EDT

Date:         Fri Jun 24 09:06:17 1994

From: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

Message-Id: <2e0ada4a.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      "Editing Umbrella" gone wild - re. June 1 minutes

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I was really surprised by the content of changes made at the June 1 meeting of

ATA WG.  Although I was there for the first half of the meeting, I could not

stay for the second half which was supposed to be "Editorial Review".  It turns

out that several changes were made which are anything but editorial.  It was my

understanding that a "detailed review" of the entire document would be done one

piece at a time.  The goal was to eliminate confusion and clarify wording, and

add robustness to the specification.  In addition, there were some

previously-discussed and agree-to items which would be incorporated (e.g. 16.6

MB). I did not understand that we were going to make real-time technical

changes to ATA.

Some example from the minutes:

Changing Capacitance and redefining intent in section 4.6

Adding VOL and VOH requirement to DASP in 5.2.4

Deleting SPSYNC description in 5.2.15.1

Some of these changes are OK in my opinion, but some are not. The normal

procedure for technical changes is to schedule agenda items for discussion,

make postings to reflector, allow time for attendees to check with their

companies in advance, or report back to their companies and then vote in

follow-up meetings.

I don't want to slow things down,  but I will not accept calling technical

changes "Editorial" for the sake of convenience.

--

John Masiewicz -- John_Masiewicz@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\335.MSG

### Org Date:    06-24-94 08:59:16 AM

### From:        John Masiewicz@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     "Editing Umbrella" gone wild - re. June 1 minutes

### Attachments: none

From ata-owner  Fri Jun 24 12:55:25 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02061; Fri, 24 Jun 94 12:55:25 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA02054; Fri, 24 Jun 94 12:55:16 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id MAA06414; Fri, 24 Jun 1994 12:49:10 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 24 Jun 94 12:39:04 PDT

Date: 24 Jun 94 13:20:27 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: ATA-2 Rev2d Now Available

To: "scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"72020242604991/286357@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I just finished uploading the ATA-2 Rev 2d proposed draft standard to the

SCSI BBS.  I finally got it done, and I appologize for being late.

The file is 0948DR2D.DOC.  It is a Word for Windows 2.0 document.  It can

be found on the SCSI BBS  (719-574-0424) in the ATA directory.  The

document has changes noted by change bars, cross thru of deleted text, and

underlined new text.

I will bring hardcopies with me to the ATA Working Group meeting on June

28, at the Western Digital facility in Irvine.

See you there,

Steve Finch.

5723283@mcimail.com

PH:  714 573-6808

FX:  714 573-6916

From ata-owner  Fri Jun 24 15:04:29 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01349; Fri, 24 Jun 94 15:04:29 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA01342; Fri, 24 Jun 94 15:04:28 PDT

Received: with SMTP-MR; Fri, 24 Jun 1994 15:01:43 PST

Mr-Received: by mta WDUS04; Relayed; Fri, 24 Jun 1994 15:01:43 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Fri, 24 Jun 1994 22:53:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Please re-post ATA-2 w/o change bars

To: ATA@dt.wdc.com

Message-Id: <01HDXALRNLP08YA98G@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Fri, 24 Jun 1994 23:01:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;34105142604991/431878@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Steve,

please post ata-2 (2d) without underscore for new text to the BBS.

The underscored new text make the timing diagrams and tables unreadable.

Please leave the change bars and overstrike in to help us correlate the two

documents.

I would recommend that you bring copies of the non-underscored document to

the meeting.

Regards,

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Mon Jun 27 08:00:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA13756; Mon, 27 Jun 94 08:00:47 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA13749; Mon, 27 Jun 94 08:00:37 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id HAA16536; Mon, 27 Jun 1994 07:54:32 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 27 Jun 94 08:00:21 PDT

Date: 27 Jun 94 08:58:23 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: OOPS!  Wrong date.

To: "scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"32855172604991/287136@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Sorry to all for both the possible confusion and this extra message.

In my notice that the ATA-2 Rev 2d proposed working draft was available on

the SCSI BBS, I incorrectly stated the date of the next meeting as June 28.

This is incorrect, it should have been June 29 at the Western Digital

office in Irvine.

I have been asked to place the ATA-2 Rev2d document on an FTP site.  This

is fine with me, but I don't have access to FTP.  Sorry, could someone else

move it (like the owner of the FTP site)?

I have also been asked to make a version of the document available without

change bars and crossouts to indicate changes.  My answer is:  if you can

print it you probably have Word and if you have Word you can easily remove

the change bars yourself:  Edit, Select All; Tools, Revision Marks, Accept

revisions.

See you on WEDNESDAY, JUNE 29.

Steve Finch, Silicon Systems.

From ata-owner  Mon Jun 27 11:05:41 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18053; Mon, 27 Jun 94 11:05:41 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA18046; Mon, 27 Jun 94 11:05:40 PDT

Received: with SMTP-MR; Mon, 27 Jun 1994 11:00:39 PST

Mr-Received: by mta WDUS04; Relayed; Mon, 27 Jun 1994 11:00:39 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Mon, 27 Jun 1994 18:56:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: *** URGENT *** new dir to ata wg mtg ***

To: ATA@dt.wdc.com

Message-Id: <01HE194MDFCK8YAHS5@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Mon, 27 Jun 1994 18:58:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;93001172604991/434436@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

*************************************************************************

**************       U R G E N T   N O T I C E         ******************

*************************************************************************

A street fair has closed one of the access streets to WD on the 29th.

Please use the following alternate route!!!!!!!

Contact:        Tom Hann, 714 932-7472, hanan_t@a1.wdc.com

*************************************************************************

Orange County is the closest airport.

Direction:

Follow the signs from Orange County airport leading to

the 405 FWY (SOUTH). Take the 405 FWY south 7 miles to

Irvine Center Drive. Exit the FWY and take a left onto Irvine

Center Drive, which should take you back over the 405 FWY.

N E W    D I R E C T I O N S    F R O M    F W Y

------------------------------------------------------------

(Take your left into the right hand most lane, since you must)

(take an immediate right after crossing over the 405 FWY.    )

(Once you have taken a right, you must merge to the left turn)

(lane so that you can take a left on FORTUNE.                )

(                                                            )

(Once you have taken a left on fortune, you will see the     )

(Western Digital building on the left. Take another left into)

(the first driveway. As you enter the parking lot the        )

(visitors parking will be on your right against the street.  )

------------------------------------------------------------

Good Luck!!!

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Mon Jun 27 11:23:43 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18503; Mon, 27 Jun 94 11:23:43 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA18496; Mon, 27 Jun 94 11:23:42 PDT

Received: with SMTP-MR; Mon, 27 Jun 1994 11:18:48 PST

Mr-Received: by mta WDUS04; Relayed; Mon, 27 Jun 1994 11:18:48 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Mon, 27 Jun 1994 19:02:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: RE: ata-2 FTP site & files

To: ATA@dt.wdc.com, SCSI@WICHITAKS.NCR.COM

Message-Id: <01HE19R0IVPW8YA5B8@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Mon, 27 Jun 1994 19:17:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;84811172604991/434485@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

*************************************************************************

The current ATA FTP site is listed as:

dt.wdc.com

/pub/ata

The same as the ata reflector;

ata@dt.wdc.com

************************************************************************

Let's try and keep all of the ata information on the same server. It helps

prevent confusion.....

I will move the ata-2 (2d) file over to the ftp site today. If I get time

I will translate it into a postscript file also.

**************************************************************************

P.S. ATA e-mail should be sent to the ata reflector (ata.dt.wdc.com).

Anyone can send mail even if they have not yet signed up....

To sign up send e-mail to majordomo@dt.wdc.com with the following in the

body of the e-mail:

subscribe ata

It's that simple.....

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Mon Jun 27 12:49:27 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA20309; Mon, 27 Jun 94 12:49:27 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA20302; Mon, 27 Jun 94 12:49:21 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id ad18760; 27 Jun 94 12:38 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 27 Jun 94 10:29:49 MDT

Subject: ATA-2 Rev 2d is on ncrinfo.ncr.com

To: ATA Reflector <ata@dt.wdc.com>

Date: Mon, 27 Jun 94 10:28:06 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9406271238.ad18760@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Brian Berg asked that ATA-2 Rev 2d be placed on an ftp site in addition

to being placed on the SCSI BBS.  I will upload 0948dr2d.uue (uuencoded

version of the Word for Windows file) to ncrinfo.ncr.com in the

/pub/standards/ata2 directory this morning.  Once you 'get' this file,

you'll need to use uudecode on the file to restore the .doc file.

John

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Mon Jun 27 14:03:33 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22007; Mon, 27 Jun 94 14:03:33 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA22000; Mon, 27 Jun 94 14:03:31 PDT

Received: with SMTP-MR; Mon, 27 Jun 1994 14:00:37 PST

Mr-Received: by mta WDUS04; Relayed; Mon, 27 Jun 1994 14:00:37 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Mon, 27 Jun 1994 21:59:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: OOPS! ATA FTP site.

To: ATA@dt.wdc.com, SCSI@WICHITAKS.NCR.COM

Message-Id: <01HE1FC1K6W48Y9UKF@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Mon, 27 Jun 1994 22:01:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;73004172604991/434797@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

*************************************************************************

The current ATA FTP site is listed as:

fission.dt.wdc.com

/pub/ata

The same as the ata reflector;

ata@dt.wdc.com

************************************************************************

Let's try and keep all of the ata information on the same server. It helps

prevent confusion.....

I will move the ata-2 (2d) file over to the ftp site today. If I get time

I will translate it into a postscript file also.

**************************************************************************

P.S. ATA e-mail should be sent to the ata reflector (ata@dt.wdc.com).

Anyone can send mail even if they have not yet signed up....

To sign up send e-mail to majordomo@dt.wdc.com with the following in the

body of the e-mail:

subscribe ata

It's that simple.....

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Tue Jun 28 08:22:45 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA13633; Tue, 28 Jun 94 08:22:45 PDT

Received: from dell2.us.dell.com by dt.wdc.com (4.1/SMI-4.1)

id AA13609; Tue, 28 Jun 94 08:21:19 PDT

Received: from ccmail.us.dell.com (ccmail.us.dell.com [143.166.224.253]) by dell2.us.dell.com (8.6.8.1/8.6.8-DELL2-INTERNET-GATEWAY) with SMTP id FAA02776; Tue, 28 Jun 1994 05:17:49 -0500

Received: from cc:Mail by ccmail.us.dell.com

id AA772823958; Tue, 28 Jun 94 10:19:18 CST

Date: Tue, 28 Jun 94 10:19:18 CST

From: "Bill Willette" <Bill_Willette@ccmail.us.dell.com>

Encoding: 70 Text

Message-Id: <9405287728.AA772823958@ccmail.us.dell.com>

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com,

"Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Re: Cleaning Up The Mail Lists

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Bill Willette

DELL Computer Corporation

9505 Arboretum Drive

Austin, TX  78759-7299

(512) 728-9068

FAX (512) 728-3653

Bill_Willette@dell1.us.dell.com

ATAPI, ATA

***************************************************************

In a effort to clean up the e-mail reflector list to which you belong, we

are asking each of you to reply to this e-mail with the following

information:

If your system can not reply to this e-mail then send the information to:

hanan_t@a1.wdc.com

Please use the following format: (One item per line, <CR>)

Format				Example

----------------------------	------------------------------

****************************	******************************

Name				Tom Hanan

Company (or "self")		Western Digital

Address1			8105 Irvine Drive

Address2			Irvine, CA.  92718-2902

Phone #				714 932-7472

Fax #				714 932-7312

Your E-mail Address		hanan_t@a1.wdc.com

Reflectors			ATA, ATAPI, SFF, SCSI, SSA

****************************	******************************

Using the above format will allow your silent servants to use automated

scripts to contact you when your e-mail bounces. It will also be used to

audit the existing lists for bit buckets.

Please, ONLY ONE PERSON PER E-MAIL REPLY!!!!

More than one person per e-mail will cause the entire e-mail to be

ignored!!! This is not a new way to subscribe to the reflectors!!!

Failure to reply to this request may result in your accidental removal from

this list if your e-mail occasionally bounces!!!!

P.S. You might not know that your e-mail bounces since we typically

try for up to 3 days.

Regards,

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Tue Jun 28 12:50:20 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA19603; Tue, 28 Jun 94 12:50:20 PDT

Received: from gatekeeper.mcimail.com by dt.wdc.com (4.1/SMI-4.1)

id AA19596; Tue, 28 Jun 94 12:50:14 PDT

Received: by gatekeeper.mcimail.com (5.65/fma-120691);

id AA23354; Tue, 28 Jun 94 14:51:55 -0500

Received: from mcimail.com by MCIGATEWAY.MCIMail.com id am25639;

28 Jun 94 19:47 GMT

Date: Tue, 28 Jun 94 14:29 EST

From: jrubin <jrubin_+a_MOLEX-LI_+ljrubin+r%Molex-Lisle-ccMail@mcimail.com>

To: ATA <ATA@dt.wdc.com>

To: Bill Willette <Bill_Willette@ccmail.us.dell.com>

To: Dal Allan <0002501752@mcimail.com>

To: Steve Finch <0005723283@mcimail.com>

To: don clay <don_clay+adevelopment+acolorado%CONNER@mcimail.com>

To: LANE LEE <LANE_LEE+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: mark gurkowski <mark_gurkowski+adevelopment+acolorado%CONNER@mcimail.com>

To: sin <sin+_h%A1%WDSIN%Western_Digital_Corporation@mcimail.com>

To: STEVE ANDERSON <STEVE_ANDERSON+aDEVELOPMENT+aCOLORADO%CONNER@mcimail.com>

To: ATAPI <ATAPI@dt.wdc.com>

To: Tom Hanan <HANAN_T@a1.wdc.com>

Subject: Re[2]: Cleaning Up The Mail Lists

Message-Id: <34940628192943/0005337327PK1EM@mcimail.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Per your request for the following information:

Name: Jeff Rubin

Company: Molex Inc.

Address: 2222 Wellington Ct.

Address: Lisle, Il. 60532

Phone:   708-527-4367

Fax#:    708-969-1352

E-Mail Address: JRUBIN at Molex-Lis-Intl

Reflector: ATA,ATAPI,SFF,SCSI

From ata-owner  Wed Jun 29 07:59:41 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA12287; Wed, 29 Jun 94 07:59:41 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA12280; Wed, 29 Jun 94 07:59:38 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id HAA26024 for <ata@dt.wdc.com@relay>; Wed, 29 Jun 1994 07:53:32 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 29 Jun 94 08:00:12 PDT

Date: 29 Jun 94 08:41:50 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: Re: Cleaning Up The Mail Lists

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"05145192604991/288441@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Name: Steve Finch

Company:  Silicon Systems, Inc.

Address:  14351 Myford Rd.

Tustin, CA 92680-7022

Phone #:  714 573-6808

Fax #:    714 573-6916

E-Mail:   5723283@mcimail.com  (Receive)PLEASE USE THIS ADDRESS TO REACH ME

steve.finch@tus.ssi1.com (Send)

Reflectors:  ATA,ATAPI,SFF,SSA,SCSI

From ata-owner  Wed Jun 29 17:31:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA24706; Wed, 29 Jun 94 17:31:47 PDT

Received: from dell2.us.dell.com by dt.wdc.com (4.1/SMI-4.1)

id AA24569; Wed, 29 Jun 94 17:30:53 PDT

Received: from ccmail.us.dell.com (ccmail.us.dell.com [143.166.224.253]) by dell2.us.dell.com (8.6.8.1/8.6.8-DELL2-INTERNET-GATEWAY) with SMTP id OAA21665; Wed, 29 Jun 1994 14:27:17 -0500

Received: from cc:Mail by ccmail.us.dell.com

id AA772942998; Wed, 29 Jun 94 19:23:18 CST

Date: Wed, 29 Jun 94 19:23:18 CST

From: "Greg Weir" <Greg_Weir@ccmail.us.dell.com>

Encoding: 72 Text

Message-Id: <9405297729.AA772942998@ccmail.us.dell.com>

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com, HANAN_T@A1.WDC.COM

Subject: Re[2]: Cleaning Up The Mail Lists

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Greg Weir

DELL Computer Corporation

9505 Arboretum Drive

Austin, TX  78759-7299

(512) 728-9230

FAX (512) 728-3653

Greg_Weir@dell1.us.dell.com

ATAPI, ATA

***************************************************************

In a effort to clean up the e-mail reflector list to which you belong, we are

asking each of you to reply to this e-mail with the following information:

If your system can not reply to this e-mail then send the information to:

hanan_t@a1.wdc.com

Please use the following format: (One item per line, <CR>)

Format				Example

----------------------------	------------------------------

****************************	****************************** Name				Tom Hanan

Company (or "self")		Western Digital Address1			8105 Irvine

Drive

Address2			Irvine, CA.  92718-2902

Phone #				714 932-7472

Fax #				714 932-7312

Your E-mail Address		hanan_t@a1.wdc.com Reflectors			ATA, ATAPI, SFF, SCSI, SSA

****************************	******************

************

Using the above format will allow your silent servants to use automated scripts

to contact you when your e-mail bounces. It will also be used to audit the

existing lists for bit buckets.

Please, ONLY ONE PERSON PER E-MAIL REPLY!!!!

More than one person per e-mail will cause the entire e-mail to be ignored!!!

This is not a new way to subscribe to the reflectors!!!

Failure to reply to this request may result in your accidental

removal from this list if your e-mail occasionally bounces!!!!

P.S. You might not know that your e-mail bounces since we typically

try for up to 3 days.

Regards,

----------------------------------------------------------------------

-----

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Thu Jun 30 07:54:36 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10609; Thu, 30 Jun 94 07:54:36 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA10600; Thu, 30 Jun 94 07:54:28 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id HAA01127; Thu, 30 Jun 1994 07:48:22 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 30 Jun 94 07:48:46 PDT

Date: 30 Jun 94 08:35:19 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: Proposed ATA-2 Working Group Agenda

To: "scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"91535103604991/289075@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The next ATA Working Group Meeting will be during the X3T10 Plenary Week.

Specifically on Thursday, July 21, 1994 from 9:00 AM to 12:00 Noon.

The announcements for the last couple of meetings of the ATA Working Group

Meetings had indicated that the meetings were for Editing of the ATA-2

standard.  During these meetings, several technical issues were discussed,

but resolutions were not accepted as the advertised purpose of the meetings

were "editorial" and not technical.  To address these issues and new

technical issues as they occur, this ATA Working Group meeting will be a

technical meeting and will address changes to the document that are of a

technical nature.

The following is a proposed agenda for this meeting:

Proposed Agenda

X3T10 ATA Working Group

9:00 A.M. - 12:00 P.M., July 21, 1994

Manchester, NH

1.  Opening Remarks

2.  Attendance and Membership, Introductions

3.  Approval of Agenda

4.  Document Distribution

5.  Old Business

5.1.	ATA-3 Project Proposal Letter Ballets (1 hr)

5.2.	ATA-2 Technical Issues discovered during editing sessions

5.3.	ATA-2 Technical Review

6.  New Business

6.1.	Termination Issues (Hanan)

7.   Meeting Schedule

Proposed Meeting Schedule:

August 27 (and 28?), 1994, San Jose, IBM

September 15, 1994, Houston, TX, Compaq

September 28, 1994, Oklahoma City, OK, Seagate

October 19, 1994, Longmont, CO, Conner

November 10, 1994, Palm Springs, CA, Western Digital

November 30, 1994, Irvine, CA, Western Digital

8.  Adjournment

See you at the meeting

Steve Finch, Chair ATA Working Group

Silicon Systems, Inc.

PH:  714 573-6808

FX:  714 573-6916

Email:  5723283@mcimail.com

From ata-owner  Fri Jul  1 07:40:33 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08848; Fri, 1 Jul 94 07:40:33 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08841; Fri, 1 Jul 94 07:40:21 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id HAA04549; Fri, 1 Jul 1994 07:35:06 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 01 Jul 1994 07:37:01 EDT

Date:         Fri Jul 01 07:37:00 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2e13ffdd.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Removable Media Commands

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Just a followup on the discussion about the Removable Media commands that we

had at the ATA WG meeting in Irvine on 29 June 94...

I found a copy of ATA Rev 3.0 dated 22 Nov 1991and it contains the same

descriptions for these commands as found in ATA 4.0 and in ATA-2.  There was

some debate as to when these commands and descriptions appeared in the ATA

specification.

To repeat what I said in Irvine:  I think all of the Removable Media commands

and their associated errors bits should be reclassified as Vendor Unique in

ATA-2.

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\479.MSG

### Org Date:    07-01-94 07:30:31 AM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Removable Media Commands

### Attachments: none

From ata-owner  Fri Jul  1 08:13:46 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA09496; Fri, 1 Jul 94 08:13:46 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA09491; Fri, 1 Jul 94 08:13:20 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id IAA06389; Fri, 1 Jul 1994 08:10:55 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 01 Jul 1994 07:55:32 EDT

Date:         Fri Jul 01 07:55:32 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2e140435.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Removable Media Commands #2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Message #2 -- I just found an ATA Rev 2.6 dated 19 Aug 91 with the same command

descriptions.  It would appear that this "junk" as been in the ATA

specification for a long time.

Just a followup on the discussion about the Removable Media commands that we

had at the ATA WG meeting in Irvine on 29 June 94...

I found a copy of ATA Rev 3.0 dated 22 Nov 1991and it contains the same

descriptions for these commands as found in ATA 4.0 and in ATA-2.  There was

some debate as to when these commands and descriptions appeared in the ATA

specification.

To repeat what I said in Irvine:  I think all of the Removable Media commands

and their associated errors bits should be reclassified as Vendor Unique in

ATA-2.

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\481.MSG

### Org Date:    07-01-94 07:34:14 AM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Removable Media Commands #2

### Attachments: none

From ata-owner  Fri Jul  1 08:16:05 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA09638; Fri, 1 Jul 94 08:16:05 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA09622; Fri, 1 Jul 94 08:15:47 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id IAA06421; Fri, 1 Jul 1994 08:11:57 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 01 Jul 1994 07:55:40 EDT

Date:         Fri Jul 01 07:55:40 1994

From: "Hale Landis" <Hale_Landis@notes.seagate.com>

Message-Id: <2e14043d.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Hale Landis" <Hale_Landis@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Removable Media Commands #3

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Message #3 (and final message in this thead!) -- These commands are not in the

CAM ATA Rev 2.1 dated 11 June 90.

Message #2 -- I just found an ATA Rev 2.6 dated 19 Aug 91 with the same command

descriptions.  It would appear that this "junk" as been in the ATA

specification for a long time.

Just a followup on the discussion about the Removable Media commands that we

had at the ATA WG meeting in Irvine on 29 June 94...

I found a copy of ATA Rev 3.0 dated 22 Nov 1991and it contains the same

descriptions for these commands as found in ATA 4.0 and in ATA-2.  There was

some debate as to when these commands and descriptions appeared in the ATA

specification.

To repeat what I said in Irvine:  I think all of the Removable Media commands

and their associated errors bits should be reclassified as Vendor Unique in

ATA-2.

--

Hale Landis -- Hale_Landis@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\487.MSG

### Org Date:    07-01-94 07:47:13 AM

### From:        Hale Landis@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Removable Media Commands #3

### Attachments: none

From ata-owner  Fri Jul  1 17:56:28 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA21572; Fri, 1 Jul 94 17:56:28 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA21565; Fri, 1 Jul 94 17:56:23 PDT

Received: from worf.qntm.com (worf.qntm.com [146.174.250.100]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id RAA08026; Fri, 1 Jul 1994 17:50:11 -0700

Received: from mail.qntm.com by worf.qntm.com with SMTP

(1.37.109.8/16.2) id AA01036; Fri, 1 Jul 1994 17:55:59 -0700

Received: from qm_smtpgw by mail with SMTP

(1.37.109.8/16.2) id AA10758; Fri, 1 Jul 1994 17:54:35 -0700

Message-Id: <n1439037011.59716@qm_smtpgw.qntm.com>

Date: 1 Jul 1994 17:58:54 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: Proposed ATA-2 Working G

To: "ata@dt.wdc.com" <ata%dt.wdc.com@hp427u.tus.ssi1.com>,

"scsi@wichitaks.ncr.com" <scsi%wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0FC2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>Proposed ATA-2 Working Group Agenda

I was not at the last meeting in Irvine, but wonder about the outlined

schedule for ATA-2 meetings.

First, they seem to be every 2 to 3 weeks - why so often?

Second, we seem to be spreading them throughput the country.  Was

this at the request of people  or what?  I think we have been

very productive with meetings in San Jose, and it has allowed

companies to send engineers to the meetings who otherwise would

not be able to travel.

Jim

From ata-owner  Mon Jul  4 07:25:21 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28023; Mon, 4 Jul 94 07:25:21 PDT

Received: from olivea.ATC.Olivetti.Com by dt.wdc.com (4.1/SMI-4.1)

id AA28016; Mon, 4 Jul 94 07:25:18 PDT

Received: by olivea.ATC.Olivetti.Com (5.65/1.34)

id AA10607; Mon, 4 Jul 94 07:25:17 -0700

Received: from iconet.Ico.Olivetti.Com by olivea.ATC.Olivetti.Com (4.1/SMI-4.1)

id AA10603; Mon, 4 Jul 94 07:25:11 PDT

Received: from wgpo03.ico.olivetti.com by iconet.Ico.Olivetti.Com;

Received: by gwpcrd.ico.olivetti.com with Microsoft Mail

id <2E1834E8@gwpcrd.ico.olivetti.com>; Mon, 04 Jul 94 16:12:24 CET

From: BIANCHINI Giovanni <bianchini@gwpcrd.ico.olivetti.com>

To: ATA <ATA@dt.wdc.com>, atapi-owner <atapi-owner@dt.wdc.com>,

ATAPI <ATAPI@dt.wdc.com>

Subject: RE: Cleaning Up The Mail Lists

Date: Mon, 04 Jul 94 15:10:00 CET

Message-Id: <2E1834E8@gwpcrd.ico.olivetti.com>

Encoding: 10 TEXT

X-Mailer: Microsoft Mail V3.0

Content-Type: text

Content-Length: 167

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Giovanni Bianchini

Olivetti

Via Montalenghe, 8

10010 Scarmagno, TO - Italy

+39-125-527611

+39-125-527278

bianchin@iconet.ico.olivetti.com

ATA, ATAPI, SFF, SCSI, SSA

From ata-owner  Tue Jul  5 10:45:37 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29016; Tue, 5 Jul 94 10:45:37 PDT

Received: from dub-img-2.compuserve.com by dt.wdc.com (4.1/SMI-4.1)

id AA29009; Tue, 5 Jul 94 10:45:32 PDT

Received: from localhost by dub-img-2.compuserve.com (8.6.4/5.940406sam)

id NAA24041; Tue, 5 Jul 1994 13:45:30 -0400

Date: 05 Jul 94 12:49:28 EDT

From: <JEFFE@fdc.mhs.compuserve.com>

To: ATA Reflector <ata@dt.wdc.com>, SCSI Reflector <scsi@wichitaks.ncr.com>

Subject: ATA-2 Working Group Meetings

Message-Id: <940705164928_555063.0_DHL2-1@CompuServe.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

- - Mail - -

July  5, 1994         9:43am   MAIL IS -

FROM: Jeff Epstein                    Viewed Ntfy Urgent

TO: ATA Reflector

SCSI Reflector

SUBJECT: ATA-2 Working Group Meetings

COPY: Mark Raymond

Erv Hauck

Jack Allweiss

Mark Woithe

Jeff Epstein

*************************************************************************

To: ATA REFLECTOR

From: JEFF EPSTEIN - FUTURE DOMAIN CORP. IRVINE, CA

Jim McGrath from Quantum complained about the ATA working group

meetings not all being held in San Jose so more Engineers could attend.

JIM: That's just the point.  Believe it or not, not all Engineering takes

place in Silicon Valley!  Spreading out the meeting place locations allows

even more Engineers to attend who would not otherwise be able to attend.

Thanks, Jeff Epstein

Future Domain Corp.

internet: jeffe@fdc.mhs.compuserve.com

***********************************************************************

From ata-owner  Wed Jul  6 12:19:29 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28540; Wed, 6 Jul 94 12:19:29 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA28533; Wed, 6 Jul 94 12:19:17 PDT

Received: from mail.qntm.com by worf.qntm.com with SMTP

(1.37.109.8/16.2) id AA11895; Wed, 6 Jul 1994 12:18:02 -0700

Received: from qm_smtpgw by mail with SMTP

(1.37.109.8/16.2) id AA13098; Wed, 6 Jul 1994 12:16:29 -0700

Message-Id: <n1438625290.23122@qm_smtpgw.qntm.com>

Date: 6 Jul 1994 12:22:16 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: ATA-2 Working Group Meet

To: "ATA Reflector" <ata@dt.wdc.com>, JEFFE@fdc.mhs.compuserve.com,

"SCSI Reflector" <scsi@WichitaKS.NCR.COM>

X-Mailer: Mail*Link SMTP/QM 3.0.0FC2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>ATA-2 Working Group Meetings

The problem Jeff, is that spreading out standards meetings

to different locations "to get more engineers involved" does

not work.  What happens is that some people from that location,

who otherwise would not come to the meeting, come to that one

meeting.  Since they are not familiar with the previous work done,

and will not be there for subsequent meetings, they usually cannot

contribute very well.  The result is a SMALLER core of commited

people attending all the meetings and doing all the work (and

making all the decisions) - not, in my experience, an improvement.

ATA-2 at this point does not require any "brainstorming" - where

getting a low more inpout from other people who have not been

involved is valuable.  Rather, it requires some detailed, painstaking,

line by line review and discussion - something the new method will

probably not promote.  For the first stages of ATA-3 that may be

appropriate, but our first priority is to close out ATA-2 as quickly

as possible.

Jim

From ata-owner  Wed Jul  6 13:15:04 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29572; Wed, 6 Jul 94 13:15:04 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA29565; Wed, 6 Jul 94 13:14:59 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qLdMn-0002LmC; Wed, 6 Jul 94 16:14 EDT

Received: by ptlnor3.ptltd.com; Wed, 6 Jul 94 16:20:33 EDT

Date: Wed, 6 Jul 94 19:14:33 PDT

Message-Id: <E706+d0l4ia@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: ATA@dt.wdc.com, SCSI@WichitaKS.NCR.COM

Subject: Meetings

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Jim

It seems that the reflectors are once again being used for

political purposes.  I guess now would be a good time to air my gripe

concerning the location of ATA meetings.  I have not been participating in

the ATA standards meetings because they are almost never held locally.  My

feeling is that by always holding the meetings in the same location you

generate a pseudo standard which represents LOCAL inductry ideas.  Many

drive manufacturers are located in northern california, but to generate

reasonable standard users must also be present.  This applies to the nitty

details and exact wordings of the standard.

Curtis E. Stevens [Curtis_Stevens@BanNet.PTLTD.COM]

From ata-owner  Wed Jul  6 14:00:59 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00648; Wed, 6 Jul 94 14:00:59 PDT

Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA00641; Wed, 6 Jul 94 14:00:53 PDT

Received: with SMTP-MR; Wed, 6 Jul 1994 13:57:36 PST

Mr-Received: by mta WDUS04; Relayed; Wed, 06 Jul 1994 13:57:36 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Wed, 6 Jul 1994 21:45:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: ATA Meeting Locations

To: ATA@dt.wdc.com

Message-Id: <01HEDZXM3EPQ8WYKHA@wdus03.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Wed, 6 Jul 1994 21:59:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;63753160704991/450390@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To minimize reflector e-mail regarding Working Group locations I am

proposing that those of you with comments send them to me and I will

take them to the plenary meeting in July.

DO NOT REPLY TO THIS E-MAIL.

SEND YOUR COMMENTS TO:  hanan_t@a1.wdc.com

Regards,

Tom Hanan

ATA Reflector Manager

P.S. I may just print out all of the comments and bring them to the meeting

so don't put anything in your e-mail you don't want the world to see.

If you will be at the next meeting then you DO NOT need to send me

your comments.

From ata-owner  Wed Jul  6 20:29:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08289; Wed, 6 Jul 94 20:29:49 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA08282; Wed, 6 Jul 94 20:29:41 PDT

Received: from mail.qntm.com by worf.qntm.com with SMTP

(1.37.109.8/16.2) id AA13788; Wed, 6 Jul 1994 20:29:06 -0700

Received: from qm_smtpgw by mail with SMTP

(1.37.109.8/16.2) id AA14256; Wed, 6 Jul 1994 20:27:32 -0700

Message-Id: <n1438595827.95338@qm_smtpgw.qntm.com>

Date: 6 Jul 1994 20:32:57 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: Meetings

To: ATA@dt.wdc.com,

"curtis_stevens@bannet.ptltd.co" <curtis_stevens@bannet.ptltd.com>,

SCSI@WichitaKS.NCR.COM

X-Mailer: Mail*Link SMTP/QM 3.0.0FC2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>Meetings

Curtis,

I don't see the topic of meeting locations as being "political" - meetings

have to be approved this month in New Hamshire, and like any topic

requiring approval should be discussed on email first (to reduce

discussion in New Hamshire).

My problem is that I do not see how holding one meeting in, say, Houston

will get engagement from people at that site.  People may show

for that meeting, but if they are not showing now for San Jose

then they are very unlikely to follow the meeting to the next location.

For instance, anything you idscussed last week in Irvine will almost

certainly be discussed (and changed) at a subsequent non-Irvine

meeting.  In order to make sure things are as you want them, you will

have to travel to all the meetings.  I would suggest in that case that

day trips to San Jose would be easier for you.

If people only show for the meeting when it is in their location, then

far fewer people will attend every meeting consistently.  A high and

consistent attendence level is required in order to close out the last

stages of a standard (e.g. ATA-2).  As I mentioned, ATA-3 may be

different, but potentially revisiting every item at every meeting

because the people change is NOT a good way to get out of ATA-2

and into ATA-3.

Note that this is an argument for one site being the central site

for closing the standard work.  San Jose is a good choice for this

site because it IS the largest concentration of drive AND system

manufacturers in the country.   If you have to pick one site, it

is the best one.  The only real drawback is for people out east

(e.g. IBM folks), since the travel is real bad.  The only fallback

I can see there is to require everyone to travel to a central location

(e.g. Dallas or Chicago).  Note that the Fibre Channel people are

doing that (they use Denver).

Jim

From ata-owner  Thu Jul  7 10:03:07 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23704; Thu, 7 Jul 94 10:03:07 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA23697; Thu, 7 Jul 94 10:03:04 PDT

Received: from mail.qntm.com by worf.qntm.com with SMTP

(1.37.109.8/16.2) id AA20456; Thu, 7 Jul 1994 10:03:02 -0700

Received: from qm_smtpgw by mail with SMTP

(1.37.109.8/16.2) id AA04830; Thu, 7 Jul 1994 10:01:26 -0700

Message-Id: <n1438546991.32330@qm_smtpgw.qntm.com>

Date: 7 Jul 1994 10:06:52 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: ATA Meeting Frequency

To: "ATA" <ATA@dt.wdc.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0FC2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  9:54 AM

OFFICE MEMO         ATA Meeting Frequency                 Date:  7/7/94

I have received a lot of feedback on the topic of the proposed ATA

meeting locations, but nothing on the frequency.  The proposal to meet

once every 3 weeks seems to be very heavy and unneeded for ATA-2

activities (and probably ATA-3 as well for the time being).  I would

suggest less frequent, but perhaps longer, meetings.  For instance, in a

2 day meeting I think we can get more done than in two 1 day meetings.

Wil local printers and copiers, we can go through multiple drafts in

almost real time.

Note that a fewer number of longer meetings will also enhance the

consistency of membership during the review process, something

I think is key to getting ATA-2 to bed as quickly as possible.

Jim

From ata-owner  Thu Jul  7 15:10:20 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00404; Thu, 7 Jul 94 15:10:20 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA00385; Thu, 7 Jul 94 15:10:13 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id an29523; 7 Jul 94 18:09 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 7 Jul 94 16:03:18 MDT

Received: from cc:Mail by ccmgate1bb.FTCollinsCO.NCR.COM

id AA773622161 Thu, 07 Jul 94 16:02:41 MDT

Date: Thu, 07 Jul 94 16:02:41 MDT

From: dsnyder@ccmgate1bb.ftcollinsco.NCR.COM

Message-Id: <9406077736.AA773622161@ccmgate1bb.FTCollinsCO.NCR.COM>

To: ATA@dt.wdc.com

Subject: Re: ATA Meeting Frequency

Sender: ata-owner@dt.wdc.com

Precedence: bulk

unsubsribe ata dennis.snyder@ftcollinsco.ncr.com

From ata-owner  Fri Jul  8 11:17:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23354; Fri, 8 Jul 94 11:17:26 PDT

Received: from risc2.sam.com by dt.wdc.com (4.1/SMI-4.1)

id AA23347; Fri, 8 Jul 94 11:17:22 PDT

Received: from cheetah.sam.com by risc2.sam.com (4.1/SMI-4.1)

id AA27197; Fri, 8 Jul 94 11:26:35 PDT

Received: by cheetah.sam.com (4.1/SMI-4.1)

id AA01286; Fri, 8 Jul 94 11:19:58 PDT

Date: Fri, 8 Jul 94 11:19:58 PDT

From: jeff@sam.com (Jeff Liu)

Message-Id: <9407081819.AA01286@cheetah.sam.com>

Apparently-To: ATA@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

unsubsribe ata jeff@sam.com

From ata-owner  Fri Jul  8 15:14:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28419; Fri, 8 Jul 94 15:14:26 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA28412; Fri, 8 Jul 94 15:14:18 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id PAA17376; Fri, 8 Jul 1994 15:07:58 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 7 Jul 94 07:26:17 PDT

Date: 7 Jul 94 08:09:37 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: ATA Meetings and Tom Hanan's suggestion

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>,

"scsi@wichitaks.ncr.com" <scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com>

Message-Id: <"73905170704991/291769@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I prefer to receive information, complaints, etc. from people about meeting

locations and would prefer all to see these comments.  Please do not send

to one person and allow possible filtering of information.

I believe it is ANSI policy to distribute meetings geographically.  In the

past, we have had mostly volunteers to host from the Bay Area in

California, but recently we have had volunteers from other locals.

Recently, we have also had a lack of volunteers for the Bay Area.  The

result is that the tentative schedule has been described.

Remember that the final schedule is what is authorized by X3T10, not the

working group although the working group has a very significant voice in

the matter.

My questions are:

Are there Bay Area volunteers?

Who, besides Jim McGrath, object to geographic distribution of meetings?

Who would object to Bay Area only locations?

Please, voice over the reflector as this is an ATA subject.

-------

My personal opinion goes for geographic distribution.  It helps me as I

visit customers through out the country and I always schedule visits to

customers when traveling to these meetings, killing two stones with one

bird (so to speak).

----

Steve FINCH

SSi

------

714 573-6808

Fx: 714 573-6916

email:  5723283@mcimail.com

From ata-owner  Fri Jul  8 15:53:46 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29311; Fri, 8 Jul 94 15:53:46 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA29300; Fri, 8 Jul 94 15:53:24 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qMOnB-0002LEC; Fri, 8 Jul 94 18:53 EDT

Received: by ptlnor3.ptltd.com; Fri, 8 Jul 94 18:53:14 EDT

Date: Fri, 8 Jul 94 21:53:06 PDT

Message-Id: <E706+GXR5ia@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: ATA@dt.wdc.com

Subject: ...no subject...

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Steve -

I believe very strongly that there should be a geographic

distribution to the meetings.  Without geographic distribution some people

will be permanently excluded from the standards process.  What I hear

coming from Jim McGrath is that he will reject anything that happens

outside the bay area because he does not want to go elswhere for the

meetings...

Curtis E. Stevens

From ata-owner  Fri Jul  8 16:29:29 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00116; Fri, 8 Jul 94 16:29:29 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA00106; Fri, 8 Jul 94 16:29:14 PDT

Received: from gatekeeper.nsc.com (gatekeeper.nsc.com [139.187.71.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id QAA17810; Fri, 8 Jul 1994 16:22:53 -0700

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA24883 for scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com; Fri, 8 Jul 94 16:28:40 -0700

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA02117 for TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com; Fri, 8 Jul 94 16:28:38 -0700

Received: from bach.nsc.com by berlioz.nsc.com (4.1/SMI-4.1)

id AA23346; Fri, 8 Jul 94 16:28:38 PDT

Date: Fri, 8 Jul 94 16:28:38 PDT

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9407082328.AA23346@berlioz.nsc.com>

To: TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com,

ata@dt.wdc.com@hp427u.tus.ssi1.com,

scsi@wichitaks.ncr.com@hp427u.tus.ssi1.com

Subject: Re:  ATA Meetings and Tom Hanan's suggestion

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I prefer the ATA Working Group meetings to be held in the Bay Area and Southern

California. I think that this will resolve most of the objections - especially,

considering the fact that every second month we're holding a working group

meeting with the ANSI Plenary that is well distributed geographically.

-Robbie Shergill

National Semiconductor Corp.

From ata-owner  Fri Jul  8 18:34:08 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02701; Fri, 8 Jul 94 18:34:08 PDT

Received: from wdus04.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA02694; Fri, 8 Jul 94 18:34:07 PDT

Received: with SMTP-MR; Fri, 8 Jul 1994 18:27:30 PST

Mr-Received: by mta WDUS04; Relayed; Fri, 08 Jul 1994 18:27:30 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Sat, 9 Jul 1994 01:36:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: suggestions

To: ATA@dt.wdc.com

Message-Id: <01HEH1YTHM2S8YCHBW@wdus04.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Sat, 9 Jul 1994 02:26:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;03728180704991/455705@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Well I don't know about the rest of you but I think it is time to get back

to work.

The problem is simple, our last few meetings have brought into

question a lot of material that most assumed would not be addressed in

ata-2. So instead of straight forward editorial meetings we were debating

the usefulness of ata-2 and it's various technical shortcomings.

SO BE IT:

1) If we have so many gripes about ata-2 as it stands today lets start

seeing some e-mail so those members who can only participate via e-mail

can understand the issues. Suggestions on how to solve the issues would

also be welcome.

2) Lets start keeping a list of open technical issues so the group can make

informed decisions at the plenary about which items should be addressed

in the working group meetings. Once an issue is closed the history of

how it was closed will help it stay that way.

3) Lets start taking advantage of the information technology we have by

airing our technical issues and reply's BEFORE the meetings. When

consensus is not apparent we can call for straw votes over the reflector

before the plenary and working group meetings. That way we can

concentrate our time at the meetings on the unresolved issues instead of

re-hashing the things we agree on.

4) We need to move back to requiring issues on the agenda be submitted to

the group via the reflector or paper at the meeting. Open sections for

new items are fine but should be addressed on a AS-TIME-PERMITS basis.

PLEASE GET YOUR ISSUES ON THE REFLECTOR AT LEAST A WEEK BEFORE THE

MEETING SO THERE IS TIME FOR THE DUST TO SETTLE BEFORE THE MEETING.

REMEMBER IF YOU WANT ACTION ON AN ITEM YOU MUST ALLOW PEOPLE TIME

TO RESEARCH IT.

When we review the agenda we should be ruthless in removing items which

the group is not yet prepared to resolve. We can share our opinions over

e-mail before the next meeting.

5) We need to maintain a list of individual editorial and technical changes

resolved at each meeting on the ata FTP site and the BBS. The list could

be included in the published min or sent via e-mail and the mailings.

The goal of these SUGGESTIONS is to help us identify and resolve issues.

This is after all the purpose of our meetings.

Please feel free to reply over the reflector with your own suggestions, or

comments and improvements on mine.

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7312

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Fri Jul  8 19:57:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04436; Fri, 8 Jul 94 19:57:49 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA04429; Fri, 8 Jul 94 19:57:38 PDT

Received: from worf.qntm.com (worf.qntm.com [146.174.250.100]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id TAA18589; Fri, 8 Jul 1994 19:51:16 -0700

Received: from mail.qntm.com by worf.qntm.com with SMTP

(1.37.109.8/16.2) id AA04220; Fri, 8 Jul 1994 19:56:42 -0700

Received: from qm_smtpgw by mail with SMTP

(1.37.109.8/16.2) id AA13203; Fri, 8 Jul 1994 19:55:04 -0700

Message-Id: <n1438424972.71359@qm_smtpgw.qntm.com>

Date: 8 Jul 1994 20:04:11 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: ATA Meetings and Tom Han

To: "ata@dt.wdc.com" <ata%dt.wdc.com@hp427u.tus.ssi1.com>,

"scsi@wichitaks.ncr.com" <scsi%wichitaks.ncr.com@hp427u.tus.ssi1.com>,

"Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0FC2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>ATA Meetings and Tom Hanan's suggestion

From Steve:

Are there Bay Area volunteers?

Quantum will continue to volunteer as a host for meetings at the

Crown Sterling Suites hotel in Milpitas, as we did in the days of

intense local bus activity.  Given that we probably hosted 6 or 8

meetings in a row at one point, we are use to it.  I have not been

forceful in volunteering recently since it seemed that some others

(e.g. Adaptec, just a few blocks from the Crown Sterling Suites)

wanted a go at it.  But you can always count on us as an AT host

(I even have it in my budget!)

Jim

From ata-owner  Fri Jul  8 20:22:08 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04960; Fri, 8 Jul 94 20:22:08 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA04953; Fri, 8 Jul 94 20:22:00 PDT

Received: from mail.qntm.com by worf.qntm.com with SMTP

(1.37.109.8/16.2) id AA04287; Fri, 8 Jul 1994 20:21:50 -0700

Received: from qm_smtpgw by mail with SMTP

(1.37.109.8/16.2) id AA14710; Fri, 8 Jul 1994 20:20:12 -0700

Message-Id: <n1438423464.61659@qm_smtpgw.qntm.com>

Date: 8 Jul 1994 20:27:40 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: ATA meeting locations

To: "ATA" <ATA@dt.wdc.com>,

"curtis_stevens@bannet.ptltd.co" <curtis_stevens@bannet.ptltd.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0FC2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>ATA meeting locations

...What I hear coming from Jim McGrath is that he will

reject anything that happens outside the bay area because

he does not want to go elswhere for the meetings...  -- Curtis

You are not hearing correctly.  I have more travel budget than salary -

I am on the road 50% of the time, and generally have no problem

coming to meetings.  I assume Steve, Tom, Larry, and a few others

are in a similar situation.

You seem to believe that attending one meeting in your local can

have a lot of effect on the resulting standard.  My experience (based

on 8 years of standards work) is that occasional meeting participation

never has significant impact on the standard.  (An exception is if you

represent a company very key to the success of the standard).  You simply

cannot keep current with all the issues, and track them to make sure

what you thought was agreed to is not overturned in a subsequent

meeting because someone new is attending that meeting or someone

old has a new argument.

Instead, having a lot of new people at every meeting will definitely

slow down the process, since by its nature issues will be constantly

re-raised.  Given that ATA-2 is SUPPOSE to be a quick, slight modification

of ATA-1 (re edit it to make it cleaner; add a couple of features; roll

in some SFF stuff), and that the market needs a standard bringing

together a lot of the work we have done in SFF during the past year,

I don't see any reason to make the process more complicated than it has

been (note this is exactly the sort of stuff the SFF people object to

all the time).  Remember that this was supose to have been completed

last month!

I think Larry might argue that the standards process is by its nature

slow, and delay is not a vice.  Here I disagree - we have many mechanisms

to get input in the standards process, from public review to email.

Reducing the productivity of working groups does not seem to be a good

way to insure "fairness" - it costs us too much.

Since ATA-3 is more open ended at this point, I have no firm opinions

on that topic - efficiency of execution is not important in a project

that has yet to define its direction.

Jim

From ata-owner  Mon Jul 11 06:52:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08958; Mon, 11 Jul 94 06:52:26 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08951; Mon, 11 Jul 94 06:52:04 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id GAA03848; Mon, 11 Jul 1994 06:49:40 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Mon, 11 Jul 1994 06:50:36 EDT

Date:         Mon Jul 11 06:50:35 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e2123fc.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: Proposed ATA-2 Working G

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Jim McGrath wrote regarding working group meetings:

>First, they seem to be every 2 to 3 weeks - why so often?

If you are correct in your calculation I would join your inquiry or objection.

>Second, we seem to be spreading them throughput the country.  Was

>this at the request of people  or what?  I think we have been

>very productive with meetings in San Jose, and it has allowed

>companies to send engineers to the meetings who otherwise would

>not be able to travel.

While I doubt I would join you on this, the proper way to settle this issue

would be for you to introduce a motion that all X3T10 working group meetings

must be in San Jose and that you are elected the permanent host for all

meetings. I would not object to a friendly amendment that the only allowed

location be expanded to include Milpitas since the border police are not

obtrusive.

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\356.MSG

### Org Date:    07-11-94 06:26:37 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: Proposed ATA-2 Working G

### Attachments: none

From ata-owner  Mon Jul 11 11:47:19 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15176; Mon, 11 Jul 94 11:47:19 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA15169; Mon, 11 Jul 94 11:47:13 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id af20552; 11 Jul 94 14:46 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 11 Jul 94 11:45:01 MDT

Received: from cc:Mail by ccmgate1bb.FTCollinsCO.NCR.COM

id AA773952303 Mon, 11 Jul 94 11:45:03 MDT

Date: Mon, 11 Jul 94 11:45:03 MDT

From: jlohmeyer@ccmgate1bb.ftcollinsco.NCR.COM

Message-Id: <9406117739.AA773952303@ccmgate1bb.FTCollinsCO.NCR.COM>

To: scsi@WichitaKS.NCR.COM, ata@dt.wdc.com

Subject: Multi-Media SCSI-2 Demo Thursday Morning Bedford, NH

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Janet Burnham-Sauter of Global Engineering Documents will be

present at the Bedford, NH X3T10 meetings to demonstrate a

new product concept that Global is considering:  Electronic

multimedia documents (essentially hypertext).  She will have

initial versions of some of our documents, including SCSI-2.

The demo will take place Thursday morning, July 21, from

10:00 am until noon.  Please stop by during that time to see

for yourself.  The product is a Windows (tm) application that

works much like the Windows help system.

The plan is to offer a subscription service that would

include periodic document updates on CDROM.  For this

purpose, I am requesting that the document editors bring

electronic copies of their mostly-recently distributed

documents to the meeting next week.  Janet will take these

back for conversion.

From ata-owner  Mon Jul 11 12:15:39 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15842; Mon, 11 Jul 94 12:15:39 PDT

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA15831; Mon, 11 Jul 94 12:15:36 PDT

From: uunet!bannet.ptltd.com!curtis_stevens

Received: from ptltd.ptltd.com by relay1.UU.NET with SMTP

(relay) id QQwycq24473; Mon, 11 Jul 1994 13:14:41 -0400

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qNOpu-0002KSC; Mon, 11 Jul 94 13:08 EDT

Received: by ptlnor3.ptltd.com; Mon, 11 Jul 94 13:08:36 EDT

Date: Mon, 11 Jul 94 16:09:07 PDT

Message-Id: <E706+cmL6ia@ptlnor3.ptltd.com>

To: uunet!DT.WDC.COM!ATA

Subject: ATA Meetings

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Jim

If all the issues the standard is attempting to address are well

understood then I agree, a single meeting place is fine.  The fact is that

nothing is well understood and the meetings are an attempt to comprimise

on issues which few people fully understand.  If I can only attend 2 or 3

meetings a year I will have a much better understanding of the issues

because I will have a chance to listen to and participate in the

confrontations/conversations that take place.  This gives someone, such as

myself, a much better chance ti IMPLEMENT the standard in the spirit in

which it was written.  I do not think it is necessary to explain the

problems which can happen when a part of the standard is mis interpreted.

Curtis E. Stevens

From ata-owner  Thu Jul 14 15:20:59 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15441; Thu, 14 Jul 94 15:20:59 PDT

Received: from maxtor.com (zeus.maxtor.com) by dt.wdc.com (4.1/SMI-4.1)

id AA15434; Thu, 14 Jul 94 15:20:56 PDT

Received: from sanjose3.maxtor.com ([134.6.34.200]) by maxtor.com (4.1/SMI-4.1)

id AA14539; Thu, 14 Jul 94 15:21:45 PDT

Message-Id: <9407142221.AA14539@maxtor.com>

Received: by sanjose3.maxtor.com with VINES ; Thu, 14 Jul 94 15:16:02 PDT

Date: Thu, 14 Jul 94 14:28:23 PDT

From: Ray=Heineman%ENG12%MCO@sanjose3.maxtor.com

To: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To: ATA Working Group

11111111111111111111111111111111111111111111111111111111

There has been some confusion regarding the IDLE command

Para. 8.11 as specified in revision "C" of the ATA2 spec.

The issue may just be editorial.

As defined issuing an IDLE command with a non-zero sector

count value,  enables the automatic idle mode timeout.

When the timer expires the device shall enter Idle Mode.

In past specifications issuing a IDLE command with timer

has entered the Standby Mode.

Is this a editorial problem?

22222222222222222222222222222222222222222222222222222222

The CHECK POWER MODE command as defined in the current

specification determines only STANDBY (00h) or IDLE/ACTIVE (FFh).

Due to improvements in system and drive power management, the

CHECK POWER MODE command needs to be enhanced to report the

current power mode state.

A proposed change would define a value for each power mode

state.

Value		Power Mode

FFh		ACTIVE

80h		IDLE

00h		STANDBY

BXH		VENDOR UNIQUE

******************************************************

Ray Heineman

MAXTOR Corp.

PHONE 303-678-2591

EMAIL ray_heineman@maxtor.com

From ata-owner  Thu Jul 14 17:36:35 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18476; Thu, 14 Jul 94 17:36:35 PDT

Received: from cirrus.com by dt.wdc.com (4.1/SMI-4.1)

id AA18468; Thu, 14 Jul 94 17:36:32 PDT

Received: from sunstorm.corp.cirrus.com (sunstorm1) by cirrus.com with SMTP id AA22104

(5.65c/IDA-1.4.4 for <ata@dt.wdc.com>); Thu, 14 Jul 1994 17:36:04 -0700

Received: from sundrive.corp.cirrus.com by sunstorm.corp.cirrus.com with SMTP id AA23922

(5.65c/IDA-1.4.4); Thu, 14 Jul 1994 17:35:29 -0700

Received: from novmailpc.cirrus.com by sundrive.corp.cirrus.com (4.1-Cirrus/S)

id AA02309; Thu, 14 Jul 94 17:35:27 PDT

Received: from cc:Mail by novmailpc.cirrus.com

id AA774232747 Thu, 14 Jul 94 17:39:07 PST

Date: Thu, 14 Jul 94 17:39:07 PST

From: chen@corp.cirrus.com

Message-Id: <9406147742.AA774232747@novmailpc.cirrus.com>

To: TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com, ata@dt.wdc.com,

scsi@wichitaks.ncr.com

Subject: cc:Mail SMTPLINK 2.0 Undeliverable Message

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Message is undeliverable.

Reason: HOST Unreachable

To: TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com,

ata@dt.wdc.com,

scsi@wichitaks.ncr.com,

"R. S. Shergill" <rss@berlioz.nsc.com>

Subject: Re[2]: ATA Meetings and Tom Hanan's suggestion

Original text follows:

--------------------

To: TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com,

ata@dt.wdc.com,

scsi@wichitaks.ncr.com,

"R. S. Shergill" <rss@berlioz.nsc.com>

Subject: Re[2]: ATA Meetings and Tom Hanan's suggestion

Steve,

I would echo Robbie's comment on ATA meeting location.  Why don't we

make a straw poll?

Joe Chen

Cirrus Logic, Inc.

______________________________ Reply Separator _________________________________

Subject: Re:  ATA Meetings and Tom Hanan's suggestion

Author:  "R. S. Shergill" <rss@berlioz.nsc.com> at novmailpc

Date:    7/8/94 4:28 PM

I prefer the ATA Working Group meetings to be held in the Bay Area and Southern

California. I think that this will resolve most of the objections - especially,

considering the fact that every second month we're holding a working group

meeting with the ANSI Plenary that is well distributed geographically.

-Robbie Shergill

National Semiconductor Corp.

From ata-owner  Fri Jul 15 06:38:53 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04365; Fri, 15 Jul 94 06:38:53 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA04358; Fri, 15 Jul 94 06:38:50 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA10454; Fri, 15 Jul 1994 09:38:49 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9407150938.tn455413@aol.com>

To: ata@dt.wdc.com

Date: Fri, 15 Jul 94 09:38:47 EDT

Subject: Project Proposal ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

July 14, 1994

----------------------------------------------------------------------------

X3T10/94-022r3

Project Proposal

For a New

X3 Standard

ATA Packet Interface

(ATAPI)

July 13, 1994

IDENTIFICATION OF PROPOSED PROJECT

TITLE:  ATA Packet Interface (ATAPI)

PROPOSER:  X3T10

DATE SUBMITTED:  March 17, 1994  X3T10 forwarding date.

PROJECT TYPE:  D - Development of standards within X3 TC.

JUSTIFICATION OF PROPOSED STANDARD

NEEDS:

As the popularity of the ATA interface has increased, its

application area has grown outside the originally intended

purpose of direct attachment of small disk drives to desktop

computers.

ATAPI is needed to provide a packet interface capable of

delivering SCSI command descriptor blocks so that other

peripherals can be attached to the system via ATA.  Cuurently CD-

ROM and tape devices are planning to use ATAPI.

RECOMMENDED SCOPE OF STANDARD:

The proposed ATAPI  standard maintains a high degree of

compatibility with the current ATA interface while providing

enhancements to extend its usability to other device types.

The ATAPI standard aims to:

a)   provide a means to support any device that implements

SCSI commands through the definition of a packet

protocol.

b)   investigate enhancements to provide overlapped commands

using a multi-thread protocol

c)   allow a  shared cable connection with ATA disk drives.

This proposed standard will not define the command sets used over

ATAPI; those are separate projects.

EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft

AT Attachment standards to provide additional capabilities.

EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO

CURRENT AND POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to insure that the AT

Attachment has a compatible growth path.  This will insure that

current investments in AT Attachment are provided with more

stability in the face of technological developments.

DESCRIPTION OF PROPOSED PROJECT:

TYPE OF DOCUMENT:  Standard.

DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATAPI standard is for use in closed systems.

RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATAPI standard:

(1)  Solicit continuing participation by the present AT

Attachment participants through X3T10 procedures and new

participants through press releases.  Invite comments by

end-user organizations and invite proposals from

organizations that may have a contribution to a viable

ATAPI standard.

(2)  Establish functional requirements for ATAPI functional

additions along with downward compatibility requirements.

(3)  Prepare a draft standard based on proposals submitted and

other information gathered during the initial

investigation.

(4)  Consider the results of ATAPI testing as may be available

to the committee through the voluntary efforts of the

various participants in X3T10.

(5)  Submit the draft proposed standard to X3 for further

processing.

RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN

SUBJECT MATTER:

The current membership of X3T10 includes representatives from all

parts of the computer industry from semiconductor chip

manufacturers to large mainframe system manufacturers as well as

Government agencies.  The members of X3T10 have expressed their

desire to participate and cooperate in the development of this

proposed standard.

There are sufficient resources to complete the development of

this standard without delaying work on other standards.

RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical

Committee X3T10 which is responsible for developing the draft ATA

Extensions standard.

ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets for two days bi-monthly. Specific

task ad hoc groups are called as may be required for one to three

days between the regular meetings but their results are not

binding.

TARGET DATE FOR DPANS TO X3 (MILESTONE 10):  December 1995

ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 10

years.

IMPLEMENTATION IMPACTS

IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATAPI standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATAPI standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATAPI testing as may

be available to the committee through the voluntary efforts of

the various participants in X3T10.  With this method all costs

are borne by the organizations of the various participants and

have for the most part been mainly an adjunct of their normal

development costs.

LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will

be made under Milestone 7 of the SD-2.

CLOSELY RELATED STANDARDS ACTIVITIES

EXISTING STANDARDS:  None.

X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface -- Project 791-D

ATA Extensions -- Project 948-D

X3 STUDY GROUPS:  None.

OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

It is anticipated that this standard will be proposed to

JTC1/SC25/WG4.

OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:

None.

RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

RECOMMENDATIONS FOR CLOSE LIAISON:  None.

Regards,

Lawrence J. Lamers

Adaptec                    TEL: 408-957-7817

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@AOL.COM

From ata-owner  Fri Jul 15 06:39:08 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04373; Fri, 15 Jul 94 06:39:08 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA04366; Fri, 15 Jul 94 06:39:05 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA10484; Fri, 15 Jul 1994 09:39:05 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9407150939.tn455419@aol.com>

To: ata@dt.wdc.com

Date: Fri, 15 Jul 94 09:39:03 EDT

Subject: Project Proposal ATA-3

Sender: ata-owner@dt.wdc.com

Precedence: bulk

July 14, 1994

----------------------------------------------------------------------------

X3T10/94-019r3

Project Proposal

For a New

X3 Standard

ATA-3

(ATA-3)

July 13, 1994

IDENTIFICATION OF PROPOSED PROJECT

TITLE:  ATA-3 Physical Interface (ATA3-PH)

PROPOSER:  X3T10

DATE SUBMITTED:  August 1, 1994  X3T10 forwarding date.

PROJECT TYPE:  D - Development of standards within X3 TC.

JUSTIFICATION OF PROPOSED STANDARD

NEEDS:

The standard for AT Attachment has been completed and the draft

for the ATA Extensions (ATA-2) is near completion.  It has become

evident in the past few months that the industry desires further

improvements in this interface.

ATA-3 is needed to continue the evolution of the ATA interface.

The ATA-3 development is needed to reduce confusion in the

industry and the risk of incompatiblitiy and mismatched system

integration that can occur without a standard.

RECOMMENDED SCOPE OF STANDARD:

The proposed ATA-3 standard maintains a high degree of

compatibility with the ATA-2 interface and migrates the interface

to meet tomorrows needs in an evolutionary way.  ATA-3 is

intended to be primarily a low-cost, efficient disk drive

interface.

The goals of ATA-3 are:

a)   reduced complexity by removing unused functions and

features.

b)   improve compatibility through improved documentation of

registers, commands, and protocol.

c)   address cable/connector issues related to data integrity

at higher transfer rates.

d)   investigate increased transfer rates.

EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft

ATA-2 standard to provide additional capabilities.

EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO

CURRENT AND POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to insure that the AT

Attachment has an upward, compatible growth path.  This will

insure that current investments in AT Attachment are provided

with more stability in the face of technological developments.

DESCRIPTION OF PROPOSED PROJECT:

TYPE OF DOCUMENT:  Standard.

DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATA-3 standard is for use in closed systems.

RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATA-3 standard:

(1)  Solicit continuing participation by the present AT

Attachment participants through X3T10 procedures and new

participants through press releases.  Invite comments by

end-user organizations and invite proposals from

organizations that may have a contribution to a viable

ATA-3 standard.

(2)  Establish functional requirements for ATA-3 functional

additions along with downward compatibility requirements.

(3)  Prepare a draft standard based on proposals submitted and

other information gathered during the initial

investigation.

(4)  Consider the results of ATA-3 testing as may be available

to the committee through the voluntary efforts of the

various participants in X3T10.

(5)  Submit the draft proposed standard to X3 for further

processing.

RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN

SUBJECT MATTER:

The current membership of X3T10 includes representatives from all

parts of the computer industry from semiconductor chip

manufacturers to large mainframe system manufacturers as well as

Government agencies.  The members of X3T10 have expressed their

desire to participate and cooperate in the development of this

proposed standard.

There are sufficient resources to complete the development of

this standard without delaying work on other standards.

RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical

Committee X3T10 which is responsible for developing the draft ATA

Extensions standard.

ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets for two days bi-monthly. Specific

task ad hoc groups are called as may be required for one to three

days between the regular meetings but their results are not

binding.

TARGET DATE FOR DPANS TO X3 (MILESTONE 10):  December 1995

ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 10

years.

IMPLEMENTATION IMPACTS

IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATA-3 standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATA-3 standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATA-3 testing as may

be available to the committee through the voluntary efforts of

the various participants in X3T10.  With this method all costs

are borne by the organizations of the various participants and

have for the most part been mainly an adjunct of their normal

development costs.

LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will

be made under Milestone 7 of the SD-2.

CLOSELY RELATED STANDARDS ACTIVITIES

EXISTING STANDARDS:  None.

X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface -- Project 791-D

ATA Extensions -- Project 948-D

X3 STUDY GROUPS:  None.

OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

It is anticipated that this standard will be proposed to

JTC1/SC25/WG4.

OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:

None.

RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

RECOMMENDATIONS FOR CLOSE LIAISON:  None.

Regards,

Lawrence J. Lamers

Adaptec                    TEL: 408-957-7817

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@AOL.COM

From ata-owner  Fri Jul 15 10:17:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA09245; Fri, 15 Jul 94 10:17:42 PDT

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA09238; Fri, 15 Jul 94 10:17:38 PDT

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA22917 for ata@dt.wdc.com; Fri, 15 Jul 94 10:17:36 -0700

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA28501 for ata@dt.wdc.com; Fri, 15 Jul 94 10:17:35 -0700

Received: from bach.nsc.com by berlioz.nsc.com (4.1/SMI-4.1)

id AA05648; Fri, 15 Jul 94 10:17:23 PDT

Date: Fri, 15 Jul 94 10:17:23 PDT

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9407151717.AA05648@berlioz.nsc.com>

To: ata@dt.wdc.com, sff_reflector.sun.com@berlioz.nsc.com

Subject: ATA Working Group Minutes - June 1994

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Doc. No. X3T10/94-141R0

Date:    July 1, 1994

Project:

Ref. Doc.:

Reply to: S. Finch

Accredited Standards Committee*

X3, Information Processing Systems

To:         Membership of X3T10

From:       Shergill/Lamers/Finch

Subject:    Minutes of X3T10 ATA  Working Group June 29, 1994

Agenda

1. Opening Remarks                            2

2. Attendance and Membership, Introductions   2

3. Approval of Agenda                         3

4. Document Distribution                      3

5. Old Business                               3

5.1  ATA-2 Scope:                             3

5.2 Project Proposals Comment Resolution [94-019,94-020,94-021,94-022]

(Finch)                                       3

5.3 ATA-2 Editing                             4

6. New Business                               5

7. Action Items                               5

8. Meeting Schedule                           6

9. Adjournment                                6

Results of Meeting

1. Opening Remarks

Steve Finch convened the meeting at 9:00 am.  Steve thanked Tom Hanan of

Western Digital for hosting the meeting.

As is customary, the people attending introduced themselves.  A copy of the

attendance list was circulated for attendance and corrections.

It was stated that the meeting had been authorized by X3T10 and would be

conducted under the X3 rules.  Ad hoc meetings take no final actions, but

prepare recommendations for approval by the X3T10 task group.  The voting

rules for the meeting are those of the parent committee, X3T10.  These rules

are:  one vote per company; and any participating company member may vote.

The minutes of this meeting will be posted to the X3T10 BBS and the ATA

Reflector and will be included in the next X3T10 committee mailing.

2. Attendance and Membership, Introductions

Attendance at working group meetings does not count toward minimum attendance

requirements for X3T10 membership.  Working group meetings are open to any

person or company to attend and to express their opinion on the subjects being

discussed.

The following people attended the meeting.

Name                Company             Email Address

----                -------             -------------

Richard Kalish      Adaptec             rkalish@aol.comt

Lawrence J. Lamers  Adaptec             ljlamers@aol.com

Carlos Orosco       AST                 carlos.orosco@ast.com

Joseph Chen         Cirrus Logic        chen@cirrus.com

Steve Anderson      Conner Peripherals  steve.anderson@conner.com

Dan Colegrove       IBM                 colegrove@vnet.ibm.com

Tony Goodfellow     GSI

Ray Heineman        Maxtor              ray_heineman@maxtor.com

Robbie Shergill     NationalSemicon     rss@berlioz.nsc.com

Mike Carpenter      Qlogic              M.carpenter@qlc.com

Hank Davenport      Samsung

John Masiewicz      Seagate Technology  masiewicz@notes.seagate.com

Hale Landis         Seagate Technology  landis@notes.seagate.com

Gene Milligan       Seagate Technology  Gene_Milligan@notes.seagate.com

Stephen Finch       Silicon Systems     5723283@mcimail.com

Tom Hanan           Western Digital     t_hanan@dt.wdc.com

3. Approval of Agenda

The agenda was approved as proposed.

4. Document Distribution

Minutes of previous meeting

New Table for Drivers and Termination (Hanan)

ATA-2 Rev 2d with and without change-bars

5. Old Business

5.1  ATA-2 Scope:

John Masiewicz said that last meeting went beyond the stated purpose of the

meeting with many of the changes. Particularly, capacitance and spindle-sync.

Tom Hanan had also received same feedback from his people back home.

Steve Finch agreed that the last meeting was advertised as an editorial

review.

John Masiewicz also asserted that termination should be left for ATA-3. Ray

Heineman said why not define it because Mode 4 seems to need it. John

responded that we have discussed it before and agreed that you can do Mode 4

without it. Tom agreed with this and asked that this issue be added to the

agenda for the July meeting. This was done, but as an ATA-3 item.

5.2 Project Proposals Comment Resolution [94-019,94-020,94-

021,94-022] (Finch)

Hale Landis restated what he had said at the last meeting. It is his personal

opinion that ATA-3 should be the next extension of ATA - should not include

ATAPI. Host adapter requirements are different for ATA-2 than ATAPI.

Tom Hanan took floor - having talked to a lot of people, he too has heard that

the two are very different. But they do co-exist on the same cable so it'd be

nice for them to agree. So, Tom wants two different documents but a lot of

cooperation between people drafting them. Tom wants people to work under DADI

for anything else. Joe Chen said that since ATAPI has to share the cable, we

need close communications and cooperation tween the two developments. At the

logical level the ATA side doesn't need to care what ATAPI does. Steve Finch,

not as chair, wants to maintain low-level conformance between ATAPI and ATA-3.

Hale said that a CD-ROM on the same cable with an ATA-2 disk drive may cause

problems. Larry Lamers that we are essentially hearing silicon, drive and

system guys' views. As a silicon guy he understands the desire for common

silicon. He also understands the system guys' need for software commonality.

But, the drive guy is under severe cost pressure and this doesn't work with

the other two guys' desire for a universal interface. They rather have an

interface optimized for the disk drive and another for all other device types.

He also doesn't think other devices will co-exist with the drive on the same

cable. Richard Kalish asked why ATAPI cannot refer to the ATA-3 document at

the physical level. We should coordinate the two so they stay consistant.

Larry doesn't see why they have to agree - drive's needs may be divergent.

Dan Colegrove said that people are going to demand being able to hang CD-ROMs

and tapes on the same cable - one plug should be able to support both. Joe

Chen said that speed mismatch exists even between ATA and ATA-2 disk drives.

Some HBAs detect the drv bit and use different timing for each. Hale asserted

again that you can't do mode 3 timing on a cable with an old device. Tony

Goodfellow did not agree with this viewpoint - said that there are systems out

today that do this. Robbie Shergill agreed with Hale's assertion; but Tom

Hanan pointed out that CD-ROMs can become mode 3.

Hale said that he doesn't have a problem with two different documents proposed

today by Tom, except that ATA-3 will probably pick up an optional cable/conn

that will be different for higher performance and CD-ROMs will not want to

share this connection.

Carlos Orosco said that ATAPI must exist on the IDE cable else it'll die.

The group, at this point, worked out a proposed ATA-3 scope to take to the

plenary in July. Basically, used Tom's proposal but made the ATA scope more

general and added that ATA-3 will reserve

command codes for ATAPI. Two issues outstanding at this point (when time ran

out): how to handle redefinition of some register bits when the two projects

want to use them differenly with their different protocols; and where will the

ATAPI commands go.

5.3 ATA-2 Editing

The Rev. 2d document was edited in a page-by-page manner. All of the edits

from the last meeting were revisited and struck or re-done if they were deemed

technical changes.

3.1.4 Data Block: changed to use the term "sector"

Added the definition for "sector"

Editor will change the term ECC throughout to reflect the fact that this

is really vendor unique.

3.2.5 Byte Ordering: This section will be kept after corrections as worked out

in this group.

4.1 Say "other" device instead of "second" drive.

4.5 Made the cable length spec a maximum spec.

4.6 The group decided to remove note 1 from Table 3 because its addition

during last meeting was deemed to be more than editorial.

Moved Cin and Cout specs to the AC Table.

Changed the editor's note to a proper note.

5.2.4 DASP-:  The paragraph added at the last meeting was struck because it

was deemed to be more than editorial. It will be reconsidered at the next

month's meeting as a technical item.

5.2.10 INTRQ: Joe Chen wanted the ATA note put back in. This was taken out a

long time ago. Putting it back in now was considered to be a technical change.

Joe was asked to explain the meaning of that note and the group will consider

adding it.

5.2.15.1 SPSYNC:  Left the change intact. Hale will explain.

10.4.1 PIO Data Transfers: In first para, change "...compatible with ATA" to

"... compatible with Modes 0, 1, or 2".

Figure 5 - remove note 3 from t5, note 4 chang t5 and t7 to t7 and t8

10.4.3 tE needs to be added to timing diagram (tD-tGR).  tFmax note 1 should

be note 2

Annex F - Swap interface number of 2 and 3.

6.1.1 para 5 & 6 - revised wording to clarify LBA and CHS sector addressing.

Title changed to Addressing Modes.

6.2.6 - Add phrase Tbits 7-4 are not defined by this standard and are

recommended  to be written as zero.  A future standard may reserve these

bitsU.

6.2.9  Hale Landis suggested that the removable media bits 5 & 3 be removed.

He also suggested that the commands in section 8 be made vendor specific.

6.2.13 Joe Chen suggested changing the CORR bit to vendor specific.

7.1 Change to T BSY shall be set within 400 nsec of the the negation of...U

Hale to propose revised wording.  He reqeusted that 2 minutes be checked as

the maximum time to drive ready.

7.2 This section is poorly defined.  Hale requested a better definition.

7.3.2 Ray Heineman stated that there is a problem with Check Power Mode which

actually only checks standby mode.  Added power states should be included

other than standby.

HaleUs re-written power management descirptions have not been incorporated.

7.3.6 Delete Tidle timer expirationU in path 1 description.

7.4 Changed to 7.3.7

7.5 Hale is back to his old ways - he wants to eliminate Table 8 and have the

error information reflected in the command descriptions.

8.  Hale wants to add a column to define what type of protocol is used for the

command:  PIO read or write or DMA read or write or non-data.

6. New Business

None.

7. Action Items

Larry Lamers to revise project proposals for ATA-3.

Hale Landis to post document comments to reflector.

Ray Heineman to make proposal for power management reporting.

Steve Finch to incorporate changes into ATA-2.

8. Meeting Schedule

July 21, 1994 at Sheraton in Manchester, NH.

August 31 - September 1, 1994 in Silicon Valley, CA

Sept 15  Houston, TX, (Plenary)

Sept 28  Oklahoma City, OK. Host: Seagate

Oct 19   Longmont, CO. Host: Maxtor

Nov 30   Irvine, CA. Host: WD

9. Adjournment

The meeting adjourned at approximately 4:30 PM.

*Operating under the procedures of The American National Standards Institute.

X3 Secretariat, Computer and Business Equipment Manufacturers Association

(CBEMA)

1250 Eye Street NW, Suite 200, Washington, DC 20005-3922

Telephone: 202-737-8888 (Press 1 twice) FAX: 202-638-4922 or 202-628-2829

From ata-owner  Fri Jul 15 14:58:05 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15255; Fri, 15 Jul 94 14:58:05 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA15248; Fri, 15 Jul 94 14:57:53 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id OAA14387; Fri, 15 Jul 1994 14:45:33 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 15 Jul 1994 14:08:28 EDT

Date:         Fri Jul 15 14:08:28 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e26d09c.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: *** No Subject *** - but it was ATA Power Modes

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Ray Heineman wrote:

>A proposed change would define a value for each power mode

>state.

>Value  Power Mode

>FFh  ACTIVE

>80h  IDLE

>00h  STANDBY

>BXH  VENDOR UNIQUE

I trust this is a proposal for ATA-3. In any case the proposal is incomplete.

The protocols currently specified during idle and standby need to be expanded

for active, and vendor unique.  Is BXH (presumably BXh) 16 values and are other

values reserved?  I gather these  values refer to the sector count register.

Gene Milligan

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\254.MSG

### Org Date:    07-15-94 06:29:27 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: *** No Subject *** - but it was ATA Power Modes

### Attachments: none

From ata-owner  Mon Jul 18 08:32:10 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27777; Mon, 18 Jul 94 08:32:10 PDT

Received: from inet-gw-3.pa.dec.com by dt.wdc.com (4.1/SMI-4.1)

id AA27770; Mon, 18 Jul 94 08:32:07 PDT

Received: from peyoti.adv.shr.dec.com by inet-gw-3.pa.dec.com (5.65/27May94)

id AA20420; Mon, 18 Jul 94 08:26:54 -0700

Received: by peyoti.adv.shr.dec.com (5.65/DEC-Ultrix/4.3)

id AA09349; Mon, 18 Jul 1994 11:30:13 -0400

Message-Id: <9407181530.AA09349@peyoti.adv.shr.dec.com>

To: ata@dt.wdc.com

Cc: hall@peyoti.enet.dec.com

Subject: ATA SSWG meeting location ?

Date: Mon, 18 Jul 94 11:30:13 -0400

From: hall@peyoti.adv.shr.dec.com

X-Mts: smtp

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The ATA SSWG is meeting this Thursday, 9-12 in Manchester NH.

Anyone know the exact location  ?

Thanks,

Dana Hall

Digital Equipment Corp.

From ata-owner  Mon Jul 18 13:27:07 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA03976; Mon, 18 Jul 94 13:27:07 PDT

Received: from maxtor.com ([134.6.32.3]) by dt.wdc.com (4.1/SMI-4.1)

id AA03959; Mon, 18 Jul 94 13:26:40 PDT

Received: from sanjose3.maxtor.com ([134.6.34.200]) by maxtor.com (4.1/SMI-4.1)

id AA01522; Mon, 18 Jul 94 13:29:29 PDT

Message-Id: <9407182029.AA01522@maxtor.com>

Received: by sanjose3.maxtor.com with VINES ; Mon, 18 Jul 94 13:24:11 PDT

Date: Mon, 18 Jul 94 13:20:44 PDT

From: Ray=Heineman%ENG12%MCO@sanjose3.maxtor.com

Subject: Check Power Mode Proposal

To: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

TO:		ata@dt.wdc.com@internet

FROM:		ray_heineman@maxtor.com

DATE:		7/18/94

SUBJECT:	Check Power Mode Proposal

The purpose of this proposal is to enhance the Check Power

Mode command.

At the drive level we are continually improving power

management for energy star and battery life requirements.

The Check Power Mode command as defined in the current

specification identifies either a standby mode (00h) or

idle/active mode (FFh).

We have had a number of requests from users to add an

additional power state.

The proposal includes the following:

**************************************************************

Para. 7.3.2.  Power Management Commands

The Check Power Mode command allows a host to determine if a

device is currently in, going to or leaving Standby, Idle, or

Active mode.

Para. 8.4.  CHECK POWER MODE

If the device is in, going to, or recovering from the Standby,

Idle, or Active mode the device shall set BSY, set the sector

count register to code (XXh), clear BSY, and generate an interrupt.

+=========================+

| Code |    Power Mode	  |

+------+------------------+

| 00h  |     Standby	  |

| 80h  |      Idle        |

| FFh  |     Active       |

+------+------------------+

Table XX - Power Mode Codes

***************************************************************

If at all possible it would be benifical if this proposal could

be included in the ATA2 specification.  Maxtor currently supplies

products with this proposal.

From ata-owner  Mon Jul 18 16:27:20 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA07972; Mon, 18 Jul 94 16:27:20 PDT

Received: from netcom.com (netcom8.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA07847; Mon, 18 Jul 94 16:26:47 PDT

Received: by netcom.com (8.6.8.1/SMI-4.1/Netcom)

id QAA13751; Mon, 18 Jul 1994 16:27:00 -0700

From: penman@netcom.com (Duncan Penman)

Message-Id: <199407182327.QAA13751@netcom8.netcom.com>

Subject: Re: ATA SSWG meeting location ?

To: ata@dt.wdc.com

Date: Mon, 18 Jul 1994 16:26:58 -0700 (PDT)

In-Reply-To: <9407181530.AA09349@peyoti.adv.shr.dec.com> from "hall@peyoti.adv.shr.dec.com" at Jul 18, 94 11:30:13 am

X-Mailer: ELM [version 2.4 PL23]

Mime-Version: 1.0

Content-Type: text/plain; charset=US-ASCII

Content-Transfer-Encoding: 7bit

Content-Length: 223

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Location: Sheraton Tara Wayfarer in Bedford

121 South River Road

603-622-3766

The handout we got last nite says it will be in the Pavilion Room,

but that could presumably change.

Cheers,

Duncan Penman

From ata-owner  Mon Jul 25 06:18:20 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA16149; Mon, 25 Jul 94 06:18:20 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA16142; Mon, 25 Jul 94 06:18:15 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA02182; Mon, 25 Jul 1994 09:18:12 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9407250918.tn129941@aol.com>

To: ata@dt.wdc.com

Date: Mon, 25 Jul 94 09:18:12 EDT

Subject: ATAPI/ATA Meetings

Sender: ata-owner@dt.wdc.com

Precedence: bulk

July 25, 1994

-------------------------------------------------------------------------

*************************************************************************

*******************   M E E T I N G   N O T I C E S  ********************

*************************************************************************

SUBJECT:	ATAPI Study Group Meeting - August 30, 1994

ATA-2 Working Group Meeting - August 31- September 1, 1994

DATE:		8/30-9/1/94

TIME:		9:30 AM - 5 PM

PLACE:		Maxtor Corporation

251 River Oaks Parkway

San Jose, CA  95134

(408) 432-1700 (ask for BLDG 251 Lobby if you are lost)

HOST:		Ray Heineman  email: ray_heineman@maxtor.com

*************************************************************************

Steve Finch is chairing the ATA-2 meeting.

Tom Hanan is chairing the ATAPI meeting.

Ray Heinman is hosting the meetings.

This notice prepared by Larry Lamers - Don't blame Ray if there are any

errors.

DIRECTIONS:

FROM San Jose Airport:

The roads have changed a bit since Maxtor last hosted a meeting in

San Jose.  There are two exits from the airport - the main exit

and one that goes to Coleman Avenue.

Use the main exit/entrance from the airport.

Turn left onto Guadalupe Parkway. Stay to the right.

Take the North First Street exit off Guadalupe.

Turn left (north) at North First Street.

Turn right (east) at River Oaks Parkway.

Go through the intersection with Zanker Road; Maxtor

will be on your left about a 1/4 mile beyond the

intersection.

Bldg 251 is the one in the center. Come to that lobby.

Recovery options:

Fail to get on Guadalupe Parkway - keep going straight (east)

until you pass under Highway 101 expressway and then turn

left at next intersection (North First Street).

Miss North First Street exit off Guadalupe Parkway - you will

merge onto the Highway 101 expressway.  Take the first exit

(Trimble Road).  Stay on Trimble until you come to North

First Street and then turn left.

River Oaks Parkway is the first light (second right) on North

First Street north of Montague Expressway.

FROM San Francisco Airport:

Take Highway 101 Expressway south.

Take the 237 east exit.

Turn right (south) at North First Street.

Turn left (east) at River Oaks Parkway.

FROM Oakland Airport:

Take 880 south.

Take Montague west exit.

Turn right at North First Street.

Turn right at River Oaks Parkway.

---------------------------------------------------------------------------

Regards,

Lawrence J. Lamers

Adaptec                    TEL: 408-957-7817

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@AOL.COM

From ata-owner  Tue Jul 26 12:16:40 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22390; Tue, 26 Jul 94 12:16:40 PDT

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA22377; Tue, 26 Jul 94 12:16:38 PDT

Received: from hp427u.tus.ssi1.com by relay1.UU.NET with SMTP

id QQxagi01779; Tue, 26 Jul 1994 15:12:45 -0400

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id MAA05254 for <ata@dt.wdc.com@relay>; Tue, 26 Jul 1994 12:06:01 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 26 Jul 94 12:10:02 PDT

Date: 26 Jul 94 12:59:23 PDT

From: "Steve Finch" <uunet!vax2t.tus.ssi1.com!TFINCHS%A1.VAX2T.mrouter>

Subject: ATA WG Aug. 31 - Sep. 1

To: "ata@dt.wdc.com" <uunet!hp427u.tus.ssi1.com!ata@dt.wdc.com>

Message-Id: <"32959162704991/300499@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

A meeting announcement has already been distributed for the ATA Working

Group Meeting on August 31 and September 1.  Below you will find a proposed

agenda for that meeting.

The ATA-2 working draft document was uploaded to the SCSI BBS today.  You

will find it in the ATA file area.  The document is 0948DR2F.DOC.  It is a

Word For Windows 2.0 document file.  This is the document that will be

reviewed at the working group meeting.  Document 0848DR2E (Rev 2E) was

distributed at the Plenary meetings last week.  The differences between 2E

and 2F are very small:

1.  In clause 8.11, third paragraph, last sentence, we changed "enter

the Idle Mode" to "enter the Standby Mode".

2.  Table and Figure numbers were updated so that they are now numbered

consecutively throughout the document.

3.  I attempted to find all cross references within the document and

updated the references to point at what I believe are the correct clauses

and/or tables.

4.  Re-paginated to improve appearance.

The next ATA Working Group will focus on the final review of the ATA-2

document.  The Working Group agreed to hear a proposal on the addition of

series termination to the ATA-2 document.  This is the only proposal that

is planned for the meeting.

Any new proposals will be heard if time permits, i.e., after the review of

the ATA-2 document is completed.  The Chair believes that the review

process may not be completed, so if new proposals are brought to the

meeting, they will be distributed and added to the end of the agenda, but

probably will not be addressed at the meeting due to time limitations.

Such proposals will be carried forward to future meetings.

I truely believe that ATA-2 is near completion and, with a concerted

effort, we should be able to complete our work very soon.  Once ATA-2 goes

forward, we can start work on ATA-3 in earnest.

Proposed Agenda

X3T10 ATA Working Group

9:30 A.M. - 5:00 P.M., August 31, 1994

9:30 A.M. - 5:00 P.M., September 1, 1994

San Jose, CA

1.  Opening Remarks

2.  Attendance and Membership, Introductions

3.  Approval of Agenda

4.  Document Distribution

5.  Old Business

5.1.	Termination Issues (Hanan)

5.2.	ATA-2 Technical Review

6.  New Business

(Note:  new subjects will be addressed only if time permits)

7.   Meeting Schedule

September 15, 1994, Houston, TX, Compaq

October 19, 1994, Longmont, CO, Conner

November 10, 1994, Palm Springs, CA, Western Digital

8.  Adjournment

Meeting Location:

Maxtor Corporation

251 River Oaks Parkway

San Jose, CA  95134

(408) 432-1700 (ask for BLDG 251 Lobby if you are lost)

HOST:

Ray Heineman  email: ray_heineman@maxtor.com

From ata-owner  Wed Jul 27 09:53:04 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18904; Wed, 27 Jul 94 09:53:04 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA18892; Wed, 27 Jul 94 09:52:54 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA00299; Wed, 27 Jul 1994 12:52:50 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9407271250.tn241934@aol.com>

To: ata@dt.wdc.com

Date: Wed, 27 Jul 94 12:50:25 EDT

Subject: Project Proposal for ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

July 27, 1997

-----------------------------------------------------------------------------

X3T10/94-022r4

Remember to edit Header A (Shft-F8,P,H,A,E)

Project Proposal

For a New

X3 Standard

ATA Packet Interface

(ATAPI)

July 21, 1994

IDENTIFICATION OF PROPOSED PROJECT

TITLE:  ATA Packet Interface (ATAPI).

PROPOSER:  X3T10.

DATE SUBMITTED:  September 15, 1994.

PROJECT TYPE:  D - Development of standards within an X3 TC.

JUSTIFICATION OF PROPOSED STANDARD

NEEDS:

As the popularity of the ATA interface has increased, its

application area has grown outside the originally intended

purpose of direct attachment of small disk drives to desktop

computers.

ATAPI is needed to provide a packet interface capable of

delivering SCSI command descriptor blocks so that other

peripherals can be attached to the system via ATA.  Currently CD-

ROM and tape devices are planning to use ATAPI.

RECOMMENDED SCOPE OF STANDARD:

The proposed ATAPI standard maintains a high degree of

compatibility with the current ATA interface.

The ATAPI standard aims are:

a)   provide a means to transfer SCSI commands through the

definition of a packet protocol.

b)   investigate enhancements to provide overlapped commands

using a multi-thread protocol

c)   facilitate a  shared cable connection with ATA disk

drives via normative references to ATA, ATA-2 and ATA-3.

d)   to extend the use of ATA to support devices other than

traditional ATA devices.

This proposed standard will not define the command sets used over

ATAPI; those are separate projects.  The application for the

proposed standard does not require compliance with the SCSI-3

Architectural Model (SAM).

EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft

AT Attachment standards to provide additional capabilities.

EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO

CURRENT AND POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to ensure that the AT

Attachment has a compatible growth path.  This will ensure that

current investments in AT Attachment are provided with more

stability in the face of technological developments.

DESCRIPTION OF PROPOSED PROJECT:

TYPE OF DOCUMENT:  Standard.

DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATAPI standard is for use in closed systems.

RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATAPI standard:

1)   Solicit continuing participation by the present AT

Attachment participants through X3T10 procedures and new

participants through press releases.  Invite comments by end-

user organizations and invite proposals from organizations

that may have a contribution to a viable ATAPI standard.

2)   Establish functional requirements for ATAPI functional

additions along with downward compatibility requirements.

3)   Prepare a draft proposed standard based on proposals

submitted and other information gathered during the initial

investigation.

4)   Consider the results of ATAPI testing as may be available

to the committee through the voluntary efforts of the various

participants in X3T10.

5)   Submit the draft proposed standard to X3 for further

processing.

RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN

SUBJECT MATTER:

The current membership of X3T10 includes representatives from all

parts of the computer industry from semiconductor chip

manufacturers to large mainframe system manufacturers as well as

Government agencies.  The members of X3T10 have expressed their

desire to participate and cooperate in the development of this

proposed standard.

There are sufficient resources to complete the development of

this standard without delaying work on other standards.

RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical

Committee X3T10 which is responsible for the ATA Extensions

project.

ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets bi-monthly. Specific task ad hoc

groups are called as may be required for one to three days

between the regular meetings but their results are not binding.

TARGET DATE FOR dpANS TO X3 (MILESTONE 10):  December 1995

ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 5

years.

IMPLEMENTATION IMPACTS

IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATAPI standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATAPI standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATAPI testing as may

be available to the committee through the voluntary efforts of

the various participants in X3T10.  With this method all costs

are borne by the organizations of the various participants and

have for the most part been mainly an adjunct of their normal

development costs.

LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will

be made under Milestone 7 of the SD-2.

CLOSELY RELATED STANDARDS ACTIVITIES

EXISTING STANDARDS:  None.

X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface (ATA)-- Project 791-D

ATA Extensions (ATA-2)-- Project 948-D

AT Attachment -3 (ATA-3)

X3 STUDY GROUPS:  None.

OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

This standard will be considered for proposal to JTC1/SC25/WG4.

OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:

None.

RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

RECOMMENDATIONS FOR CLOSE LIAISON:  None.

From ata-owner  Wed Jul 27 09:53:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18919; Wed, 27 Jul 94 09:53:16 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA18893; Wed, 27 Jul 94 09:53:00 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA00397; Wed, 27 Jul 1994 12:52:58 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9407271250.tn241953@aol.com>

To: ata@dt.wdc.com

Date: Wed, 27 Jul 94 12:50:54 EDT

Subject: ATA Working Group Minutes

Sender: ata-owner@dt.wdc.com

Precedence: bulk

July 27, 1994

----------------------------------------------------------------------------

Minutes of ATA Working Group - July 21, 1994       X3T10/94-153r0

2

Accredited Standards Committee*

X3, Information Processing Systems

Doc.             No.

X3T10/94-153R0

Date:           July

21, 1994

Project:

Ref. Doc.:

Reply to: S. Finch

To:    Membership of X3T10

From:  Lamers/Finch

Subject:  Minutes of X3T10 ATA  Working Group July 21, 1994

Agenda

1. Opening Remarks                                         2

2. Attendance and Membership, Introductions                2

3. Approval of Agenda                                      3

4. Document Distribution                                   3

5. Old Business                                            3

5.1 Project Proposals Comment Resolution [94-019,94-022]

(Finch)                                               3

5.2 ATA-2 Technical Issues discovered during editing

sessions                                              3

5.3 ATA-2 Technical Review                            3

6. New Business                                            3

6.1 Termination Issues                                3

6.2 Check Power Mode (Heineman)                       3

7. Meeting Schedule                                        3

8. Adjournment                                             4

Results of Meeting

1.   Opening Remarks

Steve Finch convened the meeting at 9:00 am.  Steve thanked Ralph

Weber of DEC for hosting the meeting.

As  is customary, the people attending introduced themselves.   A

copy  of  the  attendance list was circulated for attendance  and

corrections.

It  was stated that the meeting had been authorized by X3T10  and

would  be conducted under the X3 rules.  Ad hoc meetings take  no

final  actions, but prepare recommendations for approval  by  the

X3T10 task group.  The voting rules for the meeting are those  of

the  parent  committee, X3T10.  These rules are:   one  vote  per

company; and any participating company member may vote.

The  minutes of this meeting will be posted to the SCSI  BBS  and

the  ATA  Reflector  and will be included in the  next  committee

mailing.

2.   Attendance and Membership, Introductions

Attendance at working group meetings does not count toward

minimum attendance requirements for X3T10 membership.  Working

group meetings are open to any person or company to attend and to

express their opinion on the subjects being discussed.

The following people attended the meeting.

Richard Kalish      Adaptec             rkalish@rahul.net

Lawrence J. Lamers  Adaptec             ljlamers@adaptec.com

Ken Scherzinger     Amphenol Spectra

Strip

Dennis Pak          Apple Computer

John Lohmeyer       AT&T/NCR            john.lohmeyer@ftcollins

Microelectronics    co.ncr.com

Joseph Chen         Cirrus Logic        chen@cirrus.com

Ed Gardner          DEC

Wayne Buckholdt     DPT

Dal Allan           ENDL                250-752@mcimail.com

Robert Liu          Fujitsu

Rene Macare         Future Domain       renem@fdc.mhs.compuserv

e.com

Mark Raymond        Future Domain

Nancy Cheng         Hitachi Computer    n.chen@hitachi.com

Products

Motoyasu Tsunoda    Hitachi Micro       mtsunoda@hmsi.hitachi.c

Systems             om

David McFadden      Honda

Dan Colegrove       IBM                 colegrove@vnet.ibm.com

John Sceible        IBM

Geoffrey Barton     Iomega

Dean Wallace        Linfinity

Ray Heineman        Maxtor              ray_heineman@maxtor.com

Peter Brown         Oak

Mike Carpenter      Qlogic              m_carpenter@qlc.com

Jim McGrath         Quantum Corp        JMCGRATH@QNTM.COM

John Masiewicz      Seagate Technology  masiewicz@notes.seagate

.com

Gene Milligan       Seagate Technology  milligan@notes.seagate.

com

Gene Milligan       Seagate Technology  Gene_Milligan@notes.sea

gate.com

Stephen Finch       Silicon Systems     5723283@mcimail.com

Paul Aloisi         Unitrode            Aloisi@uicc.com

Integrated Circuit

Tom Hanan           Western Digital     t_hanan@dt.wdc.com

Duncan Penman       Zadian

Technologies

3.   Approval of Agenda

The agenda was approved as proposed.

4.   Document Distribution

Copies of  ATA-2 Proposed Working Draft revision 2e was

distributed.

5.   Old Business

5.1  Project Proposals Comment Resolution [94-019,94-022] (Finch)

Revised proposals were reviewed and modified.  These proposals

reflected the agreement reached at the prior working group

meeting to establish two projects, one for ATA-3 disk and one for

ATAPI.  The working group unanimously agreed that the chair ask

X3T10 to replace the previous four ATA-3/ATAPI related project

proposals with these new, revised project proposals and go

throught the appropriate approval processing.

5.2  ATA-2 Technical Issues discovered during editing sessions

Ray Heineman noted that the phrase `the device shall enter the

idle mode.' in 8.11 par 3 to `the device shall enter the standby

mode'.  This error will be corrected.

5.3  ATA-2 Technical Review

Jim McGrath moved and John Masciewicz seconded that ATA-2

revision 2e be recommeded for acceptance as the working draft for

project 948D.  Passed 13:0.

6.   New Business

6.1  Termination Issues

Tom Hanan stated that implementation of mode 4 in many cases

benefits from use of series termination on the device.

Jim McGrath moved and Tom Hanan seconded that the ATA Working

Group consider a proposal for an informative annex for the ATA-2

standard be considered at the next Working Group meeting on

August 30th, 1994 providing that the proposed informative annex

be posted to the ATA reflector prior to August 17, 1994 and be

made available in hard copy at the meeting on August 30, 1994.

Passed 9:4.

6.2  Check Power Mode (Heineman)

Ray Heineman presented a proposal for check power mode

enhancements.

7.   Meeting Schedule

August 31 - September 1, 1994 in Milpitas, CA

September 15, 1994 in Houston, TX

October 19, 1994 in Longmont, CO

November 10, 1994 in Palm Springs, CA

Please note that the two day meeting will be in two locations.

August 31, 1994 will be at the IBM San Jose facility and the

September 1, 1994 meeting will be hosted by Adaptec.

8.   Adjournment

The meeting adjourned at approximately 12:00 PM.

_______________________________

*Operating under the procedures of The American National

Standards Institute.

X3 Secretariat, Computer and Business Equipment Manufacturers

Association (CBEMA)

1250 Eye Street NW, Suite 200, Washington, DC 20005-3922

Telephone: 202-737-8888 (Press 1 twice) FAX: 202-638-4922 or 202-

628-2829

From ata-owner  Wed Jul 27 09:53:15 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18912; Wed, 27 Jul 94 09:53:15 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA18905; Wed, 27 Jul 94 09:53:08 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA00331; Wed, 27 Jul 1994 12:52:53 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9407271250.tn241940@aol.com>

To: ata@dt.wdc.com

Date: Wed, 27 Jul 94 12:50:39 EDT

Subject: ATA-3 Project Proposal

Sender: ata-owner@dt.wdc.com

Precedence: bulk

July 27, 1994

-----------------------------------------------------------------------------

X3T10/94-019r4

Remember to edit Header A (Shft-F8,P,H,A,E)

Project Proposal

For a New

X3 Standard

AT Attachment - 3 Interface

(ATA-3)

July 21, 1994

IDENTIFICATION OF PROPOSED PROJECT

TITLE:  AT Attachment - 3 Interface (ATA-3)

PROPOSER:  X3T10

DATE SUBMITTED:  September 15, 1994

PROJECT TYPE:  D - Development of standards within an X3 TC.

JUSTIFICATION OF PROPOSED STANDARD

NEEDS:

The standard for AT Attachment has been completed and the draft

for the ATA Extensions (ATA-2) is near completion.  It has become

evident in the past few months that the industry desires further

improvements in this interface.  ATA-3 is needed to continue the

evolution of the ATA interface.

RECOMMENDED SCOPE OF STANDARD:

The proposed ATA-3 standard maintains a high degree of

compatibility with the ATA-2 interface and migrates the interface

to meet tomorrow's needs in an evolutionary way.  ATA-3 is

primarily intended to be a low-cost, efficient disk drive

interface.

The goals of ATA-3 are:

a)   reduced complexity by removing unused functions and

features.

b)   enhance compatibility through improved documentation of

registers, commands, and protocol.

c)   investigate increased transfer rates.

d)   address issues related to data integrity at higher

transfer rates.

e)   allow ATA and ATA-2 devices to exist on an ATA-3 physical

cable.

EXISTING PRACTICE IN AREA OF PROPOSED STANDARD:

The proposed project involves evolutionary expansion of the draft

ATA-2 standard to provide additional capabilities.

EXPECTED STABILITY OF PROPOSED STANDARD WITH RESPECT TO

CURRENT AND POTENTIAL TECHNOLOGICAL ADVANCE:

The nature of the proposed project is to ensure that the AT

Attachment has an upward, compatible growth path.  This will

ensure that current investments in AT Attachment are provided

with more stability in the face of technological developments.

DESCRIPTION OF PROPOSED PROJECT:

TYPE OF DOCUMENT:  Standard.

DEFINITION OF CONCEPTS AND SPECIAL TERMS:  None.

EXPECTED RELATIONSHIP WITH APPROVED X3 REFERENCE MODELS:

The ATA-3 standard is for use in closed systems.

RECOMMENDED PROGRAM OF WORK:

The following program of work is planned for the ATA-3 standard:

1)   Solicit continuing participation by the present AT

Attachment participants through X3T10 procedures and new

participants through press releases.  Invite comments by end-

user organizations and invite proposals from organizations

that may have a contribution to a viable ATA-3 standard.

2)   Establish functional requirements for ATA-3 functional

additions along with downward compatibility requirements.

3)   Prepare a draft proposed standard based on proposals

submitted and other information gathered during the initial

investigation.

4)   Consider the results of ATA-3 testing as may be available

to the committee through the voluntary efforts of the various

participants in X3T10.

5)   Submit the draft proposed standard to X3 for further

processing.

RESOURCES - INDIVIDUALS AND ORGANIZATIONS COMPETENT IN

SUBJECT MATTER:

The current membership of X3T10 includes representatives from all

parts of the computer industry from semiconductor chip

manufacturers to large mainframe system manufacturers as well as

Government agencies.  The members of X3T10 have expressed their

desire to participate and cooperate in the development of this

proposed standard.

There are sufficient resources to complete the development of

this standard without delaying work on other standards.

RECOMMENDED X3 DEVELOPMENT TECHNICAL COMMITTEE:

It is recommended that the development work be done in Technical

Committee X3T10 which is responsible for the ATA Extensions

project.

ANTICIPATED FREQUENCY AND DURATION OF MEETINGS:

Technical Committee X3T10 meets bi-monthly. Specific task ad hoc

groups are called as may be required for one to three days

between the regular meetings but their results are not binding.

TARGET DATE FOR dpANS TO X3 (MILESTONE 10):  December 1995

ESTIMATED USEFUL LIFE OF STANDARD:

It is anticipated that this standard will have a life of over 5

years.

IMPLEMENTATION IMPACTS

IMPACT ON EXISTING USER PRACTICES AND INVESTMENTS:

The proposed ATA-3 standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

IMPACT ON SUPPLIER PRODUCTS AND SUPPORT:

The proposed ATA-3 standard will provide an upward growth path

complementary to the existing practices and investments.  It is

likely that any isolated negative impacts would occur in any case

through non-standard evolution or revolution.

TECHNIQUES AND COSTS FOR COMPLIANCE VERIFICATION:

The committee will consider the results of ATA-3 testing as may

be available to the committee through the voluntary efforts of

the various participants in X3T10.  With this method all costs

are borne by the organizations of the various participants and

have for the most part been mainly an adjunct of their normal

development costs.

LEGAL CONSIDERATIONS:

There are no known legal considerations.  A Call for Patents will

be made under Milestone 7 of the SD-2.

CLOSELY RELATED STANDARDS ACTIVITIES

EXISTING STANDARDS:  None.

X3 STANDARDS DEVELOPMENT PROJECTS:

AT Attachment Interface -- Project 791-D

ATA Extensions -- Project 948-D

X3 STUDY GROUPS:  None.

OTHER RELATED DOMESTIC STANDARDS EFFORTS:  None.

ISO/IEC JTC 1 STANDARDS DEVELOPMENT PROJECTS:

This standard will be considered for proposal to JTC1/SC25/WG4.

OTHER RELATED INTERNATIONAL STANDARDS DEVELOPMENT PROJECTS:

None.

RECOMMENDATIONS FOR COORDINATING LIAISON:  None.

RECOMMENDATIONS FOR CLOSE LIAISON:  None.

From ata-owner  Thu Jul 28 06:31:12 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA13289; Thu, 28 Jul 94 06:31:12 PDT

Received: from maxtor.com (zeus.maxtor.com) by dt.wdc.com (4.1/SMI-4.1)

id AA13282; Thu, 28 Jul 94 06:31:09 PDT

Received: from sanjose3.maxtor.com ([134.6.34.200]) by maxtor.com (4.1/SMI-4.1)

id AA10415; Thu, 28 Jul 94 06:33:50 PDT

Message-Id: <9407281333.AA10415@maxtor.com>

Received: by sanjose3.maxtor.com with VINES ; Thu, 28 Jul 94 06:27:49 PDT

Date: Thu, 28 Jul 94 06:26:57 PDT

From: Ray=Heineman%ENG12%MCO@sanjose3.maxtor.com

Subject: Meeting Notices

To: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

July 28, 1994

*************************************************************************

*******************   M E E T I N G   N O T I C E S  ********************

*************************************************************************

SUBJECT:	ATAPI Study Group Meeting (August 30th,1994)

ATA-2 Working Group Meeting (August 31st & September

1st,1994)

DATE:		8/30, 8/31 & 9/1/94

TIME:		9:00 AM - 5 PM

PLACE:		Maxtor Corporation

251 River Oaks Parkway

San Jose, CA  95134

(408) 432-1700 (ask for BLDG 251 Lobby if you are lost)

HOST:		Ray Heineman

*************************************************************************

Steve Finch is chairing the ATA-2 meeting.

Tom Hanan is chairing the ATAPI meeting.

Ray Heineman is hosting the meetings.

DIRECTIONS:

FROM San Jose Airport:

There are two exits from the airport - the main exit

and one that goes to Coleman Avenue.

Use the main exit/entrance from the airport.

Turn left onto Guadalupe Parkway. Stay to the right.

Take the North First Street exit off Guadalupe.

Turn left (north) at North First Street.

Turn right (east) at River Oaks Parkway.

Go through the intersection with Zanker Road; Maxtor

will be on your left about a 1/4 mile beyond the

intersection.

Bldg 251 is the one in the center. Come to that lobby.

FROM San Francisco Airport:

Take Highway 101 Expressway south.

Take the 237 east exit.

Turn right (south) at North First Street.

Turn left (east) at River Oaks Parkway.

FROM Oakland Airport:

Take 880 south.

Take Montague west exit.

Turn right at North First Street.

Turn right at River Oaks Parkway.

HOTELS:

There are many choices, the most convenant are:

Sheraton Hotel

Milpitas

Hiway 880 & Montague Expressway

Holiday Inn

Milpitas

Hiway 880 & Hiway 237

************************************************************************

Regards

Ray Heineman

Maxtor Corporation

Phone 303 678 2591

Fax 303 678 2308

E-mail ray_heineman@Maxtor.com

From ata-owner  Fri Jul 29 15:36:34 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22464; Fri, 29 Jul 94 15:36:34 PDT

Received: from gate.cmd.com by dt.wdc.com (4.1/SMI-4.1)

id AA22457; Fri, 29 Jul 94 15:36:32 PDT

Received: by gate.cmd.com (5.57/UMCP-Hack/05-27-92)

id AA05516; Fri, 29 Jul 94 15:29:32 -0700

Date: Fri, 29 Jul 94 15:29:32 -0700

From: sam@cmd.com (Sam Chien-shin Lin)

Message-Id: <9407292229.AA05516@gate.cmd.com>

To: ata@dt.wdc.com

Subject: Bug in ATA-2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

On page 70 of ATA-2 (X3T9.2/0948D Revision 2c), minimum 8-bit DIOR-/DIOW-

Pulse Width (t2) is listed as 290ns for Mode 2.  However, the Mode 2

cycle time is only 240ns.  The cycle time is greater than the active

time, and the recovery time (t2i) is -50ns!

What is the proper minimum recovery time (t2i)?

Sam Lin

Senior Software Engineer

CMD Technology, Inc.

1 Vanderbilt

Irvine, CA  92718

714-454-0800 x230

FAX 714-459-9654

From ata-owner  Mon Aug  1 09:48:01 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04733; Mon, 1 Aug 94 09:48:01 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA04726; Mon, 1 Aug 94 09:47:58 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id ag19438; 1 Aug 94 11:57 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 1 Aug 94 09:54:31 MDT

Subject: Correction to X3T10 Minutes

To: ATA Reflector <ata@dt.wdc.com>,

SCSI Reflector List <scsi@WichitaKS.NCR.COM>

Date: Mon, 1 Aug 94 9:56:17 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9408011157.ag19438@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Rev 0 of the X3T10 minutes incorrectly state that there was no P1285 report.

I've corrected rev 1 as follows:

12.5    IEEE P1285 Status [Pak]

Dennis Pak provide an electronic report (see 94-155).

Sorry about the confusion,

John

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Fri Aug  5 23:32:19 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10962; Fri, 5 Aug 94 23:32:19 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA10955; Fri, 5 Aug 94 23:32:07 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id XAA27398; Fri, 5 Aug 1994 23:30:32 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 05 Aug 1994 13:33:57 EDT

Date:         Fri Aug 05 13:33:57 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e427806.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: ATA WG Aug. 31 - Sep. 1

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I understood we had agreed to finish the ATA-2 review first. Consequently I

strongly urge that topics 5.1 and 5.2 be reversed.

Gene Milligan

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\244.MSG

### Org Date:    08-05-94 01:21:59 PM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: ATA WG Aug. 31 - Sep. 1

### Attachments: none

From ata-owner  Sun Aug  7 07:55:44 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA16159; Sun, 7 Aug 94 07:55:44 PDT

Received: from mail02.prod.aol.net by dt.wdc.com (4.1/SMI-4.1)

id AA16150; Sun, 7 Aug 94 07:55:40 PDT

Received: by mail02.prod.aol.net

(1.38.193.5/16.2) id AA09226; Sun, 7 Aug 1994 10:55:38 -0400

From: LJLamers@aol.com

X-Mailer: America Online Mailer

Message-Id: <9408071055.tn742263@aol.com>

To: ata@dt.wdc.com

Date: Sun, 07 Aug 94 10:55:37 EDT

Subject: Alternate Member Letter Ballot Notice

Sender: ata-owner@dt.wdc.com

Precedence: bulk

8/5/94

----------------------------------------------------------------------------

TO: Alternate Members of X3T10

RE: Letter Ballots in Progress

Two letter ballots are in progress; one for the ATA-3 project proposal

and one for the ATAPI project proposal.  Principal members have been

sent their letter ballots along with a copy of each project proposal.

The balloting period closes August 29, 1994 at 12:00 noon.  All ballots

should be returned to John Lohmeyer, Chair of X3T10.

All comments on the project proposals should be attached to your letter

ballot.  Do not post your comments on the reflector as this will

invalidate your companies vote.  John has requested that he receive

the comments electronically either via an email to him or a posting

on the SCSI BBS.

The project proposals have been posted to the ATA reflector.  The

proposals and a copy of the ballots will be in the next X3T10

mailing.

Regards,

Lawrence J. Lamers

Adaptec                    TEL: 408-957-7817

691 Milpitas Bvld          FAX: 408-957-7193

Milpitas, CA   95035       EMAIL: LJLAMERS@AOL.COM

From ata-owner  Mon Aug  8 13:33:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA21637; Mon, 8 Aug 94 13:33:47 PDT

Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

id AA21624; Mon, 8 Aug 94 13:32:14 PDT

Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.7/8.6.4) with ESMTP id NAA02283 for <ata@dt.wdc.com>; Mon, 8 Aug 1994 13:23:48 -0700

Received: from zebra.unix.portal.com (zebra.unix.portal.com [156.151.1.103]) by portal.unix.portal.com (8.6.5/8.6.5) with ESMTP id NAA27861 for <ata@dt.wdc.com>; Mon, 8 Aug 1994 13:23:41 -0700

Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id NAA19937 for ata@dt.wdc.com; Mon, 8 Aug 1994 13:05:21 -0700

Received:  from endl by netmail.com (UUPC/extended 1.12j) with UUCP

for ata@dt.wdc.com; Mon, 08 Aug 1994 12:18:48 -0700

Received:  by endl.com (UUPC/extended 1.12b);

Mon, 08 Aug 1994 12:15:37 pdt

Message-Id: <2e468459.endl@endl.com>

Date:      Mon, 08 Aug 1994 12:15:37 pdt

From: "Dal Allan" <dal@endl.com>

To: ata@dt.wdc.com

Subject:   New_agenda_item

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:  ATA reflector

Subject:  Working Group Agenda: Proposed New Item

The following is from Gene Milligan's ISO report to the X3T10 plenary.

> Concerning the failed ATA project proposal the other countries

> indicated that there was no need for international standards for

> interfaces internal to a cabinet. WG 4 decided not to request Fast

> Track.

Secondly, and not reported in the minutes is that John said he would not

forward ATA unless the ATA working group directed him to. We met on the

morning of the plenary and this matter was not raised, and that would have

been the right forum to discuss it, prior to X3T10 beginning.

Thirdly, I want to see ATA as an ISO standard, and the present system has

rejected it twice. Once as a New Work Item because there were not enough

countries interested and again at the latest meeting of SC25/WG4 when they

decided not to Fast Track the ANSI standard.

The following is from the plenary minutes.

> Gene stated his opposition to sending our standards through ECMA because it

> prohibits small companies from participating in the international standards

> approval process.  Tom Hanan raised an ISO 9001 concern that standards like

> ATA must be recognized by ISO.  Gene said that ANSI standards are sufficient

> for ISO 9001.  John said that ISO 9001 does not prevent the use of national

> standards.

Gene is dead wrong in his statement.

The proposal I made to X3T10 and to X3T11 does not involve any companies

having to become members of ECMA. Free is definitely not 'prohibitive' by

anybody's measurement.

On the other hand, there are over 60 members of X3T10 and 40 members of

X3T11 paying $300/year to ANSI for international representation and we can't

even get our standards accepted as work items or for Fast Track. That comes

to over $30,000 a year by my calculations and getting so little for it makes

that repugnant if not prohibitive.

There are no costs involved in sending ATA to ECMA as an experiment to see

whether we want to work with ECMA. This is something I am pursuing to come

up with an alternative and faster way for American interface standards to

become ISO standards.

If the folks on this reflector want a copy of the proposal as presented to

X3T10 distributed, let me know. If there is wide enough response, I will

broadcast it on the reflector.

Bottom Line: Do the active participants in ATA want to see ATA standards

become ISO standards?

We certainly need to discuss this sometime so that the ATA working group can

give direction to John Lohmeyer.

I ask that a maximum time of 1 hour be set aside at the 8/31-9/1 working

group to discuss this matter and suggest a specific start time be named.

First thing in the morning of 8/31 or 9/1 would be preferred as some of

those who have an interest in this subject will either not have time to sit

through both days waiting for the subject or cannot offer the technical

insight on ATA-2 that a review entails.

Dal

From ata-owner  Tue Aug  9 17:06:06 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA26099; Tue, 9 Aug 94 17:06:06 PDT

Received: from uunet.UUCP by dt.wdc.com (4.1/SMI-4.1)

id AA26064; Tue, 9 Aug 94 17:05:58 PDT

From: uunet!ccmgate1bb.ftcollinsco.NCR.COM!dsnyder

Received: from ncrhub1.NCR.COM by relay1.UU.NET with SMTP

id QQxcgs12353; Tue, 9 Aug 1994 19:42:52 -0400

Received: from ncrcolo by ncrhub1.NCR.COM id aq17904; 9 Aug 94 17:31 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 9 Aug 94 14:50:25 MDT

Received: from cc:Mail by ccmgate1bb.FTCollinsCO.NCR.COM

id AA776465591 Tue, 09 Aug 94 13:53:11 MDT

Date: Tue, 09 Aug 94 13:53:11 MDT

Message-Id: <9407097764.AA776465591@ccmgate1bb.FTCollinsCO.NCR.COM>

To: uunet!dt.wdc.com!ata

Subject: Re: New_agenda_item

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Please remove me from your distribution list. Thankyou

Dennis Snyder

NCR

Colorado Springs

dennis.snyder@ftcollinsco.ncr.com

From ata-owner  Wed Aug 10 17:14:03 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA25844; Wed, 10 Aug 94 17:14:03 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA25837; Wed, 10 Aug 94 17:13:57 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id bh18010; 10 Aug 94 14:36 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 10 Aug 94 12:25:48 MDT

Subject: ATA Rev 4c available

To: ATA Reflector <ata@dt.wdc.com>

Date: Wed, 10 Aug 94 12:25:13 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9408101436.bh18010@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

ASCII copies of ATA Rev 4c are available for download from the SCSI BBS

(719-574-0424) and for anonymous ftp from ncrinfo.ncr.com.  This file

includes all editorial changes, but otherwise is technically the same

as ATA Rev 4a, the last revision distributed to the committee.

Available 'flavors' are:

Name        Size   Description

______________________________________________________________________________

ata-r4c.ibm 188786 ATA Rev 4c (8-bit ASCII--includes line drawing chars)

ata-r4c.ibz 48107  ATA Rev 4c (ZIP'd 8-bit ASCII--includes line drawing chars)

ata-r4c.txz 47729  ATA Rev 4c (ZIP'd plain 7-bit ASCII)

ata-r4c.txt 188777 ATA Rev 4c (plain 7-bit ASCII)     [Only flavor on ftp site]

Paper copies from ANSI should be available soon.  When ordering from ANSI

reference X3.221-1994.

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Thu Aug 11 09:06:41 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14289; Thu, 11 Aug 94 09:06:41 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-3.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA14281; Thu, 11 Aug 94 09:06:33 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id ae20375; 11 Aug 94 12:06 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 11 Aug 94 10:05:53 MDT

Subject: Should there be an ISO ATA standard?

To: ATA Reflector <ata@dt.wdc.com>

Date: Thu, 11 Aug 94 10:01:40 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

In-Reply-To: <2e468459.endl@endl.com>; from "Dal Allan" at Aug 8, 94 12:15 pm

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9408111206.ae20375@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I am trying to stay neutral on the issue of whether ATA should become an

ISO standard.  Frankly, I have no particular reason to support or not

support this activity.  The real question we should be addressing is

whether going through this effort will help solve any problems.

Gene indicated that the SC25/WG4 members were quite happy to use the

ANSI ATA standard.  They only need ISO standards when the standard affects

box-to-box connections.  They were not rejecting ATA -- only the

duplication of effort when it is not necessary.

As for ISO 9001 requiring usage of ISO standards, I believe this is not

true.  9001 deals with ensuring compliant organizations have quality

systems and use them.  I am following up on this point with our ISO 9001

coordinator.

Given these circumstances, why should X3T10 want to pursue an international

version of the ATA standard?  We would be giving up some measure of control

over the document with no identifiable gains.  All the arguments about

ANSI's International Program Fee and whether or not ECMA participation

is free really are not relevant until we can answer this question.

As for the experimental value of sending ATA to ECMA, X3T11 is already

conducting this experiment with one of their documents.  Does X3T10 need to

conduct a second experiment?

Just for the record, when Dal talks about directing me to send a document

to ECMA, he means in my role as a member of a company that is an ECMA

member.  I cannot send it to ECMA as chair of X3T10.  I am not even sure

whether I can send it as John Lohmeyer.  I would have to find out who is

representing AT&T on ECMA and ask him/her to sent it to ECMA.

I request that if the ATA working group does address this topic, that they

focus on identifying why we should pursue an international version of the

ATA standard.  Please do not spend a lot of time on the mechanics of how

to get it there.  I'm sure that the committee officers can find a way if

we can clearly state the needs.

John

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Thu Aug 11 21:12:41 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29601; Thu, 11 Aug 94 21:12:41 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA29594; Thu, 11 Aug 94 21:12:31 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id cm19379; 11 Aug 94 14:59 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 11 Aug 94 12:01:02 MDT

Subject: September '94 X3T10 Meeting Information

To: SCSI Reflector List <scsi@WichitaKS.NCR.COM>,

ATA Reflector <ata@dt.wdc.com>

Date: Thu, 11 Aug 94 12:00:47 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9408111459.cm19379@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

September '94 Meeting Map

(7/21/94 Version)

Monday 9/12        9   10  11  12  1   2   3   4   5   6   7   8

+===========================================+

| SCSI CAM Working Group                    |  (25)

+===============================+-----------+

| SSA electrical interface WG   | SSA UIG   |  (30)

+================+==============+-----------+

| XOR commands SG|  MMC WG      |              (20)

+================+==============+

Tuesday 9/13       9   10  11  12  1   2   3   4   5   6   7   8

+===========================================+

|    X3T10.1                                |  (40)

+===================+=======================+

|      SCSI-3 WG        |  (40)

+===================+=======================+

| SCC & RAB Joint Meeting       |              (40)

+===============================+

Wednesday 9/14     9   10  11  12  1   2   3   4   5   6   7   8   9

+===============================================+

| SCSI-3 WG                                     |  (45)

+===============================================+

Thursday 9/15      9   10  11  12  1   2   3   4   5   6   7   8   9

+===============+===============================+

| ATA WG        | X3T10 Plenary                 |  (40/80)

+===============+===============================+

Friday 9/16    8   9   10  11  12  1   2

+===========----------+

| X3T10 (*) / SFF     |                          (80)

+===========----------+

(*) If X3T10 meeting runs until Friday, Small Form Factor will follow

the X3T10 meeting; Otherwise, SFF starts at 8:30 am.

Meetings marked with == are X3T10-authorized meetings.

Meetings marked with -- are authorized by other groups and may change

without notice.

Meeting Map Abbreviation Key:

ATA:              AT Attachment (a.k.a. IDE)

CAM:              Common Access Method

Gen Edit Sess:    General Editing Session

El:               Electrical

I/F:              Interface

LB:               Letter Ballot

P1394:            IEEE High Performance Serial Bus (not an X3 activity)

Plenary:          Full Committee Meeting

RAB:              RAID Advisory Board (not an X3 activity)

RAID:             Redundant Array of Independent Devices

SCC:              SCSI-3 Controller Commands (RAID)

SFF:              Small Form Factor Industry Group (not an X3 activity)

SG:               Study Group

SSA:              Serial Storage Interface

SSP:              Serial Storage Protocol (Protocol for SSA)

UIG:              User Interest Group (not an X3 activity)

WG:               Working Group (same as Ad Hoc Group)

X3T10:            Lower Level Interfaces Technical Committee (was X3T9.2)

X3T10.1:          SSA Task Group of X3T10 (was X3T9.7)

##############################################################################

Meeting Announcement

Accredited Standards Committee Meetings

(X3T10, X3T10.1, and SFF Meetings)

SEPTEMBER 12-16, 1994

HOSTED BY:      Compaq Computer

LOCATION:       Hotel Sofitel

425 N. Sam Houston Pkwy.

Houston, TX 77060

Ph:  (713) 445-9000  or (800) 231-4612

Fax: (713) 445-0629

ROOM RATE:      $95

GROUP NAME:     ANSI X3T10

CUT-OFF DATE:   August 11, 1994

DIRECTIONS:     The hotel is near Houston Intercontinental Airport --

ask for driving directions when you rent your car.

Hotel shuttle bus is available.

HOST CONTACT:   Bill Galloway

internet: billg@bangate.compaq.com

##############################################################################

Draft Agenda X3T10 Meeting #5                          X3T10/94-006r0

Houston, TX September 15-16, 1994                        August 2, 1994

1.  Opening Remarks

2.  Approval of Agenda

3.  Attendance and Membership

4.  Approval of Minutes - July Meeting Bedford, NH (X3T10/94-150r1)

5.  Document Distribution

6.  Call for Patents

7.  Review of Old Action Items

8.  Subgroup and/or Project Status Reports

8.1     Project Status Summary [Lohmeyer]

8.2     X3T10.1 Task Group Report

8.3     SCSI-2 Status {Project 375-M} [Lamers]

8.4     SCSI Common Access Method (CAM) {Project 792-D} [Dallas]

8.5     AT Attachment (ATA) Status {Project 791-D} [Lamers]

8.6     ATA Extensions (ATA-2) Status {Project 948-D} [Finch]

8.7     SCSI-3 Architecture Model (SAM) Status {Project 994-D} [Monia]

8.8     SCSI-3 Parallel Interface (SPI) Status {Project 855-D} [Lamers]

8.9     SCSI-3 Interlocked Protocol (SIP) Status {Project 856-D} [Penman]

8.10    SCSI-3 Generic Packetized Protocol (GPP) Status {Project 991-D}

[Stephens]

8.11    SCSI-3 Serial Bus Protocol (SBP) Status {Project 992-D} [Marazas]

8.12    SCSI-3 Fiber Channel Protocol (FCP) Status {Project 993-D}

[Snively]

8.13    SCSI-3 Primary Commands (SPC) Status {Project 995-D} [Weber]

8.14    SCSI-3 Block Commands (SBC) Status {Project 996-D} [Tewell]

8.15    SCSI-3 Stream Commands (SSC) Status {Project 997-D} [Lappin]

8.16    SCSI-3 Graphics Commands (SGC) Status {Project 998-D} [open]

8.17    SCSI-3 Medium Changer Commands (SMC) Status {Project 999-D}

[Oetting]

8.18    Directly-Addressable Device Interface (DADI) Status {Project

964-D} [open]

8.19    SCSI Common Access Method -2 (CAM-2) Status {Project 990-D}

[Dallas]

8.20    SCSI-3 Controller Commands (SCC) Status {Project 1047-D} [Penokie]

8.21    Multimedia Command Set (MMC) Status {Project 1048-D} [Harris]

8.22    SCSI-3 Fast-20 Parallel Interface (Fast-20) {Project ____-D}

[Lamers]

9.  Old Business

9.1     Technical Information Bulletins

9.2     Resolution of FCP Letter Ballot Comments

9.3     Resolution of SAM Letter Ballot Comments

9.4     Resolution of SBP Letter Ballot Comments

9.5     CAM Forwarding for Second Public Review

9.6     X3T10 Procedures [Lamers]

9.7     Annual Publication of Documents [Milligan]

9.8     Fast-20 Node Capacitance 20 pF vs. 25 pF

10. New Business

10.1    Resolution of ATA-3 Letter Ballot Comments (94-165)

10.2    Resolution of ATAPI Letter Ballot Comments (94-166)

11. ISO [Milligan]

12. Liaison Reports

12.1    X3T11 Liaison Report [Cummings]

12.2    SFF Industry Group [Allan]

12.3    IEEE P1394 [Marazas]

12.4    IT8 [Lohmeyer]

12.5    IEEE P1285 Status [Pak]

12.6    PCMCIA Report [McLean]

12.7    AIIM C21 [Podio]

13. Review of Action Items

14. Meeting Schedule

14.1    Authorization of Working Group Meetings

15. Adjournment

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Thu Aug 11 21:55:35 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00623; Thu, 11 Aug 94 21:55:35 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA00529; Thu, 11 Aug 94 21:54:19 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id VAA24434; Thu, 11 Aug 1994 21:49:29 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 11 Aug 1994 15:47:18 EDT

Date:         Thu Aug 11 15:47:17 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e4a8047.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: New_agenda_item

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Dal sent out a message which mixes up facts and misunderstandings. Since I am

featured in the misunderstandings of fact or procedures I guess I should

respond to set the record straight:

>The following is from Gene Milligan's ISO report to the X3T10 plenary.

> Concerning the failed ATA project proposal the other countries

> indicated that there was no need for international standards for

> interfaces internal to a cabinet. WG 4 decided not to request Fast

> Track.

That is a fact. However it is abbreviated in terms of the  impact of the

decision. WG 4 has no decision. There seems to be an implication from the rest

of Dal's discussion that they rejected fast track for ATA. That is not

something in their power to decide. The decision to submit for fast track is

strictly up to an authorized proposer. Their decision was that they would not

request that ANSI submit it for fast track.

>Secondly, and not reported in the minutes is that John said he would not

>forward ATA unless the ATA working group directed him to. We met on the

>morning of the plenary and this matter was not raised, and that would have

>been the right forum to discuss it, prior to X3T10 beginning.

That is a misunderstanding or an omission which can be misinterpreted. John

(Lohmeyer) has forwarded  ATA and it is being published as an ANSI standard.

However undoubtedly Dal was referring to something to do with ISO or ECMA. If

it is ISO we don't forward but that is close enough. To propose it for a CD we

need a project approval from ISO/IEC JTC 1. We proposed the project to them.

They did not vote it down but there were not five countries who volunteered to

participate in its development. So in effect it died for lack of support and

not for opposition. The second way we can get an ISO standard voted on is to

recommend that ANSI submit it for fast track. They can do this after it is a

published standard of a Member Body of ISO or IEC.  If something is submitted

for Fast Track it then goes out for a DIS letter ballot. There is some

dissension at the moment on what happens, according to the procedures, after

the DIS ballot. It is clear that if all Member Bodies vote Yes without comment

that the standard is then sold as an ISO/IEC standard. However if there are No

votes and Comments the debate starts. Our past practice and presumption is that

at that point the DIS is processed as it would have been if it had come through

the non-fast track path. The other interpretation of the procedures is that the

vote is strictly up/down. If the vote passes regardless of comments it is

published exactly the same as the submitted standard. If the vote fails it is

not published as an ISO/IEC standard. (I think we are better off not seeking a

definitive ruling on this until such time that someone enforces a voted down

ruling because there is precedent for continuing with changes to reach an

acceptable IS. The pre-ruling, especially without a case at hand has the danger

of adding support of the up/down theory.)

The issue was raised long ago, I think at the Newport meeting and there have

been numerous working groups  since that could have discussed it. I don't

actually agree that a technical working group is the correct forum but I would

not object.

>Thirdly, I want to see ATA as an ISO standard, and the present system has

>rejected it twice. Once as a New Work Item because there were not enough

>countries interested and again at the latest meeting of SC25/WG4 when they

>decided not to Fast Track the ANSI standard.

At this point the correct way to do that Dal, would be to make a motion at

X3T10 that upon publication of the ANSI Standard that we recommend that ANSI

submit it to JTC 1 for fast tracking. However in order for the motion to pass,

I think it would be beneficial for you to provide the information on why you

want ISO/IEC to publish it in spite of the indication by Denmark, Germany, and

Japan that there was no need for an International Standard. The other thing to

consider the Fast Track voting takes place completely asynchronously with any

meetings. Consequently it would be necessary to have reasons for correspondence

in an attempt to have the DIS letter ballot not be voted down. As I pointed

out, SC25/WG4 has not rejected ATA any times let alone two times.

The following is from the plenary minutes.

> Gene stated his opposition to sending our standards through ECMA because it

> prohibits small companies from participating in the international standards

> approval process.  Tom Hanan raised an ISO 9001 concern that standards like

> ATA must be recognized by ISO.  Gene said that ANSI standards are sufficient

> for ISO 9001.  John said that ISO 9001 does not prevent the use of national

> standards.

That is a fact from several accounts. I made a statement close to that and the

minutes does include it.  It is a presumption however on my part that small

(US) companies would not want to pay multiple thousands of dollars to join ECMA

and participate in the meetings in Europe. I omitted an additional reason

however. The additional reason is that I am concerned that ISO/IEC JTC 1 may

view this as flaunting the process and react negatively on other projects we

have with them. This is a delicate period in that we are attempting to get the

participation issue ruled in our favor regarding Fibre Channel and some

additional HIPPI projects. If we don't we do have numerous members, presumably

the same ones who participate in ECMA, who should be able to influence cou

ntries like France and England to check the box that they will participate. The

participation can be by Email and by responding to CD and DIS ballots (which

they would need to do for fast track).

>Gene is dead wrong in his statement.

I don't agree. But I am open to hearing where I am wrong.

>The proposal I made to X3T10 and to X3T11 does not involve any companies

>having to become members of ECMA. Free is definitely not 'prohibitive' by

>anybody's measurement.

I agree you don't have to become members. But that is not participating. Free

is not participating.

From ata-owner  Sat Aug 13 23:21:33 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA26622; Sat, 13 Aug 94 23:21:33 PDT

Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

id AA26615; Sat, 13 Aug 94 23:21:30 PDT

Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.7/8.6.4) with ESMTP id XAA08295 for <ata@dt.wdc.com>; Sat, 13 Aug 1994 23:21:27 -0700

Received: from zebra.unix.portal.com (zebra.unix.portal.com [156.151.1.103]) by portal.unix.portal.com (8.6.5/8.6.5) with ESMTP id XAA21119 for <ata@dt.wdc.com>; Sat, 13 Aug 1994 23:21:25 -0700

Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id WAA01546 for ata@dt.wdc.com; Sat, 13 Aug 1994 22:32:55 -0700

Received:  from endl by netmail.com (UUPC/extended 1.12j) with UUCP

for ata@dt.wdc.com; Sat, 13 Aug 1994 15:27:40 -0700

Received:  by endl.com (UUPC/extended 1.12b);

Sat, 13 Aug 1994 15:23:14 pdt

Message-Id: <2e4d47d2.endl@endl.com>

Date:      Sat, 13 Aug 1994 15:23:14 pdt

From: "Dal Allan" <dal@endl.com>

To: ata@dt.wdc.com

Subject:   ATA_ISO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:  ATA Members

Subject:  ECMA

Well, the storm that descended on my email came as a surprise.

In the May plenary when the proposal to work with ECMA was made, a straw

poll supported the idea of processing one of our standards as an experiment.

There were two suitable candidates, ATA and CAM. Gene liked the idea of CAM

and John demurred on ATA because of a concern there might be copyright

issues. My memory is that there was little interest in doing so with CAM,

and that ATA was the preferred choice if there were no copyright issues. It

was learned later that there are no copyright issues.

The July plenary lasted so long that I went to get something to eat before

the ISO/ECMA agenda item and returned after it completed (looks like I have

to start eating lunch on plenary days).

From secondhand reports and the July minutes, I learned that Gene had raised

objections that had been covered in the original proposal and that John

would do nothing with ATA unless the working group directed him to.

The purpose of my email was to find out if this subject of making ATA an ISO

standard is worth pursuing, not start a finger-pointing war with the present

officers. It's not that long since I was an officer of X3T9.2 and it is

tough enough without hostilities. That was not the intent, and if it was

taken as such by John and Gene then I apologize for the misunderstanding.

Until this year I felt it would be a conflict of interest to pursue some

alternative method of accelerating the ISO process. That is no longer an

issue, hence the proposals made in May and June to X3T10 and X3T11.

X3T10 and X3T11 are separate committees with separate membership and

separate officers. The idea of the straw poll in May was to find out if

X3T10 wanted to participate in forwarding a document to ECMA.

The fact that X3T11 is conducting an experiment does not change the poll in

X3T10 in May that favored processing an X3T10 document through ECMA as a

trial to see how the process might work.

In his reply, John stated correctly that there are two issues here and we

should not mix them up.

The first is whether members want ATA to be an ISO standard or not. That was

why time was requested on the ATA working group agenda.

Gene objected in his email to my word usage:

> ..... SC25/WG4 has not rejected ATA any times let alone two times.

ATA has been discussed at WG4 twice.

- As a New Work Item which failed for lack of support.

- When WG4 felt there was no need to have it as an ISO standard.

Maybe 'rejection' was the wrong word, but the result is still the same: no

action on ATA as an ISO standard.

Unless the participants in ATA want ATA as an ISO standard there is no point

in arguing about this subject further. At this time, I have seen no answer

on whether it will be on the agenda at the end of this month or not.

The second is whether rocking the boat on processing ISO standards is worth

doing. Obviously, I think so.

John and Gene both commented about how to use the present system. My reason

for pursuing an alternative is that the present system has not worked well

for us in the past, so a change is worth trying. I have no vested interest

in ECMA, and I don't know if working with ECMA will be any better than ANSI

and JTC1/SC25. I don't see how it could be any worse.

I don't even know how ECMA wants to work the issues of a relationship with

X3T10 and X3T11, that is a decision for ECMA. Remember, this was a proposal

for a trial, not a commitment. Both sides have to figure things out.

However, as stated in the May proposal there is the potential of:

1. More meeting opportunities to take action on international standards.

2. Participation in international activities available to all members.

3. Direct editorial support from the ECMA support organization.

4. Lower costs for processing of ISO standards.

As to why I called Gene's comments made in the plenary about the proposal

'dead wrong', I stand by them. The May proposal covered the fact that small

companies were not required to join ECMA to participate as ECMA meetings

held in the United States were to be open to all members of X3T10 and X3T11.

.pm1

.lm3

Quote from Gene's email of 8/11:

It is a presumption on my part that small (US) companies would not want

to.....participate in the meetings in Europe.

Quote from proposal to X3T10 in May 1994:

Although ECMA has European in its name, meetings are not restricted to

being in Europe and holding meetings in the United States is a consid

-------------------------------------

eration. Many many years ago, during SCSI-1 development there was a joint

meeting of ISO (then under Dara Hekimi) in Washington DC.

It would be possible for every member to become more involved with ISO

standards, if ECMA met once per year with X3T10 (in an odd month) and once

----------------------------------------------

with X3T11 (in an even month). We would need an International Represent-

ative to attend the General Assembly meetings in Geneva twice a year.

Quote from Gene's email of 8/11:

It is a presumption on my part that small (US) companies would not want

pay multiple thousands of dollars to join ECMA.....

Quote from proposal to X3T10 in May 1994:

Jan indicated that ECMA would be interested in supporting our activities,

which raises the question of why. First, ECMA supports itself by member-

ship fees, and there is a strong possibility that more companies will join

ECMA if interface activities were included in its scope of ISO standard-

ization. Secondly, I suggested that the direct costs for holding meetings

in the United States that we can all attend be covered by the members

-----------------

(this cost is estimated to be in the ballpark of $100/member/year).

.lm1

On the first item as to whether members want to see ATA sent to ISO or not,

we can only wait to find out if it makes the ATA working group agenda.

The second item on the general issues of processing documents is something

that we can cover at the next plenary.

Dal

From ata-owner  Tue Aug 16 10:35:30 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA03926; Tue, 16 Aug 94 10:35:30 PDT

Received: from io.org by dt.wdc.com (4.1/SMI-4.1)

id AA03919; Tue, 16 Aug 94 10:35:26 PDT

Received: from r-node.io.org by io.org with smtp

(Smail3.1.28.1 #1) id m0qaSPq-000u40C; Tue, 16 Aug 94 13:35 WET DST

Date: Tue, 16 Aug 1994 13:35:20 -0400 (EDT)

From: Myles White <myles@io.org>

Subject: External ATA connection

To: ata@dt.wdc.com

Message-Id: <Pine.3.89.9408161332.D12275-0100000@r-node.io.org>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Sender: ata-owner@dt.wdc.com

Precedence: bulk

While not part of the official ATA/ATAPI working group, I've been on the

reflector for several months now, following research into the new ATA

standards.

One of the most common reactions I get from the user community to a

discussion of ATA/ATAPI v/s SCSI is disappointment over the lack of an

external device connection.

People appreciate the higher hard drive capacity, the improved

throughput, the promise of less expensive devices, the inclusion of

CD-ROM and the promise of tape. Plug-and-play is a big feature, but the

tone of the conversation changes quickly when it is revealed that CD-ROM

and tape will have to remain as internal devices.

Is anyone in the working group thinking about adding an external

connector to the ATA/ATAPI standard? If not, is the difficulty technical,

financial or political?

Myles White       | e-mail: myles@io.org   | Freelance writer

Assistant Editor  | CIS:    75570,2744     | Events Coordinator

Toronto Computes! | voice:  (416) 465-0852 | Showfest Productions

Toronto, Canada   | fax:    (416) 778-5492 | (Computer Fest shows)

From ata-owner  Wed Aug 17 12:17:46 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA05341; Wed, 17 Aug 94 12:17:46 PDT

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA05334; Wed, 17 Aug 94 12:17:42 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA01791; Wed, 17 Aug 1994 12:11:06 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qaq8h-000gJTC; Wed, 17 Aug 94 12:55 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 17 Aug 1994 12:00:40 EDT

Date:      Wed, 17 Aug 1994 12:00:40 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e523428.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   address/phone changes

Sender: ata-owner@dt.wdc.com

Precedence: bulk

FYI...  I have moved to Colorado and have new addresses and phone

numbers...

Hale Landis

PO Box 204

Niwot, CO 80544

Phone 303-530-2198 (use until 2 Sep 94)

Phone 303-530-2580 (use after 2 Sep 94)

Email landis@sugs.tware.com

--

------------------------------------------

Hale Landis                Boulder, CO USA

landis@sugs.tware.com         303-530-2580

------------------------------------------

From ata-owner  Wed Aug 17 21:21:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA16820; Wed, 17 Aug 94 21:21:26 PDT

Received: from io.org by dt.wdc.com (4.1/SMI-4.1)

id AA16813; Wed, 17 Aug 94 21:21:20 PDT

Received: from nudge.io.org by io.org with smtp

(Smail3.1.28.1 #1) id m0qayyT-000u4hC; Thu, 18 Aug 94 00:21 WET DST

Date: Thu, 18 Aug 1994 00:19:55 -0400 (EDT)

From: Myles White <myles@io.org>

Subject: re:External ATA connection

To: curtis_stevens@bannet.ptltd.com

Cc: ata@dt.wdc.com

In-Reply-To: <E706+,fIIia@ptlnor3.ptltd.com>

Message-Id: <Pine.3.89.9408180053.C8338-0100000@nudge.io.org>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Thanks - as guessed, several folks have provided similar answers -- i.e.,

an external IDE (ATA/ATAPI) connector can't or isn't desirable to be done

because of the cable length restriction due to the termination

problem, and a suspicion that if these things were solved, all you'd have

is basically another flavour of SCSI (I'm oversimplifying) with less

performance and compatibility--and virtually no cost savings.

Today.

One of the things I've learned about the computer industry over the

years, however, is that if there's a need for something (and particularly

if there's an appetite for it), there are scads of bright people who will

figure out a way to do whatever is required. Usually all that's needed is

a suggestion that the "whatever" may be desirable.

It's also occurred to me that while I may not be able to contribute any

technical expertise to these discussions (as already demonstrated in our

earlier correspondence, I am NOT and engineer), I may be able to

contribute some philisophical points.

Given the tone of the replies I've had so far (they've all been gentle

and positive) and keeping in mind that you folks are busy, I may still

chime in from time to time.

Okay?

Myles White       | e-mail: myles@io.org   | Freelance writer

Assistant Editor  | CIS:    75570,2744     | Events Coordinator

Toronto Computes! | voice:  (416) 465-0852 | Showfest Productions

Toronto, Canada   | fax:    (416) 778-5492 | (Computer Fest shows)

From ata-owner  Mon Aug 22 15:14:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA05974; Mon, 22 Aug 94 15:14:49 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA05965; Mon, 22 Aug 94 15:14:38 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id ai02458; 22 Aug 94 15:43 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 22 Aug 94 13:39:42 MDT

Subject: Volume and File Structure Standardization Meeting

To: SCSI Reflector List <scsi@WichitaKS.NCR.COM>,

ATA Reflector <ata@dt.wdc.com>

Date: Mon, 22 Aug 94 13:40:47 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

Cc: Lynn Barra <lbarra@cbema.org>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9408221543.ai02458@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Lynn Barra sent me the attached meeting notice for a meeting on

standardizing the Volume and File Structure on removable media.

John

#############################################################################

!!!  VFS MEETING NOTICE -- SEPTEMBER 12, 1994  !!!

I would like to invite anyone interested in data interchange and

storage on removable media to attend an open industry-wide

meeting on future "Volume and File Structure" (VFS) standardization

efforts.

We are exploring the interest level and need for development of

additional national and international VFS standards and the

feasibility of forming an ANSI Accredited Technical Committee (TC)

for Volume and File Structure.

The proposed VFS TC would combine the optical media responsibility

of Task Group X3B11.1 and the magnetic media responsibility of the

X3B5 VFS Ad hoc (and possibly the CD-ROM responsibility currently

under NISO).  This VFS TC would also have US TAG responsibility for

ISO/IEC JTC 1 SC15.

The meeting will address both the requirements for standards and

the expected participation needed for formation of a new Technical

Committee.

The meeting will be held in Tacoma, WA on September 12, 1994 in

the Executive Board Room of the Sheraton Tacoma from 1:00 p.m. to

6:00 p.m.

Sheraton Tacoma

1320 Broadway Plaza

Tacoma, WA 98402

phone: 206-572-3200

fax:	206-591-4105

A limited number of rooms are available under the group ID:

"AMERICAN NATIONAL STANDARDS INSTITUTE" at a rate of $75

single and $85 double plus tax.  Please use this ID and register ASAP.

Directions from Seattle/Tacoma Airport:

I-5 to exit 133 (City Center)

Highway 705 toward (City Center) downtown, Exit at "A" Street

"A" Street to "11th" Street, Left onto "11th" Street

"11th" Street (2 blocks) to Broadway, Left onto Broadway(2 blocks).

Any individuals or organizations who have an interest or are

materially affected are invited to participate.  Please contact the

chairman of X3B11.1 as stated below.

Mr. Jim Owens

Chairman X3B11.1

M/S 019-72B

Wang Laboratories Inc.

1 Industrial Ave.

Lowell, MA 01851

phone: 508-967-1790

fax:   508-967-2210

email: owens@wiis.wang.com

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Tue Aug 23 16:36:59 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06991; Tue, 23 Aug 94 16:36:59 PDT

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA06863; Tue, 23 Aug 94 16:35:28 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id QAA26283; Tue, 23 Aug 1994 16:02:30 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qd4KP-000gJTC; Tue, 23 Aug 94 16:28 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Tue, 23 Aug 1994 16:19:14 EDT

Date:      Tue, 23 Aug 1994 16:19:13 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5a59c2.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>,

"ATAPI Reflector"    <atapi@dt.wdc.com>

Subject:   PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Tony Pione of IBM posted the following message to the ATAPI

reflector.  I'm sending my follow up to the ATAPI and ATA

reflectors.  The text of Tony's message is preceeded by ">"

characters.

>  I would like to touch on the pre-fetch function definition next

>  week at our meeting.  There are many pre-fetching controllers out

>  there that aren't real compatible with ATAPI CD-ROM.

Yep, no surprise to me here.  It has been my opinion all along

that ATA and ATAPI were two separate interfaces with little in

common.  A close investigation of ATAPI would probably reveal

that it is very incompatible with ATA in a lot of areas and it

will become even more incompatible as ATA evolves to ATA-3.  I

fully expect that ATA-3 will require some level of intelligence

in the host adapter.  We are starting to see this today with

ATA-2 devices and new ATA PIO Mode 3+ host adapters.  It is this

low level of intelligence we see today -- the sector prefeching

for example -- that Tony has commented on here.

But just wait, the high performance ATA host adapter people will

probably come up with even more goodies that will mess up ATAPI.

ATAPI may have started as a cheap way to attach a non-ATA device

to the usually unused second drive plug on a PC's ATA host

adapter.  And this may work with an ATA-1 hard disc and an ATAPI

device on an old style ATA-1 host adapter.  Unfortunately for the

ATAPI folks, systems with ATA-1 host adapters are dinosaurs.  A

good(?) system design today would include an ATA-2 host adapter

for the hard disc and a second ATA-1 host adapter for the ATAPI

(and perhaps other ATA-1 style devices).  Two host adapters for

two different interfaces.

>  I propose that we put forth some design requirements for

>  pre-fetching controllers that won't interfere with ATAPI data

>  xfers.

Hmmm...  This may not be easy.  Most of the folks building these

host adapters don't come to the ATA meetings (and in some cases

don't even have a copy of the ATA document -- I've talked to some

of them!).  Plus, what you are talking about here may be the

functional equivalent of a CAM effort -- an effort to define how

a host adatper should interface with the host and how it should

handle various types of requests.  Indeed, that may be

appropriate for ATAPI, since one of the things ATAPI is, is

another attempt to figure out a cheap way to attach SCSI devices

to a PC.  I don't think ATA, at this time, needs such a effort --

ATA-3 may well need such an effort, but I hope not.

>  We know of one problem where the controller assumes 512 byte

>  xfers and doesn't handle the raw sector format well because of

>  it.  An extra erroneous word gets transferred between successive

>  single sector requests with this controller.

>  This is really more of an ATA global issue.

I don't think so (or I hope it doesn't become an ATA global

issue).  I really believe that we are moving to the day when

there is a single ATA device per host adapter, especially for ATA

hard discs and especially on higher performance systems.  A

single device per host adapter not because that is what ATA-3+

requires but because it solves so many of the existing ATA

problems, such as overlapped commands, master/slave issues

(especially if the ATA drive is configured as drive 1), data

transfer speed mismatch problems, and others, while remaining in

the whole completely ATA compatible.

On the other hand, if this really is an ATA global issue, then it

may well be the second nail in the coffin for ATA (over 528MB is

the first nail).  This could finally make ATA as complex,

difficult to design, implement, configure and expensive to the

point that SCSI became an attractive alternative for PC systems.

Of course that would also spell the end for ATAPI too -- why have

a SCSI subset when you can have the real thing for a similar

cost?

[Oh boy, I'm sure this is going to stir up some interesting

mail!]

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Tue Aug 23 19:41:59 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10940; Tue, 23 Aug 94 19:41:59 PDT

Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA10932; Tue, 23 Aug 94 19:41:56 PDT

Received: with SMTP-MR; Tue, 23 Aug 1994 19:36:30 PST

Mr-Received: by mta WDUS04; Relayed; Tue, 23 Aug 1994 19:36:30 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Wed, 24 Aug 1994 02:58:00 PST

From: "Rich Rutledge (714) 932-7205 [TEL]" <RUTLEDGE_R@A1.WDC.COM>

Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

To: "ata-owner@dt.wdc.com" <ATA-OWNER@dt.wdc.com>

Cc: ATA Reflector <ATA@dt.wdc.com>, ATAPI Reflector <ATAPI@dt.wdc.com>

Message-Id: <01HG9DR9LXRO8ZEFG3@wdus03.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Wed, 24 Aug 1994 03:36:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;03639132804991/541312@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hale,

We all have our own opinions and outlooks regarding a variety of matters.

With regards to IDE/ATA, my outlook is that SFF/X3T10 are chartered to

develop specifications/standards that satisfy the requests of the

customers.  It often appears that some folks assume their role is telling

the customers what they can or can not do.  My suggestion is to listen to

what the customers want and that is more low cost IDE functionality.

Having said this, the customers generally like the cost effective

functionality that IDE/ATA has long delivered for disk drives.  Lets face

it, cost is king in the mainstream PC market.  Just look at how succesful

Compaq has been with their low price strategy.  It should come as no

surprise to anybody that a need existed to extend IDE's capabilities.  The

528 Mbyte barrier was long ago overcome on AT bus ESDI controllers via

translating BIOS (did we all forget 760 Mbyte ESDI drives that too required

the 528 Mbyte barrier to be broken?).  SCSI adapters have always had to

translate LBA to Int 13 CHS.  This one was easy and hardly a nail in a

coffin.

Our video brothers deserve special thanks for driving the needs for local

bus as this has allowed us to free our disk drives from the demands of the

ISA bus.  Once local bus became a mainstream feature in new systems, it was

again pretty straightforward to exploit it.  We all know that the CPU is

more effective at doing 32 bit PIO transfers, so little surprise that the

LSI devices that connect IDE to local bus (Appian, Adaptec, CMD, Intel,

Opti, PC Tech, others) take advantage of this fact.  Doing so requires the

device to initiate its own IDE cycle, thus pre-fetch.  Now the chip needs

to know when to stop pre-fetching.  This is the issue raised by Tony

Pionne.  Many chips pre-fetch using 512 byte transfer counters as this is

the disk block size.  This is not the case for CD-ROM and Tape.

So, this is an implementation detail whereby the local bus IDE

controller/system simply must be ATAPI aware and if so, then pre-fetch must

be disabled for either the ATAPI device or for the IDE channel that has the

ATAPI device as some LSI chips have programmable features on a device basis

while others have them on a channel basis.

This is done by either the BIOS or a pre-boot driver at boot time and is

commonly known as ATAPI AWARENESS.  It goes right along with the code that

takes a look at IDENTIFY DRIVE date for geometry and data transfer timing

parameters.  Disabling pre-fetch for ATAPI devices really doesn't do much

to performance as non disk peripherals have quite low data rate needs.

This is really an implementation issue between the BIOS/driver and the

hardware.

What really happened here is a failure for the industry to COOPERATE before

it jumped into the COMPETITIVE phase.  Wouldn't it be ideal if there was a

single, standard register set for IDE bridge controller silicon!  Alas,

this snake got out of the can before it could be controlled.  This same

mistake was not repeated for PCI DMA IDE, witness the register level

specification worked out by Intel/NCR/WD/others in ADVANCE of product

shipments.  This should dramatically facilitate low cost, bus mastered,

multi-channel IDE data transfers.  Perhaps the LSI device suppliers could

get together and author a SFF proposal.  My opinion is that this is too

late.

This lack of a register level standard is one of the motivators for the

Phoenix Enhanced Disk Drive specification which provides a STANDARD way to

determine the hardware capabilities offered by the system/controller.

The other missed opportunity was to agree on a standard translation

algorithm for >528 Mbyte IDE.  Everybody remember taking a drive formated

on a Vendor A host adapter and try to run it on a machine that has a Vendor

B host adapter?  Again, another problem that could have been elinated with

a little upfront cooperation.....which was scuttled at the pre-meeting

before the 11/93 Comdex High Capacity Storage Integration Conference when

it became obvious that already completed implementations prevented

agreement....too bad as this would have made the end-users live far more

pleasent.

The past 18 months has seen multiple instances of fast paced, upfront

widespread industry cooperation to allow long term benefit, specifically:

1. Plug and Play SCSI

2. SCAM

3. ATAPI CD-ROMs (SFF-8020 and SFF-8029)

4. Bus Master IDE (SFF 8036 I think)

5. ATAPI Tape (QIC 157)

6. Local Bus IDE Mode 3/1 (SFF 8011)

7. Local Bus IDE Mode 4/2 (SFF 8033)

My own calculations indicate that 1995 will see IDE overtake SA-450 (aka

floppy) as the highest volume peripheral interface as IDE disk + IDE CD-ROM

+ IDE tape will outship 3.5" floppy + 5.25" floppy + floppy tape.  The

sheer dollar and unit volume will continue to draw industry focus to IDE

for internal storage application needs.  Until something comes along that

is both cheaper and better, it will have a hard time entering the

mainstream PC marketplace.  Look for its capabilities to continue to

increase as the PC market is dominated by making the existing thing faster

and cheaper instead of moving to something entirely new.  Examples include

ECP, Enhanced IDE, Super VGA, Intel compatible CPUs, etc.

I believe that Centronics defined the "Centronics Parallel Port" in the

mid 60's and have been out of business since the mid '80s...yet it is still

very much alive after almost 30 years.  IDE evolved from the PC/AT hard

disk controller in 1984, thus it is 10 years into its live.  Time will tell

if it too lives as long as the Centronics Parallel Port.

Rich Rutledge

Western Digital

From ata-owner  Tue Aug 23 22:41:44 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14378; Tue, 23 Aug 94 22:41:44 PDT

Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

id AA14365; Tue, 23 Aug 94 22:41:40 PDT

Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.7/8.6.4) with ESMTP id WAA24600 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:37 -0700

Received: from zebra.unix.portal.com (zebra.unix.portal.com [156.151.1.103]) by portal.unix.portal.com (8.6.5/8.6.5) with ESMTP id WAA12113 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:34 -0700

Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id WAA16918 for ata@dt.wdc.com; Tue, 23 Aug 1994 22:02:02 -0700

Received:  from endl by netmail.com (UUPC/extended 1.12j) with UUCP

for ata@dt.wdc.com; Tue, 23 Aug 1994 21:37:15 -0700

Received:  by endl.com (UUPC/extended 1.12b);

Tue, 23 Aug 1994 17:44:44 pdt

Message-Id: <2e5a97fd.endl@endl.com>

Date:      Tue, 23 Aug 1994 17:44:45 pdt

From: "Dal Allan" <dal@endl.com>

To: ata@dt.wdc.com

Subject:   1996_calendar

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:       ATA Reflector

Fibre Channel Reflector

HIPPI Reflector

SCSI Reflector

SSA Reflector

Yes, I know some of you will get 5 copies of this email but the time has

come to plan for the 1996 meeting dates. Carl Zeitler proposed dates at

X3T11 last week, and I agreed to check them out against the overall calendar

for 1996.

Most of the months work well with meeting in the second full week, but there

are some conflicts which need to be considered. In the months which could be

a problem, the preferred week is indicated by 'x' days and the alternative

is noted by 'a' days.

In April, Carl suggested X3T11 meet the week of 4/15 which happens to be tax

time. Although Easter Sunday is not a holiday it is celebrated as a family

event, so that prevents meeting the week of April 8. The week of 4/1 is a

possible alternative, and if X3T10 chose the earlier week in March, there

would be an even spacing between X3T11 and the likely X3T10 calendar.

In July, X3T10 comes immediately after the July 4 weekend, so travel on

Sunday is not the greatest. Slipping it to a week later brings it within 2

weeks of the X3T11 meeting. We try to keep 3 weeks between them for the

benefit of those who attend both. Either X3T10 should not start until

Tuesday so everyone can take Monday flights, or X3T11 should slide a week.

Thanksgiving is the fourth Thursday of November so that puts X3T11 the week

after. Flying on Sunday of Thanksgiving is to be avoided. Holding X3T11 a

week earlier or a week later are not very practical choices. Maybe X3T11

should not start until Tuesday, so everyone can take Monday flights.

If anyone knows of more events in 1996 we should be aware of, or if any of

the holidays shown are incorrect, just let me know. Please send calendar

comments to me at 250-1752@mcimail.com rather than flooding the reflectors

like I have just done.

Telling me your any preferences for weeks is not much value, as choosing the

dates is a plenary decision. I will accumulate any responses and advise them

to the respective plenaries.

Volunteers to host will be welcomed by the chairs of both X3T10 and X3T11.

The 'best' months disappear fast, Western Digital has volunteered to host in

April 1996 and AMP has volunteered to host in October 1996.

January     *h *h  2  3  4  5  6         New Years

7  x  x  x  x  x *h  X3T10

*h *h 16 17 18 19 20         Martin Luther King

21 22 23 24 25 26 27

February    28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11 12 13 14 15 16 *h

*h *h 20 21 22 23 24         President's Day

March       25 26 27 28 29  1  2

3  a  a  a  a  a  9  X3T10a

10  x  x  x  x  x 16  X3T10

17 18 19 20 21 22 23

24 25 26 27 28 29 30

April       31  a  a  a  a  5 ee  X3T11a

ee  8  9 10 11 12 13         Easter

14  x  x  x  x  x 20  X3T11

21 22 23 24 25 26 27

May         28 29 30  1  2  3  4

5  x  x  x  x  x 11  X3T10

mm 13 14 15 16 17 18         Mother's Day

19 20 21 22 23 24 *h

June        *h *h 28 29 30 31  1         Memorial Day

2  3  4  5  6  7  8

9  x  x  x  x 14 15  X3T11

ff 17 18 19 20 21 22         Father's Day

23 24 25 26 27 28 29

July        30  1  2  3 *h *h *h         July 4

*h  x  x  x  x  x 13  X3T10

14  a  a  a  a  a 20  X3T10a

21 22 23 24 25 26 27

August      28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11  a  a  a  a 16 17  X3T11a

18 19 20 21 22 23 24

25 26 27 28 29 30 *h

September   *h *h  3  4  5  6  7         Labor Day

8  x  x  x  x  x 14  X3T10

15 16 17 18 19 20 21

22 23 24 25 26 27 28

October     29 30  1  2  3  4  5

6  x  x  x  x 11 12  X3T11

13 14 15 16 17 18 19

20 21 22 23 24 25 26

November    27 28 29 30 31  1  2

3  x  x  x  x  x  9  X3T10

10 11 12 13 14 15 16

17 18 19 20 21 22 23

h 25 26 27 *h *h *h         Thanksgiving

December    *h  x  x  x  x  6  7  X3T11

8  9 10 11 12 13 14

15 16 17 18 19 20 21

22 23 24 25 26 27 28         Christmas

29 30 31

From ata-owner  Wed Aug 24 07:32:34 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23934; Wed, 24 Aug 94 07:32:34 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA23927; Wed, 24 Aug 94 07:32:32 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id HAA19202 for <ata@dt.wdc.com@relay>; Wed, 24 Aug 1994 07:33:54 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 24 Aug 94 07:16:07 PDT

Date: 24 Aug 94 08:05:22 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: ATA and ISO

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"22505142804991/314817@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Dal Allan has requested that the ATA working group discuss the issue of

submitting the ATA (ATA-1) standard to ISO using ECMA Fast-Track.

Because we, the ATA working group, have agreed to keep the August 31 -

September 1 meeting focused on the technical review of ATA-2, I will place

this issue on the agenda for the ATA working group meeting on September 15

in Houston, Texas.  This will allow for discussion before the X3T10 plenary

meeting.

I hope this meets everybodies approval.

Steve Finch

Chair, ATA Working Group

Silicon Systems Inc.

From ata-owner  Wed Aug 24 10:21:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27485; Wed, 24 Aug 94 10:21:16 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA27478; Wed, 24 Aug 94 10:21:12 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA129358868; Wed, 24 Aug 1994 10:21:09 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA259168687; Wed, 24 Aug 1994 10:18:08 -0700

Message-Id: <n1434398720.44824@qm_smtpgw.qntm.com>

Date: 24 Aug 1994 10:17:25 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: ATA Working Group Agenda

To: "ATA" <ATA@dt.wdc.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  9:40 AM

OFFICE MEMO         ATA Working Group Agenda              Date:  8/24/94

I have not yet seen an agenda, and would like to add the

following discussion items/proposals:

1) We have been requested to provide the capability of

eliminating the need for the PIO mode of data transfer completely

in the future.  This requires that the data transfer commands

other than READ/WRITE (Identify drive, Read buffer, Read long,

Format, Write buffer, Write long) have a DMA mode of transfer.

To be consistent with the previous READ DMA/WRITE DMA

implementations, this would require new op codes for DMA

versions of these commands, which would otherwise execute in

the same manner as before.  Assignment of op codes is thus the

action requested.

2)  I will be offering two different ways of implementing command

queuing for ATA drives, and request feedback.  The key requirement of

queuing is to provide for tag passing and to provide an interlock

mechanism to allow sending subsequent commands down and not have

that activity interfere with IRQs and data transfer from the drive for

previously queued commands.  One recommendation uses register space

allocated by the AT architecture to the Hard Disk in the Divice Control

Block for both tag passing and interlocking.  The second requires that

a "tag command" be sent first (a new op code with the tag in a task file

register) to both set and interlock and pass a teg value for the next,

"real" command.  We would like to brainstorm these concepts towards

the goal of drafting a formal proposal.

3)  I would like to make sure we follow up on WD's suggestion of adding some

guidance to hosts in implementing local bus modes on their systems.

I do think this would be valuable, and are willing to work with others

either next week or at other meeting times/locations if people do not

believe this is an ATA-2 topic.

Jim

From ata-owner  Wed Aug 24 12:57:09 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01049; Wed, 24 Aug 94 12:57:09 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA01042; Wed, 24 Aug 94 12:57:07 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA146358223; Wed, 24 Aug 1994 12:57:03 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA056798042; Wed, 24 Aug 1994 12:54:02 -0700

Message-Id: <n1434389365.51283@qm_smtpgw.qntm.com>

Date: 24 Aug 1994 13:02:46 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Prefetch, local bus, etc...

To: "ATA" <ATA@dt.wdc.com>, "ATAPI" <ATAPI@dc.wdc.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  12:48 PM

OFFICE MEMO         Prefetch, local bus, etc...           Date:  8/24/94

At a higher level of concern, there is no industry forum to discuss issues

relating to hardware on the system side of the bus (ATA does not really

cut it).  I would like us to either establish a new group to do this, or

have an existing group take on the responsibility of working on

system chip/BIOS/OS issues.  Any takers?

Jim

From ata-owner  Wed Aug 24 17:38:28 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA07486; Wed, 24 Aug 94 17:38:28 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA07473; Wed, 24 Aug 94 17:36:56 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA23779; Wed, 24 Aug 1994 17:30:29 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdLhk-000gJTC; Wed, 24 Aug 94 11:01 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Tue, 23 Aug 1994 22:22:23 EDT

Date:      Tue, 23 Aug 1994 22:22:22 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5aaedf.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA rev 2F -- section 3.2.5

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I recommend that the two identical sentences in this section

be changed from

"For even values of n from 2 to 512."

to

"For even values of n starting at 2."

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 17:40:22 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA07552; Wed, 24 Aug 94 17:40:22 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA07490; Wed, 24 Aug 94 17:38:55 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA23869; Wed, 24 Aug 1994 17:32:38 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdLhm-000gJTC; Wed, 24 Aug 94 11:01 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Tue, 23 Aug 1994 22:30:02 EDT

Date:      Tue, 23 Aug 1994 22:30:01 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5ab0aa.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2f -- Section 5.2.4

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Again, I may have missed something but I thought that we were

going to put in some kind of statement warning host designers

against placing improper loading on the DASP- signal because such

loading can cause the master/slave handshaking to fail?

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 17:41:12 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA07572; Wed, 24 Aug 94 17:41:12 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA07565; Wed, 24 Aug 94 17:41:09 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA23950; Wed, 24 Aug 1994 17:34:14 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdLho-000gJUC; Wed, 24 Aug 94 11:01 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Tue, 23 Aug 1994 22:40:10 EDT

Date:      Tue, 23 Aug 1994 22:40:09 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5ab30a.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2f -- Section 6.1.1

Sender: ata-owner@dt.wdc.com

Precedence: bulk

In the third paragraph, the sentence

"When a single device is attached to the interface it shall be

set as Device 0."

should be deleted.

In the sixth paragraph, last sentence, the word "Typically"

is distrubing.  It seems to imply that the form of sequential

access to sectors on the media is optional.  I don't think

this form of addressing is optional.  I think the word

"Typically" should be deleted.

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 17:44:00 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA07635; Wed, 24 Aug 94 17:44:00 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA07627; Wed, 24 Aug 94 17:43:33 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA23814; Wed, 24 Aug 1994 17:31:28 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdLhl-000gJUC; Wed, 24 Aug 94 11:01 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Tue, 23 Aug 1994 22:27:40 EDT

Date:      Tue, 23 Aug 1994 22:27:40 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5ab01d.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- sections 4.2 and 4.3

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Sorry, I have not been following all the ATA activities as

closely as I should the last few months, but...

It seems to me that we had a discussion about sections 4.2 and

4.3 once upon a time and that we were going to move the 2.5" and

1.8" connection info from the annexes into this area of the

document.  This is a technical thing, but as currently written it

would appear that to be "ATA compatible", a device must use the

connectors defined in these sections which in the real world are

used only by 3.5" and larger drives.

Did I miss something?

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 17:56:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08035; Wed, 24 Aug 94 17:56:26 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08028; Wed, 24 Aug 94 17:56:17 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA24453; Wed, 24 Aug 1994 17:45:18 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdQdU-000gJTC; Wed, 24 Aug 94 16:17 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 24 Aug 1994 15:10:33 EDT

Date:      Wed, 24 Aug 1994 15:10:33 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5b9b2a.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 7.1

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Ahhh... Here it is again... That strange statement about

what Device 0 does when there is no Device 1...

I recommend that the sentence

If Device 0 does not detect DASP- during the reset sequence

above, then Device 0 shall respond to a PIO read of the status

Register of Device 1 with a value of 00h.

be replace by

If Device 0 does not detect DASP- during the reset sequence

above, then Device 0 shall respond to a write and read

operations of the Device 1 Command Block and Control Block

registers as described in section 6.2.14.

Note that section 6.2.14 is my proposed new section -- the section

number is subject to change.

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 17:58:23 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08056; Wed, 24 Aug 94 17:58:23 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08049; Wed, 24 Aug 94 17:58:20 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA24876; Wed, 24 Aug 1994 17:47:34 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdSMp-000gJTC; Wed, 24 Aug 94 18:08 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 24 Aug 1994 16:54:28 EDT

Date:      Wed, 24 Aug 1994 16:54:27 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5bb384.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 8 -- general comments

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Many months ago I said I would rewrite what is now section 8, the

command descriptions.  I said I would do this in the style I used

for the rewrite of the power management commands.

OK...  I'm doing it now.  Expect to see several messages, each

containing a part of section 8, probably 5-10 commands per

message.

And, my recent complaint that while my new power management

introduction stuff got into ATA-2, my new power management

command descriptions did not get into ATA-2, is true.  Well...

I'm going to fix that in my rewrite.

Are you ready?  (I'm not sure I am, but here goes...)

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 18:01:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08144; Wed, 24 Aug 94 18:01:49 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08137; Wed, 24 Aug 94 18:01:40 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA24051; Wed, 24 Aug 1994 17:36:07 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdLhp-000gJTC; Wed, 24 Aug 94 11:01 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 24 Aug 1994 00:06:16 EDT

Date:      Wed, 24 Aug 1994 00:06:16 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5ac739.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Sections 6.2.x

Sender: ata-owner@dt.wdc.com

Precedence: bulk

In section 6.2.3, the sentences

At the end of the command, this register is updated to

reflect the current cylinder number.

and

At the end of the command, this register is updated to

reflect the current LBA bits 16-23.

should probably be deleted or replaced.  A possible replacement

could be

At completion of media access commands, this register may be

updated to reflect the high order bits of the last cylinder

number accessed.

and

At completion of media access commands, this register may be

updated to reflect LBA bits 16-23 of the last LBA accessed.

-----

In section 6.2.4, the sentences

At the end of the command, this register is updated to reflect

the current cylinder number.

and

At the end of the command, this register is updated to reflect

the current LBA bits 8-15.

should probably be deleted or replaced.  A possible replacement

could be

At completion of media access commands, this register may be

updated to reflect the low order bits of the last cylinder

number accessed.

and

At completion of media access commands, this register may be

updated to reflect LBA bits 8-15 of the last LBA accessed.

-----

I recommend that Section 6.2.5 be replaced with the following:

The data register is either 8-bits or 16-bits depending on the

interface width currently selected and/or the type of data

being transferred by the current command.  This register is

used only by PIO data transfer commands.

-----

In section 6.2.6, in the SRST bit description I recommend that

the sentence

Device 1 is not required to execute the DASP- handshake

procedure.

But what about Device 0?  I recommend that this sentence be

deleted.

-----

In section 6.2.8, in the L=0 descriptions, the sentence

At command completion, these bits are updated reflect the

currently selected head.

should probably be deleted or replaced.  A possible replacement

could be

At the end of media access commands, these bits may be updated

to reflect the last head number accessed.

-----

In section 6.2.8, in the L=1 descriptions, the sentence

At command completion, these bits are updated reflect the

current LBA bits 24-27.

should probably be deleted or replaced.  A possible replacement

could be

At the end of media access commands, these bits may be updated

to reflect LBA bits 24-27 of the last LBA accessed.

-----

In section 6.2.9, I recommend that the descriptions of the MC and

MCR bits should be changed to:

This bit is reserved for use by removable media devices.  The

use and meaning of this bit is vendor specific.

-----

In section 6.2.11, the second paragraph is confusing.  It implies

that a host need only check this register to determine if a

command was successful which is not true -- the Status register

should be used to make this determination.  I recommend this

paragraph be replaced with:

For media access commands, this register shall be zero at the

completion of a command if there is no error indication in the

Status register.  For media access commands that complete with

an error indication in the Status register, this register

contains the number of sectors which need to be transferred in

order to complete the request.

-----

Section 6.2.12, is confusing with respect to the contents of the

register at command completion.  I recommend that this section

be replaced with:

For media access commands in CHS mode, this register contains

the starting sector number.  For media access commands in LBA

mode, this register contains bits 0-7 of the starting LBA.

This register is used by some non-media access commands to

pass command specific information to the device.

At the completion of media access commands in CHS mode, this

register may be updated to reflect the last sector number

accessed.  At the completion of media access commands in LBA

mode, this register may be updated to reflect bits 0-7 of the

last LBA accessed.  This register is used by some non-media

access commands to pass command specific to the host.

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 18:02:21 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08155; Wed, 24 Aug 94 18:02:21 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08147; Wed, 24 Aug 94 18:02:09 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA24397; Wed, 24 Aug 1994 17:43:30 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdQdT-000gJUC; Wed, 24 Aug 94 16:17 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 24 Aug 1994 12:29:06 EDT

Date:      Wed, 24 Aug 1994 12:29:06 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5b7553.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 6.2.13, etc

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Oh boy, its Status register time and of all the 6.2.x sections

this one is the biggest problem.  It is much less work if I just

propose a complete rewrite of this section and also propose a new

section, so here goes...

First, please note that I have deleted some of the references to

the old status being valid within 400ns of BSY=0 -- this *MUST*

go away -- an ATA device *MUST* make its status valid *BEFORE*

BSY=0 if host software people are going to be able to program

this interface on faster systems!

Here is my proposed rewrite of section 6.2.13.  Please read

this carefully -- thanks!

-----begin

6.2.13.  Status register

This register contains the device status.  The contents of this

register are updated to reflect the current state of the device

and the progress of any command being executed by the device.

When BSY=0, the other bits in this register are valid and the

other Command Block register may contain meaningful information.

When BSY=1, no other bits in this register are valid and no other

Command Block registers contain any meaningful information.

For devices that implement the Power Management features, the

contents of the Status register, and all other Command Block

registers, is unpredictable while a device is in the Sleep power

saving mode.

If the host reads this register when an interrupt is pending, it

is considered to be the interrupt acknowledge and the interrupt

is cleared.

+=======+=======+=======+=======+=======+=======+=======+=======+

|   7   |   6   |   5   |   4   |   3   |   2   |   1   |   0   |

|-------+-------+-------+-------+-------+-------+-------+-------+

|  BSY  | DRDY  |  DWF  |  DSC  |  DRQ  |  CORR |  IDX  |  ERR  |

+=======+=======+=======+=======+=======+=======+=======+=======+

- BSY (Busy) is set whenever the device has control of the

Command Block Registers.  When BSY=1, a read of any Command Block

Register shall return the contents of the Status register.

The device shall not change the state of the DRQ, CORR or ERR

bits unless BSY=1, except when the last sector of a PIO data in

command has been transferred by the host the DRQ bit is cleared

without the BSY bit being set.

The BSY bit is set by the device under the following

circumstances:

a) within 400 nsec after the negation of RESET- or after SRST has

been set in the Device Control register.

b) within 400 nsec after a write of the Command register.

c) during data transfer phase of PIO data in commands if there is

another sector or block to transfer.

d) during data transfer phase of PIO data out commands following

the transfer of a sector or block.

e) during the data transfer phase of DMA commands.

The BSY bit remains set until the reset activity is completed.

For commands, either the BSY bit must be set, or if the BSY bit

is cleared, the DRQ bit must be set, until command completion.

- DRDY (Device Ready) is set to indicate that the device is

capable of responding to and attempting to execute all possible

commands codes.  Devices that implement the power management

features maintain DRDY=1 when they are in the Idle and Standby

power saving modes.  When the state of the DRDY bit changes, it

shall not change again until after the host reads the Status

register.

During any time when DRDY=0 a device must respond as follows to

commands:

a) the device shall accept and attempt to execute the EXECUTE

DEVICE DIAGNOSTIC and INITIALIZE DEVICE PARAMETERS commands.

b) the device should reject all other commands codes by setting

the ABRT bit in the Error register and setting the ERR bit in the

Status register before clearing the BSY bit to signal command

completion.  If a device accepts commands other than EXECUTE

DEVICE DIAGNOSTIC and INITIALIZE DEVICE PARAMETERS during the

time DRDY=0, the results are vendor specific.

- DWF (Device Write Fault) indicates a write fault error has been

detected.  The internal status or internal conditions that causes

this error to be indicated is vendor specific.  When the state of

the DWF bit changes, it shall not change again until after the

host reads the Status register.

- DSC (Device Seek Complete) is set to indicate that the device

has completed all activities associated with a seek operation.  A

device may clear this bit at the start of a seek operation and

set this bit at the completion of a seek operation.  The SEEK

command normally consists of only a seek operation, but seek

operations may be part of the functions performed by other

commands.  If a device does not implement a seek operation or

does not indicate the status of seek operations, this bit shall

have the same value as the DRDY bit.  When the state of the DSC

bit changes, it shall not change again until after the host reads

the Status register.

- DRQ (Data Request) indicates that the device is ready to

transfer data between the host and the device.  The DRQ bit is

not used and remains cleared during the execution of non-data

transfer commands.  During execution of PIO data transfer

commands and DMA commands, either the BSY bit must be set, or if

the BSY bit is cleared, the DRQ bit must be set, until command

completion.

The state of the DRQ bit shall not change during PIO commands

except as follows:

a) During PIO data transfer commands when the device is prepared

to transfer data the DRQ bit shall be set before the BSY bit is

cleared.

b) During PIO data transfer commands when the host has

transferred the current sector or block and there is another

sector or block to transfer the BSY bit shall be set before the

DRQ bit is cleared.

c) Following the transfer of the last sector or block of a PIO

data in command, the DRQ bit shall be cleared while the BSY bit

also remains cleared.

The DRQ bit is cleared by a reset and by a host write to the

Command register.

- CORR (Corrected Data) indicates that a correctable data error

was encountered and that the data has been corrected by the

device.  This condition does not terminate the current command.

The conditions that cause the CORR bit to be set are vendor

specific.  The CORR may be set and cleared at any time during the

execution of a data transfer command, but if it is set one or

more times during the execution of a data transfer command, it

shall be set prior to the last time the BSY bit is cleared for

the command.  The CORR bit is cleared by a reset and by a host

write to the Command register.  It is recommended that host

software ignore this bit.

- IDX (Index) is vendor specific.

- ERR (Error) indicates that an error occurred during the

execution of a command and that the contents of the Error

register are valid.  The ERR bit shall remain set and the Error

register contents shall remain valid until cleared by a reset or

by a host write to the Command register.

-----end

I propose that the following new section be added.  I'm not sure

6.2.14 is the "correct" section number.  And I know many of you

are going to ask why this such a mess -- why are there two

methods?  Please refer to the strange little note near the

beginning of the existing section 6.2.13...

Note:  If Device 1 is not detected as being present, Device 0

clears the Device 1 Status register to 00H (indicating that

the device is Not Ready).

This strange little note is the *ONLY* comment in the document

that describes this condition.  This note has resulted in

implementations that worked as described by the second method.

-----begin

6.2.14 Single device configurations

In a single device configuration where device 0 is the only

device and the host selects device 1, device 0 may respond to

accesses of the Command Block and Control Block registers in one

of two methods.  These two methods exist because previous

versions of the ATA standard did not specify the required

behaviour for this configuration.

The first method requires that device 0 implement an Error,

Status and Alternate Status register that is used whenever device

1 is selected.  The first method is the recommended

implementation.  The first method is:

1) The device 1 Error, Status and Alternate status registers are

set to 00H by a reset.

2) A write to the Device Control register shall complete as if

device 0 was the selected device.

3) A write to the Command Block register, other than the Command

register, shall complete as if device 0 was selected.

4) A write to the Command command register with a command code

other than the INITIALIZE DEVICE PARAMETERS or EXECUTE DEVICE

DIAGNOSTICS command causes the device 1 Error, Status and

Alternative status registers to be used as follows:

a) the BSY bit is set in the device 1 status registers.

b) the ABRT bit is set in the device 1 Error register.

c) the ERR bit is set in the device 1 status registers.

d) the BSY bit is cleared in the device 1 status registers.

e) if the nIEN bit in the Device Control Register is cleared,

the INTRQ signal is asserted.

5) An EXECUTE DEVICE DIAGNOSTIC command is executed as if it

addressed to device 0.

6) An INITIALIZE DEVICE PARAMETERS command is executed as if

device 1 is present and is actually executing the command.

The command shall have no effect of the device parameters of device 0.

[Editors note: Why is this you ask?  Because this command is

*NOT* allowed to set the ERR bit in the Status register at any

time or for any reason!]

7) A read of the Control Block or Command Block registers, other

than the Status or Alternate Status registers, shall complete as

if device 0 was selected.

8) A read of the Error, Status or Alternate status register

returns the value in the device 1 copy of these registers.  The

device 1 status registers will contain 00H following a reset and

the value 01H following an attempt to execute a command, other

than EXECUTE DEVICE DIAGNOSTICS or INITIALIZE DEVICE PARAMETERS,

on device 1.

The second method is:

1) A write to the Device Control register shall complete as if

device 0 was the selected device.

2) A write to the Command Block register, other than the Command

register, shall complete as if device 0 was selected.

3) A write to the Command command register is ignored.

5) A read of the Control Block or Command Block registers, other

than the Status or Alternate Status registers, shall complete as

if device 0 was selected.

6) A read of the Status or Alternate status register returns the

value 00H.

In a single device configuration where device 1 is the only

device and the host selects device 0, device 1 shall respond to

accesses of the Command Block and Control Block registers in the

same way it would if device 0 was present.  This is because

device 1 can not determine if device 0 is, or is not, present.

-----end

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 18:02:48 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08169; Wed, 24 Aug 94 18:02:48 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08156; Wed, 24 Aug 94 18:02:36 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA24361; Wed, 24 Aug 1994 17:42:02 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdQdS-000gJTC; Wed, 24 Aug 94 16:17 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 24 Aug 1994 11:50:58 EDT

Date:      Wed, 24 Aug 1994 11:50:57 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5b6c62.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>,

"ATAPI Reflector"    <atapi@dt.wdc.com>

Subject:   Hale's response to Rich

Sender: ata-owner@dt.wdc.com

Precedence: bulk

After sending my response to Rick I noticed that he had copied the

ATA and ATAPI reflectors in his message.  So here is a copy of my

response to Rick also copied to the same reflectors.

-----

On first reading, I don't disagree with anything you said,

however, I think the technology needs of future high performance

(and high capacity) hard discs will push the ATA interface, in

the ATA-3 time frame, in directions that will be fundamentally

different and probably very incompatible with ATAPI.

The ATA hard disc interface is shipping in mass volumes and will

continue to do so for a long time.  This has and will continue to

drive changes in the interface at a speed that has been and will

continue to be difficult for any committee (especially the ATA

committee!) to keep up with.  Frankly, I don't hold much hope

that the committees will be able to keep up.

I do agree that it is possible to put ATA-2 and ATAPI

intelligence into a single host adapter chip and someone is

probably doing that today.  And host software can be made aware

that certain host chip features must be enabled/disabled on a

command-by-command basis.  I just don't think we will be able to

define a "standard" way for this without a committee effort that

will be as costly as CAM as been.

As for the >528MB ATA problem, I think our efforts, while timely

and good, have fallen on deaf ears.  I have not seen a BIOS yet

that implements the MS/IBM extensions or the Phoenix enhancement

in a shipping system.  What I have seen is many BIOSes, DOS

device drivers and Windows Fastdisk implementations that simply

don't work due to design flaws or bugs.  I think this is the

first nail in the ATA coffin because the SCSI host adapter folks

solved this problem years ago.  This happened because the SCSI

folks provide their own BIOS and in many cases their own device

drivers, effectively putting themselves in charge of the entire

interface, hardware and software.  The ATA device folks have

relied on the BIOS people to supply the driver support.  This has

not to date resulted in any "standard" implementations but has

resulted in an unknown number of implementations that would

probably have to be "grandfathered" into any proposed standard.

And what a mess that would be!  That is why I call such a

standards effort the second nail in the ATA coffin.

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 18:09:17 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA08389; Wed, 24 Aug 94 18:09:17 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA08130; Wed, 24 Aug 94 18:01:39 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id RAA24675; Wed, 24 Aug 1994 17:46:38 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdQdV-000gJUC; Wed, 24 Aug 94 16:17 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 24 Aug 1994 16:01:09 EDT

Date:      Wed, 24 Aug 1994 16:01:08 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5ba705.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 7.2

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Section 7.2, all two sentences of it, are the entire discussion

of translation modes in the ATA-2 document.  I think this topic

needs much more definition than these two sentences.  Therefore,

I propose the following changes/additions to this section.

First I recommend that the 5th, 6th, 7th and 8th paragraphs in

section 6.1.1 be deleted.  Currently section 6.1.1 starts out

talking about "addressing modes" with respect to the Command

Block and Control Block registers and then in the 5th paragraph

is switches to talking about sector addressing -- very confusing.

So here is my recommended rewrite of section 7.2.

-----begin

7.2 Sector addressing modes

All addressing of data sectors recorded on the device's media is

by a logical sector address.  The mapping of logical sector

addresses to the actual physical location of the data sector on

the media is vendor specific.

An ATA device shall support at least one logical CHS translation

mode known as the default translation mode.  The device shall

enter this translation mode following a reset.  A device may

support other logical translation modes and the host may use the

INITIALIZE DEVICE PARAMETERS command to select the default CHS

mode or any of the other supported CHS modes.  The default

translation mode is described in the Identify Device information.

The current translation mode may also be described in the

Identify Device information.

A CHS address is made up of three fields:  the sector address,

the head number and the cylinder number.  Sectors are numbered

from 1 to the maximum value allowed by the current CHS

translation mode but can not exceed 255.  Heads are numbered from

0 to the maximum value allowed by the current CHS translation

mode but can not exceed 15.  Cylinders are numbered from 0 to the

maximum value allowed by the current CHS translation mode but

cannot exceed 65,535.

When the host selects a CHS translation mode using the INITIALIZE

DEVICE PARAMETERS command, the host requests the number of

sectors per logical track and the number of heads per logical

cylinder.  The device then computes the number of logical

cylinders available in requested mode.

Sequential access to logical sectors shall be accomplished by

treating the sector number as the least significant portion of

the address, the head number as the middle portion of the address

and the cylinder number as the most sigificant portion of the

address.

A device may also support the optional LBA mode of addressing.  A

device that supports LBA mode indicates this in the Identify

Device information.  A host shall not attempt to use LBA mode

unless the device indicates the mode is supported.

If a device supports LBA addressing mode, then the following

shall be supported by the device:

1) The host may select either CHS addressing in the current

translation mode or LBA addressing on a command-by-command basis

by using the LBA bit in the Device/Head register.

2) A command must complete in the same addressing mode as

selected by the host at command initiation.

3) When LBA mode is selected by the host, the Sector Number

register, Cylinder Low register, Cylinder High register and bits

HS3-HS0 of the Device/Head register contain an LBA mode address.

4) LBA mode must be supported for all media access commands,

except for the FORMAT TRACK command.  The LBA bit of the

Device/Head register shall be ignored for commands that do not

access the media.

5) Logical sectors on the device shall be linearly mapped with

the first LBA addressed sector (sector 0) being the same sector

as the first CHS addressed sector (cylinder 0, head 0, sector 1).

Irrespective of the CHS translation mode currently in effect, the

LBA address of a given logical sector does not change.  The

following is always true (heads_per_cylinder and

sectors_per_track are the current translation mode values):

LBA = ( (cylinder * heads_per_cylinder + heads )

* sectors_per_track ) + sector - 1

-----end

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 18:36:58 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA09107; Wed, 24 Aug 94 18:36:58 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA09039; Wed, 24 Aug 94 18:36:52 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA171288610; Wed, 24 Aug 1994 18:36:51 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA265578428; Wed, 24 Aug 1994 18:33:49 -0700

Message-Id: <n1434368978.77462@qm_smtpgw.qntm.com>

Date: 24 Aug 1994 18:43:36 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: ATA Rev 2f -- Section 6.

To: "ATA" <ATA@dt.wdc.com>, "Hale Landis" <landis@sugs.tware.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>ATA Rev 2f -- Section 6.1.1

>  In the sixth paragraph, last sentence, the word "Typically"

>  is distrubing.  It seems to imply that the form of sequential

>  access to sectors on the media is optional.  I don't think

>  this form of addressing is optional.  I think the word

>  "Typically" should be deleted.

Hale,

I assume the current wording is designed to cover defect management

and the possibility of surface skewing, both of which are (and should

remain) valid implementations.

Jim

From ata-owner  Wed Aug 24 23:30:22 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15162; Wed, 24 Aug 94 23:30:22 PDT

Received: from netcomsv.netcom.com (uucp6.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA15157; Wed, 24 Aug 94 23:30:17 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id XAA07308; Wed, 24 Aug 1994 23:30:16 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdY3g-000gJUC; Thu, 25 Aug 94 00:13 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Thu, 25 Aug 1994 00:16:29 EDT

Date:      Thu, 25 Aug 1994 00:16:28 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5c1b1d.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 8 (Part 2/4)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Command descriptions -- part 2 or 4.

-----begin

8.11. IDLE

TYPE - Optional - Power Management Feature Set.

PROTOCOL - Non-data command.

INPUTS - The value in the Sector Count register when the IDLE

command is issued shall determine the time period programmed into

the Standby Timer.  See Table 13.

+---------------------+------------------------------+

| Sector Count        | Corresponding                |

| Register contents   | Timeout Period               |

+---------------------+------------------------------+

|   0       (00h)     | Timeout Disabled             |

|   1 - 240 (01h-F0h) | (value * 5) seconds          |

| 241 - 251 (F1h-FBh) | ((value - 240) * 30) minutes |

| 252       (FCh)     | 21 minutes                   |

| 253       (FDh)     | Vendor unique period between |

|                     | 8 and 12 hours               |

| 254       (FEh)     | Reserved                     |

| 255       (FFh)     | 21 minutes 15 seconds        |

+---------------------+------------------------------+

Table 13 - Automatic Standby Timer Periods

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command causes the device to set BSY, enter

the Idle Mode, clear BSY, and assert INTRQ.  INTRQ is asserted

even though the device may not have fully transitioned to Idle

Mode.

If the Sector Count register is non-zero then the Standby Timer

shall be enabled.  The value in the Sector Count register shall

be used to determine the time programmed into the Standby Timer.

If the Sector Count register is zero then the Standby Timer is

disabled.

ERRORS - Command Abort - The device does not support the Power

Management command set.

8.12. IDLE IMMEDIATE

TYPE - Optional - Power Management Feature Set.

PROTOCOL - Non-data command.

INPUTS - None.

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command causes the device to set BSY, enter

the Idle Mode, clear BSY, and assert INTRQ.  INTRQ is asserted

even though the device may not have fully transitioned to Idle

Mode.

ERRORS - Command Abort - The device does not support the Power

Management command set.

8.13.   INITIALIZE DEVICE PARAMETERS

TYPE - Mandatory.

PROTOCOL - Non-data.

INPUTS - The Sector Count register specifies the number of

sectors per track, and the Device/Head register which specifies

the number of heads minus 1.

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command enables the host to set the number of

sectors per track and the number of heads minus 1, per cylinder

for the current CHS translation mode.

Upon receipt of the command, the device sets BSY, saves the

parameters, clears BSY, and generates an interrupt.

The sector and head values are not checked for validity by this

command.  If they are invalid, no error will be posted until an

attempt is made to execute a media access command which shall be

failed with an ID Not Found error.

ERRORS - None.  This command is not allow to fail or set any

errors conditions for any reason.

8.14.   NOP

TYPE - Optional.

PROTOCOL - Non-data.

INPUTS - None.

OUTPUTS - The Command Block registers, other than the Error and

Status registers, are not changed by this command.

PREREQUISITES - None.

DESCRIPTION - This command enables a host which can only perform

16-bit register accesses to check device status.  The device

shall respond as it does to an unrecognized command by setting

Abort in the Error register, Error in the Status register,

clearing Busy in the Status register, and asserting INTRQ.

NOTE:  When a 16-bit host writes to the Device/Head Register, one

byte contains the Command register, so the device sees a new

command when the intended purpose is only to select a device.

Both devices may be Busy but not necessarily Ready, e.g.  Device

0 may be ready, but not device 1. To check this possibility a

typical sequence for an 8-bit host would be:

a)  Read the Status register (wait until Busy False)

b)  Select the device (write to the Device/Head Register)

c)  Read the Status register (wait until Busy False and Ready True)

d)  Send the command (write to the Command register).

As a 16-bit host executes b and d simultaneously, a problem

occurs if the device being selected is Not Ready at the time the

command is issued.

ERRORS - This command always fails with a Command Abort error.

8.15.   READ BUFFER

TYPE - Optional.

PROTOCOL - PIO data in.

INPUTS - None.

OUTPUTS - None.

PREREQUISITES - A WRITE BUFFER command should immediately preceed

a READ BUFFER command.

DESCRIPTION - The READ BUFFER command enables the host to read

the current contents of the device's sector buffer.  When this

command is issued, the device sets BSY, sets up the sector buffer

for a read operation, sets DRQ, clears BSY, and generates an

interrupt.  The host then reads up to 512 bytes of data from the

buffer.

The READ BUFFER and WRITE BUFFER commands shall be synchronized

such that sequential WRITE BUFFER and READ BUFFER commands access

the same 512 bytes within the buffer.

ERRORS - Command Abort if the command is not supported.

8.16.   READ DMA

TYPE - Optional.

PROTOCOL - DMA.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

read.  The Sector Count register specifies the number of sectors

to be transferred.

OUTPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number registers specify the address of the last sector

transferred or the address of the sector where the first error

was detected.

PREREQUISITES - The host must initialize a slave-DMA channel.

DESCRIPTION - This command executes in a similar manner to the

READ SECTOR(S) command except for the following:

* the host initializes a slave-DMA channel prior to issuing the

command

* data transfers are qualified by DMARQ and are performed by the

slave-DMA channel

* the device issues only one interrupt per command to indicate

that data transfer has terminated and status is available.

During the DMA transfer phase of a Read DMA command, the device

shall provide status of BSY or DRQ until the command is

completed.

At command completion, the Command Block Registers contain the

sector address of the last sector transferred.

An unrecoverable error encountered during execution of a READ DMA

command results in the termination of the command.  At command

completion, the Command Block Registers contain the sector

address of the sector where the first error occurred.

ERRORS - Command Abort if the command is not supported.  Other

errors possible are Bad Block, Uncorrectable data error, ID Not

Found and Address Mark Not Found.

8.17.   READ LONG

TYPE - Manadatory.

PROTOCOL - PIO data in.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

read.  The Sector Count register shall not specify a value other

than 1.

OUTPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number registers specify the address of the last sector

transferred or the address of the sector where the first error

was detected.

PREREQUISITES - The SET FEATURES subcommand to enable more than

4 vendor specific bytes must be executed prior to the READ LONG

command if more than 4 vendor specific bytes are to be transferred.

DESCRIPTION - The READ LONG command performs similarly to the

READ SECTOR(S) command except that it returns the data and a

number of vendor specific bytes appended to the data field of the

desired sector.  During a READ LONG command, the device does not

check to determine if there has been a data error.  Only single

sector read long operations are supported.

The transfer of the vendor specific bytes shall be one byte at a

time over bits DD0-7 only (8-bits wide).

ERRORS - Bad Block, ID Not Found Command Abort and Address Mark

Not Found.

8.18.   READ MULTIPLE command

TYPE - Optional.

PROTOCOL - PIO data in.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

read.  The Sector Count register specifies the number of sectors

to be transferred.

OUTPUTS - If not error, the Cylinder Low, Cylinder High,

Device/Head and Sector Number registers specify the address of

the last sector transferred.  These registers are undefined if an

error condition is indicated at command completion.

PREREQUISITES - A successful SET MULTIPLE MODE command must

preceed a READ MULTIPLE command.

DESCRIPTION - The READ MULTIPLE command performs similarly to the

READ SECTOR(S) command.  Interrupts are not generated on every

sector, but on the transfer of a block which contains the number

of sectors defined by a SET MULTIPLE MODE command.

Command execution is identical to the READ SECTOR(S) operation

except that the number of sectors defined by a SET MULTIPLE MODE

command are transferred without intervening interrupts.  DRQ

qualification of the transfer is required only at the start of

the data block, not on each sector.

The block count of sectors to be transferred without intervening

interrupts is programmed by the SET MULTIPLE MODE command, which

shall be executed prior to the READ MULTIPLE command.

When the READ MULTIPLE command is issued, the Sector Count

register contains the number of sectors (not the number of blocks

or the block count) requested.

If the number of requested sectors is not evenly divisible by the

block count, as many full blocks as possible are transferred,

followed by a final, partial block transfer.  The partial block

transfer shall be for n sectors, where

n = Remainder (SECTOR COUNT / block count)

If the READ MULTIPLE command is attempted before the SET MULTIPLE

MODE command has been executed or when READ MULTIPLE commands are

disabled, the READ MULTIPLE operation shall be rejected with an

Aborted Command error.

Device errors encountered during READ MULTIPLE commands are

posted at the beginning of the block or partial block transfer,

but DRQ is still set and the data transfer shall take place as it

normally would, including transfer of corrupted data, if any.

The contents of the Command Block Registers following the

transfer of a data block which had a sector in error are

undefined.  The host should retry the transfer as individual

requests to obtain valid error information.

Subsequent blocks or partial blocks are transferred only if the

error was a correctable data error.  All other errors cause the

command to stop after transfer of the block which contained the

error.  Interrupts are generated when DRQ is set at the beginning

of each block or partial block.

ERRORS - Bad Block, Uncorrectable Data Error, ID Not Found

Command Abort and Address Mark Not Found.

8.19.   READ SECTOR(S)

TYPE - Manadatory.

PROTOCOL - PIO data in.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

read.  The Sector Count register specifies the number of sectors

to be transferred.

OUTPUTS - If no error, the Cylinder Low, Cylinder High,

Device/Head and Sector Number registers specify the address of

the last sector transferred.  If the comamnd ends with an error,

these registers contain the address of the sector where the first

error was detected and the Sector Count register contains the

number of sectors remaining to be transferred in order to

complete the original request.

PREREQUISITES - None.

DESCRIPTION - This command reads from 1 to 256 sectors as

specified in the Sector Count register.  A SECTOR COUNT of 0

requests 256 sectors.  The transfer begins at the sector

specified in the Sector Number register.

If the device is not already on the desired track, an implied

seek is performed.

DRQ is always set prior to data transfer regardless of the

presence or absence of an error condition.

The types of errors that are not retried when no retries is

requested by the host is vendor specific.

ERRORS - Bad Block, Uncorrectable Data Error, ID Not Found

Command Abort and Address Mark Not Found.

8.20.   READ VERIFY SECTOR(S)

TYPE - Manadatory.

PROTOCOL - Non-data.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

verified.  The Sector Count register specifies the number of

sectors to be verified.

OUTPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number registers specify the address of the last sector verified

or the address of the sector where the first error was detected.

PREREQUISITES - None.

DESCRIPTION - This command is identical to the READ SECTOR(S)

command, except that DRQ is never set, and no data is transferred

to the host.

When the requested sectors have been verified, the device clears

BSY and generates an interrupt.

If an error occurs, the verify terminates at the sector where the

error occurs.  The Command Block Registers contain the sector

address of the sector where the error occurred.  The Sector Count

register shall contain the number of sectors not yet verified.

The types of errors that are not retried when no retries is

requested by the host is vendor specific.

ERRORS - Bad Block, Uncorrectable Data Error, ID Not Found

Command Abort and Address Mark Not Found.

-----end

yawn... zzz...zzz...zzz...

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Aug 24 23:44:25 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15424; Wed, 24 Aug 94 23:44:25 PDT

Received: from uucp6.netcom.com by dt.wdc.com (4.1/SMI-4.1)

id AB15157; Wed, 24 Aug 94 23:31:16 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id XAA07122; Wed, 24 Aug 1994 23:29:24 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdY3f-000gJTC; Thu, 25 Aug 94 00:13 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Thu, 25 Aug 1994 00:15:53 EDT

Date:      Thu, 25 Aug 1994 00:15:52 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5c1afa.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 8 (part 1/4)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Command descriptions -- part 1 or 4.

-----begin

8.      Command Descriptions

Commands are issued to the device by loading the pertinent

registers in the command block with the needed parameters, and

then writing the command code to the Command register.

The manner in which a command is accepted varies.  There are

three classes (see table 10) of command acceptance, all

predicated on the fact that to receive a command, BSY=0:

################################################################

#

#Hale's note:  I think all this command class stuff is a bunch

#of junk, especially Classes 2 and 3. I think these should be

#removed and all replaced by a statement like:

#

#Upon receipt of a command, the device sets BSY within 400

#nsec.  Following BSY=1, the status presented by the device

#depends on the type of command:  PIO data in, PIO data out,

#non-data transfer or DMA.  See the individual command

#descriptions and Section 9 for the protocol followed by each

#command and command type.

#

#Next I would replace the class column in table 10 with a

#type column (PIO, DMA, non-data)

#

################################################################

* Upon receipt of a Class 1 command, the device sets BSY within

400 nsec.

* Upon receipt of a Class 2 command, the device sets BSY within

400 nsec, sets up the sector buffer for a write operation,

sets DRQ within 700 usec, and clears BSY within 400 nsec of

setting DRQ.

* Upon receipt of a Class 3 command, the device sets BSY within

400 nsec, sets up the sector buffer for a write operation,

sets DRQ within 20 msec, and clears BSY within 400 nsec of

setting DRQ.

NOTE:  DRQ may be set so quickly on Class 2 and Class 3 that the

BSY transition is too short for BSY=1 to be recognized.

################################################################

#

#Hale's note:  I have replaced the "class" column with the

#"protocol" column and labeled the "type" column so you can see

#what I mean.

#

################################################################

The proto column codes represent the command protocol used:

DM - A DMA command.

ND - A non data command.

PI - A PIO data in command.

PO - A PIO data out command.

VS - A Vendor specific command.

The typ column codes represent the command type:

O - Optional -- the implementation of this command is optional.

M - Manadatory -- all ATA devices must implement this command.

R - Reserved for use in future ATA standards.

V - Reserved for vendor specific use.

+=====-======================================-Command-=====Parameters Used====+

|proto|                                  |typ| code  | FR | SC | SN | CY | DH |

|-----+--------------------------------------+-------+----+----+----+----+----|

|  VS | ACKNOWLEDGE MEDIA CHANGE         | O |  DBh  |    |    |    |    |  D |

|  VS | BOOT - POST-BOOT                 | O |  DCh  |    |    |    |    |  D |

|  VS | BOOT - PRE-BOOT                  | O |  DDh  |    |    |    |    |  D |

|  ND | CHECK POWER MODE                 | O |98h E5h|    |  y |    |    |  D |

|  VS | DOOR LOCK                        | O |  DEh  |    |    |    |    |  D |

|  VS | DOOR UNLOCK                      | O |  DFh  |    |    |    |    |  D |

|  PO | DOWNLOAD MICROCODE               | O |  92h  |  y |  y |  y |  y |  D |

|  ND | EXECUTE DEVICE DIAGNOSTIC        | M |  90h  |    |    |    |    |  D*|

|  PO | FORMAT TRACK                     | M |  50h  |  * |  y |    |  y |  y |

|  PI | IDENTIFY DEVICE                  | M |  ECh  |    |    |    |    |  D |

|  ND | IDLE                             | O |97h E3h|    |  y |    |    |  D |

|  ND | IDLE IMMEDIATE                   | O |95h E1h|    |    |    |    |  D |

|  ND | INITIALIZE DEVICE PARAMETERS     | M |  91h  |    |  y |    |    |  y |

|  ND | NOP                              | O |  00h  |    |    |    |    |  y |

|  PI | READ BUFFER                      | O |  E4h  |    |    |    |    |  D |

|  DM | READ DMA (w/retry)               | O |  C8h  |    |  y |  y |  y |  y |

|  DM | READ DMA (w/o retry)             | O |  C9h  |    |  y |  y |  y |  y |

|  PI | READ LONG (w/retry)              | M |  22   |    |  y |  y |  y |  y |

|  PI | READ LONG (w/o retry)            | M |  23   |    |  y |  y |  y |  y |

|  PI | READ MULTIPLE                    | O |  C4h  |    |  y |  y |  y |  y |

|  PI | READ SECTOR(S) (w/retry)         | M |  20   |    |  y |  y |  y |  y |

|  PI | READ SECTOR(S) (w/o retry)       | M |  21   |    |  y |  y |  y |  y |

|  ND | READ VERIFY SECTOR(S) (w/retry)  | M |  40   |    |  y |  y |  y |  y |

|  ND | READ VERIFY SECTOR(S) (w/o retry)| M |  41   |    |  y |  y |  y |  y |

|  ND | RECALIBRATE                      | M |  1xh  |    |    |    |    |  D |

|  ND | SEEK                             | M |  7xh  |    |    |  y |  y |  y |

|  ND | SET FEATURES                     | O |  EFh  |  y |    |    |    |  D |

|  ND | SET MULTIPLE MODE                | O |  C6h  |    |  y |    |    |  D |

|  ND | SLEEP                            | O |99h E6h|    |    |    |    |  D |

|  ND | STANDBY                          | O |96h E2h|    |  y |    |    |  D |

|  ND | STANDBY IMMEDIATE                | O |94h E0h|    |    |    |    |  D |

|  PO | WRITE BUFFER                     | O |  E8h  |    |    |    |    |  D |

|  DM | WRITE DMA (w/retry)              | O |  CAh  |    |  y |  y |  y |  y |

|  DM | WRITE DMA (w/o retry)            | O |  CBh  |    |  y |  y |  y |  y |

|  PO | WRITE LONG (w/retry)             | M |  32   |  * |  y |  y |  y |  y |

|  PO | WRITE LONG (w/o retry)           | M |  33   |  * |  y |  y |  y |  y |

|  PO | WRITE MULTIPLE                   | O |  C5h  |  * |  y |  y |  y |  y |

|  PO | WRITE SAME                       | O |  E9h  |  y |  y |  y |  y |  y |

|  PO | WRITE SECTOR(S) (w/retry)        | M |  30   |  * |  y |  y |  y |  y |

|  PO | WRITE SECTOR(S) (w/o retry)      | M |  31   |  * |  y |  y |  y |  y |

|  PO | WRITE VERIFY                     | O |  3Ch  |  * |  y |  y |  y |  y |

|  VS | Vendor specific                  | V |  9Ah  |    |    |    |    |    |

|  VS | Vendor specific                  | V | C0-C3h|    |    |    |    |    |

|  VS | Vendor specific                  | V |  8xh  |    |    |    |    |    |

|  VS | Vendor specific                  | V |F0h-FFh|    |    |    |    |    |

|  -- | Reserved: all remaining codes    | R |       |    |    |    |    |    |

|-----+-----------------------------------------------------------------------|

|     | CY = Cylinder registers         SC = Sector Count register            |

|     | DH = Device/Head register       SN = Sector Number register           |

|     | FR = Features register (see command descriptions for use)             |

|     | y - the register contains a valid parameter for this command.         |

|     |     For the Device/Head register, y means both the device and         |

|     |     head parameters are used.                                         |

|     | D - only the device parameter is valid and not the head parameter.    |

|     | D* - Addressed to device 0 but both devices execute it.               |

|     | * - Maintained for compatibility (see 6.2.10)                         |

+=============================================================================+

Table 10 - Command Codes and Parameters (Part 2 of 2)

Each command description in the following sections contain the following

subsections:

TYPE - Indicates if the command is manadatory or optional.

PROTOCOL - Indicates which protocol is used by the command.

INPUTS - Describes the Command Block register data that the host

must supply.

OUTPUTS - Describes the Command Block register data that is

returned by the device at the end of a command.

PREREQUISITES - Any prerequisite commands or conditions that must

be meet before the command can be issued.

DESCRIPTION - The description of the commands function(s).

ERRORS - Describes the error conditions that may cause the

command to fail.

#####################################################################

#

#Hale's note: At one of the meeting I suggested that the "removable

#media" commands be made Vendor Specific.  I have done that here.

#

#These commands were never properly described and there are few but

#very different implementations of these commands.  So why not make

#them VS?

#

#####################################################################

8.1.    ACKNOWLEDGE MEDIA CHANGE (removable)

TYPE - Optional.

PROTOCOL - Vendor specific.

INPUTS - Vendor specific.

OUTPUTS - Vendor specific.

PREREQUISITES - Vendor specific.

DESCRIPTION - This command is reserved for use by removable

media devices.  The implementation of this command is vendor

specific.

ERRORS - If the device does not support this command, the device

returns a Command Abort error.  All other error conditions are

vendor specific.

8.2.    BOOT - POST-BOOT (removable)

TYPE - Optional.

PROTOCOL - Vendor specific.

INPUTS - Vendor specific.

OUTPUTS - Vendor specific.

PREREQUISITES - Vendor specific.

DESCRIPTION - This command is reserved for use by removable

media devices.  The implementation of this command is vendor

specific.

ERRORS - If the device does not support this command, the device

returns a Command Abort error.  All other error conditions are

vendor specific.

8.3.    BOOT - PRE-BOOT (removable)

TYPE - Optional.

PROTOCOL - Vendor specific.

INPUTS - Vendor specific.

OUTPUTS - Vendor specific.

PREREQUISITES - Vendor specific.

DESCRIPTION - This command is reserved for use by removable

media devices.  The implementation of this command is vendor

specific.

ERRORS - If the device does not support this command, the device

returns a Command Abort error.  All other error conditions are

vendor specific.

8.4. CHECK POWER MODE

TYPE - Optional - Power Management Feature Set.

PROTOCOL - Non-data command.

INPUTS - None.

OUTPUTS - The Sector Count register is set to 0 (00h) if the

device is going to, in or leaving Standby mode.  The Sector Count

register is set to 255 (FFh) if the device is in Active or Idle

mode.

PREREQUISITES - None.

DESCRIPTION - If the device is in, going to, or recovering from

the Standby Mode the device shall set BSY, set the Sector Count

register to 0 (00h), clear BSY, and assert INTRQ.

If the device is in Active or Idle Mode, the device shall set BSY,

set the Sector Count register to 255 (FFh), clear BSY, and assert

INTRQ.

ERRORS - Command abort if the device does not support the Power

Management command set.

8.5.    DOOR LOCK (removable)

TYPE - Optional.

PROTOCOL - Vendor specific.

INPUTS - Vendor specific.

OUTPUTS - Vendor specific.

PREREQUISITES - Vendor specific.

DESCRIPTION - This command is reserved for use by removable

media devices.  The implementation of this command is vendor

specific.

ERRORS - If the device does not support this command, the device

returns a Command Abort error.  All other error conditions are

vendor specific.

8.6.    DOOR UNLOCK (removable)

TYPE - Optional.

PROTOCOL - Vendor specific.

INPUTS - Vendor specific.

OUTPUTS - Vendor specific.

PREREQUISITES - Vendor specific.

DESCRIPTION - This command is reserved for use by removable

media devices.  The implementation of this command is vendor

specific.

ERRORS - If the device does not support this command, the device

returns a Command Abort error.  All other error conditions are

vendor specific.

8.7.    DOWNLOAD MICROCODE

TYPE - Optional.

PROTOCOL - PIO data out.

INPUTS - The head bits of the Device/Head register shall always

be set to zero.  The Cylinder High and Low registers shall be set

to zero.  The Sector Number and Sector Count registers are used

together as a 16-bit sector count value.  The Feature register

specifies the subcommand code.

OUTPUTS - Vendor specific.

PREREQUISITES - Vendor specific.

DESCRIPTION - This command enables the host to alter the device's

microcode.  The data transferred using the DOWNLOAD MICROCODE

command is vendor specific.

All transfers shall be an integer multiple of the sector size.

The size of the data transfer is determined by the contents of

the Sector Number Register and the Sector Count register.  The

Sector Number Register shall be used to extend the Sector Count

register, to create a sixteen bit sector count value.  The Sector

Number Register shall be the most significant eight bits and the

Sector Count register shall be the least significant eight bits.

A value of zero in both the Sector Number Register and the Sector

Count register shall indicate no data is to be transferred.  This

allows transfer sizes from 0 bytes to 33 553 920 bytes, in 512

byte increments.

The Features register shall be used to determine the effect of

the DOWNLOAD MICROCODE command.  The values for the Feature

Register are:

01H - download is for immediate, temporary use

07H - save downloaded code for immediate and future use

All other values are reserved.

ERRORS - Command Abort if the device does not support this command. All

other error conditions are vendor specific.

8.8.    EXECUTE DEVICE DIAGNOSTIC

TYPE - Mandatory.

PROTOCOL - Non-data.

INPUTS - None, except that the device selection bit in the Device/Head

register is ignored.

OUTPUTS -

The diagnostic code written to the Error register is a unique 8-bit code as

shown in table 11, and not as the single bit flags defined in 7.2.9.

If Device 1 fails diagnostics, Device 0 "OR's" 80h with its own status and loads

that code into the Error register.  If Device 1 passes diagnostics or there is

no Device 1 connected, Device 0 "OR's" 00h with its own status and loads that

code into the Error register.

+=======-==================================+

| Code  |                                  |

|-------+----------------------------------|

|  01h  | No error detected                |

|  02h  | Formatter device error           |

|  03h  | Sector buffer error              |

|  04h  | ECC circuitry error              |

|  05h  | Controlling microprocessor error |

|  8xh  | Device 1 failed                  |

+==========================================+

Table 11 - Diagnostic Codes

PREREQUISITES - None.

#############################################################

#

#Hale's note:  The description of this command has never been

#correct (or even made any sense!).  I think I've fixed that

#here.

#

#############################################################

DESCRIPTION - This command shall perform the internal diagnostic

tests implemented by the device.  See also clauses 6.2.4 and

6.2.13.  The DEV bit is ignored.  Both devices, if present, shall

execute this command.

If Device 1 is present, it performs the following functions:

a) Drive 1 sets BSY within 400 nsec after the EXECUTE DEVICE

DIAGNOSTIC command was received.

b) Drive 1 negates PDIAG- within 1 msec after command received.

c) Drive 1 performs internal diagnostics.

d) Drive 1 posts diagnostic results to the Error Register

e) Drive 1 clears BSY when ready to accept commands that do not

require DRDY=1.

f) Drive 1 asserts PDIAG- to indicate that it is ready to accept

commands.

NOTE: Drive 1 must clear BSY and assert PDIAG- within 5

seconds from the time that the EXECUTE DEVICE DIAGNOSTIC command

was received.

g) Drive 1 sets DRDY when ready to accept any command.

NOTE: Steps e), f) and g) may occur at the same time.

If Device 0 is present, it performs the following functions:

a) Drive 0 sets BSY within 400 nsec after the EXECUTE DEVICE

DIAGNOSTIC command was received.

b) Drive 0 performs internal diagnostics.

c) Drive 0 waits up to 6 seconds for Drive 1 to assert PDIAG-.

d) Drive 0 posts diagnostic results to the Error Register

e) Drive 0 clears BSY when ready to accept commands that do not

require DRDY=1.

NOTE: Drive 0 must clear BSY within 6 seconds from the

time that the EXECUTE DEVICE DIAGNOSTIC command was received.

f) Drive 0 sets DRDY when ready to accept any command.

NOTE: Steps e) and f) may occur at the same time.

ERRORS - None.  All error information is returned as a diagnostic

code in the Error register.

8.9.    FORMAT TRACK

TYPE - Manadatory.

PROTOCOL - PIO data out.

INPUTS - Vendor specific.

OUTPUTS - Vendor specific.

PREREQUISITES - Vendor specific.

DESCRIPTION - The implementation of the FORMAT TRACK command is

vendor specific.  It is recommend that system implementations not

utilize this command.

ERRORS - Vendor specific.

8.10.   IDENTIFY DEVICE

TYPE - Manadatory

PROTOCOL - PIO data in.

INPUTS - None.

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - The IDENTIFY DEVICE command enables the host to

receive parameter information from the device.

When the command is issued, the device sets BSY, prepares to

transfer the 256 words or device identification data to the host,

sets DRQ, clears BSY, and generates an interrupt.  The host can

then transfer the data by reading the Data register.

The parameter words in the buffer have the arrangement and

meanings defined in Table 12.  All reserved bits or words shall

be zero.  The F/V column indicates if the word or part of a word

has fixed (F) contents that do not change, variable (V) contents

that may change depending on the device state or the commands

executed by the device, X for words with vendor specific data

which may be fixed or variable, and R for reserved words which

shall be zero.

ERRORS - None.

+=======-==============================================================+===+

|  Word |                                                              |F/V|

|-------+--------------------------------------------------------------|---|

|    0  | General configuration bit-significant information:           |   |

|       |   15  0   reserved for non-magnetic devices                  | F |

|       |   14  Vendor specific (obsolete)                             | F |

|       |   13  Vendor specific (obsolete)                             | F |

|       |   12  Vendor specific (obsolete)                             | F |

|       |   11  Vendor specific (obsolete)                             | F |

|       |   10  Vendor specific (obsolete)                             | F |

|       |    9  Vendor specific (obsolete)                             | F |

|       |    8  Vendor specific (obsolete)                             | F |

|       |    7  1=removable cartridge device                           | F |

|       |    6  1=fixed device                                         | F |

|       |    5  1=spindle motor control option implemented             | F |

|       |    4  Vendor specific (obsolete)                             | F |

|       |    3  Vendor specific (obsolete)                             | F |

|       |    2  Vendor specific (obsolete)                             | F |

|       |    1  Vendor specific (obsolete)                             | F |

|       |    0  0=reserved                                             | F |

|    1  | Number of cylinders                                          | F |

|    2  | Reserved                                                     | R |

|    3  | Number of heads                                              | F |

|    4  | Vendor specific (obsolete)                                   | X |

|    5  | Vendor specific (obsolete)                                   | X |

|    6  | Number of sectors per track                                  | F |

|  7-9  | Vendor specific                                              | X |

| 10-19 | Serial number (20 ASCII characters, 0000h=not specified)     | F |

|   20  | Vendor specific (obsolete)                                   | X |

|   21  | Buffer size in 512 byte increments (0000h=not specified)     | F |

|   22  | # of vendor specific bytes avail on READ/WRITE LONG cmds     | F |

|       |  (0000h=not spec'd)                                          |   |

| 23-26 | Firmware revision (8 ASCII characters, 0000h=not specified)  | F |

| 27-46 | Model number (40 ASCII characters, 0000h=not specified)      | F |

|   47  | 15-8  Vendor specific                                        | X |

|       |  7-0  00h = READ/WRITE MULTIPLE commands not implemented     | F |

|       |       xxh = Maximum number of sectors that can be transferred| F |

|       |             per interrupt on READ AND WRITE MULTIPLE commands|   |

|   48  | 0000h = cannot perform doubleword I/O                        | F |

|       | 0001h = can perform doubleword I/O                           | F |

|   49  | Capabilities                                                 |   |

|       | 15-13  0=reserved                                            | R |

|       |    12  1=Reserved (for advanced PIO mode support)            | R |

|       |    11  1=IORDY supported                                     | F |

|       |        0=IORDY may be supported                              | F |

|       |    10  1=IORDY can be disabled                               | F |

|       |     9  1=LBA supported                                       | F |

|       |     8  1=DMA supported                                       | F |

|       |  7- 0  Vendor specific                                       | X |

|   50  | Reserved                                                     | R |

|   51  | 15-8  PIO data transfer cycle timing mode                    | F |

|       |  7-0  Vendor specific                                        | X |

|   52  | 15-8  DMA data transfer cycle timing mode                    | F |

|       |  7-0  Vendor specific                                        | X |

|   53  | 15-2  Reserved                                               | R |

|       |    1  1=the fields reported in words 64-70 are valid         | F |

|       |       0=the fields reported in words 64-70 are not valid     | F |

|       |    0  1=the fields reported in words 54-58 are valid         | F |

|       |       0=the fields reported in words 54-58 may be valid      | F |

|   54  | Number of current cylinders                                  | V |

|   55  | Number of current heads                                      | V |

|   56  | Number of current sectors per track                          | V |

| 57-58 | Current capacity in sectors                                  | V |

|   59  | 15-9  Reserved                                               | R |

|       |    8  1 = Multiple sector setting is valid                   | V |

|       |  7-0  xxh = Current setting for number of sectors that can be| V |

|       |             transferred per interrupt on R/W multiple command|   |

| 60-61 | Total number of user addressable sectors (LBA mode only)     | F |

|   62  | 15-8  Single word DMA transfer mode active                   | V |

|       |  7-0  Single word DMA transfer modes supported               | F |

|   63  | 15-8  Multiword DMA transfer mode active                     | V |

|       |  7-0  Multiword DMA transfer modes supported                 | F |

|   64  | 15-8  Reserved                                               | R |

|       |  7-0  Advanced PIO Transfer Modes Supported                  | F |

|   65  | Minimum Multiword DMA Transfer Cycle Time Per Word           |   |

|       | 15-0  Cycle time in nanoseconds                              | F |

|   66  | Manufacturer's Recommended Multiword DMA Transfer Cycle Time |   |

|       | 15-0  Cycle time in nanoseconds                              | F |

|   67  | Minimum PIO Transfer Cycle Time Without Flow Control         |   |

|       | 15-0  Cycle Time in nanoseconds                              | F |

|   68  | Minimum PIO Transfer Cycle Time With IORDY Flow Control      |   |

|       | 15-0  Cycle Time in nanoseconds                              | F |

| 69-70 | Reserved (for advanced PIO mode support)                     | R |

| 71-127| Reserved                                                     | R |

|128-159| Vendor specific                                              | X |

|160-255| Reserved                                                     | R |

+======================================================================+===+

Table 12 - Identify Device Information (Part 2 of 2)

[Editor's Note:  There are currently restrictions by some

operating systems and BIOSs that make it desireable to set limits

or impose restrictions on some of the parameters returned by the

Identify Device command.  These restrictions are not currently

included in this proposal and must be addressed before this

document is complete.]

##########################################################

#

#Hale's note:  As you may recall I asked that each word be

#described in sections 8.10.x -- I have done this here.

#

##########################################################

8.10.1. Word 1: Number of cylinders

The number of user-addressable cylinders in the default

translation mode.

8.10.2. Word 2: Reserved.

8.10.3. Word 3: Number of heads

The number of user-addressable heads in the default translation

mode.

8.10.4. Word 4: Vendor specific data.

8.10.5. Word 5: Vendor specific data.

8.10.6. Word 6: Number of sectors per track

The number of user-addressable sectors per track in the default

translation mode.

8.10.7. Words 7-9: Vendor specific data.

8.10.8. Words 10-19: Serial Number

The contents of this field are right justified and padded with

spaces (20h).

8.10.9. Word 20: Vendor specific data.

8.10.10. Word 21: Buffer size.

The size of the device buffer in 512-byte increments:

0000H = not specified

0001H = 512 bytes

0002H = 1024 bytes

...

8.10.11. Word 22: Vendor specific bytes on READ/WRITE LONG commands

If the contents of this field are set to a value other than 4,

the only way to use this information is via the SET FEATURES

subcommand that enables more than 4 bytes to be transferred.

8.10.12. Word 23-26: Firmware revision

The contents of this field are left justified an padded with

spaces (20h).

8.10.13.        Words 27-46: Model number

The contents of this field are left justified and padded with

spaces (20h).

8.10.14.        Word 47: READ/WRITE MULTIPLE support.

Bits 7-0 of this word define the maximum number of sectors per

block that the device supports for READ/WRITE MULTIPLE commands.

If a device supports the READ/WRITE MULTIPLE and SET MULTIPLE

MODE commands, these bits contain a non-zero value.  If the

device does not support the READ/WRITE MULTIPLE and SET MULTIPLE

MODE commands, these bits shall be zero.

8.10.15.        Word 48: Double Word I/O support.

This word is not used by ATA-2 devices and shall be set to zero.

8.10.16.        Word 49: Capabilities

8.10.16.1.      IORDY Support

Bit 11 of word 49 is used to help determine whether a device

supports IORDY.  If this bit is set to one, then the device

supports IORDY operation.  If this bit is zero, the device may

support IORDY.  This insures backward compatibility.

If a device supports PIO Mode 3, then this bit must be set.

8.10.16.2.      IORDY Can Be Disabled

Bit 10 of word 49 is used to indicate a device's ability to

enable or disable the use of IORDY.  If this bit is set to one,

then the device supports the disabling of IORDY.

8.10.16.3.      LBA supported

Bit 9 of word 49 is used to indicate if the device supports LBA

mode addressing.  If this bit is set, words 60-61 must be valid.

8.10.16.4.      DMA supported

Bit 9 of word 49 is used to indicate if the device supports the

READ/WRITE DMA commands.

8.10.17.        Word 50: Reserved.

8.10.18.        Word 51: PIO data transfer cycle timing mode

The PIO transfer timing for each ATA device falls into categories

which have unique parametric timing specifications.  To determine

the proper device timing category, compare the Cycle Time

specified in figure 5 with the contents of this field.  The value

returned in Bits 15-8 should fall into one of the mode 0 through

mode 2 categories specified in figure 5, and if it does not, then

Mode 0 shall be used to serve as the default timing.

Note:  For backwards compatibility with BIOSs written before Word

64 was defined for advanced modes, a device reports in Word 51

the highest original PIO mode (i.e.  PIO mode 0, 1, or 2) it can

support.

8.10.19.        Word 52: Single Word DMA data transfer cycle timing mode

The DMA transfer timing for each ATA device falls into categories

which have unique parametric timing specifications.  To determine

the proper device timing category, compare the Cycle Time

specified in figure 6 with the contents of this field.  The value

returned in Bits 15-8 should fall into one of the categories

specified in figure 6 (i.e. 0, 1, or 2), and if it does not, then

Mode 0 shall be used to serve as the default timing.

The contents of this word shall be ignored if Words 62 or 63 are

supported.

8.10.20.         Word 53: Field Validity

Word 53, bit 0 defines whether the fields contained in words 54

through 58 are guaranteed to be valid.  If this bit is not set,

the fields contained in words 54 through 58 may be valid.

Word 53, bit 1 will be set if any of the fields reported in words

64 through 70 are valid.  This bit will be reset if the fields

reported in words 64-70 are not valid.  Any device which supports

PIO Mode 3 or above, or supports Multiword DMA Mode 1 or above,

must set bit 1 of word 53 and support the fields contained in

words 64 through 70.

8.10.21.        Word 54: Number of current cylinders

The number of user-addressable cylinders in the current

translation mode.

8.10.22.        Word 55: Number of current heads

The number of user-addressable heads in the current translation

mode.

8.10.23.        Word 56: Number of current sectors per track

The number of user-addressable sectors per track in the current

translation mode.

8.10.24.        Word 57-58: Current capacity in sectors

The current capacity in sectors excludes all sectors used for

device-specific purposes.  The number of sectors of available

capacity shall be calculated as:

(Number of current cylinders) * (Number of current heads) * (Number

of current sectors per track)

8.10.25.        Word 59: Multiple sector setting

If bit 8 is set, then bits 7-0 reflect the number of sectors

currently set to transfer on a READ/WRITE MULTIPLE command.  If

word 47 bits 7-0 are zero then word 59 bits 8-0 shall also be

zero.

8.10.26.        Word 60-61: Total number of user addressable sectors

If the device supports LBA Mode, these words reflect the total

number of user addressable sectors.  This value does not depend

on the current device geometry.  If the device does not support

LBA mode, these words shall be set to 0.

8.10.27.        Word 62: Single word DMA transfer

The low order byte identifies by bit all of the Modes which are

supported e.g. if Mode 0 is supported, bit 0 is set.  The high

order byte contains a single bit set to indicate which mode is

active.

8.10.28.        Word 63: Multiword DMA transfer

The low order byte identifies by bit all of the Modes which are

supported e.g. if Mode 0 is supported, bit 0 is set.  The high

order byte contains a single bit set to indicate which mode is

active.

8.10.29.        Word 64: Flow Control PIO Transfer Modes Supported

Bits 7 through 0 of word 64 of the Identify Device parameter

information is defined as the Advanced PIO Data Transfer

Supported Field.  This field is bit significant.  Any number of

bits may be set in this field by the device to indicate which

Advanced PIO Modes that it is capable of supporting.

Of these bits, bits 7 through 2 are Reserved for future Advanced

PIO Modes.  Bit 0, if set, indicates that the device supports PIO

Mode 3. Bit 1, if set, indicates that the device supports PIO

Mode 4.

Note:  For backwards compatibility with BIOSs written before Word

64 was defined for advanced modes, a device reports in Word 51

the highest original PIO mode (i.e.  PIO mode 0, 1, or 2) it can

support.

8.10.30.        Word 65: Minimum Multiword DMA Transfer Cycle Time Per Word

Word 65 of the parameter information of the IDENTIFY DEVICE

command is defined as the Minimum Multiword DMA Transfer Cycle

Time Per Word.  This field defines, in nanoseconds, the minimum

cycle time that the device can support when performing Multiword

DMA transfers on a per word basis.

If this field is supported, bit 1 of word 53 must be set.

Any device which supports Multiword DMA Mode 1 or above shall

support this field, and the value in word 65 shall not be less

than the minimum cycle time reported by the fastest DMA mode

supported by the device.

If bit 1 of word 53 is set because a device supports a field in

Words 64-70 other than this field and the device does not support

this field, the device shall return a value of zero in this

field.

8.10.31.        Word 66: Manufacturer's Recommended Multiword DMA Cycle Time

Word 66 of the parameter information of the IDENTIFY DEVICE

command is defined as the Manufacturer's Recommended Multiword

DMA Transfer Cycle Time.  This field defines, in nanoseconds, the

minimum cycle time per word during a single sector host transfer

while performing a multiple sector READ DMA or WRITE DMA commands

over all locations on the media under nominal conditions.

If a host runs at a faster cycle rate by operating at a cycle

time of less than this value, the device may negate DMARQ for

flow control.  The rate at which DMARQ is negated could result in

reduced throughput despite the faster cycled rate.  Transfer at

this rate does not ensure that flow control will not be used, but

implies that higher performance MAY result.

If this field is supported, bit 1 of word 53 must be set.

Any device which supports Multiword DMA Mode 1 or above shall

support this field, and the value in word 66 shall not be less

than the value in word 65.

If bit 1 of word 53 is set because a device supports a field in

Words 64-70 other than this field and the device does not support

this field, the device shall return a value of zero in this

field.

8.10.32.        Word 67: Minimum PIO Transfer Cycle Time Without Flow Control

Word 67 of the parameter information of the IDENTIFY DEVICE

command is defined as the Minimum PIO Transfer Without Flow

Control Cycle Time.  This field defines, in nanoseconds, the

minimum cycle time that, if used by the host, the device

guarantees data integrity during the transfer without utilization

of flow control.

Any device may support this field, and if this field is

supported, Bit 1 of word 53 shall be set.

Any device which supports PIO Mode 3 or above must support this

field, and the value in word 67 shall not be less than the value

reported in word 68.

If bit 1 of word 53 is set because a device supports a field in

Words 64-70 other than this field and the device does not support

this field, the device shall return a value of zero in this

field.

8.10.33.        Word 68: Minimum PIO Transfer Cycle Time With IORDY

Word 68 of the parameter information of the IDENTIFY DEVICE

command is defined as the Minimum PIO Transfer With IORDY Flow

Control Cycle Time.  This field defines, in nanoseconds, the

minimum cycle time that the device can support while performing

data transfers while utilizing IORDY flow control.

Any device may support this field, and if this field is

supported, Bit 1 of word 53 must be set.

Any device which supports PIO Mode 3 or above must support this

field, and the value in word 68 shall not be less than the

fastest PIO mode reported by the device.

If bit 1 of word 53 is set because a device supports a field in

Words 64-70 other than this field and the device does not support

this field, the device shall return a value of zero in this

field.

8.10.34.  Words 69-70: Reserved for future PIO modes.

Words 69 and 70 are reserved for future advanced PIO modes.

(Editor's note:  It is intended that this field be used for future specification

of an alternative flow control mechanism, currently being referred to as

advanced PIO flow control.)

8.10.35.  Words 71-127: Reserved.

Words 69 and 70 are reserved for future advanced PIO modes.

8.10.36.  Words 128-159: Reserved.

8.10.37.  Words 160-255: Vendor specific.

-----end

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 07:06:22 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23394; Thu, 25 Aug 94 07:06:22 PDT

Received: from netcomsv.netcom.com (uucp4.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA23387; Thu, 25 Aug 94 07:06:10 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id GAA13270; Thu, 25 Aug 1994 06:58:51 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdfBH-000gJTC; Thu, 25 Aug 94 07:49 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Thu, 25 Aug 1994 07:54:03 EDT

Date:      Thu, 25 Aug 1994 07:53:58 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5c865b.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Cc: "Satish Kholay"      <kholay_s@vlsi.com>

Subject:   please send meeting notice to ...

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Satish Kholay at VLSI, kholay_s@vlsi.com, needs a copy of next

week's meeting notice.  I can't seem to put my cursor on one

anywhere.  Would someone please send him a copy for me?  Of

course he will probably get 25 of them but that's OK.  His email

was messed up until recently and he missed the notice before.

Thanks!

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 07:30:23 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23860; Thu, 25 Aug 94 07:30:23 PDT

Received: from zeus.maxtor.com by dt.wdc.com (4.1/SMI-4.1)

id AA23853; Thu, 25 Aug 94 07:30:21 PDT

Received: from sanjose3.maxtor.com ([134.6.34.200]) by zeus.maxtor.com (8.6.9/8.6.9) with SMTP id HAA21068 for <ata@dt.wdc.com>; Thu, 25 Aug 1994 07:31:14 -0700

From: Ray=Heineman%ENG12%MCO@sanjose3.maxtor.com

Message-Id: <199408251431.HAA21068@zeus.maxtor.com>

Received: by sanjose3.maxtor.com with VINES ; Thu, 25 Aug 94 07:23:38 PDT

Date: Thu, 25 Aug 94 07:23:31 PDT

Subject: ATA2 Meeting Notice

To: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

8/25/94

*************************************************************************

*******************   M E E T I N G   N O T I C E S  ********************

*************************************************************************

SUBJECT:	ATAPI Study Group Meeting (August 30th,1994)

ATA-2 Working Group Meeting (August 31st & September

1st,1994)

DATE:		8/30, 8/31 & 9/1/94

TIME:		9:00 AM - 5 PM

PLACE:		Maxtor Corporation

251 River Oaks Parkway

San Jose, CA  95134

(408) 432-1700 (ask for BLDG 251 Lobby if you are lost)

HOST:		Ray Heineman

*************************************************************************

Steve Finch is chairing the ATA-2 meeting.

Tom Hanan is chairing the ATAPI meeting.

Ray Heineman is hosting the meetings.

DIRECTIONS:

FROM San Jose Airport:

There are two exits from the airport - the main exit

and one that goes to Coleman Avenue.

Use the main exit/entrance from the airport.

Turn left onto Guadalupe Parkway. Stay to the right.

Take the North First Street exit off Guadalupe.

Turn left (north) at North First Street.

Turn right (east) at River Oaks Parkway.

Go through the intersection with Zanker Road; Maxtor

will be on your left about a 1/4 mile beyond the

intersection.

Bldg 251 is the one in the center. Come to that lobby.

FROM San Francisco Airport:

Take Highway 101 Expressway south.

Take the 237 east exit.

Turn right (south) at North First Street.

Turn left (east) at River Oaks Parkway.

FROM Oakland Airport:

Take 880 south.

Take Montague west exit.

Turn right at North First Street.

Turn right at River Oaks Parkway.

HOTELS:

There are many choices, the most convenant are:

Sheraton Hotel

Milpitas

Hiway 880 & Montague Expressway

Holiday Inn

Milpitas

Hiway 880 & Hiway 237

************************************************************************

Regards

Ray Heineman

Maxtor Corporation

Phone 303 678 2591

Fax 303 678 2308

E-mail ray_heineman@Maxtor.com

From ata-owner  Thu Aug 25 07:45:27 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA24244; Thu, 25 Aug 94 07:45:27 PDT

Received: from zeus.maxtor.com by dt.wdc.com (4.1/SMI-4.1)

id AA24237; Thu, 25 Aug 94 07:45:25 PDT

Received: from sanjose3.maxtor.com ([134.6.34.200]) by zeus.maxtor.com (8.6.9/8.6.9) with SMTP id HAA21748 for <ata@dt.wdc.com>; Thu, 25 Aug 1994 07:49:34 -0700

From: Ray=Heineman%ENG12%MCO@sanjose3.maxtor.com

Message-Id: <199408251449.HAA21748@zeus.maxtor.com>

Received: by sanjose3.maxtor.com with VINES ; Thu, 25 Aug 94 07:41:59 PDT

Date: Thu, 25 Aug 94 07:41:38 PDT

Subject: ATA2 Section 8

To: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

There seems to be a couple of paragraphs missing

in section 8. Command Description.

There use to be two paragraphs which explained:

If a new command is issued to a drive which has an

uncompleted command in progress, the drive shall

immediately respond to the new command.

What happened to these paragraphs?

From ata-owner  Thu Aug 25 09:22:03 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA26360; Thu, 25 Aug 94 09:22:03 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA26353; Thu, 25 Aug 94 09:22:01 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA20077; Thu, 25 Aug 1994 09:21:59 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA23065; Thu, 25 Aug 1994 10:20:33 -0600

Message-Id: <9408251620.AA23065@hpgrla.gr.hp.com>

From: HLandis@CMS.gr.hp.com (Hale Landis)

Date: Tue, 23 Aug 94 16:19

To: ata@dt.wdc.com, atapi@dt.wdc.com

Subject: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 17:52

>X-Envelope-To: iandavies@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 17:52

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA18427; Tue, 23 Aug 1994 16:50:32 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA06991; Tue, 23 Aug 94 16:36:59 PDT

>Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

>	id AA06863; Tue, 23 Aug 94 16:35:28 PDT

>Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

>	id QAA26283; Tue, 23 Aug 1994 16:02:30 -0700

>Received: from sugs by tware.tware.com with uucp

>	(Smail3.1.28.1 #3) id m0qd4KP-000gJTC; Tue, 23 Aug 94 16:28 MDT

>Received:  by sugs.tware.com (UUPC/extended 1.11q);

>           Tue, 23 Aug 1994 16:19:14 EDT

>Date:      Tue, 23 Aug 1994 16:19:13 EDT

>From: "Hale Landis" <landis@sugs.tware.com>

>Message-Id: <2e5a59c2.sugs@sugs.tware.com>

>Organization: SUGS

>Reply-To: "Hale Landis" <landis@sugs.tware.com>

>To: "ATA Reflector"      <ata@dt.wdc.com>,

>        "ATAPI Reflector"    <atapi@dt.wdc.com>

>Subject:   PIO Mode 3+ ATA Host Adapters and ATAPI

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Tony Pione of IBM posted the following message to the ATAPI

reflector.  I'm sending my follow up to the ATAPI and ATA

reflectors.  The text of Tony's message is preceeded by ">"

characters.

>  I would like to touch on the pre-fetch function definition next

>  week at our meeting.  There are many pre-fetching controllers out

>  there that aren't real compatible with ATAPI CD-ROM.

Yep, no surprise to me here.  It has been my opinion all along

that ATA and ATAPI were two separate interfaces with little in

common.  A close investigation of ATAPI would probably reveal

that it is very incompatible with ATA in a lot of areas and it

will become even more incompatible as ATA evolves to ATA-3.  I

fully expect that ATA-3 will require some level of intelligence

in the host adapter.  We are starting to see this today with

ATA-2 devices and new ATA PIO Mode 3+ host adapters.  It is this

low level of intelligence we see today -- the sector prefeching

for example -- that Tony has commented on here.

But just wait, the high performance ATA host adapter people will

probably come up with even more goodies that will mess up ATAPI.

ATAPI may have started as a cheap way to attach a non-ATA device

to the usually unused second drive plug on a PC's ATA host

adapter.  And this may work with an ATA-1 hard disc and an ATAPI

device on an old style ATA-1 host adapter.  Unfortunately for the

ATAPI folks, systems with ATA-1 host adapters are dinosaurs.  A

good(?) system design today would include an ATA-2 host adapter

for the hard disc and a second ATA-1 host adapter for the ATAPI

(and perhaps other ATA-1 style devices).  Two host adapters for

two different interfaces.

>  I propose that we put forth some design requirements for

>  pre-fetching controllers that won't interfere with ATAPI data

>  xfers.

Hmmm...  This may not be easy.  Most of the folks building these

host adapters don't come to the ATA meetings (and in some cases

don't even have a copy of the ATA document -- I've talked to some

of them!).  Plus, what you are talking about here may be the

functional equivalent of a CAM effort -- an effort to define how

a host adatper should interface with the host and how it should

handle various types of requests.  Indeed, that may be

appropriate for ATAPI, since one of the things ATAPI is, is

another attempt to figure out a cheap way to attach SCSI devices

to a PC.  I don't think ATA, at this time, needs such a effort --

ATA-3 may well need such an effort, but I hope not.

>  We know of one problem where the controller assumes 512 byte

>  xfers and doesn't handle the raw sector format well because of

>  it.  An extra erroneous word gets transferred between successive

>  single sector requests with this controller.

>  This is really more of an ATA global issue.

I don't think so (or I hope it doesn't become an ATA global

issue).  I really believe that we are moving to the day when

there is a single ATA device per host adapter, especially for ATA

hard discs and especially on higher performance systems.  A

single device per host adapter not because that is what ATA-3+

requires but because it solves so many of the existing ATA

problems, such as overlapped commands, master/slave issues

(especially if the ATA drive is configured as drive 1), data

transfer speed mismatch problems, and others, while remaining in

the whole completely ATA compatible.

On the other hand, if this really is an ATA global issue, then it

may well be the second nail in the coffin for ATA (over 528MB is

the first nail).  This could finally make ATA as complex,

difficult to design, implement, configure and expensive to the

point that SCSI became an attractive alternative for PC systems.

Of course that would also spell the end for ATAPI too -- why have

a SCSI subset when you can have the real thing for a similar

cost?

[Oh boy, I'm sure this is going to stir up some interesting

mail!]

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 10:10:54 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27555; Thu, 25 Aug 94 10:10:54 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA27547; Thu, 25 Aug 94 10:10:52 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA24330; Thu, 25 Aug 1994 10:10:51 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA26935; Thu, 25 Aug 1994 11:09:24 -0600

Message-Id: <9408251709.AA26935@hpgrla.gr.hp.com>

From: HLandis@CMS.gr.hp.com (Hale Landis)

Date: Tue, 23 Aug 94 16:19

To: ata@dt.wdc.com, atapi@dt.wdc.com

Subject: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 17:52

>X-Envelope-To: johnmoor@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 17:52

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA18427; Tue, 23 Aug 1994 16:50:32 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA06991; Tue, 23 Aug 94 16:36:59 PDT

>Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

>	id AA06863; Tue, 23 Aug 94 16:35:28 PDT

>Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

>	id QAA26283; Tue, 23 Aug 1994 16:02:30 -0700

>Received: from sugs by tware.tware.com with uucp

>	(Smail3.1.28.1 #3) id m0qd4KP-000gJTC; Tue, 23 Aug 94 16:28 MDT

>Received:  by sugs.tware.com (UUPC/extended 1.11q);

>           Tue, 23 Aug 1994 16:19:14 EDT

>Date:      Tue, 23 Aug 1994 16:19:13 EDT

>From: "Hale Landis" <landis@sugs.tware.com>

>Message-Id: <2e5a59c2.sugs@sugs.tware.com>

>Organization: SUGS

>Reply-To: "Hale Landis" <landis@sugs.tware.com>

>To: "ATA Reflector"      <ata@dt.wdc.com>,

>        "ATAPI Reflector"    <atapi@dt.wdc.com>

>Subject:   PIO Mode 3+ ATA Host Adapters and ATAPI

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Tony Pione of IBM posted the following message to the ATAPI

reflector.  I'm sending my follow up to the ATAPI and ATA

reflectors.  The text of Tony's message is preceeded by ">"

characters.

>  I would like to touch on the pre-fetch function definition next

>  week at our meeting.  There are many pre-fetching controllers out

>  there that aren't real compatible with ATAPI CD-ROM.

Yep, no surprise to me here.  It has been my opinion all along

that ATA and ATAPI were two separate interfaces with little in

common.  A close investigation of ATAPI would probably reveal

that it is very incompatible with ATA in a lot of areas and it

will become even more incompatible as ATA evolves to ATA-3.  I

fully expect that ATA-3 will require some level of intelligence

in the host adapter.  We are starting to see this today with

ATA-2 devices and new ATA PIO Mode 3+ host adapters.  It is this

low level of intelligence we see today -- the sector prefeching

for example -- that Tony has commented on here.

But just wait, the high performance ATA host adapter people will

probably come up with even more goodies that will mess up ATAPI.

ATAPI may have started as a cheap way to attach a non-ATA device

to the usually unused second drive plug on a PC's ATA host

adapter.  And this may work with an ATA-1 hard disc and an ATAPI

device on an old style ATA-1 host adapter.  Unfortunately for the

ATAPI folks, systems with ATA-1 host adapters are dinosaurs.  A

good(?) system design today would include an ATA-2 host adapter

for the hard disc and a second ATA-1 host adapter for the ATAPI

(and perhaps other ATA-1 style devices).  Two host adapters for

two different interfaces.

>  I propose that we put forth some design requirements for

>  pre-fetching controllers that won't interfere with ATAPI data

>  xfers.

Hmmm...  This may not be easy.  Most of the folks building these

host adapters don't come to the ATA meetings (and in some cases

don't even have a copy of the ATA document -- I've talked to some

of them!).  Plus, what you are talking about here may be the

functional equivalent of a CAM effort -- an effort to define how

a host adatper should interface with the host and how it should

handle various types of requests.  Indeed, that may be

appropriate for ATAPI, since one of the things ATAPI is, is

another attempt to figure out a cheap way to attach SCSI devices

to a PC.  I don't think ATA, at this time, needs such a effort --

ATA-3 may well need such an effort, but I hope not.

>  We know of one problem where the controller assumes 512 byte

>  xfers and doesn't handle the raw sector format well because of

>  it.  An extra erroneous word gets transferred between successive

>  single sector requests with this controller.

>  This is really more of an ATA global issue.

I don't think so (or I hope it doesn't become an ATA global

issue).  I really believe that we are moving to the day when

there is a single ATA device per host adapter, especially for ATA

hard discs and especially on higher performance systems.  A

single device per host adapter not because that is what ATA-3+

requires but because it solves so many of the existing ATA

problems, such as overlapped commands, master/slave issues

(especially if the ATA drive is configured as drive 1), data

transfer speed mismatch problems, and others, while remaining in

the whole completely ATA compatible.

On the other hand, if this really is an ATA global issue, then it

may well be the second nail in the coffin for ATA (over 528MB is

the first nail).  This could finally make ATA as complex,

difficult to design, implement, configure and expensive to the

point that SCSI became an attractive alternative for PC systems.

Of course that would also spell the end for ATAPI too -- why have

a SCSI subset when you can have the real thing for a similar

cost?

[Oh boy, I'm sure this is going to stir up some interesting

mail!]

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 10:55:14 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28613; Thu, 25 Aug 94 10:55:14 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA28601; Thu, 25 Aug 94 10:55:11 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA28763; Thu, 25 Aug 1994 10:55:10 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA29815; Thu, 25 Aug 1994 11:53:43 -0600

Message-Id: <9408251753.AA29815@hpgrla.gr.hp.com>

From: HLandis@CMS.gr.hp.com (Hale Landis)

Date: Tue, 23 Aug 94 16:19

To: ata@dt.wdc.com, atapi@dt.wdc.com

Subject: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 17:52

>X-Envelope-To: CHADTHOM@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 17:52

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA18427; Tue, 23 Aug 1994 16:50:32 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA06991; Tue, 23 Aug 94 16:36:59 PDT

>Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

>	id AA06863; Tue, 23 Aug 94 16:35:28 PDT

>Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

>	id QAA26283; Tue, 23 Aug 1994 16:02:30 -0700

>Received: from sugs by tware.tware.com with uucp

>	(Smail3.1.28.1 #3) id m0qd4KP-000gJTC; Tue, 23 Aug 94 16:28 MDT

>Received:  by sugs.tware.com (UUPC/extended 1.11q);

>           Tue, 23 Aug 1994 16:19:14 EDT

>Date:      Tue, 23 Aug 1994 16:19:13 EDT

>From: "Hale Landis" <landis@sugs.tware.com>

>Message-Id: <2e5a59c2.sugs@sugs.tware.com>

>Organization: SUGS

>Reply-To: "Hale Landis" <landis@sugs.tware.com>

>To: "ATA Reflector"      <ata@dt.wdc.com>,

>        "ATAPI Reflector"    <atapi@dt.wdc.com>

>Subject:   PIO Mode 3+ ATA Host Adapters and ATAPI

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Tony Pione of IBM posted the following message to the ATAPI

reflector.  I'm sending my follow up to the ATAPI and ATA

reflectors.  The text of Tony's message is preceeded by ">"

characters.

>  I would like to touch on the pre-fetch function definition next

>  week at our meeting.  There are many pre-fetching controllers out

>  there that aren't real compatible with ATAPI CD-ROM.

Yep, no surprise to me here.  It has been my opinion all along

that ATA and ATAPI were two separate interfaces with little in

common.  A close investigation of ATAPI would probably reveal

that it is very incompatible with ATA in a lot of areas and it

will become even more incompatible as ATA evolves to ATA-3.  I

fully expect that ATA-3 will require some level of intelligence

in the host adapter.  We are starting to see this today with

ATA-2 devices and new ATA PIO Mode 3+ host adapters.  It is this

low level of intelligence we see today -- the sector prefeching

for example -- that Tony has commented on here.

But just wait, the high performance ATA host adapter people will

probably come up with even more goodies that will mess up ATAPI.

ATAPI may have started as a cheap way to attach a non-ATA device

to the usually unused second drive plug on a PC's ATA host

adapter.  And this may work with an ATA-1 hard disc and an ATAPI

device on an old style ATA-1 host adapter.  Unfortunately for the

ATAPI folks, systems with ATA-1 host adapters are dinosaurs.  A

good(?) system design today would include an ATA-2 host adapter

for the hard disc and a second ATA-1 host adapter for the ATAPI

(and perhaps other ATA-1 style devices).  Two host adapters for

two different interfaces.

>  I propose that we put forth some design requirements for

>  pre-fetching controllers that won't interfere with ATAPI data

>  xfers.

Hmmm...  This may not be easy.  Most of the folks building these

host adapters don't come to the ATA meetings (and in some cases

don't even have a copy of the ATA document -- I've talked to some

of them!).  Plus, what you are talking about here may be the

functional equivalent of a CAM effort -- an effort to define how

a host adatper should interface with the host and how it should

handle various types of requests.  Indeed, that may be

appropriate for ATAPI, since one of the things ATAPI is, is

another attempt to figure out a cheap way to attach SCSI devices

to a PC.  I don't think ATA, at this time, needs such a effort --

ATA-3 may well need such an effort, but I hope not.

>  We know of one problem where the controller assumes 512 byte

>  xfers and doesn't handle the raw sector format well because of

>  it.  An extra erroneous word gets transferred between successive

>  single sector requests with this controller.

>  This is really more of an ATA global issue.

I don't think so (or I hope it doesn't become an ATA global

issue).  I really believe that we are moving to the day when

there is a single ATA device per host adapter, especially for ATA

hard discs and especially on higher performance systems.  A

single device per host adapter not because that is what ATA-3+

requires but because it solves so many of the existing ATA

problems, such as overlapped commands, master/slave issues

(especially if the ATA drive is configured as drive 1), data

transfer speed mismatch problems, and others, while remaining in

the whole completely ATA compatible.

On the other hand, if this really is an ATA global issue, then it

may well be the second nail in the coffin for ATA (over 528MB is

the first nail).  This could finally make ATA as complex,

difficult to design, implement, configure and expensive to the

point that SCSI became an attractive alternative for PC systems.

Of course that would also spell the end for ATAPI too -- why have

a SCSI subset when you can have the real thing for a similar

cost?

[Oh boy, I'm sure this is going to stir up some interesting

mail!]

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 10:55:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28640; Thu, 25 Aug 94 10:55:49 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA28633; Thu, 25 Aug 94 10:55:47 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA28806; Thu, 25 Aug 1994 10:55:46 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA29850; Thu, 25 Aug 1994 11:54:20 -0600

Message-Id: <9408251754.AA29850@hpgrla.gr.hp.com>

From: HLandis@CMS.gr.hp.com (Hale Landis)

Date: Tue, 23 Aug 94 16:19

To: ata@dt.wdc.com, atapi@dt.wdc.com

Subject: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From atapi-owner@dt.wdc.com Tue 23 Aug 1994 18:04

>X-Envelope-To: johnmoor@cms.gr.hp.com

>Return-Path: <atapi-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 18:04

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA18942; Tue, 23 Aug 1994 17:02:44 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA07003; Tue, 23 Aug 94 16:37:47 PDT

>Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

>	id AA06863; Tue, 23 Aug 94 16:35:28 PDT

>Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

>	id QAA26283; Tue, 23 Aug 1994 16:02:30 -0700

>Received: from sugs by tware.tware.com with uucp

>	(Smail3.1.28.1 #3) id m0qd4KP-000gJTC; Tue, 23 Aug 94 16:28 MDT

>Received:  by sugs.tware.com (UUPC/extended 1.11q);

>           Tue, 23 Aug 1994 16:19:14 EDT

>Date:      Tue, 23 Aug 1994 16:19:13 EDT

>From: "Hale Landis" <landis@sugs.tware.com>

>Message-Id: <2e5a59c2.sugs@sugs.tware.com>

>Organization: SUGS

>Reply-To: "Hale Landis" <landis@sugs.tware.com>

>To: "ATA Reflector"      <ata@dt.wdc.com>,

>        "ATAPI Reflector"    <atapi@dt.wdc.com>

>Subject:   PIO Mode 3+ ATA Host Adapters and ATAPI

>Sender: atapi-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Tony Pione of IBM posted the following message to the ATAPI

reflector.  I'm sending my follow up to the ATAPI and ATA

reflectors.  The text of Tony's message is preceeded by ">"

characters.

>  I would like to touch on the pre-fetch function definition next

>  week at our meeting.  There are many pre-fetching controllers out

>  there that aren't real compatible with ATAPI CD-ROM.

Yep, no surprise to me here.  It has been my opinion all along

that ATA and ATAPI were two separate interfaces with little in

common.  A close investigation of ATAPI would probably reveal

that it is very incompatible with ATA in a lot of areas and it

will become even more incompatible as ATA evolves to ATA-3.  I

fully expect that ATA-3 will require some level of intelligence

in the host adapter.  We are starting to see this today with

ATA-2 devices and new ATA PIO Mode 3+ host adapters.  It is this

low level of intelligence we see today -- the sector prefeching

for example -- that Tony has commented on here.

But just wait, the high performance ATA host adapter people will

probably come up with even more goodies that will mess up ATAPI.

ATAPI may have started as a cheap way to attach a non-ATA device

to the usually unused second drive plug on a PC's ATA host

adapter.  And this may work with an ATA-1 hard disc and an ATAPI

device on an old style ATA-1 host adapter.  Unfortunately for the

ATAPI folks, systems with ATA-1 host adapters are dinosaurs.  A

good(?) system design today would include an ATA-2 host adapter

for the hard disc and a second ATA-1 host adapter for the ATAPI

(and perhaps other ATA-1 style devices).  Two host adapters for

two different interfaces.

>  I propose that we put forth some design requirements for

>  pre-fetching controllers that won't interfere with ATAPI data

>  xfers.

Hmmm...  This may not be easy.  Most of the folks building these

host adapters don't come to the ATA meetings (and in some cases

don't even have a copy of the ATA document -- I've talked to some

of them!).  Plus, what you are talking about here may be the

functional equivalent of a CAM effort -- an effort to define how

a host adatper should interface with the host and how it should

handle various types of requests.  Indeed, that may be

appropriate for ATAPI, since one of the things ATAPI is, is

another attempt to figure out a cheap way to attach SCSI devices

to a PC.  I don't think ATA, at this time, needs such a effort --

ATA-3 may well need such an effort, but I hope not.

>  We know of one problem where the controller assumes 512 byte

>  xfers and doesn't handle the raw sector format well because of

>  it.  An extra erroneous word gets transferred between successive

>  single sector requests with this controller.

>  This is really more of an ATA global issue.

I don't think so (or I hope it doesn't become an ATA global

issue).  I really believe that we are moving to the day when

there is a single ATA device per host adapter, especially for ATA

hard discs and especially on higher performance systems.  A

single device per host adapter not because that is what ATA-3+

requires but because it solves so many of the existing ATA

problems, such as overlapped commands, master/slave issues

(especially if the ATA drive is configured as drive 1), data

transfer speed mismatch problems, and others, while remaining in

the whole completely ATA compatible.

On the other hand, if this really is an ATA global issue, then it

may well be the second nail in the coffin for ATA (over 528MB is

the first nail).  This could finally make ATA as complex,

difficult to design, implement, configure and expensive to the

point that SCSI became an attractive alternative for PC systems.

Of course that would also spell the end for ATAPI too -- why have

a SCSI subset when you can have the real thing for a similar

cost?

[Oh boy, I'm sure this is going to stir up some interesting

mail!]

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 10:56:14 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28711; Thu, 25 Aug 94 10:56:14 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA28703; Thu, 25 Aug 94 10:56:11 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA28860; Thu, 25 Aug 1994 10:56:10 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA29880; Thu, 25 Aug 1994 11:54:44 -0600

Message-Id: <9408251754.AA29880@hpgrla.gr.hp.com>

From: RRutledg@CMS.gr.hp.com (Rich Rutledge)

Date: Wed, 24 Aug 94 02:58

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com, ATA-OWNER@dt.wdc.com

Cc: ATA@dt.wdc.com, ATAPI@dt.wdc.com

Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 20:55

>X-Envelope-To: CHADTHOM@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 20:55

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA25602; Tue, 23 Aug 1994 19:53:17 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA10940; Tue, 23 Aug 94 19:41:59 PDT

>Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA10932; Tue, 23 Aug 94 19:41:56 PDT

>Received: with SMTP-MR; Tue, 23 Aug 1994 19:36:30 PST

>Mr-Received: by mta WDUS04; Relayed; Tue, 23 Aug 1994 19:36:30 -0800 (PST)

>Alternate-Recipient: prohibited

>Disclose-Recipients: prohibited

>Date: Wed, 24 Aug 1994 02:58:00 PST

>From: "Rich Rutledge (714) 932-7205 [TEL]" <RUTLEDGE_R@A1.WDC.COM>

>Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

>To: "ata-owner@dt.wdc.com" <ATA-OWNER@dt.wdc.com>

>Cc: ATA Reflector <ATA@dt.wdc.com>, ATAPI Reflector <ATAPI@dt.wdc.com>

>Message-Id: <01HG9DR9LXRO8ZEFG3@wdus03.wdc.com>

>Mime-Version: 1.0

>Content-Type: TEXT/PLAIN

>Posting-Date: Wed, 24 Aug 1994 03:36:00 PST

>Importance: normal

>Priority: normal

>X400-Mts-Identifier: [;03639132804991/541312@WDUSA]

>A1-Type: MAIL

>Hop-Count: 0

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Hale,

We all have our own opinions and outlooks regarding a variety of matters.

With regards to IDE/ATA, my outlook is that SFF/X3T10 are chartered to

develop specifications/standards that satisfy the requests of the

customers.  It often appears that some folks assume their role is telling

the customers what they can or can not do.  My suggestion is to listen to

what the customers want and that is more low cost IDE functionality.

Having said this, the customers generally like the cost effective

functionality that IDE/ATA has long delivered for disk drives.  Lets face

it, cost is king in the mainstream PC market.  Just look at how succesful

Compaq has been with their low price strategy.  It should come as no

surprise to anybody that a need existed to extend IDE's capabilities.  The

528 Mbyte barrier was long ago overcome on AT bus ESDI controllers via

translating BIOS (did we all forget 760 Mbyte ESDI drives that too required

the 528 Mbyte barrier to be broken?).  SCSI adapters have always had to

translate LBA to Int 13 CHS.  This one was easy and hardly a nail in a

coffin.

Our video brothers deserve special thanks for driving the needs for local

bus as this has allowed us to free our disk drives from the demands of the

ISA bus.  Once local bus became a mainstream feature in new systems, it was

again pretty straightforward to exploit it.  We all know that the CPU is

more effective at doing 32 bit PIO transfers, so little surprise that the

LSI devices that connect IDE to local bus (Appian, Adaptec, CMD, Intel,

Opti, PC Tech, others) take advantage of this fact.  Doing so requires the

device to initiate its own IDE cycle, thus pre-fetch.  Now the chip needs

to know when to stop pre-fetching.  This is the issue raised by Tony

Pionne.  Many chips pre-fetch using 512 byte transfer counters as this is

the disk block size.  This is not the case for CD-ROM and Tape.

So, this is an implementation detail whereby the local bus IDE

controller/system simply must be ATAPI aware and if so, then pre-fetch must

be disabled for either the ATAPI device or for the IDE channel that has the

ATAPI device as some LSI chips have programmable features on a device basis

while others have them on a channel basis.

This is done by either the BIOS or a pre-boot driver at boot time and is

commonly known as ATAPI AWARENESS.  It goes right along with the code that

takes a look at IDENTIFY DRIVE date for geometry and data transfer timing

parameters.  Disabling pre-fetch for ATAPI devices really doesn't do much

to performance as non disk peripherals have quite low data rate needs.

This is really an implementation issue between the BIOS/driver and the

hardware.

What really happened here is a failure for the industry to COOPERATE before

it jumped into the COMPETITIVE phase.  Wouldn't it be ideal if there was a

single, standard register set for IDE bridge controller silicon!  Alas,

this snake got out of the can before it could be controlled.  This same

mistake was not repeated for PCI DMA IDE, witness the register level

specification worked out by Intel/NCR/WD/others in ADVANCE of product

shipments.  This should dramatically facilitate low cost, bus mastered,

multi-channel IDE data transfers.  Perhaps the LSI device suppliers could

get together and author a SFF proposal.  My opinion is that this is too

late.

This lack of a register level standard is one of the motivators for the

Phoenix Enhanced Disk Drive specification which provides a STANDARD way to

determine the hardware capabilities offered by the system/controller.

The other missed opportunity was to agree on a standard translation

algorithm for >528 Mbyte IDE.  Everybody remember taking a drive formated

on a Vendor A host adapter and try to run it on a machine that has a Vendor

B host adapter?  Again, another problem that could have been elinated with

a little upfront cooperation.....which was scuttled at the pre-meeting

before the 11/93 Comdex High Capacity Storage Integration Conference when

it became obvious that already completed implementations prevented

agreement....too bad as this would have made the end-users live far more

pleasent.

The past 18 months has seen multiple instances of fast paced, upfront

widespread industry cooperation to allow long term benefit, specifically:

1. Plug and Play SCSI

2. SCAM

3. ATAPI CD-ROMs (SFF-8020 and SFF-8029)

4. Bus Master IDE (SFF 8036 I think)

5. ATAPI Tape (QIC 157)

6. Local Bus IDE Mode 3/1 (SFF 8011)

7. Local Bus IDE Mode 4/2 (SFF 8033)

My own calculations indicate that 1995 will see IDE overtake SA-450 (aka

floppy) as the highest volume peripheral interface as IDE disk + IDE CD-ROM

+ IDE tape will outship 3.5" floppy + 5.25" floppy + floppy tape.  The

sheer dollar and unit volume will continue to draw industry focus to IDE

for internal storage application needs.  Until something comes along that

is both cheaper and better, it will have a hard time entering the

mainstream PC marketplace.  Look for its capabilities to continue to

increase as the PC market is dominated by making the existing thing faster

and cheaper instead of moving to something entirely new.  Examples include

ECP, Enhanced IDE, Super VGA, Intel compatible CPUs, etc.

I believe that Centronics defined the "Centronics Parallel Port" in the

mid 60's and have been out of business since the mid '80s...yet it is still

very much alive after almost 30 years.  IDE evolved from the PC/AT hard

disk controller in 1984, thus it is 10 years into its live.  Time will tell

if it too lives as long as the Centronics Parallel Port.

Rich Rutledge

Western Digital

From ata-owner  Thu Aug 25 10:55:56 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28662; Thu, 25 Aug 94 10:55:56 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA28655; Thu, 25 Aug 94 10:55:54 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA28821; Thu, 25 Aug 1994 10:55:53 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA29859; Thu, 25 Aug 1994 11:54:27 -0600

Message-Id: <9408251754.AA29859@hpgrla.gr.hp.com>

From: HLandis@CMS.gr.hp.com (Hale Landis)

Date: Tue, 23 Aug 94 16:19

To: ata@dt.wdc.com, atapi@dt.wdc.com

Subject: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From atapi-owner@dt.wdc.com Tue 23 Aug 1994 18:04

>X-Envelope-To: CHADTHOM@cms.gr.hp.com

>Return-Path: <atapi-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 18:04

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA18942; Tue, 23 Aug 1994 17:02:44 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA07003; Tue, 23 Aug 94 16:37:47 PDT

>Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

>	id AA06863; Tue, 23 Aug 94 16:35:28 PDT

>Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

>	id QAA26283; Tue, 23 Aug 1994 16:02:30 -0700

>Received: from sugs by tware.tware.com with uucp

>	(Smail3.1.28.1 #3) id m0qd4KP-000gJTC; Tue, 23 Aug 94 16:28 MDT

>Received:  by sugs.tware.com (UUPC/extended 1.11q);

>           Tue, 23 Aug 1994 16:19:14 EDT

>Date:      Tue, 23 Aug 1994 16:19:13 EDT

>From: "Hale Landis" <landis@sugs.tware.com>

>Message-Id: <2e5a59c2.sugs@sugs.tware.com>

>Organization: SUGS

>Reply-To: "Hale Landis" <landis@sugs.tware.com>

>To: "ATA Reflector"      <ata@dt.wdc.com>,

>        "ATAPI Reflector"    <atapi@dt.wdc.com>

>Subject:   PIO Mode 3+ ATA Host Adapters and ATAPI

>Sender: atapi-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Tony Pione of IBM posted the following message to the ATAPI

reflector.  I'm sending my follow up to the ATAPI and ATA

reflectors.  The text of Tony's message is preceeded by ">"

characters.

>  I would like to touch on the pre-fetch function definition next

>  week at our meeting.  There are many pre-fetching controllers out

>  there that aren't real compatible with ATAPI CD-ROM.

Yep, no surprise to me here.  It has been my opinion all along

that ATA and ATAPI were two separate interfaces with little in

common.  A close investigation of ATAPI would probably reveal

that it is very incompatible with ATA in a lot of areas and it

will become even more incompatible as ATA evolves to ATA-3.  I

fully expect that ATA-3 will require some level of intelligence

in the host adapter.  We are starting to see this today with

ATA-2 devices and new ATA PIO Mode 3+ host adapters.  It is this

low level of intelligence we see today -- the sector prefeching

for example -- that Tony has commented on here.

But just wait, the high performance ATA host adapter people will

probably come up with even more goodies that will mess up ATAPI.

ATAPI may have started as a cheap way to attach a non-ATA device

to the usually unused second drive plug on a PC's ATA host

adapter.  And this may work with an ATA-1 hard disc and an ATAPI

device on an old style ATA-1 host adapter.  Unfortunately for the

ATAPI folks, systems with ATA-1 host adapters are dinosaurs.  A

good(?) system design today would include an ATA-2 host adapter

for the hard disc and a second ATA-1 host adapter for the ATAPI

(and perhaps other ATA-1 style devices).  Two host adapters for

two different interfaces.

>  I propose that we put forth some design requirements for

>  pre-fetching controllers that won't interfere with ATAPI data

>  xfers.

Hmmm...  This may not be easy.  Most of the folks building these

host adapters don't come to the ATA meetings (and in some cases

don't even have a copy of the ATA document -- I've talked to some

of them!).  Plus, what you are talking about here may be the

functional equivalent of a CAM effort -- an effort to define how

a host adatper should interface with the host and how it should

handle various types of requests.  Indeed, that may be

appropriate for ATAPI, since one of the things ATAPI is, is

another attempt to figure out a cheap way to attach SCSI devices

to a PC.  I don't think ATA, at this time, needs such a effort --

ATA-3 may well need such an effort, but I hope not.

>  We know of one problem where the controller assumes 512 byte

>  xfers and doesn't handle the raw sector format well because of

>  it.  An extra erroneous word gets transferred between successive

>  single sector requests with this controller.

>  This is really more of an ATA global issue.

I don't think so (or I hope it doesn't become an ATA global

issue).  I really believe that we are moving to the day when

there is a single ATA device per host adapter, especially for ATA

hard discs and especially on higher performance systems.  A

single device per host adapter not because that is what ATA-3+

requires but because it solves so many of the existing ATA

problems, such as overlapped commands, master/slave issues

(especially if the ATA drive is configured as drive 1), data

transfer speed mismatch problems, and others, while remaining in

the whole completely ATA compatible.

On the other hand, if this really is an ATA global issue, then it

may well be the second nail in the coffin for ATA (over 528MB is

the first nail).  This could finally make ATA as complex,

difficult to design, implement, configure and expensive to the

point that SCSI became an attractive alternative for PC systems.

Of course that would also spell the end for ATAPI too -- why have

a SCSI subset when you can have the real thing for a similar

cost?

[Oh boy, I'm sure this is going to stir up some interesting

mail!]

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 10:56:34 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28738; Thu, 25 Aug 94 10:56:34 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA28730; Thu, 25 Aug 94 10:56:32 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA28887; Thu, 25 Aug 1994 10:56:30 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA29905; Thu, 25 Aug 1994 11:55:03 -0600

Message-Id: <9408251755.AA29905@hpgrla.gr.hp.com>

From: RRutledg@CMS.gr.hp.com (Rich Rutledge)

Date: Wed, 24 Aug 94 02:58

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com, ATA-OWNER@dt.wdc.com

Cc: ATA@dt.wdc.com, ATAPI@dt.wdc.com

Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From atapi-owner@dt.wdc.com Tue 23 Aug 1994 21:07

>X-Envelope-To: johnmoor@cms.gr.hp.com

>Return-Path: <atapi-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 21:06

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA26136; Tue, 23 Aug 1994 20:03:20 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA10952; Tue, 23 Aug 94 19:42:47 PDT

>Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA10932; Tue, 23 Aug 94 19:41:56 PDT

>Received: with SMTP-MR; Tue, 23 Aug 1994 19:36:30 PST

>Mr-Received: by mta WDUS04; Relayed; Tue, 23 Aug 1994 19:36:30 -0800 (PST)

>Alternate-Recipient: prohibited

>Disclose-Recipients: prohibited

>Date: Wed, 24 Aug 1994 02:58:00 PST

>From: "Rich Rutledge (714) 932-7205 [TEL]" <RUTLEDGE_R@A1.WDC.COM>

>Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

>To: "ata-owner@dt.wdc.com" <ATA-OWNER@dt.wdc.com>

>Cc: ATA Reflector <ATA@dt.wdc.com>, ATAPI Reflector <ATAPI@dt.wdc.com>

>Message-Id: <01HG9DR9LXRO8ZEFG3@wdus03.wdc.com>

>Mime-Version: 1.0

>Content-Type: TEXT/PLAIN

>Posting-Date: Wed, 24 Aug 1994 03:36:00 PST

>Importance: normal

>Priority: normal

>X400-Mts-Identifier: [;03639132804991/541312@WDUSA]

>A1-Type: MAIL

>Hop-Count: 0

>Sender: atapi-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Hale,

We all have our own opinions and outlooks regarding a variety of matters.

With regards to IDE/ATA, my outlook is that SFF/X3T10 are chartered to

develop specifications/standards that satisfy the requests of the

customers.  It often appears that some folks assume their role is telling

the customers what they can or can not do.  My suggestion is to listen to

what the customers want and that is more low cost IDE functionality.

Having said this, the customers generally like the cost effective

functionality that IDE/ATA has long delivered for disk drives.  Lets face

it, cost is king in the mainstream PC market.  Just look at how succesful

Compaq has been with their low price strategy.  It should come as no

surprise to anybody that a need existed to extend IDE's capabilities.  The

528 Mbyte barrier was long ago overcome on AT bus ESDI controllers via

translating BIOS (did we all forget 760 Mbyte ESDI drives that too required

the 528 Mbyte barrier to be broken?).  SCSI adapters have always had to

translate LBA to Int 13 CHS.  This one was easy and hardly a nail in a

coffin.

Our video brothers deserve special thanks for driving the needs for local

bus as this has allowed us to free our disk drives from the demands of the

ISA bus.  Once local bus became a mainstream feature in new systems, it was

again pretty straightforward to exploit it.  We all know that the CPU is

more effective at doing 32 bit PIO transfers, so little surprise that the

LSI devices that connect IDE to local bus (Appian, Adaptec, CMD, Intel,

Opti, PC Tech, others) take advantage of this fact.  Doing so requires the

device to initiate its own IDE cycle, thus pre-fetch.  Now the chip needs

to know when to stop pre-fetching.  This is the issue raised by Tony

Pionne.  Many chips pre-fetch using 512 byte transfer counters as this is

the disk block size.  This is not the case for CD-ROM and Tape.

So, this is an implementation detail whereby the local bus IDE

controller/system simply must be ATAPI aware and if so, then pre-fetch must

be disabled for either the ATAPI device or for the IDE channel that has the

ATAPI device as some LSI chips have programmable features on a device basis

while others have them on a channel basis.

This is done by either the BIOS or a pre-boot driver at boot time and is

commonly known as ATAPI AWARENESS.  It goes right along with the code that

takes a look at IDENTIFY DRIVE date for geometry and data transfer timing

parameters.  Disabling pre-fetch for ATAPI devices really doesn't do much

to performance as non disk peripherals have quite low data rate needs.

This is really an implementation issue between the BIOS/driver and the

hardware.

What really happened here is a failure for the industry to COOPERATE before

it jumped into the COMPETITIVE phase.  Wouldn't it be ideal if there was a

single, standard register set for IDE bridge controller silicon!  Alas,

this snake got out of the can before it could be controlled.  This same

mistake was not repeated for PCI DMA IDE, witness the register level

specification worked out by Intel/NCR/WD/others in ADVANCE of product

shipments.  This should dramatically facilitate low cost, bus mastered,

multi-channel IDE data transfers.  Perhaps the LSI device suppliers could

get together and author a SFF proposal.  My opinion is that this is too

late.

This lack of a register level standard is one of the motivators for the

Phoenix Enhanced Disk Drive specification which provides a STANDARD way to

determine the hardware capabilities offered by the system/controller.

The other missed opportunity was to agree on a standard translation

algorithm for >528 Mbyte IDE.  Everybody remember taking a drive formated

on a Vendor A host adapter and try to run it on a machine that has a Vendor

B host adapter?  Again, another problem that could have been elinated with

a little upfront cooperation.....which was scuttled at the pre-meeting

before the 11/93 Comdex High Capacity Storage Integration Conference when

it became obvious that already completed implementations prevented

agreement....too bad as this would have made the end-users live far more

pleasent.

The past 18 months has seen multiple instances of fast paced, upfront

widespread industry cooperation to allow long term benefit, specifically:

1. Plug and Play SCSI

2. SCAM

3. ATAPI CD-ROMs (SFF-8020 and SFF-8029)

4. Bus Master IDE (SFF 8036 I think)

5. ATAPI Tape (QIC 157)

6. Local Bus IDE Mode 3/1 (SFF 8011)

7. Local Bus IDE Mode 4/2 (SFF 8033)

My own calculations indicate that 1995 will see IDE overtake SA-450 (aka

floppy) as the highest volume peripheral interface as IDE disk + IDE CD-ROM

+ IDE tape will outship 3.5" floppy + 5.25" floppy + floppy tape.  The

sheer dollar and unit volume will continue to draw industry focus to IDE

for internal storage application needs.  Until something comes along that

is both cheaper and better, it will have a hard time entering the

mainstream PC marketplace.  Look for its capabilities to continue to

increase as the PC market is dominated by making the existing thing faster

and cheaper instead of moving to something entirely new.  Examples include

ECP, Enhanced IDE, Super VGA, Intel compatible CPUs, etc.

I believe that Centronics defined the "Centronics Parallel Port" in the

mid 60's and have been out of business since the mid '80s...yet it is still

very much alive after almost 30 years.  IDE evolved from the PC/AT hard

disk controller in 1984, thus it is 10 years into its live.  Time will tell

if it too lives as long as the Centronics Parallel Port.

Rich Rutledge

Western Digital

From ata-owner  Thu Aug 25 10:56:04 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28684; Thu, 25 Aug 94 10:56:04 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA28675; Thu, 25 Aug 94 10:56:02 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA28843; Thu, 25 Aug 1994 10:56:00 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA29868; Thu, 25 Aug 1994 11:54:34 -0600

Message-Id: <9408251754.AA29868@hpgrla.gr.hp.com>

From: RRutledg@CMS.gr.hp.com (Rich Rutledge)

Date: Wed, 24 Aug 94 02:58

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com, ATA-OWNER@dt.wdc.com

Cc: ATA@dt.wdc.com, ATAPI@dt.wdc.com

Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 20:55

>X-Envelope-To: johnmoor@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 20:55

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA25602; Tue, 23 Aug 1994 19:53:17 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA10940; Tue, 23 Aug 94 19:41:59 PDT

>Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA10932; Tue, 23 Aug 94 19:41:56 PDT

>Received: with SMTP-MR; Tue, 23 Aug 1994 19:36:30 PST

>Mr-Received: by mta WDUS04; Relayed; Tue, 23 Aug 1994 19:36:30 -0800 (PST)

>Alternate-Recipient: prohibited

>Disclose-Recipients: prohibited

>Date: Wed, 24 Aug 1994 02:58:00 PST

>From: "Rich Rutledge (714) 932-7205 [TEL]" <RUTLEDGE_R@A1.WDC.COM>

>Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

>To: "ata-owner@dt.wdc.com" <ATA-OWNER@dt.wdc.com>

>Cc: ATA Reflector <ATA@dt.wdc.com>, ATAPI Reflector <ATAPI@dt.wdc.com>

>Message-Id: <01HG9DR9LXRO8ZEFG3@wdus03.wdc.com>

>Mime-Version: 1.0

>Content-Type: TEXT/PLAIN

>Posting-Date: Wed, 24 Aug 1994 03:36:00 PST

>Importance: normal

>Priority: normal

>X400-Mts-Identifier: [;03639132804991/541312@WDUSA]

>A1-Type: MAIL

>Hop-Count: 0

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Hale,

We all have our own opinions and outlooks regarding a variety of matters.

With regards to IDE/ATA, my outlook is that SFF/X3T10 are chartered to

develop specifications/standards that satisfy the requests of the

customers.  It often appears that some folks assume their role is telling

the customers what they can or can not do.  My suggestion is to listen to

what the customers want and that is more low cost IDE functionality.

Having said this, the customers generally like the cost effective

functionality that IDE/ATA has long delivered for disk drives.  Lets face

it, cost is king in the mainstream PC market.  Just look at how succesful

Compaq has been with their low price strategy.  It should come as no

surprise to anybody that a need existed to extend IDE's capabilities.  The

528 Mbyte barrier was long ago overcome on AT bus ESDI controllers via

translating BIOS (did we all forget 760 Mbyte ESDI drives that too required

the 528 Mbyte barrier to be broken?).  SCSI adapters have always had to

translate LBA to Int 13 CHS.  This one was easy and hardly a nail in a

coffin.

Our video brothers deserve special thanks for driving the needs for local

bus as this has allowed us to free our disk drives from the demands of the

ISA bus.  Once local bus became a mainstream feature in new systems, it was

again pretty straightforward to exploit it.  We all know that the CPU is

more effective at doing 32 bit PIO transfers, so little surprise that the

LSI devices that connect IDE to local bus (Appian, Adaptec, CMD, Intel,

Opti, PC Tech, others) take advantage of this fact.  Doing so requires the

device to initiate its own IDE cycle, thus pre-fetch.  Now the chip needs

to know when to stop pre-fetching.  This is the issue raised by Tony

Pionne.  Many chips pre-fetch using 512 byte transfer counters as this is

the disk block size.  This is not the case for CD-ROM and Tape.

So, this is an implementation detail whereby the local bus IDE

controller/system simply must be ATAPI aware and if so, then pre-fetch must

be disabled for either the ATAPI device or for the IDE channel that has the

ATAPI device as some LSI chips have programmable features on a device basis

while others have them on a channel basis.

This is done by either the BIOS or a pre-boot driver at boot time and is

commonly known as ATAPI AWARENESS.  It goes right along with the code that

takes a look at IDENTIFY DRIVE date for geometry and data transfer timing

parameters.  Disabling pre-fetch for ATAPI devices really doesn't do much

to performance as non disk peripherals have quite low data rate needs.

This is really an implementation issue between the BIOS/driver and the

hardware.

What really happened here is a failure for the industry to COOPERATE before

it jumped into the COMPETITIVE phase.  Wouldn't it be ideal if there was a

single, standard register set for IDE bridge controller silicon!  Alas,

this snake got out of the can before it could be controlled.  This same

mistake was not repeated for PCI DMA IDE, witness the register level

specification worked out by Intel/NCR/WD/others in ADVANCE of product

shipments.  This should dramatically facilitate low cost, bus mastered,

multi-channel IDE data transfers.  Perhaps the LSI device suppliers could

get together and author a SFF proposal.  My opinion is that this is too

late.

This lack of a register level standard is one of the motivators for the

Phoenix Enhanced Disk Drive specification which provides a STANDARD way to

determine the hardware capabilities offered by the system/controller.

The other missed opportunity was to agree on a standard translation

algorithm for >528 Mbyte IDE.  Everybody remember taking a drive formated

on a Vendor A host adapter and try to run it on a machine that has a Vendor

B host adapter?  Again, another problem that could have been elinated with

a little upfront cooperation.....which was scuttled at the pre-meeting

before the 11/93 Comdex High Capacity Storage Integration Conference when

it became obvious that already completed implementations prevented

agreement....too bad as this would have made the end-users live far more

pleasent.

The past 18 months has seen multiple instances of fast paced, upfront

widespread industry cooperation to allow long term benefit, specifically:

1. Plug and Play SCSI

2. SCAM

3. ATAPI CD-ROMs (SFF-8020 and SFF-8029)

4. Bus Master IDE (SFF 8036 I think)

5. ATAPI Tape (QIC 157)

6. Local Bus IDE Mode 3/1 (SFF 8011)

7. Local Bus IDE Mode 4/2 (SFF 8033)

My own calculations indicate that 1995 will see IDE overtake SA-450 (aka

floppy) as the highest volume peripheral interface as IDE disk + IDE CD-ROM

+ IDE tape will outship 3.5" floppy + 5.25" floppy + floppy tape.  The

sheer dollar and unit volume will continue to draw industry focus to IDE

for internal storage application needs.  Until something comes along that

is both cheaper and better, it will have a hard time entering the

mainstream PC marketplace.  Look for its capabilities to continue to

increase as the PC market is dominated by making the existing thing faster

and cheaper instead of moving to something entirely new.  Examples include

ECP, Enhanced IDE, Super VGA, Intel compatible CPUs, etc.

I believe that Centronics defined the "Centronics Parallel Port" in the

mid 60's and have been out of business since the mid '80s...yet it is still

very much alive after almost 30 years.  IDE evolved from the PC/AT hard

disk controller in 1984, thus it is 10 years into its live.  Time will tell

if it too lives as long as the Centronics Parallel Port.

Rich Rutledge

Western Digital

From ata-owner  Thu Aug 25 10:56:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28782; Thu, 25 Aug 94 10:56:49 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA28774; Thu, 25 Aug 94 10:56:46 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA28914; Thu, 25 Aug 1994 10:56:44 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA29923; Thu, 25 Aug 1994 11:55:18 -0600

Message-Id: <9408251755.AA29923@hpgrla.gr.hp.com>

From: RRutledg@CMS.gr.hp.com (Rich Rutledge)

Date: Wed, 24 Aug 94 02:58

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com, ATA-OWNER@dt.wdc.com

Cc: ATA@dt.wdc.com, ATAPI@dt.wdc.com

Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From atapi-owner@dt.wdc.com Tue 23 Aug 1994 21:07

>X-Envelope-To: CHADTHOM@cms.gr.hp.com

>Return-Path: <atapi-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 21:06

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA26136; Tue, 23 Aug 1994 20:03:20 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA10952; Tue, 23 Aug 94 19:42:47 PDT

>Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA10932; Tue, 23 Aug 94 19:41:56 PDT

>Received: with SMTP-MR; Tue, 23 Aug 1994 19:36:30 PST

>Mr-Received: by mta WDUS04; Relayed; Tue, 23 Aug 1994 19:36:30 -0800 (PST)

>Alternate-Recipient: prohibited

>Disclose-Recipients: prohibited

>Date: Wed, 24 Aug 1994 02:58:00 PST

>From: "Rich Rutledge (714) 932-7205 [TEL]" <RUTLEDGE_R@A1.WDC.COM>

>Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

>To: "ata-owner@dt.wdc.com" <ATA-OWNER@dt.wdc.com>

>Cc: ATA Reflector <ATA@dt.wdc.com>, ATAPI Reflector <ATAPI@dt.wdc.com>

>Message-Id: <01HG9DR9LXRO8ZEFG3@wdus03.wdc.com>

>Mime-Version: 1.0

>Content-Type: TEXT/PLAIN

>Posting-Date: Wed, 24 Aug 1994 03:36:00 PST

>Importance: normal

>Priority: normal

>X400-Mts-Identifier: [;03639132804991/541312@WDUSA]

>A1-Type: MAIL

>Hop-Count: 0

>Sender: atapi-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Hale,

We all have our own opinions and outlooks regarding a variety of matters.

With regards to IDE/ATA, my outlook is that SFF/X3T10 are chartered to

develop specifications/standards that satisfy the requests of the

customers.  It often appears that some folks assume their role is telling

the customers what they can or can not do.  My suggestion is to listen to

what the customers want and that is more low cost IDE functionality.

Having said this, the customers generally like the cost effective

functionality that IDE/ATA has long delivered for disk drives.  Lets face

it, cost is king in the mainstream PC market.  Just look at how succesful

Compaq has been with their low price strategy.  It should come as no

surprise to anybody that a need existed to extend IDE's capabilities.  The

528 Mbyte barrier was long ago overcome on AT bus ESDI controllers via

translating BIOS (did we all forget 760 Mbyte ESDI drives that too required

the 528 Mbyte barrier to be broken?).  SCSI adapters have always had to

translate LBA to Int 13 CHS.  This one was easy and hardly a nail in a

coffin.

Our video brothers deserve special thanks for driving the needs for local

bus as this has allowed us to free our disk drives from the demands of the

ISA bus.  Once local bus became a mainstream feature in new systems, it was

again pretty straightforward to exploit it.  We all know that the CPU is

more effective at doing 32 bit PIO transfers, so little surprise that the

LSI devices that connect IDE to local bus (Appian, Adaptec, CMD, Intel,

Opti, PC Tech, others) take advantage of this fact.  Doing so requires the

device to initiate its own IDE cycle, thus pre-fetch.  Now the chip needs

to know when to stop pre-fetching.  This is the issue raised by Tony

Pionne.  Many chips pre-fetch using 512 byte transfer counters as this is

the disk block size.  This is not the case for CD-ROM and Tape.

So, this is an implementation detail whereby the local bus IDE

controller/system simply must be ATAPI aware and if so, then pre-fetch must

be disabled for either the ATAPI device or for the IDE channel that has the

ATAPI device as some LSI chips have programmable features on a device basis

while others have them on a channel basis.

This is done by either the BIOS or a pre-boot driver at boot time and is

commonly known as ATAPI AWARENESS.  It goes right along with the code that

takes a look at IDENTIFY DRIVE date for geometry and data transfer timing

parameters.  Disabling pre-fetch for ATAPI devices really doesn't do much

to performance as non disk peripherals have quite low data rate needs.

This is really an implementation issue between the BIOS/driver and the

hardware.

What really happened here is a failure for the industry to COOPERATE before

it jumped into the COMPETITIVE phase.  Wouldn't it be ideal if there was a

single, standard register set for IDE bridge controller silicon!  Alas,

this snake got out of the can before it could be controlled.  This same

mistake was not repeated for PCI DMA IDE, witness the register level

specification worked out by Intel/NCR/WD/others in ADVANCE of product

shipments.  This should dramatically facilitate low cost, bus mastered,

multi-channel IDE data transfers.  Perhaps the LSI device suppliers could

get together and author a SFF proposal.  My opinion is that this is too

late.

This lack of a register level standard is one of the motivators for the

Phoenix Enhanced Disk Drive specification which provides a STANDARD way to

determine the hardware capabilities offered by the system/controller.

The other missed opportunity was to agree on a standard translation

algorithm for >528 Mbyte IDE.  Everybody remember taking a drive formated

on a Vendor A host adapter and try to run it on a machine that has a Vendor

B host adapter?  Again, another problem that could have been elinated with

a little upfront cooperation.....which was scuttled at the pre-meeting

before the 11/93 Comdex High Capacity Storage Integration Conference when

it became obvious that already completed implementations prevented

agreement....too bad as this would have made the end-users live far more

pleasent.

The past 18 months has seen multiple instances of fast paced, upfront

widespread industry cooperation to allow long term benefit, specifically:

1. Plug and Play SCSI

2. SCAM

3. ATAPI CD-ROMs (SFF-8020 and SFF-8029)

4. Bus Master IDE (SFF 8036 I think)

5. ATAPI Tape (QIC 157)

6. Local Bus IDE Mode 3/1 (SFF 8011)

7. Local Bus IDE Mode 4/2 (SFF 8033)

My own calculations indicate that 1995 will see IDE overtake SA-450 (aka

floppy) as the highest volume peripheral interface as IDE disk + IDE CD-ROM

+ IDE tape will outship 3.5" floppy + 5.25" floppy + floppy tape.  The

sheer dollar and unit volume will continue to draw industry focus to IDE

for internal storage application needs.  Until something comes along that

is both cheaper and better, it will have a hard time entering the

mainstream PC marketplace.  Look for its capabilities to continue to

increase as the PC market is dominated by making the existing thing faster

and cheaper instead of moving to something entirely new.  Examples include

ECP, Enhanced IDE, Super VGA, Intel compatible CPUs, etc.

I believe that Centronics defined the "Centronics Parallel Port" in the

mid 60's and have been out of business since the mid '80s...yet it is still

very much alive after almost 30 years.  IDE evolved from the PC/AT hard

disk controller in 1984, thus it is 10 years into its live.  Time will tell

if it too lives as long as the Centronics Parallel Port.

Rich Rutledge

Western Digital

From ata-owner  Thu Aug 25 11:03:12 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29109; Thu, 25 Aug 94 11:03:12 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA29097; Thu, 25 Aug 94 11:03:09 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA29358; Thu, 25 Aug 1994 11:03:08 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA00525; Thu, 25 Aug 1994 12:01:41 -0600

Message-Id: <9408251801.AA00525@hpgrla.gr.hp.com>

From: RRutledg@CMS.gr.hp.com (Rich Rutledge)

Date: Wed, 24 Aug 94 02:58

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com, ATA-OWNER@dt.wdc.com

Cc: ATA@dt.wdc.com, ATAPI@dt.wdc.com

Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 20:55

>X-Envelope-To: iandavies@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 20:55

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA25602; Tue, 23 Aug 1994 19:53:17 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA10940; Tue, 23 Aug 94 19:41:59 PDT

>Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA10932; Tue, 23 Aug 94 19:41:56 PDT

>Received: with SMTP-MR; Tue, 23 Aug 1994 19:36:30 PST

>Mr-Received: by mta WDUS04; Relayed; Tue, 23 Aug 1994 19:36:30 -0800 (PST)

>Alternate-Recipient: prohibited

>Disclose-Recipients: prohibited

>Date: Wed, 24 Aug 1994 02:58:00 PST

>From: "Rich Rutledge (714) 932-7205 [TEL]" <RUTLEDGE_R@A1.WDC.COM>

>Subject: RE: PIO Mode 3+ ATA Host Adapters and ATAPI

>To: "ata-owner@dt.wdc.com" <ATA-OWNER@dt.wdc.com>

>Cc: ATA Reflector <ATA@dt.wdc.com>, ATAPI Reflector <ATAPI@dt.wdc.com>

>Message-Id: <01HG9DR9LXRO8ZEFG3@wdus03.wdc.com>

>Mime-Version: 1.0

>Content-Type: TEXT/PLAIN

>Posting-Date: Wed, 24 Aug 1994 03:36:00 PST

>Importance: normal

>Priority: normal

>X400-Mts-Identifier: [;03639132804991/541312@WDUSA]

>A1-Type: MAIL

>Hop-Count: 0

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

Hale,

We all have our own opinions and outlooks regarding a variety of matters.

With regards to IDE/ATA, my outlook is that SFF/X3T10 are chartered to

develop specifications/standards that satisfy the requests of the

customers.  It often appears that some folks assume their role is telling

the customers what they can or can not do.  My suggestion is to listen to

what the customers want and that is more low cost IDE functionality.

Having said this, the customers generally like the cost effective

functionality that IDE/ATA has long delivered for disk drives.  Lets face

it, cost is king in the mainstream PC market.  Just look at how succesful

Compaq has been with their low price strategy.  It should come as no

surprise to anybody that a need existed to extend IDE's capabilities.  The

528 Mbyte barrier was long ago overcome on AT bus ESDI controllers via

translating BIOS (did we all forget 760 Mbyte ESDI drives that too required

the 528 Mbyte barrier to be broken?).  SCSI adapters have always had to

translate LBA to Int 13 CHS.  This one was easy and hardly a nail in a

coffin.

Our video brothers deserve special thanks for driving the needs for local

bus as this has allowed us to free our disk drives from the demands of the

ISA bus.  Once local bus became a mainstream feature in new systems, it was

again pretty straightforward to exploit it.  We all know that the CPU is

more effective at doing 32 bit PIO transfers, so little surprise that the

LSI devices that connect IDE to local bus (Appian, Adaptec, CMD, Intel,

Opti, PC Tech, others) take advantage of this fact.  Doing so requires the

device to initiate its own IDE cycle, thus pre-fetch.  Now the chip needs

to know when to stop pre-fetching.  This is the issue raised by Tony

Pionne.  Many chips pre-fetch using 512 byte transfer counters as this is

the disk block size.  This is not the case for CD-ROM and Tape.

So, this is an implementation detail whereby the local bus IDE

controller/system simply must be ATAPI aware and if so, then pre-fetch must

be disabled for either the ATAPI device or for the IDE channel that has the

ATAPI device as some LSI chips have programmable features on a device basis

while others have them on a channel basis.

This is done by either the BIOS or a pre-boot driver at boot time and is

commonly known as ATAPI AWARENESS.  It goes right along with the code that

takes a look at IDENTIFY DRIVE date for geometry and data transfer timing

parameters.  Disabling pre-fetch for ATAPI devices really doesn't do much

to performance as non disk peripherals have quite low data rate needs.

This is really an implementation issue between the BIOS/driver and the

hardware.

What really happened here is a failure for the industry to COOPERATE before

it jumped into the COMPETITIVE phase.  Wouldn't it be ideal if there was a

single, standard register set for IDE bridge controller silicon!  Alas,

this snake got out of the can before it could be controlled.  This same

mistake was not repeated for PCI DMA IDE, witness the register level

specification worked out by Intel/NCR/WD/others in ADVANCE of product

shipments.  This should dramatically facilitate low cost, bus mastered,

multi-channel IDE data transfers.  Perhaps the LSI device suppliers could

get together and author a SFF proposal.  My opinion is that this is too

late.

This lack of a register level standard is one of the motivators for the

Phoenix Enhanced Disk Drive specification which provides a STANDARD way to

determine the hardware capabilities offered by the system/controller.

The other missed opportunity was to agree on a standard translation

algorithm for >528 Mbyte IDE.  Everybody remember taking a drive formated

on a Vendor A host adapter and try to run it on a machine that has a Vendor

B host adapter?  Again, another problem that could have been elinated with

a little upfront cooperation.....which was scuttled at the pre-meeting

before the 11/93 Comdex High Capacity Storage Integration Conference when

it became obvious that already completed implementations prevented

agreement....too bad as this would have made the end-users live far more

pleasent.

The past 18 months has seen multiple instances of fast paced, upfront

widespread industry cooperation to allow long term benefit, specifically:

1. Plug and Play SCSI

2. SCAM

3. ATAPI CD-ROMs (SFF-8020 and SFF-8029)

4. Bus Master IDE (SFF 8036 I think)

5. ATAPI Tape (QIC 157)

6. Local Bus IDE Mode 3/1 (SFF 8011)

7. Local Bus IDE Mode 4/2 (SFF 8033)

My own calculations indicate that 1995 will see IDE overtake SA-450 (aka

floppy) as the highest volume peripheral interface as IDE disk + IDE CD-ROM

+ IDE tape will outship 3.5" floppy + 5.25" floppy + floppy tape.  The

sheer dollar and unit volume will continue to draw industry focus to IDE

for internal storage application needs.  Until something comes along that

is both cheaper and better, it will have a hard time entering the

mainstream PC marketplace.  Look for its capabilities to continue to

increase as the PC market is dominated by making the existing thing faster

and cheaper instead of moving to something entirely new.  Examples include

ECP, Enhanced IDE, Super VGA, Intel compatible CPUs, etc.

I believe that Centronics defined the "Centronics Parallel Port" in the

mid 60's and have been out of business since the mid '80s...yet it is still

very much alive after almost 30 years.  IDE evolved from the PC/AT hard

disk controller in 1984, thus it is 10 years into its live.  Time will tell

if it too lives as long as the Centronics Parallel Port.

Rich Rutledge

Western Digital

From ata-owner  Thu Aug 25 12:07:54 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00890; Thu, 25 Aug 94 12:07:54 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA00883; Thu, 25 Aug 94 12:07:52 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA04295; Thu, 25 Aug 1994 12:07:50 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA02279; Thu, 25 Aug 1994 13:06:24 -0600

Message-Id: <9408251906.AA02279@hpgrla.gr.hp.com>

From: DAllan@CMS.gr.hp.com (Dal Allan)

Date: Tue, 23 Aug 94 17:44

To: ata@dt.wdc.com

Subject: 1996_calendar

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 23:57

>X-Envelope-To: iandavies@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 23:57

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA01179; Tue, 23 Aug 1994 22:55:36 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA14378; Tue, 23 Aug 94 22:41:44 PDT

>Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA14365; Tue, 23 Aug 94 22:41:40 PDT

>Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.7/8.6.4) with ESMTP id WAA24600 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:37 -0700

>Received: from zebra.unix.portal.com (zebra.unix.portal.com [156.151.1.103]) by portal.unix.portal.com (8.6.5/8.6.5) with ESMTP id WAA12113 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:34 -0700

>Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id WAA16918 for ata@dt.wdc.com; Tue, 23 Aug 1994 22:02:02 -0700

>Received:  from endl by netmail.com (UUPC/extended 1.12j) with UUCP

>           for ata@dt.wdc.com; Tue, 23 Aug 1994 21:37:15 -0700

>Received:  by endl.com (UUPC/extended 1.12b);

>           Tue, 23 Aug 1994 17:44:44 pdt

>Message-Id: <2e5a97fd.endl@endl.com>

>Date:      Tue, 23 Aug 1994 17:44:45 pdt

>From: "Dal Allan" <dal@endl.com>

>To: ata@dt.wdc.com

>Subject:   1996_calendar

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

To:       ATA Reflector

Fibre Channel Reflector

HIPPI Reflector

SCSI Reflector

SSA Reflector

Yes, I know some of you will get 5 copies of this email but the time has

come to plan for the 1996 meeting dates. Carl Zeitler proposed dates at

X3T11 last week, and I agreed to check them out against the overall calendar

for 1996.

Most of the months work well with meeting in the second full week, but there

are some conflicts which need to be considered. In the months which could be

a problem, the preferred week is indicated by 'x' days and the alternative

is noted by 'a' days.

In April, Carl suggested X3T11 meet the week of 4/15 which happens to be tax

time. Although Easter Sunday is not a holiday it is celebrated as a family

event, so that prevents meeting the week of April 8. The week of 4/1 is a

possible alternative, and if X3T10 chose the earlier week in March, there

would be an even spacing between X3T11 and the likely X3T10 calendar.

In July, X3T10 comes immediately after the July 4 weekend, so travel on

Sunday is not the greatest. Slipping it to a week later brings it within 2

weeks of the X3T11 meeting. We try to keep 3 weeks between them for the

benefit of those who attend both. Either X3T10 should not start until

Tuesday so everyone can take Monday flights, or X3T11 should slide a week.

Thanksgiving is the fourth Thursday of November so that puts X3T11 the week

after. Flying on Sunday of Thanksgiving is to be avoided. Holding X3T11 a

week earlier or a week later are not very practical choices. Maybe X3T11

should not start until Tuesday, so everyone can take Monday flights.

If anyone knows of more events in 1996 we should be aware of, or if any of

the holidays shown are incorrect, just let me know. Please send calendar

comments to me at 250-1752@mcimail.com rather than flooding the reflectors

like I have just done.

Telling me your any preferences for weeks is not much value, as choosing the

dates is a plenary decision. I will accumulate any responses and advise them

to the respective plenaries.

Volunteers to host will be welcomed by the chairs of both X3T10 and X3T11.

The 'best' months disappear fast, Western Digital has volunteered to host in

April 1996 and AMP has volunteered to host in October 1996.

January     *h *h  2  3  4  5  6         New Years

7  x  x  x  x  x *h  X3T10

*h *h 16 17 18 19 20         Martin Luther King

21 22 23 24 25 26 27

February    28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11 12 13 14 15 16 *h

*h *h 20 21 22 23 24         President's Day

March       25 26 27 28 29  1  2

3  a  a  a  a  a  9  X3T10a

10  x  x  x  x  x 16  X3T10

17 18 19 20 21 22 23

24 25 26 27 28 29 30

April       31  a  a  a  a  5 ee  X3T11a

ee  8  9 10 11 12 13         Easter

14  x  x  x  x  x 20  X3T11

21 22 23 24 25 26 27

May         28 29 30  1  2  3  4

5  x  x  x  x  x 11  X3T10

mm 13 14 15 16 17 18         Mother's Day

19 20 21 22 23 24 *h

June        *h *h 28 29 30 31  1         Memorial Day

2  3  4  5  6  7  8

9  x  x  x  x 14 15  X3T11

ff 17 18 19 20 21 22         Father's Day

23 24 25 26 27 28 29

July        30  1  2  3 *h *h *h         July 4

*h  x  x  x  x  x 13  X3T10

14  a  a  a  a  a 20  X3T10a

21 22 23 24 25 26 27

August      28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11  a  a  a  a 16 17  X3T11a

18 19 20 21 22 23 24

25 26 27 28 29 30 *h

September   *h *h  3  4  5  6  7         Labor Day

8  x  x  x  x  x 14  X3T10

15 16 17 18 19 20 21

22 23 24 25 26 27 28

October     29 30  1  2  3  4  5

6  x  x  x  x 11 12  X3T11

13 14 15 16 17 18 19

20 21 22 23 24 25 26

November    27 28 29 30 31  1  2

3  x  x  x  x  x  9  X3T10

10 11 12 13 14 15 16

17 18 19 20 21 22 23

h 25 26 27 *h *h *h         Thanksgiving

December    *h  x  x  x  x  6  7  X3T11

8  9 10 11 12 13 14

15 16 17 18 19 20 21

22 23 24 25 26 27 28         Christmas

29 30 31

From ata-owner  Thu Aug 25 12:08:12 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00916; Thu, 25 Aug 94 12:08:12 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA00909; Thu, 25 Aug 94 12:08:09 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA04318; Thu, 25 Aug 1994 12:08:09 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA02295; Thu, 25 Aug 1994 13:06:42 -0600

Message-Id: <9408251906.AA02295@hpgrla.gr.hp.com>

From: DAllan@CMS.gr.hp.com (Dal Allan)

Date: Tue, 23 Aug 94 17:44

To: ata@dt.wdc.com

Subject: 1996_calendar

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 23:57

>X-Envelope-To: CHADTHOM@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 23:57

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA01179; Tue, 23 Aug 1994 22:55:36 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA14378; Tue, 23 Aug 94 22:41:44 PDT

>Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA14365; Tue, 23 Aug 94 22:41:40 PDT

>Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.7/8.6.4) with ESMTP id WAA24600 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:37 -0700

>Received: from zebra.unix.portal.com (zebra.unix.portal.com [156.151.1.103]) by portal.unix.portal.com (8.6.5/8.6.5) with ESMTP id WAA12113 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:34 -0700

>Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id WAA16918 for ata@dt.wdc.com; Tue, 23 Aug 1994 22:02:02 -0700

>Received:  from endl by netmail.com (UUPC/extended 1.12j) with UUCP

>           for ata@dt.wdc.com; Tue, 23 Aug 1994 21:37:15 -0700

>Received:  by endl.com (UUPC/extended 1.12b);

>           Tue, 23 Aug 1994 17:44:44 pdt

>Message-Id: <2e5a97fd.endl@endl.com>

>Date:      Tue, 23 Aug 1994 17:44:45 pdt

>From: "Dal Allan" <dal@endl.com>

>To: ata@dt.wdc.com

>Subject:   1996_calendar

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

To:       ATA Reflector

Fibre Channel Reflector

HIPPI Reflector

SCSI Reflector

SSA Reflector

Yes, I know some of you will get 5 copies of this email but the time has

come to plan for the 1996 meeting dates. Carl Zeitler proposed dates at

X3T11 last week, and I agreed to check them out against the overall calendar

for 1996.

Most of the months work well with meeting in the second full week, but there

are some conflicts which need to be considered. In the months which could be

a problem, the preferred week is indicated by 'x' days and the alternative

is noted by 'a' days.

In April, Carl suggested X3T11 meet the week of 4/15 which happens to be tax

time. Although Easter Sunday is not a holiday it is celebrated as a family

event, so that prevents meeting the week of April 8. The week of 4/1 is a

possible alternative, and if X3T10 chose the earlier week in March, there

would be an even spacing between X3T11 and the likely X3T10 calendar.

In July, X3T10 comes immediately after the July 4 weekend, so travel on

Sunday is not the greatest. Slipping it to a week later brings it within 2

weeks of the X3T11 meeting. We try to keep 3 weeks between them for the

benefit of those who attend both. Either X3T10 should not start until

Tuesday so everyone can take Monday flights, or X3T11 should slide a week.

Thanksgiving is the fourth Thursday of November so that puts X3T11 the week

after. Flying on Sunday of Thanksgiving is to be avoided. Holding X3T11 a

week earlier or a week later are not very practical choices. Maybe X3T11

should not start until Tuesday, so everyone can take Monday flights.

If anyone knows of more events in 1996 we should be aware of, or if any of

the holidays shown are incorrect, just let me know. Please send calendar

comments to me at 250-1752@mcimail.com rather than flooding the reflectors

like I have just done.

Telling me your any preferences for weeks is not much value, as choosing the

dates is a plenary decision. I will accumulate any responses and advise them

to the respective plenaries.

Volunteers to host will be welcomed by the chairs of both X3T10 and X3T11.

The 'best' months disappear fast, Western Digital has volunteered to host in

April 1996 and AMP has volunteered to host in October 1996.

January     *h *h  2  3  4  5  6         New Years

7  x  x  x  x  x *h  X3T10

*h *h 16 17 18 19 20         Martin Luther King

21 22 23 24 25 26 27

February    28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11 12 13 14 15 16 *h

*h *h 20 21 22 23 24         President's Day

March       25 26 27 28 29  1  2

3  a  a  a  a  a  9  X3T10a

10  x  x  x  x  x 16  X3T10

17 18 19 20 21 22 23

24 25 26 27 28 29 30

April       31  a  a  a  a  5 ee  X3T11a

ee  8  9 10 11 12 13         Easter

14  x  x  x  x  x 20  X3T11

21 22 23 24 25 26 27

May         28 29 30  1  2  3  4

5  x  x  x  x  x 11  X3T10

mm 13 14 15 16 17 18         Mother's Day

19 20 21 22 23 24 *h

June        *h *h 28 29 30 31  1         Memorial Day

2  3  4  5  6  7  8

9  x  x  x  x 14 15  X3T11

ff 17 18 19 20 21 22         Father's Day

23 24 25 26 27 28 29

July        30  1  2  3 *h *h *h         July 4

*h  x  x  x  x  x 13  X3T10

14  a  a  a  a  a 20  X3T10a

21 22 23 24 25 26 27

August      28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11  a  a  a  a 16 17  X3T11a

18 19 20 21 22 23 24

25 26 27 28 29 30 *h

September   *h *h  3  4  5  6  7         Labor Day

8  x  x  x  x  x 14  X3T10

15 16 17 18 19 20 21

22 23 24 25 26 27 28

October     29 30  1  2  3  4  5

6  x  x  x  x 11 12  X3T11

13 14 15 16 17 18 19

20 21 22 23 24 25 26

November    27 28 29 30 31  1  2

3  x  x  x  x  x  9  X3T10

10 11 12 13 14 15 16

17 18 19 20 21 22 23

h 25 26 27 *h *h *h         Thanksgiving

December    *h  x  x  x  x  6  7  X3T11

8  9 10 11 12 13 14

15 16 17 18 19 20 21

22 23 24 25 26 27 28         Christmas

29 30 31

From ata-owner  Thu Aug 25 12:08:06 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00905; Thu, 25 Aug 94 12:08:06 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA00895; Thu, 25 Aug 94 12:08:03 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA04309; Thu, 25 Aug 1994 12:08:02 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA02289; Thu, 25 Aug 1994 13:06:35 -0600

Message-Id: <9408251906.AA02289@hpgrla.gr.hp.com>

From: DAllan@CMS.gr.hp.com (Dal Allan)

Date: Tue, 23 Aug 94 17:44

To: ata@dt.wdc.com

Subject: 1996_calendar

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 23:57

>X-Envelope-To: johnmoor@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 23:57

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA01179; Tue, 23 Aug 1994 22:55:36 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA14378; Tue, 23 Aug 94 22:41:44 PDT

>Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA14365; Tue, 23 Aug 94 22:41:40 PDT

>Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.7/8.6.4) with ESMTP id WAA24600 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:37 -0700

>Received: from zebra.unix.portal.com (zebra.unix.portal.com [156.151.1.103]) by portal.unix.portal.com (8.6.5/8.6.5) with ESMTP id WAA12113 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:34 -0700

>Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id WAA16918 for ata@dt.wdc.com; Tue, 23 Aug 1994 22:02:02 -0700

>Received:  from endl by netmail.com (UUPC/extended 1.12j) with UUCP

>           for ata@dt.wdc.com; Tue, 23 Aug 1994 21:37:15 -0700

>Received:  by endl.com (UUPC/extended 1.12b);

>           Tue, 23 Aug 1994 17:44:44 pdt

>Message-Id: <2e5a97fd.endl@endl.com>

>Date:      Tue, 23 Aug 1994 17:44:45 pdt

>From: "Dal Allan" <dal@endl.com>

>To: ata@dt.wdc.com

>Subject:   1996_calendar

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

To:       ATA Reflector

Fibre Channel Reflector

HIPPI Reflector

SCSI Reflector

SSA Reflector

Yes, I know some of you will get 5 copies of this email but the time has

come to plan for the 1996 meeting dates. Carl Zeitler proposed dates at

X3T11 last week, and I agreed to check them out against the overall calendar

for 1996.

Most of the months work well with meeting in the second full week, but there

are some conflicts which need to be considered. In the months which could be

a problem, the preferred week is indicated by 'x' days and the alternative

is noted by 'a' days.

In April, Carl suggested X3T11 meet the week of 4/15 which happens to be tax

time. Although Easter Sunday is not a holiday it is celebrated as a family

event, so that prevents meeting the week of April 8. The week of 4/1 is a

possible alternative, and if X3T10 chose the earlier week in March, there

would be an even spacing between X3T11 and the likely X3T10 calendar.

In July, X3T10 comes immediately after the July 4 weekend, so travel on

Sunday is not the greatest. Slipping it to a week later brings it within 2

weeks of the X3T11 meeting. We try to keep 3 weeks between them for the

benefit of those who attend both. Either X3T10 should not start until

Tuesday so everyone can take Monday flights, or X3T11 should slide a week.

Thanksgiving is the fourth Thursday of November so that puts X3T11 the week

after. Flying on Sunday of Thanksgiving is to be avoided. Holding X3T11 a

week earlier or a week later are not very practical choices. Maybe X3T11

should not start until Tuesday, so everyone can take Monday flights.

If anyone knows of more events in 1996 we should be aware of, or if any of

the holidays shown are incorrect, just let me know. Please send calendar

comments to me at 250-1752@mcimail.com rather than flooding the reflectors

like I have just done.

Telling me your any preferences for weeks is not much value, as choosing the

dates is a plenary decision. I will accumulate any responses and advise them

to the respective plenaries.

Volunteers to host will be welcomed by the chairs of both X3T10 and X3T11.

The 'best' months disappear fast, Western Digital has volunteered to host in

April 1996 and AMP has volunteered to host in October 1996.

January     *h *h  2  3  4  5  6         New Years

7  x  x  x  x  x *h  X3T10

*h *h 16 17 18 19 20         Martin Luther King

21 22 23 24 25 26 27

February    28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11 12 13 14 15 16 *h

*h *h 20 21 22 23 24         President's Day

March       25 26 27 28 29  1  2

3  a  a  a  a  a  9  X3T10a

10  x  x  x  x  x 16  X3T10

17 18 19 20 21 22 23

24 25 26 27 28 29 30

April       31  a  a  a  a  5 ee  X3T11a

ee  8  9 10 11 12 13         Easter

14  x  x  x  x  x 20  X3T11

21 22 23 24 25 26 27

May         28 29 30  1  2  3  4

5  x  x  x  x  x 11  X3T10

mm 13 14 15 16 17 18         Mother's Day

19 20 21 22 23 24 *h

June        *h *h 28 29 30 31  1         Memorial Day

2  3  4  5  6  7  8

9  x  x  x  x 14 15  X3T11

ff 17 18 19 20 21 22         Father's Day

23 24 25 26 27 28 29

July        30  1  2  3 *h *h *h         July 4

*h  x  x  x  x  x 13  X3T10

14  a  a  a  a  a 20  X3T10a

21 22 23 24 25 26 27

August      28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11  a  a  a  a 16 17  X3T11a

18 19 20 21 22 23 24

25 26 27 28 29 30 *h

September   *h *h  3  4  5  6  7         Labor Day

8  x  x  x  x  x 14  X3T10

15 16 17 18 19 20 21

22 23 24 25 26 27 28

October     29 30  1  2  3  4  5

6  x  x  x  x 11 12  X3T11

13 14 15 16 17 18 19

20 21 22 23 24 25 26

November    27 28 29 30 31  1  2

3  x  x  x  x  x  9  X3T10

10 11 12 13 14 15 16

17 18 19 20 21 22 23

h 25 26 27 *h *h *h         Thanksgiving

December    *h  x  x  x  x  6  7  X3T11

8  9 10 11 12 13 14

15 16 17 18 19 20 21

22 23 24 25 26 27 28         Christmas

29 30 31

From ata-owner  Thu Aug 25 12:23:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01487; Thu, 25 Aug 94 12:23:42 PDT

Received: from VNET.IBM.COM by dt.wdc.com (4.1/SMI-4.1)

id AA01436; Thu, 25 Aug 94 12:23:38 PDT

Message-Id: <9408251923.AA01436@dt.wdc.com>

Received: from TOROLAB6 by VNET.IBM.COM (IBM VM SMTP V2R2) with BSMTP id 6039;

Thu, 25 Aug 94 15:22:02 EDT

Date: Thu, 25 Aug 94 15:19:34 EDT

From: sandra@VNET.IBM.COM

To: dallan@cms.gr.hp.com, ata@dt.wdc.com

Subject: 1996_calendar

Sender: ata-owner@dt.wdc.com

Precedence: bulk

hello, can you please remove me from your distribution list.

Thank you

<<<<------------------------<Forwarded Note>------------------------>>>>

Received: from dt.wdc.com by vnet.IBM.COM (IBM VM SMTP V2R2) with TCP;

Thu, 25 Aug 94 15:17:02 EDT

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA00916; Thu, 25 Aug 94 12:08:12 PDT

Received: from hp.com by dt.wdc.com (4.1/SMI-4.1)

id AA00909; Thu, 25 Aug 94 12:08:09 PDT

Received: from hpgrla.gr.hp.com by hp.com with SMTP

(1.36.108.7/15.5+IOS 3.13) id AA04318; Thu, 25 Aug 1994 12:08:09 -0700

Received: from cms.gr.hp.com by hpgrla.gr.hp.com with SMTP

(1.38.193.4/15.5+ECS 3.3) id AA02295; Thu, 25 Aug 1994 13:06:42 -0600

Message-Id: <9408251906.AA02295@hpgrla.gr.hp.com>

From: DAllan@CMS.gr.hp.com (Dal Allan)

Date: Tue, 23 Aug 94 17:44

To: ata@dt.wdc.com

Subject: 1996_calendar

Sender: ata-owner@dt.wdc.com

Precedence: bulk

------------------------- Original message header:

>From ata-owner@dt.wdc.com Tue 23 Aug 1994 23:57

>X-Envelope-To: CHADTHOM@CMS.GR.HP.COM

>Return-Path: <ata-owner@dt.wdc.com>

>Received: from relay.hp.com ([15.255.152.2]) by cms.lvld.hp.com (SMTPSRV); Tue 23 Aug 1994 23:57

>Received: from fission.dt.wdc.com by relay.hp.com with SMTP

>	(1.37.109.8/15.5+ECS 3.3) id AA01179; Tue, 23 Aug 1994 22:55:36 -0700

>Received: by dt.wdc.com (4.1/SMI-4.1)

>	id AA14378; Tue, 23 Aug 94 22:41:44 PDT

>Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

>	id AA14365; Tue, 23 Aug 94 22:41:40 PDT

>Received: from portal.unix.portal.com (portal.unix.portal.com [156.151.1.100]) by nova.unix.portal.com (8.6.7/8.6.4) with ESMTP id

WAA24600 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:37 -0700

>Received: from zebra.unix.portal.com (zebra.unix.portal.com [156.151.1.103]) by portal.unix.portal.com (8.6.5/8.6.5) with ESMTP id

WAA12113 for <ata@dt.wdc.com>; Tue, 23 Aug 1994 22:41:34 -0700

>Received: from jmcbride.UUCP (uucp@localhost) by zebra.unix.portal.com (8.6.4/8.6.4) with UUCP id WAA16918 for ata@dt.wdc.com; Tue,

23 Aug 1994 22:02:02 -0700

>Received:  from endl by netmail.com (UUPC/extended 1.12j) with UUCP

>           for ata@dt.wdc.com; Tue, 23 Aug 1994 21:37:15 -0700

>Received:  by endl.com (UUPC/extended 1.12b);

>           Tue, 23 Aug 1994 17:44:44 pdt

>Message-Id: <2e5a97fd.endl@endl.com>

>Date:      Tue, 23 Aug 1994 17:44:45 pdt

>From: "Dal Allan" <dal@endl.com>

>To: ata@dt.wdc.com

>Subject:   1996_calendar

>Sender: ata-owner@dt.wdc.com

>Precedence: bulk

------------------------- End of message header.

To:       ATA Reflector

Fibre Channel Reflector

HIPPI Reflector

SCSI Reflector

SSA Reflector

Yes, I know some of you will get 5 copies of this email but the time has

come to plan for the 1996 meeting dates. Carl Zeitler proposed dates at

X3T11 last week, and I agreed to check them out against the overall calendar

for 1996.

Most of the months work well with meeting in the second full week, but there

are some conflicts which need to be considered. In the months which could be

a problem, the preferred week is indicated by 'x' days and the alternative

is noted by 'a' days.

In April, Carl suggested X3T11 meet the week of 4/15 which happens to be tax

time. Although Easter Sunday is not a holiday it is celebrated as a family

event, so that prevents meeting the week of April 8. The week of 4/1 is a

possible alternative, and if X3T10 chose the earlier week in March, there

would be an even spacing between X3T11 and the likely X3T10 calendar.

In July, X3T10 comes immediately after the July 4 weekend, so travel on

Sunday is not the greatest. Slipping it to a week later brings it within 2

weeks of the X3T11 meeting. We try to keep 3 weeks between them for the

benefit of those who attend both. Either X3T10 should not start until

Tuesday so everyone can take Monday flights, or X3T11 should slide a week.

Thanksgiving is the fourth Thursday of November so that puts X3T11 the week

after. Flying on Sunday of Thanksgiving is to be avoided. Holding X3T11 a

week earlier or a week later are not very practical choices. Maybe X3T11

should not start until Tuesday, so everyone can take Monday flights.

If anyone knows of more events in 1996 we should be aware of, or if any of

the holidays shown are incorrect, just let me know. Please send calendar

comments to me at 250-1752@mcimail.com rather than flooding the reflectors

like I have just done.

Telling me your any preferences for weeks is not much value, as choosing the

dates is a plenary decision. I will accumulate any responses and advise them

to the respective plenaries.

Volunteers to host will be welcomed by the chairs of both X3T10 and X3T11.

The 'best' months disappear fast, Western Digital has volunteered to host in

April 1996 and AMP has volunteered to host in October 1996.

January     *h *h  2  3  4  5  6         New Years

7  x  x  x  x  x *h  X3T10

*h *h 16 17 18 19 20         Martin Luther King

21 22 23 24 25 26 27

February    28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11 12 13 14 15 16 *h

*h *h 20 21 22 23 24         President's Day

March       25 26 27 28 29  1  2

3  a  a  a  a  a  9  X3T10a

10  x  x  x  x  x 16  X3T10

17 18 19 20 21 22 23

24 25 26 27 28 29 30

April       31  a  a  a  a  5 ee  X3T11a

ee  8  9 10 11 12 13         Easter

14  x  x  x  x  x 20  X3T11

21 22 23 24 25 26 27

May         28 29 30  1  2  3  4

5  x  x  x  x  x 11  X3T10

mm 13 14 15 16 17 18         Mother's Day

19 20 21 22 23 24 *h

June        *h *h 28 29 30 31  1         Memorial Day

2  3  4  5  6  7  8

9  x  x  x  x 14 15  X3T11

ff 17 18 19 20 21 22         Father's Day

23 24 25 26 27 28 29

July        30  1  2  3 *h *h *h         July 4

*h  x  x  x  x  x 13  X3T10

14  a  a  a  a  a 20  X3T10a

21 22 23 24 25 26 27

August      28 29 30 31  1  2  3

4  x  x  x  x  9 10  X3T11

11  a  a  a  a 16 17  X3T11a

18 19 20 21 22 23 24

25 26 27 28 29 30 *h

September   *h *h  3  4  5  6  7         Labor Day

8  x  x  x  x  x 14  X3T10

15 16 17 18 19 20 21

22 23 24 25 26 27 28

October     29 30  1  2  3  4  5

6  x  x  x  x 11 12  X3T11

13 14 15 16 17 18 19

20 21 22 23 24 25 26

November    27 28 29 30 31  1  2

3  x  x  x  x  x  9  X3T10

10 11 12 13 14 15 16

17 18 19 20 21 22 23

h 25 26 27 *h *h *h         Thanksgiving

December    *h  x  x  x  x  6  7  X3T11

8  9 10 11 12 13 14

15 16 17 18 19 20 21

22 23 24 25 26 27 28         Christmas

29 30 31

<<<<--------------------<End of Forwarded Note>--------------------->>>>

From ata-owner  Thu Aug 25 12:28:55 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01574; Thu, 25 Aug 94 12:28:55 PDT

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA01566; Thu, 25 Aug 94 12:28:37 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA25297; Thu, 25 Aug 1994 12:06:20 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdjo8-000gJUC; Thu, 25 Aug 94 12:45 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Thu, 25 Aug 1994 12:49:18 EDT

Date:      Thu, 25 Aug 1994 12:49:17 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5ccb8e.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 8 (part 3/4)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Command descriptions -- part 3 of 4.

-----begin

8.21.   RECALIBRATE

TYPE - Mandatory.

PROTOCOL - Non-data.

INPUTS - None.

OUTPUTS - If the command is executed in CHS addressing mode,

Cylinder High, Cylinder Low and the head portion of Device/Head

shall be zero.  The Sector Number register shall be 1. If the

command is executed in LBA addressing mode, the Cylinder High,

Cylinder Low, the head portion of the Device/Head and the Sector

Number register shall be zero.

PREREQUISITES - None.

DESCRIPTION - This command moves the read/write heads to cylinder

0. Upon receipt of the command, the device sets BSY and performs

a seek operation to cylinder zero.  The device then waits for the

seek to complete before updating status, clearing BSY and

generating an interrupt.

Devices that implement power management may not perform the seek

operation if the current power state is Idle or Standby.

ERRORS - If the device cannot reach cylinder 0, a Track 0 Not

Found error is posted.

8.22.   SEEK

TYPE - Manadatory.

PROTOCOL - Non-data.

INPUTS - The Cylinder High, Cylinder Low, head portion of the

Device/Head register and the Sector Number register contain the

logical sector number in CHS or LBA mode to which the device

should move the read/write heads.

OUTPUTS - Same as the input registers.

PREREQUISITES - None.

DESCRIPTION - This command initiates a seek operation to the

physical cylinder and head that contains the target logcial

sector.  If the device supports the indication of seek in

progress and seek complete by clearing and setting the DSC bit in

the Status register, then the device shall set DSC=0 when the

seek operation is started and shall not set DSC=1 until the seek

operation has completed.  The device may clear BSY and generate

an interrupt before the seek is completed.

Devices that implement power management may not perform the seek

operation if the current power state is Idle or Standby.

If another command is issued to the device while BSY=0 but DSC=1,

the device sets BSY=1, waits for the seek to complete, and then

begins execution of the command.

ERRORS -  ID Not Found and Command Abort.

8.23.   SET FEATURES

TYPE - Optional.

#############################################################

#

#Hale's note:  Oh, come on now, we all know that Set Features

#really isn't optional any more!

#

#############################################################

PROTOCOL - Non-data.

INPUTS - The Feature register contains a subcommand code as

described in table 14.  Some subcommands use other registers,

such as the Sector Count register to pass additional information

to the device.

OUTPUTS - See the subcommand descriptions.

PREREQUISITES - None.

DESCRIPTION - This command is used by the host to establish the

following parameters which affect the execution of certain device

features as shown in table 14.

+=====+=============================================================+

| 01h | Enable 8-bit data transfers (see 6.2.5)                     |

| 02h | Enable write cache *                                        |

| 03h | Set transfer mode based on value in Sector Count register   |

| 33h | Disable retry *                                             |

| 44h | Length of vendor specific bytes on READ LONG/WRITE LONG cmds|

| 54h | Set cache segments to Sector Count register value *         |

| 55h | Disable read look-ahead feature                             |

| 66h | Disable reverting to power on defaults (see 8.23)           |

| 77h | Disable ECC *                                               |

| 81h | Disable 8-bit data transfers (see 6.2.5)                    |

| 82h | Disable write cache *                                       |

| 88h | Enable ECC *                                                |

| 99h | Enable retries *                                            |

| AAh | Enable read look-ahead feature                              |

| ABh | Set maximum prefetch using Sector Count register value *    |

| BBh | 4 bytes of vendor specific byts on READ LONG/WRITE LONG cmds|

| CCh | Enable reverting to power on defaults (see 8.23)            |

|-----+-------------------------------------------------------------|

|     |  *These feature definitions are vendor specific             |

+=====+=============================================================+

Table 14 - Set Features register Definitions

All values not contained in Table 14 are reserved for future

definition.

At power on, or after a hardware reset, the default mode is the

same as that represented by values greater than 80h.  A setting

of 66h allows settings of greater than 80h which may have been

modified since power on to remain at the same setting after a

software reset.

A host can choose the transfer mechanism by Set Transfer Mode and

specifying a value in the Sector Count register.  The upper 5

bits define the type of transfer and the low order 3 bits encode

the mode value.

PIO Default Transfer Mode                               00000 000

PIO Default Transfer Mode, Disable IORDY                00000 001

PIO Flow Control Transfer Mode x                        00001 nnn

Single Word DMA Mode x                                  00010 nnn

Multiword DMA Mode x                                    00100 nnn

Reserved                                                01000 nnn

where "nnn" is a valid mode number for the associated transfer

type.

(Editor's note:  It is intended that the reserved values be used

for future specification of an alternative flow control

mechanism.)

If a device supports this specification, and receives a SET

FEATURES command with a Set Transfer Mode parameter and a Sector

Count register value of "00000 000", it shall set its default PIO

transfer mode.  If the value is "00000 001" and the device

supports disabling of IORDY, then the device shall set its

default PIO transfer mode and disable IORDY.

See vendor specification for the default mode of the commands

which are vendor specific.

Devices reporting support for Multi Word DMA Transfer Mode 1 must

also support Multi Word DMA Transfer Mode 0.

Support of IORDY is mandatory when PIO Mode 3 is the current mode

of operation.

ERRORS - If the device does not support the command or if any

input value is is not supported or is invalid, the device posts

an Aborted Command error.

8.24.   SET MULTIPLE MODE

TYPE - Optional.

PROTOCOL - Non-data.

INPUTS - The Sector Count register contains number of sectors per

block to use on all following READ/WRITE MULTIPLE commands.

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command enables the device to perform READ AND

WRITE MULTIPLE operations and establishes the block count for

these commands.

The Sector Count register is loaded with the number of sectors

per block.  Devices shall support block sizes of 2, 4, 8, and 16

sectors, if their buffer size is at least 8,192 bytes, and may

also support other block sizes.  Upon receipt of the command, the

device sets BSY=1 and checks the Sector Count register.

If the Sector Count register contains a valid value and the block

count is supported, the value is loaded for all subsequent READ

MULTIPLE and WRITE MULTIPLE commands and execution of those

commands is enabled.

If the Sector Count register contains 0 when the command is

issued, READ AND WRITE MULTIPLE commands are disabled.

At power on, or after a hardware reset, the default mode is READ

AND WRITE MULTIPLE disabled.  If Disable Default has been set in

the Features register then the mode remains the same as that last

established prior to a software reset, otherwise it reverts to

the default of disabled.

ERRORS - If the device does not support the READ/WRITE MULTIPLE

and SET MULTIPLE MODE commands or if a block count is not

supported, an Aborted Command error is posted, and READ MULTIPLE

and WRITE MULTIPLE commands are disabled.

8.25.   SLEEP

TYPE - Optional - Power Management Feature Set.

PROTOCOL - Non-data command.

INPUTS - None.

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command is the only way to cause the device to

enter Sleep Mode.

This command causes the device to set BSY, prepare to enter Sleep

mode, clear BSY and assert INTRQ.  The host shall read the Status

register in order to clear the interrupt and allow the device to

enter Sleep mode.  In Sleep mode the interface becomes inactive.

The host shall not attempt to access the Command Block registers

while the device is in Sleep mode.

Because some host systems may not read the Status register and

clear the interrupt, a device may automatically deassert INTRQ

and enter Sleep mode after a vendor specified time period of not

less than 2 seconds.

The only way to recover from Sleep Mode is with a software reset

or a hardware reset.

A device shall not power on in Sleep Mode nor remain in Sleep

Mode following a reset sequence.

ERRORS - Command Abort - The device does not support the Power

Management command set.

8.26.   STANDBY

TYPE - Optional - Power Management Feature Set.

PROTOCOL - Non-data command.

INPUTS - The value in the Sector Count register when the STANDBY

command is issued shall determine the time period programmed into

the Standby Timer.  See Table ??.

+---------------------+------------------------------+

| Sector Count        | Corresponding                |

| Register contents   | Timeout Period               |

+---------------------+------------------------------+

|   0       (00h)     | Timeout Disabled             |

|   1 - 240 (01h-F0h) | (value * 5) seconds          |

| 241 - 251 (F1h-FBh) | ((value - 240) * 30) minutes |

| 252       (FCh)     | 21 minutes                   |

| 253       (FDh)     | Vendor unique period between |

|                     | 8 and 12 hours               |

| 254       (FEh)     | Reserved                     |

| 255       (FFh)     | 21 minutes 15 seconds        |

+---------------------+------------------------------+

Table ?? - Automatic Standby Timer Periods

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command causes the device to set BSY, enter

the Standby Mode, clear BSY, and assert INTRQ.  INTRQ is asserted

even though the device may not have fully transitioned to Standby

Mode.

If the Sector Count register is non-zero then the Standby Timer

shall be enabled.  The value in the Sector Count register shall

be used to determine the time programmed into the Standby Timer.

If the Sector Count register is zero then the Standby Timer is

disabled.

ERRORS - Command Abort if the device does not support the Power

Management command set.

8.27.   STANDBY IMMEDIATE

TYPE - Optional - Power Management Feature Set.

PROTOCOL - Non-data command.

INPUTS - None.

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command causes the device to set BSY, enter

the Standby Mode, clear BSY, and assert INTRQ.  INTRQ is asserted

even though the device may not have fully transitioned to Standby

Mode.

ERRORS - Command Abort - The device does not support the Power

Management command set.

8.28.   WRITE BUFFER

TYPE - Optional.

PROTOCOL - PIO data out.

INPUTS - None.

OUTPUTS - None.

PREREQUISITES - None.

DESCRIPTION - This command enables the host to overwrite the

contents of one sector in the the device's sector buffer When

this command is issued, the device sets BSY, sets up the sector

buffer for a write operation, sets DRQ, clears BSY, and waits for

the host to write 512 bytes of data.  Once the host has written

the data, the device sets BSY, save the buffer data, clears BSY

and generates an interrupt.

The READ BUFFER and WRITE BUFFER commands shall be synchronized

within the device such that sequential WRITE BUFFER and READ

BUFFER commands access the same 512 bytes within the buffer.

ERRORS - Command Abort if the command is not supported.

8.29.   WRITE DMA

TYPE - Optional.

PROTOCOL - DMA.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

written.  The Sector Count register specifies the number of

sectors to be transferred.

OUTPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number registers specify the address of the last sector

transferred or the address of the sector where the first error

was detected.

PREREQUISITES - The host must initialize a slave-DMA channel.

DESCRIPTION -

This command executes in a similar manner to WRITE SECTOR(S)

except for the following:

* the host initializes a slave-DMA channel prior to issuing the

command

* data transfers are qualified by DMARQ and are performed by the

slave-DMA channel

* the device issues only one interrupt per command to indicate

that data transfer has terminated and status is available.

During the execution of a Write DMA command, the device shall

provide status of BSY or DRQ until the command is completed.

At command completion, the Command Block Registers contain the

sector address of the last sector transferred.

An unrecoverable error encountered during the execution of a

Write DMA command results in the termination of the command.  At

command completion, the Command Block Registers contain the

sector address of the sector address of the sector where the

first error occurred.

ERRORS - Command Abort if the command is not supported.  Other

errors possible are Bad Block, Uncorrectable data error, ID Not

Found and Address Mark Not Found.

8.30.   WRITE LONG

TYPE - Manadatory.

PROTOCOL - PIO data out.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

written.  The Sector Count register shall not specify a value

other than 1.

OUTPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number registers specify the address of the last sector

transferred or the address of the sector where the first error

was detected.

PREREQUISITES - The SET FEATURES subcommand to enable more than

4 vendor specific bytes must be executed prior to the WRITE LONG

command if more than 4 vendor specific bytes are to be transferred.

DESCRIPTION - This command is similar to the WRITE SECTOR(S)

command except that it writes the data and the vendor specific

bytes as supplied by the host; the device does not generate the

vendor specific bytes itself.  Only single sector Write Long

operations are supported.

The transfer of the vendor specific bytes shall be one byte at a

time over bits DD0-7 only (8-bits wide).

ERRORS - Bad Block, ID Not Found Command Abort and Address Mark

Not Found.

-----end

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 12:29:11 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01582; Thu, 25 Aug 94 12:29:11 PDT

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA01567; Thu, 25 Aug 94 12:28:55 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA25424; Thu, 25 Aug 1994 12:08:18 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdjoA-000gJTC; Thu, 25 Aug 94 12:45 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Thu, 25 Aug 1994 12:49:40 EDT

Date:      Thu, 25 Aug 1994 12:49:40 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5ccba5.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Section 8 (part 4/4)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Command descriptions -- part 4 of 4.

-----begin

8.31.   WRITE MULTIPLE command

TYPE - Optional.

PROTOCOL - PIO data out.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

written.  The Sector Count register specifies the number of

sectors to be transferred.

OUTPUTS - If not error, the Cylinder Low, Cylinder High,

Device/Head and Sector Number registers specify the address of

the last sector transferred.  These registers are undefined if an

error condition is indicated at command completion.

PREREQUISITES - A successful SET MULTIPLE MODE command must

preceed a READ MULTIPLE command.

DESCRIPTION - This command is similar to the WRITE SECTOR(S)

command. interrupts are not generated on every sector, but on the

transfer of a block which contains the number of sectors defined

by SET MULTIPLE MODE.

Command execution is identical to the WRITE SECTOR(S) operation

except that the number of sectors defined by the SET MULTIPLE

MODE command are transferred without intervening interrupts.  DRQ

qualification of the transfer is required only at the start of

the data block, not on each sector.

The block count of sectors to be transferred without intervening

interrupts is programmed by the SET MULTIPLE MODE command, which

shall be executed prior to the WRITE MULTIPLE command.

When the WRITE MULTIPLE command is issued, the Sector Count

register contains the number of sectors (not the number of blocks

or the block count) requested.

If the number of requested sectors is not evenly divisible by the

block count, as many full blocks as possible are transferred,

followed by a final, partial block transfer.  The partial block

transfer is for n sectors, where

n = Remainder (SECTOR COUNT / block count)

If the WRITE MULTIPLE command is attempted before the SET

MULTIPLE MODE command has been executed or when WRITE MULTIPLE

commands are disabled, the Write Multiple operation shall be

rejected with an Aborted Command error.

Device errors encountered during WRITE MULTIPLE commands are

posted after the attempted device write of the block or partial

block transferred.  The Write command ends with the sector in

error, even if it was in the middle of a block.  Subsequent

blocks are not transferred in the event of an error.

The contents of the Command Block Registers following the

transfer of a data block which had a sector in error are

undefined.  The host should retry the transfer as individual

requests to obtain valid error information.

Interrupts are generated when DRQ is set at the beginning of each

block or partial block.

ERRORS - Bad Block, Uncorrectable Data Error, ID Not Found

Command Abort and Address Mark Not Found.

8.32.   WRITE SAME

TYPE - Optional.

PROTOCOL - PIO data out.

INPUTS - The Feature register contains a sub command code, either

22H or DDH.  If the Feature register contains 22H, the Cylinder

Low, Cylinder High, Device/Head and Sector Number specify the the

starting CHS or LBA sector address to be written.  The Sector

Count register specifies the number of sectors to be written (not

the number of sectors transferred by the host).  If the Feature

register contains code DDH, the Cylinder High, Cylinder Low, head

portion of the Device/Head, Sector Number and Sector Count

registers are not used.

OUTPUTS - If no error, the Cylinder Low, Cylinder High,

Device/Head and Sector Number registers specify the address of

the last sector written.  If the comamnd ends with an error,

these registers contain the address of the sector where the first

error was detected and the Sector Count register contains the

number of sectors remaining to be transferred in order to

complete the original request.

PREREQUISITES - None.

DESCRIPTION - This command executes in a similar manner to WRITE

SECTOR(S) except that only one sector of data is transferred.

The contents of the sector are written to the media one or more

times.

NOTE:  The WRITE SAME command allows for initialization of part

or all of the medium to the specified data with a single command.

If the Features register is 22h, the device shall write that part

of the medium specified by the Sector Count, Sector Number,

Cylinder Low, Cylinder High and Device/Head registers.  If the

Features register contains DDh, the device shall initialize all

the user accessible medium.

The device issues an interrupt to indicate that the command is

complete.  Any error encountered during execution results in the

termination of the write operation.

ERRORS - If the Feature register contains a value other than 22h

or DDh, the command shall be rejected with an aborted command

error.  Other errors possible are Bad Block, Uncorrectable Data

Error, ID Not Found and Address Mark Not Found.

8.33.   WRITE SECTOR(S)

TYPE - Manadatory.

PROTOCOL - PIO data out.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

written.  The Sector Count register specifies the number of sectors

to be transferred.

OUTPUTS - If no error, the Cylinder Low, Cylinder High,

Device/Head and Sector Number registers specify the address of

the last sector transferred.  If the comamnd ends with an error,

these registers contain the address of the sector where the first

error was detected and the Sector Count register contains the

number of sectors remaining to be transferred in order to

complete the original request.

PREREQUISITES - None.

DESCRIPTION - This command writes from 1 to 256 sectors as

specified in the Sector Count register.  A SECTOR COUNT of 0

requests 256 sectors.

If the device is not already on the desired track, an implied

seek operation is performed.

The types of errors that are not retried when no retries is

requested by the host is vendor specific.

ERRORS - Bad Block, Uncorrectable Data Error, ID Not Found

Command Abort and Address Mark Not Found.

8.34.   WRITE VERIFY

TYPE - Optional.

PROTOCOL - PIO data out.

INPUTS - The Cylinder Low, Cylinder High, Device/Head and Sector

Number specify the the starting CHS or LBA sector address to be

written.  The Sector Count register specifies the number of sectors

to be transferred.

OUTPUTS - If no error, the Cylinder Low, Cylinder High,

Device/Head and Sector Number registers specify the address of

the last sector transferred.  If the comamnd ends with an error,

these registers contain the address of the sector where the first

error was detected and the Sector Count register contains the

number of sectors remaining to be transferred in order to

complete the original request.

PREREQUISITES - None.

DESCRIPTION - This command is similar to the WRITE SECTOR(S)

command, except that each sector is verified immediately after

being written.  The verify operation is a read without transfer

and a check for data errors.  Any errors encountered during the

verify operation are posted.  Multiple sector WRITE VERIFY

commands write all the requested sectors and then verify all the

requested sectors before generating the final interrupt.

ERRORS - Bad Block, Uncorrectable Data Error, ID Not Found

Command Abort and Address Mark Not Found.

-----end

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Thu Aug 25 19:35:57 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA11354; Thu, 25 Aug 94 19:35:57 PDT

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA11346; Thu, 25 Aug 94 19:35:37 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id TAA10257; Thu, 25 Aug 1994 19:28:46 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qdqj6-000gJTC; Thu, 25 Aug 94 20:09 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Thu, 25 Aug 1994 20:13:06 EDT

Date:      Thu, 25 Aug 1994 20:13:05 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e5d3393.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2F -- Annex A and B

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Well, this is just about it.  The control of your email will be

returned to you shortly after receiving this messsage!  I'm sure

many of you are happy to heard this news.

Here is the latest version of Annexes A and B. These Annexes have

been under discussion for awhile now and the previous version of

this message, 94-065-r2 dated 24 April 94, is what you will find

in the ATA-2 Rev 2F document.

-----begin 94-065-r3

Master/Slave Handshaking

Document 94-065-r3

25 August 94

by Hale Landis

This is a major revision since 94-065-r2 (and ATA-2 rev 2f).  I

have been trying to make this whole mess easy to understand.  I

hope I have done that with this version.  I also added the

recommendation that a host allow up to 2 minutes for DRDY=1 based

on the discussion started by Conner at our previous meetings.

I suggest that you review this carefully -- it is relatively

complex and I could have make a mistake (yep, it can happen).

----- Annex A -----

Annex A Diagnostic and reset considerations from a device

hardware standpoint

(informative)

This annex describes the algorithm and timing relationships for

devices 0 and 1 during the processing of power on and hardware

resets, software resets, and the EXECUTE DEVICE DIAGNOSTIC

command.

The timing assumes the following:

a) DASP- is asserted by Device 1 and received by Device 0 at

power-on or hardware reset to indicate the presence of Device

1. At all other times it is asserted by Device 0 or Device 1 to

indicate when a device is active.

b) PDIAG- is asserted by Device 1 and detected by Device 0. It

is used by Device 1 to indicate to Device 0 that it has

completed diagnostics and is ready to accept commands from the

Host (BSY bit is cleared).  This does not indicate that the

device is ready, only that it can accept commands.  This line

may remain asserted until the next reset occurs or an EXECUTE

DEVICE DIAGNOSTIC command is received.

A.1 Power on and hardware resets

A.1.1 Power on and hardware resets - device 0

a) Host asserts RESET- for a minimum of 25 usec.

b) Device 0 sets BSY within 400 nsec after RESET- is negated.

c) Device 0 negates DASP- within 1 msec after RESET- is negated.

d) Device 0 samples for at least 450 msec for DASP- to be

asserted from Device 1. This sampling starts 1 ms after RESET-

is negated.

e) Device 0 performs hardware initialization and diagnostics.

f) Device 0 may revert to its default condition.

g) If Device 0 detected that DASP- was asserted during step

d), then Device 0 waits up to 31 seconds for Device 1 to

assert PDIAG-.  If PDIAG- is asserted within 31 seconds,

Device 0 sets bit 7=0 in the Error Register, else Device 0

sets bit 7=1 in the Error Register.

If DASP- assertion was not detected in step d), Device 0 sets

bit 7=0 in the Error Register.

NOTE:  Device 0 must remember if Device 1 was detected in step

d) because this information is need in order to process any

Software reset or EXECUTE DEVICE DIAGNOSTIC command later.

h) Device 0 posts diagnostic results to bits 6-0 of the Error

Register.

i) Device 0 clears BSY when ready to accept commands that do not

require DRDY=1.

NOTE:  Device 0 must clear BSY within 31 seconds from the time

that RESET- is negated.

j) Device 0 sets DRDY when ready to accept any command.

NOTE:  Steps i) and j) may occur at the same time.  While no

maximum time is specified for DRDY=1 to occur a host

is advised to allow up to 2 minutes for DRDY=1.  See Figure 9.

A.1.2 Power on and hardware resets - device 1

a) Host asserts RESET- for a minimum of 25 usec.

b) Device 1 sets BSY within 400 nsec after RESET- is negated.

c) Device 1 negates DASP- within 1 msec after RESET- is negated.

d) Device 1 negates PDIAG- before asserting DASP-.

e) Device 1 asserts DASP- within 400 msecs after RESET- is

negated.

f) Device 1 performs hardware initialization and diagnostics.

g) Device 1 may revert to its default condition.

h) Device 1 posts diagnostic results to the Error Register.

i) Device 1 clears BSY when ready to accept commands that do not

require DRDY=1.

j) If Device 1 passed its diagnostics without error in step

f), Device 1 asserts PDIAG-.  If those diagnostics failed,

Device 1 does not assert PDIAG- and continues to the next

step.

NOTE:  Device 1 must clear BSY, and optionally assert PDIAG-,

within 30 seconds from the time RESET- is negated.

k) Device 1 sets DRDY when ready to accept any command.

NOTE:  Steps i), j) and k) may occur at the same time.  While

no maximum time is specified for DRDY=1 to occur, a host

is advised to allow up to 2 minutes for DRDY=1.  See Figure 9.

l) Device 1 negates DASP- after the first command is received or

negates DASP- if no command is received within 31 seconds after

RESET- is asserted.

__

RESET- ___/  \_________________________________________

____________________

BSY    ____/                    \______________________

___

DRDY   ____________________________________________/

|<---------------->|<---------------->|

Max for device 0    Minimun time is

is 31 seconds.     0 seconds.

Max for device 1    No maximum time

is 30 seconds.     is specified.

Figure 9- BSY and DRDY timing for power on and hardware resets

A.2 Software reset

A.2.1  Software reset - device 0

a) Host sets SRST=1 in the Device Control register.

b) Device 0 sets BSY within 400 nsec after detecting that SRST=1.

c) Device 0 performs hardware initialization and diagnostics.

d) Device 0 may revert to its default condition.

e) Device 0 posts diagnostic results to the Error Register.

f) Device 0 waits for the host to set SRST=0.

g) If Device 0 detected that Device 1 is present during the

most recent power on or hardware reset sequence, then Device 0

waits up to 31 seconds from the time that SRST=0 for Device 1

to assert PDIAG-.  If PDIAG- is asserted within 31 seconds,

Device 0 sets bit 7=0 in the Error Register, else Device 0

sets bit 7=1 in the Error Register.

If device 1 was not detected during the most recent power up

or hardware reset sequence, then Device 0 sets bit 7=0 in the

Error register.

h) Device 0 clears BSY when ready to accept commands that do not

require DRDY=1.

NOTE:  Device 0 must clear BSY within 31 seconds from the time

that the host sets SRST=0.

i) Device 0 sets DRDY when ready to accept any command.

NOTE:  Steps h) and i) may occur at the same time.  While

no maximum time is specified for DRDY=1 to occur, a host

is advised to allow up to 2 minutes for DRDY=1.  See Figure 10.

A.2.2 Software reset - device 1

a) Host sets SRST=1 in the Device Control register.

b) Device 1 set BSY within 400 nsec after detecting

that SRST=1.

c) Device 1 negates PDIAG- within 1 msec after detecting that

SRST=1.

d) Device 1 perform hardware initialization and diagnostics.

e) Device 1 may revert to its default condition.

f) Device 1 posts diagnostic results to the Error Register.

g) Device 1 waits for the host to set SRST=0.

h) Device 1 clears BSY when ready to accept commands that do not

require DRDY=1.

i) If Device 1 passed its diagnostics without error in step

d), Device 1 asserts PDIAG-.  If those diagnostics failed,

Device 1 does not assert PDIAG- and continues to the next

step.

NOTE:  Device 1 must clear BSY, optionally assert PDIAG-,

within 30 seconds from the time the host sets SRST=0.

j) Device 1 sets DRDY when ready to accept any command.

NOTE:  Steps h), i) and j) may occur at the same time.  While

no maximum time is specified for DRDY=1 to occur, a host

is advised to allow up to 2 minutes for DRDY=1.  See Figure 10.

__

SRST ___/  \_________________________________________

_____________________

BSY  ___/                     \______________________

___

DRDY ____________________________________________/

|<---------------->|<---------------->|

Max for device 0    Minimun time is

is 31 seconds.     0 seconds.

Max for device 1    No maximum time

is 30 seconds.     is specified.

Figure 10- BSY and DRDY timing for software reset

A.3 EXECUTE DEVICE DIAGNOSTIC Command Execution

A.3.1 EXECUTE DEVICE DIAGNOSTIC command execution - device 0

a) Device 0 sets BSY within 400 nsec after the EXECUTE DEVICE

DIAGNOSTIC command is received.

b) Device 0 performs hardware initialization and diagnostics.

c) Device 0 resets the Command Block registers to default

condition.

d) Device 0 posts diagnostic results to bits 6-0 of the Error

Register.

e) If Device 0 detected that Device 1 is present during the

most recent power on or hardware reset sequence, then Device 0

waits up to 6 seconds from the time that the EXECUTE DEVICE

DIAGNOSTIC command was received for Device 1 to assert PDIAG-.

If PDIAG- is asserted within 6 seconds, Device 0 sets bit 7=0

in the Error Register, else Device 0 sets bit 7=1 in the Error

Register.

If device 1 was not detected during the most recent power up

or hardware reset sequence, then Device 0 sets bit 7=0 in the

Error register.

f) Device 0 clears BSY when ready to accept commands that do not

require DRDY=1.

NOTE:  Device 0 must clear BSY within 6 seconds from the time

that the EXECUTE DEVICE DIAGNOSTIC command was received.

g) Device 0 sets DRDY when ready to accept any command.

NOTE:  Steps f) and g) may occur at the same time.  While

no maximum time is specified for DRDY=1 to occur, a host

is advised to allow up to 2 minutes for DRDY=1.  See Figure 11.

A.3.2 EXECUTE DEVICE DIAGNOSTIC command execution - device 1

a) Device 1 sets BSY within 400 nsec after the EXECUTE DEVICE

DIAGNOSTIC command is received.

b) Device 1 negates PDIAG- within 1 msec after the command

received.

c) Device 1 performs hardware initialization and diagnostics.

d) Device 1 resets the Command Block registers to the default

condition.

e) Device 1 posts diagnostic results to the Error Register.

f) Device 1 clears BSY when ready to accept commands that do not

require DRDY=1.

g) If Device 1 passed its diagnostics without error in step

c), Device 1 asserts PDIAG-.  If those diagnostics failed,

Device 1 does not assert PDIAG- and continues to the next

step.

NOTE:  Device 1 must clear BSY and assert PDIAG- within 5

seconds from the time RESET- is asserted.

h) Device 1 sets DRDY when ready to accept any command.

NOTE:  Steps f), g) and h) may occur at the same time.  While

no maximum time is specified for DRDY=1 to occur, a host

is advised to allow up to 2 minutes for DRDY=1.  See Figure 11.

__________________

BSY  ___/                  \______________________________

___________

DRDY _________________________________________/

|<---------------->|<---------------->|

Max for device 0    Minimun time is

is  6 seconds.     0 seconds.

Max for device 1    No maximum time

is  5 seconds.     is specified.

Figure 11- BSY and DRDY timing for Diagnostic command

----- end Annex A -----

----- Annex B -----

Annex B. Diagnostic and reset considerations from a device

firmware stanpoint

(informative)

B.1. Power on and hardware reset (RESET-)

DASP- is read by Device 0 to determine if Device 1 is present.

If Device 1 is present Device 0 will read PDIAG- to determine

when it is valid to clear BSY and whether Device 1 has powered on

or reset without error, otherwise Device 0 clears BSY whenever it

is ready to accept commands.  Device 0 may assert DASP- to

indicate device activity.

B.2. Software reset

If Device 1 is present Device 0 will read PDIAG- to determine

when it is valid to clear BSY and whether Device 1 has reset

without any errors, otherwise Device 0 will simply reset and

clear BSY.  DASP- is asserted by Device 0 (and Device 1 if it is

present) in order to indicate device active.

B.3. Device diagnostic command

If Device 1 is present, Device 0 will read PDIAG- to determine

when it is valid to clear BSY and if Device 1 passed or failed

the EXECUTE DEVICE DIAGNOSTIC command, otherwise Device 0 will

simply execute its diagnostics and then clear BSY.  DASP- is

asserted by Device 0 (and Device 1 if it is present) in order to

indicate the device is active.

B.4. Truth table

In all the above cases:  Power on, RESET-, software reset, and

the EXECUTE DEVICE DIAGNOSTIC command the Device 0 Error register

is calculated as follows:

+-----------+------------+-----------+-----------+

|  Device 1 |    PDIAG-  |  Device 0 |  Error    |

|  Present? |  Asserted? |  Passed   |  Register |

+-----------+------------+-----------+-----------+

|    Yes    |    Yes     |    Yes    |    01h    |

|    Yes    |    Yes     |    No     |    0xh    |

|    Yes    |    No      |    Yes    |    81h    |

|    Yes    |    No      |    No     |    8xh    |

|    No     | (not read) |    Yes    |    01h    |

|    No     | (not read) |    No     |    0xh    |

+-----------+------------+-----------+-----------+

Where x indicates the appropriate Diagnostic Code for the

Power on, RESET-, software reset, or device diagnostics error.

Table 14 - Reset Error register Values

B.5. Power on or hardware reset algorithm

NOTE:  In the following algrorithms, the notation 1* refers to

the Drive 1 status register copy that Drive 0 keeps when there is

no Drive 1 present.

B.5.1. Algorithm for Device 0

1) Power on or hardware reset is detected by the device's

hardware.

2) The hardware should automatically do the following within 400

nsec:

a) Set up the hardware to report both Device 0 and Device 1*

status registers.

b) Set the Device 0 Status register to 80h.

c) Set the Device 1* Status register to 80h.

3) The device must determine if it is Device 0 or Device 1. This

can be done at least two different ways:  by jumper or by

using the CSEL signal.  This algorithm is for Device 0.

4) Set up PDIAG- and DASP:

a) Set up PDIAG- as an input.

b) Release DASP- and set up DASP- as an input.

c) Monitor DASP- for 450 msec or until DASP- is asserted by

Device 1.

d) If DASP- is asserted within 450 msec, note that Device 1 is

present and set up the hardware so it reports Device 0 status

only and remember the Device 1 is present.

e) If DASP- is not asserted within 450 msec, note that Device

1 is not present and set up the hardware so it reports both

Device 0 and 1* status registers and remember that Device 1 is

not present.

f) Set up DASP- as an output.

g) Assert DASP-.

NOTE:  Steps 1-4 must complete within 450 msec.

5) Perform any remaining time critical hardware initialization

including starting the spin up of the device if needed.

Complete all the hardware initialization and diagnostic tests

needed to get the device ready, including:

a) Set the Sector Count register to 01h.

b) Set the Sector Number register to 01h.

c) Set the Cylinder Low register to 00h.

d) Set the Cylinder High register to 00h.

e) Set the Device/Head register to 00h.

6) If Device 1 was detected in step 4:

a) Monitor PDIAG- for 31 seconds or until PDIAG- is asserted

by Device 1.

b) If PDIAG- is asserted within 31 seconds, set bit 7=0 in the

Error register.

c) If PDIAG- is not asserted within 31 seconds, set bit 7=1 in

the Error register.

7) Post Device 0's initialization and diagnostic results:

a) If Device 0 completed all initialization and diagnostics

without error, set bits 6-0 of the Error register to 1.

b) If Device 0 failed any initialization or diagnostics, set

bits 6-0 of the Error register to a value of 2 or greater

indicating the type of failure.

8) Set the status register to 00h and set Drive 1* Status register

to 00H.

NOTE:  Steps 1-8 must complete within 31 seconds.

9) Finish initialization and optionally finish spin up.

10) Post Ready status:

a) Set the Status register to 40h or if spin up is complete

set the status register to 50h.

b) Negate DASP-

11) If not previously done, wait for spin up to complete and then set the

status register to 50h.

B.5.2. Algorithm for Device 1

1) Power on or hardware reset is detected by the device's

hardware.

2) The hardware should automatically do the following within 400

nsec:

a) Set up the hardware to report both Device 0 and Device 1*

status registers.

b) Set the Device 0 Status register to 80h.

c) Set the Device 1* Status register to 80h.

3) The device must determine if it is Device 0 or Device 1. This

can be done at least two different ways:  by jumper or by

using the CSEL signal.  This algorithm is for Device 1.

4) Set up PDIAG- and DASP:

a) Negate the PDIAG- signal.

b) Set up PDIAG- as an output.

c) Assert the DASP- output.

d) Set up DASP- as an output.

e) Set up the hardware so it reports Device 1 status only.

NOTE:  Steps 1-4 must complete within 400 msec.

5) Perform any remaining time critical hardware initialization

including starting the spin up of the device if needed.

Complete all the hardware initialization and diagnostic tests

needed to get the device ready, including:

a) Set the Sector Count register to 01h.

b) Set the Sector Number register to 01h.

c) Set the Cylinder Low register to 00h.

d) Set the Cylinder High register to 00h.

e) Set the Device/Head register to 00h.

6) Post Device 1's initialization and diagnostic results:

a) If Device 1 completed all initialization and diagnostic

without error, Set the Error register to 1 and assert

PDIAG-.

b) If Device 1 failed any initialization or diagnostics, set

the Error register to a value of 2 or greater indicating the

type of failure and do not assert PDIAG-.

7) Set the status register to 00.

NOTE:  Steps 1-7 must complete within 30 seconds.

8) Finish initialization and optionally finish spin up.

9) Post Ready status:  Set the Status register to 40h or if spin

up is also complete set the Status register to 50h.

10) If not previously done, wait for spin up to complete and then set the

status register to 50h.

11) Negate DASP- when the Command register is written or after 31

seconds.

B.6 Software Reset Algorithm

B.6.1 Algorithm for Device 0

1) SRST=1 is detected by the device's hardware.

2) The hardware should set 80h in the Status register within 400

nsec.

3) If there is no Device 1, the hardware should set the Device 1*

Status register to 80h

NOTE:  Steps 1-3 must happen within 1 msec.

4) Assert DASP-.

5) Finish all the hardware initialization needed to place the

device in reset including all diagnostics.

6) Wait for SRST=0.

7) Reset the Command Block registers:

a) Set the Sector Count register to 01h.

b) Set the Sector Number register to 01h.

c) Set the Cylinder Low register to 00h.

d) Set the Cylinder High register to 00h.

e) Set the Device/Head register to 00h.

8) If Device 1 is present:

a) Monitor PDIAG- for 31 seconds or until PDIAG- is asserted

by Device 1.

b) If PDIAG- is asserted within 31 seconds, set bit 7=0 in the

Error register.

c) If PDIAG- is not asserted within 31 seconds, set bit 7=1 in

the Error register.

9) Post Device 0's initialization and diagnostic results:

a) If Device 0 completed all initialization and diagnostics

without error, set bits 6-0 of the Error register the value 1.

b) If Device 0 failed any initialization or diagnostics, set

bits 6-0 of the Error register to a value of 2 or greater

indicating the type of failure.

NOTE: Steps 7-9 must complete within 31 seconds.

10) Set the Status register to 50h and set Drive 1* Status

register to 00h.

B.6.2 Algorithm for Device 1

1) SRST=1 is detected by the device's hardware.

2) The hardware should set 80h in the Status register within 400

nsec.

3) Negate the PDIAG- signal.

NOTE:  Steps 1-3 must complete within 1 msec.

4) Assert DASP-.

5) Finish all the hardware initialization needed to place the

device in reset including all diagnostics.

6) Wait for SRST=0.

7) Reset the Command Block registers:

a) Set the Sector Count register to 01h.

b) Set the Sector Number register to 01h.

c) Set the Cylinder Low register to 00h.

d) Set the Cylinder High register to 00h.

e) Set the Device/Head register to 00h.

8) Post Device 1's initialization and diagnostic results:

a) If Device 1 completed all initialization and diagnostics

without error, set bits 6-0 of the Error register the value 1

and assert PDIAG-.

b) If Device 1 failed any initialization or diagnostics, set

bits 6-0 of the Error register to a value of 2 or greater

indicating the type of failure and do not assert PDIAG-.

NOTE: Steps 7-9 must complete within 30 seconds.

9) Set the Status register to 50h.

B.7.  EXECUTE DEVICE DIAGNOSTIC Command Algorithm

B.7.1. Algorithm for Device 0

1) The EXECUTE DEVICE DIAGNOSTIC command is received.

2) The hardware should set 80h in the Status register within 400

nsec.

3) The hardware should set 80h in the Device 1* Status register.

NOTE:  Steps 1-3 must complete within 1 msec.

4) Assert DASP-.

5) Perform all the device diagnostics and note the results.

6) Finish all the hardware initialization needed to get the

device ready to receive any type of command from the host

including:

a) Set the Sector Count register to 01h.

b) Set the Sector Number register to 01h.

c) Set the Cylinder Low register to 00h.

d) Set the Cylinder High register to 00h.

e) Set the Device/Head register to 00h.

7) If Device 1 is present:

a) Monitor PDIAG- for 6 seconds or until PDIAG- is asserted by

Device 1.

b) If PDIAG- is asserted within 6 seconds, set bit 7=0 in the

Error register.

c) If PDIAG- is not asserted within 6 seconds, set bit 7=1 in

the Error register.

8) Post Device 0's initialization and diagnostic results:

a) If Device 0 completed all initialization and diagnostics

without error, set bits 6-0 of the Error register the value 1.

b) If Device 0 failed any initialization or diagnostics, set

bits 6-0 of the Error register to a value of 2 or greater

indicating the type of failure.

NOTE: Steps 1-8 must complete within 6 seconds.

9) Set the Status register to 50h and set Drive 1* Status

register to 00h.

10) Assert INTRQ.

B.7.2. Algorithm for Device 1

1) The EXECUTE DEVICE DIAGNOSTIC command is received.

2) The hardware should set 80h in the Status register within 400

nsec.

3) Negate the PDIAG- signal.

NOTE:  Steps 1-3 must complete within 1 msec.

4) Assert DASP-.

5) Perform all the device diagnostics and note the results.

6) Finish all the hardware initialization needed to get the

device ready to receive any type of command from the host

including:

a) Set the Sector Count register to 01h.

b) Set the Sector Number register to 01h.

c) Set the Cylinder Low register to 00h.

d) Set the Cylinder High register to 00h.

e) Set the Device/Head register to 00h.

7) Post Device 1's initialization and diagnostic results:

a) If Device 1 completed all initialization and diagnostics

without error, set bits 6-0 of the Error register the value 1

and assert PDIAG-.

b) If Device 1 failed any initialization or diagnostics, set

bits 6-0 of the Error register to a value of 2 or greater

indicating the type of failure and do not assert PDIAG-.

NOTE: Steps 1-7 must complete within 5 seconds.

8) Set the Status register to 50h.

----- end Annex B -----

-----end 94-065-r3

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Sat Aug 27 17:25:27 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02903; Sat, 27 Aug 94 17:25:27 PDT

Received: from wdus03.wdc.com by dt.wdc.com (4.1/SMI-4.1)

id AA02857; Sat, 27 Aug 94 17:24:18 PDT

Received: with SMTP-MR; Sat, 27 Aug 1994 17:16:49 PST

Mr-Received: by mta WDUS04; Relayed; Sat, 27 Aug 1994 17:16:49 -0800 (PST)

Alternate-Recipient: prohibited

Disclose-Recipients: prohibited

Date: Sun, 28 Aug 1994 01:12:00 PST

From: "Tom Hanan (714)932-7472" <HANAN_T@A1.WDC.COM>

Subject: Multi-threding (Proposal) R0.1

To: ATAPI@dt.wdc.com, ATA@dt.wdc.com

Message-Id: <01HGEU52WK9W8ZEW5G@wdus03.wdc.com>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN

Posting-Date: Sun, 28 Aug 1994 01:16:00 PST

Importance: normal

Priority: normal

X400-Mts-Identifier: [;94617172804991/549070@WDUSA]

A1-Type: MAIL

Hop-Count: 0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

1.0 Overview

This proposal is intended to provide a basis for extensions to the

existing ATA and ATAPI protocols needed to support Multi-threading.

The need to provide Multi-threading extensions to ATA and ATAPI devices

is driven by system manufactures desire to allow inexpensive, high

performance hard disk drives, ATAPI CD-ROM and ATAPI tape devices to

share a single low cost ATA connection.

This proposal has been put forward by Western Digital in an effort to

address industry wide compatibility issues related to the specifics of

implementing high performance Multi threading on ATA host systems and

peripherals.

It is the intent of the editor of this document to publish this document

as an SFF extension to the existing ATA-2 and SFF ATAPI 1.2 documents.

It is also the intent of the editor to work closely with the editors of

the ATAPI protocol and ATA-3 documents to incorporate the appropriate

sections of this proposal within their respective documents.

Comments or suggestions regarding the technical content of this proposal

should be posted on the ATA / SFF ATAPI reflectors.

Comments or suggestions regarding editorial or administrative issues

related to this document should be sent to:

Tom Hanan

Western Digital

8105 Irvine Center Drive

Irvine CA  92718

PH:     714 932-7472

FAX:    714 932-7314

E-Mail  hanan_t@a1.wdc.com

P.S. I hope I get more e-mail on the technical content of the document

than either the format or the politics of which sections will be

included in which X3T10 document. Let's get the compatibility issues

worked out before we decide where the information should go......

Regards,

Tom Hanan

I)	Traditional

Cmd       Data      Data          Cmd       Data

+--+--+   +--+--+   +--+--+--+    +--+--+   +--+--+--+

|SL|IC|o o|RS|TD|o o|RS|TD|RL|    |SL|IC|o o|RS|TD|RL|

+--+--+   +--+--+   +--+--+--+    +--+--+   +--+--+--+

|B0|R1|B1 |B0|In|B1 |B0|In|B0     |B0|R1|B1 |B0|In|B0

|s0|  |   |D1|  |   |D1|  |D0     |s0|  |   |D1|  |D0

|  |  |   |IA|  |   |IA|  |R1     |  |  |   |Ia|  |R1

II)	Overlapped

Cmd                                   Poll                     Poll

+--+--+--+                            +--+--+                  +--+--+

|SL|IC|RL| o o o o o o o o o o o o o o|SL|RL|o o o o o o o o o |SL|RL|

+--+--+--+                            +--+--+                  +--+--+

|B0|R1|B0                             |B0|B0                   |B0|B0|

|s1|  |D0                             |s1|D0                   |s1|D0|

|  |  |C0                             |  |C0                   |R1|C1|

=======================================================================

New

III)	Shared IRQ (ATA-3)

Cmd                           Data                 Data

+--+--+--+                  +--+--+--+        +--+--+--+

|ST|IC|RL| o o o o o o o o o|SL|TD|RL| o o o o|SL|TD|RL|

+--+--+--+                  +--+--+--+        +--+--+--+

|B0|R1|B0                   |B0|In|B0         |B0|In|B0

|s0|  |D0                   |s0|  |D0         |s0|  |D0

|  |  |R0                   |D1|  |R0         |D1|  |R1

|C0                   |A1|  |C0         |A1|  |C1

IV) Tagged Queing (ATAPI Only)

Cmd(1)    Cmd(2)             Data(2)               Data(1)

+--+--+   +--+--+--+        +--+--+--+         +--+--+--+

|SL|IC|o o|SL|IC|RL| o o o o|SL|TD|RL| o o o o |SL|TD|RL|

+--+--+   +--+--+--+        +--+--+--+         +--+--+--+

|B0|R1|B0 |B0|R1|B0         |B0|In|B0          |B0|In|B0

|s1|  |D0 |s1|  |D0         |s1|A0|D0          |s1|A0|D0

|t1|  |R1 |t2|  |R0         |D1|  |R0          |D1|  |R0

|tt|  |C0 |tt|  |C0         |T2|  |C1          |T1|  |C1

|A1|  |            |A1|  |

=======================================================================|

+------+------+------++----------------+----------------+

| BSY  | DRDY | DRQ  || New Cmd        | Change DRV     |

+------+------+------++----------------+----------------+

|  0   |   0  |   0  || Ignore New Cmd |       OK       |

+--------------------++----------------+----------------+

|  0   |   1  |   0  || Accept New Cmd |       OK       |

+--------------------++----------------+----------------+

|  0   |   1  |   1  || Abort Cur Cmd  | ?????????????  |

|      |      |      || Accept New Cmd |                |

+--------------------++----------------+----------------+

B=BSY,  D=DRQ, s=DRV, R=DRDY, C=DSC or POST, A=ATTN(IDX,Status:bit1),

Ia=HIRQ Asserted, In=HIRQ Negated, t(n) or T(N)=Tag, tt or TT= Tag Type,

ST=Set Taskfile, IC= Issue Command, RS= Read Status, TD=Transfer Data.

2.0 Shared IRQ:

The shared IRQ protocol is designed to allow more than one device on a

given IDE cable to process commands and data concurrently. To achieve

this goal the function of several bits within the task file have been

enhanced. Specifically the function and behavior of DRDY, DSC and IDX

have been changed as follows:

2.1 Shared IRQ Signal Timing:

+Issue New Cmd  + Release + Sel  + Xfer Data  +

RL +     IC        +    RL   +  SL  +    TD      + ICorRL

================================================================

__________________     h   +                     __

DRV    _/a+               +XXXXX_________________________/  XXXX

____________________         ____________________________

nIEN   __/b               e\_______/j      +            +\______

+__ c           +         +__k

cmd    ___/A0\______________________/A2\________________________

+   __       ___+         +   ___m        p___q

BSY    ______/  \_____/  d\____________/   \________/   \_______

+       ____     _________________n_______o   +

DRQ    __________/    \____XXXXX            XXXXXXXX________XXXX

+               +         +      +n           +

Data   ___________/\/\______________________/\/\/\/\____________

+               +        i+      +            +

Status __#__________________#_____#_______________________#_____

________________         +      n________p   + ______

T File ___/                \_______________/        \____/______

+               +    ____j       +            +

INTRQ  _______________________/g \_\____________________________

+               +   __________l  +            +

Attn   ______________________/f     +   \________________XXXXXXX

a-e) Issue Cmd to Device 1

f-j) Release Cable

k-l) Select Device 0

n-q) Transfer Device 0 Data

2.1.1 a-d) Issue Command

a) The IC-Issue Command phase of a transfer is started by slecting the

device too which the command will be sent. If the previous phase was an

IC to the same device the host may not need to Release and Select the

device again in order to issue it another tagged que command. Device 1

is used in the example above.

b) For backward compatibility the host must disable the assertion of

interupts using nIEN after selecting the device to which it intends to

issue a command. nIEN may already be disabling the interupts if the

previous phase was also an IC to the same device. Writing to nIEN with

any device selected shall prevent all devices on the cable from

asserting interupts to the host. Disabling interupts is neccissary to

prevent interupts from non selected device from confusing host drivers

while they are issuing commmands. Note that disabling interupts while

issuing ata and atapi commands requires atapi devices supporting shared

interupts to have hardware accelerated command packet DRQ capabilities.

If the host intends to issue an ATAPI Tagged command to the device then

it must set up the tt-Tag Type and Tn-Tag number in the task file,

before issuing the A0 ATAPI Packet command to the device.

c) After the host has disabled interupts it can issue a shared IRQ

command to the selected peripheral. Once the command has been issued it

will continue normally until BSY and DRQ are deasserted.

d) When BSY and DRQ are both deasserted the selected device is

signalling the host system that it is releasing controll of the cable.

2.1.2 e) Release Cable

If the host system has no other commands for that device or the devices

DRDY bit indicates that the device is not ready for another command, the

host should poll the other devices on the cable before enabling

interupts using nIEN. This is necissary to detect other devices capable

of accepting commands or devices which have pending interupts to the

host which are disabled because of nIEN. If no ther devices on the cable

require servicing then the host system can enable host interupts using

nIEN and wait for an interupt. The host system may also wish to launch a

timer thread to allow it to poll the devices on the cable for another

opportunity to issue more commands to the devices before the next

interupt from a device on the cable.

Note that the cables task file becomes invalid from the time that the

host uses nIEN to disable host interupts until nIEN is used to re-enable

host interupts or the device is selected using the ATA-3 A2 Select

Device command. (See j-n)

2.1.3 f-g) Request Attention

While the cable is Released, I.E. the host is not issuing a command ,

interupts are enabled, and no device has been selected to process an

interupt, any device on the cable may assert ATTN:IDX and assert the

interupt signal to the host to request attention from the host. Note

that the assertion of ATTN in the status register is independant of nIEN

and must preceed the assertion of the interupt signal.

2.1.4 i-j) Identify Device requesting Attention

Upon reciept of an interupt the host system must use the DRV bit to poll

the devices in the cable to determine which device is requesting

attention.

Depending on the IRQ mode selected using the Set Features command the

interupt signal asserted by the device may be iether a 1-2us pulse or a

signal negated by a read to the devices status register.

2.1.5 j-n) SL - Selection

Devices configured for Shared IRQ must be selected to enable their task

file for interupt processing. This is true for command termination and

data transfer interupts. In both cases the device will assert ATTN until

they are selected by the host using the ATA-3 A2 Device Select command.

Once the host has determined which device requires attention the host

must disable host interupts from the devices on the cable using nIEN

before issueing the ATA-3 Select Device command.

2.1.6 n-q) Transfer Data

Data transfer occures as defigned by the protocol for the non A2 command

issued to the device. The Transfer data phase in complete when the

device again releases the cable by de-asserting BSY and DRQ.

2.2 New ATA-3 Command Definitions:

Several new commands have been defigned to support Shared IRQ.

2.2.1 Select Device (A2h)

This command allows the host to assign control of the cable to a

specific device. The device is considered to have been selected only if,

after the command is issued to the device, the device de-asserts BSY

without aborting the A2 command. Devices aborting the A2 command do not

support the shared IRQ protocol and may need to be reset before they

will again function according to the traditional IDE protocol.

One issue yet to be resolved is how long the host system should be

forced to wait for BSY to be de-asserted when the host issues an A2-

Select Device command. Because nIEN has been set to disable interrupts

an interrupt can not be generated by the device to indicate that it is

done processing the Select Device command. Since it is assumed that

initial shared IRQ devices will not hardware accelerate the Select

Device command, the host system must poll for the de-assertion of BSY.

To prevent degradation of host performance it is recommended that the

host system poll for 50us after which it should generate a timer thread

to return control to the driver not more than 500us latter. The host

should continue to use a timer driven polling sequence for up to 3ms at

which point the host should reset the devices on the cable assuming a

catastrophic command failure has occured.

2.2.2 Set Features:

Enable Shared IRQ Protocol:

Disable Shared IRQ Protocol:

These two commands are used to enable and disable the shared IRQ

protocol within the devices on the cable. These commands are designed to

allow the OS drivers to identify devices compatible with the shared IRQ

protocol and configure them for maximum performance. The device will

abort or accept these commands to identify it's compatibility with the

feature.

Peripherals shipped with Shared IRQ protocol enabled may lock up BIOS

and OS drivers which are not designed to utilize this feature when they

are unable to abort an active command because DRDY remains = 0. BIOS and

drivers which issue an SRST as a result of a DRDY time-out are 100%

compatible with drives shipped with Shared IRQ enabled.

Enable OCI-Open Collector IRQ Protocol:

Disable OCI-Open Collector IRQ Protocol:

These two commands are used to enable and disable the open collector IRQ

protocol within the devices on the cable. These commands are designed to

allow the BIOS or adapter drivers to identify devices compatible with

the open collector protocol and configure the host and peripherals for

maximum performance. The device will abort or accept these commands to

identify it's compatibility with the feature.

2.3 New Bit & Signal Definitions:

2.3.1 DRDY:

DRDY has been changed so that it can be used to indicate when the

peripheral is ready to accept another command into it's command Que. To

achieve this functionality the peripheral no longer aborts the current

command when it receives a new command with DRDY=1. When the peripherals

que is full then it must indicate this condition to the host system by

de-asserting DRDY. Commands received while DRDY=0 shall be ignored by

the peripheral. Note that de-asserting DRDY may prevent the host from

aborting a command by issuing another command before the first command

has completed.

2.3.2 DSC (POST):

DSC has been changed so that it can be used to indicate when the

peripheral has completed the command for which it is reporting status.

In the case of Tagged ATAPI commands the DSC bit is used in conjunction

with the tag to identify which command in the que has completed.

2.3.3 IDX (ATTN):

IDX has been changed so that it can be used to indicate which

peripherals are asserting HIRQ. This bit mimics the peripherals internal

IRQ bit and is unaffected by the state of nIEN. Note that this bit may

also be used to poll the peripherals without the use of interrupts.

3.0 Tagged Queuing (ATAPI Only):

The tagged queuing protocol for ATAPI is designed to allow the

peripheral to process multiple commands concurrently. To achieve this

goal the function of several TAG fields within the ATAPI task file have

been defined. Specifically the function and behavior of Tag and Tag Type

have been defined as follows:

If only SIMPLE QUEUE TAG commands are used, the peripheral may execute

the commands in any order that is deemed desirable.

If ORDERED QUEUE TAG commands are used, the peripheral shall execute the

commands in the order received with respect to other commands received

with ORDERED QUEUE TAGs. All commands received with a SIMPLE QUEUE TAG

Type prior to a command received with an ORDERED QUEUE TAG type, shall

be executed before that command with the ORDERED QUEUE TAG type.

All commands received with a SIMPLE QUEUE TAG Type after a command

received with an ORDERED QUEUE TAG Type, regardless of initiator,

shall be executed after that command with the ORDERED QUEUE TAG type.

A command received with a HEAD OF QUEUE TAG type is placed first in the

queue, to be executed next.  A command received with a HEAD OF QUEUE TAG

message shall be executed prior to any queued I/O process.  Consecutive

commands received with HEAD OF QUEUE TAG type are executed in a last-in-

first-out order.

A command received without a queue tag type, while there are any

tagged I/O commands in the command queue, shall be aborted with a Tag of

00h.

An ATAPI peripheral that supports shared IRQ, but not tagged queuing,

shall use DRDY to accept one tagged command at a time. All peripherals

supporting Shared IRQ shall return the tag issued to the peripheral with

the command.

3.1 Typical sequences for tagged queuing:

An ATAPI command using tagged queuing uses the following sequences for

normal execution. The host system selects the device and monitors DRDY

until the device is ready to accept another command (DRDY=1). The host

then sets up the task file for the command including the Tag and Tag

Type fields. Note the Tag and Tag Type fields are specified as 0 for non

tagged ATAPI commands. Once the Task file is configured then the host

reads the status register again to verify that the peripheral is ready

to accept a command. If DRDY=1 then the host writes to the command

register to issue the tagged command. The host is then free to issue

more queued commands to this peripheral or select another device on the

cable.

3.2 Aborting Tagged Commands:

The host may abort a specific tagged command in the peripherals que by

issuing a new command to the peripheral with the tag used for the

command to be aborted. The new command is placed into the que using the

order defined by the new tag type. Note that the tag type is not

required to match the original tag.

Peripherals can abort tagged commands using the existing aborted command

sequence with the additional requirement that the peripheral identify

the aborted command with the commands tag.

Cmd(1)    Cmd(2)            Data(2)              Data(2)

+--+--+   +--+--+--+        +--+--+--+         +--+--+--+

|ST|IC|o o|ST|IC|DC| o o o o|SL|TD|DC| o o o o |SL|TD|DC|

+--+--+   +--+--+--+        +--+--+--+         +--+--+--+

|B0|R1|B0 |B0|R1|B0         |B0|In|B0          |B0|In|B0

|s1|  |D0 |s1|  |D0         |s1|A0|D0          |s1|A0|D0

|t1|  |R1 |t2|  |R0         |D1|  |R0          |D1|  |R0

|tt|  |C0 |tt|  |C0         |A1|  |C0          |A1|  |C1

|T2|  |            |T2|

Abort(1)

+--+--+

|SL|DC|

+--+--+

|B0|B0

|s1|D0

|D0|R0

|T1|C1

|E1

3.3 Example of Tag Que Ordering:

An example of the execution of five queued Commands is described to

demonstrate how tagged queuing operates.  The five commands are defined

in the table below.  At the time the Commands are first being executed,

it is assumed that the actuator is in position to access logical block

10 000.

Commands in order received by target

+==========-============-============-==========-============-=========+

|  Command |  Queue tag |  Queue tag |  Logical |   Transfer |  Status |

|          |   message  |    value   |   block  |    length  |         |

|          |            |            |  address |            |         |

|----------+------------+------------+----------+------------+---------|

|   READ   |    SIMPLE  |     01h    |   10 000 |    1 000   |  Queued |

|   READ   |    SIMPLE  |     02h    |      100 |        1   |  Queued |

|   READ   |   ORDERED  |     03h    |    1 000 |    1 000   |  Queued |

|   READ   |    SIMPLE  |     04h    |   10 000 |        1   |  Queued |

|   READ   |    SIMPLE  |     05h    |    2 000 |    1 000   |  Queued |

+======================================================================+

The optimum order would require that those blocks close to the actuator

position be the first blocks accessed, followed by those increasingly

far from the actuator position.  However, the command with queue tag 03h

is an ordered command, so that all simple Commands transferred

previously must be executed before, while all simple Commands

transferred after the ordered command must be executed after the ordered

command.

If a target supports an optimizing algorithm the actual order in which

the Commands are executed could be as shown in the table below.

Commands in order of execution

+==========-============-============-==========-============-=========+

|  Command |  Queue tag |  Queue tag |  Logical |   Transfer |  Status |

|          |   message  |    value   |   block  |    length  |         |

|          |            |            |  address |            |         |

|----------+------------+------------+----------+------------+---------|

|   READ   |    SIMPLE  |     01h    |   10 000 |    1 000   |  Queued |

|   READ   |    SIMPLE  |     02h    |      100 |        1   |  Queued |

|   READ   |   ORDERED  |     03h    |    1 000 |    1 000   |  Queued |

|   READ   |    SIMPLE  |     05h    |    2 000 |    1 000   |  Queued |

|   READ   |    SIMPLE  |     04h    |   10 000 |        1   |  Queued |

+======================================================================+

Commands with queue tag values 01h and 02h are executed in the order

received since the actuator is already in position to execute command

01h.  Command 02h must be executed before Command 04h or 05h because

the ordered Command 03h was transmitted after Commands 01h and 02h

but before Commands 04h and 05h.  Command 03h is then executed after

Command 02h.  The Commands 04h and 05h are executed after the

ordered Command 03h.  Command 05h is executed before Command 04h

because the actuator is in position to access block 2 000 after

executing Command 03h.  Command 04h is executed last.

As an example of the operation of the HEAD OF QUEUE TAG Command,

consider that a new Command, identified by a HEAD OF QUEUE TAG message

with a queue tag of 08h, is transmitted to the target while the ordered

Command 03h is being executed.  The Command 03h continues execution, but

the new HEAD OF QUEUE TAG Command is placed in the queue for execution

before all subsequent Commands.  In this case, the queue for execution

after the ordered Command 03h was executed would appear as shown in the

table below.

Modified by HEAD OF QUEUE TAG message

+==========-============-============-==========-============-=========+

|  Command |  Queue tag |  Queue tag |  Logical |   Transfer |  Status |

|          |   message  |    value   |   block  |    length  |         |

|          |            |            |  address |            |         |

|----------+------------+------------+----------+------------+---------|

|   READ   |   ORDERED  |     03h    |    1 000 |    1 000   |Executing|

|   READ   | HEAD OF QUE|     08h    |        0 |        8   |  Queued |

|   READ   |    SIMPLE  |     05h    |    2 000 |    1 000   |  Queued |

|   READ   |    SIMPLE  |     04h    |   10 000 |        1   |  Queued |

+======================================================================+

To obtain maximum performance gains using tagged queuing requires

careful implementation of the queuing algorithms in the target.  In

addition, host systems should allow a maximum number of simple Commands

to be executed with a minimum number of ordered Commands.

---------------------------------------------------------------------------

_/    _/ _/_/_/  _/_/_/  Western Digital Corporation

_/ _/ _/ _/   _/ _/       Tom Hanan

_/_/_/_/ _/   _/ _/        Ph.  714 932-7472,  Fax. 714 932-7314

_/    _/ _/_/_/  _/_/_/     E-Mail hanan_t@a1.WDC.com

-------------------------------- END --------------------------------------

From ata-owner  Mon Aug 29 13:26:34 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22352; Mon, 29 Aug 94 13:26:34 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA22344; Mon, 29 Aug 94 13:26:28 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id NAA28439; Mon, 29 Aug 1994 13:19:01 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Mon, 29 Aug 1994 10:05:43 EDT

Date:         Mon Aug 29 10:05:42 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e61eb37.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: Prefetch, local bus, etc...

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Jim McGrath wrote:

>At a higher level of concern, there is no industry forum to discuss issues

>relating to hardware on the system side of the bus (ATA does not really

>cut it).  I would like us to either establish a new group to do this, or

>have an existing group take on the responsibility of working on

>system chip/BIOS/OS issues.  Any takers?

Actually there is (are). If you literally meant ATA then the host side

equivalents would be various IEEE groups (projects). They have an international

counterpart in JTC 1/SC 26. If you actually meant SFF then the groups would be

ones like EISA, PCI, VITA, PCMCIA etc. IEEE has a boot standard but I accept

the homily that BIOS writers don't participate (although I have no idea if the

homily is true). OS in the sense of UNIX is dealt with by IEEE and languages by

X3 and perhaps IEEE.

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\696.MSG

### Org Date:    08-29-94 06:28:19 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: Prefetch, local bus, etc...

### Attachments: none

From ata-owner  Mon Aug 29 13:28:33 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22361; Mon, 29 Aug 94 13:28:33 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA22353; Mon, 29 Aug 94 13:28:28 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id NAA28669; Mon, 29 Aug 1994 13:22:31 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Mon, 29 Aug 1994 10:05:51 EDT

Date:         Mon Aug 29 10:05:51 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e61eb40.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: ATA Working Group Agenda

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Jim McGrath wrote:

> 1) We have been requested to provide the capability of

>     eliminating the need for the PIO mode of data transfer completely

>     in the future.  This requires that the data transfer commands

>     other than READ/WRITE (Identify drive, Read buffer, Read long,

>     Format, Write buffer, Write long) have a DMA mode of transfer.

>     To be consistent with the previous READ DMA/WRITE DMA

>     implementations, this would require new op codes for DMA

>     versions of these commands, which would otherwise execute in

>     the same manner as before.  Assignment of op codes is thus the

>     action requested.

Another alternative would be to eliminate or not use some of those commands.

Jim further wrote:

>3)  I would like to make sure we follow up on WD's suggestion of adding some

>     guidance to hosts in implementing local bus modes on their systems.

>     I do think this would be valuable, and are willing to work with others

>     either next week or at other meeting times/locations if people do not

>     believe this is an ATA-2 topic.

We have agreed that the period has gone down on ATA-2 except for needed

editing. Therefore this is a clear ATA-3 topic and should be completed as early

as the contributors are able to. I say clear, not really knowing the scope. It

could be it should be contributed to IEEE but let's not worry about that until

a paper is completed.

Gene

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\702.MSG

### Org Date:    08-29-94 06:40:22 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: ATA Working Group Agenda

### Attachments: none

From ata-owner  Mon Aug 29 13:37:06 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22585; Mon, 29 Aug 94 13:37:06 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA22578; Mon, 29 Aug 94 13:37:00 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id NAA29032; Mon, 29 Aug 1994 13:27:12 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Mon, 29 Aug 1994 10:06:03 EDT

Date:         Mon Aug 29 10:06:02 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e61eb4b.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: ATA Rev 2F -- Section 6.2.13, etc

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hale wrote:

>In a single device configuration where device 1 is the only

>device and the host selects device 0, device 1 shall respond to

>accesses of the Command Block and Control Block registers in the

>same way it would if device 0 was present.  This is because

>device 1 can not determine if device 0 is, or is not, present.

It could be construed that this is part of the second, non recommended method.

This is especially true readers who don't want to be contaminated by

non-recommended methods and consequently don't read them. I suggest moving it

ahead of the device 0 description.

You gotta not use "must" in your proposals. Please globally change "must" to

"shall".

Gene

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\710.MSG

### Org Date:    08-29-94 07:07:27 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: ATA Rev 2F -- Section 6.2.13, etc

### Attachments: none

From ata-owner  Tue Aug 30 12:54:43 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA20432; Tue, 30 Aug 94 12:54:43 PDT

Received: from alink-gw.apple.com by dt.wdc.com (4.1/SMI-4.1)

id AA20413; Tue, 30 Aug 94 12:53:30 PDT

Received: by alink-gw.apple.com (921113.SGI.UNSUPPORTED_PROTOTYPE/7-Oct-1993-eef)

id AA03316; Tue, 30 Aug 94 12:53:24 -0700

for ATAPI@DT.WDC.COM

Date: 30 Aug 94 19:48 GMT

From: YANG1@AppleLink.Apple.COM (Matsushita/Panasonic, C Yang,CST)

Subject: Duplicate Copy received !

To: ATA@dt.wdc.com, ATAPI@dt.wdc.com

Message-Id: <778276403.6108386@AppleLink.Apple.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Dear Mr. Reflector:

Even today, I am still receiving one (only one, used to be a few !) duplicate

copy for each file. It seems somebody has the same problem as I have. Is this

problem being solved now ?

Best Regards.

From ata-owner  Tue Aug 30 16:32:29 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA25256; Tue, 30 Aug 94 16:32:29 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA25240; Tue, 30 Aug 94 16:32:16 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id QAA27397; Tue, 30 Aug 1994 16:07:22 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Tue, 30 Aug 1994 12:46:57 EDT

Date:         Tue Aug 30 12:46:56 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e636282.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      ATA-2 Comments

Sender: ata-owner@dt.wdc.com

Precedence: bulk

8/30/94

To: Steve Finch and John Lohmeyer

Cc: Everybody Else

Subject: Comments on X3T10 948D Rev 2f

I could not find sufficient time to completely review the subject document.

Consequently I thought it would be most productive for me to rely upon earlier

reviews and comments on prior material and to focus on those portions that were

not in ATA earlier. Consequently my comments are in two major portions. The

first portion addresses the comments I made on ATA. The second portion are new

comments.

With regard to the comments I made many moons ago on X3T9.2 791D Rev 4A. I

find the following of those comments using the old numbers (omitted numbers

were resolved in ATA-2 if not in ATA)to still be an issue:

Mainly in order of occurrence in the document the comments are:

3) Delete page ii(now 4) document status.

6) In conventions the statement for "Names" seems to be from and applicable to

SCSI not ATA.

9) I think the 31 seconds in the second paragraph of page 9 (now 5.2.4 on page

24, fourth paragraph) is in error. I believe it should be 30 seconds

(neglecting the issue as to whether or not BIOS implementations will actually

allow such a long time). (Added comment: My recollection was that Drive 1 had

400 ms to set it and another 30 sec to reset it. Consequently Drive 0 only has

to wait 31 seconds for it to be negated (0.4S +30S +0.6S [committee slack]).

(See Figure 9)

17) On page 18 (now 35 sec 6.2.13) the state of the DRDY bit after an error but

before the Status Register is read is not clearly stated. The English seems to

imply that the DRDY bit which was One before the error remains One after the

error and then is changed to zero after the Status Register is read. This is

not the case.

18) The DSC bit is nearly as awkward.

19) (In 7.1 Reset response) It is not clear to me that there should be a

requirement that DASP- be asserted for 30 seconds. I believe it should just be

asserted for at least 450 msec and until PDIAG- is asserted which must be

within 30 seconds.

24) The last paragraph (of the note in) of 9.13 (now 8.14)states a problem

without offering a solution. In fact the NOP is the solution for the stated

problem and the paragraph needs to be turned around to state this or deleted.

In particular I believe it is the PCMCIA case for which this function is

required.

25) Section 9.15 (now 8.16) needs to be more clearly stated to identify the

point at which data transfers cease upon error. A possibility is "... results

in the termination of data transfer after transfer of the sector without an

error prior to the sector with the detected error. The sector in error is not

transferred. The drive ...". A similar change should be made in 9.18 (now 8.19

and now OK).

32) The statement at the end of 10.1.1 (now 9.1.1) that transfer of data with

errors is at the option of the host appears to be in conflict with contrary

statements in section 9 (now 8.19 etc.).

35) In Annex A the first three key bullets (now the second a), b) c))are not

just timing assumptions. I would replace "The timing assumes the following:"

with "The diagnostic and reset architecture requires the following:".

36) In A.1.1 add "is" before negated in the third line. Change the sixth line

to Drive 0 waits up to 1 msec then samples ...". (Added comment: tP in Figure 9

is a maximum. There is no need for Device 0 to wait beyond the point where

device 0 is guaranteed to be not driving the line. It makes no difference if

Device 1 is detected early. The time constraint falls on allowing time for

PDIAG to be negated.)

38) There is no requirement that PDIAG- be deasserted by drive 1 prior to

asserting DASP-. Consequently change the first line of A.1.2.1 to "Drive 1

negates PDIAG- within 1 msec." (The change has now been partially made but not

sufficient. It is important that the deassertion of PDIAG be bounded. This is

also a problem in Figure 9. Please clean up Figure 9 to include the negation of

DASP within 1 ms by Device 1.)

40) A.1.1.2 and A1.2.2 Should be changed to "After up to 1 msec, Drive 0 waits

...".

41) In the seventh line of A.1.2.1 and the third line of A.2.2.1 a more

accurate statement is "Drive 1 asserts PDIAG- to indicate that it has completed

its diagnostic routines (within 30 seconds from the trailing edge of RESET-)."

(Thank you for making the change but the trailing edge was missed.)

42) Change "A.2.1A.2.1" "A.2.1". Also either change the "(within 31 seconds)"

to "(within 30 seconds)" or delete it. (Added comment: Part one taken care of

but not part two. Please note that I am assuming a software reset does not come

into play until after a hardware reset has already established drive one

present or not. It may be desirable to treat it the same as a hardware reset

which would infer the 31 seconds perhaps. If you stick with 31 seconds please

change Figure 9 to reflect that BSY may extend out to tS for drive 0. Also

A.2.1 needs to account for the single drive being drive 1 unless we protect the

end user.)

New Comments:

1) In the second sentence of 6.2.8 change contents bits to contents of bits.

2) In 8.9 change systme to system. This suggests a spell checker should be run.

3) Figure 9 is confusing in that there is an implication that all the signals

shown on the lower half are from Device 1. However tR applies to device 0 also.

tR should be drawn in the top half also.

4) Why isnt the note in item (e) of all the A.3. sections in direct conflict

with A.1.(g), and Figure 9?

5) Regarding A.2 isnt there a problem that BIOSs dont wait 31 seconds after a

soft reset?

6) In D.2 why is ATA (X3.221) called out rather than ATA-2?

7) Annex D is titled Informative but uses Normative shall language. Make it

consistent. If it is made Normative be sure to label it Optional.

8) In D.3.8 at first blush the requirement to not resample SELATA across a

reset is unreasonable. Please insert the phrase Because RESET polarity is

changed by the initial state of SELATA- ahead of the device shall not resample

....

9) The DOOR LOCK and DOOR UNLOCK commands do not seem to include any guidance

that justifies the last line of D.4.1. and D.4.2.

10) In E.1 change sectors in the first two instances to 512 Byte sectors.

11) In the first full paragraph of E.1 replace , and these with which.

12) At the end of the first full paragraph of E.1 and elsewhere the term 528 MB

is used. This was defined just above to be 512 MB. The Annex should be made

consistent.

13) Should the third full paragraph of E.1 add the phrase as well as by LBA

addressing?

14) In the last paragraph of E.1 replace specification with annex.

15) In the first paragraph of E.2 replace mode to use and determine with mode

to determine.

16) Change the first sentence of the second paragraph of E.2 to The rules in

this annex are defined to facilitate compatibility. Delete the last sentence as

it is redundant to E.1.

17) The last sentence of E.3.2 seems to be a nonsequitur.

18) Annex E should be identified as Normative.

19) In Annex F replace provide information on which command with shows which

command.

20) In the notes of Table 18 what does it mean for a command code to be

identical to a unique command?

21) In Annex F the meaning of Reserved is problematic.

22) The note in Table 19 is inside out. The commands appear once with two

command codes. It appears the note was from when it was intended to do a

complete sort and then that concept was partially abandoned.

23) In Section 2 replace X3T9.2/93-791D with X3.221. But why is it a normative

reference? Doesnt ATA-2 replace ATA? Are we making conflicts between the two

normative? I think I would make it an informative reference.

24) The change to 3.1.11 Reserved is a technical change, is in agreement with

the X3T10 vote (albeit perhaps only on SCSI), and I am in complete agreement

with it. Please delete the Editors note.

25) In 4.2.1 the part numbers are for the cable. The part numbers and term

mating came from the Wren II AT specification and we used the term mating to

mean what the customer attached to the drive (the center of the universe).

26) Regarding the Editors note about feeling uneasy, this is the way ANSI is

publishing ATA and is the way ESDI was. If you guys, not me, go along with Dals

desire to finish ATA in ECMA, you may need to deal with it there. Certainly I

would expect ISO/IEC to complain - although there are ways around it in some

cases. A SFF callout, which exists, would be better but technically has a

similar problem with ANSI and certainly ISO/IEC. At this point I would just

delete the Editors note.

27) In 7.3.6 change Figure ? to Figure 5.

28) I think it makes good marketing sense in most segments to implement all

those power modes. But I dont see why I have to for the ATA drives I am

installing in Boulder Dam. Why did they become mandatory? In ATA they were

optional.

29) In 8.7 add a phrase that DOWNLOAD MICROCODE is extremely optional.

30) Is the Editors note below Table 12 referring to Annex E or additional

restrictions? Are they teed up somewhere? If they have not been teed up

somewhere or if they have been designated for ATA-3 then we should consider it

complete.

31) The stuff on page 72 should be labeled.

32) Is Figure 11 and 12 misplaced? Were they intended to be in Section 10?

G.E. Milligan

X3T10 Member

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\1244.MSG

### Org Date:    08-30-94 12:10:25 PM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     ATA-2 Comments

### Attachments: none

From ata-owner  Tue Aug 30 17:33:18 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA26563; Tue, 30 Aug 94 17:33:18 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA26556; Tue, 30 Aug 94 17:33:12 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA263893188; Tue, 30 Aug 1994 17:33:09 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA032862997; Tue, 30 Aug 1994 17:29:57 -0700

Message-Id: <n1433854403.11608@qm_smtpgw.qntm.com>

Date: 30 Aug 1994 17:38:44 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: AT Command Queuing

To: "ATA" <ATA@dt.wdc.com>, "ATAPI" <ATAPI@dt.wdc.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  5:13 PM

OFFICE MEMO         AT Command Queuing                    Date:  8/30/94

Comments on the WD proposal:

I have read the proposal once lightly and have three observations.

First, I am withholding comment on the low level signal issues

regarding overlapped commands (I have to try out more scenerios

first).  But on the issue of tagged command queuing I have two

comments:

First, tagged command queuing must be offered on ATA as well

as ATAPI.  Indeed, it is probably more important for disk drives

to institute queuing than, say, tape drives.  Therefore for the

tag value, I propose that it be passed in register 03F3h.  This

is part of the device control block of registers which is currently

decoded by all ATA/ATAPI devices.  Most of the registers in this

area are used for the floppy disk, with a few for the hard disk.

But while 3F2h and 03F4h 8 bit registers are used, 03F3 is not

used to my knowledge.  03F3h thus becomes an 8 bit read/write

register for the tag value.

Second, I would remove all provisions for tag types in ATA/ATAPI

(i.e. head of queue, ordered commands).  All queued commands would

be treated as simple commands.  Based on our SCSI experience, this

would greatly simplify the device implementation.  And in practice

we found little use for head of queue (which really does nothing

in the final analysis to insure quick execution of the command) or

ordered commands (which are better handled by simply not queuing

the command at the drive to begin with).  Note that all of this results

in a slight shift in complexity from the drive to the driver (i.e. delaying

sending down ordered commands), but that is probably the right tradeoff

to make.

Jim

From ata-owner  Wed Aug 31 06:33:58 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA11205; Wed, 31 Aug 94 06:33:58 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA11198; Wed, 31 Aug 94 06:33:50 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id GAA16865; Wed, 31 Aug 1994 06:30:28 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Wed, 31 Aug 1994 06:17:51 EDT

Date:         Wed Aug 31 06:17:51 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e6458d0.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: AT Command Queuing

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I agree with Jim's suggestion of not having different queue tag types. It seems

essential that the code complexity be kept at a minimum to retain ATA's

differentiation.

Gene

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\1482.MSG

### Org Date:    08-31-94 06:06:53 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: AT Command Queuing

### Attachments: none

From ata-owner  Thu Sep  1 07:13:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA10667; Thu, 1 Sep 94 07:13:47 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA10648; Thu, 1 Sep 94 07:10:38 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id GAA06354; Thu, 1 Sep 1994 06:45:26 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Thu, 01 Sep 1994 06:32:47 EDT

Date:         Thu Sep 01 06:32:47 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e65add0.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Forwarding of Comments I received on 2F.

Sender: ata-owner@dt.wdc.com

Precedence: bulk

1) The 2nd line in Annex E, "E. Identify Drive Data Requirements" should be

changed to "E: Identify Device ..." to be consistent with the rest of the

document.

Concerning my comment:

>12) At the end of the first full paragraph of E.1 and elsewhere the term 528 MB

>is used. This was defined just above to be 512 MB. The Annex should be made

>consistent.

2)  First, a definition of MB should be made, as most use 2^20, but some use

10^6

(even DOS is not consistent).

The 512 MB in line 4 is wrong in either base. In 10^6 it would be 528MB, and in

2^20, it would be 504MB (528,482,304/1,048,576). This is the number you would

see if you created a single partition disk with FDISK. I believe that Hale used

1024*16*64*512/2^20 to get this number, however you can only achieve 63

sectors/track in the ATA world.

Subsequent to his definition, he appears to be consistent with using 10^6 based

numbers.

8 GB does not have enough resolution to determine what base is being used (

10^6=8.46GB, or 2^20=8.26GB). This is probably not material to the

specification as it is not subsequently used as a cross over point.

I (the originator - and I agree (GEM)) recommend that a base be defined first

(10^6 or 2^20),

and then consistently used. 2^20 has the advantage of producing round numbers,

however requires translation for those accustomed to using 10^6 based numbers.

3)  Also in this paragraph ( third full paragraph of E.1 ),  "Identify Drive"

should be changed

to "Identify Device" to be consistent with the rest of the document.

ADDITIONAL ISSUES

4) In 6.2.13 Status Register, under BSY b), and c), various commands are shown

in capital letters. However, Read, Read Verify, and Write are in lower case.

Also,

since there is no read or write command defined in ATA, shouldn't they be READ

SECTOR(S) and WRITE SECTOR(S) ?

2) Sections 6.2.13, 9.1, and 9.2 need a lot of clarification. 6.2.13 implies

that status bits will be reset upon reading, when in fact none of the interface

chips we use do this. In general the status remains until the next command is

issued, regardless of the number of times that the status register is read.

6.2.13 BSY c) requires BSY to be toggled between sectors for multi-sector write

operations, but nowhere in this text is there a read command corollary.

However, 9.1 describes inter-sector BSY's.

Also, there is no minimum time requirement for BSY to be set (which is OK). We

have had implementors try to poll on BSY instead of using interrupts for data

transfer. Since this is not a latched status bit, on fast drives it may only be

set for nano-seconds before the next DRQ is set, and therefore never be seen by

the host polling routing. A warning should be placed in the text (6.2.13, or

9.1 and 9.2) indicating that you can not rely on this bit for polling.

3) 9.2.1 shows BSY=1 after command issue (although not described in 9.2).

Today's chips have an auto-write feature which goes right into data transfer

without raising BSY. This notation should be removed from the diagram.

4) 6.2.13 -CORR   I know that I (I is the originator not GEM) mentioned this

once before, but any drive

formatted with 63 sector/track (which most over 400 MB are) reporting this

status will cause an fallacious error (possibly uncorrectable) due to a

long-standing bug in DOS. This status bit should be made vendor specific, like

IDX. It has outlived its usefulness.

I (GEM) have forwarded these comments since they were E-Mailed to me without

being posted

on the reflector.

Gene

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\229.MSG

### Org Date:    09-01-94 06:25:04 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Forwarding of Comments I received on 2F.

### Attachments: none

From ata-owner  Thu Sep  1 10:46:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA15415; Thu, 1 Sep 94 10:46:42 PDT

Received: from ncrhub1.NCR.COM (h192-127-251-16.NCR.COM) by dt.wdc.com (4.1/SMI-4.1)

id AA15408; Thu, 1 Sep 94 10:46:29 PDT

Received: from ncrcolo by ncrhub1.NCR.COM id ai15058; 1 Sep 94 12:50 EDT

Received: by ncrcolo.FtCollinsCO.NCR.com; 1 Sep 94 10:47:40 MDT

Subject: ATA-3 & ATAPI Project Proposal Letter Ballot Results

To: ATA Reflector <ata@dt.wdc.com>, ATAPI Reflector <atapi@dt.wdc.com>

Date: Thu, 1 Sep 94 10:49:14 MDT

From: John Lohmeyer <jlohmeye@ncr-mpd.ftcollinsco.NCR.COM>

X-Mailer: ELM [version 2.3 PL11]

Message-Id:  <9409011250.ai15058@ncrhub1.NCR.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Letter Ballot results on the ATA-3 and ATAPI Project

Proposals

The ATA-3 Project Proposal (X3T10/94-019 r4) passed: 50-0-0-

10.

The Yes ballot from Peter Brown of Oak Technology included

the following comment:

"I think that the initial proposal offered a better

partition and would be more useful to people attempting to

build products adhering to the specifications. I further

think that the reason that the proposal was changed was an

attempt to orphan ATAPI.  I object to this tactic."

The following organizations failed to return their ballots:

AMD, CMD, Compaq, Fujitsu, Harbor Electronics, Hitachi,

Linfinity Micro, Maxoptix, Qlogic, and Quantum.

The ATAPI Project Proposal (X3T10/94-022 r4) passed: 47-3-0-

10.  We are obligated to attempt to resolve the negatives.

The No ballots from Scott Smyers of Apple, Peter Brown of

Oak Technology, and Bob Snively of Sun Microsystems included

the following comments:

Apple:

The use of ATAPI is primarily intended to be a "low-cost"

interface for CD-ROM and tape devices.  I don't understand

the concept behind "overlapped commands using a multi-

threaded protocol."  This scope will have a direct impact on

the development time, the availability, and the cost of

ATAPI devices.

We would like to maintain ATAPI as a "low-cost", relatively

simple interface.  We would request to remove item b) of the

Recommended Scope from the project proposal, dated July 21,

1994.

Oak Technology:

I voted NO on the ATAPI proposal. I will change my NO vote

to a yes if either the scope statement explicitly states

that ATAPI may be used for hard disk drives or if 2.2.d is

deleted.

Although 2.2.d is ambiguous it could be easily argued that

"other than traditional ATA devices" excludes ATAPI HDD's.

Section 4.1 and 4.2 points-out the possibility of

"nonstandard evolution or

revolution" in the case that HDD is specifically excluded

from the spec. The scope of the document should accept the

inevitability of ATAPI HDD's.

Sun Microsystems:

I regret that I must vote no on the project proposal for the

ATA Packet Interface as the proposal is presently worded. By

making the changes described in the following paragraphs, my

vote would be changed to a yes vote.

1) Section 2.2

The last sentence of the clause now reads:

"The application for the proposed standard does not require

compliance with the SCSI-3 Architectural Model (SAM)."

It is very important that the ATAPI definition provide a

software interface that is compatible with standard SCSI

devices and drivers. At the same time, it is clear that

ATAPI implementations integrate a large number of the

functional interfaces defined by SAM. For this reason, the

referenced sentence should be rewritten to read:

"The proposed standard shall define services and functions

compliant with those visible to the application client

defined by the SCSI-3 Architectural Model (SAM). An ATAPI

device may internally implement those services in a manner

that does not comply with the SAM."

The following organizations failed to return their ballots:

AMD, CMD, Compaq, Fujitsu, Harbor Electronics, Hitachi,

Linfinity Micro, Maxoptix, Qlogic, and Quantum.

--

John Lohmeyer                      E-Mail:  John.Lohmeyer@FtCollinsCO.NCR.COM

NCR Microelectronics                Voice:  719-573-3362

1635 Aeroplaza Dr.                    Fax:  719-597-8225

Colo Spgs, CO 80916              SCSI BBS:  719-574-0424 300--14400 baud

From ata-owner  Fri Sep  2 13:21:37 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA17964; Fri, 2 Sep 94 13:21:37 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA17954; Fri, 2 Sep 94 13:21:21 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA213887279; Fri, 2 Sep 1994 13:21:20 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA098567083; Fri, 2 Sep 1994 13:18:03 -0700

Message-Id: <n1433610313.67566@qm_smtpgw.qntm.com>

Date: 2 Sep 1994 13:26:20 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: ATA Working Group Meeting

To: "ATA" <ATA@dt.wdc.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  1:10 PM

OFFICE MEMO         ATA Working Group Meeting             Date:  9/2/94

There will be an ATA working group meeting, hosted by Quantum:

Date:    September 27 and 28, Tuesday and Wednesday

Time:   9:30 am to 6 pm on Tuesday, 9 am to 4 pm on Wednesday

Place:   Crown Sterling Suites Hotel, Milpitas, CA

Agenda: Complete review of ATA-2 (we are now up to section 7)

plus any ATA-3 items as time permits.

Hope to see you there.

Jim

From ata-owner  Fri Sep  2 13:41:55 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18453; Fri, 2 Sep 94 13:41:55 PDT

Received: from netcomsv.netcom.com (uucp6.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA18316; Fri, 2 Sep 94 13:41:08 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id NAA08539; Fri, 2 Sep 1994 13:28:34 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qget2-000gJTC; Fri, 2 Sep 94 14:07 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Fri, 02 Sep 1994 14:10:20 EDT

Date:      Fri, 02 Sep 1994 14:10:20 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e676a8c.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>,

"ATAPI Reflector"    <atapi@dt.wdc.com>,

"SFF Reflector"      <sff_reflector@Sun.COM>

Subject:   Phone number correction, etc.

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I have decided that I really don't need four phone bills to pay

each month, three is enough for now, so... one of the numbers I

previously posted will never be my phone number.  Here is the

latest address/phone info for me...

Hale Landis

PO Box 204

Niwot, CO 80544

Phone 303-548-0567

Email landis@sugs.tware.com

Also, I just checked air fares from Denver to Houston for the

15-16 Sep 94 ATA and SFF meetings.  They want about 0.5 year of

my income for that trip!  I will not be at the Houston meetings

unless another air fare war starts real soon.

I understand there will be another ATA WG meeting later in Sep in

San Jose.  If so, I will be there.  (What is this I hear:  you

didn't even get to section 7 on Thursday?)

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Fri Sep  2 13:54:26 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA18727; Fri, 2 Sep 94 13:54:26 PDT

Received: from hp427u.tus.ssi1.com by dt.wdc.com (4.1/SMI-4.1)

id AA18718; Fri, 2 Sep 94 13:54:23 PDT

Received: from vax2t.tus.ssi1.com (vax2t.tus.ssi1.com [146.252.23.2]) by hp427u.tus.ssi1.com (8.6.7/8.6.6) with SMTP id NAA16547 for <ata@dt.wdc.com@relay>; Fri, 2 Sep 1994 13:55:43 -0700

Received: by vax2t.tus.ssi1.com via MESSAGE-ROUTER ; 2 Sep 94 13:46:33 PDT

Date: 2 Sep 94 14:41:34 PDT

From: "Steve Finch" <TFINCHS%A1.VAX2T.mrouter@vax2t.tus.ssi1.com>

Subject: ATA-2 Rev 2g available

To: "ata@dt.wdc.com" <ata@dt.wdc.com@hp427u.tus.ssi1.com>

Message-Id: <"43141220904991/320181@VAX2T"@vax2t.tus.ssi1.com>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

This morning I uploaded Revision 2g of the ATA-2 proposed standard.  This

document is now available on the X3T10 BBS.  I placed two files there:

0948DR2G.DOC is in Word For Windows 6.0 format; 0948DR2G.ZIP is a PKZIP

copy of the same file.

From ata-owner  Fri Sep  2 15:47:21 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA21423; Fri, 2 Sep 94 15:47:21 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA21416; Fri, 2 Sep 94 15:47:13 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id PAA07971; Fri, 2 Sep 1994 15:30:59 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 02 Sep 1994 13:17:14 EDT

Date:         Fri Sep 02 13:17:14 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e675e1b.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Action Item - Policy and Procedure for Timely Publication

Sender: ata-owner@dt.wdc.com

Precedence: bulk

X3T10 Standards Development Policy and Procedure

Proposed

9/2/94

Background:

The bulk of X3T10 projects and resources are directed towards the computer

industry. This industry is characterized by dynamic growth, diverse

applications, exponential performance growth rates, fierce competition, and

short product life cycles. As a key and central aspect of these

characteristics, the customers demand standards. Timeliness is paramount.

Consequently it makes little difference whether the standards be de jure or de

facto as long as they are responsive.

Although there has been tweaking at the margins of the X3 process there has

been no major overhaul to adjust to the dynamics of our industry. It is

important for us to align our process with the facts of the industry. At the

same time we must maintain the safeguards of due process that have been a

hallmark of X3. The following Policy and Procedure is defined to achieve both

needs.

Policy:

It is the policy of X3T10 to: focus standards development within the defined

scope of X3T10; control the development schedule to achieve timely publication;

and to provide regular updates of adopted standards which need enhancements to

fuel advancements in the industry. It is also the policy of X3T10 to tailor the

activity within the general provisions of the X3 Policies and Procedures.

Procedures:

Introduction:

These procedures are in accordance with the X3-SD2. They add additional

definitions which facilitate timely, proactive standards within the scope of

X3T10.

Project Initiation:

A proposal for a new project may be made by any participant of X3T10. The

proposal can take any form that the proposer thinks will be effective in

persuading the committee that it is a worthy project. Preliminary discussion of

the merits of the project should include both E-Mail and Plenary discussion.

If a majority of the membership does not object to the initial proposal, the

Chair shall authorize a study effort to assess the requirements for the

proposed standard, to generate a schedule for the development and publication,

and to develop working papers. Although the study effort in terms of ad hoc or

working group meetings will require regular re-authorization, there is no

requirement for participants to stop development work unless an actual new

project proposal is rejected by the OMC and/or X3.

When the study effort has developed a new project proposal in accordance with

the X3 SD-3, X3T10 should take a forwarding action on the new project proposal.

The dpANS should not be forwarded ahead of the approval of the new project by

the OMC.

If a majority of the membership does object to the initial proposal, remaining

proponents are free, but not necessarily encouraged to seek additional support

for the proposal or to adjust the proposal to better fit the views of X3T10.

The proposer can also move the adoption of the proposal for a new project in

spite of formidable odds against its adoption.

There are also some special circumstances for the automatic initiation of new

project study programs as outlined later in these procedures. The automatic

initiation is a pivotal aspect of the regular publication policy.

Project Control:

The Chair shall appoint subject to approval by a simple majority a project

leader for each new project for which the Chair has authorized a study project

and/or which has been approved by X3. The project leader may or may not be the

document editor. The project leader shall be responsible for providing a

progress report including schedule adherence, key issues, and corrective

actions at each plenary.

The document editor shall be responsible for maintenance of the document

reflecting agreements of X3T10. As a part of that responsibility, the document

editor shall make available at each meeting a listing of specific items (a

portion of the document covered by a numbered individual proposal [e.g. Caching

Page change proposal X3T10/99-8357]) accepted but not yet included in the

current working document.

Working Document Acceptance:

When a revised or new document is moved for acceptance, the project editor, or

proposer if there is no project editor, shall identify the technical changes

which are newly included in the revision to be accepted. The identification can

take the form of a listing of item document numbers which are newly

incorporated. This requirement is facilitated by the fact that the project

editor is not authorized to make technical changes without submitting an item

proposal as would any other participant. For technical items which are

instigated verbally through a meeting motion rather than an item paper, such

items shall be identified by reference to the meeting minutes document number.

Since working document acceptance is not final action, only a simple majority

is required for acceptance.

Working Document Stabilization:

There can be significant expense involved to test a standard. Since the burden

for testing falls on a voluntary membership, it is important that X3T10 assist

this process by providing guidance as to the stability of a dpANS. While there

is no requirement that this be indicated ahead of a forwarding vote, for

complex projects, X3T10 should take a vote to stabilize when they judge the

dpANS ready for testing by implementors. Since the goal is prompt publication,

the reason for a stabilization period is to prepare for publication while

confirmation testing is carried out. The required vote for stabilization is a

majority of the membership (this includes 2/3 of those voting). While

stabilized the following special voting rules apply:

1) Correction of a technical error shown by inspection or testing to be

classified as a technical error - simple majority

2) Correction of an editorial error - no vote required

3) Inclusion of a previously accepted item which is on the Editors List of

Accepted Items but not yet edited into the document - no vote required

4) Inclusion of a previously accepted item which is not on the Editors List of

Accepted Items but not yet edited into the document - simple majority

5) Inclusion of a new item - majority of the membership

6) Inclusion of better wording - simple majority

7) Change from stabilized to unstabilized - majority of the membership

Automatic Initiation of a New Study Effort:

The purpose of this section is to facilitate agreement to publish a sound

standard and yet account for the need for evolutionary enhancements. There are

three periods in the processing of a dpANS which are subject to the automatic

initiation: Stabilized Period; Public Review; and X3 Letter Ballot. The latter

two periods shall be interpreted as commencing with X3T10 forwarding and ending

with provision of the standard to the BSR.

If during the stabilization period a new item receives a simple majority but

not a majority of the membership, a follow up vote shall be taken to initiate a

study effort for a dpANS - (n+1). The study effort is authorized with a simple

majority. Any other subsequent items which receive a simple majority but not a

majority of the membership are automatically assigned to the study for dpANS -

(n+1). Note that study may be a full fledged development effort depending upon

the availability of resources. For dpANS - (n+1) studies, a preliminary

schedule for publication should be targeted early in the study.

If problems develop such that X3T10 destabilizes a project, any dpANS - (n+1)

items are then eligible for consideration in dpANS - (n).

Comments that arise in the Public Review and X3 letter ballot should be

handled similarly to items arising during the stabilization period.

1) Correction of a technical error shown by inspection or testing to be

classified as a technical error - simple majority determines if dpANS should be

recalled for correction or if an errata should be published with the ANS.

2) Correction of an editorial error - no vote required

3) Inclusion of a previously accepted item which is on the Editors List of

Accepted Items but left out of the document - automatic new study effort or

automatic inclusion on the dpANS-(n+1) list.

4) Inclusion of a previously accepted item which is not on the Editors List of

Accepted Items but not yet edited into the document - automatic new study

effort or automatic inclusion on the dpANS-(n+1) list.

5) Inclusion of a new item - treated as a new proposal for dpANS-(n+1) or (7)

applies

6) Inclusion of better wording - simple majority providing it does not cause

additional public review or a new X3 ballot. Otherwise subject to (7)

7) Recall of the dpANS for any reason except (1) - majority of the membership

Machine Readable Revision Control:

X3T10 shall provide on a forward looking basis a means for software

interrogation of the revision level of compliant standard implementations.

X3T10 shall also by a majority of the membership determine which document

revisions are deemed appropriate for reporting. There is no requirement for

revisions ahead of stabilization to be required for reporting, but the

revisions at stabilization and forwarding shall as a minimum be included in the

appropriate list. For standards which become subject to the dpANS-(n+1)

provisions the reporting shall accommodate a distinction of revisions extending

throughout the series. In the case of layered standards, the physical layer

revision provision may not be appropriate at that layer, however an upper layer

protocol standard shall provide provision for the reporting of revisions of

lower layers without the facility. This provision is included to facilitate the

concept of regular publication.

Extensive Public Review:

In order to further facilitate the timely publication of standards, X3T10

shall proactively seek wide dissemination of  proposals, issues, and documents

not just during the formal and essentially too late Public Review but

throughout the development process.

The Chair shall request volunteers to provide at least a X3T10 Internet

Reflector. If appropriate, additional volunteers should be sought to provide

Reflectors on a dpANS-(n+1) basis. If volunteers can not be found,

consideration should be given to purchasing the service by a special assessment

to the mailing fee. Admission to meetings, mailings, and reflectors shall be

open to all that comply with X3T10 and Internet rules. In addition all accepted

working documents shall be made available via an FTP sight providing a

volunteer has provided that service. At the option of X3T10 a BBS may be used

in addition to or in lieu of the Internet.

Press releases announcing new X3T10 projects shall include information on how

to access at least the X3T10 principal reflector and/or BBS along with the

information that is routinely included in new project press releases.

Revision of the X3T10 Policies and Procedures:

0) Adoption by a majority of the membership with immediate effectivity (but

not implemented retroactively) and subject to amendment or cancellation if not

ratified by X3.

1) Substantive change - majority of the membership with immediate effectivity

(unless otherwise specified) and subject to subsequent ratification by X3.

2) Correction of an editorial error - no vote required

3) Inclusion of better wording - simple majority

4) Discontinuance of the Policy and/or Procedure for any reason except

disapproval by X3 - majority of the membership with immediate effectivity.

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\350.MSG

### Org Date:    09-02-94 12:48:56 PM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Action Item - Policy and Procedure for Timely Publication

### Attachments: none

From ata-owner  Fri Sep  2 17:51:43 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA24018; Fri, 2 Sep 94 17:51:43 PDT

Received: from dub-img-2.compuserve.com by dt.wdc.com (4.1/SMI-4.1)

id AA24011; Fri, 2 Sep 94 17:51:40 PDT

Received: from localhost by dub-img-2.compuserve.com (8.6.4/5.940406sam)

id UAA12950; Fri, 2 Sep 1994 20:50:39 -0400

Date: 02 Sep 94 20:46:26 EDT

From: Pat Mercer - SyQuest Technology <74754.1370@compuserve.com>

To: <ata@dt.wdc.com>

Subject: ATA-2 rev. 2g field reference error.

Message-Id: <940903004625_74754.1370_FHU105-1@CompuServe.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

To:      Steve Finch

-----------------------------------------

Section 8.8    EXECUTE DEVICE DIAGNOSTIC contains the following:

>>

The diagnostic code written to the Error register is a unique 8-bit code

as shown in table 11, and not as the single bit flags defined in 7.2.9.

<<

I believe the reference to 7.2.9 is ASCII rather than a field, and

should be a field referencing section 6.2.8 ERROR REGISTER.

Pat

From ata-owner  Fri Sep  2 20:11:29 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA26889; Fri, 2 Sep 94 20:11:29 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA26882; Fri, 2 Sep 94 20:11:26 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA274711883; Fri, 2 Sep 1994 20:11:24 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA047121686; Fri, 2 Sep 1994 20:08:07 -0700

Message-Id: <n1433585709.48284@qm_smtpgw.qntm.com>

Date: 2 Sep 1994 20:21:14 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: ATA-2 rev 2g

To: "ATA" <ATA@dt.wdc.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  8:07 PM

OFFICE MEMO         ATA-2 rev 2g                          Date:  9/2/94

I just downloaded it to my Mac and tried to import it into

Microsoft Word for the Mac (rev 5.1a) but a lot of the table

type information was totally screwed up.  Anyone else have a

problem?

Jim

From ata-owner  Fri Sep  2 20:21:30 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27081; Fri, 2 Sep 94 20:21:30 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA27074; Fri, 2 Sep 94 20:21:28 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA275132485; Fri, 2 Sep 1994 20:21:26 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA053032288; Fri, 2 Sep 1994 20:18:09 -0700

Message-Id: <n1433585107.84470@qm_smtpgw.qntm.com>

Date: 2 Sep 1994 20:28:36 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: ATA Command Queuing

To: "ATA" <ATA@dt.wdc.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Subject:                              Time:  8:10 PM

OFFICE MEMO         ATA Command Queuing                   Date:  9/2/94

In the search for unuded registers for ATA command queuing, I

was told that some ATA tape drives are using address 03F3h.

This is good input, although I would like some confirmation (and

if this is the case, why has not anyone asked to reserve that byte

in the ATA document?).  In the control block range, I see the following

usage:

03F0    unused

03F1    unused

03F2    floppy

03F3    tape(?)

03F4    floppy

03F5    floppy

03F6    hard disk (bits 7-3 reserved)

03F7    floppy

Note that if worse came to worse we have bits 7-3 of an already

allocated register, which gives us 5 bits or 32 tags.

Jim

From ata-owner  Tue Sep  6 07:07:40 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA25984; Tue, 6 Sep 94 07:07:40 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA25616; Tue, 6 Sep 94 06:52:37 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id GAA01205; Tue, 6 Sep 1994 06:45:24 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Tue, 06 Sep 1994 06:35:45 EDT

Date:         Tue Sep 06 06:35:45 1994

From: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

Message-Id: <2e6c4602.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      Re: Forwarding of Comments I

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Based on the addressing on this I conluded Jim did not send it to the ATA

reflector. So I am forwarding thhe following to the ATA reflector.

To: gene milligan

cc:

From: jmcgrath @ qntm.com @ INTERNET

Date: 09/02/94 09:25:15 PM

Subject: Re: Forwarding of Comments I

Message-Id: <n1433588673.67738@qm_smtpgw.qntm.com>

Date: 2 Sep 1994 19:32:32 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: Forwarding of Comments I

To: "Gene Milligan" <Gene_Milligan@notes.seagate.com>,

"SCSI" <SCSI@WICHITAKS.NCR.COM>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Reply to:   RE>Forwarding of Comments I received on 2F.

>  2)  First, a definition of MB should be made, as most use 2^20, but

>  some use 10^6

>  (even DOS is not consistent).

I agree that ATA-2 should define such terms as megabyte, but would suggest

using

10^6.  I believe this is consistent with what we decided in SCSI (where mega

means

1,000,000 times) and is consistent with all drive capacity specifications I

am

aware of (no one advertizes their drive using 2^20 simply because it creates

a lower, and thus less marketable, number).

> 4) 6.2.13 -CORR   I know that I (I is the originator not GEM) mentioned

this

> once before, but any drive

> formatted with 63 sector/track (which most over 400 MB are) reporting this

> status will cause an fallacious error (possibly uncorrectable) due to a

> long-standing bug in DOS. This status bit should be made vendor specific,

like

> IDX. It has outlived its usefulness.

We went over CORR, but think we ended up leaving it in.  I do not rember your

point

being raised though.

Jim

### Message trace information:

### IGATE1 Version 8, IGATE2 Version 9

### MsgFileName: m:\mgate\inbound\19240000.MIN

### From:        "Jim McGrath" <jmcgrath@qntm.com>

### To:          gene_milligan@notes.seagate.com

--

Gene Milligan -- Gene_Milligan@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\566.MSG

### Org Date:    09-06-94 06:33:39 AM

### From:        Gene Milligan@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     Re: Forwarding of Comments I

### Attachments: none

From ata-owner  Tue Sep  6 14:54:35 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06965; Tue, 6 Sep 94 14:54:35 PDT

Received: from cirrus.com by dt.wdc.com (4.1/SMI-4.1)

id AA06958; Tue, 6 Sep 94 14:54:26 PDT

Received: from sunstorm.corp.cirrus.com (sunstorm1) by cirrus.com with SMTP id AA24758

(5.65c/IDA-1.4.4 for <ATA@dt.wdc.com>); Tue, 6 Sep 1994 14:53:34 -0700

Received: from nova.corp.cirrus.com (nova2.corp.cirrus.com) by sunstorm.corp.cirrus.com with SMTP id AA22934

(5.65c/IDA-1.4.4 for <ATA%dt.wdc.com@sunstorm.corp.cirrus.com>); Tue, 6 Sep 1994 14:53:31 -0700

X-Nvlenv-01Date-Transferred:  6-Sep-1994 14:52:52 -0700; at nova.cirrus

X-Nvlenv-01Date-Posted: 06-Sep-1994 14:57:20 -0400; at fs2.cirrus

Priority: Urgent

X-Nvl-Content-Type: MULTIPART; 1-S;  $----Novell--Attachment----$

Date: 06 Sep 94 14:56:00 PDT

From: chen@corp.cirrus.com

To: ATA@dt.wdc.com ("ATA")

Subject: ATA Annex A and B

Message-Id: <B06D672E016E2276@-SMF->

Reply-To: chen@corp.cirrus.com

References: <B16D672E026E2276@-SMF->

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Form: Memo

Text: (36 lines follow)

I have reviewed in details on the annex A, Diag and reset consideration from

a device

hardware standpoint (informative).  The review includes new items from Hale.

It is my

believe that this section is better be in the main text rather then in the

annex,

described as "informative."  The reasons are as followed:

1. To avoid any conflict in the main text and annex.  There were major

differences/conflicts

in ATA-1 in this area.  Pull them together can minimize the possibility of

error.

2. Annex A describes major portion of the ATA protocol, including reset

sequence and master

slave handshake.  It should not be treated as "informative" as it stated.

This protocol need to

be in the main text.

3. The annex and Hale's modified version are in good shape as to its

completeness.  I feel

comfortable to pull it in with a few clarification.

I also read annex B, diag and reset considerations from device firmware

standpoint, and feel

that there are not "necessary."  There are many duplications with annex A.

With proper

editing, this annex can be substituted by annex A.  This is only my feeling.

I would like to

know how you people feel.  Any thought?

--Joseph Chen, Cirrus Logic, Inc.

Use Proportional Font: true

Attachment Count: 0

--$----Novell--Attachment----$

X-NVL-Content-Type: UNKNOWN

X-NVL-Content-Typename: UNKNOWN

X-NVL-Content-Charset: X-IBM-437

X-NVL-Content-Filename: ATTRIBS.BND

X-NVL-Content-Transfer-Encoding: X-UUENCODE

begin 777 ATTRIBS.BND

Y0F5Y;VYD(%!A8VME9"!!='1R:6)U=&5S`([]""\M"@``````0F5Y;VYD(%!R;W!R:65T87)Y($1A

Y=&$:`````!$```````0`!``S`0``````````````````````````5&5X=+$$"@H`````````````

Y(0$#`+$$&@$!``$````9``$``0`!`+$$````````8/\```````"\`@``````````35,@4V%N<R!3

Y97)I9@```````````````````````````(```0`!``$``0`"`%@``0!9`*\``0"P``(!`0`#`3P!

Y`0`]`3T!`0`^`9H!`0";`>P!`0#M`>T!`0#N`4D"`0!*`JH"`0"K`K\"`0#``L`"`0#!`AP#`0`=

Y`U`#`0!1`U$#`0!2`ZX#`0"O`P8$`0`'!&0$`0!E!(P$`0"-!(T$`0".!(X$`0"/!+`$`0"Q!+$$

Y`0"R!+($````````````````9``!:`$!T`(!.`0!H`4!"`<!<`@!V`D!0`L!J`P!$`X!>`\!X!`!

(2!(!L!-4````

end

--$----Novell--Attachment----$--

From ata-owner  Wed Sep  7 05:46:52 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23434; Wed, 7 Sep 94 05:46:52 PDT

Received: from VNET.IBM.COM by dt.wdc.com (4.1/SMI-4.1)

id AA23427; Wed, 7 Sep 94 05:46:47 PDT

Message-Id: <9409071246.AA23427@dt.wdc.com>

Received: from BCRVMPC2 by VNET.IBM.COM (IBM VM SMTP V2R2) with BSMTP id 9394;

Wed, 07 Sep 94 08:46:35 EDT

Date: Wed, 7 Sep 94 08:46:17 EDT

From: apione@VNET.IBM.COM

To: ATA@dt.wdc.com

Subject: Joe Chen's note about ATA document

Sender: ata-owner@dt.wdc.com

Precedence: bulk

ATA committee:

I agree with Joe that merging the Annex A with the main body of the

ATA document would lead to greater clarity. There is a lot of stuff in the

Annex that is required by ATA, but isn't even touched on in the main document.

Regards, Tony

From ata-owner  Wed Sep  7 08:01:39 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA25888; Wed, 7 Sep 94 08:01:39 PDT

Received: from netcomsv.netcom.com (uucp3.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA25881; Wed, 7 Sep 94 08:01:25 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id IAA18656; Wed, 7 Sep 1994 08:01:48 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qiO9g-000gJUC; Wed, 7 Sep 94 08:39 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 07 Sep 1994 08:41:46 EDT

Date:      Wed, 07 Sep 1994 08:41:45 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e6db50a.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   Annex A and B -- I agree

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I agree with the folks that say Annex A should be pulled into the

main body of the document -- probably into the "Protocol"

section.

I also agree that Annex B is a little redundant and I would not

oppose deleting it.

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Sep  7 09:37:57 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA27957; Wed, 7 Sep 94 09:37:57 PDT

Received: from dub-img-1.compuserve.com by dt.wdc.com (4.1/SMI-4.1)

id AA27950; Wed, 7 Sep 94 09:37:52 PDT

Received: from localhost by dub-img-1.compuserve.com (8.6.4/5.940406sam)

id MAA14946; Wed, 7 Sep 1994 12:37:51 -0400

Date: 07 Sep 94 12:34:22 EDT

From: Thomas Newman <71246.1573@compuserve.com>

To: ATA Reflector <ata@dt.wdc.com>

Subject: Annex A & B

Message-Id: <940907163422_71246.1573_CHL58-2@CompuServe.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Here's some background info on the Annex A/B discussion.

ATA's Annex A and B both describe the Master/Slave protocol.

At the time they were developed, both were summitted to the

working group. Instead of picking one for inclusion in the draft,

it was decided that they both be included because they address

the subject from two different directions. Annex A provided the

information in a general format. Annex B was aimed at the

programmer who would have to implement the protocol in his/her

device.

I wrote the Annex A portion with the intent of showing the exact

protocol required. Because of this it contains information that's

implied in the main body of the document. I would vote to bring

this information back into the document either by moving the

Annex into the main document or by adding more details to the

document

The problem I have with standards (and a lot of programming

manuals) is the lack of examples (I understand that examples are

not part of a standard). Annex A and B are really the

"example" portion of ATA standard for Master/Slave operation.

-----------------------------------------------------

Tom Newman, Mission Peak Designs

From ata-owner  Wed Sep  7 10:40:25 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29464; Wed, 7 Sep 94 10:40:25 PDT

Received: from arl-img-2.compuserve.com by dt.wdc.com (4.1/SMI-4.1)

id AA29457; Wed, 7 Sep 94 10:40:20 PDT

Received: from localhost by arl-img-2.compuserve.com (8.6.4/5.940406sam)

id NAA04686; Wed, 7 Sep 1994 13:40:18 -0400

Date: 07 Sep 94 13:39:04 EDT

From: <JEFFE@fdc.mhs.compuserve.com>

To: ATA Reflector <ata@dt.wdc.com>

Subject: ATA-2 Spec, comment

Message-Id: <940907173904_555063.0_EHF23-2@CompuServe.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

- - Mail - -

September  7, 1994   10:25am   MAIL IS -

FROM: Jeff Epstein                    Viewed Ntfy Urgent

TO: ATA Reflector

SUBJECT: ATA-2 Spec, comment

To: ATA Committee members

Subject: ATA-2 Spec, Annex A

I feel that the spec with its's separate Annex A is in the correct form as

is.  It should not be merged into the main body of the spec.  The reason I

feel this way is because the main body is a 'spec' and the annexes are like

application notes.

Think of the ATA-2 spec in the same way we all treat an IC chip spec. The

chip manufacturer tells you how it works in the spec and provides app notes

on using it.  The ATA-2 annexes describe proper/preferred use of the

interface.  They talk about host and drive interaction.

Thanks,

Jeff Epstein

Future Domain Corp.

From ata-owner  Wed Sep  7 13:08:38 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02610; Wed, 7 Sep 94 13:08:38 PDT

Received: from netcomsv.netcom.com (uucp6.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA02597; Wed, 7 Sep 94 13:07:00 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id MAA08123; Wed, 7 Sep 1994 12:58:40 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qiSvE-000gJTC; Wed, 7 Sep 94 13:44 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 07 Sep 1994 10:03:34 EDT

Date:      Wed, 07 Sep 1994 10:03:34 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e6dc836.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   BSY after Command reg is written

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The ATA and ATA-2 allow a device up to 400ns to set BSY after the

command register is written by the host.  This is probably some

hold-over from the MFM controller days.  I wonder if it isn't

time to change this?  Here is why...

Recently I was asked by one of the BIOS developers about this

because on really fast systems they have had to put a delay in

the BIOS to allow the device time to set BSY after the BIOS

writes the Command register.  Without this delay the the BIOS was

seeing no change (no BSY, no DRQ) in the Status register for some

period of time (probably less than 400ns) after writing the

Command register.

I'm not a hardware person, but from a software standpoint, I want

to see BSY (or DRQ) set in the Status register no matter how

short the delay between the time my software writes the Command

register and then reads the Status register.  I also realize that

even hardware needs time to react to events.

Do you have any idea how hard it is to time sub-microsecond

things like this in most systems?  Generally you do it by coding

some number of instructions that really don't do anything (like

shifting a register) because most systems don't include timer

hardware with anything better than millisecond (microsecond if

you are real lucky) resolution.  So you might code a delay that

works on a system today only to find it doesn't work on the same

make/model system tomorrow because of some subtle hardware

change.

I wonder if we could change ATA-2 to say something like:

When the host writes the Command register, the device shall

set the BSY bit in the Status register such that if the host

accesses the Status register using the fastest PIO timing

supported by the device, the BSY bit shall be set in the

Status register.

and eliminate the old 400ns value.  Note that this same 400ns

value is used in the reset descriptions but I'm not that

concerned about resets -- they are a "special" case anyway.

I have lots of ATA test software (much of it I have written) and

I have never seen the problem the BIOS people see on any newer

drives.  In my opinion, if everyone's hardware shipping today can

meet the above requirement, then lets change ATA-2 now.

So what do all you hardware designers say?

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Sep  7 13:51:00 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA03599; Wed, 7 Sep 94 13:51:00 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA03592; Wed, 7 Sep 94 13:50:55 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qiTxC-0002TuC; Wed, 7 Sep 94 16:50 EDT

Received: by ptlnor3.ptltd.com; Wed, 7 Sep 94 16:50:59 EDT

Date: Wed, 7 Sep 94 19:50:54 PDT

Message-Id: <E706+iSWPia@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: ATA@dt.wdc.com

Subject: re:BSY after Command reg is written

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I have to agree with Hale on this one.  I think 400ns is way too long.  I

had to create my sub-microsecond delays by doing dummy OUT instructions.

I ran into my problem obviously because a drive was waiting the full 400ns

to set the status.  I have been told by several drive manufacturers that

statuses should now be responding in the 20-30ns range.  As a BIOS company

we will have to continue to support the 400ns number for quite some time

to maintain compatibility, but it would be a welcome push for the future

to lower this number to, say, 20ns?

Curtis E. Stevens

----------------------[Reply - Original Message]----------------------

Sent by:"Hale Landis" <landis@sugs.tware.com>

The ATA and ATA-2 allow a device up to 400ns to set BSY after the

command register is written by the host.  This is probably some

hold-over from the MFM controller days.  I wonder if it isn't

time to change this?  Here is why...

Recently I was asked by one of the BIOS developers about this

because on really fast systems they have had to put a delay in

the BIOS to allow the device time to set BSY after the BIOS

writes the Command register.  Without this delay the the BIOS was

seeing no change (no BSY, no DRQ) in the Status register for some

period of time (probably less than 400ns) after writing the

Command register.

I'm not a hardware person, but from a software standpoint, I want

to see BSY (or DRQ) set in the Status register no matter how

short the delay between the time my software writes the Command

register and then reads the Status register.  I also realize that

even hardware needs time to react to events.

Do you have any idea how hard it is to time sub-microsecond

things like this in most systems?  Generally you do it by coding

some number of instructions that really don't do anything (like

shifting a register) because most systems don't include timer

hardware with anything better than millisecond (microsecond if

you are real lucky) resolution.  So you might code a delay that

works on a system today only to find it doesn't work on the same

make/model system tomorrow because of some subtle hardware

change.

I wonder if we could change ATA-2 to say something like:

When the host writes the Command register, the device shall

set the BSY bit in the Status register such that if the host

accesses the Status register using the fastest PIO timing

supported by the device, the BSY bit shall be set in the

Status register.

and eliminate the old 400ns value.  Note that this same 400ns

value is used in the reset descriptions but I'm not that

concerned about resets -- they are a "special" case anyway.

I have lots of ATA test software (much of it I have written) and

I have never seen the problem the BIOS people see on any newer

drives.  In my opinion, if everyone's hardware shipping today can

meet the above requirement, then lets change ATA-2 now.

So what do all you hardware designers say?

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

=====================================================================

From ata-owner  Wed Sep  7 15:04:19 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA05305; Wed, 7 Sep 94 15:04:19 PDT

Received: from netcomsv.netcom.com (uucp6.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA05298; Wed, 7 Sep 94 15:04:06 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id OAA25647; Wed, 7 Sep 1994 14:58:20 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qiUsz-000gJTC; Wed, 7 Sep 94 15:50 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Wed, 07 Sep 1994 15:09:03 EDT

Date:      Wed, 07 Sep 1994 15:09:03 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e6e0fcf.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   ATA Rev 2f/2g Annex E comment

Sender: ata-owner@dt.wdc.com

Precedence: bulk

As has been pointed out by Gene Milligan and perhaps others, the

reference to 512MB in Annex E is *incorrect* -- the *correct*

value is 528MB.  The correct values are in the final, also the

current, SFF-8019 document, version 2.0, dated 23 May 94.  It

would appear that Annex E was created just prior to the time when

the final SFF-8019 was published.

Just to be sure Annex E is correct, I will review it and also

compare it to SFF-8019 rev 2.0 in infinite detail before the WG

meeting at the end of this month.

As for defining what a megabyte is, I guess we should do that,

but I've never felt that should really be necessary!  A mega is

1,000,000 of something!  If we define what megabyte is, does that

also mean we must also define what a megahertz is?  Or a

megabuck?  And then what about kilo?  Must we also define

kilobyte as 1,000 bytes?  And kilohertz too?  If I ever find that

memory vendor that started all this mega is 2^20 crap, I'm not

sure what I'll do!  However, you should all write to the

publisher of your favorite dictionary and tell them that a mega

is 1,000,000 and that if they want to define mega as something

else, like megabyte as 2^20, they should show it as "slang" and

not an accepted meaning for the prefix mega!  Get those letters

in the mail today!

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Wed Sep  7 15:23:05 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA05644; Wed, 7 Sep 94 15:23:05 PDT

Received: from dub-img-1.compuserve.com by dt.wdc.com (4.1/SMI-4.1)

id AA05616; Wed, 7 Sep 94 15:23:00 PDT

Received: from localhost by dub-img-1.compuserve.com (8.6.4/5.940406sam)

id SAA07435; Wed, 7 Sep 1994 18:22:59 -0400

Date: 07 Sep 94 18:07:42 EDT

From: <JEFFE@fdc.mhs.compuserve.com>

To: ATA Reflector <ata@dt.wdc.com>

Subject: BSY after Command Reg Written

Message-Id: <940907220742_555063.0_EHF68-2@CompuServe.COM>

Sender: ata-owner@dt.wdc.com

Precedence: bulk

- - Mail - -

September  7, 1994    3:00pm   MAIL IS -

FROM: Jeff Epstein                    Viewed Ntfy Urgent

TO: ATA Reflector

SUBJECT: BSY after Command Reg Written

TO: ATA Committee Members

SUBJECT: BSY Bit Set after Command Reg Written

I agree with Hale's proposal.  Lets go for it.

Jeff Epstein, Future Domain

From ata-owner  Wed Sep  7 15:52:55 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06448; Wed, 7 Sep 94 15:52:55 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA06438; Wed, 7 Sep 94 15:52:47 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qiVr5-0002TwC; Wed, 7 Sep 94 18:52 EDT

Received: by ptlnor3.ptltd.com; Wed, 7 Sep 94 18:53:00 EDT

Date: Wed, 7 Sep 94 21:52:54 PDT

Message-Id: <E706+4FYPia@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: ATA@dt.wdc.com

Subject: re:BSY after Command Reg Written

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Jeff

I would like to nail this down a little bit more tightly than

Hales purposal.  Hales purposal gives a Standard Mode drive 600ns to

respond!  There is no reason that the status can't be set simply after a

few gate delays.  I really would like to see the wording changed from

400ns to 40ns, or maybe even 20ns.  180ns for mode 3, or even 120ns for

mode 4 is still a very long time.  There is no excuse for a status to take

that long to be set.

Curtis E. Stevens

----------------------[Reply - Original Message]----------------------

Sent by:<JEFFE@fdc.mhs.compuserve.com>

- - Mail - -

September  7, 1994    3:00pm   MAIL IS -

FROM: Jeff Epstein                    Viewed Ntfy Urgent

TO: ATA Reflector

SUBJECT: BSY after Command Reg Written

TO: ATA Committee Members

SUBJECT: BSY Bit Set after Command Reg Written

I agree with Hale's proposal.  Lets go for it.

Jeff Epstein, Future Domain

=====================================================================

Sent by:"Hale Landis" <landis@sugs.tware.com>

The ATA and ATA-2 allow a device up to 400ns to set BSY after the

command register is written by the host.  This is probably some

hold-over from the MFM controller days.  I wonder if it isn't

time to change this?  Here is why...

Recently I was asked by one of the BIOS developers about this

because on really fast systems they have had to put a delay in

the BIOS to allow the device time to set BSY after the BIOS

writes the Command register.  Without this delay the the BIOS was

seeing no change (no BSY, no DRQ) in the Status register for some

period of time (probably less than 400ns) after writing the

Command register.

I'm not a hardware person, but from a software standpoint, I want

to see BSY (or DRQ) set in the Status register no matter how

short the delay between the time my software writes the Command

register and then reads the Status register.  I also realize that

even hardware needs time to react to events.

Do you have any idea how hard it is to time sub-microsecond

things like this in most systems?  Generally you do it by coding

some number of instructions that really don't do anything (like

shifting a register) because most systems don't include timer

hardware with anything better than millisecond (microsecond if

you are real lucky) resolution.  So you might code a delay that

works on a system today only to find it doesn't work on the same

make/model system tomorrow because of some subtle hardware

change.

I wonder if we could change ATA-2 to say something like:

When the host writes the Command register, the device shall

set the BSY bit in the Status register such that if the host

accesses the Status register using the fastest PIO timing

supported by the device, the BSY bit shall be set in the

Status register.

and eliminate the old 400ns value.  Note that this same 400ns

value is used in the reset descriptions but I'm not that

concerned about resets -- they are a "special" case anyway.

I have lots of ATA test software (much of it I have written) and

I have never seen the problem the BIOS people see on any newer

drives.  In my opinion, if everyone's hardware shipping today can

meet the above requirement, then lets change ATA-2 now.

So what do all you hardware designers say?

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

=====================================================================

From ata-owner  Thu Sep  8 15:40:05 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04752; Thu, 8 Sep 94 15:40:05 PDT

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA04722; Thu, 8 Sep 94 15:40:01 PDT

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA14931 for ata@dt.wdc.com; Thu, 8 Sep 94 15:39:59 -0700

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA07260 for ata@dt.wdc.com; Thu, 8 Sep 94 15:39:57 -0700

Received: from bach.nsc.com by berlioz (4.1/SMI-4.1)

id AA24910; Thu, 8 Sep 94 15:30:56 PDT

Date: Thu, 8 Sep 94 15:30:56 PDT

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9409082230.AA24910@berlioz>

To: ata@dt.wdc.com, landis@sugs.tware.com

Subject: Re:  BSY after Command reg is written

Sender: ata-owner@dt.wdc.com

Precedence: bulk

What exactly do we want to achieve? Better performance by removing/reducing the

BIOS delay; or elimination of any artificial BIOS delay today and forever. The

latter objective is much more desirable, but I don't think that it is

achievable. In addition, the BIOS software is going to have to keep supporting

the 400ns limit for quite a while for compatibility reasons.

The reason for my pessimism is that I am not convinced that all hardware

implementations in existence today can support the suggested 20-40 ns max

delay time. The strobe signals are often time synchronized to the drive's

internal buffer clock signal and there may be some silicon out there that is

using the synchronized strobes to write the non-data task-file registers. If

this is the case, then I can easily see such a silicon taking 100-200 nsec

max in setting BSY bit with today's technology and clock rates. This would

mean that we can improve the performance of the BIOS software by 200 ns - that's

about it. Is that worth it?

These are just some quick thoughts - just to present a "hardware guy's"

mindset. (Since you asked, Hale).

-Robbie Shergill

National Semiconductor

From ata-owner  Thu Sep  8 16:36:46 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06200; Thu, 8 Sep 94 16:36:46 PDT

Received: from gatekeeper.nsc.com by dt.wdc.com (4.1/SMI-4.1)

id AA06192; Thu, 8 Sep 94 16:36:41 PDT

Received: from nsc.nsc.com by gatekeeper.nsc.com (5.65/fma-120691) with SMTP;

id AA17177 for ata@dt.wdc.com; Thu, 8 Sep 94 16:36:39 -0700

Received: from berlioz.nsc.com by nsc.nsc.com (5.65/1.34) with SMTP

id AA09450 for ata@dt.wdc.com; Thu, 8 Sep 94 16:36:38 -0700

Received: from bach.nsc.com by berlioz (4.1/SMI-4.1)

id AA26076; Thu, 8 Sep 94 16:24:36 PDT

Date: Thu, 8 Sep 94 16:24:36 PDT

From: rss@berlioz.nsc.com (R. S. Shergill)

Message-Id: <9409082324.AA26076@berlioz>

To: ata@dt.wdc.com, scsi@wichitaks.ncr.com

Subject: ATA Ad-hoc Working Group Minutes - 08/94

Cc: sff_reflector@sun.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Accredited Standards Committee*

X3, Information Processing Systems

Doc. No.   X3T10/94-____R0

Date:            Sept 8, 1994

Project:

Ref. Doc.:

Reply to:  S. Finch

To:       Membership of X3T10

From:     Shergill/Finch

Subject:  Minutes of X3T10 ATA Ad-hoc Working Group Aug 31-Sep 1, 1994

Agenda

1. Opening Remarks

2. Attendance and Membership, Introductions

3. Approval of Agenda

4. Document Distribution

5. Old Business

5.1  Termination Issues (Hanan)

5.2  ATA-2 Technical Review

6. New Business

6.1  Multi-threading (Hanan)

7. Meeting Schedule

8. Adjournment

Results of Meeting

1.  Opening Remarks

Steve Finch convened the meeting at 9:00 am.  Steve thanked Ray Heineman

of Maxtor for hosting the meeting.

As is customary, the people attending introduced themselves.  A copy of

the attendance list was circulated for attendance and corrections.

It was stated that the meeting had been authorized by X3T10 and would be

conducted under the X3 rules.  Ad hoc meetings take no final actions,

but prepare recommendations for approval by the X3T10 task group.  The

voting rules for the meeting are those of the parent committee, X3T10.

These rules are:  one vote per company; and any participating company

member may vote.

The minutes of this meeting will be posted to the X3T10 BBS and the ATA

Reflector and will be included in the next X3T10 committee mailing.

2.  Attendance and Membership, Introductions

Attendance at working group meetings does not count toward minimum

attendance requirements for X3T10 membership.  Working group meetings

are open to any person or company to attend and to express their opinion

on the subjects being discussed.

The following people attended the meeting.

Name                    Company                Email Address

Norm Harris            Adaptec

Richard Kalish       Adaptec                rkalish@aol.com

Lawrence J. Lamers   Adaptec                   ljlamers@aol.com

Joseph Chen              Cirrus Logic           chen@cirrus.com

Nicos Syrimis     Cirrus Logic

Lane Lee               Conner Peripherals

Jeff Epstein     Future Domain

Mark Raymond     Future Domain

Dan Colegrove    IBM                colegrove@vnet.ibm.com

Tony Pione                 IBM

Ray Heineman     Maxtor                     ray_heineman@maxtor.com

Ron Roberts               Maxtor

Tom Newman             Mission Peak Designs

Robert Griffith          National Semiconductor

Robbie Shergill          National Semiconductor  rss@berlioz.nsc.com

Curt Allred              Nexcom Tech.

Karl Schuh             Nexcom Tech.

Peter Brown            Oak Technology

Phil Verinski    Oak Technology

Charles Yang     Panasonic

Mike Carpenter           Qlogic                      M.carpenter@qlc.com

Farbod Falakfarsa      Quantum

James McGrath    Quantum

Don Powlison    Quantum

John Masiewicz           Seagate Technology      masiewicz@notes.seagate.com

Hale Landis

Stephen Finch    Silicon Systems         5723283@mcimail.com

Yas Hashimoto      Toshiba America

Devon Worrell    Western Digital

Tom Hanan              Western Digital           t_hanan@dt.wdc.com

(Note: Not all attendees were present for both days).

3.  Approval of Agenda

Item 5.1 was struck because Hanan did not put his proposal on the

Reflector at least two weeks prior to this meeting as was agreed to at

the last meeting. Tom was asked by Steve to bring this in as an ATA-3

item.

Item 6.1 was added for the second day, if time permits.

The agenda was approved as amended.

4.  Document Distribution

ATA-2 Working Draft, Rev 2f  - Finch, SSI.

Re: Items for ATA agenda - McGrath, Quantum.

Re: WD Command Queuing Proposal - McGrath, Quantum.

Hale Landis's Comments - Landis.

5.  Old Business

5.1  Item removed from agenda.

5.2  ATA-2 Technical Review:

The working draft revision 2f was reviewed in a section-by-section

manner.  Rest of this section is intended to be a complete list of

changes agreed upon. Please refer to the Working Draft revision 2g for a

complete and accurate description of the changes.

Revision Page will be deleted when the document is finalized (was a

comment by Gene Milligan)

Throughout the document, Steve will explicitly say "bit xxx" instead of

"xxx".

Throughout the document, Steve will change 'x' to 'r' for all reserved

bits.

Section 3:

Added glossary item - Command acceptance: a command is considered

accepted whenever the host writes to the cmd reg. and the device

currently selected has its BSY bit set to 0. An exception exists for the

execute diagnostics cmd. (see description of execute diag cmd).

Added glossary Item - Drive Selection: a drive is selected when the DEV

bit in the Drive/Head register is equal to the device number assigned to

the device by means of a dev0/dev1 jumper or switch or use of the CSEL

signal.

(moved here from section 6.1.1)

Section 3.2.5

Tom Newman said that the byte ordering appears reversed - others didn't

agree.Tom Hanan wanted a note added - this is not the order in the PC.

All agreed, Hanan wrote the note and it was added. Tom Newman was asked

to check the actual byte ordering on a disk drive.

On Day-2:  Tom Newman reported back. His results showed that the byte

ordering is reverse of what the document currently contains. Confusion

was due to the difference in byte-ordering between the actual data

transfers at the interface and the way Identify Data is represented.

Agreed to change this section to address only the byte ordering for 8

and 16-bit data transfers - not the byte-ordering related to fields in

Identify Device data. Also worked out an advisory note saying that some

systems will swap bytes. Voted on the note's inclusion: For: 13,

Against: 4. The note was accepted.

Changed "n from 2 to 512" to "n starting at 2"

Paragraph 2: changed "will" to "shall" and also stressed "on the

interface" again.

Sections 4.2 and 4.3

Editor will soften the language to say that the connector shown here is

one example of several connectors. Agreed that we will completely define

the connection schemes in ATA-3.

Section 4.5

Table 3, second line, description, changed "Rise" to "Fall".

Section 5.2.4  DASP-

Third paragraph, second sentence: deleted.

Added back the language from rev 2d concerning the requirement for the

host to current limit on the LED load. (John Masiewicz comment).

Sections 5.2.6 and 5.2.7

Added after the last sentence "The device shall not act on the data

until after it is latched".

(Tony Pione and Devon Worrell had pointed out this problem).

5.2.10 INTRQ

Joe Chen wanted to add back the option of clearing INTRQ automatically

at the end of the data block transfer - except for the last block

transfer.  Hale didn't like this option because it means that

interlocking is lost. Tom Newman pointed out that there are other ways

to interlock. Jim mcGrath pointed out that the hardware implementation

of today require a dead-time to be inserted in order to ensure

interlock. The group voted on the concept behind Joe's proposal - 3 in

favor (cirrus, conner, toshiba), 14 against. Joe was asked to bring this

issue to the plenary meeting if he wants to pursue this further.

5.2.11  IOCS16-

Fourth bullet point:

Jim McGrath: should it be "not valid" or "not asserted"? He wanted it to

be "not asserted". Others disagreed. The group agreed unanimously to

change "is not valid" to "shall not be used by the host".

Another isssue raised was whether an editor's note should be added

stating that IOCS16's switching at mode 3 or higher speeds is not

recommended. Vote - 5 for, 6 against. Not accepted.

5.2.15 second para, last sentence: changed "made by...." to "vendor

specific".

5.2.15.2  CSEL

Tom Hanan brought up the issue that some people want to use CSEL to

select the devices electronically -  so we should specify that the csel

should be used only at the time of reset. Added the sentence: "host

shall maintain CSEL at a stable level for at least 31 seconds after

the deassertion of Reset-". Also changed the terms "grounded" and "open"

to "negated" and "asserted" in the two bullets.

6.1 changed the title to Device Addressing Considerations and combined

with subsection 6.1.1.

Paragraph 3, first sentence: deleted everything after Device/Head

Register (see clause 6.2.8) because the other methods are configuration,

not selection. Robbie Shergill asked that the last sentence of para 3

should be deleted because it is not consistant with figure 1. This

opened up the issue of whether a single drive can exist as drive

1. Tom Hanan claimed that some people are implementing systems with a

slave drive only. Steve Finch asserted that this cannot be done without

violating the spec. This issue was not resolved and the sentence in

question was left intact.

Fourth paragraph was moved to glossary.

6.2 table 7: Issue was raised whether "not used" be changed to

"reserved"? After a lengthy discussion the group decided not to do so.

The Device Address register entry was replaced with a note saying that

only bit 7 has to be kept high impedance and the rest of the bits are

obseleted.

6.2.3/6.2.4/6.2.8/6.2.12

mcgrath asked that Cyl/Head/Sector Number updating requirement be

removed because the auto-increment circuitry is costly and the updating

serves no real purpose. Hale and Hanan agreed with this. Changed the

wording to state that the updating is only required for media access

cmds that end in an error. Added an editor's note stating that all

commands ended with the CSH updated prior to this spec, but this spec

doesn't require that.

6.2.5  Data Register

Changed wording as suggested by Hale in his document.

6.2.6  Device Control Register

Made bits 7-3 reserved. Voted on making bit 3 reserved: 12 in favor, 1

opposed.

6.2.7 Device Address Register

Eliminated this section. Added a note to table 7 instead.

6.2.8 Device/Head Register

Removed second sentence.

Made bits 7 and 5 reserved.

Removed the "binary encoded" term.

Changed the name of the L bit to sector address mode select

Changed the name of the DEV bit to device address

6.2.9 Error register

BBK bit: Ray Heineman wanted to eliminate this bit because Format Track

has been made vendor specific and it is the only command that uses this

bit. Hale wanted to retain it because he may need to use it for commands

other than format track.

Voted: make it reserved? For: 10, opposed: 4.

MC and MCR bits: Hale wanted to remove the description only and say that

these are for removable media and the implementation is vendor-specific.

Joe Chen disagreed - these bits have been in ATA-1 and some people are

using them.

Voted - in favor of Hale's alternate wording: 12, against:1.

6.2.11 Sector Count register

Accepted Hale's alternate description which says essentially that just

because SCNT=0 doesn't mean that the command completed successfully.

Deleted the reference to Format Track cmd.

6.2.12 Sector Number register

Accepted Hale's alternate description.

6.2.13 Status register

Joe Chen proposed making bits 2, 4 and 5 vendor-specific.

CORR bit: McGrath wanted to make it "reserved" altogether. Colegrove

wanted to retain it because his users monitor this bit to keep track of

media degradation. Shergill asserted that it is the definition of

"correcatble error" that varies from vendor to vendor so just make this

vendor specific. This was agreed to.

DWF bit: Hale doesn't mind making this vendor specific, but wants to

retain the portion that this bit should stay set 'till the status is

read. Voted to accept Hale's wording. 13 for, 3 against.

DSC bit:  Agreed to accept Hale's recommendation and make it equal to

DRDY bit because BIOSes generally look for this state in the Status

register. McGrath asked for, and all agreed, that a note should be added

that in ATA-3 this bit will be reclaimed. Additionally, this bit's name

was changed to Bit4.

Next, Hale's introductory statements for this register were amended and

accepted.

Gene Milligan had commented that the clearing of error bits is not

clearly defined and this causes the hosts to never know what to believe.

Hanan agreed and suggested that the ERR bit and the Error register

should be required to hold their state until the next command. All

agreed on the following language:

"Once this bit is set, the device shall not clear this bit or change

the contents of the command block registers, except the data reg and the

vendor specific bits in the Status register, until a new command is

accepted by the device, the SRST bit is set to one, or RESET- is

asserted."

Another of Gene's comments was also accepted - it should be noted

that the BSY=1 state can be very short so the host sw should not rely on

detecting this state.

The group next considered Hale's proposals pertaining to the Status

register. Hale recommends that DRQ, ERR and CORR bits be changed only

while BSY=1. chen suggested that just BSY and DRQ's relationship should

be covered here and ERR and CORR bits should not be covered. Finch

asserted that the better way to specify this is that there are only

certain registers or bits that the device can change while BSY=0. These

bits are: DRDY, DWF, bit 4, CORR, IDX in the status register and the

Data register. This change was accepted.

A lengthy discussion took place on the issue of BSY's state during

data transfers. In the end, the group agreed to Hale's five conditions

for when BSY must be set but changed the language. Sum total of it all

is that from 400ns max after command acceptance and until command

completion, if DRQ is not set then BSY must be. The list has been made a

set of requiremnts for the events when the device must set BSY. Robbie

Shergill raised the point that the device should also not be allowed to

set BSY at any other times - which was the original intention of this

list in ATA-1. This was agreed to and Steve will add a sentence at the

end.

DRDY bit: accepted Hale's re-wording with few changes: deleted the

requirement that it stays stable 'till status is read. A discussion took

place concerning whether DRDY=1 means that the device is ready to ACCEPT

a command - given our new definition of command acceptance. This issue

was not resolved.

ERR bit: Dan Colegrove rewrote the paragraph in response to Gene's

comment of this morning. It specified the list of bits and regs that

must not change once ERR bit is set and until the next command.

6. New Business

No new business items were addressed as the technical review was

not completed within the allocated time.

7. Meeting Schedule

9/15: Houston, TX.  Host: Compaq.

9/27-28: Milpitas, CA. Host: Quantum  (was previously scheduled

to be held at Longmont).

11/10: Palm Springs, CA.  Host: Western Digital

8. Adjournment

The meeting adjourned at approximately 4:00 PM on Sept 1, 1994.

------------------------------------------------------------------------

*Operating under the procedures of The American National Standards

Institute.

X3 Secretariat, Computer and Business Equipment Manufacturers

Association (CBEMA)

1250 Eye Street NW, Suite 200, Washington, DC 20005-3922

Telephone: 202-737-8888 (Press 1 twice)

FAX: 202-638-4922 or 202-628-2829

------------------------------------------------------------------------

From ata-owner  Fri Sep  9 09:46:55 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA26404; Fri, 9 Sep 94 09:46:55 PDT

Received: from netcomsv.netcom.com (uucp6.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA26142; Fri, 9 Sep 94 09:37:50 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id JAA24648; Fri, 9 Sep 1994 09:28:21 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qj8QP-000gJTC; Fri, 9 Sep 94 10:03 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Fri, 09 Sep 1994 10:05:56 EDT

Date:      Fri, 09 Sep 1994 10:05:54 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e706bc4.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "Steve Timm"         <stevetim@microsoft.com>

Cc: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   Please clarify ATA LBA requirements

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hello Steve,

I don't know if you are the right person at MS to send this to

but I'm sure you know the person(s) with the answer(s).  Please

note that I am copying the ATA reflector too.

I'm sending this because Richard Hoehnle of Seagate tells me that

MS says that drives >528MB will not be supported unless the drive

and the system BIOS supports LBA.  If this is true, what is this

all about?

(Please note that this could all be a misinterpretation of

something someone else said but I would still like to see a

clarification from MS.)

There should be *NO* requirement for LBA support in the drive or

in the BIOS in order to support >528MB.  This is not a

requirement for SCSI host adapter BIOSes that support drives over

528MB for many years now.  Why is it a requirement for ATA drives

over 528MB?

As you know, the tradtional BIOS supports only CHS style

addressing.  In order to support LBA style addressing at the INT

13 interface, the BIOS must support the MS/IBM Extensions.  I

don't see the BIOS people doing that today.

What I do see (in some cases), is BIOS support for CHS

translation (cylinders < 1024, heads > 16 at the INT 13

interface), like the SCSI people have been doing for years, with

some BIOS support for the Phoenix Enhancement.

It would appear that if anything settles out over the next year

or two it will be this:  BIOS will support CHS translation and

not much more, unfortunately, not all implementations will be the

same.

(Even if all the above is someone misinterpretation, I would like

to make the following observations.  Please note that I don't

work for Seagate anymore so the following is just Hale Landis

speaking.)

While last year's "Big Disk" conference at Comdex, sponsored by

Seagate and Western Digital, was widely attended and lots of

ideas were put forth, the conference was at best, a failure.

Yes, the disk drive vendors did agree to the Identify Drive data

requirements (SFF-8019, now part of ATA-2), but I have seen *NO*,

repeat *NO*, agreements on the software side.  I know that

Phoenix is pushing their Enhancements specification, and I think

that is good, but I see no common agreement on this.

If we want to head-off total failure of ATA drives over 528MB in

the marketplace, someone in the software area had better step

forward soon and get some industry agreement on the software

requirements for >528MB ATA drives.  If you don't think this is a

problem, I suggest you start by reading the

comp.sys.ibm.pc.hardware.storage news group on the Internet.

A software specification/standard/agreement should spell-out in

detail which BIOS calls are required and exactly how drive

configuration data is computed and returned by those calls.  The

current software specification don't do all this.  It seems to me

that if >528MB drives are to be widely support, some of the

MS/IBM Extensions and/or Phoenix Enhancement calls must be made

*mandatory*.

I'm sure everyone in the disk drive industry would appreciate any

help you can give towards finding a solution for the >528MB ATA

software support problems.

Thanks,

Hale

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Fri Sep  9 10:57:01 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA28056; Fri, 9 Sep 94 10:57:01 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA28046; Fri, 9 Sep 94 10:56:16 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id KAA26036; Fri, 9 Sep 1994 10:38:27 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 09 Sep 1994 09:21:22 EDT

Date:         Fri Sep 09 09:21:22 1994

From: "Richard Hoehnle" <Richard_Hoehnle@notes.seagate.com>

Message-Id: <2e706153.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "Richard Hoehnle" <Richard_Hoehnle@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      10^6 Nonsense

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Both Hale and Jim have argued the case for 10^6 notation in Annex E, when, in

fact, Base 10 numbering is nowhere to be found in an ATA disk drive. Sectors

(512 bytes) are not Base 10, clusters are not Base 10, registers are not Base

10, addresses are not Base 10, software does not allocate space in Base 10,

etc. The only place Base 10 is used is in marketing hype to exploit the total

size of the drive to the end user.

Since a standards document is a tool for the engineer and seldom seen by the

end user, don't add confusion for the engineer by requiring him(her) to convert

from Base 10 to Base 2 to understand the crossover points being referenced in

the text. Megabyte=2^20 has been well understood by most engineers for 30 years

(IBM System/360, circa 1964).

Remember, only marketing has to explain to the end user why the Base 10

capacity drive just prematurely ran out of space due to BASE 2 allocation.

--

Richard Hoehnle -- Richard_Hoehnle@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\934.MSG

### Org Date:    09-09-94 08:54:00 AM

### From:        Richard Hoehnle@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     10^6 Nonsense

### Attachments: none

From ata-owner  Fri Sep  9 13:52:49 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA02033; Fri, 9 Sep 94 13:52:49 PDT

Received: from netmail2.microsoft.com by dt.wdc.com (4.1/SMI-4.1)

id AA02026; Fri, 9 Sep 94 13:52:47 PDT

Received:  by netmail2.microsoft.com (5.65/25-eef)

id AA11805; Fri, 9 Sep 94 13:53:42 -0700

Message-Id: <9409092053.AA11805@netmail2.microsoft.com>

Received: by netmail2 using fxenixd 1.0 Fri, 09 Sep 94 13:53:42 PDT

X-Msmail-Message-Id:  AB834A0A

X-Msmail-Conversation-Id:  AB834A0A

From: Steve Timm (SYS) <stevetim@microsoft.com>

To: landis@sugs.tware.com

Date: Tue,  6 Sep 94 13:55:31 PDT

Subject: RE: Please clarify ATA LBA requirements

Cc: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hale,

Microsoft encourages vendors to support LBA for the

reasons you point out.  A single addressing scheme is

desirable.  However, today we support BIOS translation,

LBA, and DM Driver.

If your IDE drive is visible in the real mode then Chicago

will limit the size of the drive to what is seen by the BIOS.

BIOS can bypass the 528MB limit either by implementing INT 13

extensions/Phoenix EDD or by providing disk geometry

translation that our protect mode driver understands. if you

implement geometry translation scheme then you should support

the extended disk parameter tables (to inform us about the

actual and INT 13 geometry) in the format defined by MS, WD,

Phoenix and Compaq (Actually compaq invented it and everybody

else just published it)

----------

From: "Hale Landis"  <landis@sugs.tware.com>

To: Steve Timm (SYS)

Cc: "ATA Reflector"  <ata@dt.wdc.com>

Subject: Please clarify ATA LBA requirements

Date: Friday, September 09, 1994 10:05AM

Hello Steve,

I don't know if you are the right person at MS to send this to

but I'm sure you know the person(s) with the answer(s).  Please

note that I am copying the ATA reflector too.

I'm sending this because Richard Hoehnle of Seagate tells me that

MS says that drives >528MB will not be supported unless the drive

and the system BIOS supports LBA.  If this is true, what is this

all about?

(Please note that this could all be a misinterpretation of

something someone else said but I would still like to see a

clarification from MS.)

There should be *NO* requirement for LBA support in the drive or

in the BIOS in order to support >528MB.  This is not a

requirement for SCSI host adapter BIOSes that support drives over

528MB for many years now.  Why is it a requirement for ATA drives

over 528MB?

As you know, the tradtional BIOS supports only CHS style

addressing.  In order to support LBA style addressing at the INT

13 interface, the BIOS must support the MS/IBM Extensions.  I

don't see the BIOS people doing that today.

What I do see (in some cases), is BIOS support for CHS

translation (cylinders < 1024, heads > 16 at the INT 13

interface), like the SCSI people have been doing for years, with

some BIOS support for the Phoenix Enhancement.

It would appear that if anything settles out over the next year

or two it will be this:  BIOS will support CHS translation and

not much more, unfortunately, not all implementations will be the

same.

(Even if all the above is someone misinterpretation, I would like

to make the following observations.  Please note that I don't

work for Seagate anymore so the following is just Hale Landis

speaking.)

While last year's "Big Disk" conference at Comdex, sponsored by

Seagate and Western Digital, was widely attended and lots of

ideas were put forth, the conference was at best, a failure.

Yes, the disk drive vendors did agree to the Identify Drive data

requirements (SFF-8019, now part of ATA-2), but I have seen *NO*,

repeat *NO*, agreements on the software side.  I know that

Phoenix is pushing their Enhancements specification, and I think

that is good, but I see no common agreement on this.

If we want to head-off total failure of ATA drives over 528MB in

the marketplace, someone in the software area had better step

forward soon and get some industry agreement on the software

requirements for >528MB ATA drives.  If you don't think this is a

problem, I suggest you start by reading the

comp.sys.ibm.pc.hardware.storage news group on the Internet.

A software specification/standard/agreement should spell-out in

detail which BIOS calls are required and exactly how drive

configuration data is computed and returned by those calls.  The

current software specification don't do all this.  It seems to me

that if >528MB drives are to be widely support, some of the

MS/IBM Extensions and/or Phoenix Enhancement calls must be made

*mandatory*.

I'm sure everyone in the disk drive industry would appreciate any

help you can give towards finding a solution for the >528MB ATA

software support problems.

Thanks,

Hale

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Fri Sep  9 15:26:42 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA04224; Fri, 9 Sep 94 15:26:42 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA04217; Fri, 9 Sep 94 15:26:37 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qjEOv-0002UcC; Fri, 9 Sep 94 18:26 EDT

Received: by ptlnor3.ptltd.com; Fri, 9 Sep 94 18:26:46 EDT

Date: Fri, 9 Sep 94 21:26:41 PDT

Message-Id: <E706+U2CQia@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: Landis@Sugs.Tware.COM

Cc: ATA@dt.wdc.com, SCSI@WichitaKS.NCR.COM

Subject: >528

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Hale

I just received a second E-Mail stating that BIOS in general does

not support drives >528MB and that no consistent methods are being used to

solve the problem.  You are also attempting to propagate the the rumor

that LBA is required to access a drive with >1024 cylinders.  I am issuing

this E-Mail as information on the Phoenix NuBIOS product.

1. Phoenix has already made available to its customers the INT 13

Extensions designed by Microsoft and IBM.  These extensions have been

tested by the Chicago group at Microsoft.

2. Phoenix supports the Enhanced Drive Parameter Table which was

originally implemented by Compaq and documented in the Western Digital

Implementation Guide.  Chicago, Windows NT and OS/2 are documented as

gaining full access to drives with more than 1024 cylinders when the

Enhanced Drive Parameter Table is present.  This table contains both the

geometry reported by the drive (PHYSICAL GEOMETRY) and the geometry used

by the BIOS (LOGICAL GEOMETRY).  This means that at least Phoenix, Compaq

and IBM are shipping BIOS that works with large drives.  I have seen BIOS

from other vendors that appear to support this capability as well.

3. Phoenix does NOT require LBA to support drive with more than 1024

cylinders (or greater than 528MB).  We perform CHS translation on drives

using geometries and translation techniques which place the data in the

same sectors that LBA would have used.  This means that the translation

remains transparent to other software which may access the drive by

bypassing the BIOS.

As far as Phoenix is concerned, INT 13 Fn's 0-19 will remain intact until

the 8.4GB barrier is reached sometime next year.  The extensions are

already being shipped and operate on drives regardless of their "LBA"

capability.  The BIOS automatically translates if a drive does not support

LBA.  I expect that at least 1 version of DOS which uses the Extensions

will be in place by the time >8.4GB media becomes available to the public.

Current non-DOS OS software only uses INT 13 for booting the machine.  I

know that at least one OS to be realeased shortly will install a master

boot record which uses INT 13 extensions if they are present and I expect

others to follow.

Curtis E. Stevens

From ata-owner  Fri Sep  9 16:27:50 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA05641; Fri, 9 Sep 94 16:27:50 PDT

Received: from worf.qntm.com by dt.wdc.com (4.1/SMI-4.1)

id AA05626; Fri, 9 Sep 94 16:27:41 PDT

Received: from mail (mail.qntm.com) by worf.qntm.com with ESMTP

(1.37.109.11/16.2) id AA098393248; Fri, 9 Sep 1994 16:27:29 -0700

Received: from qm_smtpgw.qntm.com (qm_smtpgw) by mail with SMTP

(1.37.109.11/16.2) id AA020853038; Fri, 9 Sep 1994 16:23:58 -0700

Message-Id: <n1432994346.63801@qm_smtpgw.qntm.com>

Date: 9 Sep 1994 16:37:44 U

From: "Jim McGrath" <jmcgrath@qntm.com>

Subject: Re: 10^6 Nonsense

To: "ATA" <ATA@dt.wdc.com>,

"Richard Hoehnle" <Richard_Hoehnle@notes.seagate.com>

X-Mailer: Mail*Link SMTP/QM 3.0.0

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Reply to:   RE>10^6 Nonsense

Richard,

I have to disaggre.  SCSI is base 10, even though it also commonly

uses 512 byte sectors as well.  The key here is that in the real

world the prefixes kilo- and mega- mean 1,000 and 1,000,000.  This

is the metric system - which has been around a lot longer than

computers :-).

Jim

From ata-owner  Fri Sep  9 16:57:32 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA06444; Fri, 9 Sep 94 16:57:32 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA06437; Fri, 9 Sep 94 16:57:29 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qjFom-0002QwC; Fri, 9 Sep 94 19:57 EDT

Received: by ptlnor3.ptltd.com; Fri, 9 Sep 94 19:57:39 EDT

Date: Fri, 9 Sep 94 22:57:34 PDT

Message-Id: <E706+gNDQia@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: ATA@dt.wdc.com

Subject: re:Re: 10^6 Nonsense

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I have to agree with Hale and Jim on this one.  Our BIOS uses a 1 million

MEGA and has from day one.  Users understand this representation.  When

MSDOS 6.22 says a drive has 549,257,182 bytes free, it is very easy to say

you have 549MB free.  As far as the specification is concerned, I don't

know anybody that only works in incrmenets og MEGABYTES, everything I see

works at the BYTE level.  The 2^20 MEGABYTE uses 20 bits, this boundary is

pretty useless as far as I am concerned (speaking as a programmer)...

2^24 has some meaning, I have an intimate relationship with 2^16 and 2^32;

these represent real "boundaries" when it comes to addressing whatever.

ATA is using MEGABYTE (MB) simply as a short form of notation, not a

notation which represents meaningful boundaries.  My understanding has

been that ATA uses a 1 million byte MB, if that assumption is wrong and

there actually is no real concensus, then I would strongly push for 1MB =

1,000,000.

Shortly, the 1MB unit is going to be useless because the 1GB unit is going

to take over.  Lets handle this one now and fix on 1GB = 1,000,000,000.

These prefixes are well understood by a large population to have a base 10

meaning.

Curtis E. Stevens

----------------------[Reply - Original Message]----------------------

Sent by:"Jim McGrath" <jmcgrath@qntm.com>

Reply to:   RE>10^6 Nonsense

Richard,

I have to disaggre.  SCSI is base 10, even though it also commonly

uses 512 byte sectors as well.  The key here is that in the real

world the prefixes kilo- and mega- mean 1,000 and 1,000,000.  This

is the metric system - which has been around a lot longer than

computers :-).

Jim

=====================================================================

From ata-owner  Fri Sep  9 20:42:16 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA11468; Fri, 9 Sep 94 20:42:16 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA11461; Fri, 9 Sep 94 20:42:13 PDT

Received: from seagate.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id UAA03516; Fri, 9 Sep 1994 20:33:08 -0700

Received:  by notes.seagate.com (UUPC/extended 1.11q);

Fri, 09 Sep 1994 15:57:14 EDT

Date:         Fri Sep 09 15:57:13 1994

From: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

Message-Id: <2e70be1a.seagate@notes.seagate.com>

Organization: Seagate Technology

Reply-To: "John Masiewicz" <John_Masiewicz@notes.seagate.com>

To: ata@dt.wdc.com

Subject:      BSY after Command Reg is written - Sanity Check

Sender: ata-owner@dt.wdc.com

Precedence: bulk

In response to Hale's posting (BSY after Command reg is written) and the

subsequent responses:

1)  First of all, let's stop talking about BUSY timeouts of zero or 20

nanoseconds! You CANNOT ever use this because the system will never be able to

take advantage of those numbers anyway.  This is because in PIO mode 3, there

is a I/O read and write recovery time of 70 nS.  In other words, any attempt to

even begin a read within 70 nS of the trailing edge of write will be a SERIOUS

violation of the timing protocol.  Even in PIO mode 4, this number is 25 nS

minimum.  There should be no argument on this.  Why even consider a timing

requirement which cannot possibly be used (or even tested) on any system?

2)  Secondly, You CANNOT change a hard specification and cause potentially

millions of current-technology devices to become non-compliant.  If we want to

make non-backward compatible changes, save it for ATA-3, and let us finish

ATA-2.

2) OK, moving to the gray area between 70 nS and 25 nS.  Most current devices

today (and many still in development and qualification, I'm sure)  were

designed to operate in PIO mode 3 according to the SFF 8011 specification. In

fact, that specification is the de-facto definition of FAST ATA.  In that spec,

there is only PIO mode 3, with the 70 nS recovery time.

3)  In ATA-1 (modes 0-2) the minimum I/O R/W time for 8-bit reads was 290 nS

with about another 100 nS minimum recovery time. Mode 0 is 600 nS cycle time,

so the 400 nS BUSY time is very reasonable there.  We have been recommending

for some time now that the hosts continue to do all 8-bit non-data transfers

using the older modes.  This is to prevent compatibility problems with older

devices.  In fact, some committee members have argued that you MUST do 8-bit

I/O in the slower modes.

4)  Moving on to the argument that faster modes should have faster timeouts,

remember that use of the faster modes directly implies that you will have a

controller chip at the PCI or other local bus. (yes, there are some adapters

that plug into the ISA bus, but they can't take advantage of  the faster

performance anyway) These PCI or local bus controllers employ some type of

translating the 8-bit ATA transfers to the 32-bit busses when doing status

reads.  In this operation, the host must perform some type of I/O operation

through the controller.

Does anyone out there know haw fast a Pentium can do an I/O read after a

write?  I'm guessing, that when all the machine cycles are added up and the

controller is considered, that it's more an 100 nS.  Let's be realistic here.

There is no reason to put an unusable spec onto the devices.

5)  I'm really curious about the comment that "timing loops" are needed to time

out 400 nS.  I would also like someone to tell me how many 8-bit  I/O reads to

the device (with instruction prefetches, etc) can be accomplished in 400 nS.

It seems to me that it's only a few instructions.  Remember, the host can start

reading immediately, and if the device has responded, it done.  It does not

have to wait 400 nS to start reading, so there is no performance issue.  What

we are talking about is a few lines of code.

6)  I totally agree with Robbie's (Nat'l Semi) comments about hardware

overhead.  In our designs, we do have some overhead.  Even if you only count

the time to get the read decoded and propagate the status out, you have some

overhead.  We also employ some synchronization logic to ensure that the

BUSY/DRQ relationship is synchronous with each other, as required by the spec.

This overhead can typically be one or two clocks.  Believe me, there are

millions of devices out in the world with similar overhead.

My conclusions:

This is not a performance issue.

This is a firmware overhead issue.

The proposals are not backward compatible.

Timeouts faster than the system can actually use are useless.

My Recommendations:

Do not accept proposals to change fundamental ATA timing protocols.

Proposals should be made in ATA-3 working groups, not ATA-2.

John Masiewicz

Seagate

--

John Masiewicz -- John_Masiewicz@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\1044.MSG

### Org Date:    09-09-94 01:30:12 PM

### From:        John Masiewicz@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     BSY after Command Reg is written - Sanity Check

### Attachments: none

From ata-owner  Fri Sep  9 22:34:00 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA14010; Fri, 9 Sep 94 22:34:00 PDT

Received: from netcomsv.netcom.com (uucp2-b.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA13885; Fri, 9 Sep 94 22:33:24 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id WAA06666; Fri, 9 Sep 1994 22:28:24 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qjKfA-000gJTC; Fri, 9 Sep 94 23:07 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Fri, 09 Sep 1994 23:09:43 EDT

Date:      Fri, 09 Sep 1994 23:09:40 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e712377.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: curtis_stevens@bannet.ptltd.com

Cc: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   Re: >528

Sender: ata-owner@dt.wdc.com

Precedence: bulk

On Fri, 9 Sep 94 21:26:41 PDT, curtis_stevens@bannet.ptltd.com wrote:

> I just received a second E-Mail stating that BIOS in general does

> not support drives >528MB and that no consistent methods are being used to

> solve the problem.

This is true and it is what I was trying to say and it *is* what

is happening in the market place.  I have my own (very strong)

personal opinions about what is *really* happening in the real

world:  I think >528MB may well do permanent damage to the ATA

disk drive market due to the lack of a common implementation

based on a single clear specification.

> You are also attempting to propagate the the rumor

> that LBA is required to access a drive with >1024 cylinders.

No, I did not say that I thought LBA was required. If fact I have

always argued that LBA is *not* required.  I was asking MS to

clarify a statement that was attributed to MS that implied that

MS was requiring LBA for >528MB.  I think Steve Timm's response

was clear and states that LBA is desired but *not* required

for >528MB today.

As for what Phoenix is doing, I think it is great.  Too bad its

not in an SFF or ANSI document so the whole world can implement

the same thing.  And I agree about the 8GB boundary but I also

know that this one will probably get specified and documented

since it affects both ATA *and* SCSI.  I'm very sure the disk

drive vendors will not let *both* the ATA and SCSI markets be

damaged by the lack of specifications/standards from the software

industry.

Disk drive capacity is related to the amount of data that can be

placed on a single platter.  A single platter drive is always the

cheapest and smallest capacity drive in the market.  The industry

is in the 300MB per platter range today with every expectation that

1GB per platter is not too many years away.  This means that even

an "entry level" system will some day have a 1GB drive, be it ATA

or SCSI or whatever.  I sure hope we have our current software

problems solved by the time this happens!

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Sat Sep 10 14:01:01 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA01204; Sat, 10 Sep 94 14:01:01 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA01197; Sat, 10 Sep 94 14:00:56 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qjZXU-0002QwC; Sat, 10 Sep 94 17:00 EDT

Received: by ptlnor3.ptltd.com; Sat, 10 Sep 94 17:01:01 EDT

Date: Sat, 10 Sep 94 20:00:52 PDT

Message-Id: <E706+2uVQiA@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: ATA@dt.wdc.com

Subject: re:BSY after Command Reg is written - Sanity Check

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

The size of the Message Text is  6137(> 6K), So it is kept in the First AttachmenJohn

The reason this is coming up now is because I recently had to fix

a software failure which we traced to the BUSY status not being set after

a command was issued.  I insert a couple dummy 8 bit OUT instructions in

the code and the problem went away.  This would be OK except some chipsets

will let you program a faster I/O timing, I have seen 8, 10, 12 & 16 Mhz.

PCI and Local Bus are a reality which changes some of the rules when it

comes to assumptions on timing.  66mhz PCI is now a reality, this has a

cycle time of 15ns, even on an 8 bit read!  Drives may not be able to

respond quite that fast, but they shoul be able to reasonably beat the

400ns number.  PIO Mode 5 is not far away, if I am not mistaken it is

supposed to have a cycle time of less the 100ns (90ns?).  I think it is

ludicrous to have a 400ns response time in the spec. when even ATA-2 has

Mode 4 running at 120ns.

Curtis E. Stevens

----------------------[Reply - Original Message]----------------------

Sent by:"John Masiewicz" <John_Masiewicz@notes.seagate.com>

In response to Hale's posting (BSY after Command reg is written) and the

subsequent responses:

1)  First of all, let's stop talking about BUSY timeouts of zero or 20

nanoseconds! You CANNOT ever use this because the system will never be

able to

take advantage of those numbers anyway.  This is because in PIO mode 3,

there

is a I/O read and write recovery time of 70 nS.  In other words, any

attempt to

even begin a read within 70 nS of the trailing edge of write will be a

SERIOUS

violation of the timing protocol.  Even in PIO mode 4, this number is 25

nS

minimum.  There should be no argument on this.  Why even consider a timing

requirement which cannot possibly be used (or even tested) on any system?

2)  Secondly, You CANNOT change a hard specification and cause potentially

millions of current-technology devices to become non-compliant.  If we

want to

make non-backward compatible changes, save it for ATA-3, and let us finish

ATA-2.

2) OK, moving to the gray area between 70 nS and 25 nS.  Most current

devices

today (and many still in development and qualification, I'm sure)  were

designed to operate in PIO mode 3 according to the SFF 8011 specification.

In

fact, that specification is the de-facto definition of FAST ATA.  In that

spec,

there is only PIO mode 3, with the 70 nS recovery time.

3)  In ATA-1 (modes 0-2) the minimum I/O R/W time for 8-bit reads was 290

nS

with about another 100 nS minimum recovery time. Mode 0 is 600 nS cycle

time,

so the 400 nS BUSY time is very reasonable there.  We have been

recommending

for some time now that the hosts continue to do all 8-bit non-data

transfers

using the older modes.  This is to prevent compatibility problems with

older

devices.  In fact, some committee members have argued that you MUST do

8-bit

I/O in the slower modes.

4)  Moving on to the argument that faster modes should have faster

timeouts,

remember that use of the faster modes directly implies that you will have

a

controller chip at the PCI or other local bus. (yes, there are some

adapters

that plug into the ISA bus, but they can't take advantage of  the faster

performance anyway) These PCI or local bus controllers employ some type of

translating the 8-bit ATA transfers to the 32-bit busses when doing status

reads.  In this operation, the host must perform some type of I/O

operation

through the controller.

Does anyone out there know haw fast a Pentium can do an I/O read after a

write?  I'm guessing, that when all the machine cycles are added up and

the

controller is considered, that it's more an 100 nS.  Let's be realistic

here.

There is no reason to put an unusable spec onto the devices.

5)  I'm really curious about the comment that "timing loops" are needed to

time

out 400 nS.  I would also like someone to tell me how many 8-bit  I/O

reads to

the device (with instruction prefetches, etc) can be accomplished in 400

nS.

It seems to me that it's only a few instructions.  Remember, the host can

start

reading immediately, and if the device has responded, it done.  It does

not

have to wait 400 nS to start reading, so there is no performance issue.

What

we are talking about is a few lines of code.

6)  I totally agree with Robbie's (Nat'l Semi) comments about hardware

overhead.  In our designs, we do have some overhead.  Even if you only

count

the time to get the read decoded and propagate the status out, you have

some

overhead.  We also employ some synchronization logic to ensure that the

BUSY/DRQ relationship is synchronous with each other, as required by the

spec.

This overhead can typically be one or two clocks.  Believe me, there are

millions of devices out in the world with similar overhead.

My conclusions:

This is not a performance issue.

This is a firmware overhead issue.

The proposals are not backward compatible.

Timeouts faster than the system can actually use are useless.

My Recommendations:

Do not accept proposals to change fundamental ATA timing protocols.

Proposals should be made in ATA-3 working groups, not ATA-2.

John Masiewicz

Seagate

--

John Masiewicz -- John_Masiewicz@notes.seagate.com

-------------------------------------------------------------------------

Seagate Technology   -   920 Disc Drive   -   Scotts Valley, CA 95066 USA

Main Phone 408-438-6550   -   Email Problems postmaster@notes.seagate.com

Technical Support: BBS 408-438-8771  Fax 408-438-8137  Voice 408-438-8222

-------------------------------------------------------------------------

### OGATE Version 8 message trace and attachment information:

### MsgFileName: m:\mgate\outbound\1044.MSG

### Org Date:    09-09-94 01:30:12 PM

### From:        John Masiewicz@SEAGATE

### To:          ata @ dt.wdc.com @ internet

### Subject:     BSY after Command Reg is written - Sanity Check

### Attachments: none

=====================================================================

From ata-owner  Mon Sep 12 09:58:10 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA20218; Mon, 12 Sep 94 09:58:10 PDT

Received: from netcomsv.netcom.com (uucp11.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA20211; Mon, 12 Sep 94 09:58:07 PDT

Received: from localhost by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id JAA05464; Mon, 12 Sep 1994 09:57:22 -0700

Received: from cc:Mail by aspi.advansys.com

id AA779389029 Mon, 12 Sep 94 09:57:09

Date: Mon, 12 Sep 94 09:57:09

From: "Pham, Al" <alpham@advansys.com>

Message-Id: <9408127793.AA779389029@aspi.advansys.com>

To: ata@dt.wdc.com

Subject: Re: 10^6 Nonsense

Sender: ata-owner@dt.wdc.com

Precedence: bulk

It is ridiculous comparing computer terms to standard or common

(Metric) system.  Computer terms are very much arbitrary.  From

day one, 1K was (and still is) 1024.  A byte is 8-bit, a nibble

is 4.  If MB is define as 1,000,000 then how does one convert to

KB ?

You can not look up in a dictionary and tell people that what

Mega and Kilo mean.  Just try to look up the word "nibble" and

tell that to computer engineer what it means !!!

The word will mean what it is defined.  So be consistent.  Is MB

10 times KB ?  Then if KB is 1024 then MB must be 20^2.

-- AL Pham --

From ata-owner  Mon Sep 12 11:01:47 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA21841; Mon, 12 Sep 94 11:01:47 PDT

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA21638; Mon, 12 Sep 94 10:53:47 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id KAA17041; Mon, 12 Sep 1994 10:27:06 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qkEyS-000gJTC; Mon, 12 Sep 94 11:15 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Mon, 12 Sep 1994 11:16:56 EDT

Date:      Mon, 12 Sep 1994 11:16:55 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e7470e8.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "Steve Timm"         <stevetim@microsoft.com>,

"ATA Reflector"      <ata@dt.wdc.com>

Subject:   question about LBA usage

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I have another question about LBA usage on drives >528MB.  Note

that I have copied the ATA Reflector again.  And yes, I'll admit

it, I am "nit picking" but there is so much misinformation going

around about this that I want to get the facts correct.

First a definition:

Translating BIOS:  a BIOS that reports to its caller that a drive

>528MB has <1024 cylinders and >16 heads.

My question is:

How does using LBA mode *alone* at the ATA interface solve the

>528MB problem?

Some background questions that lead up to this question:

1) Is it not true that the partition records created by FDISK

must contain the proper starting and ending CHS locations for

each partition?

2) Is is not true that those starting and ending CHS locations

are limited to cylinder 1023 or less just as all of the

traditional INT 13 calls are limited?

3)Is it not true that if a drive is >528MB, then the partition

record must contain translated CHS starting and ending locations?

4) Is it not true that FDISK depends on the BIOS to provide the

total disk size (INT 13 AH=08) and that the only way the BIOS can

report the size of a drive >528MB is my using a translated CHS or

by using the MS/IBM extensions?

5) Is it not true that even if the BIOS supports the MS/IBM

Extensions, the BIOS must still use translated values for INT 13

AH=08?

6) If the answer to questions 1-5 are "true", then isn't the

basic requirement that the BIOS must support CHS translation?  Of

course the BIOS in question could be the system BIOS or an INT 13

replacement supplied on an adapter card or via a software driver.

7) And then is it not also true that LBA is not a *basic* requirement

for the support of drives >528MB?  Isn't using LBA just a "nice thing"

that reduces a little software overhead in a BIOS or driver read/write

call?  Isn't the support of translated CHS the *basic* requirement

today for drives >528MB?

8) In your respond Friday you said:

[...] However, today we support BIOS translation, LBA, and DM

Driver.  [...]

Would it not be more correct to say something like:

Today we support BIOS translation and DM Driver (which does

CHS translation).  If the drive supports LBA addressing then

some system overhead can be eliminated on each disk read/write

call.

And make it more clear that it is the translated CHS that is the

*first* requirement in order to support >528MB?

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------

From ata-owner  Mon Sep 12 11:30:15 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA22562; Mon, 12 Sep 94 11:30:15 PDT

Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

id AA22514; Mon, 12 Sep 94 11:28:47 PDT

Received: from jobe.shell.portal.com (jobe.shell.portal.com [156.151.3.4]) by nova.unix.portal.com (8.6.7/8.6.5) with ESMTP id LAA00793; Mon, 12 Sep 1994 11:28:42 -0700

Received: from localhost (bberg@localhost) by jobe.shell.portal.com (8.6.4/8.6.5) id LAA22425; Mon, 12 Sep 1994 11:28:39 -0700

Date: Mon, 12 Sep 1994 11:28:39 -0700

From: Brian A Berg <bberg@shell.portal.com>

Message-Id: <199409121828.LAA22425@jobe.shell.portal.com>

To: alpham@advansys.com

Subject: Re: 10^6 Nonsense

Cc: ata@dt.wdc.com

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I hear you regarding all this business about 10^6 vs. 2^20, for example.

I have found it best to use "k" for 1000 and "K" for 1024.  Similarly for

"m" and "M".  In addition, how many times have we all seen bit and byte

counts ambigously stated?  I use "b" for bit and "B" for byte, but of

course the rest of the world does not necessarily follow my convention,

so I have come to use "bit" and "Byte", e.g., Mbit and kByte.

Indeed, as has been pointed out, marketeering has caused "mega" to mean

1,000,000, and I can live with that I guess.  However, since all of these

terms have ambiguities, the bottom line should be to at least eliminate

these ambiguities.  This can best be done with footnotes and/or clear

descriptions in standards, marketing blurbs, etc.

I suggest what I have outlined in the first paragraph, along with an

explanatory footnote or some accompanying text which ELIMINATES AMBIGUI-

TIES, i.e., allow for no assumptions about what is stated.  Thank you all

for your attention.

============================================================================

Brian A. Berg           E-mail: bberg@bswd.com

Berg Software Design     Voice: 408.741.5010

P.O. Box 3488              FAX: 408.741.5234

Saratoga, CA 95070 USA   Focus: software consulting: SCSI, optical storage

From ata-owner  Mon Sep 12 11:49:33 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA23025; Mon, 12 Sep 94 11:49:33 PDT

Received: from ptltd.ptltd.com (spock_red.ptltd.com) by dt.wdc.com (4.1/SMI-4.1)

id AA23018; Mon, 12 Sep 94 11:49:29 PDT

Received: from ptlnor3.ptltd.com by ptltd.ptltd.com with smtp

(Smail3.1.27.1 #1) id m0qkGRB-0002UnC; Mon, 12 Sep 94 14:49 EDT

Received: by ptlnor3.ptltd.com; Mon, 12 Sep 94 14:49:19 EDT

Date: Mon, 12 Sep 94 17:49:13 PDT

Message-Id: <E706+c88RiA@ptlnor3.ptltd.com>

From: curtis_stevens@bannet.ptltd.com

To: ATA@dt.wdc.com

Subject: re:Re: 10^6 Nonsense

X-Incognito-Sn: 258

X-Incognito-Format: VERSION=1.71 ENCRYPTED=NO

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I actually use a similiar notation, but I think we should be aware that

the definition of computer terms is very dependent on the computer

archetecture.  Have worked on machines with 36 bit words and 9 bit bytes!

I have also seen Mega Byte defined as 1000K (1,024,000) as well.  I think

the problem needs to be cleared up with a definition of terms, and in this

case the definition of a megabyte should be 1,000,000.  I also think that

gigabyte should be 1,000,000,000.

Curtis E. Stevens

----------------------[Reply - Original Message]----------------------

Sent by:Brian A Berg <bberg@shell.portal.com>

I hear you regarding all this business about 10^6 vs. 2^20, for example.

I have found it best to use "k" for 1000 and "K" for 1024.  Similarly for

"m" and "M".  In addition, how many times have we all seen bit and byte

counts ambigously stated?  I use "b" for bit and "B" for byte, but of

course the rest of the world does not necessarily follow my convention,

so I have come to use "bit" and "Byte", e.g., Mbit and kByte.

Indeed, as has been pointed out, marketeering has caused "mega" to mean

1,000,000, and I can live with that I guess.  However, since all of these

terms have ambiguities, the bottom line should be to at least eliminate

these ambiguities.  This can best be done with footnotes and/or clear

descriptions in standards, marketing blurbs, etc.

I suggest what I have outlined in the first paragraph, along with an

explanatory footnote or some accompanying text which ELIMINATES AMBIGUI-

TIES, i.e., allow for no assumptions about what is stated.  Thank you all

for your attention.

==========================================================================

==

Brian A. Berg           E-mail: bberg@bswd.com

Berg Software Design     Voice: 408.741.5010

P.O. Box 3488              FAX: 408.741.5234

Saratoga, CA 95070 USA   Focus: software consulting: SCSI, optical

storage

=====================================================================

From ata-owner  Mon Sep 12 13:37:08 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA25297; Mon, 12 Sep 94 13:37:08 PDT

Received: from nova.unix.portal.com by dt.wdc.com (4.1/SMI-4.1)

id AA25290; Mon, 12 Sep 94 13:37:05 PDT

Received: from hobo.corp.portal.com (hobo.online.portal.com [156.151.5.5]) by nova.unix.portal.com (8.6.7/8.6.5) with ESMTP id NAA10725 for <ata@dt.wdc.com>; Mon, 12 Sep 1994 13:37:00 -0700

From: KANEMATSU-SANYO@cup.portal.com

Received: from localhost (pccop@localhost) by hobo.corp.portal.com (8.6.4/1.63) id NAA16068 for ata@dt.wdc.com; Mon, 12 Sep 1994 13:36:58 -0700

To: ata@dt.wdc.com

Subject: ATAPI MEETING

Lines: 6

Date: Mon, 12 Sep 94 13:36:57 PDT

Message-Id: <9409121336.4.15843@cup.portal.com>

X-Origin: The Portal System (TM)

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I AM INTERESTED IN ACQUIRING A COPY OF THE MEETING MINUTES OF THE LAST MEET-

ING HELD AUGUST 30, 1994 AND ALSO ACCESSING INFORMATION FROM THE REFLECTOR

BULLETIN BOARD.

THANKS FOR YOUR HELP!

PETER KOIWAI

From ata-owner  Mon Sep 12 13:48:22 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA25576; Mon, 12 Sep 94 13:48:22 PDT

Received: from milpitas.adaptec.com by dt.wdc.com (4.1/SMI-4.1)

id AA25569; Mon, 12 Sep 94 13:48:15 PDT

Received: from eng.adaptec.com ([162.62.20.6]) by milpitas.adaptec.com (4.1/SMI-4.1)

id AA27923; Mon, 12 Sep 94 13:45:14 PDT

Received: from holodeck by eng.adaptec.com (4.1/SMI-4.1)

id AA06496; Mon, 12 Sep 94 13:46:33 PDT

Date: Mon, 12 Sep 94 13:46:33 PDT

From: lroysh@eng.adaptec.com (Roy Shenfield x2473)

Message-Id: <9409122046.AA06496@eng.adaptec.com>

To: ATA@dt.wdc.com

Subject: Re: 10^6 vs. 2^20

Sender: ata-owner@dt.wdc.com

Precedence: bulk

I've been reading the mail regarding the 10^6 vs. 2 ^20 controversy.

I've always referred as the 10^6 notation as 1 MB decimal, and the 2^20

as 1 MB binary.

Regardless of which is chosen to represent capacity, the important thing

is consistency.

Unfortunately several of the DOS utilities report memory and capacity in

the binary format (FDISK, MEM). Chkdsk of course reports the actual number

of bytes (xxx,xxx,xxx), so there's inconsistency there. Windows File Manager

uses the binary notation for free and total disk space, and an actual number

for the file selected directory file total. I suspect many other programs

that report capacity also do in an inconsistent manner.

I got burned by this at a previous company where we were upping the capacity

of our drive from (decimal notation) 1 GB to 1.2 GB (this was a SCSI drive).

The 1.2 GB number was very critical for a potential customer. When we gave

the drive to our system test people and they were going to run DOS on it

FDISK reported the drive had 1.14 GB (binary notation). Thinking the drive

would fail to meet the customer requirement, they stopped shipment of

evaluation units to the customer, and caused a major fire drill for a day

and a half until we got them straightned out.

So whatever is decided should be done with some thought to what is already

out there as a defacto standard or maybe a defacto non-standard.

---------------------------------

Roy Shenfield

Senior Staff Engineer

Adaptec

email: lroysh@eng.adaptec.com

From ata-owner  Mon Sep 12 16:28:54 1994

Return-Path: <ata-owner>

Received: by dt.wdc.com (4.1/SMI-4.1)

id AA29580; Mon, 12 Sep 94 16:28:54 PDT

Received: from netcomsv.netcom.com (uucp5.netcom.com) by dt.wdc.com (4.1/SMI-4.1)

id AA29573; Mon, 12 Sep 94 16:28:40 PDT

Received: from tware.UUCP by netcomsv.netcom.com with UUCP (8.6.4/SMI-4.1)

id PAA04978; Mon, 12 Sep 1994 15:57:23 -0700

Received: from sugs by tware.tware.com with uucp

(Smail3.1.28.1 #3) id m0qkJqP-000gJTC; Mon, 12 Sep 94 16:27 MDT

Received:  by sugs.tware.com (UUPC/extended 1.11q);

Mon, 12 Sep 1994 16:29:16 EDT

Date:      Mon, 12 Sep 1994 16:29:16 EDT

From: "Hale Landis" <landis@sugs.tware.com>

Message-Id: <2e74ba1d.sugs@sugs.tware.com>

Organization: SUGS

Reply-To: "Hale Landis" <landis@sugs.tware.com>

To: "ATA Reflector"      <ata@dt.wdc.com>

Subject:   another 2 cents on kilo/mega

Sender: ata-owner@dt.wdc.com

Precedence: bulk

Lets not forget that kilo, mega, nano, micro, etc are all ISO

defined prefixes that predate the computer industry by many many

many years.

The *entire* world understands that kilo means 1,000 (kilometer,

kilohertz, etc).  It is only *some* people in the computer

industry that use kilo to mean 1024 and then only when talking

about bits and bytes.  This was *never* the proper usage for this

prefix.  Same goes for mega.

I agree with those that say we should put something into the

glossary or into a note that says kilo is 1,000 and mega is

1,000,000 (and giga is 1,000,000,000).  However, you may find

that if the document ever gets to ISO, they will delete this

because its not needed -- everyone knows what kilo and mega

are when they are reading an ISO document (or when they are

reading an ANSI document since the US uses the same ISO defined

prefixes)!

--

-------------------------------------

Hale Landis             Niwot, CO USA

landis@sugs.tware.com    303-548-0567

-------------------------------------