The Sender ADI-145 and ADI-450 radios are modifiable for TX and RX

entirely thru the keyboard.  There is no hardware modifications

required for the transmit.  Both radios, to my understanding, use the

same key strokes for extending the TX and RX ranges.  After performing

the keystroke mod on my ADI-145, I was able to increase the TX and

RX range from 130.00 to 169.00 Mhz.  The ADI-450 range should extend

from 430-470 Mhz using the same keyboard strokes.

1. Start by turning the radio on while holding the function key down.

2. Turn the radio off and while pressing the 3/STEP key on the keyboard,

turn the radio back on, the display should now read 145.000 with a

"m" in the lower left corner of the LCD display.

3. Hold the function key down and push the 3/STEP key, the display should

read 10.  Turn the channel selector knob clockwise until the display

read 05.

4. Hold the function key down and press number 9 on the keypad, the display

should now read 00.60.

5. Using the keypad direct entry, enter 30.63, then turn the channel selector

knob clockwise until the frequency reads 30.635.

6. Holding the function key down, press the 3/STEP key, the display should

read 05.  Turn the channel selector knob so the display reads 10.

7. Holding the function key down, press the O/SET key on the keypad.

8. Now press number 7 on the keypad, the display should be blank.

9. Using the keypad direct entry, enter 30.63 and the press the PTT bar.

The display should now read 130.00.  You Mod is now done.

10. To change your TX and RX widths, hold the function key down and press

the O/SET key on the keypad.  Release, then press the number 7 key

by itself.  The display should go blank (as in step #8).  Now using

direct entry, enter your RX and TX limits directory from the keypad.

For instance, if you were to input 40.69, your TX and RX would be

in the range from 140.00-169.00.

Rickie Belitz,  KD4RKP