CATY

Control program for Yaesu CAT system transceivers.

Version 1.0

Written in Turbo Pascal for PCDOS/MSDOS computers

by D. J. Wilke, N3HGQ.

PRELIMINARY DOCUMENTATION - February 4, 1990.

OVERVIEW:

CATY is a program designed to take command of your Yaesu radio,

equipped with the CAT (Computer-Aided Tuning) system, from your

personal computer.  This version was written to take control of

the FT-767GX.  The software exploits the full implementation

of the CAT system, allowing control of VFO/memory data entry,

mode selection and tuning.  Complete status reporting from the

radio is displayed on screens showing all parameters grouped in

a logical, straightforward manner.

In addition to the memory channels designed into the radio, CATY

provides "Expanded Memory" for your Yaesu.  100 additional memory

channels per data file can be copied to the internal VFO from disk,

allowing unlimited libraries of frequencies, modes, offsets and

identifying data to be archived.  These additional channels reside

in a RAM array once loaded from disk, and can be scrolled and

selected instantaneously.  Repeater offsets can be set automatically

from the stored data, eliminating the tedious procedure at the

radio's	front panel.  An initial library of data files is provided

for both VFO and memory tuning, containing known active services

such as international shortwave stations, amateur repeaters, AMTOR,

standard time signals, radioteletype, CW, FAX, packet and standard

broadcast.

All controllable parameters are entered from full-screen menus and

function keys, with the most often-used functions occupying prominent

positions.  Between commands to the radio, the software returns

control to the operator, allowing manual operation to resume.  An on-

screen real-time clock displays time in both local and UTC formats.

HARDWARE REQUIREMENTS:

IBMPC/XT/AT or compatible with at least 256k memory.

MDA/CGA/EGA/VGA display adapter and monitor.

RS-232 serial communications adapter configured as COM1: or COM2:.

Yaesu FT-767 transceiver with CAT system.

TTL to RS-232 interface for Yaesu radio.

THE FILES:

The distribution disk contains the following files:

PROGRAMS:

CATY767.EXE  - The main executable program.

LOADY.EXE    - Command line-driven memory channel loader.

SOURCE CODE:

CATY767.PAS  - TP5 source for the main program.

Y767.GLO     - TP5 global equates and setup parameters.

Y767GLO.PAS  - TP5 unit to access global references.

Y767DISP.PAS - TP5 unit for display routines.

Y767COMM.PAS - TP5 unit for serial communications.

Y767INST.PAS - TP5 unit for receiver control routines.

Y767UTIL.PAS - TP5 unit for utility routines.

Y767RTC.PAS  - TP5 unit for interrupt driven real-time clock.

ASCII FILES:

CATY.DOC     - What you're reading now.

CATY1.CFG    - Config file for COM1:, rename to CATY.BAT.

CATY2.CFG    - Config file for COM2:, rename to CATY.BAT.

CATY.BAT     - Batch file to load CATY767.EXE with cmd line options.

LOADY1.BAT   - Batch file to load memories through COM1:.

LOADY2.BAT   - Batch file to load memories through COM2:.

ASCII DATABASE FILES for VFO (capacity 100 entries per file):

Y767AMB.DAT  - Standard AM broadcast VFO database.

Y767PAK.DAT  - Packet radio HF/VHF VFO database.

Y7672MR.DAT  - 2 meter voice repeater VFO database.

Y767SWA.DAT  - International shortwave 2-11.5 mHz. VFO database.

Y767SWB.DAT  - International shortwave 11.5-28 mHz. VFO database.

Y767TTA.DAT  - Commercial radio teletype 2-14.5 mHz. VFO database.

Y767TTB.DAT  - Commercial radio teletype 14.5-19.3 mHz. VFO database.

Y767TTC.DAT  - Commercial radio teletype 19.3-25.5 mHz. VFO database.

Y76725M.DAT  - 25 meter shortwave band VFO database.

Y76731M.DAT  - 31 meter shortwave band VFO database.

ASCII DATABASE FILES for MEMORIES (capacity 10 entries per file):

Y767DEF.MEM  - Default (demo) FT-767GX memory datafile.

Y767AMT.MEM  - AMTOR/SITOR channels memory datafile.

Y767CWB.MEM  - Amateur CW band edge memory datafile.

Y767PHB.MEM  - Amateur Phone band edge memory datafile.

Y767PAK.MEM  - HF/VHF packet radio memory datafile.

Y7672MR.MEM  - 2 meter voice repeater memory datafile.

Y767FAX.MEM  - Commercial Facsimile memory datafile.

Y767RTY.MEM  - Commercial radioteletype memory datafile.

INSTALLATION:

Configure your computer so that the serial RS-232 port(s) are

available as COM1: or COM2:.  The Yaesu radio MUST have an external

RS-232 interface to connect to your computer OR you must provide

a way to communicate to/from the radio using its provided TTL

interface connector.  Only three communications lines are used:

TX (Transmit Data), RX (Receive Data) and Signal Ground.  There

is no hardware handshaking in the Yaesu CAT implementation,

so the interconnections are minimal.

If COM1: is chosen, invoke the batch file CATY1.BAT; likewise

CATY2.BAT for COM2:.  These files can be edited to inform the

local/UTC clock routines in CATY which timezone to use, the

default is EST.  To change to EDT, edit the second command

line option within the corresponding batch file, for example:

<CATY767 /comport /timezone> where /comport = [/1] or [/2] and

/timezone = [/S] (Eastern Standard) or [/D] (Eastern Daylight.)

If your system does not have a real-time clock/calendar card,

edit CATY1.BAT or CATY2.BAT, adding DOS DATE and TIME commands

to set the correct local time for CATY's RTC as in the following

CATY2.BAT example:

echo off

CATY767 /2 /S

date

time

If running from floppy disk, be certain all supporting files are

present.  On hard disk systems, copy entire set of files into a

separate directory named \CATY.

USING THE OPTIONS:

The program is entirely menu and function key driven.  Upon startup,

you are presented with a full screen Main Menu and the Function

Keys on line 25.  The available options are:

KEY                       WHAT IT DOES

===  =============================================================

1    Displays radio's status as a full screen display

2    Queries operator and sets operating frequency

3    Queries operator and sets programmed frequency step DOWN

4    Queries operator and sets programmed frequency step UP

5    Selects between VFO A, VFO B or memory

6    Selects active memory channel, 0 through 9

7    Sets operating mode to LSB, USB, CW, AM, FM OR FSK

8    Selects between Ham bands and general coverage receiver

9    Sets CTCSS tone frequencies (NOT IMPLEMENTED IN THIS RELEASE)

0    Turns off Split, Clarifier, Offset functions

+    Steps UP 10 Hertz in frequency, displays result

-    Steps DN 10 Hertz in frequency, displays result

F1   Loads memories from disk datafile

F2   Queries operator and loads VFO A from disk datafile

F3   Moves down to next lower band (depends on Ham/General setting)

F4   Moves up to next higher band (depends on Ham/General setting)

F5   Toggles SPLIT on or off

F6   Toggles CLARIFIER on or off

F7   Moves selection from selected memory to VFO A

F8   Moves selection from VFO A to selected memory

F9   Swaps contents of VFO A and selected memory

F10  Quits the program and returns to DOS

QUICK MEMORY LOAD:

There is a convenient way to quickly load or change all memory

frequencies from the command line.  The datafiles ending with .MEM

can be loaded into the Yaesu's memories using a separate file named

LOADY.EXE.  Two batch files are included which allow the use of

your choice of COM port.  It is only necessary to indicate the

category of the desired service on the command line to LOADYx.BAT,

as in:

LOADY1(or 2) PAK<CR> - Loads frequencies in Y767PAK.MEM

LOADY1(or 2) FAX<CR> - Loads frequencies in Y767FAX.MEM

or

You can invoke LOADY.EXE directly, providing your own command line

option such as:

LOADY PAK /1<CR> - Loads Y767PAK.MEM through COM1:

LOADY PAK /2<CR> - Loads Y767PAK.MEM through COM2:

The default COM port is COM1: if no command line argument is

supplied.

For simplicity, the command to LOADY is shortened to the three

characters preceding the .MEM in the memory datafiles:

Y767DEF.MEM = LOADY DEF<CR> or LOADY DEF [/1] [/2]

Y767AMT.MEM = LOADY AMT<CR> or LOADY AMT [/1] [/2]

Y767CWB.MEM = LOADY CWB<CR> or LOADY CWB [/1] [/2]

Y767PHB.MEM = LOADY PHB<CR> or LOADY PHB [/1] [/2]

Y7672MR.MEM = LOADY 2MR<CR> or LOADY 2MR [/1] [/2]

Y767RTY.MEM = LOADY RTY<CR> or LOADY RTY [/1] [/2]

New datafiles can be created using your favorite ASCII editor,

just be sure to follow the structure of the included files and

don't exceed 10 entries.  The file naming convention must be

followed for LOADY.EXE to work.

RELEASE NOTES:

CATY v1.0 is no cost software.  You cannot be charged a fee

to obtain a copy of this software including documentation

and database files.  You may use the program and tinker with

the source code, but you may not sell this code or program.

You may give a copy of this software to any person who desires

it as long as there is no charge for obtaining the copy.

This software is being made available as a learning and

development vehicle for the amateur radio community.

This sofware is NOT in the public domain.

This .DOC file requires a great deal of work, as the source

code is still evolving.  The program was developed entirely in

Turbo Pascal Version 5.5 utilizing Unit files.  Future planned

enhancements include:

Support for FT757, FT747;

An automatic logging module;

A frequency scan function;

Integrated TNC control;

Integrated datafile creation;

Lookup tables for packet commands;

Linkage to existing BBS programs;

A DOS shell function; and

Ideas from contributors.

Inputs, recommendations, critiques, wish lists and bug reports

are requested.  Please keep this package intact with all data

files, source code and documentation.  Improvements and

embellishments to the source are invited.  When updating the

source, please maintain the naming convention (e.g. CATY10.ZIP,

CATY11.ZIP etc.)

This project was a lot of work, but also a very valuable learning

experience.  I hope it will be a springboard for a really solid

and useful ham radio program.  How about it guys and gals - will

you help to sweeten the code?

73,		Don Wilke, N3HGQ

4228 Mount Carmel Road

Upperco, MD 21155

(301) 374-2735

or @KA3QAC packet BBS (145.03)