Baza de date si echivalente de Semiconductori

1.5KE100P-MBR UNIDIR TRANSIL 100V 1500W

1.5KE39P-MBR UNIDIR TRANSIL 39V 1500W

1.5KE47P-MBR UNIDIR TRANSIL 47V 1500W

10.000 HC49 QUARZE 10.000.000 MHz

10.700 HC49 QUARZE 10.700.000 MHz

1001012 Echivalent: : N.L.A.

11DQ04 Echivalent: BYV10/40-MBR

12.000 HC49 QUARZE 12.000.000 MhZ

12C508 Echivalent: PIC12C508-04/P

12C509 Echivalent: PIC12C509-04/P

1488 Echivalent: MC1488N-MBR

1489 Echivalent: MC1489N-MBR

14DN487 I.C.

15/80H THYR.+D 900V 5.5A TRACE

15/85R THYR.+D 900V 5.5A RETRACE

1580H Echivalent: 15/80H

16450 Echivalent: TL16C450N-MBR

16C450 Echivalent: TC16C450N-MBR

16C550A UART CMOS 8MHz 250ns

17020H Echivalent: 17088H

17022 Echivalent: 17088H

17023 Echivalent: 17089R

17030H Echivalent: 17088H

17031R Echivalent: 17089R

17032H Echivalent: 17088H

17033R Echivalent: 17089R

17035R THYR.+D 700V 3A RETRACE

17036H Echivalent: 17088H

17052H Echivalent: 17088H

17053R Echivalent: 17089R

17058H Echivalent: 17088H

17064 Echivalent: 17088H

17065 Echivalent: 17089R

17074 Echivalent: 17088H

17075 Echivalent: 17089R

17088H THYR.+D 720V 3A TRACE

17089R THYR.+D 720V 3A RETRACE

17106 Echivalent: 17127

17127 THYR. 800V 3.5A

17150 Echivalent: 17127

17160 Echivalent: 15/80H

18.000 HC49 QUARZE 18.000.000 MHz

18.430 HC18 QUARZE 18.430.000 MHz

18022 Echivalent: 15/80H

19373L 6.3A SICHERUNG TR5 6.3A FLINK

1K60 GE-D 45V 35mA/0.2Ap =AA11

1MB-80-MBR DRAM 1MBx1 80ns CMOS

1N1188-MBR SI-D 400V 35A/480Ap

1N1188R SI-D 400V 35A/480Ap REVER

1N3891 Echivalent: 1N3893

1N3893 SI-D 400V 12A <200ns

1N3942 SI-D 100V 2A/30Ap

1N4001-ITT Echivalent: 1N4007-MBR

1N4001/DIV Echivalent: 1N4007/DIV

1N4002-ITT Echivalent: 1N4007-MBR

1N4002/DIV Echivalent: 1N4007/DIV

1N4003-ITT Echivalent: 1N4007-MBR

1N4003/DIV Echivalent: 1N4007/DIV

1N4004-MBR Echivalent: 1N4007-MBR

1N4004/DIV Echivalent: 1N4007/DIV

1N4005-MBR Echivalent: : 1N4007-MBR

1N4005/DIV Echivalent: 1N4007/DIV

1N4006-ITT Echivalent: 1N4007-MBR

1N4006/DIV Echivalent: 1N4007/DIV

1N4007-MBR SI-D 1000V 1A/50Ap

1N4007-PHI Echivalent: 1N4007/DIV

1N4007/DIV SI-D 1000V 1A/50Ap

1N4008 Echivalent: EM516

1N4010 Echivalent: 0.4W 6V2

1N4148-MBR SI-D 75V 0.15A/0.5Ap 4ns

1N4148AMO-MBR Echivalent: 1N4148-MBR

1N4148TR-MBR Echivalent: 1N4148-MBR

1N4149-MBR SI-D 100V 0.15A 0.5W 4ns

1N4150 Echivalent: BAV10-MBR

1N4151 Echivalent: 1N4148-MBR

1N4152 Echivalent: 1N4148-MBR

1N4153 Echivalent: 1N4148-MBR

1N4154 Echivalent: 1N4151

1N4370 Echivalent: 0.4W 2V4

1N4371A Echivalent: 0.4W 2V7

1N4372 Echivalent: 0.4W 3V

1N4446 Echivalent: 1N4148-MBR

1N4447 Echivalent: 1N4148-MBR

1N4448 Echivalent: 1N4148-MBR

1N4449 Echivalent: 1N4148-MBR

1N4450 Echivalent: 1N4148-MBR

1N4451 Echivalent: 1N4148-MBR

1N4454 Echivalent: 1N4148-MBR

1N4728 Echivalent: 1.3W 3V3

1N4729 Echivalent: 1.3W 3V6

1N4730 Echivalent: 1.3W 3V9

1N4731 Echivalent: 1.3W 4V3

1N4732 Echivalent: 1.3W 4V7

1N4733 Echivalent: 1.3W 5V1

1N4734 Echivalent: 1.3W 5V6

1N4735 Echivalent: 1.3W 6V2

1N4736 Echivalent: 1.3W 6V8

1N4737 Echivalent: 1.3W 7V5

1N4738 Echivalent: 1.3W 8V2

1N4739 Echivalent: 1.3W 9V1

1N4740 Echivalent: 1.3W 10V

1N4741 Echivalent: 1.3W 11V

1N4742 Echivalent: 1.3W 12V

1N4743 Echivalent: 1.3W 13V

1N4744 Echivalent: 1.3W 15V

1N4745 Echivalent: 1.3W 16V

1N4746 Echivalent: 1.3W 18V

1N4747 Echivalent: 1.3W 20V

1N4748 Echivalent: 1.3W 22V

1N4749 Echivalent: 1.3W 24V

1N4750 Echivalent: 1.3W 27V

1N4751 Echivalent: 1.3W 30V

1N4752 Echivalent: 1.3W 33V

1N4821 Echivalent: BY227S-DIO

1N4933 Echivalent: 1N4936

1N4934 Echivalent: 1N4936

1N4935 Echivalent: 1N4936

1N4936 SI-D 400V 1A/50Ap 150ns

1N4937 SI-D 600V 1A/50Ap 250ns

1N4942 Echivalent: 1N4936

1N4944 Echivalent: 1N4936

1N4946 Echivalent: 1N4937

1N5060 Echivalent: 1N5062-MBR

1N5061 Echivalent: 1N5062-MBR

1N5062-MBR SI-D 800V 2A/50Ap 6us

1N5221A Echivalent: 0.4W 2V4

1N5223A Echivalent: 0.4W 2V7

1N5225A Echivalent: 0.4W 3V

1N5226A Echivalent: 0.4W 3V3

1N5227A Echivalent: 0.4W 3V6

1N5227B Echivalent: 0.4W 3V6

1N5228A Echivalent: 0.4W 3V9

1N5229A Echivalent: 0.4W 4V3

1N5230A Echivalent: 0.4W 4V7

1N5231A Echivalent: 0.4W 5V1

1N5232A Echivalent: 0.4W 5V6

1N5234A Echivalent: 0.4W 6V2

1N5235A Echivalent: 0.4W 6V8

1N5236A Echivalent: 0.4W 7V5

1N5237A Echivalent: 0.4W 8V2

1N5239A Echivalent: 0.4W 9V1

1N5240A Echivalent: 0.4W 10V

1N5241A Echivalent: 0.4W 11V

1N5242A Echivalent: 0.4W 12V

1N5243A Echivalent: 0.4W 13V

1N5244A Echivalent: 0.4W 14V

1N5244B Echivalent: 0.4W 14V

1N5245A Echivalent: 0.4W 15V

1N5246A Echivalent: 0.4W 16V

1N5248A Echivalent: 0.4W 18V

1N5250A Echivalent: 0.4W 20V

1N5251A Echivalent: 0.4W 22V

1N5252A Echivalent: 0.4W 24V

1N5254A Echivalent: 0.4W 27V

1N5256A Echivalent: 0.4W 30V

1N5257A Echivalent: 0.4W 33V

1N5258A Echivalent: 0.4W 36V

1N5259A Echivalent: 0.4W 39V

1N5260A Echivalent: 0.4W 43V

1N5261A Echivalent: 0.4W 47V

1N5262A Echivalent: 0.4W 51V

1N5263A Echivalent: 0.4W 56V

1N5265A Echivalent: 0.4W 62V

1N5266A Echivalent: 0.4W 68V

1N5267A Echivalent: 0.4W 75V

1N5268A Echivalent: 0.4W 82V

1N5271 Echivalent: 0.4W 100V

1N5279A Echivalent: 0.4W 180V

1N5333B-MBR Dioda Zener 5W 3V3

1N5334B-MBR Dioda Zener 5W 3V6

1N5335B-MBR Dioda Zener 5W 3V9

1N5336B-MBR Dioda Zener 5W 4V3

1N5337B-MBR Dioda Zener 5W 4V7

1N5338B-MBR Dioda Zener 5W 5V1

1N5339B-MBR Dioda Zener 5W 5V6

1N5341B-MBR Dioda Zener 5W 6V2

1N5342B-MBR Dioda Zener 5W 6V8

1N5343B-MBR Dioda Zener 5W 7V5

1N5344B-MBR Dioda Zener 5W 8V2

1N5346B-MBR Dioda Zener 5W 9V1

1N5347B-MBR Dioda Zener 5W 10V

1N5348B-MBR Dioda Zener 5W 11V

1N5349B-MBR Dioda Zener 5W 12V

1N5350B-MBR Dioda Zener 5W 13V

1N5351B-MBR Dioda Zener 5W 14V

1N5352B-MBR Dioda Zener 5W 15V

1N5353B-MBR Dioda Zener 5W 16V

1N5354B-MBR Dioda Zener 5W 17V

1N5355B-MBR Dioda Zener 5W 18V

1N5356B-MBR Dioda Zener 5W 19V

1N5357B-MBR Dioda Zener 5W 20V

1N5358B-MBR Dioda Zener 5W 22V

1N5359B-MBR Dioda Zener 5W 24V

1N5360B-MBR Dioda Zener 5W 25V

1N5361B-MBR Dioda Zener 5W 27V

1N5362B-MBR Dioda Zener 5W 28V

1N5363B-MBR Dioda Zener 5W 30V

1N5364B-MBR Dioda Zener 5W 33V

1N5365B-MBR Dioda Zener 5W 36V

1N5366B-MBR Dioda Zener 5W 39V

1N5367B-MBR Dioda Zener 5W 43V

1N5368B-MBR Dioda Zener 5W 47V

1N5369B-MBR Dioda Zener 5W 51V

1N5370B-MBR Dioda Zener 5W 56V

1N5372B-MBR Dioda Zener 5W 62V

1N5374B-MBR Dioda Zener 5W 75V

1N5375B-MBR Dioda Zener 5W 82V

1N5377B-MBR Dioda Zener 5W 91V

1N5378B-MBR Dioda Zener 5W 100V

1N5379B-MBR Dioda Zener 5W 110V

1N5380B-MBR Dioda Zener 5W 120V

1N5381B-MBR Dioda Zener 5W 130V

1N5382B-MBR Dioda Zener 5W 140V

1N5383B-MBR Dioda Zener 5W 150V

1N5384B-MBR Dioda Zener 5W 160V

1N5385B-MBR Dioda Zener 5W 170V

1N5386B-MBR Dioda Zener 5W 180V

1N5387B-MBR Dioda Zener 5W 190V

1N5388B-MBR Dioda Zener 5W 200V

1N5391 Echivalent: 1N5399-MBR

1N5392 Echivalent: 1N5399-MBR

1N5393 Echivalent: 1N5399-MBR

1N5394 Echivalent: 1N5399-MBR

1N5395 Echivalent: 1N5399-MBR

1N5396 Echivalent: 1N5399-MBR

1N5397 Echivalent: 1N5399-MBR

1N5398 Echivalent: 1N5399-MBR

1N5399-DIO Echivalent: 1N5399-MBR

1N5399-MBR SI-D 1000V 1.5A/10Ap

1N5399/DIV Echivalent: : 1N5399-MBR

1N5400 Echivalent: 1N5408-MBR

1N5401 Echivalent: 1N5408-MBR

1N5402 Echivalent: 1N5408-MBR

1N5403 Echivalent: 1N5408-MBR

1N5404 Echivalent: 1N5408-MBR

1N5405 Echivalent: 1N5408-MBR

1N5406 Echivalent: 1N5408-MBR

1N5407 Echivalent: 1N5408-MBR

1N5408-MBR SI-D 1000V 3A/200Ap

1N5408/DIV SI-D 1000V 3A/200Ap

1N5449A-MOT C-D 30V 2.6-27pF 0.4W

1N5624 Echivalent: BY255-MBR

1N5625 Echivalent: BY255-MBR

1N5626 Echivalent: BY255-MBR

1N5627 Echivalent: BY255-MBR

1N5639 Echivalent: 1.5KE39P-MBR

1N5649A Echivalent: 1.5KE47P-MBR

1N5657A Echivalent: 1.5KE100P-MBR

1N5711-ST SB-D 70V 15mA 0.1ns

1N5817 Echivalent: 1N5819

1N5818 Echivalent: 1N5819

1N5819 SB-D 40V 1A/25Ap

1N5820 Echivalent: 1N5822

1N5821 Echivalent: 1N5822

1N5822 SB-D 40V 3A/80Ap STKY

1N5907 Echivalent: 1N5908-MBR

1N5908-MBR UNIDIR TRANSIL 5V 1500W

1N5913A Echivalent: 1.3W 3V3

1N5914A Echivalent: 1.3W 3V6

1N5915A Echivalent: 1.3W 3V9

1N5916A Echivalent: 1.3W 4V3

1N5917A Echivalent: 1.3W 4V7

1N5918A Echivalent: 1.3W 5V1

1N5919A Echivalent: 1.3W 5V6

1N5920A Echivalent: 1.3W 6V2

1N5921A Echivalent: 1.3W 6V8

1N5922A Echivalent: 1.3W 7V5

1N5923A Echivalent: 1.3W 8V2

1N5924A Echivalent: 1.3W 9V1

1N5925A Echivalent: 1.3W 10V

1N5926A Echivalent: 1.3W 11V

1N5927A Echivalent: 1.3W 12V

1N5928A Echivalent: 1.3W 13V

1N5929A Echivalent: 1.3W 15V

1N5930A Echivalent: 1.3W 16V

1N5931A Echivalent: 1.3W 18V

1N5932A Echivalent: 1.3W 20V

1N5933A Echivalent: 1.3W 22V

1N5934A Echivalent: 1.3W 24V

1N5935A Echivalent: 1.3W 27V

1N5936A Echivalent: 1.3W 30V

1N5937A Echivalent: 1.3W 33V

1N5938A Echivalent: 1.3W 36V

1N5939A Echivalent: 1.3W 39V

1N5940A Echivalent: 1.3W 43V

1N5941A Echivalent: 1.3W 47V

1N5942A Echivalent: 1.3W 51V

1N5943A Echivalent: 1.3W 56V

1N5944A Echivalent: 1.3W 62V

1N5945A Echivalent: 1.3W 68V

1N5946A Echivalent: 1.3W 75V

1N5947A Echivalent: 1.3W 82V

1N5948A Echivalent: 1.3W 91V

1N5949A Echivalent: 1.3W 100V

1N5950A Echivalent: 1.3W 110V

1N5951A Echivalent: 1.3W 120V

1N5952A Echivalent: 1.3W 130V

1N5953A Echivalent: 1.3W 150V

1N5954A Echivalent: 1.3W 160V

1N5955A Echivalent: 1.3W 180V

1N5956A Echivalent: 1.3W 200V

1N5985A Echivalent: 0.4W 2V4

1N5986A Echivalent: 130.34048

1N5987A Echivalent: 0.4W 3V

1N5988A Echivalent: 0.4W 3V3

1N5989A Echivalent: 0.4W 3V6

1N5990A Echivalent: 0.4W 3V9

1N5991A Echivalent: 0.4W 4V3

1N5992A Echivalent: 0.4W 4V7

1N5993A Echivalent: 0.4W 5V1

1N5994A Echivalent: 0.4W 5V6

1N5995A Echivalent: 0.4W 6V2

1N5996A Echivalent: 0.4W 6V8

1N5997A Echivalent: 0.4W 7V5

1N5998A Echivalent: 0.4W 8V2

1N5999A Echivalent: 0.4W 9V1

1N60 Echivalent: 1K60

1N6000A Echivalent: 0.4W 10V

1N6001A Echivalent: 0.4W 11V

1N6002A Echivalent: 0.4W 12V

1N6003A Echivalent: 0.4W 13V

1N6004A Echivalent: 0.4W 15V

1N6005A Echivalent: 0.4W 16V

1N6006A Echivalent: 0.4W 18V

1N6007A Echivalent: 0.4W 20V

1N6008A Echivalent: 0.4W 22V

1N6009A Echivalent: 0.4W 24V

1N6010A Echivalent: 0.4W 27V

1N6011A Echivalent: 0.4W 30V

1N6012A Echivalent: 0.4W 33V

1N6013A Echivalent: 0.4W 36V

1N6014A Echivalent: 0.4W 39V

1N6015A Echivalent: 0.4W 43V

1N6016A Echivalent: 0.4W 47V

1N6017A Echivalent: 0.4W 51V

1N6018A Echivalent: 0.4W 56V

1N6019A Echivalent: 0.4W 62V

1N6020A Echivalent: 0.4W 68V

1N6021A Echivalent: 0.4W 75V

1N6022A Echivalent: 0.4W 82V

1N6263-MBR SB-D 60V 15mA 0.1ns

1N645 SI-D 225V 0.4A

1N647 SI-D 400V 0.4A/15Ap

1N746 Echivalent: 0.4W 3V3

1N747 Echivalent: 0.4W 3V6

1N748 Echivalent: 0.4W 3V9

1N749 Echivalent: 0.4W 4V3

1N750 Echivalent: 0.4W 4V7

1N751 Echivalent: 0.4W 5V1

1N752 Echivalent: 0.4W 5V6

1N753 Echivalent: 0.4W 6V2

1N754 Echivalent: 0.4W 6V8

1N755 Echivalent: 0.4W 7V5

1N756A Echivalent: 0.4W 8V2

1N757 Echivalent: 0.4W 9V1

1N758 Echivalent: 0.4W 10V

1N759 Echivalent: 0.4W 12V

1N821 Echivalent: 1N823

1N823 REF-DIODE 6V2 5PPM 5%

1N825 Echivalent: 1N827

1N827 REF-DIODE 6V2 1PPM 5%

1N914-MBR SI-D 100V 75mA 0.5W 4ns

1N914/DIV SI-D 100V 75mA 0.5W 4ns

1N916 Echivalent: 1N914-MBR

1N958A Echivalent: 0.4W 7V5

1N959A Echivalent: 0.4W 8V2

1N960A Echivalent: 0.4W 9V1

1N961A Echivalent: 0.4W 10V

1N962A Echivalent: 0.4W 11V

1N963A Echivalent: 0.4W 12V

1N964A Echivalent: 0.4W 13V

1N965A Echivalent: 0.4W 15V

1N966A Echivalent: 0.4W 16V

1N967A Echivalent: 0.4W 18V

1N968A Echivalent: 0.4W 20V

1N969A Echivalent: 0.4W 22V

1N970A Echivalent: 0.4W 24V

1N971A Echivalent: 0.4W 27V

1N972A Echivalent: 0.4W 30V

1N973A Echivalent: 0.4W 33V

1N974A Echivalent: 0.4W 36V

1N975A Echivalent: 0.4W 39V

1N976A Echivalent: 0.4W 43V

1N977A Echivalent: 0.4W 47V

1N978A Echivalent: 0.4W 51V

1N979A Echivalent: 0.4W 56V

1N980A Echivalent: 0.4W 62V

1N981A Echivalent: 0.4W 68V

1N982A Echivalent: 0.4W 75V

1N983A Echivalent: 0.4W 82V

1N991A Echivalent: 0.4W 180V

1SS119 Echivalent: 1N4148-MBR

1SS131 Echivalent: 1N4148-MBR

1SS244 Echivalent: BAV21-MBR

1W 12V Echivalent: 1.3W 12V

1W 15V Echivalent: 1.3W 15V

1W 5V6 Echivalent: 1.3W 5V6

A109D Echivalent: UA741CN8-MBR

A110D Echivalent: UA710-14P

A1818 Echivalent: LM1818

A202D Echivalent: TDA1002/DIV

A210E Echivalent: TBA810AS/DIV

A210K Echivalent: : TBA810AS/DIV

A211D Echivalent: TAA611

A220D Echivalent: TBA120S/DIV

A223D Echivalent: TBA120U/DIV

A225D Echivalent: TDA1047-SIE

A232D Echivalent: TDA2532/DIV

A240 Echivalent: TDA440/DIV

A241D Echivalent: TDA2541-MBR

A244D Echivalent: TCA440-MBR

A255D Echivalent: TDA2593/DIV

A273 Echivalent: TCA730A

A274D Echivalent: TCA740A/DIV

A281D Echivalent: TAA981/DIV

A283D Echivalent: TDA1083/DIV

A290D Echivalent: MC1310P/DIV

A301 Echivalent: TCA205A/DIV

A4100D Echivalent: TDA4100

A4510 Echivalent: TCA4510

AA112 Echivalent: : AA116

AA114 Echivalent: 1K60

AA116 GE-D 30V 30mA DEM,N-OHM

AA117 GE-D 115V 50mA/0.5Ap

AA118 Echivalent: OA95

AA119 Echivalent: 1K60

AA132 Echivalent: AA133

AA133 GE-D 130V 50mA <1.3V

AA137 Echivalent: OA90

AA139 Echivalent: 1K60

AA143 GE-D 30V 60mA/0.2Ap 80mW

AA144 GE-D 100V 45mA/0.15Ap 80m

AAZ17 GE-D 75V 0.14A/0.25Ap <35

AAZ18 GE-D 20V 0.13A/0.3Ap <70n

AC03D-NEC TRIAC 400V 3A 15mA

AC121 GE-P 20V 0.3A 0.9W

AC122 GE-P 30V 0.2A 0.225W

AC125 GE-P 32V 0.2A

AC126 GE-P 32V 0.2A 0.5W

AC127 GE-N 32V 0.5A

AC128 GE-P 32V 1A 1W

AC128/176K GE N/P PAIRED, COOLED

AC128K GE-P 32V 1A 1W

AC131 GE-P 30V 1A 0.75W

AC132 GE-P 32V 0.2A 0.5W

AC138 GE-P 32V 1.2A 0.22W 1.5MH

AC141K GE-N 32V 1.2A 1W

AC151 GE-P 32V 0.2A 0.9W

AC153 GE-P 32V 2A 1W

AC153K GE-P 32V 2A 1W

AC176K GE-N 32V 1A 1W

AC180 GE-P 32V 1.5A 0.3W 1MHz

AC181 Echivalent: AC187

AC181K Echivalent: AC187K

AC187 GE-N 25V 1A 1W

AC187/188 Echivalent: AC187/188K

AC187/188K GE N/P PAIRED,COOLED

AC187K GE-N 25V 1A 1W COOLED

AC188 GE-P 25V 1A 1W

AC188K GE-P 25V 1A 1W

ACP2371DIP-ITT AUDIO PROCESSOR+ A/D&D/A

ACVP2205-ITT ADAPTIVE COMBFILTER VIDEO

AD133 GE-P 50V 15A 36W

AD136 GE-P 40V 10A 11W

AD139 GE-P 32V 3.5A 13W

AD142 Echivalent: : N.L.A.

AD143 Echivalent: AD142

AD148 GE-P 32V 3.5A 13.5W

AD149 GE-P 50V 3.5A 27W

AD150 Echivalent: AD149

AD161 GE-N 32V 1A 6W

AD161/162 GE N/P 32V 1A 6W PAIRED

AD162 GE-P 32V 1A 6W

AD164 Echivalent: AD162

AD165 GE-N 25V 1A 6W

AD166 GE-P 60V 5A 27.5W

AD1851N-AD 1*16B DAC 0.004%THD 16*FS

AD301 Echivalent: LM301AP-MBR

AD532JD-MBR 4-QUADR. ANAL MULTPL

AD534JH-AD 4-QUADR. ANAL MULTPL 1MHZ

AD536AKH-AD TRUE RMS-to-DC CONV 5mV A

AD558JN-AD DAC 1*8B V-OUT 1/2LSB

AD574AJN-AD ADC 1*12B 35us 1LSB

AD584JN-AD V-REF 2.5,5,7.5,10V 30PPM

AD590H Echivalent: AD590JH

AD590JH-MBR TEMP.Senzor 1uA/K 1.5'ACC

AD633JN-AD 4-QUADR. ANAL.MULTPL LOWC

AD636JN-MBR TRUE RMS-to-DC CONV .5mV

AD654JN-AD VOLT TO FREQ CONV 500kHz

AD711JN-AD OPAMP 1* 16V/us 3MHz AUDI

AD712JN-AD OPAMP 2* 16V/us 3MHz AUDI

AD736JN-AD TRUE RMS-to-DC CONV 5mV A

AD7523JN-MBR M-DAC 1*8B I-OUT 200ns

AD7524JN-MBR M-DAC 1*8B I-OUT 1/2LSB

AD7528JN-AD M-DAC 2* 8B I-OUT 1/2LSB

AD7569JN-AD CODEC 1*8B 500KSPS

AD7574JN-AD ADC 1*8B 15us CPU-COMP

AD7576JN-AD ADC 1x8 BIT 30us 1LSB

AD827JN-AD OPAMP 2* 300V/us 50MHz

AD847JN-MBR OPAMP

ADC0800PCD-MBR ADC 8B

ADC0803LCN-MBR ADC 8B 73us 0.5LSB -25/+8

ADC0804LCN-MBR ADC 8B 73us 1LSB -25/+85'

ADC0808CCN Echivalent: ADC0808N-MBR

ADC0808N-MBR ADC 8B 8-CHN 100uS

ADC0809CCN-MBR ADC 8B ACQUISITION PORT

ADC0820CCN-MBR ADC 8B

ADC0831CCN-MBR ADC 8-BIT

ADC2300-ITT Echivalent: : N.L.A.

ADC2300E Echivalent: ADC2300-ITT

ADC2301EDIP-IT Echivalent: 209.63161

ADC2310E Echivalent: : N.L.A.

ADC800 Echivalent: ADC0800PCD-MBR

ADC804 Echivalent: ADC0804LCN-MBR

ADC808 Echivalent: ADC0808N-MBR

ADC809 Echivalent: ADC0809CCN-MBR

ADC820 Echivalent: ADC0820CCN-MBR

ADM233LJN Echivalent: MAX233CPP

AF106 GE-P 25V 10mA 220MHz VHF

AF109R GE-P 20V 12mA 260MHz VHF

AF115 Echivalent: AF125

AF116 Echivalent: AF126

AF117 Echivalent: AF127

AF118 GE-P 70V 30mA 375mW 125MH

AF121 GE-P 25V 10mA 270MHz

AF124 Echivalent: : N.L.A.

AF125 GE-P 32V 10mA 75MHz

AF126 Echivalent: : N.L.A.

AF127 GE-P 32V 10mA 75MHz

AF139 GE-P 20V 10mA 550MHz

AF178 Echivalent: AF106

AF200 GE-P 25V 10mA 0.145W

AF201 GE-P 25V 10mA 0.145W

AF239S GE-P 15V 10mA 700MHz

AF279 GE-P 15V 10mA 60mW 780MHz

AF279S GE-P 20V 10mA 0.6W

AF280 GE-P UHF MIXER 550MHz

AF306 GE-P 25V 15mA 60mW 500MHz

AF367 GE-P 153V 10mA UHF 800MHz

AF379 GE-P UHF 1250MHz

AL102 GE-P 130V 6A 30W

AL103 Echivalent: AL102

AL112 GE-P 130V 6A 10W

AL113 Echivalent: AL112

AM26LS29-MBR 4xRS423 LINE DRIVER

AM26LS30-MBR 4xRS422/423 DRIVER

AM26LS31-MBR 4xRS422/485 DR. 1xENABLE

AM26LS32-MBR 4xRS422/3 RECEIVER Uin 7V

AM26LS33-MBR 4xRS422/3 RECEIVER 15V

AMF 10A AUTO SICHERUNGEN 10A

AMF 15A AUTO SICHERUNG 15A

AMF 20A AUTO SICHERUNG 20A

AMF 25A AUTO SICHERUNG 25A

AMF 30A AUTO SICHERUNG 30A

AMF 3A AUTO SICHERUNG 3AMP

AMF 4A AUTO SICHERUNG 4A

AMF 5A AUTO SICHERUNG 5AMP

AMF 7.5A AUTO SICHERUNG 7.5A

AMF ASS104 ASS. 20X5 PCS AUTO SICHER

AMF ASS90 BOX 18X5 PCS.AUTO SICHERU

AMFM 10A MINI KFZ-SICHERUNG 10A

AMFM 15A MINI KFZ-SICHERUNG 15A

AMFM 20A MINI KFZ-SICHERUNG 20A

AMFM 25A MINI KFZ-SICHERUNG 25A

AMFM 2A MINI KFZ-SICHERUNG 2A

AMFM 30A MINI KFZ-SICHERUNG 30A

AMFM 3A MINI KFZ-SICHERUNG 3A

AMFM 4A MINI KFZ-SICHERUNG 4A

AMFM 5A MINI KFZ-SICHERUNG 5A

AMFM 7.5A MINI KFZ-SICHERUNG 7.5A

AMU2481DIP-ITT AUDIO MIXER

AMU2481PLC-ITT AUDIO MIXER

AN101 FM NOISE CANCELLER

AN103 PHASE LOCKED LOOP

AN115 STEREO DECODER 14p

AN1319 Echivalent: LM319N-MBR

AN1324 Echivalent: LM324N-MBR

AN1358 Echivalent: MC1458N-MBR

AN1431T Echivalent: LM317LZ-MBR

AN1458 Echivalent: MC1458N-MBR

AN1555 Echivalent: NE555N-MBR

AN1741 Echivalent: UA741CN8-MBR

AN2010S-MAT Echivalent: : N.L.A.

AN210B-MAT AM/FM-ZF 14p

AN211 STEREO DECODER 14p

AN214Q AF AMPLIFIER 18V 4.5W

AN217BB Echivalent: AN217P

AN217P FM-AM RECEIVER SYSTEM

AN234 Echivalent: : N.L.A.

AN236 TV SUB CARRIER

AN240P-MAT TV SOUND IF AMPL.

AN241 Echivalent: CA3065E-MBR

AN252 Echivalent: AN7140

AN253P-MAT Echivalent: : N.L.A.

AN260 Echivalent: : N.L.A.

AN262 VCR PLAY/REC AMPL.16p

AN274 Echivalent: AN374P-MAT

AN277B AM/FM IF AMPLIFIER

AN278 FM-ZF 9p

AN295 Echivalent: : N.L.A.

AN301 Echivalent: : N.L.A.

AN302 VTR-LUM.+VIDEO-AGC

AN303-MAT Echivalent: : N.L.A.

AN305 VTR COLOUR APC 16p

AN3130-MAT RF MODULATOR

AN315 Echivalent: : N.L.A.

AN316 VTR DROP OUT COMPENS.

AN318 VTR SERVO

AN3211K-MAT TV/VIDEO SIGNAL PROCESSOR

AN3215NK-MAT TV/VIDEO SIGNAL AMPLIFIER

AN3230K-MAT I.C.

AN3231K-MAT VHS NTSC PROCESSOR

AN3236K-MAT IC

AN3248NK-MAT VIDEO SIGNAL PROCESSOR

AN3310K-MAT TAPE HEAD AMPLIFIER

AN3311K-MAT TAPE HEAD AMPLIFIER

AN3311S-MAT VIDEO TAPE HEAD AMPLIFIER

AN3312-MAT I.C.

AN3313-MAT TAPE HEAD AMPLIFIER

AN3320K-MAT VIDEO RECORD AMPL.4 HEAD

AN3321K-MAT VIDEO PLAYBACK/REC. CIRCU

AN3330K-MAT IC

AN3331K-MAT IC.

AN3334K-MAT IC

AN337 VTR CHROMA

AN3386NK I.C.

AN345V-MAT TV/VIDEO SIGNAL PROCESSOR

AN3500FBP-MAT IC

AN3553FBP-MAT Echivalent: : N.L.A.

AN360 AUDIO AMPL. SINGLE-CHAN.

AN362-MAT STEREO DEC./LED DRIVE 16p

AN363N-MAT STEREO DECODER 16p

AN366 Echivalent: : N.L.A.

AN374P-MAT Echivalent: : N.L.A.

AN377 Echivalent: : N.L.A.

AN3792-MAT VTR CYLINDER SERVO INTF.

AN3794-MAT CAPSTAN SERVO-INTERFACE

AN3795N-MAT CAPSTAN SERVO INTERFACE

AN3812K-MAT IC

AN3814K-MAT VTR MOTOR DRIVER 1.5A

AN3821K-MAT VTR CAPST.DRIVER 1.5A

AN3822 Echivalent: AN3821K-MAT

AN3824K Echivalent: 8305073824

AN3826NK-MAT CAPSTAN-MOTOR DRIVER 1A

AN3990K-MAT VCR I.C.

AN3991K-MAT VIDEO SIGNAL PROCESSOR

AN3991NS-MAT VIDEO SIGNAL PROCESSOR

AN4250 Echivalent: LM4250CN-MBR

AN4558 Echivalent: MC4558CN-MBR

AN5010-MAT CHANNEL SELECTION

AN5011 TV CHANNEL SELECTION

AN5020-MAT RC RECEIVER PREAMPLIFIER

AN5026K-MAT REMOTE CONTROL RECEIVER

AN5033-MAT Echivalent: : N.L.A.

AN5034 TV TUNER CONTROL

AN5070-MAT TV TUNER SWITCH+ 31V-REG

AN5071-MAT TV TUNER SWITCH+ 31V-REG

AN5111 Echivalent: : N.L.A.

AN5120N-MAT CTV VIDEO IF AMPLIFIER

AN5130 Echivalent: AN5132-MAT

AN5132-MAT CTV/VID IF-AMP+DET+AGC+AF

AN5135NK-MAT VIDEO+FM IF-AMP+ AGC+ AFC

AN5136K VIDEO/SOUND IF AMPLIFIER

AN5138NK-MAT CTV/VID IF-AMP+PLL+AGC+AF

AN5150N-MAT TV IF-AMP+AFC NEG. POLARI

AN5151N-MAT TV IF-AMP+AFC POS. POLARI

AN5210 TV/VIDEO SOUND CIRCUIT

AN5215-MAT TV SOUND IF+FM DETECTOR

AN5222-MAT TV SOUND IF+DET+AF OUTPUT

AN5250-MAT TV SOUND IF+DAT+ 2W AMP

AN5255-MAT Echivalent: AN5256-MAT

AN5256-MAT TV SOUND IF+DET+ 2W AMP

AN5260 Echivalent: : N.L.A.

AN5262-MAT TV DC VOLUME CONTROL

AN5265-MAT TV DC VOLUME CONTROL

AN5315-MAT Echivalent: : N.L.A.

AN5352N-MAT CHAR.+ PATTERN INTERFACE

AN5355-MAT TELETEXT CIRCUIT

AN5410 Echivalent: AN5411-MAT

AN5411-MAT CTV DEFLECT.SIGNAL PRC.

AN5421N-MAT TV SYNCHRON. SIGNAL. DET.

AN5431N-MAT VIDEO DEFLECTION CIRCUIT

AN5435-MAT CTV DEFLECTION SIGN.PROC.

AN5436N-MAT CTV DEFLECTION SIGN.PROC.

AN5512-MAT TV VERTICAL OUTPUT

AN5515-MAT VERT DEFL OUTPUT

AN5520-MAT Echivalent: : N.L.A.

AN5521-MAT TV VERT. DEFLECTION OUTPU

AN5532-MAT TV VERTICAL DEFL. CIRCUIT

AN5534-MAT VERTICAL PROC./DEFLECT.

AN5551-MAT GRAPHIC/VIDEO SUPPORT CIR

AN5560-MAT 50 & 60Hz INDENTIFICATION

AN5601K-MAT TV PAL/NTSC CHROMA PROC.

AN5612N-MAT CTV PAL/SECAM CHROMA 60%

AN5613-MAT COLOR DECODER CIRCUIT

AN5615-MAT TV/VIDEO SIGNAL PROCESSOR

AN5620 TV PAL CHROMA

AN5622-MAT CTV PAL CHROMA SIGN.PROC.

AN5625N-MAT PAL/NTSC CHROMA SIGN.PROC

AN5630N-MAT Echivalent: : N.L.A.

AN5633K-MAT PAL/SECAM PROCESSOR

AN5635N-MAT CTV SECAM CHROMA SIGN.PRO

AN5640-MAT CTV PAL/SECAM/NTSC DETECT

AN5650-MAT TV/VIDEO SYNC. CIRCUIT

AN5700-MAT TV TUNER BAND SWITCH

AN5701 TV TUNER (BAND) 9p

AN5710 TV TUNER

AN5712 Echivalent: : N.L.A.

AN5720 VIDEO DETECT.AMP.+DEM. 9p

AN5730 SOUND IF AMP.DETECTOR 7p

AN5732 TV SOUND IF AMP. 7p

AN5733-MAT DUAL AF-ATTENUATOR

AN5742-MAT SINGLE CHAN. AMP. 12V 0.3

AN5750-MAT TV HORZ. SYNCHR. + OSC.

AN5753-MAT TV HOR.DEFLECTION SIGN.PR

AN5760-MAT TV VERT.DEFLECTION SIGN.P

AN5763-MAT TV VERT.DEFLECT.DRIVER

AN5766K-MAT GRAPHIC/VIDEO SUPPORT CIR

AN5790N-MAT CRT DISPLAY DRIVER

AN5791-MAT CRT DISPLAY PHASE SHIFT I

AN5835-MAT ELECTRONIC AUDIO CONTROL

AN5836-MAT DC VOLUME+TONE CONTROL

AN5860-MAT ANALOG SWITCH

AN5862K-MAT ANALOG SWITCH CTV-IC

AN5900-MAT PWM REGULATOR 15kHz

AN608P-MAT WIDEB. AMP. 10MHz COM.PHA

AN612 BALANCE MODULATOR 7p

AN6130N-MAT FM NOISE SUPPRESSOR 18p

AN6135 HIFI POP NOISE SUPPR. 9p

AN6136 Echivalent: : N.L.A.

AN620 I.C.

AN6209 TV IC

AN6247NT-MAT RECORDER AUTOREVERSE-CONT

AN6249 Echivalent: AN6250

AN6250 REC.AUTO-REVERSE CONTR.7p

AN6263N-MAT RECORDER, PAUSE DETECTION

AN6270 MOTOR CONTROLER 16p

AN6291-MAT DUAL dBx NOISE REDUCT. CI

AN6306-MAT AUDIO TAPE RECORD. CIRCUI

AN6307-MAT TAPE HEAD AMPLIFIER

AN6310 VTR LIMINANCE PROCESS

AN6320N-MAT VTR HEAD AMPLIFIER

AN6326N-MAT VCR HEAD AMPLIFIER

AN6327-MAT TV/VIDEO SIGNAL PROCESSOR

AN6330-MAT VIDEO HEAD AMPLIFIER 14p

AN6331 Echivalent: AN6332

AN6332 VTR PLAY-BACK PROC. 28p

AN6340 VIDEO SERVO

AN6341N-MAT VTR CAPSTAN SERVO 16p

AN6342N-MAT VTR FREQ.DEVIDER

AN6344 VTR HEAD SERVO CONTR.

AN6345 FG DIVIDER 16p

AN6346 VC SERVO CILIND.INTERFACE

AN6350 VTR HEAD SERVO CONTR.

AN6352 VTR PITCH CONTROL

AN6356N-MAT VCR SERVO CYLINDER INTF

AN6359N-MAT VCR SERVO 20p

AN6360-MAT VTR COLOUR ACC CIRCUIT

AN6362-MAT VTR COLOR-AFC (NTSC-PAL)

AN6363 VIDEO COLOR-AFC (PAL)

AN6367NK-MAT VTR COLOR (PAL,SECAM,NTSC

AN6368-MAT VTR PAL/SECAM SIGNAL DET.

AN6371-MAT VTR COLOR APC (PAL)

AN6387-MAT Echivalent: : N.L.A.

AN6387K-MAT Echivalent: : N.L.A.

AN6514 Echivalent: : N.L.A.

AN6540-MAT LWDR POS V-REG 8V5 0.2A

AN6541-MAT LWDR POS V-REG 9V 0.3-0.6

AN6550-MAT DUAL LOW-FREQ. EQUALIZER

AN6551-MAT DUAL OP-AMP,NF-EQUAL.

AN6552-MAT Echivalent: MC1458N-MBR

AN6553 Echivalent: RC4559N-MBR

AN6554 QUADRUPLE OPERAT.AMP.14p

AN6555 DUAL AMPLIFIER +/-18V

AN6556 Echivalent: RC4559N-MBR

AN6557-MAT DUAL OPAMP

AN6558-MAT DUAL OPAMP

AN6559 Echivalent: MC1458N-MBR

AN6562 Echivalent: MC1458N-MBR

AN6564 Echivalent: LM324N-MBR

AN6572 Echivalent: MC1458N-MBR

AN6573 Echivalent: TA7504S-TOS

AN6605-MAT I.C.

AN6609N 2 SPEED DC-MOTOR DRIVER

AN6610 Echivalent: TDA1151-ST

AN6612-MAT MOTOR SPEEDCTRL.Uref=1.32

AN6636-MAT I.C.

AN6650-MAT MOTOR CONTROL

AN6651 MOTOR CONTROL

AN6652 SPEED CONTROL, Uref=1.25V

AN6671K-MAT I.C.

AN6676 Echivalent: : N.L.A.

AN6677-MAT Echivalent: : N.L.A.

AN6780S-MAT TIMER 7p SIP

AN6870N-MAT STEREO PEAK HOLDER 28p

AN6873-MAT VACCUM FLUOR.DISP. DRIVER

AN6875 LED DRIVER 5 LED's LOG.

AN6876 LED DRIVER(5 LEDS LIN) 9p

AN6878-MAT LED DRV 7-LED'S LOGARIT.

AN6879-MAT LED DECODER 7 LED's

AN6880 SERVO MOTOR CONTROL 7p

AN6881-MAT PROG. FREQUENCY DIVIDERS

AN6882 LED DRIVER, 7 LEDS

AN6884-MAT LED LEVEL METER 5 LED'S

AN6888-MAT LED DRIVER

AN6889 DRIVER 5 DOT LED DISPLAY

AN6912 Echivalent: LM339N-MBR

AN6913 DUAL COMPARATOR 9p

AN6914 Echivalent: LM393N-MBR

AN7000 Echivalent: 209.83636

AN7010K-MAT AUDIO PRE-AMPLIFIER

AN7016NK-MAT RECORDER, 2xA/W AMP. MUTE

AN7024-MAT AM-TUNER, MPX-STEREO-DEC.

AN7025K-MAT AM/FM IF AMPLIFIER

AN7062 PRE-AMPLIFIER 18p

AN7072-MAT IF AMP. + MUTE

AN7081K-MAT DUAL PRE/VOLUME/PWR GOVER

AN7082K-MAT RECORDER HEADPH. AMP.

AN7105 DUAL POWER AMP. 18p

AN7106K-MAT DUAL POWER AMP.

AN7108-MAT DUAL RECORDER -NF- V+E

AN7110-MAT 1.2W AUDIO POWER AMP. 9p

AN7111 1.2W AUDIO POWER AMP. 9p

AN7112E-MAT 1.0W AUDIO POWER AMP. 9p

AN7114E-MAT 1.0W AUDIO POWER AMP. 14p

AN7115 2.1W AUDIO POWER AMP.

AN7116-MAT 1.0W AUDIO POWER AMP.

AN7117 1.0W AUDIO POWER AMP. 9p

AN7118-MAT 3.0W BTL POWER AMP.

AN7118S-MAT 3.0 WATT AMPLIFIER

AN7120 Echivalent: : N.L.A.

AN7130-MAT 4.2W AUDIO POWER AMP. 9p

AN7131-MAT 5W AUDIO POWER AMP. 9p

AN7133N-MAT PWR AMP 2x5.8W/3E 24V 6A

AN7134NR-MAT PWR AMP 2x7.5W/3E 24V 6A

AN7135-MAT PWR AMP 2*7W /3E 15V

AN7140 5.0W AUDIO POWER AMP.

AN7141-MAT SINGLE CHANNEL AUDIO AMP.

AN7142-MAT DUAL POWER AMPLIFIER

AN7143-MAT Echivalent: : N.L.A.

AN7145M DUAL CHANNEL AUDIO AMP.

AN7146M-MAT DUAL POWER AMP.

AN7147N-MAT 2x5.8W AUDIO POWER AMPL.

AN7148-MAT DUAL 2.1W AUDIO POWER AMP

AN7149N-MAT DUAL CHANNEL AUDIO AMP.

AN7154 PWR.AMP SINGLE CHANNEL

AN7156N-MAT PWR-AMP 2*5.5W

AN7158N-MAT 7.5W DUAL POWER AMP.

AN7160 Echivalent: : N.L.A.

AN7161N-MAT 18W BTL AUDIO POWER

AN7161NFP-MAT 18W BTL AUDIO POW+COOLFIN

AN7163 18W BTL AUDIO POWER AMP.

AN7164-MAT 2xNF-E 30V 5A 30W (21V/8E

AN7166 5.8W DUAL POWER AMP.

AN7168 5.7W AUDIO POWER AMP. 12p

AN7169 5.7W AUDIO POWER AMP. 12p

AN7170 NF-E 35V 4A 18W

AN7171NK-MAT AUDIO POWER AMP 20W

AN7172K-MAT Echivalent: : N.L.A.

AN7173K-MAT PWR AMP 2x9W/4E 24V 6A

AN7174K-MAT PWR AMP 2x12.5W/4E 24V 6A

AN7176K-MAT DUAL AMPLIFIER

AN7177-MAT 20W BTL AUDIO POWER AMPL.

AN7178 5.7W AUDIO POWER AMP. 12p

AN7188K-MAT PWR AMPLIFIER

AN7190K-MAT I.C.

AN7205-MAT FM-TUNER Ucc=1,6...6V

AN7213-MAT RADIO FM FRONT END

AN7216 FM-TUNER Ucc=1,7..7V

AN7218 AM TUNER FM/AM IF AMP.16p

AN7220A-MAT AM/FM AMP.+ AFC + DEM.18p

AN7222N-MAT AM TUNER AM/FM IF AMP.

AN7223 FM/AM IF AMP.AFC/AGC

AN7224-MAT FM/AM IF AMPLIFIER AFC/AG

AN7225-MAT AM/FM IF

AN7236-MAT FM TUNER +FM TUNING LEVEL

AN7254-MAT CAR RADIO FM FRONT END

AN7256-MAT CAR RADIO FM-ZF DEMOD.

AN7273 AM/FM AMP. + AFC +DEM.

AN7310-MAT DUAL PRE-AMP.

AN7311-MAT DUAL PRE-AMP. 9p

AN7312-MAT AUDIO PRE-AMPLIFIER

AN7315 2x NF-V Ucc=1.6 ... 4.5V

AN7316-MAT RECORDER, 2xNF-V, ALC

AN7317-MAT I.C.

AN7320-MAT REC. AUT. LEVEL CONTROL

AN7330K-MAT 2 CHN GRAPHIC EQ. AMP

AN7362 I.C.

AN7363N-MAT I.C.

AN7370-MAT RECORDER IC, DOLBY C

AN7382-MAT ELECTRONIC AUDIO CONTROL

AN7384N-MAT DC VOLUME CONTROL

AN7410N-MAT STEREO DECODER

AN7411 FM STEREO, MPX-DECODER

AN7414 STEREO DECODER 18p

AN7415 STEREO DECODER 16p

AN7420 Echivalent: TA7343AP-TOS

AN7421 Echivalent: TA7342P-TOS

AN7465K CAR RADIO FM-STER. MPX-DE

AN7470-MAT FM STEREO MPX DEM.PLL 16p

AN7805 Echivalent: UA7805-MBR

AN7806 Echivalent: UA7806-MBR

AN7808 Echivalent: UA7808-MBR

AN7809 Echivalent: UA7809-MBR

AN7810 Echivalent: UA7810-MBR

AN7812 Echivalent: UA7812-MBR

AN7812R-MAT POS V-REG 12V 1A + RESET

AN7815 Echivalent: UA7815-MBR

AN7818 Echivalent: UA7818-MBR

AN7910 Echivalent: UA7910-MBR

AN8053 I.C.

AN8060-MAT LWDR NEG V-REG 4V 30MA 0.

AN8072N-MAT 5xPOS V-REG

AN8077NK IC

AN8090-MAT SW-REG 500KHz 35Vin

AN8250N-MAT STEPPING MOTOR DRIVER

AN8275N-MAT I.C.

AN829-MAT DUAL AF-ATTENUATOR

AN8370 IC RAN8370S

AN8377N-MAT 3-CHANNEL LIN. DRIVER

AN8387S-MAT CD 2 CHAN. LINE DRIV.

APU2400E39-ITT CTV AUDIO PROCESSOR

APU2400T-ITT Echivalent: : N.L.A.

APU2470DIP-ITT D2-MAC AUDIO PROCESSOR

APU2471-90 Echivalent: 209.61626

APU2471DIP-ITT Echivalent: : N.L.A.

ASY127 Echivalent: ASY27

ASY26 Echivalent: AC128

ASY27 GE-P 25V 0.2A 0.15W

ASY77 GE-P 60V 1A 0.26W 500KHz

ASZ15 GE-P 100V 8A 30W

ASZ16 Echivalent: ASZ18

ASZ17 Echivalent: ASZ18

ASZ18 GE-P 100V 8A 30W

AY-3-1015 Echivalent: : N.L.A.

AY-3-8203 TV CHANNEL SELECTOR

AY-3-8210 REMCONTROL DECODER

AY-3-8475 256 CMND IR RC RECEIVER

AY-3-8500 Echivalent: : N.L.A.

AY-3-8765 MOTOR CYCLE GAME 28p

AY-3-8910 Echivalent: M5255-OKI

AY-3-8912 TV/VIDEO GAME CIRCUIT

AY-3-8913 PROGRM.SOUND GENERAT. 40p

AY-5-1013 Echivalent: 6402-MBR

AY-5-1203A CHANNEL FADE IN 24p

AY-5-1232 DIGITAL CLOCK IC 60 Hz

AY-5-8100 Echivalent: : N.L.A.

20.000 HC49 QUARZE 20.000.000 MHz

2101A SRAM 256x4 I/O

2114-200NS SRAM 1Kx4 =TC5514APL2

2148 Echivalent: 2114-200NS

2332A CHAR.ROM C64 =901225-01

2352D Echivalent: NJM2352DD-JRC

2364A-B Echivalent: 6004-161

24C01N-MBR EEPROM 128x8BIT I2C-BUS

24C02AB1 Echivalent: 24C02N-MBR

24C02CP Echivalent: 24C02N-MBR

24C02N-MBR EEPROM 256x8 I2C BUS

24C02P Echivalent: 24C02N-MBR

24C04N-MBR EEPROM 512x8 BIT I2C BUS

24C08N-MBR EEPROM 1Kx8 BIT I2CBUS

24C16N-MBR EEPROM 2Kx8 BIT I2CBUS

24C32N-MBR EEPROM 4KX8 BIT I2C BUS

24LC16B-MBR EEPROM 2K*8 2.5-5.5V SUP

2512-2 Echivalent: 2N5672

251715-01 MEMORY CONTROLER C64-2

2532 Echivalent: 2732A-250-MBR

25F120-IR SI-D 1200V 25A/300Ap

26LS30 Echivalent: AM26LS30-MBR

26LS31 Echivalent: AM26LS31-MBR

26LS32 Echivalent: AM26LS32-MBR

27.000 HC49 QUARZE 27MHz

2708/DIV EPROM 1Kx8 SUPP.+12V+5V-5

27128-250-MBR EPROM 16Kx8 250ns 12V5

2716-350/DIV EPROM 2Kx8 350ns 25V

27256 Echivalent: 27C256-120-MBR

27256-200-MBR Echivalent: 27C256-120-MBR

27256-250-MBR Echivalent: 27256-200-MBR

2732 Echivalent: 2732A-250-MBR

2732A-250-MBR EPROM 4Kx8 250ns 21V

27512-200-MBR Echivalent: 27C512-150-MBR

27512-250-MBR Echivalent: 27C512-150-MBR

2764A-200-MBR Echivalent: 27C64-150-MBR

2764A-250 Echivalent: 2764A-200-MBR

2764A-300 Echivalent: 2764A-200-MBR

27C010-150-MBR C-EPROM 128KX8

27C020 Echivalent: 27C2001-120-MB

27C040 Echivalent: 27C4001-120-MB

27C1000-150-MB C-EPROM 128Kx8 PIN2=EN

27C1001-100-MB C-EPROM 128K*8 PIN2=A1

27C1001-120 Echivalent: 27C1001-100-MB

27C1001-150-MB Echivalent: : 27C1001-100-

27C1024-150-MB C-EPROM 64Kx16 =27C210-15

27C128-250-MBR C-EPROM 16Kx8 250ns 12V

27C2001-120-MB C-EPROM 256Kx8 5V

27C2048-150-MB C-EPROM 128Kx16 150ns 12V

27C210-15FA Echivalent: 27C1024-150-MBR

27C256 Echivalent: 27C256-120-MBR

27C256-120-MBR C-EPROM 32Kx8 120ns 12V5

27C256-200 Echivalent: 27C256-120-MBR

27C256-25 Echivalent: 27C256-120-MBR

27C256-250 Echivalent: 27C256-120-MBR

27C4001-120-MB C-EPROM 512KX8 5V

27C4048 Echivalent: 27C4002-120-MB

27C4096-120 C-EPROM 256K*16 120NS

27C512-120-MBR C-EPROM 64Kx8 120ns 12V5

27C512-150-MBR Echivalent: 27C512-120-MBR

27C512-200 Echivalent: 27C512-150-MBR

27C64-150-MBR C-EPROM 8Kx8 150ns

27C64-200-MBR Echivalent: 27C64-150-MBR

27C64-250-MBR Echivalent: 27C64-150-MBR

2816 Echivalent: 28C16A-150-MBR

2864 Echivalent: 28C64A-150-MBR

28C16A-150-MBR EEPROM 2K*8 150ns

28C16A-200 Echivalent: 28C16A-150-MBR

28C256 Echivalent: 28C256-250-MBR

28C256-150-MBR EEPROM 32Kx8 150ns

28C256-250-MBR Echivalent: 28C256-150-MBR

28C64A-150-MBR EEPROM 8Kx8 150ns

28F010-150-DIP FLASH EPROM 128K*8

2N109 GE-P 35V 0.15A 0.165W

2N1304-MBR GE-N 25V 0.3A 0.15W 10MHz

2N1305-MBR GE-P 30V 0.3A 0.15W 5MHz

2N1307-MBR GE-P 30V 0.3A 0.15W B>60

2N1595-MBR THYR. 50V 1A 10/5mA

2N1599-MBR THYR. 400V 1A 10/5mA

2N1613-MBR SI-N 75V 1A 0.8W 60MHz

2N1613/DIV SI-N 75V 0.5A 2.6W 100MHz

2N1671 Echivalent: 2N2160-TO18

2N1671A Echivalent: 2N2160-TO18

2N1671B-MBR UJT-P Ip=6uA Iv=8mA 0.45W

2N1711-MBR SI-N 75V 1A 0.8W 70MHz

2N1711/DIV SI-N 75V 0.5A 2.6W 120MHz

2N1893-MBR SI-N 120V 0.5A 0.8W

2N1893/DIV SI-N 120V 0.5A 0.8W 50MHz

2N2102-MBR SI-N 120V 1A 1W <120MHz

2N2148 GE-P 60V 5A 12.5W

2N2160-TO18 UJT-P Ip=25uA Iv=8mA .45W

2N2165 SI-P 30V 50mA 0.15W 18MHz

2N2166 SI-P 15V 50mA 0.15W 10MHz

2N2217 Echivalent: 2N2219A-MBR

2N2218 Echivalent: 2N2219A-MBR

2N2218A Echivalent: 2N2219A-MBR

2N2219 Echivalent: 2N2219A-MBR

2N2219A-MBR SI-N 40V 0.8A 0.8W 250MHz

2N2219A/DIV SI-N 40V 0.8A 0.8W 250MHz

2N2220 Echivalent: 2N2222A-MBR

2N2221A Echivalent: 2N2222A-MBR

2N2222-PL Echivalent: PN2222A-MBR

2N2222A Echivalent: 2N2222A-MBR

2N2222A-MBR SI-N 40V 0.8A 0.5W 300MHz

2N2222A-PL Echivalent: PN2222A-MBR

2N2222A/DIV SI-N 40V 0.8A 0.5W 300MHz

2N2223-MBR 2xSI-N 100V 0.5A 0.6W >50

2N2223A-MBR 2xSI-N 100V 0.5A 0.6W >50

2N2243A SI-N 120V 1A 0.8W 50MHz

2N2297 Echivalent: 2N3019-MBR

2N2324 Echivalent: 2N2326-MOT

2N2326-MOT THYR. 200V 1A 0.2mA

2N2329 THYR. 400V 1A 0.2/2mA

2N2368 Echivalent: 2N2369A-MBR

2N2369A-MBR SI-N 40V 0.2A .36W 12/18n

2N2369A/DIV SI-N 15V 0.2A 0.36W 12/18

2N2405 Echivalent: 2N2102-MBR

2N2483 Echivalent: 2N3904-MBR

2N2484 Echivalent: 2N3904-MBR

2N2646-TO18 UJT-P lp=5uA lv=4mA 0.3W

2N2647-TO18 UJT-P lp=2uA lv=8mA 0.3W

2N2857 SI-N 30V 40mA 0.2W >1GHz

2N2894-MBR SI-P 12V 0.2A 1.2W 60/90n

2N2904 Echivalent: 2N2905A-MBR

2N2904A Echivalent: 2N2905A-MBR

2N2905 Echivalent: 2N2905A-MBR

2N2905A-MBR SI-P 60V 0.6A 0.6W 45/100

2N2905A/DIV SI-P 60V 0.6A 0.6W 45/100

2N2906 Echivalent: 2N2906A

2N2906A SI-P 60V 0.6A 0.4W 45/100

2N2907 Echivalent: 2N2907A-MBR

2N2907-PL Echivalent: PN2907-MBR

2N2907A-MBR SI-P 60V 0.6A 0.4W 45/100

2N2907A-PL Echivalent: PN2907A-MBR

2N2914 Echivalent: 2N2917-MBR

2N2917-MBR SI-N 45V 0.03A >60Mz

2N2918 Echivalent: 2N2917-MBR

2N2926-MBR SI-N 25V 0.1A 0.2W 300MHz

2N2955 GE-P 40V 0.1A 0.15W 200MH

2N3019-MBR SI-N 140V 1A 0.8W 100MHz

2N3020 Echivalent: 2N3019-MBR

2N3053-CDL SI-N 60V 0.7A 5W 100MHz

2N3054-MBR SI-N 90V 4A 25W 3MHz

2N3054/DIV SI-N 90V 4A 25W 3MHz

2N3055-CDL SI-N 100V 15A 115W 800kHz

2N3055-ST SI-N 100V 15A 115W 800kHz

2N3055/DIV SI-N 100V 15A 115W 800kHz

2N3055H-BAN SI-N 100V 15A 115W 800kHz

2N3228 THYR. 200V 3.2A 15/20mA

2N3251-MBR SI-P 50V 0.2A 0.36W

2N3375 SI-N 40V 0.5A 11.6W 500MH

2N3415 Echivalent: BC337-25-MBR

2N3416 Echivalent: BC337-25-MBR

2N3417 Echivalent: BC337-25-MBR

2N3439 SI-N 450V 1A 10W 15MHz

2N3440-MBR SI-N 300V 1A 10W 15MHz

2N3441 SI-N 160V 3A 25W POWER

2N3442-MBR SI-N 160V 10A 117W 0.8MHz

2N3442/DIV SI-N 160V 10A 117W 0.8MHz

2N3495-MBR SI-P 120V 0.1A 0.6W >150M

2N3502 SI-P 45V 0.6A 0.7W 200MHz

2N3525 Echivalent: 2N4101

2N3553-PHI SI-N 65V 0.35A 7W 500MHz

2N3553/DIV SI-N 65V 0.35A 7W 500MHz

2N3563 Echivalent: PN3563-MBR

2N3568 Echivalent: BC337-25-MBR

2N3569 Echivalent: BC337-25-MBR

2N3571-MBR SI-N 30V 0.05A 0.2W 1.4GH

2N3572 Echivalent: 2N3571-MBR

2N3583-MBR SI-N 250/175V 2A 35W >10M

2N3584 Echivalent: 2N3585

2N3585 Echivalent: : N.L.A.

2N3632-PHI Echivalent: : 2N3632/DIV

2N3632/DIV SI-N 40V 0.25A 23W 400MHz

2N3642 Echivalent: BC337-25-MBR

2N3646 SI-N 40V 0.2A 0.2W

2N3672 Echivalent: 2N2907A-MBR

2N3700-MBR SI-N 140V 1A 0.5W 200MHz

2N3702 Echivalent: BC557B-MBR

2N3703 Echivalent: BC557B-MBR

2N3704 Echivalent: BC337-40-MBR

2N3705 Echivalent: BC337-16-MBR

2N3706 Echivalent: BC337-16-MBR

2N3707 SI-N 30V 0.03A 0.36W 100M

2N3708 SI-N 30V 0.03A 0.36W 80MH

2N3709 Echivalent: BC547A-MBR

2N3710 Echivalent: BC547B-MBR

2N3711 Echivalent: BC547C-MBR

2N3715 Echivalent: 2N3716-MBR

2N3716-MBR SI-N 100V 10A 150W 4MHz

2N3724 Echivalent: 2N3725-MBR

2N3725-MBR SI-N 80V 0.5A 1W 35/60ns

2N3740 SI-P 60V 4A 25W >4MHz

2N3740/DIV SI-P 60V 4A 25W >4MHz

2N3741 SI-N 80V 4A 25W >4MHz

2N3742 SI-N 300V 0.05A 1W >30MHz

2N3766 Echivalent: 2N3767-MBR

2N3767-MBR SI-N 100V 4A 20W >10MHZ

2N3771-ST SI-N 50V 30A 150W POWER

2N3771/DIV SI-N 50V 30A 150W POWER

2N3772-MBR SI-N 100V 20A 150W POWER

2N3772/DIV SI-N 100V 20A 150W POWER

2N3773-MBR SI-N 160V 16A 150W POWER

2N3773/DIV SI-N 160V 16A 150W POWER

2N3790 Echivalent: 2N3792-MBR

2N3791 Echivalent: 2N3792-MBR

2N3792-MBR SI-P 80V 10A 150W 4MHz

2N3793 Echivalent: BC337-25-MBR

2N3819-MBR N-FET 25V 20mA 0.36W

2N3820-MBR P-FET 20V 15mA 0.36W

2N3821-MBR N-FET 50V 2.5mA 0.3W

2N3823-MBR N-FET 30V 20mA 0.3W

2N3824 N-FET 50V 10mA 0.3W <250E

2N3866-MBR SI-N 55V 0.4A 1W 175MHz 1

2N3899-MBR THYR. 600V 22A

2N3903 Echivalent: 2N3904-MBR

2N3904-MBR SI-N 60V 0.2A .35W 300MHz

2N3905 Echivalent: 2N3906-MBR

2N3906-MBR SI-P 40V 0.2A .35W 250MHz

2N3909-MBR P-FET 20V 10MA 0.3W

2N3924-MBR Echivalent: : N.L.A.

2N3958-MBR N-FET 50V 5mA 0.25W

2N3963-MBR SI-P 80V 0.2A 0.36W >40MH

2N3972 N-FET 40V 50mA 1.8W

2N4001-MBR SI-N 100V 1A 15W 40MHZ

2N4030 Echivalent: 2N4033

2N4031 Echivalent: 2N4033

2N4032 Echivalent: 2N4033

2N4033 SI-P 80V 1A 0.8W 150MHz

2N4036-MBR SI-P 90V 1A 1W 60MHz

2N4037 Echivalent: BC161-16-MBR

2N4072 Echivalent: 2N3904-MBR

2N409 GE-P 13V 15mA 80mW 6.8MHz

2N4091 Echivalent: 2N4391-MBR

2N4092 Echivalent: 2N4392-MBR

2N4093 Echivalent: 2N4393-MBR

2N4101 Echivalent: : N.L.A.

2N4123 Echivalent: 2N3904-MBR

2N4124 Echivalent: 2N3904-MBR

2N4125 Echivalent: 2N3906-MBR

2N4126 SI-P 25V 200mA HF

2N4220-MBR N-FET 30V 0.2A

2N4236 SI-P 80V 3A 1W >3MHz

2N4249 Echivalent: BC556B-MBR

2N4250 Echivalent: BC557B-MBR

2N426 Echivalent: 2N427

2N426E Echivalent: ZN426E8-PLS

2N427 GE-P 30V 0.4A 0.15W B>40

2N428 GE-P 30V 0.4A 0.15W B>60

2N4286 SI-N 30V 0.05A 0.25W

2N4287 SI-N 45V 0.1A 0.25W 40MHz

2N4289 Echivalent: BC327-25-MBR

2N4291 SI-P 40V 0.2A 0.25W 150MH

2N4302 N-FET 30V 0.5mA 0.3W

2N4347 SI-N 140V 5A 100W 0.8MHz

2N4348 SI-N 140V 10A 120W >0.2MH

2N4351 N-FET 30V 30mA 0.3W 140KH

2N4391-MBR N-FET 40V 50mA 30E Up<10V

2N4392-MBR N-FET 40V 25mA 60E Up<5V

2N4393-MBR N-FET 40V 5mA 100E Up<3V

2N4398 Echivalent: 2N4399-MBR

2N4399-MBR Echivalent: : MJ4502-MBR

2N4400 Echivalent: 2N4401-MBR

2N4401-MBR SI-N 60V 0.6A 200MHz

2N4402 Echivalent: 2N4403-MBR

2N4403-MBR SI-P 40V 0.6A 200MHz

2N4416-MBR N-FET 30V 15mA VHF/UHF

2N4420 SI-N 40V 0.2A 0.36W

2N4427-ST SI-N 40V 0.4A 1W 175MHz 1

2N4441 THYR. 50V 5.1A 30mA

2N4444 Echivalent: BT151/800R-PHI

2N4859 Echivalent: BSV80-PHI

2N4870 Echivalent: 2N4871

2N4871 PUT 30V 0.05A 5uA

2N4891 Echivalent: 2N4871

2N4895 Echivalent: BFX34-MBR

2N4906 SI-P 80V 5A 87.5W >4MHz

2N4918 Echivalent: 2N4920-MBR

2N4919 Echivalent: 2N4920-MBR

2N4920-MBR SI-P 80V 1A 30W

2N4921 Echivalent: 2N4923

2N4923 SI-N 80V 1A 30W

2N4991 Echivalent: MBS4991-MOT

2N4992 Echivalent: MBS4992-MOT

2N5038-MBR SI-N 150V 20A 140W 0.5us

2N5038/DIV SI-N 150V 20A 140W 0.5us

2N5039-ST Echivalent: : 2N5038-MBR

2N5060 Echivalent: 2N5064-PHI

2N5061 Echivalent: 2N5064-PHI

2N5062 Echivalent: 2N5064-PHI

2N5064-MOT Echivalent: : 2N5064-PHI

2N5064-PHI THYR. 200V 0.8A 0.2mA 3/3

2N5086 Echivalent: BC557B-MBR

2N5087 Echivalent: BC557B-MBR

2N5090 SI-N 55V 0.4A 4W 5mA

2N5109-ST SI-N 40V 0.5A 2.5W 1.5GHz

2N5116-MBR P-FET 30V 5mA 150E Up<4V

2N5154-MBR SI-N 100V 2A 10W

2N5157 Echivalent: BUX80-MBR

2N5160 Echivalent: : N.L.A.

2N5179-CDL SI-N 20V 50mA 0.2W >1GHz

2N5190 Echivalent: 2N5192-MBR

2N5191 Echivalent: 2N5192-MBR

2N5192-MBR SI-N 80V 4A 40W 2MHz

2N5193 Echivalent: BD442-MBR

2N5194 Echivalent: BD442-MBR

2N5195 Echivalent: BD442-MBR

2N5204 THYR. 600V 22A 40/100 mA

2N5210 Echivalent: 2N3904-MBR

2N5239 Echivalent: 2N5240

2N5240 SI-N 375V 5A 100W >2MHz

2N5245 Echivalent: BF245A-MBR

2N5294 Echivalent: BD243C-MBR

2N5296 Echivalent: BD243C-MBR

2N5298 SI-N 80V 4A 36W >0.8MHz

2N5301 Echivalent: 2N5886-MBR

2N5302-ST Echivalent: 2N5886-MBR

2N5303 Echivalent: 2N5886-MBR

2N5308 N-DARL 40V 0.3A 0.4W B>7K

2N5320-MBR SI-N 100V 2A 10W AFSWITCH

2N5321 Echivalent: 2N5320-MBR

2N5322-MBR SI-P 100V 2A 10W AFSWITCH

2N5400-MBR Echivalent: 2N5401-MBR

2N5401-MBR SI-P 160V 0.6A 0.31W

2N5401-TOS Echivalent: 2N5401-MBR

2N5415-MBR Echivalent: 2N5416-MBR

2N5416-MBR SI-P 350V 1A 10W 15MHz

2N5433-MBR N-FET 25V 0.4A 0.3W 7E

2N5445 Echivalent: 2N5446

2N5446 TRIAC 600V 40A 70/100mA

2N5457-MBR N-FET 25V 1mA Up<6V

2N5458-MBR N-FET 25V 2.9mA UNI

2N5458-NSC Echivalent: 2N5458-MBR

2N5459 Echivalent: BF245C-MBR

2N5460-MBR P-FET 40V 5mA Up<6V GEN.P

2N5461-MBR P-FET 40V 9mA 0.31W

2N5462-MBR P-FET 40V 16mA Up<9V GEN.

2N5484-MBR N-FET 25V 5mA 0.31W

2N5485-MBR P-FET 25V 4mA Up<4V

2N5494 Echivalent: BD537-MBR

2N5496 Echivalent: BD243C-MBR

2N5550 Echivalent: 2N5551-MBR

2N5551-MBR SI-N 180V 0.6A 0.31W VID.

2N5589 SI-N 36V 0.6A 3W 175MHz

2N5591 Echivalent: BLY89C-PHI

2N5630 Echivalent: MJ15024-MOT

2N5631 Echivalent: MJ15024-MOT

2N5639 N-FET 30V 10mA 310mW

2N5643 Echivalent: BLY93C-PHI

2N5671 Echivalent: 2N5672

2N5672 SI-N 150V 30A 140W 0.5us

2N5679 Echivalent: 2N5680-MBR

2N5680-MBR SI-P 120V 1A 1W

2N5682-MBR SI-N 120V 1A 1W >30MHz

2N5684 SI-P 80V 50A 200W

2N5685 Echivalent: 2N5686

2N5686 SI-N 80V 50A 300W >2MHz

2N5770-MBR SI-N 30V 0.05A 0.7W >900M

2N5771-MBR SI-P 15V 50mA 625mW >850M

2N5875 Echivalent: MJ2955-MBR

2N5876-MBR SI-P 80V 10A 150W >4MHz

2N5877 Echivalent: 2N5878

2N5878 SI-N 80V 10A 150W >4MHz

2N5879 SI-N 60V 10A 150W >4MHz

2N5883 Echivalent: 2N5884-ST

2N5884-ST SI-P 80V 25A 200W AFPOWSW

2N5884/DIV SI-P 80V 25A 200W AFPOWSW

2N5885 Echivalent: 2N5886-MBR

2N5886-MBR SI-N 80V 25A 200W >4MHz

2N5886/DIV SI-N 80V 25A 200W >4MHZ

2N6012 Echivalent: BC337-40-MBR

2N6027-MBR PUT 40V 0.15A 5uA

2N6028-MOT PUT 40V 0.15A 1uA

2N6029 Echivalent: BD318/DIV

2N6030 Echivalent: 2N6031-MOT

2N6031-MOT SI-P 140V 16A 200W 1MHz

2N6031/DIV SI-P 140V 16A 200W 1MHz

2N6038 Echivalent: BD681-MBR

2N6039 Echivalent: BD681-MBR

2N6040 Echivalent: TIP107-MBR

2N6041 Echivalent: TIP107-MBR

2N6042 Echivalent: TIP107-MBR

2N6043 Echivalent: TIP102-MBR

2N6044 Echivalent: TIP102-MBR

2N6045 Echivalent: TIP102-MBR

2N6049 Echivalent: : N.L.A.

2N6050-MBR P-DARL+D 60V 12A 100W

2N6051 Echivalent: MJ2501-ST

2N6052 Echivalent: BDX64C

2N6055 Echivalent: MJ1001

2N6056 Echivalent: 2N6284-ST

2N6057 Echivalent: 2N6059-ST

2N6058 Echivalent: 2N6059-ST

2N6059-ST SI-N 100V 12A 150W

2N6071 Echivalent: 2N6075A-MBR

2N6073 Echivalent: 2N6075A-MBR

2N6075 Echivalent: 2N6075A-MBR

2N6075A-MBR TRIAC 600V 4A 20mA

2N6080-MBR Echivalent: 2N6083

2N6081 Echivalent: 2N6083

2N6082 Echivalent: 2N6083

2N6083 SI-N 36V 5A PQ=30W 175MHz

2N6098 SI-N 70V 10A 75W AFPOWSWI

2N6099 SI-N 70V 10A 75W AFPOWSWI

2N6101 Echivalent: 2N6488-MBR

2N6106 Echivalent: BD244C-MBR

2N6107 Echivalent: BD244C-MBR

2N6109-MBR SI-P 60V 7A 40W 10MHz

2N6111 Echivalent: 2N6109-MBR

2N6121 Echivalent: BD243C-MBR

2N6123 Echivalent: BD243C-MBR

2N6124-MBR SI-P 45V 4A 40W

2N6211 SI-P 275V 2A 20W 20MHz

2N6212 Echivalent: 2N6213

2N6213 SI-P 400V 2A 35W >20MHz

2N6240 Echivalent: C106D-MOT

2N6246 Echivalent: 2N6248

2N6247 Echivalent: 2N6248

2N6248 SI-P 110V 15A 125W >6MHz

2N6251 Echivalent: S2530A

2N6254 Echivalent: 2N3772-MBR

2N6255 Echivalent: BFQ43-PHI

2N6258 Echivalent: MJ15024-MOT

2N6259 Echivalent: MJ15024-MOT

2N6282 Echivalent: 2N6284-ST

2N6283 Echivalent: 2N6284-ST

2N6284-ST N-DARL 100V 20A 160W B>75

2N6285 Echivalent: 2N6287-ST

2N6286 Echivalent: 2N6287-ST

2N6287-ST P-DARL 100V 20A 160W

2N6287/DIV P-DARL 100V 20A 160W

2N6288 Echivalent: 2N6292

2N6290 Echivalent: 2N6292

2N6292 SI-N 80V 7A 40W

2N6308 Echivalent: BUX80-MBR

2N6322 Echivalent: BUX22-MBR

2N6328 Echivalent: 2N5886-MBR

2N6343 Echivalent: TIC226D-MBR

2N6349 Echivalent: BT137/800-PHI

2N6356 N-DARL 50V 20A 150W B>150

2N6388 Echivalent: BD651-MBR

2N6395 Echivalent: TIC126D-MBR

2N6398-MBR Echivalent: TIC126M-TI

2N6400 Echivalent: 2N6405-MOT

2N6401 Echivalent: 2N6405-MOT

2N6402 Echivalent: 2N6405-MOT

2N6403 Echivalent: 2N6405-MOT

2N6404 Echivalent: 2N6405-MOT

2N6405-MOT THYR. 800V 16A <30/<40mA

2N6422 SI-P 500V 2A 35W >10MHz

2N6427-MBR N-DARL 40V 0.5A 0.625W

2N6474 Echivalent: BD941

2N6476-MBR SI-P 130V 4A 16W 5MHz

2N6486 Echivalent: 2N6488-MBR

2N6487 Echivalent: 2N6488-MBR

2N6488-MBR SI-N 90V 15A 75W

2N6489 Echivalent: 2N6491-MBR

2N6490 Echivalent: 2N6491-MBR

2N6491-MBR SI-P 90V 15A 30W

2N6504 Echivalent: 2N6509-MOT

2N6505 Echivalent: 2N6509-MOT

2N6506 Echivalent: 2N6509-MOT

2N6507 Echivalent: 2N6509-MOT

2N6508 Echivalent: 2N6509-MOT

2N6509-MOT THYR. 800V 25A 40mA 2/15u

2N6517-MBR SI-N 350V 0.5A 0.625W >40

2N6520-MBR SI-P 350V 0.5A 0.625W >40

2N6545 Echivalent: BUX47A-ST

2N6547 SI-N 850/400V 15A 175W

2N6551 Echivalent: 2N6553-MBR

2N6552 Echivalent: 2N6553-MBR

2N6554 Echivalent: 2N6556-MBR

2N6555 Echivalent: 2N6556-MBR

2N6556-MBR SI-P 100V 1A 10W >75MHZ

2N6609 SI-P 160V 16A 150W 2MHz

2N6660-MBR N-FET 60V 2A 6.25W 3E

2N6661-MBR N-FET 90V 2A 6.2W 4E

2N6675 SI-N 400V 15A

2N6678-ST SI-N 400V 15A TO3

2N6686 Echivalent: BUX22-MBR

2N6716 SI-N 60V 2A 2W 50MHz

2N6718-MBR SI-N 100V 2A 2W 50MHz

2N6725-MBR N-DARL 60V 2A 1W B>15K

2N6728-MBR SI-P 60V 2A 2W >50MHz

2N692 THYR. 800V 16A 15/50mA

2N697 SI-N 60V 1A 0.6W <50MHz

2N7000 Echivalent: BS170-MBR

2N7002-MBR N-FET 60V 0.115A 0.2W 7E5

2N706 Echivalent: 2N2369A-MBR

2N708 Echivalent: 2N2369A-MBR

2N735 Echivalent: 2N3904-MBR

2N914-ST SI-N 40V 0.5A <40/40NS SW

2N918-MBR SI-N 30V 50mA 0.2W 600MHZ

2N918/DIV SI-N 15V 50mA 0.2W 900MHz

2N930 Echivalent: BC107B-MBR

2P4M-NEC THYR. 400V 2A 0.2mA 30us

2SA1006-NEC Echivalent: 2SA1006B-NEC

2SA1006A-NEC Echivalent: 2SA1006B-NEC

2SA1006B-NEC SI-P 250V 1.5A 25W 80MHz

2SA1009-NEC SI-P 350V 2A 15W

2SA101 Echivalent: 2SB681

2SA1010 Echivalent: TIP42C-MBR

2SA1011-SAN SI-P 160V 1.5A 25W 120MHz

2SA1012 Echivalent: 2SB826-SAN

2SA1013-TOS SI-P 160V 1A 0.9W 50MHz

2SA1014 Echivalent: 2SA1013-TOS

2SA1015-MBR SI-P 50V 0.15A 0.4W 80MHz

2SA1016-SAN SI-P 100V 0.05A 0.4W 110M

2SA1017-SAN SI-P 120V 50mA 0.5W 110MH

2SA1018-MAT SI-P 250V 70mA 0.75W >50M

2SA1019 Echivalent: 2SA1208-SAN

2SA1020-TOS SI-P 50V 2A 0.9W 100MHz

2SA1021-TOS Echivalent: : N.L.A.

2SA1026-MIT Echivalent: : N.L.A.

2SA1027-MIT SI-P 50V 0.2A 0.25W 100MH

2SA1029-HIT SI-P 30V 0.1A 0.2W 280MHz

2SA1034-MAT SI-P 35V 50mA 0.2W 200MHz

2SA1037-RHM SI-P 50V 0.4A 140MHz FR

2SA1038 Echivalent: 2SA970-MBR

2SA1039 Echivalent: 2SA970-MBR

2SA1048-TOS SI-P 50V 0.15A 0.2W 80MHz

2SA1049-TOS SI-P 120V 0.1A 0.2W 100MH

2SA1061-MAT SI-P 100V 6A 70W 15MHz

2SA1062 SI-N 120V 7A 80W 15MHz

2SA1065 SI-P 150V 10A 120W 50MHz

2SA1066 Echivalent: 2SB985-SAN

2SA1075-FUJ Echivalent: 2SA1215-SKN

2SA1076 Echivalent: 2SA1216-SKN

2SA1084-HIT SI-P 90V 0.1A 0.4W 90MHz

2SA1091 Echivalent: BF421-MBR

2SA1093 Echivalent: 2SA1265N-TOS

2SA1094 Echivalent: 2SA1295-SKN

2SA1095-TOS Echivalent: 2SA1295-SKN

2SA1102-SKN Echivalent: : 2SA1489-SKN

2SA1103-SKN SI-P 100V 7A 70W 20MHz

2SA1104-SKN Echivalent: : 2SA1490-SKN

2SA1105 Echivalent: 2SA1106-SKN

2SA1106-SKN Echivalent: 2SA1106/DIV

2SA1106/DIV SI-P 140V 10A 100W 20MHZ

2SA1108 Echivalent: 2SA1302-TOS

2SA1110-MAT SI-P 120V 0.5A 5W 250MHz

2SA1111-MAT SI-P 150V 1A 20W 200MHz

2SA1112-MAT SI-P 180V 1A 20W 200MHz

2SA1115-MIT SI-P 50V 0.2A 200MHz UNI

2SA1116-SKN Echivalent: 2SA1494-SKN

2SA1117-SKN Echivalent: : N.L.A.

2SA1120-TOS SI-P 35V 5A 170MHz

2SA1123-MAT SI-P 150V 50mA 0.75W 200M

2SA1124-MAT SI-P 150V 50mA 1W 200MHz

2SA1127-MAT SI-P 60V 0.1A 0.4W 200MHz

2SA1133 Echivalent: 2SB861-HIT

2SA1135 Echivalent: 2SA1489-SKN

2SA114 Echivalent: AF200

2SA1141-NEC SI-P 115V 10A 100W 90MHz

2SA1142-NEC SI-P 180V 0.1A 8W 180MHZ

2SA1144-TOS Echivalent: 2SA1360-TOS

2SA1145-TOS SI-P 150V 50mA 0.8W 200MH

2SA1146-TOS Echivalent: 2SA1265N-TOS

2SA1150-TOS SI-P 35V 0.8A 0.3W 120MHz

2SA1156-NEC SI-P 400V 0.5A 10W POWER

2SA1160-TOS SI-P 20V 2A 0.9W 150MHz

2SA1162 Echivalent: BC856A-MBR

2SA1163-TOS SI-P 120V 0.1A 100MHz

2SA1169 Echivalent: 2SA1494-SKN

2SA1170-SKN Echivalent: : 2SA1170/DIV

2SA1170/DIV SI-P 200V 17A 200W 20MHZ

2SA1175-NEC Echivalent: 2SA1048-TOS

2SA1184-TOS Echivalent: 2SA1358-TOS

2SA1185-MAT SI-P 50V 7A 60W 100MHz

2SA1186-SKN SI-P 150V 10A 100W

2SA1200-TOS SI-P 150V 50mA 0.5W 120MH

2SA1201-TOS SI-P 120V 0.8A 0.5W 120MH

2SA1206-NEC SI-P 15V 0.05A 0.6W

2SA1207-SAN SI-P 180V 70mA 0.6W 150MH

2SA1208-SAN SI-P 180V 0.07A 0.9W

2SA1209-SAN SI-P 180V 0.14A 10W

2SA1210-SAN SI-P 200V 0.14A 10W

2SA1213-TOS SI-P 50V 2A 0.5W 120MHz

2SA1215-SKN SI-P 160V 15A 150W 50MHz

2SA1216-SKN SI-P 180V 17A 200W 40MHz

2SA1220A-MBR SI-P 120V 1.2A 20W 160MHz

2SA1221-SAN SI-P 160V 0.5A 1W 45MHz

2SA1225-TOS SI-P 160V 1.5A 15W 100MHz

2SA1227A-NEC SI-P 140V 12A 120W 60MHz

2SA1232-NEC SI-P 130V 10A 100W 60MHz

2SA1241-TOS SI-P 50V 2A 10W 100MHz

2SA1242-TOS SI-P 35V 5A 1W 170MHz

2SA1244-TOS SI-P 60V 5A 20W 60MHz

2SA1246 Echivalent: BC556B-MBR

2SA1248 Echivalent: 2SA1249-SAN

2SA1249-SAN SI-P 180V 1.5A 10W 120MHz

2SA1252 Echivalent: BC856B-MBR

2SA1261-NEC SI-P 100V 10A 60W POWER

2SA1262-SKN SI-P 60V 4A 30W 15MHz

2SA1263N-TOS Echivalent: 2SA1264N-TOS

2SA1264N-TOS SI-P 120V 8A 80W 30MHz

2SA1264N/DIV SI-P 120V 8A 80W 30MHZ

2SA1265N-TOS SI-P 140V 10A 100W 30MHz

2SA1266-KEC SI-P 50V 0.15A 0.4W POWER

2SA1268-KEC SI-N 120V 0.1A 0.3W 100MH

2SA1270-KEC SI-P 35V 0.5A 0.5W 200MH

2SA1271-KEC SI-P 30V 0.8A 0.6W 120MHz

2SA1273-KEC Echivalent: KTA1273-KEC

2SA1275-KEC SI-P 160V 1A 0.9W 20MHz

2SA1276-KEC Echivalent: 2SA473-MBR

2SA1282-MIT SI-P 20V 2A 0.9W 80MHz

2SA1283-MIT SI-P 60V 1A 0.9W 85MHz

2SA1284 Echivalent: BC640-MBR

2SA1285 Echivalent: 2SA970-MBR

2SA1286-MIT SI-P 30V 1.5A 0.9W 90MHz

2SA1287-MIT SI-P 50V 1A 0.9W 90MHz

2SA1292-SAN SI-P 80V 15A 70W 100MHz

2SA1293-TOS SI-P 100V 5A 30W 0.2us

2SA1294-SKN SI-P 230V 15A 130W

2SA1294/DIV SI-P 230V 15A 130W

2SA1295-SKN SI-P 230V 17A 200W 35MHZ

2SA1296-TOS SI-P 20V 2A 0.75W 120MHz

2SA1298-TOS SI-P 30V 0.8A 0.2W 120MHZ

2SA1300-TOS SI-P 10V 2A 0.75W 140MHz

2SA1301 Echivalent: 2SA1302-TOS

2SA1302-TOS SI-P 200V 15A 150W 25MHz

2SA1302/DIV SI-P 200V 15A 150W 25MHZ

2SA1303-SKN SI-P 150V 14A 125W 50MHz

2SA1306-TOS SI-P 160V 1.5A 20W

2SA1306A-TOS SI-P 180V 1.5A 20W 100MHz

2SA1307-TOS SI-P 60V 5A 20W 0.1us

2SA1309-MAT SI-P 30V 0.1A 0.3W 80MHz

2SA1310-MAT SI-P 60V 0.1A 0.3W 200MHz

2SA1315-TOS SI-P 80V 2A 0.9W 0.2us

2SA1316-TOS SI-P 80V 0.1A 0.4W 50MHz

2SA1317-SAN SI-P 60V 0.2A 0.3W 200MHz

2SA1318-SAN SI-P 60V 0.2A 0.5W 200MHz

2SA1319-SAN SI-P 180V 0.7A 0.7W 120MH

2SA1321-TOS SI-P 250V 50mA 0.9W 100MH

2SA1328-TOS SI-P 60V 12A 40W 0.3us

2SA1329-TOS SI-P 80V 12A 40W 0.3us

2SA1345-SAN SI-N 50V 0.1A 0.3W 250MHz

2SA1346-SAN SI-P 50V 0.1A 200MHz

2SA1348-SAN SI-P 50V 0.1A 200MHz

2SA1349-TOS P-ARRAY 80V 0.1A 0.4W 170

2SA1350 Echivalent: 2SA1351-TOS

2SA1352-SAN SI-P 200V 0.1A 5W 70MHz

2SA1357-TOS SI-P 35V 5A 10W 170MHz

2SA1358-TOS SI-P 120V 1A 10W 120MHz

2SA1359-TOS SI-P 40V 3A 10W 100MHz

2SA1360-TOS SI-P 150V 50mA 5W 200MHz

2SA1361-TOS SI-P 250V 50mA 80MHz

2SA1370-SAN SI-P 200V 0.1A 1W 150MHz

2SA1371E-SAN SI-P 300V 0.1A 1W 150MHz

2SA1376-NEC SI-P 200V 0.1A 0.75W 120M

2SA1380-SAN SI-P 200V 0.1A 1.2W

2SA1381-MBR SI-P 300V 0.1A 150MHz

2SA1382-TOS SI-P 120V 2A 0.9W 0.2us

2SA1383 SI-P 180V 0.1A 10W 180MHz

2SA1386-SKN SI-P 160V 15A 130W 40MHz

2SA1386/DIV SI-P 160V 15A 130W 40MHZ

2SA1387-TOS SI-P 60V 5A 25W 80MHz

2SA1389 Echivalent: 2SA1302-TOS

2SA1392-SAN SI-P 60V 0.2A 0.4W 200MHz

2SA1396-NEC SI-P 100V 10A 30W

2SA1399-MIT SI-P 55V 0.4A 0.9W 150MHz

2SA1400-NEC SI-P 400V 0.5A 10W

2SA1402 Echivalent: 2SA1403-SAN

2SA1403-SAN SI-P 80V 0.5A 10W 800MHz

2SA1405-SAN SI-P 120V 0.3A 8W 500MHz

2SA1406-SAN SI-P 200V 0.1A 7W 400MHz

2SA1407-SAN SI-P 150V 0.1A 7W 400MHz

2SA1413-NEC SI-P 600V 1A 10W 26MHz

2SA1428-TOS SI-P 50V 2A 1W 100MHz

2SA1431-TOS SI-P 35V 5A 1W 170MHz

2SA144 Echivalent: DTA144xx

2SA1441-NEC SI-P 100V 5A 25W <300ns

2SA1442-NEC Echivalent: 2SA1443-NEC

2SA1443-NEC SI-P 100V 10A 30W

2SA1450-SAN SI-P 100V 0.5A 0.6W 120MH

2SA1451-TOS SI-P 60V 12A 30W 70MHz

2SA1460-NEC SI-P 60V 1A 1W <40NS

2SA1470-SAN SI-P 80V 7A 25W 100MHz

2SA1475-SAN SI-P 120V 0.4A 15W 500MHz

2SA1476-SAN SI-P 200V 0.2A 15W 400MHz

2SA1477-SAN SI-P 180V 0.14A 10W 150MH

2SA1488-SKN SI-P 60V 4A 25W 15MHz

2SA1489-SKN SI-P 80V 6A 60W 20MHz

2SA1490-SKN SI-P 120V 8A 80W 20MHz

2SA1491-SKN SI-P 140V 10A 100W 20MHz

2SA1492 Echivalent: 2SA1386-SKN

2SA1493 Echivalent: 2SA1494-SKN

2SA1494-SKN SI-P 200V 17A 200W 20MHz

2SA1507-SAN SI-P 180V 1.5A 10W 120MHz

2SA1515-RHM SI-P 40V 1A 0.3W 150MHz

2SA1516-TOS SI-P 180V 12A 130W 25MHz

2SA1516/DIV SI-P 180V 12A 130W 25MHZ

2SA1519-SAN SI-P 50V 0.5A 0.3W 200MHz

2SA1535-MAT Echivalent: 2SA1535A-MAT

2SA1535A-MAT SI-P 180V 1A 40W 200MHz

2SA1538-SAN SI-P 120V 0.2A 8W 400MHz

2SA1539-SAN SI-P 120V 0.3A 8W 400MHZ

2SA1540-SAN SI-P 200V 0.1A 7W 300MHz

2SA1541-SAN SI-P 200V 0.2A 7W 300MHz

2SA1553-TOS SI-P 230V 15A 150W 25MHz

2SA1566-HIT SI-N 120V 0.1A 0.15W 130M

2SA1567-SKN SI-P 50V 12A 35W 40MHz

2SA1568-SKN SI-P 60V 12A 40W

2SA1577-RHM SI-P 32V 0.5A 0.2W 200MHz

2SA1593-SAN SI-P 120V 2A 15W 120MHz

2SA1601-SHI SI-P 60V 15A 45W

2SA1606-SAN SI-P 180V 1.5A 15W 100MHz

2SA1615-NEC SI-P 30V 10A 15W 180MHz

2SA1624-SAN SI-P 300V 0.1A 0.5W 70MHz

2SA1625-NEC SI-P 400V 0.5A 0.75W

2SA1626-NEC SI-P 400V 2A 1W 0.5/2.7us

2SA1633-RHM SI-P 150V 10A 100W 20MHz

2SA1643-SKN SI-P 50V 7A 25W 75MHZ

2SA1667-SKN SI-P 150V 2A 25W 20MHz

2SA1668-SKN SI-P 200V 2A 25W 20MHz

2SA1670-SKN SI-P 80V 6A 60W 20MHz

2SA1671-SKN SI-P 120/120V 8A 75W 20MH

2SA1672-SKN SI-P 140V 10A 80W 20MHz

2SA1673-SKN SI-P 180V 15A 85W 20MHz

2SA1673/DIV SI-P 180V 15A 85W 20MHZ

2SA1680-TOS SI-P 60V 2A 0.9W 100/400n

2SA1684-NEC SI-P 120V 1.5A 20W 150MHz

2SA1693-SKN Echivalent: 2SA1694-SKN

2SA1694-SKN SI-P 120/120V 8A 80W 20MH

2SA1695-SKN SI-P 140V 10A 80W 20MHz

2SA1703-SAN SI-P 30V 1.5A 1W 180MHz

2SA1706-SAN SI-P 60V 2A 1W

2SA1708-SAN SI-P 120V 1A 1W 120MHz

2SA1726-SKN SI-P 80V 6A 50W 20MHZ

2SA1776-RHM SI-P 400V 1A 1W

2SA1803-TOS SI-P 80V 6A 55W 30MHz

2SA1837-TOS SI-P 230V 1A 20W 70MHz

2SA1930-TOS SI-P 180V 2A 20W 200MHZ

2SA1962-TOS SI-P 230V 15A 130W 25MHZ

2SA200 GE-P TRANSISTOR

2SA234 Echivalent: AF200

2SA329-SAN GE-P 15V 10mA 0.05W

2SA467-TOS SI-P 40V 0,4A 0,3W

2SA473-MBR SI-P 30V 3A 10W 100MHz

2SA483-TOS SI-P 150V 1A 20W 9MHz

2SA489 Echivalent: BD242C-MBR

2SA490 Echivalent: BD242C-MBR

2SA493-TOS SI-P 50V 0.05A 0.2W 80Mhz

2SA495-TOS SI-P 35V 0.1A 0.2W 200MHz

2SA496 Echivalent: BD140-MBR

2SA509 Echivalent: BC557B-MBR

2SA539-NEC Echivalent: 2SA1392-SAN

2SA545 Echivalent: BC640-MBR

2SA561 Echivalent: 2SA1015-MBR

2SA562 SI-P 30V 0.5A 0.5W 200MHz

2SA564 Echivalent: BC557B-MBR

2SA566-HIT SI-P 100V 0.7A 10W 100MHz

2SA603-NEC Echivalent: : N.L.A.

2SA606-NEC Echivalent: : N.L.A.

2SA608-SAN SI-N 40V 0.1A 0.1W 180MHz

2SA614-MBR SI-P 80V 1A 15W 30MHz

2SA620 SI-P 30V 0.05A 0.2W 120MH

2SA626-NEC SI-P 80V 5A 60W 15MHz

2SA628-MIT SI-P 30V 0.1A 100MHz

2SA636-NEC Echivalent: : N.L.A.

2SA639-NEC SI-P 180V 50mA 0,25W

2SA640 Echivalent: 2SA970-MBR

2SA641-NEC Echivalent: : N.L.A.

2SA642-NEC SI-P 30V 0.2A 0.25W 200MH

2SA643-MBR SI-P 40V 0.5A 0.5W 180MHz

2SA653-NEC SI-P 150V 1A 15W 5MHz

2SA659 Echivalent: 2SA984-SAN

2SA661 Echivalent: 2SA817A-TOS

2SA666 Echivalent: 2SA1127-MAT

2SA671 Echivalent: BD244C-MBR

2SA673 Echivalent: BC327-25-MBR

2SA678 Echivalent: BC327-25-MBR

2SA679 Echivalent: 2SA1265N-TOS

2SA683 Echivalent: 2SA684-MAT

2SA684-MAT SI-P 60V 1A 1W 200MHz

2SA695 Echivalent: BC640-MBR

2SA697 Echivalent: BC327-25-MBR

2SA699-MAT SI-P 40V 2A 10W 150MHz

2SA705 Echivalent: BC327-25-MBR

2SA708A-NEC SI-P 100V 0.7A 0.8W 50MHz

2SA715 Echivalent: BD140-MBR

2SA719 Echivalent: 2SA720-MAT

2SA720-MAT SI-P 60V 0.5A 0.6W 200MHz

2SA721 Echivalent: 2SA1127-MAT

2SA722 Echivalent: 2SA1127-MAT

2SA725-MIT SI-P 35V 0.1A 0.15W 100MH

2SA726-MIT Echivalent: : N.L.A.

2SA733-MBR SI-P 60V 0.15A 0.25W 50MH

2SA733Y Echivalent: KSA733

2SA738-HIT SI-P 25V 1.5A 8W 160MHz

2SA740-TOS Echivalent: : N.L.A.

2SA743 Echivalent: BD140-MBR

2SA745 Echivalent: 2SA747-SKN

2SA747-SKN Echivalent: : N.L.A.

2SA750 Echivalent: BC560B-MBR

2SA756-HIT SI-P 100V 6A 50W 20MHz

2SA761 Echivalent: 2N5322-MBR

2SA762-SNY SI-P 110V 2A 23W 80MHz

2SA763 Echivalent: : N.L.A.

2SA764-SKN Echivalent: 2SA765-SKN

2SA765-SKN SI-P 80V 6A 40W 10MHz

2SA768-SKN SI-P 60V 4A 30W 10MHz

2SA769-SKN SI-P 80V 4A 30W 10MHz

2SA769/DIV Echivalent: 2SA769-SKN

2SA770-SKN SI-P 60V 6A 40W 10MHz

2SA771 SI-P 80V 6A 40W 2MHz

2SA773 Echivalent: : N.L.A.

2SA777-MAT SI-P 80V 0.5A 0.75W 120MH

2SA778 Echivalent: 2SA778A-HIT

2SA778A-HIT SI-P 180V 0.05A 0.2W 60MH

2SA781-HIT SI-P 20V 0.2A 0.2W <80/16

2SA794-MAT SI-P 100V 0.5A 5W 120MHz

2SA794A-MAT SI-P 120V 0.5A 5W 120MHz

2SA798-MIT Echivalent: : N.L.A.

2SA808 Echivalent: 2SA756-HIT

2SA812-NEC SI-P 50V 0.1A 0.15W

2SA814 Echivalent: 2SA814/DIV

2SA814-TOS Echivalent: : N.L.A.

2SA814/DIV SI-P 120V 1A 15W 30MHZ

2SA815 Echivalent: BD240C-MBR

2SA816-TOS SI-P 80V 0.75A 1.5W 100MH

2SA817-TOS SI-P 80V 0.3A 0.6W 100MHz

2SA817A-TOS SI-P 80V 0.4A 0.8W 100MHz

2SA818 Echivalent: 2SA1145-TOS

2SA822-RHM Echivalent: 2SA1123-MAT

2SA823 Echivalent: 2SA822-RHM

2SA824 Echivalent: 2SA822-RHM

2SA836-HIT SI-P 55V 0.1A 0.2W 100MHz

2SA838-MAT SI-P 30V 30mA 0.25W 300MH

2SA839-TOS SI-P 150V 1.5A 25W 6MHz

2SA840 Echivalent: MPSA92-MBR

2SA841-TOS SI-P 60V 0.05A 0.2W 140MH

2SA844 Echivalent: BC556A-MBR

2SA850 Echivalent: BC640-MBR

2SA854 Echivalent: BC327-40-MBR

2SA858-FUJ SI-P 150V 50mA 0.5W 100MH

2SA872-HIT SI-P 90V 0.05A 0.2W 120MH

2SA872A-HIT SI-P 120V 50mA 0.3W 120MH

2SA882 Echivalent: 2SB946-MAT

2SA884-SNY SI-P 65V 0.2A 0.27W 140MH

2SA885-MAT SI-P 45V 1A 5W 200MHz

2SA886-MAT SI-P 50V 1.5A 1.2W

2SA893-HIT SI-P 90V 50mA 0.3W

2SA900-MAT SI-P 18V 1A 1.2W

2SA904-MIT Echivalent: : N.L.A.

2SA909 Echivalent: MJ15023-MOT

2SA912 Echivalent: 2SA1124-MAT

2SA913-MAT Echivalent: 2SA1111-MAT

2SA914-MAT SI-P 150V 0.05A 200MHz

2SA915-NEC SI-P 120V 0.05A 0.8W 80Mh

2SA916-NEC SI-P 160V 0.05A 1W 80MHZ

2SA921-MAT SI-P 120V 20mA 0.25W 200M

2SA927 Echivalent: BC327-25-MBR

2SA933-RHM SI-P 50V 0.1A 0.3W

2SA934-RHM SI-P 40V 0.7A 0.75W

2SA935-RHM SI-P 80V 0.7A 0.75W 150MH

2SA937-RHM SI-P 50V 0.1A 0.3W 140MHz

2SA939 Echivalent: MJE350-MBR

2SA940-MBR SI-P 150V 1.5A 25W 4MHz

2SA941-TOS SI-P 120V 0.05A 0.3W 150M

2SA942 Echivalent: 2SA941-TOS

2SA949-TOS SI-P 150V 50mA 0.8W 120MH

2SA950 Echivalent: BC327-25-MBR + KLA

2SA952 Echivalent: BC327-16-MBR

2SA953 Echivalent: BC640-MBR

2SA954 Echivalent: BC640-MBR

2SA957-SKN Echivalent: 2SA1668-SKN

2SA958-SKN Echivalent: 2SA1668-SKN

2SA965-TOS SI-P 120V 0.8A 0.9W 120MH

2SA966-TOS SI-P 30V 1.5A 0.9W 120MHz

2SA968-MBR SI-P 160V 1.5A 25W 100MHz

2SA970-MBR SI-P 120V 0.1A 100MHz

2SA973 Echivalent: BC560B-MBR

2SA979-MIT Echivalent: : N.L.A.

2SA980-SKN Echivalent: 2SA982

2SA981-SKN Echivalent: 2SA982

2SA982 SI-P 140V 8A 80W 20MHz

2SA984-SAN SI-P 60V 0.5A 0.5W 120MHz

2SA985-NEC SI-P 120V 1.5A 25W 180MHz

2SA988-NEC SI-P 120V 0.05A 0.5W

2SA991-NEC SI-P 60V 0.1A 0.5W 90MHz

2SA992 SI-P 100V 0.05A 0.2W

2SA995-MIT SI-P 100V 0.05A 0.4W 100M

2SA999 Echivalent: BC556A-MBR

2SB1009-RHM SI-P 40V 2A 10W 100MHz

2SB1010-RHM SI-P 40V 2A 0.75W 100MHz

2SB1012K-HIT P-DARL 120V 1.5A 8W

2SB1013-SAN SI-P 20V 2A 0.7W

2SB1015-TOS SI-P 60V 3A 25W 0.4us

2SB1016-TOS SI-P 100V 5A 30W 5MHz

2SB1017-TOS SI-P 80V 4A 25W 9MHz

2SB1018-TOS SI-P 100V 7A 30W 0.4us

2SB1020-TOS P-DARL+D 100V 7A 30W 0.8u

2SB1023-TOS P-DARL+D 60V 3A 20W B=5K

2SB1035-MIT SI-P 30V 1A 0.9W 100MHz

2SB1038-NEC Echivalent: 2SB1039-NEC

2SB1039-NEC SI-P 100V 4A 40W 20MHz

2SB1050-MAT SI-P 30V 5A 1W 120MHz

2SB1055-MAT SI-P 120V 6A 70W 20MHz

2SB1065-MBR SI-P 60V 3A 10W

2SB1066-RHM SI-P 50V 3A 1W 70MHz

2SB1068-NEC SI-P 20V 2A 0.75W 180MHz

2SB1071-MAT SI-P 40V 4A 25W 150MHz

2SB1077-HIT P-DARL 60V 4A 40W B>1K

2SB1086-MBR SI-P 160V 1.5A 20W 50MHz

2SB1098-NEC P-DARL+D 100V 5A 20W B=80

2SB1099-NEC P-DARL+D 100V 8A 25W B=6K

2SB1100-NEC P-DARL+D 100V 10A 30W B=6

2SB1109-HIT SI-P 160V 0.1A 1.25W

2SB1109S SI-P 160V 0.1A 1.25W

2SB1117-NEC SI-P 30V 3A 1W 280MHz

2SB1120-SAN SI-P 20V 2.5A 0.5W 250MHz

2SB1121T-SAN SI-P 30V 2A 150MHz

2SB1122 Echivalent: 2SB1123-SAN

2SB1123-SAN SI-P 60V 2A 0.5W 150MHz

2SB1132-RHM SI-P 40V 1A 0.5W 150MHz

2SB1133-SAN SI-P 60V 3A 25W 40MHz

2SB1134-SAN SI-P 60V 5A 25W 30W

2SB1135-SAN SI-P 60V 7A 30W 10MHz

2SB1136-SAN SI-P 60V 12A 30W 10MHz

2SB1140-SAN SI-P 25V 5A 10W 320MHz

2SB1141-SAN SI-P 20V 1.2A 10W 150MHz

2SB1142-SAN Echivalent: 2SB1143-SAN

2SB1143-SAN SI-P 60V 4A 10W 140MHz

2SB1146-HIT P-DARL 120V 6A 25W

2SB1149-NEC P-DARL 100V 3A 15W B=10K

2SB1151-NEC SI-P 60V 5A 20W

2SB1154-MAT SI-P 130V 10A 70W 30MHz

2SB1155 Echivalent: 2SB1156-MAT

2SB1156-MAT SI-P 130V 20A 100W

2SB1162-MAT SI-P 160V 12A 120W

2SB1163-MAT SI-P 170V 15A 150W

2SB1166-SAN SI-P 60V 8A 20W 130MHz

2SB1168-SAN SI-P 120V 4A 20W 130MHz

2SB1182-RHM SI-P 40V 2A 10W 100MHZ

2SB1184-RHM SI-P 60V 3A 15W 70MHZ

2SB1185-RHM SI-P 50V 3A 25W 70MHz

2SB1186-RHM SI-P 120V 1.5A 20W 50MHz

2SB1187-RHM SI-P 80V 3A 35W

2SB1188-RHM SI-P 40V 2A 100MHz

2SB1202-SAN SI-P 60V 3A 15W 150MHz

2SB1202-SMD Echivalent: 2SB1202-SAN

2SB1203-SAN SI-P 60V 5A 20W 130MHz

2SB1204-SAN SI-P 60V 8A 20W 130MHz

2SB1205-SAN SI-P 25V 5A 10W 320MHz

2SB1212-RHM SI-P 160V 1.5A 0.9W 50MHz

2SB1223-SAN P-DARL+D 70V 4A 20W 20MHz

2SB1236-RHM SI-P 120V 1.5A 1W 50MHz

2SB1237-RHM SI-P 40V 1A 1W 150MHz

2SB1238-RHM SI-P 80V 0.7A 1W 100MHz

2SB1240-RHM SI-P 40V 2A 1W 100MHz

2SB1243-RHM SI-P 60V 3A 1W

2SB1253-MAT Echivalent: 2SB1254-MAT

2SB1254-MAT P-DARL 160V 7A 70W

2SB1255-MAT P-DARL 160V 8A 100W B>5K

2SB1258-SKN P-DARL+D 100V 6A 30W B>1K

2SB1274-SAN SI-P 60V 3A 30W 100MHz

2SB1282-SHI P-DARL+D 100V 4A 25W 50MH

2SB1292-RHM SI-P 80V 5A 30W

2SB1302-SAN SI-P 25V 5A 320MHz

2SB1318-NEC P-DARL+D 100V 3A 1W B>200

2SB1326-RHM SI-P 30V 5A 0.3W 120MHZ

2SB1329-RHM SI-P 40V 1A 1.2W 150MHz

2SB1330-RHM SI-P 32V 0.7A 1.2W 100MHz

2SB1331-RHM SI-P 32V 2A 1.2W 100MHz

2SB1353E-RHM SI-P 120V 1.5A 1.8W 50MHz

2SB1361-MAT SI-P 150V 9A 100W 15MHZ

2SB1367 Echivalent: 2SB1016-TOS

2SB1370-RHM SI-P 60V 3A 30W 15MHz

2SB1373-MAT SI-P 160V 12A 2.5W 15MHz

2SB1375-TOS SI-P 60V 3A 25W 9MHz

2SB1382-SKN P-DARL+D 120V 16A 75W B>2

2SB1393-MAT SI-P 30V 3A 2W 30MHz

2SB1420-SKN SI-P 120V 16A 80W 50MHz

2SB1425-RHM SI-P 20V 2A 1W 90MHz

2SB1429-TOS SI-P 180V 15A 150W 10MHz

2SB1434-MAT SI-P 50V 2A 1W 110MHz

2SB1468 SI-P 60/30V 12A 25W

2SB1470-MAT P-DARL 160V 8A 150W B>5K

2SB1490 P-DARL 160V 7A 90W B>5K

2SB1493-MAT P-DARL 160/140V 7A 70W 20

2SB1503-MAT P-DARL 160V 8A 120W B>5K

2SB1531-MAT TRANSISTOR

2SB1556-TOS P-DARL 140V 8A 120W B>5K

2SB1557-TOS P-DARL 140V 7A 100W B>5K

2SB1559-SKN P-DARL 160V 8A 80W B>5K

2SB1560-SKN P-DARL 160V 10A 100W 50MH

2SB1565-RHM SI-P 80V 3A 25W 15MHZ

2SB1587-SKN P-DARL+D 160V 8A 70W B>5K

2SB1624-SKN P-DARL 110V 6A 60W B>5K

2SB175-MAT Echivalent: : N.L.A.

2SB187 Echivalent: AC151

2SB206-SHI GE-P 80V 30A 80W

2SB324 GE-P 32V 1A 0.25W

2SB337 GE-P 50V 7A 30W LF-POWER

2SB405 Echivalent: AC188K

2SB407 GE-P 30V 7A 30W

2SB471-HIT Echivalent: 2SB206-SHI

2SB474 Echivalent: : N.L.A.

2SB481 GE-P 32V 1A 6W 15KHz

2SB492-SAN GE-P 25V 2A 6W

2SB507 Echivalent: BD242C-MBR

2SB509-SAN Echivalent: : N.L.A.

2SB511E-SAN SI-P 35V 1.5A 10W 8MHz

2SB512 Echivalent: 2SB941-MAT

2SB514 Echivalent: BD242C-MBR

2SB524-MIT SI-P 60V 1.5A 10W 70MHz

2SB525-MIT Echivalent: : N.L.A.

2SB526 Echivalent: 2SB527-MIT

2SB527-MIT SI-P 110V 0.8A 10W 70MHz

2SB528 Echivalent: 2N4920-MBR

2SB529 Echivalent: BD242C-MBR

2SB531 SI-P 90V 6A 50W 8MHz

2SB536-NEC SI-P 130V 1.5A 20W 40MHz

2SB537-NEC SI-P 130V 1.5A 20W 60MHz

2SB539 Echivalent: : N.L.A.

2SB541-MBR SI-P 110V 8A 80W 9MHz

2SB544-SAN SI-P 25V 1A 0.9W 180MHz

2SB546A-NEC SI-P 200V 2A 25W 5MHz

2SB548 Echivalent: BD140-MBR

2SB549-NEC SI-P 120V 0.8A 10W 80MHz

2SB554 Echivalent: 2SB600

2SB555 Echivalent: 2SB681

2SB556 Echivalent: 2SB681

2SB557 SI-P 120V 8A 80W

2SB558 Echivalent: 2SB688-TOS

2SB56 Echivalent: AC151

2SB560-SAN SI-P 100V 0.7A 0.9W 100MH

2SB561-HIT SI-P 25V 0.7A 0.5W

2SB562 Echivalent: 2SB561-HIT

2SB564 SI-P 30V 1A 0.8W

2SB595 Echivalent: BD244C-MBR

2SB596 Echivalent: TIP42C-MBR

2SB598-SAN SI-P 25V 1A 0.5W 180MHz

2SB600 SI-P 200V 15A 200W 4MHz

2SB601-NEC P-DARL 100V 5A 30W

2SB605-NEC SI-P 60V 0.7A 0.8W 120MHz

2SB617 Echivalent: TIP42C-MBR

2SB618-NEC Echivalent: : N.L.A.

2SB621 SI-N 25V 1.5A 0.6W 200MHz

2SB621A-MAT SI-N 50V 1A 0.75W 200MHz

2SB628 Echivalent: 2SA1111-MAT

2SB631-SAN SI-P 100V 1A 8W

2SB632-SAN SI-P 25V 2A 10W 100MHz

2SB633-SAN SI-P 100V 6A 40W 15MHz

2SB637-HIT SI-P 50V 0.1A 0.3W 200MHz

2SB641-MAT SI-P 30V 0.1A 120MHz

2SB642 Echivalent: BC556B-MBR

2SB643 Echivalent: BC640-MBR

2SB644 Echivalent: BC327-25-MBR

2SB645 Echivalent: 2SB600

2SB646-HIT Echivalent: 2SB647-HIT

2SB647-HIT SI-P 120V 1A 0.9W 140MHz

2SB648 Echivalent: BF472-PHI

2SB649-MBR Echivalent: 2SB649A-MBR

2SB649A-MBR SI-P 160V 1.5A 1W 140MHz

2SB656-HIT SI-P 160V 12A 125W 20MHz

2SB673-MBR P-DARL+D 100V 7A 40W 0.8u

2SB675 Echivalent: 2SB673-MBR

2SB676-TOS P-DARL 100V 4A 30W 0.15us

2SB681 SI-N 150V 12A 100W 13MHZ

2SB686 Echivalent: BD250C-MBR

2SB688-TOS SI-P 120V 8A 80W 10MHz

2SB690 Echivalent: BD244C-MBR

2SB695-MAT Echivalent: : N.L.A.

2SB697 Echivalent: 2SB817-SAN

2SB698 Echivalent: BC327-25-MBR

2SB700 SI-P 160V 12A 100W

2SB703-NEC SI-P 100V 4A 40W 18MHz

2SB705-NEC SI-P 140V 10A 120W 17MHz

2SB705/DIV SI-P 140V 10A 120W 17MHZ

2SB707-NEC SI-P 80V 7A 40W POWER

2SB709-MAT SI-P 45V 0.1A 0.2W 80MHz

2SB716-HIT SI-P 120V 0.05A 0.75W

2SB720 Echivalent: 2SB720/DIV

2SB720-HIT Echivalent: : N.L.A.

2SB720/DIV SI-P 200V 2A 25W 100MHZ

2SB727-HIT P-DARL+D 120V 6A 50W B>1K

2SB731-NEC SI-P 60V 1A 10W 75MHz

2SB733-NEC SI-P 20V 2A 1W >50MHz

2SB734-NEC SI-P 60V 1A 1W 80MHz

2SB737-RHM Echivalent: : N.L.A.

2SB739-HIT SI-P 20/16V 2A 0.9W 80MHz

2SB740-HIT SI-P 70V 1A 0.9W

2SB744-MBR SI-P 70V 3A 10W 45MHz

2SB75 Echivalent: AC151

2SB750-MAT P-DARL+D 60V 2A 35W B>100

2SB751-MAT Echivalent: 2SB950A-MAT

2SB753-TOS SI-P 100V 7A 40W 0.4us

2SB754 Echivalent: 2SB827-SAN

2SB755-TOS Echivalent: 2SA1302-TOS

2SB756 Echivalent: 2SA1302-TOS

2SB761 Echivalent: 2SB941-MAT

2SB764-SAN SI-P 60V 1A 0.9A 150MHz

2SB765-HIT P-DARL+D 120V 3A 30W B>1K

2SB766-MAT SI-P 30V 1A 200MHz

2SB772-MBR SI-P 40V 3A 10W 80MHz

2SB774-MAT SI-P 30V 0.1A 0.4W 150MHz

2SB775-SAN SI-P 100V 6A 60W 13MHz

2SB776-SAN SI-P 120V 7A 70W 15MHz

2SB778-FEC Echivalent: 2SA1232-NEC

2SB788-MAT SI-P 120V 0.02A 0.4W 150M

2SB791-HIT P-DARL+D 120V 8A 40W B>10

2SB793 Echivalent: BC327-25-MBR

2SB794-NEC P-DARL+D 60V 1.5A 10W B=7

2SB795-NEC P-DARL+D 80V 1.5A 10W B<3

2SB808-SAN SI-P 20V 0.7A 0.25W 250MH

2SB810-NEC SI-P 30V 0.7A 0.35W 160MH

2SB815-SAN SI-P 20V 0.7A 0.25W 250MH

2SB816-SAN SI-P 150V 8A 80W 15MHz

2SB817-SAN SI-P 160V 12A 100W

2SB817F-CDL SI-P 160V 12A 90W 15MHz

2SB819-MAT SI-P 50V 1.5A 1W 150MHz

2SB822-RHM SI-P 40V 2A 0.75W 100MHz

2SB824-SAN SI-P 60V 5A 30W 30 MHz

2SB825-SAN SI-P 60V 7A 40W 10MHz

2SB826-SAN SI-P 60V 12A 40W 10MHz

2SB827-SAN SI-P 60V 7A 80W 10MHz

2SB828-SAN SI-P 60V 12A 80W 10MHz

2SB829-SAN SI-P 60V 15A 90W 20MHz

2SB834 Echivalent: BD242C-MBR

2SB835-MAT Echivalent: : N.L.A.

2SB843 Echivalent: BD242C-MBR

2SB856 Echivalent: BD242C-MBR

2SB857-MBR SI-P 50V 4A 40W NF/S-L

2SB861-HIT SI-P 200V 2A 30W

2SB863-TOS SI-P 140V 10A 100W 15MHz

2SB865-SAN P-DARL 80V 1.5A 0.9W

2SB870 Echivalent: 2SB946-MAT

2SB873-MAT SI-P 30V 5A 1W 120MHz

2SB882-SAN P-DARL+D 70V 10A 40W B>5K

2SB883-SAN P-DARL+D 70V 15A 70W B=5K

2SB884-SAN P-DARL 110V 3A 30W B=4K

2SB885-SAN P-DARL+D 110V 3A 35W B=4K

2SB891-RHM SI-P 40V 2A 5W 100MHz

2SB892-SAN SI-P 60V 2A 1W

2SB895A-MAT P-DARL 60V 1A B=8000

2SB897-NEC P-DARL+D 100V 10A 80W B>1

2SB908-TOS P-DARL+D 80V 4A 15W 0.15u

2SB909-RHM SI-P 40V 1A 1W 150MHz

2SB911 Echivalent: 2SB1240-RHM

2SB915 Echivalent: BDW84C-MBR

2SB922-SAN SI-P 120V 12A 80W 20MHz

2SB926-SAN SI-P 30V 2A 0.75W

2SB938-MAT Echivalent: 2SB938A-MAT

2SB938A-MAT P-DARL+D 60V 4A 40W B>1K

2SB940-MAT SI-P 200V 2A 35W 30MHz

2SB941-MAT SI-P 60V 3A 35W POWER

2SB945-MAT SI-P 130V 5A 40W 30MHz

2SB946-MAT SI-P 130V 7A 40W 30MHz

2SB949 Echivalent: 2SB950A-MAT

2SB950 Echivalent: 2SB950A-MAT

2SB950A-MAT P-DARL+D 80V 4A 40W B>1K

2SB951 Echivalent: BD902

2SB953 Echivalent: 2SB953A-MAT

2SB953A-MAT SI-P 50V 7A 30W 150MHz

2SB955-HIT P-DARL+D 120V 10A 50W B=4

2SB965 Echivalent: TIP34C-MBR

2SB966 Echivalent: TIP34C-MBR

2SB974-NEC Echivalent: 2SB975-NEC

2SB975-NEC P-DARL+D 100V 8A 40W B>6K

2SB976-MAT SI-P 27V 5A 0.75W 120MHz

2SB985-SAN SI-P 60V 3A 1W 150MHz

2SB986-SAN SI-P 60V 4A 10W 150MHz

2SB988-KEC SI-P 60V 3A 30W <400/2200

2SD1010-MAT SI-N 50V 50mA 0.3W 200MHz

2SD1012-SAN SI-N 20V 0.7A 0.25W 250MH

2SD1018-NEC SI-N 250V 4A 80W B>250

2SD1020 Echivalent: BC337-25-MBR

2SD1021 Echivalent: BC639-MBR

2SD1027-SHI N-DARL+D 20V 15A 100W B>1

2SD1033-NEC SI-N 200V 2A 20W 10MHz

2SD1036-FUJ SI-N 150/120V 15A 150W

2SD1044 Echivalent: 2SD1457-MAT

2SD1046 Echivalent: 2SD1047-SAN

2SD1047-SAN SI-N 160V 12A 100W 15MHz

2SD1048-SAN SI-N 20V 0.7A 0.25W 250MH

2SD1049-FUJ SI-N 120V 25A 100W

2SD1051-MAT SI-N 50V 1.5A 1W 150MHz

2SD1055-RHM SI-N 40V 2A 0.75W 100MHz

2SD1060 Echivalent: 2SD1062-SAN

2SD1061 Echivalent: 2SD1062-SAN

2SD1062-SAN SI-N 60V 12A 40W 10MHz

2SD1063 Echivalent: 2SD1064-SAN

2SD1064-SAN SI-N 60V 12A 80W

2SD1065-SAN SI-N 60V 15A 90W

2SD1071 Echivalent: BU806-MBR

2SD1073-FEC N-DARL 300V 4A 40W B>1K

2SD1088-TOS N-DARL 300V 6A 30W B>2000

2SD1094 Echivalent: S2530A

2SD1111 Echivalent: BC618-MBR

2SD1113K-HIT N-DARL+D 300V 6A 40W

2SD1128-FEC N-DARL 150V 5A 30W

2SD1133 Echivalent: BD243C-MBR

2SD1135-HIT SI-N 80V 4A 40W

2SD1136 Echivalent: 2SD1138-HIT

2SD1137 Echivalent: 2SD1138-HIT

2SD1138-HIT SI-N 200V 2A 30W

2SD1140-TOS N-DARL 30V 1.5A 0.9W

2SD1145-SAN SI-N 60V 5A 0.9W 120MHz

2SD1148-TOS SI-N 140V 10A 100W 20MHz

2SD1153-SAN SI-N 80V 1.5A 0.9W

2SD1159 Echivalent: 2SD1163A-HIT

2SD1163-HIT Echivalent: 2SD1163A-HIT

2SD1163A-HIT SI-N 300V 7A 40W

2SD1164-NEC SI-N 150V 1.5A 10W DAR+DI

2SD1168 Echivalent: BU208A-ST

2SD1173-MAT SI-N+D 1500V 5A 70W

2SD1175-MAT Echivalent: : N.L.A.

2SD1185 Echivalent: 2SD1186-HIT

2SD1186-HIT Echivalent: : N.L.A.

2SD1187-TOS SI-N 100V 10A 80W 10MHz

2SD1189-RHM SI-N 40V 2A 5W 100MHz

2SD1190 Echivalent: 2SD1192-SAN

2SD1192-SAN N-DARL+D 70V 10A 40W B=5K

2SD1196-SAN N-DARL+D 110V 8A 40W B=40

2SD1198-MAT N-DARL 30V 1A 1W 150MHz

2SD1207-SAN SI-N 60V 2A 1W

2SD1210-NEC N-DARL+D 150V 10A 80W B=5

2SD1212 Echivalent: 2SD1062-SAN

2SD1213-SAN SI-N 60V 20A 50W

2SD1225-RHM SI-N 40V 1A 1W 150MHz

2SD1227 Echivalent: 2SC3225-TOS

2SD1238-SAN SI-N 120V 12A 80W 20MHz

2SD1244-MAT SI-N+D 2500/900V 1A 50W

2SD1246-SAN SI-N 30V 2A 0.75W

2SD1247-SAN SI-N 30V 2.5A 1W

2SD1254 SI-N 130V 3A 30W

2SD1255-MAT SI-N 130V 4A 35W 30MHz

2SD1263-MAT Echivalent: 2SD1263A-MAT

2SD1263A-MAT SI-N 400V 0.75A 35W 30MHz

2SD1264-MAT SI-N 200V 2A 30W POWER

2SD1265-MAT SI-N 60V 4A 30W 25kHz

2SD1266-MAT SI-N 60V 3A 35W POWER

2SD1267-MAT SI-N 60V 4A 40W 20MHz

2SD1270-MAT SI-N 130V 5A 2W 30MHz

2SD1271-MAT SI-N 130V 7A 40W 30MHz

2SD1271/DIV Echivalent: 2SD1271-MAT

2SD1272-MAT SI-N 200V 1A 40W 25MHz

2SD1273-MAT SI-N 80V 3A 40W 50MHz

2SD1274-MAT SI-N 150V 5A 40W 40MHz

2SD1275 Echivalent: 2SD1276-MAT

2SD1276-MAT N-DARL 60V 4A 40W

2SD1277 Echivalent: BD901-MBR

2SD1279-TOS Echivalent: : N.L.A.

2SD1286-NEC N-DARL+D 60V 1A 8W B=1K-3

2SD1288-NEC SI-N 120V 7A 70W

2SD1289-NEC SI-N 120V 8A 80W

2SD1291 Echivalent: BU508D-PHI

2SD1292-RHM SI-N 120V 1A 0.9W 100MHz

2SD1293-RHM SI-N 120V 1A 1W 100MHz

2SD1297-NEC N-DARL+D 150V 25A 100W

2SD1302-MAT SI-N 25V 0.5A 0.6W 200MHz

2SD1306-HIT SI-N 30V 0.7A 150mW 250MH

2SD1308-NEC N-DARL+D 150V 8A 40W

2SD1311-NEC Echivalent: : N.L.A.

2SD1313-TOS SI-N 800V 25A 200W 6MHz

2SD1314-TOS N-DARL+D 600V 15A 150W

2SD1330-MAT SI-N 25V 0.5A 0.6W 200MHz

2SD1347-SAN SI-N 60V 3A 1W 150MHz

2SD1348-SAN SI-N 60V 4A 10W 150MHz

2SD1350 Echivalent: 2SD1350A-MAT

2SD1350A-MAT SI-N 600V 0.5A 1W 55MHz

2SD1351-KEC Echivalent: KTD1351-KEC

2SD1376K-HIT N-DARL+D 120V 1.5A 40W

2SD1378-RHM SI-N 80V 0.7A 10W 120MHz

2SD1379-RHM N-DARL 40V 2A 10W 150MHz

2SD1380-MBR SI-N 40V 2A 10W 100MHz

2SD1382-RHM SI-N 120V 1A 10W 100MHz

2SD1384-RHM SI-N 40V 2A 0.75W 100MHz

2SD1391-MAT SI-N 1500V 5A 80W

2SD1392-NEC N-DARL+D 60V 5A 30W B=800

2SD1396 Echivalent: BU508D-PHI

2SD1397-SAN SI-N+D 1500V 3.5A 50W

2SD1398-SAN SI-N+D 1500V 5A 50W

2SD1398/DIV Echivalent: 2SD1398-SAN

2SD1399-SAN SI-N+D 1500V 6A 80W

2SD1402 Echivalent: BU508A-PHI

2SD1403-SAN SI-N 1500V 6A 120W

2SD1404-TOS SI-N+D 300V 7A 25W 1us

2SD1405-TOS SI-N 50V 3A 25W 2us

2SD1406-TOS SI-N 60V 3A 25W 0.8us

2SD1407-TOS SI-N 100V 5A 30W 12MHz

2SD1408-TOS SI-N 80V 4A 30W 8MHz

2SD1409-TOS N-DARL+D 600V 6A 25W 1us

2SD1411 Echivalent: 2SD1411/DIV

2SD1411-TOS Echivalent: : N.L.A.

2SD1411/DIV SI-N 100V 7A 30W 10MHZ

2SD1412-TOS Echivalent: 2SD1411-TOS

2SD1413 Echivalent: 2SD1413/DIV

2SD1413-TOS Echivalent: : N.L.A.

2SD1413/DIV N-DARL+D 60V 3A 20W .O1US

2SD1414-TOS Echivalent: 2SD1196-SAN

2SD1415-TOS N-DARL+D 100V 7A 30W 0.8u

2SD1420 Echivalent: IRFZ44-MBR

2SD1425 Echivalent: 2SD1426-TOS

2SD1426-TOS Echivalent: : 2SD1427-TOS

2SD1426/DIV SI-N+D 1500V 3.5A

2SD1427-TOS SI-N+D 1500V 5A 80W

2SD1428-TOS SI-N+D 1500V 6A 80W

2SD1429 Echivalent: BU2508A-PHI

2SD1430 Echivalent: BU2508A-PHI

2SD1431-TOS Echivalent: : 2SD1432-TOS

2SD1432 Echivalent: 2SD1432/DIV

2SD1432-TOS Echivalent: : 2SD1433-TOS

2SD1432/DIV SI-N 1500V 6A 80W

2SD1433-TOS Echivalent: : BU2508AX-PHI

2SD1433/DIV Echivalent: 2SD1433-TOS

2SD1439-MAT SI-N+D 1500V 3A 50W

2SD1439/DIV Echivalent: 2SD1441/DIV

2SD1441-MAT SI-N+D 1500V 4A 80W

2SD1441/DIV Echivalent: : 2SD1441-MAT

2SD1446-MAT N-DARL+D 500V 6A 40W B>50

2SD1449 Echivalent: 2SD1302-MAT

2SD1453-HIT Echivalent: : N.L.A.

2SD1453/DIV SI-N 1500V 3A 50W

2SD1454 Echivalent: 2SD1455-HIT

2SD1455-HIT Echivalent: : N.L.A.

2SD1456 Echivalent: BU508A-PHI

2SD1457-MAT N-DARL+D 140V 6A 60W

2SD1458-MAT SI-N 20V 0.7A 1W

2SD1468-RHM SI-N 30V 1A 0.3..0.4W 150

2SD1486 Echivalent: 2SD1487-MAT

2SD1487-MAT Echivalent: : N.L.A.

2SD1491-NEC N-DARL+D 70V 2A 10W B>2K

2SD1494 Echivalent: 2SD1496-HIT

2SD1496-HIT SI-N 1500V 5A 50W

2SD1497 Echivalent: 2SD1497-HIT

2SD1497-02-HIT SI-N 1500V 6A 50W

2SD1497-02/DIV Echivalent: BU2508A-PHI

2SD1497-04-HIT Echivalent: : BU2508A-PHI

2SD1497-06-HIT Echivalent: BU2508A-PHI

2SD1497-HIT Echivalent: : 2SD1432-TOS

2SD1497/DIV SI-N 1500V 6A 50W

2SD1504-HIT SI-N 30V 0.5A 0.3W 300MHz

2SD1506-MBR SI-N 60V 3A 10W 90MHz

2SD1508-TOS N-DARL 30V 1.5A 10W B>400

2SD1509-TOS N-DARL+D 80V 2A 10W 0.4uS

2SD1511-MAT N-DARL 100V 1A 1W 150MHz

2SD1521-HIT N-DARL+D 50V 1.5A 2W B>2K

2SD1525-TOS N-DARL+D 100V 30A 150W

2SD1526-MAT SI-N 130V 1A 1W 200MHz

2SD1541-MAT SI-N 1500V 3A 50W

2SD1543 Echivalent: 2SD1546-TOS

2SD1544 Echivalent: 2SD1546-TOS

2SD1545 Echivalent: 2SD1546-TOS

2SD1546-TOS Echivalent: : S2000N-TOS

2SD1547 Echivalent: S2000N-TOS

2SD1548-TOS Echivalent: 2SC4542-TOS

2SD155-NEC SI-N 80V 3A 25W

2SD1553 Echivalent: 2SD1554-TOS

2SD1554-TOS SI-N+D 1500V 3.5A 40W 1us

2SD1555-TOS SI-N+D 1500V 5A 40W 1us

2SD1556-TOS SI-N+D 1500V 6A 50W 1us

2SD1563 Echivalent: 2SD1563A-RHM

2SD1563A-RHM SI-N 160V 1.5A 10W 80MHZ

2SD1564 Echivalent: 2SD1565-NEC

2SD1565-NEC N-DARL+D 100V 5A 30W

2SD1576-MAT SI-N 1500V 2.5A 48W

2SD1577-MAT SI-N 1500V 5A 80W

2SD1577/DIV SI-N 1500V 5A 80W

2SD1579-NEC N-DARL+D 150V 1.5A 1W

2SD1585 Echivalent: BD243C-MBR

2SD1589-NEC N-DARL+D 100V 5A 20W

2SD1590-NEC N-DARL+D 150V 8A 25W

2SD1595-NEC N-DARL+D 60V 5A 20W B=6K

2SD1609-MBR SI-N 160V 0.1A NF/S-L

2SD1610-HIT SI-N 200V 0.1A 1.3W 140MH

2SD1624-SAN SI-N 60V 3A .5W 150MHZ

2SD1632-MAT N-DARL+D 1500V 4A 80W

2SD1647-RHM N-DARL+D 50V 2A 25W

2SD1649-SAN SI-N+D 1500/800V 2,5A 50W

2SD1650-SAN SI-N+D 1500/800V 3.5A 50W

2SD1651-SAN SI-N+D 1500/800V 5A 60W

2SD1652-SAN SI-N+D 1500V 6A 60W 3MHz

2SD1656-SAN SI-N 1500V 6A 50W 3MHz

2SD1657 Echivalent: 2SD1330-MAT

2SD1663-MAT SI-N 1500V 5A 80W 0.5us

2SD1664-RHM SI-N 40V 1A 0.5W 150MHz

2SD1666-SAN SI-N 60V 3A 20W

2SD1667-SAN SI-N 60V 5A 25W 30MHz

2SD1668R-SAN SI-N 60V 7A 30W

2SD1669-SAN SI-N 60V 12A 30W

2SD1669/DIV Echivalent: 2SD1669-SAN

2SD1677-SAN SI-N 1500V 5A 100W 0.5us

2SD1680-MAT SI-N 330/200V 7A 70W

2SD1681-SAN SI-N 20V 1.2A 10W 150MHz

2SD1682 Echivalent: 2SD1683-SAN

2SD1683-SAN SI-N 60V 4A 10W 150MHz

2SD1684-SAN SI-N 120V 1.2A 10W 150MHz

2SD1705-MAT Echivalent: 2SD1706-MAT

2SD1706-MAT SI-N 130/80V 15A 80W 20MH

2SD1707-MAT SI-N 130/80V 20A 100W

2SD1708 Echivalent: : N.L.A.

2SD1710-SAN SI-N 1500/800V 5A 100W

2SD1718 Echivalent: 2SC3281-TOS

2SD1725-SAN SI-N 120V 4A 20W 180MHZ

2SD1729-MAT SI-N+D 1500/700V 3.5A 60W

2SD1730-MAT SI-N+D 1500/700V 5A 100W

2SD1732 Echivalent: BU2508D-PHI

2SD1739-MBR SI-N 1500/700V 6A 100W

2SD1740-FEC N-DARL 150V 5A 25W B=5000

2SD1758-RHM SI-N 40V 2A 10W 100MHz

2SD1760-RHM SI-N 60V 3A 15W 90MHz

2SD1761-RHM SI-N 80V 3A 35W

2SD1762-RHM SI-N 60V 3A 25W 70MHz

2SD1763A-RHM SI-N 120V 1.5A 20W 80MHz

2SD1764-RHM N-DARL+D 60V 2A 20W B>100

2SD1765-RHM N-DARL+D 100V 2A 20W B>1K

2SD1769-SKN N-DARL+D 120V 6A 50W

2SD1776-MAT SI-N 80V 2A 25W 40MHz

2SD1783-RHM N-DARL+D 60V 5A 30W B=2K

2SD1785-SKN N-DARL+D 120V 6A 30W 100M

2SD1790-SHI N-DARL+D 200V 4A 25W B=1K

2SD1791-SHI N-DARL 100V 7A 30W 50MHz

2SD1796-SKN N-DARL+D 60V 4A 25W

2SD180 Echivalent: 2SD895-SAN

2SD1802-SAN SI-N 60V 3A 15W 150MHz

2SD1806-SAN SI-N+D 40V 2A 15W 150MHz

2SD1809-RHM N-DARL 60V 1A 0.9W B>2K

2SD1812-RHM SI-N 160V 1.5A 0.9W

2SD1815-SAN SI-N 120V 3A 20W 180MHz

2SD1817-SAN SI-D 80V 3A 15W B>2K

2SD1825-SAN N-DARL+D 70V 4A 20W

2SD1827-SAN N-DARL+D 70V 10A 30W 20MH

2SD1830-SAN N-DARL+D 110V 8A 30W B=4K

2SD1835-SAN SI-N 60V 2A 150MHz 60/580

2SD1843-NEC N-DARL+D 60V 1A 1W B>2000

2SD1846-MAT Echivalent: 2SD1849-MAT

2SD1847-MAT SI-N+D 1500/700V 5A 100W

2SD1849-MAT SI-N+D 1500/700V 7A 120W

2SD1850 Echivalent: BU508DF-PHI

2SD1853-SAN N-DARL+D 80V 1.5A 0.7W B>

2SD1856-RHM N-DARL+D 60V 5A 25W

2SD1857-RHM SI-N 120V 1.5A 1W 80MHz

2SD1858-RHM SI-N 40V 1A 1W 150MHz

2SD1859-RHM SI-N 80V 0.7A 1W 120MHz

2SD1862-RHM SI-N 40V 2A 1W 100MHZ

2SD1863-RHM SI-N 120V 1A 1W 100MHz

2SD1864-RHM SI-N 60V 3A 1W 90MHz

2SD1877-SAN SI-N+D 1500/800V 4A 50W

2SD1878-SAN SI-N+D 1500V 5A 60W 0.3us

2SD1879 Echivalent: BU508DF-PHI

2SD188 Echivalent: : N.L.A.

2SD1880-SAN SI-N+D 1500V 8A 70W

2SD1881-SAN SI-N+D 1500V 10A 70W

2SD1883 Echivalent: BU508AF-PHI

2SD1884 Echivalent: BU508AF-PHI

2SD1885 Echivalent: 2SD1887-SAN

2SD1886-SAN Echivalent: 2SD1887-SAN

2SD1887-SAN SI-N 1500/800V 10A 70W

2SD1887/DIV SI-B 1500/800V 10A 70W

2SD1893 Echivalent: 2SD1894-MAT

2SD1894-MAT SI-N 160V 7A 70W 20MHz

2SD1895-MAT N-DARL 160V 8A 100W 20MHz

2SD1911 Echivalent: : N.L.A.

2SD1913-SAN SI-N 60V 3A 20W 100MHz

2SD1929-RHM N-DARL+D 60V 2A 1.2W

2SD1930-RHM N-DARL 100V 2A 1.2W B=500

2SD1933-RHM N-DARL+D 80V 4A 30W

2SD1941-HIT Echivalent: 2SD1739-MAT

2SD1944-RHM SI-N 80V 3A 30W 50MHz

2SD1958-SAN SI-N 200V 4.5A 30W 10MHz

2SD1959 SI-N 1400V 10A 50W

2SD1978-HIT N-DARL+D 120V 1.5A 0.9W

2SD198 SI-N 300V 1A 25W 45MHz

2SD199 Echivalent: 2SD200-MBR

2SD1991-MAT SI-N 60V 0.1A 0.4W 150MHz

2SD1992-MAT SI-N 30V 0.5A 0.6W 200MHz

2SD1994-MAT SI-N 60V 1A 1W 200MHz

2SD1996-MAT SI-N 25V 0.5A 0.6W 200MHz

2SD200-MBR SI-N 1500V 2.5A 10W

2SD2006-RHM SI-N 80V 0.7A 1.2W 120MHz

2SD2007-RHM SI-N 40V 2A 1.2W 100MHZ

2SD2010-RHM N-DARL 60V 2A 1.2W B>1000

2SD2012-TOS SI-N 60V 3A 25W 3MHz

2SD2018-MAT N-DARL+D 60V 1A 5W B>6K5

2SD2023 Echivalent: BD241C-MBR

2SD2033-RHM Echivalent: : N.L.A.

2SD2052-MAT SI-N 150V 9A 100W 20MHZ

2SD2058 Echivalent: 2SD2061-RHM

2SD2061-RHM SI-N 80V 3A 30W 8MHz

2SD2066-MAT SI-N 160V 12A 120W

2SD2088 N-DARL+D 60V 2A 0.9W B>2K

2SD2089 Echivalent: 2SD1555-TOS

2SD2095 Echivalent: 2SD2125-TOS

2SD2096 Echivalent: BD241C-MBR

2SD2125-TOS SI-N+D 1500V 5A 50W 0.2us

2SD213 SI-N 110V 10A 100W

2SD2136-MAT SI-N 60V 3A 1.5W 30MHz

2SD2137 Echivalent: 2SD2137A-MAT

2SD2137A-MAT SI-N 80V 3A 15W 30MHZ

2SD2141-SKN N-DARL+D 380V 6A 35W B>15

2SD2144-RHM SI-N 25V 0.5A B>560

2SD2151-MAT SI-N 130/80V 10A 30W 20MH

2SD2155 Echivalent: 2SC3281-TOS

2SD2159-RHM SI-N 30V 2A 1W 110MHz

2SD2231 Echivalent: : N.L.A.

2SD2250 N-DARL 160V 7A 90W B>5K

2SD2253 SI-N+D 1700V 6A 50W

2SD2255-MAT N-DARL 160V 7A 70W 20MHz

2SD2259 Echivalent: 2SD1458-MAT

2SD226 Echivalent: 2SD1266-MAT

2SD2276-MAT N-DARL 160V 8A 120W B>5K

2SD2331-MAT N-DARL+D 1500V 3A

2SD2333-MAT TRANSISTOR

2SD234 SI-N 60V 3A 25W AF-POWER

2SD2340-MAT SI-N 130V 6A 50W

2SD235 Echivalent: 2SD880-MBR

2SD2375 SI-N 80V 3A 25W B>500

2SD2386 N-DARL 140V 7A 70W B>5K

2SD2389-SKN N-DARL 160V 10A 100W B>5K

2SD2390-SKN N-DARL 160V 10A 100W 55MH

2SD2394-RHM SI-N 60V 3A 30W

2SD2395-RHM SI-N 50V 3A 25W

2SD2399-RHM N-DARL+D 80V 4A 30W B=1K-

2SD2438-SKN N-DARL+D 160V 8A 70W B>5K

2SD2493-SKN N-DARL 110V 6A 60W 60MHz

2SD2498 SI-N 1500V 6A 50W

2SD2499-TOS SI-N+D 1500V 6A 50W

2SD261 Echivalent: BC337-25-MBR

2SD287 SI-N 200V 10A 100W 8MHz

2SD288 Echivalent: BD241C-MBR

2SD299 Echivalent: 2SD1391-MAT

2SD300 Echivalent: 2SD1391-MAT

2SD313-MBR SI-N 60V 3A 30W 8MHz

2SD314 Echivalent: 2SD313-MBR

2SD315 Echivalent: BD243C-MBR

2SD317 Echivalent: 2SD1266-MAT

2SD325-SAN SI-N 35V 1.5A 10W 8MHz

2SD330 Echivalent: BD241C-MBR

2SD345 Echivalent: BD241C-MBR

2SD348 Echivalent: 2SD1403-SAN

2SD350-MBR SI-N 1500V 5A 22W

2SD350/DIV Echivalent: 2SD350A/DIV

2SD350A-MBR SI-N 1500V 5A 22W

2SD350A/DIV SI-N 1500V 5A 22W

2SD355 Echivalent: BC639-MBR

2SD359 SI-N 40V 2A 10W LOWFREQPO

2SD361-MIT SI-N 60V 1.5A 10W 70MHz

2SD365 Echivalent: 2SD1266-MAT

2SD371 Echivalent: 2SC1619

2SD380 Echivalent: BU208A-ST

2SD381-NEC SI-N 130V 1.5A 20W 60MHz

2SD382-NEC SI-N 130V 1.5A 20W 60MHz

2SD386-SAN SI-N 200V 3A 25W 8MHz

2SD386A Echivalent: 2SD386-SAN

2SD387 Echivalent: 2SD1138-HIT

2SD388 Echivalent: 2SD551

2SD389 Echivalent: 2SD1266-MAT

2SD390 Echivalent: 2SD1266-MAT

2SD400-SAN SI-N 25V 1A 0.9W

2SD401-MBR SI-N 200V 2A 20W 10MHz

2SD401A Echivalent: 2SD401-MBR

2SD414-NEC SI-N 120/80V 0.8A 10W

2SD415-NEC SI-N 120/100V 0.8A 10W

2SD424-MBR SI-N 160V 15A 150W POWER

2SD425 Echivalent: 2SD1148-TOS

2SD426 Echivalent: 2SD551

2SD427 Echivalent: 2SD427/DIV

2SD427-MBR Echivalent: : 2SD732

2SD427/DIV SI-N 120V 8A 80W 5MHZ

2SD428 Echivalent: 2SD551

2SD438-SAN SI-N 100V 0.7A 0.9W 100MH

2SD467-HIT SI-N 25V 0.7A 0.5W 280MHz

2SD468-HIT SI-N 25V 1A 0,9W 280MHz

2SD468C Echivalent: BC639-MBR

2SD471-MBR SI-N 30V 1A 0.8W UNI (EBC

2SD476 SI-N 70V 4A 40W 7MHz

2SD478-HIT SI-N 200V 2A 30W

2SD5072-SSU Echivalent: BU2508DF-PHI

2SD5080 Echivalent: BU2508AF-PHI

2SD517 Echivalent: 2SD1439-MAT

2SD525 Echivalent: TIP41C-MBR

2SD526 Echivalent: BD243C-MBR

2SD531 Echivalent: 2SD613-SAN

2SD545-MBR SI-N 25V 1.5A 0.5W

2SD549-TOS N-DARL 30V 1.5A 15W B>4K

2SD551 Echivalent: : N.L.A.

2SD552-TOS SI-N 220V 15A 150W 4MHz

2SD553-TOS SI-N 70V 7A 40W 10MHz

2SD555-NEC SI-N 400V 15A 200W 7MHz

2SD555/DIV Echivalent: :2SD555-NEC

2SD556-SKN SI-N 120V 15A 120W 8MHz

2SD560-NEC N-DARL 100V 5A 30W

2SD558-NEC Echivalent: : N.L.A.

2SD571-NEC SI-N 60V 700mA 1W 110MHz

2SD587 Echivalent: 2SD718-TOS

2SD592-MAT SI-N 30V 1A 0.75W 200MHz

2SD596-NEC SI-N 30V 0.7A 170MHz

2SD600 Echivalent: 2SD600K-SAN

2SD600K-SAN SI-N 120V 1A 8W

2SD601 Echivalent: BC848B-MBR

2SD602 Echivalent: 2SD602A-MAT

2SD602A-MAT SI-N 60V 0.5A 0.2W 200MHz

2SD608-NEC Echivalent: : 2SC2238-MBR

2SD612-SAN SI-N 25V 2A 10W 100MHz

2SD613-SAN SI-N 100V 6A 40W 15MHz

2SD617-SKN N-DARL 120V 8A 100W

2SD631 Echivalent: BUV21-MBR

2SD633 Echivalent: BD901-MBR

2SD634 Echivalent: BD901-MBR

2SD635 Echivalent: BD901-MBR

2SD636-MAT Echivalent: 2SD637-MAT

2SD637-MAT SI-N 60V 0.1A 0.4W 150MHz

2SD638-MAT Echivalent: : BC327-25-MBR

2SD639 Echivalent: BC639-MBR

2SD649 Echivalent: 2SD1632-MAT

2SD655 Echivalent: BC639-MBR

2SD661-MAT SI-N 35V 0.1A 0.4W 200MHz

2SD662-MAT SI-N 250V 0.1A 0.6W 50MHz

2SD665-TOS Echivalent: : N.L.A.

2SD666-HIT SI-N 120V 0.05A 140MHz

2SD667-HIT SI-N 120V 1A 140MHz

2SD668 Echivalent: BF471-PHI

2SD669-MBR Echivalent: 2SD669A-MBR

2SD669A-MBR SI-N 160V 1.5A 1W 140MHz

2SD676 SI-N 160V 12A 125W 8MHZ

2SD678 Echivalent: 2SD1276-MAT

2SD679 Echivalent: 2SD1276-MAT

2SD683 Echivalent: 2SD683A-TOS

2SD686 Echivalent: TIP122-MBR

2SD704 Echivalent: BD243C-MBR

2SD712 SI-N 100V 4A 30W 8MHz

2SD716 Echivalent: TIP33C-MBR

2SD717-TOS SI-N 70V 10A 80W 0.3us

2SD718-TOS SI-N 120V 8A 80W 12MHz

2SD725 SI-N 1500V 6A 50W POWER

2SD726-HIT SI-N 100V 4A 40W 10MHz

2SD727 Echivalent: 2SC2837-SKN

2SD731 SI-N 170V 7A 80W 7MHz

2SD732 SI-N 150V 8A 80W 15MHz

2SD733 Echivalent: 2SD551

2SD734-SAN SI-N 25V 0.7A 0.6W 250MHz

2SD743 Echivalent: BD243C-MBR

2SD743A Echivalent: BD243C-MBR

2SD745 Echivalent: 2SC3281-TOS

2SD756 Echivalent: 2SC2632-MAT

2SD757 Echivalent: 2SD758-HIT

2SD760-HIT Echivalent: : N.L.A.

2SD762-MAT SI-N 60V 3A 25W 25kHz

2SD763-HIT SI-N 120V 1A 0.9W

2SD768-HIT N-DARL+D 120V 6A 40W B>1K

2SD772 Echivalent: 2SD1274-MAT

2SD773-NEC SI-N 20V 2A 1W 110MHz

2SD774-NEC SI-N 100V 1A 1W 95MHz

2SD781-HIT SI-N 150V 2A 1W 0.6us

2SD786-RHM SI-N 40V 0.3A 0.25W

2SD787-HIT SI-N 20V 2A 0.9W 80MHz

2SD788-HIT SI-N 20/20V 2A 0.9W 100MH

2SD789-HIT SI-N 100/50V 1A 0.9W 80MH

2SD792 Echivalent: BU208A-ST

2SD794-MBR SI-N 70V 3A 10W 60MHz

2SD795-NEC SI-N 40V 3A 20W 95MHz

2SD797 Echivalent: BDY29

2SD798-TOS N-DARL 600V 6A 30W B>1K5

2SD799-TOS N-DARL+D 400V 6A 30W

2SD800 SI-N 750V 4A 30W 8MHz

2SD807 Echivalent: 2SC3679-SKN

2SD809-NEC SI-N 100V 1A 10W 85MHz

2SD811 Echivalent: BU526-MBR

2SD817 Echivalent: 2SD819-TOS

2SD818 Echivalent: 2SD819-TOS

2SD819-TOS SI-N 1500V 3.5A 50W

2SD820-MBR SI-N 1500V 5A 50W

2SD821 Echivalent: BU208A-ST

2SD822 SI-N 1500/600V 7A 50W

2SD822/DIV SI-N 1500V 7A 50W

2SD826-SAN SI-N 60V 5A 10W 120MHz

2SD829-HIT N-DARL+D 150V 15A 100W B=

2SD83 Echivalent: 2SC2581-SKN

2SD836 Echivalent: BD901-MBR

2SD837-MAT N-DARL 60V 4A 40W

2SD837B Echivalent: BD901-MBR

2SD838 Echivalent: S3786-TOS

2SD841 Echivalent: BUT11A-ST

2SD844-TOS SI-N 50V 7A 60W 15MHz

2SD845-TOS Echivalent: 2SD1036-FUJ

2SD849 Echivalent: 2SD1391-MAT

2SD850 SI-N 1500V 3A 25W

2SD8550C-HIT TRANSISTOR

2SD856-MAT SI-N 60V 3A 35W POWER

2SD861 Echivalent: 2SC3169-MAT

2SD863-SAN SI-N 50V 1A 0.9W

2SD864K-HIT N-DARL+D 120V 3A 30W

2SD866 Echivalent: 2SD1271-MAT

2SD867 SI-N 130V 10A 100W 3MHz

2SD868 Echivalent: 2SD869-MBR

2SD869-MBR Echivalent: : 2SD871

2SD870-MBR Echivalent: 2SD871

2SD871 SI-N+D 1500V 5A 50W

2SD876 Echivalent: 2SD1272-MAT

2SD879-SAN SI-N 30V 3A 0.75W 200MHz

2SD880-MBR SI-N 60V 3A 30W 0.8us

2SD882-MBR SI-N 30V 3A 10W

2SD886 Echivalent: 2SD1273-MAT

2SD887 Echivalent: 2SD1273-MAT

2SD889-MAT SI-N+D 1500V 4A 50W

2SD891 Echivalent: BC517-MBR

2SD892-MAT Echivalent: 2SD892A-MAT

2SD892A-MAT N-DARL 60V 0.5A 0.4W B>2K

2SD894-SAN N-DARL 30V 1.5A 10W 120MH

2SD895-SAN SI-N 100V 6A 60W 10MHz

2SD896 Echivalent: TIP33C-MBR

2SD898 Echivalent: : N.L.A.

2SD898B Echivalent: BU208D-ST

2SD900B Echivalent: : N.L.A.

2SD903 Echivalent: 2SD1398-SAN

2SD904 Echivalent: 2SD1398-SAN

2SD905 Echivalent: : N.L.A.

2SD917-MAT SI-N 330V 7A 70W POWER

2SD92-SKN SI-N 100V 3A 20W

2SD921-FEC N-DARL 200V 5A 80W B>700

2SD946-MAT N-DARL 30V 1A =

2SD947-RHM N-DARL 40V 2A 5W 150MHz

2SD950 Echivalent: 2SD1541-MAT

2SD951-MBR SI-N 1500V 3A 65W

2SD953 Echivalent: 2SD1577-MAT

2SD958-MAT SI-N 120V 0.02A 0.4W 200M

2SD965-MBR SI-N 40V 5A 0.75W 150MHz

2SD966-MAT SI-N 40V 5A 1W 150MHz

2SD968 Echivalent: 2SD968A-MAT

2SD968A-MAT SI-N 120V 0.5A 1W 120MHz

2SD969 Echivalent: 2SD1330-MAT

2SD970-HIT N-DARL+D 120V 8A 40W B>1K

2SD972-SKN N-DARL 50V 4A 30W B=3K

2SD973 Echivalent: BC639-MBR

2SD982-FEC N-DARL 200V 5A 40W B=3000

2SD985 Echivalent: 2SD986-NEC

2SD986-NEC N-DARL 150/80V 1.5A 10W

2SD998-NEC N-DARL 100V 1.5A 10W B=7K

2SH1061-HIT Echivalent: 2SC1061-HIT

2SJ103-TOS P-FET 50V 2.6mA Up<2V

2SJ109-TOS P-FET DUAL 30V Id>2.6mA

2SJ113-HIT P-FET 100V 10A 100W OE35

2SJ114-HIT Echivalent: : N.L.A.

2SJ115-TOS Echivalent: 2SJ200-TOS

2SJ117-HIT P-FET 400V 2A 40W 35ns

2SJ118 Echivalent: 2SJ119-HIT

2SJ119-HIT Echivalent: : N.L.A.

2SJ160 Echivalent: 2SJ162-HIT

2SJ161 Echivalent: 2SJ162-HIT

2SJ162-HIT P-FET 160V 7A 100W 1E

2SJ174-HIT P-FET 60V 20A 75W 235ns

2SJ175-HIT P-FET 60V 10A 25W OE25

2SJ177-HIT P-FET 60V 20A 75W <0E085

2SJ182-HIT P-FET 60V 3A 20W OE28

2SJ200-TOS P-FET 180V 10A 120W

2SJ201-TOS P-FET 200V 12A 150W

H1061-HIT Echivalent: 2SC1061-HIT

H6N80-MBR N-FET 800V 4.2A 170W 1E9

HA11122W-HIT DUAL REC. RECORD/PLAYBACK

HA11123-HIT FM IF DETEC.AM CONVRT.18p

HA11211 FM/AM TUNER 16/20p

HA11215A-HIT COLOR TV PICTURE IF 24p

HA11219

HA11221 TV VIDEO ZF

HA11223W-HIT

HA11225-HIT FM IF AMPLIFIER 16p

HA11226

HA11226-SMD Echivalent ON REQUEST

HA11227 Echivalent: AN7410N-MAT

HA11235 =DBL2009

HA1124-HIT TV SOUND IF AMPLIFIER 14p

HA11244-HIT

HA11247-HIT CTV NTSC CHROMA

HA1124A TV SOUND IF 14p

HA1125 Echivalent: CA3065E-MBR

HA11251-HIT FM/AM TUNER + DEMOD. 16p

HA1127 Echivalent: TBA331

HA1137W FM IF AMPL.

HA1138

HA11401

HA11412A-HIT

HA11414

HA11423-HIT TV BLANKING 16/20p

HA1143 Echivalent: 8905405971

HA11440-HIT COLOR TV VIDEO 16p

HA11489 Echivalent: U4647B-TFK

HA11498 Echivalent: U4647B/PCB

HA1151 AM-HF + FM 14p

HA11517BNT-HIT HOR. & VERT. DEFLECTION I

HA1156W STEREO DECODER

HA11580A-HIT

HA11701

HA11702 VC-HEAD PREAMPLIFIER

HA11703 VC-FM SIGNAL 20p

HA11706 VTR COLOR APC 18p

HA11710 VTR CHROMA 16p

HA11711

HA11713 VTR SERVO

HA11714-HIT MOTOR CONTROLER

HA11715-HIT 3 PH BI-DIR MOTOR DRIVER

HA11716 VTR LUMIN. SIGNAL

HA11717

HA11718

HA11724

HA11738

HA11741

HA11741NT-HIT VTR CHROMA 30P SMALL VERS

HA11744-HIT VIDEO-A/W AMPL.

HA11745-HIT VIDEO AMPLIFIER

HA11748-HIT

HA11749 MOTOR SPEED CONTROLER

HA11751 VTR PHASE SERVO CONTROLLE

HA11752 AUDIO AMPLIFIER

HA118019NT-HIT VIDEO REC. PRE-AMPLIFIER

HA11827NT-HIT I.C.

HA11839NT-HIT I.C.

HA11847 I.C.

HA1196 FM STEREO DECODER

HA1197 AM TUNER

HA1199-HIT AM TUNER 16p

HA12001W-HIT

HA12002W-HIT POWER AMPL. PROTECTOR

HA12003 Echivalent: MC1310P-MOT

HA12005 AUDIO LOW NOISE AMP. 16p

HA12006-HIT RECORDER A/W AMPLIFIER

HA12009-HIT 7-SEG. DEC. FOR DIG. TUNI

HA1201

HA12010-HIT FLT DRIVER

HA12016-HIT FM MPX DEMODULATOR 16p

HA12017-HIT PRE-AMP HI-FI

HA12019

HA1202 FM IF AMP. 8p

HA12026 STEREO DECODER 16p DIP

HA12038-28P DOLBY-C TYPE 28p

HA12038NT-HIT DOLBY-C TYPE

HA12044 CX NOISE REDUCTION

HA12045-HIT DOLBY NOISE REDUCTION 28p

HA12046 Echivalent: 8945900804

HA12047 DOLBY-B NOISE REDUCTION

HA12058 DOLBY B/C IC FOR TAPE DEC

HA12062 Echivalent: 8945902293

HA12064A Echivalent: 8945900736

HA12088ANT-HIT 2 CHN DOLBY B+C CIRCUIT

HA12109 Echivalent: 8945902284

HA12116 I.C.

HA12134A-HIT DOLBY NOISE REDUCTION

HA12135A-HIT DOLBY NOISE REDUCTION CIR

HA12136-HIT DOLBY NOISE REDUC.SYST.

HA12402 Echivalent: TA7613AP-TOS

HA12412-HIT FM TUNER + IF STAGES 16p

HA12413 FM/AM IF AMP.

HA12430-HIT

HA12434

HA13001-HIT

HA13006 3PH MOTOR DRIVER

HA13007-HIT MOTOR DRIVER (4x)

HA1301-HIT Echivalent: HA13001-HIT

HA13108-HIT

HA1311

HA13117-HIT PWR AMP 1x14W/4E 18V

HA13118-HIT PWR AMP 1x18W/4E 18V BTL

HA13119-HIT PWR AMP 2x5.5W/4E 18V

HA13127-HIT PWR AMP 2x16W/4E 18V

HA13128-HIT PWR AMP 2x19W/4E 18V

HA13130-HIT PWR AMP 2x16W/4E 18V

HA13135 PWR AMP 2x19W/4E 18V

HA13139

HA13150A-HIT 4-CHANNEL AUDIO AMPLIFIER

HA13151-HIT PWR AMP 4x14W/4E 18V 3A

HA13152 PWR AMP 4x14W/4E 18V 3A

HA13153A-HIT AMPLIFIER

HA13154A-HIT PWR AMPLIFIER

HA13155-HIT PWR AMP 4*33W/4E 18V

HA13157-HIT I.C.

HA13158-HIT PWR AMP

HA1317W-HIT

HA1319

HA1325-HIT PWR AMP 2W/8E 20V 1.25A

HA13403-HIT MOTOR DRIVER 3-PHASE

HA13403S-HIT Echivalent: HA13403-HIT

HA13406W-HIT MOTOR DR. 3 PH. BRUSHLESS

HA13408-HIT 9 OUTPUT INV. OC POWER DR

HA13412-HIT 3 PH. BRUSHLESS MOTOR DR.

HA13426-HIT SPINDLE MOTOR DR. FOR HDD

HA13441-HIT SPINDLE MOTOR DRIVER (HDD

HA13456A-HIT MOTOR CONTROLER

HA1350 Echivalent: HA1397-HIT

HA1364 Echivalent: TDA1190Z-ST

HA1370 Echivalent: HA1397-HIT

HA1377-HIT

HA1384-HIT PWR AMP 1x20W/4W 18V

HA1388-HIT

HA1389 5.6W POWER AMP. 10p

HA1389R 5.6W POWER AMP.10p REVERS

HA1392-HIT PWR AMP 2x6.8W/4E 20V

HA1394-HIT DUAL 6-7W.POWER AMP. 12p

HA1396 20W AUDIO POWER AMP. 12p

HA1397-HIT 20W AUDIO POWER AMP. 12p

HA1398-HIT DUAL 5.8W POWER AMP.

HA1406 AUDIO AMPL. SINGLE CHAN.

HA1452W-HIT

HA1457W-HIT

HA16617P-HIT VACUUM FLUORESCENT DRIVER

HA17324 Echivalent: LM324N-MBR

HA17339 Echivalent: LM339N-MBR

HA17358 Echivalent: LM358N-MBR

HA17384PS-HIT SMPS I-CNTRL IC 57kHz

HA17393 Echivalent: LM393N-MBR

HA17431PS-HIT AP V-REG 2.5-40V 0.15A

HA17458 Echivalent: MC1458N-MBR

HA17458GS Echivalent: MC1458N-MBR

HA17524P-HIT Echivalent: TL494CN-MBR

HA17558 Echivalent: MC1458N-MBR

HA17741 Echivalent: UA741CN8-MBR

HA17805 Echivalent: UA7805-MBR

HA17806 Echivalent: UA7806-MBR

HA17807 Echivalent: UA7807-MBR

HA17901 Echivalent: LM2901N-MBR

HA17902P-HIT QUAD OPAMP

HA17903 Echivalent: 8945900945

HA2145 Echivalent: 8905405211

HANDLE

HCF4053 Echivalent: CD4053-MBR

HCF4066BE Echivalent: CD4066-MBR

HD17558 Echivalent: NE558N-MBR

HD46850P Echivalent: 6850P-MBR

HD49733ANT VCR SERVO CONTROLER

HD614042SE02 Echivalent: HD614042SE2

HD614042SE1 CPU 4B 128RAM 4kROM

HD614042SE2 CPU 4B 128RAM 4kROM

HD614042SE30 CPU 4B 128RAM 4kROM

HD614042SE69 Echivalent: HD614042SG69

HD614042SG69 CPU 4B WITH ROM

HD614042SH89 CPU 4B WITH ROM

HD6321 Echivalent: 6821P-MBR

HD6350 Echivalent: 6850P-MBR

HD63B21P Echivalent: 68B21P-MBR

HD74LS86P Echivalent: SN74LS86-MBR

HEF4000 Echivalent: CD4000-PHI

HEF4011 Echivalent: CD4011-MBR

HEF4013 Echivalent: CD4013-MBR

HEF4046 Echivalent: CD4046-PHI

HEF4052 Echivalent: CD4052-PHI

HEF4053BP Echivalent: CD4053-PHI

HEF4066 Echivalent: CD4066-PHI

HEF4093 Echivalent: CD4093-PHI

HEF4094 Echivalent: CD4094-PHI

HEF4538BP Echivalent: CD4538-PHI

HER101 Echivalent: UF4003

HER102 Echivalent: UF4003

HER103 Echivalent: UF4003

HER104 Echivalent: UF4007

HER105 Echivalent: UF4007

HER106 Echivalent: UF4007

HER107 Echivalent: UF4007

HER108 Echivalent: UF4007

HER301 Echivalent: UF5404

HER302 Echivalent: UF5404

HER303 Echivalent: UF5404

HER304 Echivalent: UF5404

HER305 Echivalent: UF5404

HER306 Echivalent: UF5408

HER307 Echivalent: UF5408

HER308 Echivalent: UF5408

HL90222 FAIRCHILD IC

HM6116 Echivalent: 6116-100-MBR

HM9102A

HML044 Echivalent: 209.80469

HPA100R-SAN SI-N+D 1500V 10A 150W 0.2

HPA150R-SAN SI-N+D 1500V 15A 180W 0.2

HT2 Echivalent: BR100-ST

HV03-12 Echivalent: HV05-12

HV05-12 SI-D 12KV 0.5A FOR MICRO-

HV07-12 Echivalent: MW-D008

HVM12 Echivalent: HV05-12

HVR062 HIGH VOLTAGE PROTECTOR DI

HVR1X3 Echivalent: HV05-12

HVR3-12 Echivalent: HV05-12

HVR3-9 Echivalent: HV05-12

HVR6X2 Echivalent: HVR062

HY53C464LS-80 Echivalent: 41464-80-MBR

HZ20L-2 REF-DIODE 19.9V 2% 0.5MA

HZ5C2-HIT Echivalent: 0.4W 5V1

IC/TR-DATABASE HALBLEITER DATENBANK

ICL232CPE-MBR ALSO: SP232ACP

ICL7106CPL-MBR ADC 3.5-DIG 333ms+LCD DR.

ICL7106RCPL-MB ADC 3.5-DIG 333ms REVERSE

ICL7107CPL-MBR ADC 3.5-DIG 333ms+LED DR.

ICL7109CPL-MBR ADC 12B 133ms

ICL7116CPL-MBR ADC 3.5-DIG 333ms+DISPL H

ICL7117CPL-MBR ADC 3.5-DIG 7-SEGMENT OUT

ICL7126CPL-MBR ADC 3.5-DIG 333ms LOW-POW

ICL7129CPL-HAR IC.

ICL7135CPI-MBR ADC 4.5-DIG 250ms BCD OUT

ICL7136CPL-MBR ADC 3.5-DIG 333ms LOW-POW

ICL7611 Echivalent: TLC271CP-MBR

ICL7650CPA-MBR 1xOPAMP 2MHz CHOPPER STAB

ICL7650CPD-MBR OPAMP 1* 2MHz CHOPPER STA

ICL7660CPA-MBR C-MOS VOLT CONV. -1.5-10V

ICL7662CPA-MBR SINGLE DC-to-DC CONVERTER

ICL8013ACTX Echivalent: ICL8013CCTX

ICL8013CCTX 4-QUADRANT ANALOG MULTIPL

ICL8038CCPD-MB PREC.WAVEFORM GEN/VOLT CO

ICL8069CCZR-MB REF-DIODE 1.2V 50ppm

ICL8211CPA-HAR PROGR VOLT DET. 1.8-30V

ICL8212CPA-HAR PROGR VOLT DET. 1.8-30V

ICM7170IPG-MBR uPC CLOCK/CALENDAR 4MHz

ICM7207IPD-MBR CMOS TIMEBASE GENERATOR

ICM7213IPD-MBR ONE SEC./MIN. TIMEBASE GE

ICM7217AIPI-MB 4-DIG U/D CNTR + LED DR

ICM7224IPL-MBR LCD-DRIVER 4.5-DIG

ICM7555CN Echivalent: NE555NCMOS-MBR

ICM7556IPD Echivalent: NE556NCMOS-MBR

ICP ASS ASS. 15x3=45pcs

ICP-F10 HALBLEITERSICH. 0.4A 50V

ICP-F15 HALBLEITERSICH. 0.6A 50V

ICP-F20 HALBLEITERSICHEUNG 0.8A 5

ICP-F25 HALBLEITERSICH. 1A 50V AS

ICP-F38 HALBLEITERSICH. 1.5A 50

ICP-F50 HALBLEITERSICH. 2A 50V

ICP-F75 HALBLEITERSICH. 2.7A 50

ICP-N10 HALBLEITERSICH.0.4A 50V A

ICP-N15 HALBLEITERSICH.FUSE 0.6A

ICP-N20 HALBLEITERSICH. 0.8A 50V

ICP-N25 HALBLEITERSICH. 1A 50V AS

ICP-N38 HALBLEITERSICH. 1.5A 50V

ICP-N5 HALBLEITERSICH. 0.25A 50V

ICP-N50 HALBLEITERSICH. 2A 50V AS

ICP-N75 HALBLEITERSICH. 2.7A 50V

ICS1700AN NiCd CHARGER I.C.

ICS1702N NiMH CHARGER IC

IMS1630 Echivalent: 6264-100-MBR

IP3842N Echivalent: UC3842N-MBR

IR2110-IR CMOS MOTOR DRIVER

IR2403-SHR DARL.ARRAY 7x45V 0.4A

IR2406G-SHR 12-DOT LED DISPLAY DRIVER

IR2422-SHR 7XDARLINGTON TRAN. ARRAY

IR2E02-SHR NIXIE DISPLAY DRIVER

IR2E27A-SHR DOT MATRIX DISPLAY DRIVER

IR3C02A-SHR

IR3M02-SHR SMPS CIRCUIT

IR9331-SHR VOLT. TO FREQ. CONVERTER

IR94559 Echivalent: RC4559N-MBR

IRF120-MBR N-FET 100V 9.2A 60W <58/5

IRF130-MBR Echivalent: IRF240-MBR

IRF140-MBR N-FET 100V 28A 150W 0E77

IRF230-MBR N-FET 200V 9A 75W <0E4

IRF234 Echivalent: IRF340-MBR

IRF240-MBR N-FET 200V 18A 125W

IRF242 Echivalent: IRF240-MBR

IRF244-MBR Echivalent: IRF450-MBR

IRF250-MBR N-FET 200V 30A 150W

IRF251-MBR

IRF330-MBR N-FET 400V 5.5A 75W <1E

IRF331 Echivalent: IRF330-MBR

IRF340-MBR N-FET 400V 10A 125W <62/1

IRF350-MBR N-FET 400V 13A 150W 0.4E

IRF440-MBR N-FET 500V 8A 125W <0E85

IRF450-MBR N-FET 500V 13A 150W

IRF460-MBR Echivalent: IRFP460-MBR

IRF510 Echivalent: IRF520-MBR

IRF511 Echivalent: IRF520-MBR

IRF520-MBR N-FET 100V 10A 70W 0.27R

IRF522 Echivalent: IRF520-MBR

IRF523 Echivalent: IRF520-MBR

IRF530-MBR N-FET 100V 16A 90W 0.16R

IRF531 Echivalent: IRF530-MBR

IRF532 Echivalent: IRF530-MBR

IRF533 Echivalent: IRF530-MBR

IRF540-MBR N-FET 100V 28A 150W 0.077

IRF541-MBR Echivalent: IRF540-MBR

IRF542 Echivalent: IRF540-MBR

IRF610-MBR Echivalent IRF630-MBR

IRF620 Echivalent: IRF630-MBR

IRF624 Echivalent: IRF630-MBR

IRF630-MBR N-FET 200V 9A 75W 0.4E

IRF632 Echivalent: IRF630-MBR

IRF640-MBR N-FET 200V 18A 125W 0.18R

IRF641 Echivalent: IRF640-MBR

IRF642 Echivalent: IRF640-MBR

IRF644-MBR N-FET 250V 14A 125W

MC14497P-MOT PCM REMOTE CTRL TRANSM.

MC14499P-MOT 4-DIGIT 7 SEGM DEC/LED DR

MC145026P-MOT REMOTE CONTROL ENCODER

MC145027P-MOT REMOTE CONTROL DECODER

MC145028P-MOT REMOTE CONTROL DECODER

MC145041P-MOT A-D CONVERTER+CLOCK+MUX

MC145106P-MOT PLL FREQ. SYNTHESIZER

MC14511 Echivalent: CD4511-MBR

MC145151P2-MOT PHASE LOCKED LOOP

MC14516 Echivalent: CD4516-MBR

MC145166P-MOT PLL FREQUENCY SYNTHESIZER

MC14528 Echivalent: CD4528-MBR

MC14538 Echivalent: CD4538-MBR

MC145406P-MBR 3xRS232 TRANSCEIVER

MC14541 Echivalent: CD4541-MBR

MC145436AP-MOT DTMF RECEIVER

MC1455 Echivalent: NE555N-MBR

MC145503P-MOT PCM/MODEM APPLICATION

MC14551BCP Echivalent: CD4551-MBR

MC14553BCP Echivalent: CD4553-MBR

MC14556 Echivalent: CD4556-MBR

MC14568 Echivalent: CD4568-MBR

MC14573P-MOT QUAD PROGRAMMABLE OPAMP

MC14574P-MOT QUAD SINGLE SUPPLY COMP.

MC1458 Echivalent: MC1458N-MBR

MC14584 Echivalent: CD40106-MBR

MC1458D-MBR OPAMP 2x 36V 1MHz 0.8V/us

MC1458N-MBR OPAMP 2x 36V 1MHz 0.8V/us

MC1458P Echivalent: MC1458N-MBR

MC146818P-MOT CLOCK/CALENDER 24p

MC1488 Echivalent: MC1488N-MBR

MC1488N-MBR 4xRS232 DRIVER =GD75188

MC1488P Echivalent: MC1488N-MBR

MC1489N-MBR 4xRS232 RECEIVER =GD75189

MC1495BP-MOT ALSO: MC1595L-MOT

MC1496G-MBR

MC1496P-MBR BALANCED MOD.- DEMOD.

MC14C88N-MBR 4xRS232 CMOS DRIVER

MC14C89N-MBR 4xRS232 CMOS RECEIVER

MC150 Echivalent: BY228-PHI

MC1508 Echivalent: DAC08EN-MBR

MC1558N-MBR OPAMP 2x 44V 1MHz 0.8V/us

MC1590G-MOT RF/IF AUDIO AMPLIFIER

MC1595L-MOT Echivalent: MC1495BP-MOT

MC1648P-MOT VOLT. CONTROLLED OSCILL.

MC1709 Echivalent: UA741CN8-MBR

MC1723 Echivalent: UA723CN14-MBR

MC1733 Echivalent: UA733CN-MBR

MC1741 Echivalent: UA741CN8-MBR

MC1747 Echivalent: UA747CN-MBR

MC1748 Echivalent: UA748N08-MBR

MC1776 Echivalent: UA776CN-MBR

MC2681P Echivalent SCN2681AC1N4

MC2833 Echivalent: MC2833P-MOT

MC2833P-MOT FM TRANSM. FOR CORDL. PH.

MC3003 Echivalent: SN74LS32-MBR

MC3301 Echivalent: LM3301N-MBR

MC3301P-MOT Echivalent: LM3301N-MBR

MC3302P Echivalent: LM3302N-MBR

MC3303N Echivalent: LM2902N-MBR

MC3303P Echivalent: LM2902N-MBR

MC33078N-MBR DUAL OPAMP

MC33078P Echivalent: MC33078N-MBR

MC33164P5-MOT UNDERVOLTAGE SENSING CIRC

MC33166T-MOT Echivalent: MC34166T-MOT

MC3320P-MOT AUDIO DRIVER

MC3334P-MOT HIGH ENERGY IGNIT.CIRCUIT

MC3340P-MOT DC CONTROLLED ATTENUATOR

MC3346 Echivalent: CA3046-MBR

MC3356P-MOT FSK CIRCUIT

MC3357P-MOT FM RECEIVER IC 45MHz

MC3359P-MOT FM-RECEIVER IC + SCAN OUT

MC3361B-MBR FM-RECEIVER =ULN3862A LOW

MC3361BD-MOT SMD FM-RECEIVER

MC3361BP Echivalent: MC3361B-MBR

MC3362P-MOT DUAL CONVERS. FM RECEIVER

MC3363DW-MOT DUAL CONV. FM RECEIVER

MC3371P-MOT FM IF, LC TUNING IC

MC3372P-MOT FM IF, LC OR CERAM TUNING

MC3373P-MOT REMOTE CONTROL AMP/PREAMP

MC34001P Echivalent: TL071CN-MBR

MC34002P Echivalent: TL072CN-MBR

MC34004P Echivalent: TL074CN-MBR

MC3401 Echivalent: LM3900N-MBR

MC34014 Echivalent: MC34014P-MOT

MC34014P-MOT TELEPHONE SPEECH CIRCUIT

MC34018P-MOT VOICE SW. SPEAKERPHONE IC

MC3401P-MBR Echivalent: LM3900N-MBR

MC3403 Echivalent: LM324N-MBR

MC34051P-MOT 2xLINE TRANSCEIVER RS422/

MC34060AP-MOT PWM CNTRL 42V 0.25A

MC34063AP-MBR SMPS CIRCUIT, VMSO

MC34064P5-MOT POWER SUPPLY ACCESSORY DE

MC34081P Echivalent: LF411CN-MBR

MC34082P Echivalent: MC1458N-MBR

MC34083P-MOT DUAL OPAMP

MC34084P-MOT QUAD OPAMP

MC34119P-MOT PWR AMP 1*55mW/16E 3V 1%

MC34151P-MOT MOTOR DRIVER

MC34163P-MOT POWER SWITCHING REGULATOR

MC34164P-5-MOT VOLTAGE DETECTOR

MC34166T-MOT SWITCH MODE POWER SUPPLY

MC34182 Echivalent: TL062CN-MBR

MC3423 Echivalent: MC3423P-MBR

MC3423P-MBR POW. SUPPLY SUP. DEVICE

MC3450P-MBR 4xLINE RECEIVER

MC3456 Echivalent: NE556N-MBR

MC3470P-MBR HEAD AMPL. DISK DRIVE

MC3479P-MOT STEPPER MOTOR CIRCUIT

MC3486N-MBR 4xRS422/3 RECEIVER 2xENAB

MC3487N-MBR 4xRS422 DRIVER 2xENABLE

MC3488AP1-MOT LINE DRIVER

MC3556 Echivalent: NE556N-MBR

MC4024P-MOT DUAL ASTABLE MULTI VIBR.

MC4044P-MOT PHASE FREQ. DETECTOR

MC44000P-MOT Echivalent: MC44001P-MOT

MC44001P-MOT MULTISTANDARD DECODER

MC44002P-MOT COLOR DECODER CIRCUIT

MC44007P-MOT TV/VID SIGNAL PROCESSOR

MC44107P-MOT

MC44130P-MOT TV/VIDEO SOUND CIRCUIT

MC44131-MBR I.C.

MC44140P-MOT CTV CHROMA DELAY

MC44602P2-MOT CONSTANT CURRENT SOURCE

MC44603P-MOT SMPS I+V-MODE IC 250kHz

MC44802AP-MOT DIGITAL TUNER CIRCUIT

MC44802P-MOT Echivalent: MC44802AP-MOT

MC4558CN-MBR OPAMP 2x 44V 3MHz 1V/us

MC4558S Echivalent: NJM4558SD-JRC

MC4741 Echivalent: LM348N-MBR

MC6800 Echivalent: 6800P-MBR

MC6802P Echivalent: 6802P-MBR

MC6803 Echivalent: 6803P-MBR

MC6803P Echivalent: 6803P-MBR

MC6804 Echivalent: 209.11051

MC6809 Echivalent: 6809P-MBR

MC6809P Echivalent: 6809P-MBR

MC6821P Echivalent: 6821P-MBR

MC6840 Echivalent: 6840P-MBR

MC6845P Echivalent: 6845P-MBR

MC6850P Echivalent: 6850P-MBR

MC68705P3S-MOT CPU 8B 1MHz 112 RAM 1800E

MC68705R3S CPU 8B 1MHz 112RAM 1800EP

MC68B21P-MBR Echivalent: 68B21P-MBR

MC75451 Echivalent: SN75451-MBR

MC75491 Echivalent: SN75491-MBR

MC7805 Echivalent: UA7805-MBR

MC7808 Echivalent: UA7808-MBR

MC7809 Echivalent: UA7809-MBR

MC7810 Echivalent: UA7810-MBR

MC7812 Echivalent: UA7812-MBR

MC7815 Echivalent: UA7815-MBR

MC7824 Echivalent: UA7824-MBR

MC78L05 Echivalent: UA78L05-MBR

MC78XX Echivalent: UA78XX-MBR

MC7915 Echivalent: UA7915-MBR

MC7924 Echivalent: UA7924-MBR

MCA640 Echivalent: TCA640/DIV

MCA650 Echivalent: TCA650-MBR

MCM144102P-MOT SRAM 16x16 TUNING MEM

MCM14537CP-MOT SRAM 256X1 200NS

MCM2802P-MOT EEPROM 32x32BIT

MCM4517P Echivalent: 4116-150

MCR100-6 Echivalent: P0102DA-ST

MCR106-6 Echivalent: C106D-MOT

MCT210 OPTOISO 7K5V 45V >150% 4/

MCT2201 OPTOISO 5.3KV 45V >100% 1

MCU2600-ITT DIG.2000 CLOCK GENERATOR

MCU2632-ITT DIG 2000 CLOCK-GENERAT 18

MDA1044 Echivalent: TDA1044/DIV

MDA2010 Echivalent: TDA2020/DIV

MDA2020 Echivalent: TDA2020/DIV

MDA2060-ITT AMPL. 22V 3.5A 20W

MDA2061-ITT Echivalent: MDA2062-ITT

MDA2062-ITT EEPROM 128x8 OC.

MDA2062/DIV EEPROM 128x8 OC.

MDA3505 Echivalent: TDA3505/DIV

MDA4290V Echivalent: TDA4290V

MDA8442 Echivalent: TDA8442/DIV

MEA2050-ITT A/D AND BUS CONVERTER

MEA2901-ITT TUNER INTERFACE

MF 0.05A KLEINSICHERUNG FLINK TR3

MF 0.063A KLEINSICHERUNG FLINK TR3

MF 0.08A KLEINSICHERUNG FLINK TR3

MF 0.125A KLEINSICHERUNG FLINK TR3

MF 0.16A KLEINSICHERUNG FLINK TR3

MF 0.1A KLEINSICHERUNG FLINK TR3

MF 0.25A KLEINSICHERUNG FLINK TR3

MF 0.2A KLEINSICHERUNG FLINK TR3

MF 0.315A KLEINSICHERUNG FLINK TR3

MF 0.4A KLEINSICHERUNG FLINK TR3

MF 0.5A KLEINSICHERUNG FLINK TR3

MF 0.63A KLEINSICHERUNG FLINK TR3

MF 0.8A KLEINSICHERUNG FLINK TR3

MF 1.25A KLEINSICHERUNG FLINK TR3

MF 1.6A KLEINSICHERUNG FLINK TR3

MF 1A KLEINSICHERUNG FLINK TR3

MF 2.5A KLEINSICHERUNG FLINK TR3

MF 2A KLEINSICHERUNG FLINK TR3

MF 3.15A KLEINSICHERUNG FLINK TR3

MF 4A KLEINSICHERUNG FLINK TR3

MF 5A KLEINSICHERUNG FLINK TR3

MF10CCN-NSC DUAL FILTER, 2nd ORDER

MFE201-MOT

MGF1302-MIT N-FET 6V 0.1A 0.3W 4GHz

MHB193-TSL

MIC5801 Echivalent: UCN5801A

MJ1000 Echivalent: MJ1001

MJ10001-MBR N-DARL+D 500V 20A 175W B>

MJ10005-MBR N-DARL+D 500/400V 20A 175

MJ1001 N-DARL 80V 8A 90W =BDX63A

MJ10012-MBR N-DARL+D 600V 10A 175W

MJ10015-MOT Echivalent: MJ10016-MOT

MJ10016-MOT N-DARL+D 500V 50A 250W 1U

MJ11011 Echivalent: MJ11015-ST

MJ11012 Echivalent: MJ11016-ST

MJ11013 Echivalent: MJ11015-ST

MJ11014 Echivalent: MJ11016-ST

MJ11015-MBR P-DARL 120V 30A 200W

MJ11016-MBR N-DARL 120V 30A 200W

MJ11031 Echivalent: MJ11033-MOT

MJ11032-MOT N-DARL 120V 50A 300W

MJ11033-MOT P-DARL 120V 50A 300W

MJ15001 Echivalent: MJ15003-MBR

MJ15002 Echivalent: MJ15004-MOT

MJ15003-MBR SI-N 140V 20A 250W 3MHz

MJ15003/DIV SI-N 140V 20A 250W 3MHz

MJ15004-MOT SI-P 140V 20A 250W 3MHz

MJ15004/DIV SI-P 140V 20A 250W 3MHz

MJ15015-MOT SI-N 120V 15A 180W 0.8MHz

MJ15015/DIV Echivalent MJ15015-MOT

MJ15016-MOT SI-P 120V 15A 180W 0.8MHz

MJ15016/DIV SI-P 120V 15A 180W 0.8MHz

MJ15022-MOT SI-N 350/200V 16A 250W

MJ15022/DIV SI-N 350/200V 16A 250W

MJ15023-MOT SI-P 350V 16A 250W 4MHz

MJ15023/DIV SI-P 350V 16A 250W 4MHZ

MJ15024-MOT SI-N 250V 16A 250W

MJ15024/DIV Echivalent MJ15024-MOT

MJ15025-MOT SI-P 400V 16A 250W 4MHz

MJ15025/DIV SI-P 400V 16A 250W 4MHZ

MJ16018-MBR SI-N 1500V 10A 175W

MJ2500 Echivalent: MJ2501-ST

MJ2501-ST P-DARL 80V 10A 150W

MJ2955-MBR SI-P 100V 15A 150W 4MHz

MJ2955/DIV SI-P 100V 15A 150W 4MHZ

MJ3000 Echivalent: MJ3001-ST

MJ3001-ST N-DARL 80V 10A 150W

MJ4032-ST P-DARL 100V 10A 150W

MJ4033 Echivalent: MJ4035-ST

MJ4034 Echivalent: MJ4035-ST

MJ4035-ST N-DARL 100V 16A 150W

MJ413-MBR SI-N 400V 10A 125W > 2.5M

MJ4502-MBR SI-P 100V 30A 200W

MJ802-MBR SI-N 90V 30A 200W

MJ802/DIV SI-N 90V 30A 200W

MJ900 Echivalent MJ901-ST

MJ901-MBR Echivalent MJ2501-ST

MJE13004-MBR SI-N 300V 4A 75W TO220

MJE13005-MBR SI-N 300V 8A 75W

MJE13005/DIV SI-N 300V 8A 75W

MJE13006 Echivalent: MJE13007-MBR

MJE13007-MBR SI-N 400V 8A 80W

MJE13008 Echivalent: MJE13009-MBR

MJE13009-MBR SI-N 400V 12A 100W

MJE13009/DIV SI-N 400V 12A 100W

MJE15003 Echivalent: MJ15003-MBR

MJE15028-MBR Echivalent: MJE15030-MBR

MJE15029 Echivalent: MJE15031-MBR

MJE15030-MBR SI-N 150V 8A 50W 30MHz

MJE15031-MBR SI-P 150V 8A 50W 30MHz

MJE16002-MOT

MJE18004-MOT SI-N 450V 5A 100W 13MHz

MJE18006-MOT SI-N 450V 6A 100W 14MHz

MJE18008-MBR SI-N 450V 8A 125W 0.3US

MJE200 Echivalent: BD441-MBR

MJE210-MBR SI-P 40V 5A 15W >65MHZ

MJE243-MBR SI-N 100V 4A 15W >40MHz

MJE250 Echivalent: MJE253-MBR

MJE253-MBR SI-P 100V 4A 15W >40MHz

MJE270-MBR N-DARL 100V 2A 15W >16MHz

MJE271-MBR P-DARL 100V 2A 15W B>1K5

MJE2955T-MBR SI-P 70V 10A 90W AFPOWER

MJE2955T/DIV SI-P 70V 10A 90W

MJE3055T-MBR SI-N 70V 10A 90W AFPOW.

MJE3055T/DIV Echivalent MJE3055T-MBR

MJE340-MBR SI-N 300V 0.5A 20W VIDPOW

MJE340/DIV SI-N 300V 0.5A 20W

MJE350-MBR SI-P 300V 0.5A 20W VIDPOW

MJE350/DIV SI-P 300V 0.5A 20W

MJE5850-MOT SI-P 350/300V 8A 80W

MJE700 Echivalent: BD682-MBR

MJE800-MBR N-DARL+D 60V 4A 40W B>750

MJE8502-MBR SI-N 700V 5A 80W B>750

MJF18004-MOT SI-N 450V 5A 35W 13MHz

MJF18006-MOT Echivalent MJF18008-MOT

MJF18008-MOT SI-N 450V 8A 45W 0.3us

MJF18204-MOT SI-N 600V 5A 35W 13MHz

MJM2352 Echivalent: NJM2352DD-JRC

MJW16018-MOT SI-P 800V 10A 150W 3MHZ

MJW16206-MOT SI-N 1200V 12A 150W 3MHz

MJW16212-MOT SI-N 650V 10A 150W

MK3883N4 Echivalent: Z80ADMA-MBR

MK3884N4 Echivalent: Z80ASIO-0-MBR

MK3885N4 Echivalent: Z80ASIO-1-MBR

MK50992 Echivalent: KS5805A

MK5156 Echivalent: M5156F1-MBR

ML8204 TWO TONE RINGER

ML8205-MBR TWO TONE RINGER

ML926

MM53107N-NSC

MM5318N-NSC DIGITAL CLOCK

MM53200N-NSC DIGITAL CODE TRANSCEIVER

MM5369N-NSC

MM5387AA/N-NSC

MM5402N-NSC DIGITAL CLOCK 40p

MM5426 Echivalent: 209.10123

MM5439N-NSC

MM5450N-MBR LED DISPLAY DRIVER

MM5451N-NSC LCD DISPLAY DRIVER

MM5455N-NSC CONSUMER CLOCK CIRCUIT

MM5456N Echivalent: 209.10123

MM5457N-NSC DIGITAL CLOCKS

MM5480N-NSC

MM5616 Echivalent: CD4016-MBR

MM5840N-NSC TV CLOCK DIGITAL

MM74C922 Echivalent: SN74C922-NSC

MM74C923N Echivalent: SN74C923N

MM74C925 Echivalent: SN74C925-NSC

MM80C98N-NSC Echivalent: CD40098-PHI

MN1219-MAT EEPROM 32X16 BIT

MN1220T-MAT EEPROM 64*10 10us

MN1226-MAT MEMORY IC

MN1228-MAT EPROM 256x16 10us

MN1250BJC

MN1276-MAT IC RMN1276

MN1280-MAT ANALOG SIGN.COND.PEAKHOLD

MN152451GWA CPU

MN15245KWC MICROPROCESSOR+ROM ONWA

MN15245SAY1 uCOMPUTER

MN15261PDU IC RMN15261PDU

MN1527KMA Echivalent: 209.70258

MN15283VJU MICROPROCESSOR WITH ROM-C

MN1551CMR1-MAT

MN3004-MAT

MN3005-MAT 4096 STAGE 200ms 15V

MN3007-MAT ALSO: TDA2107-PHI

MN3008-MAT 2048 STAGE 102.4ms 15V

MN3010-MAT 512-STAGE ANAL.SHIFT REG.

MN3011-MAT 3328-STAGE DELAY

MN3101-MAT CMOS CLOCK GEN/DR. 15V

MN3102-MAT CLOCK GEN/DRIVER 5V

MN3207-MAT 1024 STAGE 51.2ms 5V

MN3208-MAT CLOCK GENERATOR

MN3802A-MAT

MN4264P-15 DRAM 16KX4 150NS

MN53200 Echivalent: MM53200N-NSC

MN6030B-MAT I.C.

MN6030ES-MAT I.C.

MN6076-MAT CLOCK CIRCUIT

MN6163A-MAT CHROMA SIGNAL PROCESSOR

MN6168VIA-MAT VTR DIG.SERVO DRIVER

MN650-SKN QUAD FET OPAMP

MN6748KAA-MAT I.C.

MOC3010 OPTO-TRIAC DR 250V 1A

MOC3020 Echivalent K3020P

MOC3021 OPTO-TRIAC DR. 400V IFT=1

MOC3023 OPTO-TRIAC DR. 400V IFT=5

MOC3041 OPTO-TRIAC DR.400V 15mA Z

MOC3042 OPTO-TRIAC DR.400V 10mA Z

MOC3043 OPTO-TRIAC DR. 400V 5mA Z

MOC3062 OPTO-TRIAC DRVR 600V 1A/1

MOC8101 OPTO

MP4501-TOS P/N-ARRAY 4x

MPF102-MBR N-FET 25V 2mA Up<8V

MPF910 Echivalent: VN10KM-MBR

MPS2222A Echivalent: PN2222A-MBR

MPS3640-MOT SI-P 12V 80mA 635mW 500MH

MPS802 64K - 297 UPGRADE COMM.

MPSA05 Echivalent: MPSA06-MBR

MPSA06-MBR SI-N 80V 0.5A 0.625W DRIV

MPSA10 SI-N 40V 0.1A 0.21W 50MHz

MPSA106E TRANSISTOR

MPSA12-MBR N-DARL 20V 0.5A 0.625W

MPSA13 Echivalent: MPSA14-MBR

MPSA18-MBR SI-N 45V 0.2A 625mW 100MH

MPSA14-MBR SI-N 30V 0.5A 0.625W

MPSA42-MBR SI-N 300V 0.5A 0.625W

MPSA43 Echivalent: MPSA42-MBR

MPSA44-MBR SI-N 500V 0.3A 625mW 20MH

MPSA55-MBR Echivalent: MPSA56-CDL

MPSA56-MBR SI-P 80V 0.5A 0.625W DRIV

MPSA70-MBR SI-P 40V 0.1A 0.35W >125M

MPSA92-MBR SI-P 300V 0.5A 0.625W

MPSA93 Echivalent: MPSA92-MBR

MPSH10-MBR SI-N 25V 40mA 0.35W 650MH

MPSU01 Echivalent: MPSU06

MPSU05 Echivalent: MPSU06

MPSU06

MPSU10

MPSU45-MBR

MPSU51 Echivalent: MPSU06

MPSU55 Echivalent: MPSU56-MOT

MPSU56

MPSU60

MPSU95-MBR

MPX10DP-MOT PRESSURE/STRAIN GAUGE SEN

MR500 Echivalent: 1N5408-MBR

MR501 Echivalent: 1N5408-MBR

MR502 Echivalent: 1N5408-MBR

MR504 Echivalent: 1N5408-MBR

MR506 Echivalent: 1N5408-MBR

MR508 Echivalent: 1N5408-MBR

MR510 Echivalent: 1N5408-MBR

MR750 Echivalent: MR756-MBR

MR751 Echivalent: MR756-MBR

MR752 Echivalent: MR756-MBR

MR754 Echivalent: MR756-MBR

MR756-MBR SI-D 600V 6A/400Ap

MR820 Echivalent: MR826-DIO

MR821 Echivalent: MR826-DIO

MR822 Echivalent: MR826-DIO

MR824 Echivalent: MR826-DIO

MR826-DIO SI-D 600V 5A/300Ap 300ns

MR851 Echivalent: BYW95C-PHI

MR852 Echivalent: BYW95C-PHI

MR854 Echivalent: BYW95C-PHI

MR856 Echivalent: BYW95C-PHI

MRF237-MOT SI-N 36V 0.6A 4W 174MHz

MRF237/DIV Echivalent: MRF237-MOT

MRF238 Echivalent: BLW31-PHI

MRF455-MOT SI-N 36V 15A 60W 30MHz

MRF475 SI-N 20V 4A 4W 50MHz

MSA-0685 WIDEBAND AMPLIFIER =MAR6

MSA0885 Echivalent: MSA-0885

MSM16911 Echivalent: ER5911

MSM2128 Echivalent: 6116-100-MBR

MSM4030 Echivalent: CD4030-MBR

MSM5255RS-OKI Echivalent: M5255-OKI

MSM5840-141 Echivalent: 209.11156

MSM5840-84 Echivalent: 209.81792

MSM6322-OKI PITCH CONTROLER

MSM6965 I.C.

MSP2400PLC-ITT MULTISTD SOUND PROCESSOR

MSP2410DIP-ITT MULTISTANDARD SOUND PROCE

MSP3400BDIP-IT MULTISTD SOUND PROCESSOR

MSP3410BDIP-IT MULTISTD SOUND PROCESSOR

MT8870DIP DMTF GENERATOR/RECEIVER =

MT8880DIP DTMF TRANSCEIVER

MTP3N50-MBR Echivalent: IRF820-MBR

MTP3N60-MBR Echivalent STP3NA60-MBR

MTP6N60-MBR Echivalent: P6N60-MBR

MUR115-MOT Echivalent: MUR120-MOT

MUR120-MOT SI-D 200V 1A/35AP 25NS

MUR1510 Echivalent: BYV79/200-PHI

MUR1520 Echivalent: BYV79/200-PHI

MUR410-MOT Echivalent: MUR420-MBR

MUR420-MOT SI-D 100V 4A 25ns FRCR

MUR430 Echivalent: MUR460-MBR

MUR440 Echivalent: MUR460-MBR

MUR450 Echivalent: MUR460-MBR

MUR460-MBR SI-D 600V 4A/70Ap 50ns

MUR840 Echivalent: BYT08P/400-MBR

MUR880-MOT SI-D 800V 8A/100Ap

MV104 Echivalent: BB204B-MBR

MV601DP-PLS

MV8870DP Echivalent: MT8870DIP

N413M-NEC DIAC 29-37V 2A

N82S100N PROG.LOGIC ARRAY 50ns

N82S131-PHI TTL-PROM 512x4 30ns

NDSU45 Echivalent: MPSU45-MBR

NE4558D Echivalent: MC1458D-MBR

NE4558N Echivalent: MC4558CN-MBR

NE5007N-PHI DAC 8B

NE5008N-PHI DAC 8B

NE5018N-PHI DAC 1x8B 2us 0.2% UP-INTF

NE5019N-PHI DAC 1x8B 0.1% 2us UP-INTF

NE5020N-PHI DAC 1x10B 0.1% 5us

NE5037N-PHI ADC 1x6B 9us

NE5044N-PHI

NE5090N-PHI ADDRES. RELAY DR. 8x 0.2A

NE521N-PHI V-COMP 2x 14V 12ns DIFFER

NE522N-PHI V-COMP 2x 14V 15ns DIFFER

NE527N-PHI V-COMP 1x 30V 15ns

NE529N-MBR V-COMP 1x 30V 10ns

NE530N-PHI

NE531N-PHI OPAMP 1x 44V 1MHz 35V/us

NE532N Echivalent: LM358N-MBR

NE542N-PHI Echivalent: LM387N-MBR

NE544N-PHI

NE545B-PHI Echivalent: NE645N-MBR

NE5512N-PHI OPAMP 1x 32V 3MHz 1V/us

NE5514N-PHI OPAMP 4x 32V 3MHz 1V/us

NE5517N-PHI DUAL TRANSCOND.OPAMP 16p

NE5521N-PHI LVDT SIGNAL CONDITIONER

NE5532AN-MBR OPAMP 2x 44V 10MHz 9V/us

NE5532N-MBR OPAMP 2x 44V 10MHz 9V/us

NE5532P Echivalent: NE5532N-MBR

NE5533AN-PHI OPAMP 2x 44V 10MHz 13V/us

NE5533N-PHI OPAMP 2x 44V 10MHz 13V/us

NE5534AN-MBR OPAMP 1x 44V 10MHz 13V/us

NE5534N-MBR OPAMP 1x 44V 10MHz 13V/us

NE5535N-MBR DUAL HIGH SLEW RATE OPAMP

NE5537N-PHI SAMPLE+HOLD AMP. LOW LEAK

NE5539N-PHI OPAMP 1x 24V 350MHz 600V/

NE555-CMOS Echivalent: NE555NCMOS-MBR

NE555D-MBR SINGLE TIMER 8p SMD

NE555H-MBR TIMER TO99 HOUSING

NE555N-MBR RC TIMER 1x 16V 500KHz

NE555N/DIV RC TIMER 1x 16V 500KHz

NE555NCMOS-MBR C-MOS VERSION NE555

NE555P Echivalent: NE555N-MBR

NE5560N-PHI PWM CNTRL IC 100KHz

NE5561N-PHI PWM CNTRL IC 100KHz

NE5562N-PHI PWM CNTRL IC 600KHz

NE5568N-PHI SMPS CNTRL IC 100KHz 20mA

NE556D-MBR DUAL TIMER 14p SMD

NE556N-MBR RC TIMER 2x =SM8330/8336

NE556NCMOS-MBR C-MOS VERSION NE556

NE558N-MBR RC TIMER 4x 16V

NE5592N-PHI VIDEO-AMP 2x 16V 110MHz

NE564N-PHI PHASE LOCKED LOOP 5V 50MH

NE565N-MBR Echivalent NE565N/DIV

NE565N/DIV PHASE LOCKED LOOP

NE566N-MBR FUNCTION GENERATOR 24V

NE567F TONE/FREQ.DECODER PLL

NE567N-MBR TONE/FREQ.DECODER PLL

NE570N-PHI ANALOG COMPANDER 16p

NE571N-MBR ANALOG COMPANDER =UPC1571

NE572N-PHI DUAL-COMPANDER

NE575N-MBR

NE587N-PHI LED DECODER/DRIVER BCD-IN

NE5900N-PHI CALL PROGRESS DECODER

NE590N-PHI ADDRES.PERIPH.DR. 8x 0.3A

NE591N-PHI ADDRES.PERIPH.DR. 8x 0.3A

NE592N08-MBR VIDEO-AMP 16V 120MHz

NE592N14-MBR VIDEO-AMP 16V 120MHz

NE594N-MBR VAC.FLUORESC.DISPL DRV.

NE602 Echivalent: NE602AN-PHI

NE602AN-PHI DOUBLE BAL.MIXER+OSC.

NE604AN-PHI Echivalent: NE614AN-PHI

NE605N-PHI LOW POWER MIXER FM IF SYS

NE612AN-PHI Echivalent: SA612AN-PHI

NE614AN-PHI LOW POWER FM IF SYST.

NE645B Echivalent: NE645N-MBR

NE645N-MBR

NE646B-MBR

NE646N Echivalent: NE646B-MBR

NE650N-MBR DOLBY B+C NOISE RED.16p

NE652 Echivalent: TEA0652-PHI

NE654N-MBR PRE-AMP.+DOLBY IC 24p

NE7555 Echivalent: NE555NCMOS-MBR

NJM2043D-JRC DUAL LOW NOISE PRE-AMP.

NJM2043DD-JRC Echivalent: NJM2043D-JRC

NJM2043L-JRC DUAL LOW NOISE PRE-AMP

NJM2043LD-JRC Echivalent: NJM2043L-JRC

NJM2043S-JRC Echivalent: NJM2043LD-JRC

NJM2043SD-JRC Echivalent: NJM2043S-JRC

NJM2068D-JRC DUAL OPAMP 15V 0.5W

NJM2068DD-JRC Echivalent: NJM2068D-JRC

NJM2068L-JRC DUAL OPAMP 15V

NJM2068LD-JRC Echivalent: NJM2068L-JRC

NJM2068S-JRC Echivalent NJM2068LD-JR

NJM2068SD-JRC Echivalent: NJM2068S-JRC

NJM2073D-JRC DUAL AUDIO AMPLIFIER

NJM2073DD-JRC Echivalent: NJM2073D-JRC

NJM2352D-JRC SWITCHING REGULATOR

NJM2352DD-JRC Echivalent: NJM2352D-JRC

NJM2360D-JRC DC-DC CONVERTER CONTROLLE

NJM2360DD-JRC Echivalent: NJM2360D-JRC

NJM2903D-JRC Echivalent: LM393N-MBR

NJM2904 Echivalent: LM2904N-MBR

NJM386D-JRC Echivalent: LM386N-MBR

NJM4556D-JRC DUAL HI CURRENT 18V

NJM4556DD-JRC Echivalent: NJM4556D-JRC

NJM4558L-JRC DUAL OPAMP SR=1V

NJM4558LD-JRC Echivalent: NJM4558L-JRC

NJM4558S-JRC DUAL OPAMP SR=1V ALSO:K

NJM4558SD-JRC Echivalent: NJM4558S-JRC

NJM4559D-JRC Echivalent: RC4559N-MBR

NJM4560D-JRC DUAL OPAMP 15V 0.5W

NJM4560DD-JRC Echivalent: NJM4560D-JRC

NJM4560M-JRC Echivalent UPC4560G-NEC

NJM4560MD-JRC Echivalent: NJM4560M-JRC

NJM4560S-JRC DUAL OPAMP 15V 0.5W

NJM4560SD-JRC Echivalent: NJM4560S-JRC

NJM4565L-JRC DUAL OPAMP 15V 0.5W

NJM4565LD-JRC Echivalent: NJM4565L-JRC

NJM4580L-JRC DUAL OPAMP 15V 0.8W

NJM5532D-JRC Echivalent: NE5532AN-MBR

NMC9306-NSC Echivalent: 9306-MBR

NVM3060-ITT EEPROM 512*8

O.3W 12V Echivalent: 0.4W 12V

OA202 SI-D 150V 0.16A 3.5us

OA47 Echivalent: AAZ18

OA90 GE-D 20V 8mA/45mAp <3.2V

OA91 GE-D 115V 50MA/0.15AP <3V

OA95 GE-D 115V 50mA/0.15Ap <2V

OEC0010C/D I.C.

OEC0017 I.C.

OEC1021A Echivalent: OEC1021B

OEC1021B I.C.

OEC3008 I.C.

OEC8012

OEC8034 I.C.

OEC9005 I.C.

OEC9009 I.C.

OEC9011 I.C.

OF498 Echivalent: BY228-PHI

OF517 Echivalent: 0.3W 1V5

OF799 Echivalent: BY228-PHI

OFWK6260 Echivalent: TDA4452-TFK

OM2060-PHI WIDEBAND AMPLIFIER

OM335-PHI Echivalent: OM2070-PHI

OM350-PHI NO LONGER AVAILABLE

OM360L-PHI HYBRID AMP. 12V 1GHz 23dB

OM361-PHI Echivalent OM2064-PHI

OM931-PHI HYBRID IC VHF/UHF WIDEBAN

OM961-PHI Echivalent: OM991-PHI

ON4046 Echivalent: BU508A-PHI

ON4147 Echivalent: BU508A-PHI

ON4152 Echivalent: BU508D-PHI

ON4213 Echivalent: BUT11A-ST

ON4223 Echivalent: BU508D-PHI

ON4226 Echivalent: BUT11A-ST

ON4302 Echivalent: BU508AF-PHI

ON4341 Echivalent: BUT12A-PHI

ON4359-PHI N-DARL+D 120V 4A 40W >10M

ON4436 Echivalent: 130.61714

ON4466 Echivalent: BUZ90A-SIE

ON4508-PHI TRANSISTOR

ON4521 Echivalent: BUZ90A-SIE

ON4542-PHI SELECTED BU508A ON HFE

ON4673 Echivalent: BU2508AF-PHI

ON4843-PHI TRANSISTOR

ON4845 Echivalent: BU2506DF-PHI

ON4923 Echivalent: BU2525A-PHI

ON611 Echivalent: 2SC2466-HIT

ON724 Echivalent: BDX65C

ON769 Echivalent: 130.41436

ON823 Echivalent: TIP132-MBR

ON946 Echivalent: BD538-MBR

ON974 Echivalent: BU508A-PHI

OP07CP-MBR OP-AMP

OP07CT

OP249S-MBR DUAL HIGH SPEED OPAMP

OP27 Echivalent: OP27GP-MBR

OP27GP-MBR LOW NOISE HI-SPEED OPT.IC

OPA27GP Echivalent: OP27GP-MBR

P0102AA Echivalent: P0102DA-ST

P0102AB Echivalent: P0102DA-ST

P0102BA Echivalent: P0102DA-ST

P0102BB Echivalent: P0102DA-ST

P0102CB Echivalent: P0102DA-ST

P0102DA-ST THYR. 400V 0.8A 0.2mA 0.2

P0102DB-ST Echivalent: P0102DA-ST

P0201MA-ST THYR. 600V 0.8A 10uA 0.2m

P3N60 Echivalent: STP3NA60-MBR

P3NA60 Echivalent: STP3NA60-MBR

P4NA60FI Echivalent: STP4NA60F-ST

P4NA80FI Echivalent: STP4NA80F-ST

P5N60 Echivalent: P6N60-MBR

P600A Echivalent: P600K-MBR

P600B Echivalent P600K-MBR

P600D Echivalent: P600K-MBR

P600G Echivalent: P600K-MBR

P600J Echivalent: P600K-MBR

P600K-MBR SI-D 800V 6A/400Ap

P600K/DIV Echivalent P600K-MBR

P600M-MBR SI-D 1000V 6A/400Ap

P6N60-MBR N-FET 600V 6A 125W 1E8

P6NA60 Echivalent: P6N60-MBR

P8031AH Echivalent: 8031AH-MBR

P8032AH Echivalent: 8032DIP-MBR

P8052AH Echivalent: 8052DIP-MBR

P80C31 Echivalent: 80C31DIP-MBR

P82B715PN-PHI I2C-BUS EXTENDER

P82C206 Echivalent: 82C206PLCC

P8749H Echivalent: 8749HD-MBR

PA0016 I.C.

PA0018 I.C.

PA2007 I.C.

PA2008 I.C.

PA2018 I.C.

PA3002 I.C. =GXX1001

PA3003 I.C.

PA3005 Echivalent: HA1384-HIT

PA3029A Echivalent: HA13151-HIT

PA3029B Echivalent: HA13151-HIT

PAL002A Echivalent: TDA7394-ST

PBL3717A Echivalent: PBL3717N-MBR

PBL3717N-MBR STEPPER MOTOR DRIVER

PBL3781 TELEPHONE SPEECH CIRCUIT

PBYR1040-MBR SB-D 40V 10A/125Ap

PBYR1545CT Echivalent: MBR1545CT-MBR

PBYR1645-PHI Echivalent: MBR1645-MBR

PBYR3040WT-MBR SB-D 40V 30A/300Ap

PC101 OPTOISO 5KV 35V 60% 6/8US

PC111 OPTOISO 5KV 35V 100% 3US

PC113 ALSO: 130.90735

PC120FB OPTO KOPPLUNG

PC121 OPTOISO 5KV 70V 4us AC-in

PC123 OPTOISO 5KV 70V 18us

PC814 NPN-OUT/AC-IN OPTOCOUPLER

PC816 OPTOCOUPLER

PC817 OPTOISO 5kV 35V 50% 18US

PC827 OPTOISO 2* 5KV 35V 50% 6u

PC849 OPTOISO 4* 35V 50% 4us

PCA84C440P401 Echivalent PCA84C640P30

PCA84C444P504 Echivalent: CTV222S-PHI

PCA84C640P016 CPU 8B 10MHZ 6K ROM 128B

PCA84C640P019 Echivalent: PCA84C640P30/2

PCA84C640P030 Echivalent: PCA84C640P30/2

PCA84C640P30/1 Echivalent: PCA84C640P30/2

PCA84C640P30/2 CPU 8B 10MHz 6KROM 128RAM

PCA84C641P068 CPU 8B 10MHz 6KROM/128RAM

PCA84C641P524 CPU 8B 10MHz 6KROM 128RAM

PCA84C840P185 CPU 8B 10MHz 8KROM 128RAM

PCA84C841P086 CPU 8B 10MHz 8KROM 128RAM

PCB8573P Echivalent: PCF8573P-PHI

PCC89 Echivalent ON REQUEST

PCD3310-PHI PULSE & DTMF DIALER + RED

PCD3311P-PHI DIALER CIRCUIT

PCD3312P-PHI DTMF MODEM GENERATOR 8p

PCD8571 Echivalent: PCF8571P-PHI

PCD8572-8P Echivalent: PCF8582C2P-PHI

PCD8582P-MBR ALSO: PCF8582E2P-PHI

PCD8584 Echivalent: PCF8584P-PHI

PCF1251-PHI VOLTAGE DETECTOR

PCF200 ELEKTRONENROHRE

PCF201 ELEKTRONENROHRE

PCF802 ELEKTRONENROHRE

PCF84C81AP049 CPU 8B 256B RAM 8K ROM=CT

PCF8570P-PHI SRAM 256x8 CMOS 12C

PCF8571P-PHI Echivalent: PCF8570P-PHI

PCF8572P-PHI Echivalent: PCF8582C2P-PHI

PCF8573P-PHI CLOCK CALENDER

PCF8574 Echivalent: PCF8574AP-PHI or P

PCF8574AP-PHI REMOTE 8-BIT I/O EXP. I2C

PCF8574P-PHI REMOTE 8BIT I/O EXP. I2C

PCF8581P-PHI Echivalent: PCF8582C2P-PHI

PCF8582AP-PHI Echivalent: PCF8582C2P-PHI

PCF8582C2P-PHI EEPROM 256x8 250ns I2C 2-

PCF8582E2P-PHI Echivalent: PCF8582C2P-PHI

PCF8583P-PHI CLOCK CALENDAR + 256B RAM

PCF8584P-PHI I2C BUS CONTROLER

PCF8591P-PHI 8-BIT AD/DA CONVERTER

PCF8593P-PHI REAL TIME CLOCK

PCH200 ELEKTRONENROHRE

PCL805 ELEKTRONENROHRE

PCL86 ELEKTRONENROHRE

PCM56P-BB DAC 1x16B SER.INPUT AUDIO

PCM56U-BB DAC 1x16B SER.IN AUDIO SM

PDA54-11EWA 14 SEGM.DISPL.14mm SUPERR

PDA54-11GWA 14 SEGM.DISPL.14mm GRUN C

PDA54-11HWA 14 SEGM.DISPL.14mm ROT CA

PDC54-11EWA 14 SEGM.DISPL.14mm SUPERR

PDC54-11HWA 14 SEGM.DISPL.14mm ROT CC

PDC54-11YWA 14 SEGM.DISP.14mm GELB CC

PH2222A-PHI SI-N 75V 0.8A 0.5W PLASTI

PH2369-MBR SI-N 15V 0.5A 0.5W 12/18n

PIC12C508-04/P CPU 8B 4MHz 512EPROM 25 R

PIC12C509-04/P CPU 8B 4MHz 1K ROM 41 RAM

PIC16C54A-04/P CPU 8B 4MHz 512OTP/25RAM

PIC16C54XT/P ALSO: PIC16C54A-04/P

PIC16C55HS/P CPU 8B 20MHz 512 OTP/25RA

PIC16C56XT/P CPU 8B 4MHz 1K OTP/25RAM

PIC16C57XT/P CPU 8B 4MHz 2K OTP/72RAM

PIC16C84-04/P CPU 8B 4MHz 1KEEPROM 36RA

PIC16F84-04/P CPU 8B 1MHz 68RAM 1KEEPRO

PIC600-MBR SMPS CIRCUIT

PIC625 SW-REG 60V 15A 100kHz

PL504 ELECTRONENBUIS

PL508 Echivalent ON REQUEST

PL519 ELEKTRONENROHRE

PL95 Echivalent ON REQUEST

PLL02A I.C.

PLL08A I.C.

PN2222A-MBR SI-N 75V 0.8A 0.5W PLASTI

PN2907-MBR SI-P 40V 0.6A 0.4W PLASTI

PN2907A-MBR SI-P 60V 0.6A 0.4W PLASTI

PN3563-MBR SI-N 30V 50mA 0.2W 600MHz

PN3638 SI-P 25V 0.5A 0.625W 100M

PS6002 Senzor

PSA08-11EWA 16 SEGM.DISPL.20mm SUPERR

PSA08-11GWA 16 SEGM.DISPL.20mm

PSA08-11HWA 16 SEGM.DISPL.20mm ROT CA

PSA08-11YWA 16 SEGM.DISPL.20mm

PSB4500P-SIE

PSB6520-2 Echivalent: LS1240A-ST

PSC08-11EWA 16 SEGM.DISPL.20mm SUPERR

PSC08-11HWA 16 SEGM.DISPL.20mm ROT CC

PT2014 PNP TRANSISTOR

PVPU2200-ITT CTV VIDEO PROCESSOR

PVPU2203-ITT CTV VIDEO PROCESSOR

PVPU2204DIP-IT

PY500A ELEKTRONENROHRE

Q4004LT Echivalent: Q4006LT

Q4006LT TRIAC+DIAC 400V 6A 2mA

Q4008LT TRIAC+DIAC 400V 8A 2mA

Q4010LT TRIAC+DIAC 400V 10A

R1003 Echivalent: DTC124ES-RHM

R1004 SI-N 50V 0.1A 47K/47K

R1008 Echivalent: DTC144WS-RHM

R2KN-SKN SI-D 140V 1A

R2KY-SKN SI-D 160V 1A

R2M/DIV UNIDIR TRANSIL 130V 1A

R2M-SKN UNIDIR TRANSIL 130V 1A

R4050 Echivalent: BU508A-PHI

R4051 Echivalent: BU508A-PHI

R4060 Echivalent: BU506-PHI

RB100A-RHM SB-D 40V 1A SCHOTTKY

RC4136-MBR 4xOPAMP 1MHz PRECISION

RC4151-MBR VOLTAGE TO FREQUENCY CONV

RC4152-MBR Echivalent: LM331N-NSC

RC4194N-MBR 2xAP V-REG 0.1-42V 0.2A

RC4558P Echivalent: MC4558CN-MBR

RC4559D ALSO: 209.71766

RC4559N-MBR OPAMP HIGH PERF

RC4560N-MBR Echivalent: NJM4560DD-JRC

RFP40N10-HRS N-FET 100V 40A 160W 0.04E

RFS 0.04A KLEINSICHERUNG FLINK TR5

RFS 0.05A KLEINSICHERUNG FLINK TR5

RFS 0.063A KLEINSICHERUNG FLINK TR5

RFS 0.08A KLEINSICHERUNG FLINK TR5

RFS 0.125A KLEINSICHERUNG FLINK TR5

RFS 0.16A KLEINSICHERUNG FLINK TR5

RFS 0.1A KLEINSICHERUNG FLINK TR5

RFS 0.25 MICROSICHERUNG TR5 0.25

RFS 0.25A ALSO: RFS 0.25

RFS 0.2A KLEINSICHERUNG FLINK TR5

RFS 0.315 MICROSICHERUNG TR5 0.315

RFS 0.315A ALSO: RFS 0.315

RFS 0.4 MICROSICHERUNG TR5 0.4

RFS 0.4A ALSO: RFS 0.4

RFS 0.5 MICROSICHERUNG TR5 0.5

RFS 0.5A ALSO: RFS 0.5

RFS 0.63 MICROSICHERUNG TR5 0.63

RFS 0.63A ALSO: RFS 0.63

RFS 0.8 MICROSICHERUNG TR5 0.8

RFS 0.8A ALSO: RFS 0.8

RFS 1 MICROSICHERUNG TR5 1

RFS 1.25 MICROSICHERUNG TR5 1.25

RFS 1.25A ALSO: RFS 1.25

RFS 1.6 MICROSICHERUNG TR5 1.6

RFS 1.6A ALSO: RFS 1.6

RFS 1A ALSO: RFS 1

RFS 2 MICROSICHERUNG TR5 2A

RFS 2.5 MICROSICHERUNG TR5 2.5

RFS 2.5A ALSO: RFS 2.5

RFS 2A ALSO: RFS 2

RFS 3.15 MICROSICHERUNG TR5 3.

RFS 3.15A ALSO: RFS 3.15

RFS 4 MICROSICHERUNG TR5 4

RFS 4A ALSO: RFS 4

RFT 0.05A KLEINSICHERUNG TRAGE TR5

RFT 0.063A KLEINSICHERUNG TRAGE TR5

RFT 0.08 MICROSICHERUNG TR5 0.08A

RFT 0.08A ALSO: RFT 0.08

RFT 0.1 MICROSICHERUNG TR5 0.1A

RFT 0.125 MICROSICHERUNG TR5 0.125A

RFT 0.125A ALSO: RFT 0.125

RFT 0.16 MICROSICHERUNG TR5 0.16A

RFT 0.16A ALSO: RFT 0.16

RFT 0.1A ALSO: RFT 0.1

RFT 0.2 MICROSICHERUNG TR5 0.2A

RFT 0.25 MICROSICHERUNG TR5 0.25A

RFT 0.25A ALSO: RFT 0.25

RFT 0.2A ALSO: RFT 0.2

RFT 0.315 MICROSICHERUNG TR5 0.315A

RFT 0.315A ALSO: RFT 0.315

RFT 0.4 MICROSICHERUNG TR5 0.4A

RFT 0.4A ALSO: RFT 0.4

RFT 0.5 MICROSICHERUNG TR5 0.5A

RFT 0.5A ALSO: RFT 0.5

RFT 0.63 MICROSICHERUNG TR5 0.63A

RFT 0.63A ALSO: RFT 0.63

RFT 0.8 MICROSICHERUNG TR5 0.8A

RFT 0.8A ALSO: RFT 0.8

RFT 1 MICROSICHERUNG TR5 1A

RFT 1.25 MICROSICHERUNG TR5 1.25A

RFT 1.25A ALSO: RFT 1.25

RFT 1.6 MICROSICHERUNG TR5 1.6A

RFT 1.6A ALSO: RFT 1.6

RFT 1A ALSO: RFT 1

RFT 2 MICROSICHERUNG TR5 2A

RFT 2.5 MICROSICHERUNG TR5 2.5A

RFT 2.5A ALSO: RFT 2.5

RFT 2A ALSO: RFT 2

RFT 3.15 MICROSICHERUNG TR5 3.15A

RFT 3.15A ALSO: RFT 3.15

RFT 4 MICROSICHERUNG TR5 4A

RFT 4A ALSO: RFT 4

RFT 5 MICROSICHERUNG TR5 5A

RFT 5A ALSO: RFT 5

RFT 6.3 ALSO: 254.10026

RFT-RFS ASS ASS. TR SICHERUNGEN 17 X

RGB K12-KIT K12 VIDEO AUSG.VERSTAERKE

RGP02-17-GI Echivalent: RGP02-20-GI

RGP02-18E-GI Echivalent: RGP02-20E-GI

RGP02-20E-GI SI-D 2000V 0.5A/20Ap 300n

RGP10G Echivalent: BYD33M-PHI

RGP10M Echivalent: BYD33M-PHI

RGP15K Echivalent: FR157

RGP15M Echivalent: FR157

RGP30M-DIO SI-D 1000V 3A/125Ap 500ns

RS606 Echivalent: KBU8M

RTA-3045 ABSTANDHAL. 3mm LED 4.5

RTA-3051 ABSTANDHAL. 3mm LED 5.1mm

RTA-3065 ABSTANDHALT.3mm LED 6.5mm

RTA-3070 ABSTANDHALT.3mm LED 7.0mm

RTA-3085 ABSTANDHALT.LED

RTA-3090 ABSTANDHALT.3mm LED9.0mm

RTA-3110 ABSTANDHALT.3mm LED11.0mm

RTA-3130 ABSTANDHALT.3mm LED 3.0mm

RTA-3140 ABSTANDHALT.3mm LED 14.0m

RTA-3150 ABSTANDHALT.3mm LED5.0mm

RTA-3160 ABSTANDHALT.3mm LED6mm

RTA-3170 ABSTANDHALT.3mm LED17.0mm

RTA-5045 ABSTANDHALT.5mm LED 4.5mm

RTA-5070 ABSTANDHALT.5mm LED7.0mm

RTA-5090 ABSTANDHALT.5mm LED 9mm

RTA-5120 ABSTANDHALT.5mm LED12.0mm

RTA-5127 ABSTANDHALT.5mm LED 12.7m

RTA-5140 ABSTANDHALT.5mm LED 14mm

RTA-5170 ABSTANDHALT.5mm LED17.0mm

RTA-5180 ABSTANDHALT.5mm LED18.0mm

RTA-5200 ABSTANDHALT.5mm LED20.0mm

RTF-1090 KUNSTOFF-FASSUNG LED 10mm

RTF-5010 KUNSTOFF-FASSUNGLED 5mm

RTF-5020 KUNSTOFF-FASSUNGR/REFL 5m

RTF-8080 KUNSTOFF-FASSUNG LED 8mm

RTM-1090 CHROMFASSUNG LED10mm EXCL

RTM-3010 CHROMFASSUNG LED 3mm LE

RTM-3020 CHROMFASSUNG LED 3mm LE

RTM-3030 CHROMFASSUNG LED 3mm LED

RTM-5010 CHROMFASSUNG LED 5mm LE

RTM-5020 CHROMFASSUNG LED 5mm LE

RTM-5030 CHROMFASSUNG LED 5mm LED

RTM-5040 CHROMFASSUNG LED 5mm

RTM-5050 CHROMFASSUNG LED 5mm

RTM-5060 CHROMFASSUNG LED+GLAS 5mm

RTM-8080 CHROMFASSUNG LED 8mm EX

RTM-8090 CHROMFASSUNG LED 8mm EXC

RTZ-3500 FASSUNG FUR 3MM LEDS

RUF01 Echivalent: SAA1124-ITT

RV4151NB Echivalent: RC4151-MBR

S0402DH-MBR THYR. 400V 4A 0.2uA

S0402MH-ST THYR. 600V 4A 0.2uA

S041P Echivalent: SO41P-SIE

S042P Echivalent: SO42P-SIE

S0807DH Echivalent: TIC116D-MBR

S0807MH Echivalent: TIC116M-MBR

S0807NH Echivalent: TIC116N-MBR

S1207DH Echivalent: TIC126D-MBR

S1207MH Echivalent: BT151/800R-PHI

S1207NH Echivalent: BT151/800R-PHI

S175 RF PWR AMP TRANSIST0R

S1854-TOS DRIVER STAGE 112.5/117.5V

S186P Echivalent BPW41N /50

S1998A LED ALARM CLOCK 40p

S2000A-TOS Echivalent: S2000A3

S2000A3 Echivalent BU2508A-PHI

S2000AF Echivalent: S2000AF-ST

S2000AF-ST SI-N 1500V 8A 50W 0.7us

S2000AF-TOS Echivalent: S2000N-TOS

S2000AF1 Echivalent: S2000AF-ST

S2000AF3-TOS Echivalent: S2000N-TOS

S2000AFI Echivalent: S2000AF-ST

S2000DF Echivalent: S2055N-TOS

S2000N-TOS SI-N 1500V 8A 50W 0.7us

S2003 Echivalent: SB20-03B-SAN

S200D Echivalent: 1N4148-MBR

S201S01-SHA Echivalent: S201S05-SHA

S201S02 Echivalent: S201S06-SHA

S201S05-SHA TRIAC 600V 3A 15mA

S201S06-SHA TRIAC 600V 3A 15MA +ZCROS

S202DS4 Echivalent: S202S02

S202S02-SHA TRIAC 600V 8A 15mA+ZERO C

S202SE2 Echivalent: S202S02

S2055A-TOS Echivalent: BU2508D-PHI

S2055AF-TOS Echivalent: S2055N-TOS

S2055F Echivalent: S2055N-TOS

S2055N-TOS SI-N+D 1500V 8A 50W 0.3us

S2060D Echivalent: TIC106M-MBR

S21ME4 Echivalent: TLP3061

S233CP-SIE MASK PROGR. ROM

S2512DH Echivalent: TYN688-ST

S2514NH Echivalent: 2N6509-MOT

S2530 Echivalent: S2530A

S2530A SI-N 1000V 10A 100W

S2560 Echivalent: BF256C-MBR

S2800D Echivalent: TIC126D-MBR

S2800M Echivalent: TIC126M-MBR

S2818 Echivalent: BU208D-ST

S2818A Echivalent: BU208D-ST

S3786-TOS TRANSISTOR

S3900S Echivalent: 15/80H

S3901S Echivalent: 15/85R

S4015L Echivalent: TYN616-ST

S45-M4X25 Echivalent: S45-M4X30

S65-B1.7X4B Echivalent: S65-B1.7X5

S8050 Echivalent: SS8050

S8550 Echivalent: SS8550

S921TS Echivalent: BF493-MBR

SA03-11EWA DISPL. 7mm C.A.SUPER-ROT

SA03-11GWA DISPL. 7mm C.A. GRUEN

SA03-11HWA DISPL. 7mm C.A. ROT

SA03-11YWA DISPL. 7mm C.A. GELB

SA04-11EWA DISPL.10mm C.A. SUPER-ROT

SA04-11GWA DISPL.10mm C.A. GRUEN

SA04-11HWA DISPL.10mm C.A. ROT

SA04-11YWA DISPL.10mm C.A. GELB

SA08-11EWA DISPL.20mm C.A. SUPER-ROT

SA08-11GWA DISPL.20mm C.A. GRUEN

SA08-11HWA DISPL.20mm C.A. ROT

SA08-11YWA DISPL.20mm C.A. GELB

SA10-21EWA DISPL.25mm C.A. SUPER-ROT

SA10-21GWA DISPL.25mm C.A. GRUEN

SA10-21HWA DISPL.25mm C.A. ROT

SA10-21YWA DISPL.25mm C.A. GELB

SA1013 Echivalent: 2SA1013-TOS

SA1359 Echivalent: 2SA1359-TOS

SA23-12EWA DISPL.57mm C.A. SUPER-ROT

SA23-12GWA DISPL.57mm C.A. GRUEN

SA23-12HWA DISPL.57mm C.A. ROT

SA23-12YWA DISPL.57mm C.C.

SA39-11GWA DISPLAY KBR 10mm CA GRUEN

SA39-11HWA DISPLAY KBR 10MM CA ROT

SA40-19EWA DISPLAY 100mm SUPERROT C-

SA40-19SRWA DISPLAY 100mm HYPERROT C-

SA40-19YWA DISPLAY 100mm GELB C-A

SA52-11EWA DISPL.13mm C.A. SUPER-ROT

SA52-11GWA DISPL.13mm C.A. GRUEN

SA52-11HWA DISPL.13mm C.A. ROT

SA52-11YWA DISPL.13mm C.A. GELB

SA56-11EWA DISPL.14mm C.A. SUPER-ROT

SA56-11GWA DISPL.14mm C.A. GRUEN

SA56-11HWA DISPL.14mm C.A. ROT

SA602AN Echivalent: NE602AN-PHI

SA612AN-PHI DOUBLE BALANCE MIXER+OSC.

SAA1000-ITT US REMOTE CRT.TRANS/RECV.

SAA1004-ITT FREQUENCY DEVIDER 14p

SAA1005-ITT

SAA1006-MBR DIODE MATRIX ENCODER

SAA1008 8-STEP BINARY DIVIDER 18p

SAA1010 US REM.CRT.DECOD.15 CODES

SAA1021 Echivalent: SAA1121-ITT

SAA1024-ITT

SAA1025-MBR US REM. CRT RECIEV=M1025B

SAA1027-PHI MOTOR DRIVE CIRCUIT

SAA1029-PHI Echivalent SAA1029/DIV

SAA1029/DIV LOGIC+INTFACE CRT. =D410D

SAA1042 Echivalent: SAA1042V-MOT

SAA1042AV-MOT

SAA1042V-MOT STEPPER MOTOR DRIVER 20V

SAA1050 Echivalent: SAA1250-1-ITT

SAA1051 Echivalent: SAA1251

SAA1056-PHI PLL FREQ. SYNTHESIZER 16p

SAA1057-MBR TUNING PLL FREQ.SYNTH.

SAA1058-MBR AM/FM FREQUENCY SYNTH.16p

SAA1060-PHI LED-DISPLAY INTERFACE 24p

SAA1061-PHI LED-DISPLAY INTERFACE

SAA1062-PHI LED-DISPLAY INTERFACE 24P

SAA1063-PHI DISPLAY DRIVER 0.9W

SAA1064-PHI VCR 4-DIGIT LED DRIVER

SAA1070 FREQ COUNTER/INTERFACE

SAA1071 Echivalent: SAA1271-ITT

SAA1073-ITT UHF DIVIDER/TUNER

SAA1074-ITT

SAA1075-ITT TV DIGITAL TUNING 18p

SAA1076 Echivalent: SAA1276

SAA1082P Echivalent: SAA3027P-PHI

SAA1084P Echivalent 209.80581

SAA1086P I.C.

SAA1089P I.C.

SAA1094-2-MBR TELEPH.SOUND ALARM

SAA1097 Echivalent: 209.81786

SAA1101P-PHI UNIVERSAL SYNC GENERATOR

SAA1121-ITT

SAA1124-ITT US REM-CNTRL TRANSM=RUF01

SAA1130-ITT US TRANSMITTER 30 CODES 1

SAA1174-ITT

SAA1220-ITT

SAA1250-1-ITT Echivalent: SAA1250-ITT

SAA1250-ITT IR REMOTE CRT TRANSM.

SAA1251 IR REMOTE CRT RECEIV.24p

SAA1271-ITT ADAPT.IC INFRARED RCV.18p

SAA1272A-ITT IR RC ADAPTER

SAA1272D REM CONTR IC

SAA1274 Echivalent: SAA1074-ITT

SAA1276-ITT CHARACTER GENERATOR

SAA1280

SAA1281

SAA1292-ITT

SAA1293-2-ITT Echivalent: SAA1293-3/DIV

SAA1293-3-ITT Echivalent: SAA1293-3/DIV

SAA1293-3/DIV TV REMOTE CNTRL RECEIVER

SAA1293A-3-ITT TV REMOTE CONTR. RECEIVER

SAA1293A-3/DIV TV REMOTE CONTR. RECEIVER

SAA1293A-6-ITT REMOTE CNTRL RECEIVER

SAA1294-ITT

SAA1300-PHI TUNER SWITCHING CIRC. 9p

SAA1310P-PHI CNTRL INTF FOR VHS RECORD

SAA1350-ITT IR REMOTE CRT TRANSM. 18p

SAA1351-ITT

SAA1900S-ITT TOY ORGAN IC 24p

SAA3004P-MBR IR RC TRANSM.448 COMM.

SAA3005 Echivalent: M3005AB1-ST

SAA3006P-PHI IR RC TRANSM.32x64CODES R

SAA3007P-PHI IR RC TRANSMITTER(455 KC)

SAA3008P-PHI IR TRANSMIT.20x64CODES RE

SAA3009P-PHI

SAA3010P-PHI HIGH PERFORM.TRANSM.RC5 2

SAA3027P-PHI IR REMOTE CTRL.TRANSM.RC5

SAA3028P-PHI

SAA3049P-PHI IR RECEIVER 32x64 CODES

SAA4700-PHI VPS DATALINE PROC.

SAA5000P-PHI

SAA5010 RC RECEIVER/DECODER 24p

SAA5012

SAA5020-PHI TXT TIMING CIRCUIT

SAA5030-PHI TXT VIDEO PROCESSOR 24p

SAA5040A-PHI TXT ACQUISITION+CNTR. 28p

SAA5040B-PHI TXT ACQUISITION+CNTR. 28p

SAA5041-PHI TXT ACQUISITION+CNTR. 28p

SAA5042-PHI TXT ACQUISITION+CNTR. 28p

SAA5050-PHI Echivalent 209.10046

SAA5051-PHI TXT CHARACT.GEN.(BRD) 28p

SAA5052-PHI TXT CHARACT.GEN.(SWE)

SAA5053-PHI TXT CHARACT.GEN.(ITALIAN)

SAA5190-PHI TELETEXT DECODER/PROCESSO

SAA5230-PHI TXT VIDEO PROCESSOR

SAA5231-MBR TXT VIDEO PROC. =SDA5231

SAA5235

SAA5240P/A-PHI COMP. CONTR. TXT CIRCUIT

SAA5240P/B

SAA5241P/B-PHI Echivalent: 209.82785

SAA5243P/E-PHI ECCT(WEST EUROPA)CIRC.

SAA5243P/H-PHI ECCT(EAST EUROPE)CIRC.

SAA5243P/L-PHI ECCT(ARABIC+UK)CIRC.

SAA5244APA-PHI SINGLE CHIP TXT DEVICE

SAA5244P/A-PHI Echivalent: SAA5244APA-PHI

SAA5246AP/E-PH TV I.C.

SAA5246AP/H Echivalent: 209.31215

SAA5246P/E-PHI

SAA5248ZP-PHI TXT / VPS DECODER

SAA5250P-PHI DATA ACQ.INTERFACE FOR TX

SAA5254P/E-PHI INTEGR. VIP AND TELETEXT

SAA5295P-MBR I.C.

SAA5351 CRT CONTROLER EUROM 50Hz

SAA5534 Echivalent: TDA1541-PHI

SAA7000-PHI CD-MUTING/INTERPOLATION I

SAA7020 CD ERROR CORRECTION 40p

SAA7030-PHI

SAA7210P-PHI DECODER FOR CDs 40p

SAA7220P/A-PHI DIF. FILTER FOR CD'S

SAA7274P-PHI AUDIO DIGITAL INP.CIRC.24

SAA7280P-PHI NICAM DECODER

SAA7310 ZIE KLADBLOK

SAA7321 Echivalent: 209.61708

SAA9042P/A Echivalent: SAA9042A-PHI

SAA9050-8A-PHI PAL/NTSC DEC.(DIG)I2C BUS

SAA9057-PHI CLOCK FOR DIG.VIDEO SIGN.

SAA9068WP-PHI PICTURE IN PICTURE CIRCUI

SAB0600-SIE

SAB0601-SIE

SAB1009 Echivalent: SAB1009BP

SAB1009BP 900Mhz WIDE BAND AMPL.14p

SAB1016 CHARACTER FADE IN 24p

SAB1018P-PHI

SAB1046P 1GHz FREQ.DIVIDER 14p

SAB1165N-PHI SINGLE CLOCK PRESCALER

SAB1256 Echivalent 209.60756

SAB2011 Echivalent: SAB3021

SAB2013P Echivalent 209.10017

SAB2015P TV TUNING CIRCUIT

SAB2016P TV TUNING CIRCUIT

SAB2021 Echivalent: SAB3021

SAB2022P TV AFT CIRCUIT

SAB2024P

SAB3011 Echivalent: SAB3021

SAB3012 IR RECEIVER, 2x64 CODES

SAB3013-PHI Echivalent 209.10102

SAB3013-SMD

SAB3013/DIV Echivalent: 209.10102

SAB3013P Echivalent: SAB3013-PHI

SAB3017 REMOTE CTR. INTERFACE 18p

SAB3021 RC TRANSM.2x64CODES =U807

SAB3027 Echivalent: SAA3027P-PHI

SAB3034 Echivalent: 209.10169

SAB3035-PHI TUNING FREQ.SYNTH.

SAB3037-PHI

SAB3042 IR RC RECEIVER 16p

SAB3044

SAB3209-SIE IR RC RECEIVER

SAB3210 IR RC TRANSMITTER

SAB4209

SAB5089 Echivalent: SBA5089-ITT

SAB600 Echivalent: SAB0600-SIE

SAB6456A-PHI 1 GHz PRESCALER 64/256

SAB8031 Echivalent: 8031AH-MBR

SAB8032 Echivalent: 8032DIP-MBR

SAB8048 CPU 8B 64B RAM 1K ROM

SAB80535 Echivalent: 80C535N-SIE

SAB80535N Echivalent: 80C535N-SIE

SAB80C535 Echivalent: 80C535N-SIE

SAB80C535N Echivalent: 80C535N

SAB8259 Echivalent: 8259AP-MBR

SAB8288 Echivalent: 8288-MBR

SAB8726-PHI SINGLE CLOCK PRESCALER

SAD1009P-PHI Echivalent: 209.83331

SAE0700-SIE Echivalent: SAE0700/DIV

SAE0700/DIV

SAE0800-SIE SOUND GENERATOR CIRCUIT

SAE700 Echivalent: SAE0700-SIE

SAE800 Echivalent: SAE0800-SIE

SAF0300-ITT

SAF1031P Echivalent: SAF1039P-PHI

SAF1032P-PHI Echivalent 209.10008

SAF1039P-PHI

SAF1054 PREAMP + ALC

SAF1092-ITT SPEEDMTR+MILEAGE INDIC.18

SAF1135P-PHI

SAK215-ITT Echivalent: SAK215/DIV

SAK215/DIV PULSE SHAPER REV-COUNT

SAL41B DIODE

SAS560 Echivalent: SAS560S/DIV

SAS560S/DIV Senzor FOR 4 KEYS+ PRESET

SAS560T Echivalent: SAS560S/DIV

SAS570 QUAD TOUCH AMPLIFIER

SAS570S IMPROVED SAS570 (EXTENSIO

SAS570T

SAS580 QUAD TOUCH AMPLIFIER

SAS590 4xTOUCH AMPL. EXTENSION

SAS660 Echivalent: SAS6600

SAS6600 QUAD TOUCH AMPL + PRESET

SAS6610 4-TOUCH AMPL+PRESET+DIODE

SAS670 Echivalent: SAS6700

SAS6700 QUAD TOUCH AMPL. EXTENSIO

SAS6710 Senzor FOR 4 KEYS + DIODE

SB120 Echivalent: 1N5819

SB130 Echivalent: 1N5819

SB140 Echivalent: 1N5819

SB150 Echivalent: SB160

SB160 SB-D 60V 1A/25Ap

SB20-03B-SAN SB-D 30V 2A/20Ap STKY

SB3100 SB-D 100V 3A/100AP

SB320 Echivalent: SB350

SB330-MBR Echivalent: SB350

SB340 Echivalent: SB350

SB350 SB-D 50V 3A/100Ap STKY

SB520 Echivalent: SB550

SB530 Echivalent: SB550

SB540 Echivalent: SB550

SB550 SB-D 50V 5A/85Ap STKY

SBA5089-ITT Echivalent: UM95089

SC03-12EWA DISPL. 7mm C.C. SUPER-ROT

SC03-12GWA DISPL. 7mm C.C. GRUEN

SC03-12HWA DISPL. 7mm C.C. ROT

SC03-12YWA DISPL. 7mm C.C. GELB

SC04-11EWA DISPL.10mm C.C. SUPER-ROT

SC04-11GWA DISPL.10mm C.C. GRUEN

SC04-11HWA DISPL.10mm C.C. ROT

SC08-11EWA DISPL.20mm C.C. SUPER-ROT

SC08-11GWA DISPL.20mm C.C. GRUEN

SC08-11HWA DISPL.20mm C.C. ROT

SC08-11YWA DISPL.20mm C.C. GELB

SC10-21EWA DISPL.25mm C.C. SUPER-ROT

SC10-21GWA DISPL.25mm C.C. GRUEN

SC10-21HWA DISPL.25mm C.C. ROT

SC10-21YWA DISPL.25mm C.C. GELB

SC107 Echivalent: BC107B-MBR

SC23-12EWA DISPL.57mm C.C. SUPER-ROT

SC23-12GWA DISPL.57mm C.C. GRUEN

SC23-12HWA DISPL.57mm C.C. ROT

SC23-12YWA DISPL.57mm C.C. GELB

SC39-11HWA DISPLAY KBR 10mm CK ROT

SC40-19EWA DISPLAY 100mm SUPERROT C-

SC40-19GWA DISPLAY 100mm GRUEN C-C

SC40-19SRWA DISPLAY 100mm HYPERROT C-

SC40-19YWA DISPLAY 100mm GELB C-C

SC52-11EWA DISPL.13mm C.C. SUPER-ROT

SC52-11GWA DISPL.13mm C.C. GRUEN

SC52-11HWA DISPL.13mm C.C. ROT

SC52-11YWA DISPL.13mm C.C. GELB

SC56-11EWA DISPL.14mm C.C. SUPER-ROT

SC56-11GWA DISPL.14mm C.C. GRUEN

SC56-11HWA DISPL.14mm C.C. ROT

SC56-11YWA DISPL.14mm C.C. GELB

SCN2661AC1N28 ENH.PROG.COMMUNICATE INTF

SCN2681AC1N40 DUAL ASYNCHR. REC./TRANS.

SCN68000CAN64 Echivalent: 68000CP10

SD340 Echivalent: BD140/DIV

SD3624A IC

SDA2001-SIE

SDA2002-SIE

SDA2003-SIE TV TUNING uCOMP.+ROM 40p

SDA2004-SIE TV TUNING 7-SEGM.DRIV.18p

SDA2005-SIE CHARACT.INSERT CTRL=SM311

SDA2006-SIE

SDA2007-SIE IR RC RECEIVER

SDA2008-SIE IR RC TRANSMITTER

SDA2010A002 CPU 8-BIT 2K ROM

SDA2010A006 Echivalent: SDA2010A022

SDA2010A016 Echivalent: SDA2010A022

SDA2010A019 Echivalent: SDA2010A022

SDA2010A021 Echivalent: 8305158018

SDA2010A022 CPU 8B 2kROM 64BRAM

SDA2010A024

SDA2010A025 CPU 8B 2 kB ROM

SDA2011 Echivalent: SDA2011A...

SDA2011A003 Echivalent: SDA2011A005

SDA2011A004 Echivalent: SDA2011A007

SDA2011A005 CPU 8B 2k ROM 64BRAM

SDA2011A007

SDA2014

SDA2023A002 Echivalent: SDA2023A502

SDA2023A003 CPU 8B 8kB ROM 128B RAM

SDA2023A502 CPU 8B 8k ROM =A502

SDA2030 Echivalent: SDA2030A...

SDA2030A009

SDA2040A002

SDA20561A008 Echivalent: 8305157568

SDA20562A001 Echivalent: 8945903447

SDA20563A502 CPU 8B WITH RAM & ROM

SDA20563A504 CPU 8B WITH RAM & ROM

SDA2080A007 CPU 8B 64K RAM + 2K ROM

SDA2080A031 Echivalent: SDA2080A034

SDA2080A034 =SDA2083A536 8945903644 (

SDA2080A035 Echivalent: SDA2080A034

SDA2080A036 CPU 8B 2k ROM 64B RAM

SDA2083 Echivalent: SDA2083A...

SDA2083A003 Echivalent: SDA2023A003

SDA2083A005 Echivalent: SDA2083A015

SDA2083A010 Echivalent: SDA2083A015

SDA2083A014

SDA2083A015 CPU 8B 8kB ROM 128B RAM =

SDA2083A514 Echivalent: SDA2083A014

SDA2083A515 Echivalent: SDA2083A015

SDA2083A523 CPU 8B 8kB ROM 128B RAM

SDA2083A534

SDA2083A536 Echivalent: SDA2080A034

SDA2084A006 Echivalent: SDA2084A012

SDA2084A007 CPU 8B WITH ROM

SDA2084A010 Echivalent: SDA2084A012

SDA2084A012 CPU 8-BIT WITH ROM

SDA2087N-SIE SYSTEM CONTROL PROCESSOR

SDA2112-2-SIE PHASE LOCKED LOOP

SDA2116-SIE EEPROM 128x8 BIT

SDA2120-SIE RADIO P.L.L. TUNING

SDA2121-SIE CMOS PHASE LOCKED LOOP

SDA2124-SIE

SDA2130 Echivalent: SDA2131-SIE

SDA2131-SIE LED DRIVER 16CH

SDA2208-2 Echivalent: SDA2208-3-SIE

SDA2208-3-SIE

SDA2216 Echivalent: SDA2516-5-SIE

SDA2218-SIE Echivalent FOR SDA2208

SDA2506-3 Echivalent: SDA2506-5-SIE

SDA2506-5-SIE

SDA2516 Echivalent: SDA2516-5-SIE

SDA2516-2 Echivalent: SDA2516-5-SIE

SDA2516-5-SIE ALSO: 24C01N-MBR

SDA2526-2 Echivalent: SDA2526-5-SIE

SDA2526-5-SIE ALSO: 24C02N-MBR

SDA2546-5-SIE IC

SDA2586-5-SIE Echivalent 24C08N-MBR

SDA3002-SIE

SDA30161-SIE

SDA3202-2-SIE Echivalent SDA3202-3-SI

SDA3202-2X-SIE FREQ.SYNTH.1.3GHz I2C-BUS

SDA3202-3-SIE ALSO: U6202B-TFK, TSA5511

SDA3202-SMD Echivalent: SDA3202-2X-SIE

SDA3205-SIE

SDA3206-MBR IR-FB TRANSMITTER

SDA3208-SIE REMOTE CNTRL TRANSMITTER

SDA3212-SIE

SDA3302 Echivalent: SDA3302-5-SIE

SDA3302-5-SIE 1.3GHz PLL TUNING I2C BUS

SDA3302-5X-SIE Echivalent: TSA5511AT-PHI

SDA3302-5X6-SI Echivalent: TSA5511T-PHI

SDA3402 Echivalent: SDA3302-5-SIE

SDA3412-SIE PHASE LOCKED LOOP

SDA3526-2 NO LONGER AVAILABLE

SDA3586-5-SIE I2C SERIAL EEPROM 1KX8

SDA4212-SIE PHASE LOCKED LOOP

SDA5231 Echivalent: SAA5231-MBR

SDA5241-SIE TXT PROCESSOR

SDA5243-2-SIE TXT PROCESSOR

SDA5273S-SIE

SDA5640-SIE DATA SLICER FOR VPS

SDA5642-SIE ONE CHIP VPS DECODER

SDA9045 Echivalent: TDA9045-PHI

SDA9064-SIE VERTICAL DEFECTION PROCES

SDA9087-2-SIE TV/VIDEO SIGNAL PROCESSOR

SDA9088-1-SIE TV/VIDEO SIGNAL PROCESSOR

SDA9088-2-SIE TV/VIDEO SIGNAL PROCESSOR

SDA9205-2-SIE ADC 3*8B 30MHz

SDH209B HYBRID ASSEMBLY

SE115N-SKN OUTPUT ERROR AMPL. 115.2V

SE135N-SKN TRANSISTOR

SE140N-SKN OUTPUT ERROR AMPL. 141.2V

SF0R5D43-TOS Echivalent: SF0R5J43-TOS

SF0R5G43-TOS Echivalent: SF0R5J43-TOS

SF0R5J43-TOS THYR. 600V 7A 1W

SF173 Echivalent: BF173

SF2D41-TOS THYR. 200V 2A 0.2mA

SF359 Echivalent: BF459/DIV

SF88 Echivalent: SF90

SF90 Echivalent: SF90-6/6 or SF90-8

SF91 Echivalent: SF91-6/5 or SF91-8

SFB3B Echivalent: 209.80261

SFC2104M Echivalent: LM104H-MBR

SFC2111M Echivalent: LM111H

SFC2301ADC Echivalent: LM301AH

SFC2311 Echivalent: LM311N-MBR

SFC2815EC Echivalent: UA7815-MBR

SFGB321-1 Echivalent: SFGB3211

SFH205 PHOTO DIODE

SFH309 FOTO TRANS. 35V <2.5mA A

SFH415T-SIE LED IR 16mW 34 Degr

SFH506-30 Echivalent: TFMS5300-TFK

SFH506-36 Echivalent: TFMS5360

SFH506/36 Echivalent: TFMS5360

SFH600-3 Echivalent: CNY17-4

SFH601G-3 Echivalent CNY17-3

SFH606-3 Echivalent: 8945902460

SFOR5D43-TOS Echivalent: SF0R5J43-TOS

SFPI-6/6 Echivalent: SFP1-6/6

SG1495N Echivalent: MC1495BP-MOT

SG201T Echivalent: LM301AH

SG2524 Echivalent: TL494CN-MBR

SG264A-SNY THYR. SPECIAL SONY

SG3524 Echivalent: SG3524N-MBR

SG3524D-MBR PWM CNTRL 4.6-5.4V 300KHz

SG3524N-MBR PWM CNTRL 4.6-5.4V 300KHz

SG3524NS25 Echivalent: 209.80585

SG3525AN-MBR PWM CNTRL 8-35V 500KHz 2%

SG3526N-MBR PWM CNTRL IC

SG3527AN-MBR PWM CNTRL 500KHz 0.1A 2%

SG3842 Echivalent: UC3842N-MBR

SG3843 Echivalent: UC3843N-MBR

SG3844 Echivalent: UC3844N-MBR

SG613-SNY THYR. SPECIAL SONY

SGSF313-ST SI-N 450V 7A 70W 0.3us

SGSF313XI-ST SI-N 1000V 5A 25W 0.3us

SGSF344-ST SI-N 600V 7A 85W

SGSF444-ST Echivalent: SGSF445-ST

SGSF445-ST SI-N 600V 7A 95W

SGSF464-ST SI-N 600V 10A 140W

SGSIF344-ST SI-N 600V 7A 35W

SGSIF444-ST SI-N 600V 7A 55W

SI1225H Echivalent: STK4133II-SAN

SI3120C-SKN V-REG 12V 1.5A 18W

SI3122V-SKN POS V-REG 12V 2A 50W

SI3132V-SKN V-REG

SI3240C-SKN V-REG 24V 1.5A 18W, 45Vin

SI7300A-SKN STEPPER MOTOR DRIVER

SI7330A-SKN

SI7660DJ Echivalent: ICL7660CPA-MBR

SI8100D-SKN I.C.

SI9503-SKN IC

SID30/15 Echivalent: BYD33M-PHI

SIM107 HYBRID IC

SIWBA60S Echivalent: S1WB(A)60

SK104 Echivalent: BYV10/40-MBR

SK3/16-SKR SI-D 1600V 1.8A/180Ap

SK3875-SKN PWR AMP

SK3GL04-SKR SI-D 400V 3.8A/175Ap 300n

SK3GL06-SKR Echivalent: BY500/1000-DIO

SK3GL08-SKR Echivalent: BY500/1000-DIO

SK3GL10-SKR Echivalent: BY500/1000-DIO

SK64 37.5 Echivalent: KL TO220S

SKE4F1/02 Echivalent: BYD33M-PHI

SKE4F1/10 Echivalent: BYD33M-PHI

SKE4F2/04 Echivalent: BY500/1000-DIO

SKE4F2/06 Echivalent: BY500/1000-DIO

SKE4F2/08 Echivalent: BY500/1000-DIO

SKE4F2/10 Echivalent: BY500/1000-DIO

SKE4G2/06 Echivalent: BYW95C-PHI

SKS1/04 Echivalent: SK3/16-SKR

SKS1/16 Echivalent: SK3/16-SKR

SKT24/04C-SKR THYR. 400V 50A 0.1A 2/80u

SL1430DP-PLS

SL1430DP/DIV Echivalent: SL1430DP-PLS

SL1431DP-PLS

SL1432DP-PLS

SL1451DP-PLS SATELITE PLL FM DET. 0.7G

SL1452DP-PLS SATELLITE LIN. FM DET. 1G

SL1454DP-PLS SATELLITE FM DET. 150MHz

SL1455DP-PLS

SL400 Echivalent: SBC3322

SL442DP-PLS I.C.

SL471DP-PLS DIGITALE DEMULTIPLEXER 16

SL486DP-PLS IR-RC PREAMPLIFIER

SL490BDP-PLS

SL490DP Echivalent: SL490BDP-PLS

SL5501 OPTOISO 3K5V 30V >15% 20/

SL560CDP-PLS SINGLE RF AMPLIFIER

SL6440CDP-PLS

SL6601CDP-PLS

SL8588 Echivalent: LM301AH

SL950 I.C.

SLA1012-SKN DIODE-ARRAY 120V 0.4A/2Ap

SLA4031-SKN Echivalent: SLA4061-SKN

SLA4061-SKN N-DARL 120V 5A 25W POWER

SLA4390-SKN DARLINGTON ARRAY

SLB0586A-SIE Echivalent: SLB0587-SIE

SLB0587-SIE SW LIGHT DIMMER HALOGEN

SLB586 Echivalent: SLB0587-SIE

SLB586A Echivalent: SLB0587-SIE

SM311 Echivalent: SDA2005-SIE

SM559B-SIE

SMD 24K Echivalent: 051.10243

SMD 4001 Echivalent: CD4001-SMD

SMD 4012 Echivalent: CD4012-SMD

SMD 4016 Echivalent: CD4066-SMD

SMD 40163 Echivalent: CD40163-SMD

SMD 4017 Echivalent: CD4017-SMD

SMD 4023 Echivalent: CD4023-SMD

SMD 40244 Echivalent: CD40244-SMD

SMD 40374 Echivalent: CD40374-SMD

SMD 4040 Echivalent: CD4040-SMD

SMD 4052 Echivalent: CD4052-SMD

SMD 4053 Echivalent: CD4053-SMD

SMD 4060 Echivalent: CD4060-SMD

SMD 4066 Echivalent: CD4066-SMD

SMD 4069 Echivalent: CD4069U-SMD

SMD 4071 Echivalent: CD4071-SMD

SMD 4093 Echivalent: CD4093-SMD

SMD 4541 Echivalent: CD4541-SMD

SMD 74HCT04 Echivalent: SN74HCT04-SMD

SMD 74LS04 Echivalent: 74LS04-SMD

SMD ASS RES Echivalent: 310.31938

SMICA TO3P Echivalent: SMICA SOT93

SMM105

SMP60N06-14 Echivalent: SUP70N06-14

SMQA1074 Echivalent: SMQA1038

SMR40000-SSU SMPS CNTRL

SMR40200C-SSU SMPS CONTROLER

SN16848-TI

SN16861-TI Echivalent 209.71966

SN16862-TI Senzor FOR 4 KEYS

SN16880N

SN16889P-TI LOGIC LEVEL IND =TL489

SN16966-TI

SN28654

SN29722 I.C.

SN29733 Senzor IC

SN29744 TV/VIDEO IC (PNP-TUNER)

SN29760N I.C.

SN29764-TI 7 SEGMENT DECODER

SN29767

SN29770BN-TI

SN29771BN-TI

SN29772BN-TI ANALOG SWITCH

SN29773BN-TI ANALOG SWITCH

SN29788-TI

SN29791N-TI OCTAL TOUCH AMPLIFIER

SN29799-TI

SN29848BN Echivalent: SN16848-TI

SN29861NG Echivalent: SN16861-TI

SN29862AN Echivalent: SN16862-TI

SN5473J Echivalent: SN7473-MBR

SN72709N Echivalent: UA741N14/DIV

SN72710N Echivalent: TL710CP-TI

SN72741 Echivalent: UA741N14/DIV

SN7400 Echivalent SN74LS00-MBR

SN7400 Echivalent: SN7400

SN7401-MBR Echivalent: SN74LS01-MBR

SN7402 Echivalent SN74LS02-MBR

SN7402 Echivalent: SN7402

SN7403 Echivalent: SN74LS03-MBR

SN7404 Echivalent SN74LS04/DIV

SN7404/DIV Echivalent: SN7404DIV

SN7405 Echivalent: SN74LS05-MBR

SN7406-MBR HEX INV BUFF DRIV OP. COL

SN7406-MBR Echivalent: SN7406MBR

SN7406/DIV HEX INV.BUFF.DRIV.OP.COLL

SN7406/DIV Echivalent: SN7406DIV

SN7407-MBR HEX BUFF DR. OC =M53207P

SN7407-MBR Echivalent: SN7407MBR

SN7407/DIV Echivalent SN7407-MBR

SN7407/DIV Echivalent: SN7407DIV

SN7408 Echivalent SN74LS08-MBR

SN7408 Echivalent: SN7408

SN7409 Echivalent SN74LS09-MBR

SN7409 Echivalent: SN7409

SN7410 Echivalent: SN74LS10-MBR

SN74100-MBR Echivalent: SN74100MBR

SN74104-MBR Echivalent: SN74104MBR

SN74104-MBR SINGLE J-K-TYPE FLIP-FLOP

SN74105-MBR Echivalent: SN74105MBR

SN74105-MBR SINGLE J-K-TYPE FLIP-FLOP

SN74107-MBR Echivalent SN74LS107-MB

SN74107-MBR Echivalent: SN74107MBR

SN74109 Echivalent: SN74LS109-MBR

SN7411 Echivalent: SN74LS11-MBR

SN74110-TI

SN74110-TI Echivalent: SN74110TI

SN74111-MBR DUAL J-K FLIP FLOP

SN74111-MBR Echivalent: SN74111MBR

SN74112 Echivalent: SN74LS112-MBR

SN74116-MBR

SN74116-MBR Echivalent: SN74116MBR

SN7412-MBR Echivalent: SN7412MBR

SN7412-MBR Echivalent: SN74LS12-MBR

SN74121-MBR MONOSTABLE MULTIVIBRATOR

SN74121-MBR Echivalent: SN74121MBR

SN74121/DIV MONOSTABLE MULTIVIBRATOR

SN74121/DIV Echivalent: SN74121DIV

SN74122 Echivalent: SN74LS122-MBR

SN74123 Echivalent SN74LS123-MB

SN74125-MBR Echivalent: SN74LS125-MBR

SN74126 Echivalent: SN74LS126-MBR

SN74128-MBR 50 Ohm LINE DRIVER

SN74128-MBR Echivalent: SN74128MBR

SN7413 Echivalent: SN74LS13-MBR

SN74132 Echivalent: SN74LS132-MBR

SN74133 Echivalent: SN74LS133-MBR

SN74134 Echivalent: SN74S134-MBR

SN74135 Echivalent: SN74S135-MBR

SN74136 Echivalent: SN74LS136-MBR

SN74137 Echivalent: SN74LS137-MBR

SN74138 Echivalent: SN74LS138-MBR

SN74139 Echivalent: SN74LS139-MBR

SN7414 Echivalent: SN74LS14-MBR

SN74141-MBR BCD DEC. DECOD/DRIVER

SN74141-MBR Echivalent: SN74141MBR

SN74142-MBR

SN74142-MBR Echivalent: SN74142MBR

SN74143-TI 4 BIT 7 SEGM.DRIV.15mA

SN74143-TI Echivalent: SN74143TI

SN74144-MBR LED DISPLAY DRIVER

SN74144-MBR Echivalent: SN74144MBR

SN74145 Echivalent: SN74LS145-MBR

SN74147 Echivalent: SN74LS147-MBR

SN74148 Echivalent: SN74LS148-MBR

SN7415 Echivalent: SN74LS15-MBR

SN74150-MBR 1OF16 DATA SELECT/MULTIPL

SN74150-MBR Echivalent: SN74150MBR

SN74151-MBR Echivalent: SN74LS151-MBR

SN74152 Echivalent: SN74LS152-MBR

SN74153 Echivalent: SN74LS153-MBR

SN74154-MBR Echivalent: SN74LS154-MBR

SN74155 Echivalent: SN74LS155-MBR

SN74156 DECODERS/MULTIPLEXERS

SN74156 Echivalent: SN74156

SN74157 Echivalent SN74LS157-MB

SN74158 Echivalent: SN74LS158-MBR

SN74159-MBR 4 BIT BIN. DECODER/DEMUX.

SN74159-MBR Echivalent: SN74159MBR

SN7416-MBR HEX INV/BUFF/DRIV. OPEN C

SN7416-MBR Echivalent: SN7416MBR

SN74160-MBR Echivalent: SN74LS160-MBR

SN74161-MBR Echivalent SN74LS161-MB

SN74161-MBR Echivalent: SN74161MBR

SN74162 Echivalent: SN74LS162-MBR

SN74163 Echivalent: SN74LS163-MBR

SN74164-MBR Echivalent: SN74LS164-MBR

SN74165 Echivalent: SN74LS165-MBR

SN74166-MBR Echivalent: SN74LS166-MBR

SN74168 Echivalent: SN74LS168-MBR

SN7417-MBR HEX BUFF./DRIVERS OPEN C.

SN7417-MBR Echivalent: SN7417MBR

SN7417/DIV HEX BUFF./DRIVERS OPEN C.

SN7417/DIV Echivalent: SN7417DIV

SN74170 Echivalent: SN74LS170-MBR

SN74172-MBR 8-BIT TO 4K-BIT SRAM

SN74172-MBR Echivalent: SN74172MBR

SN74173-MBR Echivalent: SN74173MBR

SN74173-MBR QUAD D-FL/FL

SN74174-MBR Echivalent SN74LS174-MB

SN74175-MBR Echivalent SN74LS175-MB

SN74175-MBR Echivalent: SN74175MBR

SN74176-MBR ASYNC. DECIMAL UP COUNTER

SN74176-MBR Echivalent: SN74176MBR

SN74177-MBR ASYNCH. BIN. 4B UP-COUNTE

SN74178-MBR Echivalent: SN74178MBR

SN74178-MBR SINGLE 4-BIT SHIFT REG.

SN74179-MBR 4 BIT SHIFT REG. PARALLEL

SN74179-MBR Echivalent: SN74179MBR

SN74180-MBR 9 BIT ODD/EVEN PARITY CHE

SN74180-MBR Echivalent: SN74180MBR

SN74180/DIV 9 BIT ODD/EVEN PARITY CHE

SN74180/DIV Echivalent: SN74180DIV

SN74181 Echivalent: SN74LS181-MBR

SN74182 Echivalent: SN74LS182-MBR

SN74185

SN74185 Echivalent: SN74185

SN7419 Echivalent: SN74LS19-MBR

SN74190 Echivalent: SN74LS190-MBR

SN74191 Echivalent: SN74LS191-MBR

SN74192 Echivalent: SN74LS192-MBR

SN74193 Echivalent: SN74LS193-MBR

SN74194 Echivalent: SN74LS194-MBR

SN74195-MBR Echivalent: SN74LS195-MBR

SN74196-MBR Echivalent: SN74LS196-MBR

SN74197-MBR Echivalent: SN74LS197-MBR

SN74198-MBR BI-DIR 8-BIT SHIFT REGIST

SN74198-MBR Echivalent: SN74198MBR

SN74199-MBR 8 BIT BIDIRECTIONAL UNI.

SN74199-MBR Echivalent: SN74199MBR

SN7420 Echivalent: SN74LS20-MBR

SN7421 Echivalent: SN74LS21

SN7422-MBR Echivalent: SN74LS22-MBR

SN74221 Echivalent: SN74LS221-MBR

SN7423 EXP.DUAL 4-INP.POS.NOR WS

SN7423 Echivalent: SN7423

SN74244 Echivalent: SN74LS244-MBR

SN74245 Echivalent: SN74LS245-MBR

SN74246-MBR

SN74246-MBR Echivalent: SN74246MBR

SN74247 Echivalent: SN74LS247

SN74248 Echivalent: SN74LS248-MBR

SN7425-MBR DUAL 4-INP.POS-NOR GATES

SN7425-MBR Echivalent: SN7425MBR

SN7425/DIV Echivalent: SN7425-MBR

SN74251 Echivalent: SN74LS251

SN74257 Echivalent: SN74LS257-MBR

SN74259 Echivalent: SN74LS259-MBR

SN7426 Echivalent: SN74LS26

SN74265-MBR QUAD 2-INP. AND GATE & IN

SN74266 Echivalent: SN74LS266-MBR

SN7427 Echivalent: SN74LS27-MBR

SN74273 Echivalent: SN74LS273-MBR

SN74279 Echivalent: SN74LS279-MBR

SN7428 Echivalent SN74LS28-MBR

SN74283 Echivalent: SN74LS283-MBR

SN74284-MBR 4 BY 8 BIT MULTIPLIER

SN74284-MBR Echivalent: SN74284MBR

SN74285-MBR

SN74285-MBR Echivalent: SN74285MBR

SN74293 Echivalent: SN74LS293-MBR

SN74298-MBR Echivalent: SN74298MBR

SN7430-MBR Echivalent SN74LS30-MBR

SN7432 Echivalent: SN74LS32-MBR

SN7433 Echivalent: SN74LS33-MBR

SN74365-MBR HEX BUFFER

SN74365-MBR Echivalent: SN74365MBR

SN74366 Echivalent: SN74LS366-MBR

SN74367-MBR HEX BUS DRIVER

SN74367-MBR Echivalent: SN74367MBR

SN74368-MBR HEX BUS DRIVER/INVERTER

SN74368-MBR Echivalent: SN74368MBR

SN7437 Echivalent: SN74LS37-MBR

SN74373 Echivalent: SN74LS373-MBR

SN74374 Echivalent: SN74LS374-MBR

SN7438 QUAD 2-INP. NAND BUFF. OC

SN7438 Echivalent: SN7438

SN7439-MBR QUAD 2-INPUT NAND

SN7439-MBR Echivalent: SN7439MBR

SN74390 Echivalent: SN74LS390-MBR

SN74393-MBR Echivalent: SN74LS393-MBR

SN7440 DUAL 4-INPUT NAND BUFFER

SN7440 Echivalent: SN7440

SN7441-MBR BCD-DECIM. DECODER+DRIVER

SN7441-MBR Echivalent: SN7441MBR

SN7442 Echivalent SN74LS42-MBR

SN7443 Echivalent SN74LS43

SN7443 Echivalent: SN7443

SN7444 EXCESS 3-GRAY TO DECIMAL

SN7444 Echivalent: SN7444

SN7445-MBR BCD DEC. DECOD./DRIVER

SN7445-MBR Echivalent: SN7445MBR

SN7446 BCD 7-SEGM.DECOD./DRIV.OC

SN7446 Echivalent: SN7446

SN7447-MBR BCD 7-SEGM. DECOD./DRIVER

SN7447-MBR Echivalent: SN7447MBR

SN7447/DIV Echivalent SN7447-MBR

SN7447/DIV Echivalent: SN7447DIV

SN7448 Echivalent: SN74LS48-MBR

SN7449 Echivalent: SN74LS49-MBR

SN74490 Echivalent: SN74LS490-MBR

SN7450-MBR DUAL 2 WIDE 2-INPUT AND-O

SN7450-MBR Echivalent: SN7450MBR

SN7450/DIV DUAL 2 WIDE 2-INP. AND-OR

SN7450/DIV Echivalent: SN7450DIV

SN7451 DUAL 2-WIDE 2-INP AND/OR/

SN7451 Echivalent: SN7451

SN7453 AND-OR-INVERT 2-2-2-2 INP

SN7453 Echivalent: SN7453

SN7454-TI 2 INPUT AND/NOR

SN7454-TI Echivalent: SN7454TI

SN7457 NO LONGER AVAILABLE

SN7460 DUAL 4-INPUT EXPANDERS

SN7460 Echivalent: SN7460

SN74689 Echivalent: SN74LS689-MBR

SN7470-MBR AND GATED EDGE TRIGGER FF

SN7470-MBR Echivalent: SN7470MBR

SN7472-MBR AND GATED JK MASTER SLAVE

SN7472-MBR Echivalent: SN7472MBR

SN7473-MBR DUAL JK FLIPFLOP

SN7473-MBR Echivalent: SN7473MBR

SN7473/DIV Echivalent SN7473-MBR

SN7473/DIV Echivalent: SN7473DIV

SN7474 Echivalent: SN74LS74

SN7475 Echivalent: SN74LS75-MBR

SN7476-MBR DUAL JK FLIPFLOP

SN7476-MBR Echivalent: SN7476MBR

SN7476/DIV DUAL JK FLIPFLOP

SN7476/DIV Echivalent: SN7476DIV

SN7480

SN7480 Echivalent: SN7480

SN7482-MBR 2 BIT FULL ADDER

SN7482-MBR Echivalent: SN7482MBR

SN7483 Echivalent SN74LS83-MBR

SN7483 Echivalent: SN7483

SN7484-MBR Echivalent: SN7484MBR

SN7484-MBR SRAM 16x1 BIT

SN7485 Echivalent SN74LS85-MBR

SN7486 Echivalent: SN74LS86-MBR

SN7489 16 BIT RAM

SN7489 Echivalent: SN7489

SN7490 Echivalent: SN74LS90-MBR

SN7491 Echivalent: SN74LS91-MBR

SN7492 Echivalent: SN74LS92-MBR

SN7493 Echivalent: SN74LS93-MBR

SN7494/DIV 4-BIT SHIFT REGISTER

SN7494/DIV Echivalent: SN7494DIV

SN7495 Echivalent: SN74LS95-MBR

SN7496 Echivalent: SN74LS96-MBR

SN7497-MBR BINARY MULTIPLIER

SN7497-MBR Echivalent: SN7497MBR

SN74ABT534-MBR OCTAL D-FF INV. TS

SN74AC4040 ASYNCH. UP COUNTER

SN74AC574-MBR OCTAL D-TYPE FLIP FLOP

SN74ACT04-MBR HEX INVERTERS

SN74ACT14-MBR HEX SCHMITT TRIGERS INVER

SN74ALS00-MBR QUAD 2-INPUT NAND GATE

SN74ALS373 OCTAL D-TYPE LATCHES

SN74ALS373 Echivalent: SN74ALS373

SN74ALS374-MBR OCTAL D-TYPE FLIP FLOPS

SN74ALS374-MBR Echivalent: SN74ALS374MBR

SN74ALS573-MBR OCTAL D-LATCH/BUS DRIVER

SN74ALS573-MBR Echivalent: SN74ALS573MBR

SN74ALS574-MBR OCTAL D-TYPE FLIPFLOP

SN74ALS574-MBR Echivalent: SN74ALS574MBR

SN74ALS576-MBR OCTAL D-TYPE FLIPFLOP

SN74ALS74-MBR DUAL D-TYPE FLIP-FLOPS

SN74ALS74-MBR Echivalent: SN74ALS74MBR

SN74C00-NSC CMOS QUAD 2-INP NAND GATE

SN74C00-NSC Echivalent: SN74C00NSC

SN74C02-NSC CMOS QUAD 2-INPUT NOR GAT

SN74C02-NSC Echivalent: SN74C02NSC

SN74C04 Echivalent: CD4069U-MBR

SN74C08-NSC CMOS QUAD 2-INP AND GATE

SN74C08-NSC Echivalent: SN74C08NSC

SN74C14-NSC Echivalent: CD40106-MBR

SN74C157-MBR CMOS 2-INPUT DIGITAL MULT

SN74C161 Echivalent: CD40161-MBR

SN74C174 Echivalent: CD40174-MBR

SN74C175 Echivalent: CD40175-MBR

SN74C192 Echivalent: CD40192-PHI

SN74C193 Echivalent: CD40193-MBR

SN74C20 Echivalent: CD4012-MBR

SN74C221-NSC CMOS DUAL MONOST.MULTIVIB

SN74C221-NSC Echivalent: SN74C221NSC

SN74C244-NSC DUAL 4B NON-INV. BUFFER/D

SN74C32-NSC QUAD OR GATE

SN74C32-NSC Echivalent: SN74C32NSC

SN74C373 Echivalent: CD40373-PHI

SN74C374-NSC Echivalent: CD40374-MBR

SN74C42-NSC CMOS DIGITAL DECODER/DRIV

SN74C42-NSC Echivalent: SN74C42NSC

SN74C48-NSC CMOS BCD-7 SEGM DECODER

SN74C48-NSC Echivalent: SN74C48NSC

SN74C73N-NSC DUAL J-K TYPE FLIP-FLOP

SN74C73N-NSC Echivalent: SN74C73NNSC

SN74C74-NSC DUAL D-TYPE FLIP-FLOP

SN74C74-NSC Echivalent: SN74C74NSC

SN74C76-NSC CMOS DUAL J-K FLIP-FLOP

SN74C76-NSC Echivalent: SN74C76NSC

SN74C85N-NSC DIG. MAGNITUDE COMPORATOR

SN74C85N-NSC Echivalent: SN74C85NNSC

SN74C86-MBR CMOS QUAD 2-INPUT EXOR

SN74C90-NSC CMOS ASYNC. UP COUNTER

SN74C90-NSC Echivalent: SN74C90NSC

SN74C902-NSC CMOS 6xNON-INVERTING BUFF

SN74C906-NSC CMOS HEX BUFFER OPEN DR.

SN74C906-NSC Echivalent: SN74C906NSC

SN74C911-NSC CMOS LCD CONTROLER

SN74C912-NSC 6-DIG. BCD LED DISP. DRIV

SN74C914-NSC CMOS HEX.SCHMITT TRIGGER

SN74C914-NSC Echivalent: SN74C914NSC

SN74C915-NSC CMOS 7-SEGMENT-to-BCD CON

SN74C922-NSC CMOS 16-KEY ENCODER

SN74C922-NSC Echivalent: SN74C922NSC

SN74C923N CMOS 20-KEY ENCODER

SN74C925-NSC CMOS 4-DIG. CNTR+7-SEG.DR

SN74C925-NSC Echivalent: SN74C925NSC

SN74C926-NSC CMOS 4-DIG. CNTR+7-SEG.DR

SN74C926-NSC Echivalent: SN74C926NSC

SN74C928-NSC CMOS DIG. COUNT/DECOD/DRI

SN74C928-NSC Echivalent: SN74C928NSC

SN74C93-NSC CMOS 4-BIT BINARY COUNTER

SN74F00-MBR QUAD 2-INPUT NAND GATE

SN74F00-MBR Echivalent: SN74F00MBR

SN74F02-MBR QUAD 2-INPUT NOR GATE

SN74F02-MBR Echivalent: SN74F02MBR

SN74F04-MBR BUF.&BUS/LOGIC DRIVER INV

SN74F04-MBR Echivalent: SN74F04MBR

SN74F08-MBR QUAD 2-INPUT AND GATE

SN74F08-MBR Echivalent: SN74F08MBR

SN74F10-MBR Echivalent: SN74F10MBR

SN74F10-MBR THREE 3-INPUT NAND GATE

SN74F11-MBR Echivalent: SN74F11MBR

SN74F11-MBR THREE 3-INPUT AND GATE

SN74F138-MBR DIG. DEMULTIPLEXER 8-LINE

SN74F138-MBR Echivalent: SN74F138MBR

SN74F14-MBR INVERTING SCHMITT TRIGGER

SN74F14-MBR Echivalent: SN74F14MBR

SN74F157-MBR DIG. MULTIPLEXER 2-LINES

SN74F157-MBR Echivalent: SN74F157MBR

SN74F161-MBR BIN. UP COUNTER 4-BITS

SN74F161-MBR Echivalent: SN74F161MBR

SN74F193-MBR BIN. 4BIT UP/DOWN COUNTER

SN74F193-MBR Echivalent: SN74F193MBR

SN74F240-MBR BUF.&BUS/LOGIC DRIVER INV

SN74F240-MBR Echivalent: SN74F240MBR

SN74F244-MBR OCTAL BUFFER/DRIVERS

SN74F244-MBR Echivalent: SN74F244MBR

SN74F245-MBR OCTAL BI-DIREC.BUS DRIVER

SN74F245-MBR Echivalent: SN74F245MBR

SN74F32-MBR QUAD 2-INPUT OR GATE

SN74F32-MBR Echivalent: SN74F32MBR

SN74F374-MBR OCTAL D FLIP-FLOP

SN74F374-MBR Echivalent: SN74F374MBR

SN74F38-MBR BUF.&BUS/LOG. DRIV. NAND

SN74F38-MBR Echivalent: SN74F38MBR

SN74F74-MBR DUAL D-TYPE FLIP-FLOPS

SN74F74-MBR Echivalent: SN74F74MBR

SN74HC00-MBR QUAD 2-INPUT NAND GATE

SN74HC00-MBR Echivalent: SN74HC00MBR

SN74HC01-MBR QUAD 2-INPUT NAND GATE

SN74HC02-MBR QUAD 2-INPUT NOR GATE

SN74HC02-MBR Echivalent: SN74HC02MBR

SN74HC03-MBR QUAD 2-INPUT NAND GATE

SN74HC03-MBR Echivalent: SN74HC03MBR

SN74HC04-MBR 6x INVERTING BUFFER

SN74HC04-MBR Echivalent: SN74HC04MBR

SN74HC08-MBR QUAD 2-INPUT AND GATE

SN74HC10-MBR Echivalent: SN74HC10MBR

SN74HC10-MBR TRIPLE 3-INPUT NAND GATE

SN74HC107 J-K TYPE FLIPFLOP

SN74HC109-MBR DUAL JK EDGE TRIGG. FL.FL

SN74HC109-MBR Echivalent: SN74HC109MBR

SN74HC11-MBR Echivalent: SN74HC11MBR

SN74HC11-MBR TRIPLE 3-INPUT AND GATE

SN74HC113-MBR DUAL J-K TYPE FLIP FLOP

SN74HC123-MBR DUAL MONOSTABLE MULTIVIBR

SN74HC125-MBR QUAD NON-INV. BUFFER/DRIV

SN74HC132-MBR QUAD 2-INP SCHMITT TRIGGE

SN74HC132-MBR Echivalent: SN74HC132MBR

SN74HC133-MBR NAND GATE 13 INPUT

SN74HC133-MBR Echivalent: SN74HC133MBR

SN74HC137-MBR 3 TO 8 LINE DEMULTIPLEXER

SN74HC138-MBR 3-to-8 LINE DEMULTIPLEXER

SN74HC138-MBR Echivalent: SN74HC138MBR

SN74HC139-MBR 2-to-4 LINE DEMULTIPLEXER

SN74HC14-MBR HEX SCHMITT TRIGGER

SN74HC14-MBR Echivalent: SN74HC14MBR

SN74HC148-MBR DIGITAL ENCODER

SN74HC148-MBR Echivalent: SN74HC148MBR

SN74HC151-MBR 8-INPUT DIG. MULTIPLEXER

SN74HC153-MBR DUAL 4-to-1 SELECTOR/MUX

SN74HC153-MBR Echivalent: SN74HC153MBR

SN74HC154S-MBR 4-to-16 LINE DEMUX SMALL

SN74HC154W-MBR 4-TO-16 LINE DEMUX WIDE

SN74HC155-MBR LINE DECODER/DEMULTIPLEX.

SN74HC155-MBR Echivalent: SN74HC155MBR

SN74HC157-MBR 2-INPUT DIGITAL MULTIPLEX

SN74HC163-MBR SYNCHRONOUS UP COUNTER

SN74HC164-MBR Echivalent: SN74HC164MBR

SN74HC164-MBR SINGLE 8-BIT SHIFT REGIST

SN74HC165-MBR 8 BIT SHIFT REG.PARAL-IN

SN74HC166-MBR 8-BIT PISO SHIFT REGISTER

SN74HC166-MBR Echivalent: SN74HC166MBR

SN74HC173-MBR QUAD D-TYPE FLIP-FLOP

SN74HC173-MBR Echivalent: SN74HC173MBR

SN74HC174-MBR HEX D-TYPE FLIP-FLOP

SN74HC174-MBR Echivalent: SN74HC174MBR

SN74HC175-MBR QUAD D-TYPE FLIPFLOP

SN74HC191-MBR PRESET SYNC.4-BIT COUNTER

SN74HC191-MBR Echivalent: SN74HC191MBR

SN74HC192-MBR SYNCH.PROG.DECADE COUNTER

SN74HC193-MBR SYNCHRONOUS UP/DOWN COUNT

SN74HC20-MBR 2X4-INPUT NAND GATE

SN74HC21-MBR DUAL 4-INPUT AND GATE

SN74HC21-MBR Echivalent: SN74HC21MBR

SN74HC221-MBR DUAL MONOSTABLE MULTIVIBR

SN74HC238-MBR 3-TO-8 LINE DEMULTIPLEXER

SN74HC240-MBR DUAL 4-BIT INVERTING BUFF

SN74HC241-MBR DUAL 4-BIT NON-INVERT. BU

SN74HC244-MBR BUFFER/DRIVER

SN74HC244-MBR Echivalent: SN74HC244MBR

SN74HC245-MBR OCTAL BUS TRANSCEIVER

SN74HC245-MBR Echivalent: SN74HC245MBR

SN74HC251-MBR 8-INPUT DIGITAL MULTIPLEX

SN74HC253-MBR DUAL 4-INP.DIG. DEMULTIPL

SN74HC257-MBR QUAD 2-INPUT MULTIPLEXER

SN74HC257-MBR Echivalent: SN74HC257MBR

SN74HC259-MBR 8-BIT LATCH

SN74HC273-MBR OCTAL D-TYPE FLIP-FLOP

SN74HC273-MBR Echivalent: SN74HC273MBR

SN74HC279-MBR QUAD S-R-TYPE FLIP-FLOP

SN74HC279-MBR Echivalent: SN74HC279MBR

SN74HC280-MBR 9-BIT PARITY GEN./CHECKER

SN74HC283-MBR 4-BIT FULL ADDER

SN74HC283-MBR Echivalent: SN74HC283MBR

SN74HC294-MBR PRESCALER/FREQUENCY DIVID

SN74HC299-MBR 8-BIT UNIVERSAL SHIFT REG

SN74HC299-MBR Echivalent: SN74HC299MBR

SN74HC30-MBR 8-INPUT NAND GATE

SN74HC32-MBR QUAD 2-INPUT OR GATE

SN74HC32-MBR Echivalent: SN74HC32MBR

SN74HC365-MBR NON-INV. BUFFER/DRIVER

SN74HC365-MBR Echivalent: SN74HC365MBR

SN74HC366-MBR HEX BUFFER/LINE DRIVER

SN74HC366-MBR Echivalent: SN74HC366MBR

SN74HC373 Echivalent: SN74HC373MBR

SN74HC373-MBR OCTAL LATCH

SN74HC374-MBR OCTAL 3-STATE FLIP-FLOP

SN74HC374-MBR Echivalent: SN74HC374MBR

SN74HC375-MBR 4-BIT D-TYPE LATCH

SN74HC377-MBR OCTAL D-TYPE FLIPFLOP

SN74HC390-MBR DUAL DECADE COUNTER

SN74HC393-MBR ASYNCHRONOUS UPCOUNTER

SN74HC4016-MBR QUAD BILATERAL SWITCHES

SN74HC4016-MBR Echivalent: SN74HC4016MBR

SN74HC4017-MBR JOHNSON DECADE COUNTER

SN74HC4020-MBR ASYNCHRONOUS UP COUNTER

SN74HC4020-MBR Echivalent: SN74HC4020MBR

SN74HC4024-MBR ASYNCHRONOUS UPCOUNTER

SN74HC4046-MBR PHASE LOCKED LOOP + VCO

SN74HC4049-MBR HEX INVERT. LEVEL SHIFTER

SN74HC4049-MBR Echivalent: SN74HC4049MBR

SN74HC4050-MBR 6xNON-INVERTING BUFFER

SN74HC4052-MBR 4-CHAN. ANALOG MULTIPLEXE

SN74HC4053-MBR 3X2-CHANNEL ANALOG MULTIP

SN74HC4059-MBR SYNCHR. DOWN COUNTER

SN74HC4060-MBR ASYNCHRONOUS UPCOUNTER

SN74HC4066-MBR QUAD ANALOG SWITCH

SN74HC4067-MBR 16-CHANNEL ANALOG MULTIPL

SN74HC4075 TRIPLE 3-INPUT OR-GATE

SN74HC4078 8-INPUT OR/NOR GATE

SN74HC4078 Echivalent: SN74HC4078

SN74HC4094-MBR Echivalent: SN74HC4094MBR

SN74HC4094-MBR SINGLE 8-BIT SHIFT REG.

SN74HC42-MBR BCD-to-DECIMAL DECODER

SN74HC4316-MBR ANALOG SWITCH

SN74HC4316-MBR Echivalent: SN74HC4316MBR

SN74HC4514S-MB 4-16 LINE DEC.+LTCH SMALL

SN74HC4514S-MB Echivalent: SN74HC4514SMB

SN74HC4514W-MB 4-16 LINE DEC+LTCH WIDE B

SN74HC4514W-MB Echivalent: SN74HC4514WMB

SN74HC4518-MBR SYNCHRONOUS UPCOUNTER

SN74HC4538-MBR DUAL MONOSTABLE MULTIVIBR

SN74HC4543-SMD BCD TO 7-SEGM.LTCH/DEC/DR

SN74HC4543-SMD Echivalent: SN74HC4543SMD

SN74HC533-MBR 8-BIT D-TYPE LATCH

SN74HC540-MBR OCTAL BUFFER/DRIVER

SN74HC540-MBR Echivalent: SN74HC540MBR

SN74HC563-MBR 8-BIT D-TYPE LATCH/INV.

SN74HC563-MBR Echivalent: SN74HC563MBR

SN74HC573-MBR 8-BIT D-TYPE LATCH

SN74HC573-MBR Echivalent: SN74HC573MBR

SN74HC574-MBR OCTAL D-TYPE FLIP FLOP

SN74HC589-MBR Echivalent: SN74HC589MBR

SN74HC589-MBR SINGLE 8BIT SHIFT REGISTE

SN74HC590-MBR 8 BIT BINARY COUNTER

SN74HC590-MBR Echivalent: SN74HC590MBR

SN74HC595-MBR 8BIT SHIFT REG+LATCH OUTP

SN74HC597-MBR SINGLE 8-BIT SHIFT REGIST

SN74HC646S-MBR 8-BIT BUS TRANSCEIVER

SN74HC646S-MBR Echivalent: SN74HC646SMBR

SN74HC646W-MBR 8-BIT BUS TRANCEIVER

SN74HC646W-MBR Echivalent: SN74HC646WMBR

SN74HC682-MBR MAGNITUDE COMPARATOR

SN74HC688-MBR MAGNITUDE COMPARATOR

SN74HC73-MBR DUAL J-K TYPE FLIP FLOP

SN74HC74-MBR DUAL D-TYPE FLIP-FLOP

SN74HC74-MBR Echivalent: SN74HC74MBR

SN74HC75-MBR 4 BIT BISTABLE LATCH

SN74HC75-MBR Echivalent: SN74HC75MBR

SN74HC76-MBR DUAL J-H-TYPE FLIP-FLOP

SN74HC76-MBR Echivalent: SN74HC76MBR

SN74HC85-MBR 4 BIT COMPARATOR

SN74HC85-MBR Echivalent: SN74HC85MBR

SN74HC86-MBR QUAD 2-INPUT XOR

SN74HC86-MBR Echivalent: SN74HC86MBR

SN74HC93-MBR SYNCHRONOUS UPCOUNTER

SN74HCT00-MBR QUAD INP NAND GATE

SN74HCT00-MBR Echivalent: SN74HCT00MBR

SN74HCT00/DIV Echivalent: SN74HCT00-MBR

SN74HCT02-MBR QUAD 2 INP NOR GATE

SN74HCT02-MBR Echivalent: SN74HCT02MBR

SN74HCT03-MBR QUAD 2-INPUT NAND GATE

SN74HCT03-MBR Echivalent: SN74HCT03MBR

SN74HCT04-MBR HEX INVERTER

SN74HCT04-MBR Echivalent: SN74HCT04MBR

SN74HCT04-SMD HEX INVERTER

SN74HCT04-SMD Echivalent: SN74HCT04SMD

SN74HCT05-MBR HEX INVERTERS

SN74HCT05-MBR Echivalent: SN74HCT05MBR

SN74HCT08-MBR QUAD 2 INP AND GATE

SN74HCT08-MBR Echivalent: SN74HCT08MBR

SN74HCT10-MBR Echivalent: SN74HCT10MBR

SN74HCT10-MBR TRIPLE 3- INPUT NAND GATE

SN74HCT10/DIV Echivalent: SN74HCT10-MBR

SN74HCT107-MBR DUAL JK MASTER-SLAVE F.F.

SN74HCT107-MBR Echivalent: SN74HCT107MBR

SN74HCT109-MBR DUAL JK EDGE TRIG. FL.FL.

SN74HCT11-MBR Echivalent: SN74HCT11MBR

SN74HCT11-MBR TRIPLE 3 INPUT AND GATE

SN74HCT112-MBR DUAL JK-FLIP FLOP

SN74HCT112-MBR Echivalent: SN74HCT112MBR

SN74HCT123-MBR Echivalent: SN74HCT123MBR

SN74HCT123-MBR RETRIG.MONOSTAB.MULTIVIB.

SN74HCT125-MBR LINE DRIVER 14p

SN74HCT125-MBR Echivalent: SN74HCT125MBR

SN74HCT126-MBR QUAD LINE DRIVERS

SN74HCT126-MBR Echivalent: SN74HCT126MBR

SN74HCT132-MBR QUAD 2 INP SCHMITT TRIGG

SN74HCT132-MBR Echivalent: SN74HCT132MBR

SN74HCT137-MBR 3 TO 8 LINE DEMULTIPLEXER

SN74HCT137-MBR Echivalent: SN74HCT137MBR

SN74HCT138-MBR 1 TO 8 DECOD/DEMULTIPLEX

SN74HCT138-MBR Echivalent: SN74HCT138MBR

SN74HCT138/DIV 1 TO 8 DECODER/DEMUX

SN74HCT138/DIV Echivalent: SN74HCT138DIV

SN74HCT139-MBR DUAL 1 OF 4 DECODER/DEMUL

SN74HCT139-MBR Echivalent: SN74HCT139MBR

SN74HCT139/DIV Echivalent SN74HCT139-M

SN74HCT139/DIV Echivalent: SN74HCT139DIV

SN74HCT14-MBR HEX SCHMITT TRIGG

SN74HCT14-MBR Echivalent: SN74HCT14MBR

SN74HCT147-MBR PRIORITY ENCODER

SN74HCT147-MBR Echivalent: SN74HCT147MBR

SN74HCT148-MBR Echivalent: SN74HC148-MBR

SN74HCT151-MBR 8 LINE TO 1 LINE MPX 16p

SN74HCT151-MBR Echivalent: SN74HCT151MBR

SN74HCT153-MBR 2x4 DATA SELECTOR MULT.

SN74HCT153-MBR Echivalent: SN74HCT153MBR

SN74HCT154-MBR 4 TO 16 LINE DEMUX WIDE

SN74HCT154-MBR Echivalent: SN74HCT154MBR

SN74HCT155-MBR Echivalent SN74HC155-MB

SN74HCT155-MBR Echivalent: SN74HCT155MBR

SN74HCT157-MBR QUAD 2 INP NON INV MULTIP

SN74HCT157-MBR Echivalent: SN74HCT157MBR

SN74HCT158 QUAD 2 LINE TO 1 LINE MPX

SN74HCT158 Echivalent: SN74HCT158

SN74HCT160 Echivalent: SN74HCT160

SN74HCT160 SYNCHR.PRG.DECADE COUNTER

SN74HCT161-MBR Echivalent: SN74HCT161MBR

SN74HCT161-MBR SYNCH.PROG.BINARY COUNTER

SN74HCT162-MBR Echivalent: SN74HCT162MBR

SN74HCT162-MBR SYN.DEC.COUNTER+CLEAR 16p

SN74HCT163-MBR Echivalent: SN74HCT163MBR

SN74HCT163-MBR SYNCHR.PRG.DECADE COUNTER

SN74HCT164-MBR 8BIT SHIFT REG.+PARAL.OUT

SN74HCT164-MBR Echivalent: SN74HCT164MBR

SN74HCT165-MBR 8 BIT SHIFT REG.PAR.INPUT

SN74HCT165-MBR Echivalent: SN74HCT165MBR

SN74HCT166-MBR 8-BIT SER. IN/PAR. OUT SH

SN74HCT166-MBR Echivalent: SN74HCT166MBR

SN74HCT173-MBR 4 D-TYPE FLIP FLOPS

SN74HCT173-MBR Echivalent: SN74HCT173MBR

SN74HCT174-MBR 6 D-FLIP FLOPS

SN74HCT174-MBR Echivalent: SN74HCT174MBR

SN74HCT175 QUAD FLIP-FLOP

SN74HCT175 Echivalent: SN74HCT175

SN74HCT190 Echivalent: SN74HCT190

SN74HCT190 SYNCHR.PRG.DECADE COUNTER

SN74HCT191-MBR Echivalent: SN74HCT191MBR

SN74HCT191-MBR SYNCHR. UP/DOWN COUNTERS

SN74HCT192 Echivalent: SN74HCT192

SN74HCT192 SYNCH.PROG.DECADE COUNTER

SN74HCT193-MBR Echivalent: SN74HCT193MBR

SN74HCT193-MBR SYNCH.PROG.BINARY COUNTER

SN74HCT194-MBR 4 BIT UNIVERSAL SHIFT REG

SN74HCT195-MBR 4 BIT UNIVERSAL SHIFT REG

SN74HCT194-MBR Echivalent: SN74HCT194MBR

SN74HCT195-MBR Echivalent: SN74HCT195MBR

SN74HCT20 DUAL 4 INP NAND GATE

SN74HCT20 Echivalent: SN74HCT20

SN74HCT21 DUAL QUAD AND GATES 14p

SN74HCT21 Echivalent: SN74HCT21

SN74HCT221-MBR DUAL MONOSTAB.MULTIV. 16p

SN74HCT221-MBR Echivalent: SN74HCT221MBR

SN74HCT237 3-TO-8 LINE DECODER 16p D

SN74HCT237 Echivalent: SN74HCT237

SN74HCT238 DIGITAL D-MULTIPLEX 8LINE

SN74HCT238 Echivalent: SN74HCT238

SN74HCT240-MBR OCTAL 3ST INV/DRIVER

SN74HCT240-MBR Echivalent: SN74HCT240MBR

SN74HCT240/DIV OCTAL 3ST INV/DRIVER

SN74HCT240/DIV Echivalent: SN74HCT240DIV

SN74HCT241-MBR 8 BIT BUS DRIVER

SN74HCT241-MBR Echivalent: SN74HCT241MBR

SN74HCT243 QUAD BUS TRANCEIVER

SN74HCT243 Echivalent: SN74HCT243

SN74HCT244-MBR OCTAL 3ST NON INV DRIVER

SN74HCT244-MBR Echivalent: SN74HCT244MBR

SN74HCT245-MBR OCTAL BUS TRANSCEIVER

SN74HCT245-MBR Echivalent: SN74HCT245MBR

SN74HCT251-MBR 8 TO 1 LINE MULTIPLEXER

SN74HCT251-MBR Echivalent: SN74HCT251MBR

SN74HCT257-MBR DIG. MULTIPLEXER 2-LINE

SN74HCT257-MBR Echivalent: SN74HCT257MBR

SN74HCT259-MBR 8 BIT LATCH

SN74HCT259-MBR Echivalent: SN74HCT259MBR

SN74HCT27-MBR 3 TRIPLE NOR GATE 14p

SN74HCT27-MBR Echivalent: SN74HCT27MBR

SN74HCT273-MBR OCTAL D-TYPE FLIP-FLOP

SN74HCT273-MBR Echivalent: SN74HCT273MBR

SN74HCT279 Echivalent: SN74HC279-MBR

SN74HCT299-MBR 8-BIT SHIFT REG, PAR. IN/

SN74HCT299-MBR Echivalent: SN74HCT299MBR

SN74HCT30-MBR 8-INPUT NAND GATE

SN74HCT30-MBR Echivalent: SN74HCT30MBR

SN74HCT32-MBR QUAD 2INP OR GATE

SN74HCT32-MBR Echivalent: SN74HCT32MBR

SN74HCT32/DIV Echivalent: SN74HCT32-MBR

SN74HCT356-MBR DIG. MULTIPLEXER 8-LINE

SN74HCT356-MBR Echivalent: SN74HCT356MBR

SN74HCT365-MBR HEX BUFF.WTH COMM.ENABLE

SN74HCT365-MBR Echivalent: SN74HCT365MBR

SN74HCT367-MBR 6 BUS LINE DRIVERS 16p

SN74HCT367-MBR Echivalent: SN74HCT367MBR

SN74HCT368-MBR 6xINVER.BUS LINEDRIVERS

SN74HCT368-MBR Echivalent: SN74HCT368MBR

SN74HCT373-MBR OCTAL LATCH

SN74HCT373-MBR Echivalent: SN74HCT373MBR

SN74HCT374-MBR OCTAL 3-STATE FLIP-FLOP

SN74HCT374-MBR Echivalent: SN74HCT374MBR

SN74HCT374/DIV OCTAL 3-STATE FLIP-FLOP

SN74HCT374/DIV Echivalent: SN74HCT374DIV

SN74HCT377 EIGHT D-FLIP FLOPS 20p

SN74HCT377 Echivalent: SN74HCT377

SN74HCT390-MBR DUAL DECADE COUNTER

SN74HCT390-MBR Echivalent: SN74HCT390MBR

SN74HCT393-MBR DUAL BINARY COUNTER

SN74HCT393-MBR Echivalent: SN74HCT393MBR

SN74HCT4002-MB DUAL 4 INPUT NOR GATE

SN74HCT4002-MB Echivalent: SN74HCT4002MB

SN74HCT40103 8-BIT SYNC. DOWN COUNTER

SN74HCT40103 Echivalent: SN74HCT40103

SN74HCT4017 Echivalent: SN74HCT4017

SN74HCT4017-MB DECADE COUNTER

SN74HCT4024-MB 7 STAGE RIPPLE COUNTER

SN74HCT4024-MB Echivalent: SN74HCT4024MB

SN74HCT4040-MB 12 BIT BINARY COUNTER

SN74HCT4040-MB Echivalent: SN74HCT4040MB

SN74HCT4046-MB PHASE LOCKED LOOP + VCO

SN74HCT4046-MB Echivalent: SN74HCT4046MB

SN74HCT4050-MB Echivalent: SN74HC4050-MBR

SN74HCT4051-MB 8-CHN ANALOG MUX/DEMUX

SN74HCT4051-MB Echivalent: SN74HCT4051MB

SN74HCT4053-MB Echivalent: SN74HCT4053MB

SN74HCT4053-MB TRIP.2-CHN ANAL.MUX/DEMUX

SN74HCT4059-MB Echivalent: SN74HCT4059MB

SN74HCT4059-MB SYNCHRONOUS DOWN COUNTER

SN74HCT4060-MB 14 STAGE COUNTER/DIV/OSC.

SN74HCT4060-MB Echivalent: SN74HCT4060MB

SN74HCT4066-MB QUAD ANALOG SWITCH

SN74HCT4066-MB Echivalent: SN74HCT4066MB

SN74HCT4067-MR 16-CHANNEL ANALOG MULTIPL

SN74HCT4075-MB TRIPLE 3-INPUT OR GATE

SN74HCT4094-MB 8 BIT UNIVERS.BUS REGIST.

SN74HCT4094-MB Echivalent: SN74HCT4094MB

SN74HCT42-MBR 1 TO 10 DECODER

SN74HCT42-MBR Echivalent: SN74HCT42MBR

SN74HCT4316-MB ANALOG SWITCH

SN74HCT4316-MB Echivalent: SN74HCT4316MB

SN74HCT4511-MB BCD-7SEG.LATCH/DEC/DRIVER

SN74HCT4511-MB Echivalent: SN74HCT4511MB

SN74HCT4514-MB DIDITAL DECODER/DEMUX

SN74HCT4514-MB Echivalent: SN74HCT4514MB

SN74HCT4518-MB DUAL BCD COUNTER

SN74HCT4518-MB Echivalent: SN74HCT4518MB

SN74HCT4520-MB DUAL BINARY COUNTER

SN74HCT4520-MB Echivalent: SN74HCT4520MB

SN74HCT4538-MB DUAL PREC.MONO.MULTIVIB.

SN74HCT4538-MB Echivalent: SN74HCT4538MB

SN74HCT4543-MB BCD TO 7-SEGM.LTCH/DECO/D

SN74HCT4543-MB Echivalent: SN74HCT4543MB

SN74HCT534-MBR OCTAL D-TYPE FLIPFLOP

SN74HCT540-MBR OCTAL BUFFERS/DRIVERS 20p

SN74HCT540-MBR Echivalent: SN74HCT540MBR

SN74HCT541-MBR OCT. BUS DRIVER

SN74HCT541-MBR Echivalent: SN74HCT541MBR

SN74HCT563-MBR 8-BIT D-TYPE LATCH

SN74HCT563-MBR Echivalent: SN74HCT563MBR

SN74HCT564-MBR OCTAL D-TYPE FLIP FLOP

SN74HCT564-MBR Echivalent: SN74HCT564MBR

SN74HCT573-MBR OCTAL D-TYPE LATCH

SN74HCT573-MBR Echivalent: SN74HCT573MBR

SN74HCT574-MBR 8-BIT D-TYPE FLIP-FLOP 20

SN74HCT574-MBR Echivalent: SN74HCT574MBR

SN74HCT590 Echivalent: SN74HC590-MBR

SN74HCT595-MBR 8BIT SHIFT REG+LATCH OUTP

SN74HCT595-MBR Echivalent: SN74HCT595MBR

SN74HCT640-MBR 8-BIT BUS TRANSCEIVER

SN74HCT640-MBR Echivalent: SN74HCT640MBR

SN74HCT646S-MB 8-BIT BUS TRANSCEIVER

SN74HCT646S-MB Echivalent: SN74HCT646SMB

SN74HCT646W-MB 8-BIT BUS TRANSCEIVER WID

SN74HCT646W-MB Echivalent: SN74HCT646WMB

SN74HCT652-MBR SINGLE 8-BIT BUS TRANSCEI

SN74HCT688-MBR 8 BIT MAGNITUDE COMPARATO

SN74HCT688-MBR Echivalent: SN74HCT688MBR

SN74HCT73-MBR DUAL JK FLIP FLOP

SN74HCT73-MBR Echivalent: SN74HCT73MBR

SN74HCT74 DUAL D FLIP/FLOP

SN74HCT74 Echivalent: SN74HCT74

SN74HCT75-MBR 4 BIT BISTABLE LATCH

SN74HCT75-MBR Echivalent: SN74HCT75MBR

SN74HCT85-MBR 4 BIT COMPARATOR

SN74HCT85-MBR Echivalent: SN74HCT85MBR

SN74HCT86 QUAD EXCLUSIVE OR GATE

SN74HCT86 Echivalent: SN74HCT86

SN74HCT93-MBR 4 BIT BINARY COUNTER

SN74HCT93-MBR Echivalent: SN74HCT93MBR

SN74HCU04-MBR 6 INVERTERS 14p

SN74HCU04-MBR Echivalent: SN74HCU04MBR

SN74LS00-MBR QUAD 2-INPUT NAND GATE

SN74LS00-MBR Echivalent: SN74LS00MBR

SN74LS00/DIV QUAD 2-INPUT NAND GATE

SN74LS00/DIV Echivalent: SN74LS00DIV

SN74LS01-MBR QUAD 2-INP.NAND GATE O.C.

SN74LS01-MBR Echivalent: SN74LS01MBR

SN74LS02-MBR QUAD 2-INPUT NOR GATE

SN74LS02-MBR Echivalent: SN74LS02MBR

SN74LS02/DIV QUAD 2-INP NOR GATE=DL002

SN74LS02/DIV Echivalent: SN74LS02DIV

SN74LS03-MBR QUAD 2-INP.NAND GATE O.C.

SN74LS03-MBR Echivalent: SN74LS03MBR

SN74LS03/DIV Echivalent: SN74LS03-MBR

SN74LS04-MBR HEX INVERTER

SN74LS04-MBR Echivalent: SN74LS04MBR

SN74LS04-SMD HEX INVERTER

SN74LS04-SMD Echivalent: SN74LS04SMD

SN74LS04/DIV HEX INVERTER =DL004D

SN74LS04/DIV Echivalent: SN74LS04DIV

SN74LS05-MBR HEX INVERTER OPEN COLL.

SN74LS05-MBR Echivalent: SN74LS05MBR

SN74LS06-MBR HEX BUFFER

SN74LS06-MBR Echivalent: SN74LS06MBR

SN74LS07-MBR HEX BUFFER

SN74LS07-MBR Echivalent: SN74LS07MBR

SN74LS08-MBR QUAD 2-INPUT AND GATE

SN74LS08-MBR Echivalent: SN74LS08MBR

SN74LS08/DIV QUAD 2-INPUT AND GATE

SN74LS08/DIV Echivalent: SN74LS08DIV

SN74LS09-MBR QUAD 2-INP.AND GATE O.C.

SN74LS09-MBR Echivalent: SN74LS09MBR

SN74LS10-MBR Echivalent: SN74LS10MBR

SN74LS10-MBR TRIPLE 3-INPUT NAND GATE

SN74LS107-MBR DUAL JK FLIP FLOP

SN74LS107-MBR Echivalent: SN74LS107MBR

SN74LS109-MBR DUAL JK EDGE TRIGG.FL.FL.

SN74LS109-MBR Echivalent: SN74LS109MBR

SN74LS11-MBR Echivalent: SN74LS11MBR

SN74LS11-MBR TRIPLE 3-INPUT AND GATE

SN74LS11/DIV Echivalent: SN74LS11DIV

SN74LS11/DIV TRIPLE 3-INPUT AND GATE

SN74LS112-MBR DUAL JK FL/FL NEG.EDGE 16

SN74LS112-MBR Echivalent: SN74LS112MBR

SN74LS113-MBR DUAL JK FL/FL NEG.EDGE

SN74LS113-MBR Echivalent: SN74LS113MBR

SN74LS114-MBR DUAL FLIP FLOP

SN74LS114-MBR Echivalent: SN74LS114MBR

SN74LS12-MBR Echivalent: SN74LS12MBR

SN74LS12-MBR TRIPLE 3-INP.NAND GATE OC

SN74LS121 Echivalent: SN74121-MBR

SN74LS122-MBR Echivalent: SN74LS122MBR

SN74LS122-MBR RETRIGG.MONOST.MULTIVIBR.

SN74LS123-MBR DUAL RETR.MONOST.MULTIVIB

SN74LS123-MBR Echivalent: SN74LS123MBR

SN74LS123/DIV DUAL RETR.MONOST.MULTIVIB

SN74LS123/DIV Echivalent: SN74LS123DIV

SN74LS124 Echivalent: SN74LS629-MBR

SN74LS125-MBR QUAD 3-ST.BUFF.LOW ENABLE

SN74LS125-MBR Echivalent: SN74LS125MBR

SN74LS126-MBR QUAD 3-ST.BUFF.HIGH ENABL

SN74LS126-MBR Echivalent: SN74LS126MBR

SN74LS128 Echivalent: SN74128-MBR

SN74LS13-MBR DUAL NAND SCHMITT TRIGGER

SN74LS13-MBR Echivalent: SN74LS13MBR

SN74LS132-MBR QUAD 2-INP.SCHMITT TRIGG.

SN74LS132-MBR Echivalent: SN74LS132MBR

SN74LS132/DIV QUAD 2-INPUT SCHMITT TRIG

SN74LS132/DIV Echivalent: SN74LS132DIV

SN74LS133-MBR 13-INPUT NAND GATE

SN74LS133-MBR Echivalent: SN74LS133MBR

SN74LS134 Echivalent: SN74S134-MBR

SN74LS135 Echivalent: SN74S135-MBR

SN74LS136-MBR QUAD EXCLUSIVE-OR OPEN C.

SN74LS136-MBR Echivalent: SN74LS136MBR

SN74LS137-MBR 3-TO-8 LINE DECOD/DEMULTI

SN74LS137-MBR Echivalent: SN74LS137MBR

SN74LS138-MBR 1-OF-8 DECOD/DEMULTIPLEX.

SN74LS138-MBR Echivalent: SN74LS138MBR

SN74LS139-MBR DUAL 1-0F-4 DECOD/DEMULT.

SN74LS139-MBR Echivalent: SN74LS139MBR

SN74LS14-MBR HEX SCHMITT TRIGGER

SN74LS14-MBR Echivalent: SN74LS14MBR

SN74LS14/DIV Echivalent SN74LS14-MBR

SN74LS14/DIV Echivalent: SN74LS14DIV

SN74LS140 Echivalent: SN74S140-MBR

SN74LS141 Echivalent: SN74141-MBR

SN74LS143-MBR Echivalent: SN74143-TI

SN74LS145-MBR BCD DEC.DECODER DRIVER

SN74LS145-MBR Echivalent: SN74LS145MBR

SN74LS147-MBR 10-T0-4 LINE ENCODER

SN74LS147-MBR Echivalent: SN74LS147MBR

SN74LS148-MBR 8 TO 3 LINE ENCODER

SN74LS148-MBR Echivalent: SN74LS148MBR

SN74LS15-MBR Echivalent: SN74LS15MBR

SN74LS15-MBR TRIPLE 3-INP.AND GATE O.C

SN74LS150 Echivalent: SN74150-MBR

SN74LS151-MBR 8-INPUT MULTIPLEXER

SN74LS151-MBR Echivalent: SN74LS151MBR

SN74LS152-MBR 1 TO 8 DATA SEL./MULTIPL.

SN74LS152-MBR Echivalent: SN74LS152MBR

SN74LS153-MBR DUAL 4-INPUT MULTIPLEXER

SN74LS153-MBR Echivalent: SN74LS153MBR

SN74LS154-MBR 4-TO-16 LINE DECOD/DEMULT

SN74LS154-MBR Echivalent: SN74LS154MBR

SN74LS155-MBR DUAL 1-OF-4 DECODER

SN74LS155-MBR Echivalent: SN74LS155MBR

SN74LS156-MBR DUAL 1-OF-4 DECODER O.C.

SN74LS156-MBR Echivalent: SN74LS156MBR

SN74LS157-MBR QUAD 2-INP.MULTIPL.NONINV

SN74LS157-MBR Echivalent: SN74LS157MBR

SN74LS157/DIV QUAD 2-INPUT MULTI.NONINV

SN74LS157/DIV Echivalent: SN74LS157DIV

SN74LS158-MBR QUAD 2-INP.MULTIPL.INVERT

SN74LS158-MBR Echivalent: SN74LS158MBR

SN74LS159 Echivalent: SN74159-MBR

SN74LS16 Echivalent: SN7416-MBR

SN74LS160-MBR BCD DECADE COUNTER AS.RES

SN74LS160-MBR Echivalent: SN74LS160MBR

SN74LS161-MBR 4-BIT BINARY COUNT.AS.RES

SN74LS161-MBR Echivalent: SN74LS161MBR

SN74LS162-MBR Echivalent: SN74LS162MBR

SN74LS162-MBR SYN.4 BIT DECADE COUNTER

SN74LS163-MBR 4-BIT BIN.COUNT.SYNCH.RES

SN74LS163-MBR Echivalent: SN74LS163MBR

SN74LS164-MBR 8-BIT SIPO SHIFT REGISTER

SN74LS164-MBR Echivalent: SN74LS164MBR

SN74LS165-MBR 8-BIT PISO SHIFT REGISTER

SN74LS165-MBR Echivalent: SN74LS165MBR

SN74LS166-MBR 8-BIT PISO SHIFT REGISTER

SN74LS166-MBR Echivalent: SN74LS166MBR

SN74LS168-MBR 4-BIT UP-DOWN SYN.DECAD.C

SN74LS168-MBR Echivalent: SN74LS168MBR

SN74LS169-MBR Echivalent: SN74LS169MBR

SN74LS169-MBR SYN 4 BIT UP/DOWN (BINARY

SN74LS170-MBR 4 BY 4 REGISTER FILES

SN74LS170-MBR Echivalent: SN74LS170MBR

SN74LS173-MBR QUAD D REGISTER 3-STATE.C

SN74LS173-MBR Echivalent: SN74LS173MBR

SN74LS174-MBR HEX D FLIP FLOP WTH CLEAR

SN74LS174-MBR Echivalent: SN74LS174MBR

SN74LS175-MBR QUAD D FL.FL.WITH CLEAR

SN74LS175-MBR Echivalent: SN74LS175MBR

SN74LS176 Echivalent: SN74176-MBR

SN74LS18-MBR

SN74LS18-MBR Echivalent: SN74LS18MBR

SN74LS180-MBR Echivalent: SN74180-MBR

SN74LS181-MBR ARITHEMETIC UNIT/FUNCTION

SN74LS181-MBR Echivalent: SN74LS181MBR

SN74LS182-MBR

SN74LS182-MBR Echivalent: SN74LS182MBR

SN74LS183-MBR DUAL CARRY SAVE FULL ADDE

SN74LS183-MBR Echivalent: SN74LS183MBR

SN74LS185 Echivalent: SN74185

SN74LS189-MBR Echivalent SN7489

SN74LS189-MBR Echivalent: SN74LS189MBR

SN74LS19-MBR Echivalent: SN74LS19MBR

SN74LS19-MBR SIX INVER.SCHMITT-TRIGGER

SN74LS190-MBR Echivalent: SN74LS190MBR

SN74LS190-MBR UP/DOWN DECADE COUNTER

SN74LS191-MBR Echivalent: SN74LS191MBR

SN74LS191-MBR UP/DOWN BINARY COUNTER

SN74LS192-MBR Echivalent: SN74LS192MBR

SN74LS192-MBR UP/DOWN DECADE COUNTER

SN74LS192/DIV Echivalent SN74LS192-MB

SN74LS192/DIV Echivalent: SN74LS192DIV

SN74LS193-MBR Echivalent: SN74LS193MBR

SN74LS193-MBR UP/DOWN BINARY COUNTER

SN74LS194-MBR 4-BIT RI/LEFT SHIFT REGIS

SN74LS194-MBR Echivalent: SN74LS194MBR

SN74LS195-MBR 4-BIT SHIFT REGISTER

SN74LS195-MBR Echivalent: SN74LS195MBR

SN74LS196-MBR DECADE COUNTER

SN74LS196-MBR Echivalent: SN74LS196MBR

SN74LS197-MBR PRESETABLE BINARY

SN74LS197-MBR Echivalent: SN74LS197MBR

SN74LS198 Echivalent: SN74198-MBR

SN74LS199 Echivalent: SN74199-MBR

SN74LS20-MBR DUAL 4-INPUT NAND GATE

SN74LS20-MBR Echivalent: SN74LS20MBR

SN74LS20/DIV DUAL 4-INPUT NAND GATE

SN74LS20/DIV Echivalent: SN74LS20DIV

SN74LS21 DUAL 4-INPUT AND GATE

SN74LS21 Echivalent: SN74LS21

SN74LS22-MBR DUAL 4-INP.NAND GATE O.C.

SN74LS22-MBR Echivalent: SN74LS22MBR

SN74LS221-MBR DUAL MONSTABLE MULIVIBR.

SN74LS221-MBR Echivalent: SN74LS221MBR

SN74LS23 Echivalent: SN7423

SN74LS24-MBR

SN74LS24-MBR Echivalent: SN74LS24MBR

SN74LS240-MBR OCTAL 3-ST.DRIVER INVERT.

SN74LS240-MBR Echivalent: SN74LS240MBR

SN74LS241-MBR OCTAL 3-ST.DRIVER NON.INV

SN74LS241-MBR Echivalent: SN74LS241MBR

SN74LS242-MBR QUAD BUS TRANSCEIVER

SN74LS242-MBR Echivalent: SN74LS242MBR

SN74LS243-MBR QUAD BUS TRANSCEIVER

SN74LS243-MBR Echivalent: SN74LS243MBR

SN74LS244-MBR OCTAL 3-ST.DRIVER NON INV

SN74LS244-MBR Echivalent: SN74LS244MBR

SN74LS244/DIV OCTAL 3-ST. DRIVER NON IN

SN74LS244/DIV Echivalent: SN74LS244DIV

SN74LS245-MBR OCTAL BUS TR.CEIVER NOINV

SN74LS245-MBR Echivalent: SN74LS245MBR

SN74LS246-MBR Echivalent: SN74246-MBR

SN74LS247 BCD TO 7SEGM.DECOD =D347D

SN74LS247/DIV Echivalent: SN74LS247DIV

SN74LS248-MBR BCD DECODER 16P

SN74LS248-MBR Echivalent: SN74LS248MBR

SN74LS249-MBR BCD-7 SEGMENT DEC.+DRIVER

SN74LS249-MBR Echivalent: SN74LS249MBR

SN74LS25 Echivalent: SN7425-MBR

SN74LS251 8-INP.MULTIPLEXER 3-STATE

SN74LS251 Echivalent: SN74LS251

SN74LS253-MBR DUAL 4-INP.MULTIPL.3-STA.

SN74LS253-MBR Echivalent: SN74LS253MBR

SN74LS256-MBR DUAL 4 BIT ADRESS.LATCH

SN74LS256-MBR Echivalent: SN74LS256MBR

SN74LS257-MBR QUAD 2-INP.MULTIPL.3-STA.

SN74LS257-MBR Echivalent: SN74LS257MBR

SN74LS258-MBR QUAD DATA SEL/MPX

SN74LS258-MBR Echivalent: SN74LS258MBR

SN74LS259-MBR 8 BIT ADRESSABLE LATCHES

SN74LS259-MBR Echivalent: SN74LS259MBR

SN74LS26 QUAD 2-INP.NAND GATE H.V.

SN74LS26/DIV Echivalent: SN74LS26DIV

SN74LS260-MBR DUAL 5-INPUT NOR GATE

SN74LS260-MBR Echivalent: SN74LS260MBR

SN74LS261-MBR 2x4-BIT MULTIPLIER

SN74LS266-MBR QUAD EXCLUS.NOR GATE OP.C

SN74LS266-MBR Echivalent: SN74LS266MBR

SN74LS27-MBR Echivalent: SN74LS27MBR

SN74LS27-MBR TRIPLE 3-INPUT NOR GATE

SN74LS27-SMD Echivalent: SN74LS27SMD

SN74LS273-MBR OCTAL D FL.FL.WITH CLEAR

SN74LS273-MBR Echivalent: SN74LS273MBR

SN74LS275-MBR 7-BIT WALLACE TREE ELEMEN

SN74LS275-MBR Echivalent: SN74LS275MBR

SN74LS279-MBR QUAD SET/RESET LATCH

SN74LS279-MBR Echivalent: SN74LS279MBR

SN74LS28-MBR QUAD 2-INPUT NOR BUFFER

SN74LS28-MBR Echivalent: SN74LS28MBR

SN74LS280-MBR 9 BIT ODD/EVEN PARITY

SN74LS280-MBR Echivalent: SN74LS280MBR

SN74LS283-MBR 4-BIT FULL ADDER

SN74LS283-MBR Echivalent: SN74LS283MBR

SN74LS290-MBR DECADE COUNTER

SN74LS290-MBR Echivalent: SN74LS290MBR

SN74LS292-MBR PROG. 32-BIT BIN. FREQ. D

SN74LS292-MBR Echivalent: SN74LS292MBR

SN74LS293-MBR 4-BIT BIN. COUNTER

SN74LS293-MBR Echivalent: SN74LS293MBR

SN74LS294-MBR

SN74LS294-MBR Echivalent: SN74LS294MBR

SN74LS295-MBR 4 BIDIRECTIONAL UNIVERSAL

SN74LS295-MBR Echivalent: SN74LS295MBR

SN74LS297-MBR PHASE LOCKED LOOP FILTER

SN74LS298-MBR QUAD 2-INP.MULIP.WTH.STOR

SN74LS298-MBR Echivalent: SN74LS298MBR

SN74LS299-MBR 8-BIT UNI PIPO SHIFT REG.

SN74LS299-MBR Echivalent: SN74LS299MBR

SN74LS30-MBR 8-INPUT NAND GATE

SN74LS30-MBR Echivalent: SN74LS30MBR

SN74LS30/DIV Echivalent: SN74LS30-MBR

SN74LS30/DIV Echivalent: SN74LS30DIV

SN74LS32-MBR QUAD 2-INPUT OR GATE

SN74LS32-MBR Echivalent: SN74LS32MBR

SN74LS32/DIV QUAD 2-INPUT OR GATE

SN74LS32/DIV Echivalent: SN74LS32DIV

SN74LS320-MBR

SN74LS320-MBR Echivalent: SN74LS320MBR

SN74LS321-MBR Echivalent: SN74LS321MBR

SN74LS321-MBR SINGLE CRYSTAL CONT. OSCI

SN74LS322-MBR 8-BIT SHIFT REG.+SIGN EXT

SN74LS322-MBR Echivalent: SN74LS322MBR

SN74LS323-MBR 8 BIT BI.SHIFT 20p

SN74LS323-MBR Echivalent: SN74LS323MBR

SN74LS324-MBR SINGLE VOLT. CONTR. OSCIL

SN74LS33-MBR QUAD 2-INP.NOR GATE O.C.

SN74LS33-MBR Echivalent: SN74LS33MBR

SN74LS347-MBR BCD-to-7-SEGMENT DEC./DRI

SN74LS348-MBR 8 TO 3 LINE DECODER

SN74LS348-MBR Echivalent: SN74LS348MBR

SN74LS352-MBR DUAL 4 TO 1 LINE

SN74LS352-MBR Echivalent: SN74LS352MBR

SN74LS353-MBR DUAL 4-TO1 DATA SEL.3-ST.

SN74LS353-MBR Echivalent: SN74LS353MBR

SN74LS363-MBR

SN74LS363-MBR Echivalent: SN74LS363MBR

SN74LS365-MBR HEX BUFF.WTH COMM.ENABLE

SN74LS365-MBR Echivalent: SN74LS365MBR

SN74LS366-MBR HEX INV. WTH COMM.ENABLE

SN74LS366-MBR Echivalent: SN74LS366MBR

SN74LS367-MBR HEX BUFF.4-BIT&2-BIT 3-ST

SN74LS367-MBR Echivalent: SN74LS367MBR

SN74LS368-MBR HEX INV.4-BIT&2-BIT 3-ST.

SN74LS368-MBR Echivalent: SN74LS368MBR

SN74LS37-MBR QUAD 2-INPUT NAND BUFFER

SN74LS37-MBR Echivalent: SN74LS37MBR

SN74LS373-MBR OCTAL TRANSPARANT LATCH

SN74LS373-MBR Echivalent: SN74LS373MBR

SN74LS374-MBR OCTAL D FLIP FLOP 3-STATE

SN74LS374-MBR Echivalent: SN74LS374MBR

SN74LS375-MBR OCTAL D-FLIP FLOP

SN74LS375-MBR Echivalent: SN74LS375MBR

SN74LS377-MBR OCTAL D FL.FL.WTH ENABLE

SN74LS377-MBR Echivalent: SN74LS377MBR

SN74LS378-MBR HEX D-TYPE FLIP FLOP

SN74LS378-MBR Echivalent: SN74LS378MBR

SN74LS379-MBR QUAD DUAL FLIP/FLOP

SN74LS379-MBR Echivalent: SN74LS379MBR

SN74LS38-MBR QUAD 2-INP.NAND BUFFER OC

SN74LS38-MBR Echivalent: SN74LS38MBR

SN74LS386-MBR EXCLUSIVE OR GATE (EXOR)

SN74LS386-MBR Echivalent: SN74LS386MBR

SN74LS39-MBR Echivalent: SN7439-MBR

SN74LS390-MBR DUAL 74LS90

SN74LS390-MBR Echivalent: SN74LS390MBR

SN74LS393-MBR 4-BIT BINAIRY UPCOUNTER

SN74LS393-MBR Echivalent: SN74LS393MBR

SN74LS395-MBR 4 BIT UNIVERS.SHIFT REG.

SN74LS395-MBR Echivalent: SN74LS395MBR

SN74LS396-MBR PIPELINE REGISTER

SN74LS396-MBR Echivalent: SN74LS396MBR

SN74LS399-MBR QUAD 2 INPUT MBX -STORAGE

SN74LS399-MBR Echivalent: SN74LS399MBR

SN74LS40-MBR DUAL 4-INPUT NAND BUFFER

SN74LS40-MBR Echivalent: SN74LS40MBR

SN74LS41-MBR Echivalent: SN7441-MBR

SN74LS42-MBR BCD TO 1 OF 10 DECODER

SN74LS42-MBR Echivalent: SN74LS42MBR

SN74LS423-MBR MONOSTABLE MULTIVIBRATOR

SN74LS43 EXCESS 3-to-DECIMAL DECOD

SN74LS43 Echivalent: SN74LS43

SN74LS445-MBR BCD TO DECIMAL DEC./DRIVE

SN74LS445-MBR Echivalent: SN74LS445MBR

SN74LS45 Echivalent: SN7445-MBR

SN74LS46 Echivalent: SN7446

SN74LS465 Echivalent: DM71LS95-NSC

SN74LS47-MBR BCD 7SEGM.DECOD/DRIVER OC

SN74LS47-MBR Echivalent: SN74LS47MBR

SN74LS471-MBR Echivalent: SN74LS471MBR

SN74LS471-MBR TTL-PROM 256x8 T.S. 60ns

SN74LS472 Echivalent: SN74S472-MBR

SN74LS48-MBR BCD 7SEGM.DECOD/DRIVER

SN74LS48-MBR Echivalent: SN74LS48MBR

SN74LS49-MBR

SN74LS49-MBR Echivalent: SN74LS49MBR

SN74LS490-MBR DUAL DECADE COUNTERS

SN74LS490-MBR Echivalent: SN74LS490MBR

SN74LS50 Echivalent: SN7450-MBR

SN74LS51-MBR DUAL AND-OR-INVERT GATE

SN74LS51-MBR Echivalent: SN74LS51MBR

SN74LS53 Echivalent: SN7453

SN74LS533-MBR 8-BIT D-LATCH WITH ENABLE

SN74LS54-MBR 4 WIDE AND-OR

SN74LS54-MBR Echivalent: SN74LS54MBR

SN74LS540 OCTAL BUFFERS/DRIVERS=DL5

SN74LS540 Echivalent: SN74LS540

SN74LS541 OCT. BUS DRIVER

SN74LS541 Echivalent: SN74LS541

SN74LS55-MBR 2-WIDE 4-INP.AND-OR-INV.G

SN74LS55-MBR Echivalent: SN74LS55MBR

SN74LS56-MBR FREQUENCY DIVIDER 50:1

SN74LS573-MBR OCTAL D-TYPE LATCH

SN74LS573-MBR Echivalent: SN74LS573MBR

SN74LS574-MBR OCTAL D-FLIP-FLOP

SN74LS574-MBR Echivalent: SN74LS574MBR

SN74LS590-MBR 8-BIT BINARY COUNTER

SN74LS590-MBR Echivalent: SN74LS590MBR

SN74LS592-MBR 8-BIT SYNCHR. UP COUNTER

SN74LS595-MBR SINGLE 8-BIT SHIFT REGIST

SN74LS597-MBR SINGLE 8-BIT SHIFT REGIST

SN74LS60 Echivalent: SN7460

SN74LS620-MBR OCTAL BUS INVERTER 20P

SN74LS620-MBR Echivalent: SN74LS620MBR

SN74LS621-MBR OCTAL BUS DRIVER NON INV

SN74LS621-MBR Echivalent: SN74LS621MBR

SN74LS623-MBR OCTAL BUS NON INV 20P

SN74LS623-MBR Echivalent: SN74LS623MBR

SN74LS624-MBR Echivalent: SN74LS624MBR

SN74LS624-MBR VCO 2-PHASE ENABLE/RANGE

SN74LS628-MBR VOLTAGE CONTROLLED OCILLA

SN74LS629-MBR IMPROVED 74LS124BLE

SN74LS629-MBR Echivalent: SN74LS629MBR

SN74LS63-MBR Echivalent: SN74LS63MBR

SN74LS63-MBR SIX CURRENT SenzorS

SN74LS640-MBR 8-BIT BI-DIREC.INV.BUS DR

SN74LS640-MBR Echivalent: SN74LS640MBR

SN74LS641-MBR OCTAL BUS TRANSCEIVER

SN74LS641-MBR Echivalent: SN74LS641MBR

SN74LS642-MBR OCTAL BUS TRANSCEIVERS

SN74LS642-MBR Echivalent: SN74LS642MBR

SN74LS645-MBR OCTAL BUS TRANSCEIVERS

SN74LS645-MBR Echivalent: SN74LS645MBR

SN74LS646-MBR 8 BIT BI-DIRE.BUS DRIVER

SN74LS646-MBR Echivalent: SN74LS646MBR

SN74LS669-MBR 4BIT SYNC.BINARY COUNTER

SN74LS669-MBR Echivalent: SN74LS669MBR

SN74LS670-MBR 4x4-BIT REGISTER FILE

SN74LS670-MBR Echivalent: SN74LS670MBR

SN74LS674-MBR SINGLE SHIFT REG. PAR-IN

SN74LS682-MBR 8-BIT MAGNITUDE COMPARATO

SN74LS682-MBR Echivalent: SN74LS682MBR

SN74LS688-MBR 8BIT MAGNITUDE COMPORATOR

SN74LS688-MBR Echivalent: SN74LS688MBR

SN74LS689-MBR 8-BIT MAGNITUDE COMP.

SN74LS70 Echivalent: SN7470-MBR

SN74LS72 Echivalent: SN7472-MBR

SN74LS73-MBR DUAL JK FLIP FLOP

SN74LS73-MBR Echivalent: SN74LS73MBR

SN74LS74 DUAL D FLIP FLOP=1582606

SN74LS74 Echivalent: SN74LS74

SN74LS75-MBR 4-BIT BI-STABLE LATCH

SN74LS75-MBR Echivalent: SN74LS75MBR

SN74LS76-MBR DUAL JK FLIP FLOP

SN74LS76-MBR Echivalent: SN74LS76MBR

SN74LS77-MBR QUAD. D-TYPE LATCHES

SN74LS77-MBR Echivalent: SN74LS77MBR

SN74LS78-MBR DUAL JK FLIP FLOP

SN74LS78-MBR Echivalent: SN74LS78MBR

SN74LS795 Echivalent: DM71LS95-NSC

SN74LS80 Echivalent: SN7480

SN74LS810 Echivalent: SN74ALS810-MBR

SN74LS82 Echivalent: SN7482-MBR

SN74LS83-MBR 4-BIT FULL ADDER

SN74LS83-MBR Echivalent: SN74LS83MBR

SN74LS83/DIV 4-BIT FULL ADDER

SN74LS83/DIV Echivalent: SN74LS83DIV

SN74LS85-MBR 4-BIT MAGNITUDE COMPARAT.

SN74LS85-MBR Echivalent: SN74LS85MBR

SN74LS85/DIV 4-BIT MAGNITUDE COMPARAT.

SN74LS85/DIV Echivalent: SN74LS85DIV

SN74LS86-MBR QUAD EXCLUSIVE OR GATE

SN74LS86-MBR Echivalent: SN74LS86MBR

SN74LS86/DIV QUAD EXCLUSIVE OR GATE

SN74LS86/DIV Echivalent: SN74LS86DIV

SN74LS89 Echivalent: SN7489

SN74LS90-MBR DECADE COUNTER

SN74LS90-MBR Echivalent: SN74LS90MBR

SN74LS91-MBR 8 BIT SERIAL SHIFT REGIST

SN74LS91-MBR Echivalent: SN74LS91MBR

SN74LS92-MBR DIVIDE BY 12 COUNTER

SN74LS92-MBR Echivalent: SN74LS92MBR

SN74LS93-MBR 4-BIT BINARY COUNTER

SN74LS93-MBR Echivalent: SN74LS93MBR

SN74LS94 Echivalent: SN7494/DIV

SN74LS95-MBR 4-BIT SHIFT REGISTER

SN74LS95-MBR Echivalent: SN74LS95MBR

SN74LS96-MBR 5 BIT SHIFT REGISTER 16p

SN74LS96-MBR Echivalent: SN74LS96MBR

SN74LS97-MBR Echivalent: SN7497-MBR

SN74S00 Echivalent: SN74F00-MBR

SN74S04 Echivalent: SN74F04-MBR

SN74S112-MBR DUAL JK FLIP-FLOPS

SN74S112-MBR Echivalent: SN74S112MBR

SN74S134-MBR 12-INPUT NAND GATE

SN74S135-MBR QUAD 2-INPUT XOR/XNOR GAT

SN74S140-MBR DUAL 4-INP NAND GATE

SN74S175-MBR QUAD D-FLIP FLOPS 16p DIP

SN74S175-MBR Echivalent: SN74S175MBR

SN74S196-MBR Echivalent: SN74S196MBR

SN74S287 Echivalent: 82S129-PHI

SN74S387-MBR PROM 256x4 65ns

SN74S387-MBR Echivalent: SN74S387MBR

SN74S472-MBR Echivalent: 82S147-PHI

SN74S74 Echivalent: SN74F74-MBR

SN75107-MBR DUAL DIF. LINE RECEIVER

SN75107/DIV DUAL DIF LINE RECEIVER

SN75110 DUAL DIF. LINE DRIVER

SN75113N-MBR LINE DRIVER

SN75115-MBR DUAL LINE RECEIVER O.C.

SN75116-MBR DRIV./REC. CCITT V11 & X2

SN75150 Echivalent: SN75150N8-MBR

SN75150N14

SN75150N8-MBR 2xRS232 DRIVER

SN75150N8/DIV 2xRS232 DRIVER

SN75154N-MBR 4xRS232 RECEIVER

SN75154N/DIV 4xRS232 RECEIVER

SN75172N-MBR Echivalent: AM26LS31-MBR

SN75173 Echivalent: AM26LS32-MBR

SN75174N-TI Echivalent: MC3487N-MBR

SN75175 Echivalent: MC3486N-MBR

SN75176N-MBR 1xRS422/85 TRANSCEIVER LP

SN75179-MBR DRIV./REC. V11, X27 & RS-

SN75182N-MBR BUS/LINE RECEIVER

SN75183-MBR LINE DRIVER

SN75188 Echivalent: MC1488N-MBR

SN75189 Echivalent: MC1489N-MBR

SN75365N-MBR TTL-TO-MOS TRANSLATOR

SN75450-MBR PERIPHERAL DRIVER

SN75451-MBR PERIPHERAL DRIVER

SN75452-MBR DUAL POWER DRIVER

SN75453BP-MBR 300mA PERIPHERAL DRIVER

SN75462N-MBR DUAL POWER DRIVER

SN75468 Echivalent: ULN2003AN-MBR

SN75469-MBR 7*N-DARL DR. 100V 0.35A

SN75477N-TI DUAL POWER DRIVER OC 0.3A

SN75491-MBR MOS-TO-LED 4xSEGMENT DR.

SN75492 MOS-TO-LED 6xDIGIT DRIVER

SN75518N-TI ALSO: 209.83171

SN75733N-TI Echivalent: UA733CN-MBR

SN75C188-MBR Echivalent: MC14C88N-MBR

SN75C189-MBR Echivalent: MC14C89N-MBR

SN76001ANQ Echivalent: TAA611B12

SN76110N STEREO DECODER

SN76115Q Echivalent: MC1310P-MOT

SN76116N Echivalent: UA758N-MBR

SN76131 Echivalent: UA739PC/DIV

SN76227 Echivalent: MC1327AP-MBR

SN76360NS Echivalent TDA2560

SN76396

SN76514N-TI RF MODULATOR/DEMODULATOR

SN76544N-TI

SN76600P Echivalent: MC1350P-MOT

SN76622 Echivalent: TBA120U-PHI

SN76623 Echivalent: TBA120T-MBR

SN76660 Echivalent: TBA120

SN76666 Echivalent: CA3065E-MBR

SN76670 TV SOUND ZF

SN76709

SN76720N-TI

SN76727 I.C.

SN76730 Echivalent: TBA730

SO41E-SIE

SO41P-SIE

SO42P-SIE

SO42P/DIV FM-MIXER 200MHz

SO437 I.C.

SO498

SONG1 Echivalent 209.10005

SONG2 Echivalent 209.10007

SP231ACP 2xRS232 TRANSCEIVER

SP232ACP 2xRS232 TRANSCEIVER

SP233ACP Echivalent: MAX233CPP

SP4632-PLS

SP4653DP-PLS 1 GHz %256 PRESCALER

SP485CS Echivalent: LTC485CN8-MBR

SP5000ADP-PLS RF FREQUENCY SYNTHESIZER

SPH4690 Echivalent: TDA4605-3 +IRFBC30

SPH4692 Echivalent: TDA4605-3 +IRFBC30

SPM109B I.C. =SPM109-1

SPU2220-ITT SECAM CHROMA PROCESSOR UN

SPU2221-ITT

SPU2243DIP-ITT

SPU2243PLC-ITT SECAM PROCESSOR UNIT

SPU632 Echivalent: U665B-TFK

SR1040 Echivalent: PBYR1040-MBR

SR150 Echivalent: SB160

SR160 Echivalent: SB160

SR1640 Echivalent: PBYR1645-PHI

SR2M Echivalent: R2M-SKN

SR3040 Echivalent: PBYR3040WT-MBR

SR350 Echivalent: SB350

SR50 Echivalent: BSR50-PHI

SR5040 SB-D 40V 50A/400Ap

SR550 Echivalent: SB550

SR840 SB-D 40V 8A/90Ap

SRAM1K*4 Echivalent: 2114-200NS

SRAM2K*8 Echivalent: 6116-100-MBR

SRAM32K*8 Echivalent: 62256-70-MBR

SRAM512K*8 Echivalent: KM684000ALP-7L

SRAM8K*8 Echivalent: 6264-70-MBR

SS8050 SI-N 40V 1.5A 1W 100MHz

SS8550 SI-P 40V 1.5A 1W 100MHz

SS9012 Echivalent: KTC9012-MBR

SSH6N80 Echivalent: H6N80-MBR

SSM2017P SELF CONTAINED AUDIO PREA

SSM2024P TRANSCONDUCTANCE AMPLIFIE

SSM2210P 2xSI-N 40V 20mA 0.5W 200M

SSM2220P-AD 2xSI-P 36V 20mA 0.5W 190M

SSP6N60A Echivalent: P6N60-MBR

ST13005 Echivalent: MJE13005-MBR

ST13007 Echivalent: MJE13007-MBR

ST24C02AB1 Echivalent: 24C02N-MBR

ST24C02CP Echivalent: 24C02N-MBR

ST24C04B1 Echivalent: 24C04N-MBR

ST93C06 Echivalent: 93C06-MBR

ST93C46 Echivalent: 93C46P-MBR

STA301A-SKN N-ARRAY 3x60V 4A B>1K

STA341M-SKN P/N-ARRAY 30V 1A B>100

STA401A-SKN N-ARRAY 4x60V 4A B>1K

STA402A-SKN P-ARRAY 4x50V 4A B>1K

STA403A-SKN N-ARRAY 4x100V 4A B>1K

STA434A-SKN P/N-ARRAY 2*60V 4A 20W B>

STA441C-SKN N-ARRAY 4x160V 1.5A B>40

STA451C-SKN P/N-ARRAY 2x60V 3A B>40

STA471A-SKN N-ARRAY 4x60V 2A B>2K

STA8012-SKN TRANSISTOR ARRAY

STA901M-SKN TRANSISTOR ARRAY

STH15NA50-ST Echivalent: STW15NA50-ST

STH15NA50F-ST Echivalent: STW15NA50-ST

STK0029-SAN

STK0030-SAN Echivalent: STK0040-SAN

STK0039-SAN POWER AMP 1x35W 45V 20kHz

STK0040-SAN

STK0050-SAN

STK0060 Echivalent: STK0080-SAN

STK0070 Echivalent: STK0080-SAN

STK0080-SAN PWR AMP 1*80W 65V 0.1%

STK0100-SAN

STK011-SAN

STK015-SAN

STK020-SAN

STK078-SAN

STK078G

STK080-SAN 1x30W 27V POWER AMP 20kHz

STK082-SAN

STK082G Echivalent: STK084G-SAN

STK083G Echivalent: STK084G-SAN

STK084-SAN

STK084G-SAN 1x50W 35V POWER AMP 20kHz

STK085 Echivalent: STK086-SAN

STK086-SAN 1x70W 42V POWER AMP 20kHz

STK086G-SAN

STK1035 Echivalent: STK1049-SAN

STK1049-SAN AUDIO AMPL. 45W

STK1050-SAN 1x50W 35V POWER AMP 20kHz

STK1050II-SAN PWR AMP 1X50W 39V 20KHZ

STK1060-SAN

STK1060II

STK1070-SAN

STK1070II-SAN NO LONGER AVAILABLE

STK11006 Echivalent: STR11006-SKN

STK2030-SAN HYBRID POWER AMPLIFIER

STK2038II-SAN

STK2040-SAN

STK2058IV-SAN PWR AMP 2x50W/4E 59V

STK2101-SAN FM CAR RADIO NOISE SUPPRE

STK2125 Echivalent: STK2129-SAN

STK2129-SAN Echivalent 209.81183

STK2135 Echivalent: STK2155-SAN

STK2139 Echivalent: STK2155-SAN

STK2145 Echivalent: STK2155-SAN

STK2230-SAN 2x30W 30V POWER AMP 20kHz

STK2240-SAN

STK2250-SAN 2x50W 37V POWER AMP 20kHz

STK3041-SAN 2-CHN. AUDIO PRE-AMP 41V

STK3042-SAN

STK3042II-SAN Echivalent: STK3042III-SAN

STK3042III-SAN 2x50W 43V POWER AMP 20kHz

STK3062-SAN Echivalent: STK3062III-SAN+kla

STK3062II-SAN Echivalent: STK3062III-SAN

STK3062III-SAN

STK3062IV Echivalent: 209.73065

STK3082-SAN Echivalent: STK3082III-SAN

STK3082II-SAN Echivalent: STK3082III-SAN

STK3082III-SAN PRE-AMP. 65V .005% 20kHz

STK3102II-SAN PWR AMP 2*59W 75V 20kHz

STK3102III-SAN 2x100W 60V POWER AMPLIFIE

STK3102IV Echivalent: 209.73453

STK3122III-SAN 2x120W 66V POWER AMP 20kH

STK3152 Echivalent: STK3152III-SAN

STK3152III-SAN 2x150W 72V POWER AMP 20kH

STK3156-SAN

STK3161-SAN

STK392-040-SAN SMPS CONTROLER 100kHz

STK401-080-SAN PWR AMP 2*45W/6E 45V 0.4%

STK401-120-SAN PWR AMP 2x80W/6E 61V

STK4017-SAN 1x6.5W 26V POWER AMP 100k

STK4019-SAN 1x10W 32V POWER AMP 100kH

STK4024II-SAN 1x20W 23V POWER AMP 50kHz

STK4025-SAN 1x25W 48V POWER AMP 100kH

STK4026-SAN PWR AMP 1x25W 38V 50KHz

STK4026II-SAN PWR AMP 1x25W 38V 50KHz 0

STK4026V-SAN PWR AMP 1x25W 38V 50KHz .

STK4028-SAN

STK4028II-SAN POWER AMPLIFIER

STK4028V Echivalent: STK4028X-SAN

STK4028X-SAN PWR AMP 1*30W/8E 29V .008

STK4030II-SAN PWR AMP 1*35W/8E 45V 0.3%

STK4036II-SAN 1x50W 35V POWER AMP 50kHz

STK4036V-SAN 1X50W 35V POWER AMP 50KHZ

STK4036X-SAN 1x50W 36V POWER AMP 50kHz

STK4036XI-SAN 1x50W 50V POWER AMP.50kHz

STK4038II-SAN 1x60W 38V POWER AMP 50kHz

STK4040II-SAN 1x70W 42V POWER AMP 50kHz

STK4040V-SAN PWR AMP 1x70W/8E 42V 50KH

STK4040X-SAN 1x70W 42V POWER AMP 50kHz

STK4040XI-SAN 1x70W 43V POWER AMP 50kHz

STK4042II-SAN 1x80W 45V POWER AMP 50kHz

STK4042XI-SAN PWR AMP 1X80W 45V 50KHZ

STK4044II-SAN 1x100W 51V POWER AMP 50kH

STK4044V-SAN 1x100W 51V POWER AMP 50kH

STK4044X Echivalent: STK4044XI-SAN

STK4044XI-SAN 1x100W 51V POWER AMP 50kH

STK4046V-SAN 1x120W 55V POWER AMP 50kH

STK4046XI-SAN 1x120W 55V POWER AMP 50kH

STK4048 Echivalent: STK4048II-SAN

STK4048II-SAN 1x150W 59V POWER AMP 50kH

STK4048V-SAN 1x150W 60V POWER AMP 50kH

STK4048XI-SAN 1x150W 60V POWER AMP 50kH

STK405-050A POWER AMPLIFIER

STK4050 Echivalent: STK4050II-SAN

STK4050II-SAN 1x200W 66V POWER AMP 50kH

STK4050V-SAN 1x200W 66V POWER AMP 50kH

STK4060-SAN

STK4065-SAN 2x25W 13V BTL AMP 30kHz

STK4101II-SAN PWR AMP 2X6W(13V/8E) 20.5

STK4111II-SAN 2x10W 17V POWER AMP 50kHz

STK4112II-SAN 2x10W 17V POWER AMP 50kHz

STK4121II-SAN 2x15W 20V POWER AMP 50kHz

STK4122II-SAN 2x15W 20V POWER AMP 50kHz

STK413 Echivalent: STK4332-SAN

STK4131II-SAN 2x20W 23V POWER AMP 50kHz

STK4131V-SAN PWR AMP 2x20W/4E 23V 50kH

STK4132 Echivalent: STK4132II-SAN

STK4132II-SAN 2x20W 23V POWER AMP 50kHz

STK4133II-SAN PWR AMP 2x20W/4E 33V 1%

STK4140II-SAN PWR AMP 2*25W/4E 40V 1%

STK4141 Echivalent: STK4141II-SAN

STK4141II-SAN 2x25W 26V POWER AMPL 50kH

STK4141V-SAN 2x25W 27V POWER AMP 50kHz

STK4142II-SAN 2x25W 26V POWER AMP 50kHz

STK4147 Echivalent: STK4147II-SAN

STK4147II-SAN 3x25W 26V POWER AMP 50kHz

STK415 Echivalent: STK435A-SAN

STK4150II-SAN PWR AMP 2*35W/4E 42V 1%

STK4151II-SAN 2x30W 27V POWER AMP 50kHz

STK4151V-SAN 2x30W 28V POWERAMP 50kHz

STK4151X-SAN PWR AMP 2x30W/8E 29V 0.02

STK4152II-SAN 2x30W 27V POWER AMP 50kHz

STK4161II-SAN 2x35W 30V POWER AMP 50kHz

STK4161V-SAN 2x35W 30V POWER AMP 50kHz

STK4162II-SAN 2x35W 30V POWER AMP 50kHz

STK4164II-SAN PWR AMP 2x30.5W/8E 50KHz

STK4171II-SAN 2x40W 32V POWER AMPL 50kH

STK4171V-SAN 2x40W 32V POWER AMP 50kHz

STK4172II-SAN 2x40W 32V POWER AMP 50kHz

STK4181 Echivalent: STK4181II-SAN

STK4181II-SAN 2x45W 33V POWER AMP 50kHz

STK4181V-SAN 2x45W 34W POWER AMP 50kHz

STK4182II-SAN 2x45W 33V POWER AMP 50kHz

STK419-140-SAN PWR AMP

STK419-150-SAN PWR AMP

STK4191 Echivalent: STK4191II-SAN

STK4191II-SAN 2x50W 35V POWER AMP 50kHz

STK4191V-SAN 2x50W 35V POWER AMP 50kHz

STK4191X-SAN 2x50W 35V POWER AMP 50KHz

STK4192 Echivalent: STK4192II-SAN

STK4192II-SAN 2x50W 35V POWER AMP 50kHz

STK4197II PWR AMPLIFIER

STK4201II-SAN Echivalent: STK4211II-SAN

STK4201V-SAN Echivalent: STK4211V-SAN

STK4204V-SAN 2xPWR AMP 2x60W/8E 39V

STK4211II-SAN 2x70W 42V POWER AMP 50kHz

STK4211V-SAN 2x70W 43V POWER AMP 50kHz

STK4221II-SAN 2x80W 45V POWER AMP 50kHz

STK4231II-SAN 2x100W 51V POWER AMP 50kH

STK4231V-SAN 2x100W 51V POWER AMP 50kH

STK4241 Echivalent: STK4241V-SAN

STK4241II-SAN PWR AMP 2x120W/8E 53V 50K

STK4241V-SAN 2x120W 54V POWER AMP 50kH

STK4272-SAN HYBRID POWER AMPLIFIER

STK4273-SAN

STK430-SAN Echivalent: STK436A-SAN

STK4301-SAN PWR AMP 2*22W 31V 50kHz

STK430II-SAN Echivalent: STK430III-SAN

STK430III-SAN 2x5W POWER AMPL 20kHz

STK4311-SAN 2x28W 35V POWER AMP 50kHz

STK433 Echivalent: STK433A-SAN

STK4332-SAN 2x5W 23V POWER AMP 50kHz

STK433A-SAN 2x5W 23V POWER AMP 10kHz

STK435 Echivalent: STK435A-SAN

STK4352-SAN 2x7W 27V POWER AMP 50 kHz

STK435A-SAN 2x7W 27V POWER AMP 20kHz

STK4362-SAN 2x10W 33V POWER AMP 100kH

STK436A-SAN 2x10W 32V POWER AMP 20kHz

STK437 Echivalent: STK4372-SAN

STK4372-SAN 2x12W 35V POWER AMP 100kH

STK439-SAN 2x15W 39V POWER AMP 20kHz

STK4392-SAN 2x15W 39V POWER AMP 100kH

STK441-SAN 2x20W 44V POWER AMP 20kHz

STK4412-SAN 2x20W 44V POWER AMP 100kH

STK443-SAN 2x25W 49V POWER AMP 20kHz

STK4432-SAN 2x25W 49V POWER AMP 100kH

STK457-SAN Echivalent STK459-SAN

STK459-SAN Echivalent STK463-SAN

STK460-SAN Echivalent: STK461-SAN

STK461-SAN 2x20W 23V POWER AMP 20kHz

STK463-SAN 2x25W 26V POWER AMP 20kHz

STK465A-SAN 2x30W 28V POWER AMP 20kHz

STK4773 Echivalent: STK4793-SAN

STK4793-SAN 2x15W 22V POWER AMP 100kH

STK4803-SAN 2x20W 24V POWER AMP 100kH

STK4813-SAN 2x20W 24V POWER AMP 100kH

STK4833-SAN 2x25W 25V POWER AMP 100kH

STK4843-SAN

STK4853-SAN

STK4873-SAN 2x35W 30V POWER AMP 100kH

STK50322-SAN I.C.

STK5314-SAN VCR REGULATOR 12.05+12.0V

STK5315-SAN VCR REGULATOR

STK5322H-SAN

STK5323-SAN VCR REGULATOR

STK5324-SAN VCR REGULATOR 12.0+12.0V

STK5325-SAN VCR REGULATOR 12.0+14.0V

STK5326-SAN VCR REGULATOR 12.0+13.0V

STK5330-SAN VCR REGULATOR

STK5331-SAN VCR REGULATOR

STK5332-SAN 3 OUT VCR REG. 13/6/5.1V

STK5333S-SAN VOLTAGE REGULATOR

STK5335-SAN 4 OUT VCR REGULATOR

STK5336-SAN VCR REGULATOR

STK5337-SAN 3 OUT VCR REG. 12.3/6.1/5

STK5338-SAN 3 OUT VCR REGULATOR

STK5339-SAN 3 OUT VCR REG. 12.3/5.8/5

STK5340-SAN VCR REGULATOR

STK5342-SAN 3 OUT VCR REG. 12.3/6/5.2

STK5343-SAN VCR REGULATOR

STK5352

STK5353-SAN VOLTAGE REGULATOR

STK5361-SAN VOLTAGE REGULATOR

STK5362-SAN VCR REGULATOR 5.1+9.0V

STK5372-SAN Echivalent: STK5372H-SAN

STK5372H-SAN 3 OUT VCR REG. 12.1/12/5.

STK5373-SAN VCR REGULATOR

STK5383-SAN I.C.

STK5391-SAN VCR REGULATOR

STK5392-SAN VCR REGULATOR

STK54041 Echivalent: STR54041-SKN

STK5421-SAN 4 OUT VCR REGULATOR

STK5422-SAN 4 OUT VCR REGULATOR

STK5431-SAN SWITCHING REGULATOR

STK5434-SAN VCR REGULATOR

STK5436-SAN VCR REGULATOR

STK5441-SAN 3 OUT VCR REG. 12/9/5.5V

STK5443-SAN VOLT. REG 12V 9V AND 5.8V

STK5446-SAN VCR REGULATOR

STK5451-SAN 3 OUT VCR REG.+OUTP. SEL.

STK5461-SAN 3 OUT VCR REG. 12/12/5.1V

STK5462-SAN 3 OUT VCR REG. 16/9/5.1V

STK5464-SAN 3 OUT VCR REG. 12/12/5.3V

STK5466-SAN 3 OUT VCR REG. 12/12/5.3V

STK5467-SAN 3 OUT VCR REG. 12.1/12.1/

STK5468-SAN 3 OUT VCR REG. 12.8/12.1/

STK5471-SAN 3 OUT VCR REGULATOR

STK5472-SAN VCR REGULATOR

STK5473-SAN 3 OUT VCR REG. 12.9/12.2/

STK5474-SAN VCR REGULATOR

STK5476-SAN 3 OUT VCR REG. 12/12/5.3V

STK5477-SAN 3 OUT VCR REG. 12/12/5.1V

STK5478-SAN 3 OUT VCR REG. 12/9/5.5V

STK5479-SAN 3 OUT VCR REG. 12.3/12.2/

STK5481-SAN 4 OUT VCR REGULATOR

STK5482-SAN 4 OUT VCR REG. 15/5.8/12/

STK5486-SAN 5 OUT VCR REGULATOR

STK5487-SAN 4-OUTPUT REG. FOR VTR

STK5490-SAN 4 OUT VCR REG.

STK561F-SAN 2W AUDIO AMP+ 115V 1A REG

STK563F-SAN 1.8W AUDIO AMP+ 135V 1A R

STK5720-SAN VOLTAGE REGULATOR 9.5V/2A

STK5730-SAN VCR REGULATOR

STK58041 Echivalent: STR58041-SKN

STK583F-SAN 4W AUDIO AMP+ 135V 1A REG

STK6327-SAN GRAPHIC EQUALIZER

STK6328A-SAN 2xGRAPHICS EQUALIZER 12dB

STK6431-SAN

STK6607-SAN MOTOR DRIVER

STK6712AIV-SAN 4-PH MOT.DR+PR HIGH 52V 2

STK6712BIV-SAN 4-PH MOT.DR+PR LOW 52V 2.

STK6722-SAN 50V 2.5A UNIP.MOTOR DRIVE

STK6732-SAN

STK6875-SAN DC BI-DIR MOTOR DR 50V 12

STK6922-SAN

STK6932-SAN 4 CHN 18V 1A AC MOTOR DR.

STK6962-SAN

STK6972H-SAN 2 CHN 30V 2A DC MOTOR DR.

STK6981-SAN STEPPER MOTOR DR. 30V 1.5

STK6982 Echivalent: STK6982H-SAN

STK6982B-SAN 30V 2.5A UNIP. MOTOR DRIV

STK6982H-SAN 52V 2.5A UNIP. MOTOR DRIV

STK6982HII-SAN 52V 2.5A UNIP.MOTOR DRIVE

STK713-680-SAN SW-REG 330W MOSFET

STK7216-SAN 2 OUT CHOP-REG. 9/1A+12/3

STK7217-SAN 2 OUT CHOP-REG. 9/1.5A+12

STK7225-SAN

STK7226-SAN

STK7233

STK7251-SAN CHOP-REGULATOR

STK7253-SAN VOLTAGE REGULATOR

STK730-060-SAN SW-REG CTV 110W MOSFET

STK7308-SAN SW-REG CTV 115V 80W

STK7309-SAN SW-REG CTV 115V 80W

STK7310-SAN SW-REG CTV 115V 100W

STK732C-SAN CHOPPER REG. 15V 1A

STK73405II-SAN OFF LINE SWITCH REGULATOR

STK73410-SAN SW-REG CTV 115V 100W

STK73410II-SAN SW-REG CTV 115V 100W

STK7348-SAN SW-REG CTV 115V 80W

STK7356-SAN SW-REG CTV 115V 60W

STK7358-SAN SW-REG CTV 115V 80W

STK7359-SAN SW-REG CTV 115V

STK73605I-SAN SW-REG VCR 15V 40W

STK73907-SAN SW-REG CTV 900V 180W

STK73908-SAN SW-REG CTV 900V 210W

STK7404-SAN SWITCHING REGULATOR 40W

STK7406-SAN SWITCHING REGULATOR 60W

STK7408-SAN SWITCHING REGULATOR 80W

STK7410-SAN OFFL. SWITCHING REG. 100W

STK7458-SAN I.C.

STK746-SAN I.C.

STK7554-SAN

STK7561A-SAN 2 OUT CHOP-REG.5V/2A+12V/

STK7563F-SAN 2 OUT CHOP-REG.5V/3A+24V/

STK7573-SAN 2xCHOP. +5V/1A +24V/2A

STK7576-SAN VOLT.REG.

STK770-SAN 1 OUT CHOP-REG. 5-24V/2A

STK7702-SAN HYBRID I.C.

STK7703-SAN

STK7707-SAN VOLTAGE REGULATOR

STK772-SAN 1 OUT CHOP-REG. 5-24V/3.2

STK780-SAN 1 OUT CHOP-REG. 5-24V/4A

STK78603-SAN HI-RES COLOR PROC+DRIVER

STK795-SAN 1 OUT CHOP-REG. 5V/3A

STK79801-SAN DISPLAY PROC.&VERT DEFL.

STK79905C-SAN HYBRID I.C.

STK8040 Echivalent: STK8050-SAN

STK8050-SAN

STK8250-SAN

STP2N80 Echivalent: STP4NA80-ST

STP2N80F Echivalent: STP4NA80F-ST

STP3NA60-MBR N-FET 600V 2.9A 80W <4E

STP3NA60F-ST N-FET 600V 2.1A 40W <4E

STP4N40FI Echivalent: STP4NA60F-ST

STP4NA40FI Echivalent: STP4NA60F-ST

STP4NA60-MBR N-FET 600V 4.3A 100W <2E2

STP4NA60F-ST N-FET 600V 2.7A 40W <2E2

STP4NA80-ST N-FET 800V 4A 110W <3E

STP4NA80F-ST N-FET 800V 2.5A 45W <3E

STP6NA60 Echivalent: P6N60-MBR

STR10006-SKN SMPS CNTRL IC

STR1096-SKN

STR11006-SKN POS V-REG 7.15V 0.8A 27W

STR1195-SKN I.C.

STR12006-SKN IC

STR1229-SKN I.C.

STR16006-SKN V-REG 7.15V 0.8A 27W EXT.

STR17006-SKN VOLTAGE REGULATOR

STR1706 Echivalent: STRD1706-SKN

STR1806 Echivalent: STRD1806-SKN

STR1816 Echivalent: STRD1816-SKN

STR20005-SKN SW-REG 5.1V 2A 55W 30KHz

STR20012-SKN SW-REG 12V 2A 55W 30KHz

STR20013-SKN SW-REG 13V 2A 55W 30KHz

STR2005-SKN SW-REG 5.1V 2A 75W 25KHz

STR2012-SKN SINGLE DC-to-DC VOLT. CON

STR2013-SKN SW-REG 13V 2A 75W 25KHz

STR2015-SKN SW-REG 15V 2A 75W 25KHz

STR2024-SKN SW-REG 24V 2A 75W 25KHz

STR2105-SKN SWITCHING REGULATOR

STR2124-SKN SWITCHING REGUL. 24V 3A

STR30110-SKN POS V-REG 110V 3.5A 27W

STR30115-SKN VOLTAGE REGULATOR

STR30123-SKN POS V-REG 123V 3.5A 27W

STR30125-SKN POS V-REG 125V 3.5A 27W

STR30130-SKN POS V-REG 130V 3.5A 27W

STR3105-SKN HYBRID I.C.

STR3123-SKN POS V-REG 123V 3.5A 40W

STR3125-SKN POS V-REG 125V 3.5A 40W

STR3130-SKN POS V-REG 130V 3.5A 40W

STR370-SKN POS V-REG 110V 3.5A 25W

STR371-SKN POS V-REG 115V 3.5A 25W

STR380-SKN POS V-REG 123V 3.5A 25W

STR381-SKN POS V-REG 130V 3.5A 25W

STR40090-SKN RH-IX0465CEZZ

STR40115-SKN I.C.

STR4090A-SKN HYBRID I.C.

STR41090-SKN POS V-REG 115V 6A 27W

STR4211-SKN POS V-REG 111V 4A 40W

STR43111-SKN I.C.

STR440-SKN POS V-REG 92V 4A 25W

STR441-SKN POS V-REG 95V 4A 25W

STR44115-SKN HYBRID I.C.

STR442-SKN POS V-REG 102V 4A 25W

STR450A-SKN POS V-REG 115V 5A 25W

STR451 Echivalent: STR451-SKN

STR451-KIT V-REG 103V 5A 25W MOD-KIT

STR451-SKN POS V-REG 103V 5A 25W

STR45111-SKN I.C.

STR453A-SKN POS V-REG 130V 5A 25W

STR454-SKN

STR455-SKN POS V-REG 110V 5A 25W

STR456A-SKN POS V-REG 112V 5A 25W

STR470A-SKN POS V-REG 103V 4A 25W

STR50020-SKN I.C.

STR50092-SKN POS V-REG 92V 6A 27W

STR50103-SKN Echivalent: STR50103A-SKN

STR50103A-SKN POS V-REG 103.4V 6A 27W

STR50112A-SKN POS V-REG

STR50113-SKN VOLT-REG

STR50115A-SKN POS V-REG 115.4V 6A 27W=B

STR50213-SKN POS V-REG 113.3V 0.8A 27W

STR50330-SKN I.C.

STR51213-SKN I.C.

STR5214-SKN I.C.

STR53041-SKN VOLTAGE REGULATOR

STR54041-SKN POS V-REG 114.5V 6A 27W

STR54041M-SKN Echivalent: STR54041-SKN

STR5412-SKN POS V-REG 112V 5A 40W

STR55041-SKN VOLTAGE REGULATOR

STR56041-SKN VOLT-REG

STR56707 Echivalent: STRS6707-SKN

STR56708-SKN Echivalent: STRS6708-SKN

STR5707-SKN Echivalent: STRS5707-SKN

STR5741-SKN Echivalent: STRS5741-SKN

STR58041-SKN I.C.

STR59041-SKN I.C.

STR5941 Echivalent: STR59041-SKN

STR60001-SKN IC

STR6008X-SKN Echivalent: STRD6008X-SKN

STR6020-SKN POS V-REG 111V 4A 40W

STR61001-SKN I.C.

STR6307-SKN Echivalent: STRS6307-SKN

STR6309 Echivalent: STRS6309-SKN

STR6545 Echivalent: STRS6545-SKN

STR6549-SKN Echivalent: STRM6549-SKN

STR6707-SKN Echivalent: STRS6707-SKN

STR6708-SKN Echivalent: STRS6708-SKN

STR7001-SKN SW-REG 5.1V 6A 100W

STR80145-SKN VOLTAGE DOUBLER 500V 5A

STR81145 VOLTAGE DOUBLER 500V 10A

STR81159-SKN V-REG

STR8124-SKN I.C.

STR9005-SKN POS V-REG 5V 4A 75W

STR9012-SKN POS V-REG 12V 4A 75W

STR90120-SKN VOLT-REG

STRD1005T-SKN VOLTAGE REGULATOR

STRD1706-SKN I.C.

STRD1806-SKN IC PANASONIC

STRD1806E Echivalent: STRD1806EA

STRD1806EA

STRD1816-SKN IC PANASONIC

STRD1906-SKN I.C.

STRD4412-SKN I.C.

STRD4420-SKN SWITCH REGULATOR

STRD4512-SKN I.C.

STRD5095A-SKN I.C.

STRD5441-SKN I.C.

STRD5541-SKN SWITCH REGULATOR

STRD6008X-SKN I.C.

STRD6009E I.C.

STRD6108-SKN VOLT-REG

STRD6202-SKN I.C.

STRD6601-SKN I.C.

STRD6602-SKN I.C.

STRD6801-SKN I.C.

STRD6802-SKN I.C

STRM6523-SKN I.C.

STRM6545-SKN IC.

STRM6546-SKN VOLTAGE REGULATOR

STRM6549-SKN VOLTAGE REGULATOR

STRM6559-SKN VOLT-REG

STRS5706-SKN I.C.

STRS5707-SKN VOLTAGE REGULATOR

STRS5717-SKN VOLTAGE REGULATOR

STRS5741-SKN I.C.

STRS5841-SKN I.C.

STRS5941-SKN IC.

STRS6301-SKN I.C.

STRS6307-SKN IC.

STRS6309-SKN VOLTAGE REGULATOR

STRS6525-SKN VOLT-REG

STRS6533-SKN I.C.

STRS6545-SKN I.C.

STRS6707-SKN SW-REG 85-265V 140W

STRS6708-SKN SW-REG 85-265V 180W

STRS6709-SKN SW-REG

STRZ2152-SKN VOLT-REG

STV2102B-ST PAL DEFLECTION PROCESSOR

STV2110A-ST Echivalent: STV2110B-ST

STV2110B-ST PAL/SECAM DEFLECTION PROC

STV2118A-ST Echivalent: STV2118B-ST

STV2118B-ST COLOR DECODER PAL/NTSC/SE

STV2145-ST E-W AND VERT. CTRL I2C-BU

STV2151A-ST COLOR DECODER PAL/NTSC/SE

STV2160-ST VIDEO SCANNING PSI PROC.

STV5180D-ST

STV8224A-ST CTV IF AMPLIFIER

STV9379-ST V-DEFL BOOSTER 90V 2A INT

STV9379A-ST V-DEFL BOOSTER 90V 2.6A I

STV9379F-ST Echivalent STV9379FA-ST

STV9379FA-ST V-DEFL BOOSTER 90V 2.6A E

STW15NA50-ST N-FET 500V 14.6A 190W 0.0

SUP70N06-14 N-FET 60V 70A 142W 0.014E

SVI3101D

SVI3102A Echivalent: SVI3102D-MAT

SVI3102B Echivalent: SVI3102D-MAT

SVI3102C Echivalent: SVI3102D-MAT

U1096B-TFK

U116-TFK

U117-TFK

U120-TFK I.C.

U123P-TFK PHOTO PULSE AMPLIFIER 6p

U143M-TFK 7 SEGM.LED DRIVER 16p

U165M-TFK I.C.

U2008B-TFK AC POWER CNTRL REGULATOR

U2067B-TFK

U208B-TFK VACUUM CLEANER MOTOR CNTR

U209B-TFK TACHO MOTOR CNTRL IC

U210B-TFK UNIV. MOTOR SPEED CNTRL I

U211B-TFK UNIV. TACHO AC MOTOR CNTR

U212B-TFK PHASE CONTROL I.C.

U214D Echivalent: 2114-200NS

U217B-TFK LOW-COST ZERO CROSS SWITC

U221B-TFK

U2343B-TFK PLL-STEREO-DECODER 3,5..1

U237B-TFK

U2400B-TFK AUTOMATIC NiCd CHARGER

U2401B-TFK

U2402B-TFK CHARGE CONT. FOR BATTERIE

U243B-TFK

U244B-TFK LED BAND DRIVER(5 LEDS)

U246B-TFK TELEPHONE AMPL.

U247B-TFK

U2501B-TFK

U2505B-TFK IR RECEIVER AMPLIFIER

U2510B-TFK ALL BAND AM/FM RECEIVER

U254B-TFK

U2560B-TFK I.C.

U2561B-TFK I.C.

U257B-TFK

U263B1-TFK

U265B Echivalent: U665B-TFK

U267B-TFK EXTENSION FOR U237

U2822B-TFK DUAL CHAN. AUDIO PWR AMP.

U2829B-TFK 2xFM IF SOUND DEMODULATOR

U2840B-TFK FM IF AMP+SWITCH+VOL.CNTR

U3009M ADC 8B

U3036M Echivalent: U336M-TFK

U3090M-TFK = 209.72226

U310 Echivalent: J310-MBR

U318M-TFK IR RC RECEIVER 28p

U321M-TFK

U327MD-TFK IR RC TRANSMITTER 24p

U329M-TFK

U334M-TFK

U336M-TFK

U337M-TFK TV/FB/PCM RECEIVER

U338M-TFK TV RC PCM RECEIVER

U353M-TFK FET SIGNAL SWITCH

U401B-TFK RECORDER HIGH COM SYST.24

U4062B-TFK FM/VHF TUNER/FRONT 300MHz

U4076B-TFK THREE TONE RINGER

U408B-TFK RECORDER COMPANDER 20p

U4119B-TFK Echivalent: TDA1072A-MBR

U416B-TFK AM/FM PORTABLE RADIO

U417B-TFK Echivalent: TDA1083-TFK

U418B-TFK

U420B-TFK

U427B-TFK IR DIODE DRIVER 8p

U440-MBR 2xN-FET 25V 30mA 0.35W

U4439B Echivalent: TDA4439-TFK

U4453B-TFK VIDEO OUTPUT STAGE & AFC

U4490-TFK

U450B-TFK 3 FREQ.TELEPHONE CALL

U4606B-TFK Echivalent: TEA5040-ST

U4646B-TFK Echivalent: TEA5040-ST

U4647B-KIT Echivalent: TEA5040-ST

U4647B-TFK Echivalent: TEA5040-ST

U4647B/PCB Echivalent: TEA5040-ST

U4647J-TFK

U4744B-TFK TV SAW-FILTER AMPLIFIER

U4777B-TFK VHF-MIXER+2 INT. OSC.

U4985B-A Echivalent: TDA4210-TFK

U555C Echivalent: 2708/DIV

U6052B-TFK MULTIPLEX SYSTEM RECEIVER

U6060B-TFK

U6202B-TFK PLL TUNING I.C. 1.3GHz

U6204B Echivalent: U6204B-TFK

U6204B-FP-TFK

U6204B-TFK PLL TUNING IC 1.3GHz I2C-

U6316B-TFK PLL TUNING IC 1GHz 3-W BU

U643B-TFK CAR BLINK GEN. 1W 0.03E

U664B-SIL-TFK Echivalent U891BS-TFK

U664BS-TFK 1 GHz FREQ.DIV.1:64 SELF

U684B-TFK 1 GHz FREQ. DIVIDER 1:64

U7805 Echivalent: UA7805-MBR

U804D Echivalent: 209.10102

U8060B Echivalent: TDA3560-MBR

U807D Echivalent: SAB3021

U813BS-TFK 1.1GHz DIV. 1:64/128/256

U813BSSP-TFK 1.1GHz DIV. 1:64/128/256

U820 Echivalent: U820B-TFK

U820B-TFK PWR AMP 1.1W/8E 16V

U821B-TFK

U829B-TFK TV FM IF SOUND DEMODULATO

U8442B-TFK

U865BS-TFK

U891BSAFP-TFK SMD VERSION U891BS

UA1458 Echivalent: MC1458N-MBR

UA148 Echivalent: UA741N14/DIV

UA1488 Echivalent: MC1488N-MBR

UA1489 Echivalent: MC1489N-MBR

UA180 Echivalent: UAA180

UA3086 Echivalent: CA3086-MBR

UA324 Echivalent: LM324N-MBR

UA3403 Echivalent: LM324N-MBR

UA4006 Echivalent: UAA4006ADP-ST

UA592 Echivalent: NE592N8-MBR

UA709-14P Echivalent: UA741CN14/DIV

UA709-8P Echivalent: UA741CN8-MBR

UA709CP Echivalent: UA741CN8-MBR

UA709H Echivalent: UA741H-MBR

UA709TO Echivalent: UA741H-MBR

UA710-14P COMP. 1x 21V HIGH SPEED

UA710TO OPAMP to99

UA711-14P Echivalent: LM711N-MBR

UA723CN14-MBR AP V-REG 2-37V 0.15A 3%

UA723CN14/DIV AP V-REG 2-37V 0.15A 3%

UA723H Echivalent: UA723TO

UA723N Echivalent: UA723CN14-MBR

UA723TO AP V-REG 2-37V 0.15A 3%

UA725 Echivalent: LM725CN-NSC

UA7300 DOLBY NOISE REDUCTION

UA733CN-MBR DIF. VIDEO-AMP 10V 120MHz

UA739PC OPAMP 2x 18V LOW NOISE

UA741CH Echivalent: UA741H-MBR

UA741CN8-MBR OPAMP 1x 36V 1MHz GEN.PU.

UA741CN8/DIV OPAMP 1x 36V 1MHz GEN PU

UA741CP Echivalent: UA741CN8-MBR

UA741D-MBR OPAMP 1x 36V 1MHz GEN.PUR

UA741H-MBR OPAMP 1x 36V 1MHz GEN.PUR

UA741H/DIV OPAMP 1x 36V 1MHz GEN.PUR

UA741N14/DIV 1xOPAMP 3-18V 1MHz GEN.PU

UA747CN-MBR OPAMP 2x 36V 1MHz GEN.PUR

UA747CN/DIV OPAMP 2x 36V 1MHz GEN.PU.

UA748N08-MBR 1xOPAMP 3-18V 1MHz DECOMP

UA748TO SINGLE BIP OP AMP TO99

UA749 Echivalent: UA739PC/DIV

UA758N-MBR ALSO: LM1800N-NSC

UA758N/DIV ADDITIONAL DIGITAL IC

UA758PC Echivalent: UA758N/DIV

UA776CN-MBR 1xOPAMP 1.2-18V 1MHz uPOW

UA7805-MBR POS V-REG 5V 1.5A 4%

UA7805/DIV POS V-REG 5V 1A

UA7805F-MBR POS V-REG 5V 1A 4%

UA7805KC/DIV POS V-REG 5V 1.5A 4%

UA7806-MBR POS V-REG 6V 1.5A 4%

UA7806F-MBR POS V-REG 6V 1A ISOLATED

UA7806KC-MBR POS V-REG 6V 1.5A 4%

UA7807-MBR POS V-REG 7V 1A

UA7808-MBR POS V-REG 8V 1.5A 4%

UA7808F-MBR POS V-REG 8V 1A ISOLATED

UA7808KC POS V-REG 8V 1.5A 4%

UA7809-MBR POS V-REG 9V 1.5A 4%

UA7809/DIV POS V-REG 9V 1.5A 4%

UA7809F-MBR POS V-REG 9V 1A ISOLATED

UA7810-MBR POS V-REG 10V 1.5A

UA7812-MBR POS V-REG 12V 1.5A 4%

UA7812/DIV Echivalent UA7812-MBR

UA7812F-MBR POS V-REG 12V 1A

UA7812KC/DIV POS V-REG 12V 1.5A 4%

UA7812R Echivalent: AN7812R-MAT

UA7815-MBR POS V-REG 15V 1.5A 4%

UA7815/DIV POS V-REG 15V 1A

UA7815F-MBR POS V-REG 15V 1A 4%

UA7815KC POS V-REG 15V 1.5A 4%

UA7818-MBR POS V-REG 18V 1.5A 4%

UA7818F-MBR POS V-REG 18V 1A ISOLATED

UA7820-ST POS V-REG 20V 1A

UA7824-MBR POS V-REG 24V 1.5A 4%

UA7824/DIV Echivalent UA7824-MBR

UA7824KC-MBR POS V-REG 24V 1.5A

UA7824KC/DIV POS V-REG 24V 1.5A

UA78509 Echivalent: UA78S09-ST

UA78L02 POS V-REG 2V 0.1A

UA78L05-MBR POS V-REG 5V 0.1A

UA78L05/DIV POS V-REG 5V 0.1A

UA78L06-MBR POS V-REG 6V 0.1A

UA78L06/DIV POS V-REG 6V 0.1A

UA78L08-MBR POS V-REG 8V 0.1A

UA78L08/DIV Echivalent UA78L08-MBR

UA78L09-MBR POS V-REG 9V 0.1A

UA78L09/DIV POS V-REG 9V 0.1A

UA78L10-MBR POS V-REG 10V 0.1A

UA78L12-MBR POS V-REG 12V 0.1A

UA78L12/DIV POS V-REG 12V 0.1A

UA78L15-MBR POS V-REG 15V 0.1A

UA78L15/DIV POS V-REG 15V 0.1A

UA78L18-MBR POS V-REG 18V 0.1A

UA78L20 POS V-REG 20V 0.1A

UA78L24 POS V-REG 24V 0.1A

UA78M05 Echivalent: UA7805-MBR

UA78M08 Echivalent: UA7808-MBR

UA78M12 Echivalent: UA7812-MBR

UA78M15 Echivalent: UA7815-MBR

UA78M18 Echivalent: UA7818-MBR

UA78M20 Echivalent: UA7820-ST

UA78S05-ST POS V-REG 5V 2A 4%

UA78S09-ST POS V-REG 9V 2A 4%

UA78S10-ST POS V-REG 10V 2A 4%

UA78S12-ST POS V-REG 12V 2A 4%

UA78S15-ST POS V-REG 15V 2A 4%

UA78S18-ST POS V-REG 18V 2A 4%

UA78S24-ST POS V-REG 24V 2A 4%

UA78S40-MBR SMPS CNTRL IC 1.5A 100kHz

UA78S75-MBR POS V-REG 7.5V 2A

UA78T05-MBR POS V-REG 5V 3A

UA78T12-MOT POS V-REG 12V 3A

UA78T15-MBR POS V-REG 15V 3A

UA7905-MBR NEG V-REG 5V 1.5A 4%

UA7905KC NEG V-REG 5V 1.5A 4%

UA7906-MBR NEG V-REG 6V 1.5A 4%

UA7908-MBR NEG V-REG 8V 1.5A 4%

UA7908F-MBR NEG V-REG 8V 1A ISOLATED

UA7909-MBR NEG V-REG 9V 1A

UA7909/DIV Echivalent UA7909-MBR

UA7910-MBR NEG V-REG 10V 1A

UA7910F-MBR NEG V-REG 10V 1A

UA7912-MBR NEG V-REG 12V 1.5A 4%

UA7912/DIV Echivalent UA7912-MBR

UA7915-MBR NEG V-REG 15V 1.5A 4%

UA7915KC-MBR NEG V-REG 15V 1.5A 4%

UA7918-MBR NEG V-REG 18V 1.5A 4%

UA7920-ST NEG V-REG 20V 1A

UA7924-MBR NEG V-REG 24V 1.5A 4%

UA7924/DIV NEG V-REG 24V 1.5A

UA79L05-MBR NEG V-REG 5V 0.1A

UA79L05-TOS Echivalent: UA79L05-MBR

UA79L06-MBR NEG V-REG 6V 0.1A

UA79L08 NEG V-REG 8V 0.1A

UA79L09-MBR NEG V-REG 9V 0.1A

UA79L10-MBR NEG V-REG 10V 0.1A

UA79L12-MBR NEG V-REG 12V 0.1A

UA79L15-MBR NEG V-REG 15V 0.1A

UA79L18-MBR NEG V-REG 18V 0.1A

UA79L24-MBR NEG V-REG 24V 0.1A

UA79M12 Echivalent: UA7912-MBR

UA855D Echivalent: Z80APIO/DIV

UA857D Echivalent: Z80ACTC/DIV

UA858D Echivalent: Z80ADMA-MBR

UA880D Echivalent: Z80BCPU-MBR

UA9334 Echivalent: NE5090N-PHI

UA9368 Echivalent: DM9368N-NSC

UA9602PC-MBR DUAL MONOSTABLE MULTIVIB.

UA9636ACP-MBR 2xLINE DRIVER+ADJ. S.R.

UA9637ACP-MBR DUAL DIFFERENTIAL LINE RC

UA9638CP-TI 2xH.S. DIFF. LINE DRIVER

UA9639CP-MBR DUAL DIFF. LINE RECEIVER

UA9667 Echivalent: ULN2003AN-MBR

UA9668 Echivalent: ULN2004AN-MBR

UAA1004DP-MOT VOLTAGE SWITCH CIRCUIT 8p

UAA1006-PHI

UAA1008ADC TV STATION SEARCHING

UAA1008ADP-MOT

UAA1009 ARI DECODER 16p

UAA1016B-MOT POWER SUPPLY SUPPORT DEV.

UAA1300-PHI

UAA145-TFK THYRISTOR 3-PHASE CNTRL I

UAA145/DIV THYRISTOR 3-PHASE CNTRL I

UAA146-TFK Echivalent: UAA145-TFK

UAA170-SIE

UAA170/DIV Echivalent: UAA170-SIE

UAA180 12 LED DISPLAY ENCODER=A2

UAA190 TV TUNING INDICATOR 8p

UAA2000A LIN-IC PLL-SYNTH. TV-TUN.

UAA2003-MBR PLL INT. FOR FREQ. SYNTH.

UAA2016P-MOT ZERO CROS. TRIG/PHASE CNT

UAA2022-MOT 16 SEGM.LED DRIVER 24p

UAA4000-ST

UAA4000S

UAA4002DP/DIV

UAA4006ADP-ST

UAA4006BDP-ST

UAA4006DP-ST

UAA4009-ST

UB855D Echivalent: Z80BPIO-MBR

UB8563D Echivalent: Z80ADART-MBR

UB857D Echivalent: Z80ACTC-MBR

UB880D Echivalent: Z80BCPU-MBR

UC2842AN-MBR SMPS CIRCUIT

UC2844N-MBR SMPS CIRCUIT

UC3524 Echivalent: SG3524N-MBR

UC3842AN-MBR PWM I-CNTRL 5V 2% 52KHz

UC3842N-MBR PWM I-CNTRL 5V 2% 52KHz

UC3843AN-MBR PWM I-CNTRL 5V 2% 52KHz

UC3843N-MBR PWM I-CNTRL 5V 2% 52KHz

UC3844AN-MBR PWM I-CNTRL 5V 2% 52KHz

UC3844N-MBR PWM I-CNTRL 5V 2% 52KHz

UC3844N/DIV PWM I-CNTRL 5V 2% 52KHZ

UC3845AN-MBR PWM I-CNTRL 5V 2% 52KHz

UC3845N-MBR PWM I-CNTRL 5V 2% 52KHz

UC3846N-MBR PWM I-CNTRL

UC3851N-MBR PWM I-CNTRL 32V .5% 50KHZ

UC3854N-MBR POWER FACTOR CONTROLER

UC3906N BATTERY CHARGER

UCN5801A LATCHED PHERIPHERAL DRIVE

UDN2540B PERIPHERAL DRIVER

UDN2540H Echivalent: 209.73191

UDN2580A Echivalent: 209.60886

UDN2953B-SPR FULL-BRIDGE DC MOTOR DRVR

UDN2981A Echivalent: TD62783AP-TOS

UDN2982 Echivalent: M54562P-MIT

UDN2983A-MBR 8-CHANNNEL SOURCE DRIVER

UDN2984A 8-CHANNEL SOURCE DRIVER

UDN2987A-MBR OCTAL PERIPHERAL DRIVER

UDN6118A Echivalent: NE594N-MBR

UF4001 Echivalent: UF4003

UF4002 Echivalent: UF4003

UF4003 SI-D 200V 1A/50Ap 50ns

UF4004 Echivalent: UF4007

UF4005 Echivalent: UF4007

UF4006 Echivalent: UF4007

UF4007 SI-D 1000V 1A/50Ap 75ns

UF5401 Echivalent: UF5404

UF5402 Echivalent: UF5404

UF5403 Echivalent: UF5404

UF5404 SI-D 400V 3A/150Ap 50ns

UF5406 Echivalent: UF5408

UF5407 Echivalent: UF5408

UF5408 SI-D 1000V 3A/150Ap 75ns

UGN3140U-MBR HALL-EFFECT SWITCH

UL1211 Echivalent: LA1201

UL1242 Echivalent: TBA120S-MBR

UL1497 Echivalent: TCA150T-BAN

UL1970N Echivalent: UAA170/DIV

UL1980N Echivalent: UAA180

ULN2001AN-MBR 7xN-DR. 50V 0.5A TTL INP

ULN2002AN-MBR 7xN-DR. 50V 0.5A 14-25V

ULN2003AN-MBR 7xN-DR. 50V 0.5A=TD62003

ULN2003AN/DIV Echivalent: ULN2003AN-MBR

ULN2004AN-MBR 7xN-DR. 50V 0.5A 6-15V

ULN2011A 7xNPN DARLINGT.ARRAY

ULN2024A-MBR Echivalent: SN75469-MBR

ULN2064B-MBR 4xN-DARL 50V 1.5A TTL-INP

ULN2065B-ST 4xN-DARL 80V 1.5A TTL-INP

ULN2066B-MBR 4xN-DARL 50V 1.5A 6-15V

ULN2067B Echivalent: ULN2066B-MBR

ULN2068B-MBR 4xN-DARL 50V 1.5A TTL INP

ULN2069B-MBR QUAD DARLINGTON DRIVER

ULN2074B-MBR 4xN-DARL 50V 1.5A TTL INP

ULN2075B-MBR 4xN-DARL 80V 1.5A TTL-INP

ULN2083 Echivalent: CA3083-HAR

ULN2204 Echivalent: TDA1083-TFK

ULN2244 Echivalent: UA758N-MBR

ULN2801A-MBR 8xN-DR. 50V 0.5A TTL-INP

ULN2802A-MBR 8xN-DR. 50V 0.5A 14-25V

ULN2803A-MBR 8xN-DR. 50V 0.5A=TD62083A

ULN2804A-MBR 8xN-DR. 50V 0.5A 6-15V

ULN2805A-MBR 8xN-DR. 50V 0.5A 6-15V

ULN2813A OCTAL DARLINGTON DRIVER

ULN2823A Echivalent: L603C-ST

ULN3718M-MBR SINGLE CHANNEL AUDIO AMP.

ULN3810 Echivalent: MC1310P-MOT

ULN3812 Echivalent: UA758N-MBR

ULN3862A Echivalent: MC3361B-MBR

UM3166 Echivalent: UM66TxxL

UM3481A

UM3484A-UMC 2-MELODY GENERATOR

UM3511A-UMC

UM3561A-UMC 3 SIRENE GENERATOR

UM3750 Echivalent: UM3758-120A-UM

UM3758-120A-UM PROG. ENCODER/DECODER

UM5010-07-UMC

UM6114 Echivalent: 2114-200NS

UM6116 Echivalent: 6116-100-MBR

UM61256-15NS CACHE RAM 32x8K 15ns

UM62256 Echivalent: 62256-70-MBR

UM6264 Echivalent: 6264-100-MBR

UM6502 Echivalent: 6502A-MBR

UM6532 Echivalent: 6532P

UM66T01S-MBR

UM66T19L-MBR MELODY GEN: FOR ELISE

UM66T20L-UMC MELODY GENERATOR

UM66T32L-MBR MELODY GEN: WALTZ

UM66T68L-MBR MELODY GEN: IT'S A SMALL

UM6845 Echivalent: 6845P-MBR

UM7106 Echivalent: ICL7106CPL-MBR

UM7106R Echivalent: ICL7106RCPL-MB

UM7107A/B Echivalent: ICL7107CPL-MBR

UM8250 Echivalent: 8250A-MBR

UM82C11 Echivalent: 82C11P-MBR

UM82C206 Echivalent: 82C206PLCC

UM86409 Echivalent: MM53200N-NSC

UM91210C-UMC TONE/PULSE SWITCHABLE DIA

UM9151-3 DMTF TELF. DIALER =FT9151

UM91531-UMC TONE PULSE DIALER+BCD INP

UM91611-UMC

UM95089 DTMF TONE DECODER

UMC3481 Echivalent: UM3481A

UMC3484 Echivalent: UM3484A-UMC

UMC91210C-UMC Echivalent: UM91210C-UMC

UN101 IC

UN102 I.C.

UN1111 Echivalent: 8945705151

UN1112 Echivalent: 8945705343

UN4112 Echivalent: 8945705343

UN4214 I.C.

UPA2003 Echivalent: ULN2003AN-MBR

UPA53C-NEC Echivalent: M54516P-MIT

UPA63H-NEC 2xN-FET 60V Idss>20mA 0.5

UPA68HA-NEC

UPA81C-NEC N-ARRAY 8x40V 0.4A B>1K

UPC1001

UPC1004C-NEC FM-KF

UPC1018C-NEC AM TUNER AM/FM-IF AMP.

UPC1020H-NEC

UPC1023 Echivalent: UPC1024H-NEC

UPC1024H-NEC PREAMP LOW NOISE

UPC1025H-NEC

UPC1026C-NEC STEREO DECODER 14p

UPC1028 Echivalent: TA7130P-TOS

UPC1031H-NEC VERTICAL DEFLECTION 10p

UPC1032 Echivalent: LA3160-SAN

UPC1035C-NEC RECORDER FUNCT.CONTROL 14

UPC1037H-NEC SINGLE SIDE BAND AMPL.

UPC1042C Echivalent: TL494CN-MBR

UPC1043C-NEC MOTOR CONTROLER 16p

UPC1094C-NEC SMPS CIRCUIT

UPC1158HA-NEC RECORDER ALC 7p

UPC1161C-NEC STEREO DECODER 16p

UPC1165 Echivalent: KA2213-SSU

UPC1167C-NEC FM-DEM + AFC + AGC

UPC1170 PRE AMPL.+ ALC 8p

UPC1171C-NEC AM TUNER IF AMP. 14p

UPC1173C-NEC FM MPX DEMOD.FM TUNER 16p

UPC1176C-NEC FM NOISE SUPPRESSOR 16p

UPC1177H-NEC

UPC1178C-NEC

UPC1181H-NEC AMPLIFIER 5.8W

UPC1182H-NEC =UPC1181 REVERSED PINNING

UPC1185H-NEC PWR AMP 2*5.8W

UPC1186 Echivalent: LA3160-SAN

UPC1187V-NEC STEREO DECODER

UPC1188H-NEC 20W POW.AMPL.10P(=HA1350)

UPC1191V-NEC AM-HF + ZF + AGC

UPC1197C-NEC STEREO DECODER

UPC1204C-NEC REC/PLAYB AMPL

UPC1208C Echivalent: UPC1167C-NEC

UPC1210C-NEC

UPC1211V-NEC FM IF + PLL DETECTOR 19p

UPC1212C-NEC 1.0W POWER AMPLIFIER 8p

UPC1213C-NEC 2.4W POWER AMPLIFIER 8p

UPC1215V-NEC AM TUNER + AMPL. + AGC

UPC1216 NF AMPL

UPC1223C-NEC STEREO DECODER

UPC1225H-NEC 30-50W POW.AMP.DRIVER 12p

UPC1227V-NEC STEREO DECODER

UPC1228HA-NEC DUAL PRE-AMPLIFIER 8p

UPC1230H2-NEC 20W BTL POWER AMP. 12p H

UPC1235C-NEC STEREO DECODER

UPC1237HA-NEC POWER AMPLIFIER PROTECTOR

UPC1238V-NEC 10W POWER AMPL. to220/5p

UPC1241H-NEC POWER AMPLIFIER

UPC1242H-NEC POWER AMPLIFIER

UPC1245V-NEC FM-ZF + AGC + AFC

UPC1251 Echivalent: LM358N-MBR

UPC1253H2-NEC LEVEL Senzor, RECORDER-DB

UPC1255-NEC

UPC1263-NEC Echivalent KA2214-SSU

UPC1270H-NEC

UPC1277H-NEC

UPC1278H-NEC

UPC1288V-NEC

UPC1290C-NEC SPST QUAD (NO)(LOG.LEVEL0

UPC1297CA-NEC Echivalent 209.72874

UPC1298V-NEC 50 - 80W POWER AMP DRIVER

UPC1310V-NEC AUDIO DUAL AMPL. 7W

UPC1313HA-NEC DUAL CHANNEL PREAMP.

UPC1316C-NEC DUAL 1.2W POWER AMP 14P

UPC1318AV-NEC PWR AMP 2x23W 160kHz

UPC1320 Echivalent: BA1320-RHM

UPC1330HA-NEC REC/PB AUDIO HEAD SWITCH

UPC1335V-NEC

UPC1342V-NEC AUDIO PREAMP & OUTPUT AMP

UPC1350C-NEC PREAMP. + ALC 14p

UPC1352C-NEC

UPC1353C-NEC 2.4W TV SOUND AMPLIFIER

UPC1356C2-NEC

UPC1360C-NEC CHANNEL SELECTOR 24p

UPC1361C-NEC CHANNEL SELECTOR

UPC1362C-NEC 12 CHANNEL SELECTOR 20p

UPC1363C-NEC TV Senzor IC

UPC1363CA-NEC CHANNEL SEL 24PIN SMALL!!

UPC1364C2-NEC CHROMINANCE PROCESSOR

UPC1365C-NEC CHROM.+ LUMIN.PROCES. 28p

UPC1366C-NEC VIDEO IF PROCESSOR 14p

UPC1367C-NEC

UPC1373HA-NEC IR AMPL/RECEIVER

UPC1377C-NEC HORIZONTAL/VERTICAL DEFL.

UPC1378H-NEC

UPC1379C-NEC SYNCHRO.SIGNAL PROCES.

UPC1380C

UPC1382C Echivalent: CA3065E-MBR

UPC1384C-NEC

UPC1387C-NEC RGB INP.FAST BLANK.OUTP.L

UPC1391HA-NEC SOUND IF PROCESSOR 8p

UPC1394C-NEC SWITCHING REGULATOR

UPC1397C-NEC CHROMA PROCESSOR

UPC1406HA-NEC DUAL ATTENUATOR

UPC1420CA-NEC COLOR DEMODULATOR

UPC1421CA-NEC I.C.

UPC1423

UPC14308 Echivalent: UA7808-MBR

UPC1447-NEC

UPC1458 Echivalent: MC1458N-MBR

UPC1474HA-NEC REMOTE CONTROL RECEIVER

UPC1470H-NEC MOTOR SPEED REGULATOR

UPC1484CA-NEC I.C.

UPC1488H-NEC VERTICAL DEFLECTION 9-22'

UPC1490HA-NEC INFRARED TRANSM./RECEIVER

UPC1498H-NEC

UPC1504C-NEC PG/FG HIGH SENSE AMPL.

UPC1505C-NEC I.C.

UPC1513HA-NEC I.C.

UPC1514CA-NEC I.C.

UPC1520CA-NEC I.C.

UPC1521HA-NEC IC.

UPC1524 Echivalent: HA11745-HIT

UPC1525-NEC I.C.

UPC1533HA-NEC I.C.

UPC1534 Echivalent: HA11744-HIT

UPC1536

UPC1571C-NEC TELECOM COMPANDER

UPC1701C-NEC IC.

UPC1892CT-NEC SURROUND SOUND PROCESSOR

UPC2002V-NEC Echivalent: TDA2002-ST

UPC2311C-NEC

UPC2500AH-NEC SINGLE CHANNEL AUDIO AMP.

UPC251 Echivalent: MC1458N-MBR

UPC30C-NEC

UPC311 Echivalent: LM311N-MBR

UPC324 Echivalent: LM324N-MBR

UPC339C Echivalent: LM339N-MBR

UPC339G Echivalent: LM339D-MBR

UPC358 Echivalent: LM358N-MBR

UPC393 Echivalent: LM393N-MBR

UPC41C-NEC

UPC451 Echivalent: LM224N-MBR

UPC451C Echivalent: LM224N-MBR

UPC4557C-NEC DUAL OPAMP, HIGH PERFOR.

UPC4558 Echivalent: MC4558CN-MBR

UPC4559 Echivalent: RC4559N-MBR

UPC4560C-NEC Echivalent: NJM4560DD-JRC

UPC4560G-NEC SMD DUAL OP-AMP 10MHz

UPC4570C-NEC DUAL OPAMP

UPC4570HA-NEC DUAL OPAMP

UPC494C Echivalent: TL494CN-MBR

UPC5121-NEC

UPC554C-NEC STEREO DECODER 14p

UPC555H Echivalent: NE555H-MBR

UPC563 AMPLIFIER 17V 2.5A 5.2W

UPC566

UPC571H-NEC 5.5W AUDIO AMPLIFIER 12p

UPC574J-NEC REF-DIODE 33V FOR TUNER

UPC575C2-NEC 2.0W POWER AMPLIFIER 8p

UPC576 3W AUDIO AMPL. 100kHz

UPC577H-NEC FM IF AMPLIFIER

UPC580C-NEC

UPC585C-NEC

UPC587 Echivalent: MC1310P-MOT

UPC592H2 NF-V + EQUALIZER

UPC741G Echivalent: UA741D-MBR

UPC7805H Echivalent: UA7805-MBR

UPD1708AG223 Echivalent UPD1708AG226

UPD1708AG226

UPD1719G011 CPU 4B 256RAM 2KROM+ADC+L

UPD1719G014 CPU 4B 256RAM 2KROM+ADC+L

UPD1937C-NEC IR-REM.CTR. RECEIVER 20 C

UPD1943G-NEC SMD IR SENDER/TRANSMITTER

UPD1986C-NEC IR TRANSMITTER 27COMMANDS

UPD1987-NEC REMOTE CONTROL RECEIVER

UPD4016C1 Echivalent: 6116-100-MBR

UPD4066 Echivalent: CD4066-MBR

UPD416 Echivalent: 4116-150

UPD424256C70 Echivalent: 44256-70-MBR

UPD6121G001 SMD REMOTE CTRL IR REC/TR

UPD6142C001 I.C.

UPD6251C-NEC EEPROM

UPD6450CX002 ON SCREEN DISPLAY CIRCUIT

UPD70108 Echivalent: V20 8MHZ

UPD70116 Echivalent: V30 8MHZ

UPD71051 Echivalent: 82C51-MBR

UPD71054C Echivalent: 82C54P-MBR

UPD71055C Echivalent: 82C55AP-MBR

UPD75104CW087 Echivalent: UPD75104CW87

UPD75104CW87 SYSTEM CONTROL PROCESSOR

UPD75108G76 Echivalent: 209.61994

UPD765C-NEC FLOPPY DISK CONTROLER

UPD780C Echivalent: Z80BCPU-MBR

UPD780C-1 Echivalent: Z80BCPU-MBR

UPD8035 Echivalent: 8035DIP-MBR

UPD8039 Echivalent: 8039DIP-MBR

UPD8049A42 Echivalent: 209.10171

UPD8049C163 Echivalent: 900.10252

UPD8049C568 Echivalent: 209.10543

UPD8243C Echivalent: 8243C-MBR

UPD8748HC Echivalent: 8748HC-MBR

UPD8748HD Echivalent: 8748HD-MBR

UPD8749HC Echivalent: 8749HD-MBR

V20 8MHZ CPU 8B 8MHz=UPD70108C8

V30 8MHZ CPU 16B 8MHz=UPD70116C8

V53C104FP70 Echivalent: 44256-70-MBR

VAD2150DIP-ITT A/D I.C. 7 BIT CTV 25MHz

VC5022X I.C. MONO ANA

VCU2100-ITT

VCU2133ADIP-IT HIGH SPEED (DE)CODER 40p

VCU2133APLC-IT HIGH SPEED AD/DA CODEC

VCU2136A46-ITT Echivalent: VCU2136ADIP-ITT

VCU2136ADIP-IT

VN10KM-MBR N-FET 60V 0.31A 5E Up<2.5

VN66AF-SLX Echivalent: VN66AFD-SLX

VN66AFD-SLX N-FET 60V 2A 12W 3E Up<2.

VN88AF Echivalent: VN88AFD-SLX

VN88AFD-SLX N-FET 80V 1.3A 20W Up<2.5

VPU2200 Echivalent: PVPU2200-ITT

VPU2203 Echivalent: PVPU2203-ITT

VPU2204 Echivalent: PVPU2204DIP-IT

VSP2860DIP-ITT

X0102BA Echivalent: P0102DA-ST

X0250C Echivalent: LA4422-SAN

X0403DE Echivalent: TIC106M-MBR

X0403ME Echivalent: TIC106M-MBR

X0403NE Echivalent: TIC106N-MBR

X0640C Echivalent: LA7830-SAN

X2401P Echivalent: 24C01N-MBR

X2402P Echivalent: 24C02N-MBR

X2404P Echivalent: 24C04N-MBR

X2408P Echivalent: 24C08N-MBR

X2416P Echivalent: 24C16N-MBR

X2444P EEPROM 16x16 BIT 1MHz

X24C01 Echivalent: 24C01N-MBR

X24C02 Echivalent: 24C02N-MBR

X24C02P Echivalent: 24C02N-MBR

X24C04 Echivalent: 24C04N-MBR

X24C08 Echivalent: 24C08N-MBR

X24C16 Echivalent: 24C16N-MBR

X24C16P Echivalent: 209.62524

XC401007P Echivalent: ZC401007P-MOT

XC488AO Echivalent: BA6353S-RHM

XC86413B

XC86602P Echivalent: 8305686603

XC86607P-MOT

XC86617P Echivalent 8305686617

XC86620P-MOT

XC94308P Echivalent: 8945902458

XR13600 Echivalent: LM13600N-MBR

XR2003 Echivalent: ULN2003AN-MBR

XR2201 Echivalent: ULN2001AN-MBR

XR2203CP Echivalent: ULN2003AN-MBR

XR2204CP Echivalent: ULN2004AN-MBR

XR2205 Echivalent: ULN2004AN-MBR

XR2206CP-MBR MONOLITHIC FUNCTION-GEN

XR2207CP-MBR VOLT CONTROLLED OSCILL

XR2209CP-MBR PRECISION OSCILLATOR

XR2211CP-MBR FSK DEM/TONE DECODER

XR2264CP-MBR

XR2567CP-MBR

XR3403 Echivalent: LM324N-MBR

XR4151 Echivalent: RC4151-MBR

XR4194N Echivalent: RC4194N-MBR

XR4741 Echivalent: LM348N-MBR

XR8038CP Echivalent: ICL8038CCPD-MB

YM3020 DAC 1*16B 6V 3.5US

YM3623B DIGITAL AUDIO CIRCUIT

YM3805 CD-PLAYER CONTROLER

YM3811 IC

YM3815H

YTF530 Echivalent: IRF530-MBR

YTF640 Echivalent: IRF640-MBR

YTF840 Echivalent: IRF840-MBR

Z0105MA-MBR Echivalent Z0107MA-MBR

Z0107MA-MBR TRIAC 600V 0.8A 7mA 2us

Z0405MF-MBR TRIAC 600V 4A 5MA

Z0840006PSC Echivalent: Z80BCPU-MBR

Z10/160B Echivalent: 1.3W 160V

Z80ACPU-MBR Echivalent: Z80BCPU-MBR

Z80ACPU/DIV Echivalent: Z80BCPU-MBR

Z80ACTC-MBR PROG.CNTR/TIMER 4MHz

Z80ACTC/DIV PROG.CNTR/TIMER 4MHz=UA85

Z80ADART-MBR DUAL ASYN.REC.TRANSM.4MHz

Z80ADMA-MBR DMA CONTROLER 4MHz

Z80APIO-MBR Echivalent: Z80BPIO-MBR

Z80APIO/DIV PARALL.I/O.CONT.4MHz=UA85

Z80ASIO-0-MBR SER. I/O CNTRL 4MHz Z8440

Z80ASIO-1-MBR SER. I/O CNTRL 4MHz Z8441

Z80ASIO-2-MBR SER. I/O CNTRL 4MHz Z8442

Z80BCPU-MBR CPU 8B 6MHz

Z80BCTC-MBR PROG.CNTR/TIMER 6MHz

Z80BPIO-MBR DUAL 8-BIT I/O PORT CNTR

Z80BSIO-0-MBR SER. I/O CNTRL 6MHz Z8440

Z80BSIO-1-MBR SER. I/O CNTRL 6MHz Z8441

Z80BSIO-2-MBR SER. I/O CNTRL 6MHz Z8442

Z80CPU Echivalent: Z80BCPU-MBR

Z80CTC Echivalent: Z80ACTC-MBR

Z80DART Echivalent: Z80ADART-MBR

Z80DMA Echivalent: Z80ADMA-MBR

Z80PIO Echivalent: Z80BPIO-MBR

Z80SIO Echivalent: Z80ASIO-0-MBR

Z8400 Echivalent: Z80BCPU-MBR

Z8410 Echivalent: Z80ADMA-MBR

Z8420 Echivalent: Z80BPIO-MBR

Z8430 Echivalent: Z80ACTC-MBR

Z8440 Echivalent: Z80ASIO-0-MBR

Z8441 Echivalent: Z80ASIO-1-MBR

Z8442 Echivalent: Z80ASIO-2-MBR

Z8681AB1-ST Echivalent: Z8681B1-ST

Z8681B1-ST CPU 8B 8MHZ 144B RAM

ZC401007P-MOT Echivalent 8945902458

ZC84236P Echivalent: 8305212844

ZC84237P-MOT

ZC84285P-MOT ALSO: 8945901582

ZC86617P Echivalent: XC86617P

ZC86622P-MOT Echivalent: 8945902766

ZC86625P Echivalent: 8945902766

ZC88654P Echivalent: 8305684654

ZC89501P

ZC89504 Echivalent: 8305689504

ZGS 0.05A SICHERUNG 6X32 FLINK 0.05

ZGS 0.063A SICHERUNG 6X32 FLINK 0.06

ZGS 0.08A SICHERUNG 6X32 FLINK 0.08

ZGS 0.125A SICHERUNG 6X32 FLINK 0.12

ZGS 0.16A SICHERUNG 6X32 FLINK 0.16

ZGS 0.1A SICHERUNG 6X32 FLINK 0.1A

ZGS 0.25A SICHERUNG 6X32 FLINK 0.25

ZGS 0.2A SICHERUNG 6X32 FLINK 0.2A

ZGS 0.315A SICHERUNG 6X32 FLINK 0.31

ZGS 0.4A SICHERUNG 6X32 FLINK 0.4A

ZGS 0.5A SICHERUNG 6X32 FLINK 0.5A

ZGS 0.63A SICHERUNG 6X32 FLINK 0.63

ZGS 0.8A SICHERUNG 6X32 FLINK 0.8A

ZGS 1.25A SICHERUNG 6X32 FLINK 1.25

ZGS 1.6A SICHERUNG 6X32 FLINK 1.6A

ZGS 10A SICHERUNG 6X32 FLINK 10A

ZGS 12A SICHERUNG 6X32 FLINK 12A

ZGS 15A SICHERUNG 6X32 FLINK 15A

ZGS 16A SICHERUNG 6X32 FLINK 16A

ZGS 1A SICHERUNG 6X32 FLINK 1A 2

ZGS 2.5A SICHERUNG 6X32 FLINK 2.5A

ZGS 20A SICHERUNG 6X32 FLINK 20A

ZGS 2A SICHERUNG 6X32 FLINK 2A 2

ZGS 3.15A SICHERUNG 6X32 FLINK 3.15

ZGS 4A SICHERUNG 6X32 FLINK 4A 2

ZGS 5A SICHERUNG 6X32 FLINK 5A 2

ZGS 6.3A SICHERUNG 6X32 FLINK 6.3A

ZGS 8A SICHERUNG 6X32 FLINK 8A 2

ZGT 0.032A SICHERUNG 6X32 TRAGE 0.03

ZGT 0.05A SICHERUNG 6X32 TRAGE 0.5A

ZGT 0.063A SICHERUNG 6X32 TRAGE 0.06

ZGT 0.08A SICHERUNG 6X32 TRAGE 0.08

ZGT 0.125A SICHERUNG 6X32 TRAGE 0.12

ZGT 0.16A SICHERUNG 6X32 TRAGE 0.16

ZGT 0.1A SICHERUNG 6X32 TRAGE 0.1A

ZGT 0.25A SICHERUNG 6X32 TRAGE 0.25

ZGT 0.2A SICHERUNG 6X32 TRAGE 0.2A

ZGT 0.315A SICHERUNG 6X32 TRAGE 0.31

ZGT 0.4A SICHERUNG 6X32 TRAGE 0.4A

ZGT 0.5A SICHERUNG 6X32 TRAGE 0.5A

ZGT 0.63A SICHERUNG 6X32 TRAGE 0.63

ZGT 0.8A SICHERUNG 6X32 TRAGE 0.8A

ZGT 1.25A SICHERUNG 6X32 TRAGE 1.25

ZGT 1.6A SICHERUNG 6X32 TRAGE 1.6A

ZGT 10A SICHERUNG 6X32 TRAGE 10A

ZGT 12A SICHERUNG 6X32 TRAGE 12A

ZGT 15A SICHERUNG 6X32 TRAGE 15A

ZGT 16A SICHERUNG 6X32 TRAGE 16A

ZGT 1A SICHERUNG 6X32 TRAGE 1A 2

ZGT 2.5A SICHERUNG 6X32 TRAGE 2.5A

ZGT 2A SICHERUNG 6X32 TRAGE 2A W

ZGT 3.15A SICHERUNG 6X32 TRAGE 3.15

ZGT 4A SICHERUNG 6X32 TRAGE 4A 2

ZGT 5A SICHERUNG 6X32 TRAGE 5A 2

ZGT 6.3A SICHERUNG 6X32 TRAGE 6.3A

ZGT 8A SICHERUNG 6X32 TRAGE 8A 2

ZH1 KUNSTST.SICHERUNGSHALTER

ZH2 SICHERUNGSHALTER METAL 5x

ZH3 SICHERUNGSH.5x20 BAJONET

ZH4 SICHERUNGSHALT.5x20 SCREW

ZH5 SICHERUNGSH.6x32 SCREW.CA

ZH6 SICHERUNGSH.5x20 SCREW CA

ZH6S =SICHERUNGSHALTER6 KLEIN

ZH8 SICHERUNGSHALTER

ZH9 IN LINE SICHERUNGSHALTER

ZKS 0.05A SICHERUNG 5X20 FLINK 0.05

ZKS 0.063A SICHERUNG 5X20 FLINK 0.06

ZKS 0.08A SICHERUNG 5X20 FLINK 0.08

ZKS 0.125A SICHERUNG 5X20 FLINK 0.12

ZKS 0.16A SICHERUNG 5X20 FLINK 0.16

ZKS 0.1A SICHERUNG 5X20 FLINK 0.1A

ZKS 0.25A SICHERUNG 5X20 FLINK 0.25

ZKS 0.2A SICHERUNG 5X20 FLINK 0.2A

ZKS 0.315A SICHERUNG 5X20 FLINK 0.31

ZKS 0.4A SICHERUNG 5X20 FLINK 0.4A

ZKS 0.5A SICHERUNG 5X20 FLINK 0.5A

ZKS 0.63A SICHERUNG 5X20 FLINK 0.63

ZKS 0.8A SICHERUNG 5X20 FLINK 0.8A

ZKS 1.25A SICHERUNG 5X20 FLINK 1.25

ZKS 1.6A SICHERUNG 5X20 FLINK 1.6A

ZKS 10A SICHERUNG 5X20 FLINK 10A

ZKS 12A SICHERUNG 5X20 FLINK 12A

ZKS 16A SICHERUNG 5X20 FLINK 16A

ZKS 1A SICHERUNG 5X20 FLINK 1A

ZKS 2.5A SICHERUNG 5X20 FLINK 2.5A

ZKS 2A SICHERUNG 5X20 FLINK 2A

ZKS 3.15A SICHERUNG 5X20 FLINK 3.15

ZKS 4A SICHERUNG 5X20 FLINK 4A

ZKS 5A SICHERUNG 5X20 FLINK 5A

ZKS 6.3A SICHERUNG 5X20 FLINK 6.3A

ZKS 8A SICHERUNG 5X20 FLINK 8A

ZKS ASS ASS.SICHERUNGEN 5x20 FLIN

ZKT 0.032A SICHERUNG 5X20 TRAGE 0.03

ZKT 0.04A SICHERUNG 5X20 TRAGE 0.04

ZKT 0.05A SICHERUNG 5X20 TRAGE 0.05

ZKT 0.063A SICHERUNG 5X20 TRAGE 0.06

ZKT 0.08A SICHERUNG 5X20 TRAGE 0.08

ZKT 0.125A SICHERUNG 5X20 TRAGE 0.12

ZKT 0.16A SICHERUNG 5X20 TRAGE 0.16

ZKT 0.1A SICHERUNG 5X20 TRAGE 0.1A

ZKT 0.25A SICHERUNG 5X20 TRAGE 0.25

ZKT 0.2A SICHERUNG 5X20 TRAGE 0.2A

ZKT 0.315A SICHERUNG 5X20 TRAGE 0.31

ZKT 0.4A SICHERUNG 5X20 TRAGE 0.4A

ZKT 0.5A SICHERUNG 5X20 TRAGE 0.5A

ZKT 0.63A SICHERUNG 5X20 TRAGE 0.63

ZKT 0.8A SICHERUNG 5X20 TRAGE 0.8A

ZKT 1.25A SICHERUNG 5X20 TRAGE 1.25

ZKT 1.6A SICHERUNG 5X20 TRAGE 1.6A

ZKT 10A SICHERUNG 5X20 TRAGE 10A

ZKT 12A SICHERUNG 5X20 TRAGE 12A

ZKT 16A SICHERUNG 5X20 TRAGE 16A

ZKT 1A SICHERUNG 5X20 TRAGE 1A

ZKT 2.5A SICHERUNG 5X20 TRAGE 2.5A

ZKT 2A SICHERUNG 5X20 TRAGE 2A

ZKT 3.15A SICHERUNG 5X20 TRAGE 3.15

ZKT 4A SICHERUNG 5X20 TRAGE 4A

ZKT 5A SICHERUNG 5X20 TRAGE 5A

ZKT 6.3A SICHERUNG 5X20 TRAGE 6.3A

ZKT 8A SICHERUNG 5X20 TRAGE 8A

ZKT ASS ASS.SICHERUNGEN 5x20 TRAG

ZN409E-PLS

ZN414Z-PLS

ZN415E Echivalent: ZN416E-PLS

ZN426E8-PLS

ZN427E ADC 8B UP-COMP

ZN428E8-PLS

ZN429E8-PLS

ZN447E-PLS ADC 8-BIT 8.5us 0.12%

ZN449E-MBR

ZN458-PLS REF-DIODE 2.45V 30PPM

ZPD 30V Echivalent: 0.4W 30V

ZPD30 Echivalent: 0.4W 30V

ZPY33 Echivalent: 1.3W 33V

ZPYXXX Echivalent: 1.3W XXV

ZTE 1V5 Echivalent: 0.3W 1V5

ZTE 2V Echivalent: 0.3W 2V

ZTE2 Echivalent: 0.3W 2V

ZTK33B REF-DIODE 33V 100PPM 3%

ZTX108 Echivalent: BC547A-MBR

ZTX213 SI-P 45V 0.2A 0.3W 350MHz

ZTX300 Echivalent: BC337-25-MBR

ZTX301 Echivalent: BC337-25-MBR

ZTX302 Echivalent: BC337-25-MBR

ZTX342 SI-N 120V 0.1A 0.3W

ZTX450 SI-N 60V 1A 1W 150MHz

ZTX500 Echivalent: BC327-25-MBR

ZTX501 Echivalent: BC327-25-MBR

ZTX502 Echivalent: BC327-25-MBR

ZTX550 SI-P 60V 1A 1W >150MHz

ZTX649 Echivalent: ZTX651

ZTX650 Echivalent: ZTX651

ZTX651 Echivalent: ZTX653

ZTX652 Echivalent: ZTX653

ZTX653 SI-N 120V 2A 1W >140MHz

ZTX749 Echivalent: ZTX753

ZTX750 Echivalent: ZTX753

ZTX751 Echivalent: ZTX753

ZTX752 Echivalent: ZTX753

ZTX753 SI-P 120V 2A 1W TO92

ZTX753M1TA SI-P 120V 2A 1W

ZY68-ITT Echivalent: 2W 68V

B064D Echivalent: TL064CN/DIV

B081 Echivalent: TL071CN-MBR

B083D Echivalent: TL083CN-MBR

B084D Echivalent: TL084CN

B1000C10000F Echivalent: B1000C15000F

B1000C15000F BRIDGE RECT.SQUARE FASTON

B1000C1500R BRIDGE RECT. ROUND =W10M

B1000C25000F BRIDGE RECT.SQUARE FASTON

B1000C35000F BRIDGE RECT.SQUARE FASTON

B1000D BRIDGE RECT. DIL =DB107

B100C10000F Echivalent: B100C15000F

B100C15000F BRIDGE RECT.SQUARE FASTON

B100C1500R BRIDGE RECT. ROUND =W01M

B100C25000F BRIDGE RECT.SQUARE FASTON

B100C35000F Echivalent: B400C35000F

B100D Echivalent: B200D

B1060 Echivalent: MBR1060-MBR

B110D Echivalent: UA710-14P

B125C1000 Echivalent: B250C2300A OF B

B125C1500A Echivalent: B250C2300A

B125C1500B Echivalent: B250C2300B

B125C1500R Echivalent: B250C1500R

B125C2300A Echivalent: B250C2300A

B125C2300B Echivalent: B250C2300B

B125C25000 Echivalent: B400C25000F

B125C3200A Echivalent: B250C3700A

B125C3200B Echivalent: B380C3700B

B125C3700A Echivalent: B250C3700A

B125C3700B Echivalent: B380C3700B

B125C5000 Echivalent: B125C5000A or B

B125C5000A Echivalent: B250C5000A

B125C5000B Echivalent: B250C5000B

B125C7000 Echivalent: B125C7000A or B

B125C7000A Echivalent: B250C7000A

B125C7000B Echivalent: B500C7000B

B125D Echivalent: B250D

B125R Echivalent: B250C1500R

B125S Echivalent: B500S

B165 Echivalent: L165V-ST

B200C10000F Echivalent: B200C15000F

B200C15000F BRIDGE RECT.SQUARE FASTON

B200C1500R Echivalent: B400C1500R

B200C25000F BRIDGE RECT.SQUARE FASTON

B200C35000F Echivalent: B400C35000F

B200C35000W Echivalent: B400C35000W

B200D BRIDGE RECT. DIL =DB103

B250C1000 Echivalent: B250C1500R

B250C10000 Echivalent: B400C15000F

B250C1500 Echivalent: B250C2300A OF B

B250C1500A Echivalent: B250C2300A

B250C1500B Echivalent: B250C2300B

B250C1500R Echivalent: B400C1500R

B250C2000 Echivalent: B250C2300A OF B

B250C2300A BRIDGE RECT. -ww+

B250C2300B BRIDGE RECT. -w+w

B250C25000 Echivalent: B400C25000F

B250C3200A Echivalent: B250C3700A

B250C3200B Echivalent: B380C3700B

B250C3700 Echivalent: B250C3700A or B

B250C3700A BRIDGE RECT. -ww+

B250C3700B Echivalent: B380C3700B

B250C5000 Echivalent: B250C5000A or B

B250C5000A BRIDGE RECT. -ww+

B250C5000B BRIDGE RECT. -w+w

B250C7000 Echivalent: B250C7000A or B

B250C7000A BRIDGE RECT. -ww+

B250C7000B Echivalent: B500C7000B

B250C800 Echivalent: B250C1500R

B250D Echivalent: B400D

B250R Echivalent: B250C1500R

B250S Echivalent: B500S

B260D Echivalent: TDA1060/DIV

B2761 Echivalent: TAA2761A

B3370V Echivalent: LM337T/DIV

B380C1000 Echivalent: B380C2300A

B380C1500 Echivalent: B380C2300A or B

B380C1500A Echivalent: B380C2300A

B380C1500B Echivalent: B380C2300B

B380C1500R Echivalent: B1000C1500R

B380C2000 Echivalent: B380C3700A

B380C2300A BRIDGE RECT. -ww+

B380C2300B BRIDGE RECT. -w+w

B380C3200A Echivalent: B380C3700A

B380C3200B Echivalent: B380C3700B

B380C3700 Echivalent: B380C3700A or B

B380C3700A BRIDGE RECT. -ww+

B380C3700B BRIDGE RECT. -w+w

B380C5000 Echivalent: B380C5000A or B

B380C5000A BRIDGE RECT. -ww+

B380C5000B Echivalent: B500C5000B

B380C7000 Echivalent: B380C7000A or B

B380C7000A Echivalent: B500C7000A

B380C7000B Echivalent: B500C7000B

B380D Echivalent: B500D

B380R Echivalent: B380C1500R

B380S Echivalent: B500S

B4002 Echivalent: UAA4002DP/DIV

B400C10000F Echivalent: B400C15000F

B400C15000F BRIDGE RECT.SQUARE FASTON

B400C1500R BRIDGE RECT. 1.5A

B400C25000F BRIDGE RECT.SQUARE FASTON

B400C35000F BRIDGE RECT.SQUARE FASTON

B400C35000W BRIDGE RECT.SQUARE WIRE

B400D BRIDGE RECT. DIL =DB104

B40C1000 Echivalent: B40C1000B

B40C1000B Echivalent: B80C2300B

B40C1000R Echivalent: B80C1500R

B40C1500A Echivalent: B40C2300A

B40C1500B Echivalent: B80C2300B

B40C1500R Echivalent: B80C1500R

B40C2300A BRIDGE RECT. -ww+

B40C2300B Echivalent: B80C2300B

B40C3200A Echivalent: B80C3700A

B40C3200B Echivalent: B80C3700B

B40C3700 Echivalent: B80C3700A or B

B40C3700A Echivalent: B80C3700A

B40C3700B Echivalent: B80C3700B

B40C5000 Echivalent: B80C5000A or B

B40C5000A Echivalent: B80C5000A

B40C5000B Echivalent: B80C5000B

B40C7000 Echivalent: B80C7000A or B

B40C7000A Echivalent: B80C7000A

B40C7000B Echivalent: B80C7000B

B40C800A Echivalent: B80C2300A

B40C800B Echivalent: B80C2300B

B40D Echivalent: B80D

B40R Echivalent: B80C1500R

B40S Echivalent: B500S

B500C1500 Echivalent: B500C2300A B

B500C1500A Echivalent: B500C2300A

B500C1500B Echivalent: B500C2300B

B500C1500R Echivalent: B1000C1500R

B500C2300A BRIDGE RECT. -ww+

B500C2300B BRIDGE RECT. -w+w

B500C3200A Echivalent: B500C3700A

B500C3200B Echivalent: B500C3700B

B500C3700A BRIDGE RECT. -ww+

B500C3700B BRIDGE RECT. -w+w

B500C5000 Echivalent: B500C5000A or B

B500C5000A BRIDGE RECT. -ww+

B500C5000B BRIDGE RECT. -w+w

B500C7000 Echivalent: B500C7000A or B

B500C7000A BRIDGE RECT. -ww+

B500C7000B BRIDGE RECT. -w+w

B500D BRIDGE RECT. DIL

B500R Echivalent: B500C1500R

B500S BRIDGE RECT. SMD

B50C10000F Echivalent: B100C15000F

B50C1500R Echivalent: B100C1500R

B50C25000F Echivalent: B100C25000F

B50C35000F Echivalent: B400C35000F

B50D Echivalent: B200D

B556D Echivalent: NE556N-MBR

B560C1500R Echivalent: B1000C1500R

B600C10000F Echivalent: B600C15000F

B600C15000F BRIDGE RECT.SQUARE FASTON

B600C1500R Echivalent: B1000C1500R

B600C25000F BRIDGE RECT.SQUARE FASTON

B600C35000F BRIDGE RECT.SQUARE FASTON

B600D Echivalent: B1000D

B615 Echivalent: TCA315A

B654D Echivalent: SN28654

B761D Echivalent: TAA761A/DIV

B800C10000F Echivalent: B1000C15000F

B800C15000F Echivalent: B1000C15000F

B800C1500R Echivalent: B1000C1500R

B800C25000F Echivalent: B1000C25000F

B800C35000F Echivalent: B1000C35000F

B800D Echivalent: B1000D

B80C1000 Echivalent: B100C1500R

B80C1500 Echivalent: B80C2300A B

B80C1500A Echivalent: B80C2300A

B80C1500B Echivalent: B80C2300B

B80C1500R Echivalent: B100C1500R

B80C2300A BRIDGE RECT. -ww+

B80C2300B BRIDGE RECT. -w+w

B80C25000 Echivalent: B400C25000F

B80C3200A Echivalent: B80C3700A

B80C3200B Echivalent: B80C3700B

B80C35000W Echivalent: B400C35000W

B80C3700 Echivalent: B80C3700A or B

B80C3700A BRIDGE RECT. -ww+

B80C3700B BRIDGE RECT. -w+w

B80C5000 Echivalent: B80C5000A or B

B80C5000A BRIDGE RECT. -ww+

B80C5000B BRIDGE RECT. -w+w

B80C7000 Echivalent: B80C7000A or B

B80C7000A BRIDGE RECT. -ww+

B80C7000B BRIDGE RECT. -w+w

B80C800 Echivalent: B80C2300A OF B

B861D Echivalent: TAA861A

BA-V1084A500F Echivalent: VP-0067

BA100 Echivalent: 1N4148-MBR

BA10324 Echivalent: LM324N-MBR

BA10339 Echivalent: LM339N-MBR

BA10358 Echivalent: LM358N-MBR

BA10393 Echivalent: LM393N-MBR

BA12003 Echivalent: ULN2003AN-MBR

BA12004 Echivalent: ULN2004AN-MBR

BA125 Echivalent: 1N5449A-MOT

BA127D Echivalent: 1N4148-MBR

BA1310F Echivalent: MC1310P-MOT

BA1320-RHM STEREO MPX DEMOD.

BA1330 Echivalent: AN7410N-MAT

BA1332-RHM STEREO DECODER

BA1332L-RHM STEREO DECODER

BA1335-RHM Echivalent: : N.L.A.

BA1350-RHM STEREO MPX+SOFT MUTING

BA1356-RHM STEREO DECODER 16pQ

BA1360-RHM STEREO DECODER

BA1404-RHM FM STEREO TRANSMITTER

BA145 Echivalent: BA159-MBR

BA147-TFK SI-D 15V 0.15A

BA148 Echivalent: BA159-MBR

BA15218-RHM OPAMP 2x 36V 10MHz 3V/us

BA15218N-RHM OPAMP 2x 36V 10MHz 3V/us

BA155 Echivalent: BAV21-MBR

BA157 Echivalent: BA159-MBR

BA158 Echivalent: BA159-MBR

BA159-DIO Echivalent: BA159-MBR

BA159-MBR SI-D 1000V 1A/35Ap 300ns

BA159/DIV Echivalent: : BA159-MBR

BA1604-RHM PLL TONE DECODER 9V 500kH

BA17805T Echivalent: UA7805-MBR

BA182 C-D 35V 0.8-2.1pF VHF

BA204 Echivalent: BAW62-PHI

BA216 Echivalent: BA315-PHI

BA220 SI-D 10V 0.2A/0.4Ap 4ns

BA222-RHM TIMER RC 18V 0.2A

BA225-RHM MONOST. MULTIVIB. 2x POS-

BA2266A-RHM RADIO SERVO CNTRL 10V

BA236-RHM DUAL RC-TYPE TIMER

BA243 C-D 20V 1-3pF 1R 0.1A

BA243G C-D 20V 1-3PF 1R 0.1A

BA244 C-D 35V 1-3pF 0R5 0.1A

BA282-MBR C-D 35V 0.9-2pF 0R7 0.1A

BA283-MBR C-D 35V 0.9-2pF 1R2 0.1A

BA301-RHM PRE-AMPLIFIER 0.15W

BA306-RHM AMPLIFIER

BA311 HIGH VOLT.PRE-AMP.7p

BA3114-RHM CAR AUDIO LINE AMPLIFIER

BA312 Echivalent: BA311

BA3122 BA3122N-RHM

BA313-RHM REC/PLAYB.PRE-AMP+ALC

BA314 Echivalent: BA314-RHM

BA314-PHI Dioda Zener 0.77V 0.25A

BA314-RHM REC/PLAYB.PRE-AMP+ALC 9p

BA315-PHI Dioda Zener 0.7V 0.25A

BA316 Echivalent: BA317

BA317 SI-D 30V 0.1A/0.2Ap 4ns

BA318-PHI Echivalent: 1N4148-MBR

BA329 Echivalent: LA3160-SAN

BA328 Echivalent: LA3161-SAN

BA3306-RHM NF AMPLIFIER

BA3308-RHM DUAL AMPL.+ EQUAL. + ALC.

BA331 Echivalent: CA3046-MBR

BA3312N-RHM PREAMP ALC REC/PLAY BACK

BA333-RHM REC.PLAYB.PRE-AMP.+ALC

BA335-RHM AUDIO LEVEL Senzor

BA336-RHM TAPE RECORD-FINDER 9p

BA337-RHM AUTO REVERSE CONTROLER

BA338-RHM BLANK SECTION DETECTOR

BA340 PRE-AMPLIFIER 16p

BA3402-RHM 2x RECORDER AUTOREVERSE

BA3406AL-RHM DUAL PRE-AMP

BA3410AF-RHM RECORDER CIRCUIT

BA3416BL-RHM DUAL PRE-AMP REC/P.B.

BA3420AL-RHM RECORDER, 2xA/W AMPLIFIER

BA3422S-RHM RECORD/PLAYBACK AMPLIFIER

BA343-RHM DUAL AMP.+ ALC

BA3505F-RHM DUAL PRE/POWER AMPL

BA3506A-RHM DUAL PRE/POWER AMPL

BA3516-RHM 3V POWER PRE-AMP

BA3518F-RHM POWER AMP. FOR HEADPHONE

BA3520-RHM 3V DUAL PREPOWER AMPL.

BA3521-RHM AMPLIFIER

BA3704 RECORDER MEMORY

BA3706-RHM RECORDER MEMORY 16p

BA3812L-RHM 5 FREQ GRAPHIC EQUALIZER

BA3822LS-RHM GRAPHIC EQUALIZER

BA3824 Echivalent: BA3824LS-RHM

BA3824LS-RHM GRAPH EQUAL STEREO 5 pnts

BA3900-RHM PWR SUPPLY FOR CAR 4-OUTP

BA3904A-RHM PWR SUPPLY FOR CAR, 4-OUT

BA3906-RHM REGULATOR FOR CAR AUDIO

BA3910B-RHM SYSTEM POWER SUPPLY

BA3913-RHM V-REG 6-OUTPUTS

BA3918-RHM 6xPOS V-REG+BACKUP MODE

BA3920-RHM GENERAL REF. FOR RADIO

BA3924-RHM 4XPOS V-REG. VIN 5-22V

BA3930-RHM I.C.

BA3932-RHM 6xPOS V-REG Vin=6.5-22V

BA3933-RHM 6xPOS V-REG Vin=6.5-22V

BA3936-RHM 5xPOS V-REG Vin=6.5-22V

BA3960-RHM V-REG, DUAL CIRCUIT, VAR.

BA401 AM IF AMPLIFIER

BA402 FM IF AMPLIFIER 7p

BA403 Echivalent: TA7130P-TOS

BA4110-RHM FM-ZF+AFC+AGC

BA4112-RHM FM-IF AMPLIFIER

BA4113N-RHM FM-IF SYSTEM

BA4210 AM/FM IF AMPLIFIER

BA4220 AM/FM-ZF+DEMOD. Ucc=3..14

BA4232AL-RHM Echivalent: : N.L.A.

BA4234L FM/AM IF SYSTEM

BA4236L-RHM AM FM IF TAPE REC.

BA4402-RHM FM-HF

BA4403-RHM FM FRONT END

BA4404-RHM FM-FRONT END+AFC 1-9V

BA4405-RHM FM FRONT/END

BA4410-RHM I.C.

BA4412-RHM RADIO CONSUMER FM/AM

BA4422 Echivalent: LA1186N-SAN

BA4558 Echivalent: MC4558CN-MBR

BA4560-RHM OPAMP 2x 36V 10MHz 4V/us

BA479-TFK SI-D 30V 50mA 100MHz

BA481-PHI SI-D 4V 30mA 13R 1.1pF

BA482 SI-D 35V 0.1A 0.7R 1.2PF

BA5101 REC. INPUT AUTO LEVEL CTR

BA5102 Echivalent: BA5102AP-RHM

BA5102A Echivalent: BA5102AP-RHM

BA5102AP-RHM SWITCHLES REC/PLAY AMP.

BA5104A-RHM SWITCHLES REC/PLAY.AMP.16

BA511 Echivalent: BA511A-RHM

BA5112-RHM VCR, AUDIO RECORD/PLAYBAC

BA5115-RHM VTR AUDIO REC/PLAY SYST.

BA5115L-RHM VTR AUDIO REC/PLAY SYST.

BA511A-RHM 5.8W POWER AMP.

BA514 Echivalent: : N.L.A.

BA516 Echivalent: BA526-RHM

BA518 1.5W/9V POWER AMP. 8p

BA5204-RHM 3V. DUAL POWER AMP.

BA5208AF-RHM I.C.

BA521-RHM 5.8W POWER AMP.

BA524 Echivalent: : N.L.A.

BA526-RHM 700mW/6V. POWER AMP.

BA527-RHM 800mW/6V.POWER AMP.

BA532 Echivalent: : N.L.A.

BA534-RHM 2.3W/9V. POWER AMP. 10p

BA536 Echivalent: BA5406-RHM

BA5402A Echivalent: : N.L.A.

BA5404-RHM HEADPHONE AMPLIFIER 12V

BA5406-RHM 5W/12V. DUAL POW.AMP.

BA5410-RHM DUAL 5.2W POWER AMPLF.12V

BA5412-RHM DUAL 1.5W POWER AMPLIF.9V

BA5413-RHM DUAL 5.4W POWER AMPLIF.12

BA5414 Echivalent: : N.L.A.

BA5415A-RHM PWR AMP 1*5.4W/3E 18V

BA5417-RHM PWR AMP 2X1.3W/8E 9V

BA546-RHM 550mW/6V. POWER AMP. 12p

BA547 Echivalent: BA518

BA6104-RHM LED LIN.METER(5 LEDS)

BA6109-RHM MOTOR DR. 3-MODE 18V 0.8A

BA6110-RHM OPAMP V-CNTRL 34V

BA612-RHM NPN DARL DR. 5x 30V 0.4A

BA6122A-RHM SW-REG 5V+9V 41kHz

BA6124 Echivalent: AN6884-MAT

BA6125-RHM 5 SEGM. BAR GRAPH DR. LIN

BA6129AF-RHM MEMORY BACK-UP I.C.

BA6137-RHM LED VU METER (5 LEDS) 9p

BA6138 SQUARE-LAW COMPR. AMPL.

BA614-RHM CURRENT DR. 6x 24V 0.1A

BA6144 Echivalent: BA6137-RHM

BA6146-RHM BARGR./DISPLAY DRIVER

BA6149LS-RHM 6-CHAN. SWITCHING REG.

BA6154-RHM LED VU METER (5 LED's)

BA6161N-RHM SMPS IC, V-MODE SINGLE OU

BA618-RHM LED DRIVER 7x 16V 0.1A

BA6191-RHM LOADING MOTOR DRIVER 2.2W

BA6208-RHM MOTOR LCD DRIVER

BA6209 Echivalent: BA6209-RHM

BA6209-RHM MOTOR DR. 4-MODE 18V 1.6A

BA6209N Echivalent: : BA6209-RHM

BA6209U Echivalent: BA6209-RHM

BA6218-RHM MOTOR DR. 4-MODE 18V 0.7A

BA6219B-RHM MOTOR DR. 4-MODE 24V 2.2A

BA6219BFP-RHM MOTOR DR. 4-MODE 24V 2.2A

BA6220-RHM MOTOR CONTROLER

BA6222-RHM REVERSIBLE MOTOR DRIVER

BA6227-RHM CONTROL SP. MT 3V REC.

BA6229-RHM MOTOR DR. 4-MODE 24V 1.2A

BA6235-RHM 3V ELECTR SUPERVISOR

BA6238A-RHM MOTOR DR. 2x REV. 20V 1.6

BA6239A-RHM MOTOR DR. 2x REV. 20V 1.2

BA6247 Echivalent: BA6247H-RHM or N-R

BA6247H-RHM 2 REV. MOT.DR. + COOLING

BA6247N-RHM 2 REV. MOT. DR. NO COOLIN

BA6248 3-INP BI-DIR MOTOR DRIVER

BA6259N-RHM 2 CH.REVERSE MOTOR DRIVER

BA6280AF SMD DRIVER MOTOR PWM CD

BA6286-RHM DC MOTOR DRIVER 1A 2W

BA6290A-RHM POWER DRIVER

BA6294-RHM CD/CD-ROM DRIVER I.C.

BA6296FP-RHM CD-ROM DRIVER

BA6297FP-RHM SERVO CIRCUIT MOTOR DRIVE

BA6298FP-RHM CD/CD-ROM QUAD POWER DRVR

BA6301-RHM VELOCITY SERVO 16p

BA6302A-RHM VELOCITY SERVO 16p

BA6303-RHM VELOCITY SERVO

BA6303F-RHM VELOCITY SERVO

BA6304 WIRE REMOTE CONTROL

BA6305-RHM VELOCITY SERVO 8p

BA631-RHM FM INTERCOM 10V

BA6321 CNTRL SERVO SP VTR

BA6328 I.C.

BA6334-RHM 16 MODE CTV/VCR DETECTOR

BA6353S-RHM SERVO DRIVER

BA6397FP-RHM CD/CD-ROM QUAD PWR DRIVER

BA6398FP-RHM QUAD CD POWER DRIVER 1.7W

BA6405-RHM D.C. MOTOR DRIVER

BA6410-RHM MOTOR DR. 2-PHASE 20V 1.2

BA6411-RHM MOTOR DR. 2-PHASE 20V 1.2

BA6413-RHM 2-PHASE MOTOR DRIVER

BA6418N-RHM DRIVER MOTOR REVERSIBLE

BA6430S-RHM I.C.

BA6432S-RHM 3 PHASE MOTOR DRIVER

BA6435S-RHM 3 PH MOTOR DRIVER

BA6439P-RHM MOTOR DRIVER

BA6459FS-RHM 3-PHASE FULL WAVE MOTOR D

BA656 LEDBAR DRIVER 5CH 9p

BA6565A-RHM TELEPHONE TONE RINGERS

BA6566-RHM SPEECH NETWORK

BA658 VU-MTR BAR LEVEL DRIV.16p

BA664-RHM TRANS.ARRAY 6x 20V 0.1A

BA668A Echivalent: 209.82537

BA681A-RHM LED LIN.METER(12 LEDS)18p

BA682 Echivalent: BA682-DIODE

BA682-DI Echivalent: BA682-DIODE

BA682-DIODE C-D 35V 0.9-2pF 0R7 0.1A

BA682A-RHM LEDBAR DRIVER 12CH

BA683A-RHM LED METER (12 LEDS) 18p

BA683FILM Echivalent: BA683

BA6886-RHM DC MOTOR DRIVER

BA6887-RHM IC.

BA6887V3 GOTO: BA6887-MBR

BA695-RHM AM/FM LED/LCD-METER 3 LED

BA6993-RHM Echivalent: LM393N-MBR

BA6993N-RHM DUAL COMPARATOR SIL

BA7001-RHM VC ANALOG SWITCH

BA7004-RHM TEST PATTERN GENERATOR 5p

BA7007-RHM PAL SECAM DISCRIMINATOR

BA7021-RHM LINEAIR SPEC. FUNCTION

BA7025L-RHM SECAM DISCRIMINATOR

BA7026L-RHM VIDEO/AUDIO SIGNAL SWITCH

BA707-RHM POS V-REG 3V3 10mA 1mV/'C

BA7100-RHM SECAM DISCRIMINATOR

BA7107 SECAM CHROMA PROCESSOR

BA7107S-RHM SECAM CHROMA PROCESSOR

BA715-RHM OPAMP 2x 36V LOW POWER

BA718-RHM OPAMP 2x 18V LOW POWER

BA7200-RHM I.C.

BA7212S-RHM VIDEO SIGNAL PB/REC AMPL

BA7252S-RHM Echivalent: : N.L.A.

BA7253S AMP VID SIG PB REC

BA7254S-RHM VIDEO SIGNAL AMPLIFIER

BA7255-RHM I.C.

BA7258AS-RHM Y-SIGNAL PROCESSOR

BA7274S-RHM PRE/REC AMPLIFIER

BA728-RHM OPAMP 2x 18V LOW POWER

BA728N-RHM OPAMP 2x 18V LOW POWER

BA7604N-RHM VIDEO SIGNAL SWITCHER

BA7630S-RHM VIDEO C+ DECODER

BA7631-RHM SWITCH&SCRAMBLED SIGNAL D

BA7631F-RHM SWITCH&SCRAMBLED SIGNAL D

BA7746S-RHM PRE/REC AMPLIFIER

BA7751ALS-RHM VC-A/W AMPL SWITCHLESS

BA7752LS-RHM Echivalent: : N.L.A.

BA7755A-RHM VCR AUDIO HEAD SWITCH

BA7765AS-RHM NORMAL AUDIO SIGNAL PROC.

BA7766AS-RHM VTR AUDIO SIGNAL PROC.

BA7767AS-RHM NORMAL AUDIO SIGNAL PROC.

BA7795LS-RHM VCR AUDIO PROCESSOR

BA7797F-RHM AUDIO SIGNAL PROCESSOR

BA806 PLL MOTOR CONTROLER

BA8204 Echivalent: ML8204

BA8205-RHM Echivalent: ML8205-MBR

BA8420-RHM VC SERVO CNTRL SP. PLAYBA

BA843-RHM Echivalent: : N.L.A.

BA8500-RHM CAPSTAN-SERVO DRIVER

BA8504 Echivalent: : N.L.A.

BAR10 SB-D 20V 35MA/100MAP

BAR43S 2xSB-D 30V 0.1A/0.75Ap 5n

BAS11-MBR SI-D 300V 0.3A/4Ap 1us

BAS16-MBR SI-D 75V 0.25A 6ns A6

BAS19 Echivalent: BAS21-MBR

BAS20 Echivalent: BAS21-MBR

BAS21-MBR SI-D 250V 0.2A <50ns

BAS32L-PHI Echivalent: LL4148-MBR

BAS33 SI-D 40V 0.1A

BAT.CL.SN-4 Echivalent: BAT.CL.SN-2

BAT.CL.SN-8 Echivalent: BAT.CL.SN-2

BAT17-MBR SB-D 4V 30mA STKY 15R 1pF

BAT41-MBR SB-D 100V 0.1A

BAT42 Echivalent: BAT85-MBR

BAT43 Echivalent: BAT85-MBR

BAT46-MBR SB-D 100V 0.15A

BAT47 Echivalent: BAT48-MBR

BAT48-MBR SB-D 40V 0.35A 10ns

BAT49-MBR SB-D 80V 0.5A

BAT54A-MBR 2xSB-D 30V 0.2A

BAT82-MBR SB-D 50V 30mA STKY <1ns

BAT83-MBR SB-D 60V 3mA STKY <1nS

BAT85-MBR SB-D 30V 0.2A STKY 10pF

BAT86-PHI SB-D 50V 0.2A STKY 8pF =

BAV10-MBR SI-D 60V 0.3A/0.6Ap 6ns

BAV100 Echivalent: BAV103-MBR

BAV101 Echivalent: BAV103-MBR

BAV102 Echivalent: BAV103-MBR

BAV103-MBR SI-D 250V 0.2A 5pF 50ns

BAV17 Echivalent: BAV18-MBR

BAV18-MBR SI-D 50V 0.2A/.05Ap 50ns

BAV19 Echivalent: BAV21-MBR

BAV20-MBR Echivalent: BAV21-MBR

BAV21-MBR SI-D 200V 0.2A/0.5Ap 50ns

BAV23-MBR 2xSI-D 250V 0.2A <50nS

BAV70-MBR 2xSI-D 70V 0.2A 6ns A4

BAV99-MBR 2xSI-D 70V 0.2A 6ns A7

BAW27-MBR Echivalent: 1N 4148-MBR

BAW56-PHI 2xSI-D 70V 0.2A 6ns A1

BAW62-PHI SI-D 75V 0.2A/0.45Ap 4ns

BAW75-TFK SI-D 35V 0.15A/2Ap 2ns

BAW76-TFK SI-D 75V 0.15A/2Ap 2ns

BAX12-MBR SI-D 90V 0.4A/0.8Ap 50ns

BAX13 SI-D 50V 0.15A 0.5W 4ns

BAX14 Echivalent: BAX18-MBR

BAX16 Echivalent: BAV21-MBR

BAX17 SI-D 200V 0.2A <120ns

BAX18-MBR SI-D 75V 0.4A/2Ap

BAY21 SI-D 350V 0.25A 1us

BAY46 Echivalent: BA159-MBR

BB103 C-D 30V 11.3-30pF

BB104 Echivalent: BB204B-MBR

BB105B C-D 30V 2-18pF 0R8 UHF

BB105G C-D 30V 1.8-18pF 1R2 UHF

BB106 C-D 30V 4-20pF 0R4 VHF

BB109G C-D 30V 4.3-32pF 0R5 VHF

BB109G-TAPE Echivalent: BB109G

BB112-PHI C-D 12V 17-560pF AM TUNIN

BB119 C-D 10V 15.3-19pF 1E5

BB121A-MBR C-D 30V 1.9-18pF 0.6R

BB122 Echivalent: BB121A-MBR

BB139 C-D 30V 4.3-29pF 0.5R

BB142 C-D 30V 12pF FM/VHF

BB204B-MBR Echivalent: : N.L.A.

BB204G-MBR 2xC-D 30V 14-39pF 0R4 GRE

BB205B C-D 30V 11pF

BB205G C-D 30V 1.8-17pF 1R2 UHF

BB207 C-D TUNING DIODE

BB209-MBR C-D 30V 2.6-31pF 0R85

BB212-PHI Echivalent: : N.L.A.

BB221 C-D 28V 1.8-17pF 0R6 470M

BB222 Echivalent: BB405B-PHI

BB304G C-D 2V 1.7-46pF GREEN

BB329 Echivalent: BB529-ITT

BB405B-PHI C-D 30V 1.8-18pF 0.75R

BB405G-PHI Echivalent: BB405B-PHI

BB409-SIE C-D 28V 5-32pF 0.4R VHF

BB505B Echivalent: BB405B-PHI

BB505G Echivalent: BB405G-PHI

BB529-ITT C-D 28V 2.5-12PF 0R85 300

BB609A Echivalent: BB609B

BB804-MBR 2xC-D 18V 42-46.5pF 0.2E

BB809-MBR C-D 28V 4-46pF VHF-TUNING

BB811-MBR Echivalent: : N.L.A.

BC107 Echivalent: BC107B-MBR

BC107A Echivalent: BC107B-MBR

BC107B Echivalent: BC107B-MBR

BC107B-MBR SI-N 50V 0.2A 0.3W 250MHz

BC107B-PHI Echivalent: BC107B-MBR

BC107B/DIV SI-N 50V 0.2A 0.3W 250MHz

BC107C SI-N 50V 0.2A 0.3W 250MHz

BC108 Echivalent: BC107C

BC108A Echivalent: BC107B-MBR

BC108B Echivalent: BC107B-MBR

BC108C Echivalent: BC107C

BC109 Echivalent: BC109B-CDL

BC109B-CDL Echivalent: BC109B-MBR

BC109B-MBR SI-N 30V 0.2A 0.3W 300MHz

BC109B/DIV SI-N 30V 50mA 0.3W 300MHz

BC109C-MBR SI-N 30V 0.2A 0.3W 150MHz

BC109C/DIV Echivalent: : BC109C-MBR

BC117 SI-N 120V 50mA 0.3W >60MH

BC119 SI-N 60V 1A 0.8W 10MHz

BC125 Echivalent: BC337-25-MBR

BC129 Echivalent: BC547B-MBR

BC130 Echivalent: BC547B-MBR

BC135 SI-N 45V 0.2W >200MHz

BC136 SI-N 60V 0.5A 0.3W >60MHz

BC137 Echivalent: BC327-25-MBR

BC138 Echivalent: BC141-10-CDL

BC139-MBR SI-P 40V 0.5A 0.7W

BC140-10 Echivalent: BC141-10-CDL

BC140-16 Echivalent: BC141-16-MBR

BC141-10-MBR SI-N 100V 1A 0.75W 50MHz

BC141-10/DIV SI-N 100V 1A 0.65W 50MHz

BC141-16-MBR SI-N 100V 1A 0.75W 50MHz

BC141-16/DIV SI-N 100V 1A 0.65W 50MHz

BC141-6 Echivalent: BC141-10-CDL

BC142 SI-N 80V 1A 0.8W

BC143 SI-P 60V 1A 0.7W AF/DRIVE

BC146-PHI SI-N 20V 50mA 50mW 150MHz

BC146/DIV SI-N 20V 0.1A 50mW 150MHz

BC147A Echivalent: BC547A-MBR

BC147B Echivalent: BC547B-MBR

BC148B Echivalent: BC547B-MBR

BC148C Echivalent: BC547C-MBR

BC149C Echivalent: BC550C-MBR

BC157B Echivalent: BC557B-MBR

BC158C Echivalent: BC557C-MBR

BC159 Echivalent: BC560B-MBR

BC159B Echivalent: BC560B-MBR

BC159C Echivalent: BC560C-MBR

BC160-10 Echivalent: BC161-16-MBR

BC160-16 Echivalent: BC161-16-MBR

BC161-10 Echivalent: BC161-16-MBR

BC161-16-MBR SI-P 60V 1A 0.75W 50MHz

BC161-16/DIV SI-P 60V 1A 0.65W 50MHz

BC161-6 Echivalent: BC161-16-MBR

BC167B Echivalent: BC547B-MBR

BC167C Echivalent: BC547C-MBR

BC168B Echivalent: BC547B-MBR

BC168C Echivalent: BC547C-MBR

BC169 Echivalent: BC550B-MBR

BC170A Echivalent: BC547A-MBR

BC170B Echivalent: BC547B-MBR

BC170C Echivalent: BC547C-MBR

BC171A Echivalent: BC547A-MBR

BC171B Echivalent: BC547B-MBR

BC172A Echivalent: BC547A-MBR

BC172B Echivalent: BC547B-MBR

BC172C Echivalent: BC547C-MBR

BC173C Echivalent: BC550C-MBR

BC174A Echivalent: BC546A-MBR

BC174B Echivalent: BC546B-MBR

BC177A-MBR SI-P 50V 0.1A 0.3W 130MHz

BC177A/DIV SI-P 50V 0.1A 0.3W 130MHz

BC177B-MBR SI-P 50V 0.1A 0.3W 130MHz

BC177B/DIV SI-P 50V 0.1A 0.3W 130MHz

BC177C SI-P 50V 0.1A 0.3W 130MHz

BC178B Echivalent: BC177B/DIV

BC178C Echivalent: BC177C

BC179B Echivalent: BC177B/DIV

BC179C Echivalent: BC178C

BC181 Echivalent: BC557B-MBR

BC182A Echivalent: BC546A-MBR

BC182B Echivalent: BC546B-MBR

BC182C Echivalent: BC546C-MBR

BC182L Echivalent: BC546A-MBR

BC183A Echivalent: BC547A-MBR

BC183B Echivalent: BC547B-MBR

BC183C Echivalent: BC547C-MBR

BC184B Echivalent: BC550B-MBR

BC184C Echivalent: BC550C-MBR

BC184L Echivalent: BC550C-MBR

BC186 Echivalent: BC557B-MBR

BC187 Echivalent: BC557B-MBR

BC190 SI-N 70V 0.1A 0.3W 250MHz

BC192 Echivalent: BC328-25-ITT

BC204 Echivalent: BC557B-MBR

BC204B Echivalent: BC557B-MBR

BC205 Echivalent: BC557B-MBR

BC207 Echivalent: BC547A-MBR

BC208 Echivalent: BC547B-MBR

BC209 Echivalent: BC550B-MBR

BC211A Echivalent: : N.L.A.

BC212 Echivalent: BC556B-MBR

BC212B Echivalent: BC556B-MBR

BC212L Echivalent: BC556B-MBR

BC213B Echivalent: BC557B-MBR

BC213C Echivalent: BC557C-MBR

BC214B Echivalent: BC560B-MBR

BC214C Echivalent: BC560C-MBR

BC231 Echivalent: BC327-16-MBR

BC232 Echivalent: BC337-25-MBR

BC237A Echivalent: BC547B-MBR

BC237B Echivalent: BC547B-MBR

BC237C Echivalent: BC547C-MBR

BC238A Echivalent: BC547A-MBR

BC238B Echivalent: BC547B-MBR

BC238C Echivalent: BC547C-MBR

BC239B Echivalent: BC550B-MBR

BC239C Echivalent: BC550C-MBR

BC243C Echivalent: BD243C-MBR

BC244C Echivalent: BD244C-MBR

BC245C-MBR Echivalent: TIP33C-MBR

BC250A Echivalent: BC560B-MBR

BC250B Echivalent: BC560B-MBR

BC250C Echivalent: BC560C-MBR

BC251 Echivalent: BC557C-MBR

BC251A Echivalent: BC557A-MBR

BC251B Echivalent: BC557B-MBR

BC251C Echivalent: BC557C-MBR

BC252A Echivalent: BC557A-MBR

BC252B Echivalent: BC557B-MBR

BC252C Echivalent: BC557C-MBR

BC255 Echivalent: BC337-25-MBR

BC256 Echivalent: BC556B-MBR

BC257B Echivalent: BC557B-MBR

BC258 Echivalent: BC547B-MBR

BC259A Echivalent: BC560B-MBR

BC259C Echivalent: BC560B-MBR

BC260 Echivalent: BC557C-MBR

BC261 Echivalent: BC557B-MBR

BC264A Echivalent: BF245C-MBR

BC264B Echivalent: BF245B-MBR

BC264C Echivalent: BF245B-MBR

BC264D Echivalent: BF245B-MBR

BC266 Echivalent: BC557B-MBR

BC267 Echivalent: BC337-25-MBR

BC268 Echivalent: BC337-25-MBR

BC285 SI-N 120V 0.1A 0.36W 80MH

BC286 Echivalent: BC141-16-MBR

BC287 Echivalent: BC161-16-MBR

BC297 Echivalent: BC327-16-MBR

BC300-CDL SI-N 120V 0.5A 6W 120MHz

BC300/DIV SI-N 120V 0.5A 6W 120MHz

BC301 Echivalent: BC300-CDL

BC302 Echivalent: BC300-CDL

BC303-CDL SI-P 85V 1A 6W 75MHz

BC303/DIV SI-P 85V 1A 0.85W

BC304 Echivalent: BC303/DIV

BC307 Echivalent: BC557A-MBR

BC307A Echivalent: BC557A-MBR

BC307B Echivalent: BC557B-MBR

BC307C Echivalent: BC557C-MBR

BC308 Echivalent: BC560B-MBR

BC308A Echivalent: BC560B-MBR

BC308B Echivalent: BC560B-MBR

BC308C Echivalent: BC557C-MBR

BC309B Echivalent: BC560B-MBR

BC309C Echivalent: BC557C-MBR

BC313 SI-P 60V 1A 4W 50MHz

BC315 Echivalent: BC557B-MBR

BC317 Echivalent: BC547B-MBR

BC318 Echivalent: BC547B-MBR

BC318C Echivalent: BC547C-MBR

BC319 Echivalent: BC550B-MBR

BC320 Echivalent: BC557B-MBR

BC323 SI-N 100V 5A 0.8W 100MHz

BC325 Echivalent: BC640-MBR

BC326 Echivalent: BC640-MBR

BC327 Echivalent: BC327-16 -25 -40

BC327-16-MBR SI-P 50V 0.8A 625mW 100MH

BC327-25-MBR SI-P 50V 0.8A 625mW 100MH

BC327-40-MBR SI-P 50V 0.8A 625mW 100MH

BC328 Echivalent: BC328-16 -25 -40

BC328-16-ITT Echivalent: BC327-16-MBR

BC328-25-ITT Echivalent: BC328-25-ROL

BC328-25-ROL Echivalent: : BC327-25-MBR

BC328-40-ITT Echivalent: BC327-40-MBR

BC330 Echivalent: BC329

BC336 SI-P 25V 50mA 0.31W 50MHz

BC337 Echivalent: BC337-16 -25 -40

BC337-16-MBR SI-N 50V 0.8A 625mW 150MH

BC337-25-MBR SI-N 50V 0.8A 625mW 150MH

BC337-40-MBR SI-N 50V 0.8A 0.625W 150M

BC337-40-TOS Echivalent: BC337-40-MBR

BC338 Echivalent: BC338-16 -25 -40

BC338-16-ITT Echivalent: BC337-16-MBR

BC338-25-ITT Echivalent: BC337-25-MBR

BC338-40 Echivalent: BC337-40-MBR

BC340 Echivalent: BC141-10-CDL

BC341 Echivalent: BC141-10-CDL

BC345 Echivalent: BC640-MBR

BC347 Echivalent: BC546B-MBR

BC348 Echivalent: BC546B-MBR

BC349 Echivalent: BC546B-MBR

BC350 Echivalent: BC557B-MBR

BC351 Echivalent: BC557B-MBR

BC360 Echivalent: BC161-16-MBR

BC361 Echivalent: BC161-16-MBR

BC368-MBR SI-N 20V 1A 0.8W 100MHz

BC369-MBR SI-P 20V 1A 0.8W

BC369-PHI Echivalent: 130.44593

BC372 Echivalent: BC879-MBR

BC373 Echivalent: BC879-MBR

BC375 Echivalent: 130.60491

BC376-PHI SI-P 25V 1A 0.625W 150MHz

BC382 Echivalent: BC550B-MBR

BC383 Echivalent: BC550B-MBR

BC384 Echivalent: BC550B-MBR

BC393-CDL SI-P 180V 10mA 40mW

BC407 Echivalent: BC547B-MBR

BC394 Echivalent: BF299-MBR

BC408 Echivalent: BC547B-MBR

BC413B Echivalent: BC550B-MBR

BC414B Echivalent: BC550B-MBR

BC414C Echivalent: BC550C-MBR

BC415A Echivalent: BC560A-MBR

BC415B Echivalent: BC560B-MBR

BC415C Echivalent: BC560C-MBR

BC416B Echivalent: BC560B-MBR

BC416C Echivalent: BC560C-MBR

BC417 Echivalent: BC557B-MBR

BC431 Echivalent: BC639-MBR

BC432 Echivalent: BC640-MBR

BC440 Echivalent: BC639-MBR

BC441 SI-N 75V 2A 1W

BC447 Echivalent: BC449-MBR

BC448-MBR SI-P 80V 0.3A 0.625W >100

BC449-MBR SI-N 100V 0.3A 0.625W

BC450 SI-P 100V 0.3A 0.625W

BC451 SI-N 50V 0.1A 0.3W >150MH

BC460 Echivalent: BC461

BC477 Echivalent: BC556B-MBR

BC461 SI-P 75V 2A 1W

BC478 Echivalent: BC557B-MBR

BC479 Echivalent: BC557B-MBR

BC485-MBR SI-N 45V 1A 0.625W 200MHz

BC487B-MBR SI-N 60V 1A 0.625W 200MHz

BC488 SI-P 60V 0.1A 625mW >135M

BC489-MBR SI-N 80V 1A 0.625W 200MHz

BC490-MBR SI-P 80V 1A 0.625W 200MHz

BC516-MBR P-DARL 40V 0.4A 0.625W

BC517-MBR N-DARL 40V 0.4A 0.625W

BC527 Echivalent: BC640-MBR

BC528 Echivalent: BC640-MBR

BC536 TRANSISTOR

BC537 Echivalent: BC538

BC538 SI-N 80V 1A 0.625W 100MHz

BC546A-MBR SI-N 80V 0.2A 0.5W

BC546B-MBR SI-N 80V 0.2A 0.5W

BC546C-MBR SI-N 80V 0.1A 0.5W

BC546C/DIV Echivalent: BC546C-MBR

BC547A-MBR SI-N 50V 0.2A 0.5W

BC547B-MBR SI-N 50V 0.2A 0.5W 300MHz

BC547C-MBR SI-N 50V 0.2A 0.5W 300MHz

BC548A-ITT Echivalent: BC547A-MBR

BC548B-ITT Echivalent: BC547B-MBR

BC548C-ITT Echivalent: BC547C-MBR

BC549B-ITT Echivalent: BC550B-MBR

BC549C-ITT Echivalent: BC550C-MBR

BC549C-TOS Echivalent: BC550C-MBR

BC550B-MBR SI-N 50V 0.2A 0.5W

BC550C-MBR SI-N 50V 0.2A 0.5W

BC556A-MBR SI-P 60V 0.2A 0.5W

BC556B-MBR SI-P 80V 0.2A 0.5W

BC557A-MBR SI-P 50V 0.2A 0.5W

BC557B-MBR SI-P 50V 0.2A 0.5W

BC557C-MBR SI-P 50V 0.2A 0.5W

BC558A-ITT Echivalent: BC557A-MBR

BC558B-ITT Echivalent: BC557B-MBR

BC558C-ITT Echivalent: BC557C-MBR

BC559A Echivalent: BC560B-MBR

BC559B Echivalent: BC560B-MBR

BC559C Echivalent: BC560C-MBR

BC560 Echivalent: BC560B-MBR

BC560A-MBR Echivalent: : BC560B-MBR

BC560B-MBR SI-P 50V 0.2A 0.5W

BC560C-MBR SI-P 50V 0.2A 0.5W

BC582 Echivalent: BC337-25-MBR

BC617 Echivalent: BC618-MBR

BC618-MBR N-DARL 80V 1A 0.625W B>10

BC635 Echivalent: BC639-MBR

BC636 Echivalent: BC640-MBR

BC637 Echivalent: BC639-MBR

BC639-MBR SI-N 80V 1A 0.8W 100MHz

BC638 Echivalent: BC640-MBR

BC640-MBR SI-P 80V 1A 0.8W 130MHz

BC728 Echivalent: BC640-MBR

BC807 Echivalent: BC807-25-MBR

BC807-25-MBR SI-P 50V 0.5A 0.25W 5B

BC807-40-MBR SI-P 45V 0.5A 0.3W 100MHz

BC808-25 Echivalent: BC807-25-MBR

BC808-40 Echivalent: BC807-40-MBR

BC817-16-MBR SI-N 50V 0.5A 0.25W 6A

BC817-25-MBR SI-N 50V 0.5A 0.25W 6B

BC817-40-MBR SI-N 50V 0.5A 0.25W 6C

BC818 Echivalent: BC817-16-MBR

BC818-16 Echivalent: BC817-16-MBR

BC818-25 Echivalent: BC817-25-MBR 6F

BC818-40 Echivalent: BC817-40-MBR

BC828 SI-P 50V 0.8A 0.8W 100MHz

BC846-MBR Echivalent: BC846B-MBR

BC846B-MBR SI-N 80V 0.1A 0.25W 1B

BC847 Echivalent: BC847B-MBR

BC847A-MBR SI-N 50V 0.1A 0.2W

BC847B-MBR SI-N 50V 0.1A 0.25W 1F

BC847BR-MBR SI-N 50V 0.1A 0.25W SMD

BC847C-MBR SI-N 50V 0,1A 0.25W 1G

BC848 Echivalent: BC848B-MBR

BC848A Echivalent: BC848B-MBR

BC848B-MBR Echivalent: : BC847B-MBR

BC848C Echivalent: BC847C-MBR

BC848C-SMD Echivalent: BC847C-MBR

BC849B Echivalent: BC849C-MBR

BC849C-MBR SI-N 30V 0.1A 0.25W 2C

BC850B Echivalent: BC850C-MBR

BC850C-MBR SI-N 45V 0.1A 0.25W 2G

BC856A-MBR SI-P 65V 0.1A 150MHz 3A

BC856B-MBR SI-P 65V 0.1A 150MHz 3B

BC857 Echivalent: BC857B-MBR

BC857A-MBR SI-P 50V 0.1A 150MHz

BC857B-MBR SI-P 45V 0.1A 0.2W 3F

BC857BR-MBR SI-P 45V 0.1A 0.2W

BC857C-MBR SI-P 45V 0.1A 0.25W 3G

BC858A Echivalent: BC857A-MBR

BC858B Echivalent: BC857B-MBR

BC858BR Echivalent: BC857BR-MBR

BC858C-MBR Echivalent: BC857C-MBR

BC858SMD Echivalent: BC857C-MBR

BC859A Echivalent: BC859B-MBR

BC859B-MBR SI-P 30V 0.1A 0.25W 4B

BC859C Echivalent: BC860C-MBR

BC860A Echivalent: BC860C-MBR

BC860B SI-P 50V 0.1A 4F

BC860C-MBR SI-P 50V 0.1A 4G

BC868-MBR SI-N 25V 1A 60MHz

BC869-PHI SI-P 25V 1A 1W 60MHz

BC869C Echivalent: BC869-PHI

BC875 Echivalent: BC879-MBR

BC876-MBR Echivalent: BC880-MBR

BC877 Echivalent: BC879-MBR

BC878 Echivalent: BC880-MBR

BC879-MBR N-DARL 100V 1A 0.8W

BC880-MBR P-DARL 100V 1A 0.8W

BCP68-MBR SI-N 20V 1A 1.5W 60MHz

BCV27-MBR N-DARL 40V 0.5A 0.25W B>1

BCW60A Echivalent: BC848B-MBR

BCW60B Echivalent: BC848B-MBR

BCW60C Echivalent: BC848B-MBR

BCW60D Echivalent: BC847C-MBR

BCW61A Echivalent: BC857B-MBR

BCW61B Echivalent: BC857B-MBR

BCW61C Echivalent: BC857B-MBR

BCW61D Echivalent: BC857C-MBR

BCW91 Echivalent: : N.L.A.

BCX17-MBR SI-P 50V 0.5A 100MHz T1

BCX17R-MBR SI-P 50V 0.5A 100MHZ T4

BCX19-MBR SI-N 50V 0.5A 300mW 200MH

BCX38B-MBR N-DARL 80V 0.8A 1W B>4000

BCX51 Echivalent: BDX53-MBR

BCX52 Echivalent: BCX53-MBR

BCX53-MBR SI-P 100V 1A 50MHZ

BCX56-MBR SI-N 100V 1A 130MHz

BCX70 Echivalent: BC847B-MBR

BCX71 Echivalent: BC856B-MBR

BCY56-PHI Echivalent: : BC107B-PHI

BCY58 Echivalent: BCY59

BCY59 SI-N 45V 0.2A 1W 250MHz

BCY70-PHI Echivalent: BCY71-MBR

BCY71-MBR SI-P 45V 0.2A 0.35W

BCY72 SI-P 30V 0.2A 0.35W

BCY77 Echivalent: BCY79

BCY78 Echivalent: BCY79

BCY79 SI-P 45V 0.2A 1W 180MHz

BCY85 SI-N 100V 0.2A 0.3W

BD106 Echivalent: BD109

BD109 SI-N 60V 3A 15W

BD115-MBR SI-N 245V 0.15A 0.8W

BD127 Echivalent: BD129

BD128 Echivalent: BD129

BD129 SI-N 400V 0.5A 17.5W

BD130 Echivalent: 2N3055-ST

BD131-MBR SI-N 70V 3A 15W >60MHz

BD131/DIV SI-N 70V 3A 15W

BD132-MBR SI-P 45V 3A 15W >60MHz

BD132/DIV SI-P 45V 3A 15W

BD133 Echivalent: BD131-MBR

BD134 Echivalent: BD238-MBR

BD135 Echivalent: BD139-MBR

BD135-10 Echivalent: BD139-16

BD135-16 Echivalent: BD139-16

BD136 Echivalent: BD140-MBR

BD136-10 Echivalent: BD140-16

BD136-16 Echivalent: BD140-16

BD137 Echivalent: BD139-MBR

BD137-10 Echivalent: BD139-16

BD137-16 Echivalent: BD139-16

BD138 Echivalent: BD140-MBR

BD138-10 Echivalent: BD140-16

BD138-16 Echivalent: BD140-16

BD139 Echivalent: BD139-MBR

BD139- Echivalent: BD139-MBR

BD139-10 Echivalent: BD139-16

BD139-16 SI-N 80V 1.5A 12.5W 50MHz

BD139-MBR SI-N 80V 1.5A 12.5W 50MHz

BD139-PHI Echivalent: BD139-MBR

BD139-ST Echivalent: BD139-MBR

BD139/DIV SI-N 80V 1.5A 12.5W 50MHz

BD140 Echivalent: BD140-MBR

BD140- Echivalent: BD140-MBR

BD140-10 Echivalent: BD140-16

BD140-16 SI-P 80V 1.5A 12.5W 50MHz

BD140-MBR SI-P 80V 1.5A 12.5W 50MHz

BD140-ST Echivalent: BD140-MBR

BD140/DIV SI-P 80V 1.5A 12.5W 50MHz

BD141 SI-N 140V 8A 117W

BD142 SI-N 50V 15A 117W

BD150 Echivalent: BD115-MBR

BD157 Echivalent: BD159-MBR

BD158 Echivalent: BD159-MBR

BD159-MBR SI-N 375V 0.5A 20W

BD160 SI-N 250V 5A 25W

BD162 Echivalent: BD237-MBR

BD163 Echivalent: BD237-MBR

BD165 Echivalent: BD237-MBR

BD166 Echivalent: BD238-MBR

BD167 Echivalent: BD237-MBR

BD168 Echivalent: : BD236

BD169 Echivalent: BD237-MBR

BD170 Echivalent: BD238-MBR

BD175 Echivalent: BD179-MBR

BD176 Echivalent: BD180-MBR

BD177 Echivalent: BD179-MBR

BD178 Echivalent: BD180-MBR

BD179-MBR SI-N 80V 3A 30W

BD179/DIV SI-N 80V 3A 30W

BD180-MBR SI-P 80V 3A 30W >2MHz

BD180/DIV SI-P 80V 3A 30W >2MHz

BD181 Echivalent: BD183

BD182 Echivalent: 2N3055-ST

BD183 SI-N 85V 15A 117W

BD186 Echivalent: BD244C-MBR

BD187 Echivalent: BD239C-MBR

BD188 Echivalent: BD442-MBR

BD189 Echivalent: BD441-MBR

BD190 Echivalent: BD442-MBR

BD201 SI-N 60V 8A 55W

BD201F-PHI SI-N 60V 8A 32W

BD202 Echivalent: BD538-MBR

BD203 Echivalent: BD537-MBR

BD204 Echivalent: BD538-MBR

BD204F-PHI SI-P 60/60V 8A 60W >7MHz

BD207 Echivalent: BD537-MBR

BD208 Echivalent: BD538-MBR

BD222 Echivalent: BD537-MBR

BD225 Echivalent: BD538-MBR

BD226 Echivalent: BD230-MBR

BD227 Echivalent: BD231-MBR

BD228 Echivalent: BD230-MBR

BD229 Echivalent: BD231-MBR

BD230-MBR SI-N 100V 1.5A 12.5W

BD230/DIV SI-N 100V 1.5A 12.5W

BD231-MBR SI-P 100V 1.5A 12.5W

BD231/DIV SI-P 100V 1.5A 12.5W

BD232 SI-N 300V 0.25A 7W

BD233 Echivalent: BD237-MBR

BD234 Echivalent: BD238-MBR

BD235 Echivalent: BD237-MBR

BD236 Echivalent: BD238-MBR

BD237-MBR SI-N 100V 2A 25W 3MHz

BD237/DIV SI-N 100V 2A 25W 3MHz

BD238-MBR SI-P 100V 2A 25W 3MHz

BD238/DIV SI-P 100V 2A 25W 3MHz

BD239A Echivalent: BD239B

BD239B Echivalent: BD239C-MBR

BD239C-MBR SI-N 100V 2A 30W 3MHz

BD239C/DIV SI-N 100V 2A 30W 3MHz

BD239E Echivalent: TIP29E-MBR

BD240 SI-P 45V 2A 30W

BD240A Echivalent: BD240C-MBR

BD240B Echivalent: BD240C-MBR

BD240C-MBR SI-P 100V 2A 30W 3MHz

BD240C/DIV SI-P 100V 2A 30W 3MHz

BD241 Echivalent: BD241C-MBR

BD241A Echivalent: BD241C-MBR

BD241B Echivalent: BD241C-MBR

BD241C-MBR SI-N 100V 3A 40W 3MHz

BD241C/DIV Echivalent: : BD241C-MBR

BD241D-MBR SI-N 120V 3A 40W 3MHz

BD242 Echivalent: : BD242C-MBR

BD242A Echivalent: BD242C-MBR

BD242B Echivalent: BD242C-MBR

BD242C-MBR SI-P 100V 3A 40W 3MHz

BD242C/DIV SI-P 100V 3A 40W 3MHz

BD243 Echivalent: BD243C-MBR

BD243A Echivalent: BD243C-MBR

BD243B Echivalent: BD243C-MBR

BD243C-MBR SI-N 100V 6A 65W 3MHz

BD243C/DIV SI-N 100V 6A 65W

BD243D Echivalent: BD243F

BD243F SI-N 200V 6A 65W 3MHz

BD244 Echivalent: BD244C-MBR

BD244A Echivalent: BD244C-MBR

BD244B Echivalent: BD244C-MBR

BD244C-MBR SI-P 100V 6A 65W 3MHz

BD244C/DIV Echivalent: : BD244C-MBR

BD244F SI-P 200V 6A 65W 3MHZ

BD245A Echivalent: TIP33C-MBR

BD245B Echivalent: TIP33C-MBR

BD245C-MBR Echivalent: TIP33C-MBR

BD245C/DIV Echivalent: TIP33C-MBR

BD246 Echivalent: TIP34C-MBR

BD246A Echivalent: TIP34C-MBR

BD246B Echivalent: TIP34C-MBR

BD246C-MBR Echivalent: TIP34C-MBR

BD249 Echivalent: TIP35C-MBR

BD249A Echivalent: TIP35C-MBR

BD249B Echivalent: TIP35C-MBR

BD249C-MBR Echivalent: TIP35C-MBR

BD250A Echivalent: BD250C-MBR

BD250C-MBR Echivalent: TIP36C-MBR

BD250C/DIV SI-P 100V 25A 125W 3MHz

BD262 Echivalent: BD682-MBR

BD263 Echivalent: BD679A-MBR

BD266 Echivalent: BD652-MBR

BD267 Echivalent: BD651-MBR

BD277 SI-P 45V 7A 70W >10MHz

BD278 Echivalent: BD201

BD291 Echivalent: BD201

BD292 Echivalent: BD538-MBR

BD293 Echivalent: BD537-MBR

BD294 Echivalent: BD538-MBR

BD301 Echivalent: BD303

BD302 SI-P 60V 8A 55W >3MHz

BD303 SI-N 60V 8A 55W >3MHz

BD311 Echivalent: 2N3716-MBR

BD312 Echivalent: BD314

BD313 Echivalent: 2N3716-MBR

BD314 SI-P 80V 10A 150W

BD315 Echivalent: BD317/DIV

BD317/DIV SI-N 100V 16A 200W 1MHz

BD318/DIV SI-P 100V 16A 200W

BD329 SI-N 32V 3A 15W 130MHz

BD330 SI-P 32V 3A 15W

BD331 Echivalent: BD335

BD332 Echivalent: BD336-MBR

BD333 Echivalent: BD335

BD334 Echivalent: BD336-MBR

BD335 N-DARL 100V 6A 60W

BD336-MBR P-DARL 100V 6A 60W

BD337-MBR N-DARL+D 120V 6A 60W >10M

BD361 Echivalent: BD329

BD362 SI-P 32V 3A 15W

BD369 Echivalent: 2N5884-ST

BD371B SI-N 60V 1,5A 2,5W

BD375 Echivalent: BD237-MBR

BD376 Echivalent: BD237-MBR

BD377 Echivalent: BD237-MBR

BD378 Echivalent: BD238-MBR

BD379-MBR Echivalent: BD237-MBR

BD380-MBR Echivalent: BD238-MBR

BD385 SI-N 60V 1A 10W >250MHz

BD386 Echivalent: BD387

BD387 SI-N 80V 1A 10W >250MHz

BD410 SI-N 500V 1A 20W

BD411 N-DARL 50V 2A 10W B>25K

BD433 Echivalent: BD441-MBR

BD434 Echivalent: BD442-MBR

BD435 Echivalent: BD441-MBR

BD436 Echivalent: BD442-MBR

BD437 Echivalent: BD441-MBR

BD438 Echivalent: BD442-MBR

BD439 Echivalent: BD441-MBR

BD440 Echivalent: BD442-MBR

BD441-MBR SI-N 80V 4A 36W 3MHz

BD441/DIV SI-N 80V 4A 36W 3MHz

BD442-MBR SI-P 80V 4A 36W 3MHz

BD442/DIV SI-P 80V 4A 36W 3MHz

BD443 Echivalent: BD237-MBR

BD505 Echivalent: BD509

BD507 Echivalent: BD509

BD509 Echivalent: : N.L.A.

BD512 Echivalent: : N.L.A.

BD515 SI-N 45V 2A 10W 160MHz

BD533-ST Echivalent: BD537-MBR

BD534 Echivalent: BD538-MBR

BD535 Echivalent: BD537-MBR

BD536 Echivalent: BD538-MBR

BD536FI Echivalent: 130.63508

BD537-MBR SI-N 80V 8A 50W

BD537/DIV Echivalent: BD537-MBR

BD538-MBR SI-P 80V 4A 50W >3MHz

BD539-MBR SI-N 40V 5A 45W

BD540 Echivalent: BD277

BD543B Echivalent: BD543C-MBR

BD543C-MBR SI-N 100V 8A 70W 3MHz

BD543C/DIV SI-N 100V 8A 70W

BD544 Echivalent: BD912-MBR

BD545 SI-N 40V 15A 85W 3MHz

BD546 Echivalent: TIP36C-MBR

BD561 Echivalent: BD441-MBR

BD562-MBR Echivalent: BD442-MBR

BD590 Echivalent: BD592

BD595 Echivalent: BD911-MBR

BD597 Echivalent: BD911-MBR

BD599 Echivalent: BD911-MBR

BD607 Echivalent: BD911-MBR

BD608 Echivalent: : N.L.A.

BD609 Echivalent: BD911-MBR

BD610 Echivalent: BD912-MBR

BD618 Echivalent: BD442-MBR

BD637 SI-N 100V 2A 3OW >3MHz

BD638 SI-P 100V 2A 30W >3MHz

BD643 Echivalent: BD651-MBR

BD644 Echivalent: BD652-MBR

BD645 Echivalent: BD651-MBR

BD646 Echivalent: BD652-MBR

BD647 Echivalent: BD651-MBR

BD648-MBR P-DARL 80V 8A 62,5W

BD649 Echivalent: BD651-MBR

BD650 Echivalent: BD652-MBR

BD651-MBR N-DARL 120V 8A 62.5W

BD651/DIV N-DARL 120V 8A 62.5W

BD652-MBR P-DARL 120V 8A 62.5W

BD652/DIV P-DARL 120V 8A 62.5W

BD663 Echivalent: BD911-MBR

BD675 Echivalent: BD681-MBR

BD675A Echivalent: BD679A-MBR

BD676 Echivalent: BD682-MBR

BD676A Echivalent: BD680A-MBR

BD677 Echivalent: BD681-MBR

BD677A Echivalent: BD679A-MBR

BD678 Echivalent: BD682-MBR

BD678A Echivalent: BD680A-MBR

BD679 Echivalent: BD681-MBR

BD679A-MBR N-DARL+D 80V 4A 40W

BD679A/DIV Echivalent: : BD679A-MBR

BD680 Echivalent: BD682-MBR

BD680A-MBR P-DARL+D 80V 4A 40W

BD680A/DIV P-DARL+D 80V 4A 40W

BD681-MBR N-DARL+D 100V 4A 40W B>75

BD681/DIV N-DARL+D 100V 4A 40W

BD682-MBR P-DARL+D 100V 4A 40W

BD682/DIV P-DARL+D 100V 4A 40W

BD683 N-DARL 120V 4A 40W

BD684 P-DARL 120V 4A 40W

BD697 Echivalent: BD901-MBR

BD698 Echivalent: BD902

BD698A Echivalent: BD902

BD699 Echivalent: BD901-MBR

BD700 Echivalent: BD902

BD701 Echivalent: BD901-MBR

BD702 Echivalent: BD902

BD705 Echivalent: BD711-MBR

BD706-ST Echivalent: BD712-MBR

BD707 Echivalent: BD711-MBR

BD708 Echivalent: BD712-MBR

BD709 Echivalent: BD711-MBR

BD710 Echivalent: BD712-MBR

BD711-MBR SI-N 100V 12A 75W

BD712-MBR SI-P 100V 12A 75W POWER

BD722 SI-P 80V 4A 36W >3MHz

BD737 Echivalent: BD441-MBR

BD743 Echivalent: BD911-MBR

BD743C-MBR SI-N 110V 15A 90W >5MHz

BD744 Echivalent: BD744C

BD744C SI-P 110V 15A 90W 5MHz

BD745 Echivalent: TIP35C-MBR

BD746 Echivalent: BD250C-MBR

BD750/DIV SI-P 100V 20A 200W

BD751/DIV SI-N 100V 20A 200W

BD785 Echivalent: BD791

BD786 Echivalent: BD792

BD787 Echivalent: BD791

BD788 Echivalent: BD792

BD789 Echivalent: BD791

BD790 Echivalent: BD792

BD791 SI-N 100V 4A 15W

BD792 SI-P 100V 4A 15W

BD801-MBR SI-N 100V 8A 65W >3MHz

BD805 Echivalent: BD911-MBR

BD806 Echivalent: BD912-MBR

BD807 Echivalent: BD911-MBR

BD808 Echivalent: BD912-MBR

BD809 Echivalent: BD911-MBR

BD810 Echivalent: BD912-MBR

BD825 Echivalent: BD829-MBR

BD826 Echivalent: BD830-MBR

BD827 Echivalent: BD829-MBR

BD828 Echivalent: BD830-MBR

BD829-MBR SI-N 100V 1A 8W

BD830-MBR SI-P 100V 1A 8W 75MHz

BD839 SI-N 45V 1.5A 10W 125MHz

BD843 SI-N 100V 1.5A 10W >150MH

BD875 Echivalent: BD879

BD877 N-DARL 80V 1A 9W 200MHz

BD879 N-DARL 100V 1A 9W 200MHz

BD880 P-DARL 100V 1A 200MHz

BD895 Echivalent: BD901-MBR

BD897 Echivalent: BD901-MBR

BD898 Echivalent: BD902

BD899 Echivalent: BD901-MBR

BD900 Echivalent: BD902

BD901-MBR N-DARL+D 100V 8A 70W

BD901/DIV N-DARL 100V 8A 70W

BD902 P-DARL 100V 8A 70W

BD905 Echivalent: BD911-MBR

BD906 Echivalent: BD912-MBR

BD907 Echivalent: BD911-MBR

BD908 Echivalent: BD912-MBR

BD909 Echivalent: BD911-MBR

BD910 Echivalent: BD912-MBR

BD911-MBR SI-N 100V 15A 90W

BD912-MBR SI-P 100V 15A 90W

BD933 Echivalent: BD941

BD934 Echivalent: BD942

BD935 Echivalent: BD941

BD936 Echivalent: BD942

BD937 Echivalent: BD941

BD938 Echivalent: BD942

BD939 Echivalent: BD941

BD939F SI-N 120V 3A 19W 3MHz

BD940 Echivalent: BD942

BD941 SI-N 140V 3A 30W 3MHz

BD941F-MBR Echivalent: : 130.43526

BD942 SI-P 140V 3A 30W 3MHz

BD943 SI-N 22V 5A 40W 3MHz

BD944 Echivalent: BD948

BD945 Echivalent: BD243C-MBR

BD947 Echivalent: BD243C-MBR

BD948 SI-P 45V 5A 40W 3MHz

BD949 Echivalent: BD951

BD951 SI-N 80V 5A 40W >3MHz

BD956 SI-P 120V 5A 40W 3MHz

BD977 Echivalent: BD879

BDT60 Echivalent: BDT62C

BDT61-MBR N-DARL+D 60V 4A 50W >10MH

BDT61C-MBR N-DARL+D 120V 4A 50W >10M

BDT61F-MBR N-DARL+D 60V 4A

BDT62B Echivalent: BDT62C

BDT62C P-DARL 120V 10A 90W B>1K

BDT63B Echivalent: BDT63C

BDT63C N-DARL 120V 10A 90W B>1K

BDT64C/DIV P-DARL 120V 12A 125W B>1K

BDT65B Echivalent: BDT65C

BDT65C N-DARL 120V 12A 125W B>1K

BDT85A SI-N 100V 15A 125W 20MHz

BDT86A SI-P 100V 15A 125W 20MHz

BDT87 SI-N 120V 15A 125W 10MHz

BDT88 SI-P 120V 12A 117W

BDT95 Echivalent: : N.L.A.

BDT95A SI-N 100V 10A 90W 4MHz

BDT96 Echivalent: BDT96A

BDT96A SI-P 100V 10A 90W 4MHz

BDV64 Echivalent: BDV64B-MBR

BDV64A Echivalent: BDV64B-MBR

BDV64B-MBR Echivalent: TIP147-MBR

BDV64C P-DARL+D 120V 20A 125W B>

BDV65 Echivalent: BDV65B-MBR

BDV65B-MBR N-DARL+D 100V 20A 125W B>

BDV65C-MBR N-DARL+D 120V 20A 125W B>

BDV65C/DIV N-DARL+D 120V 20A 125W B>

BDV66B-MBR Echivalent: BDV66C/DIV

BDV66C-MBR Echivalent: BDV66C/DIV

BDV66C/DIV P-DARL+D 120V 16A 200W 7M

BDV66D P-DARL+D 160V 16A 200W

BDV67B Echivalent: BDV67D

BDV67C Echivalent: BDV67D

BDV67D Echivalent: : N.L.A.

BDV93 Echivalent: TIP33C-MBR

BDV94 Echivalent: TIP34C-MBR

BDV95 Echivalent: TIP33C-MBR

BDW22 Echivalent: BDW22C-MBR

BDW22A Echivalent: BDW22C-MBR

BDW22B Echivalent: BDW22C-MBR

BDW22C-MBR SI-P 100V 10A 90W >3MHZ

BDW23A Echivalent: BDX53C-MBR

BDW23B Echivalent: BDW23C

BDW23C N-DARL+D 100V 6A 50W

BDW24C Echivalent: BDX54C-MBR

BDW41 Echivalent: BDW93C-MBR

BDW42-MBR N-DARL 100V 15A 85W B>1K

BDW46 P-DARL 80V 15A 85W B>1K

BDW47-MBR P-DARL 100V 15A 85W B>1K

BDW51 Echivalent: BDW51C

BDW51C SI-N 100V 15A 125W >3MHz

BDW52C Echivalent: BD318/DIV

BDW83 Echivalent: BDW83C-MBR

BDW83C-MBR N-DARL 100V 15A 150W

BDW83C/DIV N-DARL 100V 15A 150W

BDW83D-MBR N-DARL+D 120V 15A 150W

BDW84C-MBR P-DARL 100V 15A 150W

BDW84C/DIV P-DARL 100V 15A 150W

BDW84D-MBR P-DARL+D 120V 15A 150W >1

BDW93A Echivalent: BDW93C-MBR

BDW93B Echivalent: BDW93C-MBR

BDW93C-MBR N-DARL 100V 12A 80W Echivalent:

BDW93CF-ST N-DARL 100V 12A 40W ISOLA

BDW94A Echivalent: BDW94C-MBR

BDW94B Echivalent: BDW94C-MBR

BDW94C-MBR P-DARL 100V 12A 80W

BDX11 SI-N 160V 10A 117W >0.8MH

BDX16A SI-P 140V 3A 25W 800KHz

BDX18-MBR Echivalent: MJ2955-MBR

BDX20 SI-P 160V 10A 117W >4MHz

BDX22 Echivalent: 2SC2233-MBR

BDX32 SI-N 1700V 4A 40W

BDX33 Echivalent: BDX33C-MBR

BDX33A Echivalent: BDX33C-MBR

BDX33B Echivalent: BDX33C-MBR

BDX33C-MBR N-DARL 100V 10A 70W

BDX34 Echivalent: BDX34C-MBR

BDX34A Echivalent: BDX34C-MBR

BDX34B Echivalent: BDX34C-MBR

BDX34C-MBR P-DARL 100V 10A 70W

BDX34C-RCA Echivalent: BDX34C-MBR

BDX35 Echivalent: BDX37-PHI

BDX36 Echivalent: BDX37-PHI

BDX37-PHI SI-N 80V 5A 15W 350ns

BDX42 Echivalent: BDX44-PHI

BDX43 Echivalent: BDX44-PHI

BDX44-PHI N-DARL+D 90V 1A 5W 1.5us

BDX45 Echivalent: BDX47-PHI

BDX46 Echivalent: BDX47-PHI

BDX47-PHI P-DARL 90V 1A 5W

BDX50-MBR SI-N 160V 16A 150W >800KH

BDX53 Echivalent: BDX53C-MBR

BDX53A Echivalent: BDX53C-MBR

BDX53B Echivalent: BDX53C-MBR

BDX53C-MBR N-DARL 100V 6A 60W B=500

BDX53C/DIV Echivalent: BDX53C-MBR

BDX53F-MBR N-DARL 160V 6A 60W B=500

BDX54 Echivalent: BDX54C-MBR

BDX54A Echivalent: BDX54C-MBR

BDX54B Echivalent: BDX54C-MBR

BDX54C-MBR P-DARL 100V 6A 60W B=500

BDX54C/DIV P-DARL 100V 6A 60W B=500

BDX54F-MBR P-DARL 160V 6A 60W B=500

BDX62A Echivalent: BDX62C/DIV

BDX62C/DIV P-DARL 120V 8A 90W

BDX63A Echivalent: BDX63C

BDX63B Echivalent: BDX63C

BDX63C N-DARL 140V 8A 90W

BDX64B Echivalent: BDX64C

BDX64C P-DARL 120V 12A 117W B>1K

BDX65A Echivalent: BDX65C

BDX65B Echivalent: BDX65C

BDX65C N-DARL 120V 12A 117W

BDX66A Echivalent: BDX66C

BDX66B Echivalent: BDX66C

BDX66C P-DARL 120V 16A 150W

BDX66C-MBR P-DARL 120V 16A 150W

BDX67A Echivalent: BDX67C-MBR

BDX67B Echivalent: BDX67C-MBR

BDX67C-MBR N-DARL 120V 16A 150W

BDX67C/DIV N-DARL 120V 16A 150W

BDX68B Echivalent: : MJ11015-MBR

BDX68C Echivalent: MJ11015-ST

BDX69B Echivalent: MJ11016-MBR

BDX71 SI-N 70V 10A 75W >0.8MHz

BDX73 Echivalent: BD711-MBR

BDX75 SI-N 45V 16A 75W >0.8MHz

BDX77-MBR SI-N 100V 8A 60W >7MHz

BDX78 Echivalent: BD538-MBR

BDX85C Echivalent: BDX87C-ST

BDX86C Echivalent: BDX88C-MBR

BDX87C-ST N-DARL 100V 12A 120W

BDX88C-MBR P-DARL 100V 12A 120W

BDX94 SI-P 80V 8A 90W >4MHz

BDX95-MBR SI-N 100V 8A 90W >4MHz

BDX96 SI-P 100V 8A 90W >4MHz

BDY20 SI-N 100V 15A 117W 1MHz

BDY29 SI-N 100V 30A 220W POWER

BDY33D Echivalent: BYD33D-PHI

BDY33M Echivalent: BYD33M-PHI

BDY56 SI-N 180V 15A 115W >10MHz

BDY57 Echivalent: BDY58-ST

BDY58-ST Echivalent: : BDY58/DIV

BDY58/DIV SI-N 160V 25A 175W

BDY73-MBR SI-N 100V 15A 115W >8KHz

BDY73B Echivalent: BDY73-MBR

BDY81B Echivalent: BD537-MBR

BDY83B SI-P 50V 4A 36W 3MHz

BDY90-ST SI-N 120V 10A 60W 0.35us

BDY90/DIV SI-N 120V 10A 60W 0.35us

BDY91 Echivalent: BDY90-ST

BDY92 Echivalent: BDY90-ST

BESCHERMHOES Echivalent: CMP-PROT

BF068 Echivalent: BFQ68

BF072 Echivalent: B-F 072

BF075 Echivalent: BFQ75

BF088 Echivalent: BFQ88

BF115 SI-N 50V 30mA 0.15W

BF120 SI-N 220V 50mA 0.3W

BF125 SI-N AM/FM V/M/O/ZF450MHz

BF127 Echivalent: BF199-CDL

BF152-MBR SI-N 30V 0.2W 800MHz VHF-

BF155 SI-N 40V 20mA 600MHz

BF161 SI-N 50V 20mA 550MHz

BF163 SI-N 40V 20mW 600MHz

BF164 SI-N 40V 0.2W 600MHz

BF166 SI-N 40V 20MA 0.175W 500M

BF167 Echivalent: BF198

BF173 SI-N 40V 25mA 0.23W 600MH

BF177 Echivalent: BF259-MBR

BF178 Echivalent: BF259-MBR

BF179B Echivalent: BF259-MBR

BF180 SI-N 30V 20mA 675MHz .15W

BF181 Echivalent: BF182

BF182 SI-N 25V 20mA 650MHz

BF183 Echivalent: BF763

BF184 SI-N 20V 30mA 260MHz

BF185 Echivalent: BF494-MBR

BF186 SI-N 190V 0.06A 0.8W 120M

BF189 SI-N 30V 25mA 270MHz

BF194 Echivalent: BF494-MBR

BF195 SI-N 30V 30mA 200MHz .22W

BF196 Echivalent: BF198

BF197 Echivalent: BF199-CDL

BF198 Echivalent: : BF199-CDL

BF199 Echivalent: BF199-CDL

BF199-CDL SI-N 40V 25mA 0.3W 550MHz

BF199-MBR Echivalent: BF199-CDL

BF200 SI-N 30V 20mA 0.15W 500MH

BF223 Echivalent: BF199-CDL

BF224 SI-N 45V 50mA 0.25W 450MH

BF225 Echivalent: BF198

BF240-MBR SI-N 40V 25mA 0.25W 400MH

BF241 Echivalent: BF240-MBR

BF244A-MBR N-FET 30V 25mA 0.3W

BF244B Echivalent: BF244C-MBR

BF244C-MBR N-FET 30V 25mA 0.3W

BF245A-MBR N-FET 30V 25mA 0.3W

BF245A/DIV Echivalent: : BF245A-MBR

BF245B-MBR N-FET 30V 25mA 0.3W

BF245B/DIV N-FET 30V 25mA 0.3W

BF245C-MBR N-FET 30V 0.1A 0.3W 170MC

BF245C/DIV Echivalent: : BF245C-MBR

BF246 Echivalent: BF246A B C

BF246A Echivalent: BF246C-MBR

BF246B Echivalent: BF247B

BF246C-MBR N-FET 25V 25mA 0.25W

BF247A Echivalent: BF247B

BF247B N-FET 25V 25mA 0.25W

BF247C N-FET 25V 25mA 0.25W

BF253 SI-N 30V 30mA 150MHz

BF254 SI-N 30V 30mA 260MHz .22W

BF255 SI-N 20V 30mA 200MHz .22W

BF256A-MBR N-FET 30V 7mA Vgs<7.5

BF256A/DIV Echivalent: BF256B/DIV

BF256B-MBR N-FET 30V 13mA

BF256B/DIV N-FET 30V 13mA

BF256C-MBR N-FET 30V 10mA 0.25W=S256

BF256C/DIV N-FET 30V 10mA 0.25W

BF257 Echivalent: BF259-MBR

BF258 Echivalent: BF259-MBR

BF259-MBR SI-N 300V 0.1A 0.8W 90MHz

BF259S-MBR SI-N 300V 0.1A 0.8W 90MHz

BF271 SI-N 40V 30mA 240mW 1GHz

BF272 Echivalent: 2N2906A

BF297 Echivalent: BF299-MBR

BF298 Echivalent: BF299-MBR

BF299-MBR SI-N 300V 0.1A 0.625W

BF310 Echivalent: BF960-MBR

BF311 Echivalent: BF199-CDL

BF314 Echivalent: BF496

BF316 SI-P UHF-M/O 550..660MHz

BF320 Echivalent: 2N3820-MBR

BF324-MBR SI-P 30V 25mA 450MHz .25W

BF336 Echivalent: BF337-MBR

BF337-MBR Echivalent: : BF259-MBR

BF338 Echivalent: BF259-MBR

BF339-MBR SI-P VHF-V/M/O 500MHz

BF340 Echivalent: BF343

BF341 Echivalent: BF343

BF342 Echivalent: BF343

BF343 SI-P 35V 35mA >80MHz .25W

BF353 Echivalent: BFR84-PHI

BF357 SI-N 30V 0.05A 1.6GHz

BF362-MBR SI-N UHF-V 800MHz

BF370-PHI SI-N 40V 0.1A 0.5W >500MH

BF372 Echivalent: 2N2906A

BF374 Echivalent: BF199-CDL

BF377 SI-N 15V 25mA 1.3GHz

BF378 Echivalent: BF377

BF391 Echivalent: BF393-CDL

BF392 Echivalent: BF393-MBR

BF393-MBR SI-N 300V 0.5A 0.65W

BF398 Echivalent: MPSA92-MBR

BF410B-PHI N-FET 20V 0.7mA

BF410C-MBR N-FET 20V 12mA AMPLIF

BF411 SI-N 110V 0.05A 0.3W

BF412 Echivalent: BF299-MBR

BF413 Echivalent: BF299-MBR

BF414 Echivalent: BF324-MBR

BF415 Echivalent: BF759

BF416 Echivalent: BF762

BF417 SI-N 300V 0.2A 6W 50MHz

BF418 SI-P 300V 0.2A 6W 50MHz

BF419-MBR SI-N 300V 0.1A 6W

BF420-MBR SI-N 300V 0.1A 0.83W

BF421-MBR SI-P 300V 0.1A 0.83W

BF422-MBR Echivalent: BF420-MBR

BF423 Echivalent: BF421-MBR

BF424 SI-P 30V 25mA 300MHz

BF435 SI-P 160V 0.2A 0.625W 80M

BF440 SI-P 40V 25mA 250MHz

BF441B SI-P 40V 25mA 250MHz

BF450 SI-P 40V 25mA 375MHz .25W

BF451 Echivalent: BF450

BF455 AM/FM-V/M/O 400MHz

BF457 Echivalent: BF459-MBR

BF458 Echivalent: BF459-MBR

BF459-MBR SI-N 300V 0.1A 10W 90MHz

BF459/DIV SI-N 300V 0.1A 10W 90MHz

BF460 Echivalent: : N.L.A.

BF461 Echivalent: BF462

BF462 SI-N 350V 0.5A 10W 45MHz

BF463 Echivalent: : N.L.A.

BF468-MBR Echivalent: : N.L.A.

BF469 Echivalent: BF471-PHI

BF470 Echivalent: BF472-PHI

BF471-PHI SI-N 300V 0.1A 2W 60MHz

BF471/DIV Echivalent: : BF471-PHI

BF472-PHI SI-P 300V 30mA 2W 60MHz

BF472/DIV SI-P 300V 30mA 2W 60MHz

BF479 SI-P 30V 50mA .16W 1.4GHz

BF480 Echivalent: 2SC2466-HIT

BF483 Echivalent: BF487-PHI

BF485 Echivalent: BF487-PHI

BF487-PHI SI-N 400V 0.05A 0.83W

BF491 Echivalent: BF493-MBR

BF492 Echivalent: BF493-MBR

BF493-MBR SI-P 300V 0.5A 0.625W

BF494-MBR SI-N 20V 30mA 260MHz

BF495C-MBR SI-N 30V 30mA 200MHz 0.3W

BF496 SI-N 30V 20mA 0.3W 550MHz

BF506 SI-P 40V 30mA 0.3W 550MHz

BF507 SI-N 30V 20mA 0.5W >750MH

BF509 SI-P 40V 30mA 0.3W 750MHz

BF516 SI-P 35V 20mA 850MHz

BF569 SI-P 40V 30mA 280mW 850MH

BF583-PHI Echivalent: BF585-PHI

BF585-PHI SI-N 350V 0.05A 5W 70MHz

BF587-MBR SI-N 400V 0.05A 5W >70MHz

BF606A-PHI Echivalent: : N.L.A.

BF617 Echivalent: BF759

BF618 Echivalent: BF762

BF622-PHI SI-N 250V 0.1A 2W

BF679/DIV SI-P 40V 30mA .16W 880MHz

BF680 SI-P 40V 30mA .16W 750MHz

BF681 Echivalent: BF979

BF689 SI-N 15V 25mA 0.2W 1GHz

BF689K-PHI SI-N 25V 25mA 0.36W 0.2GH

BF717 Echivalent: BF759

BF718 Echivalent: BF762

BF757 Echivalent: BF869-MBR

BF758 SI-N 300V 0.5A 2W

BF759 SI-N 350V 0.5A 10W VID-PO

BF762 Echivalent: : N.L.A.

BF763 SI-N 15V 25mA 0.36W 1.8GH

BF770 Echivalent: BF770A-SIE

BF770A-SIE SI-N 15V 0.05A 5.5 GHZ

BF787 Echivalent: BF759

BF788 Echivalent: BF759

BF790 Echivalent: BF762

BF791 SI-P 300V 0.1A 5W

BF792 Echivalent: BF762

BF799 SI-N 30V 35mA 280mW 800MH

BF817-MBR Echivalent: BSV81

BF818 Echivalent: BSV81

BF819-PHI SI-N 250V 0.1A 1.2W

BF820-MBR SI-N 300V 25mA >60MHz

BF821-PHI SI-P 300V 25mA 0.31W

BF822 Echivalent: BF820-MBR

BF823 Echivalent: BF821-PHI

BF840-MBR SI-N 40V 25mA 0.28W 380MH

BF844-MOT SI-N 450V 0.3A 625mW >50M

BF857 Echivalent: BF859-MBR

BF858 Echivalent: BF859-MBR

BF859-MBR SI-N 300V 0.1A 2.5W

BF869-MBR Echivalent: BF871-MBR

BF870 Echivalent: BF872-MBR

BF871-MBR SI-N 300V 0.1A 1.8W

BF872-MBR SI-P 300V 0.1A 1.6W 60MHz

BF872/DIV Echivalent: BF872-MBR

BF881 SI-N 400V 0.03A >60MHz

BF883S-TFK SI-N 275V 0.05A 7W >60MHZ

BF891 SI-P 400V 30mA <60MHz

BF896 Echivalent: 130.41773

BF900 Echivalent: BF961-MBR

BF900S Echivalent: BF961-MBR

BF905 Echivalent: BF961-MBR

BF907 Echivalent: BF961-MBR

BF910 N-FET-DG 20V 50mA 0.33W

BF926-PHI SI-P 20V 25mA 350MHz 17dB

BF914 Echivalent: BF509

BF936 Echivalent: BF324-MBR

BF939 SI-P 30V 220mA 750MHz .25

BF959-MBR SI-N 20V 0.1A 1.1GHz

BF960-MBR N-FET-DG 20V 25mA .8GHz 1

BF961-MBR N-FET-DG 20V 30mA .2GHz 2

BF961/DIV N-FET-DG 20V 30mA .2GHz 2

BF963 Echivalent: BF964

BF964 N-FET-DG 20V 30mA .2GHz 2

BF965 Echivalent: BF964

BF966 Echivalent: BF966S-MBR

BF966/DIV N-FET-DG 20V 30mA .8GHz 1

BF966S-MBR N-FET-DG 20V 30mA .2W .8G

BF967 SI-P 30V 20mA 900MHz .16W

BF968 SI-P UHF TRANS. 1100MHz

BF969 Echivalent: BF679/DIV

BF970-TFK SI-P 35V 30mA 0.3W 1GHz

BF970/DIV SI-P 35V 30mA .16W 850MHz

BF979 SI-P 20V 50mA 0.3W 1.75GH

BF980A-PHI N-FET-DG 18V 30mA UHF

BF981-MBR N-FET-DG 20V 20mA VHF

BF982-MBR N-FET-DG 20V 40mA 200MHz

BF989-PHI N-FET 20V 30mA 0.2W

BF990A-PHI N-FET-DG 18V 30mA 0.2W

BF991-MBR N-FET-DG 20V 20mA UHF

BF992-MBR N-FET 20V 40mA 0.2W

BF994S-MBR N-FET-DG 20V 30mA 200MHz

BF996S-MBR N-FET-DG 20V 30mA 800MHz

BF997 Echivalent: BF996S-MBR

BF998-MBR N-FET-DG 12V 30mA 800MHz

BF999-MBR N-FET 20V 30MA 0.2W 300MH

BFG135-PHI SI-N 25V 0.15A 1W BIPOLAR

BFG196 Echivalent: BFG198-PHI

BFG198-PHI SI-N 20V 0.1A 1W 8GHZ

BFG65-MBR Echivalent: : N.L.A.

BFG65/DIV SI-N 10V 50mA 0.3W 8GHz

BFG71-PHI TRANSISTOR =ON4357

BFG91A-PHI Echivalent: : N.L.A.

BFG94-PHI SI-N 15V 60mA 0.7W BIPOLA

BFG96-PHI SI-N 20V 75mA 0.7W 800MHz

BFG97-PHI SI-N 20V 0.1A 0.5W BIPOLA

BFO68 Echivalent: BFQ68-PHI

BFQ10-PHI N-FET 30V 30mA 250mW

BFQ162-PHI SI-N 20V 0.5A 3W 1GHz

BFQ232-PHI SI-N 100V 0.3A 1GHz

BFQ232A-PHI SI-N 115V 0.3A 800MHz

BFQ235A-PHI SI-N 115V 0.3A 3W 800MHz

BFQ252-PHI SI-P 100V 0.3A 3W

BFQ252A-PHI SI-P 115V 0.3A 800MHz

BFQ255-PHI SI-P 100V 0.3A 3W 1GHz

BFQ255A-PHI SI-P 115V 0.3A 3W 800MHz

BFQ262-PHI SI-P 100V 0.4A 5W 1GHz

BFQ262A-PHI SI-P 115V 0.4A 5W 800MHz

BFQ33C-PHI SI-N 7V 20mA 0.14W 12.5GH

BFQ34-PHI SI-N 18V 0.15A 2.7W 4GHz

BFQ34T-PHI Echivalent: : N.L.A.

BFQ42-PHI Echivalent: : N.L.A.

BFQ43-PHI SI-N 18V 1.2A 4W 175MHz 1

BFQ65-MBR SI-N 10V 50mA 0.3W 8GHz 8

BFQ68 Echivalent: BFQ68-PHI

BFQ68-PHI SI-N 18V 0.3A 4.5W 4GHz

BFQ69-SIE Echivalent: : N.L.A.

BFR134-PHI Echivalent: : N.L.A.

BFR15 Echivalent: BFT66

BFR29-PHI N-FET 30V 10mA Up<4V

BFR34A Echivalent: BFR90-MBR

BFR36-ST SI-N 40V 200mA 0.8W 1.3GH

BFR35AP-SIE SI-N 12V 30mA 4.9GHz 14dB

BFR37 SI-N 30V 50mA 0.25W 1.4GH

BFR38 SI-P 40V 20mA 0.2W 1GHz

BFR39-MBR SI-N 90V 1A 0.8W >100MHz

BFR40 SI-N 70V 1A 0.8W >100MHz

BFR41 Echivalent: BFR40

BFR64 Echivalent: BFQ34-PHI

BFR79 SI-P 90V 1A 0.8W >100MHz

BFR80 Echivalent: BC640-MBR

BFR81 Echivalent: BC640-MBR

BFR84-PHI Echivalent: BFR84/DIV

BFR84/DIV N-FET-DG 20V 50MA 0.3W

BFR90-MBR SI-N 15V 30mA 5GHz 19.5dB

BFR90/02 Echivalent: BFR90-MBR

BFR90A-MBR SI-N 15V 30mA 5.5GHz 16dB

BFR90A/DIV SI-N 15V 30mA 5.5GHz 16dB

BFR91-MBR SI-N 12V 50mA 5GHz 18dB

BFR91/02 Echivalent: BFR91-MBR

BFR91/SH Echivalent: BFQ69-SIE

BFR91A-MBR SI-N 12V 50mA 6GHz 14dB

BFR91A/DIV Echivalent: : BFR91A-MBR

BFR92-MBR SI-N 15V 30mA 5GHz 19.5dB

BFR92A-MBR SI-N 15V 30mA 5.5GHz 16dB

BFR92R-MBR SI-N 15V 30mA 5GHz REVERS

BFR93 Echivalent: BFR93A-MBR

BFR93A-MBR SI-N 15V 50mA 6GHz 14dB

BFR95-MBR SI-N 25V 0.15A 1.5W 3.5GH

BFR96-MBR SI-N 15V 75mA 5GHz 16dB

BFR96/02 Echivalent: BFR96-MBR

BFR965 Echivalent: BFR96S-MBR

BFR96S-MBR SI-N 15V 0.1A 5.5GHz 11dB

BFR96S/DIV SI-N 15V 0.1A 5.5GHz 11dB

BFR96TS Echivalent: BFR96S-MBR

BFR97 Echivalent: 2N3866-MBR

BFS17 SI-N 15V 25mA 1GHz E1

BFS19-MBR SI-N 30V 30mA 260MHz

BFS20-PHI SI-N 30V 25mA 450MHz G1

BFS22A-PHI SI-N 3V 0.75A 4W 175MHz

BFS23A-PHI SI-N 36V 0.5A 4.5W 500MHz

BFT25-PHI SI-N 8V 6.5mA 50mW 500NHz

BFT43 SI-N 125/100V 1A 0.8W

BFT45-PHI SI-P 250V 0.5A 0.75W 70MH

BFT66 SI-N 15V 30mA 4.5GHz 12dB

BFT79-MBR SI-P 90V 1A 0.8W >100MHZ

BFT95 SI-P UHF 15V 25mA 3.6-5GH

BFT96 Echivalent: : N.L.A.

BFT98-SIE Echivalent: : N.L.A.

BFW10-MBR N-FET 30V 20mA AMPL.

BFW11-MBR N-FET 30V 10mA AMPL.

BFW11/DIV N-FET 30V 10mA AMPL.

BFW12-PHI N-FET 30V 5mA AMPL.

BFW16A-MBR SI-N 25V 0.3A 1.5W 1.2GHz

BFW16A/DIV SI-N 25V 0.15A 1.5W 1.2GH

BFW17A-PHI Echivalent: BFW17A/DIV

BFW17A/DIV SI-N 25V 0.3A 1.5W 1.1GHz

BFW30-PHI SI-N 10V 0.1A 0.25W 1.6GH

BFW43-ST SI-P 150V 0.1A 0.4W 150MH

BFW44 SI-P 150V 0.1A 0.7W 50MHz

BFW92-MBR SI-N 15V 50mA 0.3W 1.6GHz

BFW92A-MBR SI-N 15V 25mA 3.2GHz 13dB

BFW93/02-PHI Echivalent: : N.L.A.

BFW99 Echivalent: BFT66

BFX29 Echivalent: BC161-16-MBR

BFX34-MBR SI-N 60V 5A 0.87W

BFX37-MBR SI-P 90V 0.1A 0.36W 70MHz

BFX38-MBR SI-P 55V 1A 0.8W B>85

BFX40 SI-P 75V 1A 0.8W B>85

BFX48-ST SI-P 30V 0.1A 0.36W

BFX55 SI-N 60V 0.4A 2.2W 700MHz

BFX59 Echivalent: BFX89-MBR

BFX84 Echivalent: BFX85-CDL

BFX85-CDL SI-N 100V 1A 0.8W

BFX87 Echivalent: 2N2905A-MBR

BFX88 Echivalent: 2N2905A-MBR

BFX89-MBR SI-N 15V 50mA 0.2W 1.3GHz

BFX93 Echivalent: BC550C-MBR

BFY33 Echivalent: 2N1613-MBR

BFY34 Echivalent: 2N1613-MBR

BFY39 SI-N 45V 0.1A 0.3W 150MHz

BFY46 Echivalent: 2N1711-MBR

BFY50-MBR SI-N 80V 1A 0.7W 55/175ns

BFY51-MBR SI-N 60V 1A 0.7W

BFY52 SI-N 40V 1A 0.8W 100MHz

BFY56 SI-N 60V 1A 0.8W

BFY57 Echivalent: BF259-MBR

BFY64-MBR SI-P 40V 0.6A 0.7W

BFY88-MBR SI-N 25V 25mA 850MHz

BFY90 Echivalent: BFY90-ST

BFY90-PHI Echivalent: BFY90-ST

BFY90-ST SI-N 15V 25mA 2GHz 8dB

BFY90/DIV SI-N 15V 25mA 2GHz 8dB

BGX885N-PHI CATV AMPL. 860MHz 17dB

BGY58 Echivalent: BGY88-PHI

BGY58/03 Echivalent: BGY88-PHI

BGY88-PHI CATV AMP. 450MHz 35dB

BGY89-PHI CATV AMP. 450MHz 38dB

BL2710A SPANNUNGSSTABI-ERSATZ IC4

BL2711 POWER SUPPLY ASSEMBLY

BLV30-PHI Echivalent: : N.L.A.

BLW29-PHI Echivalent: : N.L.A.

BLW31-PHI Echivalent: : N.L.A.

BLW32-PHI SI-N 50V 0.65A 10W 3.5GHz

BLW60C-PHI SI-N 18V 9A 100W 650MHz

BLX15-PHI SI-N 110V 6.5A 195W 275MH

BLX68-PHI Echivalent: : N.L.A.

BLX91A-PHI Echivalent: : N.L.A.

BLY87C-PHI SI-N 36V 1.53A 20W 175MHz

BLY88C-PHI SI-N 18V 3A 36W 850MHz

BLY89C-PHI SI-N 18V 6A 73W 800MHz

BLY93C-PHI SI-N 65V 2A 25W 175MHz

BLY94-PHI SI-N 65V 6A 50W 175MHz

BMR78 Echivalent: BMR808

BMR90 Echivalent: BMR95

BN5115A I.C.

BNC-PLIER Echivalent: CRIMP-PLIER

BOX KM25 Echivalent: BOX KM25B

BP103-2 PHOTO-N 50V <0.16mA 850nm

BP103-3 PHOTO-N 50V <0.25mA 850nm

BP104 PHOTO-D 20V 17uA 950nm

BPW14 Echivalent: BPW14C

BPW14A Echivalent: BPW14C

BPW14B Echivalent: BPX43-5

BPW14C PHOTO-N 32V 50mA 780nm

BPW17N PHOTO-N 32V 50mA 780nm

BPW20R PHOTO-D 5V 61uA 7.5'

BPW21R Echivalent: BPW20R

BPW24R PHOTO-D 50V 5nA 870nm

BPW34 PHOTO-D 32V 80nA 880nm

BPW39B PHOTO-N 32V 50MA 780NM

BPW40 PHOTO-N 32V 100mA 780nm

BPW41N PHOTO-D 32V 925nm

BPW42 Echivalent: SFH309

BPW43 PHOTO-D 32V 900nm

BPW50 PHOTO-D 32V 925nm

BPW76B Echivalent: BPX38-3

BPX38-2 Echivalent: BPX38-3

BPX43-4 Echivalent: BPX43-5

BPX43-5 PHOTO-N 50V <2.5mA 880nm

BPX61 GEN.PURPOSE PHOTO-DIODE

BPX63 SIL. PHOTODIODE 7V 800nm

BPX65 SIL. PHOTODIODE 50V 850nm

BPX81 Echivalent: BPX81-2

BPX81-2 PHOTO-N 32V <0.50mA 850nm

BPY62-3 PHOTO-N 50V <2.0mA 850nm

BR100-ST Echivalent: DB3-ST

BR100/03-PHI DIAC 28-36V 2A

BR100/DIV Echivalent: : BR100-ST

BR101-PHI THYR. TRIGGER 50V 0.175A

BR103 Echivalent: P0102DA-ST

BR203 THYR. 30V 1A 0.2mA 1/10uS

BR303 THYR. 30V 1A 0.2mA 1/13uS

BR303T THYR. 30V 1A 0.2mA ROUND

BR403 THYR. 30V 1A 2mA 1/13us

BRT12H-SIE OPTO TRIAC 60V 0.3A

BRX44 Echivalent: BRX49-MBR

BRX45 Echivalent: BRX49-MBR

BRX46 Echivalent: BRX49-MBR

BRX47-MOT Echivalent: BRX49-MBR

BRX48 Echivalent: BRX49-MBR

BRX49-MBR THYR. 400V 0.8A 0.2mA

BRX64 Echivalent: C106D-MOT

BRX65 Echivalent: : C106M-MOT

BRY39-PHI THYR. TRIGGER 70V 0.25A

BRY49-TFK Echivalent: BRY39-PHI

BRY55 Echivalent: P0102DA-ST

BRY55/200 Echivalent: P0102DA-ST

BRY55/300 Echivalent: P0102DA-ST

BRY55/400 Echivalent: P0102DA-ST

BRY55/600 Echivalent: P0201MA-ST

BRY56-MBR PUT 70V 0.175A

BS107-MBR N-FET 200V 0.13A 0.8W 26R

BS108-MBR N-FET 200V 0.23A 0.8W 8E

BS170-MBR N-FET 60V 0.3A 0.8W 5R

BS170/DIV Echivalent: BS170-MBR

BS208-MBR P-FET 200V 0.2A 0.8W

BS250-MBR P-FET 45V 0.18A 0.83W

BSN254A-PHI N-FET 250V 0.3A 1W <7E

BSN274-PHI N-FET 270V 0.25A 1W <8E

BSN304-PHI N-FET 300V 0.25A 1W <8E

BSR14-PHI SI-N 75V 0.8A <35/285ns

BSR31-PHI SI-P 70V 1A B>100

BSR50-PHI N-DARL 60V 2A 0.8W 350MHz

BSR51 Echivalent: BC879-MBR

BSR52 Echivalent: BC879-MBR

BSR60 P-DARL 45V 1A 0.8W

BSS123-MBR N-FET 100V 0.17A 13/29ns

BSS38-MBR SI-N 120V 0.1A 0.2W

BSS44-MBR SI-P 65V 5A 5W

BSS50 Echivalent: BSS52-MBR

BSS51 Echivalent: BSS52-MBR

BSS52-MBR N-DARL 100V 1A 0.8W

BSS60 Echivalent: BSS62-PHI

BSS61 Echivalent: BSS62-PHI

BSS62-PHI P-DARL+D 100V 1A 0.8W

BSS68-PHI SI-P 60/40V 0.8A <50/110n

BSS89-MBR N-FET 240V 0.3A 1W 6R

BSS91-MBR N-FET 200V 0.35A 1.5W <6E

BSS92-MBR P-FET 200V 0.15A 1W 38/45

BSTB0140 Echivalent: TIC106M-MBR

BSTB0246 Echivalent: TIC116N-MBR

BSTB0440 Echivalent: 17088H

BSTC0233 Echivalent: TIC116N-MBR

BSTC0240 Echivalent: TIC116N-MBR

BSTC0246 Echivalent: TIC116N-MBR

BSTC1026 Echivalent: S0402DH-MBR

BSTC1040 Echivalent: TIC116M-MBR

BSTC1046 Echivalent: TIC116N-MBR

BSTC1233 Echivalent: : N.L.A.

BSTC3146M Echivalent: TIC116N-MBR

BSTCC0246H Echivalent: 15/80H

BSTCC0246R Echivalent: 15/85R

BSV52-MBR SI-N 20V 0.1A 225mW 400MH

BSV57B Echivalent: : N.L.A.

BSV80-PHI N-FET 40V 10mA 0.35W

BSV81 N-FET 30V 50mA 0.2W 100E

BSW43-MBR SI-N 60V 0.2A 0.3W B>180

BSW66A Echivalent: BSW68A-MBR

BSW67A Echivalent: BSW68A-MBR

BSW68A-MBR SI-N 150V 2A 5W 130MHZ

BSW85/DIV SI-N 75V 0.5A 0.5W 250MHZ

BSX19 Echivalent: 2N2369A-MBR

BSX20 SI-N 40V 0.5A .36W 7/18ns

BSX21 Echivalent: BSS38-MBR

BSX26-MBR SI-N 40V 0.5A 0.36W

BSX29-MBR SI-P 12V 0.2A 0.36W 25/35

BSX32-MBR SI-N 65V 1A 0.8W 35/40ns

BSX45 Echivalent: BSX47

BSX46 Echivalent: BSX47

BSX47 SI-N 120V 1A 5W

BSX52-MBR SI-N 25V 0.2A 0.3W B>180

BSX59-PHI SI-N 45V 1A 0.8W

BSX60-MBR Echivalent: BSX59-PHI

BSX61 Echivalent: BSX60-MBR

BSX88 SI-N 40V 0.5A 0.36W

BSY55 Echivalent: BSY56

BSY56 SI-N 120V 0.5A 0.8W 100MH

BT100 Echivalent: BT151/500R-PHI

BT100A Echivalent: TIC116M-MBR

BT106 THYR. 700V 10A 20mA

BT112/750R Echivalent: TD3F800R-AEG

BT113/700R Echivalent: TD3F700R

BT119 THYR.+D 700V 3A 40mA TRAC

BT120 THYR.+D 700V 3A 40mA RETR

BT134/600-PHI TRIAC 600V 5A <35mA

BT136/500-PHI Echivalent: BT136/600-PHI

BT136/600-PHI TRIAC 600V 4A 35/70mA

BT136/800-PHI TRIAC 800V 4A 35/70mA

BT136F/800-PHI TRIAC 800V 4A 35/70mA ISO

BT137/500-PHI Echivalent: BT137/600-PHI

BT137/600-PHI TRIAC 600V 8A 35/70mA

BT137/800-PHI TRIAC 800V 8A 35/70mA

BT138/500-PHI Echivalent: BT138/800-PHI

BT138/600-PHI Echivalent: BT138/800-PHI

BT138/800-PHI TRIAC 800V 12A 35/70mA

BT138F/800-PHI TRIAC 800V 12A 35/70mA IS

BT139/500 Echivalent: BT139/600-PHI

BT139/600-PHI TRIAC 600V 16A 35/70mA

BT139/800-PHI TRIAC 800V 16A 35/70mA

BT139F/800-PHI TRIAC 800V 16A 35/70mA IS

BT145/500R Echivalent: 2N6509-MOT

BT145/600R Echivalent: 2N6509-MOT

BT145/800R-PHI Echivalent: 2N6509-MOT

BT149F Echivalent: BT149G-PHI

BT149G-PHI THYR. 600V 0.5A 0.2mA

BT149M Echivalent: BT149G-PHI

BT151/500R-PHI Echivalent: BT151/800R-PHI

BT151/600R Echivalent: BT151/800R-PHI

BT151/650R-PHI Echivalent: BT151/800R-PHI

BT151/800R-PHI THYR. 800V 12A 15mA 2uS

BT151F/500R Echivalent: BT151F/800R-PHI

BT151F/650R Echivalent: BT151F/800R-PHI

BT151F/800R-PH THYR. 800V 12A 15mA/1V5

BT152/400R Echivalent: BT152/600R-PHI

BT152/600R-PHI THYR. 600V 20A 50mA 2/35u

BT152/800R-PHI THYR. 800V 20A 50mA 2/35u

BT153-PHI Echivalent: TIC106M-MBR

BT169D-PHI THYR. 400V 0.5A 0.2/5mA

BT9151-3 Echivalent: UM9151-3

BTA04/400S Echivalent: BTA08/600SW-ST

BTA04/600S Echivalent: BTA08/600SW-ST

BTA06/400B-ST TRIAC 400V 6A 50/100mA

BTA06/600A-ST TRIAC 600V 6A 25/25mA

BTA06/700B-ST TRIAC 700V 6A 50/100mA

BTA08/400B-ST Echivalent: BTA08/400C-ST

BTA08/400C-ST TRIAC 400V 8A 25/50mA

BTA08/400S-ST Echivalent: BTA08/600SW-ST

BTA08/600B Echivalent: BTA08/700B-ST

BTA08/600SW-ST TRIAC 600V 8A 10mA

BTA08/700B-ST TRIAC 700V 8A 100/50mA

BTA10/400B-ST TRIAC 400V 10A 50/100mA

BTA10/600B Echivalent: BTA12/600B-ST

BTA10/700B-ST Echivalent: BT138/800-PHI

BTA12/400B Echivalent: BTA12/600B-ST

BTA12/600B-ST TRIAC 600V 12A 50/100mA

BTA12/800B-ST TRIAC 800V 12A 100/50mA

BTA16-800B Echivalent: BTA16/800B-ST

BTA16/400B Echivalent: BTA16/600B-ST

BTA16/600B-ST TRIAC 600V 16A 50/100mA

BTA16/700B-ST TRIAC 700V 16A 100mA

BTA16/800B-ST TRIAC 800V 16A 100/50mA

BTA25/400 Echivalent: BTA25/600A-ST

BTA25/600A-ST TRIAC 600V 30A 100/150mA

BTA25/600B-ST TRIAC 600V 30A 50/100mA

BTA25/700A-ST TRIAC 700V 25A 100/150mA

BTA26/400A-ST TRIAC 400V 25A 100/150mA

BTA26/600A Echivalent: BTA26/700B-ST

BTA26/700B-ST TRIAC 700V 25A 50/100mA

BTA40/600 Echivalent: BTA40/700B-ST

BTA40/600B Echivalent: BTA40/700B-ST

BTA40/700B-ST TRIAC 700V 40A 50/100mA

BTA41/600B-ST TRIAC 600V 40A 80/100mA

BTB08400B Echivalent: BTA08/400C-ST

BTB08600B Echivalent: BTA08/700B-ST

BTB12/600 Echivalent: BTA12/600B-ST

BTB15/600 Echivalent: BTA16/600B-ST

BTB15/700B Echivalent: BTA16/700B-ST

BTF-050 Echivalent: BT F-050

BTF050 Echivalent: BT F-050

BTL-080 Echivalent: BT L-080

BTL-130 Echivalent: BT L-130

BTS121A-SIE N-FET 100V 22A 95W 0.1E

BTT185 Echivalent: B-TT 185

BTT195 Echivalent: B-TT 195

BTW27/600R THYR. 600V 4.7A 50MA

BTW68/800-MBR THYR. 800V 25A 50mA

BTX18/500-MBR THYR. 500V 1.6A 5MA

BU1008ADF-MOT Echivalent: BU2508DF-PHI

BU1008AF-MOT Echivalent: BU2508AF-PHI

BU104-ST Echivalent: BU406D-MBR

BU104/DIV Echivalent: BU608/DIV

BU104DP-ST Echivalent: BU406D-MBR

BU105-PHI Echivalent: BU208A-ST

BU105/DIV Echivalent: BU205-RFT

BU106 SI-N 325V 10A 50W

BU107 SI-N 300V 10A 50W

BU108/DIV Echivalent: BU208A-ST

BU108A/DIV Echivalent: BU208A/DIV

BU109 SI-N 330V 10A 85W

BU109DP Echivalent: : N.L.A.

BU110 SI-N 150V 10A 30W 15MHz

BU111 Echivalent: BU326A/DIV

BU124 Echivalent: BU124A-MBR

BU124A-MBR SI-N 400V 10A 50W 6MHZ

BU125-ST SI-N 130/60V 5A 0.8W 100M

BU125S Echivalent: BU125-ST

BU126-RFT Echivalent: BU326A/DIV

BU126-ST Echivalent: BU326A-MBR

BU128-ST Echivalent: BU128/DIV

BU128/DIV SI-N 300/200V 10A 62W

BU133/DIV SI-N 750/250V 3A 30W TO3

BU134 Echivalent: BU326A-MBR

BU137 Echivalent: BUY69A-ST

BU143 Echivalent: BUX48A-MBR

BU1506DX-PHI SI-N+D 1500V 5A 32W 0.5US

BU1508AX-PHI SI-N 1500V 8A 35W 0.6US

BU1508DF-PHI Echivalent: BU1508DX-PHI

BU1508DX-PHI SI-N+D 1500V 8A 35W 0.6US

BU180/DIV Echivalent: BU180A/DIV

BU180A/DIV SI-N+D 400V 10A

BU180E N-DARL 1500V 5A 12W

BU184-ST Echivalent: : BU806-MBR

BU189 N-DARL 330V 8A 60W

BU204 Echivalent: BU208A-ST

BU205-EI Echivalent: BU208A-ST

BU206 Echivalent: BU209/DIV

BU207 Echivalent: BU208A-ST

BU208 Echivalent: BU208A-ST

BU208- Echivalent: BU208A-ST

BU208-02 Echivalent: BU208A-ST

BU208/02 Echivalent: BU208A-ST

BU208A Echivalent: BU208A-ST

BU208A-CDL Echivalent: : BU208A-ST

BU208A-ST SI-N 1500V 8A 150W THIN B

BU208A-TOS Echivalent: : N.L.A.

BU208A/DIV SI-N 1500V 5A 130W 4MHz

BU208B SI-N 700V 5A 80W 7MHz

BU208D-ST SI-N+D 1500V 8A 150W

BU208D/DIV SI-N 1500V 8A 150W

BU209-KON Echivalent: BU209/DIV

BU209/DIV SI-N 1700V 4A 12.5W TO3

BU210 Echivalent: BU626/DIV

BU223 Echivalent: BUY69A-ST

BU225 Echivalent: : N.L.A.

BU226-TFK Echivalent: BU226/DIV

BU226/DIV SI-N 2000V 1.5A 10W TO3

BU2506DF-PHI SI-N+D 1500V 5A 45W 0.4us

BU2506DX-PHI SI-N+D 1500V 5A 45W 0.4US

BU2508A-PHI SI-N 1500V 8A 125W 0.4us

BU2508AF-PHI SI-N 1500V 8A 45W 0.4us

BU2508AW Echivalent: BU2508A-PHI

BU2508AX-PHI SI-N 1500V 8A 45W 0.4US

BU2508D-PHI SI-N+D 1500V 8A 125W 0.4u

BU2508DF-PHI SI-N+D 1500V 8A 45W 0.4us

BU2508DW Echivalent: BU2508D-PHI

BU2508DX-PHI SI-N+D 1500V 8A 45W 0.4US

BU2520A-PHI Echivalent: BU2525A-PHI

BU2520AF-PHI SI-N 1500V 10A 45W 0.2us

BU2520AX-PHI SI-N 1500V 10A 45W 0.2US

BU2520DF-PHI SI-N+D 1500V 10A 45W 0.35

BU2520DX-PHI SI-N+D 1500V 10A 45W 0.35

BU2522AF-PHI Echivalent: BU2525AF-PHI

BU2525A-PHI SI-N 1500V 12A 0.2us

BU2525AF-PHI SI-N 1500V 12A 45W 0.2us

BU2525AW Echivalent: BU2525A-PHI

BU2525AX-PHI SI-N 1500V 12A 45W 0.2US

BU2525D-PHI SI-N+D 1500V 12A 0.2us

BU2525DW Echivalent: BU2525D-PHI

BU2527AF-PHI SI-N 1500V 12A 45W 0.2us

BU2527AX Echivalent: BU2527AX-PHI

BU2527AX-PHI SI-N 1500V 12A 45W 0.2US

BU2722AF-PHI SI-N 1700V 10A 45W

BU2735AS-RHM POWER SUPPLY CONTROLER

BU2751S IC FOR AKAI

BU2820S-RHM IC

BU2896K-RHM I.C.

BU290 Echivalent: BUZ90-SIE

BU312-MBR SI-N 280/150V 6A 25W

BU323-MBR Echivalent: BU323A-MBR

BU323A-MBR Echivalent: MJ10012-MBR

BU325-ST SI-N 200/200V 3A 25W

BU326/DIV Echivalent: BU326A/DIV

BU326A Echivalent: BU326A-MBR

BU326A-MBR SI-N 900V 6A 75W

BU326A/DIV SI-N 900V 6A 75W

BU326S-RFT SI-N 800/400V 6A 60W TO3

BU406-MBR SI-N 400V 7A 65W 0.75us

BU406/DIV SI-N 400V 7A 60W

BU406D-MBR SI-N+D 400V 7A 65W 0.75us

BU406D/DIV SI-N+D 400V 7A 65W 0.75us

BU406F-PHI Echivalent: : N.L.A.

BU407-MBR SI-N 330V 7A 65W 0.75us

BU407-SSU Echivalent: BU407-MBR

BU407D-MBR SI-N+D 330V 7A 65W 0.75us

BU407D-MEV Echivalent: BU407D-MBR

BU407D-PHI Echivalent: BU407D-MBR

BU407F-MBR Echivalent: : BU406F-PHI

BU408-ST Echivalent: BU406-MBR

BU408D-PHI Echivalent: BU406D-MBR

BU408D-ST Echivalent: BU406D-MBR

BU409-ST Echivalent: BU406-MBR

BU409D-ST SI-N+D 250V 7A 60W

BU412 SI-N+D 280V 8A

BU413 SI-N 330V 10A 60W TO3

BU414B SI-N+D 900V 8A 60W

BU415A SI-N 800V 12A 120W TO3

BU415B SI-N+D 800V 12A 120W

BU426-PHI Echivalent: BU426A-MBR

BU426/DIV Echivalent: BU426A/DIV

BU426A-MBR SI-N 900V 6A 114W

BU426A-PHI Echivalent: BU426A-MBR

BU426A-TFK Echivalent: BU426A-MBR

BU426A-TI Echivalent: BU426A-MBR

BU426A/DIV SI-N 900/400V 6A 70W

BU426E-MBR SI-N 800V 6A 70W TO220

BU426V SI-N 800/375V 6A 70W

BU433 SI-N 375V 6A 70W

BU4xxxB Echivalent: CD4xxx-MBR

BU500-MOT Echivalent: BU500/DIV

BU500/DIV SI-N 1500V 6A 75W

BU500D SI-N+D 1500/700V 6A 75W

BU505-ST SI-N 1500V 2.5A 75W 0.9us

BU505D-PHI SI-N+D 1500V 2.5A 75W

BU505DF-PHI SI-N+D 1500V 2.5A 20W

BU506-PHI SI-N 700V 5A 100W POWER

BU506AF Echivalent: BU508AXI-ST

BU506D-PHI SI-N+D 700V 5A 100W POWER

BU506DF-PHI SI-N+D 1500V 5A 20W POWER

BU508-PHI Echivalent: BU508A-PHI

BU508A Echivalent: BU508A-ST

BU508A-PHI SI-N 1500V 8A 125W 0.7us

BU508A-SAN SI-N 1500V 8A 125W 0.7us

BU508A-ST SI-N 1500V 8A 125W 0.7us

BU508A/DIV Echivalent: BU508A-PHI

BU508AF Echivalent: BU508AF-PHI OR -ST

BU508AF-PHI SI-N 1500V 8A 34W 0.7us

BU508AF-SAN SI-N 1500V 8A 34W 0.7us

BU508AF-SSU Echivalent: BU508AF-PHI OR -ST

BU508AF-ST SI-N 1500V 8A 34W 0.7us

BU508AF1 Echivalent: BU508AF-ST

BU508AG-SAN Echivalent: ON4542-PHI

BU508AM-SAN Echivalent: BU508A-SAN

BU508AT Echivalent: : BU508AXI-ST

BU508AW Echivalent: BU508A-PHI

BU508AXI-ST Echivalent: : N.L.A.

BU508D Echivalent: BU508D-PHI

BU508D-PHI SI-N+D 1500V 8A 125W 0.7u

BU508D-ST SI-N+D 1500V 8A 125W 0.7u

BU508D-TFK Echivalent: BU508DR/DIV

BU508D/DIV Echivalent: BU508D-ST

BU508DF Echivalent: BU508DF-ST

BU508DF-PHI SI-N+D 1500V 8A 34W 0.7uS

BU508DF-ST SI-N+D 1500V 8A 34W 0.7us

BU508DFI Echivalent: BU508DF-ST

BU508DR-TFK Echivalent: BU508DR/DIV

BU508DR/DIV SI-N+D 1500V 8A 125W+Rbe=

BU508DW Echivalent: BU508D-PHI

BU508F Echivalent: BU508AF-PHI OR -ST

BU508V Echivalent: BU508V-PHI

BU508V-PHI Echivalent: : BU508A-PHI

BU508V/DIV Echivalent: BU508V-PHI

BU522-MBR N-DARL 400/375V 7A 75W

BU526-MBR SI-N 800V 8A 86W

BU526/DIV Echivalent: BU526A/DIV

BU526A/DIV SI-N 900/460V 8A 86W

BU536 SI-N 1100V 8A 62W

BU536-ST Echivalent: BU536

BU546-RFT Echivalent: BU546/DIV

BU546/DIV SI-N 1350V 6A 100W TO3

BU603 SI-N 1350V 5A 100W 0.7us

BU606-ST Echivalent: BU608/DIV

BU606D-ST SI-N+D 400V 7A 90W

BU606D/DIV SI-N+D 400V 7A 90W

BU607-ST Echivalent: BU608/DIV

BU607D-ST Echivalent: BU606D-ST

BU607D/DIV Echivalent: BU606D-ST

BU608 Echivalent: BU608/DIV

BU608-ST Echivalent: BU608/DIV

BU608/DIV SI-N 400V 6A 90W TO3

BU608D-ST SI-N+D 400V 7A 90W

BU608D/DIV SI-N+D 400V 7A 90W

BU626-RFT Echivalent: BU626/DIV

BU626/DIV Echivalent: : BU626A/DIV

BU626A/DIV SI-N 1000V 10A 100W

BU705 Echivalent: BU705-TFK

BU705-PHI Echivalent: : BU705-TFK

BU705-TFK SI-N 1500V 2.5A 75W 0.7us

BU706DF-PHI SI-N+D 1500V 5A 32W 0.7us

BU706F-PHI SI-N 1500V 5A 32W 0.7us

BU724 Echivalent: : N.L.A.

BU800/DIV Echivalent: BU208A-ST

BU800S Echivalent: BU208A-ST

BU801-ST SI-N+D 600V 3A 40W

BU806-MBR N-DARL+D 400V 8A 60W 0.35

BU806/DIV SI-N+D 400V 8A

BU806FI-ST SI-N+D 400V 8A

BU807-PHI Echivalent: BU806-MBR

BU807F-PHI Echivalent: BU806FI-ST

BU808DF-ST N-DARL+D 1500/700V 5A 50W

BU808DFI Echivalent: BU808DF-ST

BU810-MBR N-DARL+D 600V 7A 75W

BU824 N-DARL+D 650V 0.5A

BU826 Echivalent: : BU826/DIV

BU826/DIV N-DARL+D 800V 6A 125W 0.2

BU826A-PHI Echivalent: BU826A/DIV

BU826A/DIV N-DARL+D 900V 6A 125W 0.2

BU902 Echivalent: BU508A-ST

BU903-ST Echivalent: BU508A-ST

BU903-TFK Echivalent: BU508A-ST

BU903/DIV Echivalent: BU508A-ST

BU908-ST Echivalent: BU2508A-PHI

BU908-TFK Echivalent: BU2508A-PHI

BU908/DIV Echivalent: BU2508A-PHI

BU908A Echivalent: BU2508AF-PHI

BU908AF Echivalent: BU2508AF-PHI

BU910-ST Echivalent: BU912-ST

BU911-ST Echivalent: BU912-ST

BU912-ST Echivalent: : N.L.A.

BU920 Echivalent: BU931-ST

BU920P N-DARL 350V 10A 120W

BU921 Echivalent: BU931-ST

BU921P SI-N 400/450V 10A 120W

BU921T-ST Echivalent: BU931T-ST

BU926 Echivalent: TIPL763A-MBR

BU930 Echivalent: BU932/DIV

BU931-ST SI-N 500V 15A 175W

BU931R Echivalent: BU931-ST

BU931T-ST SI-N 450V 10A 125W

BU931ZP-ST Echivalent: BU941ZP-ST

BU932/DIV N-DARL 500V 15A 175W

BU932P-MBR Echivalent: BU941P-ST

BU932P/DIV N-DARL 500V 15A 125W

BU932R Echivalent: BU932/DIV

BU941P-ST N-DARL 500V 15A 150W

BU941ZP-ST N-DARL 350V 15A 150W

BUF405-ST Echivalent: BUF405A-ST

BUF405A-ST SI-N 1000/450V 7.5A 80W

BUF405AF-ST SI-N 1000V 7.5A ISOLATED

BUF405AHS Echivalent: BUF405A-ST

BUF405AXI-ST Echivalent: BUF405AF-ST

BUF410-ST SI-N 850V 15A 125W

BUH1015-ST SI-N 1500V 14A 160W 0.11u

BUH1015HI-ST SI-N 1500V 14A 70W 0.11us

BUH1215-ST SI-N 1500V 19A 200W 0.11u

BUH313 Echivalent: BUH315-ST

BUH313D Echivalent: BUH315D-ST

BUH313DX Echivalent: BUH315D-ST

BUH314D SI-N TRANSISTOR

BUH315-ST SI-N 1500V 5A 50W

BUH315D-ST SI-N+D 1500/700V 5A 50W

BUH315DX Echivalent: BUH315D-ST

BUH417-ST Echivalent: : N.L.A.

BUH515-ST SI-N 1500V 8A 60W 3.9us

BUH515D-ST SI-N+D 1500/700V 8A 60W

BUH517-ST SI-N 1700V 8A 60W 3.9us

BUH517D-ST SI-N+D 1700/700V 8A 60W

BUH713 Echivalent: BUH715-ST

BUH715-ST SI-N 1500V 10A 60W

BUK436/800A Echivalent: : BUK436/800B-

BUK436/800B-PH N-FET 800V 4A 125W <4E

BUK437/400A Echivalent: BUK437/400B-PHI

BUK437/400B-PH N-FET 400V 14A 180W

BUK437/600A Echivalent: BUK437/600B-PH

BUK437/600B-PH N-FET 600V 9A 180W <1E2

BUK438/800A Echivalent: BUK438/800B-PHI

BUK438/800B-PH N-FET 800V 7.6A 220W 2E

BUK443 Echivalent: BUK443/60B-PHI

BUK443/60B-PHI N-FET 60V 13A 25W

BUT70-MBR SI-N 200V 40A 200W

BUT72 SI-N 400V 40A 200W

BUT76A-TFK Echivalent: : BUT76A/DIV

BUT76A/DIV SI-N 1000V 10A 100W 0.8us

BUT90-MBR SI-N 200V 50A 250W

BUT92 SI-N 350/250V 50A 250W

BUT93 SI-N 600V 4A 55W 9MHz

BUV18-ST SI-N 120V 47A 250W 1.5us

BUV19 Echivalent: BUV20-MBR

BUV20-MBR SI-N 160V 50A 250W 1.5us

BUV21-MBR SI-N 250/200V 40A 250W

BUV22 Echivalent: BUX22-MBR

BUV23-MBR SI-N 325V 40A 250W

BUV24 SI-N 400V 30A 250W

BUV25 SI-N 500V 20A 250W

BUV26 SI-N 180V 14A 85W 1.8us

BUV27-MBR SI-N 240V 12A 65W 40ns

BUV26A SI-N 200V 20A 85W

BUV27A/DIV SI-N 300V 12A 65W 0.4us

BUV28-MBR SI-N 400V 10A 65W 40ns

BUV28/DIV SI-N 400V 10A 65W 40ns

BUV28A-MBR SI-N 450V 10A 65W 40NS

BUV37 Echivalent: : BU941P-ST

BUV46 Echivalent: BUV46A-MBR

BUV46A-MBR SI-N 1000/450V 6A 85W

BUV47 Echivalent: BUV47A

BUV47A Echivalent: BUV48A-MBR

BUV48 Echivalent: BUV48A-MBR

BUV48A-MBR SI-N 1000V 15A 150W 0.8us

BUV48A/DIV Echivalent: BUV48A-MBR

BUV48AF-MBR SI-N 1000V 15A 65W

BUV48B Echivalent: BUV48C-ST

BUV48C-ST SI-N 1200/700V 15A 150W

BUV48C/DIV SI-N 1200/700V 15A 150W

BUV48CF-ST SI-N 1200V 15A 65W

BUV50-MBR SI-N 250V 25A 150W

BUV56A-MBR SI-N 1000V 10A 70W

BUV61-MBR SI-N 300V 50A 250W

BUV62 Echivalent: BUX22-MBR

BUV70 SI-N 1300/550V 10A 140W

BUV90-MBR Echivalent: BUV90/DIV

BUV90/DIV N-DARL+D 650V 10A 125W

BUV93 SI-N 600/350V 2A 15W 12MH

BUV98A-MBR SI-N 1000V 30A 150W 5MHz

BUW11 Echivalent: BUW11A-MBR

BUW11A-MBR SI-N 1000V 5A 100W 0.8us

BUW11AF-PHI SI-N 1000V 5A 32W 0.8us

BUW11F Echivalent: BUW11AF-PHI

BUW12 SI-N 850V 8A 125W 0.8us

BUW12A-MBR SI-N 1000V 8A 125W 0.8us

BUW12A/DIV SI-N 1000V 8A 125W 0.8us

BUW12F SI-N 850V 8A 34W 0.8us

BUW13 SI-N 850V 15A 175W 0.8us

BUW13A-MBR SI-N 1000V 15A 175W 0.8us

BUW13A/DIV SI-N 1000V 15A 175W 0.8us

BUW23-MBR SI-P 450V 10A 125W <300NS

BUW26 SI-N 800V 10A 125W 20MHz

BUW32A-ST Echivalent: BUW42-MBR

BUW34 Echivalent: BUX80-MBR

BUW35 Echivalent: BUX80-MBR

BUW36 Echivalent: BUX81

BUW41B Echivalent: BUT56A/DIV

BUW42-MBR SI-P 400V 15A 150W

BUW44 Echivalent: BUX48A-MBR

BUW45 Echivalent: BUX48A-MBR

BUW46 Echivalent: BUX48A-MBR

BUW48-ST SI-N 120V 30A 150W 1.5us

BUW49-MBR SI-N 160V 30A 150W

BUW50-ST SI-N 250V 25A 150W TO-218

BUW72 SI-N 450V 10A 100W

BUW81A N-DARL 800V 10A 80W

BUW84 SI-N 800V 2A 50W 0.4us

BUW85-PHI SI-N 1000V 2A 50W 0.4us

BUX10-ST SI-N 160V 25A 150W 1.5us

BUX10/DIV SI-N 160V 25A 150W 1.5us

BUX11-ST Echivalent: BUX12-ST

BUX12-ST SI-N 300V 20A 150W

BUX12/DIV Echivalent: : BUX12-MBR

BUX13-MBR SI-N 400V 15A 150W >8MHZ

BUX20-MBR SI-N 160V 50A 350W 1.5us

BUX21-MBR Echivalent: : BUX22

BUX22-MBR SI-N 300V 40A 250W POWSWI

BUX23-MBR SI-N 400/325V 30A 350W

BUX24 SI-N 450/400V 20A 350W >8

BUX27 Echivalent: BUW11A-MBR

BUX32B SI-N 1000V 8A 150W

BUX37 N-DARL 400V 15A 35W

BUX39 Echivalent: BUX39/DIV

BUX39-MBR Echivalent: : N.L.A.

BUX39/DIV SI-N 120/90V 30A 120W 8MH

BUX40 SI-N 160V 20A 120W 1.2us

BUX41 SI-N 250V 15A 120W

BUX41N SI-N 220/160V 18A 120W

BUX42 SI-N 300V 12A 120W

BUX47 Echivalent: BUX48A-MBR

BUX47A-ST Echivalent: BUX48A-MBR

BUX47A/DIV Echivalent: BUX48A/DIV

BUX48 Echivalent: BUX48A-MBR

BUX48A-MBR SI-N 1000V 15A 175W 0.8us

BUX48A/DIV Echivalent: BUX48A-MBR

BUX51-MBR SI-N 300/200V 3.5A 10W >8

BUX54 SI-N 450V 2A 10W >8MHz

BUX55 SI-N 450V 2A 10W 8MHz

BUX66 SI-P 200/150V 2A 35W >20M

BUX77 SI-N 100V 5A 40W >2.5MHz

BUX80-MBR SI-N 800V 10A 100W

BUX80/DIV SI-N 800V 10A 100W =BUS12

BUX81 SI-N 1000V 10A 100W

BUX82 SI-N 800V 6A 60W

BUX84-MBR SI-N 800V 2A 40W 0.4us

BUX84-PHI Echivalent: BUX84-MBR

BUX84/DIV SI-N 800V 2A 40W 0.4us

BUX85-MBR SI-N 1000V 2A 40W 0.4us

BUX85/DIV Echivalent: BUX85-MBR

BUX85F-PHI SI-N 1000V 2A 18W 0.4us

BUX86 Echivalent: BUX87-MBR

BUX86F Echivalent: BUX86P-PHI

BUX86P-PHI SI-N 800V 0.5A 20W 0.4us

BUX87 Echivalent: BUX87-MBR

BUX87-MBR SI-N 1000V 0.5A 20W 0.4us

BUX87P-PHI SI-N 1000V 0.5A 20W 0.4us

BUX88-MBR SI-N 1500V 12A 160W 7MHz

BUX98 Echivalent: BUX98A-MBR

BUX98A-MBR SI-N 450V 30A 250W

BUX98C-MBR SI-N 1200V 30A 250W 5MHz

BUY18S SI-N 80/40V 10A 20W

BUY47-MBR SI-N 150/120V 7A 10W 90MH

BUY47A Echivalent: BUV48A-MBR

BUY48-MBR Echivalent: : N.L.A.

BUY49P-ST SI-N 250V 3A 10W

BUY49S-MBR SI-N 250V 3A 10W 50MHZ

BUY69A-ST SI-N 1000V 10A 100W 1us

BUY69A/DIV SI-N 1000V 10A 100W

BUY69B Echivalent: BUY69A-ST

BUY69C Echivalent: BUY69A-ST

BUY70A SI-N 1000/400V 10A 75W

BUY71 SI-N 2200V 2A 40W HORDEFL

BUY72 SI-N 280/200V 10A 60W

BUY89-MBR SI-N 1500V 6A 80W

BUZ10-ST N-FET 50V 20A 80W 0.08R

BUZ100-SIE N-FET 50V 60A 250W 0.18E

BUZ10A-ST Echivalent: BUZ10-ST

BUZ11-MBR N-FET 50V 36A =BUK455-50A

BUZ11A-MBR N-FET 50V 27A 90W 0.055R

BUZ11AF Echivalent: 2SK1190-SKN

BUZ12 Echivalent: IRFZ44-MBR

BUZ14 N-FET 50V 39A 125W

BUZ15-MBR N-FET 50V 45A 125W

BUZ171-MBR P-FET 50V 8A 40W 0.3R

BUZ20-MBR Echivalent: IRF530-MBR

BUZ21-MBR N-FET 100V 21A =BUK455-10

BUZ215-SIE N-FET 500V 5A 75W <1E5

BUZ22-SIE N-FET 100V 34A 125W 0.055

BUZ24-MBR Echivalent: : N.L.A.

BUZ24/DIV Echivalent: : BUZ24-MBR

BUZ25-ST Echivalent: BUZ24-MBR

BUZ30A-SIE N-FET 200V 7A 75W

BUZ31 Echivalent: : IRF640-MBR

BUZ310-MBR N-FET 1000V 2.5A 75W <5E

BUZ311 Echivalent: BUZ310-MBR

BUZ32-ST Echivalent: IRF640-MBR

2SC1000-TOS SI-N 55V 0.1A 0.2W 80MHz

2SC1008-MBR SI-N 80V 0.7A 0.8W 75MHz

2SC1012 Echivalent: 2SC1012A-MAT

2SC1012A-MAT SI-N 250V 60mA 0.75W >80M

2SC1013 Echivalent: 2SC1014-MIT

2SC1014-MIT SI-N 50V 1.5A 7W

2SC1017-MIT SI-N 75V 1A 60mW 120MHz

2SC1025 Echivalent: : N.L.A.

2SC1030 SI-N 150V 6A 50W

2SC1046-CDL SI-N 1000V 3A 25W

2SC1047-MAT SI-N 30V 20mA 0.4W 650MHz

2SC1050 SI-N 300V 1A 40W

2SC1051 SI-N 150V 7A 60W 8MHz

2SC1060 Echivalent: BD241C-MBR

2SC1061-HIT SI-N 50V 3A 25W 8MHz=H106

2SC1070-NEC SI-N 30V 20mA 900MHz

2SC1079 Echivalent: 2SD551

2SC1080 SI-N 110V 12A 100W 4MHz

2SC109 SI-N 50V 0.6A 0.6W

2SC1093-NEC TRANSISTOR

2SC1095 Echivalent: 2SC1226-MAT

2SC1096-NEC SI-N 40V 3A 10W 60MHz

2SC1098 Echivalent: BD241C-MBR

2SC1106 SI-N 350V 2A 80W

2SC1107 Echivalent: BD243C-MBR

2SC1114-SKN SI-N 300V 4A 100W 10MHz

2SC1114/DIV SI-N 300V 4A 100W 10MHz

2SC1115-SKN SI-N 140V 10A 100W 10MHz

2SC1116-MBR SI-N 180V 10A 100W 10MHz

2SC1124-SNY Echivalent: : N.L.A.

2SC1161 SI-P 160V 12A 120W

2SC1162-MBR SI-N 35V 1.5A 10W 180MHz

2SC1166-TOS Echivalent: 2SC1627-TOS

2SC1172 SI-N 1500V 5A 50W

2SC1173 Echivalent: BD243C-MBR

2SC1175 Echivalent: BC547B-MBR

2SC1195 SI-N 200V 2.5A 100W

2SC1209 Echivalent: BC639-MBR

2SC1210 Echivalent: BC639-MBR

2SC1211 Echivalent: BC639-MBR

2SC1212 Echivalent: BD237-MBR

2SC1213C-MBR SI-N 50V 0.5A 0.4W UNI

2SC1214-HIT SI-N 50V 0.5A 0.6W 50MHz

2SC1215-MAT SI-N 30V 50mA 0.4W 1.2GHZ

2SC1216-NEC SI-N 40V 0.2A 0.3W <20/40

2SC1222 Echivalent: BC550B-MBR

2SC1226-MAT SI-N 40/50V 2A 10W 150MHz

2SC1238-MIT SI-N 35V 0.15A 5W 1.7GHz

2SC1239-MIT Echivalent: : N.L.A.

2SC1247A-FUJ SI-N 50V 0.5A 0.4W 60MHz

2SC1306 Echivalent: 2SC1969-MIT

2SC1307-NEC Echivalent: 2SC1969-MIT

2SC1308-MBR SI-N 1500V 7A 50W

2SC1312-MIT SI-N 35V 0.1A 0.15W 100MH

2SC1313 Echivalent: BC550B-MBR

2SC1317 Echivalent: 2SC1318-MAT

2SC1318-MAT SI-N 60V 0.5A 0.6W 200MHz

2SC1325 Echivalent: BU208A-ST

2SC1327-MAT Echivalent: 2SC2634-MAT

2SC1328-MAT Echivalent: 2SC2634-MAT

2SC1343-HIT SI-N 150V 10A 100W 14MHz

2SC1345-HIT SI-N 55V 0.1A 0.1W 230MHz

2SC1346 Echivalent: 2SC1318-MAT

2SC1347 Echivalent: 2SC1318-MAT

2SC1358 Echivalent: BU208A-ST

2SC1359-MAT SI-N 30V 30mA 0.4W 250MHz

2SC1360-MAT SI-N 50V 0.05A 1W >300MHz

2SC1362-SNY SI-N 50V 0.2A 0.25W 140MH

2SC1364 Echivalent: BC546B-MBR

2SC1368-HIT SI-N 25V 1.5A 8W 180MHz

2SC1382-TOS SI-N 80V 0.75A 5W 100MHz

2SC1383 Echivalent: 2SC1384-MAT

2SC1384-MAT SI-N 60V 1A 1W 200MHz

2SC1385-HIT Echivalent: : N.L.A.

2SC1393-MBR SI-N 30V 20mA 250 mW 700M

2SC1398-MAT SI-N 70V 2A 15W

2SC1399-NEC Echivalent: : N.L.A.

2SC1400-NEC Echivalent: 2SC3244E-MIT

2SC1402 Echivalent: 2SD427-MBR

2SC1403 Echivalent: 2SC3855-SKN

2SC1406 Echivalent: 2SC1384-MAT

2SC1407 Echivalent: 2SC1384-MAT

2SC1413A-MBR Echivalent: : 2SC1413A/DIV

2SC1413A/DIV SI-N 1200V 5A 50W

2SC1419-HIT SI-N 50V 2A 20W 5MHz

2SC1426-NEC SI-N 35V 0.2A 2.7GHz

2SC143 Echivalent: 2N2369A-MBR

2SC1431-SNY SI-N 110V 2A 23W 80MHz

2SC1432 N-DARL 30V 0.3A 0.3W B=40

2SC1438 Echivalent: 2SC1439-FUJ

2SC1439-FUJ SI-N 150V 50mA 0.5W 130MH

2SC1444-SKN Echivalent: 2SC1445-SKN

2SC1445-SKN SI-N 100V 6A 40W 10MHz

2SC1446 SI-N 300V 0.1A 10W 55MHz

2SC1447-TOS SI-N 300V 0.15A 20W 80MHz

2SC1448-TOS SI-N 150V 1.5A 25W 3MHz

2SC1449-NEC SI-N 40V 2A 5W 60MHZ

2SC1450-MAT SI-N 150V 0.4A 20W

2SC1454 SI-N 300V 4A 50W 10MHz

2SC1473 Echivalent: BF420-MBR

2SC1474-3-SNY Echivalent: 2SC1474-4-SNY

2SC1474-4-SNY SI-N 20V 2A 0.75W 80MHz

2SC1475-SNY Echivalent: BC639-MBR

2SC1490 Echivalent: 2N6083

2SC1491 Echivalent: 2N6083

2SC1501-MAT SI-N 300V 0.1A 10W 55MHz

2SC1505-NEC SI-N 300V 0.2A 15W

2SC1507-NEC SI-N 300V 0.2A 15W 80MHz

2SC1509-MAT SI-N 80V 0.5A 1W 120MHz

2SC1514-HIT Echivalent: : N.L.A.

2SC1515-HIT SI-N 200V 0.05A 0.2W 110M

2SC1520-NEC SI-N 300V 0.2A 12,5W

2SC1545-RHM N-DARL 40V 0.3A 0.3W B=1K

2SC1546 Echivalent: BC517-MBR

2SC1565 Echivalent: 2SC2592-MAT

2SC1567-MAT SI-N 100V 0.5A 5W 120MHz

2SC1568 Echivalent: BD139-MBR

2SC1569 Echivalent: 2SC1505-NEC

2SC1570-SAN SI-N 55V 0.1A 0.2W 100MHz

2SC1571-SAN SI-N 40V 0.1A 0.2W 100MHz

2SC1573-MAT SI-N 200V 0.1A 1W 80MHz

2SC1576-TOS Echivalent: : N.L.A.

2SC1577-SKN SI-N 500V 8A 80W 7MHz

2SC1583-MIT SI-N 50V 0.1A 0.4W 100MHz

2SC1586-SKN Echivalent: : MJ15024-MBR

2SC1617 Echivalent: 2SD718-TOS

2SC1619 SI-N 100V 6A 50W 10MHz

2SC1623-NEC SI-N 60V 0.1A 0.2W 250MHz

2SC1624 SI-N 120V 1A 15W 30MHz

2SC1625 Echivalent: 2SC1624

2SC1626 Echivalent: 2SC3421Y-TOS

2SC1627-TOS SI-N 80V 0.4A 0.8W 100MHz

2SC1663 Echivalent: BF460 MPSA10

2SC1669 Echivalent: 2SC3073-TOS

2SC1670 Echivalent: BF393-CDL

2SC1674-NEC SI-N 30V .02A 600MC RF/IF

2SC1675-NEC SI-N 50V .03A 0.25W

2SC1678-TOS SI-N 65V 3A 3W

2SC1681 Echivalent: 2SC2240-TOS

2SC1683 Echivalent: 2SC2592-MAT

2SC1684 Echivalent: 2SC1685-MAT

2SC1685-MAT SI-N 60V 0.1A 150MC UNI

2SC1688-MAT SI-N 50V 30mA 0.4W 550MHz

2SC1708 Echivalent: 2SC1708A-MIT

2SC1708A-MIT SI-N 120V 50mA 0.2W 150MH

2SC1722 Echivalent: 2SC1505-NEC

2SC1729-MIT SI-N 35V 3.5A 16W 500MHz

2SC1730-MBR SI-N 30V 0.05A 1.1GHz UHF

2SC1735 Echivalent: 2SD667-HIT

2SC1740-RHM SI-N 40V 100mA 0.3W

2SC1741-RHM SI-N 40V 0.5A 0.3W 250MHz

2SC1755 Echivalent: 2SC1756-SAN

2SC1756-SAN SI-N 300V 0.2A >50MHz

2SC1760-SNY SI-N 100V 1A 7.9W 80MHz

2SC1775-HIT Echivalent: 2SC1775A-HIT

2SC1775A-HIT SI-N 120V 0.05A 0.2W UNI

2SC1781-HIT SI-N 50V 0.5A 0.35W

2SC1788 Echivalent: 2SD1302-MAT

2SC1815-MBR SI-N 50V 0.15A 0.4W 80MHz

2SC1815BL-MBR SI-N 60V 0.15A 0.4W B>350

2SC1815GR-MBR SI-N 60V 0.15A 0.4W B>200

2SC1815Y-MBR SI-N 60V 0.15A 0.4W B>120

2SC1816 Echivalent: 2SC2075-TOS

2SC1817 Echivalent: 2SC1969-MIT

2SC1826 Echivalent: 2SC1827-SKN

2SC1827-SKN SI-N 100V 4A 30W 10MHz

2SC1832-SKN N-DARL 500V 15A 150W B>10

2SC1841-NEC SI-N 120V 0.05A 0.5W

2SC1844-NEC SI-N 60V 0.1A 0.5W 100MHz

2SC1845-NEC SI-N 120V 0.05A 0.5W

2SC1846-MAT SI-N 120V 0.05A 0.5W

2SC1847-MAT SI-N 50V 1.5A 1.2W

2SC1855-HIT SI-N 20V 20mA 0.25W 550MH

2SC1856 Echivalent: 2SC1855-HIT

2SC1871-NEC SI-N 450V 15A 150W <1/3us

2SC1875 Echivalent: BU208A-ST

2SC1879-HIT N-DARL+D 120V 2A 0.8W B>1

2SC1885 Echivalent: BF420-MBR

2SC1890-HIT SI-N 90V 0.05A 0.3W 200MH

2SC1893 Echivalent: 2SC1895-TOS

2SC1895-TOS SI-N 1500V 6A 50W 2MHz

2SC1903 Echivalent: 2SC2911-SAN

2SC1904-FUJ Echivalent: 2SC2911-SAN

2SC1906-MBR SI-N 19V 0.05A 0.3W

2SC1907-HIT SI-N 30V 0.05A 1100MHz

2SC1909 Echivalent: : N.L.A.

2SC1913-MAT SI-N 150V 1A 15W 120MHz

2SC1914-MIT SI-N 90V 50mA 0.2W 150MHz

2SC1915-MIT Echivalent: 2SD1292-RHM

2SC1921-HIT SI-N 250V 0.05A 0.6W

2SC1922-MBR SI-N 1500V 2.5A 50W

2SC1923-TOS SI-N 30V 20mA 10mW 550MHz

2SC1929-MAT SI-N 300V 0.4A 25W 80MHz

2SC1940 Echivalent: 2SC1941-NEC

2SC1941-NEC SI-N 160V 50mA 0.8W

2SC1942 Echivalent: BU208A-ST

2SC1944-MIT SI-N 80V 6A PQ=16W

2SC1945-MIT SI-N 80V 6A 20W

2SC1946A-MIT SI-N 35V 7A 50W

2SC1947-MIT SI-N 35V 1A 4W/175MHz

2SC1953-MAT SI-N 150V 0.05A 1.2W 70MH

2SC1957-NEC SI-N 40V 1A 1.8W/27MHz

2SC1959-MBR SI-N 30V 0.5A 0.5W 200MHz

2SC1964 Echivalent: 2SC2075-TOS

2SC1966 Echivalent: 2SC1967-MIT

2SC1967-MIT SI-N 35V 2A 8W 470MHz

2SC1968 SI-N 35V 5A 3W 470MHz

2SC1969-MIT SI-N 60V 6A 20W

2SC1969/DIV SI-N 60V 6A 20W

2SC1970-MIT SI-N 40V 0.6A 5W

2SC1971-MIT SI-N 35V 2A 12.5W

2SC1972-MIT SI-N 35V 3.5A 25W

2SC1973-MBR Echivalent: : N.L.A.

2SC1975-MAT SI-N 120V 2A 3.8W 50MHz

2SC1980-MAT SI-N 120V 20mA 0.25W 200M

2SC1983 Echivalent: BD241C-MBR

2SC1984-SKN SI-N 100V 3A 30W B=700

2SC1985-SKN SI-N 80V 6A 40W 10MHz

2SC1986 Echivalent: TIP41C-MBR

2SC2001 Echivalent: BC337-25-MBR

2SC2002 Echivalent: BC639-MBR

2SC2003 Echivalent: BC639-MBR

2SC2021 Echivalent: 2SC1740-RHM

2SC2023-SKN SI-N 300V 2A 40W 10MHz

2SC2023/DIV SI-N 300V 2A 40W 10MHZ

2SC2026-NEC SI-N 30V 0.05A 0.25W

2SC2027 SI-N 1500/800V 5A 50W

2SC2028-FUJ Echivalent: 2SC1957-NEC

2SC2029 Echivalent: 2SC2075-TOS

2SC2036-TOS SI-N 80V 1A PQ=1..4W

2SC2037-NEC Echivalent: : N.L.A.

2SC2053-MIT SI-N 40V 0.3A 0.6W 500MHz

2SC2055-MIT SI-N 18V 0,3A 0,5W

2SC2058-RHM SI-N 40V 0.05A 0.25W

2SC2060-RHM SI-N 40V 0.7A 0.75W 150MH

2SC2061-RHM SI-N 80V 1A 0.75W 120MHz

2SC2063 Echivalent: 2SC2058-RHM

2SC2068-TOS SI-N 300V 0.05A 95MHz

2SC2071 Echivalent: BF471-PHI

2SC2073-MBR SI-N 150V 1.5A 25W 4MHz

2SC2076 Echivalent: 2SC1359-MAT

2SC2078-SAN SI-N 80V 3A 10W 150MHz

2SC2085-MAT Echivalent: 2SC1446

2SC2086-MIT SI-N 75V 1A 0.45W/27MHz

2SC2088-TOS Echivalent: 2SC2389-RHM

2SC2092 Echivalent: 2SC2092/DIV

2SC2092-HIT Echivalent: : N.L.A.

2SC2092/DIV SI-N 75V 3A 5W 27MHZ

2SC2094-MIT SI-N 40V 3.5A PQ>15W 175M

2SC2097-MIT SI-N 50V 15A PQ=85W

2SC2098 Echivalent: MRF475

2SC2120-MBR SI-N 30V 0.8A 0.6W 120MHz

2SC2122 SI-N 800V 10A 50W

2SC2141-SNY Echivalent: : N.L.A.

2SC2153 Echivalent: 2SC1215-MAT

2SC2166-MIT SI-N 75V 4A 12.5W RFPOWER

2SC2166/DIV SI-N 75V 4A 12.5W RFPOWER

2SC2167-SKN Echivalent: 2SC2168-SKN

2SC2168-SKN SI-N 200V 2A 30W 10MHz

2SC2200-TOS SI-N 500V 7A 40W 1US

2SC2209-MAT SI-N 50V 1.5A 10W 150MHz

2SC2216-TOS SI-N 45V 50mA 0.3W 300MHz

2SC2228-SAN SI-N 160V 0.05A 0.75W >50

2SC2229-MBR SI-N 200V 50mA 0.8W 120MH

2SC2230-TOS SI-N 200V 0.1A 0.8W 50MHz

2SC2233-MBR SI-N 200V 4A 40W 8MHz

2SC2235-TOS SI-N 120V 0.8A 0.9W 120MH

2SC2236-TOS SI-N 30V 1.5A 0.9W 120MHz

2SC2237-MIT SI-N 35V 2A PQ>7.5W 175MH

2SC2238-MBR SI-N 160V 1.5A 25W 100MHZ

2SC2238/DIV SI-N 160V 1.5A 25W 100MHZ

2SC2240-TOS SI-N 120V 50mA .3W 100MHz

2SC2258 Echivalent: BF459-MBR

2SC2260 Echivalent: 2SC2261-SKN

2SC2261-SKN SI-N 180V 8A 80W 15MHz

2SC2267-HIT SI-N 400/360V 0.1A 0.4W

2SC2270-TOS SI-N 50V 5A 10W 100MHz

2SC2271-SAN SI-N 300V 0.1A 0.9W 50MHz

2SC2274 Echivalent: BC639-MBR

2SC2275-NEC SI-N 120V 1.5A 25W 200MHz

2SC2278-HIT Echivalent: : N.L.A.

2SC2283 SI-N 38V 0.75A 2.8W(500MH

2SC2287-NEC SI-N 38V 1.5A 7.1W 175MHz

2SC2291-MIT Echivalent: : N.L.A.

2SC2295-MAT SI-N 30V 0.03A 0.2W 250MH

2SC2307-SKN SI-N 500V 12A 100W 18MHz

2SC2308-HIT SI-N 55V 0.1A 0.2W 230MHz

2SC2310-MBR SI-N 55V 0.1A 0.2W 230MHz

2SC2312-MIT SI-N 60V 6A 18.5W/27MHz

2SC2314-SAN SI-N 45V 1A 5W

2SC2315-SKN Echivalent: : N.L.A.

2SC2320-MIT SI-N 50V 0,2A 0,3W

2SC2328A-TOS Echivalent: : N.L.A.

2SC2329-NEC SI-N 38V 0.75A 2W(175MHz)

2SC2330-NEC Echivalent: : N.L.A.

2SC2331-NEC SI-N 150V 2A 15W POWER

2SC2333-NEC SI-N 500/400V 2A 40W

2SC2334-MBR SI-N 150V 7A 40W POWER =K

2SC2335-MBR SI-N 500V 7A 40W POWER

2SC2336-NEC Echivalent: : 2SC2336B-NEC

2SC2336A-NEC Echivalent: 2SC2336B-NEC

2SC2336B-NEC SI-N 250V 1.5A 25W 95MHz

2SC2337 Echivalent: MJ15024-MOT

2SC2344-SAN SI-N 180V 1.5A 25W 120MHz

2SC2347-TOS SI-N 15V 50mA 250mW 650MH

2SC2362-SAN SI-N 120V 50mA 0.4W 130MH

2SC2363-SAN SI-N 120V 50mA 0.5W 130MH

2SC2365 SI-N 600V 6A 50W POWER

2SC2369-NEC SI-N 25V 70mA 0.25W 4.5GH

2SC2371-NEC Echivalent: BF459-MBR

2SC2373 Echivalent: MJE13007-MBR

2SC2375 Echivalent: 2SC2910-SAN

2SC2383-TOS SI-N 160V 1A 0.9W 100MHz

2SC2389-RHM SI-N 120V 50mA 0.3W 140MH

2SC2407-NEC SI-N 35V 0.15A 0.16W 500M

2SC2412-RHM SI-N 50V 0.1A 180MHz

2SC2433-FUJ SI-N 120V 30A 150W 80MHz

2SC2440 SI-N 450V 5A 40W

2SC2441 Echivalent: 2SC2911-SAN

2SC245-NEC Echivalent: : N.L.A.

2SC2458-TOS SI-N 50V 0.15A 0.2W 80MHz

2SC2466-HIT SI-N 30V 0.05A 2.2GHz

2SC2481-TOS Echivalent: : N.L.A.

2SC2482-MBR SI-N 300V 0.1A 0.9W 50MHz

2SC2485-MAT SI-N 100V 6A 70W 15MHz

2SC2486-MAT SI-N 120V 7A 80W 15MHz

2SC2491-SKN SI-N 100V 6A 40W 15MHz

2SC2492 Echivalent: MJ15003-MBR

2SC2497-MAT SI-N 70V 1.5A 5W 150MHz

2SC2498-TOS SI-N 30V 0.05A 0.3W 3.5GH

2SC2500 Echivalent: 2SD1207-SAN

2SC2501 Echivalent: 2SC2502-SHI

2SC2502-SHI Echivalent: : N.L.A.

2SC2508-TOS SI-N 40V 6A 50W 175MHZ

2SC2510-TOS SI-N 55V 20A 250W(28MHZ)

2SC2512-HIT SI-N 30V 50mA 900MHz TUNE

2SC2516-NEC SI-N 150V 5A 30W <0.5/2us

2SC2517-NEC SI-N 150V 5A 30W <0.5/2us

2SC2525-FUJ Echivalent: 2SC3264-SKN

2SC2526-FUJ Echivalent: 2SC3264-SKN

2SC2538-MIT SI-N 40V 0.4A 0.7W

2SC2539-MIT SI-N 35V 4A 17W 175MHz

2SC2542-FEC SI-N 450V 5A 40W

2SC2546 Echivalent: 2SC2240-TOS

2SC2547-HIT SI-N 120V 0.1A 0.4W

2SC2551-TOS SI-N 300V 0.1A 0.4W 80MHz

2SC2552-TOS SI-N 500V 2A 20W

2SC2553-TOS SI-N 500V 5A 40W 1us

2SC2555 Echivalent: BU426A-MBR

2SC2562-TOS SI-N 60V 5A 25W 0.1us

2SC2563-TOS SI-N 120V 8A 80W 90MHz

2SC2564 Echivalent: 2SC3280-MBR

2SC2565 Echivalent: 2SC3281-TOS

2SC2570A-NEC SI-N 25V 70mA 0.6W

2SC2577-SKN Echivalent: 2SC2581-SKN

2SC2578-SKN Echivalent: : 2SC2579-SKN

2SC2579-SKN Echivalent: : 2SC2581-SKN

2SC2579/DIV SI-N 160V 8A 80W 20MHz

2SC2580 Echivalent: 2SC2581-SKN

2SC2581-SKN Echivalent: : 2SC3855-SKN

2SC2581/DIV SI-N 200V 10A 100W

2SC2588-NEC Echivalent: : N.L.A.

2SC2590-MAT SI-N 120V 0.5A 5W 250MHz

2SC2591-MAT Echivalent: 2SC2592-MAT

2SC2592-MAT SI-N 180V 1A 20W 250MHz

2SC2603-MIT SI-N 50V 0.2A 0.3W

2SC2607 Echivalent: 2SC2608

2SC2608 Echivalent: : N.L.A.

2SC2610-HIT SI-N 300V 0.1A 0.8W 80MHz

2SC2611-MBR SI-N 300V 0.1A 0.8W 80MHz

2SC2621E-SAN SI-N 300V 0.2A 10W >50MHz

2SC2625-FEC SI-N 450V 10A 80W

2SC2626 Echivalent: BUW13A-MBR

2SC2630-MIT SI-N 35V 14A 100W

2SC2631-MAT SI-N 150V 50mA 0,75W 160M

2SC2632-MAT SI-N 150V 50mA 1W 160MHz

2SC2634-MAT SI-N 60V 0.1A 0.4W 200MHz

2SC2653-MAT SI-N 350V 0.2A 15W >50MHz

2SC2654-NEC SI-N 40V 7A 40W

2SC2655-TOS SI-N 50V 2A 0.9W 0.1us

2SC2656-FEC SI-N 450V 7A 80W <1.5/4.5

2SC2660-MAT SI-N 200V 2A 30W 30MHz

2SC2665 Echivalent: 2SC3855-SKN

2SC2668-TOS SI-N 30V 20mA 0.1W 550MHz

2SC2671-MAT SI-N 15V 80mA 0.6W 5.5GHz

2SC2682-NEC SI-N 180V 0.1A 8W 180MHz

2SC2688 Echivalent: BF459-MBR

2SC2690-MBR SI-N 120V 1.2A 20W 160MHz

2SC2694-MIT SI-N 35V 20A 140W

2SC2704 Echivalent: 2SC3423-TOS

2SC2705-TOS SI-N 150V 50mA 0.8W 200MH

2SC2706-TOS SI-N 140V 10A 100W 90MHZ

2SC2706/DIV SI-N 140V 10A 100W 90MHZ

2SC2712-TOS SI-N 50V 0.15A 0.15W 80MH

2SC2714-TOS SI-N 30V 20mA 0.1W 550MHz

2SC2717-TOS SI-N 30V 50mA 0.3W 300MHz

2SC2719 Echivalent: BC639-MBR

2SC2724-MIT SI-N 30V 30mA 200MHz

2SC2738-MAT Echivalent: 2SC3868-MAT

2SC2749-NEC SI-N 500V 10A 100W 50MHz

2SC2750-NEC SI-N 150V 15A 100W POWER

2SC2751-NEC SI-N 500V 15A 120W 50MHz

2SC2752-NEC SI-N 500V 0.5A 10W <1/3.5

2SC2753-TOS SI-N 17V 0.07A 0.3W 5GHz

2SC2759-NEC SI-N 30V 50mA 0.2W 2.3GHz

2SC2773 Echivalent: 2SC3264-SKN

2SC2774-SKN Echivalent: 2SC3858-SKN

2SC2785 Echivalent: BC546B-MBR

2SC2786-NEC SI-N 20V 20mA 600MHz

2SC2787-NEC SI-N 50V 30mA 0.3W 250MHz

2SC2789-SHI Echivalent: 2SC4138-SKN

2SC2791 SI-N 900V 5A 100W

2SC2792-TOS SI-N 850V 2A 80W

2SC2793-TOS SI-N 900V 5A 100W

2SC2802-NEC SI-N 300V 0.2A 10W 80MHz

2SC2808-RHM SI-N 100V 50mA 0.5W 140MH

2SC2810-SKN SI-N 500V 7A 50W 18MHz

2SC2812-SAN SI-N 55V 0.15A 0.2W 100MH

2SC2814-SAN SI-N 30V 0.03A 320MHz F

2SC2824 Echivalent: 2SC3421Y-TOS

2SC2825-SKN SI-N 80V 6A 70W B>500

2SC2832 Echivalent: 2SC3972-MAT

2SC2833 Echivalent: 2SC3211-MAT

2SC2834A-MAT Echivalent: : N.L.A.

2SC2837-SKN SI-N 150V 10A 100W 70MHz

2SC2838 Echivalent: 2SC3264-SKN

2SC2839-SAN SI-N 20V 30mA 0.15W 320MH

2SC2851-MAT SI-N 36V 0.3A 1W 1.5GHz

2SC2873-TOS SI-N 50V 2A 0.5W 120MHz

2SC2877 Echivalent: 2SC3422Y-TOS

2SC2878-TOS SI-N 20V 0.3A 0.4W 30MHz

2SC2879-TOS SI-N 45V 25A PEP=100W 28M

2SC2882-TOS SI-N 90V 0.4A 0.5W 100MHz

2SC288A-NEC SI-N 35V 20mA 0.15W

2SC2898-MBR SI-N 500V 8A 50W

2SC2898/DIV SI-N 500V 8A 50W

2SC2901-NEC SI-N 40V 0.2A 0.6W <12/18

2SC2908-NEC SI-N 200V 5A 50W 50MHz

2SC2909 Echivalent: 2SC2910-SAN

2SC2910-SAN SI-N 160V 70mA 0.9W 150MH

2SC2911-SAN SI-N 180V 140mA 10W 150MH

2SC2912-SAN SI-N 200V 140mA 10W 150MH

2SC2921 Echivalent: 2SC2922-SKN

2SC2922-SKN SI-N 180V 17A 200W 50MHz

2SC2923-MAT SI-N 300V 0.1A 140MHz

2SC2928-HIT SI-N 1500V 5A 50W

2SC2938 Echivalent: 2SC2939-SHI

2SC2939-SHI SI-N 500V 10A 100W 2.5us

2SC2944-FEC Echivalent: : N.L.A.

2SC2958-NEC SI-N 160V 0.5A 1W

2SC2979-HIT SI-N 800V 3A 40W

2SC2979/DIV SI-N 800V 3A 40W

2SC2987-NEC SI-N 140V 12A 120W 60MHz

2SC2988-MAT SI-N 36V 0.5A 175MHz

2SC2999-SAN SI-N 20V 30mA 750MHz

2SC3001-MIT SI-N 20V 3A PQ=7W(175MHz)

2SC3012 Echivalent: 2SC2837-SKN

2SC3019-MIT SI-N 35V 0.4A 0.6W 520MHz

2SC3020-MIT SI-N 35V 1A 10W

2SC3022-MIT SI-N 35V 7A 50W

2SC3025-HIT Echivalent: 2SC3026-HIT

2SC3026-HIT SI-N 1700V 5A 50W POWER

2SC3030-FEC N-DARL 900V 7A 80W

2SC3039-MBR SI-N 500V 7A 52W

2SC3040 Echivalent: BU508A-ST

2SC3042-SAN SI-N 500/400V 12A 100W

2SC3052F-MIT SI-N 50V 0.2A 0.15W 200MH

2SC3063-MAT SI-N 300V 0.1A 1.2W 140MH

2SC3066-SAN Echivalent: 2SC3067-SAN

2SC3067-SAN 2xSI-N 130V 50mA 0.5W 160

2SC3068-SAN SI-N 30V 0.3A Ueb=15V B>8

2SC3070 Echivalent: 2SC3225-TOS

2SC3071-SAN SI-N 120V 0.2A Ueb=15V B>

2SC3073-TOS SI-N 30V 3A 15W 100MHz

2SC3074-TOS SI-N 60V 5A 20W 120MHZ

2SC3075-TOS SI-N 500V 0.8A 10W 1/1.5U

2SC3089-SAN SI-N 800V 7A 80W

2SC3101-MIT SI-N 250V 30A 200W 25MHz

2SC3102-MIT SI-N 35V 18A 170W 520MHz

2SC3112-TOS SI-N 50V 0.15A 0.4W 100MH

2SC3113 Echivalent: 2SC3112-TOS

2SC3116-SAN SI-N 180V 0.7A 10W 120MHz

2SC3117-SAN SI-N 180V 1.5A 10W 120MHz

2SC3133-MIT SI-N 60V 6A 1.5W 27MHz

2SC3148-TOS SI-N 900V 3A 40W 1us

2SC3148/DIV SI-N 900V 3A 40W 1US

2SC3150-SAN SI-N 900V 3A 50W 15MHz

2SC3150/DIV Echivalent: : 2SC3150-SAN

2SC3151 Echivalent: BUW11A-MBR

2SC3152 Echivalent: 2SC3153-SAN

2SC3153-SAN SI-N 900V 6A 100W

2SC3153/DIV Echivalent: : 2SC3153-SAN

2SC3155 Echivalent: 2SC3153-SAN

2SC3156-SAN Echivalent: 2SC3153-SAN

2SC3157-NEC SI-N 150V 10A 60W

2SC3158-NEC SI-N 500V 7A 60W

2SC3164-SHI SI-N 500V 10A 100W

2SC3169-MAT SI-N 500V 2A 25W >8MHz

2SC3175-SAN SI-N 400V 7A 50W 40MHz

2SC3178-FUJ SI-N 1200V 2A 60W

2SC3179-SKN SI-N 60V 4A 30W 15MHz

2SC3180N-TOS SI-N 80V 6A 60W 30MHz

2SC3181N-TOS SI-N 120V 8A 80W 30MHz

2SC3181N/DIV SI-N 120V 8A 80W 30MHZ

2SC3182N-TOS SI-N 140V 10A 100W 30MHz

2SC3182N/DIV SI-N 140V 10A 100W 30MHZ

2SC3195-KEC SI-N 30V 20mA 0.1W 550MHz

2SC3197 Echivalent: KTC3197-KEC

2SC3198 Echivalent: KTC3198-KEC

2SC3199-KEC SI-N 60V 0.15A 0.2W 130MH

2SC3200-KEC SI-N 120V 0.1A 0.3W 100MH

2SC3202 SI-N 35V 0.5A 0.5W 300MHz

2SC3203-KEC SI-N 35V 0.8A 0.6W 120MHz

2SC3205-KEC SI-N 30V 2A 1W 120MHz

2SC3206-KEC SI-N 150V 0.5A 0.8W 120MH

2SC3210-MAT SI-N 500V 10A 100W 1us

2SC3211-MAT SI-N 800V 5A 70W >3MHz

2SC3212-MAT SI-N 800V 7A 3W 3.5MHz

2SC3225-TOS SI-N 40V 2A 0.9W 1us

2SC3231-KEC SI-N 200V 4A 40W 8MHZ

2SC3240-MIT SI-N 50V 25A 110W 30MHz

2SC3242-MIT SI-N 20V 2A 0.9W 80MHz

2SC3244E-MIT SI-N 100V 0.5A 0.9W 130MH

2SC3245 Echivalent: 2SC3245A-MIT

2SC3245A-MIT SI-N 150V 0.1A 0.9W 200MH

2SC3246-MIT SI-N 30V 1.5A 0.9W 130MHz

2SC3247-MIT SI-N 50V 1A .9W 130MHz B>

2SC3253 Echivalent: 2SC3258-TOS

2SC3257-TOS SI-N 250V 10A 40W 1/3.5us

2SC3258-TOS SI-N 100V 5A 30W 120MHz

2SC3259-SHI Echivalent: : N.L.A.

2SC3260-SHI N-DARL 800V 3A 50W B>10

2SC3261 Echivalent: 2SC3262-SHI

2SC3262-SHI N-DARL 800V 10A 100W

2SC3263-SKN SI-N 230V 15A 130W

2SC3264-SKN SI-N 230V 17A 200W 60MHz

2SC3271-RHM SI-N 300V 1A 5W 80MHz

2SC3277-SAN SI-N 500V 10A 90W 20MHz

2SC3279-TOS SI-N 10V 2A 0.75W 150MHz

2SC3280-MBR SI-N 160V 12A 120W 30MHz

2SC3280-TOS Echivalent: : N.L.A.

2SC3281-TOS SI-N 200V 15A 150W 30MHz

2SC3281/DIV SI-N 200V 15A 150W 30MHZ

2SC3284-SKN SI-N 150V 14A 125W 60MHz

2SC3284/DIV SI-N 150V 14A 125W 60MHZ

2SC3293-SAN N-DARL+D 50V 1.2A 20W 180

2SC3297-TOS SI-N 30V 3A 15W 100MHz

2SC3298-TOS Echivalent: : 2SC4159-SAN

2SC3298/DIV Echivalent: 2SC3298-TOS

2SC3299-TOS SI-N 60V 5A 20W 0.1us

2SC3300-SKN SI-N 100V 15A 100W

2SC3303-TOS SI-N 100V 5A 20W 0.2us

2SC3306-TOS SI-N 500V 10A 100W 1us

2SC3307-TOS SI-N 900V 10A 150W 1us

2SC3309-TOS SI-N 500V 2A 20W 1us

2SC3309/DIV SI-N 500V 2A 20W 1US

2SC3310-TOS SI-N 500V 5A 30W 1us

2SC3311-MAT SI-N 60V 0.1A 0.3W 150MHz

2SC3320-FEC SI-N 500V 15A 80W

2SC3318 Echivalent: 2SC3320-FEC

2SC3326-TOS SI-N 20V 0.3A 0.15W 30MHz

2SC3327-TOS SI-N 50V 0.3A 0.2W 30MHZ

2SC3328-TOS SI-N 80V 2A 0.9W 100MHz

2SC3330-SAN SI-N 60V 0.2A 0.3W 200MHz

2SC3331-MBR SI-N 60V 0.2A 0.5W 200MHz

2SC3332-SAN SI-N 180V 0.7A 0.7W 120MH

2SC3334-TOS SI-N 250V 50mA 0.9W 100MH

2SC3345-TOS SI-N 60V 12A 40W 90MHz

2SC3346-TOS SI-N 80V 12A 40W 0.2us

2SC3353A-MAT Echivalent: : N.L.A.

2SC3355-NEC SI-N 20V 0.1A 0.6W 6.5GHz

2SC3356-NEC SI-N 20V 0.1A 0.2W 7GHz

2SC3377-RHM SI-N 40V 1A 0.6W 150MHz

2SC3378-TOS SI-N 120V 0.1A 0.2W 100MH

2SC3379-MIT SI-N 20V 1.5A PQ=3W

2SC3381-TOS 2xSI-N 80V 0.1A 0.4W 170M

2SC3383-SAN SI-N 60V 0.2A 0.5W 250MHz

2SC3397-SAN SI-N 50V 0.1A 250MHz 46K/

2SC3399-SAN SI-N 50V 0.1A 250MHz

2SC3400-SAN SI-N 50V 0.1A 250MHz 22K/

2SC3401-SAN SI-N 50V .1A 46K/23KOHM

2SC3402-SAN SI-N 50V 0.1A 250MHz 10K/

2SC3405-TOS SI-N 900V 0.8A 20W 1us

2SC3409 SI-N 900V 2A 80W .8uS

2SC3416-SAN SI-N 200V 0.1A 5W 70MHz

2SC3419-TOS SI-N 40V 0.8A 5W 100MHz

2SC3420-TOS SI-N 50V 5A 10W 100MHz

2SC3421O-TOS SI-N 120V 1A 1.5W BJT O-G

2SC3421Y-TOS SI-N 120V 1A 10W 120MHz

2SC3422Y-TOS SI-N 40V 3A 10W 100MHz

2SC3423-TOS SI-N 150V 50mA 5W 200MHz

2SC3425-TOS SI-N 500V 0.8A 10W

2SC3446-SAN SI-N 800V 7A 40W 18MHz

2SC3447-SAN SI-N 800V 5A 50W 18MHz

2SC3456-SAN SI-N 1100/800V 1.5A 40W

2SC3457-SAN SI-N 1100V 3A 50W

2SC3458 Echivalent: 2SC3459-SAN

2SC3459-SAN Echivalent: 2SC3460-SAN

2SC3460-SAN SI-N 1100V 6A 100W

2SC3460/DIV Echivalent: 2SC3460-SAN

2SC3461-SAN SI-N 1100/800V 8A 120W

2SC3463 Echivalent: 2SC3460-SAN

2SC3464 Echivalent: 2SC3461-SAN

2SC3466-SAN SI-N 1200/650V 8A 120W

2SC3467-SAN SI-N 200V 0.1A 1W 150MHz

2SC3468-SAN SI-N 300V 0.1A 1W 150MHz

2SC3481-SAN Echivalent: : 2SC3482

2SC3482 Echivalent: : N.L.A.

2SC3486-SAN SI-N 1500V 6A 120W

2SC3502-MBR SI-N 200V 0.1A 1.2W

2SC3503-SAN SI-N 300V 0.1A 7W 150MHz

2SC3504-SAN SI-N 70V 0.05A 0.9W 500MH

2SC3505-FUJ SI-N 900V 6A 80W

2SC3506 Echivalent: BUW11A-MBR

2SC3507-MAT SI-N 1000/800V 5A 80W

2SC3509-MAT N-DARL+D 900V 10A 100W 0.

2SC3514-NEC SI-N 180V 0.1A 10W 200MHz

2SC3518-NEC SI-N 60V 5A 10W

2SC3519 Echivalent: 2SC3263-SKN

2SC3520-SKN SI-N 500V 18A 130W 18MHZ

2SC3526-MAT SI-N 110V 0.15A 7A 30W 1u

2SC3527 Echivalent: 2SC3528-MAT

2SC3528-MAT SI-N 500V 20A 125W

2SC3546 Echivalent: 2SC3280-MBR

2SC3549 SI-N 900V 3A 40W

2SC3552-SAN SI-N 1100V 12A 150W 15MHz

2SC3559 Echivalent: 2SC3752-SAN

2SC3568-NEC SI-N 150V 10A 30W

2SC3571-NEC SI-N 500V 7A 30W

2SC3577-MAT SI-N 850V 5A 80W 6MHz

2SC3581 SI-N 55V 0.4A 0.9W 150MHz

2SC3591-SAN SI-N 400V 7A 50W

2SC3595-SAN SI-N 30V 0.5A 5W 2GHz

2SC3596-SAN SI-N 80V 0.3A 8W 700MHz

2SC3597-SAN SI-N 80V 0.5A 10W 800MHz

2SC3598 Echivalent: 2SC3599-SAN

2SC3599-SAN SI-N 120V 0.3A 8W 500MHz

2SC3600-SAN SI-N 200V 0.1A 7W 400MHz

2SC3601-SAN SI-N 200V 0.15A 7W 400MHz

2SC3605 Echivalent: 2SC2570A-NEC

2SC3608-TOS SI-N 20V 0.08A 6.5GHz

2SC3611 SI-N 50V 0.15A 4W 300MHZ

2SC3616-NEC SI-N 25V 0.7A 250MHz

2SC3621-TOS SI-N 150V 1.5A 10W 100MHz

2SC3623-NEC SI-N 60V 0.15A 0.25W B=1K

2SC3632-NEC SI-N 600V 1A 10W 30MHz

2SC3636-SAN SI-N 900/500V 7A 80W

2SC3642-SAN SI-N 1200V 6A 100W 200ns

2SC3655-SAN SI-N 50V 0.1A 0.4W 46/23K

2SC3656-SAN SI-N 50V 0.1A 0.4W 10K/10

2SC3657 Echivalent: BUW11A-MBR

2SC3659 SI-N+D 1700/800V 5A 50W

2SC366 Echivalent: 2SC1959-MBR

2SC3668 SI-N 50V 2A 1W 100MHz

2SC3669-TOS SI-N 80V 2A 1W 0.2us

2SC3675-SAN SI-N 1500/900V 0.1A 10W

2SC3678-SKN SI-N 900V 3A 80W

2SC3679-SKN SI-N 900/800V 5A 100W

2SC3679/DIV SI-N 900/800V 5A 100W

2SC3680-SKN SI-N 900/800V 7A 120W 6MH

2SC3680/DIV SI-N 900/800V 7A 120W 6MH

2SC3684-SAN SI-N+D 1500V 10A 150W

2SC3685 Echivalent: 2SC3688-SAN

2SC3686 Echivalent: 2SC3688-SAN

2SC3687 Echivalent: 2SC3688-SAN

2SC3688-SAN SI-N 1500V 10A 150W 0.2us

2SC3688/DIV SI-N 1500V 10A 150W 0.2US

2SC369 Echivalent: 2SC2240-TOS

2SC3691 Echivalent: 2SC3692-NEC

2SC3692-NEC SI-N 100V 7A 30W <300/180

2SC371 Echivalent: 2SC1815-MBR

2SC3715 Echivalent: 2SD2125-TOS

2SC372-TOS Echivalent: 2SC1815-MBR

2SC3729 Echivalent: BU508A-PHI

2SC373-TOS SI-N 35V 0.1A 0.2W B>200

2SC3746-SAN SI-N 80V 5A 20W 100MHz

2SC3748-SAN SI-N 80V 10A 30W 100/600n

2SC3752-SAN SI-N 1100/800V 3A 30W

2SC3781-SAN SI-N 120V 0.4A 15W 500MHz

2SC3782-SAN SI-N 200V 0.2A 15W 400MHz

2SC3783-TOS SI-N 800V 5A 100W

2SC3787-SAN SI-N 180V 0.14A 10W 150MH

2SC3788-SAN SI-N 200V 0.1A 5W 150MHz

2SC3789-SAN SI-N 300V 0.1A 7W 70MHz

2SC3790-SAN SI-N 300V 0.1A 7W 150MHZ

2SC3792-SAN SI-N 50V 0.5A 0.5W 250MHz

2SC3795A-MAT SI-N 900V 5A 40W

2SC3795B Echivalent: 2SC3795A-MAT

2SC380 Echivalent: 2SC380TM-TOS

2SC3807-SAN SI-N 30V 2A 15W 260MHz

2SC3808-SAN N-DARL 80V 2A 170MHz B>80

2SC380TM-TOS SI-N 30V 50mA 0.3W 100MHz

2SC3811-MAT SI-N 40V 0.1A 0.4W 450MHz

2SC382 Echivalent: BF198

2SC3831-SKN SI-N 500V 10A 100W

2SC3832 Echivalent: BUT56A/DIV

2SC3833-SKN SI-N 500/400V 12A 100W

2SC3842-FUJ SI-N 600V 10A 70W 32MHZ

2SC3844-FUJ SI-N 600V 15A 75W 30MHZ

2SC3844A Echivalent: 2SC3844-FUJ

2SC3851-SKN SI-N 80V 4A 25W 15MHz

2SC3852-SKN SI-N 80V 3A 25W 15MHz

2SC3853 Echivalent: 2SC2579-SKN

2SC3854 Echivalent: 2SC3855-SKN

2SC3855-SKN SI-N 200V 10A 100W 20MHz

2SC3856 Echivalent: 2SC3263-SKN

2SC3857-SKN SI-N 200V 15A 150W 20MHz

2SC3858-SKN SI-N 200V 17A 200W 20MHz

2SC3866-FEC SI-N 900V 3A 40W

2SC3868-MAT SI-N 500V 1.5A 25W 0.7us

2SC388 Echivalent: 2SC388A-MBR

2SC3883-HIT SI-N+D 1500V 6A 50W

2SC3884-TOS Echivalent: 2SC3884A-TOS

2SC3884A-TOS SI-N 1500V 6A 50W

2SC3885A Echivalent: 2SC3886A-TOS

2SC3886A-TOS SI-N 1500V 8A 50W 0.1us

2SC3886A/DIV SI-N 1500V 8A 50W 0.1US

2SC388A-MBR SI-N 25V 50mA 0.3W 300MHz

2SC3890-SKN SI-N 500V 7A 30W 500NS

2SC3892 Echivalent: 2SC3892A-TOS

2SC3892A-TOS SI-N+D 1500V 7A 50W 0.4us

2SC3893-TOS Echivalent: 2SC3893A-TOS

2SC3893A-TOS SI-N+D 1500V 8A 50W

2SC3895-SAN SI-N 1500/800V 8A 70W

2SC3896-SAN SI-N 1500V 8A 70W

2SC3897-SAN SI-N 1500V 10A 70W

2SC3902-SAN SI-N 180V 1.5A 10W 120MHZ

2SC3907-TOS SI-N 180V 12A 130W 30MHz

2SC3927-SKN SI-N 900V 10A 120W

2SC394-TOS SI-N 25V 0.1A 200MC RF

2SC3940-MAT SI-N 30V 1A 1W 200MHz

2SC3943-MAT SI-N 110V 0.15A 2W 300MHz

2SC3944-MAT SI-N 150V 1A 40W 300MHz

2SC3948-FUJ SI-N 850V 10A 75W 20MHZ

2SC3950-SAN SI-N 30V 0.5A 5W

2SC3952-SAN SI-N 80V 0.5A 10W 700MHz

2SC3953-SAN SI-N 120V 0.2A 8W 400MHz

2SC3954-SAN SI-N 120V 0.3A 8W 400MHz

2SC3955-SAN SI-N 200V 0.1A 7W 300MHz

2SC3956-SAN SI-N 200V 0.2A 7W 70MHz

2SC3964-TOS SI-N 40V 2A 1.5W 1us

2SC3972-MAT SI-N 800/500V 5A 40W

2SC3973A-MAT SI-N 900V 7A 45W

2SC3973B-MAT Echivalent: 130.60851

2SC3979 Echivalent: 2SC3979A-MAT

2SC3979A-MAT SI-N 800V 3A 2W 10MHz

2SC3987-SAN N-DARL+D 50V 3A 15W

2SC3995 Echivalent: BU2525AF-PHI

2SC3996-SAN SI-N 1500/800V 15A 180W

2SC3996/DIV SI-N 1500/800V 15A 180W

2SC3997-SAN Echivalent: 2SC3998-SAN

2SC3998-SAN SI-N 1500V 25A 250W POWER

2SC3999-NEC SI-N 300V 0.1A 0.75W 300M

2SC4004-MAT SI-N 900/800V 1A 30W <1/4

2SC4020-SKN SI-N 900V 3A 50W 1us

2SC4024-SKN SI-N 100V 10A 35W B>300

2SC4028 Echivalent: BUV70

2SC4029-TOS SI-N 230V 15A 150W 30MHz

2SC4043-RHM SI-N 20V 50mA 0.15W 3.2GH

2SC4046-HIT SI-N 120V 0.2A 8W 350MHz

2SC4052-SHI SI-N 600V 3A 40W 20MHZ

2SC4055 Echivalent: BUT56A/DIV

2SC4056-SHI SI-N 600V 8A 45W

2SC4058 Echivalent: 2SC4059-SHI

2SC4059-SHI SI-N 600/450V 15A 130W

2SC4064-SKN SI-N 50V 12A 35W 40MHz

2SC4064/DIV SI-N 50V 12A 35W 40MHZ

2SC4075 Echivalent: 2SC1505-NEC

2SC4106 Echivalent: BUT56A/DIV

2SC4107-SAN SI-N 500/400V 10A 60W

2SC4119-SAN N-DARL+D 1500V 15A 250W B

2SC4122-SAN Echivalent: 2SC4123-SAN

2SC4123-SAN SI-N+D 1500V 7A 60W

2SC4124 Echivalent: 2SC4125-SAN

2SC4125-SAN SI-N+D 1500/800V 10A 70W

2SC4131-SKN SI-N 100V 15A 60W 18MHz

2SC4135-SAN SI-N 120V 2A 15W 200MHz

2SC4137-RHM SI-N 25V 0.1A 300MHz

2SC4138-SKN SI-N 500V 10A 80W <1/3.5u

2SC4153-SKN SI-N 200V 7A 30W 0.5us

2SC4157-TOS SI-N 600V 10A 100W

2SC4159-SAN SI-N 180V 1.5A 15W 100MHz

2SC4161-SAN SI-N 500V 7A 30W

2SC4169-SAN N-DARL+D 50V 1.2A 1W B=4K

2SC4199-SKN SI-N 1400V 10A 100W

2SC4200 SI-N 20V 0.6A 5W 2.5GHz

2SC4204-SAN SI-N 30V 0.7A 0.6W

2SC4231-SHI SI-N 1200/800V 2A 30W

2SC4234 Echivalent: 2SC4235-SHI

2SC4235-SHI SI-N 1200/800V 3A 80W

2SC4236-SHI SI-N 1200/800V 6A 100W

2SC4237-SHI SI-N 1200/800V 10A 150W

2SC4242-FEC SI-N 450/400V 7A 40W

2SC4256-SAN SI-N 1500V 10A 175W 6MHz

2SC4278-RHM SI-N 150V 10A 100W 30MHz

2SC4288-TOS Echivalent: 2SC4288A-TOS

2SC4288A-TOS Echivalent: : N.L.A.

2SC4288A/DIV SI-N1600/600V 12A 200W

2SC4289 Echivalent: 2SC4298A-TOS

2SC4289A-TOS SI-N 1500V 16A 200W

2SC4290 Echivalent: 2SC4290A-TOS

2SC4290A-TOS SI-N 1500V 20A 200W

2SC4293-SAN Echivalent: 2SD1730-MAT

2SC4297-SKN SI-N 500V 12A 75W 10MHz

2SC4298-SKN SI-N 500V 15A 80W 10MHz

2SC4300-SKN SI-N 900V 5A 75W 1/6us

2SC4304-SKN SI-N 800V 3A 35W

2SC4308-HIT SI-N 30V 0.3A 0.6W 2.5GHz

2SC4313-SHI SI-N 900V 10A 100W 0.5us

2SC4370-KEC Echivalent: 2SC3298-TOS

2SC4381-SKN SI-N 150V 2A 25W 15MHz

2SC4382-SKN SI-N 200V 2A 25W 15MHz

2SC4385 Echivalent: 2SC4386-SKN

2SC4386-SKN SI-N 160/120V 8A 75W 20MH

2SC4387-SKN SI-N 200V 10A 80W 20MHz

2SC4388-SKN SI-N 200V 15A 85W 20MHz

2SC4408-TOS SI-N 80V 2A 0.9W 100/600n

2SC4418 Echivalent: 2SC3795A-MAT

2SC4429-SAN SI-N 1100/800V 8A 60W

2SC4430-SAN SI-N 1100V 12A 65W 15MHZ

2SC4431-NEC SI-N 120V 1.5A 20W 150MHz

2SC4436 Echivalent: 2SC3486-SAN

2SC4439 SI-N 180V 0.3A 8W 400MHZ

2SC4466 Echivalent: 2SC4467-SKN

2SC4467-SKN SI-N 160/120V 8A 80W 20MH

2SC4468-SKN SI-N 200V 10A 80W 20MHz

2SC4484-SAN SI-N 30V 2.5A 1W 250MHZ

2SC4488-SAN SI-N 120V 1A 1W 120MHZ

2SC4511-SKN SI-N 120V 6A 30W 20MHZ

2SC4512-SKN SI-N 120V 6A 50W 20MHZ

2SC4517-SKN SI-N 900V 3A 30W 6MHz

2SC4517A-SKN SI-N 1000V 3A 30W 0.5us

2SC4531-TOS SI-N+D 1500V 10A 50W

2SC4532-TOS SI-N 1700V 10A 200W 2uS

2SC4538-FEC SI-N 900V 5A 80W

2SC454-HIT SI-N 30V 0.1A 230MHz

2SC4542-TOS SI-N 1500V 10A 50W

2SC4547-SAN N-DARL+D 85V 3A 30W B>2K

2SC4557-SKN SI-N 900V 10A 80W <1/5.5u

2SC4560-TOS SI-N 1500V 10A 80W

2SC458-MBR SI-N 30V 0.1A 230MC UNI

2SC4582-SHI SI-N 600V 100A 65W 20MHz

2SC460-HIT SI-N 30V 0.1A 0.2W 230MHz

2SC461-HIT SI-N 30V 0.1A 0.2W 230MHz

2SC4688-TOS TRANSISTOR

2SC4692 Echivalent: 2SC4747-HIT

2SC4706 Echivalent: BUV48A-MBR

2SC4742-HIT Echivalent: : 2SC3883-HIT

2SC4744-HIT SI-N 1500V 6A POWER

2SC4745-HIT SI-N 1500V 6A

2SC4747-HIT SI-N 1500V 10A 50W 0.3us

2SC4747/DIV SI-N 1500V 10A 50W 0.3US

2SC4758 SI-N 1500V 8A 50W HI-RES

2SC4762-TOS TRANSISTOR

2SC4769-SAN SI-N+D 1500V 7A 60W

2SC4770-SAN SI-N 1500/800V 7A 60W

2SC4793-TOS SI-N 230V 1A 2W 100MHz

2SC4804-SHI SI-N 900V 3A 30W 0.3us

2SC4820-SKN SI-N 450V 6A 30W 12MHz

2SC4826-SAN SI-N 200V 3A 1.3W 300MHz

2SC4833 Echivalent: 2SC4834-SHI

2SC4834-SHI SI-N 500V 8A 45W <0.3/1.4

2SC485-TOS Echivalent: : N.L.A.

2SC4883 Echivalent: 2SC4883A-SKN

2SC4883A-SKN SI-N 180V 2A 20W 120MHZ

2SC4890 Echivalent: 2SC4891-SAN

2SC4891-SAN SI-N 1500V 15A 75W

2SC4908-SKN SI-N 900V 3A 35W 1us

2SC4916 Echivalent: 2SC3892A-TOS

2SC4924-SAN SI-N 800V 10A 70W

2SC4927-HIT TRANSISTOR

2SC495 Echivalent: 2SC3419-TOS

2SC496 Echivalent: 2SC3419-TOS

2SC4977-FEC SI-N 450V 7A 40W

2SC5002-SKN SI-N 1500V 7A 80W

2SC5003-RHM SI-N+D 1500V 7A 80W

2SC5027-TOS SI-N 1100V 3A 50W 0.3us

2SC5030 SI-N 50V 5A 1.3W 150MHZ

2SC5047-SAN SI-N 1600V 25A 250W

2SC5045-SAN SI-N 1600V 15A 75W

2SC5048-TOS SI-N 1500V 12A 50W 0.3us

2SC5070-SAN SI-N 30V 2A 1.5W B>800

2SC508 Echivalent: 2SC2233-MBR

2SC5086-TOS SI-N 20V 80MA 7GHZ

2SC5088 Echivalent: BU2508AF-PHI

2SC509-TOS SI-N 35V 0.5A 0.6W 60MHz

2SC5129-TOS TRANSISTOR

2SC5144 SI-N 1700V 20A 200W

2SC5148-TOS SI-N 1500V 8A 50W 0.2us

2SC5149-TOS SI-N+D 1500V 8A 50W 0.2us

2SC5150-TOS SI-N 1700V 10A 50W 03us

2SC5171 SI-N 180V 2A 20W 200MHZ

2SC5196 Echivalent: 2SC5198

2SC5198 SI-N 140V 10A 100W 30MHZ

2SC5207 SI-N 1500V 10A 50W 0.4us

2SC5242-TOS SI-N 230V 15A 130W 30MHZ

2SC5243-MAT TRANSISTOR

2SC5244 Echivalent: 2SC5244A

2SC5244A SI-N 1600V 30A 200W

2SC5250 Echivalent: 2SC5250-HIT

2SC5250-HIT TRANSISTOR

2SC5251-HIT TRANSISTOR

2SC5294-MAT SI-N TRANSISTOR

2SC5296-SAN SI-N+D 1500V 8A 60W

2SC5297-SAN SI-N 1500V 8A 60W

2SC5299-SAN SI-N 1500V 10A 70W 0.2US

2SC535-HIT SI-N 20V 20mA 0.1W 0.700M

2SC536-MBR SI-N 40V 0.1A 180MC UNI

2SC538 Echivalent: 2SC1685-MAT

2SC5386 TRANSISTOR

2SC549 Echivalent: 2N3553-PHI

2SC558 Echivalent: BUY72

2SC586 Echivalent: 2SD1274-MAT

2SC620-MIT SI-N 50V 0.2A 0.25W UNI

2SC633-SNY Echivalent: : N.L.A.

2SC634 Echivalent: BC546B-MBR

2SC636 Echivalent: 2N3632-PHI

2SC643-MBR SI-N 1100V 2.5A 50W

2SC644-MAT SI-N 30V 50mA 0.25W

2SC645-MAT SI-N 30V 30mA 0.14W 200MH

2SC6492 Echivalent: LMC6492BEN-NSC

2SC680A-HIT Echivalent: : N.L.A.

2SC681 Echivalent: BU110

2SC710-MIT SI-N 30V 0.03A 200MHz

2SC711-MIT SI-N 30V 0.05A 150MHz

2SC712-MIT SI-N 30V 0.5A 150MHz

2SC717-HIT SI-N 30V 50mA 0.2W 600MHz

2SC730-MIT SI-N 40V 0.4A PQ=1.5W

2SC732-TOS SI-N 50V 0.15A 0.4W 150MH

2SC733-TOS Echivalent: 2SC1815-MBR

2SC734-TOS Echivalent: : N.L.A.

2SC735-TOS SI-N 35V 0.4A 0.3W UNI

2SC738 Echivalent: BF198

2SC752-TOS SI-N 15V 100mA 0.1W

2SC756-SNY SI-N 40V 4A 10W 65MHz

2SC774 Echivalent: BC141-10-CDL

2SC782-TOS Echivalent: : N.L.A.

2SC783-TOS Echivalent: 2SC782-TOS

2SC784-TOS SI-N 40V 0.02A 500MC RF

2SC789 Echivalent: 2SD880-MBR

2SC790 Echivalent: 2SD880-MBR

2SC792 Echivalent: 2SC1576-TOS

2SC793 Echivalent: TIP33C-MBR

2SC8050 Echivalent: SS8050

2SC815 SI-N 60V 0.2A 0.25W 200MH

2SC828-MAT SI-N 30V 0.05A 0.25W UNI

2SC829-MAT SI-N 30V 30mA 0.4W 230MHz

2SC839-NEC SI-N 50V 0.03A 250MHz

2SC8550 Echivalent: SS8550

2SC867-SNY SI-N 400V 1A 23W 8MHz

2SC867A Echivalent: 2SC867-SNY

2SC869-MIT SI-N 160V 30mA 0.2W 150MH

2SC875 Echivalent: BC141-16-MBR

2SC897 Echivalent: : N.L.A.

2SC898A-HIT SI-N 150V 7A 80W 15MHz

2SC900-NEC SI-N 30V 0.03A 100MHz

2SC9012 Echivalent: KTC9012-MBR

2SC9013 Echivalent: KTC9013-KEC

2SC9014 Echivalent: KTC9014-KEC

2SC9015 Echivalent: ktc9015-KEC

2SC9018 Echivalent: KTC9018-KEC

2SC930-SAN SI-N 15V 0.03A 300MC RF

2SC936-HIT SI-N 1000V 1A 22W POWER

2SC940 Echivalent: BU608/DIV

2SC941-TOS SI-N 35V 20mA 0.2W 120MHz

2SC943-NEC SI-N 60V 0.2A 0.3W 220MHZ

2SC945-MBR SI-N 50V 0.1A 250MC UNI

2SC982-TOS N-DARL 40V 0.3A 0.4W

2SC983-TOS Echivalent: : N.L.A.

T-STV2160 Echivalent: STV2160-ST

T0505MH Echivalent: BT136/600-PHI

T0509MH-ST TRIAC 600V 5A 10mA=TAG232

T0605MH Echivalent: TIC216M-MBR

T0805DH Echivalent: TIC225D-MBR

T0808DJ Echivalent: BTA08/600SW-ST

T0808MJ Echivalent: BTA08/600SW-ST

T0810DJ Echivalent: BTA08/400C-ST

T0812DH Echivalent: BT137/600-PHI

T0812MH Echivalent: BT137/600-PHI

T1012NJ Echivalent: BT138F/800-PHI

T1210MH Echivalent: BT138/800-PHI

T1210NH Echivalent: BT138/800-PHI

T1213DH Echivalent: TIC236M-MBR

T1213DJ Echivalent: BTA12/600B-ST

T1213MJ Echivalent: BTA12/600B-ST

T1612MH Echivalent: BT139/600-PHI

T1612NH Echivalent: BT139/800-PHI

T1613DH Echivalent: TIC246D-MBR

T1613DJ Echivalent: BTA16/600B-ST

T1613MH Echivalent: TIC246M-MBR

T1613MJ Echivalent: BTA16/600B-ST

T2513MK Echivalent: BTA25/600B-ST

T2800D Echivalent: BT137/600-PHI

T4013BK Echivalent: BTA40/700B-ST

T40993 Echivalent: KS5805B

T4700D TRIAC 400V 15A 80/60mA

T9054V THYR. 600V 5A 0.2mA

TA07-11EWA SUPERROT DOTMATRIX 18mm

TA07-11HWA ROT DOTMATRIX 18mm C.A.

TA07-11YWA GELB DOTMATRIX 18mm C.A.2

TA12-11EWA SUPERROT DOTMATRIX 30mm

TA12-11GWA GRUEN DOTMATRIX 30mm

TA12-11HWA ROT DOTMATRIX 30mm C.A.

TA12-11YWA GELB DOTMATRIX 30mm CA 2

TA20-11EWA SUPERROT DOTMATRIX 50mm C

TA20-11GWA GRUEN DOTMATRIX 50mm C.A.

TA20-11HWA ROT DOTMATRIX 50mm C.A.

TA2003P-TOS FM DETECTOR

TA2031-TOS IC

TA2040 IC

TA24-11EWA SUPERROT DOTMATRIX 60mm C

TA24-11HWA ROT DOTMATRIX 60mm C.A.

TA6210 Echivalent: KIA6210AH-KEC

TA7046P-TOS AM/FM IF STAGES

TA7060AP-TOS FM IF AMPLIFIER 7p

TA7061AP-TOS FM IF AMPLIFIER

TA7063P-TOS FM MPX DEMOD. 7p

TA7066P-TOS PRE AMPLIFIER 7p

TA7069P-TOS I.C.

TA7070P-TOS Echivalent: LA1364

TA7075P-TOS TV+VC VIDEO-ZF+AGC 14p D

TA7089P-TOS AN V-REG 3.3-33V 0.2A

TA7092P-TOS PWR AMP 3.5W

TA7108

TA7117P-TOS 4-CHANNEL STEREO DECODER

TA7119P-TOS AM/FM IF STAGES

TA7120P DUAL AUDIO PRE AMPL.

TA7124 TV VIDEO AMPL.

TA7129P-TOS

TA7130P-TOS FM IF AMPLIFIER

TA7137 Echivalent: LA3210-SAN

TA7139P-TOS LOW NOISE PRE-AMPLIFIER

TA7140P-TOS SMALL OUTPUT POWER AMP.7p

TA7150P-TOS COLOUR TV CHROMA UNIT

TA7156 Echivalent: MC1310P-MOT

TA7157 Echivalent: MC1310P-MOT

TA7176AP Echivalent: CA3065E-MBR

TA7177

TA7178

TA7179-TOS TRACKING REG.8-20V. 14p

TA7192P-TOS CTV CHROMA CIRCUIT

TA7193P-TOS Echivalent KA2151

TA7200

TA7204P-TOS

TA7205AP-TOS

TA7208

TA7214P-TOS 2XPWR AMP 2X4.8W/4E 18V

TA7217AP-TOS Echivalent: KIA7217AP-KEC

TA7217P Echivalent: KIA7217AP-KEC

TA7222AP-TOS PWR AMP 1x5.8W 18V

TA7223P-TOS 1.0W POWER AMPL. 14p

TA7226P-TOS

TA7227P-TOS Echivalent 209.80684

TA7229P-TOS

TA7230P-TOS PWR AMP 2x4W/4E 20V

TA7232P-TOS PWR AMP. 2x2.2W 12V

TA7233P-TOS PWR AMP. 2x4.5W 12V

TA7236P-TOS DUAL PWR AMP 2x7W/4E 15V

TA7237AP-TOS

TA7240AP-TOS PWR AMP 2x5.8W/4E 18V

TA7241AP-TOS PWR AMP 2x5.8W/4E 18V REV

TA7242P-TOS VERTICAL OSCILLATOR 10p

TA7243P-TOS SIF+DET+VOL+3W AUDIO AMP.

TA7245BP-TOS 3-PH MOTOR DR. 26V 1.2A

TA7245CP-TOS MOTOR DRIVER 3-PH. BI-DIR

TA7245F-TOS MOTOR CONTROLER

TA7248P-TOS 3-PH MOTOR DR. 35V 0.9A

TA7250BP-TOS PWR AMP 1*30W/2E 18V 10%

TA7251BP-TOS PWR AMP 1*30W/2E 18V 10%

TA7252AP-TOS PWR AMP 1*9.6W/2E 18V 0.1

TA7256P-TOS DUAL PWR OP-AMP 18V 0.5A

TA7257P-TOS DC MOTOR BRIDGE DRIV. 1.5

TA7259P-TOS 3-PHASE MOTOR DRIVER

TA7262P-TOS BI-DIREC. DC MOTOR DRIVER

TA7263P-TOS DUAL POWER AMPL. 12p

TA7264P-TOS TA7263 WITH REV PINFIG

TA7265AP-TOS I.C.

TA7267BP-TOS CV CCV STOP SHORT BRAKE

TA7268P-TOS PWR AMP 11W/8E 45V 3.2A

TA7269P-TOS PWR AMP. 2x7.8W 12V

TA7270P-TOS PWR AMP 2x5.8W/4E 18V

TA7271P-TOS PWR AMP 2x5.8W/4E 18V REV

TA7272P-TOS DUAL MOTOR DRIVER AMPL.

TA7273P-TOS

TA7274P-TOS 12.0W POWER AMPL. 7p

TA7279P-TOS DC MOTOR/BRIDGE DRIVER

TA7280P-TOS PWR AMP 2*5.8W/4E 18V 10%

TA7281P-TOS PWR AMP 2*5.8W/4E 18V REV

TA7282AP-TOS PWR AMP. 2x4.6W 15V

TA7283AP-TOS PWR AMP. 2x4.5W 15V

TA7284P-TOS MOTOR CONTR. 9 - 24V

TA7288P-TOS MOTOR CONTR. 9 - 25V

TA7291P-TOS MOTOR CONTR. 4.5-18V 10p

TA7291S-TOS MOTOR CONTROLER

TA7292P-TOS PWR AMP 2x5.8W/4E 18V

TA7299P-TOS PWR AMP 2x5.8W/4E 18V

TA7302P-TOS FM IF AMPLIFIER 7p

TA7303P-TOS FM IF AMPL. 9p

TA7307 RECORDER NF-V

TA7310P-TOS

TA7312

TA7313 Echivalent: AN7112E-MAT

TA7314P-TOS NO LONGER AVAILABLE

TA7315P-TOS TV TUNER BANDSELECTOR 9p

TA7317P-TOS LOUDSPEAKER PROTECTOR 9p

TA7318P-TOS

TA7320P-TOS BALANCED MOD./DEMOD. 9p

TA7321

TA7322P-TOS PRE AMPLIFIER 9p

TA7323 STEREO DECODER 9p

TA7324P-TOS MUTING FOR POP NOISE 9p

TA7325P-TOS PRE AMPL. + MUTING 9p

TA7326 LONG-HOUR TIMER IC

TA7328AP-TOS PRE AMPL. WITH ALC 12p

TA7330P-TOS PRE AMPL. WITH ALC

TA7331P-TOS PWR AMP 1x0.2W/4E 5V

TA7333P-TOS RC TRANSMITTER 3-MODE

TA7335P-TOS FM FRONT END

TA7336P-TOS PWR AMP 1x0.34W/8E 9V

TA7337P-TOS TV SOUND LIMITER/DEMOD

TA7339P-TOS I.C.

TA7340P-TOS MOTOR CTRL, HYSTERISIS AM

TA7341P-TOS PROGRAMM. SKIP IC

TA7342P-TOS FM MPX DEMODULATOR

TA7343AP-TOS FM MPX DECODER

TA7347P-TOS AUDIO/VIDEO SWITCH 7p

TA7348P-TOS AUDIO/VIDEO SWITCH 9p

TA7349P-TOS MUTING POP NOISE

TA7350P-TOS VERT. DEFL. SYNC. INSERTI

TA7353-TOS I.C.

TA7354P-TOS BRIDGE DRIVER

TA7355P-TOS

TA7358AP-TOS Echivalent LA1185-SAN

TA7358P-TOS FM-TUNER

TA7359P-TOS DUAL PRE-AMP 9p

TA7361AP-TOS MUTING I.C.

TA7362P-TOS MUTING I.C.

TA7364P-TOS ANALOG SIGNAL CONDITIONER

TA7366P-TOS LED METER DRIVER (LOG)

TA7368P-TOS PWR AMP 1x0.72W/4E 10V

TA7370P-TOS FM/PLL/MPX ST DEC Ucc=3V

TA7371F-TOS

TA7373F-TOS SMD STEREO MULTIPLEX

TA7374P-TOS STEREO MULTIPLEXER

TA7375P-TOS Echivalent: LA3161-SAN

TA7376P-TOS PWR AMP 2x0.4W/8E 6V

TA7378P-TOS FM/TV RF-AMPL. MIX

TA7401AP-TOS FM STEREO DEMODULATOR

TA7402P-TOS AM STEREO TUNER DECODER

TA7403 3 STAGE AMPL/MUTING

TA7404 QUADRATOR DEMODULATOR

TA7405P-TOS 2 CH REV HEAD SEL-SWITCH

TA7411AP-TOS FM-ZF MUTING

TA7415P-TOS CD OUTPUT AMPLIFIER

TA7417AP-TOS DUAL AMP. ALC MICROPHONE

TA7424AP-TOS IC

TA7502P Echivalent: UA741CN8-MBR

TA7504S-TOS OPAMP INTERNAL COMPENSATE

TA75061 Echivalent: TL061CN

TA75071 Echivalent: TL071CN-MBR

TA7521P DUAL OPAMP

TA75339P Echivalent: LM339N-MBR

TA75358P-TOS OPAMP

TA75393 Echivalent: LM393N-MBR

TA75393S-TOS V-COMP 2X36V 1.3US EXT.TE

TA75458P Echivalent: MC1458N-MBR

TA75458S Echivalent: TA75558S-TOS

TA75558P Echivalent: MC4558CN-MBR

TA75558S-TOS DUAL AMPLIFIER

TA75559S-TOS OPAMP 2X32V EXT. TEMP.

TA7555P-TOS Echivalent: NE555N-MBR

TA75902 Echivalent: LM2902N-MBR

TA7604 Echivalent: AN7410N-MAT

TA7606P-TOS ZERO-CROSS SWITCH/TEMP. C

TA7607AP-TOS PIF + AFT REVERSE AGC 16p

TA7609P-TOS SYNC.SEP AFC+OSC DRIV.

TA7611AP-TOS PIF + AFT FORWARD AGC

TA7612AP-TOS LED DISPLAY ENCODER

TA7613AP-TOS FM IF AMPLIFIER 16p

TA7614AP-TOS

TA7616P-TOS AM TUNER

TA7617AP-TOS RECORDER DRIVER CONTROL

TA7621P-TOS SECAM CHROMA PROCESSOR 28

TA7622AP-TOS

TA7628P-TOS PRE AMPL. WITCH ALC 16p

TA7629P-TOS VTR AMPLIFIER 16p

TA7630P-TOS TV TONE CONTROL

TA7634AP-TOS I.C.

TA7639P-TOS 2XREC./PLAYB. AMPLIFIER

TA7640AP-TOS AM IF TUNER/IF AMPL.

TA7641BP-TOS AM RADIO SINGLE CHIP

TA76431S Echivalent: TL431CLP-MBR

TA7644BP-TOS NTSC VIDEO/CHROMA DEFL.

TA76494P Echivalent: TL494CN-MBR

TA76524P Echivalent: SG3524N-MBR

TA7654P-TOS LED DISPLAY ENCODER 14p

TA7657P-TOS RADIO CONTROL RECEIV. 14p

TA7658P Echivalent: LA3220-SAN

TA7659P-TOS PIF + AFT FORWARD AGC

TA7660P-TOS PIF + AFT FORWARD AGC

TA7666P-TOS LED DRIVER (2x5 LEDS)

TA7668BP-TOS DUAL PRE AMP. + ALC 16p

TA7670P-TOS NTSC TV CONTROLER

TA7673P-TOS VHF RF MODULATOR FOR VTR

TA7676AP-TOS CTV-NTSC-RGB SYSTEMSWITCH

TA7676P-TOS RGB/PAL/SECAM SYSTEM SWIT

TA7678 Echivalent: KA2913A-SSU

TA7679 I.C.

TA7680AP-TOS TV/VIDEO TONE AMP+AGC 24p

TA7681AP-TOS VOL. + AF AMPLIFIER

TA7687AP-TOS AM/FM RADIO TUNER 16p

TA7688P-TOS PWR AMP 2x0.038W/4E 5V

TA7698AP-TOS CHROMA PAL/NTSC CONTROLLE

TA7699AP-TOS

TA7704AP-TOS FM IF SYST. QUAD. DET.

TA7705P-TOS DUAL AUDIO PREAMP=DBL1045

TA7708 PRE/LINE AMPL

TA7709P-TOS DUAL PRE-AMP 16p

TA7710P-TOS AUDIO/VIDEO I.C.

TA7719P-TOS PREAMP/LINE AMP

TA7727P-TOS SINGLE AUDIP PREAMPL.

TA7736P-TOS DC-MOTOR-Tr 4..15V 1A

TA7739P-TOS AUDIO I.C.

TA7741P-TOS ALSO: 209.11411

TA7743P-TOS BRUSHL. MOTOR CONTR.4-15V

TA7745P-TOS 3-PHASE DRIVER Iout=1A ma

TA7750P-TOS 2-INPUT AUDIO/VIDEO SWITC

TA7757P-TOS AM/FM IF SYST. 3V

TA7758P-TOS AM/FM MUTING I.C.

TA7761P-TOS TELECOM TUNER SYSTEM

TA7764P-TOS I.C.

TA7765AF-TOS AM/FM IF SYST. 1.5V

TA7769P-TOS PWR AMP 2x11W/4E 9V

TA7772P-TOS 2-CHAN. VIDEO PREAMPLIFIE

TA7778P-TOS RGB INTERFACE

TA7780BN-TOS

TA7784P-TOS 2-CHANNEL AMP.

TA7792P-TOS AM/FM TUNING SINGLE CHIP

TA7796P-TOS GRPH EQUALIZER 5-BAND

TA7796Z-TOS 5 BAND GRAPHIC EQUILIZER

TA78005AP Echivalent: UA7805-MBR

TA78012AP Echivalent: UA7812-MBR

TA78xxS Echivalent: UA78xxF

TA790xxS Echivalent: UA79xxF

TA8000-TOS WATCH DOG TIMER 5V 0.01A

TA8101N-TOS TRACKING SERVO CD-PLAYER

TA8102P-TOS CD-MOTOR CNTRL 4.5-12V

TA8105N-TOS 1-CHIP PRE+POWER AMP. IC

TA8108AP-TOS AM/FM IF STAGES

TA8110AP-TOS AM-FM AMPLIER Ucc=1.8..8V

TA8119P-TOS AMPLIFIER

TA8122AN-TOS AM/FM STEREO TUNER SYSTEM

TA8125 I.C.

TA8127N-TOS AM/FM STEREO TUNER SYSTEM

TA8129Z-TOS DIFFERENTIAL 6-STAGE AMP.

TA8132AN I.C.

TA8150N-TOS 1-CHIP FM PROCESSOR

TA8158F-TOS RF AMPLIFIER MIXER

TA8162SN-TOS 2-CHANNEL TAPE PREAMP

TA8164P-TOS AM/FM AMPLIFIER (MONO)

TA8167N-TOS ONE CHIP TUNER SYSTEM

TA8173AP-TOS

TA8176SN-TOS FM-TUNER

TA8184P-TOS VOLUME + TONE CONTROL

TA8189P-TOS DOUBLE RECORD. AMP. =N

TA8192F-TOS PWR AMPLIFIER 2 CHN

TA8194Z-TOS MUTING I.C.

TA8200AH-TOS PWR AMP 2x13W/8E 37V

TA8201AK-TOS PWR AMP 1x14W/4E 18V

TA8202K-TOS LIN-I.C. 18V 1.5A 2x2.9W

TA8205AH-TOS PWR AMP 2x15W/4E 18V

TA8207K-TOS AMPLIFIER

TA8208H-TOS PWR AMP 2x5.8W/4E 13.2V

TA8210AH-TOS PWR AMP 2x19W/4E 18V

TA8210H Echivalent: TA8210AH-TOS

TA8211AH-TOS PWR AMP 2x6W/8E 30V

TA8212AH Echivalent: TA8212F-TOS

TA8212F-TOS 4-CHAN. CD MOTOR DRVR 14.

TA8214K-TOS 4xPOS V-REG 8.3V .1/.1/.3

TA8215AH-TOS Echivalent: TA8215H-TOS

TA8215H-TOS PWR AMP 2*15W/4E 18V

TA8215L-TOS PWR AMP 2x15W/4E 18V

TA8216H-TOS PWR AMP 2x13W/4E 24V

TA8217P-TOS PWR AMP 2x2.5W/4E 12V

TA8218AH-TOS PWR AMP 3x6W/8E 30V

TA8218H Echivalent: TA8218AH-TOS

TA8220H-TOS PWR AMP 2x19W/4E 18V

TA8221AH-TOS PWR AMP 2x19W/4E 18V

TA8221AL-TOS PWR AMP 2x19W/4E 18V

TA8221H Echivalent: TA8221AH-TOS

TA8221L Echivalent: TA8221AL-TOS

TA8223 Echivalent: TA8223K-TOS

TA8223K-TOS PWR AMP 2x6.5W/4E 15V

TA8225H-TOS PWR AMP 1x23W/4E 18V

TA8225L-TOS PWR AMP 1x24W/4E 18V

TA8227P-TOS PWR AMP 2x2.5W/4E 12V

TA8229K-TOS PWR AMP 2x2.5W/4E 15V

TA8231L-TOS PWR AMPLIFIER

TA8232H-TOS I.C.

TA8238K-TOS I.C.

TA8248K-TOS 2XPWR AMP 2X4.6W/4E 20V

TA8251AH-TOS IC

TA8255AH-TOS I.C.

TA8316AS-TOS IGBT GATE DRIVER

TA8400P-TOS 2-CHANNEL 4 MODE DRIVER

TA8403K-TOS 1 CHIP CTV I.C.

TA8405S-TOS BRIDGE DRIVER

TA8406P-TOS DUAL AMPLIFIER 18V 0.5A

TA8409 Echivalent: TA8409S-TOS

TA8409S-TOS MOTOR DRIVER

TA8410K-TOS DUAL PWR AMP 9V 0.6A

TA8410P-TOS DUAL PWR OPAMP 9V 0.6A

TA8424F-TOS DC MOTOR DRIVER

TA8427K-TOS CTV IC

TA8428K-TOS CTV VERT. DEFL. OUTPUT ST

TA8432K-TOS CTV VERT. OUT. DEFL. SYST

TA8445K-TOS CTV VERT. DEFLECT. SYSTEM

TA8449P-TOS QUAD POWER OPAMP

TA8604P-TOS PAL/NTSC/SECAM CHROMA SIG

TA8605N-TOS VHS VIDEO SIGNAL PROCES.

TA8606N-TOS VTR LUMINANCE

TA8607P-TOS VHS FM SIGNAL PROCESSOR

TA8611AN-TOS PIF+SIF REVERSE RF AGC

TA8615N-TOS MULTI COLOR SYSTEM SWITCH

TA8617S-TOS VC SERVO CONTROLER

TA8618S-TOS FG CONTROL AMPLIFIER

TA8628N-TOS 2-INPUT A/V SWITCH 2CH VO

TA8631N-TOS MONITOR RGB-PROCESSOR

TA8632N-TOS

TA8637BP-TOS VHF RF MODULATOR + REG.

TA8644N-TOS VHS CHROMA PAL/NTSC/SECAM

TA8646AN-TOS COLOR TV VIDEO & SOUND IC

TA8653N-TOS IC.

TA8659AN-TOS PAL/NTSC/SECAM DECODER

TA8659CN-TOS PAL/NTSC/SECAM DECODER

TA8659N Echivalent: TA8659AN-TOS

TA8680N-TOS CTV VIDEO+TONE IC (NTSC)

TA8690AN-TOS PAL/NTSC INTERFACE CIRCUI

TA8690N Echivalent: TA8690AN-TOS

TA8691N-TOS 1-CHIP CTV PAL SYSTEM

TA8701AN-TOS REV. RF AUDIO GAIN CONTRO

TA8718N-TOS CHROMA PAL CONTROLER

TA8720AN-TOS IC.

TA8721SN-TOS DUAL TONE AMPLIFIER

TA8725AN-TOS CTV NTSC PLL CIRCUIT

TA8739P-TOS BUS CONTR. DEFL. PROCESSO

TA8750AN-TOS SECAM CHROMA PROCESSOR

TA8751AN-TOS AUDIO/VIDEO INTERFACE SWI

TA8751N Echivalent: TA8751AN-TOS

TA8757AN-TOS VHS CHROMA SIGNAL PROCES.

TA8759AN-TOS Echivalent: TA8759BN-TOS

TA8759BN-TOS CHROMA PAL/SECAM/NTSC CNT

TA8783N-TOS I.C.

TA8807N-TOS

TA8808BN-TOS CHROMA PAL/NTSC CNTRL

TA8859AP-TOS WIDE TV I2C-BUS

TA8867AN-TOS CHROMA PAL/NTSC CONTROLLE

TA8872N-TOS

TAA2761 Echivalent: TAA2761A

TAA2761A Echivalent: TAA2765A/DIV

TAA2765A/DIV 2xOPAMP 15V 70mA -25 +85'

TAA350A-PHI

TAA450 Echivalent 209.80253

TAA4765A-SIE

TAA4765A/DIV Echivalent TAA4765A-SIE

TAA521A OPAMP

TAA550B REF-DIODE 33V FOR TUNER

TAA611 Echivalent: TAA611A,B,C

TAA611B12 NF AMPLIFIER 1.0W 14p

TAA611C11 NF AMPLIFIER 3.3W 14p

TAA611C12 NF-E 15V 1A 2,1W (12V/8E)

TAA611T12 Echivalent: TAA611B12

TAA630S

TAA661A-ST

TAA661B AMPLIFIER DEMODULATOR 14p

TAA691-ST TV SOUND DEMODULATOR+AMP.

TAA761A/DIV AMPLIFIER 18V 70 mA

TAA761DIP-MBR AMPLIFIER 18V 70 mA

TAA765A-SIE

TAA780-ITT VOLTREG FIXED POSITIVE

TAA861A AMPLIFIER 20V 70mA

TAA865 Echivalent TAA865A

TAA865A Echivalent TAA765A/DIV

TAA930-PHI TV TONE FREQ. TUSSENTRAP

TAA970 SINGLE AUDIO PREAMP.

TAA981/DIV RF AMPLIFIER 14p

TAE1453A-SIE

TAG0102AA Echivalent: P0102DA-ST

TAG103X Echivalent: BR203

TAG203/350 Echivalent: TIC206D-MBR

TAG225/400 Echivalent: TIC116D-MBR

TAG232/600 Echivalent: T0509MH-ST

TAG31 Echivalent: TIC246D-MBR

TAG425/400 Echivalent: TIC225D-MBR

TAG621/600 Echivalent: TIC116M-MBR

TAG621/800 Echivalent: TIC116N-MBR

TAG626/600 Echivalent: TIC116M-MBR

TAG626/700 Echivalent: TIC116N-MBR

TAG633/400 Echivalent: S0402DH-MBR

TAG633/600 Echivalent: S0402MH-ST

TAG665/800 Echivalent: TIC126N-MBR

TAG92B Echivalent: Z0105MA-MBR

TAG93D Echivalent: Z0105MA-MBR

TAGE0102AA Echivalent: P0102DA-ST

TBA120 FM/IF AMPL+DEMOD. LC-FILT

TBA120A-MBR FM/IF AMPL+DEMOD. LC-FILT

TBA120AS/DIV FM/IF AMPL+DEMOD. NF-V

TBA120S-MBR FM/IF AMPL+DEMOD NF-V

TBA120S/DIV FM/IF AMPL+DEMOD. NF-V

TBA120T-MBR FM/IF AMPL+DEMOD. CER.FIL

TBA120T/DIV FM/IF AMPL+DEMOD. CER.FIL

TBA120U-PHI FM/IF AMPL+DEMOD VC-SIGN.

TBA120U-TFK FM/IF AMPL+DEMOD VC-SIGNA

TBA120U/DIV FM/IF AMPL+DEMOD. VC-SIGN

TBA120UB-SIE Echivalent: TBA120U-PHI

TBA121-2-SIE Echivalent U2840B-TFK

TBA130-2-SIE

TBA1441 Echivalent TBA440

TBA2110-MBR VCO/FSK DEMOD.+ IR AMPL.

TBA221 Echivalent: UA741H-MBR

TBA221B Echivalent: UA741CN8-MBR

TBA222B-SIE

TBA229-2-SIE Echivalent: U2829B-TFK

TBA231A Echivalent: UA739PC

TBA271B Echivalent: TAA550B

TBA2800-ITT IR PRE-AMPLIFIER 14p

TBA281G Echivalent: UA723TO

TBA311A17

TBA331 Echivalent: CA3046-MBR

TBA341 Echivalent: CA3054-HAR

TBA395

TBA396 Echivalent: SN76396

TBA400D-ST

TBA440C Echivalent: TBA440P-MBR

TBA440N Echivalent: TBA1441

TBA440P-MBR

TBA460Q AF/FM (PRE) AMPLIFIER

TBA480 IF AMPL.+ FM DEMOD. 16p

TBA480Q IF AMPL.+ FM DEMOD. 16pQ

TBA490

TBA500-PHI

TBA510

TBA520 TV PAL SYNCH.DEMOD.

TBA530/DIV RGB MATRIX PREAMPL.

TBA540-MBR REFERENCE OSCILLATOR

TBA540/DIV REFERENCE OSCILL. 16p

TBA540Q REFERENCE OSCILL. 16pQ

TBA560B

TBA560C

TBA570A/DIV AM/FM RECEIVER

TBA570AQ AM/FM RECEIVER

TBA625BX5

TBA651 AM TUNER + IF STAGE

TBA673 TELEPH.RING (DE)MODULATOR

TBA730 TV-VERT. OSCILLATOR

TBA750A Echivalent: TBA750C-MBR

TBA750B Echivalent: TBA750C-MBR

TBA750C-MBR

TBA750Q Echivalent: TBA750C-MBR

TBA760 TV VERT.SYNCHR.

TBA780 Echivalent: CA3065E-MBR

TBA790T/DIV PWR AMP 1*2.5W 14V

TBA800/DIV PWR AMP 1x5W/16E 30V 20kH

TBA810AP Echivalent: TBA810P-MBR

TBA810AS/DIV PWR AMP 1x7W/16E 20V 20kH

TBA810AT-TFK PWR AMP 1x7W/16E 20V 20kH

TBA810P-MBR PWR AMP 1x7W/16E 20V 20kH

TBA810S/DIV PWR AMP 1x7W/16E 20V 20kH

TBA810SH/DIV Echivalent: TBA810S/DIV

TBA820L Echivalent: TBA820M-ST

TBA820M-ST PWR AMP 1x2W/8E 16V 20kHz

TBA820M/DIV PWR AMP 1x2W/8E 16V 20KHz

TBA820P PWR AMP 1x2W/8E 16V 20kHz

TBA820T Echivalent: TBA820M-ST

TBA860Q I.C.

TBA915 PWR AMP 1*0.6W 17V 6KHZ

TBA920/DIV SYNC.SEPARATOR+OSC 5% 12V

TBA920S-MBR SYNC.SEPARATOR+OSC 1.5% 1

TBA940

TBA950/2 Echivalent: TBA950:2X/DIV

TBA950:2X/DIV TV SYNC. SEPARATOR +OSC

TBA970/DIV TV VIDEO

TBA990-MBR TV PAL SYNCHR.DEMOD.

TBA990Q TV PAL SYNCHR.DEMOD. 16pQ

TBB1458B Echivalent: MC1458N-MBR

TBB1469-SIE FM RADIO RECEIVER CIRCUIT

TBB741G Echivalent: UA741CN8-MBR

TC07-11EWA SUPERROT DOTMATRIX 18mm

TC07-11GWA GRUEN DOTMATRIX 18mm C.C.

TC07-11HWA ROT DOTMATRIX 18mm C.C.

TC07-11YWA GELB DOTMATRIX 18mm CC 2

TC12-11HWA ROT DOTMATRIX 30mm C.C.

TC12-11YWA GELB DOTMATRIX 30mm CC 2

TC14433EPG ADC 3.5-DIG 40US DUAL-RAM

TC20-11EWA SUPERROT DOTMATRIX 50mm C

TC20-11GWA GRUEN DOTMATRIX 50mm C.C.

TC20-11HWA ROT DOTMATRIX 50mm C.C.

TC24-11EWA SUPERROT DOTMATRIX 60mm C

TC24-11HWA ROT DOTMATRIX 60mm C.C.

TC24-11YWA GELB DOTMATRIX 60mm C.C.

TC4001 Echivalent: CD4001-MBR

TC4011 Echivalent: CD4011-MBR

TC4013 Echivalent: CD4013-MBR

TC4021BP Echivalent: CD4021-MBR

TC4039 Echivalent: CD4039-MBR

TC4052 Echivalent: CD4052-MBR

TC4053 Echivalent: CD4053-MBR

TC4053BP Echivalent: CD4053-MBR

TC4066 Echivalent: CD4066-MBR

TC4066BP Echivalent: CD4066-MBR

TC4081BP Echivalent: CD4081-MBR

TC4093 Echivalent: CD4093-MBR

TC4093BP Echivalent: CD4093-MBR

TC4094BP Echivalent: CD4094-MBR

TC5082P-TOS DIVIDER 10.24MHz

TC5090AP-TOS ADC BINARY OUTP.

TC511000AP-80 Echivalent: 1MB-80-MBR

TC51256 Echivalent: 41256-80-MBR

TC5501P-TOS SRAM 256x4 5V

TC5514 Echivalent: 2114-200NS

TC5516 Echivalent: 6116-100-MBR

TC5517 Echivalent: 6116-100-MBR

TC55257 Echivalent: 62256-70-MBR

TC5565 Echivalent: 6264-70-MBR

TC89101P-TOS EEPROM 128x8 OR 64x16 250

TC9020P-TOS ON SCREEN DISPLAY CIRCUIT

TC9100P-TOS PLL FREQ.-SYNTHESIZER

TC9106 Echivalent: TC9106BP-TOS

TC9106BP-TOS PLL FREQUENCY SYNTHESIZER

TC9121P-TOS RECORDER CONTROLER

TC9122P-TOS BINARY COUNTER

TC9123P-TOS I.C.

TC9124AP-TOS

TC9125BP-TOS AM/FM/TV-TUN. PLL-SYNTH.

TC9130P-TOS SWITCH FF OUT 4-CHANN.

TC9134P-TOS RC RECEIVER 32 CODES

TC9135P-TOS 6-CIRCUITS RESET TOUCH SW

TC9137P-TOS AM/FM DIG.TUNING PLL SYNT

TC9138AP-TOS PROGR. SEARCH FOR CASS.

TC9142P-TOS QUARZ PLL MOTOR CONTROL

TC9143P-TOS RECORDER CONTROL

TC9145P-TOS 6 CIRC. ANALOG SWITCH

TC9147BP-TOS FM/MW/LM TUNING SYSTEM

TC9148P-TOS IR RC-TRANSM. 18 FUNCTION

TC9149P-TOS IR RC-RECEIV. 10 FUNCTION

TC9150P-TOS IR RC-RECEIV. 18 FUNCTION

TC9151P-TOS RC DECODER

TC9152P-TOS 5x FEATHER TOUCH SWITCH

TC9153AP-TOS ELECTONIC VOLUME CNTRL

TC9154AP-TOS VOLUME AUD.SIG.GAIN CONTR

TC9155P-TOS FREQ.BANDPASS/TONE CONTR.

TC9158P-TOS FREQ.DISPLAY DRIVER

TC9162N-TOS ANALOG SW-ARRAY 6*2+2*1

TC9163N-TOS ANALOG SW-ARRAY 4*2+2*2

TC9164N-TOS ANALOG SW-ARRAY 2*4+4*2

TC9167P-TOS RECORDER MUSIC SELECTION

TC9169AP-TOS UP/DOWN GRAPHIC EQUALIZER

TC9171P-TOS PHASE LOCKED LOOP

TC9172P-TOS PLL AM/FM DIGITAL TUNING

TC9174P-TOS I/O INTERFACE DIG. TUNING

TC9176P ELECT. VOLUME REGELING

TC9177P-TOS ELECTR.VOLUME SWITCH 20p

TC9217P-TOS PLL FOR 140MHz AUDIO

TC9227P-TOS PLL DIGITAL TUNING SYSTEM

TC9305P-TOS LOGIC CONTROLER FOR AUDI

TC9310

TCA0372-SMD Echivalent: TCA0372DW-MOT

TCA0372DP1-MOT DUAL OPAMP 1.1MHz

TCA0372DP2-MOT DUAL OPAMP +/-15-40V

TCA0372DW-MOT DUAL OPAMP

TCA0372N16-MOT Echivalent: TCA0372DP2-MOT

TCA0372N8-MOT Echivalent: TCA0372DP1-MOT

TCA1004 AUDIO POWER AMPL. OUTP.ST

TCA105N/DIV THRESHOLD COMP. 5-30V 50m

TCA150T-BAN PWR AMP 1*5W 18V

TCA205A/DIV INDUCTIVE PROXIMITY SWITC

TCA220-MBR TRIPLE AMPL. HIGH SPEED

TCA240

TCA250-MBR

TCA270S-MBR TV VIDEO AMPLIFIER =CA270

TCA290A-MBR STEREO DECODER

TCA3089 Echivalent: CA3089E-MBR

TCA315A OPAMP 15V 70mA EXT.TEMP =

TCA3189 Echivalent: CA3189E-HAR

TCA325 OPAMP 15V 70mA EXT.TEMP

TCA350

TCA420A-PHI FM AMPL. + AGC

TCA440-MBR Echivalent TCA440/DIV

TCA440/DIV AM RECEIVER (=A244D)

TCA4500A-MBR STEREO DECODER

TCA4510 STEREO DECODER

TCA4511-SIE PLL CAR STEREO DECODER

TCA520B-PHI SINGLE OPAMP

TCA540

TCA540Q

TCA5500-MOT STEREO DC AF CONTROL 18p

TCA580 GYRATOR SWITCH REEL REPL.

TCA640/DIV TV CHROMA PAL/SECAM =MCA6

TCA650-MBR TV PAL/SECAM SYNC.DEM.

TCA650/DIV TV PAL/SECAM SYNC. DEMOD

TCA660B-MBR TV ADJUST CONTROL

TCA671-SIE

TCA700Y-ITT POS V-REG 10V 0.22A (CAR)

TCA730A AF DC CONTROL

TCA740A/DIV AF DC TONE CONTROL=D81401

TCA750

TCA780 Echivalent: TCA785-SIE

TCA785-SIE PHASE CNTRL IC FOR TRIAC

TCA800DP-PLS TV SYNCHR.DEMODULATOR 16p

TCA800Q

TCA810A US RECEIVER DECODER

TCA830S AUDIO POWER AMPL.2W 12p

TCA871-SIE Echivalent: TCA671-SIE

TCA900 MOTOR SPEED REG.

TCA910 MOTOR SPEED REG.

TCA940E/DIV AUDIO POWER AMPL.10W 12pQ

TCA955-SIE

TCA965B-SIE WINDOW DISCRIMINATOR

TCA971-SIE

TCA991 Echivalent: TCA971-SIE

TCDT1101G OPTOISO 6KV 32V 40%

TCEP100Q I.C.

TCM3105N-TI

TCM5089N-TI Echivalent: UM95089

TCM80A IC

TCST1000 OPTICAL SWITCH

TCST2000 OPTICAL SWITCH

TD129 Echivalent: 1N4148-MBR

TD190 Echivalent: BA159-MBR

TD3F700R THYR.+D 700V 8A 50us

TD3F800H-AEG THYR.+D 800V 8A 50mA 3uS

TD3F800R-AEG

TD6104P-TOS PRESCALER TUNING SYSTEM

TD62001 Echivalent: ULN2001AN-MBR

TD62003AP Echivalent: ULN2003AN-MBR

TD62004P Echivalent: ULN2004AN-MBR

TD62064AP Echivalent: ULN2064B-MBR

TD62083AP Echivalent: ULN2803A-MBR

TD62084AP Echivalent: ULN2804A-MBR

TD62104 Echivalent: ULN2004AN-MBR

TD62105P Echivalent: M54517P-MIT

TD62308 Echivalent: M54567P-MIT

TD62504P-TOS 7xNPN TRANS. ARRAY COM.EM

TD62506 Echivalent: LB1216-SAN

TD62783AP-TOS PWR DR. 8*50V 0.35A=UDN29

TD6301AN-TOS

TD6301AP-TOS LED/LCD FREQ.DISPL.DRIVER

TD6304AP-TOS RECORDER PLUNGER DRIVER

TD6308AP-TOS RECORDER, PLUNGER DRIVER

TD6350P-TOS TV FREQ.SYNTH.+PRESCALER

TD6358N-TOS FREQ.-SYNTHESIZER+PRESCAL

TD6358P-TOS FREQ. SYNTHESIZER+PRESCAL

TD6359N-TOS FREQ. SYNTHESIZER+PRESCAL

TD6359P-TOS PRESC.+ PLL + BAND SWITCH

TD6360N-TOS MULTI SYSTEM SERVO.

TDA0200SP Echivalent: L200CV-ST

TDA1001A NOISE SUPPRESSOR 16p

TDA1001B-PHI NOISE SUPPRESSOR 16p

TDA1002/DIV PLAY/REC.AMPL. + ALC 16p

TDA1003A-PHI SPEED CTR. + ERASE 16p

TDA1004A

TDA1005A-PHI

TDA1008-PHI

TDA1009

TDA1010A-PHI PRE+PWR AMP 1*10W/2E 18V

TDA1011-PHI PRE+PWR AMP 1*6.5W/4E 24V

TDA1011A-PHI ALSO: 209.80934

TDA1011Q 2-6W AUDIO (PRE)AMP. 15kH

TDA1013A-PHI 4W AUDIO AMPLIFIER 9p

TDA1013B-PHI PWR AMP 1*10W/16E 40V+DC

TDA1015-PHI PRE+PWR AMP 1*4W/4E 18V

TDA1016-PHI REC/PLAY.PRE+PWE AMP 2W/4

TDA1020-PHI 12W AUDIO POWER AMPL. 9p

TDA1022-PHI

TDA1023-PHI TRIAC TRIGGER PROP.CNTRL

TDA1026

TDA1028/DIV QUAD OPAMP (AF SELECT)16p

TDA1029-PHI DUAL OPAMP (AF SELECT)

TDA1029/DIV DUAL OPAMP (AF SELECT)

TDA1034 Echivalent: NE5534N-MBR

TDA1037

TDA1038-PHI I.C.

TDA1041E-ST SPEED CONTROLER 8p

TDA1041P SPEED CONTROLER 14p

TDA1044-ITT

TDA1044/DIV V-DEFLECTION SYSTEM 27V

TDA1044F-ITT TV VA-SYNC.+OSC.+OUTP.STA

TDA1044U-ITT

TDA1046/DIV AM RECEIVER + DEMOD.+NF-V

TDA1047-SIE FM IF AMPL.+ DEM

TDA1047/DIV FM IF AMPL.+ DEM. =A225D

TDA1048G-MBR TV AM TONE AMPLIFIER

TDA1051 Echivalent: TCA150T-BAN

TDA1053-MBR

TDA1054M-ST PLAY/REC. AMPL. + ALC

TDA1054M/DIV PLAY/REC.AMPL.+ ALC 16p

TDA1057 REF-DIODE 33V

TDA1059B-PHI MOTOR SPEED REGULATOR

TDA1060-PHI SMPS CNTRL IC TV

TDA1060/DIV SMPS CNTRL IC TV =B260D

TDA1062-TFK

TDA1068 AUTORADIO NOISE INVERTER

TDA1072A-MBR AM RECEIVER IC

TDA1074A-PHI DUAL TANDEM ELEC.PTM.18p

TDA1077-PHI DTMF GENERATOR/RECEIVER

TDA1078 Echivalent: NE5539N-PHI

TDA1082-PHI EAST-WEST CORRECT.DRV.

TDA1083-TFK AM/FM AUDIO AMPLIFIER

TDA1083/DIV AM/FM AUDIO AMPLIFIER =A2

TDA1085A-MOT Echivalent: TDA1085A/DIV

TDA1085A/DIV UNIVERSAL SPEED CNTR

TDA1085C-MOT UNIVERSAL SPEED CNTR.

TDA1087-TFK 2xNPN TRANS. VHF MIXER

TDA1092

TDA1093 TUNING INTERF. FOR TDA106

TDA1097

TDA1151-ST MOTOR SPEED REG. TO126

TDA1152 Echivalent: TDA1151-ST

TDA1154-MBR SPEED CONTROLER

TDA1170D-MBR TDA1170 IN DIP16

TDA1170H VERT DEFL HOR FINDIP!

TDA1170N-MBR VERT DEFL 12pQ FINDIP

TDA1170N-ST Echivalent: TDA1170N-MBR

TDA1170N/DIV TV/VIDEO CIRCUIT

TDA1170S-ST Echivalent TDA1170S/DIV

TDA1170S-TFK Echivalent TDA1170S/DIV

TDA1170S/DIV V-DEFLECTION SYSTEM 27V

TDA1170SH/DIV V-DEFLECTION SYSTEM 27V

TDA1175-ST

TDA1175M-ST TV VERT. DEFLECTION

TDA1175P-ST TV VERT. DEFLECTION SYST.

TDA1180P-ST TV HORIZ.SYNCHR.+OSC

TDA1185A-MOT AC SPEED CONTROLER 14p

TDA1190Z-ST

TDA1200 Echivalent: CA3089E-MBR

TDA120T Echivalent: TBA120T-MBR

TDA1220A-ST Echivalent: TDA1220B-MBR

TDA1220B-MBR

TDA1235 TV SOUND AMPLIFIER 18p

TDA1236 TV SOUND AMPLIFIER 18p

TDA1251 CONSTANT CURRENT-SOURCE

TDA1270Q/DIV TV VERT.SYNCHR.+OSC 12PQ

TDA1301T-PHI DIGITAL CD SERVO CONTROLL

TDA1308-PHI PWR AMP 1*60mW/32E 7V HEA

TDA1308T-PHI PWR AMP 1X60W/32E 7V HDPH

TDA1311A-PHI DAC 2*16BIT CONT. CALIBRA

TDA1327 Echivalent: MC1327AP-MBR

TDA1405-ST POS V-REG 5V 1.2A =L129

TDA1412 POS V-REG 12V

TDA1415

TDA1432-PHI

TDA1490 Echivalent: L149V-ST

TDA1506-PHI Echivalent 209.81177

TDA1508 RECORD.SERVO+AUTOREV.18p

TDA1510AQ-PHI PWR AMP 2*12W/2E 18V BTL

TDA1510Q-PHI 2x12W POWER AMPL.13p(bent

TDA1510S1 Echivalent: TDA1510AQ-PHI

TDA1512Q Echivalent: TDA1512AQ-PHI

TDA1514A-PHI PWR AMP 1*65W/4E 30V

TDA1514AQ-PHI NF-AMPL 27.5V 42W (8E)

TDA1514N4 Echivalent: 209.70938

TDA1514N7 Echivalent: 209.61576

TDA1514Q-PHI Echivalent: TDA1514AQ-PHI

TDA1515A Echivalent TDA1515AQ-PH

TDA1515AQ-PHI Echivalent TDA1515BQ-PH

TDA1515BQ-PHI PWR AMP 2*12W/2E 18V+PROT

TDA1515Q Echivalent: TDA1515AQ-PHI

TDA1516AQ-PHI Echivalent: TDA1516BQ-PHI

TDA1516BQ-PHI PWR AMP 2*12W/2E 18V BTL

TDA1516Q-PHI Echivalent: TDA1516BQ-PHI

TDA1517-PHI PWR AMP 2*6W/4E 18V

TDA1518BQ-PHI PWR AMP 2*12W/2E 18V BTL

TDA1518Q-PHI Echivalent: TDA1518BQ-PHI

TDA1519-PHI PWR AMP 2*6W/2E 18V

TDA1519A-PHI PWR AMP 2*11W/4E 18V BTL

TDA1519AQ-PHI PWR AMP 2x11W/4E 18V BTL

TDA1519B-PHI PWR AMP 2*6W/4E 18V BTL

TDA1520A-PHI Echivalent: TDA1520B-PHI

TDA1520AQ Echivalent: TDA1520B-PHI

TDA1520B-PHI Echivalent 209.71743

TDA1520Q ALSO: TDA1520BQ-PHI

TDA1521-PHI PWR AMP 2*12W/8E 21V BTL

TDA1521A-PHI PWR AMP 2*6W/8E 21V BTL

TDA1521AQ-PHI PWR AMP 2*6W/8E 21V BTL

TDA1521Q-PHI PWR AMP 2*12W/8E 21V BTL

TDA1522 STEREO PLAYBACK AMP/EQ 9p

TDA1524A-PHI STEREO TONE/VOL.CONTR.

TDA1524A/DIV STEREO TONE/VOL.CTR.

TDA1526-PHI STEREO TONE/VOLUME CTR.

TDA1533 Echivalent: 209.80827

TDA1540P-PHI Echivalent 209.81453

TDA1541-PHI DAC 2x16B STEREO

TDA1541A-PHI DAC 2x16B 1LSB 0.5us

TDA1542-PHI ACTIVE POST FILTERING IC

TDA1543-PHI DAC 2x16B 0.5us I2S-FORMA

TDA1543A-PHI DAC 2x16B 0.5us JAP.FORMA

TDA1546T-PHI FILTER DSP BITSTR. CONV.

TDA1547-PHI DAC 2*20B SERIAL INPUT

TDA1550 Echivalent: ZTK33B

TDA1552 Echivalent: TDA1552Q-PHI

TDA1552Q-PHI PWR AMP 2*22W/4E 18V BTL

TDA1553 Echivalent: TDA1553Q-PHI

TDA1553AQ-PHI Echivalent TDA1553Q-PHI

TDA1553CQ-PHI Echivalent TDA1553Q-PHI

TDA1553Q-PHI PWR AMP 2*22W /4E 18V BTL

TDA1554Q-PHI PWR AMP 2*22W/4E(4*11W/2E

TDA1555Q-PHI PWR AMP 2*22W/4E(4*11W/2E

TDA1556Q-PHI PWR AMP 2x22W/4E 18V BTL

TDA1557Q-PHI PWR AMP 2x22W/4E 18V BTL

TDA1558 Echivalent: TDA1558Q-PHI

TDA1558Q-PHI PWR AMP 2*22W/4E(4*11W/2E

TDA1559 Echivalent: TDA1059B-PHI

TDA1560 Echivalent: TDA1560Q-PHI

TDA1560Q-PHI PWR AMP 1*40W/8E 18V

TDA1561Q-PHI PWR AMP 2*23W/4E 18V

TDA1571 BALANCED MIXER MODULT.16p

TDA1572-PHI AM CAR RADIO RECEIVER 18p

TDA1574-PHI INTEGRATED FM TUNER

TDA1576-PHI FM/IF AMPL.+DETECTOR

TDA1578A-PHI TIME PLL STEREO DEC.

TDA1579-PHI TRAFIC WARNING DECOD.

TDA1589 TRAFIC CONTROL TONE 18p

TDA1591-PHI PLL STEREO DEC.+NOISE CAN

TDA1594-PHI LED DISP.DRIVER, 11 LEDS

TDA1596-PHI FM/IF AMPL./DETECTOR

TDA1598-PHI TIME PLL STEREO DECOD.18p

TDA1600-PHI MULTI-FUNC.OSC SWITCH 24p

TDA1602A-PHI DOUBLE DECK PLAYB./RECORD

TDA1670 Echivalent: TDA1675A-ST

TDA1670A Echivalent: TDA1675A-ST

TDA1672A-ST

TDA1675 Echivalent: TDA1675A-ST

TDA1675A-ST TV VERT.DEFL. + OSCILL.

TDA1701-ITT

TDA1770-KIT Echivalent MODULE

TDA1770A-ST Echivalent TDA1770-KIT

TDA1771-ST VERTICAL DEFLECTION I.C.

TDA1870A ALSO: TDA1675A-ST

TDA1872A-ST Echivalent TDA1675A-ST

TDA1904-ST 4W AUDIO AMPLIFIER 16p

TDA1905-ST 6W AUDIO AMP.+ MUTING 16p

TDA1908-ST

TDA1908A-ST

TDA1910-ST 10W AUDIO AMPLIFIER 11p

TDA1940-MBR

TDA2002 Echivalent: TDA2002-ST

TDA2002-ST AUDIO AMPL. VERTICAL 8W

TDA2002/DIV 8W AUDIO AMPL. VERTICAL

TDA2002H-ST AUDIO AMPL. HORIZONTAL 8W

TDA2002H/DIV Echivalent: TDA2002/DIV

TDA2002V Echivalent: TDA2002-ST

TDA2003 Echivalent: TDA2003-ST

TDA2003-ST AUDIO AMPL. 10W VERTICAL

TDA2003/DIV 10W AUDIO AMPL. VERTICAL

TDA2003H-ST AUDIO AMPL. HORIZONT. 10W

TDA2003V Echivalent: TDA2003-ST

TDA2004-ST Echivalent: TDA2004R-ST

TDA2004R-ST STEREO AMPL. 2x10W

TDA2005M-ST Echivalent: TDA2005S-ST

TDA2005R-ST PWP AMP 1X20W/4E 28V BRDG

TDA2005S-ST PWR AMP 2x10W/4E 28V

TDA2006 Echivalent: TDA2006-MBR

TDA2006-MBR 12W AUDIO AMPLIFIER VERT.

TDA2006/DIV Echivalent: TDA2006H/DIV

TDA2006H-ST 12W AUDIO AMPLIFIER HOR.

TDA2006H/DIV Echivalent: TDA2006H-ST

TDA2006V Echivalent: TDA2006-MBR

TDA2007-ST PWR AMP 2*6W/4E

TDA2007A-ST PWR AMP 2*6W/4E SC-PROT

TDA2008-ST Echivalent TDA2008/DIV

TDA2008/DIV AUDIO AMPL. 1*12W

TDA2009-ST 2x11W STEREO AMPL. 11pQ

TDA2009A-ST 2x11W STEREO AMPL. 11p

TDA2009S-ST Echivalent: TDA2009-ST

TDA200SP Echivalent: L200CV-ST

TDA2010N/DIV Echivalent: TDA2020/DIV

TDA2010Q NF AMPL 18V 3.5A 12W

TDA2020-ST ALSO: TDA2020/DIV

TDA2020/DIV 20W AUDIO POWER AMP.=MDA2

TDA2020AD2 Echivalent: TDA2020-ST

TDA2020DQ Echivalent: 209.80687

TDA2030 Echivalent: TDA2030-ST

TDA2030-ST 18W 18V AUDIO AMPL. VERT.

TDA2030/DIV PWR AMP 1x14W/4E 18V VERT

TDA2030A Echivalent: TDA2030AV-ST

TDA2030AV-ST 18W 22V AUDIO AMPL. VERT.

TDA2030AV/DIV Echivalent: TDA2030AV-ST

TDA2030H-MBR 18W 18V AUDIO AMPL. HOR

TDA2030V Echivalent: TDA2030-ST

TDA2040-ST AUDIO AMPL. 22W 4A 20V

TDA2040/DIV PWR AMP 1*22W TO220/5p

TDA2040H-ST Echivalent TDA2040-ST

TDA2040H/DIV Echivalent: TDA2040H-ST

TDA2040V Echivalent: TDA2040-ST

TDA2050H Echivalent: TDA2050V-ST

TDA2050P Echivalent: TDA2050V-ST

TDA2050V-ST 25W HIFI AUDIO AMPLIFIER

TDA2051 Echivalent: TDA2051V-ST

TDA2051V-ST PWR AMP. 1x40W

TDA2052H Echivalent: TDA2052V-ST

TDA2052V-ST PWR AMP 1x65W HIFI

TDA2054M RECORDER PLAY/REC.AMP 16p

TDA2104-PHI Echivalent: MN3004-MAT

TDA2105 Echivalent MN3005-MAT

TDA2107-PHI Echivalent: MN3007-MAT

TDA2108-PHI Echivalent: MN3008-MAT

TDA2140-TFK

TDA2148-SIE TV-AM SOUND (FRANCE)

TDA2165-MBR Echivalent: TEA2165-ST

TDA2170 Echivalent: TDA2170-ST

TDA2170-KIT Echivalent KIT

TDA2170-ST Echivalent TDA2170-KIT

TDA2220-ST AM/FM AMPLIFIER 20p

TDA2270-KIT ERSATZSCHALTUNG

TDA2270-ST Echivalent: TDA2270-KIT

TDA2320AN-ST STEREO PREAMPLIFIER 8p

TDA2320N-ST STEREO PREAMPLIFIER 8p

TDA2440-3 TV/VIDEO IF STAGE + AGC

TDA2451-2-SIE

TDA2460-2-SIE TV AM/FM MULTISTANDARD 20

TDA2501-PHI PAL-NTSC ENCODER 16p

TDA2502-PHI MOTOR CONTROL 18p

TDA2503-PHI

TDA2504-PHI FM MODEM 8mm VTRs 24p

TDA2505-PHI

TDA2506-PHI SECAM ENCODER 24p

TDA2506T-PHI SMD SECAM ENCODER 24p

TDA2507-PHI SECAM FM MOD/CONTROLER

TDA2507T-PHI SMD FM VCR MOD/CONTR. 16p

TDA2510

TDA2514 Echivalent: TDA2514A

TDA2514A

TDA2515-PHI FM MOD./DEMOD.8mm VTRs 40

TDA2523-PHI

TDA2525-PHI

TDA2525Q Echivalent: TDA2525-PHI

TDA2530-MBR RGB MATRIX PREAMP. 15V

TDA2532/DIV TV RGB MATRIX =A232D

TDA2540 IF AMPL./DEMOD.(NPN) 16p

TDA2540Q Echivalent: TDA2540

TDA2541-MBR IF AMP./DEMOD.(PNP)

TDA2541/DIV IF AMPL./DEMOD.(PNP)

TDA2541Q IF AMPL./DEMOD.(PNP)

TDA2542-MBR IF AMPL./DEMOD.(PNP) 16p

TDA2543-PHI AM SOUND IF CIRCUIT

TDA2544 IF AMPL./DEMOD.(MOS) 16p

TDA2545A-PHI QUASI-SPLIT SOUND CIR.

TDA2545Q QUASI-SPLIT SOUND CIR.16p

TDA2546A-PHI QUASI SPLIT SOUND CIR.1

TDA2548-PHI IF AMPL./DEMOD.(PNP) 16p

TDA2549-PHI IF AMPL+DEMOD.FOR TV 24p

TDA2555-PHI DUAL FM DEMOD.

TDA2556-PHI QUASI SPLIT SOUND CIR.

TDA2557-PHI DUAL FM DEMODULATOR

TDA2558-PHI QUASI SPLIT SOUND+DEMODU.

TDA2560 Echivalent TDA2560-3-SI

TDA2560-3-SIE TV PAL VIDEO + CHROMA

TDA2560Q

TDA2574V Echivalent TDA2574VB

TDA2574VB I.C.

TDA2577A-PHI TV HOR/VERT SYNCHR.

TDA2578A-PHI TV HOR/VERT SYNCHR. 18p

TDA2579A-PHI SYNCHR.CIRC.(625 LINES)18

TDA2579B-PHI TV HA/VA COMB.

TDA2581-PHI Echivalent: TDA2581/DIV

TDA2581/DIV SMPS CNTRL TV-SN

TDA2581Q-PHI Echivalent: TDA2581Q/DIV

TDA2581Q/DIV SMPS CNTRL TV-SN

TDA2582-PHI SMPS CNTRL IC

TDA2582/DIV SMPS CNTRL IC

TDA2590 Echivalent: TDA2593-MBR

TDA2591 IMPROVED TDA2590 16p

TDA2591Q-PHI IMPROVED TDA2591 16pQ

TDA2593-MBR HOR. COMBINATION

TDA2593/DIV Echivalent TDA2593A-TFK

TDA2593A-TFK HORIZONTAL COMBINAT.

TDA2593Q HORIZONTAL COMBINAT 16 pQ

TDA2594-PHI

TDA2595-PHI HOR.COMB.+TRANSM.&PROT.

TDA2596

TDA2600-PHI CTV-VERT. DEFLECTION DIP1

TDA2611A-PHI PWR AMP 1*10W/16E 35V

TDA2611AQ-PHI PWR AMP 1*10W/16E 35V

TDA2613-PHI PWR AMP 1*10W/4E 42V

TDA2613Q-PHI PWR AMP 1*10W/4E 42V

TDA2614-PHI PWR AMP 2*6W

TDA2615-PHI PWR AMP 2*6W/8E 21V BTL

TDA2616-PHI PWR AMP 2x12W/8E 21V

TDA2616Q-PHI PWR AMP 2x12W/8E 21V

TDA2620

TDA2630-PHI

TDA2631-MBR

TDA2640 Echivalent: TDA2640-PHI

TDA2640-2-SIE Echivalent: TDA2640-PHI

TDA2640-PHI

TDA2652 IC =8305302265

TDA2653-KIT ERSATZSCHALTUNG

TDA2653/DIV VERTICAL DEFLECTION SYSTE

TDA2653A-PHI VERT.DEFLECT.CIRCUIT

TDA2653Q-PHI Echivalent: TDA2653A-PHI

TDA2654-PHI VERT.DEFLECT.CIRCUIT 9p

TDA2655-KIT Echivalent MODULE

TDA2655A-KIT ERSATZSCHALTUNG

TDA2655B-KIT Echivalent KIT

TDA2658-PHI TV VERT.DEFL.OSCIL.OUTPUT

TDA2670 TV AMPLIFIER 16p

TDA2710-1 VTR COLOR SIGNAL 16p

TDA2721 VTR 8.8MHz + MIXER 16p

TDA2730 FM LIMITER/DEMOD. VTRs 16

TDA2740 VTR DROP OUT DETECTOR 16p

TDA2750-PHI BUS INTERFACE

TDA2790

TDA2791-PHI TV SOUND

TDA2791Q TV SOUND

TDA2795 TV SOUND DECODER 18p

TDA2820M NF AMPL

TDA2822 Echivalent: TDA2822M-ST

TDA2822-ST PWR AMP 1W LOW VOLTAGE

TDA2822M-ST 1W LOW VOLT.AMPLIFIER 8p

TDA2822S-ST 1W LOW VOLT. AMPLIFIER

TDA2824S-ST

TDA2840-SIE TV QUASI PARALL.TONE 14p

TDA2910-MBR I.C.

TDA3030B-MOT

TDA3047-PHI IR RC PRE-AMPLIFIER 16p

TDA3048-PHI

TDA3060 Echivalent: TDA1060-PHI

TDA3082 Echivalent: CA3082-MBR

TDA3083 Echivalent: CA3083-HAR

TDA3190-MBR COMPL.TV SOUND CHANN.

TDA3301B-MOT

TDA3303-MOT Echivalent: TDA3301B-MOT

TDA3310 5 CH TRANSISTOR ARRAY

TDA3320-MBR Echivalent: TCA971-SIE

TDA3410 2xREC/PLAY AMPLIFIER 16p

TDA3420-ST DUAL PREAMP.LOW NOISE 16p

TDA3500

TDA3501/DIV COLOR DECODER CIRCUIT

TDA3502 VTR+TV RGB AMP. 28p

TDA3504-PHI VIDEO CONTR.COMB.

TDA3505-MBR VIDEO CNTRL COMB=TDA3505G

TDA3505/DIV VIDEO CTRL. COMB.=MDA3505

TDA3505G-MBR VIDEO CONTROL COMB.

TDA3506-MBR

TDA3507-PHI

TDA3508-PHI

TDA3510/DIV PAL DECODER =MDA3510

TDA3520/DIV TV SECAM DECODER 28p

TDA3521-MBR CTV SECAM DECODER

TDA3530/DIV SECAM DECODER =XA055

TDA3540-PHI

TDA3541-PHI IF AMP + DEMOD(PNP) 16p

TDA3541/DIV Echivalent: TDA3541-PHI

TDA3560-MBR Echivalent TDA3561A-PHI

TDA3560/DIV Echivalent TDA3560-MBR

TDA3561/DIV CTV PAL DECODER

TDA3561A-PHI TV PAL DECODER 28p

TDA3561A/DIV Echivalent: TDA3561A-PHI

TDA3561AQ Echivalent: TDA3561A-PHI

TDA3561B-PHI TV PAL DECODER 28p

TDA3562-TFK Echivalent: TDA3562A/DIV

TDA3562A Echivalent: TDA3562A/DIV

TDA3562A-PHI Echivalent TDA3562A/DIV

TDA3562A-ST Echivalent TDA3562A-PHI

TDA3562A-TFK Echivalent: TDA3562A/DIV

TDA3562A/DIV TV PAL/NTSC DEC

TDA3562AP Echivalent: TDA3562A/DIV

TDA3563A-PHI

TDA3564-PHI

TDA3565-PHI TV PAL DECODER 18p

TDA3566-PHI PAL/NTSC DECODER

TDA3566A-PHI PAL/NTSC DECODER

TDA3566P Echivalent: TDA3566-PHI

TDA3567-PHI CTV PAL/NSTC DECODER

TDA3569-PHI DECODER/ENCODER PAL VIDEO

TDA3570-PHI TV NTSC DECODER 28p

TDA3576A Echivalent: TDA3576B-PHI

TDA3576B-PHI CTV-SYNCHR. + OSC.

TDA3580-MBR

TDA3590-PHI TV SECAM/PAL TRANSCOD.24p

TDA3590A-PHI TV SECAM/PAL TRANSCOD.24p

TDA3591A-PHI

TDA3592A-PHI TV SECAM/PAL TRANSCOD.24p

TDA3601Q-PHI 6xPOS V-REG

TDA3602-PHI 3xPOS V-REG

TDA3603 Echivalent: TDA3603P-PHI

TDA3603P-PHI MULTIPLE V-REG WITH SWITC

TDA3611-PHI CUSTOM I.C.

TDA3640-PHI SMPS CONTROLER

TDA3645-PHI

TDA3650-PHI CTV VA-O, VA-E

TDA3650Q Echivalent: TDA3650-PHI

TDA3651-PHI Echivalent: TDA3653B-PHI

TDA3651A-PHI Echivalent: TDA3653C-PHI

TDA3651AQ-PHI Echivalent: TDA3653CQ-PHI

TDA3651Q Echivalent: TDA3653B-PHI

TDA3652AQ Echivalent TDA3654-PHI

TDA3652Q Echivalent: TDA3654Q-PHI

TDA3653 Echivalent: TDA3653B-PHI

TDA3653A Echivalent: TDA3653C-PHI

TDA3653AQ Echivalent: TDA3653CQ-PHI

TDA3653B-PHI VERT.DEFL. & GUARD 10-40V

TDA3653C-PHI VERT.DEFLECT.+GUARD 9P

TDA3653CQ-PHI VERTICAL DEFLECTION+GUARD

TDA3653Q Echivalent: TDA3653CQ-PHI

TDA3654-PHI VERT.DEFLECT. + GUARD 9p

TDA3654A Echivalent: TDA3654-PHI

TDA3654Q-PHI VERT.DEFL.+GUARD 9P.Q.

TDA3700 Echivalent: TDA3701

TDA3701 PAL SYNCHR.PROCES.28p

TDA3710 PAL CHROM.SIGNAL PROC.28p

TDA3720 VC SECAM PROCESSOR 18p

TDA3724-PHI SECAM IDENTIFICAT.CIRC.18

TDA3725 SECAM CHROMIN.PROCES. 18p

TDA3730-PHI FREQ.DEMOD.+DROP OUT 28p

TDA3740-PHI

TDA3755-PHI PAL/NTSC SYNC.PROCESS.

TDA3760-PHI PAL CHROM.SIGNAL PROC.28p

TDA3766-PHI I.C.

TDA3770 Echivalent: TDA3771-PHI

TDA3771-PHI VIDEO PROCESSOR 18p

TDA3780 VTR FREQ.MODULATOR 18p

TDA3791

TDA3800A Echivalent TDA3800G-PHI

TDA3800G-PHI STEREO/DUAL SOUND PROC.

TDA3800GS-PHI

TDA3803A-PHI STEREO/DUAL SOUND DEC.

TDA3810-PHI STEREO/PSEUDO STEREO

TDA3826-PHI

TDA3827-PHI TV SOUND MODULATOR

TDA3840-PHI TV IF AMP+DEMOD. 20p

TDA3842-PHI MULTIST.TV IF AMP.+DEMOD.

TDA3843-PHI AM TV SOUND DEMOD.

TDA3845-PHI VCR QUASI-SPLIT SOUND 16p

TDA3856-PHI VCR QUASI-SPLIT SOUND PRO

TDA3857-PHI TV/VIDEO SOUND CIRCUIT

TDA3858-PHI TV/VIDEO SOUND CIRCUIT

TDA3950A-MOT TV PAL/CHROMA + OSC 14p

TDA4001-SIE AM-RADIO + DEMODULATOR

TDA4050B-SIE IR RC PREAMPLIFIER

TDA4050B/DIV IR RC PREAMPLIFIER

TDA4052-SIE ALSO: 8945901380

TDA4060-SIE

TDA4092-ST 2 DIGIT DECODER DRIV. 24p

TDA4100 AM/FM RADIO AMPLIFIER 22p

TDA4150-SIE

TDA4173AF-TFK Echivalent: TDA8177-ST

TDA4180 IR/US RECEIVER AMPL. 8P

TDA4200-SIE FM-DEMOD +STOP IMPULS 18P

TDA4210-TFK CAR RADIO FM IF IC =U4985

TDA4250-MBR Echivalent: TDA4250B-TFK

TDA4250B-TFK

TDA4260-SIE TV + AFC AMPLIFIER 8p

TDA4280T Echivalent: TDA4280U-SIE

TDA4280U-SIE TV QUASI PARR.TONE 18p

TDA4281T Echivalent: TDA4283T-SIE

TDA4282T-SIE CTV VIDEO AMPLIFIER

TDA4283T-SIE QUASI-PARALLEL SOUND TV-F

TDA4290-2-SIE

TDA4290V TONE CONTROL STEREO 16p

TDA4292-SIE

TDA440 Echivalent: TDA440S-MBR

TDA440-MBR VIDEO IF AMPLIFIER 15V

TDA440/DIV VIDEO IF AMPLIFIER 15V

TDA4400 IF SYST.+AFC(PNP) 16p

TDA4405 Echivalent: TDA440S-MBR

TDA440S-MBR IF SYST.+PREAMP.(PNP) 16p

TDA440S/DIV Echivalent TDA440S-MBR

TDA4420-MBR COLOR TV VIDEO IF AMPL.

TDA4421-TFK

TDA4422 Echivalent: TDA5600-MBR

TDA4426-TFK IF SYST.+ AFC(PNP) 18p

TDA4427-MBR

TDA4427A Echivalent: TDA4427-MBR

TDA4431-ST TV AFC CONTROL SIGN.IDENT

TDA4431-TFK TV AFC CONTROL 14p

TDA4432-TFK

TDA4433-ST TV AFC CONTRL 14p

TDA4437-TFK TV-VIDEO AMPL. AFC INV. 1

TDA4437S-TFK TV CIRCUIT

TDA4439-TFK TV VIDEO ZF

TDA4440-MBR

TDA4442-TFK IF SYST.+ AFC 16p

TDA4443-MBR MULTI IF SYST.+ AGC

TDA4445A-TFK Echivalent TDA4445B-TFK

TDA4445B-MBR QUASI PARAL.SOUND AM/FM-D

TDA4445F-TFK QUASI PARAL.SOUND PROC

TDA4450-TFK TV VIDEO-ZF AGC NPN-TUNER

TDA4452-TFK VIDEO IF AMPLIF FOR TV/VC

TDA4453-5 TDA4453-TFK

TDA4453-TFK TV-VIDEO AMP. MULTISTANDA

TDA4454-TFK VIDEO-IF D2MAC PLL DETECT

TDA4455A-TFK VIDEO-IF D2MAC PLL DETECT

TDA4480-2 Echivalent: TDA4480-TFK

TDA4480-TFK MLTI STD STEREO SOUND PRO

TDA4481-TFK MLTI STD MONO SOUND PROC.

TDA4482-TFK

TDA4483-TFK TV/VCR SOUND PROCESSING I

TDA4484-MBR

TDA4500-PHI SIGNAL COMB. B/W TV 28p

TDA4501-PHI SIGNAL COMB. COLOUR TV 28

TDA4502A-PHI

TDA4503-PHI

TDA4504 Echivalent: TDA4504B-PHI

TDA4504A-PHI

TDA4504B-PHI TV/CTV SIGNAL PROCESSOR

TDA4505 Echivalent: TDA4505E-PHI

TDA4505A-PHI Echivalent: TDA4505E-PHI

TDA4505E-PHI TV SIGNAL/SOUND PROCES.28

TDA4505K-PHI

TDA4505M-PHI CTV SIGNAL PROCESSOR

TDA4510-PHI TV PAL DECODER 16p

TDA4532-PHI VIDEO PALL/SECAM/NTSC ENC

TDA4550-PHI

TDA4554-PHI I.C.

TDA4555-MBR TV COLOUR DEC. NEG-SIGNAL

TDA4555/DIV COLOR DECODER CIRCUIT

TDA4556-MBR TV COLOUR DEC. POS-SIGNAL

TDA4556/DIV TV COLOUR DEC. POS-SIGNAL

TDA4557-MBR TV PAL/SECAM/NTSC DEC

TDA4560 Echivalent: TDA4565-MBR

TDA4565-MBR CHROMA VIDEO CTI IC

TDA4565/DIV CHROMA VIDEO CTI IC

TDA4566-PHI CTV Y-DELAY 550..820ns

TDA4568-PHI LUMINANCE PROCESSOR

TDA4570-PHI NTSC DECODER

TDA4580-PHI VTR CONTROL COMB.

TDA4580/DIV VTR CONTROL COMB.

TDA4600 Echivalent: TDA4601-ST

TDA4600-2/DIV SMPS CONTROL IC

TDA4600-2D-SIE Echivalent: TDA4601B-ST

TDA4600-3 Echivalent: TDA4601-ST

TDA4600-MOT Echivalent: TDA4601-ST

TDA4601-SIE SMPS CNTRL 350W 90kHz BIP

TDA4601-ST Echivalent TDA4601-SIE

TDA4601/DIV SMPS CNTRL 350W 90kHz BIP

TDA4601B-ST SMPS CNTRL 120W 90kHz BIP

TDA4601D-SIE Echivalent: TDA4601B-ST

TDA4601D/DIV SMPS CNTRL 350W 90kHz BIP

TDA4605 Echivalent: TDA4605-GS

TDA4605-15 Echivalent: TDA4605-SIE

TDA4605-15-SIE Echivalent: TDA4605-SIE

TDA4605-2-SIE SMPS CNTRL 165kHz MOS

TDA4605-2/DIV Echivalent TDA4605-2-SI

TDA4605-3-SIE SMPS CNTRL 200kHz MOS

TDA4605-5 Echivalent: TDA4605-2-SIE

TDA4605-GS SMPS CNTRL IC =GL8905

TDA4605-SIE SMPS CNTRL 200kHz

TDA4605-ST SMPS CNTRL 200kHz MOS

TDA4610-KIT Echivalent FOR TDA4610

TDA4650-PHI CTV PAL/SECAM

TDA4651-PHI CTV PAL/SECAM

TDA4660-PHI VCR 64ms DELAY LINE

TDA4660T-PHI Echivalent TDA4662T-PHI

TDA4661-PHI VCR CMOS DELAY LINE

TDA4662-PHI TV/VIDEO DELAY LINE CIRC.

TDA4662T-PHI TV/VIDEO DELAY LINE CIRC.

TDA4665-PHI TV/VIDEO SIGNAL PROCESSOR

TDA4670-PHI PICTURE SIGNAL IMPR. IC

TDA4671-PHI TV/VIDEO SIGNAL PROCESSOR

TDA4680-PHI VIDEO PROCESSOR +WHITE CN

TDA4681-PHI VIDEO PROC. I2C, NTSC, JA

TDA4685-PHI VIDEO/AUDIO SIGNAL PROCES

TDA4686-PHI

TDA4687-PHI VIDEO PROCESSOR

TDA4700A-SIE Echivalent: TDA4700A/DIV

TDA4700A/DIV SMPS CNTRL IC (BIP. TRANS

TDA4710H-PHI Echivalent 209.61671

TDA4718-SIE Echivalent: TDA4718A-SIE

TDA4718A-SIE SMPS CNTRL IC (BIP.TRANS)

TDA4720-PHI VIDEO CORRECTION CIRCUIT

TDA4725-PHI CHROMA PROCESSOR

TDA4780-PHI TV/VIDEO SIGNAL PROCESSOR

TDA4800-PHI VERT.DEFLECTION CIRC.

TDA4810-PHI

TDA4814A-SIE SMPS CNTRL IC (BIP.TRANS)

TDA4820T-PHI TV/VIDEO SYNC. CIRCUIT

TDA4850-PHI H/V DEF.CTR FOR MONITORS

TDA4850/DIV H/V DEF. CTR FOR MONITORS

TDA4851-PHI HOR/VERT.CTR.FOR MONITORS

TDA4852-PHI MONITOR H/V SYNC PROCESSO

TDA4854-PHI MONITOR H/V SYNC PROCESSO

TDA4855-PHI AUTOSYNC DEFLECTION CONTR

TDA4858-PHI ECON. AUTOSYNC DEFL.CTRL

TDA4860-PHI V-DEFL. AMPL.140Hz 30V

TDA4866-PHI V-DEFL. AMPL. 140Hz 25V

TDA4880-PHI CRT/VIDEO CONTROLER

TDA4881-PHI MONITOR VIDEO-AMP. 70MHz

TDA4882-PHI CRT/VIDEO CONTROLER

TDA4884-PHI SINGLE VIDEO AMPLIFIER

TDA4918A-SIE

TDA4919A

TDA4920-SIE

TDA4925-SIE PWR AMP 3x5.2W/4E 25V 2A

TDA4930-SIE

TDA4935-SIE

TDA4940-SIE TV STEREO TONE

TDA4941-SIE

TDA4942-SIE TV STER.MATRIX+REC CON.16

TDA4944-SIE TV STEREO MATRIX 14p

TDA4950-ST E/W PARABOLIC SQUARE CRT

TDA4951-TFK TV EAST/WEST CORR. CIRCUI

TDA5030A-PHI VHF MIXER/OSCILATOR

TDA5030A/DIV VHF MIXER/OSCILLATOR

TDA5140 Echivalent: TDA5140A-PHI

TDA5140A-PHI MOTOR DR. 3x 0.85A

TDA5330T-PHI VHF/UHF/HYP/ MIX/OSCILL.

TDA5331T-PHI SMD TDA5330 MIRROR PINS

TDA5332T-PHI 2xMIXER/OSC. 20p

TDA5400-SIE

TDA5500-SIE

TDA5510-2-SIE Echivalent 8945901476

TDA5600-MBR

TDA5620-SIE

TDA5630-MBR CTV SECAM DECODER 24p DIP

TDA5631-MBR 3-BAND MIXER/OSCIL. 3V

TDA5660P-SIE

TDA5666-5-SIE VIDEO-MODULATOR FM AUDIO

TDA5700 AM/FM RADIO AMPL. 16p

TDA5701 DAC 1x8B

TDA5702 DAC 1x8B

TDA5708-PHI PHOTO DIODE SIGN.PROC.28p

TDA5708T-PHI PHOTO DIODE SIGN.PROC SMD

TDA5709-PHI RADIAL ERROR SIGN.PROC.20

TDA5709T-PHI SMD RAD. ERROR SIGN.PROC.

TDA5820-SIE MULTI TV/VIDEO IF AMP 22p

TDA5830-2-SIE

TDA5832-SIE

TDA5850-SIE VIDEO SWITCH

TDA5850/DIV VIDEO SWITCH

TDA5931 Echivalent: TDA5931-6

TDA5931-6 CTV VIDEO IF AMPLIFIER

TDA5940-2-SIE

TDA6100Q-PHI Echivalent: TDA6101Q-PHI

TDA6101Q-PHI VIDEO AMPLIFIER 8MHz

TDA6103Q-PHI VIDEO AMPLIFIER

TDA6106Q-PHI VIDEO OUTPUT AMPLIFIER

TDA6107Q-PHI VIDEO AMP 3* 4.5MHZ

TDA6111Q-PHI 16MHz VIDEO OUTPUT AMPL.9

TDA6130-5X-SIE RF MIXER SMD

TDA6130-SIE RF-MIXER

TDA6160-2S-SIE

TDA6160-2X-SIE AM/FM IF AMPLIFIER

TDA6600-2-SIE TV STEREO SOUND DEC.+MATR

TDA6610-2 Echivalent: TDA6610-5-SIE

TDA6610-5-SIE TV STEREO SOUND PROCESSOR

TDA6611-SIE Echivalent: TDA6612-2-SIE

TDA6612-2-SIE 1 CHIP STEREO PROCESSOR

TDA6612-5 Echivalent: TDA6612-2-SIE

TDA6812-5-SIE IC

TDA7000-PHI MONO FM RADIO CIRCUIT 18p

TDA7010T-PHI MONO FM RADIO CIRCUIT 16p

TDA7020T-PHI Echivalent: TDA7021T-PHI

TDA7021T-PHI STEREO/MONO FM CIRC.16p S

TDA7040T-PHI SMD PLL STEREO DECODER 8P

TDA7050-PHI PWR AMP 2*75mW/32E 6V BTL

TDA7050T-PHI PWR AMP 2*75mW/32E 6V BTL

TDA7052-PHI PWR AMP 1*1W/8E 18V

TDA7052A-PHI PWR AMP 2*1W/8E 18V DC-VO

TDA7053-PHI PWR AMP 2*1W/8E 18V BTL

TDA7053A-PHI DUAL BTL POWER DRIVER

TDA7056-PHI PWR AMP 1*5W/8E BTL

TDA7056A-PHI PWR AMP 1*5W/8E + DC VOL

TDA7057AQ-PHI PWR AMP 2*5W

TDA7057Q-PHI PWR AMP 2*3W/16E 18V BTL

TDA7072-PHI POWER/DRIVER AMPLIFIER

TDA7073-PHI DUAL POWER DRIVER 16p

TDA7073A-PHI DUAL BTL POWER DRIVER

TDA7088T-PHI AM/FM RECEIVER CIRCUIT

TDA7211-ST 3V. FM TUNER

TDA7212

TDA7220-ST AM-FM RECEIVER 16p S

TDA7220/DIV AM-FM RECEIVER

TDA7222-ST RADIO IF TUNING+SIGNAL PR

TDA7222A Echivalent: TDA7222-ST

TDA7230A-ST STEREO DEC.+HEADPH.AMP.

TDA7231A-ST 1.6W LOW VOLT.AMPL. 8p

TDA7232-ST

TDA7233-ST MUTING I.C. 16V 1A 1,6W

TDA7240AH-ST Echivalent: TDA7240AV-ST

TDA7240AV-ST 20W BRIDGE AMPLIFIER

TDA7241BV-ST AMPLIFIER 28V 3.5A 20W

TDA7241V Echivalent: TDA7241BV-ST

TDA7245-ST 5W AUDIOAMP. + MUTE 18p

TDA7245A-ST PWR AMP 5W

TDA7250-ST 15 TO 80W STEREO DRIV.20p

TDA7253-ST PWR.AMP 8W

TDA7256-ST AMPLIFIER 28V 5,5A 22W

TDA7262-ST PWR AMP 2*20W 80kHz

TDA7263-ST PWR AMP 2*12W

TDA7263M-ST PWR AMP 2*12W

TDA7264-ST PWR AMP 2*25W +MUTE

TDA7265-ST PWR AMP 2*25W +MUTE

TDA7267-ST PWR AMP 1*2W + STAND-BY

TDA7270S-ST

TDA7272A-ST 1A AUTOREV.SPEED REG.20p

TDA7273-ST STEREO CASS.PLAYBACK SYST

TDA7274-ST LOW VOLT.SPEED REG.

TDA7275A-ST 1.5A MOTOR SPEED REG.

TDA7282-ST STEREO PREAMPLIFIER

TDA7284-ST DUAL APLIFIER + ALC

TDA7285-ST STEREO PLAYB+MOTOR CTRL

TDA7294V-ST PWR AMP 2*100W

TDA7295-ST PWR AMP 2*80W/4E 40V

TDA7300-ST

TDA7300D-ST AUDIO PROCESSOR

TDA7302-ST DIG-CNTRL STEREO AUDIO PR

TDA7310-ST AUDIO PROCESSOR

TDA7312-ST DIG-CTRL STEREO AUDIO PRO

TDA7313-ST DIGITAL AUDIO PROC+LOUDNE

TDA7313D-ST DIGITAL AUDIO PROC+LOUDNE

TDA7314-ST DIG CTRL ST. AUDIO PROC+L

TDA7317-ST GRAPHIC EQUALIZER

TDA7318-ST DIG-CNTRL STEREO AUDIO PR

TDA7318D-ST SMD DIG-CTRL STEREO AUDIO

TDA7330A-ST RADIO I.C. CONSUMER AM/FM

TDA7340S-ST AUDIO SIGNAL PROCESSOR

TDA7350-ST PWR AMP 2x11W/2E 13V 0.5%

TDA7350A-ST PWR AMP 2*11W/2E BRDG

TDA7360-ST DUAL AMPLIFIER 3.5A 11W

TDA7361-ST

TDA7362-ST PWR AMP 2*11W + CLIPP-DET

TDA7365-ST DUAL CHANNEL AUDIO AMPL.

TDA7370V-ST 4x5.5W AUDIO AMP+CLIP.DET

TDA7372A-ST PWR AMP 4*6W FOR CAR RADI

TDA7374BV-ST 2x17W AUDIO AMPLIFIER

TDA7374V-ST Echivalent: TDA7374BV-ST

TDA7375H-ST Echivalent: TDA7375V-ST

TDA7375V-ST PWR AMP 4* BRIDGE

TDA7376B-ST PWR AMP 2*25W + PROG CLIP

TDA7384-ST Echivalent: TDA7384A-ST

TDA7384A-ST PWR AMP 4x25W FOR CAR-APP

TDA7385-ST PWR AMP 4*22W

TDA7386-ST PWR AMP 4*28W

TDA7394-ST PWR AMP 2*30W/4E =PAL002A

TDA7396-ST PWR AMP 1*45W BRIDGE FOR

TDA7770 RECORDER FUNCTION CONTROL

TDA780 Echivalent: CA3065E-MBR

TDA8000-PHI SMART CARD INTERFACE

TDA8114-ST VTR PROCESS.INTERFACE 20p

TDA8115-ST DUAL MOTOR DRIVER to220/7

TDA8116-ST

TDA8120B-ST MULTI VIDEO IF SYST.

TDA8124-ST VIDEO IF INTERFACE

TDA8133-ST V-REG 5.1+8V +DISABLE&RES

TDA8135-ST 2xPOS V-REG 5.1+ADJ 0.6A

TDA8137-ST 2xPOS V-REG 5.1V 1A

TDA8138-ST V-REG 5.1+12V 1A+DIS&RST

TDA8138A-ST V-REG 5.1+12V 1A +DISABLE

TDA8138B-ST V-REG 5.1+12V 1A + RESET

TDA8139-ST V-REG 5.1+ADJ. 1A+DIS&RST

TDA8140-ST DEFLECTION POWER TRANS.DR

TDA8140-TFK Echivalent: TDA8140-ST

TDA8143-ST DEFLECT.POW.TRANS.DRIVER

TDA8145-ST E/W CORRE. FOR SQUARE CRT

TDA8146-ST TV EAST-WEST CORRECTION

TDA8153-KIT Echivalent O.O.P.

TDA8160-ST FLAH MODE INFRARED RECEIV

TDA8170-ST V-DEFL AMP 35V 2.5A INTRE

TDA8171-ST V-DEFL AMP 40V 2.5A INTRE

TDA8172-ST V-DEFL AMP 35V 2.5A EXTRE

TDA8173-ST V-DEFL AMP 35V 2.5A INTRE

TDA8174-ST V-DEFL AMP. 35V 6A

TDA8174W-ST V-DEFL AMP 35V 6A C

TDA8175-ST V-DEFL AMP 35V 2.5V

TDA8177-ST V-DEFL AMP 40V 2.5A

TDA8178F Echivalent: TDA8178FS-ST

TDA8178FS-ST

TDA8179FS-ST Echivalent: STV9379F-ST

TDA8179S-ST Echivalent: STV9379-ST

TDA8185-ST H/V TV PROCESSOR

TDA8190-ST

TDA8191-ST TV SOUND CHANNEL

TDA8192-ST TV SOUND CHANNEL

TDA8196-ST SWITCH DC VOLUME CONTR.8p

TDA8198-ST DUAL AUDIO SWITCH+ DC VOL

TDA8199-ST ELECTRONIC AUDIO CONTROL

TDA8204B-ST NICAM DECODER

TDA8205-ST NICAM DECODER

TDA8212-ST CTV IF-SOUND AMPLIFIER

TDA8213-ST VIDEO IF+DC VOL.CNTRL+AFC

TDA8214B-ST HOR/VERT. DEFLECTION

TDA8215A-ST Echivalent: TDA8215B-ST

TDA8215B-ST H/V DEFLECTION 20p

TDA8216-ST CTV PAL DECODER

TDA8217-ST

TDA8218-ST Echivalent TDA8214B-ST

TDA8303-PHI SOUND IF

TDA8304-PHI MULTI STANDARD COMB.

TDA8305-PHI Echivalent: TDA8305A-PHI

TDA8305A-PHI CTV SMALL SIGN.COMBINATIO

TDA8305A/DIV Echivalent: TDA8305A-PHI

TDA8310-PHI PIP IF AMPLIFIER

TDA8340-PHI TV IF AMPL.+DEMOD. 16p

TDA8341-PHI TV IF AMPL.+ DEMOD. 16p

TDA8349 Echivalent TDA8349A-PHI

TDA8349A-PHI IF-STAGES

TDA8350Q-PHI DC-COUPLED VERT.DEFLEC.

TDA8350Q3-PHI Echivalent: TDA8350Q-PHI

TDA8351-PHI V-DEFL. AMPL. 120Hz 25V

TDA8354Q-PHI VERT DEFL IC FOR 25 TO 20

TDA8356-PHI V-DEFL. AMPL. 120Hz 25V

TDA8360N3-PHI Echivalent TDA8360N5-PH

TDA8360N4-PHI Echivalent TDA8360N5-PH

TDA8360N5-PHI ONE CHIP TV, PAL ONLY

TDA8361 Echivalent: TDA8361N3-PHI

TDA8361AN3-PHI TV SUBSYSTEM & SYSTEM

TDA8361EN4-PHI Echivalent: TDA8361EN5-PHI

TDA8361EN5-PHI TV SUBSYSTEM & SYSTEM

TDA8361N1-PHI Echivalent: TDA8361N3-PHI

TDA8361N3-PHI ONE CHIP PAL/NTSC TV SET

TDA8361N5 ONE CHIP PAL/NTSC TV SET

TDA8362 Echivalent: TDA8362N3-PHI

TDA8362AN1-PHI Echivalent: TDA8362AN2-PHI

TDA8362AN2-PHI Echivalent TDA8362AN3-P

TDA8362AN3-PHI ONE CHIP TV SYSTEM

TDA8362BN3-PHI Echivalent: TDA8362BN5-PHI

TDA8362BN4-PHI Echivalent TDA8362BN5-P

TDA8362BN5-PHI ONE CHIP TV FOR KOREAN TV

TDA8362EN5-PHI ONE CHIP TV SYSTEM

TDA8362N1-PHI Echivalent: TDA8362N3-PHI

TDA8362N2-PHI Echivalent: TDA8362N3-PHI

TDA8362N3-PHI Echivalent TDA8362N5-PH

TDA8362N4-PHI Echivalent: TDA8362N5-PHI

TDA8362N5-PHI 1-CHIP MULTISTANDARD CTV

TDA8363-PHI Echivalent 209.90191

TDA8366 Echivalent: TDA8366N3-PHI

TDA8366N2-PHI Echivalent: TDA8366N3-PHI

TDA8366N3-PHI ONE CHIP MULTISTD. I2C-BU

TDA8369-PHI

TDA8370-PHI TV/VIDEO SYNC. CIRCUIT

TDA8372A-PHI

TDA8374-PHI BUS-CNTRL 1-CHIP TV, MULT

TDA8374A-PHI BUS-CNTRL 1-CHIP TV, MULT

TDA8375A-PHI BUS-CNTRL 1-CHIP TV, MULT

TDA8376-PHI ONE CHIP MULTISTD. I2C-BU

TDA8376A-PHI 1 CHIP MULTISTD I2C-BUS 1

TDA8380-PHI PWM CNTRL IC 100KHz

TDA8380A-PHI PWM CONTROL IC 100KHz

TDA8385-PHI

TDA8390-PHI Echivalent: TDA8390A-PHI

TDA8390A-PHI PAL DECODER/RGB MATRIX

TDA8391-PHI

TDA8395P-PHI ALIGNMENT FREE SECAM DEC.

TDA8403K Echivalent: TA8403K-TOS

TDA8405-PHI STEREO/DUAL SOUND PROC.

TDA8415-PHI TV/VIDEO STEREO/DUAL PROC

TDA8416-PHI TV SYSTEM SUPPORT

TDA8417-PHI TV&VCR SOUND PROC.

TDA8420-PHI Echivalent TDA8421-PHI

TDA8421-PHI HIFI AUDIO PROCESSOR

TDA8424-PHI HIFI STEREO PROCESSOR I2C

TDA8425-PHI SOUND PROC SPATIAL/PSEUDO

TDA8427K Echivalent: TA8427K-TOS

TDA8432-PHI CCT VERT DEFL PROCESS.

TDA8433-PHI

TDA8440-PHI VIDEO/AUDIO SWITCH 18p

TDA8442-PHI CTV DEC BUS INTERF.16p

TDA8442/DIV CTV DEC BUS INTERF

TDA8443-PHI RGB/YUB SWITCH

TDA8443B-PHI RGB/YUB SWITCH

TDA8444P-PHI DAC 8x6-BIT I2C-BUS

TDA8451-PHI P2CCD DELAY LINE+MATRIX 1

TDA8451A Echivalent: TDA8451-PHI

TDA8452-PHI BUS INTERFACE

TDA8453A-PHI P2CCD FILTER COMB.18p

TDA8461-PHI PAL/NTSC DECODER

TDA8490-PHI DECAM DECODER

TDA8501-PHI PAL/NTSC DECODER

TDA8540-PHI VIDEO SWITCH MATRIX 4x4

TDA8560Q-PHI PWR AMP 2*40W/2E 18V BTL

TDA8561Q-PHI PWR AMP 2*24W/4E(4*12W/2E

TDA8562Q-PHI PWR AMP 4*12W

TDA8563Q-PHI PWR AMP

TDA8567Q-PHI PWR AMP 4*16W/4E 16V

TDA8568Q-PHI PWR AMP 4*16W/4E 14.4V

TDA8702-PHI DAC 1x8-BIT 0.5LSB 30MHz

TDA8703-PHI ADC 1x8-BIT 40MHz

TDA8708-PHI Echivalent: TDA8708A-PHI

TDA8708A-PHI ADC 1x8B 32MHZ 0.5LSB

TDA8709-PHI 32MHz VIDEO INPUT INTERFA

TDA8718N Echivalent: TA8718N-TOS

TDA8730-PHI PLL FM DEMOD. FOR DSB SIG

TDA8732-PHI NICAM-728 DEMOD.20p

TDA8735-PHI PLL FREQUENCY SYNTHESIZER

TDA8740-PHI RF REC. CIRC. DET. DEMOD.

TDA8741-PHI SOUND CIRC. + NOISE REDUC

TDA8745-PHI SATELITE SOUND CIRCUIT

TDA8808T-PHI PHOTO DIODE SIGNAL PRC.

TDA8809T-PHI PHOTO DIODE SIGNAL PRC.28

TDA9045-PHI

TDA9080-PHI

TDA9102B Echivalent: TDA9102C-ST

TDA9102C-ST H/V DEFL. PR. FOR MONITOR

TDA9103-ST H/V DEFLECTION PROCESSOR

TDA9103B-ST H/V DEFELECTION PROCESSOR

TDA9105-ST H/V DEFL CTRL FOR MULTISY

TDA9108-ST

TDA9141-PHI COLOR DECODER CIRCUIT

TDA9151 Echivalent: TDA9151B-PHI

TDA9151B-PHI L PROCESSOR/DEFLECTOR

TDA9160A-PHI COLOR DECODER CIRCUIT

TDA9162-PHI DECODER + SYNC. PROCESSOR

TDA9302H-ST V-DEFL. AMP 35V 1.8A

TDA9400 Echivalent: TDA9403-ITT

TDA9403-ITT TV LINE CIRCUIT 16p

TDA9503-ITT

TDA9513-ITT

TDA9610H-PHI RADIO SIGNAL PROCESSOR

TDA9800-PHI CHROMA DEMOD./SEPARATOR

TDA9802-PHI COLOR TV VIDEO IF AMPL.

TDA9803-PHI COLOR TV VIDEO IF AMPLIFI

TDA9806-PHI VIF-PLL & FM-PLL DEMODULA

TDA9811-PHI MULTISTD VIF-PLL

TDA9813T-PHI SINGLE STANDARD VIF-PLL

TDA9815-PHI MULTISTD MAC VIF-PLL PROC

TDA9818-PHI VIDEO IF AMPLIF

TDA9820-PHI TV FM SOUND DEMODULATOR

TDA9821-PHI RF RECIEVING CIRCUIT

TDA9830-PHI TV AM SOUND IF CIRCUIT

TDA9840-PHI TV STEREO/DUAL SOUND PROC

TDA9860-PHI TV/VIDEO SOUND CIRCUIT

TDA9875-PHI DIG TV STEREO SOUND PROC

TDB0124 Echivalent: LM124N-MBR

TDB0146DP Echivalent: LM346N-MBR

TDB1030-PHI

TDB1080-PHI

TDB1080T-PHI SMD AM IF+FM DEMOD.

TDB2033 IR RC PREAMPLIFIER 16p

TDB7910N-ST 1xPWR OPAMP 500mA

TDE1647DP Echivalent: TDE1747DP-ST

TDE1737DP-ST POWER SWITCH 45V/0.5A

TDE1747DP-ST POWER SWITCH 45V/0.5A

TDE1767DP-ST POWER SWITCH 45V 0.5A

TDSG1150 DISPLAY 7mm C.A. GRUN

TDSG1160 DISPLAY 7mm C.C. GRUN

TDSG3110 DISPLAY 10mm C.A. GRUN

TDSG3120 DISPLAY 10mm C.C. GRUN

TDSG3160 DISPLAY 10mm C.C. GRUN

TDSO1150 DISPL. 7mm C.A. ROT H.EFF

TDSO1160 DISPL. 7mm C.C. ROT H.EFF

TDSO3150 DISPL. 10mm C.A. ROT H.EF

TDSO3160 DISPL. 10mm C.C. ROT H.EF

TDSR1150 DISPLAY 7mm C.A. ROT

TDSR1160 DISPLAY 7mm C.K ROT

TDSR3150 DISPLAY 10mm C-A ROT

TDSR5150 ALSO: TDSR5156

TDSR5160 DISPLAY 13mm CC ROT 8

TDSY3160 DISPLAY 10mm C.C. GELB

TDSY5150 DISPLAY 13mm C.A. GELB

TE1299 Echivalent: 2N3716-MBR

TE1300 Echivalent: BD314

TEA0652-PHI DOLBY B&C NOISE REDUCT.18

TEA0653T-PHI DOLBY B+C 20-MDIP

TEA0654 Echivalent: NE654N-MBR

TEA0655-PHI

TEA0657-PHI DOLBY B/C NOISE REDUCTION

TEA0665-PHI DOLBY B/C, AV 6dB,THD .3%

TEA1002-PHI PAL COLOUR ENCODER

TEA1007-TFK LOW COST AC-MOTOR CNTRL

TEA1009-ITT IR RECEIVER AMP.

TEA1010

TEA1014-ST

TEA1015P I.C.

TEA1017-PHI 13-BIT SERIE-PARALLEL CON

TEA1024-TFK TRIAC CNTRL ZERO VOLT SWI

TEA1030B-ST

TEA1035-ST ANALOG SWITCH

TEA1039-PHI PWM CNTRL IC 100KHz

TEA1041T-PHI POWER SUPPLY MONITOR 1.33

TEA1042PN-PHI TELEPH. HANDSFREE SPEAKER

TEA1045-ITT TELEPHONE SUBSET AMP. 18p

TEA1060-PHI DTMF CIRCUIT+DIALLER

TEA1060/DIV DTMF CIRCUIT+DIALER

TEA1061-PHI TELEPHONE DIAL+TRANSM.18p

TEA1061/DIV TELEPHONE DIAL+TRANSMIT.

TEA1062-PHI TELEPHONE TRANSMISSION C.

TEA1062A-PHI TELEPHONE CIRCUIT

TEA1064A-PHI DTMF + DIALER + SPEECH IF

TEA1064AT-PHI DTMF + DIALER + SPEECH IF

TEA1067-PHI LOW.VOLT.SPEECH/TRANSM.18

TEA1068-PHI DTMF DIALLER/INTERFACE

TEA1080P-PHI PHONE SET PERIPHERALS 8p

TEA1081-PHI P-SUB FOR TELEPHONE SETS

TEA1087

TEA1090-PHI POWERPLUG

TEA1091-MBR IC

TEA1093-PHI TELEPHONE HANDS FREE IC

TEA1100-PHI BAT./POWER SUPPLY MONITOR

TEA1101-PHI BATTERY MONITOR NICd/NIMH

TEA1102-PHI NIMH CHARGER CONTROL+MON

TEA1330-ST STEREO DECODER 16p

TEA2000-PHI

TEA2014A-ST VIDEO SIGNAL SWITCH

TEA2014A/DIV VIDEO SIGNAL SWITCH

TEA2018A-ST SMPS CNTRL TV 0.5A

TEA2019-ST

TEA2024-ST

TEA2025B-ST 1W STEREO AMPLIFIER 16p

TEA2026A Echivalent: TEA2029A-TFK

TEA2026C-ST Echivalent: TEA2029C-ST

TEA2026CV-TFK Echivalent TEA2029CV-TF

TEA2028-ST Echivalent: TEA2028B-ST

TEA2028B-ST DEFLECT.+ SMPS PROCES.

TEA2029A-TFK

TEA2029C-ST

TEA2029CV-TFK TIMING PROC.+ SMPS CTRL I

TEA2029F-MBR Echivalent N.L.A

TEA2031A-ST E/W PARABOLIC SQUARE CRT

TEA2037A-ST H/V DEFLECTION CIRC.16p

TEA2114-ST VIDEO SIGNAL SWITCH

TEA2130-ST H/V DEFL. AUTO VCR SWITCH

TEA2164-ST SMPS CNTRL IC

TEA2164/N-ST

TEA2164G-ST Echivalent: TEA2164/N-ST

TEA2164N-ST Echivalent: TEA2164/N-ST

TEA2165-ST Echivalent TEA2164-ST

TEA2165A-ST

TEA2260-ST SMPS CNTRL TV 1.2A

TEA2261-ST SMPS CNTRL TV 1.2A

TEA2262-ST SMPS CONTROLER

TEA3717DP-ST MOTOR DRIVER

TEA3718DP-ST STEPPER MOTOR DR. 1.5A

TEA3718SDP-ST STEPPER MOTOR DR. 1.5A

TEA3718SP-ST STEPPER MOTOR DR. 1.5A

TEA5030 CTV VIDEO SIGNAL

TEA5031D-ST

TEA5040-ST WIDEBAND VIDEOPROCES.

TEA5040S-ST WIDEBAND VIDEO PROCESSOR

TEA5040T Echivalent: TEA5040-ST

TEA5101A-ST Echivalent: TEA5101B-ST

TEA5101B-ST TV RGB AMPLIFIER

TEA5110-ST 2xPOS V-REG 5V

TEA5114A-ST 2xRGB SWITCH 20Mhz 16p

TEA5115-ST 5xRGB SWITCH 25MHz

TEA5116-ST TV RGB SWITCH

TEA5170-ST SMPS CNTRL TV

TEA5500-PHI CODED LOCK.CIRC. SEC.SYST

TEA5501-PHI CODED LOCK.CIRC. SEC. SYS

TEA5550 AM CAR RADIO RECEIVER 16p

TEA5560 FM/IF SYSTEM 9p

TEA5570-PHI AM/FM RADIO RECEIVER 16p

TEA5580-PHI PLL ECON. STEREO-DECOD.16

TEA5581-PHI PLL STEREO DEC.+SOURCE SE

TEA5591-PHI AM/FM RADIO Ucc=1,8..15V

TEA5591A-PHI AM/FM RADIO RECEIVER

TEA5620-ST TV PAL DECODER

TEA5630-ST TV SECAM DECODER

TEA5640C-ST Echivalent TEA5640E-ST

TEA5640E-ST PAL/SECAM/NTSC DECODER

TEA5640F-ST

TEA5652-ST WIDEBAND VIDEO PROCESSOR

TEA5701-ST Echivalent 209.73594

TEA6000 FM/IF SYSTEM+MICROPRC.18p

TEA6100-PHI FM uCOMP TUNING INTERFACE

TEA6101-PHI ANTENNA DIVERSITY IC

TEA6200-PHI AM UPCONV.RADIO 10.7MHz I

TEA6300-PHI CAR RADIO PREAMP. 28p

TEA6310T-PHI HIFI AUD.PROC.(FADER) I2C

TEA6320-PHI SOUND FADER CNTRL CIRCUIT

TEA6330T-PHI SOUND FADER CONTROL

TEA6360-PHI 5-BAND STEREO EQUALIZER

TEA6414A-ST

TEA6415 Echivalent: TEA6415B

TEA6415B Echivalent TEA6415C-ST

TEA6415C-ST VIDEO MATRIX SWITCH I2C

TEA6417-ST VIDEO MATRIX SWITCH I2C-B

TEA6420-ST MATRIX AMPL/PRE-AMPL

TEA6422-ST AUDIO MATRIX I2C-BUS

TEA7531DP-ST SPEAKERPHONE CIRCUIT

TEA7532DP-ST TELELPHONE MONITOR AMPL.

TEA8170-TFK Echivalent: TDA8170-ST

TEA8172-TFK Echivalent: TDA8172-ST

TFA1001W

TFMS1380-TFK Echivalent TSOP1138-TFK

TFMS3300-TFK Echivalent: TFMS5300-TFK

TFMS4300-TFK Echivalent: TFMS5300-TFK

TFMS4360-TFK Echivalent: TFMS5360

TFMS5300-TFK Echivalent: TSOP1730-TFK

TFMS5360 PHOTO MOD. 36kHz VERTICAL

TFMT5300-TFK Echivalent: TSOP1730TB1

THD200FI-ST SI-N 1500V 10A 60W

TIC106D-MBR Echivalent: TIC106M-MBR

TIC106M-MBR THYR. 600V 5A 0.2mA 2uS

TIC106N-MBR THYR. 800V 5A 0.2mA 2uS

TIC116D-MBR THYR. 400V 8A 20mA 1/11uS

TIC116M-MBR THYR. 600V 8A 20mA 1/11uS

TIC116N-MBR THYR. 800V 8A 20mA 1/11uS

TIC126D-MBR THYR. 400V 12A 20mA 1us

TIC126M-MBR THYR. 600V 12A 20mA 1uS

TIC126N-MBR THYR. 800V 12A 20mA 1uS

TIC201M-MBR TRIAC 600V 2.5A 5/25mA

TIC206D-MBR TRIAC 400V 4A 5/10mA

TIC206M-MBR TRIAC 600V 4A 5/10mA

TIC216D Echivalent: TIC216M-MBR

TIC216M-MBR TRIAC 600V 6A 5/10mA

TIC225D-MBR TRIAC 400V 8A 5/30mA

TIC225M-MBR TRIAC 600V 8A Ih<30mA

TIC226D-MBR TRIAC 400V 8A 50mA

TIC226M-MBR TRIAC 600V 8A 50mA

TIC226N-MBR TRIAC 800V 8A 50mA

TIC236D-MBR Echivalent: TIC236M-MBR

TIC236M-MBR TRIAC 600V 12A 50mA

TIC236N-MBR TRIAC 800V 12A 50mA

TIC246D-MBR Echivalent TIC246M-TI

TIC246M-MBR TRIAC 600V 16A 50mA

TIC253D-MBR Echivalent: TIC253M-TI

TIC253M-MBR TRIAC 600V 20A 50mA

TIC263D-MBR Echivalent: TIC263M-MBR

TIC263D/DIV Echivalent: TIC263M-MBR

TIC263M-MBR TRIAC 600V 25A 50mA

TIC263M/DIV Echivalent: TIC263M-MBR

TIC45 Echivalent: 2N5064-PHI

TIC47-MBR Echivalent: 2N5064-PHI

TICP106D-MBR THYR. 400V 2A 5mA

TIL111 OPTOKOUPLER 30V 8% 5/5us

TIL112 OPTOKOUPLER 20V 2% 2/2us

TIL113 OPTO KOUPLER

TIL116 Echivalent: 4N25

TIL117 Echivalent: CNY17-3

TIL118 Echivalent: 4N27

TIL119 Echivalent: 4N33

TIL124 Echivalent: CNY17-3

TIL125 Echivalent: CNY17-3

TIL127 OPTKOUPLER 30V 1000% 0.3m

TIL306 SEGMENT LED DISPLAY

TIL309 SEVEN SEGMENT LED DISPLAY

TIL311

TIP100 Echivalent: TIP102-MBR

TIP102-MBR N-DARL 100V 8A 80W

TIP102/DIV N-DARL 100V 8A 80W

TIP105 Echivalent: TIP107-MBR

TIP106 Echivalent: TIP107-MBR

TIP107-MBR P-DARL 100V 15A 80W

TIP107/DIV P-DARL 100V 15A 80W

TIP110 Echivalent: TIP112-MBR

TIP111 Echivalent: TIP112-MBR

TIP112-MBR N-DARL 100V 2A 50W

TIP112/DIV N-DARL 100V 2A 50W

TIP115 Echivalent: TIP117-MBR

TIP116 Echivalent: TIP117-MBR

TIP117-MBR P-DARL 100V 2A 50W

TIP117/DIV P-DARL 100V 2A 50W

TIP120 Echivalent: TIP122-MBR

TIP121 Echivalent: TIP122-MBR

TIP122-MBR N-DARL 100V 5A 65W

TIP122/DIV N-DARL 100V 5A 65W

TIP125 Echivalent: TIP127-MBR

TIP126 Echivalent: TIP127-MBR

TIP127-MBR P-DARL 100V 5A 65W

TIP127/DIV P-DARL 100V 5A 65W

TIP130 Echivalent: TIP132-MBR

TIP131 Echivalent: TIP132-MBR

TIP132-MBR N-DARL 100V 8A 70W

TIP135 Echivalent: TIP137-MBR

TIP136 Echivalent: TIP137-MBR

TIP137-MBR P-DARL 100V 8A 70W

TIP140 Echivalent: TIP142-MBR

TIP141 Echivalent: TIP142-MBR

TIP142-MBR N-DARL 100V 10A 125W

TIP142T-MBR N-DARL 100V 10A 80W

TIP145 Echivalent: TIP147-MBR

TIP146 Echivalent: TIP147-MBR

TIP147-MBR P-DARL+D 100V 10A 125W

TIP150 Echivalent: TIP152-MBR

TIP151 Echivalent: TIP152-MBR

TIP152-MBR N-DARL+D 400/400V 7A 80W

TIP160 Echivalent: TIP162

TIP162 N-DARL 380V 10A 3W

TIP29 Echivalent: BD239C-MBR

TIP2955-MBR SI-P 100V 15A 90W NF/S-L

TIP29A Echivalent: BD239C-MBR

TIP29B Echivalent: BD239C-MBR

TIP29C Echivalent: BD239C-MBR

TIP29E-MBR SI-N 180V 2A 30W >3MHz=BD

TIP30 Echivalent: BD240C-MBR

TIP3055-MBR SI-N 100V 15A 90W NF/S-L

TIP3055/DIV SI-N 100V 15A 90W NF/S-L

TIP30A Echivalent: BD240C-MBR

TIP30B Echivalent: BD240C-MBR

TIP30C Echivalent: BD240C-MBR

TIP31A Echivalent: BD241C-MBR

TIP31B Echivalent: BD241C-MBR

TIP31C Echivalent: BD241C-MBR

TIP32 Echivalent: BD242C-MBR

TIP32A Echivalent: BD242C-MBR

TIP32B Echivalent: BD242C-MBR

TIP32C Echivalent: BD242C-MBR

TIP33A Echivalent TIP33C-MBR

TIP33B Echivalent: TIP33C-MBR

TIP33C-MBR SI-N 115V 10A 80W

TIP34A Echivalent: TIP34C-MBR

TIP34B Echivalent: TIP34C-MBR

TIP34C-MBR SI-P 100V 10A 80W 3MHz

TIP35C-MBR SI-N 100V 25A 125W 3MHz

TIP36A Echivalent: TIP36C-MBR

TIP36B Echivalent: TIP36C-MBR

TIP36C-MBR SI-P 100V 25A 125W 3MHz

TIP36C/DIV SI-P 100V 25A 125W 3MHZ

TIP41A Echivalent: TIP41C-MBR

TIP41B Echivalent: TIP41C-MBR

TIP41C-MBR SI-N 100V 6A 65W 3MHz

TIP41C/DIV SI-N 100V 6A 65W 3MHz

TIP42 Echivalent TIP42C-MBR

TIP42A Echivalent: TIP42C-MBR

TIP42B Echivalent: TIP42C-MBR

TIP42C-MBR SI-P 140V 6A 65W

TIP42C/DIV SI-P 140V 6A 65W

TIP47 Echivalent: TIP50-MBR

TIP48 Echivalent: TIP50-MBR

TIP49 Echivalent: TIP50-MBR

TIP50-MBR SI-N 400V 1A 40W 2us

TIP51 Echivalent: TIP54-MBR

TIP54-MBR SI-N 500V 3A 100W >2.5MHz

TIP55A Echivalent: BUW12A-MBR

TIP58A Echivalent: BUV48A-MBR

TIPL751A-MBR

TIPL760-MBR SI-N 850/400V 4A 75W

TIPL760A-MBR SI-N 100V 4A 80W 12MHz

TIPL761A-MBR SI-N 1000V 4A 100W

TIPL762A-MBR SI-N 800V 6A 120W POWER

TIPL762A/DIV SI-N 800V 6A 120W POWER

TIPL763-MBR Echivalent TIPL763A-TI

TIPL763A-MBR SI-N 1000V 8A 120W 8MHz

TIPL790A-MBR SI-N 150V 10A 70W 10MHz

TIPL791A-MBR SI-N 450V 4A 75W

TIS43 Echivalent: 2N2646-TO18

TK15062 TV/AV SWITCH FOR SAMSUNG

TL022CP Echivalent: LM358N-MBR

TL044CN-TI QUAD OPAMP

TL061CN-MBR SINGLE LOW POWER OPAMP

TL061CP Echivalent: TL061CN

TL062CD-MBR SMD DUAL LOW POWER OPAMP

TL062CN-MBR DUAL LOW POWER OPAMP 8p

TL064CN-MBR QUAD LOW POWER OPAMP 14p

TL064CN/DIV QUAD LOW POWER OPAMP

TL066CP-MBR QUAD LOW POWER OPAMP

TL071CD-MBR 1xOPAMP 5-18V 4MHz LOW-NO

TL071CN-MBR 1xOPAMP 5-18V 4MHz LOW-NO

TL071CP Echivalent: TL071CN-MBR

TL072CN-MBR DUAL LOW NOISE OPAMP 8p

TL074CN-MBR QUAD LOW NOISE OPAMP 14p

TL080CP SINGLE OPAMP SR 13V VI 10

TL081CN-MBR Echivalent: TL071CN-MBR

TL081CN/DIV Echivalent TL071CN-MBR

TL082CN-MBR Echivalent TL072CN-MBR

TL082CN/DIV Echivalent TL072CN-MBR

TL083CN-MBR Echivalent: TL083CN/DIV

TL083CN/DIV DUAL OPAMP

TL084CN Echivalent: TL074CN-MBR

TL16C450N-MBR ACIA CMOS =82C50

TL170 Echivalent: DN6838-MAT

TL272CP-TI Echivalent: TLC272CP-MBR

TL331CP-TI

TL430CLP-TI V-REG 2.75-30V 1-100MA

TL431CLP-MBR V-REG 2.5-36V 1-100mA=KA4

TL431CLP/DIV V-REG 2.5-36V 1-100mA

TL489 Echivalent: SN16889P-TI

TL494CN-MBR PWM CNTRL IC

TL495CN-MBR PWM CONTROL IC

TL497ACN-MBR SW-REG 35V 0.5A

TL507CP-TI ADC 7B

TL594CN-MBR SMPS CONTRL 300KHz 0.2A 1

TL710CN-TI Echivalent: UA710-14P

TL710CP-TI

TL7702ACP-MBR V-SUPPLY SUPERVISER 2V

TL7705ACP-MBR V-SUPPLY SUPERVISER 5V

TL7709ACP V-SUPPLY SUPERVISER 9V

TL7712ACP V-SUPPLY SUPERVISER 12V

TL7715ACP-MBR V-SUPPLY SUPERVISER 15V

TL783C-TI AP V-REG 1.25-125V 0.7A

TLBR5410 5mm ROT BLINKING

TLC2201ACP-TI LOW OFFSET/BIAS OPAMP

TLC271CP-MBR PROG.LOW POWER OPAMP 8p

TLC272CP-MBR CMOS DUAL OPAMP

TLC274CN-MBR CMOS QUAD OPAMP

TLC336B Echivalent: TLC336T-ST

TLC336D Echivalent: TLC336T-ST

TLC336T-ST TRIAC 600V 3A 5/5mA

TLC372CP-TI DUAL SINGLE-SUPPLY COMP.

TLC374CN-TI QUAD SINGLE SUPPLY COMP.

TLC549CP-TI ADC 8-B 25us + S&H

TLC555 Echivalent: NE555NCMOS-MBR

TLC555CP Echivalent: NE555NCMOS-MBR

TLC556 Echivalent: NE556NCMOS-MBR

TLC556CN Echivalent: NE556NCMOS-MBR

TLC7555CP Echivalent: NE555NCMOS-MBR

TLE4201A1-SIE

TLE4903F-SIE

TLFO3100

TLHR5400 Echivalent: LED5 R-TFK

TLN115 LED 5mm INFRARED

TLP3042 TRIAC DRIVER OPTOCOUPLER

TLP3061 OPTO KOUPLER =TLP3063

TLP3063 OPTO-TRIAC 600V 50MA 5MA

TLP504A OPTOISO 2* 2.5KV 30V 50%

TLP532 OPTOISO 2.5KV 55V >50% 3U

TLP541G OPTO-THYR. DR. 400V 70MA

TLP550 OPTOISO 2.5KV 5V 30% 0.3/

TLP581 NPN OUT/DC IN OPTOKOUPLER

TLP620 OPTOISO 5KV 55V >50% 3US

TLP621-1 Echivalent PC816

TLP621GRLF2 OPTOKOPPLER

TMM2016 Echivalent: 6116-100-MBR

TMP47C1637N215 CPU 4B =TMP47C1637NU215

TMP47C200N2572 CPU 4B 128 RAM + ROM

TMP47C200N2576 CPU 4B 128 RAM + ROM

TMP47C400N6219 CPU 4B 256 RAM + ROM

TMP47C432-8188 Echivalent: 310.31849

TMP47C432-8189 Echivalent: 310.31851

TMP47C432N8188 Echivalent: 310.31849

TMP47C432N8189 Echivalent: 310.31851

TMP47C432P8187 Echivalent: 310.31849

TMP47C432P8188 Echivalent: 310.31849

TMP47C432P8189 Echivalent: 310.31851

TMP47C433N3848

TMP47C434N3122 Echivalent: 209.32171

TMP47C434N3142 Echivalent: 209.32118

TMP47C434N3146

TMP47C434N3414 Echivalent: 310.31947

TMP47C434N3415 Echivalent: 310.31982

TMP47C434N3526 CPU 4B ROM+RAM

TMP47C434N3537 Echivalent: 310.31947

TMP47C434N3548 Echivalent: 209.60229

TMP47C434N3555 Echivalent: 310.31848

TMP47C434NR214 CPU 4B WITH ROMCODE FUNAI

TMP47C434NR217 Echivalent: 209.32171

TMP47C434NR221 Echivalent 310.31982

TMP47C434NR231 CPU 4B WITH ROMCODE

TMP47C434NR234 Echivalent: 209.33286

TMP47C634N2412 Echivalent: 310.31927

TMP47C634N2414 Echivalent: 310.31927

TMP47C634N2415 Echivalent: 310.31927

TMP47C634N2416 Echivalent: 310.31927

TMP47C634N2474 Echivalent: 310.31927

TMP47C634N2475 Echivalent: 310.31927

TMP47C634N2476 Echivalent: 310.31927

TMP47C634N2675 Echivalent: 310.31928

TMP47C634N2676 Echivalent: 310.31928

TMP47C634N2691 CPU 4B WITH ROM

TMP47C634N2692 CPU 4B WITH ROM

TMP47C634N2697 CPU 4B WITH ROM

TMP47C634NR331 CPU 4B WITH ROM

TMP47C634NR461 CPU 4B WITH ROMCODE

TMP47C634NR491 CPU 4B WITH ROMCODE

TMP47C634NR514 CPU 4B WITH ROM

TMP47C834NR021 CPU 4-BIT 256B RAM + ROM

TMP47C834NR067 CPU 4-BIT 256B RAM + ROM

TMP47C834NR071 CPU 4-BIT 256B RAM + ROM

TMP47C834NR165 ALSO: 209.33275

TMP47C837NU411 CPU 4B 256B RAM + ROM

TMP47C837NU412 CPU 4B 256B RAM + ROM

TMP8035 Echivalent: 8035DIP-MBR

TMS1000-096 Echivalent: 209.10153

TMS1000-117 Echivalent: 209.80488

TMS1000-233 TMS1000NLPMP0233

TMS1000NLPM TMS1000NLP MP0186 =209...

TMS1024NLL-TI I/O EXPANDER

TMS1025

TMS1943N2L-TI

TMS1944 4 BIT UPC

TMS1965

TMS3450 Echivalent LM8560-SAN

TMS3477NL-TI UPC SPEECH DEVICE

TMS3613 Echivalent: 209.80542

TMS36131 Echivalent: 209.80542

TMS3701BN-TI D/A CONVERTER

TMS3712

TMS3748

TMS3753NL-TI

TMS3755DNL-TI

TMS3757 Echivalent: 209.82419

TMS3763ANL28 Echivalent: 209.82385

TMS3763CNL-TI

TMS4256-12NL Echivalent: 41256-100-MBR

TMS9129NL-TI NON-VGA VIDEO CONTROLER

TMS9929A Echivalent: 209.11038

TOP201YAI OFF-LINE PWM SW 20-45W

TOP202YAI OFF-LINE PWM SW 30-60W

TOP204YAI OFF-LINE PWM SW 60-100W

TOP210PFI OFF LINE PWM SW 8W

TP3057N-NSC A-LAW CODEC FILTER

TP5088N-NSC DIALER CIRCUIT

TP5089 Echivalent: UM95089

TPU2732-ITT

TPU2732S-ITT TXT PROC. UNIT SCANDINAVI

TPU2734-ITT

TPU2735DIP-ITT TXT PROCESSOR

TS082010-ST Echivalent: P0102DA-ST

TS082060-ST Echivalent: P0201MA-ST

TS271 Echivalent: TLC271CP-MBR

TS272 Echivalent: TLC272CP-MBR

TS274 Echivalent: TLC274CN-MBR

TS555IN Echivalent: NE555NCMOS-MBR

TS556CN Echivalent: NE556NCMOS-MBR

TSA5511-PHI FREQ.SYNTH.1.3GHz I2C-bus

TSA5511AT-PHI FREQ.SYNTH.1.3GHz I2C-BUS

TSA5511T-PHI FREQ.SYNTH. 1.3GHz I2C-BU

TSA5512-PHI PLL BI-DIREC.SYNT. 1.3 GH

TSA5512AT-PHI PLL BI-DIREC.SYNT. 1.3 GH

TSA5512T-PHI PLL BI-DIREC.SYNT. 1.3 GH

TSA5514-PHI PLL BIDIR. SYNTHES. 1.3GH

TSA5514AT-PHI PLL BIDIR. SYNTHES. 1.3GH

TSA6057-PHI PLL FREQ SYNTH. IIC BUS

TSC7106CPL Echivalent: ICL7106CPL-MBR

TSC7106RCPL Echivalent: ICL7106RCPL-MBR

TSC7126CPL Echivalent: ICL7126CPL-MBR

TSC7136 Echivalent: ICL7136CPL-MBR

TSOP1138-TFK PHOTO MOD 38kHz 2.4KBIT

TSOP1730-TFK PHOTO MOD. 30kHz VERTICAL

TSOP1730TB1 PHOTO MOD. 30kHz HORIZONT

TUA1574-SIE FM TUNER CIRCUIT

TUA2000-4-SIE TV VHF TUNER <=400MHz

TUA2007X Echivalent: TDA5330T-PHI

TUA2017X Echivalent: TDA5331T-PHI

TUA2019-5X-SIE VHFI/VHFII/UHF TUNER IC

TV13S SI-D 13KV 7.4mA/1.5Ap

TV18S SI-D 18KV 4.8mA/1.5Ap

TV20D2-AEG SI-D 20KV 5.7mA

TV20S SI-D 20KV 5.7mA

TY40163 I.C.

TY45018 Echivalent: MC44130P-MOT

TYN1004 Echivalent: TYN1012

TYN1006 Echivalent: TYN1012

TYN1010 Echivalent: TYN1012

TYN1012 THYR. 1000V 12A <15/30mA

TYN408 Echivalent: TIC126D-MBR

TYN412 Echivalent: TIC126D-MBR

TYN416 Echivalent: TIC126N-MBR

TYN608-ST Echivalent: TIC116M-MBR

TYN610 Echivalent: TIC126M-MBR

TYN612-ST Echivalent: TIC126M-MBR

TYN616-ST THYR. 600V 16A 25mA 2/70u

TYN685 Echivalent: TYN688-ST

TYN688-ST THYR. 400V 20A 25mA 2/70u

TYN690-ST THYR. 600V 20A 25mA 2/70u

TYN808 Echivalent: BT151/800R-PHI

TYN812-ST Echivalent: BT151/800R-PHI

TYN825 Echivalent: 2N6509-MOT

TZ D-072 Senzor Temperatura 72C.

TZ D-077 Senzor Temperatura 77C.

TZ D-084 Senzor Temperatura 84C.

TZ D-087 Senzor Temperatura 87C.

TZ D-091 Echivalent TZ D-093

TZ D-093 Senzor Temperatura 93C.

TZ D-098 Senzor Temperatura 98C.

TZ D-100 Senzor Temperatura 100C.

TZ D-104 Senzor Temperatura 104C.

TZ D-109 Senzor Temperatura 109C.

TZ D-117 Senzor Temperatura 117C.

TZ D-121 Senzor Temperatura 121C.

TZ D-128 Senzor Temperatura 128C.

TZ D-141 Senzor Temperatura 141C.

TZ D-152 Senzor Temperatura 152C.

TZ D-167 Senzor Temperatura 167C.

TZ D-184 Senzor Temperatura 184C.

TZ D-192 Senzor Temperatura 192C.

TZ D-216 Senzor Temperatura 216C.

TZ D-228 Senzor Temperatura 228C.

TZ D-240 Senzor Temperatura 240C.