http://www.tubes.ru/techinfo/AmateurRadio/gu-81m.html

GU-81M

PENTODE

The GU-81M pentode is used in self-excited oscillation and power amplification circuits of RF equipment.

GENERAL
Cathode: directly heated, carbonized thoriated tungsten.
Envelope: glass, with base.
Height: at most 260 mm.
Diameter: at most 202 mm.
Mass: at most 1 kg.

1, 2 - cathode; 3 - grid 1; 4 - grid 2; 5 - cathode centre tap; 6, C3 - grid 3; A - anode

| OPERATING ENVIRONMENTAL CONDITIONS |  |
| --- | --- |
| Ambient temperature, °C | -10 to +55 |
| Relative humidity at up to +25 °C, % | 98 |

| BASIC DATA Electrical Parameters |  |
| --- | --- |
| Filament voltage V | 12.6 |
| Filament current, A, at most | 11 |
| Mutual conductance (at anode voltage 2 kV, grid 2 voltage 600 V, anode current 200 mA), mA/V | 4.5-6.5 |
| Gain coefficient (grid 1 -grid 2) (at anode voltage 2 kV, grid 2 voltages 600 and 500 V, anode current 200 mA) | 2.5-4 |
| Bias voltage (at anode voltage 2 kV, grid 2 voltage 600 V), V | 116-160 |
| Interelectrode capacitance, pF: |  |
| input | 25-32 |
| output | 21-26 |
| grid 1-anode, at most | 0.1 |
| grid 1-grid 3 | 1-4 |
| Output power (at anode voltage 2 kV, grid 2 voltage 600 V, bias voltage -200 V, grid 1 drive voltage amplitude 300 V, anode current, at least 450 mA, grid 1 current at most 20 mA, grid 2 current, at most 220 mA), W, at least | 700 |

| Limit Operating Values |  |
| --- | --- |
| Filament voltage, V | 11.6-13.4 |
| Anode voltage, V: |  |
| at frequencies not above 6 MHz | 3 |
| at frequencies not above 24 MHz | 2.5 |
| at frequencies not above 50 MHz | 1.5 |
| Grid 2 voltage, V | 600 |
| Anode current (average value), A | 0.6 |
| Grid 1 current (average value), A | 0.02 |
| Grid 2 current (average value), A | 0.2 |
| Dissipation, W: |  |
| anode | 450 |
| anode (momentary dissipation) | 600 |
| grid 2 | 120 |
| grid 1 | 10 |
| Envelope temperature, °C | 350 |

Averaged Anode Characteristic Curves:
Uf = 12.6V; Ug2 = 0.5kV;
— • — (Pa max)

Averaged Grid 2-Anode Characteristic Curves:
Uf = 12.6V; Ug2 = 0.5kV

Averaged Characteristic Curves:
Uf = 12.6V; Ug2 = 0.6kV

Averaged Grid-Anode Characteristic Curves:
Uf = 12.6V; Ug2 = 0.5kV

Averaged Grid-Anode Characteristic Curves:
Uf = 12.6V; Ug2 = 0.6kV

Averaged Characteristic Curves:
Uf = 12.6V; Ua = 2.5kV;
_____ anode-grid;
_ _ _ grid 2

Main Menu | All Types | Contact Us | Order Form | Warranty | Quality | Company Profile