DOCUMENTATION FILE FOR CWIT MORSE PROGRAM

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Version 1.00      3 October 1993.

(C) David Fraser ZL3AI 1993.

CWIT is a morse  training  program  designed  to be  used  by

students with little previous computer experience or keyboard

skills, and no previous morse knowledge.

It  has been designed to be interesting and fun to use, as an

alternative  or  supplement  to existing ways to learn Morse.

Acknowledgements.

~~~~~~~~~~~~~~~~~  The following  have  been  influential  or

helpful in the development of this program:

My wife Ann ZL3TNT and son Abed, for initial impetus.

Bob ZL2CA for clarifying and defining objectives.

Tony ZL3UAC for thoroughly evaluating unusual concepts.

Vee ZL3TVS and Caroline for encouraging user feedback.

Jesse and Peter Baker for extensive software trials.

NZART Branch 68 (North Canterbury) members in general.

Margaret Mills' Morse booklet.

The Morseman ZL1AN's series of articles in Break-In magazine.

Alan ZL1AMW and contributors for the use of Data News text.

Introduction.

~~~~~~~~~~~~~  Like  Hamdisk,  CWIT  (pronounced QUIT)  arose

from the efforts of other members  of  my  family  (wife  Ann

ZL3TNT and 10 year old son Abed who passed the September 1993

Amateur Exam) to acquire more Amateur  Radio  qualifications.

Much reading and listening to comments  about  various  morse

training  schemes  was  done,  particularly  Margaret  Mills'

course,  the Morseman articles in the Break-In magazine,  and

the members of  the  North  Canterbury  Amateur  Radio  Club.

Release.

~~~~~~~~ This is a shareware program and may be freely copied

evaluated and used.  Donations will not be refused.

Farnsworth Speed.

~~~~~~~~~~~~~~~~~ The Farnsworth speed is 14 wpm, which means

that at speeds below 14 wpm the individual letters  are  sent

at a 14 wpm speed, but  the  gaps  between  the  letters  are

lengthened appropriately.

Multiple users.

~~~~~~~~~~~~~~~ CWIT supports multiple users.   The user name

should follow the "CWIT" on the command line, eg "CWIT John",

or "CWIT Ann", etc. This enables  very  easy  interfacing  to

your existing menu system, and CWIT will  save  and  retrieve

each user's parameters.  Also, now  that  the  program  knows

your name, it can be a bit more friendly.

Options.

~~~~~~~~ The morse speed, audio frequency, game sound effects

and which serial port (if any) to  be  used  are  set  up  by

pressing the F2 function  key  from  the  main  menu.   These

options are saved to a file which is the name of the user, so

various users can have their own configuration.   If you wish

to listen  to  morse  without  disturbing  the  rest  of  the

household,  try selecting an audio frequency of say 50 Hz and

see what that sounds like.  Generally it sounds like a rather

quiet buzz,  which,  although not perfect,  may enable you to

practice when  the  normal  800 hz  (or  whatever)  would  be

objectionable.   For the same reason, the game sound  effects

can be turned on or off.

Serial port.

~~~~~~~~~~~~ The serial port to be used for  hand  key  input

and / or  output  to  a  transmitter  is  set  up  using  the

"F2-Configure"  function.   You may select any serial port up

to COM 8.  If you enter zero, no port will be used.

A hand key may be connected to a Serial port so that you  can

practice sending morse with a hand key.  You  should  aim  to

make your morse sound the same as that which the  computer's,

and also a graphical representation of your  keying  will  be

displayed on the screen.  Ideally the dashes should be  three

times the length of the dots, and the gaps between  the  dots

or dashes making up a single letter should be a  dot  length.

Between letters there should be a gap equal in  length  to  a

dash.

There are two outputs: the actual morse keying itself,  and a

Tx/Rx output to switch from Tx to Rx.  There is  a  delay  of

one second between  Tx  switch  on  and  the  keying  output.

Wiring connections are identical to BayCom modem connections.

Function          9 pin       25 pin     Quiescent    Active

~~~~~~~~          ~~~~~       ~~~~~~     ~~~~~~~~~    ~~~~~~

Morse output      pin 4       pin 20       -12v        +12v

Tx/Rx output      pin 7       pin 4        -12v        +12v

Signal ground     pin 5       pin 7

Hand key, between 7 & 8       4 & 5

Serial ports 1 and 2  have  been  tested,  but  it  has  been

difficult to get access to a computer with 3 or  more  ports,

so these have not been tested in actual practice.   They  are

set up correctly (in theory), so if anyone uses them I  would

be interested to know if they do in fact  operate  correctly.

Elapsed time.

~~~~~~~~~~~~~ The elapsed time in various  menu  options  (if

available) is displayed  in  the  top  right  corner  of  the

screen.  Note that when in the main menu, the time  displayed

is the total elapsed  time.   You  may  find  it  helpful  to

allocate a certain amount of time for each session.

Screen saver.

~~~~~~~~~~~~~ If the program is left running in  a  menu  for

more than three minutes the screen is blanked and is replaced

by a picture of a bouncing Cwit.  This  is  done  to  prevent

'burning' of the screen, and to snap the user out of it.

Feedback.

~~~~~~~~~ I am very interested to hear good or  bad  comments

about the program, so  don't  feel  hesitant.   I  am  not  a

regular user or teacher of morse code so I  fully  expect  to

have  to further refine it,  especially if users find it dull

or boring to use!

----------------//----------------

David Fraser       Phone (O3) 314-6825

PO Box 48

Waipara 827O       Packet radio ZL3AI @ ZL3AC.#80.CHC.NZL.OC

New Zealand.