MDMTEST1.EXE and MDMTEST2.EXE are simple programs written to help you

with tuning the RXB of your TCM3105 based modem. I found this voltage

to be critical for modem's performace and how -cd option of ax25.com

driver works.

Bad RXB causes the output of the TCM3105 to be asymetric.

Suppose you receive a FSK signal modulated with square wave

like: 010101010101010101... With proper RXB you get square wave

with 50% duty cycle out of pin 8 of TCM3105. With bad RXB

positive pulses are of different length then negative ones.

mdmtest1.exe takes modem's output and measures the lengths

of positive and negative pulses. Then it computes the deviation

from full bit periods (for 1200 bps the bit period is 1sek/1200 = 0.83 ms)

For synchronous transmition the pulse width should be a multiply

of bit period. The program displays the average deviation

from this rule. Ideally the deviation should be zero when a clean packet

signal is being received. See it yourself...

mdmtest2.exe does similar thing but it takes the period between

modem's output transitions of same polarity. This value is used

in improved algorithm for -cd option and it should in principle

be insensitive to small RXB variation. But again: see it yourself...

Pawel Jalocha, SP9VRC