��������������������������������������������ͻ

�      TEEBAL version 1.50, 5 January 1996   �

�                                            �

�         Gary E.J. Bold, ZL1AN @ ZL1AB      �

�          Phone (09) 483 7240 (Home)        �

�                (09) 373 7599 (Work)        �

�                (09) 373 7445 (FAX)         �

��������������������������������������������ͼ

TEEBAL designs and/or investigates high-pass TEE transmatches.  Matches are

computed with respect to 50 ohms.   Reactive loads may be specified.

Inductor losses are included, and power lost in the inductor is calculated.

Both capacitor and inductor values can be varied interactively while

viewing SWR, and the input impedance of the transmatch shown as series

resistance plus reactance.

1. INITIALISATION:

TEEBAL may be initialised in one of two ways.

1.1 If "Automatic" mode is initially selected, you'll be prompted for

- a load resistance in ohms,

- a load reactance in ohms,  (+ve inductive, -ve capacitive)

- a characteristic impedance in ohms,

- a frequency in MHz,

- an inductor Q value.

A transmatch will be designed, using the lossless design equations,

approximately satisfying these parameters.   See later for

explanations, and what you can do next.   The operating screen will

then appear.

1.2 If "Manual mode" is initially selected, you'll be prompted for

parameter values

- C1 and C2 in pF,

- L in uH,

- Inductor Q,

- load resistance in ohms,

- load  reactance in ohms,  (+ve inductive, -ve capacitive)

- frequency in MHz.

If <enter> is given for C1 and/or C2 values, 100 pF will be used.

If <enter> is given for Inductor Q, a million (infinite) will be used.

The operating screen will then appear.

2. THE OPERATING SCREEN.

Appearance:  At the right, the function of various keys is explained.

left/right arrows:   Vary the value of C2 down and up by deltaC.

up/down arrows:   Vary the value of C1 down and up by deltaC.

f3:   Set deltaC to 0.1 pF

f4:   Set deltaC to 1 pF

f5/f6:   Vary the coil inductance down/up in 0.1 uH steps.

f7/f8:   Vary the load resistance down/up in 1 ohm steps.

Z,X:   Vary the load reactance  down/up in 1 ohm steps.

f10:   Show characteristic impedance,percentage of input

power lost in inductor, voltages across capacitors,

minimum possible value of load reactance.

ESC:   Stop the program.

END:   Re-start the program immediately.

f1:   Show this help screen.

Above this, the selected load resistance, reactance, and operating

frequency in MHz are shown.

At the top left of the screen, the current values of L, C1 and C2 (input

and output capacitors) are shown.   These are immediately updated in

response to the arrow keys as indicated above.   Below, the current SWR

(standing wave ratio with respect to 50 ohms) and current input impedance

of the transmatch are shown.   Resistive and reactive components are shown

separately.   POSITIVE and NEGATIVE reactance are INDUCTIVE and CAPACITIVE,

respectively.

If a transmatch is designed by TEEBAL itself, (first initialisation option)

AND the Q of the inductor is FINITE, the calculated parameters will NOT

give an EXACT match (There are no simple design equations which take

account of inductor losses).   However, the match will be close, and the

"true" matching values of the two capacitors can be found by empirical

adjustment with the arrow keys.   Near the matching point, you'll find that

Changing C1 predominantly varies the REACTIVE component,

Changing C2 predominantly varies the RESISTIVE component

of the input impedance.   The match can be adjusted by monitoring EITHER

the input resistance and reactance (easier when far away from a match) or

the SWR (easiest when close to a match).   Even when changing the

capacitors in steps of 0.1 pF (after pressing f3), an "exact" match

cannot generally be obtained to the 4 significant figures of SWR

displayed.   However no SWR meter can display to this accuracy anyway,

and a match to 2 decimal places is close enough.

When a close match has been obtained (or at any time) pressing f10 shows

the percentage of power currently lost in the transmatch; thus,

4% means 4 watts dissipated if 100 watts is input,

the current characteristic impedance of the transmatch, as;

- a NORMALISED value (wrt 1 ohm, design uses this)

- an UN-NORMALISED value wrt 50 ohms, the actual value used.

The rms voltage developed across C1 and C2 for 100 watts input;

The mimumum (most negative) permitted value of load reactance for

this value of characteristic impedance.

If the input power is NOT 100 watts, the voltages shown across each

capacitor, Vc, must be scaled by Vc * sqr(P/100), where

Vc is the voltage shown on the screen,

sqr() is the square root operator,

P is the actual power.

Thus, if the power input is 400 watts, the capacitor voltages shown must be

doubled.

3. DETAILS OF SPECIFYING THE DESIGN.

For initial design, choose the option that lets TEEBAL design the

transmatch for you.   You then have to specify only the characteristic

impedance (R0), the coil Q, the load impedance and operating frequency.

If the load is resistive, or has a POSITIVE reactive component, you must

specify a characteristic impedance which is LARGER than the larger of 50

ohms OR the load resistance.

If the load has a NEGATIVE reactive component, you will have to specify

an even larger value than this - the output capacitor has to be able to

compensate.   If your entered value is too small, the minimum acceptable

value will be shown, and you'll be prompted for another.

To get the ``equal reactance'' design (capacitors identical) for a purely

resistive load, choose the value of the characteristic impedance as the

load resistance plus 50 ohms.

For ``reasonable'' capacitor values, and to give some adjustment leeway,

specify a characteristic impedance somewhat greater the minimum.

Increasing R0 DECREASES the value of capcitors required, but INCREASES

transmatch losses for a given Q.

4. SPECIFYING AND CHANGING THE LOAD IMPEDANCE.

The load is specified by its SERIES equivalent circuit, that is, a

resistance in series with a reactance.  INDUCTIVE reactance is positive,

CAPACITIVE reactance is negative.   Resistance and reactance are entered

separately.

Either can be subsequently varied.

Load RESISTANCE can be increased/decreased with f7/f8.

Load REACTANCE  can be increased/decreased with Z/X.

If the REACTANCE is changed, the value of C2, the output capacitance,

is automatically changed to compensate, and immediately reflected in the

new value shown on the screen.

making the reactance more POSITIVE (inductive),  DECREASES C2.

making the reactance more NEGATIVE (capacitive), INCREASES C2.

In either case, the MATCH DOES NOT CHANGE - if the tuner is matched, it

will stay matched.  Note that there is a MAXIMUM NEGATIVE value for the

reactance (shown when f10 is pressed) which cannot be exceeded.  At this

value, the output capacitance is infinite, and if it goes more negative

than this, an INDUCTOR would be required.  TEEBAL does not cope with this,

and you wouldn't want it either.

Thus, if the load specified has a more negative reactance than this

critical value, a LARGER value of R0 (characteristic impedance) must be

selected (you'll have to re-start the program) or you can alternatively

increase the inductor value using f6 at this point to increases R0.

If the load RESISTANCE is changed, the bridge WILL become unbalanced.  The

changed situation is not shown until either capacitance is subsequently

changed with the arrow keys.   The bridge must now be re-balanced.

5. THEORY.

The match is computed by considering the TEE match as two L matching

units back-to-back.   The input L is matched to an output "characteristic

impedance", R0, which can be thought of as a "virtual resistance" seen at

the centre of the transmatch.  The output L is matched to a source having

R0 as its resistance.   The two resulting L inductors are combined in

parallel to give the overall inductance required in the transmatch.

For resistive loads, theory shows that R0 MUST be larger than the larger

of the load resistance OR the system source resistance (here 50 ohm).

Selecting R0 close to (but greater than) the larger of these will always

give LOWER transmatch losses, but LARGER capacitor values, and LOWER

voltages across them.

A LARGER R0 gives LARGER values of inductance and SMALLER capacitors, but

with HIGHER voltage across them.

The design equations are:

XC1 = sqr(50*(R0 - 50))     - reactance of input capacitor

XC2 = sqr(RL*(R0 - RL))     - reactance of output capacitor

XL1 = (50*R0)/XC1

XL2 = (RL*R0)/XC2

XL  = XL1 // XL2            - reactance of inductance.

These equations don't account for finite inductor Q, the effect of which

is modelled as a resistance in series with the inductor.   A closer match

can always be found by adjusting the capacitor values.   However, if R0 is

too close to its limiting (lower) value, an exact match may not be

possible.   Increase the inductance value and try again.

The power dissipated in the transmatch is found by performing series to

parallel transformations (using Bold's equations) on the inductor and its

series loss resistor, and the load and series output capacitance.   The

ratio of the resulting equivalent parallel resistances is equal to the

inverse ratio of the power dissipated in each.  The total power in both

is normalised to 100 watts.  This is exact, and not just an application

of the approximate expression

k = (sqr(R0/50 - 1) + sqr(R0/R2 - 1))/Q

The voltage across a capacitor is found using

Vc = I*Xc       where I is rms current through a capacitor of reactance Xc.

I1 (input capacitor current) is found by assuming that this causes 100

watts to be dissipated in 50 ohms.  I2 (output capacitor current)

assuming that this causes 100 watts to be dissipated in the load

resistance.   The relations are

VC1 = sqr(P*(R0 - 50))

VC2 = sqr(P*(R0 - R2))      where P = input power

6. TYPICAL PARAMETER VALUES:

"Receiving spaced" capacitors are adequate for "reasonable" loads for

powers up to 100 watts or so.   Assume that such capacitors break down

(arc) when greater than about 500 volt rms is developed across them.

Minimum values of such capacitors are typically 10 - 20 pF.

Maximum values of such capacitors are typically 100 - 350 pF.

On lower frequency bands, sections of multi-ganged capacitors can be

connected in parallel, when the capacitance values ADD.

Inductance values required typically vary from 10 uH (80 metres) to 1 uH

or less (10 metres).

A good quality (air-spaced, at least one coil diameter from metal)

inductor will have Q of up to 200.   About 100 is a good all-round value.

Toroidal inductors wound on cores may have Q's of 50 or less.

7. DESIGN EXAMPLE.

Start the progrm:   When asked if TEEBAL is to design the transmatch,

enter "y" or just <enter>.  Enter

load resistance          = 200 ohms

characteristic impedance = 1000 ohms

frequency, MHz           = 3.7

Inductor Q               = 100

The operating screen will immediately display these values:

L   = 6.765 uH

C1  = 197.4 pF

C2  = 107.5 pF

SWR = 1.0636

Adusting C1 and C2 will allow the SWR to be reduced to 1.0010, with

C1 = 199.2 pF

C2 = 104.7 pF

Pressing f10 then shows that 6.25% of the input power is lost in the

inductor, and the resulting characteristic impedance = 983 ohms.   This

is much larger than the load, so if we have extra capacitance capability,

we can try reducing this.

Lower L to 5.065 uH (f5).  Re-balance, and find

C1 = 289.9 pF

C2 = 172.2 pF

SWR = 1.0006

Pressing f10 again shows that 4.17% of power is lost, and the

characteristic impedance has dropped to 491 ohms.   If we have even more

capacitance, we could reduce it further by decreasing L again.